Curriculum of National Certificate Level-2 in Construction Machinery

Level-2 Equivalent to G-III

Curriculum

National Vocational and Technical Training Commission (NAVTTC)

Government of Pakistan

Contents

1.	Introduction	4

2.	Purpose of the qualification	5

3.	Overall objectives of training program	5

4.	Entry level of trainees	6

5.	Minimum qualification for teachers	6

6.	Duration of the course:	6

7.	Description and structure of the course	8

8.	Scheme of Studies.	9

9.	Detail of Modules	10

National Certificate Level-2 in Construction Machinery	10

9.1 Workshop Practice - I	10

9.1.1	Maintain Tools & Equipment.	10

9.1.2	Apply technique of Hand and Power Tools	12

9.1.3	Perform Metal/Bench Work	14

9.1.4	Perform cutting on Metal using Circular/Power Heck Saw	17

9.1.5	Perform grinding operations	19

9.1.6	Perform Tool & Cutter Grinding	22

9.1.7	Perform Basic Lathe Machine Operations	24

9.1.8	Perform Shaper, Planar and Slotter Machining Operations	32

9.1.9	Perform CNC Lathe Operations	36

9.1.10	Perform Pre-Welding Operations	40

9.1.11	Perform Shielded Metal Arc Welding (SMAW)/electric welding	42

9.1.12	Perform different welding joints	48

9.1.13	Perform Forging Operations	51

9.2 Engineering Drawing and CAD-1	53

9.2.1	Perform Basic Manual Drawing	53

9.2.2	Construct different Engineering Curves	59

9.2.3	Construct multi-view drawings	65

9.2.4	Install CAD software	70

9.2.5	Manage display in CAD software	73

9.2.6	Perform function of CAD general commands	76

9.2.7	Perform Different Cad operation	81

9.3 Occupational Health, Safety and Environment	86

9.3.1	Read and Develop Career Professionalism	86

9.3.2	Apply basic Occupational Health & Safety regarding heavy machinery	89

9.3.3	Perform general health, safety and environment practices	93

10.	List of Machinery, Tools & Equipment.	98

10.1. Workshop Practice-I	98

10.2. Basic Engineering Drawing and CAD/CAM	100

10.3. Occupational Health, Safety and Environment	100

11.	Members of the Curriculum Development Committee.	101

# Introduction

Construction Machinery Operators and Technicians are in demand across the country and abroad. Their services are required for everything from road and bridge construction, back hoe operation, bulldozing, loading and grading, dredging and heavy paving, to excavating and much, much more. This is a good career opportunity for a reliable and responsible individual with a strong work ethic. Heavy Machine Operators and Technicians not only work on regular construction building jobs, but also on infrastructure projects (roads, bridges, Dams and ports, otherwise called non-building construction), and in mining and timber operations.

In order to build the capacity of technical and vocational training institutes in Pakistan through provision of demand driven competencies-based trainings in construction sector the NAVTTC, and TEVT Sector Support Program (TSSP) have joined hands together to develop qualifications for construction sector. These qualifications will not only build the capacity of existing workers of this sector but also support the youth to acquire skills best fit for this sector. The benefits and impact of development of these qualifications will be on both demand and supply side.

Based upon this demand of industry these competency‐based qualifications for Construction Machinery are developed under National Vocational Qualification Framework (NVQF) (Level 2).  The qualifications mainly cover competencies along with related knowledge and professional skills which are essential for getting a job or self-employed.

The National Vocational & Technical Training Commission (NAVTTC) has approved the Qualification Development Committee (QDC). The QDC consists experts from the relevant industries from different geographical locations across Pakistan and academicians who were consulted during the development process to ensure input and ownership of all the stakeholders. The National Competency Standards could be used as a referral document for the development of curricula to be used by training institutions.

Construction Machinery curriculum will prepare students to efficiently operate heavy machines such as Dozers, Loaders, Excavator and Graders and to perform basic preventive maintenance on most types of heavy equipment. Coursework includes Safety, Heavy Machines operations, grades, Legal & environmental concerns & equipment maintenance. Graduates of this program may find employment with state and local government agencies and private contractors engaged in highway or other construction activities.

In fact, no sphere of life may be identified that does not include the contribution of construction machinery. Thus, the importance of construction machinery may be determined according to its usefulness in our daily life. Therefore, industry expectations for skilled workforce are also dynamic which can only be managed through setting relevant competency standards in collaboration with the leading industries. Being cognizant of this fact, National Vocational & Technical Training Commission (NAVTTC) developed competency standards for Mechanical technology with Specialization in Construction Machinery under National Vocational Qualifications Framework (NVQF). These competency standards have been developed by a Qualifications Development Committee (QDC) and validated by the Qualifications Validation Committee (QVC) having representation from the leading Construction, Real States & Builder industry of the country.

# Purpose of the qualification

The purpose of these qualifications is to set high professional standards for Construction Machinery. The specific objectives of developing these qualifications are as under:

- Improve the professional competence of the trainees

- Provide opportunities for recognition of skills attained through non-formal or informal pathways

- Improve the quality and effectiveness of training and assessment for Construction Machinery.

- Enable the existing workforce to capacitate themselves in new technologies and method so that train persons in this field can maintain construction machinery for its longer

# Overall objectives of training program

The Construction Machinery qualification of level 2 consists of both the theoretical and practical details and having the following Occupations

- Workshop Practice-1

- Engineering Drawing and CAD-I

- Occupational Health, Safety and Environment

# Entry level of trainees

The entry for National Certificate level 2, in Construction Machinery are

- A person having Matric certificate with science subjects

# Minimum qualification for teachers

- B. Tech in Mechanical Technology 4 Years

- B.Sc. / B.E. in Mechanical Engineering 4 Years

- D. A. E. in Mechanical Technology with 3 Years teaching experience

- Must be able to communicate effectively

Medium of instruction

English, and Urdu.

# Duration of the course:

The proposed Modules are composed of 23 competencies that will be covered in 600 Learning hours. Duration of the course is proposed to be Three years. The total weightage for technical competencies is 600 hours. A total of 200 hours has been reserved for allied subjects i.e. Islamic studies, English, Mathematics, Physics and Chemistry.

The details of technical Modules are given as under:

Level - 2 = 6 months.

Level - 3 = 6 months.

Level - 4 = 1 Year.

Level - 5 = 1 Year.

The distribution of contact hours and credit hours in level-2 is given below:

Level - 2 = 6 months (16 weeks)

Total.	600 hours. & 60 Credits

Theory.	120 hours (20 %)

Practice.	480 hours (80 %)

# Description and structure of the course

Following is the structure of the course:

- National Qualification Certificate level-2 in “Construction Machinery”,

      - Workshop Practice-1

| Code | Competency Standards | TH | PR | Total | Credits | Category |
| --- | --- | --- | --- | --- | --- | --- |
| CS-01 | Maintain Tools & Equipment. | 6 | 21 | 27 | 2.7 | Technical |
| CS-02 | Apply technique of Hand and Power Tools | 4 | 21 | 25 | 2.5 | Technical |
| CS-03 | Perform Metal/Bench Work | 5 | 33 | 38 | 3.8 | Technical |
| CS-04 | Perform cutting on Metal Circular/Power Hack Saw | 4 | 18 | 22 | 2.2 | Technical |
| CS-05 | Perform Grinding Operations | 7 | 24 | 31 | 3.1 | Technical |
| CS-06 | Perform Tool & Cutter Grinding | 4 | 15 | 19 | 1.9 | Technical |
| CS-07 | Perform Lathe Machine Operations | 10 | 45 | 55 | 5.5 | Technical |
| CS-08 | Perform Shaper Machining Operations | 4 | 15 | 19 | 1.9 | Technical |
| CS-09 | Perform CNC Lathe Operations | 8 | 30 | 38 | 3.8 | Technical |
| CS-10 | Perform Pre-Welding Operations | 6 | 18 | 24 | 2.4 | Technical |
| CS-11 | Perform Shielded Metal Arc Welding (SMAW)/Electric welding | 7 | 24 | 31 | 3.1 | Technical |
| CS-12 | Perform different welding joints | 5 | 18 | 23 | 2.3 | Technical |
| CS-13 | Perform Forging Operations | 3 | 15 | 18 | 1.8 | Technical |
|  | Total | 73 | 297 | 370 | 37 |  |

      - Engineering Drawing and CAD-I

| Code | Competency Standards | TH | PR | Total | Credits | Category |
| --- | --- | --- | --- | --- | --- | --- |
| CS-14 | Perform Basic Manual Drawing | 6 | 21 | 27 | 2.7 | Technical |
| CS-15 | Construct different Engineering Curves | 5 | 21 | 26 | 2.6 | Technical |
| CS-16 | Construct multi-view drawings | 6 | 33 | 39 | 3.9 | Technical |
| CS-17 | Install CAD software | 4 | 15 | 19 | 1.9 | Technical |
| CS-18 | Manage display in CAD software | 4 | 15 | 19 | 1.9 | Technical |
| CS-19 | Perform function of CAD general commands | 8 | 30 | 38 | 3.8 | Technical |
| CS-20 | Perform Different CAD operations | 8 | 30 | 38 | 3.8 | Technical |
| Total |  | 41 | 165 | 206 | 20.6 |  |

      - Occupational Health, Safety and Environment

| Code | Competency Standards | TH | PR | Total | Credits | Category |
| --- | --- | --- | --- | --- | --- | --- |
| CS-21 | Read and Develop Career Professionalism | 2 | 6 | 8 | 0.8 | Generic |
| CS-22 | Apply basic Occupational Health & Safety regarding heavy machinery | 2 | 6 | 8 | 0.8 | Generic |
| CS-23 | Perform general health, safety and environment practices | 2 | 6 | 8 | 0.8 | Generic |
|  | Total | 6 | 18 | 24 | 2.4 |  |
|  | Grand Total of Level-2 | 120 | 480 | 600 | 60 |  |

# Scheme of Studies.

National Certificate Level-2 in Construction Machinery

| S.No. | Occupation or Subject | Category | Total Contact Hours |  |  | Credit | Periods Per Week |  |  | No. of Weeks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Th | Pr. | Total |  | Th | Pr | Total |  |
|  | Workshop Practice-1 | Technical | 73 | 297 | 370 | 37 | 4.56 | 18.56 | 23.13 | 16 |
|  | Engineering Drawing and CAD-I | Technical | 41 | 165 | 206 | 20.6 | 2.56 | 10.31 | 12.88 | 16 |
|  | Occupational Health, Safety and Environment | Generic | 6 | 18 | 24 | 2.4 | 0.38 | 1.13 | 1.50 | 16 |
|  |  | Allied |  |  |  |  |  |  |  | 16 |
|  |  | Allied |  |  |  |  |  |  |  | 16 |
|  | Total |  | 120 | 480 | 600 | 60 | 7.50 | 30 | 37.5 |  |

- Detail of Modules

## National Certificate Level-2 in Construction Machinery

## 9.1 Workshop Practice - I

- Maintain Tools & Equipment.

Objective: This module covers the skills and knowledge required to Arrange Tools and Equipment, Maintain Tool Box, calibrate measuring tools and Manage Inventory of tools and equipment.

Duration: 27 Hours									Theory: 6 Hours			Practice: 21 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Arrange Tools and Equipment | Trainee will be able to: - Identify tools and equipment - Interpret job card - Prepare list of tools and Collect equipment as per requirement - Collect tools and equipment from store. - Identify insulated tools and equipment | - Explanation of Tool and Equipment orientation - Difference between hazard and risk - State risks in Storing Place - Safety precautions for Tool and Equipment. Practical activity: Identify Measuring and Cutting Tools. | Theory- 01 hrs Practice- 03 hrs Total- 04 hrs | Measuring Tool Marking Tool Calibrating tool Cutting Tool Holding Devices Safety Equipment | Class Room and workshop Store |
| - Maintain Toolbox | Trainee will be able to: - Check physical conditions of tools and equipment before use - Perform preventive maintenance as per standards - Perform corrective maintenance of tools as per requirements. - Clean tools and equipment after use - Place tools and equipment at appropriate place. | - Describe clamping/holding methods - Describe basic measuring /Marking /cutting tools - Define Machine shop tool their working and safety procedure during work. Practical activity: Maintain Toolbox | Theory-01 hrs Practice- 06 hrs Total- 07 hrs | Measuring Tool Marking Tool Calibrating tool Cutting Tool Holding Devices Tool Boxes | Class Room and workshop Store |
| - Calibrate measuring tools | Trainee will be able to - Check calibration status of the measuring tools - Perform calibration of measuring tools as per standards - Record calibration test results | - Clamping/holding methods - Type of measuring tools. - Calibrating Devices Practical activity: Perform Calibration for Measuring Tool | Theory-2hrs Practice- 6 hrs Total- 08 hrs | Measuring Tool Marking Tool Calibrating tool Cutting Tool Holding Devices | Class Room and workshop Store |
| - Manage Inventory of tools and equipment | Trainee will be able to - Check tools and equipment as per record - Report for faulty tools and equipment to supervisor - Generate demand for deficit tools and equipment - Maintain all records of tools and Equipment | - Describe the safety procedure during maintenance. - State the purpose of cleaning & maintenance of work area. - Explanation of the disposal of waste material Practical activity: Make List of Tool and Equipment present in Store. | Theory-02 hrs Practice- 06 hrs Total- 08 hrs | Measuring Tool Marking Tool Calibrating tool Cutting Tool Holding Devices Safety Equipment | Class Room and workshop Store |

- Apply technique of Hand and Power Tools

Objective: This module covers the skills and knowledge required to determine job requirements, sequence of operations and apply technique of hand and power tool.

Duration: 25 Hours									Theory: 04 Hours			Practice: 21 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Use of hand tools | Trainee will be able to: - Select hand tools appropriate to the task requirements. - Use hand tools to produce desired outcomes to job specifications which may include finish, tension, size or shape. - Adhere all safety requirements before, during and after use. - Identify and mark unsafe or faulty tools for repair according to designated procedures before, during and after use. - Maintain tools, including hand sharpening according to standard operational procedures, principles and techniques. - Store hand tools safely in appropriate location according to standard operational procedures and manufacturers’ recommendations. | - Explanation of Tool and Equipment orientation - Difference between hazard and risk - State risks in Storing Place - Safety precautions for Tool and Equipment. - Knowledge & Understanding - Applications of different hand tools. - Procedures for marking unsafe or faulty tools for repair - Clamping /securing methods | Theory- 02 hrs Practice- 09 hrs Total- 11 hrs | Measuring Tool Marking Tool Calibrating tool Cutting Tool Holding Devices Safety Equipment Work bench, Bench vice, Hammer, Tri-square, Hand hacksaw, Scriber, Vernier caliper, Flat File, Number/alphabet punch | Class Room and workshop |
| - Use of power tools | Trainee will be able to: - Select power tools appropriate to the task requirements. - Use power tools for a determined sequence of operations-which may include clamping, alignment and adjustment to produce desired outcomes-to job specifications which may include finish, size or shape. - Adhere all safety requirements to before, during and after use. - Identify and mark unsafe or faulty tools for repair according to designated procedures before, during and after use. - Maintain tools, including hand sharpening according to standard operational procedures, principles and techniques. - Store power tools safely in appropriate location according to standard operational procedures and manufacturers’ recommendations. | - Describe clamping/holding methods - Describe basic measuring /Marking /cutting tools - Define Machine shop tool their working and safety procedure during work. - Adjustment/alignments to a range of power tools - Tool sharpening techniques for a range of power tools - Hazards and control measures associated with using hand and power tools - Benefits and limits of cutting and shaping metal with auxiliary equipment | Theory-02 hrs Practice-12 hrs Total- 14 hrs | Measuring Tool, Marking Tool, Calibrating tool, Cutting Tool, Holding Devices, Tool Boxes, Work bench, Bench vice, Hammer, Tri-square, Hand hacksaw, Scriber, Vernier caliper, Flat File, Number/alphabet punch, | Class Room and workshop |

- Perform Metal/Bench Work

Objective: This module covers the skills and knowledge required to Develop Name Plate manually, carry out Sawing, Prepare Inside Caliper, Prepare Bottle Opener, Prepare Dovetail Joint, Prepare Tri Square (small size), Cut Threads on Work Piece, Prepare Funnel, Prepare Drawer Handle, Cut Pipe Threads and Prepare spanner (small size).

Duration: 38 Hours									Theory: 05 Hours			Practice: 33 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Develop Name Plate manually | Trainee will be able to: - Select marking tools - Hold the sheet in vice. - Cut sheet as per drawing - Perform surface finishing with file - Level the surface with tri-square - Mark the plate as per name requirements - Punch the marked area - Perform finishing with sand paper | - Basic measurement - Marking /cutting tools - Types of Files Practical activity: Make Name Plate. | Theory-01 Hrs Practice-06 Hrs Total- 07 Hrs | Work bench Bench vices Hammer Tri-square Hand hacksaw Scriber Vernier caliper Flat File Number/alphabet punch Metal Sheet | Class Room And Lab |
| - Prepare Dovetail Joint | Trainee will be able to: - Select marking tools - Cut sheet as per drawing - Perform surface finishing with file - Level the surface of both work pieces with tri-square - Mark both work pieces according to drawing - Create outer notch on work piece using flat file and hacksaw - Create inner notch using hacksaw and chisel - Compare both pieces by inserting outer notch into inner notch - Perform finishing with sand paper | - Types of fits. - Types of Chisels. - Clamping Devices Practical activity: Make Dovetail Joint | Theory-01 Hrs Practice-06 Hrs Total- 07 Hrs | Work bench Bench vices Hammer Tri-square Hand hacksaw Scriber Vernier caliper Flat File Metal Sheet Metal working chisel | Class Room And Lab |
| - Cut Threads on Work Piece with tap and die | Trainee will be able to: - Identify different kind of taps & die according to requirement - Identify the work piece clamping method. - Apply tap and die alignment. - Apply lubricants while threading. - Avoid unwanted engraving and slips. - Interpret proper threading procedure | - Taps & die - calculation for drill size for internal threading. - Threading by die and taps Practical activity: Cut Threads With tap and Die | Theory-01 Hrs Practice-09 Hrs Total- 10 Hrs | Work bench Bench vices Hammer Tri-square Hand hacksaw Scriber Vernier caliper Flat File Tap set Tap set | Class Room And Lab |
| - Cut Pipe Threads | Trainee will be able to: - Select marking tools - Cut pipe as per drawing - Select die as per pipe size - Set die into die holder - Select relevant vice for pipe clamping - Perform pipe threading using appropriate method - Perform finishing with sand paper | - Clamping of work piece. - Thread Cutting - Types of Threading Practical activity: Cut Pipe Threads | Theory-01 Hrs Practice-06 Hrs Total- 07 Hrs | Work bench Bench vices Pipe vice | Class Room And Lab |
| - Prepare Funnel | Trainee will be able to: - Select marking tools - Cut sheet as per drawing - Perform surface finishing with file - Mark the sheet according to drawing - Cut the sheet with hand shear - Create radius of funnel using appropriate tools - Perform flat lock seam bend using bench vice - Perform finishing with sand paper. | - use of hand shear - Files - Marking - Cutting - Rolling - Bending Practical activity: Prepare Funnel | Theory-01 Hrs Practice-06 Hrs Total- 07 Hrs | Hand Shearing Machine. Anvil Hammer Sand Paper Metal Sheet | Class Room And Lab |

- Perform cutting on Metal using Circular/Power Heck Saw

Objective: This module covers the skills and knowledge required to Prepare the job for Sawing and Carry out Sawing at different angles.

Duration: 22 Hours									Theory: 04 Hours			Practice: 18 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Prepare the job for Sawing and Carry out Sawing operation | Trainee will be able to: - Mark the job according to given drawing - Select appropriate blade according to job requirement - Set blade in frame of hacksaw as per procedure - Ensure the work piece is clamped firmly and properly - Adopt methods and techniques for sawing that is appropriate to job requirement - Follow marked line during sawing to ensure accuracy. | - Basic measurement - Types of hacksaw frames - Basic measuring /Marking /cutting tools - Clamping/holding methods - Define methods and techniques of sawing. Practical activity: Cut a Job according to given parameters | Theory-02 Hrs Practice-09 Hrs Total- 11 Hrs | Work bench Bench vice Tri-square Hand hacksaw with blade Scriber Flat File Vernier caliper | Class Room and Lab |
| - Carry out Sawing at differet angles | Trainee will be able to: - Mark the job according to given drawing - Select appropriate blade according to job requirement - Set blade in frame of metal circular saw as per procedure. - Check the blade tightness and rotating side. - Check the work piece is clamped firmly and properly - Adopt methods and techniques for sawing that is appropriate to job requirement - Follow marked line during sawing to ensure accuracy. | - Basic measurement - Types of hacksaw frames - Basic measuring /Marking /cutting tools - Clamping/holding methods - Define methods and techniques of sawing Practical activity: Perform Sawing operation on different angles | Theory-02 Hrs Practice-09 Hrs Total- 11 Hrs | Work bench Bench vice Tri-square Hand hacksaw with blade Scriber Flat File Vernier caliper | Class Room And Lab |

- Perform grinding operations

Objective: This module covers the skills and knowledge required to Adjust Grinding wheel, adjust tool Rest and Prepare a single point cutting tool for Basic machining purposes.

Duration: 31 Hours									Theory: 07 Hours			Practice: 24 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Adjust Grinding wheel | Trainee will be able to: - Select the right tool for job - Remove the safety guard of grinding wheel. - Select the wheel type according to material/speed. - Mount the wheel on the spindle. - Balance the wheel - Put back the safety covers. - Ensure that wheel is properly mounted. | - Basic measuring - Types of grinding wheels - Grinding machine types. - Marking /cutting tools Practical activity: Adjust Grinding Wheel | Theory-01 Hrs Practice-03 Hrs Total- 04 Hrs | Measuring devices. Grinder Wheel Grinder Machine Set of Spanners Dressing Tool Safety Hamlet Gloves | Class Room And Lab |
| - Adjust tool Rest | Trainee will be able to: - Select the right tool for job - Remove the safety guard of grinding wheel. - Mount the tool rest. - Balance the wheel - Put back the safety covers - Ensure that tool rest is properly mounted and balanced | - Camping/holding methods. - Holding Devices - Tool Rest Practical activity: Adjust Tool Rest | Theory-01 Hrs Practice-03 Hrs Total- 04 Hrs | Measuring devices. Grinder Wheel Grinder Machine Set of Spanners Dressing Tool Safety Hamlet Gloves | Class Room And Lab |
| - Prepare a single point cutting tool for Basic machining purposes | Trainee will be able to: - Select marking tools - Cut the material as per requirement. - Perform surface finishing with file - Level the surface with tri-square - Give the angle to the tool according to cutting requirement. | - Different cutting tool materials. - Single Point Cutting Tools - Multipoint Cutting Tool Practical activity: Prepare a single point cutting tool | Theory-02 Hrs Practice-06 Hrs Total- 08 Hrs | Measuring devices. Grinder Wheel Grinder Machine Set of Spanners Dressing Tool Safety Hamlet Gloves Single point cutting tool material. | Class Room And Lab |
| - Dress the Wheel | Trainee will be able to: - Select dressing tools - Ensure that wheel is properly mounted. - Use dressing tool to remove the material from grinding wheel which is creating problems. - Balance the grinding wheel. | - Dressing tools. - Loading - Glazing - Dressing Practical activity: Dress the wheel | Theory-01 Hrs Practice-03 Hrs Total- 04 Hrs | Measuring devices. Grinder Wheel Grinder Machine Set of Spanners Dressing Tool Single point cutting tool Safety Hamlet Gloves | Class Room And Lab |
| - Perform Angle Grinding for finishing | Trainee will be able to: - Select marking tools - Cut the material as per requirement. - Perform surface finishing with angle grinder machine - Level the surface with tri-square. | - Safety during handling the angle grinder - Angle grinder working. - Angle Grinder Types. - Usages Practical activity: Perform Angle Grinding | Theory-01 Hrs Practice-03 Hrs Total- 04 Hrs | Measuring devices. Grinder Wheel Angle Grinder Machine Set of Spanners Dressing Tool Single point cutting tool Single point cutting tool Safety Hamlet Gloves | Class Room And Lab |
| - Operate surface grinding | Trainee will be able to: - Select the suitable size and type of grinding wheel. - Mount the work piece over the holding devices to ensure proper clamping. - Dress the wheel with diamond tip if required. - Identify reference points on work piece before grinding. - Adjust depth of cut according to speed of machine table. - Use coolant continuously to avoid over heating of the job. - Observe personal and workplace safety. | - Methods and techniques for surface grinding. - Safety precautions - Finishing Process Practical activity | Theory- 01 Hrs Practice-06 Hrs Total- 07 Hrs | Surface Grinding Machine Holding Devices Wheel Dresser Grinding Wheels Wheel Dresser Stand Measuring Tools Adjustable Wrench Allen Key Set | Class Room and Workshop |

- Perform Tool & Cutter Grinding

Objective: This module covers the skills and knowledge required to Adjust Grinding wheel, adjust tool Rest and Operate the Tool and cutter grinding.

Duration: 19 Hours									Theory: 04 Hours			Practice: 15 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Adjust Grinding wheel | Trainee will be able to: - Select the right tool for job - Remove the safety guard of grinding wheel. - Select the wheel type according to material/speed. - Mount the wheel on the spindle. - Balance the wheel - Put back the safety covers. - Ensure that wheel is properly mounted. | - Types of grinding wheels for tool and cutter grinder. - lamping/holding methods. - Safe distance from grinding wheel for tool rest. - Procedure of mounting of work-piece on to related attachments - Grinding Wheel angles. - Procedure of sharpening of tools and cutters - Safety guidelines and precautions Practical activity: Adjust Grinding wheel | Theory-01 Hrs Practice-03 Hrs Total- 04 Hrs | Tool and cutting grinding Machine Tool and cutting grinding Machine Bench vices Hammer Scriber Vernier calliper Set of spanners Offhand Grinding Machine Diamond dresser tool Grinding attachment | Class Room and Workshop |
| - Adjust tool Rest | Trainee will be able to: - Select the right tool for job - Remove the safety guard of grinding wheel. - Mount the tool rest. - Balance the wheel - Put back the safety covers - Ensure that wheel is properly mounted. | - Types of grinding wheels for tool and cutter grinder. - lamping/holding methods. - Safe distance from grinding wheel for tool rest. - Procedure of mounting of work-piece on to related attachments - Grinding Wheel angles. - Procedure of sharpening of tools and cutters - Safety guidelines and precautions Practical activity: Adjust tool Rest | Theory-01 Hrs Practice-06 Hrs Total- 07 Hrs | Tool and cutting grinding Machine Tool and cutting grinding Machine Bench vices Hammer Scriber Vernier caliper Set of spanners Offhand Grinding Machine Diamond dresser tool Grinding attachment | Class Room and Workshop |
| - Operate the Tool and cutter grinding | Trainee will be able to: - Select the suitable size, type and shape of grinding wheel. - Mount work piece on correct attachment for required procedure. - Adjust the attachments according to different types of tools and cutter grinding. - Follow procedure for sharpening of tools and cutter that is safe and appropriate. - Observe personal and safety precautions. | - Types of grinding wheels for tool and cutter grinder. - lamping/holding methods. - Safe distance from grinding wheel for tool rest. - Procedure of mounting of work-piece on to related attachments - Grinding Wheel angles. - Procedure of sharpening of tools and cutters - Safety guidelines and precautions Practical activity: Sharp the drill bit | Theory-02 Hrs Practice-06 Hrs Total-08 Hrs | Tool and cutting grinding Machine Tool and cutting grinding Machine Bench vices Hammer Scriber Vernier calliper Set of spanners Offhand Grinding Machine Diamond dresser tool Grinding attachment | Class Room and Workshop |

- Perform Basic Lathe Machine Operations

Objective: This module covers the knowledge and skills required to Perform cantering operations, perform facing Operations, perform turning operations, perform drilling or boring operations, Perform step turning operations, Perform knurling Operations, Taper turning by tail stock off-set method, Taper turning by plain taper turning attachment, Taper turning by telescopic taper turning attachment and Perform Internal and External threading Operations.

Duration: 55 Hours								Theory: 10 Hours				Practice: 45 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Perform centering operations | Trainee will be able to: - Select facing tools according to job requirement. - Mount and set the required work-holding devices, work piece and cutting tools. - Follow the correct specifications for the part or component to be produced. - Select safe procedures and tools to accomplish the work. - Adjust the operating parameters (e.g. speed and feed) of machine tool for centering the job. - Ensure all safety mechanisms are in place. | - Safety precautions involved in work. - Types of Lathe Machine - Methods and techniques of adjusting operating parameters of machine tool. Practical activity: Center the job in Lathe Machine | Theory-01 Hrs Practice-03 Hrs Total- 04 Hrs | Lathe Machine Cutting Tools Measuring Tools Personal Protective Equipment Files Vernier Caliper Threading Tools Threading Tools Lathe Attachments | Class Room And Lab |
| - Perform facing Operations | Trainee will be able to: - Select facing tools according to job requirement. - Mount and set the required work-holding devices, work piece and cutting tools. - Follow the correct specifications for the part or component to be produced. - Select safe procedures and tools to accomplish the work. - Adjust the operating parameters (e.g. speed and feed) of machine tool to achieve the work specification. - Ensure all safety mechanisms are in place. | - Speed and feed of Lathe Machine. - Use of holding and cutting tools - Reading and interpreting work specifications, drawings and sketches. - Facing Practical activity: Perform Facing Operation | Theory-1 Hrs Practice-03 Hrs Total- 04 Hrs | Lathe Machine Cutting Tools Measuring Tools Personal Protective Equipment Files Vernier Caliper Threading Tools Threading Tools Lathe Attachments | Class Room And Lab |
| - Perform Turning Operation | Trainee will be able to: - Obtain and follow work specifications, drawings or sketches to accomplish the work. - Set up and adjust the machine as per work specifications and procedures. - Ensure the components produced have the required quality and within the specified dimensional accuracy. - Shut down the machine and equipment on conclusion of the machining activities. | - Speed and feed of Lathe Machine. - Use of holding and cutting tools - Reading and interpreting work specifications, drawings and sketches. - Turning Types - Method and technique of setting up and adjusting the lathe machine - Techniques to check quality of component produced - Procedure of shutting down of machine and equipment after closure of activities. - Safety precautions and procedures need to be observed during work. Practical activity: Perform Turning Operation | Theory-1 Hrs Practice-03 Hrs Total- 04 Hrs | Lathe Machine Cutting Tools Measuring Tools Personal Protective Equipment Files Vernier Caliper Threading Tools Threading Tools Lathe Attachments | Class Room And Lab |
| - Perform Drilling And boring operations | Trainee will be able to: - Select drill or boring tools according to drawings. - Mount and set the required work-holding devices, work piece and cutting tools. - Adjust the RPM of machine according to the cutting speed. - Perform the boring operation according to the drawing. - Check quality of the component produced at different intervals. - Observe personal and workplace safety. | - Types of drilling and boring tools and their function. - Procedure of mounting and setting up of work-holding devices, work pieces and cutting tools. - Drilling Operation - Boring procedures - Safety precautions and procedures. Practical activity: Perform Drilling And boring operations | Theory-01 Hrs Practice-06 Hrs Total- 07 Hrs | Lathe Machine Cutting Tools Measuring Tools Personal Protective Equipment Files Vernier Caliper Threading Tools Threading Tools Lathe Attachments | Class Room And Lab |
| - Perform Step turning operations | Trainee will be able to: - Mount and set the required work-holding devices, work piece and cutting tools. - Select and adjust appropriate speeds and feeds of turning machine. - Produce a component which matches the work specifications using appropriate methods and techniques. - Check quality of the component produced at different intervals. - Follow safety precautions to ensure safe work and to avoid any injury | - Speed and feed of Lathe Machine. - Use of holding and cutting tools - Reading and interpreting work specifications, drawings and sketches. - Turning Types - Method and technique of setting up and adjusting the lathe machine - Techniques to check quality of component produced - Procedure of shutting down of machine and equipment after closure of activities. - Safety precautions and procedures need to be observed during work. Practical activity: Perform Step turning operations | Theory-01 Hrs Practice-03 Hrs Total- 04 Hrs | Lathe Machine Cutting Tools Measuring Tools Personal Protective Equipment Files Vernier Caliper Threading Tools Threading Tools Lathe Attachments | Class Room And Lab |
| - Perform Knurling Operations | Trainee will be able to: - Select the knurling tool according to drawing. - Set the tool and work piece in the machine according to procedure. - Adapt methods and techniques to produce proper knurling on work piece. - Select and adjust appropriate speeds and feeds of lathe machine. - Use coolants during knurling to achieve smooth impression on work piece. - Observe personal and workplace safety. | - Types of knurling tools - Types of knurling. - Methods of knurling. - Importance of using coolants during knurling. Practical activity: Perform Knurling Operations | Theory-01 Hrs Practice-03 Hrs Total- 04 Hrs | Lathe Machine Cutting Tools Measuring Tools Personal Protective Equipment Files Vernier Caliper Threading Tools Threading Tools Lathe Attachments | Class Room And Lab |
| - Taper turning by tail stock off-set method | Trainee will be able to: - Loosen tailstock clamp out. - Offset tailstock required amount. - Centre cutting tool. - Setup cutting tool for parallel turning. - Starting at small diameter take excessive cuts until the taper is .05 to .06 in oversize. - Check taper for accuracy using a taper ring gauge. | - Speed and feed of Lathe Machine. - Use of holding and cutting tools - Reading and interpreting work specifications, drawings and sketches. - Turning Types - Method and technique of setting up and adjusting the lathe machine - Techniques to check quality of component produced - Procedure of shutting down of machine and equipment after closure of activities. - Safety precautions and procedures need to be observed during work. - Taper Turning Method - Calculation of tapers. Practical activity: Perform Taper turning by tail stock off-set method | Theory-01 Hrs Practice-06 Hrs Total- 07 Hrs | Lathe Machine Cutting Tools Measuring Tools Personal Protective Equipment Files Vernier Caliper Threading Tools Threading Tools Lathe Attachments | Class Room And Lab |
| - Taper turning by plain taper turning attachment | Trainee will be able to: - Remove the binding screw that cross slide to cross feed screw nut. - Tighten the lock screw and set cutting tool on center. - Set workpiece in lathe and mark length of taper. - Use binding screw to connect sliding block to side of taper attachment. - Select depth of feed cut by compound rest feed handle. - Take a light cut and recheck taper fit. - Check taper for accuracy using a taper ring gauge. | - Turning Types - Method and technique of setting up and adjusting the lathe machine - Techniques to check quality of component produced - Procedure of shutting down of machine and equipment after closure of activities. - Safety precautions and procedures need to be observed during work. - Taper Turning Method - Calculation of tapers. Practical activity: Perform Taper turning by plain taper turning attachment | Theory-01 Hrs Practice-06 Hrs Total- 07 Hrs | Lathe Machine Cutting Tools Measuring Tools Personal Protective Equipment Files Vernier Caliper Threading Tools Threading Tools Lathe Attachments | Class Room And Lab |
| - Taper turning by telescopic taper turning attachment | Trainee will be able to: - Clean and oil the guide bar. - Loose lock screws and offset end of guide bar, - Set the bar to required taper in degrees. - Tighten the lock screw and set cutting tool on center. - Set workpiece in lathe and mark length of taper and tighten connecting screw on sliding block. - Move carriage until center of attachment is opposite to length of taper. - Lock anchor bracket to lathe bed. - Take a cut and select depth of cut. - Readjust the taper attachment, take a light cut and recheck taper fit. - Check taper for accuracy using a taper ring gauge. | - Turning Types - Method and technique of setting up and adjusting the lathe machine - Techniques to check quality of component produced - Procedure of shutting down of machine and equipment after closure of activities. - Safety precautions and procedures need to be observed during work. - Taper Turning Method - Calculation of tapers. Practical activity: Perform Taper turning by telescopic taper turning attachment | Theory-01 Hrs Practice-06 Hrs Total- 07 Hrs | Lathe Machine Cutting Tools Measuring Tools Personal Protective Equipment Files Vernier Caliper Threading Tools Threading Tools Lathe Attachments | Class Room And Lab |
| - Perform Internal and External threading Operations | Trainee will be able to: - . Mount and set the required work-holding devices, work piece and cutting tools. - Select and adjust appropriate speeds and feeds of turning machine. - Produce a component which matches the work specifications using appropriate methods and techniques. - Check quality of the component produced at different intervals. - Use Proper cutting tool with required dimensions. - Follow safety precautions to ensure safe work and to avoid any injury. | - Threading Types - Threading Tool - Method of Threading - Threading Gauge - Drilling Operation on Lathe Machine - Calculation For threading Practical activity: Perform Internal and External threading Operations | Theory-01 Hrs Practice-06 Hrs Total- 07 Hrs | Lathe Machine Cutting Tools Measuring Tools Personal Protective Equipment Files Vernier Caliper Threading Tools Threading Tools Lathe Attachments | Class Room And Lab |

- Perform Shaper, Planar and Slotter Machining Operations

Objective: This module covers the knowledge and skills required to produce a squared shape work piece, Produce V shaped work piece, T-slot Machining, Machining Irregular Surfaces, Machining External Keyways and Machining Internal Keyways.

Duration: 19 Hours						                Theory: 04 Hours				         Practice: 15 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Produce a squared shape work piece | Trainee will be able to: - Identify safety hazards related with shaping operations and take appropriate steps to avoid any injury or accident. - Dial the machine vice according to job requirement. - Select point cutting tool and set machine as per requirements. - Mount cutting tool and work piece in the machine. - Check quality of the component at suitable intervals. - Shut down the machine at safe position after finishing the work. | - safety hazards related with the shaper machine operations. - Types of Shaper Machine. - Types Planar Machine - Types of Slotter Machine - Difference Between Shaper Planar and Slotter Machine - Dial indicator - Measuring Tools - Cutting Tools - Tri Square - Bevel protector. - Speed and Feed for Shaper, Planar and Slotter Machine. Practical Activity: Make a square piece job | Theory-01 Hrs Practice-03 Hrs Total- 04 Hrs | Shaper, Planar or Slotter Machine Vice Tri square Vernier Calliper Dial indicator with magnet stand Point cutting tools Personal Protective Equipment Bevel protector | Class Room and Lab |
| - Produce V shaped work piece | Trainee will be able to: - Identify safety hazards related with shaping operations and take appropriate steps to avoid any injury or accident. - Dial the machine vice according to job requirement. - Select point cutting tool and set machine according to job requirements. - Mount cutting tool and work piece in the machine. - Check quality of the component at suitable intervals. - Shut down the machine in safe position after finishing the work | - Safety hazards related with the shaper machine operations. - Types of Shaper Machine. - Types Planar Machine - Types of Slotter Machine - Difference Between Shaper Planar and Slotter Machine - Dial indicator - Measuring Tools - Cutting Tools - Tri Square - Bevel protector. - V-Shape cutting procedure. Practical Activity: Produce V shaped work piece | Theory-01 Hrs Practice-03 Hrs Total- 04 Hrs | Shaper, Planar or Slotter Machine Vice Tri square Vernier Calliper Dial indicator with magnet stand Point cutting tools Personal Protective Equipment Bevel protector | Class Room and Lab |
| - Perform T-slot Machining | Trainee will be able to: - Identify safety hazards related with shaping operations and take appropriate steps to avoid any injury or accident. - Dial the machine vice according to job requirement. - Select point cutting tool and set machine according to job requirements. - Mount cutting tool and work piece in the machine. - Check quality of the component at suitable intervals. - Shut down the machine in safe position after finishing the work | - Safety hazards related with the shaper machine operations. - Types of Shaper Machine. - Types Planar Machine - Types of Slotter Machine - Difference Between Shaper Planar and Slotter Machine - Dial indicator - Measuring Tools - Cutting Tools - Tri Square - Bevel protector. - T-Slot Practical Activity: Perform T-slot Machining | Theory-01 Hrs Practice-06 Hrs Total- 07 Hrs | Shaper, Planar or Slotter Machine Vice Tri square Vernier Calliper Dial indicator with magnet stand Point cutting tools Personal Protective Equipment Bevel protector | Class Room and Lab |
| - Machining Irregular Surfaces | Trainee will be able to: - Identify safety hazards related with shaping operations and take appropriate steps to avoid any injury or accident. - Dial the machine vice according to job requirement. - Select point cutting tool and set machine according to job requirements. - Mount cutting tool and work piece in the machine. - Use Different feed and speed of cutting and different points according to given drawing - Check quality of the component at suitable intervals. - Shut down the machine in safe position after finishing the work | - Safety hazards related with the shaper machine operations. - Types of Shaper Machine. - Types Planar Machine - Types of Slotter Machine - Difference Between Shaper Planar and Slotter Machine - Dial indicator - Measuring Tools - Cutting Tools - Tri Square - Bevel protector. Practical Activity: Machining Irregular Surfaces | Theory-01 Hrs Practice-03 Hrs Total- 04 Hrs | Shaper, Planar or Slotter Machine Vice Tri square Vernier Calliper Dial indicator with magnet stand Point cutting tools Personal Protective Equipment Bevel protector | Class Room and Lab |

- Perform CNC Lathe Operations

Objective: This module covers the knowledge and skills required to mount the Job, Generate the Program, Run Simulation, Feed the Program and Perform CNC Lathe Operations

Duration: 38 Hours			                                           Theory: 08 Hours				Practice: 30 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Mount the Job | Trainee will be able to: - Mount the work-piece by considering the working capacity of machine as well as job requirement according to the drawing/design - Select appropriate work holding device(s) in order to achieve dimensional accuracy and clamp the job firmly as per standard practice - Attain proper alignment of tool/ cutter and work-piece e.g. concentricity of rotating jobs as per set practice - Keep safe measures while mounting the work-piece so that unwanted operation by machine may not be initiated as per safety precautions. | - Work place safety and health considerations - Personal Protective Equipment - Reading Drawing. - CNC lathe machine - Use of control panel and commands - Program debugging techniques - Use of Simulation software - Possible accidents and their counteractions - Coolant types along with benefits and uses Practical Activity: Mount the Job on CNC Lathe Machine | Theory-01 Hrs Practice-03 Hrs Total- 04 Hrs | CNC Lathe Machine or Turning Centre along with Standard Accessories Lathe Tooling (Assorted Range) CNC Programming Manual | Class Room and Lab |
| - Generate the Program | Trainee will be able to: - Interpret job requirements, calculate extra material to be removed and define reference point as per drawing/design - Define absolute or incremental coordinates system, tool path strategies, machining features and tool compensation for generating the tool path as per standard procedure - Use appropriate part programming credentials (Sequence, G-codes, M-codes, coordinates, feed, speed, tooling information etc.) according to the CNC machine control unit - Keep record of generated part program in soft/hard form in order to feed into machine control unit as per standard procedure | - Cad Cam - CNC - G codes and M codes - Feed and speed - 3D Molding Practical Activity : Generate the Program | Theory-02 Hrs Practice-06 Hrs Total- 08 Hrs | CNC Programming Manual CNC Manual CAD CAM | Class Room and Lab |
| - Run Simulation | . Trainee will be able to: - Feed the generated part program into appropriate simulation platform and run simulation for checking the tool gouge according to safety measures - Run simulation and verify movements of tool/cutter to get same results as per defined sequence - Identify occurrence of errors and modify the program as per defined procedure | - Cad Cam - CNC - G codes and M codes - Feed and speed - 3D Molding - Debugging Practical Activity: Run Simulation on CNC Lathe Machine | Theory-02 Hrs Practice-06 Hrs Total- 08 Hrs | CNC Programming Manual CNC Manual CAD CAM CAM Software with Simulation Module | Class Room and Lab |
| - Feed the Program | Trainee will be able to: - Ensure proper synchronization between machine control unit and part program file as per standard operating procedure - Switch machine to receiving mode and feed the desired part program file into machine control unit for further execution as per standard operating procedure - Select the desired part program file for execution as per standard operating procedure | - Cad Cam - CNC - G codes and M codes - Feed and speed - 3D Molding | Theory-01 Hrs Practice-06 Hrs Total- 07 Hrs | CNC Programming Manual CNC Manual CAD CAM CAM Software with Simulation Module | Class Room and Lab |
| - Perform CNC Lathe Operations | Trainee will be able to: - Ensure to control the safe operation of working on CNC machines before executing part program according to the safety measures - Control the feeds, speeds and override of machine before operating according to the prescribed procedure - Switch machine to execution mode (single block or auto) and press cycle start to run the machining sequence as per prescribed method - Compare the block-wise movements of machining sequence thoroughly during operating of machine according to the part program file - Complete the job and inspect its accuracy and precision according to the drawing/design | - Work place safety and health considerations - Personal Protective Equipment - Reading Drawing. - CNC lathe machine - Use of control panel and commands - Program debugging techniques - Use of Simulation software - Possible accidents and their counteractions - Coolant types along with benefits and uses - Lathe operations such as Facing, Turning, Drilling, Grooving, Threading, Knurling, Boring etc. | Theory-02 Hrs Practice-09 Hrs Total- 11 Hrs | CNC Lathe Machine or Turning Centre along with Standard Accessories Lathe Tooling (Assorted Range) CNC Programming Manual. CAD CAM Measuring Instruments (Vernier, Inside/Outside Calipers, Micrometer, Steel Rule, Tri-Square, Bevel Protractor etc.) | Class Room and Lab |

### Perform Pre-Welding Operations

Objective: This module covers the knowledge and skills required to Set Welding Equipment, Prepare materials for welding, Cut and Prepare Edge/s of Base Materials and Prepare Welding Consumables

Duration: 24 Hours								Theory: 06 Hours				Practice: 18 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Set Welding Equipment | Trainee will be able to: - Adjust pressure of both gas cylinders with the help of regulator - Open acetylene gas knob of welding torch Make carburizing flame by increasing acetylene gas quantity - Make neutral flame by adjusting both gases at same quantity - Make oxidizing flame by increasing oxygen gas quantity - Adjust pressure of both gas cylinders with the help of regulator - Select the correct size of the nozzle - Set both gas flame of welding torch as per standard of objects on map | - Use welding torch - Identify Gas pressure regulators - Temperature and its units - Making of different flames - Use of different flames Practical Activity: Set Welding Equipment | Theory-01 Hrs Practice-03 Hrs Total- 4 Hrs | Acetylene gas cylinder Oxygen gas cylinder Welding torch | Class Room and Lab |
| - Prepare Tools and materials for welding | Trainee will be able to: - Select and obtain required materials as per job requirements - Select appropriate marking tools as per job requirements - Mark the area to be cut as per drawing/job requirements - Select relevant welding consumables as per job requirements/WPS - Prepare consumables in accordance with required specifications | - Material selection as per requirements - Use of marking tools - Selection of consumables as applied to a particular job requirement as per laid down specs Practical Activity Prepare materials for welding | Theory-03 Hrs Practice-09 Hrs Total- 12 Hrs | Material for making a job as per drawing specs Marking tools required for welding | Class Room and Lab |
| - Cut and Prepare Edge/s of Base Materials | Trainee will be able to: - Select appropriate cutting equipment as per job requirements - Set-up cutting equipment as per manufacturer’s instructions/job requirements - Cut the base material as per job specifications and dimensions provided in the drawing - Prepare edges of the base materials as per drawing/WPS - Check dimensions of the prepared edges as per drawing/WPS - Select proper tools and chemicals for cleaning - Clean the edges of the base materials as per job requirements | - Selection of cutting equipment - Use of cutting equipment - Identification of cleaning chemicals and tools - Use of measuring tools to verify the drawing specs with the actual part. Practical Activity: Cut and Prepare Edge’s of Base Materials | Theory-02 Hr. Practice-06 Hrs Total-08 Hrs | Cutting equipment Measuring tools Cleaning chemicals | Class Room and Lab |

### Perform Shielded Metal Arc Welding (SMAW)/electric welding

Objective: This module covers the knowledge and skills required to perform Shielded Metal Arc Welding (SMAW) operations in Flat (1F, 1G) and Horizontal (2F, 2G) positions at workplace. The standard covers specific knowledge of performing Shielded Metal Arc Welding (SMAW) by selecting and setting up welding equipment, installing consumables, adjusting welding parameters and making fillet and groove welds in Flat (1F, 1G) and Horizontal (2F, 2G) positions of plate. The standard also covers post welding operations comprising cleaning, measuring, inspecting and repairing welds at workplace

Duration:  31 Hours							       Theory: 07 Hours				Practice: 24 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Prepare Welding Machine and Accessories for SMAW | The trainee will be able to: - Identify welding requirements from the job, welding procedure specifications and/or technical drawings - Prepare SMAW welding machine in accordance with welding procedure specifications/ manufacturer instructions - Set up welding machine accessories and consumables as per job requirements, welding procedure specifications and/or manufacturer’s instructions - Connect welding machine to an independent power supply - Set polarity indicated in the welding procedure specifications | - Understanding of technical drawings - Electrical supply AC and DC - Specifications/ classification of electrode/s required for the job - Electrical parameters like (voltage, current etc.) and their effects on weld - Welding techniques as per WPS/instruction sheet - Welding procedure specifications (WPS) - Method of Pre- heating of base metal - Polarity setting according to standard specifications - Welding codes and standards | Theory-01 Hrs Practice-03 Hrs Total- 04 Hrs | Separate power supply Electrodes as per job requirement Work piece for welding practice Arc Welding Machine | Class Room, and Lab |
| - Make Fillet Welds on C Steel Plate (1F, 1G) | The trainee will be able to: - Adjust welding parameters (current, voltage etc.) as per welding procedure specifications/job requirements to produce acceptable weld Maintain gap between electrode and base metal as per standard practices - Prepare the work piece/job for fillet weld - Carry out welding in Flat (1F) and Flat (1G) positions following standard procedures - Follow applicable manufacturing codes and standards for acceptance criteria of visual welding defects | - Understanding of technical drawings - Electrical supply AC and DC - Specifications/ classification of electrode/s required for the job - Electrical parameters like (voltage, current etc.) and their effects on weld - Welding techniques as per WPS/instruction sheet - Welding procedure specifications (WPS) - Method of Pre- heating of base metal - Polarity setting according to standard specifications - Welding codes and standards | Theory-02 Hrs Practice-03 Hrs Total- 05 Hrs | Separate power supply Electrodes as per job requirement Work piece for welding practice Arc Welding Machine | Class Room, and Lab |
| - Make Groove Welds on Carbon Steel Plate (2F, 2G) | The trainee will be able to: - Adjust welding parameters (current, voltage etc.) as per welding procedure specifications/job requirements to produce acceptable weld - Maintain gap between electrode and base metal as per standard practices - Prepare the work piece/job for groove weld - Carry out welding in Horizontal (2F) and Horizontal (2G) positions following standard procedures - Deposit root pass as per welding procedure specifications/job requirements - Deposit filling passes as per welding procedure specifications/job requirements - Deposit capping pass as per welding procedure specifications/job requirements - Check root, filling and capping passes for any visual discontinuities as per acceptance standards - Follow applicable manufacturing codes and standards for acceptance criteria of visual welding defects | - Understanding of technical drawings - Electrical supply AC and DC - Specifications/ classification of electrode/s required for the job - Electrical parameters like (voltage, current etc.) and their effects on weld - Welding techniques as per WPS/instruction sheet - Welding procedure specifications (WPS) - Method of Pre- heating of base metal - Polarity setting according to standard specifications - Welding codes and standards | Theory-01 Hrs Practice-03 Hrs Total- 04 Hrs | Separate power supply Electrodes as per job requirement Work piece for welding practice Arc Welding Machine | Class Room, and Lab |
| - . Make Fillet Welds on Carbon Steel Plate (3F, 3G) | The trainee will be able to: - Adjust welding parameters (current, voltage etc.) as per welding procedure specifications/job requirement to produce acceptable weld - Maintain gap between electrode and base metal as per standard practices - Prepare the work piece/job for fillet weld - Carry out welding in Vertical (3F) and Vertical (3G) positions following standard procedures - Deposit root pass as per welding procedure specifications/job requirements - Deposit filling passes as per welding procedure specifications/job requirements - Deposit capping pass as per welding procedure specifications/job requirements - Check root, filling and capping passes for any visual discontinuities as per acceptance standards - Follow applicable manufacturing codes and standards for acceptance criteria of visual welding defects | - Understanding of technical drawings - Electrical supply AC and DC - Specifications/ classification of electrode/s required for the job - Electrical parameters like (voltage, current etc.) and their effects on weld - Welding techniques as per WPS/instruction sheet - Welding procedure specifications (WPS) - Method of Pre- heating of base metal - Polarity setting according to standard specifications - Welding codes and standards | Theory-01 Hrs Practice-06 Hrs Total- 07 Hrs | Separate power supply Electrodes as per job requirement Work piece for welding practice Arc Welding Machine | Class Room, and Lab |
| - Make Groove Welds on Carbon Steel Plate (4F, 4G) | The trainee will be able to: - Adjust welding parameters (current, voltage etc.) as per welding procedure specifications/job requirements to produce acceptable weld - Maintain gap between electrode and base metal as per standard practices - Prepare the work piece/job for groove weld - Carry out welding in Overhead (4F) and Overhead (4G) positions following standard procedures Deposit root pass as per welding procedure specifications/job requirements - Deposit filling passes as per welding procedure specifications/job requirements - Deposit capping pass as per welding procedure specifications/job requirements - Check root, filling and capping passes for any visual discontinuities as per acceptance standards - Follow applicable manufacturing codes and standards for acceptance criteria of visual welding defects | - Understanding of technical drawings - Electrical supply AC and DC - Specifications/ classification of electrode/s required for the job - Electrical parameters like (voltage, current etc.) and their effects on weld - Welding techniques as per WPS/instruction sheet - Welding procedure specifications (WPS) - Method of Pre- heating of base metal - Polarity setting according to standard specifications - Welding codes and standards | Theory-01 Hrs Practice-06 Hrs Total- 07 Hrs | Separate power supply Electrodes as per job requirement Work piece for welding practice Arc Welding Machine | Class Room, and Lab |
| - Perform Post Welding Operations | The trainee will be able to: - Carry out finishing work of welds following standard procedures - Inspect weld visually and mark any visual defects, as required - Carry out repair work in accordance with approved procedures, as required - Clean work area in accordance with workplace safety practices - Maintain and store tools/equipment/consumable materials in accordance with organization guidelines | - Visual welding defects - Welding codes and standards - Specifications/ classification of electrode/s required for the job | Theory-01 Hrs Practice-03 Hrs Total- 04 Hrs | Separate power supply Electrodes as per job requirement Work piece for welding practice Arc Welding Machine | Class Room, and Lab |

### Perform different welding joints

Objective: This module covers the knowledge and skills required to Practice of Making Butt Joint, Practice of making Lap Joint and Practice of making Tee Joint

Duration:  23 Hours						       Theory: 05 Hours				Practice: 18 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Practice of Making Butt Joint | The trainee will be able to: - Take M.S Flat as per drawing - Straiten it with the help of hammer and anvil - Place the work piece on welding table - Hold the electrode in electrode holder - Set the current on welding machine as per standard - Hold the electrode at right angle - Set the current at 120-125 ampere - Tack both pieces of M.S Flat for Butt Joint - Prepare the arc by touching the end of electrode with the base metal and withdrawing it to proper gap - Lean the electrode at 70º to the base metal and complete the bead to form a square butt joint | - Understanding of technical drawings - Electrical supply AC and DC - Specifications/ classification of electrode/s required for the job - Electrical parameters like (voltage, current etc.) and their effects on weld. - Types Of Joints - Welding techniques as per WPS/instruction sheet - Welding procedure specifications (WPS) - Method of Pre- heating of base metal - Polarity setting according to standard specifications - Welding codes and standards | Theory-1.5 Hrs Practice-06 Hrs Total- 7.5 Hrs | Separate power supply Electrodes as per job requirement Work piece for welding practice Arc Welding Machine | Class Room, and Lab |
| - Practice of making Lap Joint | The trainee will be able to: - Take M.S Flat as per drawing - Straiten it with the help of hammer and anvil - Mark parallel line on both pieces with the help of scriber for positioning of lap joint - Place the work piece on welding table - Place the Bottom piece on work table and place the Top plate along the marked line - Hold the electrode in electrode holder - Set the current on welding machine as per standard - Tack both work pieces of M.S Flat - Complete the bead by back hand welding technique as per standard | - Understanding of technical drawings - Electrical supply AC and DC - Specifications/ classification of electrode/s required for the job - Electrical parameters like (voltage, current etc.) and their effects on weld. - Types Of Joints - Welding techniques as per WPS/instruction sheet - Welding procedure specifications (WPS) - Method of Pre- heating of base metal - Polarity setting according to standard specifications Welding codes and standards | Theory-1.5 Hrs Practice-06 Hrs Total- 7.5 Hrs | Separate power supply Electrodes as per job requirement Work piece for welding practice Arc Welding Machine | Class Room, and Lab |
| - Practice of making Tee Joint | The trainee will be able to: - Take M.S Flat as per drawing - Straiten it with the help of hammer and anvil - Grind the work pieces on grinding machine to prepare the edges flat and parallel to each other - Place the Bottom piece on work table and place the Top plate at 90* to each other as per drawing - Hold the electrode in electrode holder - Set the current on welding machine as per standard - Tack both work pieces of M.S Flat - Lean the electrode and move along traverse direction - Complete the bead as per standard | - Understanding of technical drawings - Electrical supply AC and DC - Specifications/ classification of electrode/s required for the job - Electrical parameters like (voltage, current etc.) and their effects on weld. - Types Of Joints - Welding techniques as per WPS/instruction sheet - Welding procedure specifications (WPS) - Method of Pre- heating of base metal - Polarity setting according to standard specifications Welding codes and standards | Theory-2 Hrs Practice-06 Hrs Total- 08 Hrs | Separate power supply Electrodes as per job requirement Work piece for welding practice Arc Welding Machine | Class Room, and Lab |

### Perform Forging Operations

Objective: This module covers the knowledge and skills required to gain basic knowledge and skills required to Perform main operation of forging (Drawing down and Swaging). The standard covers specific knowledge of performing 0f Forging in drawing down and swaging.

Duration:  18 Hours							       Theory: 03 Hours				Practice: 15 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Practice of drawing | The trainee will be able to: - Make fire in available forge furnace - Put the stock in forge furnace. - Heat the stock up to bright red condition (Plastic State) - Grip the heated stock by tong and put on anvil. - Apply firm blows of sledge hammer on both sides of stock. - Put the stock again in forge furnace and repeat process - Always wear safety dress before start the performance | - Understanding of technical drawings - Specifications/ classification of Furnaces - Drawing techniques as per WPS/instruction sheet - Drawing procedure specifications (WPS) - Method of Pre- heating of base metal. - Sheet metal drawing and wire, bar, and tube drawing. Practical Activity: Perform Drawing operation | Theory-1.5 Hrs Practice-09 Hrs Total- 10.5 Hrs | Forge furnace sledge hammer Safety Dress Dies | Class Room, and Lab |
| - Practice of Swaging | The trainee will be able to: - Make fire in forge furnace - Put the round stock in forge furnace. - Heat the stock up to bright red condition. - Fix the bottom swage in hardy hole. - Put the red-hot round bar in the bottom swage. - Put the top swage over round bar with the help of handle. - Apply firm blows of sledge hammer and move round bar longitudinally and circumferentially. - Always wear safety dress before start the performance. | - Understanding of technical drawings - Specifications/ classification of Furnaces - Swaging techniques as per WPS/instruction sheet - Swaging procedure specifications (WPS) - Method of Pre- heating of base metal. - Types of Dies - Hot Working Process - Cold Working Process - Types of Hammers Practical Activity: Perform Swaging operation | Theory-1.5 Hrs Practice-06 Hrs Total- 7.5 Hrs | Forge furnace sledge hammer Safety Dress Dies | Class Room, and Lab |

# 9.2 Engineering Drawing and CAD-1

- Perform Basic Manual Drawing

Objective: This module covers the skills and knowledge required to Draw single stroke capital vertical lettering, Draw single stroke capital inclined lettering, Draw horizontal, vertical and inclined lines, Use of compass, circles, half circles, radius, Drawing Centre   lines, centres, curves, and crossing of lines, Construction of parallel-lines, perpendicular, bisects line, angles and equal division of lines, Draw round corners, circles elements, quadrilaterals inside and outside circle and Construction of angles and triangles.

Duration: 27 Hours									Theory: 6 Hours			Practice: 21 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Practice of Lettering 5 mm height | Trainee will be able to: - Prepare Drawing sheet. - Select the tools. - Use Proper pencil for lettering with holding techniques. - Draw Boundary lines as per standards. - Make title bar - Draw upper and lower lines for lettering on distance of 5mm - Start Writing Vertical Lettering with different style like Gothic, Roman and free hand lettering | - Importance of Technical Drawing. - Language of engineering terminology. - Uses of Technical Drawing - Type of Drawing - Application of Technical drawing - Drawing Pencil, their grading, sharpening and using techniques. - Style of letters. - General rules for lettering Practical activity: - Perform sheet distribution | Theory-1 hours Practice-3hours Total- 4hours | Pencil Eraser Sharpener Notepad Graph and drawing sheet Drawing Board/Table Tea-Square Set Square Geometry Box | Class Room and Drawing Hall |
| - Practice of Lettering 3 mm height | Trainee will be able to: - Prepare Drawing sheet. - Select the tools. - Use Proper pencil for lettering with holding techniques. - Draw Boundary lines as per standards. - Make title bar - Draw upper and lower lines for lettering on distance of 3mm - Start Writing Vertical Lettering with different style like Gothic, Roman and free hand lettering | - Procedure for lettering 3mm height - Techniques for lettering 3mm height Practical activity: Perform lettering on a standard distributed sheet of drawing | Theory-0.5 hours Practice-2hours Total- 2.5hours | Pencil Eraser Sharpener Notepad Graph and drawing sheet Drawing Board/Table Tea-Square Set Square Geometry Box | Class Room and Drawing Hall |
| - Use Tee Square and Set Squares for drawing horizontal, vertical and inclined lines | Trainee will be able to: - Prepare Drawing sheet. - Select the tools. - Draw Boundary lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw horizontal lines at 0 degree using standard tools - Draw inclined lines at 30, 45, 60 and 120 degree angles using standard tools - Draw vertical lines at 90 degree using standard tools | - Introduction of lines - Techniques of sketching lines at different angles and direction - Types of lines - Uses of Tee Square and Set Square Practical activity: Draw lines in different direction and at different angles | Theory-1 hours Practice-2hours Total- 3hours | Pencil Eraser Sharpener Notepad Graph and drawing sheet Drawing Board/Table Tea-Square Set Square Geometry Box | Class Room and Drawing Hall |
| - Carry out use of Tee square and for drawing centres, crossing of lines | Trainee will be able to - Prepare Drawing sheet. - Select the tools. - Draw Boundary lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Make different angle curves. - Sketch Centre line according to standard. - Draw crossing line | - Introduction to boundary lines - Techniques of sketching boundary lines - Introduction to curves Practical activity: Prepare a drawing sheet having title bar, boundary lines and center lines | Theory-0.5 hours Practice-2hours Total- 2.5hours | Pencil Eraser Sharpener Notepad Graph and drawing sheet Drawing Board/Table Tea-Square Set Square Geometry Box | Class Room and Drawing Hall |
| - Draw circles, half circles and radius | Trainee will be able to - Prepare Drawing sheet. - Select the tools. - Draw Boundary lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Make different diameter circles and half circles using compass - Make different Radius using Compass and Protector | - Uses of compass and protector - Define circles - Types of circles Practical activity: Construct circle and half circle using compass & protector | Theory-0.5 hours Practice-2hours Total- 2.5hours | Pencil Eraser Sharpener Notepad Graph and drawing sheet Drawing Board/Table Tea-Square Set Square Geometry Box | Class Room and Drawing Hall |
| - Draw round corners, figure inside and outside circle | Trainee will be able to - Prepare Drawing sheet. - Select the tools. - Draw Boundary lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Make different dia circles. - Make diagrams inside the circle that touch the circle at the tangent points - Make diagrams outside the circle that touch the circle at the tangent points | - Define inside and outside circles - Technique for drawing round corner - Technique for drawing inscribed and circumscribed circle Practical activity: Draw a square inside a circle and circle inside a square | Theory-0.5 hours Practice-2hours Total- 2.5hours | Pencil Eraser Sharpener Notepad Graph and drawing sheet Drawing Board/Table Tea-Square Set Square Geometry Box | Class Room and Drawing Hall |
| - Draw Plane geometry angles and triangles | Trainee will be able to - Prepare Drawing sheet. - Select the tools. - Draw Boundary lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw Equilateral Triangle, Isosceles triangle, Scalene Triangle, Right Triangle, Obtuse Triangle, Acute Triangle. | - Define angles - Types of angles - Procedure for constructing angles Practical activity: Draw a Isosceles and Right angle triangle | Theory-0.5 hours Practice-2hours Total- 2.5hours | Pencil Eraser Sharpener Notepad Graph and drawing sheet Drawing Board/Table Tea-Square Set Square Geometry Box | Class Room and Drawing Hall |
| - Sketch Plane Geometry quadrilateral, square, rhombus and parallelogram | Trainee will be able to - Prepare Drawing sheet. - Select the tools. - Draw Boundary lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw Square and Rectangle as per drawing - Draw Rhombus using protector and set square as per drawing - Draw Parallelogram as per drawing | - Define quadrilateral, square, rhombus and parallelogram - Technique for constructing polygon Practical activity: Draw a polygon having four sides of same/different length and angles. | Theory-0.5 hours Practice-2hours Total- 2.5hours | Pencil Eraser Sharpener Notepad Graph and drawing sheet Drawing Board/Table Tea-Square Set Square Geometry Box | Class Room and Drawing Hall |
| - Plane Geometry parallel-lines, perpendicular, bisect line and angle | Trainee will be able to - Prepare Drawing sheet. - Select the tools. - Draw Boundary lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw parallel-lines of different length - Draw perpendicular lines as per drawing - Draw bisect lines as per drawing | - Define parallel-lines, perpendicular, bisect line and angle Practical activity: Draw a rectangle from a line, bisecting line at start and end. | Theory-0.5 hours Practice-2 hours Total- 2.5hours | Pencil Eraser Sharpener Notepad Graph and drawing sheet Drawing Board/Table Tea-Square Set Square Geometry Box | Class Room and Drawing Hall |
| - Sketch Plane geometry equal division of line and some radio with the help of compass and set square | Trainee will be able to - Prepare Drawing sheet. - Select the tools. - Draw Boundary lines as per standards. - Make title bar - Draw lines of different lengths as per drawing - Open compass more than one half of drawn line - Use compass to draw an arc from one end of drawn line - Repeat this step on other end of line - Draw a perpendicular line where both arc meets - Check the half of line using tri square | - Define bisection of line - Procedure for bisection a line Practical activity: Draw a right-angle by bisecting a line at center. | Theory-0.5 hours Practice-2hours Total- 2.5hours | Pencil Eraser Sharpener Notepad Graph and drawing sheet Drawing Board/Table Tea-Square Set Square Geometry Box | Class Room and Drawing Hall |

- Construct different Engineering Curves

Objective: This module covers the skills and knowledge required to Construct inscribe and circumscribe figures, Construct a pentagon, Hexagon and Octagon by circumscribe method, Construct a pentagon, Hexagon and Octagon by inscribe method, Construct a Tangents of circles (Inside & Outside)When the center of the given circle is known and when the circle of center is not known, Construct an Ellipse by Concentric Circle Method, Rectangle Method, Oblong Method, Arcs of Circle Method, Rhombus Method and Basic Locus Method, Construct a parabola curve by Rectangle Method, Method of Tangents( Triangle Method) and Basic Locus Method, Construct a hyperbola curve, Construct a Archimedean Spiral curve, Construct a involutes curve of square rectangle hexagon and circle and Construct of cycloid, epicycloids, and hypocycloid.

Duration:26 Hours						   Theory: 5 Hours                                             Practice: 21Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Draw inscribe and circumscribe figures. | Trainee will be able to: - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw square, triangle and hexagon according to dimension | - Techniques of sketching straight lines in different directions - Define Triangles, Quadrilateral and Polygons - Describe circular arc using different line method - Describe inscribe and circumscribe figures. - Describe circular arc - Types of Geometric Shape - Two-dimensional shapes - Three-dimensional shapes Practical activity: Draw an inscribed triangle and circumscribed hexagon | Theory-0.5 hrs Practice-2hrs Total- 2.5hrs | Pencil Eraser Sharpener Notepad Graph and drawing sheet Drawing Board/Table Tea-Square Set Square Geometry Box | Class Room and Drawing Hall |
| - Draw a pentagon, Hexagon and Octagon by circumscribe method. | Trainee will be able to: - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw pentagon, Hexagon and Octagon | - Define pentagon, Hexagon and Octagon - Circumscribed method Practical activity: Draw a circumscribed octagon | Theory-.5 hrs Practice-1.5 hrs Total- 3 hrs | Pencil Eraser Sharpener Notepad Graph and drawing sheet Drawing Board/Table Tea-Square Set Square Geometry Box | Class Room and Drawing Hall |
| - Draw pentagon, Hexagon and Octagon by inscribe method | Trainee will be able to: - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw pentagon, Hexagon and Octagon. | - Define pentagon, Hexagon and Octagon - Inscribed technique Practical activity: Draw an inscribed pentagon | Theory-0.5 hrs Practice-1.5 hrs Total- 2 hrs | Pencil Eraser Sharpener Notepad Graph and drawing sheet Drawing Board/Table Tea-Square Set Square Geometry Box | Class Room and Drawing Hall |
| - Draw Tangents of circles (Inside & Outside) | Trainee will be able to: - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw Tangents Inside of a circle When the centre of the circle is known. - Draw Tangents Inside of a circle When the centre of the circle is unknown - Draw Tangents outside of a circle When the centre of the circle is known - Draw Tangents outside of a circle When the centre of the circle is unknown | - Define Tangents and normal lines to circle - Techniques to draw tangent at any point of circle Practical activity: Construct a line touching circle at any point when its center is known. | Theory-0.5 hrs Practice-1.5 hrs Total- 2 hrs | Pencil Eraser Sharpener Notepad Graph and drawing sheet Drawing Board/Table Tea-Square Set Square Geometry Box | Class Room and Drawing Hall |
| - Construct Ellipse | Trainee will be able to: - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw an Ellipse by Concentric Circle. - Draw an Ellipse by Rectangle Method - Draw an Ellipse by Oblong Method - Draw an Ellipse by Arcs of Circle Method - Draw an Ellipse by Rhombus Method. - Draw an Ellipse by Basic Locus Method | - Describe ellipse - Describe different methods of sketching ellipse Practical activity: Construct an Ellipse by Concentric circle, Rectangle, Oblong and Arc of circle, Rhombus, and locus methods. | Theory-0.5hrs Practice-2 hrs Total- 2.5 hrs | Pencil Eraser Sharpener Notepad Graph and drawing sheet Drawing Board/Table Tea-Square Set Square Geometry Box | Class Room and Drawing Hall |
| - Draw parabola curve. | Trainee will be able to: - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw a parabola curve by Rectangle - Draw a parabola curve by Method of Tangents (Triangle Method) - Draw a parabola curve by Basic Locus Method | - Describe parabola - Describe different methods of parabola Practical activity: Construct a parabola curve by rectangle method, tangent method and basic locus method. | Theory-.5 hrs Practice-2.5 hrs Total- 3 hrs | Pencil Eraser Sharpener Notepad Graph and drawing sheet Drawing Board/Table Tea-Square Set Square Geometry Box | Class Room and Drawing Hall |
| - Draw hyperbola curve | Trainee will be able to: - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw a hyperbola curve. | - Describe hyperbola curve - Describe different methods of hyperbola curve Practical activity: Draw a hyperbola curve by Eccentricity method | Theory-0.5 hrs Practice-2 hrs Total- 2.5 hrs | Pencil Eraser Sharpener Notepad Graph and drawing sheet Drawing Board/Table Tea-Square Set Square Geometry Box | Class Room and Drawing Hall |
| - Make Archimedean Spiral curve | Trainee will be able to: - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw spiral curve. | - Describe spiral curve - Technique for spiral curve Practical activity: Construct a spiral curve. | Theory-0.5 hrs Practice-2 hrs Total- 2.5 hrs | Pencil Eraser Sharpener Notepad Graph and drawing sheet Drawing Board/Table Tea-Square Set Square Geometry Box | Class Room and Drawing Hall |
| - Make involute curve | Trainee will be able to: - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw involute curve by square - Draw involute curve by rectangle - Draw involute curve by hexagon - Draw involute curve by circle | - Describe involute curve Practical activity: Construct an involute curve by square, rectangle, hexagon and circle. | Theory-0.5 hrs Practice-3 hrs Total- 3.5 hrs | Pencil Eraser Sharpener Notepad Graph and drawing sheet Drawing Board/Table Tea-Square Set Square Geometry Box | Class Room and Drawing Hall |
| - Draw cycloid, epicycloid, and hypocycloid | Trainee will be able to: - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw the generating circle and the base line equal to the circumference of the generating circle - Divide the circle and the base line in to equal number of parts - Complete the cycloid, epicycloids, and hypocycloid. | - Describe cycloid - Describe epicycloids - Describe hypocycloid - Uses of cycloid, epicycloid, and hypocycloid - Techniques for developing cycloid, epicycloid, and hypocycloid Practical activity: Construct cycloid, epicycloid and hypocycloid from circle and line. | Theory-0.5 hrs Practice-3 hrs Total- 3.5 hrs | Pencil Eraser Sharpener Notepad Graph and drawing sheet Drawing Board/Table Tea-Square Set Square Geometry Box | Class Room and Drawing Hall |

- Construct multi-view drawings

Objective: This module covers the skills and knowledge required to Sketch Orthographic projection 1st angle, Sketch Orthographic projection 3rd angle, Sketch Oblique Drawing, Construct multi view drawing of Simple Bearing, Construct multi view drawing of Open Bearing, Sketch prism, Sketch cone and Draw pyramid.

Duration: 39 Hours									Theory: 6 Hours			Practice: 33 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Sketch Orthographic projection in 1st angle of Projection | Trainee will be able to: - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in equal parts. - Draw plan view - Draw front view - Draw side view | - Purpose and principles of Orthographic Projection. - Conventional symbols with its importance. - Steps involved in paper distribution - Explain Multi view drawing of Simple 3-D object. - Explain Orthographic projection in 1st angle. - Dimensioning of drawing Practical activity: Draw plan, front, and side view of hexagonal nut in 1st angle of projection. | Theory-1 hrs Practice-5hrs Total- 6 hrs | Pencil Eraser Sharpener Notepad Graph and drawing sheet Drawing Board/Table Tea-Square Set Square Geometry Box | Class Room and Drawing Hall |
| - Sketch Orthographic projection 3rd angle of Projection | Trainee will be able to: - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in equal parts. - Draw plan view - Draw front view - Draw side view | - Explain Orthographic projection in 3rd angle. - Explain Prism, Cone and pyramid Practical activity: Draw plan, front, and side view of hexagonal nut in 3rd angle of projection. | Theory-1 hrs Practice-5 hrs Total- 6 hrs | Pencil Eraser Sharpener Notepad Graph and drawing sheet Drawing Board/Table Tea-Square Set Square Geometry Box | Class Room and Drawing Hall |
| - Sketch Oblique Drawing | Trainee will be able to: - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in equal parts - Draw the front or side view of the object. | - Basic Principle of Oblique Drawing. - Understand difference between orthographic projection and oblique projection - Types of Oblique Drawing. - Orientation of an object Practical activity: Draw front and side view of cube by oblique projection method. | Theory-01 hr. Practice-03 hrs Total-04 hrs | Pencil Eraser Sharpener Drawing Sheet Scotch Tape Pegs Drawing Board/Table. Tea-Square Set Square. Templets. Geometry Box. Scale Card | Class Room & Drawing Hall |
| - Construct multi view drawing of Simple Bearing. | Trainee will be able to: - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in equal parts. - Draw plan view of simple bearing - Draw front view of simple bearing - Draw side view of simple bearing | - Understand terminology used in simple bearing. - Use of T and set square to draw lines at different angles. - Calculate overall Length, Width and Height of simple 3-D objects - Shifting of dimensions from one view to other - Uses of different types of lines in multi view drawing Practical activity: Draw Plan, front and side view of simple bearing by 1st angle of projection. | Theory-1 hrs Practice-5 hrs Total- 6 hrs | Pencil Eraser Sharpener Drawing Sheet Scotch Tape Pegs Drawing Board/Table. Tea-Square Set Square. Templets. Geometry Box. Scale Card | Class Room & Drawing Hall |
| - Multi view drawing of Open Bearing | Trainee will be able to: - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in equal parts. - Draw plan view of open bearing - Draw front view of open bearing - Draw side view of open bearing | - Understand terminology used in open bearing. - Use of T and set square to draw lines at different angles. - Calculate overall Length, Width and Height of open bearing - Shifting of dimensions from one view to other - Uses of different types of lines in multi view of open bearing Practical activity: Draw Plan, front and side view of simple bearing by 1st angle of projection. | Theory-.5 hrs Practice-4 hrs Total- 4.5 hrs | Pencil Eraser Sharpener Notepad Graph and drawing sheet Drawing Board/Table Tea-Square Set Square Geometry Box Scale Card | Class Room & Drawing Hall |
| - Sketch prism | Trainee will be able to: - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in equal parts. - Sketch prism | - Introduction to geometrical drawing. - Concept of Solid of revolutions - Describe conic sections Practical activity: Draw a pentagonal prism. | Theory-0.5 hrs Practice-4 hrs Total- 4.5 hrs | Pencil Eraser Sharpener Notepad Graph and drawing sheet Drawing Board/Table Tea-Square Set Square Geometry Box Scale Card | Class Room & Drawing Hall |
| - Sketch cone | Trainee will be able to: - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in equal parts. - Start with a horizontal oval - draw the two sides of a triangle which meets at a common point | - cone and its type - different terminologies used in solid of revolutions Practical activity: Draw a cone from a circle. | Theory-0.5 hr Practice-4 hrs Total- 4.5 hrs | Pencil Eraser Sharpener Notepad Graph and drawing sheet Drawing Board/Table Tea-Square Set Square Geometry Box Scale Card | Class Room & Drawing Hall |
| - Draw pyramid | Trainee will be able to: - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in equal parts. - Sketch pyramid | - Define Pyramid - Procedure for sketching pyramid Practical activity: Draw Pyramid from a square. | Theory-0.5 hr Practice-03 hrs Total-3.5 hrs | Pencil Eraser Sharpener Notepad Graph and drawing sheet Drawing Board/Table Tea-Square Set Square Geometry Box Scale Card | Class Room & Drawing Hall |

- Install CAD software

Objective: This module covers the skills and knowledge required to create new file and create basic drawing, Practice loading CAD software into computer memory and Practice CAD abbreviations.

Duration: 19 Hours									Theory: 4 Hours			Practice:  15 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Practice loading CAD software into computer memory | Trainee will be able to: - Download the installer files based on the download methods available for your product. - Find and double-click the setup file to start the installation - Click Install/Uninstall on this Computer - Select any one of the following options - Typical - Full - Compact - Custom - Perform necessary steps to Install CAD software | - Hardware requirements for the CAD software. - Procedure for installation of CAD software - Basic computer skill Practical activity: Install AutoCAD software on PC/Laptop. | Theory-1 hrs Practice-5 hrs Total- 6 hrs | Computer with all accessories , AutoCAD software | Class Room and Computer Lab |
| - Practice CAD abbreviations, auto CAD release 12 of latest (e.g. A for Arc, C for circle, E for Erase etc.). | Trainee will be able to: - Create New Template - Save the File - Create Drawing - Select units as per requirements - Select drawing Limits - Click on command window - Type alphabet A and select method of arc to be used - Type alphabet C and select method of circle to be used - Type alphabet E and select erase method to be used | - Basic Drawing Settings - Unit setting - Limits setting - User coordinate system Workspace setting - Save CAD file Practical activity: Draw a line, arc and circle with command window. | Theory-1.5 hrs Practice-5 hrs Total- 6.5 hrs | Computer with all accessories ,AutoCAD software . | Class Room and Computer Lab |
| - Practice function keys for shortcuts | Trainee will be able to: - Create New Template - Save the File - Create Drawing - Select units as per requirements - Select drawing Limits - Select function key F1 to display help - Select function key F3 to turn on/off Object Snap - Select function key F4 to turn on/off 3D Object Snap - Select function key F7 to turn on/off Grid Display - Select function key F8 to lock cursor movement to Horizontal/Vertical | - Basic Drawing Settings - Unit setting - Limits setting - User coordinate system Workspace setting - Save CAD file - Uses of function key Practical activity: Draw a rectangle with object snap on, using line command by activating Grid display on. | Theory-1.5 hrs Practice-5 hrs Total- 6.5 hrs | Computer with all accessories ,AutoCAD software . | Class Room and Computer Lab |

- Manage display in CAD software

Objective: This module covers the skills and knowledge required to Practice to draw two points using Cartesian Notation on graph paper and Set-up drawing area using CAD software

Duration: 19 Hours									Theory: 4Hours			Practice: 15 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Practice to draw two points using Cartesian Notation on graph paper | Trainee will be able to: - Create New Template - Save the File - Create Drawing - Select units as per requirements - Select drawing Limits - Draw A1,1 point on x and y axis keeping origin as reference - Draw B4,2 point on x and y axis keeping origin as reference | - Basic Drawing Settings - Unit setting - Limits setting - User coordinate system - Cartesian coordinate system - Save CAD file Practical activity: Draw a rectangle with first corner at origin. | Theory-1 hours Practice-3 hours Total- 4 hours | Computer with all accessories ,AutoCAD software . | Class Room and Computer Lab |
| - Practice to draw straight line using polar coordinates on graph paper | Trainee will be able to: - Create New Template - Save the File - Create Drawing - Select units as per requirements - Select drawing Limits - Select line command from draw tool bar - Select point 1,1 on graph paper - Type @5<30 in command prompt and press enter - Type @60<0 in command prompt and press enter - Type @20<90 in command prompt and press enter - Press ESC to exit command | - Basic Drawing Settings - Unit setting - Limits setting - User coordinate system - Polar coordinate system - Save CAD file Practical activity: Draw a parallelogram using line command by polar coordinates system. | Theory-1 hours Practice-4 hours Total- 5 hours | Computer with all accessories ,AutoCAD software . | Class Room and Computer Lab |
| - Set-up drawing area using CAD software | Trainee will be able to: - Create New Template - Save the File - Create Drawing - Select units as per requirements - Select drawing Limits - Set-up drawing area as per standard operating procedure | - Basic Drawing Settings - Unit setting - Limits setting - User coordinate system - Polar coordinate system - DIMSTYLE and MTEXT commands - Describe drawing area of CAD - Scales and paper sizes Practical activity: Set the drawing area for basic CAD drawing. | Theory-1 hours Practice-4 hours Total- 5 hours | Computer with all accessories ,AutoCAD software . | Class Room and Computer Lab |
| - Practice for Turning GRID ON and Off and SNAP ON and OFF | Trainee will be able to: - Create New Template - Save the File - Create Drawing - Select units as per requirements - Select drawing Limits - Select Status Bar and click on GRID icon to turn on or turn off grid - Select Status Bar and click on SNAP icon to turn on or turn off snap | - Basic Drawing Settings - Unit setting - Limits setting - User coordinate system - Purpose and function of Grid and Snap command Practical activity: Adjust Grid and SNAP as per drawing requirement. | Theory-1 hours Practice-4 hours Total- 5 hours | Computer with all accessories ,AutoCAD software . | Class Room and Computer Lab |

- Perform function of CAD general commands

Objective: This module covers the skills and knowledge required to Draw a line with LINE command, Widen Border lines with PEDIT and Create layers and move border to its own layer.

Duration: 38 Hours									Theory: 8 Hours			Practice: 38 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required |  | Learning Place |
| --- | --- | --- | --- | --- | --- | --- |
| - Draw a line with LINE command | Trainee will be able to: - Create New Template - Save the File - Create Drawing - Select units as per requirements - Select drawing Limits - Click on LINE icon from draw tool bar - Give dimension of line in Command window OR - Specify Start Point and End Point | - Basic Drawing Settings - Unit setting - Limits setting - User coordinate system - Cartesian coordinate system - Polar coordinate system - Dimension command - Types of dimension Practical activity: Draw hexagon using line command and give dimensions. | Theory-1 hrs Practice-4 hrs Total- 5 hrs | Computer with all accessories ,AutoCAD software . |  | Class Room and Computer Lab |
| - Widen Border lines with PEDIT | Trainee will be able to: - Create New Template - Save the File - Create Drawing - Select units as per requirements - Select drawing Limits - Draw border lines - Type the command PEDIT - Select Width and type a width factor | - Basic Drawing Settings - Unit setting - Limits setting - User coordinate system - Define border lines - PEDIT Command Practical activity: Draw a line with PEDIT command. | Theory-1 hrs Practice-4 hrs Total- 5 hrs | Computer with all accessories ,AutoCAD software . |  | Class Room and Computer Lab |
| - Save Border Template (QSAVE) | Trainee will be able to: - Create New Template - Save the File - Create Drawing - Select units as per requirements - Select drawing Limits - Draw border lines - Type the command CNTL+S to quickly save border template | - Basic Drawing Settings - Unit setting - Limits setting - User coordinate system - Define border lines Practical activity: Apply save command to save drawing in the required format. | Theory-1 hrs Practice-4 hrs Total- 5 hrs | Computer with all accessories ,AutoCAD software . |  | Class Room and Computer Lab |
| - Create layers and move border to its own layer | Trainee will be able to: - Create New Template - Save the File - Create Drawing - Select units as per requirements - Select drawing Limits - Draw different layers as per standard - Draw border as per standard - Select MOVE command - Select border as per standard - Draw border on selected layer | - Basic Drawing Settings - Unit setting - Limits setting - User coordinate system - Define layer - Types of layer - MOVE command Practical activity: Draw a layer for center line. | Theory-1.5 hrs Practice-5 hos Total- 6.5 hrs | Computer with all accessories ,AutoCAD software . |  | Class Room and Computer Lab |
| - Create a layer for Title Block | Trainee will be able to: - Create New Template - Save the File - Create Drawing - Select units as per requirements - Select drawing Limits - Draw border lines - Create a layer as per standard - Draw title block using lines as per standard | - Basic Drawing Settings - Unit setting - Limits setting - User coordinate system - Define title block - Purpose of title block Practical activity: Draw title block using lines as per standard | Theory-1 hrs Practice-4 hrs Total- 5 hrs | Computer with all accessories ,AutoCAD software . | Class Room and Computer Lab |  |
| - Create Title Block | Trainee will be able to: - Create New Template - Save the File - Create Drawing - Select units as per requirements - Select drawing Limits - Draw border lines - Create title block as per standard - Specify cell alignment of title block - Specify scale of title block | - Basic Drawing Settings - Unit setting - Limits setting - User coordinate system - Define title block - Purpose of title block - Scale command Practical activity: Draw a tittle block, align the cell and scale the block. | Theory-1 hrs Practice-4 hrs Total- 5 hrs | Computer with all accessories ,AutoCAD software . |  | Class Room and Computer Lab |
| - Practice for Zoom command | Trainee will be able to: - Create New Template - Save the File - Create Drawing - Select units as per requirements - Select drawing Limits - Type ZOOM in command bar - Select required zoom type for example - All - Center - Dynamic - Extents - Previous - Scale - Window - Object | - Basic Drawing Settings - Unit setting - Limits setting - User coordinate system - DIMSTYLE and MTEXT commands - HATCHING concepts in AutoCAD - Zooming options Practical activity: Apply zoom commands for a drawing. | Theory-1.5 hrs Practice-5 hrs Total- 6.5 hrs | Computer with all accessories ,AutoCAD software . |  | Class Room and Computer Lab |

- Perform Different Cad operation

Objective: This module covers the skills and knowledge required Practice of 3D commands, Practice for filling Title Block, Start a New drawing and practice different shapes.

Duration: 38 Hours									Theory: 8 Hours			Practice: 30 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Practice of 3D commands | Trainee will be able to: - Create New Template - Save the File - Create Drawing - Select units as per requirements - Select drawing Limits - Draw border lines - Draw objects as per standard - use 3D commands to convert objects into 3D as per standards | - Extrude command - Union command - Subtraction command - Intersection command - 3D orbit command - Edit command Practical activity: Draw a rectangle with four holes and convert into 3D object. | Theory-1 hours Practice-14 hours Total- 15 hours | Computer with all accessories ,AutoCAD software . | Class Room and Computer Lab |
| - Practice for filling Title Block | Trainee will be able to: - Create New Template - Save the File - Create Drawing - Select units as per requirements - Select drawing Limits - Draw border lines - Create title block as per standard - Specify cell alignment of title block - Specify scale of title block - Select edit command to fill title block | - Create title block - Annotate command - Table command - Modify command for table Practical activity: Draw a table with a title block and the text will align in the center of cell. | Theory-1 hours Practice-2 hours Total- 3 hours | Computer with all accessories ,AutoCAD software . | Class Room and Computer Lab |
| - Practice for plotting the drawing on plotter or printer | Trainee will be able to: - Create New Template - Save the File - Create Drawing - Select units as per requirements - Select drawing Limits - Draw border lines - Select print command - Select available plotter or printer as per standard | - Plot style manager - Model and layout command - Paper sizes Practical activity: Plot a drawing with model and layout command. | Theory-1 hours Practice-3 hours Total- 4 hours | Computer with all accessories ,AutoCAD software . | Class Room and Computer Lab |
| - Start a New drawing | Trainee will be able to: - Start auto cad program - Press CTRL+ N or select New in file menu - Select units as per requirements - Select drawing limits | - Basic drawing setting - Unit setting - Limit setting Practical activity: Sett the units and limits for a new drawing in CAD. | Theory-1 hours Practice-2 hours Total- 3 hours | Computer with all accessories ,AutoCAD software . | Class Room and Computer Lab |
| - Practice with edit command to make changes in the drawing | Trainee will be able to: - Create New Template - Save the File - Create Drawing - Select units as per requirements - Select drawing Limits - Draw border lines - Type command DDEDIT - Select line whose dimension needs to be edited | - DDEDIT command Practical activity: Edit single line text and dimension text of a drawing with DDEDIT command. | Theory-1 hours Practice-3 hours Total- 4 hours | Computer with all accessories ,AutoCAD software . | Class Room and Computer Lab |
| - Draw an Angled line | Trainee will be able to: - Create New Template - Save the File - Create Drawing - Select units as per requirements - Select drawing Limits - Draw border lines - Type command ALIGNED LINE and type magnitude and angle of line - Type LINE command and type @20<45 in command bar - Select LINE icon and specify first and second point with mouse cursor on desired angle | - Define line - Line command - Polar coordinate system Practical activity: Create a line at angle using polar coordinate system. | Theory-1 hours Practice-2 hours Total- 3 hours | Computer with all accessories ,AutoCAD software . | Class Room and Computer Lab |
| - Practice with Undo Command and Redo command | Trainee will be able to: - Create New Template - Save the File - Create Drawing - Select units as per requirements - Select drawing Limits - Select CIRCLE command and draw a circle of desired dia - Press CTRL+Z to undo that circle - Press CTRL+Y to redo that circle | - Undo command - Redo command Practical activity: Delete a circle from a rectangle with undo command and bring back with redo command. | Theory-1 hours Practice-2 hours Total- 3 hours | Computer with all accessories ,AutoCAD software . | Class Room and Computer Lab |
| - Draw a circle with circle command | Trainee will be able to: - Create New Template - Save the File - Create Drawing - Select units as per requirements - Select drawing Limits - Select CIRCLE command - Select circle type e.g. center radius, center diameter, 2-point, 3-point and draw a circle of desired dia or radius | - Define circle - Techniques to draw circle - Circle Command Practical activity: Draw circle with centre radius, centre diameter, 2-point, 3-point Command. | Theory-1 hours Practice-2 hours Total- 3 hours | Computer with all accessories ,AutoCAD software . | Class Room and Computer Lab |

# 9.3 Occupational Health, Safety and Environment

- Read and Develop Career Professionalism

Objective: This Module covers the knowledge, skills and attitudes to Integrate personal objectives with organizational goals, Set and meet work priorities and Maintain professional growth and development

Duration: 5 Hours									Theory: 2 Hours				Practice: 3 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Integrate personal objectives with organizational goals | Trainee will be able to: - Personal growth and work plans are pursued towards improving the qualifications set for the profession - Maintain the Intra- and interpersonal relationships in the course of managing oneself based on performance evaluation. - Demonstrate the commitment to the organization and its goal in the performance of duties | Goal alignment: Set objective which are in line with business priorities. Goals must be smart, and time bound Practical activity: Ask your Line Manager to give you current goals of the organization and therefore identify your objectives which should be in line with business priorities e.g. Uninterrupted supply of compressed air to run the plant | Theory- 0.3 hrs Practice- 0.5 hrs Total- 0.8 hrs | Paper Pencil Eraser | Class Room |
| LU2. Set and meet work priorities | Trainee will be able to: - Competing demands are prioritized to achieve personal, team and organizational goals and objectives. - Manage work priorities and commitments in order to utilize the resources efficiently and effectively. - Practices along economic use and maintenance of equipment and facilities are followed as per established procedures | - Resources: Human, Financial, Technology, Hardware, Software - Evaluation: Performance Appraisal, Psychological Profile, Aptitude Tests. - Recognitions: Recommendations, Citations, Certificate of Appreciations, Commendations, Awards, Tangible and Intangible Rewards. Practical activity: Develop 5 objectives relevant to maintenance of equipment e.g. timely execution of periodic maintenance plans and keep your skill set up-to date Time management is the key to achieve objectives | Theory-0.3 hrs Practice- 0.5 hrs Total- 0.8 hrs | Paper Pencil Eraser | Class Room |
| LU3. Maintain professional growth and development | Trainee will be able to: - Identify the trainings and career opportunities and avail on the bases of job requirements. - Recognitions are sought/received and demonstrated as proof of career advancement - Obtained and renewed the licenses and/or certifications relevant to job and career. | - Self-Development: Identify your training needs to develop competencies in order to perform given work profile Practical activity: To make yourself successful in the current job profile, make a plan with 12 months for your self-development covering functional and leadership competencies e.g. learning the operation of honing machine (functional), engage in team work (leadership). These will be evaluated by the line manager in Appraisal along with objectives (LU1) | Theory-0.8 hrs Practice- 1 hrs Total- 1.8 hrs | Paper Pencil Eraser | Class Room |

- Apply basic Occupational Health & Safety regarding heavy machinery

Objective: This competency standard covers the skills and knowledge required to Adopt Health & Safety regulations, encourage primary safety program, Ensure Personal protective equipment (PPE), Ensure Safety documentation common to Heavy Machines, Assess Worksite hazards, Judge Dangerous Trees, protect wildlife, Adopt Safety during fire season and Perform Map orientation.

Duration: 5 Hours									Theory: 2 Hours				Practice: 3 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Adopt Health & Safety regulations. | Trainee will be able to: - Identify rights & responsibilities regarding safety - Interpret regulations & guidelines specific to Heavy Machines. - Interpret common safety rules and tips. - Identify employer safety rules and policies. | - Learn about the safety responsibilities of workers and employers according to Govt. of Pakistan (Pakistan Factory Act 1934) - Learn about the safe work method of heavy machinery - Learn common safety guidelines of the industry Practical activity: Differentiate between responsibilities of workers and employers | Theory- 0.3 hrs Practice- 0.5 hrs Total- 0.8 hrs | Paper Pencil Eraser | Class Room |
| - Encourage primary safety program | - Trainee will be able to: - Motivate by regulation. - Motivate by ethics, legitimate concern - Motivate by cost of lost time and injury Claims. - Motivate by liability organizational goals and objectives. | Learn about the financial, social and moral impact of incidents on personal life and organization Practical activity: Enlist 5 impacts each; financial, social and moral | Theory-0.3 hrs Practice- 0.5 hrs Total- 0.8 hrs | Paper Pencil Eraser | Class Room |
| - Ensure Safety documentation common to Heavy Machines. | Trainee will be able to: - Construct safety network flag person certification. - Ensure Workplace Hazardous Materials Information System. (WHMIS) - Prepare Material Safety Data Sheets (MSDS) - Ensure symbols identifying dangerous goods. - Follow safety while transporting dangerous goods | - Identify hazardous materials on site. - Enlist various types of personal protective equipment. - Define material Safety Data Sheets (MSDS). - Describe Workplace Hazardous Materials Information System. (WHMIS) Practical activity: Enlist 5 hazards related to heavy machinery, enlist mandatory PPEs and signages for construction site. | Theory-0.8 hrs Practice- 1 hrs Total- 1.8 hrs | Paper Pencil Eraser Safety signs PPEs | Class Room and Site Visit |
| - Assess Worksite hazards | - Ensure safety during Steep terrain movement. - Ensure safe terrain stability - Evaluate overhead High voltage wires. - Estimate overhead obstacles, overhangs & demolition | - Observe and identify Worksite hazards Practical activity: Enlist 8 hazards related to work site | Theory-0.8 hrs Practice- 1 hrs Total- 1.8 hrs | Paper Pencil Eraser Safety signs PPEs | Class Room and Site Visit |
| - Judge Dangerous trees. | - Identify Snags. (Snag is any dry standing Tree) - Down all snags. - Flagg (Huge snag if left standing) | - Observe and identify snags & flagg Practical activity: - Enlist snags & flag and its mitigation measure | Theory-0.8 hrs Practice- 1 hrs Total- 1.8 hrs | Paper Pencil Eraser Safety signs PPEs | Class Room and Site Visit |
| - Protect wildlife | - Identify Osprey-nest. - Mark nest on the map. - Leave the trees around for animals /bear in a den. | - Observe and identify Worksite hazards Practical activity: - In a park/forest, before starting work perform risk assessment to protect animals’ habitat. Enlist birds’ nest and squirrel trees and mark on site map | Theory-0.8 hrs Practice- 1 hrs Total- 1.8 hrs | Paper Pencil Eraser Safety signs PPEs | Class Room and Site Visit |
| - Adopt Safety during fire season | - Ensure designated fire equipment with the machine. - Ensure fire watch, fire trucks are ready to go. - Shut down the operation when humidity becoming lower than the temperature. - Calculate degree of hazards depends on the level of humidity versus temperature. | - Observe and identify Worksite hazards Practical activity: - In an open area, forest or tree cutting site, note down degree of hazards depends on the level of humidity versus temperature. | Theory-0.8 hrs Practice- 1 hrs Total- 1.8 hrs | Paper Pencil Eraser Safety signs PPEs | Class Room and Site Visit |
| - Perform Map orientation | - Identify hazards on map like Rocks, construction, pipelines, boundaries, power lines, soft ground, quarries and tree lines. - Follow map and evaluate all the issues like terrain of an area, slope percentage | - Observe and identify Worksite hazards Practical activity: - Take a blue print of drawing before digging activity and mark utilities line (water, natural gas, IT, electricity cables). After marking on map, mark on site as well. | Theory-0.8 hrs Practice- 1 hrs Total- 1.8 hrs | Paper Pencil Eraser Safety signs PPEs | Class Room and Site Visit |

- Perform general health, safety and environment practices

Objective: This competency standard covers the skills and knowledge required to Ensure personal protective equipment (PPE), Protect Tools and Equipment, Maintain First aid Box, Ensure Safeguard of Machines, Adopt Environmental Regulation, Adopt company policies and procedures, Follow federal, provincial/ territorial, and municipal legislation, Attain health & safety training, Prepare for emergencies, Respond to emergencies, Monitor activities of people, vehicles, and other equipment in area..

Duration: 5 Hours									Theory: 2 Hours				Practice: 3 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Identify hazards relevant to your task | Trainee will be able to: - Identify hazards correctly in accordance with OHS standards - Identify safety signs and symbols - Identify unsafe act and conditions | - Define Hazard. - Describe types of hazard. Practical activity: Visit an area (Mechanical Workshop) and enlist 05 different types of hazards with control measures | Theory- 0.3 hrs Practice- 0.5 hrs Total- 0.8 hrs | Paper Pencil Eraser | Class Room/ Mechanical Workshop |
| LU2. Ensure personal protective equipment (PPE) | - Arrange PPEs as per requirement - Wear proper PPE as per nature of job - Store PPE at appropriate place after use | - Knowledge and proper use of Personal Protective Equipment (PPE). Practical activity: Demonstrate how to properly wear earplugs, Helmet with chin strip and safety harness (if available) | Theory-0.3 hrs Practice- 0.5 hrs Total- 0.8 hrs | Paper Pencil Eraser PPEs | Class Room / Work site |
| LU3. Protect Tools and Equipment | - Ensure insulation of tools and equipment - Store tools and equipment safely - Clean tools on a regular basis before stacking | - Knowledge about cleaning of workplace with regards to Safety Practical activity: Visit workshop and arrange tools & equipment in 5S order, Sort, Set in order, shine, sustain | Theory-0.8 hrs Practice- 1 hrs Total- 1.8 hrs | Paper Pencil Eraser Safety signs | Class Room and wood workshop |
| LU4. Maintain First aid Box | - Ensure availability of first aid box - Check first aid box for requisite emergency medicines - Check expiry date of medicines - Perform first aid treatment against electric shock - Perform first aid treatment / bandages against minor injuries | - Knowledge about First-Aid-Box. - Usage of first aid box Practical activity: Make a checklist of First Aid Supplies and review it on monthly basis for replenishment of stock and expiry date | Theory-0.3 hrs Practice- 0.5 hrs Total- 0.8 hrs | Paper Pencil Eraser PPEs First Aid Box | Class Room / Work site |
| LU5. Ensure Safeguard of Machines | - Check safety guards of machine - Check brake of machines - Check controlling devices of machine - Perform test operation on machine | - Describe Machine & equipment safety. Develop knowledge about Lock-Out Tag Out of machine before making any intervention Practical activity: Visit mechanical workshop in supervision of your instructor and isolate the energy sources of lathe machine and check gear box | Theory-0.3 hrs Practice- 0.5 hrs Total- 0.8 hrs | Paper Pencil Eraser PPEs LOTO box | Class Room / Work site |
| LU6. Adopt Environmental Regulation | - Locate applicable permits on job site - Ensure work friendly environment - Adopt environmental regulations. | Use environmental sustainability approach in workplace Practical activity: Visit a construction site and evaluate if the work is carried about after seeking NOC from Govt. | Theory-0.3 hrs Practice- 0.5 hrs Total- 0.8 hrs | Paper Pencil Eraser PPEs First Aid Box | Class Room / Work site |
| LU7. Adopt company policies and procedures | - Ensure company policy and procedures - Adopt company procedures. | Implement and adhere Occupational Health & Safety policies & SOPs of the company. Practical activity: - Adopt safe way of working during assigned task. Go to workshop and perform wood cutting by adopting “Safe way of working” | Theory-0.3 hrs Practice- 0.5 hrs Total- 0.8 hrs | Paper Pencil Eraser | Class Room / Workshop/ Work site |
| LU8. Follow federal, provincial/ territorial, and municipal legislation | - Locate relevant section and legislation - Seek clarification of legislation - Adopt regulation of the area | - Follow Occupational Health & Safety Regulations. Practical activity: Check drainage system of your facility/any manufacturing site through its layouts, it should have separate lines for storm water, sanitary waste and process effluent | Theory-0.3 hrs Practice- 0.5 hrs Total- 0.8 hrs | Paper Pencil Eraser PPEs | Class Room / Work site |
| LU9. Attain health & safety training | - Take required health and safety training - Implement work place hazardous materials information system (WPHMIS) - Adopt first aid, cardio for respiratory, resuscitation, and CPR | - Build competency by acquiring relevant training of HSE Practical activity: Do a self-assessment of your routine job, and enlist you have any training needs for the given role. E.g. Consider basic first aid and fire-fighting training and plan it with Rescue 1122 | Theory-0.3 hrs Practice- 0.5 hrs Total- 0.8 hrs | Paper Pencil Eraser | Class Room / Work site |
| LU10. Prepare for emergencies | - Take emergency response training. Ensure emergency response exercises - Adopt first aid, cardio for respiratory, resuscitation, and CPR - Adopt first aid, cardio for respiratory, resuscitation, and CPR | - Emergency response plan development and implementation Practical activity: - Develop a Fire ERP for workers in a closed building | Theory-0.3 hrs Practice- 0.5 hrs Total- 0.8 hrs | Paper Pencil Eraser PPEs First Aid Box | Class Room / Work site |
| LU11. Respond to emergencies | - Follow emergency plan - Communicate instructions - Assess risk and determine course of action - Operate emergency equipment and supplies | - Learn to operate Fire extinguisher and usage of first aid supplies - Learn to take control as safety warden Practical activity: - Identify DCP type fire extinguisher and operate it. Perform a crepe bandage on ankle of your fellow colleague | Theory-0.3 hrs Practice- 0.5 hrs Total- 0.8 hrs | Paper Pencil Eraser PPEs First Aid Box Fire Extinguisher | Class Room / Work site |
| LU12. Monitor activities of people,vehicles, and other equipment in area | - Identify movement of others in work area - Respond to signals or traffic control person - Communicate with site person | - Assess Typical worksite Hazards. Practical activity: Perform risk assessment of vehicles routes and suggest improvements w.r.t safety | Theory-0.3 hrs Practice- 0.5 hrs Total- 0.8 hrs | Paper Pencil Eraser PPEs | Class Room / Work site |
| LU13. Investigate incident at workplace | - Identify incidents causes - Collect relevant data for evidences - Analyze the accident and plan a control measure - Implement the plan | - Investigate accident history in different areas Practical activity: Investigate slip, trip and fall (below 2 meters height) accidents by drilling down the root cause. Suggest control measure to prevent slip, trip and fall | Theory-0.3 hrs Practice- 0.5 hrs Total- 0.8 hrs | Paper Pencil Eraser | Class Room / Work site |

- List of Machinery, Tools & Equipment.

(FOR A CLASS OF 25 STUDENTS)

## 10.1. Workshop Practice-I

| S# | Tools and Equipment | Quantity | S# | Consumable Material | Quantity |
| --- | --- | --- | --- | --- | --- |
| 1 | Work bench | 05 Nos | 1 | Cutting oil | 25 Liters |
| 2 | Bench vices | 25 Nos | 2 | Grese | 10 Kg |
| 3 | Hammer | 25 Nos | 3 | Welding electrode Size 12 No and10 No | 5 Packet each |
| 4 | Tri-square | 25 Nos | 4 | Metal Filler rod | 5 Kg |
| 5 | Hand hacksaw | 25 Nos | 5 | Welding flux | 5 Packet |
| 6 | Scriber | 25 Nos | 6 | Solder wire | 0.5 kg |
| 7 | Vernier caliper | 25 Nos | 7 | Copper Filler rod | 1 kg |
| 8 | Flat File | 25 Nos | 8 | Cotton waste | 10 kg |
| 9 | Number/alphabet punch | 25 Nos | 9 | Kerosene Oil | 10 liters |
| 10 | Round file | 25 Nos |  |  |  |
| 11 | Metal working chisel | 25 Nos |  |  |  |
| 12 | Punching tools | 25 Nos |  |  |  |
| 13 | Tap & Die set | 25 Nos |  |  |  |
| 14 | Pipe cutter | 25 Nos |  |  |  |
| 15 | Pipe vice | 25 Nos |  |  |  |
| 16 | D-type bevel protector | 25 Nos |  |  |  |
| 17 | Grinding Machine | 02 Nos |  |  |  |
| 18 | Personal Protective Equipment | 25 Nos |  |  |  |
| 19 | Wheel Dresser stand | 05 Nos |  |  |  |
| 20 | Dresser | 05 Nos |  |  |  |
| 21 | Lathe Machine | 20 Nos |  |  |  |
| 22 | Cutting Tools | 20 Nos |  |  |  |
| 23 | Measuring Tools | 25 Nos |  |  |  |
| 24 | Files | 25 Nos |  |  |  |
| 25 | Thread gauges | 25 Nos |  |  |  |
| 26 | Threading Tools | 25 Nos |  |  |  |
| 27 | Drilling Machines | 02 Nos |  |  |  |
| 28 | Drill chuck with Key | 05 Nos |  |  |  |
| 29 | Machine Vice | 25 Nos |  |  |  |
| 30 | Marking Tools | 25 Nos |  |  |  |
| 31 | Drill Sleeve and Socket | 05 Nos |  |  |  |
| 32 | Counter drill | 05 Nos |  |  |  |
| 33 | Shaper, Planar or Slotter | 1 No each |  |  |  |
| 34 | Dial indicator with magnet stand | 25 Nos |  |  |  |
| 35 | Point cutting tools | 25 Nos |  |  |  |
| 36 | Bevel protector | 25 Nos |  |  |  |
| 37 | Layout tools. | 25 Nos |  |  |  |
| 38 | Measuring devices. | 25 Nos |  |  |  |
| 39 | Hand held calculator. | 25 Nos |  |  |  |
| 40 | Hand drill machine | 25 Nos |  |  |  |
| 41 | Disk grinder | 05 Nos |  |  |  |
| 42 | Pin grinder | 05 Nos |  |  |  |
| 43 | Oxygen cylinder | 10 Nos |  |  |  |
| 44 | Acetylene gas cylinder | 10 Nos |  |  |  |
| 45 | Pressure regulators | 10 Nos |  |  |  |
| 46 | Cylinder key | 10 Nos |  |  |  |
| 47 | Welding torch | 10 Nos |  |  |  |
| 48 | Rubber house pipe | 10 Nos |  |  |  |
| 49 | Back fire arrester | 10 Nos |  |  |  |
| 50 | Spark lighter | 10 Nos |  |  |  |
| 51 | Steel wire brush | 10 Nos |  |  |  |
| 52 | Pedestal grinder | 02 Nos |  |  |  |
| 53 | Solder gun | 25 Nos |  |  |  |
| 54 | Marking tools | 25 Nos |  |  |  |

## 10.2. Basic Engineering Drawing and CAD/CAM

| S# | Tools and Equipment | Quantity | S# | Consumable Material | Quantity |
| --- | --- | --- | --- | --- | --- |
| 1 | Drawing Board/Table. | 25 Nos | 1 | Graph and drawing sheet. | 500 No each |
| 2 | T-Square | 25 Nos | 2 | Templets. | 25 Nos |
| 3 | Set Square. | 25 Nos | 3 |  |  |
| 4 | Geometry Box. | 25 Nos | 4 |  |  |
| 5 | Tee-square and compass. | 25 Nos | 5 |  |  |
| 6 | Computer with all accessories | 25 Nos | 6 |  |  |
| 7 | AutoCAD software disk | 25 Nos | 7 |  |  |
| 8 | Models | 25 Nos | 8 |  |  |

## 10.3. Occupational Health, Safety and Environment

| S# | Tools and Equipment | Quantity | S# | Consumable Material | Quantity |
| --- | --- | --- | --- | --- | --- |
| 1 | Steel-toed footwear, | 25 Nos |  |  |  |
| 2 | Hard hat, | 25 Nos |  |  |  |
| 3 | Safety gloves, | 25 Nos |  |  |  |
| 4 | Appropriate safety glasses, | 25 Nos |  |  |  |
| 5 | High visibility vest, | 25 Nos |  |  |  |
| 6 | Hearing protection, | 25 Nos |  |  |  |
| 7 | Breathing apparatus, | 05 Nos |  |  |  |
| 8 | De-electric boots and gloves for protection from electrical shock. | 25 Nos |  |  |  |
| 9 | Site emergency response plan, | 05 Nos |  |  |  |
| 10 | Fire extinguishers, | 05 Nos |  |  |  |
| 11 | Fire blankets, | 25 Nos |  |  |  |
| 12 | Respirators, masks, | 25 Nos |  |  |  |
| 13 | Fire hoses, | 05 Nos |  |  |  |
| 14 | First aid kits, stretchers, | 05 Nos |  |  |  |
| 15 | Safety standard books | 05 Nos |  |  |  |

- Members of the Curriculum Development Committee.

| S# | Name | Designation | Organization |
| --- | --- | --- | --- |
| 1 | Engr Inayat Ur Rehman | DACUM Facilitator | Ex Director Academics KP-TVETA. |
| 2 | Mr. Adil Qadeer | Lecture | University of Lahore, |
| 3 | Mr. Muhammad Arsalan | Lecture | University of Lahore, |
| 4 | Engr Jamal Akbar | Associate Prof | GPI Sardar Garhi, Peshawar KP-TVETA |
| 5 | Engr Farooq Iftikhar | Metallurgy engineer | Pakistan Institute of Technology for Advanced Engineering Material, Lahore |
| 6 | Engr Umar, | Instructor | Swedish College Gujarat, P-TEVTA |
| 7 | Mr. Amir Amin | HOD | Polytechnic College Lahore, |
| 8 | Mr Muhammad Rizwan Zafar | HOD, Mechanical, | Technology college Lahore, |
| 9 | Ms Saba Sadiq | Lecture Mechanical, | UOL, Campus, Islamabad, |
| 10 | Fazal Rehman, | Associate Professor | GCT Peshawar, KP TEVA, |
| 11 | Saad Amjad | Project Engineer, | Cagrow consulting Company Lahore |
| 12 | Umar Qureshi | Director, | Pakistan National Accreditation Council, Islamabad |
| 13 | Syed Muhammad Syed Arsalan | Lecture, | GCT Railway Road Lahore |
| 14 | Amina Irfan | Lecture | MS Industrial Engineering, Lahore |
| 15 | Mr. Muhammad Yasir, | Dy. Director | NAVTTC, Islamabad |