Curriculum of National Certificate Level - 3 in Construction Machinery

Level-3 Equivalent to G-II

Curriculum

National Vocational and Technical Training Commission (NAVTTC)

Government of Pakistan

Contents

1.	Introduction	4

2.	Purpose of the qualification	5

3.	Overall objectives of training program	5

4.	Entry level of trainees	6

5.	Minimum qualification for teachers	6

6.	Duration of the course:	6

7.	Description and structure of the course	7

8.	Scheme of Studies.	8

9.	Detail of Modules.	9

National Certificate Level-3 in Construction Machinery	9

9.4 Internal Combustion Engine	9

9.4.1	Identify Engine Types, their Component and Specifications	9

9.4.2	Disassemble and assemble Basic Engine Parts	12

9.4.3	Maintain Engine Cooling System Components and Their Relationship	16

9.4.4	Inspect Engine Lubrication System and working of its Components	18

9.4.5	Locate Diesel Engine’s Fuel System Components	20

9.4.6	Maintain the Engine intake and Exhaust System and its Components	24

9.4.7	Perform Overhauling of Four Stroke Engine	26

9.5 Automotive Electrics and Electronics	28

9.5.1	Perform Basic Electricity Measurements	28

9.5.2	Identify Electromagnetism and its application	30

9.5.3	Perform Battery Service of Vehicle	37

9.5.4	Repair Engine Starting System	39

9.5.5	Repair Battery Charging System	42

9.5.6	Repair Spark Ignition System of Internal Combustion Engine	44

9.5.7	Adjust Ignition Timing of Spark Ignition Engine	46

9.5.8	Repair Distributer-Less Ignition System	48

9.5.9	Repair Lightning System of Vehicle	50

9.5.10	Repair Cooling System and Vehicle Accessories	52

9.5.11	Recognize Electrical system of Vehicle	54

9.6 Soft Skills	59

10.6.1.	Participate in Workplace Communication	59

10.6.2.	Practice Occupational Health and Safety Procedures	62

10.6.3.	Perform Work in Team Environment	65

10.	List of Machinery, Tools & Equipment.	67

10.1. Internal Combustion Engine	67

10.2. Automotive Electrics and Electronics	68

11.	Members of the Curriculum Development Committee.	69

# Introduction

Construction Machinery Operators and Technicians are in demand across the country and abroad. Their services are required for everything from road and bridge construction, back hoe operation, bulldozing, loading and grading, dredging and heavy paving, to excavating and much, much more. This is a good career opportunity for a reliable and responsible individual with a strong work ethic. Heavy Machine Operators and Technicians not only work on regular construction building jobs, but also on infrastructure projects (roads, bridges, Dams and ports, otherwise called non-building construction), and in mining and timber operations.

In order to build the capacity of technical and vocational training institutes in Pakistan through provision of demand driven competencies-based trainings in construction sector the NAVTTC, and TEVT Sector Support Program (TSSP) have joined hands together to develop qualifications for construction sector. These qualifications will not only build the capacity of existing workers of this sector but also support the youth to acquire skills best fit for this sector. The benefits and impact of development of these qualifications will be on both demand and supply side.

Based upon this demand of industry these competency‐based qualifications for Construction Machinery are developed under National Vocational Qualification Framework (NVQF) (Level 3).  The qualifications mainly cover competencies along with related knowledge and professional skills which are essential for getting a job or self-employed.

The National Vocational & Technical Training Commission (NAVTTC) has approved the Qualification Development Committee (QDC). The QDC consists experts from the relevant industries from different geographical locations across Pakistan and academicians who were consulted during the development process to ensure input and ownership of all the stakeholders. The National Competency Standards could be used as a referral document for the development of curricula to be used by training institutions.

Construction Machinery curriculum will prepare students to efficiently operate heavy machines such as Dozers, Loaders, Excavator and Graders and to perform basic preventive maintenance on most types of heavy equipment. Coursework includes Safety, Heavy Machines operations, grades, Legal & environmental concerns & equipment maintenance. Graduates of this program may find employment with state and local government agencies and private contractors engaged in highway or other construction activities.

In fact, no sphere of life may be identified that does not include the contribution of construction machinery. Thus, the importance of construction machinery may be determined according to its usefulness in our daily life. Therefore, industry expectations for skilled workforce are also dynamic which can only be managed through setting relevant competency standards in collaboration with the leading industries. Being cognizant of this fact, National Vocational & Technical Training Commission (NAVTTC) developed competency standards for Mechanical technology with Specialization in Construction Machinery under National Vocational Qualifications Framework (NVQF). These competency standards have been developed by a Qualifications Development Committee (QDC) and validated by the Qualifications Validation Committee (QVC) having representation from the leading Construction, Real States & Builder industry of the country.

# Purpose of the qualification

The purpose of these qualifications is to set high professional standards for Construction Machinery. The specific objectives of developing these qualifications are as under:

- Improve the professional competence of the trainees

- Provide opportunities for recognition of skills attained through non-formal or informal pathways

- Improve the quality and effectiveness of training and assessment for Construction Machinery.

- Enable the existing workforce to capacitate themselves in new technologies and method so that train persons in this field can maintain construction machinery for its longer

# Overall objectives of training program

The Construction Machinery qualification of level 3 consists of both the theoretical and practical details and having the following Occupations

- Internal Combustion Engine

- Automotive Electrics and Electronics

- Generic

# Entry level of trainees

The entry for National Certificate level 3, in Construction Machinery are

- A person having National Vocational Certificate level 2, in Construction Machinery.

# Minimum qualification for teachers

- B. Tech in Mechanical Technology 4 Years

- B.Sc. / B.E. in Mechanical Engineering 4 Years

- D. A. E. in Mechanical Technology with 3 Years teaching experience

- Must be able to communicate effectively

Medium of instruction

English, and Urdu.

# Duration of the course:

The proposed Modules are composed of 21 competencies that will be covered in 600 Learning hours. Duration of the course is proposed to be Three years. The total weightage for technical competencies is 600 hours. A total of 200 hours has been reserved for allied subjects i.e. Islamic studies, English, Mathematics, Physics and Chemistry.

The details of technical Modules are given as under:

Level - 2 = 6 months.

Level - 3 = 6 months.

Level - 4 = 1 Year.

Level - 5 = 1 Year.

The distribution of contact hours and credit hours in level-3 is given below:

Level – 3 = 6 months (16 weeks)

Total.	600 hours. & 60 Credits

Theory.	123 hours (20.5 %)

Practice.	477 hours (79.5 %)

# Description and structure of the course

Following is the structure of the course:

- National Qualification Certificate level-3 in “Construction Machinery”

            - Internal Combustion Engine

| Code | Competency Standards | TH | PR | TL | Credits | Category |
| --- | --- | --- | --- | --- | --- | --- |
| CS-24 | Identify Engine Types Their Component and its Specifications | 6 | 24 | 30 | 3 | Technical |
| CS-25 | Disassemble and assemble Engine Parts | 6 | 24 | 30 | 3 | Technical |
| CS-26 | Maintain Engine Cooling System Components and Their Relationship | 6 | 24 | 30 | 3 | Technical |
| CS-27 | Inspect Engine Lubrication System and working of its Components | 6 | 24 | 30 | 3 | Technical |
| CS-28 | Locate Diesel Engine’s Fuel System Components | 6 | 24 | 30 | 3 | Technical |
| CS-29 | Maintain the Engine intake and Exhaust System Components | 6 | 24 | 30 | 3 | Technical |
| CS-30 | Perform Overhauling of Four Stroke Engine | 10 | 40 | 50 | 5 | Technical |
| Total |  | 47 | 171 | 218 | 21.8 |  |

            - Automotive Electrics and Electronics

| Code | Competency Standards | TH | PR | TL | Credits | Category |
| --- | --- | --- | --- | --- | --- | --- |
| CS-31 | Perform Basic Electricity Measurements | 7 | 30 | 37 | 3.7 | Technical |
| CS-32 | Identify The Electromagnetism And Its Application | 6 | 30 | 36 | 3.6 | Technical |
| CS-33 | Perform Battery Service Of Vehicle | 4 | 18 | 22 | 2.2 | Technical |
| CS-34 | Repair Engine Starting System | 6 | 18 | 24 | 2.4 | Technical |
| CS-35 | Repair Battery Charging System | 6 | 18 | 24 | 2.4 | Technical |
| CS-36 | Repair Spark Ignition System Of Internal Combustion Engine | 6 | 27 | 33 | 3.3 | Technical |
| CS-37 | Adjust Ignition Timing of Spark Ignition Engine | 6 | 27 | 33 | 3.3 | Technical |
| CS-38 | Repair Distributer-Less Ignition System | 6 | 27 | 33 | 3.3 | Technical |
| CS-39 | Repair Lightning System of Vehicle | 6 | 21 | 27 | 2.7 | Technical |
| CS-40 | Repair Cooling System and Vehicle Accessories | 8 | 36 | 44 | 4.4 | Technical |
| CS-41 | Recognize Electrical System of vehicle | 9 | 36 | 45 | 4.5 | Technical |
| Total |  | 70 | 288 | 358 | 35.8 |  |

            - Generic

| Code | Competency Standards | TH | PR | TL | Credits | Category |
| --- | --- | --- | --- | --- | --- | --- |
| CS-42 | Participate In Workplace Communication | 2 | 6 | 8 | 0.8 | Generic |
| CS-43 | Practice Occupational Health and Safety Procedures | 2 | 6 | 8 | 0.8 | Generic |
| CS-44 | Perform Work In Team Environment | 2 | 6 | 8 | 0.8 | Generic |
| Total |  | 6 | 18 | 24 | 2.4 |  |
| Grand Total of Level-3 |  | 123 | 477 | 600 | 60 |  |

# Scheme of Studies.

National Certificate Level-3 in Construction Machinery

| S.No. | Occupation or Subject | Category | Total Contact Hours |  |  | Credit | Periods Per Week |  |  | No. of Weeks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Th | Pr. | Total |  | Th | Pr | Total |  |
|  | Internal Combustion Engine | Technical | 47 | 171 | 218 | 21.8 | 2.94 | 10.69 | 13.63 | 16 |
|  | Automotive Electrics and Electronics | Technical | 70 | 288 | 358 | 35.8 | 4.38 | 18.00 | 22.38 | 16 |
|  | Generic | Technical | 6 | 18 | 24 | 2.4 | 0.38 | 1.13 | 1.50 | 16 |
|  |  | Allied |  |  |  |  |  |  |  | 16 |
|  |  | Allied |  |  |  |  |  |  |  | 16 |
|  | Total |  | 123 | 477 | 600 | 60 | 7.69 | 29.81 | 37.50 |  |

- Detail of Modules.

## National Certificate Level-3 in Construction Machinery

## 9.4 Internal Combustion Engine

- Identify Engine Types, their Component and Specifications

Objective: This module covers the skills and knowledge required to Recognize Petrol Engine, Recognize Diesel Engine, Analyze the functioning of Diesel Engine and Analyse the CI engine Ignition system.

Duration: 25 Hours									Theory: 6 Hours			Practice: 19 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Recognize Petrol Engine | Trainee will be able to: - Locate spark plugs and it functions - Point out distributor and its function - Identify ignition coil and its functions - Identify plug leads and its functions - Identify carburetor and its function - Locate EFI system and its main components | - Types (Petrol four stroke &two stroke engine) - Differentiate petrol EFI and carburetor engine. - Aspirated (Natural, turbocharged, supercharged) | Theory-1.5Hrs Practice-5 Hrs Total- 6.5 Hrs | Measuring tools (Vernier caliper, micrometer, Hand tools trolley Petrol Engine | Class Room and IC Lab |
| - Recognize Diesel Engine | Trainee will be able to: - Identify the typical diesel engines and function of their main components. - Locate fuel injectors and its function - Point out high pressure fuel pump and its functions - Identify heater plug and its function - Identify the Common rail fuel system. - Identify HEUI fuel system | - Types (diesel four & two stroke) - Cylinder layout (single, multi, in-line, V shape, horizontally opposed). - Location of engine (front, rear, transverse, longitudinal) - Capable to explain different parts in diesel engine. | Theory-1.5Hrs Practice-05Hrs Total- 6.5 Hrs | Measuring tools (Vernier caliper, micrometer, Hand tools trolley Diesel Engine | Class Room and IC lab |
| - Point out the main components of Diesel Engine | Trainee will be able to: - Locate the cylinder, cylinder head and its basic function - Identify the function of piston. - Identify the function of connecting rod, crank shaft and valves. - Locate the flywheel, camshaft, timing gears and its function | - explain different parts in diesel engine - Cylinder layout (single, multi, in-line, V shape, horizontally opposed). - Location of engine (front, rear, transverse, longitudinal) | Theory-2Hrs Practice-4 Hrs Total- 06 Hrs | Diesel Engine Special tools (ring compressor, belt tensioner, torque wrench) | Class Room and IC Lab |
| - Analyse the Petrol/diesel engine Ignition system. | Trainee will be able to - Inject oil into the combustion chamber - Atomize the oil jet droplets - Evaporate the droplets. - Mix the vapors with hot air and form combustible mixture | - Types (Petrol four stroke, diesel four stroke). - Aspirated (Natural, turbocharged, supercharged) - Cylinder layout (single, multi, in-line, V shape, horizontally opposed). - Location of engine (front, rear, transverse, longitudinal - Differentiate petrol EFI and carburetor engine. - Explain different parts in petrol/diesel engine. | Theory-01Hrs Practice-05 Hrs Total- 06 Hrs | Special tools (ring compressor, belt tensioner, torque wrench) Measuring tools (Vernier caliper, micrometer, Hand tools trolley Diesel Engine | Class Room and IC Lab |

- Disassemble and assemble Basic Engine Parts

Objective: This module covers the skills and knowledge required to Perform Engine block disassembly procedure, Carry out Engine block assembly procedure, Perform Engine head disassembly procedure and Carry out Engine head assembly procedure

Duration: 36 Hours									Theory: 07 Hours			Practice:  Hours 29

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Disassemble Engine block. | Trainee will be able to: - Select appropriate tools. - Perform disassembly of engine according to manufacturing shop manual. - Inspect the disassemble components | - Describe the engine terms as following: Top dead Centre (TDC), bottom dead Centre (BDC), bore, stroke, engine capacity, clearance volume, swept volume, compression ratio, single overhead cam, double overhead cam, overhead valve, - Describe the main engine components and functions: Components: c cylinder blocks, liners(wet/dry), manifolds, valve operating mechanisms, timing, gears, camshafts, pistons, pushrods, connecting rods, crankshafts, flywheels, machined faces, securing devices, journals/ bearings, seals. - Describes the faults in engine as following: White smoke, black smoke, engine oil level decreasing, incorrect adjustment of tappets, incorrect adjustment of ignition timing, incorrect adjustment of engine timing. | Theory-2Hrs Practice-6 Hrs Total- 08 Hrs | • Measuring tools (Vernier caliper, micrometer, Cylinder bore gauge) • Hand tools trolley • Special tools (ring compressor, belt tensioner, torque wrench) • Engine (Petrol/ diesel) | Class Room and IC Lab |
| - Assemble Engine block | Trainee will be able to: - Select appropriate tools and perform crankshaft assembly in the engine block with all related components. - Select appropriate tools and perform piston ring assembly in the block under the specified procedure. - Assembly of the all engine components as specified procedure in the manual. | - Describe the engine terms as following: Top dead Centre (TDC), bottom dead Centre (BDC), bore, stroke, engine capacity, clearance volume, swept volume, compression ratio, single overhead cam, double overhead cam, overhead valve, - Describe the main engine components and functions: Components: cylinder blocks, liners(wet/dry), manifolds, valve operating mechanisms, timing, gears, camshafts, pistons, pushrods, connecting rods, crankshafts, flywheels, machined faces, securing devices, journals/ bearings, seals. - Describes the faults in engine as following: White smoke, black smoke, engine oil level decreasing, incorrect adjustment of tappets, incorrect adjustment of ignition timing, incorrect adjustment of engine timing. | Theory-2Hrs Practice-8 Hrs Total- 10 Hrs | • Measuring tools (Vernier caliper, micrometer, Cylinder bore gauge) • Hand tools trolley • Special tools (ring compressor, belt tensioner, torque wrench) • Engine (Petrol/ diesel) - Engine Oil - Gasket Set, - Piston Rings, - Oil Seals, - Valve Seats, Silicon, Emery Paste - Stick, - Cotton Waste | Class Room and IC lab |
| - Disassemble Engine head | Trainee will be able to: - Select appropriate tools. - Remove Head Gasket. - Carry out disassembly of engine camshaft according to specified procedure in the manual. - Carry out disassembly of engine head valve train according to specified procedure in the manual. - Dissemble Retainer Plate and Rotter coil - Inspect the disassemble components | - Describes the faults in engine as following: White smoke, black smoke, engine oil level decreasing, incorrect adjustment of tappets, incorrect adjustment of ignition timing, incorrect adjustment of engine timing. - Describe the main engine components and functions: Components: cylinder heads, liners(wet/dry), manifolds, valve operating mechanisms, timing, gears, camshafts, | Theory-1Hrs Practice-5 Hrs Total- 06 Hrs | • Measuring tools (Vernier caliper, micrometer, Cylinder bore gauge) • Hand tools trolley • Special tools (ring compressor, belt tensioner, torque wrench) • Engine (Petrol/ diesel) | Class Room and IC Lab |
| - Assemble Engine head | Trainee will be able to: - Assemble Retainer Plate and Rotter coil - Assembly of engine head valve train according to specified procedure in the manual. - Assembly of engine camshaft according to specified procedure in the manual. - Place the Head Gasket. | - Describe the main engine components and functions: Components: cylinder heads, liners(wet/dry), manifolds, valve operating mechanisms, timing, gears, camshafts, - Describes the faults in engine as following: White smoke, black smoke, engine oil level decreasing, incorrect adjustment of tappets, incorrect adjustment of ignition timing, incorrect adjustment of engine timing. | Theory-2Hrs Practice-8 Hrs Total- 10 Hrs | Measuring tools (Vernier caliper, micrometer, Cylinder bore gauge) •Hand tools trolley •Special tools (ring compressor, belt tensioner, torque wrench) •Engine (Petrol/ diesel) - Engine Oil - Gasket Set, - Piston Rings, - Oil Seals, - Valve Seats, Silicon, Emery Paste - Stick, Cotton Waste | Class Room and IC Lab |

- Maintain Engine Cooling System Components and Their Relationship

Objective: This module covers the skills and knowledge required to Identify Cooling system in the vehicle, Demonstrate Antifreeze/Inhibiters and Perform Flushing of Cooling system

Duration: 30 Hours									Theory: 6 Hours			Practice: 24 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Identify Cooling system in the vehicle. | Trainee will be able to: - Demonstrate main components of cooling system and their functions. - Locate the water pump and understand the function of its components. - Point out the thermostat Valve. - Identify the radiator, its types and understand the function of the components. - Demonstrate the radiator fan and understand its drive types including fan motor and its pump. | - Describe the operating principles and terminologies of cooling system as following: Air cooled engine, liquid cooled engine, operating temperature, thermal efficiency, pressurized system, heat exchange method (conduction, radiation, and convection), corrosion, and inhibitors/antifreeze) - Describe the functions and purpose of engine cooling system components as: Electrical motor operated fan with thermo switch, belt operated fan, temperature controlled/viscous coupling fan, pressure cap, water body/pump, drive belt, thermostat, radiator, core plug, hoses/clamps, and gasket/ seals. | Theory-02Hrs Practice-08 Hrs Total- 10 Hrs | •Hand tools trolley •Radiator leak tester •Vehicle/ simulator with a cooling system •Drive belt tensioner gauge | Class Room and IC Lab |
| - Demonstrate Antifreeze/Inhibiters | Trainee will be able to: - Identify the antifreeze its compositions and types. - Identify the rust corrosion in the engine and preventions. | • Capable of diagnose, adjust and rectify the cooling system faults. | Theory-02Hrs Practice-08 Hrs Total- 10 Hrs | •Hand tools trolley •Radiator leak tester •Vehicle/ simulator with a cooling system •Drive belt tensioner gauge | Class Room and IC Lab |
| - Perform Flushing of Cooling system | Trainee will be able to: - Remove the Coolant from engine. - Add the solution of water and Cleaning agents (Freshers) - Start the engine for specified hours. - Remove the solution - Add new coolant. | - Describe the functions and purpose of engine cooling system components as: Electrical motor operated fan with thermo switch, belt operated fan, temperature controlled/viscous coupling fan, pressure cap, water body/pump, drive belt, thermostat, radiator, core plug, hoses/clamps, and gasket/ seals. | Theory-02Hrs Practice-08 Hrs Total- 10 Hrs | •Hand tools trolley •Radiator leak tester •Vehicle/ simulator with a cooling system •Drive belt tensioner gauge •Coolants, •Sealant/gasket, • Hose Pipe, •Thermostat Valve | Class Room and IC Lab |

- Inspect Engine Lubrication System and working of its Components

Objective: This module covers the skills and knowledge required to perform Identify the Lubrication system, Analyze different Lubricants in the Engine and Sketch Lubrication system diagram

Duration: 27 Hours								Theory: 06 Hours				Practice: 21 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Identify the Lubrication system | Trainee will be able to: - Identify the main components of engine lubrication system. - Identify types of oil pumps. - Identify the main relief valve. - Identify the Temperature, pressure and level sensor of oil. - Identify Oil filter. - Identify the Oil cooler - Identify oil coolers for the high-performance engine (air cooled/ Water cooled type) | - The functions/operating principle and main components of engine lubrication system - Wet sump, dry sump, total loss, Oil pumps and its types, pressure relief valves, oil filter, oil cooler, ventilation/ PCV Valve, hoses, and oil level indicators. - Lubricant (oils viscosity, viscosity index, oil classifications, Reduction of frictional forces, cooling effect, cleaning effect, corrosion resistance, and noise reduction). | Theory-02Hrs Practice-06Hrs Total- 08 Hrs | •Hand tools trolley •Car Lift •Vehicle/ simulator with a cooling system •Sealant/gasket | Class Room and IC Lab |
| - Analyze different Lubricants in the Engine. | Trainee will be able to: - Identify the types of engine oil used in the engine with respect to viscosity. - Identify the function of oil filter and its components. - Identify the oil sump and its components. | - Lubricant (oils viscosity, viscosity index, oil classifications, Reduction of frictional forces, cooling effect, cleaning effect, corrosion resistance, and noise reduction). | Theory-02Hrs Practice-08 Hrs Total- 10 Hrs | •Hand tools trolley •Car Lift •Vehicle/ simulator with a cooling system •Sealant/gasket | Class Room and IC Lab |
| - Sketch Lubrication system diagram | Trainee will be able to: - Draw The lubrication system diagram with EGR - Draw The lubrication system diagram without EGR | - Flow of oil in Lubrication system diagram with EGR - Flow of oil in Lubrication system diagram without EGR | Theory-02Hrs Practice-07Hrs Total- 09 Hrs | Pencil Drawing Sheet Drawing Board Scale Rubber | Class Room and Drawing Hall |

- Locate Diesel Engine’s Fuel System Components

Objective: This module covers the skills and knowledge required to Identify PT (pressure time) fuel system, Locate Inline fuel injection (BOSCH) system, Inspect Common rail injection (CRI) fuel system, Demonstrate Hydraulic electronic unit injection (HEUI) system and Locate Mechanical Electronic Unit Injection (MEUI) system

Duration: 30 Hours								Theory: 08 Hours				Practice: 22 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Identify PT (pressure time) fuel system | Trainee will be able to: - Locate the diesel tank. - Demonstrate the primary fuel filters. - Identify the PT pump. - Identify the priming pump. - Identify the secondary fuel filters. - Identify the PT injectors. - Identify the pressure regulator valve. - Identify the return line. - Draw the block diagram of PT pump fuel system. | - The features of main components of engine fuel system as following. Fuel tanks, filler cap/ security, venting, fuel level sender, lines/ filters, and fuel lift pumps (mechanical/ electrical), fuel (octane rating, Octane number) | Theory-02Hrs Practice-04Hrs Total- 06 Hrs | •Toll trolley (Complete set) •Diesel engine vehicle or simulator •Safety wears | Class Room and IC Lab |
| - Locate Inline fuel injection (BOSCH) system. | Trainee will be able to: - Identify the diesel tank. - Identify the primary fuel filters. - Identify the transfer pump. - Identify the priming pump. - Identify the secondary fuel filters - Identify the BOSCH pump. - Identify the nozzle - Identify the return line. - Draw the block diagram of BOSCH pump fuel system. | - The features of main components of engine fuel system as following. Fuel tanks, filler cap/ security, venting, fuel level sender, lines/ filters, , and fuel lift pumps (mechanical/ electrical), fuel (octane rating, Octane number - The Diesel injection systems main components as following. Single/ multi-hole injectors, inline fuel injection pump, rotary fuel injection pump, fuel injection pressure, bleed points, fuel cutoff switch, governor operation, in Combustion chamber types, main features of pressure/ volume diagrams/ effects of advanced/ retarded/ no fuel injection | Theory-1.5Hrs Practice-04 Hrs Total- 5.5 Hrs | •Toll trolley (Complete set) •Diesel engine vehicle or simulator •Safety wears | Class Room and IC Lab |
| - Inspect Common rail injection (CRI) fuel system. | Trainee will be able to: - Identify the ECM. - Identify the Transfer pump. - Identify the primary pump. - Identify the secondary fuel filters - Identify the main pump. - Identify the Common Rail. - Identify the injectors. - Identify the return line. - Draw the block diagram of CRI system. | - The features of main components of engine fuel system as following. Fuel tanks, filler cap/ security, venting, fuel level sender, lines/ filters, and fuel lift pumps (mechanical/ electrical), fuel (octane rating, Octane number - The Diesel injection systems main components as following. Single/ multi-hole injectors, inline fuel injection pump, rotary fuel injection pump, fuel injection pressure, bleed points, fuel cutoff switch, governor operation, in Combustion chamber types, main features of pressure/ volume diagrams/ effects of advanced/ retarded/ no fuel injection | Theory-1.5Hrs Practice-04 Hrs Total- 5.5 Hrs | •Toll trolley (Complete set) •Diesel engine vehicle or simulator •Safety wears | Class Room and IC Lab |
| - Demonstrate Hydraulic electronic unit injection (HEUI) system | Trainee will be able to: - Identify the transfer pump. - Identify the secondary fuel filters - Identify the injectors. - Identify the pressure regulator valve. - Identify the oil pan - Identify the oil pump. - Identify the oil cooler. - Identify the oil filter - Identify the unit injection pump - Identify the injector | - The features of main components of engine fuel system as following. Fuel tanks, filler cap/ security, venting, fuel level sender, lines/ filters, , and fuel lift pumps (mechanical/ electrical), fuel (octane rating, Octane number - The Diesel injection systems main components as following. Single/ multi-hole injectors, inline fuel injection pump, rotary fuel injection pump, fuel injection pressure, bleed points, fuel cutoff switch, governor operation, in Combustion chamber types, main features of pressure/ volume diagrams/ effects of advanced/ retarded/ no fuel injection | Theory-1.5Hrs Practice-05Hrs Total- 6.5 Hrs | •Toll trolley (Complete set) •Diesel engine vehicle or simulator •Safety wears | Class Room and IC Lab |
| - Locate Mechanical Electronic Unit Injection (MEUI)system | Trainee will be able to: - Identify the ECM (main pump). - Identify the MEUI injector. - Identify the MEUI regulator valve - Identify the MEUI return line. - Draw the block diagram of PT pump fuel system. | - The features of main components of engine fuel system as following. Fuel tanks, filler cap/ security, venting, fuel level sender, lines/ filters, , and fuel lift pumps (mechanical/ electrical), fuel (octane rating, Octane number - The Diesel injection systems main components as following. Single/ multi-hole injectors, inline fuel injection pump, rotary fuel injection pump, fuel injection pressure, bleed points, fuel cutoff switch, governor operation, in Combustion chamber types, main features of pressure/ volume diagrams/ effects of advanced/ retarded/ no fuel injection | Theory-1.5Hrs Practice-05Hrs Total- 6.5 Hrs | •Toll trolley (Complete set) •Diesel engine vehicle or simulator •Safety wears |  |

- Maintain the Engine intake and Exhaust System and its Components

Objective: This module covers the skills and knowledge required to Maintain Intake system and its components, Maintain Exhaust system and its components and Identify Emission control system

Duration: 27 Hours								Theory: 06 Hours				Practice: 21 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Maintain Intake system and its components | Trainee will be able to: - Inspect the performance of main components of Intake manifold - Remove and assemble intake manifold from engine - Remove the pre cleaner and air filter - Identify the impeller of turbo charger. - Identify After cooler. - Identify inter-cooler. | - Intake system and its functions - Function of inter-cooler and after-cooler. - Operation of turbocharger. | Theory-02Hrs Practice-08 Hrs Total- 10 Hrs | •Vehicle •Tools trolley (Complete set of hand tools) •Gas analyzer •Vacuum pump | Class Room and IC Lab |
| - Maintain Exhaust system and its components | Trainee will be able to: - Inspect the performance of main components of exhaust manifold - Remove and assemble exhaust manifold from engine - Replace the muffler and EGR under the specified procedure. - Replace the gasket of muffler under the specified procedure. - Remove and assemble turbocharger. - Inspect the performance of main components of Turbocharger. | - Exhaust system and its functions. - muffler assembly operation - the exhaust gas regulator (EGR - Exhaust gas recirculation and its operation - operation of turbocharger | Theory-02Hrs Practice-08 Hrs Total- 10 Hrs | •Vehicle •Tools trolley (Complete set of hand tools) •Gas analyzer •Vacuum pump | Class Room and IC Lab |
| - Identify Emission control system | Trainee will be able to: - Identify EGR and its function. - Identify the positive crankcase ventilation (PCV). - Identify the exhaust gas recirculation system. - Identify the main components of Evaporative control system of fuel system. | - Exhaust gas regulator (EGR). - positive crank case ventilation PCV system and it operation - Exhaust gas recirculation and its operation | Theory-02Hrs Practice-05 Hrs Total- 07 Hrs | •Vehicle •Tools trolley (Complete set of hand tools) •Gas analyzer •Vacuum pump | Class Room and IC Lab |

- Perform Overhauling of Four Stroke Engine

Objective: This module covers the skills and knowledge required to Perform Overhauling of Four stroke engine cylinder head and Service the cylinder block, Piston and Piston Rings

Duration: 46 Hours								Theory: 06 Hours				Practice: 40 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Perform Overhauling of Four stroke engine cylinder head. | - Arrange proper tools, equipment and shop manual. - Disconnect cables, wires, and muffler, Air filter from cylinder head for dismantling cylinder head. - Remove cylinder head assembly. - Clean cylinder head and its components to identify the damage parts. - Install Repaired cylinder head and its attachments according to shop manual. - Test engine performance to verify servicing of cylinder head. | - Different systems of engine - Parts of engine - Diagnostic procedure of Engine - Engine dismantling procedure - Engine inspection and repair procedure - Engine assembling procedure - Engine oil circuits - Engine Timing - Manufacturers and workshop manual of given vehicle - Working principle Fuel system - Exhaust system - Euro standards | Theory-03Hrs Practice-18 Hrs Total- 21 Hrs | •Tools trolley (complete set of tools) •Compression tester •Measuring tools (Complete set for engine overhauling) •Fuel pressure Tester •Special Service Tools | Class Room and IC Lab |
| - Service the cylinder block, Piston and Piston Rings | - Arrange proper tools and equipment to service cylinder block, piston and piston rings. - Remove cylinder head assembly from the cylinder block to service cylinder block piston and piston Rings as per shop manual. - Clean cylinder block, piston and its components to remove dust. - Remove cylinder block, piston and inspect its components to identify the damage parts as per Shop manual. - Install Cylinder Block, piston, piston rings, and cylinder head and connect all the wires, cables to the engine as per shop manual. - Test engine performance to verify servicing of cylinder block, piston and piston rings as per shop manual. | - Different systems of engine - Parts of engine - Diagnostic procedure of Engine - Engine dismantling procedure - Engine inspection and repair procedure - Engine assembling procedure - Engine oil circuits - Engine Timing - Manufacturers and workshop manual of given vehicle - Working principle Fuel system - Exhaust system - Euro standards | Theory-03Hrs Practice-22 Hrs Total- 25 Hrs | - Tools trolley (complete set of tools) - Compression tester - Measuring tools (Complete set for engine overhauling) - Fuel pressure Tester - Piston ring - Engine oil - Silicon, - Oil sump - Jain, - Oil filter, - Head Gasket, - Grease, - Cotton waste. | Class Room and IC Lab |

## 9.5 Automotive Electrics and Electronics

    - Perform Basic Electricity Measurements

Objective: This module covers the skills and knowledge required to enable the learner to perform electrical related calculations and measurements, as well as conceptualize different automotive functions related to the electrical operations.

Duration: 37 Hours								Theory: 07 Hours				Practice: 30 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Basic Electricity | Trainee will be able to: - Prepare series circuit on work bench using appropriate tools. - Prepare parallel circuit on work bench using appropriate tools. - Prepare series parallel circuit on work bench using appropriate tools. | - Describe the following terms: Atom and its particles (electron, neutrons & protons), EMF, current, voltage, resistance, magnetism, electro magnetism, flux, inductance, mutual inductance, capacitance, and diode. Practical activity: - Calculation involving the series, parallel, and series parallel circuits | Theory- 04 hrs Practice- 15 hrs Total- 19 hrs | Digital Multi meter (DMM). Load (resistors) Wires, Test bench (Complete accessories) Lamps Bulb and Bulb Holder, Thimble (Male & Female), Circuit Board, Wire Clumps, Battery | Class Room and wood workshop |
| LU2. Basic Electricity Measurement | Trainee will be able to: - Measure/ record voltage by using Digital Multi METER (DMM) - Measure/ record current by using Digital Multi Meter (DMM) - Measure/ record resistance by using Digital Multi Meter. - Measure/ record continuity by using Digital Multi Meter. | - Calculation of ohm law, (V= IR) calculation of power law (P= VI). Practical activity: - Correct use of Digital multi meter (DMM) - Point out electromagnets in the vehicle. | Theory- 03 hrs Practice- 15 hrs Total- 18 hrs | Digital Multi meter (DMM). Load (resistors) Wires, Test bench (Complete accessories) Lamps Bulb and Bulb Holder, Thimble (Male & Female), Circuit Board, Wire Clumps, Battery | Class Room and wood workshop |

    - Identify Electromagnetism and its application

Objective: This module covers the skills and knowledge required to implement the electromagnet, effect on current carrying conductor in magnetic field, effect on conductor by varying the current with the help of rheostat, plot magnetic lines of forces of bar magnet, observe its various characteristics and verify the Faraday’s law and emf through induction

Duration: 36 Hours								Theory: 06 Hours				Practice: 30 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Construct Electromagnet | - Trainee will be able to: - Prepare series circuit on work bench using appropriate tools. - Prepare parallel circuit on work bench using appropriate tools. - Prepare series parallel circuit on work bench using appropriate tools. | - Magnet, magnetism & electromagnet - The functions of iron core - Strong magnet - The movement of current carrying conductor which is placed in magnetic field - Fleming’s left-hand rule - Magnetic lines of force. - Magnetic field. - Travel of magnetic lines of force with respect to each other. - Practical activity: - Construct an electromagnet and verify its operation. - Plot magnetic field of around the magnet using compass. - Verify faradays law using magnet and coil. - Implement a circuit to verify emf through induction | Theory- 0.5 hours Practice- 03 hours Total- 3.5 hours | Iron nail as core Thin coated copper wire Current carrying conductor Horse shoe magnet Power Supply Compass needle Bar Magnet Paper Lead pencil Coil Galvanometer Magnet Connecting leads Transformer | Class Room and wood work shop |
| Implement circuit to determine the effect on current carrying conductor in magnetic field. | Trainee will be able to: - Measure/ record voltage by using Digital Multi Meter - Measure/ record current by using Digital Multi Meter - Measure/ record resistance by using Digital Multi Meter. - Measure/ record continuity by using Digital Multi Meter | - Magnet, magnetism & electromagnet - The functions of iron core - Strong magnet - The movement of current carrying conductor which is placed in magnetic field - Fleming’s left-hand rule - Magnetic lines of force. - Magnetic field. - Travel of magnetic lines of force with respect to each other. - The first & 2nd law of michael faraday - The role of magnetic strength in faraday’s law - Coupled coils - Practical activity: - Construct an electromagnet and verify its operation. - Plot magnetic field of around the magnet using compass. - Verify faradays law using magnet and coil. - Implement a circuit to verify emf through induction | Theory- 0.5 hours Practice- 06 hours Total- 6.5 hours | Iron nail as core Thin coated copper wire Current carrying conductor Horse shoe magnet Power Supply Compass needle Bar Magnet Paper Lead pencil Coil Galvanometer Magnet Connecting leads Transformer | Class Room and wood workshop |
| Determine the effect on conductor by varying the current with the help of rheostat. | - Trainee will be able to: - Reduce the rheostat resistance - Record the effect on copper rod - Increase the rheostat resistance | - Travel of magnetic lines of force with respect to each other. - The first & 2nd law of michael faraday - The role of magnetic strength in faraday’s law - The role of conductor’s length or turns in faraday’s law - Practical activity: - Construct an electromagnet and verify its operation. - Plot magnetic field of around the magnet using compass. - Verify faradays law using magnet and coil. - Implement a circuit to verify emf through induction | Theory- 01 hours Practice- 03 hours Total- 04 hours | Iron nail as core Thin coated copper wire Current carrying conductor Horse shoe magnet Power Supply Compass needle Bar Magnet Paper Lead pencil Coil Galvanometer Magnet Connecting leads Transformer | Class Room and wood workshop |
| - Plot magnetic lines of forces of bar magnet | - Trainee will be able to: - Place a bar magnet on paper and outline its boundary with the help of lead pencil. - Place a compass needle at one side the magnet. - Mark points on paper where the compass needle stop. - Repeat the same procedure till compass reach at the other end of magnet. - Change the position of compass needle near the magnetic pole and repeat the procedure for P3 to P4 | - The role of magnetic strength in faraday’s law - The role of conductor’s length or turns in faraday’s law - Lenz’s law - Induction - Faraday’s first law of electro-magnetic induction - Practical activity: - Construct an electromagnet and verify its operation. - Plot magnetic field of around the magnet using compass. - Verify faradays law using magnet and coil. - Implement a circuit to verify emf through induction | Theory- 01 hours Practice- 03 hours Total- 04 hours | Iron nail as core Thin coated copper wire Current carrying conductor Horse shoe magnet Power Supply Compass needle Bar Magnet Paper Lead pencil Coil Galvanometer Magnet Connecting leads Transformer | Class Room and wood workshop |
| - Verify Faradays law by moving magnet inside the coil. | - Construct a coil with hollow iron cylinder (approximately 3 inches in length 1.5 inch in diameter.) and make 150 to 200 turns on it. - Connect Galvano meter with coil. - Move permanent magnet inside the coil, vary its speed and record the effect on reading of Galvanometer. - Hold the magnet inside the coil and do not move, now record the effect on reading of Galvano meter. | - Magnet, magnetism & electromagnet - The functions of iron core - Strong magnet - The movement of current carrying conductor which is placed in magnetic field - Fleming’s left-hand rule - Magnetic lines of force. - Magnetic field. - Travel of magnetic lines of force with respect to each other. - The first & 2nd law of michael faraday - The role of magnetic strength in faraday’s law - The role of conductor’s length or turns in faraday’s law - Lenz’s law - Induction - Faraday’s first law of electro-magnetic induction - Coupled coils - The induction of emf in 2nd coil, when voltage is applied to 1st coil - Mutual induction core - Practical activity: - Construct an electromagnet and verify its operation. - Plot magnetic field of around the magnet using compass. - Verify faradays law using magnet and coil. - Implement a circuit to verify emf through induction | Theory- 01 hours Practice- 06 hours Total- 07 hours | Iron nail as core Thin coated copper wire Current carrying conductor Horse shoe magnet Power Supply Compass needle Bar Magnet Paper Lead pencil Coil Galvanometer Magnet Connecting leads Transformer | Class Room and wood workshop |
| - Verify Faradays law by moving coil near the magnet field. | - Construct a coil with hollow iron cylinder (approximately 3 inches in length 1.5 inch in diameter.) and make 150 to 200 turns on it. - Connect Galvano meter with coil. - Fix permanent magnet and move the coil fast and slow on it and record the effect on reading of Galvano meter. - Hold the coil near the magnetic field do not move, now record the effect on reading of Galvano meter. | - Magnet, magnetism & electromagnet - The functions of iron core - Strong magnet - Magnetic lines of force. - Magnetic field. - Travel of magnetic lines of force with respect to each other. - Faraday’s first law of electro-magnetic induction - Coupled coils - The induction of emf in 2nd coil, when voltage is applied to 1st coil - Mutual induction core | Theory- 01 hours Practice- 03 hours Total- 04 hours | Iron nail as core Thin coated copper wire Current carrying conductor Horse shoe magnet Power Supply Compass needle Bar Magnet Paper Lead pencil Coil Galvanometer Magnet Connecting leads Transformer | Class Room and wood workshop |
| - Verify emf through induction. | - Take step down transformer and connect its secondary with a Galvano meter and primary winding with a DC battery through a rheostat. - Continuously variate the rheostat and observe the reading on the Galvano meter | - Magnet, magnetism & electromagnet - The functions of iron core - Strong magnet - Magnetic lines of force. - Magnetic field. - Travel of magnetic lines of force with respect to each other. - Faraday’s first law of electro-magnetic induction - Coupled coils - The induction of emf in 2nd coil, when voltage is applied to 1st coil - Mutual induction core - Practical activity: | Theory- 01 hours Practice- 06 hours Total- 07 hours | Iron nail as core Thin coated copper wire Current carrying conductor Horse shoe magnet Power Supply Compass needle Bar Magnet Paper Lead pencil Coil Galvanometer Magnet Connecting leads Transformer | Class Room and wood workshop |

    - Perform Battery Service of Vehicle

Objective: This module covers the skills and knowledge required to enable the learner to diagnose the problem related to the battery and starting system and enable the learner to troubleshoot the system if not working

Duration: 02 Hours								Theory: 09 Hours				Practice: 11 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Service of battery | Trainee will be able to: - Perform the battery test by use of hydrometer, Terminal clean, tight, and insulated, secure with battery holder as per given specification. - Inspect and change the Ground (Earth) cable from the vehicle body. - Test battery by using the load tester/battery tester and assess the battery performance. | - Lead Acid battery and its internal components: Material, acid and water ratio, plates, cells, ampere hour rating, voltage drop, corroded/ lose terminals, maintenance free batteries. Practical activity: - Troubleshoot the problems related to the vehicle battery. | Theory- 01 hours Practice- 06 hours Total- 07 hours | Vehicle or simulator. Tool box Safety goggles Gloves Battery External charger Battery load tester DMM Wires Hydrometer | Class Room and wood work shop |
| LU2. Battery replacement | Trainee will be able to: - Perform Safe removal and installation of Battery from the vehicle Inspect/service the battery and assess safety fitting and correctness under specified procedure. - Carry out safe procedure to charge the battery using an external charger. | - Describe the chemical activities in Lead Acid Battery while charging and discharging. Practical activity: - Troubleshoot the problems related to the vehicle battery. | Theory- 01 hours Practice-03 hours Total- 04 hours | Vehicle or simulator. Tool box Safety goggles Gloves Battery External charger Battery load tester DMM Wires Hydrometer | Class Room and wood work shop |

    - Repair Engine Starting System

Objective: This module covers the skills and knowledge required to enable the learner to diagnose the problem related to the battery and starting system and enable the learner to troubleshoot the system if not working

Duration: 24 Hours							Theory: 06 Hours				Practice: 18 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Repair starting system | Trainee will be able to: - Inspect the vehicle starting system and troubleshoot the following. - Low cranking speed/ not cranking, Ignition switch operation malfunctioning. - Inspect the vehicle starting system and troubleshoot the following. - Unusual noise during operation, corrosive battery terminals, safety neutral switch, incorrect pinion engagement/ disengagement, blown fuse/ relay, lose/ corroded main starting wire. | - Short circuits and battery related hazards. Wrong battery installation, touch the positive terminal with the body, un-insulated wire touches with the vehicle body, battery size not matches with the maker, battery acid injury (human eye/ body), disposal of faulty batteries. - Describe the Wiring diagram associated with the electrical/electronic symbols. - Circuit protection device (Fuse, fusible link, circuit breaker) Circuit switching device (manual switches, electromagnet switch/relay normal open and normal close, pressure switch, temperature switch) Resister (Rating, identification method, effect of change of temperature) Electronic devices (Diode, Zener diode, bipolar transistor, Thyristor, thermistor, heat sink, LED) Practical activity: - Troubleshoot the problems related to the vehicle battery. - Capable to describe the fault related to the battery and take appropriate decisions. | Theory- 03 hours Practice- 09 hours Total- 12 hours | Vehicle or simulator. Toolbox Safety goggles Gloves Battery External chargers Battery load testers DMM Wires Hydrometer | Class Room and wood workshop |
| LU2. Service Starter Motor. | Trainee will be able to: - Carryout safe procedure to replace the starter motor from the vehicle. - Service/ replace components of starter motor and perform bench testing. | - Describe the Wiring diagram associated with the electrical/electronic symbols. Circuit protection device (Fuse, fusible link, circuit breaker) Circuit switching device (manual switches, electromagnet switch/relay normal open and normal close, pressure switch, temperature switch) Resister (Rating, identification method, effect of change of temperature) Electronic devices (Diode, Zener diode, bipolar transistor, Thyristor, thermistor, heat sink, LED) - Distinguish between increase/decrease cranking current consumption, voltage drop in a system, electrical load, and reason of high ampere draw from a system/overload. - Describe the Lenz’s law of motor principle, single phase, three phase motor, AC Motor, DC Motor. - Practical activity: - Capable to troubleshoot the vehicle starting system. - Service the starter motor. | Theory- 03 hours Practice- 09 hours Total- 12 hours | Vehicle or simulator. Toolbox Safety goggles Gloves Battery External chargers Battery load testers DMM Wires Hydrometer | Class Room and wood workshop |

    - Repair Battery Charging System

Objective: This module covers the skills and knowledge required to Repair charging system and Replace alternator

Duration: 24 Hours								Theory: 06 Hours				Practice: 18 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Repair charging system | Trainee will be able to: - Inspect vehicle charging system and diagnose the following. Battery warning light indication on instrument panel, test output voltage of alternator, drive belt condition, alternator bearing noise, wire harness, fuse, and proper insulated wires. - Dismantle/ service of alternator as following; Repair connector, replace carbon brushes, Replace Rectifier Bridge, replace voltage regulator and replace stator winding. | - Describe the Purpose and function of Alternator’s following parts. - Rotor, stator windings, Voltage regulator, Rectifier Bridge, Internal cooling fan, Bearings, carbon brush, mounting/ adjustment bolts), drive belt and its types, ratio alternator/ crank pulleys, RPM, Alternator input/ output voltages. Practical activity: - Capable to troubleshoot the vehicle charging system - Capable to describe the fault related to the alternator and take appropriate decisions | Theory- 04 hrs Practice- 09 hrs Total- 13 hrs | Vehicle/ simulator Test lamp DMM Tool kit Ampere meter Fuses Relay | Class Room and wood workshop |
| LU2. Replace alternator | - Trainee will be able to: - Perform safe working practice to remove the alternator from the vehicle and refit under the specified procedure. - Perform test bench of alternator to confirm the operation. | - Describe the functions and uses of the following. - Semiconductor and its types, Rectification (Half wave/ full wave), transformer and its types, capacitor and its types. - Practical activity: - Capable to describe the fault related to the alternator and take appropriate decisions. - Capable to Service the alternator | Theory- 02 hours Practice-09 hours Total- 11 hours |  | Class Room and wood workshop |

    - Repair Spark Ignition System of Internal Combustion Engine

Objective: This module covers the skills and knowledge required to diagnose and troubleshoot conventional spark ignition system as well as conceptualize the ignition system mechanism in the engine.

Duration: 33 Hours								Theory: 06 Hours				Practice: 27 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Repair Ignition System | Trainee will be able to: - Inspect vehicle Ignition system and diagnose the following: Crank engine and test presence of spark in a system, identify faulty fuse in fuse box (Note: follow safe working procedure). - Inspect vehicle Ignition system and diagnose following: Test performance of plug cable (high tension leads) and plugs, inspect distributor cap/rotor for performance, assess correct ignition timing, inspect vacuum advancer, assess C.B. Point condition/gap and test condenser. (Note: follow safe working procedure). | - Describe the Purpose and function of ignition system’s following parts in Spark ignition system contact breaker type, spark ignition system breaker less type. Ignition switch, Ignition coil, resister, plug cable, distributer cap, rotor arm, vacuum advancer, centrifugal advancer, contact breaker assembly, condenser, high tension leads/ connectors, suppression, spark plug (construction, heat range type, size), Magnetic Reflector, Hall Effect trigger/ amplifier, differentiate the performance of contact breaker and contact breaker less type. Practical activity: - Troubleshot the vehicle ignition system (Non EFI engines) | Theory- 03 hours Practice- 15 hours Total- 18 hours | Vehicle/ Simulator Digital Multi meter (DMM). Tools box Test Lamp Timing gun | Class Room and wood workshop |
| LU2. Repair Ignition System (CDI System) | Trainee will be able to: - Inspect the vehicle Ignition system and diagnose the ignition coil performance, (Note: follow safe working procedure). - Inspect the vehicle Ignition system and diagnose the CDI of distributor. (Note: follow safe working procedure). | - Describe the Purpose and function of ignition system’s following parts in Spark ignition system contact breaker type, spark ignition system breaker less type. Ignition switch, Ignition coil, resister, plug cable, distributer cap, rotor arm, vacuum advancer, centrifugal advancer, contact breaker assembly, condenser, high tension leads/ connectors, suppression, spark plug (construction, heat range type, size), Magnetic Reflector, Hall Effect trigger/ amplifier, differentiate the performance of contact breaker and contact breaker less type. Practical activity: - Capable to adjust the ignition timing - Capable to service the ignition system. | Theory- 03 hours Practice- 12 hours Total- 15 hours | Vehicle/ Simulator Digital Multi meter (DMM). Tools box Test Lamp Timing gun | Class Room and wood workshop |

    - Adjust Ignition Timing of Spark Ignition Engine

Objective: This module covers the skills and knowledge required to diagnose and troubleshoot conventional spark ignition system as well as conceptualize the ignition system mechanism in the engine

Duration: 33 Hours								Theory: 06 Hours				Practice: 27 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Adjust Ignition Timing | Trainee will be able to: - Installation of distributer assembly under specified procedure and start the engine, adjust ignition timing with the help of timing gun. - Inspect/ Replace ignition switch, inspect/ replace ignition coil, and Repair/ replace electrical connectors under specified procedure. | - Describe primary circuit, secondary circuit of ignition system, identifies change in the spark timing in accordance to the load and speed of the engine. - Explain Faradays law of mutual induction and calculate the primary and secondary voltages and current. Practical activity: - Troubleshot the vehicle ignition system (Non EFI engines) - Capable to adjust the ignition timing | Theory- 03 hours Practice- 15 hours Total- 18 hours | Vehicle/ Simulator Digital Multi meter (DMM). Tools Box Test Lamp Timing Gun | Class Room and wood workshop |
| LU2. Repair Ignition System | Trainee will be able to: - Inspection of crank position sensor under specified procedure and starts the engine. - Inspection/replace of ignition fuse under specified procedure and starts the engine. | - Describe primary circuit, secondary circuit of ignition system, identifies change in the spark timing in accordance to the load and speed of the engine. - Explain Faradays law of mutual induction and calculate the primary and secondary voltages and current. Practical activity: - Troubleshot the vehicle ignition system (Non EFI engines) - Capable to adjust the ignition timing - Capable to service the ignition system. | Theory- 03 hours Practice- 12 hours Total- 15 hours | Vehicle/ Simulator Digital Multi meter (DMM). Tools Box Test Lamp Timing Gun | Class Room and wood workshop |

    - Repair Distributer-Less Ignition System

Objective: This module covers the skills and knowledge required to enable the learner to troubleshoot the electronic ignition related problems in the vehicle.

Duration: 33 Hours							Theory: 06 Hours				Practice: 27 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Troubleshoot Distributer Less Systems | Trainee will be able to: - Use of scanner and find out the fault related to the ignition coil and reset the trouble code by use of scanner. - Use of scanner and find out the fault related to the crank sensor and reset the trouble code by use of scanner. - Use of scanner and find out the fault related to the crank sensor and reset the trouble code by use of scanner. | - Ignition advance mechanism in Distributor less system, Dual coil system, Coil on plug system, crank sensor signals, Practical activity: - Capable to diagnose and troubleshoot the electronic ignition system. - Capable to replace faulty parts of electronic ignition system. | Theory- 03 hours Practice- 12 hours Total- 15 hours | Tools trolley (Complete set of hand tools) Scanner Test lamp DMM (Multi meter) | Class Room and wood workshop |
| LU2. Repair Distributor Less Systems | Trainee will be able to: - Use test lamp and identify the faulty fuse in the fuse box under the specified procedure. - Replace the faulty fuse with the same color and start the engine under the specified procedure. - Replace the faulty ignition coil and faulty sensor under the specified procedure. | - Ignition advance mechanism in Distributor less system, Dual coil system, Coil on plug system, crank sensor signals, Practical activity: - Capable to diagnose and troubleshoot the electronic ignition system. - Capable to replace faulty parts of electronic ignition system. | Theory- 03 hours Practice- 15 hours Total- 18 hours | Tools trolley (Complete set of hand tools) Scanner Test lamp DMM (Multi meter) | Class Room and wood workshop |

    - Repair Lightning System of Vehicle

Objective: This module covers the skills and knowledge required to enable the learner to rectify electrical related troubles IN the vehicle and understand the Wiring of the vehicle.

Duration: 27 Hours							Theory: 06 Hours				Practice: 21 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Troubleshoot the Lightening System | Trainee will be able to: - Inspect vehicle lightening system and diagnose following; under specified testing instructions, Blown fuse finding from interior or exterior fuse box. (Note: follow safe working procedure). - Inspect vehicle lightening system and diagnose following; under specified testing instructions, faulty relay and faulty flasher. (Note: follow safe working procedure). | - Identify the terms used and calculations associated with the electricity. Definitions (magnetism, self/ mutual induction). Laws (amperes rule, Fleming’s rule) Switch (manual, magnet, pressure, temperature) sender (fuel level, temperature) Practical activity: - Capable to troubleshoot the vehicle lightening system | Theory- 03 hours Practice- 09 hours Total- 12 hours | Tools trolley (Complete set of hand tools) Scanner Test lamp DMM (Multi meter) | Class Room and wood workshop |
| LU2. Repair Lights | Trainee will be able to: - Inspect vehicle lightening system and diagnose following under specified testing instructions. Inspect/ Replace head lamp, parking bulb and indicator bulb (Note: follow safe working procedure). - Inspect vehicle lightening system and diagnose following under specified testing instructions. Back light bulb, reverse light bulb. (Note: follow safe working procedure). | - Identify the terms used and calculations associated with the electricity. Definitions (magnetism, self/ mutual induction). Laws (amperes rule, Fleming’s rule) Switch (manual, magnet, pressure, temperature) sender (fuel level, temperature) Practical activity: - Capable to troubleshoot the vehicle lightening system | Theory- 03 hours Practice- 12 hours Total- 15 hours | Tools trolley (Complete set of hand tools) Scanner Test lamp DMM (Multi meter) | Class Room and wood workshop |

    - Repair Cooling System and Vehicle Accessories

Objective: This module covers the skills and knowledge required to enable the learner to rectify electrical related troubles IN the vehicle and understand the Wiring of the vehicle.

Duration: 44 Hours								Theory: 08 Hours				Practice: 36 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Repair Cooling System and Vehicle Accessories | Trainee will be able to: - Inspects the Radiator fan operation for correct operation under specified testing instructions. - Inspects the thermal switch for correct operation under specified testing instructions. - Inspects the temperature sender for correct operation under specified testing instructions. | - Identify terms used and calculations associated with the electricity. Definitions (magnetism, self/mutual induction). Laws (amperes rule, Fleming’s rule) - Describe the use of following materials in vehicle with example. Conductor (Good/ bad), Insulator, semiconductors, electrical Cable (Color coding, ampere rating, wire harness, cross sectional area, cable identification method). - Differentiates each in following. Switch (manual, magnet, pressure, temperature) sender (fuel level, temperature) Practical activity: - Capable to troubleshoot the vehicle lightening system - Capable to describe the fault related to the electrical lights and cooling system, and take appropriate decisions. | Theory- 04 hours Practice- 18 hours Total- 22 hours | Vehicle/ simulator Digital Multi Meter (DMM) Tools box Test lamp | Class Room and wood work shop |
| LU2. Repair Electrical Accessories | Trainee will be able to: - Inspects the oil warning switch operation for correct operation under specified testing instructions. - Inspects fuse, relay operation for correct operation under specified testing instructions. - Inspects the fuel sender operation for correct operation under specified testing instructions. - Inspects the wiper motor, washer motor for correct operation under specified testing instructions. - Inspects heating Elements for correct operation under specified testing instructions | - Identify terms used and calculations associated with the electricity. Definitions (magnetism, self/mutual induction). Laws (amperes rule, Fleming’s rule) - Describe the use of following materials in vehicle with example. Conductor (Good/ bad), Insulator, semiconductors, electrical Cable (Color coding, ampere rating, wire harness, cross sectional area, cable identification method). - Differentiates each in following. Switch (manual, magnet, pressure, temperature) sender (fuel level, temperature) Practical activity: - Capable to troubleshoot the vehicle lightening system - Capable to describe the fault related to the electrical lights and cooling system, and take appropriate decisions. | Theory- 04 hours Practice- 18 hours Total- 22 hours | Vehicle/ simulator Digital Multi Meter (DMM) Tools box Test lamp | Class Room and wood work shop |

    - Recognize Electrical system of Vehicle

Objective: This module covers the skills and knowledge required to competencies required to identify machine electrical system, Inspect machine electrical system,

Duration: 45 Hours								Theory: 09 Hours				Practice: 36 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Identify machine electrical system | Trainee will be able to: - Identify the wiring - Identify the sensors - Identify the solenoids - Identify the switches - Identify the fuse - Identify the relay - Identify the alternator - Identify the starting motor - Identify the battery - Identify the motors - Identify the resistors, diodes - Identify lights, horn | - Describe Electrical wiring - Describe the sensors - Describe the solenoids - Describe the switches - Describe the fuse - Describe the relay - Describe the alternator - Describe the starting motor - Describe the battery - Describe the motors - Describe the resistors, diodes - Describe the lights, horn Practical activity: - Capable to repair Electrical wiring - Capable to check the sensors - Capable to check the solenoids - Capable to check switches - Capable to check fuse - Capable to check relay | Theory- 03 hours Practice- 12 hours Total- 15 hours | Tools trolley (Complete set of Hand Tools) Special Service tools | Class Room and wood workshop |
| LU2. Inspect machine electrical system | Trainee will be able to: - Inspect the wiring - Inspect the sensors - Inspect the solenoids - Inspect the switches - Inspect the fuse - Inspect the relay - Inspect the alternator - Inspect the starting motor - Inspect the battery - Inspect the motors - Inspect the resistors, diodes - Inspect lights, horn | - Describe Electrical wiring - Describe the sensors - Describe the solenoids - Describe the switches - Describe the fuse - Describe the relay - Describe the alternator - Describe the starting motor - Describe the battery - Describe the motors - Describe the resistors, diodes - Describe the lights, horn Practical activity: - Capable to repair Electrical wiring - Capable to check the sensors - Capable to check the solenoids - Capable to check switches - Capable to check fuse - Capable to check relay | Theory- 02 hours Practice- 09 hours Total- 11 hours | Tools trolley (Complete set of Hand Tools) Special Service tools | Class Room and wood workshop |
| LU3. Troubleshoot of machine electrical system | Trainee will be able to: - Troubleshoot the wiring - Troubleshoot the sensors - Troubleshoot the solenoids - Troubleshoot the switches - Troubleshoot the fuse - Troubleshoot the relay - Troubleshoot the alternator/ - Troubleshoot the starting motor - Troubleshoot the battery - Troubleshoot the motors - Troubleshoot the resistors, diodes - Troubleshoot the lights, horn | - Describe Electrical wiring - Describe the sensors - Describe the solenoids - Describe the switches - Describe the fuse - Describe the relay - Describe the alternator - Describe the starting motor - Describe the battery - Describe the motors - Describe the resistors, diodes - Describe the lights, horn Practical activity: - Describe Electrical wiring - Describe the sensors - Describe the solenoids - Describe the switches - Describe the fuse - Describe the relay - Describe the alternator - Describe the starting motor - Describe the battery - Describe the motors - Describe the resistors, diodes - Describe the lights, horn | Theory- 02 hours Practice- 09 hours Total- 11 hours | Tools trolley (Complete set of Hand Tools) Special Service tools | Class Room and wood workshop |
| LU4. Repairing of machine electrical system | Trainee will be able to: - Repair the wiring - Repair the alternator - Repair the starting motor - Repair the motors | - Describe Electrical wiring - Describe the sensors - Describe the solenoids - Describe the switches - Describe the fuse - Describe the relay - Describe the alternator - Describe the starting motor - Describe the battery - Describe the motors - Describe the resistors, diodes - Describe the lights, horn Practical activity: - Describe Electrical wiring - Describe the sensors - Describe the solenoids - Describe the switches - Describe the fuse - Describe the relay - Describe the alternator - Describe the starting motor - Describe the battery - Describe the motors - Describe the resistors, diodes - Describe the lights, horn | Theory-02 hours Practice- 06 hours Total- 08 hours | Tools trolley (Complete set of Hand Tools) Special Service tools | Class Room and wood workshop |

## 9.6 Soft Skills

### Participate in Workplace Communication

Objective: This module covers the skills and knowledge required to Obtain and convey Workplace information, Participate in workplace meetings /discussions and Complete relevant work-related documents

Duration: 8 Hours									Theory: 2 Hours			Practice: 6 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Obtain and convey Workplace information | - Assess the Specific and relevant information from the appropriate sources - Convey and gather the information of Effective questioning, active listening and speaking skills. - Transfer the information using the appropriate medium and ideas. - Use appropriate non- verbal communication. - Identify and follow appropriate lines of communication with supervisors and colleagues. - Use the defined workplace procedures for the location and storage of information. - Carry out Personal interaction clearly and concisely. | - Appropriate sources: Team members, Suppliers, Trade personnel, Local government, Industry bodies - Forms: Personnel forms, telephone message forms, safety reports. | Theory-0.5 Hrs Practice-02 Hrs Total-2.5 Hrs |  | Class room |
| - Participate in workplace meetings and discussions | - Attend the team meetings on time - Clearly express your own opinions and listen other’s point of view without interruption. - Established the protocols in the courteous manner of meeting inputs with the meeting purpose and Workplace interactions. - Questions about simple routine workplace procedures and matters concerning working conditions of employment are asked and responded to Meetings outcomes are interpreted and implemented | - Workplace interactions: Face to face, Telephone, Electronic and two-way radio, written including electronic, memos, instruction, and forms, non-verbal including gestures, signals, signs and diagrams - Medium: Memorandum, Circular, Notice, Information discussion, Follow-up or verbal, instructions, Face to face communication. | Theory-01 Hrs Practice-02 Hrs Total-03 Hrs |  | Class room |
| - Complete relevant work-related documents | - Range of forms relating to conditions of employment are completed accurately and legibly - Workplace data is recorded on standard workplace forms and documents - Perform the basic mathematical processes that are used for routine calculations - Identify the errors in recording information on forms/ documents and properly acted upon - Reporting requirements to supervisor are completed according to organizational guidelines | - Protocols: Observing meeting, Compliance with meeting decisions, Obeying meeting instructions. - Storage: Manual filing system, Computer-based filing system | Theory-0.5 Hrs Practice-02 Hrs Total-2.5 Hrs |  | Class room |

- Practice Occupational Health and Safety Procedures

Objective: This module covers the skills and knowledge required to Identify hazards and risks, evaluate hazards and risks, Control hazards and risks and Maintain OHS awareness

Duration: 8 Hours									Theory: 2 Hours			Practice: 6 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Identify hazards and risks | - Identify the safety regulations and workplace safety and hazard control practices and procedures are clarified and explained based on organization procedures - Identify the hazards/risks in the workplace and their corresponding indicators to minimize or eliminate risk to co-workers, workplace and environment in accordance with organization procedures. - Contingency measures during workplace accidents, fire and other emergencies are recognized and established in accordance with organization procedures | - Safety regulations: Clean Air Act, Building code, National Electrical and Fire Safety Codes, Waste management statutes and rules - Types of Hazards/Risks: Physical hazards, Chemical hazards | Theory-0.5 Hrs Practice-1.5 Hrs Total-02 Hrs |  | Class room |
| - Evaluate hazards and risks | - Identify the terms of maximum tolerable limits which when exceeded will result in harm or damage are identified based on threshold limit values (TLV) - Identify the effects of the hazards. - Identify the OHS issues and/or concerns and safety hazards are reported to designated personnel in accordance with workplace requirements and relevant workplace OHS legislation. | - Contingency measures: Evacuation, Isolation, Decontamination, (Calling designed) emergency personnel. | Theory-0.5 Hrs Practice-1.5 Hrs Total-2 Hrs |  | Class room |
| - Control hazards and risks | - Identify the occupational Health and Safety (OHS) procedures for controlling hazards/risks in workplace are consistently followed - Procedures for dealing with workplace accidents, fire and emergencies are followed in accordance with organization OHS policies - Use of Personal protective equipment (PPE) in accordance with organization OHS procedures and practices. - Provide the appropriate assistance in the event of a workplace emergency in accordance with established organization protocol. | - Emergency-related drills and training: Fire drill, Earthquake drill, Basic life support/CPR, First aid, Spillage control, Decontamination of chemical and toxic, Disaster, preparedness/management - PPE: Mask, Gloves, Goggles, Hair Net/cap/bonnet, Face mask/shield, Ear muffs, Apron/Gown/coverall/jump suit, Anti-static suits. | Theory-0.5 Hrs Practice-1.5 Hrs Total-02 Hrs |  | Class room |
| - Maintain OHS awareness | - Practice the emergency-related drills and trainings as per established organization guidelines and procedures. - Update and complete the OHS personal records in accordance with workplace requirements. | - OHS personal records: Medical/Health records, Incident reports, Accident reports, OHS-related training completed. | Theory-0.5 Hrs Practice-1.5 Hrs Total-02 Hrs |  | Class room |

- Perform Work in Team Environment

Objective: This module covers the skills and knowledge required to Describe team role and scope, Identify own role and responsibility within team and Work as a team member

Duration: 8 Hours									Theory: 2 Hours			Practice: 6 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Describe team role and scope | - Identify the role and objective of the team from available sources of information. - Identify team parameters, reporting relationships and responsibilities from team discussions and appropriate external sources. | - Role and objective of team: Work activities in a team environment, initiative and judgment maybe demonstrated on the job, either individually or in a team environment. | Theory-0.5 Hrs Practice-02 Hrs Total-2.5 Hrs |  | Class room |
| - Identify own role and responsibility within team | - Identify the individual role and responsibilities within the team environment. - Identify and recognize the roles and responsibility of other team members. - Reporting relationships within team and external to team are identified | - Sources of information: Standard operating and/or other workplace procedures, Job procedures, Machine/equipment manufacturer’s specifications and instructions, Organizational or external personnel, Client/supplier instructions, Quality standards, OHS and environmental standards. | Theory-01 Hrs Practice-02 Hrs Total-03 Hrs |  | Class room |
| - Work as a team member | - Effective and appropriate forms of communications used and interactions undertaken with team members who contribute to known team activities and objectives - Effective and appropriate contributions made to complement team activities and objectives, based on individual skills and competencies and workplace context - Observed protocols in reporting using standard operating procedures - Contribute to the development of team work plans based on an understanding of team’s role and objectives. | - Workplace context: Work procedures and practices, Conditions of work environments, Legislation and industrial agreements, Standard work practice including the storage, safe handling and disposal of chemicals, Safety, environmental, housekeeping and quality guidelines. | Theory-0.5 Hrs Practice-02 Hrs Total-2.5 Hrs |  | Class room |

- List of Machinery, Tools & Equipment.

(FOR A CLASS OF 25 STUDENTS)

## 10.1. Internal Combustion Engine

| S# | Tools and Equipment | Quantity | S# | Consumable Material | Quantity |
| --- | --- | --- | --- | --- | --- |
|  | Car Lift | 01 No |  | Emery Paste and Stick, | 25 Nos |
|  | Compression tester | 05 Nos |  | Hose Pipe, | 25 Nos |
|  | Diesel engine vehicle or simulator | 01 No |  | Oil Seals, | 25 Nos |
|  | Drive belt tensioner gauge | 05 Nos |  | Coolants, | 15 Liters |
|  | Engine Petrol/ diesel | 01 No each |  | cotton waste. | 10 kg |
|  | Gas analyzer | 01 No |  | Engine Oil, | 10 Liters |
|  | Hand tools trolley | 05 Nos |  | Gasket Set, | 25 Nos |
|  | Measuring tools (Complete set for engine overhauling) | 05 Nos |  | grease, | 2 kg |
|  | Measuring tools (Vernier caliper, micrometer, Cylinder bore gauge) | 25 Nos |  | head Gasket, | 25 Nos |
|  | Pressure Tester | 05 Nos |  | oil filter, | 25 Nos |
|  | Radiator leak tester | 05 Nos |  | Oil sump Jain, | 25 Nos |
|  | Safety wears | 25 Nos |  | Piston ring, | 25 Nos |
|  | Sealant/gasket | 25 Nos |  | Sealant/gasket, | 25 Nos |
|  | Special tools (ring compressor, belt tensioner, torque wrench) | 25 Nos |  | Silicon, | 5 Packet’s |
|  | Tools trolley (Complete set of hand tools) | 25 Nos |  | Thermostat Valve | 25 Nos |
|  | Vacuum pump | 05 Nos |  | Valve Seats, | 25 Nos |
|  | Vehicle | 02 Nos |  |  |  |
|  | Vehicle/ simulator with a cooling system | 01 No |  |  |  |

## 10.2. Automotive Electrics and Electronics

| S# | Tools and Equipment | Quantity | S# | Consumable Material | Quantity |
| --- | --- | --- | --- | --- | --- |
|  | Ampere meter | 25 Nos |  | Battery | 05 Nos |
|  | Bar Magnet | 25 Nos |  | Bulb and Bulb Holder, | 25 Nos |
|  | Battery External charger | 25 Nos |  | Circuit Board, | 25 Nos |
|  | Battery load tester | 25 Nos |  | Thimble (Male & Female), | 25 Nos |
|  | Coil | 25 Nos |  | Wire Clumps, | 25 Nos |
|  | Compass needle | 25 Nos |  | Wires, | 05 Nos |
|  | Connecting leads | 25 Nos |  |  |  |
|  | Current carrying conductor | 25 Nos |  |  |  |
|  | Digital Multi Meter (DMM). | 25 Nos |  |  |  |
|  | Fuses | 25 Nos |  |  |  |
|  | Galvanometer | 05 Nos |  |  |  |
|  | Gloves | 25 Nos |  |  |  |
|  | Horse shoe magnet | 25 Nos |  |  |  |
|  | Hydrometer | 25 Nos |  |  |  |
|  | Iron nail as core | 25 Nos |  |  |  |
|  | Lamps | 25 Nos |  |  |  |
|  | Load (resistors) | 25 Nos |  |  |  |
|  | Magnet | 25 Nos |  |  |  |
|  | Power Supply | 05 Nos |  |  |  |
|  | Relay | 25 Nos |  |  |  |
|  | Safety goggles | 25 Nos |  |  |  |
|  | Scanner | 02 Nos |  |  |  |
|  | Special Service tools | 25 Nos |  |  |  |
|  | Test benches (Complete accessories) | 25 Nos |  |  |  |
|  | Test lamp | 25 Nos |  |  |  |
|  | Thin coated copper wire | 25 Nos |  |  |  |
|  | Timing gun | 05 Nos |  |  |  |
|  | Tool box | 25 Nos |  |  |  |
|  | Tools trolley (Complete set of Hand Tools) | 05 Nos |  |  |  |
|  | Transformer | 05 Nos |  |  |  |
|  | Vehicle or simulator. | 01 No |  |  |  |
|  | Wires | 05 Nos |  |  |  |

- Members of the Curriculum Development Committee.

| S# | Name | Designation | Organization |
| --- | --- | --- | --- |
| 1 | Engr Inayat Ur Rehman | DACUM Facilitator | Ex Director Academics KP-TVETA. |
| 2 | Mr. Adil Qadeer | Lecture | University of Lahore, |
| 3 | Mr. Muhammad Arsalan | Lecture | University of Lahore, |
| 4 | Engr Jamal Akbar | Associate Prof | GPI Sardar Garhi, Peshawar KP-TVETA |
| 5 | Engr Farooq Iftikhar | Metallurgy engineer | Pakistan Institute of Technology for Advanced Engineering Material, Lahore |
| 6 | Engr Umar, | Instructor | Swedish College Gujarat, P-TEVTA |
| 7 | Mr. Amir Amin | HOD | Polytechnic College Lahore, |
| 8 | Mr Muhammad Rizwan Zafar | HOD, Mechanical, | Technology college Lahore, |
| 9 | Ms Saba Sadiq | Lecture Mechanical, | UOL, Campus, Islamabad, |
| 10 | Fazal Rehman, | Associate Professor | GCT Peshawar, KP TEVA, |
| 11 | Saad Amjad | Project Engineer, | Cagrow consulting Company Lahore |
| 12 | Umar Qureshi | Director, | Pakistan National Accreditation Council, Islamabad |
| 13 | Syed Muhammad Syed Arsalan | Lecture, | GCT Railway Road Lahore |
| 14 | Amina Irfan | Lecture | MS Industrial Engineering, Lahore |
| 15 | Mr. Muhammad Yasir, | Dy. Director | NAVTTC, Islamabad |