Curriculum of National Certificate Level -5 in Construction Machinery

Curriculum

National Vocational and Technical Training Commission (NAVTTC)

Government of Pakistan

Contents

1.	Introduction	6

2.	Purpose of the qualification	7

3.	Overall objectives of training program	7

4.	Entry level of trainees	8

5.	Minimum qualification for teachers	8

6.	Duration of the course:	8

7.	Description and structure of the course	10

8.	Scheme of Studies.	15

9.	Detail of Modules Level wise	16

National Certificate Level-5 Part-1 in Construction Machinery	16

9.15 Power Generators (GEN-SET)	16

9.15.1	Analyze Parts of Generators	16

9.15.2	Analyze Main Alternator	18

9.15.3	Analyze Operation of Control Panel	20

9.15.4	Perform Preventive Maintenance	22

9.15.5	Carry out Trouble Shooting Guide	26

9.15.6	Perform Safety Practices and Procedures of Gen Set	30

9.16 Steering & Brake System of Construction Machinery	32

9.16.1	Recognize steering & brake system of bulldozer.	32

9.16.2	Disassemble and assemble steering clutch of bulldozer.	35

9.16.3	Disassemble and assemble steering control valve of bulldozer.	37

9.16.4	Test and adjust the steering and brake system of bulldozer	39

9.16.5	Analyze hydrostatic steering system of Bulldozer.	41

9.16.6	Analyze steering system of wheeled machines.	43

9.16.7	Test and adjust TOE-IN of motor grader	47

9.16.8	Test and adjust of steering system of wheel loader.	49

9.16.9	Recognize hydro vacuum brake used in motor grader.	52

9.16.10	Analyze air brake system for wheeled vehicles.	54

9.17 Hydraulic System of Construction Machinery	57

9.17.1	Demonstrate hydraulic system	57

9.17.2	Recognize components of Hydraulic system	59

9.17.3	Draw hydraulic circuits (schematics)	62

9.17.4	Maintain Hydraulic Pump	64

9.17.5	Recognize HYDRAULIC TANK	66

9.17.6	Demonstrate components of Hydraulic Cylinder	68

9.17.7	Maintain HSS (Hydro Static Steering) Pump and Motor	70

9.17.8	Troubleshoot of hydraulic system	73

9.18 Planning & management of construction machines	76

9.18.1	Prepare Critical Path Method Diagram	76

9.18.2	Calculate depreciation of construction machinery	79

9.18.3	Calculate the cost of owning for Construction machinery	81

9.18.4	Calculate the operating cost for construction machinery	83

9.18.5	Determine the productivity of construction machinery	87

9.18.6	Calculate the effect of grade on tractive effort of vehicles	90

9.19 Soft Skill	93

9.19.1	Develop Professionalism	93

National Certificate Level-5 Part-2 in Construction Machinery	96

9.20 Power Transmission of Construction Machinery	96

9.20.1	Demonstrate main clutches	96

9.20.2	Perform mechanical Transmission	99

9.20.3	Demonstrate the torque converter.	101

9.20.4	Perform different tests of torque converter	103

9.20.5	Demonstrate Torque flow transmission.	105

9.20.6	Demonstrate control valve assembly of torque flow transmission.	107

9.20.7	Analyse Hydro-shift Transmission.	109

9.20.8	Analyse Hydrostatic Transmission (HST).	111

9.21 Hydraulic Excavator	113

9.21.1	Demonstrate Hydraulic Excavator	113

9.21.2	Analyze hydraulic pump of excavator.	115

9.21.3	Maintain control valves of excavator.	117

9.21.4	Maintain hydraulic motors	119

9.21.5	Analyze hydraulic cylinders	122

9.21.6	Maintain Hydraulic Control Circuit	124

9.21.7	Maintain air conditioning system	127

9.21.8	Analyze Heating System	130

9.21.9	Analyze ventilation system	131

9.22 Final Drive, Under Carriage and Power Train/Line	133

9.22.1	Recognize the power train and track group	133

9.22.2	Perform inspection of track group	134

9.22.3	Adjust the Track chain tension	136

9.22.4	Disassemble and assemble the final drive	138

9.22.5	Recognize cutaway and differential assembly of a wheeled vehicle.	140

9.22.6	Disassemble & assemble inter-axle differential assembly	142

9.22.7	Recognize structure and type of Off-road Tires.	144

9.23 Entrepreneurship	146

9.23.1	Investigate micro business opportunities	146

9.23.2	Develop Micro Business Proposal	149

9.23.3	Develop Marketing Plan	151

9.23.4	Develop and Review Business Plan	154

9.23.5	Monitor & Review Business Performance	156

9.23.6	Organize Finances for Micro-Business	159

9.23.7	Manage human resources	162

9.23.8	Market the products and services	166

9.23.9	Negotiate for resolving business issues	172

10.	List of Machinery, Tools & Equipment.	174

11.	Members of the Curriculum Development Committee.	184

# Introduction

Construction Machinery Operators and Technicians are in demand across the country and abroad. Their services are required for everything from road and bridge construction, back hoe operation, bulldozing, loading and grading, dredging and heavy paving, to excavating and much, much more. This is a good career opportunity for a reliable and responsible individual with a strong work ethic. Heavy Machine Operators and Technicians not only work on regular construction building jobs, but also on infrastructure projects (roads, bridges, Dams and ports, otherwise called non-building construction), and in mining and timber operations.

In order to build the capacity of technical and vocational training institutes in Pakistan through provision of demand driven competencies-based trainings in construction sector the NAVTTC, and TEVT Sector Support Program (TSSP) have joined hands together to develop qualifications for construction sector. These qualifications will not only build the capacity of existing workers of this sector but also support the youth to acquire skills best fit for this sector. The benefits and impact of development of these qualifications will be on both demand and supply side.

Based upon this demand of industry these competency‐based qualifications for Construction Machinery are developed under National Vocational Qualification Framework (NVQF) (Level 5).  The qualifications mainly cover competencies along with related knowledge and professional skills which are essential for getting a job or self-employed.

The National Vocational & Technical Training Commission (NAVTTC) has approved the Qualification Development Committee (QDC). The QDC consists experts from the relevant industries from different geographical locations across Pakistan and academicians who were consulted during the development process to ensure input and ownership of all the stakeholders. The National Competency Standards could be used as a referral document for the development of curricula to be used by training institutions.

Construction Machinery curriculum will prepare students to efficiently operate heavy machines such as Dozers, Loaders, Excavator and Graders and to perform basic preventive maintenance on most types of heavy equipment. Coursework includes Safety, Heavy Machines operations, grades, Legal & environmental concerns & equipment maintenance. Graduates of this program may find employment with state and local government agencies and private contractors engaged in highway or other construction activities.

In fact, no sphere of life may be identified that does not include the contribution of construction machinery. Thus, the importance of construction machinery may be determined according to its usefulness in our daily life. Therefore, industry expectations for skilled workforce are also dynamic which can only be managed through setting relevant competency standards in collaboration with the leading industries. Being cognizant of this fact, National Vocational & Technical Training Commission (NAVTTC) developed competency standards for Mechanical technology with Specialization in Construction Machinery under National Vocational Qualifications Framework (NVQF). These competency standards have been developed by a Qualifications Development Committee (QDC) and validated by the Qualifications Validation Committee (QVC) having representation from the leading Construction, Real States & Builder industry of the country.

# Purpose of the qualification

The purpose of these qualifications is to set high professional standards for Construction Machinery. The specific objectives of developing these qualifications are as under:

- Improve the professional competence of the trainees

- Provide opportunities for recognition of skills attained through non-formal or informal pathways

- Improve the quality and effectiveness of training and assessment for Construction Machinery.

- Enable the existing workforce to capacitate themselves in new technologies and method so that train persons in this field can maintain construction machinery for its longer

# Overall objectives of training program

The Construction Machinery qualification of level 5 consists of both the theoretical and practical details and having the following Occupations

- Power Generators (Gen Set)

- Steering and Brake System

- Hydraulic System

- Planning and Management

- Power Transmission of Construction Machinery

- Hydraulic Excavator

- Final Drive, under Carriage and Power Line

- Entrepreneurial Skill

# Entry level of trainees

The entry for National Certificate level 5, in Construction Machinery are

- A person having National Vocational Certificate level 4, in Construction Machinery.

# Minimum qualification for teachers

- B. Tech in Mechanical Technology 4 Years

- B.Sc. / B.E. in Mechanical Engineering 4 Years

- D. A. E. in Mechanical Technology with 3 Years teaching experience

- Must be able to communicate effectively

Medium of instruction

English, and Urdu.

# Duration of the course:

The proposed Modules are composed of 64 competencies that will be covered in 1200 Learning hours. Duration of the course is proposed to be one year. The total weightage for technical competencies is 1200 hours. A total of 400 hours has been reserved for allied subjects i.e. Islamic studies, English, Mathematics, Physics and Chemistry.

The details of technical Modules are given as under:

Level - 2 = 6 months.

Level - 3 = 6 months.

Level - 4 = 1 Year.

Level - 5 = 1 Year.

The overall distribution of Learning hours and Credit Hours are given below:

Total.	3600 hours. & 360 Credits

Theory.	714 hours (19.83 %)

Practice.	2886 hours (80.17 %)

Level - 5 = 1 Year (16 weeks + 16 Weeks)

| Level-5 Part-1 | Level-5 Part-2 | Level-5 (Part-1 + Part-2) |
| --- | --- | --- |
| Total. 600 hours. & 60 Credits Theory. 120 hours (20 %) Practice. 480 hours (80 %) | Total. 600 hours. & 60 Credits Theory. 126 hours (21 %) Practice. 474 hours (79 %) | Total. 1237 hours. & 123.7 Credits Theory. 303 hours (24.5 %) Practice. 934 hours (75.5 %) |

# Description and structure of the course

Following is the structure of the course:

- National Qualification Certificate level-5, Part-1 in “Construction Machinery”

      - Power Generators (Gen Set)

| Code | Competency Standards | TH | PR | TL | Credits | Category |
| --- | --- | --- | --- | --- | --- | --- |
| CS-95 | Analyse parts of Generators | 3 | 18 | 21 | 2.1 | Functional |
| CS-96 | Analyse Main Alternator | 3 | 12 | 15 | 1.5 | Functional |
| CS-97 | Analyse Operation of Control Panel | 5 | 21 | 26 | 2.6 | Functional |
| CS-98 | Perform Preventive Maintenance | 6 | 27 | 33 | 3.3 | Functional |
| CS-99 | Carry out Trouble Shooting Guide | 6 | 27 | 33 | 3.3 | Functional |
| CS-100 | Perform Safety Practices and Procedures of Gen Set | 3 | 12 | 15 | 1.5 | Functional |
| Total |  | 26 | 117 | 143 | 14.3 |  |

      - Steering and Brake System

| Code | Competency Standards | TH | PR | TL | Credits | Category |
| --- | --- | --- | --- | --- | --- | --- |
| CS-101 | Recognize steering & brake system of bulldozer | 3 | 12 | 15 | 1.5 | Technical |
| CS-102 | Disassemble and assemble steering clutch of bulldozer | 3 | 24 | 27 | 2.7 | Technical |
| CS-103 | Disassemble and assemble the steering control valve of bulldozer | 3 | 12 | 15 | 1.5 | Technical |
| CS-104 | Test and adjust the steering and brake system of bulldozer | 4 | 21 | 25 | 2.5 | Technical |
| CS-105 | Analyse hydrostatic steering system of Bulldozer | 4 | 21 | 25 | 2.5 | Technical |
| CS-106 | Analyse steering system of wheeled machines | 5 | 27 | 32 | 3.2 | Technical |
| CS-107 | Test and adjust TOE-IN of motor grader | 2 | 18 | 20 | 2 | Technical |
| CS-108 | Test and adjust of steering system of wheel loader | 2 | 18 | 20 | 2 | Technical |
| CS-109 | Recognize hydro vacuum brake used in motor grader | 4 | 12 | 16 | 1.6 | Technical |
| CS-110 | Analyse air brake system for wheeled vehicles | 5 | 21 | 26 | 2.6 | Technical |
| Total |  | 35 | 186 | 221 | 22.1 |  |

      - Hydraulic System

| Code | Competency Standards | TH | PR | TL | Credits | Category |
| --- | --- | --- | --- | --- | --- | --- |
| CS-111 | Demonstrate Hydraulic System | 4 | 9 | 13 | 1.3 | Technical |
| CS-112 | Recognize components of Hydraulic System | 4 | 9 | 13 | 1.3 | Technical |
| CS-113 | Draw Hydraulic Circuits (Schematics) | 5 | 18 | 23 | 2.3 | Technical |
| CS-114 | Maintain Hydraulic Pump | 4 | 21 | 25 | 2.5 | Technical |
| CS-115 | Recognize Hydraulic Tank | 3 | 9 | 12 | 1.2 | Technical |
| CS-116 | Demonstrate components of Hydraulic Cylinder | 4 | 9 | 13 | 1.3 | Technical |
| CS-117 | Maintain HSS (Hydro Static Steering) Pump And Motor | 4 | 9 | 13 | 1.3 | Technical |
| CS-118 | Troubleshoot Hydraulic System | 4 | 12 | 16 | 1.6 | Technical |
| Total |  | 32 | 96 | 128 | 12.8 |  |

      - Planning and Management

| Code | Competency Standards | TH | PR | TL | Credits | Category |
| --- | --- | --- | --- | --- | --- | --- |
| CS-119 | Prepare CPM | 4 | 12 | 16 | 1.6 | Technical |
| CS-120 | Calculate depreciation of Construction Machinery | 4 | 12 | 16 | 1.6 | Technical |
| CS-121 | Calculate the cost of owning for Construction Machinery | 3 | 12 | 15 | 1.5 | Technical |
| CS-122 | Calculate the operation cost for Construction Machinery | 3 | 12 | 15 | 1.5 | Technical |
| CS-123 | Determine the productivity of Construction Machinery | 5 | 12 | 17 | 1.7 | Technical |
| CS-124 | Calculate the effect of grade on tractive effort of vehicles | 4 | 9 | 13 | 1.3 | Technical |
| Total |  | 23 | 69 | 92 | 9.2 |  |

      - Generic

| Code | Competency Standards | TH | PR | TL | Credits | Category |
| --- | --- | --- | --- | --- | --- | --- |
| CS-125 | Develop Professionalism | 4 | 12 | 16 | 1.6 | Generic |
| Total |  | 4 | 12 | 16 | 1.6 |  |
| Grand Total of Level-5 (Part-1) |  | 120 | 480 | 600 | 60 |  |

- National Qualification Certificate level-5, Part-2 in “Construction Machinery”

            - Transmission of Construction Machinery

| Code | Competency Standards | TH | PR | TL | Credits | Category |
| --- | --- | --- | --- | --- | --- | --- |
| CS-126 | Demonstrate main Clutches | 6 | 18 | 24 | 2.4 | Technical |
| CS-127 | Perform Mechanical Transmission | 6 | 21 | 27 | 2.7 | Technical |
| CS-128 | Demonstrate the Torque Converter | 4 | 21 | 25 | 2.5 | Technical |
| CS-129 | Perform Different Tests of Torque Converter | 6 | 21 | 27 | 2.7 | Technical |
| CS-130 | Demonstrate Torque flow transmission | 8 | 21 | 29 | 2.9 | Technical |
| CS-131 | Demonstrate control valve assembly of torque flow transmission | 4 | 18 | 22 | 2.2 | Technical |
| CS-132 | Analyse hydro shift transmission | 6 | 21 | 27 | 2.7 | Technical |
| CS-133 | Analyse Hydrostatic Transmission (HST) | 5 | 18 | 23 | 2.3 | Technical |
| Total |  | 45 | 159 | 204 | 20.4 |  |

            - Hydraulic Excavator

| Code | Competency Standards | TH | PR | TL | Credits | Category |
| --- | --- | --- | --- | --- | --- | --- |
| CS-134 | Demonstrate Hydraulic Excavator | 4 | 18 | 22 | 2.2 | Technical |
| CS-135 | Analyse Hydraulic Pump of Excavator | 6 | 21 | 27 | 2.7 | Technical |
| CS-136 | Maintain Control Valves of Excavator | 2 | 9 | 11 | 1.1 | Technical |
| CS-137 | Maintain Hydraulic Motors | 7 | 30 | 37 | 3.7 | Technical |
| CS-138 | Analyse Hydraulic Cylinders | 4 | 12 | 16 | 1.6 | Technical |
| CS-139 | Maintain Hydraulic Control Circuit | 6 | 21 | 27 | 2.7 | Technical |
| CS-140 | Maintain Air Conditioning System | 6 | 21 | 27 | 2.7 | Technical |
| CS-141 | Analyse Heating system | 6 | 21 | 27 | 2.7 | Technical |
| CS-142 | Analyse Ventilation system | 6 | 21 | 27 | 2.7 | Technical |
| Total |  | 47 | 174 | 221 | 22.1 |  |

            - Final Drive, under Carriage and Power Line

| Code | Competency Standards | TH | PR | TL | Credits | Category |
| --- | --- | --- | --- | --- | --- | --- |
| CS-143 | Recognize The Power Train And Track Group Of Bulldozer | 2 | 18 | 20 | 2 | Technical |
| CS-144 | Perform Inspection of Track Group | 2 | 18 | 20 | 2 | Technical |
| CS-145 | Adjust the Track Chain Tension | 2 | 18 | 20 | 2 | Technical |
| CS-146 | Disassemble and Assemble the Final Drive | 2 | 18 | 20 | 2 | Technical |
| CS-147 | Recognize Cutaway & Differential Model of a Wheeled Vehicle | 3 | 12 | 15 | 1.5 | Technical |
| CS-148 | Disassemble & Assemble Inter-Axle Differential Assembly | 2 | 18 | 20 | 2 | Technical |
| CS-149 | Recognize structure and types of Off-Road Tires | 3 | 12 | 15 | 1.5 | Technical |
| Total |  | 16 | 114 | 130 | 13 |  |

            - Entrepreneur

| Code | Competency Standards | TH | PR | TL | Credits | Category |
| --- | --- | --- | --- | --- | --- | --- |
| CS-150 | Investigate micro business opportunities | 2 | 3 | 5 | 0.5 | Generic |
| CS-151 | Develop a micro business proposal | 2 | 3 | 5 | 0.5 | Generic |
| CS-152 | Develop a marketing plan | 2 | 3 | 5 | 0.5 | Generic |
| CS-153 | Develop and review a business plan | 2 | 3 | 5 | 0.5 | Generic |
| CS-154 | Organize finances for the micro business | 2 | 3 | 5 | 0.5 | Generic |
| CS-155 | Manage human resources | 2 | 3 | 5 | 0.5 | Generic |
| CS-156 | Market the products and services | 2 | 3 | 5 | 0.5 | Generic |
| CS-157 | Monitor and review business performance | 2 | 3 | 5 | 0.5 | Generic |
| CS-158 | Negotiate for resolving business issues | 2 | 3 | 5 | 0.5 | Generic |
| Total |  | 18 | 27 | 45 | 4.5 |  |
| Grand Total of Level-5 (Part-2) |  | 126 | 474 | 600 | 60 |  |
| Grand Total of Level-5 |  | 246 | 954 | 1200 | 120 |  |
| Grand Total of Level-2,3,4 and Level-5 |  | 714 | 2886 | 3600 | 360 |  |

# Scheme of Studies.

National Certificate Level-5 Part-1 in Construction Machinery

| S.No. | Occupation or Subject | Category | Total Contact Hours |  |  | Credit | Periods Per Week |  |  | No. of Weeks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Th | Pr. | Total |  | Th | Pr | Total |  |
|  | Power Generators (Gen Set) | Technical | 26 | 117 | 143 | 14.3 | 1.63 | 7.31 | 8.94 | 16 |
|  | Steering and Brake System | Technical | 35 | 186 | 221 | 22.1 | 2.19 | 11.63 | 13.81 | 16 |
|  | Hydraulic System | Technical | 32 | 96 | 128 | 12.8 | 2.00 | 6.00 | 8.00 | 16 |
|  | Planning and Management | Technical | 23 | 69 | 92 | 9.2 | 1.44 | 4.31 | 5.75 | 16 |
|  | Generic | Generic | 4 | 12 | 16 | 1.6 | 0.25 | 0.75 | 1.00 | 16 |
|  |  | Allied |  |  |  |  |  |  |  |  |
|  | Total |  | 120 | 480 | 600 | 60 | 7.5 | 30 | 37.5 |  |

National Certificate Level-5 Part-2 in Construction Machinery

| S.No. | Occupation or Subject | Category | Total Contact Hours |  |  | Credit | Periods Per Week |  |  | No. of Weeks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Th | Pr. | Total |  | Th | Pr | Total |  |
|  | Power Transmission of Construction Machinery | Technical | 45 | 159 | 204 | 20.4 | 2.81 | 9.94 | 12.75 | 16 |
|  | Hydraulic Excavator | Technical | 47 | 174 | 221 | 22.1 | 2.94 | 10.88 | 13.81 | 16 |
|  | Final Drive, under Carriage and Power Line | Technical | 16 | 114 | 130 | 13 | 1.00 | 7.13 | 8.13 | 16 |
|  | Entrepreneur | Generic | 18 | 27 | 45 | 4.5 | 1.13 | 1.69 | 2.81 | 16 |
|  |  | Allied |  |  |  |  |  |  |  |  |
|  | Total |  | 126 | 474 | 600 | 60 | 7.88 | 29.63 | 37.50 |  |

- Detail of Modules Level wise

## National Certificate Level-5 Part-1 in Construction Machinery

## 9.15 Power Generators (GEN-SET)

- Analyze Parts of Generators

Objective: This module covers the skills and knowledge required to Identify parts of generator, Inspection of generator and Troubleshooting of generator

Duration: 21 Hours						     Theory: 3 Hours						 Practice: 18 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Identify parts of generator | Trainee will be able to: - Identify engine - Identify main alternator - Identify AVR (automatic voltage regulator) - Identify ATS (auto transfer switch) - Identify control panel - Identify circuit barker - Identify cables - Identify anti vibration Mountings | - Describe Engine Practical activity: - Inspect of the engine - Inspect of main alternator | Theory- 1 hour Practice- 6 hours Total- 7 hours | Generator PP&E Overall Safety shoes Safety helmet Goggles Gloves | Class room and Generator room |
| - Inspection of generator | Trainee will be able to: - Inspect of the engine - Inspect of main alternator - Inspect of AVR (automatic voltage regulator) - Inspect of ATS (auto transfer switch) - Inspect the control panel - Inspect the circuit barker - Inspect the cables - Inspect of anti-vibration mountings | - Describe main alternator Practical activity: - Inspect of AVR (automatic voltage regulator) - Inspect of ATS (auto transfer switch) - Inspect the control panel | Theory- 1 hour Practice- 6 hours Total- 7 hours | Generator PP&E Overall Safety shoes Safety helmet Goggles Gloves | Class room and Generator room |
| - Troubleshoot of generator | Trainee will be able to: - Troubleshoot engine - Troubleshoot main alternator - Troubleshoot control panel - Troubleshoot AVR (automatic voltage regulator) - Troubleshoot ATS (auto transfer switch) - Troubleshoot circuit barker - Troubleshoot cables | - Describe power train of excavator Practical activity: - Troubleshoot ATS (auto transfer switch) - Troubleshoot circuit barker - Troubleshoot cables | Theory- 1 hour Practice- 6 hours Total- 7 hours | Generator PP&E Overall Safety shoes Safety helmet Goggles Gloves | Class room and Generator room |

- Analyze Main Alternator

Objective: This module covers the skills and knowledge required to Identification of main alternator, inspection of main alternator and Troubleshooting of main alternator.

Duration: 15 Hours						     Theory: 3 Hours						 Practice: 12 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Identify main alternator | Trainee will be able to: - Identify stator - Identify rotor - Identify exciter - Identify rectifiers - Identify fan - Identify coupling disc | - Describe rotor - Describe Stator Practical activity: - Identify stator - Identify rotor | Theory- 1 hour Practice- 6 hours Total- 7 hours | Generator PP&E Overall Safety shoes Safety helmet Goggles Gloves | Class room and Generator room |
| - Inspect main alternator | Trainee will be able to: - Inspect stator - Inspect rotor - Inspect exciter - Inspect rectifiers - Inspect fan - Inspect coupling disc | - Describe exciter - Describe rectifier Practical activity: - inspect exciter - inspect rectifiers | Theory- 1 hour Practice- 3 hours Total- 4 hours | Generator PP&E Overall Safety shoes Safety helmet Goggles Gloves | Class room and Generator room |
| - Troubleshoot of main alternator | Trainee will be able to: - Troubleshoot stator - Troubleshoot rotor - Troubleshoot exciter - Troubleshoot rectifiers - Troubleshoot fan - Troubleshoot coupling disc | - Describe fan - Describe coupling disc Practical activity: - Troubleshoot fan - Troubleshoot coupling disc | Theory- 1 hour Practice- 3 hours Total- 4 hours | Generator PP&E Overall Safety shoes Safety helmet Goggles Gloves | Class room and Generator room |

- Analyze Operation of Control Panel

Objective: This module covers the skills and knowledge required to the Operation of control panel, Inspection of control panel and Troubleshooting of control panel

Duration: 26 Hours						     Theory: 5 Hours						 Practice: 21 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Perform Operation of control panel | Trainee will be able to: - Perform starting/auto starting/stopping of control panel - Check voltage, frequency, amperes, rpm, temperature, oil pressure, hour meters, load, error codes of control panel - Check fuse, relay, timer, sensors of control panel | - Knowledge of start/auto start/stop. - Knowledge of Fault Indication. Practical activity: - Capable to trouble shoot the control panel - Capable to perform the parameter setting/Calibration. | Theory- 3 hours Practice- 9 hours Total- 12 hours | Generator PP&E Overall Safety shoes Safety helmet Goggles Gloves | Class room and Generator room |
| - Inspect control panel | Trainee will be able to: - Inspect starting/auto starting/stopping of control panel - Inspect voltage, frequency, amperes, rpm, temperature, oil pressure, hour meters, load, error codes of control panel - Inspect fuse, relay, timer, sensors of control panel | - How to read the Fault Code. - How to perform task on control panel - How to check the faulty control panel Practical activity: - Inspect voltage, frequency, amperes, rpm, temperature, oil pressure, hour meters, load, error codes of control panel - Inspect fuse, relay, timer, sensors of control panel | Theory- 1 hour Practice- 6 hours Total- 7 hours | Generator PP&E Overall Safety shoes Safety helmet Goggles Gloves | Class room and Generator room |
| - Troubleshoot of control panel | Trainee will be able to: - Check Start/Stop of control panel - Check Power Supply to control panel - Check the control panel for Fault Indication. - Enter password to access setting of control panel or connect laptop to control panel for parameter setting/Programming. | - How to establish connection of control panel with Laptop. - Knowledge to perform parameter setting from control panel. - Knowledge of safety protection Practical activity: - Operate the control panel for Fault Indication. - Practice Entering password to access setting of control panel or connect laptop to control panel for parameter setting/Programming. | Theory- 1 hour Practice- 6 hours Total- 7 hours | Generator PP&E Overall Safety shoes Safety helmet Goggles Gloves | Class room and Generator room |

- Perform Preventive Maintenance

Objective: This module covers the skills and knowledge required to perform Daily maintenance of Engine, operating hour maintenance of Engine,500 HR maintenance of engine,1000 HR maintenance of engine,2000 HR maintenance of engine, Alternator preventive maintenance, Decommissioning, Dismantling and disposal.

Duration: 33 Hours						     Theory: 6 Hours						 Practice: 27 Hour

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Perform Daily maintenance of Engine | Trainee will be able to: - Drain water from fuel filter. - Check fuel level. - Empty air filter evacuator valve. - Check engine oil level. - Check Coolant Level. - Check electrolyte level of Battery. - Check for fault indication (If any). - Check on abnormal noise. - Check obstruction on crankcase breather. | - Knowledge of electrically simulating faults - Knowledge of cleaning all battery cap vents Practical activity: - Drain water from fuel filter. - Practice Checking fuel level. - Empty air filter evacuator valve. - Check engine oil level. | Theory- 1 hour Practice- 6 hours Total- 7 hours | Generator PP&E Overall Safety shoes Safety helmet Goggles Gloves | Class room and Generator room |
| - Perform 250 operating hour maintenance of Engine | Trainee will be able to: - Change Engine Oil. - Change Oil Filter. - Check hoses & Pipes for any leakage/damage. - Inspect/Adjust fan/alternator belt. - Check Indicator Air Filter (Clean Filter if necessary). | - Knowledge of tightening all exhaust connections - Knowledge of tightening all electric connections Practical activity: - Change Engine Oil. - Change Oil Filter. - Check hoses & Pipes for any leakage/damage. | Theory- 1hour Practice- 6 hours Total- 7 hours | Generator PP&E Overall Safety shoes Safety helmet Goggles Gloves | Class room and Generator room |
| - Perform 500 Hour maintenance of engine. | Trainee will be able to: - Change fuel filter (primary/secondary) - Change air filter primary. - Test earth leak relay. - Check emergency stop - Clean radiator - Check hoses, pipes & clamps (replace if necessary) - Check electrolyte level & battery terminal - Analyze coolant - Drain/clean fuel tank water & sediment. | - Types of alternator windings Practical activity: - Change air filter primary. - Test earth leak relay. - Check emergency stop - Clean radiator | Theory- 1 hour Practice- 3 hours Total- 4 hours | Generator PP&E Overall Safety shoes Safety helmet Goggles Gloves | Class room and Generator room |
| - Perform 1000 Hour maintenance of engine | Trainee will be able to - Replace air filter safety. - Replace fan/alternator belt - Check electrical system cables for wear - Check/Test glow plugs - Check torque on critical bolt connection - Check rubber flexible - Adjust engine inlet/outlet valve - Check engine protective devices | - Knowledge of instrument panel gauges and meters Practical activity: - Check rubber flexible - Adjust engine inlet/outlet valve - Check engine protective devices | Theory- 0.75 hours Practice- 3 hours Total- 3.75 hours | Generator PP&E Overall Safety shoes Safety helmet Goggles Gloves | Class room and Generator room |
| - Perform 2000 Hour maintenance of engine | Trainee will be able to: - Check fuel injectors - Check starting motor - Check turbocharger - Check water pump - Inspect charging alternator | - Knowledge of circuit breakers Practical activity: - Practice to Check turbocharger - Check water pump - Inspect charging alternator | Theory-0.75 hours Practice- 3 hours Total- 3.75 | Generator PP&E Overall Safety shoes Safety helmet Goggles Gloves | Class room and Generator room |
| - Carry out Alternator preventive maintenance | Trainee will be able to: - Inspect the alternator winding conditions and carry out general cleaning - Inspect cable connections between the alternator and circuit breaker - Measure alternator insulation resistance (After 1000 operating hour) | - Knowledge of daily start up checks Practical activity: - Inspect cable connections between the alternator and circuit breaker - Measure alternator insulation resistance (After 1000 operating hour) | Theory-0.75 hours Practice- 3 hours Total- 3.75 | Generator PP&E Overall Safety shoes Safety helmet Goggles Gloves | Class room and Generator room |
| - Decommission, Dismantle and disposal of material | Trainee will be able to: - Consult your local Dealer and local regulations regarding the disposal of materials on your generator set - Contact with companies specializing in recycling of iron, steel and copper from gen set - Consult a specialist company regarding the removal of other components like gaskets and seals of gen set etc. | - Knowledge of weekly checks - Knowledge of how to measure the insulation resistance of alternator Practical activity: - Perform special treatment for some components such as electrical cable, electronic accessories and plastics as per standard | Theory-0.75 hours Practice- 3 hours Total- 3.75 | Generator PP&E Overall Safety shoes Safety helmet Goggles Gloves | Class room and Generator room |

- Carry out Trouble Shooting Guide

Objective: This module covers the skills and knowledge required to Analyse why Engine Fails to Start, Check Engine stoppage due to high coolant temperature, Troubleshot fan malfunction, Analyse auto mode (on, off) of generator, Check Alarm For Not In Auto Mode (Standby Sets Only), Check Output Voltage, Analyse why Output Voltage low, Analyse Voltage drop when load is imposed and Check why Voltage Higher than rated Voltage

Duration: 33 Hours						     Theory: 6 Hours						   Practice: 27 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Analyze why Engine Fails to Start | Trainee will be able to: - Check all emergency stop push buttons are released - Check the stop button light is not on - Check there are no shutdown events active - Reset, if required, after remedying the indicated fault - Refer to your instructor if necessary | - Knowledge of display in Gen Set Practical activity: - Check there are no shutdown events active - Reset, if required, after remedying the indicated fault | Theory- 40 mins Practice- 3 hours Total- 3.66 hours | Generator PP&E Overall Safety shoes Safety helmet Goggles Gloves | Class room and Generator room |
| - Check Engine stoppage due to high coolant temperature. | Trainee will be able to: - Check coolant level in the radiator. - Refer to safety section before removing the radiator cap - Remove radiator cap safely using cloth - Fill water if necessary - Check the radiator - Check Thermostat - Check Water Pump - Refer to your instructor if necessary | - Knowledge of engine gauges Practical activity: - Remove radiator cap safely using cloth - Fill water if necessary - Check the radiator - Check Thermostat | Theory- 40 mins Practice- 3 hours Total- 3.66 hours | Generator PP&E Overall Safety shoes Safety helmet Goggles Gloves | Class room and Generator room |
| - Troubleshoot of fan malfunction | Trainee will be able to: - Check radiator fan working or not - Check fuse of radiator fan - Check electrical supply of radiator fan - Ensure that electric connecters are attached properly - Refer to your instructor if necessary | - Knowledge of emergency signals Practical activity: - Check fuse of radiator fan - Check electrical supply of radiator fan - Ensure that electric connecters are attached properly | Theory- 40 mins Practice- 3 hours Total- 3.66 hours | Generator PP&E Overall Safety shoes Safety helmet Goggles Gloves | Class room and Generator room |
| - Analyze auto mode (on, off) of generator | Trainee will be able to: - Check that the generator set stops - Depress emergency stop push button - Held down the stop key for 5 seconds - Skip cool down time - Refer to your instructor if necessary | - Knowledge of signal LED’s Practical activity: - Check that the generator set stops - Depress emergency stop push button - Held down the stop key for 5 seconds | Theory- 40 mins Practice- 3 hours Total- 3.66 hours | Generator PP&E Overall Safety shoes Safety helmet Goggles Gloves | Class room and Generator room |
| - Check Alarm for Not in Auto Mode (Standby Sets Only) | Trainee will be able to: - Check the module is in “auto” mode - Check emergency stop push buttons are not pressed - Refer to your instructor if necessary | - Knowledge of water pump Practical activity: - Check emergency stop push buttons are not pressed refer to your instructor if necessary | Theory- 40 mins Practice- 3 hours Total- 3.66 hours | Generator PP&E Overall Safety shoes Safety helmet Goggles Gloves | Class room and Generator room |
| - Check Output Voltage | Trainee will be able to: - Fuse on AVR broken. - AVR broken. - Rotating rectifier (diode bridge) defect. - Alternator winding burned or broken. - Bad wire connection | - Knowledge of Thermostat Practical activity: - Rotating rectifier (diode bridge) defect. - Alternator winding burned or broken. | Theory- 40 mins Practice- 3 hours Total- 3.66 hours | Generator PP&E Overall Safety shoes Safety helmet Goggles Gloves | Class room and Generator room |
| - Analyze why Output Voltage low | Trainee will be able to: - Adjust voltage on AVR. - Check alternator speed. - AVR defect. - Alternator windings damaged | - Knowledge of ATS (auto transfer switch) Practical activity: - Check alternator speed. - AVR defect. - Alternator windings damaged | Theory- 40 mins Practice- 3 hours Total- 3.66 hours | Generator PP&E Overall Safety shoes Safety helmet Goggles Gloves | Class room and Generator room |
| - Analyze Voltage drop when load is imposed | Trainee will be able to: - Check LED’s on AVR. - Check nominal power with load requirements. Yellow LED: overload or cosj = bad. - Power problem on engine.Red LED: under speed. - AVR defect. - Alternator windings | - Knowledge of cool down time Practical activity: - Power problem on engine. Red LED: under speed. - AVR defect. | Theory- 40 mins Practice- 3 hours Total- 3.66 hours | Generator PP&E Overall Safety shoes Safety helmet Goggles Gloves | Class room and Generator room |
| - Check why Voltage Higher than rated Voltage | Trainee will be able to: - Unbalanced load. - Nonlinear load, harmonics. - AVR defect. | - Knowledge of engine gauges Practical activity: - Check LED’s on AVR. - Check nominal power with load requirements. | Theory- 40 mins Practice- 3 hours Total- 3.66 hours | Generator PP&E Overall Safety shoes Safety helmet Goggles Gloves | Class room and Generator room |

- Perform Safety Practices and Procedures of Gen Set

Objective: This module covers the skills and knowledge required to Adopt Health & Safety regulations, Encourage primary safety program,

Duration: 15 Hours						     Theory: 3 Hours						 Practice: 12 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Adopt Health & Safety regulations. | Trainee will be able to: - Do not charge a frozen battery, this may cause an explosion - Ensure the generator set room is properly ventilated - Keep the room, the floor and the generator set clean. - When spills of fuel, oil, battery electrolyte or coolant occur, they should be cleaned up immediately - Never store flammable liquids near the engine - Identify rights & responsibilities regarding safety - Interpret regulations & guidelines specific to Heavy Machines. - Interpret common safety rules and tips. - Identify employer safety rules and policies. | - Define Hazard. - Describe types of hazard. - Knowledge and proper use of Personal Protective Equipment (PPE). - Describe Occupational Health & Safety Regulations. - Describe Typical worksite Hazards. Practical activity: - Practice of ensuring the generator set room is properly ventilated - Keep the room, the floor and the generator set clean. - When spills of fuel, oil, battery electrolyte or coolant occur, they should be cleaned up immediately | Theory- 1.5 hours Practice- 6 hours Total- 7.5 hours | Generator PP&E Overall Safety shoes Safety helmet Goggles Gloves | Class room and Generator room |
| - Encourage primary safety program | Trainee will be able to: - Motivate by regulation. - Motivate by ethics, legitimate concern - Motivate by cost of lost time and injury Claims. - Motivate by liability | - Describe factors affecting Health & Safety in the workplace. - Knowledge about First aid box. - Usage of first aid box accident history in different types of industries Environment safety Practical activity: - Practice Motivation by regulation. - Motivate by ethics, legitimate concern - Motivate by cost of lost time and injury Claims. | Theory- 1.5 hours Practice- 6 hours Total- 7.5 hours | Generator PP&E Overall Safety shoes Safety helmet Goggles Gloves | Class room and Generator room |

# 9.16 Steering & Brake System of Construction Machinery

    - Recognize steering & brake system of bulldozer.

Objective: This module covers the skills and knowledge required to identify main components of steering & brake system and powertrain of bulldozer.

Duration: 15 Hours							Theory: 3 Hours				Practice: 12 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Identify the layout of steering & brake system of bulldozer. | Trainee will be able to: - Identify steering and brake control lever and pedals. - Identify steering pump & filter - Identify steering control valve - Identify steering case - Identify steering clutch & brake unit - Identify the modulating steering valve. | - Describe Steering clutches structure. - Describe Steering clutches operation. - Describe hydraulic circuits of steering clutches. - Describe hydraulic circuits of brake circuit. Practical activity: - Identify steering pump & filter - Identify steering control valve - Identify steering case | Theory- 0.6 hours Practice- 2.4 hours Total- 3 hours | Bulldozer steering and brake cutaway model PP&E Coverall Safety shoes Safety helmet | Class Room and hydraulic workshop |
| - Demonstrate power Train of Bulldozer | Trainee will be able to: - Locate Engine - Locate main clutch / Torque converter - Locate Transmission and Transmission Pinion - Locate Bevel gear and Axle shaft - Locate steering clutches - Locate Final Drives - Locate sprockets and Track chains. | - Describe power train of Bulldozer Practical activity: Identify power train of bulldozer | Theory- 0.6 hours Practice- 2.4 hours Total- 3 hours | Bulldozer PP&E Coverall Safety shoes Safety helmet | Class Room and hydraulic workshop |
| - Demonstrate on cutaway model of steering clutch of CL-I type. | Trainee will be able to: - Identify clutch drum & Brake drum - Identify disc & plates of clutch - Identify bearing, cage, piston & spring of clutch assembly. | - Describe Steering clutch structure. Practical activity: Identify cutaway model of steering clutch of CL-I | Theory- 0.6 hours Practice- 2.4 hours Total- 3 hours | cutaway model of steering clutch of CL-I PP&E Coverall Safety shoes Safety helmet | Class Room and hydraulic workshop |
| - Demonstrate on cutaway model of steering clutch of CL-III type. | Trainee will be able to - Identify clutch drum & Brake drum - Identify disc & plates of steering clutch - Identify bearing, cage, piston, pressure Plate & spring of steering clutch assembly. - Demonstrate difference between CL-I and CL-III steering clutches. | - Describe Steering clutch structure. Practical activity: Identify scutaway model of steering clutch of CL-III type. | Theory- 0.6 hours Practice- 2.4 hours Total- 3 hours | cutaway model of steering clutch of CL-III type. PP&E Coverall Safety shoes Safety helmet | Class Room and hydraulic workshop |
| - Demonstrate hydraulic circuit of steering & brake system of bulldozer | Trainee will be able to - Demonstrate steering hydraulic circuits in straight crawling, right and left turning - Demonstrate brake hydraulic circuits in straight crawling, right and left turning - Identify Different Take off ports for pressure gauges | - Describe hydraulic circuit of steering clutch. - Describe hydraulic circuit of brake circuit Practical activity: | Theory- 0.6 hours Practice- 2.4 hours Total- 3 hours | Bulldozer PP&E Coverall Safety shoes Safety helmet | Class Room & hydraulic workshop |

    - Disassemble and assemble steering clutch of bulldozer.

Objective: This module covers the skills and knowledge required to disassembling and assembling of steering clutch of bulldozer.

Duration: 27 Hours								Theory: 3 Hours				Practice: 24 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Disassemble steering clutch of bulldozer. | Trainee will be able to: - Arrange proper tools to Disassemble the steering clutch - Disassemble the brake band - Disassemble the brake drum - Remove the bolts - Remove the pressure plate - Remove disc & plates - Disassemble clutch drum. - Remove springs & sleeves. - Remove clutch piston | Describe disassembling chart of steering clutch Practical activity: - Remove the bolts - Remove the pressure plate - Remove disc & plates . | Theory- 1.5 hours Practice- 12 hours Total- 13.5 hours | Tools trolley (Complete set of Hand Tools). Special Service tools steering clutch assembly PP&E Coverall Safety shoes Safety helmet | Class Room and Hydraulic workshop |
| - Assemble steering clutch of bulldozer. | Trainee will be able to: - Install clutch piston - Install springs & sleeves. - Install clutch drum - Install disc & plates. - Install pressure plate. - Install bolts. - Install the brake drum - Install brake band | Describe assembling chart of steering clutch Practical activity: - Assemble disc & plates. - Install pressure plate. - Install bolts. | Theory- 1.5 hours Practice- 12 hours Total- 13.5 hours | Tools trolley (Complete set of Hand Tools). Special Service tools steering clutch assembly PP&E Coverall Safety shoes Safety helmet | Class Room and Hydraulic workshop |

    - Disassemble and assemble steering control valve of bulldozer.

Objective: This module covers the skills and knowledge required to disassembling & assembling of steering control valve assembly of bulldozer.

Duration: 15 Hours								Theory: 3 Hours				Practice: 12 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Disassemble the steering control valve of bulldozer | Trainee will be able to: - Arrange Proper tools to Disassemble the steering control valve of bulldozer - Disassemble steering relief valve - Remove brake valve assembly from steering valve assembly. - Disassemble steering valve assembly. - Disassemble brake valve assembly | - Explain Disassembling Procedure of steering control valve assembly Practical activity: - Remove brake valve assembly from steering valve assembly. - Disassemble brake valve. | Theory- 1.5 hours Practice- 6 hours Total- 7.5 hours | Tools trolley (Complete set of Hand Tools). Special Service tools steering control valve assembly. PP&E Coverall Safety shoes Safety helmet | Class Room and Hydraulic workshop |
| - Assemble the steering control valve of bulldozer | Trainee will be able to: - Arrange Proper tools to Assemble the steering control valve of bulldozer - Inspect, clean and replace faulty parts of steering control valve of bulldozer. - Assemble brake valve parts. - Assemble steering valve parts. - Install clutch & brake valve assembly together. - Assemble relief valve. | - Explain assembling Procedure of steering control valve assembly Practical activity: - Assemble brake valve parts. - Assemble steering valve parts. | Theory- 1.5 hours Practice- 6 hours Total- 7.5 hours | Tools trolley (Complete set of Hand Tools). Special Service tools steering control valve assembly. PP&E Coverall Safety shoes Safety helmet | Class Room and Hydraulic workshop |

    - Test and adjust the steering and brake system of bulldozer

Objective: This module covers the skills and knowledge required to testing & adjusting of steering and brake system of bulldozer.

Duration: 25 Hours									Theory: 4 Hours				Practice: 21 Hour

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Test steering system of bulldozer. | Trainee will be able to: - Arrange Proper tools to Test the steering system of bulldozer. - Measure the lever pulling force. - Measure steering lever travel - Measure steering relief pressure. - Measure Left and right steering clutches disengaging pressure. | - Describe the testing procedure of steering system of Bulldozer. Practical activity: - Measure the lever pulling force. - Measure steering lever travel - Measure steering relief pressure. | Theory- 1 hours Practice- 5.25 hours Total- 6.25 hours | Tools trolley (Complete set of Hand Tools) Special Service tools compression gauge set PP&E Coverall Safety shoes Safety helmet | Class Room and hydraulic workshop |
| - Adjust steering system of bulldozer | Trainee will be able to: - Adjust the lever pulling force. - Adjust steering lever travel - Adjust steering relief pressure. - Adjust Left and right steering clutches disengaging pressure | - Describe the adjusting procedure of steering system of Bulldozer. Practical activity: - Adjust steering lever travel - Adjust steering relief pressure. - Adjust Left and right steering | Theory- 1 hours Practice- 5.25 hours Total- 6.25 hours | Tools trolley (Complete set of Hand Tools) Special Service tools compression gauge set PP&E Coverall Safety shoes Safety helmet | Class Room and hydraulic workshop |
| - Test the brake system of bulldozer | Trainee will be able to: - Arrange Proper tools to Test the brake system of bulldozer. - Measure the pedal pulling force. - Measure the brake pedal travel. - Measure left and right brake relief pressure | - Describe the testing procedure of brake system of Bulldozer. Practical activity: - Measure the brake pedal travel | Theory- 1 hours Practice- 5.25 hours Total- 6.25 hours | Tools trolley (Complete set of Hand Tools) Special Service tools compression gauge set PP&E Coverall Safety shoes Safety helmet | Class Room and hydraulic workshop |
| - Adjust the brake system of bulldozer. | Trainee will be able to - Adjust the pedal pulling force. - Adjust the brake pedal travel. - Adjust left and right brake relief pressure | - Describe the adjusting procedure of brake system of Bulldozer. Practical activity: - Adjust the pedal pulling force | Theory- 1 hours Practice- 5.25 hours Total- 6.25 hours | Tools trolley (Complete set of Hand Tools) Special Service tools compression gauge set PP&E Coverall Safety shoes Safety helmet | Class Room and hydraulic workshop |

    - Analyze hydrostatic steering system of Bulldozer.

Objective: This module covers the skills and knowledge required to identify main components and testing & adjusting of hydrostatic steering system of bulldozer.

Duration: 25 Hours								Theory: 4 Hours				Practice: 21 Hours

| + | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Demonstrate hydrostatic steering system of bulldozer | - Trainee will be able to: - Identify the main components on model. - Trace the power train on machine during straight traveling. - Trace the power train on machine during right turn. - Trace the power train on machine during pivot turning. | - Describe of hydrostatic steering system of bulldozer - Practical activity: - Identify HSS components. | Theory- 2 hours Practice- 10.5 hours Total- 12.5 hours | Bulldozer PP&E Coverall Safety shoes Safety helmet | Class Room and Hydraulic workshop |
| LU2. Test and adjust the hydrostatic steering pump oil pressure of bulldozer. | - Trainee will be able to: - Read the HSS Oil pressure on dash board monitor. - Measure HSS Oil pressure with the help of gauge. - Compare the reading with maintenance standards on shop manual - Adjust oil pressure if needed. | - Describe testing and adjusting procedure of hydrostatic steering system. - Practical activity: - Measure HSS Oil pressure with the help of gauge. | Theory- 2 hours Practice- 10.5 hours Total- 12.5 hours | Tools trolley (Complete set of Hand Tools). Special Service tools Bulldozer Pressure gauge set PP&E Coverall Safety shoes Safety helmet | Class Room and Hydraulic workshop |

    - Analyze steering system of wheeled machines.

Objective: This module covers the skills and knowledge required to identify main components and hydraulic circuits of wheel loader and motor grader.

Duration: 32 Hours								Theory: 5 Hours				Practice: 27 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Demonstrate steering system of wheel loader on machine. | Trainee will be able to: - Identify steering wheel. - Identify steering column - Identify Re-circulating ball type steering gear. - Identify steering control valve assembly. - Identify two-way restrictor valve. - Identify steering cylinder. - Identify steering pump. - Locate Hydraulic Tank | - Describe steering system of wheel loader. Practical activity: - Identify two-way restrictor valve | Theory- 0.8 hours Practice-4.5 hours Total- 5.3 hours | wheel loader PP&E Coverall Safety shoes Safety helmet | Class Room and hydraulic workshop |
| - Identify /Draw steering circuits of wheel loader. | Trainee will be able to: - Identify the steering Circuit of wheel loader in straight travelling. - Identify the steering Circuit of wheel loader in right turning. - Identify the steering Circuit of wheel loader in left turning. - Draw the steering Circuit of wheel loader in straight travelling. - Draw the steering Circuit of wheel loader in right turning. - Draw the steering Circuit of wheel loader in left turning. | - Describe steering circuits of wheel loader. Practical activity: | Theory- 0.8 hours Practice-4.5 hours Total- 5.3 hours | wheel loader PP&E Coverall Safety shoes Safety helmet | Class Room and hydraulic workshop |
| - Identify Different Valves inside the Steering control valve of wheel loader. | Trainee will be able to: - Identify the check valve in cutaway model. - Locate demand spool in cutaway model - Identify the orifice - Recognize the steering spool - Identify the safety suction valve | - Describe orbit roll type steering valve. Practical activity: Identify the safety suction valve | Theory- 0.8 hours Practice-4.5 hours Total- 5.3 hours | Steering control valve assembly of wheel loader PP&E Coverall Safety shoes Safety helmet | Class Room and hydraulic workshop |
| - Demonstrate steering system of motor grader on machine. | Trainee will be able to - Identify steering wheel. - Identify steering column. - Identify orbit roll type steering valve - Identify steering cylinder - Identify steering pump. - Point out flow control valve. - Locate hydraulic tank | - Describe steering system of motor grader. Practical activity: - Identify steering pump | Theory- 0.8 hours Practice-4.5 hours Total- 5.3 hours | motor grader PP&E Coverall Safety shoes Safety helmet | Class Room and hydraulic workshop |
| - Demonstrate Orbit Roll Type Steering Valve from cut away model | Trainee will be able to - Point out ports of orbit roll steering valves. - Point out rotor and stator of hand pump. - Locate drive shaft. - Point out center pin and x-spring. - Locate sleeve. - Locate spool. Point out check valve. | - Describe Orbit Roll Type Steering Valve assembly. Practical activity: - Identify center pin and x-spring. | Theory- 0.8 hours Practice-4.5 hours Total- 5.3 hours | cut away model of Orbit Roll Type Steering Valve assembly PP&E Coverall Safety shoes Safety helmet | Class Room & hydraulic workshop |
| - Identify /Draw Identify steering circuit of motor grader. | Trainee will be able to - Identify the steering Circuit of motor grader in straight travelling. - Identify the steering Circuit of motor grader in right turning. - Identify the steering Circuit of motor grader r in left turning. - Draw the steering Circuit of motor grader in straight travelling. - Draw the steering Circuit of motor grader in right turning. - Draw the steering Circuit of motor grader in left turning. | - Describe steering system of motor grader. Practical activity: | Theory- 0.8 hours Practice-4.5 hours Total- 5.3 hours | Motor grader PP&E Coverall Safety shoes Safety helmet | Class Room & hydraulic workshop |

    - Test and adjust TOE-IN of motor grader

Objective: This module covers the skills and knowledge required to testing and adjusting of TOE-IN of motor grader.

Duration: 20 Hours								Theory: 2 Hours				Practice: 18 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Test TOE-IN of motor grader. | Trainee will be able to: - Arrange Proper tools to Test TOE-IN of motor grader. - Stop a machine on a level surface - Measure distance between front wheels from front. - Measure distance between front wheels from rear. - Calculate difference between above two readings to find out TOE-IN. - Compare the above TOE-IN value with stander given in Shop Manual. | - Describe the testing procedure of TOE-IN of Motor Grader. Practical activity: - Measure distance between front wheels from rear. | Theory- 1 hours Practice- 9 hours Total- 10 hours | Tools trolley (Complete set of Hand Tools). Special Service tools motor grader PP&E Coverall Safety shoes Safety helmet | Class Room and Hydraulic workshop |
| - Adjust TOE-IN of motor grader | - Trainee will be able to: - Arrange Proper tools to adjust TOE-IN of motor grader. - Park the grader on level ground. - Loose the mounting lock nuts. - Turn right side tie rod clock wise or anti clock wise to get the specified TOE-IN range. - Turn left side tie rod clock wise or anti clock wise to get the specified TOE-IN range. - Tighten the mounting lock nuts. | - Describe the adjusting procedure of TOE-IN of Motor Grader. Practical activity: | Theory- 1 hours Practice- 9 hours Total- 10 hours | Tools trolley (Complete set of Hand Tools). Special Service tools motor grader PP&E Coverall Safety shoes Safety helmet | Class Room and Hydraulic workshop |

    - Test and adjust of steering system of wheel loader.

Objective: This module covers the skills and knowledge required to testing and adjusting of steering system of wheel loader.

Duration: 20 Hours								Theory: 2 Hours				Practice: 18 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Measure steering wheel play | Trainee will be able to: - Confirm the neutral position of steering wheel put mark 1 on the wheel. - Turn steering wheel clock wise up till tires on road begin to turn put mark 2 on steering wheel. - Turn steering wheel anti clock wise up till tires on road begin to turn put mark 3 on steering wheel. - Measure the straight distance between mark 2 and 3. | - Describe the testing procedure of steering wheel play of wheel loader. Practical activity: - Measure the straight distance between mark 2 and 3. | Theory- 0.3 hours Practice- 3 hours Total- 3.3 hours | Tools trolley (Complete set of Hand Tools). Special Service tools wheel loader PP&E Coverall Safety shoes Safety helmet | Class Room and hydraulic workshop |
| - Test and adjust the free play of steering wheel of wheel loader. | Trainee will be able to: - Inspect the steering linkages for looseness and free play. - Loose lock nut of steering gear box. - Adjust by turning adjustment screw in or out. | - Describe the adjusting procedure of steering wheel play of wheel loader. Practical activity: - Adjust by turning adjustment screw in or out. | Theory- 0.3 hours Practice- 3 hours Total- 3.3 hours | Tools trolley (Complete set of Hand Tools) Special Service tools wheel loader PP&E Coverall Safety shoes Safety helmet | Class Room and hydraulic workshop |
| - Measure operating force of steering wheel. | Trainee will be able to: - Hook push –pull scale to the knob of steering wheel. - Raise the bucket 400mm and remove safety bar. - Pull push-pull scale tangentially to steering wheel. - Read the value when steering wheel is moving smoothly. | - Describe orbit roll type steering valve. Practical activity: | Theory- 0.3 hours Practice- 3 hours Total- 3.3 hours | Tools trolley (Complete set of Hand Tools) Special Service tools PP&E Coverall Safety shoes Safety helmet | Class Room and hydraulic workshop |
| - Measure operating time of steering wheel | Trainee will be able to - Start the engine and raise bucket about 400mm and remove the safety bar. - Turn steering wheel up till machine is turned fully right or left at low idling. - Turn steering wheel up till machine is turned fully right or left at high idling. - Check steering oil pressure. | - Describe steering system of motor grader. Practical activity: - Check steering oil pressure | Theory- 0.3 hours Practice- 3 hours Total- 3.3 hours | Tools trolley (Complete set of Hand Tools). Special Service tools Hydraulic pressure gauge set PP&E Coverall Safety shoes Safety helmet | Class Room and hydraulic workshop |
| - Measure steering relief pressure | Trainee will be able to - Fit safety bar on the frame. - Remove plug from takeoff port of right turn of steering circuit. - Install pressure gauge. - Start the engine and run it at high idling. - Turn the steering wheel to the right and measure pressure when relief valve is actuated. | - Describe Orbit Roll Type Steering Valve assembly. Practical activity: - Install pressure gauge. | Theory- 0.3 hours Practice- 3 hours Total- 3.3 hours | Tools trolley (Complete set of Hand Tools). Special Service tools Hydraulic pressure gauge set PP&E Coverall Safety shoes Safety helmet | Class Room & hydraulic workshop |
| - Adjust steering relief pressure. | Trainee will be able to - Stop the engine. - Remove cap nut of relief valve. - Loose lock nut. - Turn adjustment screw to adjust. - Tighten screw to increase pressure - Loosen screw to decrease pressure. . | - Describe steering system of motor grader. Practical activity: - Tighten screw to increase pressure | Theory- 0.3 hours Practice- 3 hours Total- 3.3 hours | Tools trolley (Complete set of Hand Tools). Special Service tools Hydraulic pressure gauge set PP&E Coverall Safety shoes Safety helmet | Class Room & hydraulic workshop |

    - Recognize hydro vacuum brake used in motor grader.

Objective: This module covers the skills and knowledge required to main components of hydro master and drum brake of motor grader.

Duration: 16 Hours								Theory: 4 Hours				Practice: 12 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Demonstrate cutaway model of hydro master used in motor grader brake. | Trainee will be able to: - Identify main components of hydro master from cutaway model. - Demonstrate operation of hydro master | - Describe of hydro vacuum brake used in brake system of motor grader. Practical activity: | Theory- 1.3 hours Practice- 4 hours Total- 5.3 hours | cutaway model of hydro master of motor grader PP&E Coverall Safety shoes Safety helmet | Class Room and hydraulic workshop |
| - Identify/draw hydro master brake circuit. | Trainee will be able to: - Identify brake paddle - Identify master cylinder - Identify Hydro Master - Identify Vacuum pump - Identify air inlet port - Identify High pressure brake oil lines to wheels. - Draw hydro master brake circuit. | - Describe of hydro master. Practical activity: - Identify Vacuum pump - Identify Hydro Master | Theory- 1.3 hours Practice- 4 hours Total- 5.3 hours | Motor grader PP&E Coverall Safety shoes Safety helmet | Class Room and hydraulic workshop |
| - Identify drum type wheel brake of motor grader. | Trainee will be able to: - Identify wheel cylinder. - Identify return spring. - Identify brake drum. - Identify brake shoe and lining. - Identify adjuster. Draw hydro master brake circuit. | - Describe drum type brake. Practical activity: - Identify brake drum. - Identify brake shoe and lining. | Theory- 1.3 hours Practice- 4 hours Total- 5.3 hours | . Motor grader PP&E Coverall Safety shoes Safety helmet | Class Room and hydraulic workshop |

    - Analyze air brake system for wheeled vehicles.

Objective: This module covers the skills and knowledge required to main components and circuits of motor grader, wheel loader and dump truck.

Duration: 26 Hours								Theory: 5 Hours				Practice: 21 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Trace the air brake circuit of Motor Grader’s Disc Brake System | Trainee will be able to: - Identify brake valve. - Identify air compressor. - Identify air reservoir. - Identify relay valve. - Identify low pressure switch and stop lamp switch. | - Describe the testing procedure of steering wheel play of wheel loader. Practical activity: - Identify air reservoir. - Identify relay valve. | Theory- 0.8 hours Practice- 3.5 hours Total- 4.3 hours | Motor grader PP&E Coverall Safety shoes Safety helmet | Class Room and hydraulic workshop |
| - Identify components of disc type wheel brake of motor grader. | Trainee will be able to: - Identify discs. - Identify plates. - Identify spring. - Identify piston. - Identify cage. | - Describe the adjusting procedure of steering wheel play of wheel loader. Practical activity: - Identify spring. - Identify piston | Theory- 0.8 hours Practice- 3.5 hours Total- 4.3 hours | Motor grader PP&E Coverall Safety shoes Safety helmet | Class Room and hydraulic workshop |
| - Trace hydraulic type brake circuit of wheel loader. | Trainee will be able to: - Identify hydraulic tank - Identify brake valve - Identify brake and cooling fan pump - Identify slack adjuster for front and rear brake. - Identify front and rear brake. - Identify accumulator fro front and rear brake. - Identify lines. | - Describe orbit roll type steering valve. Practical activity: - Identify slack adjuster for front and rear brake. - Identify front and rear brake | Theory- 0.8 hours Practice- 3.5 hours Total- 4.3 hours | wheel loader. PP&E Coverall Safety shoes Safety helmet | Class Room and hydraulic workshop |
| - Identify components of brake valve of wheel loader. | Trainee will be able to - Identify inching valve spool. - Identify front brake valve spool - Identify rear brake valve spool | - Describe steering system of motor grader. Practical activity: - Identify front brake valve spool | Theory- 0.8 hours Practice- 3.5 hours Total- 4.3 hours | wheel loader PP&E Coverall Safety shoes Safety helmet | Class Room and hydraulic workshop |
| - Trace the Air brake circuit of Dump Truck | Trainee will be able to - Identify the brake pedal. - Identify the brake valve. - Identify the compressor. - Identify the air reservoir. - Identify the relay valve. - Identify the pressure lines from relay valve to hydraulic cylinder. - Identify the brake lines from hydraulic cylinder to wheel. - Identify brake cylinder. | - Describe Orbit Roll Type Steering Valve assembly. Practical activity: - Identify the air reservoir. - Identify the relay valve | Theory- 0.8 hours Practice- 3.5 hours Total- 4.3 hours | Dump Truck PP&E Coverall Safety shoes Safety helmet | Class Room & hydraulic workshop |
| - Test and adjust the Air brake system of Dump Truck | Trainee will be able to - Measure the brake pedal travel - Adjust the brake pedal travel. - Measure the air pressure of air reservoir - Rectify leakage in air and oil lines of air brake system. | - Describe steering system of motor grader. Practical activity: - Adjust the brake pedal travel. - Measure the air pressure of air reservoir . | Theory- 0.8 hours Practice- 3.5 hours Total- 4.3 hours | Dump Truck PP&E Coverall Safety shoes Safety helmet | Class Room & hydraulic workshop |

# 9.17 Hydraulic System of Construction Machinery

- Demonstrate hydraulic system

Objective: This module covers the skills and knowledge required to Identify Hydraulic system components and visually inspects hydraulic system.

Duration: 13 Hours									Theory: 2 Hours			Practice: 9 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Identify Hydraulic system components | Trainee will be able to: - Identify tank, pumps, valves, Test ports for pressure gauges - Identify Linkages, joy stick, actuators (cylinders), sight glasses, control levers, display panels (monitors), filters, coolers, lines, hoses, fittings. - Identify switches, solenoids, relays, sensors, electronic control modules, wiring harnesses | - State Pascal’s law - State pressure and it’s different types (Atmospheric, Gauge, Vacuum, Absolute Pressure) - State Flow rate methods of flow rate and types of flow meter. - Entrained Air, Dissolved Air, Aeration, Cavitation’s, Set/ Rated, Cracking and Surge Pressure. - The full range of hydraulic system components - Characteristics and operational function of each hydraulic system component Practical activity: Draw hydraulic system diagram and label its component/ parts. | Theory- 2hours Practice- 4.5hours Total- 6.5.5hours | Bulldozer Multi meter Training aid /board of Pressure and flow difference. | Class room & labs/Workshop |
| - Inspect Visually hydraulic system | Trainee will be able to: - Check for worn, loose, damage hydraulic pipes and hoses. - Check visually hydraulic components like pump, cylinder and tank. - Check the level of hydraulic oil - Refill the hydraulic oil if necessary | - Procedures for inspecting and testing hydraulic system components - State hydraulic oil - Describe different grades of hydraulic oil - State viscosity of hydraulic oil and viscosity index of oil - State the causes of deterioration of oil and how to prevent them. - Describe difference between Pressure and Flow. Practical activity: Identify the damage and worn parts, hydraulic line, switches, solenoids, relays, modules, check and refill hydraulic line. | Theory- 2hours Practice- 4.5hours Total- 6.5 hours | Bulldozer Multi meter Training aid /board of Pressure and flow difference. | Class room & labs/Workshop |

- Recognize components of Hydraulic system

Objective: This module covers the skills and knowledge required to identify parallel and series system, Identify Open and close center system, Understand Hydraulic Pump, Understand Hydraulic tank and Understand Hydraulic cylinder.

Duration: 13 Hours									Theory: 4 Hours				Practice: 9 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Identify parallel and series system | Trainee will be able to: - Identify the parallel circuit - Identify the series circuit - Draw simple circuit of parallel system - Draw simple circuit of series system | - Define hydraulic system of Bulldozer - Differentiate between parallel and series Hydraulic system. Practical activity: Identify and draw circuit of parallel and series system. | Theory- 1hours Practice- 1.5hours Total- 2.5hours | Tools trolley (Complete set of Hand Tools) Special Service tools. Series and Parallel circuit board. | Class room & labs/Workshop |
| - Identify Open and close center system | Trainee will be able to: - Identify the open centre system from given circuit - Identify the close centre system from given circuit - Draw simple circuit of open centre system - Draw simple circuit of close centre system | - Differentiate between open center and close center Hydraulic system | Theory- 1hours Practice- 1.5hours Total- 2.5hours | Tools trolley (Complete set of Hand Tools) Special Service tools. Series and Parallel circuit board. | Class room & labs/Workshop |
| - Understand Hydraulic Pump | Trainee will be able to: - Identify the main components of hydraulic pump from cutaway model. - Types of gear pump based on pressure range - Disassembling of hydraulic pump - Assembling of hydraulic pump | - Describe main function of hydraulic pump - Types of hydraulic Pump - Working principle of hydraulic pump Practical activity: Identify and draw the main components of hydraulic pump. | Theory- 1hours Practice- 2hours Total- 3.5hours | Tools trolley (Complete set of Hand Tools) Special Service tools. Cutaway model of Hydraulic Pump. | Class room & labs/Workshop |
| - Understand Hydraulic tank | Trainee will be able to: - Identify main relief valve, suction valve, safety valve, check valve, from cutaway model of hydraulic tank. - Identify blade lifting spools. - Identify riper spool - Identify blade tilting spool - Identify filter and strainer - Disassembling of hydraulic tank - Assembling of hydraulic tank | - Describe main function of hydraulic tank - Working principle of hydraulic tank - Define valves - Types of valves Practical activity: Identify and draw the main components of hydraulic tank. | Theory- 0.5hours Practice- 2hours Total- 2.5hours | Tools trolley (Complete set of Hand Tools) Special Service tools. Cutaway model of Hydraulic tank. | Class room & labs/Workshop |
| - Understand Hydraulic cylinder | Trainee will be able to: - Identify piston valve, wear ring, U packing, piston, piston rod and dust seal from inside the cutaway model of hydraulic cylinder. - Disassembling of hydraulic cylinder - Assembling of hydraulic cylinder | - Describe main function of hydraulic cylinder - Working principle of hydraulic cylinder - Types of hydraulic cylinder Practical activity: Identify and draw the main components of hydraulic cylinder. | Theory- 0.5hours Practice- 2hours Total- 2.5hours | Tools trolley (Complete set of Hand Tools) Special Service tools. Cutaway model of Hydraulic Cylinder. | Class room & labs/Workshop |

- Draw hydraulic circuits (schematics)

Objective: This module covers the skills and knowledge required to identify symbols used in hydraulic circuit /schematics, Identify the raising circuit of blade, Identify the lower circuit of blade, Identify the hold circuit of blade and Identify the float circuit of blade

Duration: 23 Hours									Theory: 5 Hours			Practice: 18 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Identify symbols used in hydraulic circuit /schematics | Trainee will be able to: - Identify the hydraulic symbols. - Draw different hydraulic symbols. | - Describe hydraulic circuit of different machines. - Define function and uses of hydraulic symbols - Define schematic diagram Practical activity: Draw hydraulic circuit with appropriate symbols (Schematic diagram) | Theory- 1hours Practice- 4hours Total- 5hours | Shop manual Circuit board | Class room & Workshop |
| - Identify the raising circuit of blade | Trainee will be able to: - Identify the raising circuit of blade of bulldozer - Draw the raising circuit of blade of bulldozer | - Describe the raising circuit of blade of bull dozer - Define blade - Types of blade - Explain function of blade | Theory- 1hours Practice- 5hours Total- 6hours | Shop manual Circuit board | Class room & Workshop |
| - Identify the lower circuit of blade | Trainee will be able to: - Identify the lower circuit of blade of bulldozer. - Draw the lower circuit of blade of bulldozer | - Describe the lower circuit of blade of bull dozer. | Theory- 1hours Practice- 3hours Total- 4hours | Shop manual Circuit board | Class room & Workshop |
| - Identify the hold circuit of blade | Trainee will be able to: - Identify the hold circuit of blade of bulldozer - Draw the hold circuit of blade of bulldozer. | - Describe the hold circuit of blade of bull dozer. | Theory- 1hours Practice- 3hours Total- 4hours | Shop manual Circuit board | Class room & Workshop |
| - Identify the float circuit of blade | Trainee will be able to: - Identify the float circuit of blade of bulldozer. - Draw the float circuit of blade of bulldozer. | - Describe the float circuit of blade of bull dozer. | Theory- 1hours Practice- 3hours Total- 4hours | Shop manual Circuit board | Class room & Workshop |

- Maintain Hydraulic Pump

Objective: This module covers the skills and knowledge required to identify parallel and series system, Identify Open and close center system, Understand Hydraulic Pump, Understand Hydraulic tank and Understand Hydraulic cylinder.

Duration: 25 Hours									Theory: 4 Hours			Practice: 21 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Identify Hydraulic pump | Trainee will be able to: - Identify the different types of hydraulic pump - Identify the different components of hydraulic pump from cutaway model | - State principle of hydraulic pump. - Explain nomenclature of hydraulic pump. - Enlist different types of hydraulic pump | Theory- 1.5hours Practice- 7hours Total- 8.5hours | Tools trolley (Complete set of Hand Tools) Pressure gauge set Hydraulic pump cutaway models | Class room & Workshop |
| - Inspect hydraulic pump | Trainee will be able to: - Inspect the performance of hydraulic pump - Inspect different components of hydraulic pump - Check the set pressure and compare with standard value - Measure the flow rate and compare with standard value | - Define relief recess, floating bush, side bush, radial clearances and pressure balancing groove. - Explain effects of discharge pressure, running clearances and viscosity/oil temperature on the pump capacity with the help of graphs | Theory- 1.5hours Practice- 7hours Total- 8.5hours | Tools trolley (Complete set of Hand Tools) Pressure gauge set Hydraulic pump cutaway models | Class room & Workshop |
| - Replace hydraulic pump | Trainee will be able to: - Remove hydraulic pipes of pump - Remove hydraulic pump from bulldozer - Install hydraulic pipes of pump - Install hydraulic pump on bulldozer | - Describe deterioration of pump due to dirty and foamy oil and cavitation due to aeration. | Theory- 1hours Practice- 7hours Total- 8hours | Tools trolley (Complete set of Hand Tools) Pressure gauge set Hydraulic pump cutaway models | Class room & Workshop |

- Recognize HYDRAULIC TANK

Objective: This module covers the skills and knowledge required to Identify Hydraulic tank and Inspect hydraulic tank.

Duration: 12 Hours									Theory: 3 Hours				Practice: 9 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Identify Hydraulic tank | Trainee will be able to: - Identify the open and close type of hydraulic tank - Identify the different components of hydraulic tank from cutaway model. - Identify different spools present in hydraulic tank from cutaway model - Identify different valves present in hydraulic tank pressure control (relief valve, safety valve), direction control (check valve and blade lift spool) and flow control (suction valve) from cutaway model - Identify different position of blade lift spool like raise, lower, hold and float positions | - State hydraulic tank - State types of hydraulic tank - State different components of hydraulic tank - Describe function of pressure control valve, direction control valve and flow control valve. | Theory- 1.5hours Practice- 4hours Total- 5.5hours | Tools trolley (Complete set of Hand Tools) Special Service tools Pressure gauge set Hydraulic tank cutaway model | Class room & Workshop |
| - Inspect hydraulic tank | Trainee will be able to: - Dissembling of hydraulic tank - Inspect the strainer. - Inspect the hydraulic filter - Inspect the control valve assembly present in tank - Inspect the spools used in control valve assembly - Inspect the main relief valve - Assembling of hydraulic tank | - State location, structure and function of main relief valve, safety valve, check valve and suction control valve - State different type of main relief valve poppet type, spool type and pilot operated type. - State different position of spools raise, lower, hold and float position - State flow control valve | Theory- 1.5hours Practice- 5hours Total- 6.5hours | Tools trolley (Complete set of Hand Tools) Special Service tools Pressure gauge set Hydraulic tank cutaway model | Class room & Workshop |

- Demonstrate components of Hydraulic Cylinder

Objective: This module covers the skills and knowledge required to Identify Hydraulic cylinder, inspect hydraulic cylinder and Overhauling of hydraulic cylinder.

Duration: 13 Hours									Theory: 4 Hours				Practice: 9 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Identify Hydraulic cylinder | - Identify the different types of hydraulic cylinder - Identify the different components of hydraulic cylinder from cutaway model. - Identify quick drop valve on machine. | - Define hydraulic cylinder - Enlist types of hydraulic cylinder - Enlist different components of hydraulic cylinder | Theory- 1.5hours Practice- 3hours Total- 4.5hours | Tools trolley (Complete set of Hand Tools) Special Service tools Pressure gauge set | Class room & Workshop |
| - Inspect hydraulic cylinder | - Dissembling of hydraulic cylinder - Inspect different components of hydraulic cylinder - Assembling of hydraulic cylinder - Inspect the hoses attached with hydraulic cylinder - Replace different components of hydraulic cylinder as per requirement. - Perform hydraulic drift test of work equipment of bulldozer | - Describe function and structure of each component of hydraulic cylinder piston ring, wear ring, U-packing, V-packing and dust seal. - Describe location, structure and function of quick drop valve used in hydraulic cylinder. - Describe the importance and procedure of hydraulic drift test. Also compare it with standard value in shop manual | Theory- 1.5hours Practice- 3hours Total- 4.5hours | Tools trolley (Complete set of Hand Tools) Special Service tools Pressure gauge set | Class room & Workshop |
| - Overhaul hydraulic cylinder | - Replace the dust seal of hydraulic cylinder - Replace the U-packing of hydraulic cylinder - Replace the V-packing of hydraulic cylinder - Replace the piston rings of hydraulic cylinder - Replace the wear rings of hydraulic cylinder - Assembling of hydraulic cylinder. | - Define Seal of hydraulic system - Types of packing - Define function of piston ring - Function of wear ring - Procedure for overhauling | Theory- 1hours Practice- 3hours Total- 4hours | Tools trolley (Complete set of Hand Tools) Special Service tools Pressure gauge set | Class room & Workshop |

- Maintain HSS (Hydro Static Steering) Pump and Motor

Objective: This module covers the skills and knowledge required to identify HSS (Hydro Static Steering) Pump and HSS motor, Identify CLSS (Close Load Sensing System) System and OLSS (Open Load Sensing System) System and Check HSS system.

Duration: 13 Hours									Theory: 4 Hours				Practice: 9 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Identify HSS (HYDRO STATIC STEERING) Pump and HSS motor | - Identify components of HSS system - Label the work equipment hydraulic system layout diagram - Identify work equipment controls levers - Identify scavenging pump. - Identify the HSS Pump - Identify LS, PC and PC-EPC Valves - Label the LS valve - Label the PC valve - Label the PC-EPC valve - Identify the flow of oil through diagram - Identify the HSS Motor - Identify the hydraulic circuit of HSS motor on machine - Identify the power train of HSS system on machine - Identify variable throttle valve | - Describe the function and structure of HSS Pump - Label the sectional view diagram oh HSS Pump - Describe scavenging Pump - Describe the control of delivery of HSS pump by swash plate angle - Describe the structure, operation and function of PC-EPC valve. - Describe the structure, operation and function of TVC valve. - Describe the structure, operation and function of Variable throttle valve. - Explain circuit for increasing delivery of pump also draw it circuit diagram. - Explain circuit for decreasing delivery of pump also draw it circuit diagram. - Describe the function and structure of HSS Motor - Describe the brake valve with counter balance valve, check valve and safety valve | Theory- 1.5hours Practice- 3hours Total- 4.5hours | Tools trolley (Complete set of Hand Tools) Special Service tools Pressure gauge set Bulldozer | Class room & Workshop |
| - Identify CLSS (Close Load Sensing System) System and OLSS (Open Load Sensing System) System | - Identify the CLSS system through diagram - Identify the circuit of CLSS to control Swash plate angle of HSS pump through diagram - Identify the OLSS system through diagram - Identify the circuit of OLSS to control Swash plate angle of HSS pump through diagram - Checking of HSS pressure by gauge - Checking of PPC outlet pressure by gauge | - Describe CLSS System - Describe OLSS - Describe structure, operation and function of main relief valve of steering and work equipment | Theory- 1.5hours Practice- 3hours Total- 4.5hours | Tools trolley (Complete set of Hand Tools) Special Service tools Pressure gauge set Bulldozer | Class room & Workshop |
| - Test HSS pump | - Check HSS pressure by gauge - Check PPC outlet pressure by gauge - Check direction of rotation of radiator fan from monitor screen - Read error codes displayed on monitor | - Describe structure, operation and function of steering priority valve - Describe structure, operation and function of suction safety valve | Theory- 1hours Practice- 3hours Total- 4.5hours | Tools trolley (Complete set of Hand Tools) Special Service tools Pressure gauge set Bulldozer | Class room & Workshop |

- Troubleshoot of hydraulic system

Objective: This module covers the skills and knowledge required to identify Diagnose work equipment of hydraulic system, inspect blade lift slow speed, inspect ripper lift slow speed, and inspect excessive hydraulic drift of blade lift, Diagnose excessive hydraulic drift of ripper lift.

Duration: 16 Hours									Theory: 4 Hours			Practice: 12 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Diagnose work equipment of hydraulic system | - Inspect/refill the oil level from hydraulic tank as per requirement - Inspect work equipment and HSS pump as per requirement - Inspect operation of unload valve as per requirement - Inspect/adjust set pressure or main relief valve pressure as per requirement - Inspect /adjust PPC valve as per requirement | - Describe the work equipment of the machine. - Explain the pilot pressure to operate the spools also draw circuit diagram. - Explain the structure, function and operation of PPC valve. - Explain how to read shop manual. - Explain how to read monitor screen on machine. Explain how to read codes on monitor screen | Theory- 1.5hours Practice- 3hours Total- 4.5hours | Tools trolley (Complete set of Hand Tools) Special Service tools Pressure gauge set | Class room & Workshop |
| - Inspect blade lift slow speed | - Inspect/refill the level of the hydraulic oil as per requirement - Inspect the operation of PPC valve as per requirement - Inspect operation of blade lift control valve as per requirement - Bleed air from blade lift cylinder as per requirement | - Explain different circuits of blades and ripper also draw their circuits | Theory- 1.5hours Practice- 2hours Total- 3.5 hours | Tools trolley (Complete set of Hand Tools) Special Service tools Pressure gauge set | Class room & Workshop |
| - Inspect ripper lift slow speed | - Inspect /refill the level of hydraulic oil from tank as per requirement - Inspect operation of PPC valve for ripper as per standards - Inspect operation of ripper lift control valve as per standards - Bleed air from ripper lift cylinder as per requirement | - Explain different circuits of blades and ripper also draw their circuits | Theory- 1hours Practice- 2hours Total- 3hours | Tools trolley (Complete set of Hand Tools) Special Service tools Pressure gauge set | Class room & Workshop |
| - Inspect excessive hydraulic drift of blade lift | - Inspect/repair the oil leakages in the hydraulic circuit as per requirement. - Inspect/replace the seal of blade lift control valve(spool) as per requirement - Inspect dust seal of blade lift cylinder as per requirement |  | Theory- 0hours Practice- 2.5hours Total- 2.5hours | Tools trolley (Complete set of Hand Tools) Special Service tools Pressure gauge set | Class room & Workshop |
| - Diagnose excessive hydraulic drift of ripper lift | - Inspect/repair the oil leakages in the hydraulic circuit as per requirement - Inspect/replace the seal of the ripper lift control valve as per requirement - Inspect dust seal of the ripper lift cylinder as per requirement |  | Theory- 0hours Practice- 2.5hours Total- 2.5hours | Tools trolley (Complete set of Hand Tools) Special Service tools Pressure gauge set | Class room & Workshop |

## 9.18 Planning & management of construction machines

### Prepare Critical Path Method Diagram

Objective: This module covers the skills and knowledge required to to prepare the critical path method diagram for management &  smooth flow of activities in construction work

Duration: 16 Hours								Theory: 4 Hours				Practice: 12 Hours

| Learning Unit | Learning Outcomes | Learning elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Identify the Need of planning & management | Trainee will be able to: - Enlist planning & management Principles - Mention planning & management Requirements - Identify the resources and data required for planning & management - Enlist the principles of planning & management | - Explanation of planning - State purpose of management - Description of types of planning & management - Application of principles of planning & management Practical activity: Identify principles of planning & management | Theory0.66Hrs Practice-2 Hrs Total- 2.66 Hrs | Paper Pen Pencil Eraser Sharpner | Class Room |
| - Specify Each Activity | Trainee will be able to: - Check the whole process. - Figure Out different activities. - Categorize the activities. - Enlist the activities. - Specify each activity. | - Explanation of activity - Description of types of activities Practical activity: - Identify activities for preparing Critical Path Method | Theory0.66Hrs Practice-2 Hrs Total- 2.66 Hrs | Paper Pen Pencil Eraser Sharpner Paper T & E: | Class Room |
| - Establish Dependencies (Activity Sequence) | Trainee will be able to: - Point out the independent activities. - Point out the dependent activities. - Establish the relationship between activities. - Arrange the activities in sequence. | - Description of dependent & independent activities - Importance of sequencing in Critical Path Method Practical activity: - Arrange activities of Critical Path Method in sequence | Theory0.66Hrs Practice-2 Hrs Total- 2.66 Hrs | Paper Pen Pencil Eraser Sharpner Paper T & E: | Class Room |
| - Estimate Activity Completion Time | Trainee will be able to - Identify the nature of project. - Apply three-point formula. - Interpret the data. - Estimate activity completion time. - Allot time as per nature of project. | - Explanation of technical term relevant to time. Practical activity: - Estimate time of activities | Theory0.66Hrs Practice-2 Hrs Total- 2.66 Hrs | Paper Pen Pencil Eraser Sharpner Paper | Class Room |
| - Draw the Network Diagram | Trainee will be able to - Enlist all activities in project. - Draft the network roughly. - Estimate time for each activity. - Write estimated time in each activity. | - State the purpose of network diagrams - Techniques for preparing network diagram. Practical activity: - Prepare network diagram | Theory0.66Hrs Practice-2 Hrs Total- 2.66 Hrs | Paper Pen Pencil Eraser Sharpner Paper | Class Room |
| - Identify the Critical Path | Trainee will be able to - Review the network diagram. - Calculate the earliest start and earliest finish time. - Calculate the late start and late finish time. - Trace the critical path. | - Purpose of CPM - Steps for preparation of CPM Practical activity: - Calculate the time for critical path. | Theory0.66Hrs Practice-2 Hrs Total- 2.66 Hrs | Paper Pen Pencil Eraser Sharpner Paper T & E: Calculator | Class Room |

### Calculate depreciation of construction machinery

Objective: This module covers the skills and knowledge required to calculate the depriciation of a machine due to age,wear, deterioration, and obsolescence.

Duration: 16 Hours						Theory: 4 Hours				Practice: 12 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Identify the need of depreciation. | Trainee will be able to: - Assess the importance of depreciation. - Mention the requirements for depreciation. - Identify the reasons for depreciation. - Identify the resources and data required for depreciation. | - Need for calculation of depreciation - State requirements for depreciation Practical Activity: - Identify need of depreciation | Theory-1 Hrs Practice-3 Hrs Total- 4 Hrs | Paper Pen Pencil Eraser Sharpner Paper | Class Room |
| - Estimates of the items. | Trainee will be able to: - Identify the items to be estimated. - Enlist the items to be estimated. - Estimate The purchase price of the piece of equipment (termed P). - Estimate the ‘Recovery’ period (termed N). - Estimate the salvage value (termed F). | - Purpose of estimate - State items to be estimated Practical Activity: - Estimate the items for depreciation | Theory-1 Hrs Practice-3 Hrs Total- 4 Hrs | Paper Pen Pencil Eraser Sharpner Paper T & E: Calculator | Class Room |
| - Select suitable depreciation method. | Trainee will be able to: - Identify the methods of depreciation. - Enlist the methods of depreciation. - Evaluate the estimated data. - Select the suitable method as per data. | - State the methods for calculation of depreciation - Selection of suitable method for depreciation Practical Activity: - Perform moisture content test for given sample timber | Theory-1Hrs Practice-3Hrs Total- 3Hrs | Paper Pen Pencil Eraser Sharpner Paper T & E: Calculator | Class Room |
| - Compute the depreciation. | - Trainee will be able to - Compile all the data. - Interpret the data. - Compute the depreciation by selected formula - Review the whole process. | - Interpretation of data - Practical Activity: - Calculate depreciation of construction machine | Theory-1 Hrs Practice-3 Hrs Total- 4 Hrs | Paper Pen Pencil Eraser Sharpner Paper T & E: Calculator | Class Room |

### Calculate the cost of owning for Construction machinery

Objective: This module covers the knowledge and  skills required to calculate the cost of ownership and allied fixed cost  for various construction machinery.

Duration: 15 Hours								Theory: 3 Hours				Practice: 12 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Identify the need of cost of owning. | Trainee will be able to: - Analyze the different factors affecting owning cost. - Determine the nature of cost - Enlist the components of owning cost. - Identify the need of owning cost. | - Description of owning cost - State factors affecting owning cost - Practical Activity: - Identify the purpose of owning cost | Theory-0.6 Hrs Practice-2.4 Hrs Total- 3 Hrs | Paper Pen Pencil Eraser Sharpner | Class Room |
| - Calculate the initial cost. | Trainee will be able to: - Study the importance of initial cost. - Point out the factors that affect initial cost. - Gather the data about initial cost. - Arrange the data. - Compute the data. - Calculate initial cost | - State the purpose of initial cost - State factors affecting owning cost - Practical Activity: - Determine initial cost | Theory-0.6 Hrs Practice-2.4 Hrs Total- 3 Hrs | Paper Pen Pencil Eraser Sharpner T & E: Calculator | Class Room |
| - Calculate the depreciation of machines. | Trainee will be able to: - Identify the need of depriciation. - Estimates of the items. - Select suitable depriciation method. - Compute the depriciation. | - Need for calculation of depreciation - Requirements for calculating depreciation of machines - Practical Activity: - Determine depreciation of construction machines | Theory-0.6 Hrs Practice-2.4 Hrs Total- 3 Hrs | Paper Pen Pencil Eraser Sharpner T & E: Calculator | Class Room |
| - Determine INVESTMENT (OR INTEREST) COST. | Trainee will be able to: - Study the importance of investment cost. - Point out the factors that affect investment cost. - Gather the data about investment cost. - Arrange the data. - Compute the data. - Determine the investment cost. | - State the purpose of investment cost - State factors affecting investment cost - Practical Activity: - Determine investment cost | Theory-0.6 Hrs Practice-2.4 Hrs Total- 3 Hrs | Paper Pen Pencil Eraser Sharpner T & E: Calculator | Class Room |
| - Compute the total ownership cost. | Trainee will be able to: - Review the overall process. - Compute the total ownership cost. - Check for errors if any. | - Procedure for calculation of ownership cost - Practical Activity: - Calculate ownership cost | Theory-0.6 Hrs Practice-2.4 Hrs Total- 3 Hrs | Paper Pen Pencil Eraser Sharpner T & E: Calculator | Class Room |

### Calculate the operating cost for construction machinery

Objective: This module covers the knowledge and  skills required to calculate the operating cost of construction machineery considering different aspects of cost.

Duration: 15 Hours							Theory: 3 Hours				Practice: 12 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Identify the need of operating cost. | Trainee will be able to: - Analyze the different factors affecting operating cost. - Determine the nature of cost - Enlist the components of operating cost. - Identify the need of operating cost | - Description of operating cost - State factors affecting operating cost Practical activity: - Identify the purpose of operating cost | Theory-0.25 Hrs Practice-1 Hrs Total- 1.25 Hrs | Paper Pen Pencil Eraser Sharpner | Class Room |
| - Determine repair and maintenance Cost of machines. | Trainee will be able to: - Inspect machines for repair and maintenance. - Enlist the nature of faults. - Calculate the hourly repair and maintenance cost. - Determine annual repair and maintenance cost. | - State the purpose of repair & maintenance cost - State factors affecting repair & maintenance cost Practical activity: - Determine pair & maintenance cost | Theory-0.25 Hrs Practice-1 Hrs Total- 1.25 Hrs | Paper Pen Pencil Eraser Sharpner T & E: Calculator | Class Room |
| - Determine tire cost. | Trainee will be able to: - Identify the need of tire cost. - Analyze the factors affecting tire cost. - Inspect the machines. - Determine tire cost. | - State the purpose of tire cost - State factors affecting tire cost Practical activity: - Determine tire cost | Theory-0.25 Hrs Practice-1 Hrs Total- 1.25 Hrs | Paper Pen Pencil Eraser Sharpner T & E: Calculator | Class Room |
| - Verify the consumable materials cost. | Trainee will be able to - Enlist the consumable materials. - Identify the need of consumable materials. - Analyze the market rate. | - State the purpose of consumable material cost - State factors affecting consumable material cost Practical activity: - Determine consumable material cost | Theory-0.25 Hrs Practice-1 Hrs Total- 1.25 Hrs | Paper Pen Pencil Eraser Sharpner T & E: Calculator | Class Room |
| - Calculate fuel cost. | Trainee will be able to - Inspect the engine of machine. - Identify the type of engine. - Observe the fuel consumption for different engines. - Consider factors for fuel consumption | - State the purpose of fuel cost - State factors affecting fuel cost Practical activity: - Determine fuel cost | Theory-0.25 Hrs Practice-1 Hrs Total- 1.25 Hrs | Paper Pen Pencil Eraser Sharpner T & E: Calculator | Class Room |
| - Assess lubricating oil cost. | Trainee will be able to - Inspect the engine of machine. - Identify the type of engine. - Observe the lubricating oil consumption for different engines. - Consider factors for lubricating oil consumption | - State the purpose of lubricating oil cost - State factors affecting lubricating oil cost Practical activity: - Determine lubricating oil cost | Theory-0.25 Hrs Practice-1 Hrs Total- 1.25 Hrs | Paper Pen Pencil Eraser Sharpner T & E: Calculator | Class Room |
| - Assess mobilization and demobilization cost. | Trainee will be able to - Identify the need of mobilization and demobilization. - Observe the factors affecting mobilization cost. - Enlist the components of mobilization cost. | - State the purpose of mobilization and demobilization cost - State factors affecting mobilization and demobilization cost Practical activity: - Determine mobilization and demobilization. | Theory-0.25 Hrs Practice-1 Hrs Total- 1.25 Hrs | Paper Pen Pencil Eraser Sharpner T & E: Calculator | Class Room |
| - Determine equipment operator cost. | Trainee will be able to: - Identify the number of operators. - Identify the types of machines. - Enlist the machines. - Determine equipment operator cost. | - State the purpose of equipment operator cost. - State factors affecting equipment operator cost - Practical activity: - Determine equipment operator cost. | Theory-0.25 Hrs Practice-1 Hrs Total- 1.25 Hrs | Paper Pen Pencil Eraser Sharpner T & E: Calculator |  |
| - Evaluate special item cost. | Trainee will be able to: - Identify the special items. - Enlist the special items. - Assess the need for special items | - State the purpose of special item cost. - State factors affecting special item cost - Practical activity: - Determine special item cost | Theory-0.25 Hrs Practice-1 Hrs Total- 1.25 Hrs | Paper Pen Pencil Eraser Sharpner T & E: Calculator |  |
| Select suitable method for finding operating cost. | Trainee will be able to: Identify methods for finding operating cost. Enlist the methods for finding operating cost. Analyze the data. Select the suitable method as per data. | Enlist the methods for finding operating cost. Analyze the data Practical activity: Select suitable method for operating cost as per data | Theory-0.25 Hrs Practice-1 Hrs Total- 1.25 Hrs | Paper Pen Pencil Eraser Sharpner T & E: Calculator |  |
| - Calculate operating cost. | Trainee will be able to: - Review the overall process. - Gather all the costs. - Arrange the data. - Calculate the total operating cost. - Check for errors if any. | - Interpretation of data - Practical activity: - Determine operating cost | Theory-0.25 Hrs Practice-1 Hrs Total- 1.25 Hrs | Paper Pen Pencil Eraser Sharpner T & E: Calculator |  |
| - Compare the cost calculated by different methods. | Trainee will be able to: - Enlist the methods for calculating operating cost. - Calculate operating cost by different methods. - Compare the cost calculated by different methods | - Determine operating cost by different methods - Practical activity: - Compare operating cost obtained by different methods | Theory-0.25 Hrs Practice-1 Hrs Total- 1.25 Hrs | Paper Pen Pencil Eraser Sharpner T & E: Calculator |  |

### Determine the productivity of construction machinery

Objective: This module covers the knowledge and  skills to required determine the productivity of various construction machinery.

Duration: 	17 Hours							Theory: 5 Hours				Practice: 12 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Conduct equipment productivity analysis. | Trainee will be able to: - Identify the need for productivity analysis. - Plan the productivity analysis session. - Inform the relevant persons. - Arrange the productivity analysis session. - Conduct equipment productivity analysis. - Record the information. | - Explanation of productivity of machines - Need of productivity analysis Practical activity: - Conduct productivity analysis for construction machines | Theory-0.71 Hrs Practice-1.71Hrs Total- 2.42 Hrs | Paper Pen Pencil Eraser Sharpner | Class Room |
| - Identify the productivity methods. | Trainee will be able to: - Inspect the machines. - Enlist the machines. - Analyze the specifications of machines. - Identify the productivity methods. - Select the appropriate method for calculation of productivity. | - Explanation of the techniques for productivity of machines - Inspection of construction machines for productivity Practical activity: - Select suitable method for productivity of construction machines | Theory-0.71 Hrs Practice-1.71Hrs Total- 2.42 Hrs | Paper Pen Pencil Eraser Sharpner | Class Room |
| - Determine productivity of dozer. | Trainee will be able to: - Consider the historical data. - Assess the nature of project. - Select the type of dozer. - Enlist the factors for productivity. - Apply the formula. - Compute productivity of dozer | - Factors affecting on productivity of dozer - Interpretation of data Practical activity: - Determine productivity of dozer | Theory-0.71 Hrs Practice-1.71Hrs Total- 2.42 Hrs | Paper Pen Pencil Eraser Sharpner T & E: Calculator | Class Room |
| - Determine productivity of motor grader. | Trainee will be able to - Assess the nature of project. - Assess the calculation bases of motor grader. - Enlist the factors for productivity. - Apply the formula. - Compute productivity of motor grader. | - Factors affecting on productivity of motor grader - Interpretation of data Practical activity: - Determine productivity of motor grader | Theory-0.71 Hrs Practice-1.71Hrs Total- 2.42 Hrs | Paper Pen Pencil Eraser Sharpner T & E: Calculator | Class Room |
| - Determine productivity of Wheel Loader. | Trainee will be able to - Assess the nature of project. - Enlist the factors for productivity. - Apply the formula. - Compute productivity of Wheel Loader | - Factors affecting on productivity of wheel loader - Interpretation of data Practical activity: - Determine productivity of wheel loader | Theory-0.71 Hrs Practice-1.71Hrs Total- 2.42 Hrs | Paper Pen Pencil Eraser Sharpner T & E: Calculator | Class Room |
| - Determine productivity of Road Roller/Compactor. | Trainee will be able to - Consider the historical data. - Assess the nature of project. - Select the type of road roller/compactor. - Enlist the factors for productivity. - Apply the formula. - Compute productivity of road roller/ compactor | - Factors affecting on productivity of road roller/compactor - Interpretation of data Practical activity: - Determine productivity of road roller/compactor | Theory-0.71 Hrs Practice-1.71Hrs Total- 2.42 Hrs | Paper Pen Pencil Eraser Sharpner T & E: Calculator | Class Room |
| - Determine productivity of excavator. | Trainee will be able to - Assess the nature of project. - Select the type of excavator. - Enlist the factors for productivity. - Apply the formula. - Compute productivity of excavator | - Factors affecting on productivity of excavator - Interpretation of data Practical activity: - Determine productivity of excavator | Theory-0.71 Hrs Practice-1.71Hrs Total- 2.42 Hrs | Paper Pen Pencil Eraser Sharpner T & E: Calculator | Class Room |

### Calculate the effect of grade on tractive effort of vehicles

Objective: This module covers the knowledge and  skills required to calculate the effect of grade and assess the factors affecting tractive effort of vehicles

Duration: 13 Hours							Theory: 4 Hours				Practice: 9 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Identify the aspects for analysis of tractive force. | Trainee will be able to: - Observe the grade of road. - Inspect the type of pavement - Investigate the geographical features. - Monitor weather conditions. - Assess the type of machine - Observe traffic intensity. - Assess the power of machine. | - Explanation of tractive force - Factors affecting tractive force Practical Activity: | Theory-0.66 Hrs Practice-1.5 Hrs Total- 2.16 Hrs | Paper Pen Pencil Eraser Sharpner T & E: Calculator | Class Room |
| - Evaluate the rolling resistance of machines. | Trainee will be able to: - Identify the type of machinery. - Identify the type of pavement. - Assess the condition of road surface. - Compile all the data. - Evaluate the rolling resistance of machines | - State the causes of rolling resistance - Factors affecting rolling resistance - Practical Activity: - Determine rolling resistance of machines | Theory-0.66 Hrs Practice-1.5 Hrs Total- 2.16 Hrs | Paper Pen Pencil Eraser Sharpner T & E: Calculator | Class Room |
| - Evaluate the impact of temperature on performance of IC engines. | Trainee will be able to: - Observe the working principle of IC engine. - Identify the variation in power of machines due to temperature. - Determine the performance of engine in labs. - Prepare performance report of IC engine. | - Analyze the impact of temperature on IC engines - Techniques for determining performance of IC Engine Practical activity: - Prepare performance report of IC engine for temperature | Theory-0.66 Hrs Practice-1.5 Hrs Total- 2.16 Hrs | Paper Pen Pencil Eraser Sharpner T & E: Calculator | Class Room |
| - Evaluate the impact of altitude on performance of IC engines. | Trainee will be able to: - Observe the change in air pressure with altitude. - Analyze the impact of air pressure. - Compare with sea level pressure. - Assess the impact on IC engine performance | - Analyze the impact of altitude on IC engines - Techniques for determining performance of IC Engine Practical Activity: - Prepare performance report of IC engine for altitude | Theory-0.66 Hrs Practice-1.5 Hrs Total- 2.16 Hrs | Paper Pen Pencil Eraser Sharpner T & E: Calculator | Class Room |
| - Evaluate the combine impact of temperature and altitude on IC engine performance. | Trainee will be able to: - Observe standard conditions for testing. - Identify the instrument required for performance evaluation. - Conduct the test as per specifications. - Gather all the data. - Apply formula. - Compute performance of engine. - Assess the overall impact on performance. | - Overall impact of temperature & altitude on IC engines - Techniques for assessing performance of IC engine - Practical Activity: - Prepare performance report of IC engine | Theory-0.66 Hrs Practice-1.5 Hrs Total- 2.16 Hrs | Paper Pen Pencil Eraser Sharpner T & E: Calculator | Class Room |
| - Calculate the effect of grade on tractive effort. | Trainee will be able to: - Identify the type of pavement. - Check the grade of road. - Observe the performance of machine moving upward. - Observe the performance of machine moving downward. - Gather all the data. - Apply formula. - Calculate effect of grade on tractive effort. | - Collection of data for tractive force - Interpretation of data Practical activity: - Determine tractive effort for construction machines | Theory-0.66 Hrs Practice-1.5 Hrs Total- 2.16 Hrs | Paper Pen Pencil Eraser Sharpner T & E: Calculator | Class Room |

## 9.19 Soft Skill

- Develop Professionalism

Objective: This module covers the skills and knowledge required to Create a personal vision / mission, manage your attitude, Practice self-discipline, manage time, manage your professional development and Participate in trainings and performance review

Duration: 16 Hours									Theory: 4 Hours			Practice: 12 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Create a personal vision / mission | - Clarify / prioritize self-values and consider the value of others. - Clarify expectations of yourself and expectations others have of you. - Identify what you need to do to be successful (personal standards, targets, goals, principals) - Set specific short- and long-term goals. - Translate the vision into actionable steps. - Integrate the vision into daily practice. - Recount frequently with your vision and change accordingly. | - Explain long and short-term goals. - Importance of personal vision and mission - Describe the advantages of personal vision and mission. - Explain the importance of personal and professional motivation | Theory-01 Hrs Practice-02 Hrs Total-03 Hrs |  | Class room |
| - Manage your attitude | - Challenge yourself, break old habits, and move out of your comfort zone. - Practice innovative techniques for out of the box creative thinking. - Seek out support and feedback from others on the team, in the organization / community etc. - Identify daily, weekly accomplishments. - Read inspirational material, audiotapes. | - Identify your positive attitude. - Explain the advantages of innovative ideas and techniques during job. - Explain self-planning and management techniques | Theory-0.5 Hrs Practice-02Hrs Total-2.5 Hrs |  | Class room |
| - Practice self-discipline | - Accountable for your performance. - Identify what you need to do to be successful. - Communicate your priorities to others. - Make and honor appointments with yourself and others. - Practice relaxation and energizing techniques. | - Define goals and strategies of self- development. - Explain the importance of communication. - Explain the advantages of self-discipline. | Theory-0.5 Hrs Practice-02 Hrs Total-2.5 Hrs |  | Class room |
| - Manage time | - Isolate key success activities and prioritize them. - Breakdown large tasks down into manageable action steps (set time frame). - Create or adopt action plans and follow it. - Set aside appropriate blocks of time for goal related activities. - Make the best possible use of support people / recourses to accomplish tasks. | - Explain the importance of time management to achieve different tasks. | Theory-01 Hrs Practice-02 Hrs Total-3 Hrs |  | Class room |
| - Manage your professional development | - Take inventory of your personal interests, abilities, skills, knowledge etc. - Identify and prioritize the strengths and gaps. - Use available assessment tools. - Create a personal growth strategy / career path. - Set personal goals and timeframe for achieving them. - Learn from your mistakes. | - Define concept about performance standards - Explain the importance and need of professional development | Theory-0.5 Hrs Practice-02 Hrs Total-2.5 Hrs |  | Class room |
| - Participate in trainings and performance review | - Analyze, evaluate and improve performance, and report significant issues/problems to senior management - Demonstrate to-do attitude in profession - Demonstrate understanding of skills requirements - Use the competences acquired in trainings | - Explain relevant knowledge about training / job requirements | Theory-0.5 Hrs Practice-02 Hrs Total- 2.5 Hrs |  | Class room |

## National Certificate Level-5 Part-2 in Construction Machinery

## 9.20 Power Transmission of Construction Machinery

- Demonstrate main clutches

Objective: This module covers the skills and knowledge required to Diagnose and repair clutches and transmissions by: performing a visual inspection; diagnosing and troubleshooting, repairing and verifying the repair of clutch systems and components; manual transmissions/transfer cases and components; torque converters, lock-up clutches and components; power shift transmissions and components; power shift transmission electrical and electronic controls and components; hydrostatic transmissions and components

Duration: 24 Hours								Theory: 06 Hours				Practice: 18 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Recognize main Clutch and its components. | Trainee will be able to: - Identify systems type and applications - Point out Housings - locate Clutches - Point out Discs - Point out Pressure plates - Locate Flywheels - Point out Shaft - Point out Bearings | - Classify different types of Clutches. - Describe main clutches. Practical activity - Identify main clutch components. | Theory- 1.5 hours Practice- 4.5 hours Total- 6 hours | Tool trolley (all Tools) Clutch Special service tool | Class Room Workshop |
| - Inspect main Clutch and its components. | Trainee will be able to: - Disassembly of main Clutch. - Analyze Housings - Inspect Clutches - Analyze Discs - Analyze Pressure plates - Inspect Flywheels - Analyze Shaft - Analyze Bearings - Assemble Main Clutch. | - Describe discs. - Describe plates. Practical activity: - Inspect main clutch components. | Theory- 1.5 hours Practice- 4.5 hours Total- 6 hours | Tool trolley (all Tools) Clutch Special service tool | Class Room Workshop |
| - Diagnose and troubleshoot main Clutch and its components. | Trainee will be able to: - Inspect Reservoirs - Inspect Hydraulic cylinders - Analyze Clutch discs and plates - Analyze Flywheel - Inspect Release mechanisms - Inspect Pilot/release bearings - Analyze Input shafts - Inspect Drive mechanisms | - Describe over center system. Practical activity: - Diagnose main clutch components | Theory- 1.5 hours Practice- 4.5 hours Total- 6 hours | Tool trolley (all Tools) Clutch Special service tool | Class Room Workshop |
| - Troubleshoot main Clutch and its components. | Trainee will be able to: - Repair/replace hydraulic cylinders - Replace Clutch discs and plates - Replace Flywheel assemblies - Replace/repair Release mechanisms - Replace Pilot/release bearings - Replace Input shafts - Replace Cables and linkages | - Describe Troubleshoot main clutch and its components Practical activity: - Troubleshoot main clutch components/. | Theory- 1.5 hours Practice- 4.5 hours Total- 6 hours | Tool trolley (all Tools) Clutch Special service tool | Class Room Workshop |

- Perform mechanical Transmission

Objective: This module covers the skills and knowledge required to identify Manual Transmissions Inspect manual transmissions / transfer cases and components and Troubleshoot manual transmissions / transfer cases and components

Duration: 27 Hours								Theory: 06 Hours				Practice: 21 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Recognize Manual Transmissions | Trainee will be able to: - Identify types of manual transmission. - Point out Housings - Identify Gears - Locate Shafts - Locate Synchronizers - Identify Shift mechanisms and linkages - Identify Power take-off (PTO) units - Point out Bearings - Point out Thrust washers - Locate Seals - Point out Gaskets | - Able to identify transmission. Practical activity - Identify transmission. - Identify components of transmission. | Theory- 2 hours Practice- 7 hours Total- 9 hours | Tool trolley (all Tools) Transmission Special service tool | Class Room Workshop |
| - Inspect manual transmissions / transfer cases and components | Trainee will be able to: - Disassemble transmission - Analyze Gears - Analyze Shafts - Inspect Synchronizers - Inspect Shift mechanisms and linkages - Analyze Power take-off (PTO) units - Inspect Bearings - Analyze Fluid - Inspect Thrust washers - Inspect Seals - Analyze Gaskets. | - Able to identify components of transmission. - Able to inspect components of transmission. Practical activity: - Inspect components of transmission. | Theory- 2 hours Practice- 7 hours Total- 9 hours | Tool trolley (all Tools) Transmission Special service tool | Class Room Workshop |
| - Troubleshoot manual transmissions / transfer cases and components | Trainee will be able to: - Replace Gears - Replace Shafts - Replace Synchronizers - Replace/repair Shift mechanisms and linkages - Replace Power take-off (PTO) units - Replace Bearings - Replace Thrust washers - Replace Seals - Replace Gaskets - Replace/refill Fluids. | - Able to troubleshoot components of transmission. Practical activity: - Troubleshoot components of transmission. | Theory- 2 hours Practice- 7 hours Total- 9 hours | Tool trolley (all Tools) Transmission Special service tool | Class Room Workshop |

- Demonstrate the torque converter.

Objective: This module covers the skills and knowledge required to provide skills and knowledge to recognize components of torque converter on cutaway model Analyze Torque Convertor and Troubleshoot torque converter.

Duration: 25 Hours								Theory: 04 Hours				Practice: 21 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Recognize components of torque converter on cutaway model | Trainee will be able to: - Identify Torque converter case - Identify Drive case - Identify Turbine - Identify Stator - Identify Pump - Identify Stator shaft - Identify Stator rear housing - Identify Stator front housing | - Describe torque convertor. - Describe Types of torque convertor. - Describe component of torque convertor. Practical activity - Identify torque convertor. | Theory- 1.3 hours Practice- 7 hours Total- 8.3 hours | Tool trolley (all Tools) Torque Convertor Special service tool | Class Room Workshop |
| - Analyze Torque Convertor. | Trainee will be able to: - Arrange proper tools to disassemble the torque converter. - Disassemble torque convertor. - Inspect Torque converter case - Inspect Drive case - Inspect Turbine - Inspect Stator - Inspect Pump - Inspect Stator shaft - Inspect Stator rear housing - Inspect Stator front housing | - Describe inspection of torque convertor. Practical activity: - Inspect torque convertor. | Theory- 1.3 hours Practice- 7 hours Total- 8.3 hours | Tool trolley (all Tools) Torque Convertor Special service tool | Class Room Workshop |
| - Troubleshoot torque converter. | Trainee will be able to: - Replace Torque converter case - Replace Drive case - Replace Turbine - Replace Stator - Replace Pump - Replace Stator shaft - Replace Stator rear housing - Replace Stator front housing replace Gears - Replace Shafts - Replace Synchronizers - Replace/repair Shift mechanisms and linkages - Replace Power take-off (PTO) units - Replace Bearings - Replace Thrust washers - Replace Seals - Replace Gaskets - Replace/refill Fluids. | - Describe troubleshooting of torque convertor. Practical activity: - Troubleshoot torque convertor. | Theory- 1.3 hours Practice- 7 hours Total- 8.3 hours | Tool trolley (all Tools) Torque Convertor Special service tool | Class Room Workshop |

- Perform different tests of torque converter

Objective: This module covers the skills and knowledge required to Practice stall test of torque converter and Practice pressure test of torque converter on bulldozer.

Duration: 27 Hours								Theory: 06 Hours				Practice: 21 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Practice stall test of torque converter | Trainee will be able to: - Arrange proper tools for stall test of torque converter on bulldozer. - Press brake pedal. - Select 3rd gear. - Increase Engine RPM slowly to set rpm as per manual. - Limit test to max 5 sec. - Check that the engine RPM is within limit. - Report the observation. | - Describe stall test. - Describe stall test procedure. Practical activity - perform stall test of torque convertor | Theory- 3 hours Practice- 10.5 hours Total- 13.5 hours | Tool trolley (all Tools) Torque Convertor Special service tool Pressure test gauge | Class Room Workshop |
| - Practice pressure test of torque converter | Trainee will be able to: - Arrange proper tools to Practice pressure test of torque converter on bulldozer. - Identify test points for measurement of pressures on machine. - Observe temperature within limit. - Make gear in neutral. - Measure torque converter outlet pressure at engine’s low idle & high idle RPM. - Measure Torque converter inlet pressure at engine’s low idle & high idle speed - Check that the pressure values are within limit. | - Describe pressure check procedure of torque convertor. Practical activity: - Check the pressure of torque convertor. | Theory- 3 hours Practice- 10.5 hours Total- 13.5 hours | Tool trolley (all Tools) Torque Convertor Special service tool Pressure test gauge | Class Room Workshop |

- Demonstrate Torque flow transmission.

Objective: This module covers the skills and knowledge required to identify Torque Flow Transmission, Inspect Torque Flow Transmission and Troubleshoot Torque Flow Transmission.

Duration: 29 Hours									Theory: 08 Hours				Practice: 21 Hour

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Recognize Torque Flow Transmission | Trainee will be able to: - Locate Transmission case. - Point out transmission clutch pack. - Point out direction clutch packs. - Locate speed clutch packs. - Point out transmission shaft. - Identify gears. - Point out bearings. | - Describe Torque Flow Transmission. - Describe components of Torque Flow Transmission. Practical activity - Identify components of Torque Flow Transmission. | Theory- 2 hours Practice- 7 hours Total- 09 hours | Tool trolley (all Tools) Torque Flow Transmission. Special service tool | Class Room Workshop |
| - Inspect Torque Flow Transmission. | Trainee will be able to: - Disassemble Transmission. - Analyze Transmission case. - Inspect transmission clutch pack. - Inspect Clutches. - Inspect Plates. - Analyze direction clutch packs. - Analyze speed clutch packs. - Inspect transmission shaft. - Inspect gears. - Point out bearings. | - Describe inspection of Torque Flow Transmission. Practical activity: - Inspect Torque Flow Transmission. | Theory- 3 hours Practice- 7 hours Total- 10 hours | Tool trolley (all Tools) Torque Flow Transmission. Special service tool | Class Room Workshop |
| - Troubleshoot Torque Flow Transmission. | Trainee will be able to: - Disassemble Torque Flow Transmission. - Replace transmission clutch pack. - Replace Discs. - Replace direction clutch packs. - Replace speed clutch packs. - Replace/repair transmission shaft. - Replace gears. - Replace bearings. | - Describe Troubleshoot of Torque Flow Transmission. Practical activity: - Troubleshooting of Torque Flow Transmission. | Theory- 3 hours Practice- 7 hours Total- 10 hours | Tool trolley (all Tools) Torque Flow Transmission. Special service tool | Class Room Workshop |

- Demonstrate control valve assembly of torque flow transmission.

Objective: This module covers the skills and knowledge required to identify control valve assembly of torque flow transmission, inspect control valve assembly of torque flow transmission and Troubleshoot control valve assembly of torque flow transmission.

Duration: 22 Hours								Theory: 04 Hours				Practice: 18 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Recognize control valve assembly of torque flow transmission. | Trainee will be able to: - Locate control valve assembly. - Point out transmission spool valve. - Point out main relief valve. - Point out safety valve. - Locate Modulating and Quick Return Valve. - Point out reducing valve. - Locate direction valve. Limit test to max 5 sec. | - Describe control valve assembly of Torque Flow Transmission. - Describe components of control valve assembly of Torque Flow Transmission. - Describe demand spool. - Describe control valve assembly. - Describe transmission spool valve. - Describe main relief valve. - Describe safety valve. - Describe Modulating and Quick Return Valve - Describe reducing valve. - Describe direction valve. Practical activity - identify components control valve assembly of torque flow transmission. | Theory- 1.3 hours Practice- 06 hours Total- 7.3 hours | Tool trolley (all Tools) Control valve assembly Special service tool | Class Room Workshop |
| - Inspect control valve assembly of torque flow transmission. | Trainee will be able to: - Analyze control valve assembly. - Inspect transmission spool valve. - Inspect main relief valve. - Inspect safety valve. - Inspect Modulating and Quick Return Valve. - Inspect reducing valve. - Analyze direction valve. | - Inspect Control Valve assembly Practical activity: - Inspect control valve assembly of torque flow transmission. | Theory- 1.3 hours Practice- 06 hours Total- 7.3 hours | Tool trolley (all Tools) Control valve assembly Special service tool | Class Room Workshop |
| - Troubleshoot control valve assembly of torque flow transmission. | Trainee will be able to: - Replace control valve assembly. - Repair/Replace transmission spool valve. - Repair/Replace main relief valve. - Repair/Replace out safety valve. - Repair/Replace Modulating and Quick Return Valve. - Repair/Replace reducing valve. - Repair/Replace direction valve. | - Troubleshoot Control Valve assembly Practical activity: - Troubleshooting control valve assembly of torque flow transmission. | Theory- 1.3 hours Practice- 06 hours Total- 7.3 hours | Tool trolley (all Tools) Control valve assembly Special service tool | Class Room Workshop |

- Analyse Hydro-shift Transmission.

Objective: This module covers the skills and knowledge required to provide skills and knowledge to Identify Hydro-shift Transmission, Inspect Hydro-shift Transmission and Troubleshoot Hydro-shift Transmission.

Duration: 27 Hours								Theory: 06 Hours				Practice: 21 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Recognize Hydro-shift Transmission. | Trainee will be able to: - Locate Transmission case. - Point out transmission clutch pack. - Point out direction clutch packs. - Locate speed clutch packs. - Point out transmission shaft. - Identify gears. - Point out bearings. | - Describe Hydro-shift Transmission. - Describe components of Hydro-shift Transmission. Practical activity - Identify components of Hydro-shift Transmission. | Theory- 02 hours Practice- 07 hours Total- 09 hours | Tool trolley (all Tools) Hydro-shift Transmission. Special service tool | Class Room Workshop |
| - Inspect Hydro-shift Transmission. | Trainee will be able to: - Disassemble Transmission. - Analyze Transmission case. - Inspect transmission clutch pack. - Inspect Clutches. - Inspect Plates. - Analyze direction clutch packs. - Analyze speed clutch packs. - Inspect transmission shaft. - Inspect gears. - Point out bearings. Measure torque converter outlet pressure at engine’s low idle & high idle RPM. - Measure Torque converter inlet pressure at engine’s low idle & high idle speed - Check that the pressure values are within limit. | - Describe inspection of Hydro-shift Transmission. Practical activity: - Inspect Hydro-shift Transmission. | Theory- 02 hours Practice- 07 hours Total- 09 hours | Tool trolley (all Tools) Hydro-shift Transmission. Special service tool | Class Room Workshop |
| - Troubleshoot Hydro-shift Transmission. | Trainee will be able to: - Disassemble Torque Flow Transmission. - Replace transmission clutch pack. - Replace Discs. - Replace direction clutch packs. - Replace speed clutch packs. - Replace/repair transmission shaft. - Replace gears. - Replace bearings. | - Describe Troubleshoot of Hydro-shift Transmission. Practical activity: - Troubleshooting of Hydro-shift Transmission. | Theory- 02 hours Practice- 07 hours Total- 09 hours | Tool trolley (all Tools) Hydro-shift Transmission. Special service tool | Class Room Workshop |

- Analyse Hydrostatic Transmission (HST).

Objective: This module covers the skills and knowledge required to Identify of hydrostatic transmission, Inspection of hydrostatic transmission and Troubleshoot hydrostatic transmissions

Duration: 23 Hours								Theory: 05 Hours				Practice: 18 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Recognize the hydrostatic transmission. | Trainee will be able to: - Point out Housings - Locate Pumps - Point out Motors - Point out Gears - Locate Bearings - Identify Seals - Identify Control valves - Locate Lines - Point out Hoses | - Describe hydrostatic transmissions - Describe components of hydrostatic transmissions - Describe Pump. - Describe Motor. Practical activity - identify components of hydrostatic transmissions | Theory- 1.6 hours Practice- 06 hours Total- 7.6 hours | Tool trolley (all Tools) hydrostatic transmissions Special service tool | Class Room Workshop |
| - Inspect hydrostatic transmission. | Trainee will be able to: - Inspect Housings - Analyze Pumps - Analyze Motors - Inspect Gears - Inspect Bearings - Inspect Seals - Inspect Control valves - Analyze Lines - Analyze Hoses | - Describe inspection of hydrostatic transmissions Practical activity: - inspect hydrostatic transmissions | Theory- 1.6 hours Practice- 06 hours Total- 7.6 hours | Tool trolley (all Tools) hydrostatic transmissions Special service tool | Class Room Workshop |
| - Troubleshoot hydrostatic transmissions. | Trainee will be able to: - Replace Housings - Replace/repair Pumps - Replace/repair Motors - Replace Gears - Replace Bearings - Replace Seals - Replace/repair Control valves - Replace Lines - Replace Hoses | - Describe Troubleshoot of hydrostatic transmissions Practical activity: - troubleshooting of hydrostatic transmissions | Theory- 1.6 hours Practice- 06 hours Total- 7.6 hours | Tool trolley (all Tools) hydrostatic transmissions Special service tool | Class Room Workshop |

# 9.21 Hydraulic Excavator

- Demonstrate Hydraulic Excavator

Objective: This module covers the skills and knowledge required to identify excavator and its components, different types and power train of excavator.

Duration: 22 Hours							Theory: 4 Hours				Practice: 18 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Identify components of Excavator | Trainee will be able to: - Identify bucket - Identify arm - Identify boom - Identify bucket cylinder - Identify arm cylinder - Identify boom cylinders - Identify hydraulic pipes - Identify the upper carriage - Identify the under carriage - Identify the work equipment | - Describe main components of excavator. Practical activity: - Identify the upper carriage - Identify the under carriage | Theory- 1.3 hours Practice- 6 hours Total- 7.3 hours | Excavator PP&E Coverall Safety shoes Safety helmet | Class Room and hydraulic workshop |
| - Differentiate Types of Excavator | Trainee will be able to: - Identify Crawler type excavator - Identify Wheel type excavator - Identify Hybrid type excavator | - Describe types of excavator Practical activity: | Theory- 1.3 hours Practice- 6 hours Total- 7.3 hours | . Excavators PP&E Coverall Safety shoes Safety helmet | Class Room and hydraulic workshop |
| - Identify Power Train of Excavator | Trainee will be able to: - Identify engine. - Identify damper - Identify hydraulic pump. - Identify control valve assembly. - Identify swing motor. - Identify swivel join and travel motor. - Identify hydraulic cylinders. | - Describe power train of excavator Practical activity: - Identify hydraulic pump. - Identify control valve assembly | Theory- 1.3 hours Practice- 6 hours Total- 7.3 hours | Excavator PP&E Coverall Safety shoes Safety helmet | Class Room and hydraulic workshop |

- Analyze hydraulic pump of excavator.

Objective: This module covers the skills and knowledge required to Identify Inspection and Troubleshoot of Hydraulic Pump of excavator.

Duration: 27 Hours								Theory: 6 Hours				Practice: 21 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Identify hydraulic Pump circuit | Trainee will be able to: - Identify the hydraulic tank, hydraulic pump, filters, hydraulic pipes, control valve and actuators - Identify different types of hydraulic Pumps. - Identify the Main components of hydraulic Pump - Identify the Charging Pumps/Pilot pump | - Describe Hydraulic pump system. Practical activity: Identify the Charging Pumps/Pilot pump. | Theory- 1.5 hours Practice- 5.25 hours Total- 6.75 hours | Excavator PP&E Coverall Safety shoes Safety helmet | Class Room and Hydraulic workshop |
| - Inspect Hydraulic Pump Circuit | Trainee will be able to: - Inspect the hydraulic tank, hydraulic pump, filters, hydraulic pipes and control valve. - Inspect Hydraulic pump according to set standards. - Inspect the main components of hydraulic pump. - Inspect the Charging pump/Pilot Pump. | - Describe charging pump/pilot pump system Practical activity: - Inspect the hydraulic tank, hydraulic pump, filters, hydraulic pipes and control valve. | Theory- 1.5 hours Practice- 5.25 hours Total- 6.75 hours | Excavator PP&E Coverall Safety shoes Safety helmet | Class Room and Hydraulic workshop |
| - Troubleshoot hydraulic Pump | Trainee will be able to: - Troubleshoot the hydraulic tank, hydraulic pump, filters, Hydraulic Pipes and control valve. - Troubleshoot the hydraulic pump as per requirement. - Troubleshoot charging pump/pilot pump as per requirement. - Disassemble hydraulic Pump. - Assemble hydraulic Pump. - Disassemble charging pump/pilot pump. - Assemble charging pump/pilot pump. | - Describe main pump pressure Practical activity: - Troubleshoot the hydraulic pump as per requirement. | Theory- 1.5 hours Practice- 5.25 hours Total- 6.75 hours | Excavator hydraulic Pump assembly Tool Box Special service tool PP&E Coverall Safety shoes Safety helmet | Class Room and Hydraulic workshop |
| - Adjustment of Hydraulic pump | Trainee will be able to: - Use Hydraulic pressure gauge set. - Select pressure gauge per requirement. - Install pressure gauge. - Start the Engine. - Perform pressure adjustment. - Perform hydraulic system bleeding. - Perform hydraulic pump oil flow testing. | - Describe PPC pressure Practical activity: - Select pressure gauge per requirement. | Theory- 1.5 hours Practice- 5.25 hours Total- 6.75 hours | Excavator pressure gauge set PP&E Coverall Safety shoes Safety helmet | Class Room and Hydraulic workshop |

- Maintain control valves of excavator.

Objective: This module covers the skills and knowledge required to Identify Inspection and Troubleshoot of control Valve assembly of excavator.

Duration: 11 Hours								Theory: 2 Hours				Practice: 9 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Identify control Valve | Trainee will be able to: - Identify the hydraulic tank, Control valve, filters, lines and control valve. - Identify different types of Control valve (OLSS, EOLSS, and CLSS). - Identify the Main components of control valve: Spool valves for actuators, Straight travel valve/ travel junction valve/Flow combiner valve, swing priority valve, arm throttle valve, drift prevention valve/anti-drift valve, pressure compensation valve, Bucket Flow rate control valve, safety valves, safety suction valve & suction valve - Identify main relief valve. - Identify PPC valves (Travel, Work equipment) - Identify two stage relief valves. | Describe Main control valve Practical activity: - Identify different types of Control valve (OLSS, EOLSS, and CLSS). - Identify main relief valve. | Theory- 0.6 hours Practice- 3 hours Total- 3.6 hours | Excavator PP&E Coverall Safety shoes Safety helmet | Class Room and Hydraulic workshop |
| - Inspect control valve | Trainee will be able to: - Inspect the hydraulic tank, filters, lines and control valve. - Inspect Hydraulic control valve according to set standards. - Inspect the main components of hydraulic control valve. - Inspect main relief valve. - Inspect PPC valves (Travel, Work equipment) - Inspect two stage relief valves. | - Describe Accumulators Practical activity: - Inspect Hydraulic control valve according to set standards. - Inspect the main components of hydraulic control valve. | Theory- 0.6 hours Practice- 3 hours Total- 3.6 hours | Excavator PP&E Coverall Safety shoes Safety helmet | Class Room and Hydraulic workshop |
| - Troubleshoot Control valve | Trainee will be able to: - Troubleshoot the hydraulic tank, hydraulic pump, filters, lines and control valve. - Troubleshoot the hydraulic control valve as per requirement. - Dissemble control valve. - Assemble control valve. - Troubleshoot accumulators - Remove accumulators. - Install accumulators. - Adjust the pressure of main relief valve and control valve using gauge set. - Troubleshoot PPC valves (Travel, Work equipment) - Troubleshoot two stage relief valves | - Describe PPC valves Practical activity: - Dissemble control valve. - Install accumulators. | Theory- 0.6 hours Practice- 3 hours Total- 3.6 hours | Excavator control valve assembly Tool box Special service tool PP&E Coverall Safety shoes Safety helmet | Class Room and Hydraulic workshop |

- Maintain hydraulic motors

Objective: This module covers the skills and knowledge required to Identify Inspection and Troubleshoot of hydraulic Motors (swing and travel) of excavator

Duration: 37 Hours									Theory: 7 Hours				Practice: 30 Hour

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Identify hydraulic Motors (Travel and Swing) | Trainee will be able to: - Identify the hydraulic travel motors and lines. - Identify different types of hydraulic travel Motor. - Identify the Main components of hydraulic travel Motor parking brake and brake valve - Identify the hydraulic swing motor and lines. - Identify hydraulic swing lock. - Identify the Main components of hydraulic swing Motor, Swing motor relief valve, reverse prevention valve | - Describe Hydraulic travel motor system. - Describe Hydraulic swing motor system. Practical activity: - Identify the Main components of hydraulic swing Motor, Swing motor relief valve, reverse prevention valve. - Identify hydraulic swing lock. | Theory- 1.7 hours Practice- 7.5 hours Total- 9.2 hours | Excavator PP&E Coverall Safety shoes Safety helmet | Class Room and hydraulic workshop |
| - Inspect Hydraulic Motors (Travel and Swing) | Trainee will be able to: - Inspect the hydraulic travel motors and lines. - Inspect different types of hydraulic travel Motor. - Inspect the Main components of hydraulic travel Motor - Inspect the hydraulic swing motor and Hydraulic Pipes. - Inspect different types of hydraulic swing Motor. - Inspect the Main components of hydraulic swing Motor | - Describe Hydraulic travel motor system. - Describe Hydraulic swing motor system. Practical activity: | Theory- 1.7 hours Practice- 7.5 hours Total- 9.2 hours | Excavator PP&E Coverall Safety shoes Safety helmet | Class Room and hydraulic workshop |
| - Troubleshoot hydraulic Motors (Travel and Swing) | Trainee will be able to: - Troubleshoot the hydraulic Motor - Troubleshoot the hydraulic travel motor as per requirement. - Troubleshoot hydraulic swing motor as per requirement. - Disassemble hydraulic travel motor - Assemble hydraulic travel motor - Disassemble hydraulic swing motor. - Assemble hydraulic swing motor. | - Describe Hydraulic travel motor system. - Describe Hydraulic swing motor system. Practical activity: - Disassemble hydraulic travel motor - Assemble hydraulic travel motor | Theory- 1.7 hours Practice- 7.5 hours Total- 9.2 hours | Excavator Tools trolley (Complete set of Hand Tools) Special Service tools Pressure gauge set PP&E Coverall Safety shoes Safety helmet | Class Room and hydraulic workshop |
| - Adjust Hydraulic motors (Travel and Swing). | Trainee will be able to - Perform pressure adjustment. - Perform hydraulic system bleeding. - Perform hydraulic motor oil flow testing. | - Describe Hydraulic travel motor system. - Describe Hydraulic swing motor system. Practical activity: - Perform hydraulic system bleeding | Theory- 1.7 hours Practice- 7.5 hours Total- 9.2 hours | Excavator Tools trolley (Complete set of Hand Tools) Special Service tools compression gauge set | Class Room and hydraulic workshop |

- Analyze hydraulic cylinders

Objective: This module covers the skills and knowledge required to Identify Inspection and Troubleshoot of hydraulic cylinders of excavator

Duration: 16 Hours								Theory: 4 Hours				Practice: 12 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Identify hydraulic cylinders | Trainee will be able to: - Identify main components inside the cutaway model of hydraulic cylinder. - Identify Piston Valve. - Identify packings. - Identify wear rings. - Identify dust seal. | - State hydraulic cylinder - State types of hydraulic cylinder - State different components of hydraulic cylinder - Describe function and structure of each component of hydraulic cylinder piston ring, wear ring and dust seal. Practical activity: - Identify packing’s. - Identify wear rings.. | Theory- 1.3 hours Practice- 4 hours Total- 5.3 hours | Excavator Hydraulic cylinder cutaway model PP&E Coverall Safety shoes Safety helmet | Class Room and Hydraulic workshop |
| - Inspect hydraulic cylinder | Trainee will be able to: - Inspect different components of hydraulic cylinder - Dissemble hydraulic cylinder - Assemble hydraulic cylinder - Inspect the hoses attached with hydraulic cylinder - Check creeping rate/hydraulic drift test of work equipment | - Describe hydraulic cylinder drift test and how to perform. Practical activity: - Dissemble hydraulic cylinder - Assemble hydraulic cylinder | Theory- 1.3 hours Practice- 4 hours Total- 5.3 hours | Excavator PP&E Coverall Safety shoes Safety helmet | Class Room and Hydraulic workshop |
| - Troubleshoot hydraulic cylinder | Trainee will be able to: - Troubleshoot the seals of hydraulic cylinder - Troubleshoot the piston of hydraulic cylinder - Remove hydraulic cylinder from machine - Install hydraulic cylinder on machine | - State speed test of Work equipment. Practical activity: - Remove hydraulic cylinder from machine - Install hydraulic cylinder on machine. | Theory- 1.3 hours Practice- 4 hours Total- 5.3 hours | Excavator Tools trolley (Complete set of Hand Tools). Special Service tools PP&E Coverall Safety shoes Safety helmet | Class Room and Hydraulic workshop |

- Maintain Hydraulic Control Circuit

Objective: This module covers the skills and knowledge required to Identify Inspection and Troubleshoot of hydraulic control circuit of excavator

Duration: 27 Hours								Theory: 6 Hours				Practice: 21 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Identify hydraulic control circuit | Trainee will be able to: - Identify the CLSS (Closed centre load sensing system) - Identify the OLSS (Open centre load sensing system) - Identify the EOLSS (electronic load sensing system) - Identify the LS valve - Identify the PC valve - Identify EPC valve. - Identify TVC valve. - Identify CO valve. - Identify servo motor. - Identify NC valve. - Identify Servo valve. - Identify pump regulator & speed sensing control - Identify sensors, solenoids and switches. | - Describe OLSS, CLSS and EOLSS. - Describe CO, NC valve. - Describe servo motor. - Describe LS valve. - Describe PC valve - Describe EPC valve. Practical activity: - Identify the PC valve - Identify EPC valve. - Identify TVC valve. - Identify CO valve. | Theory- 1.5 hours Practice- 5.25 hours Total- 6.75 hours | Excavator PP&E Coverall Safety shoes Safety helmet | Class Room and hydraulic workshop |
| - Inspect Hydraulic control Circuit | Trainee will be able to: - Inspect the CLSS (Closed centre load sensing system) - Inspect the OLSS (Open centre load sensing system) - Inspect the LS valve - Inspect the PC valve - Inspect EPC valve. - Inspect TVC valve. - Inspect CO valve. - Inspect NC valve. - Inspect Servo valve - Inspect servo motor. - Inspect pump regulator & speed sensing control - Inspect sensors, solenoids and switches. | - Describe Servo valve - Describe TVC valve - Describe Solenoids, sensors and switches Practical activity: | Theory- 1.5 hours Practice- 5.25 hours Total- 6.75 hours | Excavator PP&E Coverall Safety shoes Safety helmet | Class Room and hydraulic workshop |
| - Troubleshoot hydraulic Control circuit | Trainee will be able to: - Troubleshoot the CLSS (Closed centre load sensing system) - Troubleshoot the LS valve - Troubleshoot the PC valve - Troubleshoot EPC valve. - Troubleshoot TVC valve. - Troubleshoot CO valve. - Troubleshoot NC valve. - Troubleshoot Servo valve - Troubleshoot /replace servo motor - Troubleshoot pump regulator - Troubleshoot/replace sensors, solenoids and switches. | - Describe CO, NC valve. - Describe servo motor. Practical activity: | Theory- 1.5 hours Practice- 5.25 hours Total- 6.75 hours | Excavator PP&E Coverall Safety shoes Safety helmet | Class Room and hydraulic workshop |
| - Adjust Hydraulic control circuit | Trainee will be able to: - Adjust the CLSS (Closed center load sensing system) - Adjust the LS valve - Adjust the PC valve | - Describe OLSS, CLSS and EOLSS. Practical activity: - Adjust the LS valve - Adjust the PC valve | Theory- 1.5 hours Practice- 5.25 hours Total- 6.75 hours | Excavator Tools trolley (Complete set of Hand Tools). Special Service tools PP&E Coverall Safety shoes Safety helmet | Class Room and Hydraulic workshop |

- Maintain air conditioning system

Objective: This module covers the skills and knowledge required to Identify Inspection and Troubleshoot of Air conditioning system of excavator.

Duration: 27 Hours								Theory: 6 Hours				Practice: 21 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Identify Air conditioning system | Trainee will be able to: - Identify the Evaporator - Identify the Condenser - Identify the Compressor - Identify the Blower - Identify the expansion valve - Identify the pressure sensor - Identify the thermostat - Identify the low and high gas pressure hose. | - Describe the refrigerant cycle - Describe the heat, Temperature, scale of temperature - Describe Vacuum - Describe pressure - Describe the Receiver - Describe the Evaporator - Describe the Condenser - Describe the Compressor - Describe the Blower - Describe the expansion valve - Describe the pressure sensor - Describe the thermostat Practical activity: - Identify the expansion valve - Identify the pressure sensor | Theory- 1.5 hours Practice- 5.25 hours Total- 6.75 hours | Excavator PP&E Coverall Safety shoes Safety helmet | Class Room and Hydraulic workshop |
| - Inspect Air conditioning system | Trainee will be able to: - Inspect the Evaporator - Inspect the Condenser - Inspect the Compressor - Inspect the Blower - Inspect the expansion valve - Inspect the pressure sensor - Inspect the thermostat - Inspect the low and high gas pressure hose. | - Describe the refrigerant cycle - Describe the heat, Temperature, scale of temperature - Describe Vacuum - Describe pressure - Describe the Receiver - Describe the Evaporator - Describe the Condenser - Describe the Compressor - Describe the Blower - Describe the expansion valve - Describe the pressure sensor - Describe the thermostat | Theory- 1.5 hours Practice- 5.25 hours Total- 6.75 hours | Excavator PP&E Coverall Safety shoes Safety helmet | Class Room and Hydraulic workshop |
| - Troubleshoot Air conditioning system | Trainee will be able to: - Troubleshoot the hydraulic tank, hydraulic pump, filters, lines and control valve. - Troubleshoot the Evaporator - Troubleshoot the Condenser - Troubleshoot the Compressor - Troubleshoot the Blower - Troubleshoot the expansion valve - Troubleshoot the pressure sensor - Troubleshoot the thermostat - Troubleshoot the low and high gas pressure hose. | - Describe the refrigerant cycle - Describe the heat, Temperature, scale of temperature - Describe Vacuum - Describe pressure - Describe the Receiver - Describe the Evaporator - Describe the Condenser - Describe the Compressor - Describe the Blower - Describe the expansion valve - Describe the pressure sensor - Describe the thermostat | Theory- 1.5 hours Practice- 5.25 hours Total- 6.75 hours | Excavator PP&E Coverall Safety shoes Safety helmet | Class Room and Hydraulic workshop |
| - Perform Assembly Disassembly and Adjustment of Air conditioning system | Trainee will be able to: - Disassemble of Evaporator, Condenser, Compressor, Blower, Expansion valve, pressure sensor, thermostat - Assemble Evaporator, Condenser, Compressor, Blower, Expansion valve, pressure sensor, thermostat - Adjust AC refrigerant pressure and volume. - Add refrigerant | - Describe the refrigerant cycle - Describe the heat, Temperature, scale of temperature - Describe Vacuum - Describe pressure - Describe the Receiver - Describe the Evaporator - Describe the Condenser - Describe the Compressor - Describe the Blower - Describe the expansion valve - Describe the pressure sensor - Describe the thermostat Practical activity: - Adjust AC refrigerant pressure and volume | Theory- 1.5 hours Practice- 5.25 hours Total- 6.75 hours | Excavator Tools trolley (Complete set of Hand Tools). Special Service tools refrigerant PP&E Coverall Safety shoes Safety helmet | Class Room and Hydraulic workshop |

- Analyze Heating System

Objective: This module covers the skills and knowledge required to Identify Inspection and Troubleshoot of Heating system of excavator.

Duration: 27 Hours								Theory: 6 Hours				Practice: 21 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Identify Heating system | Trainee will be able to: - Identify the heating core - Identify the water pump - Identify the hot water valve - Identify heater hose | - Describe hot water cycle - Describe water pump Practical activity: - Identify the hot water valve - Identify heater hose | Theory- 2 hours Practice- 7 hours Total- 9 hours | Excavator PP&E Coverall Safety shoes | Class Room and hydraulic workshop |
| - Inspect Main control valve | Trainee will be able to: - Inspect the heating core - Inspect the water pump - Inspect the hot water valve - Inspect heater hose | - Describe hot water cycle - Describe water pump Practical activity: | Theory- 2 hours Practice- 7 hours Total- 9 hours | Excavator PP&E Coverall Safety shoes Safety helmet | Class Room and hydraulic workshop |
| - Troubleshoot Heating system | Trainee will be able to: - Troubleshooting the heating core - Troubleshooting the water pump - Troubleshooting the hot water valve - Troubleshooting heater hose | - Describe hot water cycle - Describe water pump Practical activity: | Theory- 2 hours Practice- 7 hours Total- 9 hours | Excavator PP&E Coverall Safety shoes Safety helmet | Class Room and hydraulic workshop |

- Analyze ventilation system

Objective: This module covers the skills and knowledge required to Identify Inspection and Troubleshoot of Ventilation system of excavator.

Duration: 27 Hours								Theory: 6 Hours				Practice: 21 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Identify Ventilation system | Trainee will be able to: - Identify the Ducts - Identify Motor - Identify Blower - Identify controller - Identify AC Heater switches, Relays, Resistors | - Describe motor. - Describe blower - Describe controller - Describe ac heater switches relays and resistors. Practical activity: - Identify Motor - Identify Blower - Identify controller | Theory- 2 hours Practice- 7 hours Total- 9 hours | . Excavator PP&E Coverall Safety shoes Safety helmet | Class Room and hydraulic workshop |
| - Inspect ventilation | Trainee will be able to: - Inspect the ducts - Inspect the motor - Inspect the blower - Inspect the controller - Inspect AC Heater switches, Relays, Resistors | - Describe motor. - Describe blower - Describe controller - Describe ac heater switches relays and resistors. Practical activity: - Inspect the blower - Inspect the controller | Theory- 2 hours Practice- 7 hours Total- 9 hours | Excavator PP&E Coverall Safety shoes Safety helmet | Class Room and hydraulic workshop |
| - Troubleshoot ventilation | Trainee will be able to: - Troubleshoot the motor - Troubleshoot the blower - Troubleshoot the controller - Troubleshoot AC Heater switches, Relays, Resistors | - Describe motor. - Describe blower - Describe controller - Describe ac heater switches relays and resistors. Practical activity: - Troubleshoot the blower | Theory- 2 hours Practice- 7 hours Total- 9 hours | Excavator PP&E Coverall Safety shoes Safety helmet | Class Room and hydraulic workshop |

## 9.22 Final Drive, Under Carriage and Power Train/Line

- Recognize the power train and track group

Objective: This module covers the skills and knowledge required to identify the power train of track machine and track group.

Duration: 20 Hours						Theory: 2 Hours					Practice: 18 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Identify the power train | Trainee will be able to: - Identify Damper Unit - Identify Torque Converter - Identify PTO - Identify Transmission Unit - Identify Steering Assembly - Sort out the power train units in sequence. | - Describe Power Train Practical activity: Identify transmission unit Identify steering assembly | Theory- 1 hours Practice- 9 hours Total- 10 hours | Bulldozer PP&E Safety shoes Gloves Safety Goggles Safety Helmet Masks | Class Room and workshop |
| LU2. Demonstrate the track group | Trainee will be able to: - Identify track frame. - Identify sprocket. - Identify idler wheel. - Identify carrier and track roller. - Identify track shoe. - Identify different types of track shoe. - Identify track link, pin and bushes. - Identify equalizer bar. | - Describe structure and function of components of Track Group. - Describe Equalizer bar, Track Roller, Idler Roller, Sprocket, Track Chain, Track Shoes, Recoil Spring Assembly and Track Link Assembly. Practical activity: Identify carrier roller Identify equalizer bar | Theory- 1 hours Practice- 9 hours Total- 10 hours | Bulldozer PP&E Safety shoes Gloves Safety Goggles Safety Helmet Masks | Class Room and workshop |

- Perform inspection of track group

Objective: This module covers the skills and knowledge required to inspect under carriage components and measure the track group

Duration: 20 Hours						Theory: 2 Hours					Practice: 18 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Inspect under carriage components | Trainee will be able to: - Inspect the sprocket. - Inspect the idler wheel. - Inspect the carrier roller and track roller. - Inspect the track shoe and different type of shoe. - Inspect the track link, pin and bushes. | - Fundaments of sprocket - Causes and remedies of wear of Links, pin boss side faces. - Causes and remedies of wear Pin, bushings and cracks in bushings - Causes and remedies of wear of Grouser and Plates of Track Shoes. - Causes and remedies of wear of Roller Tread and Roller Flanges. - Techniques to prolong life of undercarriage parts from operational point of view. - Techniques to prolong life of undercarriage parts from service manual and maintenance point of view. Practical activity: Inspect track roller | Theory- 1 hours Practice- 9 hours Total- 10 hours | Measuring tools Special Service tools Bulldozer Excavator PP&E Safety shoes Gloves Safety Goggles Safety Helmet Masks | Class Room and workshop |
| LU2. Measure the track group | Trainee will be able to: - Arrange proper tools to measure the track roller. - Measure the track roller by keeping in view the Standard size and wear limits. - Measure the Carrier roller by keeping in view the Standard size and wear limits. - Measure the Idler wheel by keeping in view the Standard size and wear limits. - Measure the track chain link pitch by keeping in view the Standard size and wear limits. - Measure the pin and bushing by keeping in view the Standard size and wear limits. - Measure the track chain tension by keeping in view the Standard size and wear limits. | - Measuring tools used for undercarriage components - Measurement procedure for diameter of carrier and track roller. - Measurement procedure for idler roller. - Measurement procedure for measure link pitches and link height. - Measurement procedure for pin and bushes. - Measurement procedure for grouser and plates. Practical activity: - Measure track chain tension | Theory- 1 hours Practice- 9 hours Total- 10 hours | Measuring tools Special Service tools Bulldozer Excavator PP&E Safety shoes Gloves Safety Goggles Safety Helmet Masks | Class Room and workshop |

- Adjust the Track chain tension

Objective: This module covers the skills and knowledge required to adjust the track chain tension and align the track group components

Duration: 20 Hours						Theory: 2 Hours					Practice: 18 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Adjust the Track chain tension | Trainee will be able to: - Arrange Proper Tools for adjust the track chain tension - Check track chain tension - Pump grease through grease nipple with the grease pump.(if needed) - Check track tension; continue pump grease until tension is according to requirement. - Loose lubricator to release the grease if track is tight. - Test the machine for above adjustment by moving a machine forward and reverse direction. - Readjust the Track tension (if needed). | - How to adjust track chain tension. - Structure and function of recoil spring assembly. - Procedure to adjustment of track tension of track chain of Bulldozer. Practical activity: Adjust tension of track chain | Theory- 1 hours Practice- 9 hours Total- 10 hours | Tools trolley (Complete set of Hand Tools) Measuring tools Special Service tools Grease Pump Bulldozer PP&E Safety shoes Gloves Safety Goggles Safety Helmet Masks | Class Room and workshop |
| LU2. Align the track chain | Trainee will be able to: - Arrange Proper Tools to Align the track chain - Inspect the deviation of track with respect to center of sprocket and rollers. - Adjust Idler Clearance in Sideways direction. - Test the machine for above adjustment by moving a machine forward and reverse direction. - Readjust the Idler Clearance (if Needed). | - Structure and function of recoil spring assembly. - Procedure to adjustment of track tension of track chain of Bulldozer. Practical activity: Adjust idler roller side clearance | Theory- 1 hours Practice- 9 hours Total- 10 hours | Tools trolley (Complete set of Hand Tools) Measuring tools Special Service tools Grease Pump Bulldozer PP&E Safety shoes Gloves Safety Goggles Safety Helmet Masks | Class Room and workshop |

- Disassemble and assemble the final drive

Objective: This module covers the skills and knowledge required to disassemble and assemble the final drive of track machine

Duration: 20 Hours						Theory: 2 Hours					Practice: 18 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Disassemble the final drive | Trainee will be able to: - Arrange Proper tools to Disassemble the final drive - Drain oil of final drive. - Remove drive shaft and final drive assembly. - Dismantle Sprocket hub assembly with the help of SST. - Remove wear guard of sprocket assembly. - Remove Cover Assembly. - Remove Carrier Assembly. - Remove Planetary Gear set. - Remove hub and case assembly. - Remove Cage assembly. - Remove Gear Assembly. - Remove outer race and oil seal from case. | - Structure and function of Final Drive system - Structure of Single reduction Final Drive system - Structure of Double reduction Final Drive system - Structure of planetary type Final Drive system - Disassembling procedure of final drive system Practical activity: Remove cover plate | Theory- 1 hours Practice- 9 hours Total- 10 hours | Tools trolley (Complete set of Hand Tools) Special Service tools Bulldozer PP&E Safety shoes Gloves Safety Goggles Safety Helmet Masks | Class Room and workshop |
| LU2. Assemble the final drive | Trainee will be able to: - Arrange Proper tools to assemble the final drive - Inspect, clean and replace faulty parts of final drive assembly - Press fit oil seal and outer race to case. - Install Gear Assembly. - Install Cage assembly. - Install hub and case assembly. - Install Planetary Gear set. - Install Carrier Assembly. - Install Cover Assembly. - Install wear guard of sprocket assembly - Assemble Sprocket hub assembly with the help of SST. - Install drive shaft and final drive assembly - Refill oil of final drive Assembly. | - Structure and function of Final Drive system - Structure of Single reduction Final Drive system - Structure of Double reduction Final Drive system - Structure of planetary type Final Drive system - Assembling procedure of final drive system Practical activity: Install cage assembly | Theory- 1 hours Practice- 9 hours Total- 10 hours | Tools trolley (Complete set of Hand Tools) Special Service tools Bulldozer PP&E Safety shoes Gloves Safety Goggles Safety Helmet Masks | Class Room and workshop |

- Recognize cutaway and differential assembly of a wheeled vehicle.

Objective: This module covers the skills and knowledge required to identify the power train of wheel vehicle and components of differential assembly also working principle of differential assembly

Duration: 15 Hours						Theory: 3 Hours					Practice: 12 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Identify power train of wheel vehicle | Trainee will be able to: - Identify engine. - Identify clutch/Torque convertor. - Identify transmission. - Identify universal joint. - Identify slip joint. - Identify CV (Constant velocity) joint. - Identify propeller shaft. - Identify differential assembly. - Identify axel shaft. - Identify hub. | - Power line of wheeled vehicle. - Universal joint, slip joint, propeller shaft and differential assembly. Practical activity: Identify universal joint and cv joint Identify differential assembly | Theory- 1 hours Practice- 4 hours Total- 5 hours | Differential Assembly Wheel vehicle PP&E Safety shoes Gloves Safety Goggles Safety Helmet Masks | Class Room and workshop |
| LU2. Identify differential assembly of a wheeled vehicle. | Trainee will be able to: - Identify Coupling - Identify bevel pinion and bevel gear. - Identify star pinion. - Identify side gears. - Identify cage and spider. | - Structure and function of differential assembly used in wheeled vehicles. - Working principle of differential assembly during straight travelling and turning. - Structure and function of different types of differential assembles used in wheeled vehicles. Practical activity: Identify bevel gear Identify spider | Theory- 1 hours Practice- 4 hours Total- 5 hours | Differential Assembly Wheel vehicle PP&E Safety shoes Gloves Safety Goggles Safety Helmet Masks | Class Room and workshop |
| LU3. Demonstrate cutaway model of differential assembly | Trainee will be able to: - Identify Coupling - Identify bevel pinion and bevel gear. - Identify star pinion. - Identify side gears. - Identify cage and spider. | - Structure and function of differential assembly used in wheeled vehicles. - Working principle of differential assembly during straight travelling and turning. - Structure and function of different types of differential assembles used in wheeled vehicles. Practical activity: Identify side gear | Theory- 1 hours Practice- 4 hours Total- 5 hours | Differential Assembly Wheel vehicle PP&E Safety shoes Gloves Safety Goggles Safety Helmet Masks | Class Room and workshop |

- Disassemble & assemble inter-axle differential assembly

Objective: This module covers the skills and knowledge required to disassemble and assemble the inter-axel differential assembly

Duration: 20 Hours						Theory: 2 Hours					Practice: 18 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Disassemble inter-axle differential assembly | Trainee will be able to: - Arrange Proper tools to Disassemble inter-axle differential assembly - Drain oil from differential and final Drive Case. - Dismantle Wheel Assembly. - Pull out Drive shaft - Remove inter-axle differential assembly. - Remove Coupling - Remove Bevel Pinion and cage assembly. - Remove Differential gear Assembly from Differential case. - Dismantle the differential gear assembly. | - Explain structure and function of inter axle differential assembly - Explain testing and adjustment of inter axle differential assembly Practical activity: Identify transmission unit Identify steering assembly | Theory- 1 hours Practice- 9 hours Total- 10 hours | Tools trolley (Complete set of Hand Tools) Special Service tools inter-axle differential assembly PP&E Safety shoes Gloves Safety Goggles Safety Helmet Masks | Class Room and workshop |
| LU2. Assemble inter-axle differential assembly | Trainee will be able to: - Arrange Proper tools to assemble inter-axle differential assembly. - Inspect, clean and replace faulty parts of inter-axle differential assembly - Assemble the differential gear assembly. - Install Differential gear Assembly on Differential case. - Adjust preload on bearings. - Install Bevel Pinion and cage assembly. - Install Coupling of differential. - Install inter axle differential assembly on machine. - Refit Drive shaft - Assemble Wheel Assembly. - Refill oil in differential and final Drive Case. | - Explain structure and function of inter axle differential assembly - Explain testing and adjustment of inter axle differential assembly Practical activity: Identify carrier roller - Identify equalizer bar | Theory- 1 hours Practice- 9 hours Total- 10 hours | Tools trolley (Complete set of Hand Tools) Special Service tools inter-axle differential assembly PP&E Safety shoes Gloves Safety Goggles Safety Helmet Masks | Class Room and workshop |

- Recognize structure and type of Off-road Tires.

Objective: This module covers the skills and knowledge required to identify the structure, types, coding of tires.

Duration: 15 Hours						Theory: 3 Hours					Practice: 12 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Identify structure of OFF road tires | Trainee will be able to: - Identify Tread - Identify Crown - Identify Groove. - Identify Breaker - Identify Car case - Identify Bead Section - Identify Sidewall - Identify different Tire coding | - Nomenclature of OFF Road Tires - Explain structure / parts of OFF-Road Tires. Practical activity: Identify bead section Identify Tread | Theory- 1 hours Practice- 4 hours Total- 5 hours | Measuring taps ON Road tire OFF Road tire PP&E Safety shoes Gloves Safety Goggles Safety Helmet Masks | Class Room and workshop |
| LU2. Identify different types of OFF road Tires | Trainee will be able to: - Identify Traction type tires - Identify Block type tires - Identify Rock type tires - Identify Rib type tires - Identify Snow pattern tires - Identify Smooth pattern Tires | - Explain different types of OFF Road Tires and their application on construction machines. - Selection of appropriate tire according to load and road condition. - Explain Traction and Flotation properties of OFF Road Tires. - Explain Blasting of Tires. - Describe importance of Ton Kilo meter per Hour (TKPH) value of a Tire. - Describe correct way to use Tires. - Compare Tube Type and Tube less Type Tires. - Explain special type Tires. Practical activity: Identify Block type tire | Theory- 1 hours Practice- 4 hours Total- 5 hours | Measuring taps ON Road tire OFF Road tire PP&E Safety shoes Gloves Safety Goggles Safety Helmet Masks | Class Room and workshop |
| LU3. Identify different coding of tires | Trainee will be able to: - Identify coding of OFF road Tires. - Identify coding of ON road Tires. - Identify coding of Tires in Inches System - Identify coding of Tires in Metric System | - Explain different types of OFF Road Tires and their application on construction machines. - Selection of appropriate tire according to load and road condition. - Explain Traction and Flotation properties of OFF Road Tires. Practical activity: Identify coding of OFF road tire Identify coding of ON road tire | Theory- 1 hours Practice- 4 hours Total- 5 hours | Measuring taps ON Road tire OFF Road tire PP&E Safety shoes Gloves Safety Goggles Safety Helmet Masks | Class Room and workshop |

## 9.23 Entrepreneurship

### Investigate micro business opportunities

Objective: This module covers the skills and knowledge required to develop business ideas, and to investigate market needs and factors affecting potential markets.

Duration: 5 Hours									Theory: 2 Hours				Practice: 3 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Describe business ideas | Trainee will be able to: - Gather information for business ideas from appropriate sources - List details of business ideas and opportunities - Research alternative business ideas in light of the resources available - Specify and list products and services to match business ideas - Identify and research potential customer information for business ideas - Identify and take into account financial, business and technical skills available when researching business opportunities | - Understand concept of entrepreneurship. - how to generate a business idea - various types of enterprise that exist in the community - Interpret the terms and elements involved in the concept of enterprise - Explain various motivational factors that entrepreneurs possess and utilize. - Describe the outline of small enterprise - Apply the techniques for developing creative abilities - Explain the resources of business idea - Explain the collective and creative thinking - Appreciate the importance of, and possess techniques for identifying and assessing business opportunities. - Identify the various entrepreneurial characteristics Practical activity - Propose a new business idea | Theory- 1 hours Practice- 1 hours Total- 2 hours | Stationary items | Class Room |
| - Identify market needs | Trainee will be able to: - Collect information regarding market size and potential from appropriate sources - Investigate market trends and developments to identify market needs relative to business ideas - Gather market information from primary and secondary sources to identify possible market needs in relation to business ideas - Identify ethical and cultural requirements of the market and their impact on business ideas - Identify new and emerging markets and document their features Identify and organize information on expected market growth or decline and associated risk factors | - steps for decision making and rating of decision-making skills - creativity and innovation - rational approach to make personal and business decisions - Apply the rules of negotiation for resolving business issues Practical activity: - Identify market needs of proposed business idea | Theory- 0.5 hours Practice- 1 hours Total- 1.5 hours | Stationary items | Class Room |
| - Investigate factors affecting the market | Trainee will be able to: - Identify projected changes in population, economic activity and the labor force that may affect business ideas - Identify movements in prices and projected changes in availability of resources - Review trends and developments and identify their potential impact on business ideas | - Access personal potential for becoming future entrepreneurs - Identify self- management skills and how they are important to be enterprising - Identify leadership qualities which are essential to the success of entrepreneurs Practical activity: - Identify possible trends that can affect proposed business idea | Theory- 0.5 hours Practice- 1 hours Total- 1.5 hours | Stationary items | Class Room |

### Develop Micro Business Proposal

Objective: This module covers the skills and knowledge required to develop an identified business idea, to research the feasibility of the business opportunity and to present a business idea in formats that suit a range of stakeholders.

Duration: 5 Hours							Theory: 02 Hours				Practice: 03 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Evaluate business opportunities | Trainee will be able to: - Identify and research key factors that influence viability of business ideas - Analyze business ideas in terms of personal or family needs and commitments - Evaluate impacts of emerging or changing technology, including e-commerce, on the business - Determine viability of business opportunity in line with perceived risks, resources available, financial returns and other outcomes sought - Assess and match personal skills/attributes against those perceived as necessary for a particular business opportunity - Identify and assess business risks according to resources available and personal preferences | - State and local government legislative requirements relating to business operation, especially in regard to occupational health and safety (OHS) and environmental issues, Practical activity: - Identify possible business opportunities for proposed business plan | Theory- 1 hours Practice- 1 hours Total- 2 hours | Stationary items | Class Room |
| - Detail the business idea | Trainee will be able to: - Develop an accurate description of the business idea for key stakeholders - Develop an accurate summary of the major products and/or services required to suit personal needs and requirements | - Income and expenditure costing Practical activity: - Perform income and expenditure costing of business idea | Theory- 0.5 hours Practice- 1 hours Total- 1.5 hours | Stationary items | Class Room |
| - Prepare the business overview to suit different stakeholders | Trainee will be able to: - Present an accurate list of key stakeholders and their information requirements - Determine an acceptable method of presentation of information for each stakeholder - Provide accurate customized information to target audiences | - Principles of risk assessment relevant to the business opportunity | Theory- 0.5 hours Practice- 1 hours Total- 1.5 hours | Stationary items | Class Room |

### Develop Marketing Plan

Objective: This module covers the skills and knowledge required to research, develop and present a marketing plan for an entrepreneurship business.

Duration: 5 Hours							Theory: 02 Hours				Practice: 03 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Devise marketing strategies | Trainee will be able to: - Evaluate marketing opportunity options that address organizational objectives, and evaluate their risks and returns in the selection process - Develop marketing strategies that address strengths and opportunities within the organization’s projected capabilities and resources - Develop strategies which increase resources or organizational expertise where gaps exist between current capability and marketing objectives - Develop feasible marketing strategies and communicate reasons that justifies their selection - Ensure strategies align with organization’s strategic direction - Develop a marketing performance review strategy, incorporating appropriate marketing metrics to review of organizational performance against marketing objectives | - Describe the market & marketing - Differentiate between sellers and buyers’ market - Describe the five ‘w’ of market Practical activity: - Perform Strength weakness opportunity and treat (SWOT) analysis for proposed Business plan | Theory- 1 hours Practice- 1 hours Total- 2 hours | Stationary items | Class Room |
| - Plan marketing tactics | Trainee will be able to: - Detail tactics to implement each marketing strategy in terms of scheduling, costing, accountabilities and persons responsible - Identify coordination and monitoring mechanisms for scheduled activities - Ensure tactics are achievable within organization’s projected capabilities and budget - Ensure tactics meeting legal and ethical requirements - Ensure tactics provide for ongoing review of performance against objectives and budgets, and allow marketing targets to be adjusted if necessary | - Explain the procedure for assessing the market size and demand - Explain the major factors to be considered when selecting a location for a business - Describe the basic types of business ownership and the limitation of each Practical activity: - Propose marketing tactics for proposed business plan | Theory- 0.5 hours Practice- 1 hours Total- 1.5 hours | Stationary items | Class Room |
| - Prepare and present a marketing plan | Trainee will be able to: - Ensure marketing plan meets organizational, as well as marketing, objectives and incorporates marketing approaches and a strategic marketing mix - Ensure marketing plan contains a rationale for objectives and information that supports the choice of strategies and tactics - Present marketing plan for approval in the required format and timeframe - Adjust marketing plan in response to feedback from key stakeholders and disseminate for implementation within the required timeframe | - Explain the computation of initial and working capital needed to start an enterprise - Identify the advantages and disadvantages of using various sources of capital to start an enterprise - Explain the component of cost of product - Explain the breakeven analysis for a new business - Calculate the breakeven point for various new business Practical activity: - Develop market plan for proposed business idea | Theory- 0.5 hours Practice- 1 hours Total- 1.5 hours | Stationary items | Class Room |

### Develop and Review Business Plan

Objective: This module covers the skills and knowledge required to research, develop and present a marketing plan for an entrepreneurship business.

Duration: 5 Hours							Theory: 02 Hours				Practice: 03 Hours

|  | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Determine scope of business | Trainee will be able to: - Determine scope of the business plan and associated systems is determined in consultation with specialist personnel. - Access accurate information for inform business plan development - Account for and incorporate trends and seasonal variations into the business plan. - Account for strategic goals, targets and directions of the enterprise in the development of the business plan - Comply Legal obligations in developing the business plan. | - Appreciate the importance of business plan - Explain the process of writing a business plan Practical activity: - Identify strategic goals, targets and directions of proposed business plan | Theory- 1 hours Practice- 1 hours Total- 2 hours | Stationary items | Class Room |
| - Prepare business plan | Trainee will be able to: - Develop operational goals and targets to meet the enterprise strategic plan. - Identify and incorporate supply chains into the business plan. - Identify risk management needs are within the business plan. - Incorporate trial systems in order to test budgetary impact and operational potential prior to full implementation of the business plan. - Set clear and measurable indicators of operational performance to allow for realistic analysis of performance. | - Develop feasibility for a business idea - Realize the problem that may be encountered when starting a small business/Enterprise - Develop a business plan for a small business on the standard format Practical activity: - Perform Strategic planning of proposed business plan | Theory- 0.5 hours Practice- 1 hours Total- 1.5 hours | Stationary items | Class Room |
| - Document and review business plan | Trainee will be able to: - Include fiscal and operational systems that enhance performance management and suit enterprise requirements. - Incorporate resource considerations the business plan. - Document accurately and clearly communicate business Plan to all relevant parties. - Monitor to identify strengths, weaknesses and areas for improvement performance against the business plan - Make recommendations to improve the business plan and associated systems as required. | - Evaluate the business plan in a real market satiation. Practical activity: - Identify room for improvement in proposed business plan | Theory- 0.5 hours Practice- 1 hours Total- 1.5 hours | Stationary items | Class Room |

### Monitor & Review Business Performance

Objective: This module covers the skills and knowledge required to monitor and review the commercial performance of the business, allocation and coordination of business resources and identification of performance requirements.

Duration: 5 Hours							Theory: 02 Hours				Practice: 03 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Evaluate commercial performance | Trainee will be able to: - Gather and analyze data relating to enterprise performance to identify historical and current performance. - Review and analyze operational structures to determine the suitability of organizational processes to enterprise objectives. - Evaluate enterprise strengths and weaknesses against market conditions to determine current and future capacities. - Evaluate enterprise objectives are to identify variations and scope for future development. | - Rates of return for products and/or services - Financial analysis techniques Practical activity: - Evaluate business performance | Theory- 0.5 hours Practice- 1 hours Total- 1.5 hours | Stationary items | Class Room |
| - Allocate and co-ordinate business resources | Trainee will be able to: - Identify and communicate roles and responsibilities of personnel. - Identify resource requirements for enterprise and cost them using standard financial analysis techniques. - Calculate costs of ensuring sustainability of enterprise operations and factor into business planning for the enterprise. | - Structure and operation of small businesses relevant State/Territory Occupational Health and Safety (OHS) - Legislative requirements - Environmental conditions, positive environmental practices and negative impact minimization measures Practical activity: - Calculate cost for resources required for proposed business plan | Theory- 0.5 hours Practice- 1 hours Total- 1.5 hours | Stationary items | Class Room |
| - Identify performance requirements | Trainee will be able to: - Develop realistic performance indicators within available timeframes and resources - Identify and minimize factors inhibiting performance against objectives. - Monitor and assess market conditions based on relevant data. - Prepare and incorporate strategies and programs to promote the sustainability of operations into enterprise procedures. | - Human resource requirements for the enterprise - Transport requirements for the enterprise - Enterprise/property improvement requirements. Practical activity: - Evaluate performance indicators for proposed business plan | Theory- 0.5 hours Practice- 0.5 hrs Total- 1 hr | Stationary items | Class Room |
| - Review business performance | Trainee will be able to: - Review regularly enterprise operations to identify opportunities for improvements in performance. - Monitor and anticipate impact of natural conditions on enterprise to assess sustainability of resource use. - Compare costs and estimates with resource allocation. - Determine operational plans to determine schedule of activities | - Market performance in commodities - Statutory marketing requirements Practical activity: - Review business plan to identify gaps | Theory- 0.5 hours Practice- 0.5 hrs Total- 1 hours | Stationary items | Class Room |

### Organize Finances for Micro-Business

Objective: This module covers the skills and knowledge required to investigate the financial capacity to enter into a micro business, to determine the projected cash flow, to source finances and to monitor the profitability of the business.

Duration: 5 Hours							Theory: 02 Hours				Practice: 03 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Ascertain own financial position and the ability to provide capital/equity for the business | Trainee will be able to: - Realistically detail personal, family or community financial situation in terms of funds available and commitments already incurred - Determine equity finance and assets available for micro business from personal, family or community sources | - Basic budgeting - Basic costing for the business Practical activity: - Perform Budgeting of proposed business | Theory- 0.5 hours Practice- 1 hours Total- 1.5 hours | Stationary items | Class Room |
| - Determine projected cash flow for the business | Trainee will be able to: - Determine the level of forecast business activity over a year and the business mix - Estimate establishment costs for the business and repayment schedule for borrowings - Calculate the monthly variable and fixed costs needed to conduct business activity over a year - Estimate personal drawings needed to be taken from the business - Estimate the monthly income generated by the business for a year based on price per unit item or hourly charge rate for labor - Develop a cash flow budget for the first year of business operation - Seek professional advice to estimate goods and services tax and operating finance required for the business | - Financial commitments and requirements - Financial reports and terminology Practical activity: - Develop cash flow for proposed business plan | Theory- 0.5 hours Practice- 1 hours Total- 1.5 hours | Stationary items | Class Room |
| - Source the required funds to establish the business | Trainee will be able to: - Estimate required funding to establish and run the business based on expected sales and activity levels, available finances and commitments - Investigate methods of accessing alternative sources of finance - Identify strategies for meeting financial obligations - Implement plans to access available funds as required | - Methods and relative costs of obtaining finance - Own financial position Practical activity: - Suggest a funding plan for proposed business plan | Theory- 0.5 hours Practice- 0.5 hours Total- 1 hours | Stationary items | Class Room |
| - Monitor profitability of the business | Trainee will be able to: - Maintain and review monthly expenditure and income records - Compare equity at beginning and end of a year to estimate business performance - Assess the financial viability of the business after a year of operation - Seek professional advice on depreciation, insurance and tax implications of the business | - Sources of advice and assistance. Practical activity: - Suggest strategies to monitor proposed business performance | Theory- 0.5 hours Practice- 0.5 hours Total- 1 hours | Stationary items | Class Room |

### Manage human resources

Objective: This module covers the skills and knowledge required to manage human resources and to manage and develop human resources to achieve organization’s operational objectives.

Duration: 5 Hours							Theory: 02 Hours				Practice: 03 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Lead and motivate people | Trainee will be able to: - Establish goals for people and teams to optimize achievement in work tasks. - Take into account the capabilities of people and teams. - Provide advice and support sensitive to the individual's needs to people in the performance of their duties. - Undertake activities to achieve commitment to common goals. - Recognize and encourage initiative and innovation - Recognize and communicate achievements within the organization. | - Describe the hiring method/Procedures - Describe the term & conditions of services and job description for various employments Practical activity: Suggest strategies for workforce motivation | Theory- 0.5 hours Practice- 0.5 hours Total- 1 hours | Stationary items | Class Room |
| - Undertake human resource planning | Trainee will be able to: - Determine human resource needs within the anticipated operational needs and allocated budget. - Analyze alternatives to staffing levels which clearly demonstrate returns to the organization. - Develop contingency plans for staffing which meet key provisions of the human resources plan. - Compare existing competencies of staff with the needs of the work group. - Plan staffing levels and negotiate with stakeholders within the organisational framework to achieve maximum efficiency of operations. | - Describe the characteristics of successful sales personals - communication principles - conflict resolution principles and practice - Equal Employment Opportunity Practical activity: Compute workforce required for proposed business plan | Theory- 0.5 hours Practice- 1 hours Total- 1.5 hours | Stationary items | Class Room |
| - Develop and facilitate performance | Trainee will be able to: - Negotiate performance criteria individuals, teams and work groups. - Review performance criteria as circumstances change. - Conduct performance appraisal based on clearly established and agreed performance criteria. - Identify and propose the total performance development system strategies to rectify performance shortfalls and recognize success. - Address performance problems confidentially and in a constructive and timely manner, in line with relevant organizational procedures. - Make selections, transfers and promotions in accordance with organization policies and supported with documented information. - Develop and implement mechanisms for the identification of human resource development needs within the work group taking account of the strategic plan for the organization. | - grievance procedures - interpersonal relations - leadership theory and principles Practical activity: Organize performance appraisal system for required workforce | Theory- 0.5 hours Practice- 0.5 hours Total- 1 hours | Stationary items | Class Room |
| - Facilitate training, education and development opportunities | - .Make information on planned training events widely available throughout the organization. - Include training, education and development plans as part of individual/team performance plans. - Facilitate individual/team access to, and participation in, training, education and development opportunities. - Contribute coaching and mentoring effectively to the training, education and development of personnel in an environment of change. - Enhance training, education and development opportunities of individual, team and organizational performance. - Create workplace environment is which facilitates training, education and development | - training and education principles - training need analysis Practical activity: - Organize training programs for human resource | Theory- 0.5 hours Practice- 0.5 hours Total- 1 hours | Stationary items | Class Room |

### Market the products and services

Objective: This module covers the skills and knowledge required to develop business ideas, and to investigate market needs and factors affecting potential markets.

Duration: 5 Hours							Theory: 02 Hours				Practice: 03 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Analyze market information | Trainee will be able to: - Identify, research and analyze existing or new markets for existing or new products or services using techniques to ensure reliable data - Analyze past trends and developments to determine market variability and associated risks - Develop gross margin budgets to account for market variability - Identify and evaluate competing products to determine strengths and weaknesses of own products - Monitor market environment to ensure information is current and reliable - Identify the legal, ethical and environmental constraints of the markets and their effect on the enterprise - Identify product specifications that suit market requirements and price advantage at the time - Present clear and concise information to the enterprise management team. | - Describe the life cycle of product - Identify the various ways of selecting suppliers, - Explain the inventory management of stock, raw material and finished goods etc. Practical activity: Identify market trends for proposed business plan | Theory- 0.25 hours Practice- 0.5 hour Total- 0.75 hours | Stationary items | Class Room |
| - Identify and evaluate factors to include in a marketing plan | Trainee will be able to: - Identify and evaluate production processes to ensure required product specifications are met - Identify and assess alternative selling strategies and techniques to identify marketing targets and methods - Identify and assess distribution channels and their role in your marketing strategies - Ensure the data used is reliable and the market environment and trends are substantiated - Evaluate the role of marketing professionals in providing advice | ` Practical activity: Evaluate factors for marketing of proposed business plan | Theory- 0.25 hours Practice- 0.5 hour Total- 0.75 hours | Stationary items | Class Room |
| - Develop a marketing plan for your products and services | Trainee will be able to: - Establish marketing objectives based on current and potential product specifications - Select appropriate production processes to ensure product specifications are met - Select selling strategies to ensure required prices are achieved - Select appropriate distribution channel options to ensure access to target markets is achieved efficiently and appropriately - Establish time-frames for production, distribution and selling activities - Develop a gross margin budget to demonstrate the cost effectiveness of the marketing plan - Develop partial gross margin budgets to account for market variability | - Appreciate the importance of financial record keeping in a small business - Explain techniques to keep cost as low as possible Practical activity: Compute marketing plan for proposed business products/services | Theory- 0.25 hours Practice- 0.5 hour Total- 0.75 hours | Stationary items | Class Room |
| - Determine promotional strategies | Trainee will be able to: - Prepare and record detailed plans for promotional activities - Outline objectives, level of exposure and available markets - Ensure strategies take account of time management and scheduling issues, and resource constraints - Create promotional materials that enhance the product and commercial presentation - Record and communicate priorities, responsibilities, timelines and budgets for promotional activities. | - Create promotional strategies Practical activity: Identify promotional strategies for proposed business plan | Theory- 0.25 hours Practice- 0.5 hour Total- 0.75 hours | Stationary items | Class Room |
| - Implement marketing activities | Trainee will be able to: - Schedule planned marketing activities within appropriate timeframes - Develop measurable performance targets that meet business plan objectives - Organize distribution channels and ensure product and service information is accurate and readily available to clients - Implement marketing activities within budgetary constraints to meet legal, ethical and enterprise requirements | - Develop balance sheet for a small enterprise - Explain the operating cycle concept Practical activity: Identify strategies required to implement marketing activities of proposed business plan | Theory- 0.5 hours Practice- 0.5 hour Total- 01 hours | Stationary items | Class Room |
| - Evaluate marketing performance. | Trainee will be able to: - Review the established marketing objectives to ensure they remain viable - Make an objective assessment of the marketing plan and its implementation by a comparison of valid and reliable data against the established objectives - Assess product, pricing and distribution policies in relation to market changes, marketing objectives and enterprise requirements - Identify areas of positive marketing performance and take corrective action to remedy poor marketing performance areas - Document and distribute information for continual analysis and effective planning management | - Explain the basic scheme of sales tax - Explain the assessment procedure for returns and filling of returns. Practical activity: Prepare document to evaluate marketing performance of proposed business plan. | Theory- 0.5 hours Practice- 0.5 hour Total- 1 hours | Stationary items | Class Room |

### Negotiate for resolving business issues

Objective: This module covers the skills and knowledge required to negotiate for resolving business issues

Duration: 5 Hours							Theory: 02 Hours				Practice: 03 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Develop and implement issue resolution and grievance procedures | Trainee will be able to: - Establish problem solving/issue resolution procedures within legislative requirements and organization’s guidelines. - Manage grievances and complaints are in a timely and caring way to optimize likelihood of a favorable outcome for all parties and in line with organizational objectives and procedures. - Document and communicate individual's rights and obligations under industrial awards/agreements and legislation are in a clear and concise manner and in appropriate language. - Conduct hearings, interviews and meetings within the principles of industrial democracy and participative, consultative processes. - Identify and invite all relevant industrial parties to participate in the resolution process | - Problem Solving Practical activity: - Develop procedures for issue resolution for proposed business | Theory- 1 hours Practice- 1.5 ours Total- 2.5 hours | Stationary items | Class Room |
| - Manage disciplinary matters | Trainee will be able to: - Implement disciplinary matters in accordance with organization’s procedures. - Conduct investigations in a caring and confidential manner to maintain performance and morale. - Provide feedback promptly. - Institute appeals processes in accordance with organizational procedures. - Recognize and recommend deficiencies in procedures for changes made | Techniques to maintain discipline in organization Practical activity: - Develop procedures to manage disciplinary matters for Proposed business | Theory- 1 hours Practice- 1.5 hours Total- 2.5 hours | Stationary items | Class Room |

- List of Machinery, Tools & Equipment.

(FOR A CLASS OF 25 STUDENTS)

### 10.1. List of Machinery, Tools & Equipment Level-5 (Part-1)

| S# | Tools and Equipment | Quantity | S# | Consumable Material | Quantity |
| --- | --- | --- | --- | --- | --- |
| 1 | Bulldozer (Old for disassembling Practical) | 01 No |  | Carb cleaner spray | 25 Nos |
| 2 | Circuit board | 05 Nos |  | Cotton rag | 10 kg |
| 3 | Generator | 01 No |  | Kerosene oil | 20 liters |
| 4 | Hand tools trolley | 05 Nos |  | oil seals | 25 Nos |
| 5 | Hydraulic Cylinder Cutaway model | 05 Nos |  | Silicone | 10 packets |
| 6 | Hydraulic pump cutaway models | 05 Nos |  |  |  |
| 7 | Hydraulic tank cutaway model | 05 Nos |  |  |  |
| 8 | Main Alternator | 05 Nos |  |  |  |
| 9 | Measuring scale | 05 Nos |  |  |  |
| 10 | Multimeter | 25 Nos |  |  |  |
| 11 | Pressure gauge set | 25 Nos |  |  |  |
| 12 | Series and Parallel circuit board. | 25 Nos |  |  |  |
| 13 | Shop manual | 25 Nos |  |  |  |
| 14 | Special Service tools. | 25 Nos |  |  |  |
| 15 | Tools trolley (Complete set of Hand Tools) | 05 Nos |  |  |  |
| 16 | Training aid /board of Pressure and flow difference. | 05 Nos |  |  |  |

## 10.2. List of Machinery, Tools & Equipment Level-5 (Part-2)

| Sr. # | Description | Quantity | Sr. # | Consumable | Quantity |
| --- | --- | --- | --- | --- | --- |
| 1. | Bulldozer (Old for disassembling Practical) | 01 No |  | Carb cleaner spray | 25 Nos |
| 2. | Clutch | 05 Nos |  | Cotton rag | 10 kg |
| 3. | Control valve assembly | 05 Nos |  | Kerosene oil | 20 liters |
| 4. | Crane | 01 No |  | oil seals | 25 Nos |
| 5. | Crawler excavator (Old for disassembling Practical) | 01 No |  | Silicone | 10 packets |
| 6. | Cutaway model | 05 Nos |  |  |  |
| 7. | Differential Assembly | 05 Nos |  |  |  |
| 8. | Dump Truck | 01 No |  |  |  |
| 9. | Final Drive | 05 Nos |  |  |  |
| 10. | Grease Pump | 05 Nos |  |  |  |
| 11. | Hybrid Excavator | 01 No |  |  |  |
| 12. | Hydraulic Gauge set | 05 Nos |  |  |  |
| 13. | Hydro-shift Transmission | 05 Nos |  |  |  |
| 14. | hydrostatic transmissions | 05 Nos |  |  |  |
| 15. | Measuring tools | 05 Nos each |  |  |  |
| 16. | Pressure test gauge | 05 Nos |  |  |  |
| 17. | Special Service tools | 05 Nos |  |  |  |
| 18 | Tools trolley (Complete set of Hand Tools) | 05 Nos |  |  |  |
| 19. | Torque Convertor | 05 Nos |  |  |  |
| 20. | Transmission | 05 Nos |  |  |  |
| 21. | Wheel excavator (Old for disassembling Practical) | 01 No |  |  |  |

- Members of the Curriculum Development Committee.

| S# | Name | Designation | Organization |
| --- | --- | --- | --- |
| 1 | Engr Inayat Ur Rehman | DACUM Facilitator | Ex Director Academics KP-TVETA. |
| 2 | Mr. Adil Qadeer | Lecture | University of Lahore, |
| 3 | Mr. Muhammad Arsalan | Lecture | University of Lahore, |
| 4 | Engr Jamal Akbar | Associate Prof | GPI Sardar Garhi, Peshawar KP-TVETA |
| 5 | Engr Farooq Iftikhar | Metallurgy engineer | Pakistan Institute of Technology for Advanced Engineering Material, Lahore |
| 6 | Engr Umar, | Instructor | Swedish College Gujarat, P-TEVTA |
| 7 | Mr. Amir Amin | HOD | Polytechnic College Lahore, |
| 8 | Mr Muhammad Rizwan Zafar | HOD, Mechanical, | Technology college Lahore, |
| 9 | Ms Saba Sadiq | Lecture Mechanical, | UOL, Campus, Islamabad, |
| 10 | Fazal Rehman, | Associate Professor | GCT Peshawar, KP TEVA, |
| 11 | Saad Amjad | Project Engineer, | Cagrow consulting Company Lahore |
| 12 | Umar Qureshi | Director, | Pakistan National Accreditation Council, Islamabad |
| 13 | Syed Muhammad Syed Arsalan | Lecture, | GCT Railway Road Lahore |
| 14 | Amina Irfan | Lecture | MS Industrial Engineering, Lahore |
| 15 | Mr. Muhammad Yasir, | Dy. Director | NAVTTC, Islamabad |