National Vocational Certificate in

Heavy Machine Operator “Crane Operator”

Level-2

Competency Standards

National Vocational and Technical Training Commission (NAVTTC)

Government of Pakistan

Contents

1.	Title of Qualification	3

2.	Introduction	3

3.	Purpose of the Qualification	4

4.	Main Objectives of the Qualification	4

5.	Date of Validation	5

6.	Codes of Qualifications	5

7.	Members of Qualification Development Committee	6

8.	Entry Requirements	7

9.	Summary of Competency Standards	7

Maintain Health and Safety at Work Place	8

Identify Machines & Its Attachments	10

Crane Basic Operation before starting of work	12

Initial Operation of Crane	15

Perform operational task	17

Initial position after completion of work and final check	19

GPS Navigation System	21

Complete List of Tools and Equipment	22

# Title of Qualification

National Vocational Certificate in Heavy Machine Operator “Wheel Loader and Excavator Operator” Level-2

# Introduction

Crane Operators are having high demand across the country and abroad. Their services are required for everywhere where construction and other related work is done. Crane operation are for road and bridge construction excavating, dumping of material and much more. This is a good career opportunity for a reliable and responsible individual with a strong work ethic.

Crane is not only work on regular construction building jobs, but also on infrastructure projects (roads, bridges, and ports, otherwise called non-building construction). Crane is working for transporting material vertically from one place to another especially heavy objects which are not easy to transport vertical or material by picking through boom and place the material at proper place where it is necessary.

In order to build the capacity of technical and vocational training institutes in Pakistan through provision of demand driven competencies-based trainings in construction sector the NAVTTC, and TEVT Sector Support Program (TSSP) have joined hands together to develop qualifications for construction sector. These qualifications will not only build the capacity of existing workers of this sector but also support the youth to acquire skills best fit for this sector. The benefits and impact of development of these qualifications will be on both demand and supply side.

Based upon this demand of industry these competency‐based qualifications for Heavy Machine Operator are developed under National Vocational Qualification Framework (NVQF) (Level 1 to 4). The qualifications mainly cover competencies along with related knowledge and professional skills which are essential for getting a job or self-employed.

The National Vocational & Technical Training Commission (NAVTTC) has approved the Qualification Development Committee (QDC). The QDC consists experts from the relevant industries from different geographical locations across Pakistan and academicians who were consulted during the development process to ensure input and ownership of all the stakeholders. The National Competency Standards could be used as a referral document for the development of curricula to be used by training institutions.

Crane operator curriculum will prepare students to efficiently operate crane efficiently and to perform basic preventive maintenance on most types of crane. Coursework includes Safety crane operation, Boom angle indicator and Boom operation lifting and lowering operation. Graduates of this program may find employment with state and local government agencies and private contractors engaged in highway, ,    mining sector and other construction activities.

# Purpose of the Qualification

The purpose of the training is to provide skilled manpower to improve the existing capacity of construction sector. This training will provide the requisite skills to the trainees to operate Heavy Machines specially crane. It will enable the participants to meet the challenges in the field of construction industry. Further, to improve the skill level of the Operators and prepare them for the construction industry to meet the market requirement either their competitor nationally and internationally.

The core purpose of this qualification is to produce employable in crane who could operate Heavy Machines according to national and international standards. In addition, this qualification will prepare unemployable youth to employee in construction and different other relevant sectors.

# Main Objectives of the Qualification

The crane level II consists of theoretical and practical details required to operate crane in construction industries. The main objectives of the qualification are as follows:

- Maintain Safety

- Identify Machine & its Attachments

- Develop Basic Literacy & Numeracy Skill

- Maintain crane before operation

- Operate crane after operation

- Park Machines at designated specific area

- Develop basic communication Skill.

- Apply Occupational Health & Safety.

# Date of Validation

The level II of National vocational qualification on crane operator has been validated by the Qualifications Development Committee (QDC) members on August 05 - 09, 2024 in Islamabad.

# Codes of Qualifications

The International Standard Classification of Education (ISCED) is a framework for assembling, compiling and analyzing cross-nationally comparable statistics on education and training. ISCED codes for these qualifications are assigned as follows:

# Members of Qualification Development Committee

The following members participated in the Qualification development and validation process.

| Name | Designation | Organization |
| --- | --- | --- |
| Muzammal Hussain | DACUM Expert | Assistant Director P-TEVTA, R.Y.Khan |
| Furqan Mustafa | Solution Expert Heavy Machinery | Heavy Mechanical Simulations Solution Expert SoftMount/Connect Sonsultants Islamabad |
| M. Wameq Ayaz | Member | HOD PVTC, Rawalpindi |
| Muhammad Zulqarnain | Member | Service Manager, Morgan Technology, Lahore |
| Abdul Haseeb Imtiaz | Member | Service Engineer, Jaffar Brothers Ltd, Bahawalpur |
| Engr. Bakhtyar Khan | Member | GTTC,Peshawar |
| Engr. Muhammad Saqib | Member | Lecturer, KP-TEVTA |
| Malik M.Faraz Haider | Member | CTTI, Islamabad |
| Engr. Shahid Khan | Member | CTTI, Islamabad |
| Hamza Nadeem | Facilitator | SS&C Wing, NAVTTC HQ |
| Ms. Rida Ramzan | Facilitator | SS&C Wing, NAVTTC HQ |

# Entry Requirements

The entry for National Vocational Certificate level II, in (Construction Sector) “Crane  Operator” are given below:

| Title | Entry requirements |
| --- | --- |
| National Vocational Certificate level 2, in (Construction Sector) “Crane Operator” | Entry for assessment for this qualification is open. However, entry into formal training institute for this qualification is person having National Vocational Certificate level 1, in (Construction Sector) “Helper” or Middle. |

# Summary of Competency Standards

| Sr. No. | Competency Standards | Category | Credits |
| --- | --- | --- | --- |
| 1 | Maintain Health and Safety at Work Place | Functional | 05 |
| 2 | Identify Machines & Its Attachments | Technical | 15 |
| 3 | Crane Basic Operation before starting of work | Technical |  |
| 4 | Initial Operation of Crane | Technical |  |
| 5 | Perform operational task | Technical |  |
| 6 | Initial position after completion of work and final check | Technical |  |
| 7 | GPS Navigation System | Technical |  |
| 8 | Language Skills Arabic/English/Chinese/German/Korean/Japanese *Note: The institute may choose any one of the language as per the availability of language instructor. | Generic | 10 |
| TOTAL |  |  | 30 |

# Maintain Health and Safety at Work Place

Overview: This unit describes the performance outcomes, skills and knowledge required to apply general work health and safety requirements in the workplace. Communicate  about health and safety assess at work place. It describes generic workplace health and safety responsibilities applicable to employees without managerial or supervisory responsibilities.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Work safely at work place | P1. Identify employee relevant organizational safety policies and procedures P2. Categorize and maintain tools and equipment as per work requirements P3. Follow established safety procedures during work activities P4. Report work-related incidents and accidents to supervisor P5. Take necessary measures to minimizing risks |
| CU2. Communicate work health and safety (WHS) assess at work place | P1. Report work health and safety issues with supervisor. P2. Contribute workplace OHS Meetings and other consultative processes for work health and safety management system at the workplace P3. Make suggestions for improving work health and safety procedure and policies |

| CU3. Minimize risks to personal safety at work place | P1. Identify situations and loops that may endanger the personal safety P2. Writing of report the incident regarding personal safety at work place P3. Eliminate and minimize work related hazards regarding personal safety P4. Identify and report damaged tools, machine parts and equipment for personal safety |
| --- | --- |

| CU4. Minimize risks to public safety | P1. Identify situations that may endanger the public safety P2. Document and report the incident at work sites P3. Eliminate workplace hazards at work sites P4. Notify Situation that may endanger situation for safety measures. |
| --- | --- |

Knowledge and Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

K1:   Identify the commonly used tools and equipment used at workplace.

K2:   Rights and responsibilities of employers and employees

K3:	Consultative processes to manage and monitor health and safety issues to enable active participation in maintaining a safe workplace

K4:	State potential hazards in the workplace

K5:	State commonly used hazard signs and safety symbols

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) to be competent in this competency standard:

Identify health and safety policies maintain work according to Standards and avoid any unwanted incident.

# Identify Machines & Its Attachments

Overview: This competency standard covers the skills and knowledge required to Identify crane and its sizes, identify components & Attachments, identify capacities & capabilities of Crane, Identify basic tools and supplies associated with crane and Maintain Inventory of tools and equipment.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Identify components & Attachments | P1. Identify different components of crane P2. Identify different attachments of relevant crane |
| CU2. Identify capacities & capabilities of crane | P1. Check capacity, capability of crane as per manufacturers specifications and recommendations P2. Ensure proper load capacity and capability of crane |
| CU3. Identify basic tools and supplies associated with Crane | P1. Check standard tools supplied with crane P2. Check spares/consumable materials of crane P3. Adopt manufacturers specifications for tools and supplies associated with machine |
| CU4. Maintain Inventory of tools and equipment. | P1. Check number of tools and equipment as per Logbook P2. Report for faulty tools and equipment P3. Generate demand for defective tools and equipment P4. Maintain record of all tools and equipment |

Knowledge & Understanding

The candidate must possess underpinning knowledge and understanding required to carry out tasks covered in this competency standard. Therefore, he/she must be able to:

K1:	Describe about Crane & Industry functionality.

K2:	Describe Crane Capabilities, Advantages & Limitations.

K3:	Describe Crane Attachments and their Purpose.

K4:	Describe basic Tools, Supplies & Lubricants associated with crane

K5:     Describe Inventory management procedure of tools and equipment.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence in order to be competent in this competency standard:

- Identify crane and its related Attachments

- Identify Capability and Capacity of Crane

- Ensure Capability of attachments as per standards

Machines and Its Attachments

Wheel Loader Attachments

  - Coupler

  - Hook

  - Grapples

  - Magnet

  - Jib

  - Boom Poles(Lifting Pole)

# Crane Basic Operation before starting of work

Overview: This competency standard covers the skills and knowledge required to Inspect and service Lubrication, Electrical, Hydraulic, Cooling, Fuel, steering and braking system of crane. and supporting pneumatic/ Hydraulic (Air-filled) system

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Inspect crane and service lubrication system | P1. Identify components to be inspected P2. Check low oil levels, dirty filler cap P3. Select appropriate tools P4. Identify and report leakages if any |
| CU2. Inspect and service electrical system | P1. Identify components to be inspected P2. Identify parts of crane for service needs, defects and hazardous conditions through visual/physical inspection P3. Select appropriate tools for rectification of minor defects in crane P4. Check water level of battery in crane and replace if required |
| CU3. Inspect and service hydraulic system | P1. Identify components to be inspected P2. Identify service needs, defects and hazardous conditions through visual/physical inspection P3. Identify and report leakages and noise of the hydraulic system in crane P4. Check hydraulic oil levels of Crane leavers P5. Replace hoses/pipes if damaged |
| CU4. Inspect and service cooling system | P1. Adopt appropriate safety measures. P2. Ensure unobstructed airflow through radiator core P3. Locate components to be inspected P4. Check and Adjust coolant level P5. Replace belts and hoses if necessary |
| CU5. Inspect and service air intake system | P1. Locate components to be inspected P2. Check air service indicators P3. Select appropriate tools P4. Clean primary air filter. P5. Replace intake hoses and clamps if required |

| CU6. Inspect and service fuel system |  | P1. Locate and Identify components to be inspected P2. Identify and read fuel gauges and level indicator in crane P3. Identify service needs, defects and hazardous conditions through visual/physical inspection P4. Perform basic maintenance such as cleaning of fuel strainer P5. Report fuel leakage if any |
| --- | --- | --- |
| CU7. Inspect and service operator station/Cab |  | P1. Locate and identify controls inside operator station/Cab P2. Identify missing or defective components or controls P3. Clean front/rear wind screen, windows and mirrors P4. Adjust mirrors |
|  | P5. Replace broken mirror/frame P6. Adjust seat and seat belt P7. Check nobs of all lights and indicators |  |
| CU8. Inspect safety equipment | P1. Ensure safety equipment is securely mounted P2. Replace expired fire extinguisher P3. Ensure to wearing appropriate PPE |  |
| CU9.Inspect and service supporting pneumatic/Hydraulic system | P1. Locate components to be inspected P2. Select appropriate tools P3. Identify service needs, defects and hazardous conditions through visual/physical inspection P4. Perform basic maintenance, such as choked drain valves P5. Replace air lines if damaged |  |

Knowledge and Understanding

The candidate must possess underpinning knowledge and understanding required to carry out tasks covered in this competency standard. Therefore, he/she must be able to:

K1:	Describe the functions of the different mechanical systems of a crane, as well as their maintenance requirements.

K2:	Explain Inspection & Maintenance of crane, and their associated attachments

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Inspect and service braking system in Crane

- Inspect and service hydraulic system in crane

- Inspect and service lubrication system in crane

- Inspect and service cooling system in crane

Tools and Equipment

- Basic tools, such as grease gun

- Hammer

- Screwdrivers

- Pliers

- Self-locking pliers

- Adjustable wrench

- Assorted other wrenches

- Basic supplies, such as grease, oil, window cleaner, rags, ice scraper, whisk broom

- Chain, Cable, Wires

# Initial Operation of Crane

Overview: This competency standard covers the skills and knowledge required to start engine monitor warning systems, warm up engine, cycle equipment functions, comply with Scheduled Maintenance Requirements and Maintain Logbook.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Warm up engine | P1. Monitor instrument panel P2. Warm up engine according to manufacturer’s instructions |
| CU3. Check function of different systems | P1. Activate all functions, such as brakes, steering, lights, wipers and hydraulic functions P2. Identify problems with functions P3. Perform required service |
| CU4. Comply with Scheduled Maintenance Requirements | P1. Comply with safety requirements P2. Read indicators/warning signals and remove the problem P3. Maintain record and documentation P4. Perform scheduled maintenance |
| CU5. Check Load Capacity and Rigging | P1. Obtain and review the load specifications provided. P2. Apply Outriggers P3. Ensuring that rigging is properly connected and secure for the safe operation of cranes. |
| CU6. Check the crane positioning at work place. | P1. Evaluate the lifting operation P2. Evaluate the lifting operation at site for potential hazards, obstructions, and ground conditions. P3. Ensure the crane is positioned on stable ground. |
| CU7. Check crane boom angle and length before lifting. | P1. Ensure Boom angle Operation P2. Adjust the boom and length of a crane according to the weight of the load requires. |

Knowledge and Understanding

The candidate must possess underpinning knowledge and understanding required to carry out tasks covered in this competency standard. Therefore he/she must be able to:

K1:	Describe warning systems of machines

K2:	Describe different types of machine warning features

K3:	Describe procedure of engine warming up

K4:	Describe cycle equipment functions

K5:	Enlist Techniques of Scheduled Maintenance

K6:	Describe procedure to maintain Logbook

K7: 	Describe boom angle operation

K8: 	Enlist about crane operation

K9:	Enlist about crane stability on ground

K10: 	Enlist about boom operation and function

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this  competency standard:

- Start engine monitor warning systems

- Warm up engine

- Boom operation

- Out Riggers function

- Stability of crane

- Maintenance

Tools and Equipment

- Utility documentation

- Logbooks

- Service Manuals

- Operational Manuals

# Perform operational task

Overview: Lifting weight with a crane involves using precise controls to raise the load safely while ensuring stability and proper rigging, safe and efficient lifting and positioning require following load limits and using appropriate rigging techniques to prevent accidents, uplifting is the process of carefully lowering the load back to its original position while maintaining control, and unrigging involves methodically detaching and removing the rigging equipment used to secure the load.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Perform process of lift the weight. | P1. Perform Slowly lift the load P2. Operate the crane controls to raise the load smoothly and steadily. P3. Monitor and adjust the load’s position to ensure it is lifted to the correct height and aligned properly. |
| CU2. Perform safe and efficient lifting and positioning of loads. | P1. Verify that the load is securely rigged and balanced before lifting. P2. Use proper crane controls and techniques to position the load accurately and avoid overloading. P3. Continuously monitor the load's movement and surroundings to ensure stability and prevent hazards. |
| CU3. Perform un lifting the weight. | P1. Gradually lower the load using controlled crane movements, ensuring it remains stable and level throughout the descent. P2. Continuously monitor the load and adjust the crane controls to prevent sudden shifts or sways. P3. Carefully set the load down on its designated spot, making sure it is properly secured and stable before disengaging the crane. |
| CU4. Perform unrigging process. | P1. Ensure the load is securely positioned and stable before beginning unrigging. P2. Carefully detach and remove all rigging equipment, such as slings and hooks, from the load. P3. Inspect the rigging equipment for damage and store it properly for future use. |

Knowledge and Understanding

The candidate must possess underpinning knowledge and understanding Knowledge and understanding of performing crane operations involve mastering control mechanisms, rigging procedures, and safety protocols to efficiently and safely lift, move, and position loads:

K1:	Describe the key control mechanisms a crane operator must master for efficient operations

K2:	Describe rigging procedures and safety protocols be applied to ensure safe crane operations

K3:			 Describe safety protocols must be followed to prevent accidents during crane operations

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in  this      competency standard:

- Start engine monitor warning systems

- Warm up engine

- Boom operation

- Out Riggers function

- Stability of crane

- Maintenance

Tools and Equipment

- Utility documentation

- Logbooks

- Service Manuals

- Operational Manuals

# Initial position after completion of work and final check

Overview: After completing work, lower the crane boom, stow the hook, and retract stabilizers; then, check for damage, ensure controls are neutral, and verify all systems and components are functioning properly.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1 Define Lower the Boom and Load. | P1. Perform Slowly lift the load P2. Operate the crane controls to raise the load smoothly and steadily. P3. Monitor and adjust the load’s position to ensure it is lifted to the correct height and aligned properly. |
| CU2 Describe Secure the load and rig out. | P1. Ensure the load is properly secured and positioned before unrigging, confirming it is stable and won’t shift or fall. P2. Carefully remove all rigging gear from the load, ensuring that all components are safely detached and that the load remains stable. P3. Lower the crane’s boom, retract stabilizers, and stow the hook and other attachments, ensuring the crane is properly secured and stored for future use. |
| CU3 Describe Position the Crane | P1. Bring the crane boom to its lowest position and retract any attachments, such as the hook or bucket, to ensure they are safely stored. P2. Engage the parking brake, ensure the stabilizers are fully retracted, and verify that the crane is on stable, level ground with all controls set to neutral. |
| CU4 Define Engage Parking Brake and Neutralize Controls | P1. Firmly activate the crane’s parking brake to prevent any movement or rolling while the crane is stationary. P2. Move all crane controls, including levers and joysticks, to the neutral position to ensure that no unintended movements occur. P3. Check that the parking brake is securely engaged and that all controls are in the neutral position before leaving the crane unattended. |
| CU5 Describe Lower Outriggers | P1. Use the crane’s control panel to extend the outriggers outward, ensuring they are deployed evenly and fully to provide a stable base. P2. Verify that the outriggers are properly set on stable ground and that the crane is level before proceeding with any additional tasks. |
| CU6 Describe Inspect and Clean Equipment | P1. Thoroughly examine the equipment for any signs of damage, wear, or malfunction, including checking for cracks, rust, or loose components. P2. Remove any dirt, debris, or residues from the equipment’s surfaces, including the boom, hook, and controls, using appropriate cleaning materials and techniques. P3. Inspect and top up essential fluids, such as hydraulic oil and coolant, ensuring that all levels are within recommended ranges for optimal operation. |
| CU7 Describe Store Equipment Properly | P1. Position the equipment in a designated storage area, ensuring it is stable and protected from environmental factors, such as weather or contaminants. P2. Apply protective covers to prevent dust and moisture accumulation, and lock all access points or controls to ensure security and prevent unauthorized use. |
| CU8 Describe Document and Report and safety | P1. Record key information about the completed task, including equipment usage, any issues encountered, and maintenance performed, in a log or report for future reference. P2. Immediately report any safety concerns or hazards observed during the task to the relevant authorities or safety officers to ensure they are addressed promptly and prevent potential risks. |

Knowledge and Understanding

The candidate must  ensuring the crane is correctly positioned with the boom lowered and stabilizers retracted, and conducting a thorough inspection to confirm all controls are neutral, safety systems are operational, and the crane is securely parked.

K1:	Describe crane be positioned and configured after completing a task to ensure it is secure.

K2:	Describe steps should be taken to ensure all crane controls are set to neutral before leaving the equipment.

K3:	 Describe aspects of the safety systems and crane parking should be inspected during the final check.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in  this      competency standard:

- Start engine monitor warning systems

- Warm up engine

- Boom operation

- Out Riggers function

- Stability of crane

- Maintenance

# GPS Navigation System

Overview: This competency standard covers the skills and knowledge required to operate GPS Navigation System.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Identify GPS Navigation System | - Improved route planning and efficiency - Reduced driver workload and fatigue - Enhanced situational awareness, especially on unfamiliar routes - Real-time traffic updates and alternative route suggestions - Location tracking for fleet management purposes |
| CU2. Identify Types of GPS Navigation System | - In-dash navigation systems integrated into the vehicle's dashboard - Portable navigation devices with standalone displays - Smartphone navigation apps |
| CU3. Operate a GPS Navigation System Safely | - Enter destination information, checking route details, and identifying potential hazards. - Follow voice prompts or on-screen instructions cautiously, avoiding distractions, and maintaining focus on the road - Recognize that GPS systems can provide accurate information or routes |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

|  | GPS Navigation Systems |
| --- | --- |
|  | Types of GPS Navigation Systems |
|  | GPS Navigation System Operation |
|  | Importance of Safe Operation |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Demonstrate understanding of GPS satellites, receivers, and maps.

- Identify various types of GPS devices, including in-dash systems, portable devices, and smartphone apps.

- Explain how GPS improves route planning, reduces driver workload, enhances situational awareness, and provides real-time traffic updates.

- Describe the characteristics and features of in-dash, portable, and smartphone navigation systems.

- Demonstrate proficiency in entering destinations, interpreting route details, and identifying potential hazards.

Demonstrate safe driving behaviors, such as maintaining a safe following distance, obeying traffic laws, and avoiding distractions.

# Complete List of Tools and Equipment

List of Machines and Tools

| Sr. # | Description |
| --- | --- |
| 1. | Steel-toed footwear |
| 2. | Safety Helmet |
| 3. | safety gloves |
| 4. | appropriate safety glasses |
| 5. | high visibility vest |
| 6. | hearing protection |
| 7. | breathing apparatus |
| 8. | De-electric boots and gloves for protection from electrical shock |
| 9. | fall protection, and other applicable PPE |
| 10. | Site emergency response plan |
| 11. | fire extinguishers |
| 12. | fire blankets |
| 13. | respirators, masks |
| 14. | fire hoses |
| 15. | first aid kits, stretchers, WHMIS book, and other related tools and gear |
| 16. | basic tools, such as grease gun |
| 17. | Hammer |
| 18. | Screwdrivers |
| 19. | Pliers |
| 20. | self-locking pliers |
| 21. | adjustable wrench |
| 22. | assorted other wrenches, measuring tape(100m) |
| 23. | Basic supplies, such as grease, oil, window cleaner, rags, ice scraper, whisk broom. |
| 24. | Color-code cards, utility documentation. Logbooks Service Manuals, OHS Regulation, |

| - List of consumable items required(for five candidates) |  |  |
| --- | --- | --- |
| S. No | Items | Quantity |
| 1 | Shilling | 2 |
| 2 | Belt | 2 |
| 7 | Helmet | 6 |
| 8 | First Aid box | 5 |
| 9 | Tool Box | 5 |
| 10 | Air Pressure gages | 5 |
| 11 | Jack Hammer | 1 |
| 12 | Barrication tape | 2 roll |
| 13 | Wheel wages | 10 |
| 14 | Glasses | 5 |
| Mobile Crane |  |  |
| 1 | Fuel | 300 ltr |
| 2 | Engine Oil | 20 ltr |
| 3 | Hydraulic oil | 350 ltr |
| 4 | Transmission oil | 25 ltr |
| 5 | Coolant | - |
| 6 | Gear Oil | 5ltr |
| 7 | Tandem Oil | - |
| 8 | Swing case Oil | - |
| 9 | Axel Oil | - |