Instruction Sheet for the Candidate

| Qualification | Machine Shop Supervisor |
| --- | --- |
| Competency Standard | Perform Cleaning, Oiling and Centering the Job |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within time frame (for practical demonstration & assessment): - Perform Cleaning and Oiling of the Lathe Machine - Perform Centering the Job By Tool Method - Perform Centering The Job Held In a Four Jaw Chuck of Face Plate |
| Time: 04 Hrs. | During a practical assessment, under observation by an assessor, you are required to |
| Minimum Evidence Required | Perform Cleaning and Oiling of the Lathe Machine - clean spindle threads properly - Properly clean lead screw of machine - Clean oil cap and oil plug using cotton cloth - Properly bed ways, clean cross slide and compound slide - Clean spindle bore and quill of tail stock - Properly lubricate oil caps of machine - Oil spindle threads - Use prescribed grade of oil for oiling and lubrication Perform Centering the Job By Tool Method - Prepare a M.S Round Bar as per drawing - Mount four jaw chuck on spindle of lathe machine - Clamp work piece 100mm out of chuck from one end - Clamp cutting tool on tool post of lathe machine - Touch cutting tool on surface of job and rotate chuck with hand to identify out centering - Loosen the opposite side jaw, where the job is out centered - Tighten the chuck in the direction of marked line - Repeat that step if required to center the job Perform Centering The Job Held In A Four Jaw Chuck Of Face Plate - Prepare a M.S Round Bar as per drawing - Mount four jaw chuck on spindle of lathe machine - Clamp work piece 100mm out of chuck from one end - Mount surface gauge on bed ways of lathe machine - Mark lines on surface of job with help of surface gauge to identify out centering - Loosen the opposite side jaw, where the job is out centered - Tighten the chuck in the direction of marked line - Repeat that step if required to center the job |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | Machine Shop Supervisor |
| Competency Standard | Perform Cleaning, Oiling and Centering the Job |
| Purpose of Assessment | Formative Assessment |
| Assessment Task | - Perform Cleaning and Oiling of the Lathe Machine - Perform Centering the Job By Tool Method - Perform Centering The Job Held In A Four Jaw Chuck Of Face Plate |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - clean spindle threads properly |  |  |
| - Properly clean lead screw of machine |  |  |
| - Clean oil cap and oil plug using cotton cloth |  |  |
| - Properly bed ways, clean cross slide and compound slide |  |  |
| - Clean spindle bore and quill of tail stock |  |  |
| - Properly lubricate oil caps of machine |  |  |
| - Oil spindle threads |  |  |
| - Use prescribed grade of oil for oiling and lubrication |  |  |
| - Prepare a M.S Round Bar as per drawing |  |  |
| - Mount four jaw chuck on spindle of lathe machine |  |  |
| - Clamp work piece 100mm out of chuck from one end |  |  |
| - Clamp cutting tool on tool post of lathe machine |  |  |
| - Touch cutting tool on surface of job and rotate chuck with hand to identify out centering |  |  |
| - Loosen the opposite side jaw, where the job is out centered |  |  |
| - Tighten the chuck in the direction of marked line |  |  |
| - Repeat that step if required to center the job |  |  |
| - Prepare a M.S Round Bar as per drawing |  |  |
| - Mount four jaw chuck on spindle of lathe machine |  |  |
| - Clamp work piece 100mm out of chuck from one end |  |  |
| - Mount surface gauge on bed ways of lathe machine |  |  |
| - Mark lines on surface of job with help of surface gauge to identify out centering |  |  |
| - Loosen the opposite side jaw, where the job is out centered |  |  |
| - Tighten the chuck in the direction of marked line |  |  |
| 24.Repeat that step if required to center the job |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | Machine Shop Supervisor |
| --- | --- |
| Competency Standard | Perform Cleaning, Oiling and Centering the Job |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________ Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________________________________ Assessor’s code:____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | 1. Perform Cleaning and Oiling of the Lathe Machine 2. Perform Centering the Job By Tool Method 3. Perform Centering The Job Held In A Four Jaw Chuck Of Face Plate |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | clean spindle threads properly |  |  |  |  |  |
|  | Properly clean lead screw of machine |  |  |  |  |  |
|  | Clean oil cap and oil plug using cotton cloth |  |  |  |  |  |
|  | Properly bed ways, clean cross slide and compound slide |  |  |  |  |  |
|  | Clean spindle bore and quill of tail stock |  |  |  |  |  |
|  | Properly lubricate oil caps of machine |  |  |  |  |  |
|  | Oil spindle threads |  |  |  |  |  |
|  | Use prescribed grade of oil for oiling and lubrication |  |  |  |  |  |
|  | Prepare a M.S Round Bar as per drawing |  |  |  |  |  |
|  | Mount four jaw chuck on spindle of lathe machine |  |  |  |  |  |
|  | Clamp work piece 100mm out of chuck from one end |  |  |  |  |  |
|  | Clamp cutting tool on tool post of lathe machine |  |  |  |  |  |
|  | Touch cutting tool on surface of job and rotate chuck with hand to identify out centering |  |  |  |  |  |
|  | Loosen the opposite side jaw, where the job is out centered |  |  |  |  |  |
|  | Tighten the chuck in the direction of marked line |  |  |  |  |  |
|  | Repeat that step if required to center the job |  |  |  |  |  |
|  | Prepare a M.S Round Bar as per drawing |  |  |  |  |  |
|  | Mount four jaw chuck on spindle of lathe machine |  |  |  |  |  |
|  | Clamp work piece 100mm out of chuck from one end |  |  |  |  |  |
|  | Mount surface gauge on bed ways of lathe machine |  |  |  |  |  |
|  | Mark lines on surface of job with help of surface gauge to identify out centering |  |  |  |  |  |
|  | Loosen the opposite side jaw, where the job is out centered |  |  |  |  |  |
|  | Tighten the chuck in the direction of marked line |  |  |  |  |  |
|  | 24.Repeat that step if required to center the job |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | Machine Shop Supervisor |
| --- | --- |
| Competency Standard | Perform Cleaning, Oiling and Centering the Job |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name:______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature:___________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor:________________________________________________ Assessor’s code:____________________________________________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Define lubrication. |  |  |
|  | Discuss type of oil grades. |  |  |
|  | Define oiling. |  |  |
|  | Explain parts of lathe machine. |  |  |
|  | How to use three jaw universal chuck? |  |  |
|  | Describe face plate. |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature _________________