Instruction Sheet for the Candidate

| Qualification | Machine Shop Supervisor |
| --- | --- |
| Competency Standard | Perform Drilling, Step Turning, Knurling and Boring a Straight Hole on lathe |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within time frame (for practical demonstration & assessment): - Perform Drilling On Lathe - Perform Step Turning - Perform Knurling - Perform Boring A Straight Hole |
| Time: 04 Hrs. | During a practical assessment, under observation by an assessor, you are required to |
| Minimum Evidence Required | Perform Drilling On Lathe - Prepare a M.S Round Bar as per drawing - Mount universal chuck on spindle of lathe machine - Clamp work piece 25mm out of chuck from one end - Set drilling speed by considering material of job - Face the work piece on both sides - Check the angle point of drill - Perform centre drilling operation - Clamp the drill of 5mm dia and create pilot hole - Perform drilling operation as per drawing - Unclamp the job and debar the job - Perform cleaning operation on machine Perform Step Turning - Prepare a M.S Round Bar as per drawing - Mount universal chuck on spindle of lathe machine - Clamp work piece 25mm out of chuck from one end - Clamp right hand side turning and facing tool on tool post - Check centre height of both tool nose - Set spindle speed by considering job dia and material of job - Face the work piece on one side - Mount centre drill in tail stock spindle - Perform centre hole drill on one side of job - Face other side of job to get required length - Clamp job as per drawing and support with dead centre - Turn dia of 12mm on the length of 47mm - Turn dia of 17mm on the length of 47mm - Unclamp the job and change the side - Turn dia of 23mm on the length of 47mm - Perform square shoulder turn operation on each step - Unclamp the job and debar the job - Perform cleaning operation on machine Perform Knurling - Prepare a M.S Round Bar as per drawing - Mount universal chuck on spindle of lathe machine - Clamp work piece 25mm out of chuck from one end - Clamp knurling tool on tool post - Check centre height of knurling tool - Set spindle speed 75% lower than turning speed - Set knurling tool at 90* with job axis - Set carriage feed at 0.38 to 0.76mm/revolution - Touch knurling tool with job surface and feed 0.63mm towards job - Start the machine and take light cut - Stop the machine, inspect the job and adjust if required - Start the machine on automatic feed and complete the job Perform Boring A Straight Hole - Clamp turning and facing tools in tool post - Check center height of both tools - Hold the job in chuck - Face the job on both sides and achieve the length of 48mm - Cut chamfer of 3x45* on both ends of job - Drill a through hole of 10mm - Drill a through hole of 16mm and 25mm - Clamp boring bar in tool post - Move carriage with hand to check if boring bar is not touching the sides of job - Set feed of 0.8mm to 1.2mm along longitudinal way - Touch boring bit point with internal circumference of job - Set appropriate depth of cut and complete the job with automatic feed |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | Machine Shop Supervisor |
| Competency Standard | Perform Drilling, Step Turning, Knurling and Boring a Straight Hole on lathe |
| Purpose of Assessment | Formative Assessment |
| Assessment Task | - Perform Drilling On Lathe - Perform Step Turning - Perform Knurling - Perform Boring A Straight Hole |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Prepare a M.S Round Bar as per drawing |  |  |
| - Mount universal chuck on spindle of lathe machine |  |  |
| - Clamp work piece 25mm out of chuck from one end |  |  |
| - Set drilling speed by considering material of job |  |  |
| - Face the work piece on both sides |  |  |
| - Check the angle point of drill |  |  |
| - Perform centre drilling operation |  |  |
| - Clamp the drill of 5mm dia and create pilot hole |  |  |
| - Perform drilling operation as per drawing |  |  |
| - Unclamp the job and debar the job |  |  |
| - Perform Step Turning |  |  |
| - Prepare a M.S Round Bar as per drawing |  |  |
| - Mount universal chuck on spindle of lathe machine |  |  |
| - Clamp work piece 25mm out of chuck from one end |  |  |
| - Clamp right hand side turning and facing tool on tool post |  |  |
| - Check centre height of both tool nose |  |  |
| - Set spindle speed by considering job dia and material of job |  |  |
| - Face the work piece on one side |  |  |
| - Mount centre drill in tail stock spindle |  |  |
| - Perform centre hole drill on one side of job |  |  |
| - Face other side of job to get required length |  |  |
| - Clamp job as per drawing and support with dead centre |  |  |
| - Turn dia of 12mm on the length of 47mm |  |  |
| - Turn dia of 17mm on the length of 47mm |  |  |
| - Unclamp the job and change the side |  |  |
| - Turn dia of 23mm on the length of 47mm |  |  |
| - Perform square shoulder turn operation on each step |  |  |
| - Unclamp the job and debar the job |  |  |
| - Perform cleaning operation on machine |  |  |
| - Prepare a M.S Round Bar as per drawing |  |  |
| - Mount universal chuck on spindle of lathe machine |  |  |
| - Clamp work piece 25mm out of chuck from one end |  |  |
| - Clamp knurling tool on tool post |  |  |
| - Check centre height of knurling tool |  |  |
| - Set spindle speed 75% lower than turning speed |  |  |
| - Set knurling tool at 90* with job axis |  |  |
| - Set carriage feed at 0.38 to 0.76mm/revolution |  |  |
| - Touch knurling tool with job surface and feed 0.63mm towards job |  |  |
| - Start the machine and take light cut |  |  |
| - Stop the machine, inspect the job and adjust if required |  |  |
| - Start the machine on automatic feed and complete the job |  |  |
| - Clamp turning and facing tools in tool post |  |  |
| - Check center height of both tools |  |  |
| - Hold the job in chuck |  |  |
| - Face the job on both sides and achieve the length of 48mm |  |  |
| - Cut chamfer of 3x45* on both ends of job |  |  |
| - Drill a through hole of 10mm |  |  |
| - Drill a through hole of 16mm and 25mm |  |  |
| - Clamp boring bar in tool post |  |  |
| - Move carriage with hand to check if boring bar is not touching the sides of job |  |  |
| - Set feed of 0.8mm to 1.2mm along longitudinal way |  |  |
| - Touch boring bit point with internal circumference of job |  |  |
| - Set appropriate depth of cut and complete the job with automatic feed |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | Machine Shop Supervisor |
| --- | --- |
| Competency Standard | Perform Drilling, Step Turning, Knurling and Boring a Straight Hole on lathe |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________ Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________________________________ Assessor’s code:____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Perform Drilling On Lathe - Perform Step Turning - Perform Knurling - Perform Boring A Straight Hole |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Prepare a M.S Round Bar as per drawing |  |  |  |  |  |
|  | Mount universal chuck on spindle of lathe machine |  |  |  |  |  |
|  | Clamp work piece 25mm out of chuck from one end |  |  |  |  |  |
|  | Set drilling speed by considering material of job |  |  |  |  |  |
|  | Face the work piece on both sides |  |  |  |  |  |
|  | Check the angle point of drill |  |  |  |  |  |
|  | Perform centre drilling operation |  |  |  |  |  |
|  | Clamp the drill of 5mm dia and create pilot hole |  |  |  |  |  |
|  | Perform drilling operation as per drawing |  |  |  |  |  |
|  | Unclamp the job and debar the job |  |  |  |  |  |
|  | Perform Step Turning |  |  |  |  |  |
|  | Prepare a M.S Round Bar as per drawing |  |  |  |  |  |
|  | Mount universal chuck on spindle of lathe machine |  |  |  |  |  |
|  | Clamp work piece 25mm out of chuck from one end |  |  |  |  |  |
|  | Clamp right hand side turning and facing tool on tool post |  |  |  |  |  |
|  | Check centre height of both tool nose |  |  |  |  |  |
|  | Set spindle speed by considering job dia and material of job |  |  |  |  |  |
|  | Face the work piece on one side |  |  |  |  |  |
|  | Mount centre drill in tail stock spindle |  |  |  |  |  |
|  | Perform centre hole drill on one side of job |  |  |  |  |  |
|  | Face other side of job to get required length |  |  |  |  |  |
|  | Clamp job as per drawing and support with dead centre |  |  |  |  |  |
|  | Turn dia of 12mm on the length of 47mm |  |  |  |  |  |
|  | Turn dia of 17mm on the length of 47mm |  |  |  |  |  |
|  | Unclamp the job and change the side |  |  |  |  |  |
|  | Turn dia of 23mm on the length of 47mm |  |  |  |  |  |
|  | Perform square shoulder turn operation on each step |  |  |  |  |  |
|  | Unclamp the job and debar the job |  |  |  |  |  |
|  | Perform cleaning operation on machine |  |  |  |  |  |
|  | Prepare a M.S Round Bar as per drawing |  |  |  |  |  |
|  | Mount universal chuck on spindle of lathe machine |  |  |  |  |  |
|  | Clamp work piece 25mm out of chuck from one end |  |  |  |  |  |
|  | Clamp knurling tool on tool post |  |  |  |  |  |
|  | Check centre height of knurling tool |  |  |  |  |  |
|  | Set spindle speed 75% lower than turning speed |  |  |  |  |  |
|  | Set knurling tool at 90* with job axis |  |  |  |  |  |
|  | Set carriage feed at 0.38 to 0.76mm/revolution |  |  |  |  |  |
|  | Touch knurling tool with job surface and feed 0.63mm towards job |  |  |  |  |  |
|  | Start the machine and take light cut |  |  |  |  |  |
|  | Stop the machine, inspect the job and adjust if required |  |  |  |  |  |
|  | Start the machine on automatic feed and complete the job |  |  |  |  |  |
|  | Clamp turning and facing tools in tool post |  |  |  |  |  |
|  | Check center height of both tools |  |  |  |  |  |
|  | Hold the job in chuck |  |  |  |  |  |
|  | Face the job on both sides and achieve the length of 48mm |  |  |  |  |  |
|  | Cut chamfer of 3x45* on both ends of job |  |  |  |  |  |
|  | Drill a through hole of 10mm |  |  |  |  |  |
|  | Drill a through hole of 16mm and 25mm |  |  |  |  |  |
|  | Clamp boring bar in tool post |  |  |  |  |  |
|  | Move carriage with hand to check if boring bar is not touching the sides of job |  |  |  |  |  |
|  | Set feed of 0.8mm to 1.2mm along longitudinal way |  |  |  |  |  |
|  | Touch boring bit point with internal circumference of job |  |  |  |  |  |
|  | Set appropriate depth of cut and complete the job with automatic feed |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | Machine Shop Supervisor |
| --- | --- |
| Competency Standard | Perform Drilling, Step Turning, Knurling and Boring a Straight Hole on lathe |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name:______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature:___________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor:________________________________________________ Assessor’s code:____________________________________________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Define step turning. |  |  |
|  | Explain types of turning. |  |  |
|  | Explain types of knurling. |  |  |
|  | Explain purpose of knurling. |  |  |
|  | Define boring. |  |  |
|  | Explain types of boring. |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature _________________