Instruction Sheet for the Candidate

| Qualification | Machine Shop Supervisor |
| --- | --- |
| Competency Standard | Perform Step or Counter Boring, Reaming, Tool Grinding, Taper Turning and Cutting Metric Threads |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within time frame (for practical demonstration & assessment): - Perform Step Or Counter Boring - Perform Reaming - Perform Tool Grinding - Perform Taper Turning By Compound Rest Method - Perform Cutting Metric Threads On Lathe Machine |
| Time: 04 Hrs. | During a practical assessment, under observation by an assessor, you are required to |
| Minimum Evidence Required | Perform Step Or Counter Boring - Clamp half-length of job outside chuck - Clamp facing tool in tool post - Check center height of tool - Face both ends of job as per drawing - Centre drill one side of job - Drill through hole of dia 6mm - Drill through hole of 12mm - clamp boring bar on tool post - Move carriage with hand to check if boring bar is not touching the sides of job - Set feed of 0.8mm to 1.2mm along longitudinal way - Cut the bore of 16mm throughout hole - Cut the bore dia of 20mm on length of 24mm - Check bore size on both sides Perform Reaming - Prepare a M.S Round Bar as per drawing - Mount universal chuck on spindle of lathe machine - Clamp work piece 25mm out of chuck from one end - Set drilling speed by considering material of job - Face the work piece on both sides - Check the angle point of drill - Perform centre drilling operation - Clamp the drill of 5mm dia and create pilot hole - Perform drilling operation using drill of 11.8mm dia - Clamp 12mm dia reamer in drill chuck - Insert reamer in to work piece and use cutting fluid - Turn off the machine and slowly exert reamer Perform Tool Grinding - Dress the face of grinding wheel - Hold tool bit to grind appropriate cutting edge angle - Slide tool bit right to left on face of wheel - Grind cutting edge angle and end relief angle at same time - Check end relief angle - Produce minor radius on tool nose Perform Taper Turning By Compound Rest Method - . Clamp work piece 25mm out of chuck from one end and Clamp right hand side turning tool on tool post - Check centre height of tool nose - Set spindle speed by considering job dia and material of job - Face the work piece on both sides with help of cross slide - Perform centre hole drill on one side of job - Face other side of job to get required length - Clamp job as per drawing and support with dead centre - Turn whole length of job on 23mm dia - Set compound slide on angle of 1*-54’ - Set appropriate depth of cut - Control feed with compound slide knob and cut taper on job - Check the minor dia with help of Vernier calliper Perform Cutting Metric Threads On Lathe Machine - Clamp work piece 25mm out of chuck from one end and Clamp facing and turning tool on tool post - Check centre height of tool nose - Face the work piece on both sides with help of cross slide - Perform centre hole drill on one side of job - Clamp job as per drawing and support with dead centre - Turn whole length of job on 22mm dia - Set threading tool on 90* with axis of work piece using angle gauge - Touch threading tool with job surface and set cross feed collar to zero - Set gears on spindle and lead screw as per standard - Take appropriate cuts to complete threading - Measure the pitch of threading performed |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | Machine Shop Supervisor |
| Competency Standard | Perform Step or Counter Boring, Reaming, Tool Grinding, Taper Turning and Cutting Metric Threads |
| Purpose of Assessment | Formative Assessment |
| Assessment Task | - Perform Step Or Counter Boring - Perform Reaming - Perform Tool Grinding - Perform Taper Turning By Compound Rest Method - Perform Cutting Metric Threads On Lathe Machine |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Clamp half-length of job outside chuck |  |  |
| - Clamp facing tool in tool post |  |  |
| - Check center height of tool |  |  |
| - Face both ends of job as per drawing |  |  |
| - Centre drill one side of job |  |  |
| - Drill through hole of dia 6mm |  |  |
| - Drill through hole of 12mm |  |  |
| - clamp boring bar on tool post |  |  |
| - Move carriage with hand to check if boring bar is not touching the sides of job |  |  |
| - Set feed of 0.8mm to 1.2mm along longitudinal way |  |  |
| - Cut the bore of 16mm throughout hole |  |  |
| - Cut the bore dia of 20mm on length of 24mm |  |  |
| - Check bore size on both sides |  |  |
| - Prepare a M.S Round Bar as per drawing |  |  |
| - Mount universal chuck on spindle of lathe machine |  |  |
| - Clamp work piece 25mm out of chuck from one end |  |  |
| - Set drilling speed by considering material of job |  |  |
| - Face the work piece on both sides |  |  |
| - Check the angle point of drill |  |  |
| - Perform centre drilling operation |  |  |
| - Clamp the drill of 5mm dia and create pilot hole |  |  |
| - Perform drilling operation using drill of 11.8mm dia |  |  |
| - Clamp 12mm dia reamer in drill chuck |  |  |
| - Insert reamer in to work piece and use cutting fluid |  |  |
| - Turn off the machine and slowly exert reamer |  |  |
| - Dress the face of grinding wheel |  |  |
| - Hold tool bit to grind appropriate cutting edge angle |  |  |
| - Slide tool bit right to left on face of wheel |  |  |
| - Grind cutting edge angle and end relief angle at same time |  |  |
| - Check end relief angle |  |  |
| - Produce minor radius on tool nose |  |  |
| - . Clamp work piece 25mm out of chuck from one end and Clamp right hand side turning tool on tool post |  |  |
| - Check centre height of tool nose |  |  |
| - Set spindle speed by considering job dia and material of job |  |  |
| - Face the work piece on both sides with help of cross slide |  |  |
| - Perform centre hole drill on one side of job |  |  |
| - Face other side of job to get required length |  |  |
| - Clamp job as per drawing and support with dead centre |  |  |
| - Turn whole length of job on 23mm dia |  |  |
| - Set compound slide on angle of 1*-54’ |  |  |
| - Set appropriate depth of cut |  |  |
| - Control feed with compound slide knob and cut taper on job |  |  |
| - Check the minor dia with help of Vernier calliper |  |  |
| - Clamp work piece 25mm out of chuck from one end and Clamp facing and turning tool on tool post |  |  |
| - Check centre height of tool nose |  |  |
| - Face the work piece on both sides with help of cross slide |  |  |
| - Perform centre hole drill on one side of job |  |  |
| - Clamp job as per drawing and support with dead centre |  |  |
| - Turn whole length of job on 22mm dia |  |  |
| - Set threading tool on 90* with axis of work piece using angle gauge |  |  |
| - Touch threading tool with job surface and set cross feed collar to zero |  |  |
| - Set gears on spindle and lead screw as per standard |  |  |
| - Take appropriate cuts to complete threading |  |  |
| - Measure the pitch of threading performed |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | Machine Shop Supervisor |
| --- | --- |
| Competency Standard | Perform Step or Counter Boring, Reaming, Tool Grinding, Taper Turning and Cutting Metric Threads |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________ Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________________________________ Assessor’s code:____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Perform Step Or Counter Boring - Perform Reaming - Perform Tool Grinding - Perform Taper Turning By Compound Rest Method - Perform Cutting Metric Threads On Lathe Machine |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Clamp half-length of job outside chuck |  |  |  |  |  |
|  | Clamp facing tool in tool post |  |  |  |  |  |
|  | Check center height of tool |  |  |  |  |  |
|  | Face both ends of job as per drawing |  |  |  |  |  |
|  | Centre drill one side of job |  |  |  |  |  |
|  | Drill through hole of dia 6mm |  |  |  |  |  |
|  | Drill through hole of 12mm |  |  |  |  |  |
|  | clamp boring bar on tool post |  |  |  |  |  |
|  | Move carriage with hand to check if boring bar is not touching the sides of job |  |  |  |  |  |
|  | Set feed of 0.8mm to 1.2mm along longitudinal way |  |  |  |  |  |
|  | Cut the bore of 16mm throughout hole |  |  |  |  |  |
|  | Cut the bore dia of 20mm on length of 24mm |  |  |  |  |  |
|  | Check bore size on both sides |  |  |  |  |  |
|  | Prepare a M.S Round Bar as per drawing |  |  |  |  |  |
|  | Mount universal chuck on spindle of lathe machine |  |  |  |  |  |
|  | Clamp work piece 25mm out of chuck from one end |  |  |  |  |  |
|  | Set drilling speed by considering material of job |  |  |  |  |  |
|  | Face the work piece on both sides |  |  |  |  |  |
|  | Check the angle point of drill |  |  |  |  |  |
|  | Perform centre drilling operation |  |  |  |  |  |
|  | Clamp the drill of 5mm dia and create pilot hole |  |  |  |  |  |
|  | Perform drilling operation using drill of 11.8mm dia |  |  |  |  |  |
|  | Clamp 12mm dia reamer in drill chuck |  |  |  |  |  |
|  | Insert reamer in to work piece and use cutting fluid |  |  |  |  |  |
|  | Turn off the machine and slowly exert reamer |  |  |  |  |  |
|  | Dress the face of grinding wheel |  |  |  |  |  |
|  | Hold tool bit to grind appropriate cutting edge angle |  |  |  |  |  |
|  | Slide tool bit right to left on face of wheel |  |  |  |  |  |
|  | Grind cutting edge angle and end relief angle at same time |  |  |  |  |  |
|  | Check end relief angle |  |  |  |  |  |
|  | Produce minor radius on tool nose |  |  |  |  |  |
|  | . Clamp work piece 25mm out of chuck from one end and Clamp right hand side turning tool on tool post |  |  |  |  |  |
|  | Check centre height of tool nose |  |  |  |  |  |
|  | Set spindle speed by considering job dia and material of job |  |  |  |  |  |
|  | Face the work piece on both sides with help of cross slide |  |  |  |  |  |
|  | Perform centre hole drill on one side of job |  |  |  |  |  |
|  | Face other side of job to get required length |  |  |  |  |  |
|  | Clamp job as per drawing and support with dead centre |  |  |  |  |  |
|  | Turn whole length of job on 23mm dia |  |  |  |  |  |
|  | Set compound slide on angle of 1*-54’ |  |  |  |  |  |
|  | Set appropriate depth of cut |  |  |  |  |  |
|  | Control feed with compound slide knob and cut taper on job |  |  |  |  |  |
|  | Check the minor dia with help of Vernier calliper |  |  |  |  |  |
|  | Clamp work piece 25mm out of chuck from one end and Clamp facing and turning tool on tool post |  |  |  |  |  |
|  | Check centre height of tool nose |  |  |  |  |  |
|  | Face the work piece on both sides with help of cross slide |  |  |  |  |  |
|  | Perform centre hole drill on one side of job |  |  |  |  |  |
|  | Clamp job as per drawing and support with dead centre |  |  |  |  |  |
|  | Turn whole length of job on 22mm dia |  |  |  |  |  |
|  | Set threading tool on 90* with axis of work piece using angle gauge |  |  |  |  |  |
|  | Touch threading tool with job surface and set cross feed collar to zero |  |  |  |  |  |
|  | Set gears on spindle and lead screw as per standard |  |  |  |  |  |
|  | Take appropriate cuts to complete threading |  |  |  |  |  |
|  | Measure the pitch of threading performed |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | Machine Shop Supervisor |
| --- | --- |
| Competency Standard | Perform Step or Counter Boring, Reaming, Tool Grinding, Taper Turning and Cutting Metric Threads |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name:______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature:___________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor:________________________________________________ Assessor’s code:____________________________________________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Define counter boring. |  |  |
|  | Explain types of counter boring. |  |  |
|  | Explain types of reamer. |  |  |
|  | Explain wheel dresser. |  |  |
|  | Define taper turning. |  |  |
|  | Explain types of Threads. |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature _________________