Assessors Judgment Guide

| Qualification | Machine Shop Supervisor |
| --- | --- |
| Competency Standard(s) | Perform Arc Welding |
| Candidate Details | Name:____________________________________________________ Registration/Roll Number: ________________________ Signature: ____________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________ Assessor’s code:___________________ Signature:__________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Make Arc - Make Bead - Make Open Square Butt Joint (MS Flat 5mm Thick) - Make V Groove Butt Joint - Make Lap Joint - Make Corner Joint Flat Position - Make Corner Joint Vertical Position - Make Spot Welding Practice(0.5mm MS Sheet) - Make Seam Welding Practice(0.5mm MS Sheet) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Take M.S Flat as per drawing |  |  |  |  |  |
|  | Straiten it with the help of hammer and anvil |  |  |  |  |  |
|  | Place the work piece on welding table |  |  |  |  |  |
|  | Hold the electrode in electrode holder |  |  |  |  |  |
|  | Set the current on welding machine as per standard |  |  |  |  |  |
|  | Touch the electrode with base metal |  |  |  |  |  |
|  | Ensure that pre heating is accomplished by moving short arc rapidly back and forward |  |  |  |  |  |
|  | Prepare the arc by touching the end of electrode with the base metal and withdrawing it to proper gap |  |  |  |  |  |
|  | Ensure the width and length of arc by watching the puddle |  |  |  |  |  |
|  | Set the current at different values. |  |  |  |  |  |
|  | Prepare the arc by touching the end of electrode with the base metal and withdrawing it to proper gap |  |  |  |  |  |
|  | Lean the electrode at 70* to the base metal and complete the bead to form an open square butt joint |  |  |  |  |  |
|  | Lean the electrode 20* downward and complete the joint by back hand technique |  |  |  |  |  |
|  | Grind the work pieces on pedestal grinder as per drawing |  |  |  |  |  |
|  | Complete the bead by back hand welding technique |  |  |  |  |  |
|  | Check the squares of joint with try square. |  |  |  |  |  |
|  | Mark parallel line with the help of scriber for positioning of overlap the plate |  |  |  |  |  |
|  | Grip the both part in Fixture |  |  |  |  |  |
|  | Switch on the spot welding machine and set the timer |  |  |  |  |  |
|  | Place the work piece on the lower electrode |  |  |  |  |  |
|  | Press down the pedal to make contact between the copper electrode to form spot weld for duration of 02 seconds |  |  |  |  |  |
|  | Switch the motor to drive the electrode wheels |  |  |  |  |  |
|  | Place the work piece on the lower electrode |  |  |  |  |  |
|  | Remove the slag with chipping hammer |  |  |  |  |  |
|  | Clean with wire brush |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | Machine Shop Supervisor |
| --- | --- |
| Competency Standard(s) | Perform Arc Welding |
| Candidate Details | Name:_______________________________________________________________________ Registration/Roll Number: __________________ Candidate Signature:__________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor:__________________________ Assessor’s code:_________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Define temperature of arc |  |  |
|  | Define conductivity |  |  |
|  | Define positions of electrode according to work |  |  |
|  | Describe Bead in Arc welding |  |  |
|  | Define Butt Joint |  |  |
|  | Explain importance of gap between electrode and base metal |  |  |

Feedback to the Candidate

Candidate’s Signature________________________ Assessor’s Signature _________________________

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | Machine Shop Supervisor |
| Competency Standards | Perform Arc Welding |
| Assessment Task | - Make Arc - Make Bead - Make Open Square Butt Joint (MS Flat 5mm Thick) - Make V Groove Butt Joint - Make Lap Joint - Make Corner Joint Flat Position - Make Corner Joint Vertical Position - Make Spot Welding Practice(0.5mm MS Sheet) - Make Seam Welding Practice(0.5mm MS Sheet) |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Take M.S Flat as per drawing |  |  |
| - Straiten it with the help of hammer and anvil |  |  |
| - Place the work piece on welding table |  |  |
| - Hold the electrode in electrode holder |  |  |
| - Set the current on welding machine as per standard |  |  |
| - Touch the electrode with base metal |  |  |
| - Ensure that pre heating is accomplished by moving short arc rapidly back and forward |  |  |
| - Prepare the arc by touching the end of electrode with the base metal and withdrawing it to proper gap |  |  |
| - Ensure the width and length of arc by watching the puddle |  |  |
| - Set the current at different values. |  |  |
| - Prepare the arc by touching the end of electrode with the base metal and withdrawing it to proper gap |  |  |
| - Lean the electrode at 70* to the base metal and complete the bead to form an open square butt joint |  |  |
| - Lean the electrode 20* downward and complete the joint by back hand technique |  |  |
| - Grind the work pieces on pedestal grinder as per drawing |  |  |
| - Complete the bead by back hand welding technique |  |  |
| - Check the squares of joint with try square. |  |  |
| - Mark parallel line with the help of scriber for positioning of overlap the plate |  |  |
| - Grip the both part in Fixture |  |  |
| - Switch on the spot welding machine and set the timer |  |  |
| - Place the work piece on the lower electrode |  |  |
| - Press down the pedal to make contact between the copper electrode to form spot weld for duration of 02 seconds |  |  |
| - Switch the motor to drive the electrode wheels |  |  |
| - Place the work piece on the lower electrode |  |  |
| - Remove the slag with chipping hammer |  |  |
| - Clean with wire brush |  |  |

Candidate’s Signature___________________		Assessor’s Signature_____________________

Date: ________________________________

Instruction Sheet for the Candidate

| Qualification | Machine Shop Supervisor |
| --- | --- |
| Competency Standard(s) | Perform Arc Welding |
| Purpose of Assessment | Formative |

| Candidate Details | Name________________________________________________________ Registration/Roll Number_____________________________________________ |
| --- | --- |
| Guidance for Candidate | To meet this standard you are required to complete the following within the given time frame (for practical demonstration & assessment): - Make Arc - Make Bead - Make Open Square Butt Joint (MS Flat 5mm Thick) - Make V Groove Butt Joint - Make Lap Joint - Make Corner Joint Flat Position - Make Corner Joint Vertical Position - Make Spot Welding Practice(0.5mm MS Sheet) - Make Seam Welding Practice(0.5mm MS Sheet) |
| Time: | During a practical assessment, under observation by an assessor, you are required to - Take M.S Flat as per drawing - Straiten it with the help of hammer and anvil - Place the work piece on welding table - Hold the electrode in electrode holder - Set the current on welding machine as per standard - Touch the electrode with base metal - Ensure that pre heating is accomplished by moving short arc rapidly back and forward - Prepare the arc by touching the end of electrode with the base metal and withdrawing it to proper gap - Ensure the width and length of arc by watching the puddle - Set the current at different values. - Prepare the arc by touching the end of electrode with the base metal and withdrawing it to proper gap - Lean the electrode at 70* to the base metal and complete the bead to form an open square butt joint - Lean the electrode 20* downward and complete the joint by back hand technique - Grind the work pieces on pedestal grinder as per drawing - Complete the bead by back hand welding technique - Check the squares of joint with try square. - Mark parallel line with the help of scriber for positioning of overlap the plate - Grip the both part in Fixture - Switch on the spot welding machine and set the timer - Place the work piece on the lower electrode - Press down the pedal to make contact between the copper electrode to form spot weld for duration of 02 seconds - Switch the motor to drive the electrode wheels - Place the work piece on the lower electrode - Remove the slag with chipping hammer - Clean with wire brush |
| Minimum Evidence Required |  |