Assessors Judgment Guide

| Qualification | Basic Electrical Engineering |
| --- | --- |
| Competency Standard(s) | Implement Electromagnet to See Various Effects & Verify Faraday’s Law |
| Candidate Details | Name:_______ _______________________________________ Registration/Roll Number: ________________________ Signature: ____________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________ Assessor’s code:___________________ Signature:__________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Implement Electromagnet - Implement circuit to determine the effect on current carrying conductor in magnetic field. - Determine the effect on conductor by varying the current with the help of rheostat. - Plot magnetic lines of forces of bar magnet. - Verify Faradays law by moving magnet in side coil. - Verify Faradays law by moving coil near the magnet field. - Verify EMF through induction. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Take iron nail (approximately 3 inches in length) as iron core and make 30 to 40 turns of thin coated copper wire to form a coil. |  |  |  |  |  |
|  | Connect dry cell battery with coil wound on the iron nail. |  |  |  |  |  |
|  | Bring iron nail near the iron pieces and tell the observation |  |  |  |  |  |
|  | Take copper rod (5cm in length) and connect wires across it. |  |  |  |  |  |
|  | Give DC supply to copper rod through rheostat. |  |  |  |  |  |
|  | Place current carrying copper rod inside the horse shoe magnet. |  |  |  |  |  |
|  | Reduce the rheostat resistance |  |  |  |  |  |
|  | Record the effect on copper rod. |  |  |  |  |  |
|  | Increase the rheostat resistance |  |  |  |  |  |
|  | Record the effect on copper rod. |  |  |  |  |  |
|  | Place a bar magnet on paper and outline its boundary with the help of lead pencil. |  |  |  |  |  |
|  | Place a compass needle at one side the magnet. |  |  |  |  |  |
|  | Mark points on paper where the compass needle stop. |  |  |  |  |  |
|  | Repeat the same procedure till compass reach at the other end of magnet. |  |  |  |  |  |
|  | Change the position of compass needle near the magnetic pole and repeat the procedure for P3 to P4. |  |  |  |  |  |
|  | Construct a coil with hollow iron cylinder (approximately 3 inches in length 1.5 inch in diameter.) and make 150 to 200 turns on it. |  |  |  |  |  |
|  | Connect Galvanometer with coil. |  |  |  |  |  |
|  | Move permanent magnet inside the coil fast and slow and record the effect on reading of Galvanometer. |  |  |  |  |  |
|  | Hold the magnet inside the coil and do not move, now record the effect on reading of Galvanometer. |  |  |  |  |  |
|  | Construct a coil with hollow iron cylinder (approximately 3 inches in length 1.5 inch in diameter.) and make 150 to 200 turns on it. |  |  |  |  |  |
|  | Connect Galvanometer with coil. |  |  |  |  |  |
|  | Fix permanent magnet and move the coil fast and slow on it and record the effect on reading of Galvanometer. |  |  |  |  |  |
|  | Hold the coil near the magnetic field do not move, now record the effect on reading of Galvanometer. |  |  |  |  |  |
|  | Take step down transformer and connect its secondary with a Galvanometer and primary winding with a DC battery through a rheostat. |  |  |  |  |  |
|  | Continuously vary the resistance on rheostat and observe the reading on the Galvanometer |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | Basic Electrical Engineering |
| --- | --- |
| Competency Standard(s) | Implement Electromagnet to See Various Effects & Verify Faraday’s Law |
| Candidate Details | Name:_______________________________________________________________________ Registration/Roll Number: __________________ Candidate Signature:__________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor:__________________________ Assessor’s code:_________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | What is magnet and magnetism? |  |  |
|  | What is electromagnet? |  |  |
|  | What is function of iron core |  |  |
|  | How we can make strong magnet? |  |  |
|  | How can we find the movement of current caring conductor which is placed in magnetic field? |  |  |
|  | What is Fleming’s left-hand rule? |  |  |
|  | Define magnetic lines of force. |  |  |
|  | Define magnetic field. |  |  |
|  | How magnetic lines of force travel with respect to each other |  |  |
|  | What is first law of Faraday? |  |  |
|  | What is second law of Faraday? |  |  |
|  | How can we find the direction of induced EMF? |  |  |
|  | What is the role of magnetic strength in Faraday’s Law? |  |  |
|  | What is the role of conductor’s length or turns in Faraday’s Law? |  |  |
|  | What is Lenz’s Law? |  |  |
|  | What is induction? |  |  |
|  | What is Faraday’s first law of Electro-Magnetic Induction? |  |  |
|  | What is meaning of coupled coils? |  |  |
|  | Why EMF induced in 2nd coil, when voltage is applied to 1st coil? |  |  |
|  | What is mutual induction? |  |  |
|  | What is core? |  |  |

Feedback to the Candidate

Candidate’s Signature________________________ Assessor’s Signature __________________

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | Basic Electrical Engineering |
| Competency Standards | Implement Electromagnet to See Various Effects & Verify Faraday’s Law |
| Assessment Task | - Implement Electromagnet |
|  | - Implement circuit to determine the effect on current carrying conductor in magnetic field. |
|  | - Determine the effect on conductor by varying the current with the help of rheostat. |
|  | - Plot magnetic lines of forces of bar magnet. |
|  | - Verify Faradays law by moving magnet in side coil. |
|  | - Verify Faradays law by moving coil near the magnet field. |
|  | - Verify EMF through induction. |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Take iron nail (approximately 3 inches in length) as iron core and make 30 to 40 turns of thin coated copper wire to form a coil. |  |  |
| - Connect dry cell battery with coil wound on the iron nail. |  |  |
| - Bring iron nail near the iron pieces and tell the observation |  |  |
| - Take copper rod (5cm in length) and connect wires across it. |  |  |
| - Give DC supply to copper rod through rheostat. |  |  |
| - Place current carrying copper rod inside the horse shoe magnet. |  |  |
| - Reduce the rheostat resistance |  |  |
| - Record the effect on copper rod. |  |  |
| - Increase the rheostat resistance |  |  |
| - Record the effect on copper rod. |  |  |
| - Place a bar magnet on paper and outline its boundary with the help of lead pencil. |  |  |
| - Place a compass needle at one side the magnet. |  |  |
| - Mark points on paper where the compass needle stop. |  |  |
| - Repeat the same procedure till compass reach at the other end of magnet. |  |  |
| - Change the position of compass needle near the magnetic pole and repeat the procedure for P3 to P4. |  |  |
| - Construct a coil with hollow iron cylinder (approximately 3 inches in length 1.5 inch in diameter.) and make 150 to 200 turns on it. |  |  |
| - Connect Galvanometer with coil. |  |  |
| - Move permanent magnet inside the coil fast and slow and record the effect on reading of Galvanometer. |  |  |
| - Hold the magnet inside the coil and do not move, now record the effect on reading of Galvanometer. |  |  |
| - Construct a coil with hollow iron cylinder (approximately 3 inches in length 1.5 inch in diameter.) and make 150 to 200 turns on it. |  |  |
| - Connect Galvanometer with coil. |  |  |
| - Fix permanent magnet and move the coil fast and slow on it and record the effect on reading of Galvanometer. |  |  |
| - Hold the coil near the magnetic field do not move, now record the effect on reading of Galvanometer. |  |  |
| - Take step down transformer and connect its secondary with a Galvanometer and primary winding with a DC battery through a rheostat. |  |  |
| - Continuously vary the resistance on rheostat and observe the reading on the Galvanometer |  |  |

Candidate’s Signature___________________		Assessor’s Signature_____________________

Date: ________________________________

Instruction Sheet for the Candidate

| Qualification | Basic Electrical Engineering |
| --- | --- |
| Competency Standard(s) | Implement Electromagnet to See Various Effects & Verify Faraday’s Law |
| Purpose of Assessment | Formative Assessment |

| Candidate Details | Name__________________________________________________ Registration/Roll Number_____________________________________________ |
| --- | --- |
| Guidance for Candidate | To meet this standard you are required to complete the following within the given time frame (for practical demonstration & assessment): - Implement Electromagnet - Implement circuit to determine the effect on current carrying conductor in magnetic field. - Determine the effect on conductor by varying the current with the help of rheostat. - Plot magnetic lines of forces of bar magnet. - Verify Faradays law by moving magnet in side coil. - Verify Faradays law by moving coil near the magnet field. - Verify EMF through induction. |
| Time: 3.5 hours | During a practical assessment, under observation by an assessor, you are required to - Implement Electromagnet - Take iron nail (approximately 3 inches in length) as iron core and make 30 to 40 turns of thin coated copper wire to form a coil. - Connect dry cell battery with coil wound on the iron nail. - Bring iron nail near the iron pieces and tell the observation - Implement circuit to determine the effect on current carrying conductor in magnetic field. - Take copper rod (5cm in length) and connect wires across it. - Give DC supply to copper rod through rheostat. - Place current carrying copper rod inside the horse shoe magnet. - Determine the effect on conductor by varying the current with the help of rheostat. - Reduce the rheostat resistance - Record the effect on copper rod. - Increase the rheostat resistance - Record the effect on copper rod. - Plot magnetic lines of forces of bar magnet. - Place a bar magnet on paper and outline its boundary with the help of lead pencil. - Place a compass needle at one side the magnet. - Mark points on paper where the compass needle stop. - Repeat the same procedure till compass reach at the other end of magnet. - Change the position of compass needle near the magnetic pole and repeat the procedure for P3 to P4. - Verify Faradays law by moving magnet in side coil. - Construct a coil with hollow iron cylinder (approximately 3 inches in length 1.5 inch in diameter.) and make 150 to 200 turns on it. - Connect Galvanometer with coil. - Move permanent magnet inside the coil fast and slow and record the effect on reading of Galvanometer. - Hold the magnet inside the coil and do not move, now record the effect on reading of Galvanometer. - Verify Faradays law by moving coil near the magnet field. - Construct a coil with hollow iron cylinder (approximately 3 inches in length 1.5 inch in diameter.) and make 150 to 200 turns on it. - Connect Galvanometer with coil. - Fix permanent magnet and move the coil fast and slow on it and record the effect on reading of Galvanometer. - Hold the coil near the magnetic field do not move, now record the effect on reading of Galvanometer. - Verify EMF through induction. - Take step down transformer and connect its secondary with a Galvanometer and primary winding with a DC battery through a rheostat. - Continuously vary the resistance on rheostat and observe the reading on the Galvanometer |
| Minimum Evidence Required |  |