Assessors Judgment Guide

| Qualification | Basic Electrical Engineering |
| --- | --- |
| Competency Standard(s) | Verify Ohm’s Law & Kirchhoff’s Law by Implementing Series/Parallel Circuits |
| Candidate Details | Name:_______ _______________________________________ Registration/Roll Number: ________________________ Signature: ____________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________ Assessor’s code:___________________ Signature:__________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Make series circuit and measure voltage and verify KVL - Make parallel circuit and measure current and verify KCL - Verify resistance of a resistor - Find unknown value of ohms law - Measure resistance of Rheostat - Measure resistance of voltmeter - Measure resistance of incandescent lamp |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Construct series circuit with different resistors on bread board and give supply voltage. |  |  |  |  |  |
|  | Measure the voltage across each resistor. |  |  |  |  |  |
|  | Note the value of voltage against each resistor. |  |  |  |  |  |
|  | Sum up the voltage of each resistor and verify is it equal to applied voltage. |  |  |  |  |  |
|  | Construct parallel circuit with different resistors on bread board. |  |  |  |  |  |
|  | Measure the current of each resistor as well as total current. |  |  |  |  |  |
|  | Note the value of current against each resistor. |  |  |  |  |  |
|  | Sum the reading of ampere meters and verify is it equal to the total current of circuit. |  |  |  |  |  |
|  | Select the 4 color band resistor. |  |  |  |  |  |
|  | Determine the value of resistor using color code. |  |  |  |  |  |
|  | Connect ohm meter across the resistor. |  |  |  |  |  |
|  | Note reading form ohm meter. |  |  |  |  |  |
|  | Compare both reading. |  |  |  |  |  |
|  | Construct the complex circuit using different resistors. |  |  |  |  |  |
|  | Find I, V with proper meter and determine R using ohms law |  |  |  |  |  |
|  | Find I, R with proper meter and determine V using ohm law |  |  |  |  |  |
|  | Find R, V with proper meter and determine I using ohm law |  |  |  |  |  |
|  | Take sliding Rheostat of 17 Ω and Ohmmeter. (0 to 20 or more than 17 Ω) which is available. |  |  |  |  |  |
|  | Adjust zero with shortning the ohmmeter leads. |  |  |  |  |  |
|  | Take reading of rheostat when slide is full placed at end,(whole resistance of rheostat)with the ohmmeter like digram shown above. |  |  |  |  |  |
|  | Note this result 17 Ω in your note book. |  |  |  |  |  |
|  | Repeat these reading when sliding exact in half of rheostat, and note this reading. |  |  |  |  |  |
|  | Take Voltmeter of 15, 30 V (D.C) or any range of voltmeter, which is available at your ab store. |  |  |  |  |  |
|  | Adjust zero with shortning the ohmmeter leads. |  |  |  |  |  |
|  | Take the reading of ohmmeter like shown above, and note it on your note book. |  |  |  |  |  |
|  | Take 40 W, lamp and 1 digital or analog ohmmeter. |  |  |  |  |  |
|  | Adjust zero with shortning the ohmmeter leads. |  |  |  |  |  |
|  | Connect the ohmmeter with its terminal and note its resistance, |  |  |  |  |  |
|  | Repeat this method for 60 & 100 W lamp |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | Basic Electrical Engineering |
| --- | --- |
| Competency Standard(s) | Verify Ohm’s Law & Kirchhoff’s Law by Implementing Series/Parallel Circuits |
| Candidate Details | Name:_______________________________________________________________________ Registration/Roll Number: __________________ Candidate Signature:__________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor:__________________________ Assessor’s code:_________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Describe characteristics of series circuit |  |  |
|  | Describe the use of volt meter |  |  |
|  | Describe characteristics of parallel circuit |  |  |
|  | Describe the use of ampere meter |  |  |
|  | Describe color code and its use |  |  |
|  | Describe the purpose of ohm meter |  |  |
|  | Relate current, voltage and resistor with one another |  |  |
|  | Describe the purpose of ohm meter |  |  |
|  | State Kirchhoff’s first law |  |  |
|  | State Kirchhoff’s second law |  |  |
|  | What is reheostat? |  |  |
|  | Why we used rheostat? |  |  |
|  | State law of resistance? |  |  |
|  | What is internal resistance of voltmeter |  |  |

Feedback to the Candidate

Candidate’s Signature________________________ Assessor’s Signature __________________

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | Basic Electrical Engineering |
| Competency Standards | Verify Ohm’s Law & Kirchhoff’s Law by Implementing Series/Parallel Circuits |
| Assessment Task | - Make series circuit and measure voltage and verify KVL |
|  | - Make parallel circuit and measure current and verify KCL |
|  | - Verify resistance of a resistor |
|  | - Find unknown value of ohms law |
|  | - Measure resistance of Rheostat |
|  | - Measure resistance of voltmeter |
|  | - Measure resistance of incandescent lamp |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Construct series circuit with different resistors on bread board and give supply voltage. |  |  |
| - Measure the voltage across each resistor. |  |  |
| - Note the value of voltage against each resistor. |  |  |
| - Sum up the voltage of each resistor and verify is it equal to applied voltage. |  |  |
| - Construct parallel circuit with different resistors on bread board. |  |  |
| - Measure the current of each resistor as well as total current. |  |  |
| - Note the value of current against each resistor. |  |  |
| - Sum the reading of ampere meters and verify is it equal to the total current of circuit. |  |  |
| - Select the 4 color band resistor. |  |  |
| - Determine the value of resistor using color code. |  |  |
| - Connect ohm meter across the resistor. |  |  |
| - Note reading form ohm meter. |  |  |
| - Compare both reading. |  |  |
| - Construct the complex circuit using different resistors. |  |  |
| - Find I, V with proper meter and determine R using ohms law |  |  |
| - Find I, R with proper meter and determine V using ohm law |  |  |
| - Find R, V with proper meter and determine I using ohm law |  |  |
| - Take sliding Rheostat of 17 Ω and Ohmmeter. (0 to 20 or more than 17 Ω) which is available. |  |  |
| - Adjust zero with shortning the ohmmeter leads. |  |  |
| - Take reading of rheostat when slide is full placed at end,(whole resistance of rheostat)with the ohmmeter like digram shown above. |  |  |
| - Note this result 17 Ω in your note book. |  |  |
| - Repeat these reading when sliding exact in half of rheostat, and note this reading. |  |  |
| - Take Voltmeter of 15, 30 V (D.C) or any range of voltmeter, which is available at your ab store. |  |  |
| - Adjust zero with shortning the ohmmeter leads. |  |  |
| - Take the reading of ohmmeter like shown above, and note it on your note book. |  |  |
| - Take 40 W, lamp and 1 digital or analog ohmmeter. |  |  |
| - Adjust zero with shortning the ohmmeter leads. |  |  |
| - Connect the ohmmeter with its terminal and note its resistance, |  |  |
| - Repeat this method for 60 & 100 W lamp |  |  |

Candidate’s Signature___________________		Assessor’s Signature_____________________

Date: ________________________________

Instruction Sheet for the Candidate

| Qualification | Basic Electrical Engineering |
| --- | --- |
| Competency Standard(s) | Verify Ohm’s Law & Kirchhoff’s Law by Implementing Series/Parallel Circuits |
| Purpose of Assessment | Formative Assessment |

| Candidate Details | Name__________________________________________________ Registration/Roll Number_____________________________________________ |
| --- | --- |
| Guidance for Candidate | To meet this standard you are required to complete the following within the given time frame (for practical demonstration & assessment): - Make series circuit and measure voltage and verify KVL - Make parallel circuit and measure current and verify KCL - Verify resistance of a resistor - Find unknown value of ohms law - Measure resistance of Rheostat - Measure resistance of voltmeter - Measure resistance of incandescent lamp |
| Time: 3 hours | During a practical assessment, under observation by an assessor, you are required to 1. Make series circuit and measure voltage and verify KVL - Construct series circuit with different resistors on bread board and give supply voltage. - Measure the voltage across each resistor. - Note the value of voltage against each resistor. - Sum the voltage of each resistor and verify is it equal to applied voltage. - Make parallel circuit and measure current and verify KCL - Construct parallel circuit with different resistors on bread board. - Measure the current of each resistor as well as total current. - Note the value of current against each resistor. - Sum the reading of ampere meters and verify, is it equal to the total current of circuit. - Verify resistance of a resistor - Select the 4 color band resistor. - Determine the value of resistor using color code. - Connect ohm meter across the resistor. - Note reading form ohm meter. - Compare both reading. - Find unknown value of ohms law - Construct the complex circuit using different resistors. - Find I, V with proper meter and determine R using ohms law - Find I, R with proper meter and determine V using ohm law - Find R, V with proper meter and determine I using ohm law - Measure resistance of Rheostat - Take sliding Rheostat of 17 Ω and Ohmmeter. (0 to 20 or more than 17 Ω) which is available. - Adjust zero with shortning the ohmmeter leads. - Take reading of rheostat when slide is full placed at end,(whole resistance of rheostat)with the ohmmeter like digram shown above. - Note this result 17 Ω in your note book. - Repeat these reading when sliding exact in half of rheostat, and note this reading. - Measure resistance of voltmeter - Take Voltmeter of 15, 30 V (D.C) or any range of voltmeter, which is available at your store. - Adjust zero with shortning the ohmmeter leads. - Take the reading of ohmmeter like shown above, and note it on your note book - Measure resistance of incandescent lamp - Take 40 W, lamp and 1 digital or analog ohmmeter. - Adjust zero with shortning the ohmmeter leads. - Connect the ohmmeter with its terminal and note its resistance, - Repeat this method for 60 & 100 W lamp |
| Minimum Evidence Required |  |