Instruction Sheet for the Candidate

| Qualification | Machine Shop Supervisor |
| --- | --- |
| Competency Standard | - Perform Cleaning, Oiling and Centering the Job - Perform Facing, Straight Turning, Centre Drilling - Perform Drilling, Step Turning, Knurling and Boring a Straight Hole On Lathe - Perform Step or Counter Boring, Reaming, Tool Grinding, Taper Turning and Cutting |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within time frame (for practical demonstration & assessment): - Perform Centering the job held In a Four Jaw Chuck of face plate, centre drilling, step turning, knurling and cutting metric threads on lathe machine |
| Time: 05 Hrs. | During a practical assessment, under observation by an assessor, you are required to |
| Minimum Evidence Required | - Prepare a M.S Round Bar as per drawing - Mount four jaw chuck on spindle of lathe machine - Clamp work piece 100mm out of chuck from one end - Mount surface gauge on bed ways of lathe machine - Mark lines on surface of job with help of surface gauge to identify out centering - Loosen the opposite side jaw, where the job is out centered - Tighten the chuck in the direction of marked line - Mount universal chuck on spindle of lathe machine - Clamp work piece 25mm out of chuck from one end - Clamp right hand side facing tool on tool post - Check centre height of tool nose - Set spindle speed by considering job dia and material of job - Face the work piece on both sides - Clamp centre drill in drilling chuck and mount it in tail stock - Perform centre drilling. - Clamp work piece 25mm out of chuck from one end - Clamp right hand side turning and facing tool on tool post - Check centre height of both tool nose - Set spindle speed by considering job dia and material of job - Face the work piece on one side - Mount centre drill in tail stock spindle - Perform centre hole drill on one side of job - Face other side of job to get required length - Clamp job as per drawing and support with dead centre - Turn dia of 12mm on the length of 47mm - Turn dia of 17mm on the length of 47mm - Unclamp the job and change the side - Turn dia of 23mm on the length of 47mm - Perform square shoulder turn operation on each step - Unclamp the job and debar the job - Clamp work piece 25mm out of chuck from one end - Clamp knurling tool on tool post - Check centre height of knurling tool - Set spindle speed 75% lower than turning speed - Set knurling tool at 90* with job axis - Set carriage feed at 0.38 to 0.76mm/revolution - Touch knurling tool with job surface and feed 0.63mm towards job - Start the machine and take light cut - Stop the machine, inspect the job and adjust if required - Start the machine on automatic feed and complete the job - Clamp work piece 25mm out of chuck from one end and Clamp facing and turning tool on tool post - Check centre height of tool nose - Face the work piece on both sides with help of cross slide - Perform centre hole drill on one side of job - Clamp job as per drawing and support with dead centre - Turn whole length of job on 22mm dia - Set threading tool on 90* with axis of work piece using angle gauge - Touch threading tool with job surface and set cross feed collar to zero - Set gears on spindle and lead screw as per standard - Take appropriate cuts to complete threading - Measure the pitch of threading performed |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | Machine Shop Supervisor |
| Competency Standard | 1. Perform Cleaning, Oiling and Centering the Job 2. Perform Facing, Straight Turning, Centre Drilling 3. Perform Drilling, Step Turning, Knurling and Boring a Straight Hole On Lathe 4. Perform Step or Counter Boring, Reaming, Tool Grinding, Taper Turning and Cutting |
| Purpose of Assessment | Summative Assessment |
| Assessment Task |  |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Prepare a M.S Round Bar as per drawing |  |  |
| - Mount four jaw chuck on spindle of lathe machine |  |  |
| - Clamp work piece 100mm out of chuck from one end |  |  |
| - Mount surface gauge on bed ways of lathe machine |  |  |
| - Mark lines on surface of job with help of surface gauge to identify out centering |  |  |
| - Loosen the opposite side jaw, where the job is out centered |  |  |
| - Tighten the chuck in the direction of marked line |  |  |
| - Mount universal chuck on spindle of lathe machine |  |  |
| - Clamp work piece 25mm out of chuck from one end |  |  |
| - Clamp right hand side facing tool on tool post |  |  |
| - Check centre height of tool nose |  |  |
| - Set spindle speed by considering job dia and material of job |  |  |
| - Face the work piece on both sides |  |  |
| - Clamp centre drill in drilling chuck and mount it in tail stock |  |  |
| - Perform centre drilling. |  |  |
| - Clamp work piece 25mm out of chuck from one end |  |  |
| - Clamp right hand side turning and facing tool on tool post |  |  |
| - Check centre height of both tool nose |  |  |
| - Set spindle speed by considering job dia and material of job |  |  |
| - Face the work piece on one side |  |  |
| - Mount centre drill in tail stock spindle |  |  |
| - Perform centre hole drill on one side of job |  |  |
| - Face other side of job to get required length |  |  |
| - Clamp job as per drawing and support with dead centre |  |  |
| - Turn dia of 12mm on the length of 47mm |  |  |
| - Turn dia of 17mm on the length of 47mm |  |  |
| - Unclamp the job and change the side |  |  |
| - Turn dia of 23mm on the length of 47mm |  |  |
| - Perform square shoulder turn operation on each step |  |  |
| - Unclamp the job and debar the job |  |  |
| - Clamp work piece 25mm out of chuck from one end |  |  |
| - Clamp knurling tool on tool post |  |  |
| - Check centre height of knurling tool |  |  |
| - Set spindle speed 75% lower than turning speed |  |  |
| - Set knurling tool at 90* with job axis |  |  |
| - Set carriage feed at 0.38 to 0.76mm/revolution |  |  |
| - Touch knurling tool with job surface and feed 0.63mm towards job |  |  |
| - Start the machine and take light cut |  |  |
| - Stop the machine, inspect the job and adjust if required |  |  |
| - Start the machine on automatic feed and complete the job |  |  |
| - Clamp work piece 25mm out of chuck from one end and Clamp facing and turning tool on tool post |  |  |
| - Check centre height of tool nose |  |  |
| - Face the work piece on both sides with help of cross slide |  |  |
| - Perform centre hole drill on one side of job |  |  |
| - Clamp job as per drawing and support with dead centre |  |  |
| - Turn whole length of job on 22mm dia |  |  |
| - Set threading tool on 90* with axis of work piece using angle gauge |  |  |
| - Touch threading tool with job surface and set cross feed collar to zero |  |  |
| - Set gears on spindle and lead screw as per standard |  |  |
| - Take appropriate cuts to complete threading |  |  |
| - Measure the pitch of threading performed |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | Machine Shop Supervisor |
| --- | --- |
| Competency Standard | 1. Perform Cleaning, Oiling and Centering the Job 2. Perform Facing, Straight Turning, Centre Drilling 3. Perform Drilling, Step Turning, Knurling and Boring a Straight Hole On Lathe 4. Perform Step or Counter Boring, Reaming, Tool Grinding, Taper Turning and Cutting |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________ Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________________________________ Assessor’s code:____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Prepare a M.S Round Bar as per drawing |  |  |  |  |  |
|  | Mount four jaw chuck on spindle of lathe machine |  |  |  |  |  |
|  | Clamp work piece 100mm out of chuck from one end |  |  |  |  |  |
|  | Mount surface gauge on bed ways of lathe machine |  |  |  |  |  |
|  | Mark lines on surface of job with help of surface gauge to identify out centering |  |  |  |  |  |
|  | Loosen the opposite side jaw, where the job is out centered |  |  |  |  |  |
|  | Tighten the chuck in the direction of marked line |  |  |  |  |  |
|  | Mount universal chuck on spindle of lathe machine |  |  |  |  |  |
|  | Clamp work piece 25mm out of chuck from one end |  |  |  |  |  |
|  | Clamp right hand side facing tool on tool post |  |  |  |  |  |
|  | Check centre height of tool nose |  |  |  |  |  |
|  | Set spindle speed by considering job dia and material of job |  |  |  |  |  |
|  | Face the work piece on both sides |  |  |  |  |  |
|  | Clamp centre drill in drilling chuck and mount it in tail stock |  |  |  |  |  |
|  | Perform centre drilling. |  |  |  |  |  |
|  | Clamp work piece 25mm out of chuck from one end |  |  |  |  |  |
|  | Clamp right hand side turning and facing tool on tool post |  |  |  |  |  |
|  | Check centre height of both tool nose |  |  |  |  |  |
|  | Set spindle speed by considering job dia and material of job |  |  |  |  |  |
|  | Face the work piece on one side |  |  |  |  |  |
|  | Mount centre drill in tail stock spindle |  |  |  |  |  |
|  | Perform centre hole drill on one side of job |  |  |  |  |  |
|  | Face other side of job to get required length |  |  |  |  |  |
|  | Clamp job as per drawing and support with dead centre |  |  |  |  |  |
|  | Turn dia of 12mm on the length of 47mm |  |  |  |  |  |
|  | Turn dia of 17mm on the length of 47mm |  |  |  |  |  |
|  | Unclamp the job and change the side |  |  |  |  |  |
|  | Turn dia of 23mm on the length of 47mm |  |  |  |  |  |
|  | Perform square shoulder turn operation on each step |  |  |  |  |  |
|  | Unclamp the job and debar the job |  |  |  |  |  |
|  | Clamp work piece 25mm out of chuck from one end |  |  |  |  |  |
|  | Clamp knurling tool on tool post |  |  |  |  |  |
|  | Check centre height of knurling tool |  |  |  |  |  |
|  | Set spindle speed 75% lower than turning speed |  |  |  |  |  |
|  | Set knurling tool at 90* with job axis |  |  |  |  |  |
|  | Set carriage feed at 0.38 to 0.76mm/revolution |  |  |  |  |  |
|  | Touch knurling tool with job surface and feed 0.63mm towards job |  |  |  |  |  |
|  | Start the machine and take light cut |  |  |  |  |  |
|  | Stop the machine, inspect the job and adjust if required |  |  |  |  |  |
|  | Start the machine on automatic feed and complete the job |  |  |  |  |  |
|  | Clamp work piece 25mm out of chuck from one end and Clamp facing and turning tool on tool post |  |  |  |  |  |
|  | Check centre height of tool nose |  |  |  |  |  |
|  | Face the work piece on both sides with help of cross slide |  |  |  |  |  |
|  | Perform centre hole drill on one side of job |  |  |  |  |  |
|  | Clamp job as per drawing and support with dead centre |  |  |  |  |  |
|  | Turn whole length of job on 22mm dia |  |  |  |  |  |
|  | Set threading tool on 90* with axis of work piece using angle gauge |  |  |  |  |  |
|  | Touch threading tool with job surface and set cross feed collar to zero |  |  |  |  |  |
|  | Set gears on spindle and lead screw as per standard |  |  |  |  |  |
|  | Take appropriate cuts to complete threading |  |  |  |  |  |
|  | Measure the pitch of threading performed |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | Machine Shop Supervisor |
| --- | --- |
| Competency Standard | 1. Perform Cleaning, Oiling and Centering the Job 2. Perform Facing, Straight Turning, Centre Drilling 3. Perform Drilling, Step Turning, Knurling and Boring a Straight Hole On Lathe 4. Perform Step or Counter Boring, Reaming, Tool Grinding, Taper Turning and Cutting |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name:______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature:___________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor:________________________________________________ Assessor’s code:____________________________________________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Explain parts of lathe machine. |  |  |
|  | Describe angles of turning tool. |  |  |
|  | Define step turning. |  |  |
|  | Explain types of knurling. |  |  |
|  | Define taper turning. |  |  |
|  | Explain types of Threads. |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature _________________