Instruction Sheet for the Candidate

| Qualification | AC Machines (Transformer)Level-4 |
| --- | --- |
| Competency Standard | - Verify Basic Laws of Electrical Machines - Operate Single Phase Transformer - Perform Tests On Single Phase Transformer - Operate Auto Transformer - Perform Tests On Auto Transformer - Operate Three Phase Transformer - Identify Vector Group of Three Phase Transformer - Repair/ Maintenance and Service of Transformer |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within time frame (for practical demonstration & assessment): - Calculate efficiency of three identical single phase transformers by direct method and connect them as per given vector group Yd11 and Yd1. |
| Time: 05 Hrs. | During a practical assessment, under observation by an assessor, you are required to |
| Minimum Evidence Required | - Select three identical single phase transformers. - Perform open circuit test on these transformers. - Perform short circuit test on these transformers. - Calculate iron losses and copper losses from above tests. - Calculate input powers and output powers. - Calculate efficiency of each transformer from there calculated powers. - Connect these transformers in star to delta connection. - Connect phase angle meter between primary and secondary. - Apply rated voltage to the primary side of transformer. - Measure the phase angle between primary and secondary with the help of phase angle meter (300 lag or 300 lead). |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | AC Machines (Transformer) |
| Competency Standard | 1. Verify Basic Laws of Electrical Machines 2. Operate Single Phase Transformer 3. Perform Tests On Single Phase Transformer 4. Operate Auto Transformer 5. Perform Tests On Auto Transformer 6. Operate Three Phase Transformer 7. Identify Vector Group of Three Phase Transformer 8. Repair/ Maintenance and Service of Transformer |
| Purpose of Assessment | Summative Assessment |
| Assessment Task |  |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Select three identical single phase transformers. |  |  |
| - Perform open circuit test on these transformers. |  |  |
| - Perform short circuit test on these transformers. |  |  |
| - Calculate iron losses and copper losses from above tests. |  |  |
| - Calculate input powers and output powers. |  |  |
| - Calculate efficiency of each transformer from there calculated powers. |  |  |
| - Connect these transformers in star to delta connection. |  |  |
| - Connect phase angle meter between primary and secondary. |  |  |
| - Apply rated voltage to the primary side of transformer. |  |  |
| - Measure the phase angle between primary and secondary with the help of phase angle meter (300 lag or 300 lead). |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | AC Machines (Transformer) |
| --- | --- |
| Competency Standard | 1. Verify Basic Laws of Electrical Machines 2. Operate Single Phase Transformer 3. Perform Tests On Single Phase Transformer 4. Operate Auto Transformer 5. Perform Tests On Auto Transformer 6. Operate Three Phase Transformer 7. Identify Vector Group of Three Phase Transformer 8. Repair/ Maintenance and Service of Transformer |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________ Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________________________________ Assessor’s code:____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Select three identical single phase transformers. |  |  |  |  |  |
|  | Perform open circuit test on these transformers. |  |  |  |  |  |
|  | Perform short circuit test on these transformers. |  |  |  |  |  |
|  | Calculate iron losses and copper losses from above tests. |  |  |  |  |  |
|  | Calculate input powers and output powers. |  |  |  |  |  |
|  | Calculate efficiency of each transformer from there calculated powers. |  |  |  |  |  |
|  | Connect these transformers in star to delta connection. |  |  |  |  |  |
|  | Connect phase angle meter between primary and secondary. |  |  |  |  |  |
|  | Apply rated voltage to the primary side of transformer. |  |  |  |  |  |
|  | Measure the phase angle between primary and secondary with the help of phase angle meter (300 lag or 300 lead). |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | AC Machines (Transformer) |
| --- | --- |
| Competency Standard | 1. Verify Basic Laws of Electrical Machines 2. Operate Single Phase Transformer 3. Perform Tests On Single Phase Transformer 4. Operate Auto Transformer 5. Perform Tests On Auto Transformer 6. Operate Three Phase Transformer 7. Identify Vector Group of Three Phase Transformer 8. Repair/ Maintenance and Service of Transformer |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name:______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature:___________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor:________________________________________________ Assessor’s code:____________________________________________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | What is star to Delta connection? |  |  |
|  | Explain the relation between line voltage and phase voltage in delta connection. |  |  |
|  | How to mark the polarity of the winding terminals? |  |  |
|  | Explain the relation between line current and phase current in delta connection. |  |  |
|  | What is transformation ratio? |  |  |
|  | Describe the conditions for paralleling of the transformers. |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature _________________