Instruction Sheet for the Candidate

| Qualification | AC & DC Machines (level-4) |
| --- | --- |
| Competency Standard | - Verify Basic Laws of Electrical Machines - Analyze Dc Generators - Perform Tests On Dc Generators - Analyze Dc Motors - Perform Tests On Dc Motors - Repair/ Maintenance and Service of Dc Motors and Dc Generators. |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within time frame (for practical demonstration & assessment): - Determine copper losses, friction losses and iron losses of two DC shunt generators by applying actual loading and run these two generators in parallel. - Calculate back EMF, brake horse power (BHP) and efficiency of DC shunt motor. |
| Time: 05 Hrs. | During a practical assessment, under observation by an assessor, you are required to |
| Minimum Evidence Required | - Select two DC shunt generators. - Connect ammeters in series with both winding of each generator. - Connect voltmeter in parallel with both winding of each generator.. - Apply 10-20 DC volts to the each winding of selected generators one by one. - Record the readings of voltmeters and ammeters. - Calculate Ra and Rsh of both generators. - Run the generators at rated speed with prime mover. - Turn on the load switch and record the meter reading. - Calculate input power and output power. - Calculate total losses (Copper loss, friction loss, Iron loss) - Connect the both generators in parallel according to the circuit diagram. - Place all switches in off position. - Run generator 1 at rated speed. - Reduce field resistance of generator 1 - Connect generator 1 with bus bar after attaining rated voltage. - Run generator 2 with prime mover. - Reduce the field resistance of generator 2 to attain its rated voltage. - Close the switches for parallel operation. - Calculate and record the reading of voltage and current on different loads. - Select DC shunt motor and make the connection as per circuit diagram. - Switch on the supply and move starter handle from off position to position 1. - Move the starter arm from position 1 to position 2 and continue this process till starter arm reach at its extreme position. - Record the value of speed and current on each load. - Calculate back EMF by using the recoded values and appropriate formula then stop the motor. - Make graph between speed and back EMF and interpret the result. - Connect the spring balance with one end of the rope. - Connect other end of rope with weight (W1). - Measure the radius of the pulley. - Hang the weight on the pulley of the motor and fix the spring balance end with earth. - Run the motor. - Measured the RPM of the motor. - Adjust the weight to pass full load current from motor. - Calculate spring balance weight and hanged weight (W1) - Calculate shaft torque. - Calculate BHP of motor by shaft torque. - Stop the motor. - Perform DC test to find the value armature resistance and field resistance - Run the motor at rated voltage without load. - Adjust the value of shunt regulator to attain rated speed. - Measure Field current and No load current with ampere meter. - Calculate total copper losses and iron losses. - Calculate input and output power. - Calculate efficiency from calculated power. |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | DC machines (Motor and Generator)(level-4) |
| Competency Standard | 1. Verify Basic Laws of Electrical Machines 2. Analyze Dc Generators 3. Perform Tests On Dc Generators 4. Analyze Dc Motors 5. Perform Tests On Dc Motors 6. Repair/ Maintenance and Service of Dc Motors and Dc Generators. |
| Purpose of Assessment | Summative Assessment |
| Assessment Task |  |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Select two DC shunt generators. |  |  |
| - Connect ammeters in series with both winding of each generator. |  |  |
| - Connect voltmeter in parallel with both winding of each generator.. |  |  |
| - Apply 10-20 DC volts to the each winding of selected generators one by one. |  |  |
| - Record the readings of voltmeters and ammeters. |  |  |
| - Calculate Ra and Rsh of both generators. |  |  |
| - Run the generators at rated speed with prime mover. |  |  |
| - Turn on the load switch and record the meter reading. |  |  |
| - Calculate input power and output power. |  |  |
| - Calculate total losses (Copper loss, friction loss, Iron loss) |  |  |
| - Connect the both generators in parallel according to the circuit diagram. |  |  |
| - Place all switches in off position. |  |  |
| - Run generator 1 at rated speed. |  |  |
| - Reduce field resistance of generator 1 |  |  |
| - Connect generator 1 with bus bar after attaining rated voltage. |  |  |
| - Run generator 2 with prime mover. |  |  |
| - Reduce the field resistance of generator 2 to attain its rated voltage. |  |  |
| - Close the switches for parallel operation. |  |  |
| - Calculate and record the reading of voltage and current on different loads. |  |  |
| - Select DC shunt motor and make the connection as per circuit diagram. |  |  |
| - Switch on the supply and move starter handle from off position to position 1. |  |  |
| - Move the starter arm from position 1 to position 2 and continue this process till starter arm reach at its extreme position. |  |  |
| - Record the value of speed and current on each load. |  |  |
| - Calculate back EMF by using the recoded values and appropriate formula then stop the motor. |  |  |
| - Make graph between speed and back EMF and interpret the result. |  |  |
| - Connect the spring balance with one end of the rope. |  |  |
| - Connect other end of rope with weight (W1). |  |  |
| - Measure the radius of the pulley. |  |  |
| - Hang the weight on the pulley of the motor and fix the spring balance end with earth. |  |  |
| - Run the motor. |  |  |
| - Measured the RPM of the motor. |  |  |
| - Adjust the weight to pass full load current from motor. |  |  |
| - Calculate spring balance weight and hanged weight (W1) |  |  |
| - Calculate shaft torque. |  |  |
| - Calculate BHP of motor by shaft torque. |  |  |
| - Stop the motor. |  |  |
| - Perform DC test to find the value armature resistance and field resistance |  |  |
| - Run the motor at rated voltage without load. |  |  |
| - Adjust the value of shunt regulator to attain rated speed. |  |  |
| - Measure Field current and No load current with ampere meter. |  |  |
| - Calculate total copper losses and iron losses. |  |  |
| - Calculate input and output power. |  |  |
| - Calculate efficiency from calculated power. |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | DC machines (Motor and Generator)(level-4) |
| --- | --- |
| Competency Standard | 1. Verify Basic Laws of Electrical Machines 2. Analyze Dc Generators 3. Perform Tests On Dc Generators 4. Analyze Dc Motors 5. Perform Tests On Dc Motors 6. Repair/ Maintenance and Service of Dc Motors and Dc Generators. |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________ Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________________________________ Assessor’s code:____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Select two DC shunt generators. |  |  |  |  |  |
|  | Connect ammeters in series with both winding of each generator. |  |  |  |  |  |
|  | Connect voltmeter in parallel with both winding of each generator.. |  |  |  |  |  |
|  | Apply 10-20 DC volts to the each winding of selected generators one by one. |  |  |  |  |  |
|  | Record the readings of voltmeters and ammeters. |  |  |  |  |  |
|  | Calculate Ra and Rsh of both generators. |  |  |  |  |  |
|  | Run the generators at rated speed with prime mover. |  |  |  |  |  |
|  | Turn on the load switch and record the meter reading. |  |  |  |  |  |
|  | Calculate input power and output power. |  |  |  |  |  |
|  | Calculate total losses (Copper loss, friction loss, Iron loss) |  |  |  |  |  |
|  | Connect the both generators in parallel according to the circuit diagram. |  |  |  |  |  |
|  | Place all switches in off position. |  |  |  |  |  |
|  | Run generator 1 at rated speed. |  |  |  |  |  |
|  | Reduce field resistance of generator 1 |  |  |  |  |  |
|  | Connect generator 1 with bus bar after attaining rated voltage. |  |  |  |  |  |
|  | Run generator 2 with prime mover. |  |  |  |  |  |
|  | Reduce the field resistance of generator 2 to attain its rated voltage. |  |  |  |  |  |
|  | Close the switches for parallel operation. |  |  |  |  |  |
|  | Calculate and record the reading of voltage and current on different loads. |  |  |  |  |  |
|  | Select DC shunt motor and make the connection as per circuit diagram. |  |  |  |  |  |
|  | Switch on the supply and move starter handle from off position to position 1. |  |  |  |  |  |
|  | Move the starter arm from position 1 to position 2 and continue this process till starter arm reach at its extreme position. |  |  |  |  |  |
|  | Record the value of speed and current on each load. |  |  |  |  |  |
|  | Calculate back EMF by using the recoded values and appropriate formula then stop the motor. |  |  |  |  |  |
|  | Make graph between speed and back EMF and interpret the result. |  |  |  |  |  |
|  | Connect the spring balance with one end of the rope. |  |  |  |  |  |
|  | Connect other end of rope with weight (W1). |  |  |  |  |  |
|  | Measure the radius of the pulley. |  |  |  |  |  |
|  | Hang the weight on the pulley of the motor and fix the spring balance end with earth. |  |  |  |  |  |
|  | Run the motor. |  |  |  |  |  |
|  | Measured the RPM of the motor. |  |  |  |  |  |
|  | Adjust the weight to pass full load current from motor. |  |  |  |  |  |
|  | Calculate spring balance weight and hanged weight (W1) |  |  |  |  |  |
|  | Calculate shaft torque. |  |  |  |  |  |
|  | Calculate BHP of motor by shaft torque. |  |  |  |  |  |
|  | Stop the motor. |  |  |  |  |  |
|  | Perform DC test to find the value armature resistance and field resistance |  |  |  |  |  |
|  | Run the motor at rated voltage without load. |  |  |  |  |  |
|  | Adjust the value of shunt regulator to attain rated speed. |  |  |  |  |  |
|  | Measure Field current and No load current with ampere meter. |  |  |  |  |  |
|  | Calculate total copper losses and iron losses. |  |  |  |  |  |
|  | Calculate input and output power. |  |  |  |  |  |
|  | Calculate efficiency from calculated power. |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | DC machines (Motor and Generator)(level-4) |
| --- | --- |
| Competency Standard | 1. Verify Basic Laws of Electrical Machines 2. Analyze Dc Generators 3. Perform Tests On Dc Generators 4. Analyze Dc Motors 5. Perform Tests On Dc Motors 6. Repair/ Maintenance and Service of Dc Motors and Dc Generators. |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name:______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature:___________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor:________________________________________________ Assessor’s code:____________________________________________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | What is different between shunt and separately excited generator? |  |  |
|  | Explain the conditions of paralleling of DC generators. |  |  |
|  | Why we find copper loss at full load? |  |  |
|  | Explain the effect of field current on motor speed. |  |  |
|  | Explain the effect of load on motor speed. |  |  |
|  | What is back EMF? |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature _________________