National Vocational Certificate Level 1&2 in Electrical Technology

(Domestic Wiring Electrician)

Competency Standards (CS) Document

National Vocational and Technical Training Commission (NAVTTC)

Government of Pakistan

Table of Contents

Introduction	5

Purpose of the Qualification	6

Common Courses with respective levels	7

Levelling of Core Competencies of the Qualification along with the occupations	8

Packaging of Occupations	10

Mapping of the Qualifications	11

Date of Validation	12

Date of Review	12

Codes of Qualifications	12

Members of Qualifications Development Committee	13

Members of Qualification Validation Committee	14

Entry Requirements	14

Detail of Competency Standards	15

0713E&E1Maintain Occupational Health and Safety	15

0713E&E2 Adopt Safety Regulations, Labour Protection Laws, Environmental Protection Laws at Workplace	19

0713E&E3 Maintain Tools & Equipment for Single Phase Wiring	25

0713E&E4 Make Cable/Wire Joints for Single Phase Wiring	28

0713E&E5 Prepare and Install Distribution Boards for Single Phase.	31

0713E&E6 Carryout Basic Electrical Installation for Single phase	33

0713E&E7 Install Simple Single Phase Electrical Wiring	36

0713E&E8 Install Single Phase Complex Electrical Wiring	43

0713E&E9 Perform Testing of Electrical Wiring	54

0713E&E10 Repair/ Maintenance of Electrical Installations in Single Phase	59

0713E&E11 Repair/ Maintenance and Service of Home Appliance	65

0713E&E12 Perform Cleaning, Oiling and Centering the Job	70

0713E&E13 Perform Facing, Straight Turning, Centre Drilling	73

0713E&E14 Perform Drilling, Step Turning, Knurling and Boring a Straight Hole on lathe	77

0713E&E15 Perform Step or Counter Boring, Reaming, Tool Grinding, Taper Turning and Cutting Metric Threads.	82

0713E&E16 Perform Basic Metal Work	87

0713E&E17 Perform Oxy Acetylene Welding	93

0713E&E18 Perform Wood Work	99

0713E&E19 Perform Arc Welding	106

# Introduction

The Technical and Vocational is a profession that is increasingly getting attention in Pakistan, not only among the youth seeking to enter the industry but also among adults who wish to polish their skills to develop a career out of it. Scope and demand of Electrical engineers, supervisors, operator, and technician is very high in Pakistan due to fast growing economic zone like CPEC. There are more job opportunities for electrical technician and supervisors in Pakistan than any other field of engineering. Pakistan is still facing energy crises, for need for more power houses, renewable energy sources is being felt which will definitely result in more demand of electrical engineers.

CBT based level 1-5 course in Electrical Technology with updated concepts, not only allows students to equip with growing Industrial trends but will be beneficial in capturing the job market across the globe. The course is divided into sub fields like electronics, computer engineering, power engineering, telecommunications, control systems and microelectronics in respective levels. Many of these sub disciplines overlap with other engineering branches, spanning a huge number of specializations such as hardware engineering, power electronics, avionics, mechatronics and nanotechnology.

Pakistan can boost economic growth and job creation by overcoming inefficiencies in its power sector, “Reforms that address these distortions can make better use of existing facilities. These need to focus on eliminating waste, promoting the shift towards cleaner energy and attracting private investments.”

“Power sector reform should be a top priority, as few other reforms could yield economic gains of a similar magnitude so quickly, based on findings from this report,” says Fan Zhang, Senior Economist at the World Bank. “If well designed, these reforms will directly benefit the poor by increasing access, improving reliability, and reducing cost and pollution.”

# Purpose of the Qualification

Based upon this demand of industry these competency‐based qualifications for Electrical Technology are developed under National Vocational Qualification Framework (Level 1 to 5). The qualifications mainly cover competencies along with related knowledge and professional attitude which is essential for getting a job or self-employed.

The qualifications are also in line with the vision of Pakistan’s National Skills Strategy (NSS), National TVET Policy and National Vocational Qualification Framework (NVQF). This provides policy directions, support and an enabling environment to the public and private sectors to impart training for skills development to enhance social and economic profile. The National Competency Standards could be used as a referral document for the development of curricula to be used by training institutions.

The purpose of the training is to provide the basic skills to the trainees in the field of Electrical and convert it into value added product which is acceptable by International market reducing the line losses and fit-in a skilled graduate into National Vocational Qualification Framework for his / her vertical career progression and qualification equivalencies at par with acceptable international standards.

# Common Courses with respective levels

LEVEL 3

LEVEL 2

LEVEL 4

LEVEL 5

# Levelling of Core Competencies of the Qualification along with the occupations

| Competency Standards | Occupation | NVQF Level | Category | Estimated Contact Hours |  |  | Cr Hr |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Th | Pr | Total |  |
| Level 2 |  |  |  |  |  |  |  |
| Maintain Occupational Health and Safety | Safety Supervisor | Level 2 | Functional | 6 | 24 | 30 | 3 |
| Adopt Safety Regulations, Labour Protection Laws, Environmental Protection Laws at Workplace |  | Level 2 | Functional | 6 | 24 | 30 | 3 |
| Maintain Tools & Equipment for Single Phase Wiring | Domestic Wiring Electrician | Level 2 | Technical | 6 | 24 | 30 | 3 |
| Make Cable/Wire Joints for Single Phase Wiring |  | Level 2 | Technical | 6 | 24 | 30 | 3 |
| Prepare and Install Distribution Boards for Single Phase Wiring |  | Level 2 | Technical | 6 | 24 | 30 | 3 |
| Carryout Basic Electrical Installation for Single phase |  | Level 2 | Technical | 9 | 24 | 33 | 3.3 |
| Install Simple Single Phase Electrical Wiring |  | Level 2 | Technical | 6 | 24 | 30 | 3 |
| Install Single Phase Complex Electrical Wiring |  | Level 2 | Technical | 9 | 24 | 33 | 3.3 |
| Perform Testing of Electrical Wiring |  | Level 2 | Technical | 6 | 24 | 30 | 3 |
| Repair/ Maintenance of Electrical Installations in Single Phase |  | Level 2 | Technical | 6 | 24 | 30 | 3 |
| Repair/ Maintenance and Service of Home Appliance |  | Level 2 | Technical | 6 | 24 | 30 | 3 |
| Perform Cleaning, Oiling and Centering the Job. | Machine Shop Technician | Level 2 | Technical | 6 | 27 | 33 | 3.3 |
| Perform Facing, Straight Turning, Centre Drilling |  | Level 2 | Technical | 6 | 27 | 33 | 3.3 |
| Perform Drilling, Step Turning, Knurling and Boring a Straight Hole On Lathe. |  | Level 2 | Technical | 6 | 27 | 33 | 3.3 |
| Perform Step or Counter Boring, Reaming, Tool Grinding, Taper Turning and Cutting |  | Level 2 | Technical | 6 | 27 | 33 | 3.3 |
| Perform Basic Metal Work. |  | Level 2 | Technical | 6 | 27 | 33 | 3.3 |
| Perform Wood Work. |  | Level 2 | Technical | 6 | 27 | 33 | 3.3 |
| Perform Oxy Acetylene Welding. |  | Level 2 | Technical | 6 | 27 | 33 | 3.3 |
| Perform Arc Welding. |  | Level 2 | Technical | 6 | 27 | 33 | 3.3 |
| Total |  |  |  | 120 | 480 | 600 | 60 |
| Percentage |  |  |  | 20 | 80 |  |  |

# Packaging of Occupations

The National Vocational Qualifications have been packaged as detailed below:

Level 2

(Safety supervisor, Domestic Wiring Electrician, Machine Shop Technician)

Safety supervisor

- Maintain Occupational Health and Safety

- Adopt Safety Regulations, Labour Protection Laws, Environmental Protection Laws at Workplace

- Develop Professionalism.

Domestic Wiring Electrician (Major)

- Maintain Tools & Equipment.

- Make Cable/Wire Joints

- Prepare and Install Main Distribution Boards (Single Phase)

- Carryout Basic Electrical Installation

- Install Simple Electrical Wiring

- Install Single Phase Complex Electrical Wiring

- Perform Testing of Electrical Wiring

- Repair/ Maintenance of Electrical Installations

- Repair/ Maintenance and Service of Home Appliances

Machine Shop Technician

- Perform Cleaning, Oiling and Centering the Job.

- Perform Facing, Straight Turning, Centre Drilling

- Perform Drilling, Step Turning, Knurling and Boring a Straight Hole On Lathe.

- Perform Step or Counter Boring, Reaming, Tool Grinding, Taper Turning and Cutting

- Perform Basic Metal Work.

- Perform Wood Work.

# Mapping of the Qualifications

AC DC Machines…

Power Electronics…..

Analog Electronics

Digital Electronics

Wiring

Soft Skills

Digital Skills

Entrepreneurship

Microcontrollers

Analog Electronics

Digital Electronics

Process Variable Measurements….

# Date of Validation

The level 5 of National DAE qualification on Electrical has been validated by the Qualifications Validation Committee (QVC) members on 23-25 May, 2019 and will remain valid for ten years i.e 25 May, 2029

# Date of Review

The level 5 of National DAE qualification on Electrical has been validated by the Qualifications Validation Committee (QVC) members on 23-25 May, 2019 and shall be reviewed after three years i.e. 26 May, 2022

# Codes of Qualifications

The International Standard Classification of Education (ISCED) is a framework for assembling, compiling and analyzing cross-nationally comparable statistics on education and training. ISCED codes for these qualifications are assigned as follows:

| ISCED Classification for Electrical Technology level 1-5 |  |
| --- | --- |
| Code | Description |
| 0713E&E(1) | National Vocational Certificate Level 1 in Electrical Technology |
| 0713E&E(2) | National Vocational Certificate Level 2 in Electrical Technology |
| 0713E&E(3) | National Vocational Certificate Level 3 in Electrical Technology |
| 0713E&E(4) | National Vocational Certificate Level 4 in Electrical Technology |
| 0713E&E(5) | National Vocational Diploma Level 5 in Electrical Technology |

# Members of Qualifications Development Committee

The following members participated in the qualification development of this qualification:

| S# | Name | Designation | Organization |
| --- | --- | --- | --- |
| 1 | Aijaz Ahmed Zia | Design & Application Engineer | INTECH Process Automation. Lahore |
| 2 | Amir Amin | HOD Electrical, Research Associate LUMS | City Polytechnic |
| 3 | Muhammad Faheem Anjum | Chief Instructor Electrical | P-TEVTA |
| 4 | Muhammad Asad | Instructor Electrical | P-TEVTA |
| 5 | Tahreem Javeed | Lab Engineer | The University of Lahore |
| 6 | Huma Naeem | Research Associate | COMSATS Lahore |
| 7 | Engr.Inayat Ur Rehman | DACUM Facilitator | KPK-TEVTA |
| 8 | Muhammad Yasir | NAVTTC Coordinator | NAVTTC HQ Islamabad |

# Members of Qualification Validation Committee

The following members participated in the qualifications validation of this qualification:

| S# | Name | Designation | Organization |
| --- | --- | --- | --- |
| 1 | Israr Ahmaad | DD Academia | KP-TEVTA |
| 2 | Muhammad Yasir | NAVTTC Coordinator | NAVTTC HQ Islamabad |
| 3 | Sayeda Fatima Iqbal | System Analyst | PBTE Lahore |
| 4 | Danish Khan | Calibration Engineer | NPSL(PCSIR) Islamabad |
| 5 | Ghulam Rasool Maka | Vice Principal | S TEVTA |
| 6 | Muhammad Mehboob butt | Associate Professor | P TEVTA |
| 7 | Inayat Ur Rehman | EX Professor/ Ex-Director Academics | KP-TEVTA |
| 8 | Muhammad Asad | Instructor Electrical | P-TEVTA |

# Entry Requirements

The entry level for National Vocational Certificate Level 1&2 in Electrical Technology is:

- A person having Middle certificate.

# Detail of Competency Standards

### 0713E&E1Maintain Occupational Health and Safety

Overview:

After this Competency Standard, the Trainee will be able to develop skill and competence required to maintain Occupational Health and Safety and take remedial measures to deal with the emergencies in a professional manner, thus minimizing the losses and providing a safe and healthy working environment.

| Competency Units | Performance Criteria |
| --- | --- |
| - Ensure use of personal protective equipment (PPE) | - Arrange the required personal protective equipment - Check functional condition of PPE’s - Wear personal protective equipment - Store PPE at appropriate place after use. |
| - Maintain First-aid Box | - Ensure availability of first aid box - Check first aid box for requisite emergency - Check expiry of medicines - Perform first aid treatment against electric shocks - Perform first aid treatment/bandages against minor injuries. |
| - Maintain Fire Extinguisher | - Check expiry of fire extinguisher - Operate fire extinguisher - Replace fire extinguisher - Ensure that the fire brigade is at stand by(for major emergency) |
| - Ensure Safeguard of Machines | - Maintain radiator shield - Maintain alternator fan shield - Maintain heat resister material on silencer - Cover main circuit breaker - Lock canopy doors |
| - Adopt company policies and procedures | - Ensure company’s safety policy - Adopt company safety procedure - Advocate worker with company safety policy - Implement Safety sign board as per standard |
| - Attain health & safety training | - Take required health and safety training - Implement work hazardous material information system (WHMIS) - Adopt first aid cardio respiratory, resuscitation and CPR |
| - Prepare for emergencies | - Take emergency response training - Ensure practice of emergency exercises - Check the emergency alarms - Ensure regular practice of gathering the workers in assembly area during the emergency. |
| - Respond to emergencies | - Follow emergency plan - Communicate instructions to co workers - Assess risk and determine course of action - Operate emergency equipment and supplies - Ensure that the ambulance is at stand by(for emergency) |

Knowledge & Understanding

- Factors affecting Health & Safety in the workplace.

- Personal Protective Equipment (PPE)

- First-Aid-Box.

- Emergency medicines and expiry

- Methods of treatment against electric shock

- Methods of treatment against minor injuries

- Types of Fire Extinguisher

- Uses of Fire Extinguisher

- Company policies and procedures

- Understand various safe guards

- Safety measures

- Work permit/no objection certificate(NOC)

- Types of work site Hazards

- Hazardous chemical control procedures

- Methods of first aid cardio respiratory Procedure

- Types of emergencies

- Response various types of emergencies

- Emergency equipment, supplies and their operation

- Methods of communication during

- emergency

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | Protection suite |
| 2 | Safety shoes |
| 3 | Safety goggles |
| 4 | Hearing protection |
| 5 | Respiratory mask |
| 6 | First Aid Box |
| 7 | First Aid Kit |
| 8 | Stretcher |
| 9 | Fire Buckets |
| 10 | Fire Extinguisher |
| 11 | Emergency Alarm/Bell |
| 12 | Emergency response Plan |
| 13 | Fall Protection Plan |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Types of hazards that are most likely to cause harm to health and safety

- Health and safety signs and precautions

- Techniques and methods to identify the risks of hazards at workplace

- Safety reporting procedures and documentation

- Describe fire-fighting methods

- Demonstrate use of appropriate Personal Protective Equipment (PPE) for the assigned job

- Demonstrate removal and disposal of PPE.

### 0713E&E2 Adopt Safety Regulations, Labour Protection Laws, Environmental Protection Laws at Workplace

Overview:

After this Competency Standard, the Trainee will be able to develop skill and competence required to maintain Occupational Safety, Health and Environment at the workplace according to the National and International Protection Agencies Standards and take remedial measures for Personal, Occupational and Environmental Protection. The Trainee will be able to deal with the emergencies in a professional manner, thus minimizing the losses and providing a safe and healthy working environment.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1: Implement International Safety Standards in your work environment | - Recognize Electrical Safety hazards as per International Electro- Technical Commission (IEC) Standards - Determine Environmental Pollution risk factors as per Protection Agency (EPA) standards - Identify Electrical Safety Hazards as per Institute of Electrical and Electronics Engineers (IEE) standards - Categorize the Electrical Safety Hazards as per Electrical Safety Foundation International(ESFI) standards - Identify Labor Protection Laws as per International Labor Organization(ILO) rules - Identify the steps to minimize the Electrical hazards and Environmental Pollution. - Prepare a report for all the above activity. |
| CU2: Implement National Safety Standards in your work environment | - Identify Factory associated hazard as per Chapter 3 of Factories Act, 1934 - Determine Environmental Pollution factors as per Pakistan Environmental Protection Act, 1997 - Recognize the Labor protection laws as per Labor Protection Policy 2006 - Identify the workplace hazards as per Occupational health and safety (OHS) standards - Identify the steps to minimize the Electrical hazards, Environmental Pollution and Labor Safety - Prepare a report for all the above activity. |
| CU3: Implement International and National Labor Protection Laws | - Identify Labor Protection Laws as per International Labor Organization(ILO) rules - Recognize the Labor protection laws as per Labor Protection Policy 2006 - Identify the Bonded Labor and Child Labor policy. - Determine the leaves policy and compensation policy for the Labor. - Recognize the minimum wage for the Labor - Identify the remedial steps for protection and prosperity of Labor. Prepare a report for all the above activity. |
| CU4: Implement National and International Environmental protection laws | - Determine Environmental Pollution risk factors as per Protection Agency (EPA) standards - Identify the steps to minimize the Electrical hazards and Environmental Pollution. - Determine Environmental Pollution factors as per Pakistan Environmental Protection Act, 1997 - Identify the requirements for Initial Environmental Examination (IEE) - Identify the requirements for Environmental Impact Assessment (EIA) - Prepare a report for all the above activity. |
| CU5: Prepare for emergencies | - Take emergency response training - Ensure practice of emergency exercises - Ensure the availability of first aid box and fire extinguisher - Check the expiry of medicines and fire extinguishers - Check the emergency alarms - Ensure regular practice of gathering the workers in assembly area during the emergency. |
| CU6: Respond to emergencies | - Follow emergency plan - Communicate instructions to co workers - Assess risk and determine course of action - Operate emergency equipment and supplies - Ensure that the ambulance and fire brigade is at stand by (for major emergency) |
| CU7: Adopt company policies and procedures | - Ensure company’s safety policy - Adopt company safety procedure - Advocate worker with company safety policy - Implement Safety sign board as per standard |

Knowledge & Understanding

- Factors affecting Health & Safety in the workplace.

- Personal Protective Equipment (PPE)

- First-Aid-Box.

- Emergency medicines and expiry

- Methods of treatment against electric shock

- Methods of treatment against minor injuries

- Types of Fire Extinguisher

- Uses of Fire Extinguisher

- IEC/EPA/IEE/ESFI Standards

- Factories Act 1934

- Pakistan Environmental Protection Act, 1997

- Occupational health and safety (OHS) standards

- Labor Protection Policy 2006

- Company policies and procedures

- Understand various safe guards

- Safety measures

- Work permit/no objection certificate(NOC)

- Types of work site Hazards

- Hazardous chemical control procedures

- Methods of first aid cardio respiratory Procedure

- Types of emergencies

- Response various types of emergencies

- Emergency equipment, supplies and their operation

- Methods of communication during emergency

- Factors affecting Health & Safety in the workplace.

- Personal Protective Equipment (PPE)

- First-Aid-Box.

- Emergency medicines and expiry

- Methods of treatment against electric shock

- Methods of treatment against minor injuries

- Types of Fire Extinguisher

- Uses of Fire Extinguisher

- IEC/EPA/IEE/ESFI Standards

- Factories Act 1934

- Pakistan Environmental Protection Act, 1997

- Occupational health and safety (OHS) standards

- Labor Protection Policy 2006

- Factors affecting Health & Safety in the workplace.

- Personal Protective Equipment (PPE)

- First-Aid-Box.

- Emergency medicines and expiry

- Methods of treatment against electric shock

- Methods of treatment against minor injuries

- Types of Fire Extinguisher

- Uses of Fire Extinguisher

- IEE/EIA Standards

- Factories Act 1934

- Pakistan Environmental Protection Act, 1997

- Occupational health and safety

- OHS) standards

- Labor Protection Policy 2006

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | Respiratory mask |
| 2 | First Aid Box |
| 3 | First Aid Kit |
| 4 | Stretcher |
| 5 | Fire Buckets |
| 6 | Fire Extinguisher |
| 7 | Emergency Alarm/Bell |
| 8 | Fall Protection Plan |
| 9 | International Safety Standards Manual |
| 10 | Emergency response Plan |
| 11 | National Safety Standards Manual |
| 12 | Plan |
| 13 | National Safety Standards Manual |
| 14 | WHMIS Handbook |
| 15 | International Safety Standards Manual |
| 16 | Safety shoes |
| 17 | Hearing protection |
| 18 | Safety goggles |
| 19 | Emergency response |
| 20 | Emergency Alarm/Bell |
| 21 | Fire Extinguisher |
| 22 | Fire Buckets |
| 23 | Stretcher |
| 24 | First Aid Box |
| 25 | Respiratory mask |
| 26 | First Aid Kit |
| 27 | Protection suite |
| 28 | Protection suite |
| 29 | Hearing protection |
| 30 | WHMIS Handbook |
| 31 | National Safety Standards Manual |
| 32 | International Safety Standards Manual |
| 33 | Fall Protection Plan |
| 34 | Emergency response Plan |
| 35 | Emergency Alarm/Bell |
| 36 | Fire Extinguisher |
| 37 | Fire Buckets |
| 38 | Stretcher |
| 39 | First Aid Kit |
| 40 | Respiratory mask |
| 41 | First Aid Box |
| 42 | Safety shoes |
| 43 | Safety goggles |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

:

- Types of hazards that are most likely to cause harm to health and safety

- Health and safety precautions

- Health and safety signs and symbols

- Techniques and methods to identify the risks of hazards at workplace

- Dealing with hazards to avoid any accident or injury

- Safety reporting procedures and documentation

- Describe fire-fighting methods

- Demonstrate use of appropriate Personal Protective Equipment (PPE)

### 0713E&E3 Maintain Tools & Equipment for Single Phase Wiring

Overview:

This Competency Standard covers the skills and knowledge required to arrange tools/equipment, maintain tool box, insulate tools/equipment, calibrate measuring tools and manage proper inventory of used/unused tools/equipment. Trainee will be expected to follow the procedures to maintain the tools/equipment.

| Competency Units | Performance Criteria |
| --- | --- |
| - Arrange Tools and Equipment | - Identify tools and equipment - Interpret job card - Prepare list of tools and equipment as per requirement - Collect tools and equipment from store |
| - Maintain Tool Box | - Check physical conditions of tools and equipment before use - Perform preventive maintenance as per standards - Perform corrective maintenance of tools as per requirements - Clean tools and equipment after use - Place tools and equipment at appropriate place |
| - Insulate Tools and Equipment | - Select insulated tools and equipment - Adopt insulated tools and equipment as per standards |
| - Calibrate measuring tools | - Check calibration status of the measuring tools - Perform calibration of measuring tools as per standards - Record calibration test results |
| - Manage Inventory of tools and equipment | - Check tools and equipment as per record - Report for faulty tools and equipment to supervisor - Generate demand for deficit tools and equipment - Maintain all records of tools and Equipment |

Knowledge & Understanding

- Explain Various tools and equipment and their functions

- Define Job card/work order

- How an Arrangement of tools/equipment as per job is required?

- Differentiate between corrective and preventive maintenance

- Arrange tools

- and equipment in tool box

- Storage methods of tools and equipment

- Insulation procedure

- Types of insulation

- Methods of insulated tools and equipment.

- Types of calibration

- Methods of equipment calibration

- Methods of tools and equipment inventory

- Report writing of faulty tools and equipment

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Describe functions of various tools and equipment

- Define job card

- Storage methods of tools and equipment

- Types of insulation

- Use of insulated tools and equipment

- Method of equipment calibration

- Report writing

### 0713E&E4 Make Cable/Wire Joints for Single Phase Wiring

Overview:

This Competency Standard covers the skills and knowledge required to make joints of cable during wiring and breakage of wires, Make Cross/Twist joint, Make Straight/Married joint, Make T- Joint, Make Rat tail joint, Make Britannia joint.

| Competency Units | Performance Criteria |
| --- | --- |
| - Make Cross/Twist joint | - Select the cable. - Strip the wire according to 50mm. - Twist the conductors. - Solder the conductor - Insulate the joint |
| - Make Straight/Married joint | - Select the cable. - Strip wire to 75mm according to joint requirement. - Intermingle the conductors to 60mm into each other. - Twist 60mm conductors leaving behind 15mm of each cable. - Solder the conductor. - Insulate the joint. |
| - Make T- Joint | - Select the cable. - Remove the insulation of cable 1 to 50mm from where a connection is required. - Separate conductors of cable 1 equally. - Take another 12 mm stripped wire 2. - Insert between two equally half conductors of cable 1 and twist. Half conductors clockwise and half anti-clock wise of cable 2. - Solder the joint. |
| - Make Rat tail joint | - Select the single conductor wires. - Strip both the wires to 5mm. - Twist the conductor. - Insulate the joint. - Solder the joint. |
| - Make Britannia joint | - Select the cable. - Strip both cables to 75mm. - Bend the tips of both cable right angle about to 6mm. - Hold the two cables overlap 50mm with tips in opposite direction. - Take another bare conductor of 1mm and wrap around in both directions to 6mm. - Solder the joint. - Insulate the joint. |

Knowledge & Understanding

- Define conductor.

- Differentiate between cable and wire.

- Describe the type of soldering.

- Differentiate between stripping and insulation removing.

- Describe the type of joints.

- Describe the procedure of jointing & soldering.

- Explain the composition of solder and soldering flux

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | Cables |
| 2 | Wires. |
| 3 | Wire stripper. |
| 4 | Solder wire. |
| 5 | Soldering paste |
| 6 | Plier. |
| 7 | Nose plier. |
| 8 | Insulation remover. |
| 9 | Solder. |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

Striping of cables.

Twisting of cable.

Soldering of joints

### 0713E&E5 Prepare and Install Distribution Boards for Single Phase.

Overview:

This Competency Standard covers the skills and knowledge required to prepare and install distribution board according to provide protection and load division to wiring, prepare estimate for wiring material, Prepare Distribution Board, Install Distribution Board and wiring.

| Competency Units | Performance Criteria |
| --- | --- |
| - Prepare estimate for wiring material. | - Develop Basic Drawing - Perform Estimation of Materials - Calculate Labor Cost |
| - Prepare Distribution Board | - Select Distribution board w.r.t. size, current rating, voltage, No. of C.Bs and phases. - Select Main Incoming Residual Current Circuit Breaker (RCCB) having minimum sensitivity. - Select outgoing Miniature Circuit Breaker (MCB) according to load. - Provide space for future MCB. - Select Voltmeter, ampere meter and indicator according to load and phases. - Select wire according to load and for wiring in DB. - Make neutral common for all load. - Connect accessories according to the circuit diagram. |
| - Install Distribution Board and wiring | - Fix the distribution board. - Take wire from DB to load without joint. - Distribute load equally on all phases. |

Knowledge & Understanding

- Define distribution board.

- Describe D.B. w.r.t. size, current rating, voltage, No. of C.Bs and phases etc.

- Name parts of distribution board.

- Define sensitivity of RCCB.

- Define MCB.

- Differentiate fuse and breaker.

- Define wiring accessories

- State purpose of each accessory.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Use accurate MCB for load.

- Make common neutral.

- Provide cost effective and quality oriented Bill of Quantity (BOQ).

### 0713E&E6 Carryout Basic Electrical Installation for Single phase

Overview:

This Competency standard deals with the skills and knowledge required to lay cables, perform single & three phase connections, basic electric wiring and wiring test for carrying out basic electrical Air Condition (AC) installation. The trainee will be required to follow company guidelines as well as the procedure for carrying out basic electrical AC installation.

| Competency Units | Performance Criteria |
| --- | --- |
| - Lay cables | P1 Interpret electrical drawing/document P2. Identify cables P3. Lay cables P4. Perform earthling |
| - Perform single-phase Connection | P1. Select cable gauge P2. Select cables colors P3. Connect cables P4. Insulate Joints |
| - Perform three phase Connection | P1. Select cable Gauge P2. Select cables colors P3. Connect cables P4. Insulate Joints |
| - Perform Basic Electrical wiring | P1. Measure cables as per requirement P2. Connect cables P3. Perform joints P4. Insulate Joints |
| - Conduct wiring Test | P1. Operate multi-meter for voltage and current P2. Perform continuity test P3. Perform polarity test P4. Perform earthling test P5. Perform insulation test P6. Record test results |

Knowledge & Understanding

- Explain Types of cables

- Explain Gauges of cables

- Define single phase connection

- Types of joints

- Define conductor and insulator

- Define three phase connection

- Knowledge about cable gauging

- Knowledge about color code of cables / phase sequence.

- Methods of Wiring

- Types of wiring

- Types of connections

- Types of wiring tests

- Describe various wiring test

- Color of cables

- Distinguish between different    wiring systems

- Name of necessary materials required for each type of wiring.

- Explain the uses of each type of wiring.

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | Cables |
| 2 | Solder |
| 3 | Wire stripper |
| 4 | Plier |
| 5 | Wires |
| 6 | Nose plier |
| 7 | Solder wire |
| 8 | Soldering paste |
| 9 | Insulation remover |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Color of Cables

- Single phase connection

- Three phase connection

- Color code of cables / Phase sequence

- Cable gauging

- Types of wiring

- Types of connections

### 0713E&E7 Install Simple Single Phase Electrical Wiring

Overview:

This Competency Standard covers the skills and knowledge required to make single pole switch circuit, make single pole switch socket circuit, make two-way switch circuit, make series/test lamp circuit. Make intermediate switch circuit, make of Tunnel circuit, Make of impulse switch circuit.

| Competency Units | Performance Criteria |
| --- | --- |
| - Make single pole switch circuit. | - Draw wiring diagram of single pole switch circuit. - Draw Current path diagram of single pole switch circuit. - Draw layout diagram of single pole switch circuit. - Mark on working board according to layout diagram. - Install accessories according to layout diagram. - Lay wires in duct/pipe according to layout diagram. - Make connections according to wiring diagram. - Check the circuit before connect the main supply. - Make connection with main supply. - Check the function of circuit after connect the main supply |
| - : Make single pole switch socket circuit. | - Draw wiring diagram of single pole switch socket circuit. - Draw Current path diagram of single pole switch socket circuit. - Draw layout diagram of single pole switch socket circuit. - Mark on working board according to layout diagram. - Install accessories according to layout diagram. - Lay wires in duct/pipe according to layout diagram. - Make connections according to wiring diagram. - Check the circuit before connect the main supply. - Make connection with main supply. - Check the function of circuit after connect the main supply |
| - Make two-way switch circuit. | - Draw wiring diagram of two-way switch circuit. - Draw Current path diagram of two-way switch circuit. - Draw layout diagram of two-way switch circuit. - Mark on working board according to layout diagram. - Install accessories according to layout diagram. - Lay wires in duct/pipe according to layout diagram. - Make connections according to wiring diagram. - Check the circuit before connect the main supply. - Make connection with main supply. - Check the function of circuit after connect the main supply |
| - Make series/test lamp circuit. | - Draw wiring diagram of series circuit. - Draw Current path diagram of series circuit. - Draw layout diagram of series circuit. - Mark on working board according to layout diagram. - Install accessories according to layout diagram. - Lay wires in duct/pipe according to layout diagram. - Make connections according to wiring diagram. - Check the circuit before connect the main supply. - Make connection with main supply. - Check the function of circuit after connect the main supply |
| - : Make intermediate switch circuit. | - Draw wiring diagram of intermediate switch circuit. - Draw Current path diagram of intermediate switch circuit. - Draw layout diagram of intermediate switch circuit. - Mark on working board according to layout diagram. - Install accessories according to layout diagram. - Lay wires in duct/pipe according to layout diagram. - Make connections according to wiring diagram. - Check the circuit before connect the main supply. - Make connection with main supply. - Check the function of circuit after connect the main supply |
| - Make of Tunnel circuit | - Draw wiring diagram of tunnel circuit. - Draw Current path diagram of tunnel circuit. - Draw layout diagram of tunnel circuit. - Mark on working board according to layout diagram. - Install accessories according to layout diagram. - Lay wires in duct/pipe according to layout diagram. - Make connections according to wiring diagram. - Check the circuit before connect the main supply. - Make connection with main supply. - Check the function of circuit after connect the main supply |
| - Make impulse switch circuit. | - Draw wiring diagram of impulse switch circuit. - Draw Current path diagram of impulse switch circuit. - Draw layout diagram of impulse switch circuit. - Mark on working board according to layout diagram. - Install accessories according to layout diagram. - Lay wires in duct/pipe according to layout diagram. - Make connections according to wiring diagram. - Check the circuit before connect the main supply. - Make connection with main supply. - Check the function of circuit after connect the main supply |

Knowledge & Understanding

- Define single pole switch.

- What is lamp?

- Explain single pole switch circuit and its use.

- Define current, voltage, power and resistance.

- Define single pole switch.

- What is lamp?

- Explain single pole switch socket circuit and its use.

- Define current, voltage, power and resistance.

- Describe socket

- Define single pole switch.

- What is lamp?

- Explain two-way switch circuit and its use.

- Define current, voltage, power and resistance.

- Describe two-way circuit.

- Define two-way switch

- Define single pole switch.

- What is lamp?

- Explain series circuit and its use.

- Define current, voltage, power and resistance.

- Describe socket.

- Define series circuit.

- Define parallel circuit.

- Define intermediate switch.

- What is lamp?

- Explain intermediate switch circuit and its use.

- Define current, voltage, power and resistance.

- Describe socket.

- Define series circuit.

- Define parallel circuit

- Define single pole switch.

- What is lamp?

- Explain tunnel circuit and its use

- Define current, voltage, power and resistance.

- Describe socket.

- Define series circuit.

- Define parallel circuit.

- Define two-way switch.

- What is lamp?

- Explain impulse switch circuit and its use.

- Define current, voltage, power and resistance.

- Describe socket.

- Define series circuit.

- Define parallel circuit.

- Define impulse switch.

- Define push button.

Tools and Equipment

.

| SN | Tools |
| --- | --- |
| 1 | Electrician Tool kit. |
| 2 | PVC Pipe/Duct. |
| 3 | PVC clamp. |
| 4 | Woden/PVC board. |
| 5 | PVC wire according to load |
| 6 | Circuit Breaker. |
| 7 | Screw |
| 8 | Single pole switch. |
| 9 | Lamp holder |
| 10 | Lamp |
| 11 | AVO meter. |
| 12 | Test Indicator |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

Make series/test lamp circuit.

### 0713E&E8 Install Single Phase Complex Electrical Wiring

Overview:

This Competency Standard covers the skills and knowledge required to Install kitchen circuit, Install Drawing room circuit, Install Sleeping room circuit, install hall wiring, install impulse switch, Install Bell indicator circuit, install timer/time switch circuit, prepare fluorescent tube circuit and install.

| Competency Units | Performance Criteria |
| --- | --- |
| - Install kitchen circuit | - Draw wiring diagram of single pole switch circuit. - Draw Current path diagram of single pole switch circuit. - Draw layout diagram of single pole switch circuit. - Mark on working board according to layout diagram. - Install accessories according to layout diagram. - Lay wires in duct/pipe according to layout diagram. - Make connections according to wiring diagram. - Check the circuit before connect the main supply. - Make connection with main supply. - Check the function of circuit after connect the main supply |
| - Install Drawing room circuit | - Draw wiring diagram of single pole switch circuit. - Draw Current path diagram of single pole switch circuit. - Draw layout diagram of single pole switch circuit. - Mark on working board according to layout diagram. - Install accessories according to layout diagram. - Lay wires in duct/pipe according to layout diagram. - Make connections according to wiring diagram. - Check the circuit before connect the main supply. - Make connection with main supply. - Check the function of circuit after connect the main supply |
| - Install Sleeping room circuit | - Draw wiring diagram of single pole switch circuit. - Draw Current path diagram of single pole switch circuit. - Draw layout diagram of single pole switch circuit. - Mark on working board according to layout diagram. - Install accessories according to layout diagram. - Lay wires in duct/pipe according to layout diagram. - Make connections according to wiring diagram. - Check the circuit before connect the main supply. - Make connection with main supply. - Check the function of circuit after connect the main supply |
| - Install Sleeping room circuit | - Draw wiring diagram of single pole switch circuit. - Draw Current path diagram of single pole switch circuit. - Draw layout diagram of single pole switch circuit. - Mark on working board according to layout diagram. - Install accessories according to layout diagram. - Lay wires in duct/pipe according to layout diagram. - Make connections according to wiring diagram. - Check the circuit before connect the main supply. - Make connection with main supply. - Check the function of circuit after connect the main supply |
| - Install hall wiring. | - Draw wiring diagram of single pole switch circuit. - Draw Current path diagram of single pole switch circuit. - Draw layout diagram of single pole switch circuit. - Mark on working board according to layout diagram. - Install accessories according to layout diagram. - Lay wires in duct/pipe according to layout diagram. - Make connections according to wiring diagram. - Check the circuit before connect the main supply. - Make connection with main supply. - Check the function of circuit after connect the main supply |
| - Install impulse switch. | - Draw wiring diagram of single pole switch circuit. - Draw Current path diagram of single pole switch circuit. - Draw layout diagram of single pole switch circuit. - Mark on working board according to layout diagram. - Install accessories according to layout diagram. - Lay wires in duct/pipe according to layout diagram. - Make connections according to wiring diagram. - Check the circuit before connect the main supply. - Make connection with main supply. - Check the function of circuit after connect the main supply |
| - Install Bell indicator circuit. | - Draw wiring diagram of single pole switch circuit. - Draw Current path diagram of single pole switch circuit. - Draw layout diagram of single pole switch circuit. - Mark on working board according to layout diagram. - Install accessories according to layout diagram. - Lay wires in duct/pipe according to layout diagram. - Make connections according to wiring diagram. - Check the circuit before connect the main supply. - Make connection with main supply. - Check the function of circuit after connect the main supply |
| - Install timer/time switch circuit. | - Draw wiring diagram of single pole switch circuit. - Draw Current path diagram of single pole switch circuit. - Draw layout diagram of single pole switch circuit. - Mark on working board according to layout diagram. - Install accessories according to layout diagram. - Lay wires in duct/pipe according to layout diagram. - Make connections according to wiring diagram. - Check the circuit before connect the main supply. - Make connection with main supply. - Check the function of circuit after connect the main supply |
| - Make fluorescent tube circuit and install. | - Draw wiring diagram of single pole switch circuit. - Draw Current path diagram of single pole switch circuit. - Draw layout diagram of single pole switch circuit. - Mark on working board according to layout diagram. - Install accessories according to layout diagram. - Lay wires in duct/pipe according to layout diagram. - Make connections according to wiring diagram. - Check the circuit before connect the main supply. - Make connection with main supply. - Check the function of circuit after connect the main supply |
| - Perform Domestic Electrical Work | - Install wire for grounding - Perform pre Installation module based Tests. - Install conduits or Trunks in the building as per drawing - Install electrical wiring, as per drawing and standard. - Install main box and distribution box. - Install Electric Breakers in main box, as per load for safety measures - Install the appliances used in electric work as per drawing - Install emergency light system in main box, as per standard color scheme - Install Main Distribution Board. - Test the Installation and obtain Test Certificate. |

Knowledge & Understanding

- Define single pole switch.

- What is lamp?

- Explain single pole switch circuit and its use.

- Define current, voltage, power and resistance.

- Describe socket.

- Define series circuit.

- Define parallel circuit.

- Define impulse switch.

- Define power plug.

- Explain the function of exhaust

- Define single pole switch.

- What is lamp?

- Explain single pole switch circuit and its use.

- Define current, voltage, power and resistance.

- Describe socket.

- Define series circuit.

- Define parallel circuit.

- Define impulse switch.

- Define power plug.

- Explain the function of exhaust.

- Explain the function of AC and Heater.

- Define single pole switch.

- What is lamp?

- Explain single pole switch circuit and its use.

- Define current, voltage, power and resistance.

- Describe socket.

- Define series circuit.

- Define parallel circuit.

- Define impulse switch.

- Define power plug.

- Explain the function of exhaust.

- Explain the function of AC and Heater.

- Define single pole switch.

- What is lamp?

- Explain single pole switch circuit and its use.

- Define current, voltage, power and resistance.

- Describe socket.

- Define series circuit.

- Define parallel circuit.

- Define impulse switch.

- Define power plug.

- Explain the function of exhaust.

- Explain the function of AC and Heater.

- Define single pole switch.

- What is lamp?

- Explain single pole switch circuit and its use.

- Define current, voltage, power and resistance.

- Describe socket.

- Define series circuit.

- Define parallel circuit.

- Define impulse switch.

- Define power plug.

- Explain the function of exhaust.

- Explain the function of AC and Heater.

- Explain multimedia.

- Explain Mic system.

- Explain Screen install on stage.

- Define single pole switch.

- What is lamp?

- Explain single pole switch circuit and its use.

- Define current, voltage, power and resistance.

- Describe socket.

- Define series circuit.

- Define parallel circuit.

- Define impulse switch.

- Define power plug.

- Explain the function of exhaust.

- Explain the function of AC and Heater.

- Explain multimedia.

- Explain Mic system.

- Explain Screen install on stage

- Define single pole switch.

- What is lamp?

- Explain single pole switch circuit and its use.

- Define current, voltage, power and resistance.

- Describe socket.

- Define series circuit.

- Define parallel circuit.

- Define impulse switch.

- Define power plug.

- Explain bell circuit

- Define single pole switch.

- What is lamp?

- Explain single pole switch circuit

- And its use.

- Define current, voltage, power and resistance.

- Describe socket.

- Define series circuit.

- Define parallel circuit.

- Define impulse switch.

- Define power plug.

- Explain timer circuit.

- Define single pole switch.

- What is lamp?

- Explain single pole switch circuit and its use.

- Define current, voltage, power and resistance.

- Describe socket.

- Define series circuit.

- Define parallel circuit.

- Define impulse switch.

- Define power plug.

- Explain fluorescent tube circuit

- Describe the safety of installation

- Explain the Protective measures against too high a contact voltage and conduct electrical accidents.

- Describe Fundamentals of installation engineering

- Describe the Power overhead-line systems.

- Describe Low-voltage switchgear and distribution systems.

- Explain Mounting and connection of motor testing of installed plant.

Tools and Equipment

.

| SN | Tools |
| --- | --- |
| 1 | Electrician Tool kit. |
| 2 | Single pole switch. |
| 3 | Lamp holder |
| 4 | Lamp |
| 5 | Woden/PVC board. |
| 6 | PVC Pipe/Duct. |
| 7 | PVC clamp. |
| 8 | Screw |
| 9 | PVC wire according to load. |
| 10 | Circuit Breaker. |
| 11 | AVO meter. |
| 12 | Test Indicator. |
| 13 | Exhaust |
| 14 | Fan |
| 15 | Series board. |
| 16 | Impulse switch. |
| 17 | Power plug |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

Install timer/time switch circuit.

### 0713E&E9 Perform Testing of Electrical Wiring

Overview:

This Competency Standard covers the skills and knowledge required to Perform Earth leakage Test, Perform Open Circuit Test, Perform Short Circuit Test, Perform Continuity/Loop Test, Perform Visual Test, Perform Insulation Test, Perform Polarity Test, Perform Earth Resistance Test, Perform Murray Loop Test, and Perform BLAVIER & Earth Loop Test

| Competency Units | Performance Criteria |
| --- | --- |
| - Perform Earth leakage Test. | - Ensure the socket must be at least 13A for RCD plug in. - Adjust the sensitivity of RCD. - Ensure tripping time must not exceed than 200msec. - Plug the RCD in socket and switch on socket outlet. - Ensure P-N and P-E light up. - Press the test button. - Ensure that RCD will trip and reading must be shown less than 200msec. - Move the selection switch to 180o. - Press the test button. - Ensure that RCD will trip and reading must be shown less than 200msec. - Move selection switch to 00. - Press the test button. - Ensure that RCD will trip and reading must be shown less than 200msec. |
| - Perform Open Circuit Test | - Disconnect the supply source, neutral conductor and earth conductor from distribution fuse box. - Connect ohmmeter between starting and ending terminals of live conductor. - Ensure the reading must be near to 0 ohm. - Repeat P2 to P3 for neutral and earth conductor. |
| - Perform Short Circuit Test | - Disconnect the supply source, neutral conductor and earth conductor from distribution fuse box. - Connect one terminal of ohmmeter with live conductor. - Connect the other terminals of ohmmeter with neutral conductor. - Ensure the reading must be infinity (No reading). - Connect ohmmeter between live conductor and earth conductor. - Ensure the reading must be infinity (No reading) |
| - Perform Continuity/Loop Test | - Connect all metal clad switches, metal parts, conduits with earth. - Open the main switch. - Turn on all the switches. - Connect earth continuity tester with conduit and independent earth. - Measure the resistance value with tester which must not be more than 1 ohm. |
| - Perform Visual Test | - Check the wires joints. - Check if there is any spark. - Check over heating of wires. - Check all the appliances are on or fluctuating. - Check all indications of meters. - Check Voltages on all phases. - Check the DB and Breakers. - Check the main supply is coming properly. |
| - Perform Insulation Test | - Switch off the main breaker. - Short circuit all lighting connections. - Connect merger between line and earth. - Rotate the merger to generate the voltage. - Read the merger dial and ensure that the insulation resistance should not be less than 50M ohm of any single circuit. |
| - Perform Polarity Test | - Switch off the main switch. - Disconnect all loads. - Switch on all circuit control switches. - Connect ohm meter between main line and all the terminal of sockets and load points one by one. - Ensure meter reading value must be less than 1 ohm. |
| - Perform Earth Resistance Test | - Place 3 electrodes in earth at distance of 10m apart between every electrode. - Connect terminal E of earth resistance tester to first electrode. - Connect terminal P of earth resistance tester to the second electrode. - Connect terminal C of the earth resistance tester with third electrode. - Measure the resistances after applying specific voltage. - Repeat P1 to P5 by interacting earthling regions. - Ensure 3 readings must be equal. |
| - Perform Murray Loop Test | - Connect galvanometer with Wheatstone bridge. - Connect positive terminal of DC source with Wheatstone bridge. - Connect negative terminal of DC source with ground. - Connect remaining two terminal of Wheatstone bridge with two cores of the cable. - Short the other end of these two cores of cable. - Note the reading of galvanometer. - Calculate Rx. - Repeat the P1 to P7 for other cores of the cable. |
| - Perform BLAVIER & Earth Loop Test | - Connect galvanometer with Wheatstone bridge. - Connect positive terminal of DC source with Wheatstone bridge. - Connect negative terminal of DC source with ground. - Connect remaining two terminal of Wheatstone bridge with two cores of the cable. - Short the other end of these two cores of cable. - Note the reading of galvanometer. - Calculate Rx. - Repeat the P1 to P7 for other cores of the cable. |

Knowledge & Understanding

- Define RCD

- Define sensitivity

- Explain P-E and P-N

- Explain the function of RCD.

- Explain the function of distribution box.

- Explain the function of fuse.

- Explain which meters are used for open circuit and short circuits test.

- What does it mean by 0 ohm reading in open circuit test?

- What does it mean by no reading

- In short circuit test?

- Explain the purpose of continuity test.

- Explain the purpose of polarity test.

- Explain the purpose of insulation resistance test.

- Explain the purpose of Murray loop test.

- Explain the purpose of BLAVIER’s loop test.

- Describe why Wheatstone bridge is attached in Murray loop test.

- What is the purpose of DC supply in Murray loop test?

- Explain the purpose of Galvanometer.

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | Sockets |
| 2 | RCD |
| 3 | Loads |
| 4 | Supply source |
| 5 | Distribution Box |
| 6 | Ohmmeter |
| 7 | Earth resistance tester |
| 8 | Continuity |
| 9 | Tester. |
| 10 | Breakers |
| 11 | Megger |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

Perform Murray Loop Test

### 0713E&E10 Repair/ Maintenance of Electrical Installations in Single Phase

Overview:

This Competency Standard identifies the competencies required to Analyze Fault in Electrical Installations, troubleshooting of electrical equipment’s and Carry out Preventive Maintenance. Carry out repair and maintenance of electrical installations at workplace in accordance with the manufacturer’s instructions and organizational requirements.

| Competency Units | Performance Criteria |
| --- | --- |
| - Analyze Fault in Electrical Installations Troubleshooting of electrical equipment’s | - Inspect visually the electrical wiring, fixtures, equipment, soldering, connection, appliances and machinery for discovering the faults and defects - Check the installation for consistency with the electrical drawing - Draw the layout of equipment’s before disassembling of electrical components - Check the faulty components with scope or meter - Re assembled the components as per drawing and the installation - Check the fault indication at relay for HT installation - Test electrical equipment as specified in the manufacturer’s manual and record the results - Prepare a list of items/material(s) required for repair /replacement as per specifications - Draw circuit diagram of electrical equipment be disassembling - Make necessary adjustments in the control and protective switchgear - Replace defective control & protective switch gear, cables and accessories with standard items - Replace defective earth electrode & faulty/damaged earthling conductors - Test installed electrical equipment for safe and optimum performance according to standards & regulations - Record the results of the test performed on a standard format |
| - Carry out Preventive Maintenance | - Perform Preventive maintenance as specified by the manufacturers or SOP - Check out the equipment logging sheet and updates logging - Update Maintenance/service records as per requirement |

Knowledge & Understanding

- The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard.

- Interpretation of layout diagrams, technical sketches, graphic symbols and wiring diagrams, and manufacturer’s specifications etc.

- Types of electrical tools used for troubleshooting and preventive maintenance purposes

- Types of electrical measuring

- instruments used in testing electrical installations

- Types of electrical wiring systems for domestic& industrial purposes

- Methods of tracing the fault

- Types of electrical control and protective switchgear and accessories used in electrical circuits

- Types and principles of operation of circuit breakers used in electrical installations and their applications

- Types of electrical wires and cables and their ratings

- Types of electrical accessories and their application on electrical installations

- Types of earthling systems used in domestic& industrial electrical installations

- Importance of testing electrical installations Importance of corrective & preventive maintenance

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | Specific Gravity Chart |
| 2 | Splicing Machine |
| 3 | Spring tension checking meter |
| 4 | Tachometer |
| 5 | Soldering Lead |
| 6 | Soldering Iron |
| 7 | Set of Screw Drivers |
| 8 | Set of Nose Pliers |
| 9 | Safety Shoes |
| 10 | Ring Spanner Set |
| 11 | RPM Meter |
| 12 | Safety Helm |
| 13 | Safety Goggles |
| 14 | Safety Belt |
| 15 | IR Temperature Gun |
| 16 | Mini Hydraulic Press Machine |
| 17 | OTDR Mete |
| 18 | Multimeter |
| 19 | Overall Combination |
| 20 | Phase Sequence Meter |
| 21 | Philips Screw drivers Set |
| 22 | L Scale |
| 23 | Lux Meter |
| 24 | Lugs Punching Machine (Hydraulic and Manual) |
| 25 | Medgar (Insulation Tester) |
| 26 | Micron Meter |
| 27 | Magnetic Conductor |
| 28 | Ammeter |
| 29 | Hydrometer |
| 30 | Hole saw |
| 31 | Handsaw |
| 332 | Hammer |
| 33 | Hacksaw |
| 34 | Grinder |
| 35 | Battery Charger |
| 36 | Gloves |
| 37 | Generator |
| 38 | Flux |
| 39 | Filler gauge |
| 40 | Files (set) |
| 41 | Earth Tester |
| 42 | Battery Cleaning Ki |
| 43 | Duct Rod |
| 44 | Disk Grinder |
| 45 | Combination Plier Set |
| 46 | Clamp Meter |
| 47 | Circuit Boards |
| 48 | Bearing Puller |
| 49 | Bench Vice |
| 50 | Chisel |
| 51 | Cells tester |
| 52 | Cable Knife |
| 53 | Cable / Wire Gauge |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

  - Trace out the fault and take corrective action

  - Carry out the preventive maintenance

  - Update the service/repair record

### 0713E&E11 Repair/ Maintenance and Service of Home Appliance

Overview:

This Competency Standard identifies the competencies required to Diagnose fault and repair electric iron, Repair/ maintenance of Pedestal Fan and Ceiling Fan, Repair/ maintenance of Juicer and Kitchen and Repair/ maintenance Refrigerator /Air conditioner. Carry out repair and maintenance of home appliances in accordance with the manufacturer’s instructions.

| Competency Units | Performance Criteria |
| --- | --- |
| - Diagnose fault and repair electric iron | - Dismantle the electric Iron for internal tests/servicing/repairs according to manufacturer’s instructions - Check the continuity of wire/switch/protective device by using specified test - Inspect visual mechanical defect such as, loose connection, short circuit, insulation and temperatures. - Check the thermostats by using specified test instruments to detect defects. - Diagnose the faulty area - Clean the parts of the iron with specified cleaning material - Repair/replace the faulty parts of iron as per diagnosed fault. - Re assembled the iron and check iron in test bench as per standard. |
| - Repair/ maintenance of Pedestal Fan and Ceiling Fan. | - Dismantle the fan for internal tests/servicing/repairs according to manufacturer’s instructions - Check the continuity of wire/switch/protective device by using specified test - Inspect visual mechanical defect such as, loose connection, short circuit, insulation and temperatures. - Check the speed and capacitor - Check the winding by using specified test instruments to detect defects. - Diagnose the faulty parts - Clean the parts of the fan with specified cleaning material - Repair/replace the faulty parts of fan as per diagnosed fault. - Rewind the winding - Assembled and test fan as per standard. |
| - Repair/ maintenance of Juicer and Kitchen | - Dismantle the juicer/grinder for internal tests/servicing/repairs according to manufacturer’s instructions - Check the continuity of wire/switch/protective device by using specified test - Inspect visual mechanical defect such as, loose connection, short circuit, insulation and temperatures. - Check the winding by using specified test instruments to detect defects. - Diagnose the faulty components - Clean the parts of the fan with specified cleaning material - Repair/replace the faulty parts as per diagnosed fault. - Rewind the winding if wind is burnt - Re assembled the juicer/grinder and check juicer in test bench as per standard. |
| - Repair/ maintenance Refrigerator /Air conditioner | - Turn off the Refrigerator /Air conditioner - Dismantle the Refrigerator /Air conditioner for internal tests/servicing/repairs according to manufacturer’s instructions. - Inspect visual mechanical defect such as, loose connection, short circuit, insulation and Check the cooling system of air conditioner. - Check the thermostat glitches - check the temperature/heat or condenser coil and identify the problems - Check the Helium gas pressure with pressure gauge - check the leaking or dirty ducts - Check the drainage of water - replace dirty filters - check ducts and thermostat with specified test equipment’s - Diagnose the faulty components - Clean the parts of the fan with specified cleaning material - Repair/replace the faulty parts as per diagnosed fault. - Refill the gas up to required standard (10 to 20 psi) - Before refilling gas create vacuum in compressor or zero gas pressure - Assembled Refrigerator /Air conditioner and performed test bench as per standard. |

Knowledge & Understanding

- Understand Type, Construction and working principal of iron

- Types of electrical measuring instruments used in testing electric iron

- Working principal of thermostat Fundamentals of transformer.

- Understand Type, Construction and working principal of fan

- Types of electrical measuring instruments used in testing electric iron

- Working principal of capacitor

- Understand Type, Construction and working principal of fan

- Types of electrical measuring instruments used in testing electric iron

- Working principal of capacitor

- Understand Construction and working principal of Refrigerator /Air conditioner fan

- Working principal of electrical measuring instruments used in testing Refrigerator /Air conditioner.

- Study and working principal of condenser and compressor

- Study about helium gas

- Study about pressure gauges

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | Electric iron |
| 2 | Fan |
| 3 | Juicer |
| 4 | Kitchen |
| 5 | Refrigerator |
| 6 | Air conditioner |
| 7 | Ammeter |
| 8 | Battery Charger |
| 9 | Battery Cleaning Kit |
| 10 | Bearing Puller |
| 11 | Clamp Meter |
| 12 | Disk Grinder |
| 13 | Combination Plier Set |
| 14 | Bench Vice |
| 15 | Duct Rod |
| 16 | Cable / Wire Gauge |
| 17 | Cable Cutter |
| 18 | Cells tester |
| 19 | Cable Knife |
| 20 | Chisel |
| 21 | Circuit Boards |
| 22 | Earth Tester |
| 23 | Files (set |
| 24 | Filler gauge |
| 25 | Flux |
| 26 | Gloves |
| 27 | Grinder |
| 28 | Hydrometer |
| 29 | Philips Screw Drivers Set |
| 30 | Phase Sequence Meter |
| 31 | Overall Combination |
| 32 | IR Temperature Gun |
| 33 | OTDR Meter |
| 34 | Ring Spanner Set |
| 35 | RPM Meter |
| 36 | All related Safety tools |
| 37 | Set of Nose Pliers |
| 38 | Set of Screw Drivers |
| 39 | Soldering Iron |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Troubleshooting of starting system of Refrigerator /Air conditioner

- Replacement of capacitor and bearings of fan

### 0713E&E12 Perform Cleaning, Oiling and Centering the Job

Overview:

This competency standard covers the skills and knowledge required to perform cleaning and oiling of the lathe machine, perform centering the job by tool method and perform centring the job held in a four jaw chuck of face plate.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Perform Cleaning and Oiling of the Lathe Machine | - Properly clean spindle threads - Properly clean lead screw of machine - Clean oil cap and oil plug using cotton cloth - Properly bed ways, clean cross slide and compound slide - Clean spindle bore and quill of tail stock - Properly lubricate oil caps of machine - Oil spindle threads - Use prescribed grade of oil for oiling and lubrication |
| CU2. Perform Centring the Job By Tool Method | - Prepare a M.S Round Bar as per drawing - Mount four jaw chuck on spindle of lathe machine - Clamp work piece 100mm out of chuck from one end - Clamp cutting tool on tool post of lathe machine - Touch cutting tool on surface of job and rotate chuck with hand to identify out centring - Loosen the opposite side jaw, where the job is out centred - Tighten the chuck in the direction of marked line - Repeat that step if required to centre the job |
| CU3. Perform Centring The Job Held In A Four Jaw Chuck Of Face Plate | - Prepare a M.S Round Bar as per drawing - Mount four jaw chuck on spindle of lathe machine - Clamp work piece 100mm out of chuck from one end - Mount surface gauge on bed ways of lathe machine - Mark lines on surface of job with help of surface gauge to identify out centring - Loosen the opposite side jaw, where the job is out centred - Tighten the chuck in the direction of marked line - Repeat that step if required to centre the job |

Knowledge & Understanding

- Discuss oil grades.

- Explain parts of lathe machine

- Define lubrication

- Define oiling

- What three jaw universal chuck?

- Define how to use surface gauge

- Describe face plate

- How to use four jaw independent chuck?

- Define how to use surface gauge

- Describe face plate

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | Mobile Oil |
| 2 | Kerosene Oil |
| 3 | Tool post key |
| 4 | Steel rule |
| 5 | Chuck key |
| 6 | Steel bar |
| 7 | Wood block |
| 13 | Cotton Glove |
| 14 | Lathe too |
| 18 | Three jaw universal chuck |
| 19 | Four jaw independent |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Perform Centreing The Job Held In A Four Jaw Chuck Of Face Plate

### 0713E&E13 Perform Facing, Straight Turning, Centre Drilling

Overview:

This competency standard covers the skills and knowledge required to perform facing, perform straight turning, and perform centre drilling.

| Competency Units | Performance Criteria |
| --- | --- |
| - Perform Facing | - Prepare a M.S Round Bar as per drawing - Mount universal chuck on spindle of lathe machine - Clamp work piece 25mm out of chuck from one end - Clamp right hand side facing tool on tool post - Check centre height of tool nose - Set spindle speed by considering job dia and material of job - Face the work piece on both sides with help of cross slide - Check the length of job - Perform cleaning operation on machine |
| - Perform Straight Turning | - Prepare a M.S Round Bar as per drawing - Mount universal chuck on spindle of lathe machine - Clamp work piece 25mm out of chuck from one end - Clamp right hand side turning tool on tool post - Check centre height of tool nose - Set spindle speed by considering job dia and material of job - Face the work piece on both sides with help of cross slide - Take a trial cut to straighten the job - Turn off the machine and take measurement of diameter of work piece - Set the cross feed collar to zero - Set appropriate cut and turn the diameter of work piece - Set cutting depth of 0.5mm for finishing the job and complete the job - Measure the diameter of job - Perform cleaning operation on machine |
| - Perform Centre Drilling | - Prepare a M.S Round Bar as per drawing - Mount universal chuck on spindle of lathe machine - Clamp work piece 25mm out of chuck from one end - Clamp right hand side facing tool on tool post - Check centre height of tool nose - Set spindle speed by considering job dia and material of job - Face the work piece on both sides - Clamp centre drill in drilling chuck and mount it in tail stock spindle - Turn on the machine and slowly feed centre drill towards work piece using tail stock hand wheel - Slide tail stock spindle Back and - Forth and complete centre drilling operation - Use cutting oil during operation - Perform cleaning operation on machine |

Knowledge & Understanding

- What is four jaw independent chuck

- Define how to use surface gauge

- Define cutting speed

- Define cutting feed

- Describe angles of facing tool

- Explain types of turning

- Describe centre drilling operation

- Explain purpose of centre drilling

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | Three jaw universal chuck |
| 2 | Speed change lever |
| 3 | Centre drill |
| 4 | Drill chuck |
| 5 | Drill chuck key |
| 6 | Vernier calliper |
| 7 | Tool post key |
| 8 | Steel rule |
| 9 | Chuck key |
| 12 | Right hand Turing tool |
| 14 | Dead centre |
| 16 | Right hand side facing tool |
| 17 | Surface gauge |
| 19 | Speed change lever |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

Perform Facing

### 0713E&E14 Perform Drilling, Step Turning, Knurling and Boring a Straight Hole on lathe

Overview :

This competency standard covers the skills and knowledge required to Perform Drilling On Lathe, Perform Step Turning, Perform Knurling and Perform Boring A Straight Hole.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Perform Drilling On Lathe | - Prepare a M.S Round Bar as per drawing - Mount universal chuck on spindle of lathe machine - Clamp work piece 25mm out of chuck from one end - Set drilling speed by considering material of job - Face the work piece on both sides - Check the angle point of drill - Perform centre drilling operation - Clamp the drill of 5mm dia and create pilot hole - Perform drilling operation as per drawing - Unclamp the job and debar the job - Perform cleaning operation on machine |
| CU2. Perform Step Turning | - Prepare a M.S Round Bar as per drawing - Mount universal chuck on spindle of lathe machine - Clamp work piece 25mm out of chuck from one end - Clamp right hand side turning and facing tool on tool post - Check centre height of both tool nose - Set spindle speed by considering job dia and material of job - Face the work piece on one side - Mount centre drill in tail stock spindle - Perform centre hole drill on one side of job - Face other side of job to get required length - Clamp job as per drawing and support with dead centre - Turn dia of 12mm on the length of 47mm - Turn dia of 17mm on the length of 47mm - Unclamp the job and change the side - Turn dia of 23mm on the length of 47mm - Perform square shoulder turn operation on each step - Unclamp the job and debar the job - Perform cleaning operation on machine |
| CU3. Perform Knurling | - Prepare a M.S Round Bar as per drawing - Mount universal chuck on spindle of lathe machine - Clamp work piece 25mm out of chuck from one end - Clamp knurling tool on tool post - Check centre height of knurling tool - Set spindle speed 75% lower than turning speed - Set knurling tool at 90* with job axis - Set carriage feed at 0.38 to 0.76mm/revolution - Touch knurling tool with job surface and feed 0.63mm towards job - Start the machine and take light cut - Stop the machine, inspect the job and adjust if required - Start the machine on automatic feed and complete the job |
| CU4. Perform Boring A Straight Hole | - Clamp turning and facing tools in tool post - Check center height of both tools - Hold the job in chuck - Face the job on both sides and achieve the length of 48mm - Cut chamfer of 3x45* on both ends of job - Drill a through hole of 10mm - Drill a through hole of 16mm and 25mm - Clamp boring bar in tool post - Move carriage with hand to check if boring bar is not touching the sides of job P10. Set feed of 0.8mm to 1.2mm along longitudinal way P11. Touch boring bit point with internal circumference of job P12. Set appropriate depth of cut and complete the job with automatic feed |

Knowledge & Understanding

- Describe job materials

- Define RPM.

- Describe drilling operation

- Explain purpose of pilot hole drill

- Define step turning

- Define cutting feed

- Describe r.p.m for step turning

- Explain types of turning

- Define types of knurling

- Describe r.p.m for knurling

- Explain purpose of knurling

- Define types of boring

- Describe r.p.m for boring

- Explain purpose of boring bar

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | Three jaw universal chuck |
| 2 | Oil can |
| 3 | Speed change lever |
| 4 | Boring bar |
| 5 | Vernier calliper |
| 6 | Dead centre |
| 7 | Steel rule |
| 8 | Surface gauge |
| 9 | Tool post key |
| 10 | Chuck key |
| 13 | Lever |
| 15 | Knurling tool |
| 20 | Drill chuck |
| 21 | Drill chuck key |
| 22 | Centre drill |
| 23 | Flat file |
| 24 | Drills of required dia |
| 25 | Speed change lever |
| 40 | Right hand side facing tool |
| 41 | Right hand Turing tool |
| 42 | Speed change lever |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

Perform Boring a Straight Hole

### 0713E&E15 Perform Step or Counter Boring, Reaming, Tool Grinding, Taper Turning and Cutting Metric Threads.

Overview:

This competency standard covers the skills and knowledge required to perform step or counter boring, perform reaming, perform tool grinding, perform taper turning by compound rest method and perform cutting metric threads on lathe machine.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Perform Step Or Counter Boring | - Clamp half-length of job outside chuck - Clamp facing tool in tool post - Check center height of tool - Face both ends of job as per drawing - Centre drill one side of job - Drill through hole of dia 6mm - Drill through hole of 12mm - clamp boring bar on tool post - Move carriage with hand to check if boring bar is not touching the sides of job - Set feed of 0.8mm to 1.2mm along longitudinal way - Cut the bore of 16mm throughout hole - Cut the bore dia of 20mm on length of 24mm - Check bore size on both sides |
| CU2. Perform Reaming | - Prepare a M.S Round Bar as per drawing - Mount universal chuck on spindle of lathe machine - Clamp work piece 25mm out of chuck from one end - Set drilling speed by considering material of job - Face the work piece on both sides - Check the angle point of drill - Perform centre drilling operation - Clamp the drill of 5mm dia and create pilot hole - Perform drilling operation using drill of 11.8mm dia - Clamp 12mm dia reamer in drill chuck - Insert reamer in to work piece and use cutting fluid - Turn off the machine and slowly exert reamer |
| CU3. Perform Tool Grinding | P1. Dress the face of grinding wheel P2. Hold tool bit to grind appropriate cutting edge angle P3. Slide tool bit right to left on face of wheel P4. Grind cutting edge angle and end relief angle at same time P5. Check end relief angle P6. Produce minor radius on tool nose |
| CU4. Perform Taper Turning By Compound Rest Method | P1. Clamp work piece 25mm out of chuck from one end and Clamp right hand side turning tool on tool post P2. Check centre height of tool nose P3. Set spindle speed by considering job dia and material of job P4. Face the work piece on both sides with help of cross slide P5. Perform centre hole drill on one side of job P6. Face other side of job to get required length P7. Clamp job as per drawing and support with dead centre P8. Turn whole length of job on 23mm dia P9. Set compound slide on angle of 1*-54’ P10. Set appropriate depth of cut P11. Control feed with compound slide knob and cut taper on job P12.Check the minor dia with help of Vernier calliper |
| CU5. Perform Cutting Metric Threads On Lathe Machine | P1. Clamp work piece 25mm out of chuck from one end and Clamp facing and turning tool on tool post P2. Check centre height of tool nose P3. Face the work piece on both sides with help of cross slide P4. Perform centre hole drill on one side of job P5. Clamp job as per drawing and support with dead centre P6. Turn whole length of job on 22mm dia P7. Set threading tool on 90* with axis of work piece using angle gauge P8. Touch threading tool with job surface and set cross feed collar to zero P9. Set gears on spindle and lead screw as per standard P10. Take appropriate cuts to complete threading P11. Measure the pitch of threading performed |

Knowledge & Understanding

- Define types of counter boring

- Describe r.p.m for counter boring

- Explain purpose of counter boring bar.

- Define types of reamer.

- Describe r.p.m for reaming.

- Explain purpose of reaming

- Define types of wheel dresser.

- Describe r.p.m for grinding

- Explain types of grinder

- Define types of taper turning

- Describe r.p.m for taper turning

- Explain purpose of taper turning

- Explain purpose of Threading

- Describe types of Threads

- Define angle of Thread

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | Three jaw universal chuck |
| 2 | Grinding gauge |
| 3 | Tool bit |
| 4 | Oil stone |
| 5 | Wheel dresser |
| 6 | Chuck key |
| 7 | Pedestal grinder |
| 8 | Speed change lever |
| 9 | Dead centre |
| 1 | Reamer Dia 12mm |
| 11 | Oil can |
| 12 | Vernier calliper |
| 13 | Surface gauge |
| 14 | Steel rule |
| 15 | Tool post key |
| 16 | Chuck key |
| 17 | Speed change lever |
| 18 | angle gauge |
| 20 | Oil can |
| 22 | Boring bar |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Perform Knurling

### 0713E&E16 Perform Basic Metal Work

Overview:

This competency standard covers the skills and knowledge required to Develop Name Plate manually, Carry out Sawing, prepration of Inside Calliper, prepration of Bottle Opener, Preparation of Dove Tail Joint, Prepration of small size Tri Square, Produce Threads on Work Piece, prepration of Funnel, prepration of Drawer Handle, Produce Pipe Threads, prepration of spanner(small size).

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Develop Name Plate manually | P1. Select marking tools P2. Cut sheet as per drawing P3. Perform surface finishing with file P4. Level the surface with tri-square P5. Mark the plate as per name requirements P6. Punch the marked area P7. Perform finishing with sand paper |
| CU2. Carry out Sawing | P1. Mark the job according to given drawing P2. Select appropriate blade according to job requirement P3. Set blade in frame of hacksaw as per procedure P4. Ensure the work piece is clamped firmly and properly P5. Adopt methods and techniques for sawing that is appropriate to job requirement P6. Follow marked line during sawing to ensure accuracy. |
| CU3. Preparation of Inside Calliper | P1. Cut sheet as per drawing P2. Mark the job according to given drawing P3. Adopt appropriate methods and techniques of filing P4. Perform bending operation using hammer and bench vice. P5. Perform drilling operation using twist drill according job drawing P6. Perform riveting operation using cross peen hammer P7. Perform finishing with sand paper |
| CU4. Preparation of Bottle Opener | P1. Select marking tools P2. Cut sheet as per drawing P3. Perform surface finishing with file P4. Level the surface with tri-square P5. Mark radius as per drawing P6. Develop radius as per drawing P7. Make the notch with round file P8. Perform finishing with sand paper |
| CU5. Preparation of Dovetail Joint | P1. Select marking tools P2. Cut sheet as per drawing P3. Perform surface finishing with file P4. Level the surface of both work pieces with tri-square P5. Mark both work pieces according to drawing P6. Create outer notch on work piece using flat file and hacksaw P7. Create inner notch using hacksaw and chisel P8. Compare both pieces by inserting outer notch into inner notch P9. Perform finishing with sand paper. |
| CU6. Prepration of small size Tri Square | P1. Select marking tools P2. Cut sheet as per drawing P3. Perform surface finishing with file P4. Level the surface of both work pieces with tri-square P5. Mark both work pieces according to drawing P6. Cut 45* on one end of both work pieces and file accordingly P7. Create a narrow cut in handle to fit blade P8. Drill a hole in handle as per drawing and rivet that hole P9. Check the firm ability of handle and blade P10. Perform finishing with sand paper. |
| CU7. Produce Threads on Work Piece | P1. Identify different kind of taps & die according to requirement P2. Identify the work piece clamping method. P3. Apply tap and die alignment. P4. Apply lubricants while threading. P5. Avoid unwanted engraving and slips. P6. Identify proper threading procedure |
| CU8. Preparation of Funnel | P1. Select marking tools P2. Cut sheet as per drawing P3. Perform surface finishing with file P5. Mark the sheet according to drawing P6. Cut the sheet with hand shear P7. Create radius of funnel using appropriate tools P8. Perform flat lock seam bend using bench vice P9.Perform finishing with sand paper. |
| CU9. Preparation of Drawer Handle | P1. Select marking tools P2. Cut MS Round as per drawing P3. Mark the round bar according to drawing P4. Bend the bar using bench vice according given radius P5. Check the radius on both ends of bar P6.Perform finishing with sand paper. |
| CU10. Produce Pipe Threads | P1. Select marking tools P2. Cut pipe as per drawing P3. Select die as per pipe size P5. Set die into die holder P6. Select relevant vice for pipe clamping P7. Perform pipe threading using appropriate method P8.Perform finishing with sand paper. |
| CU11. Preparation of spanner(small size) | P1. Select marking tools P2. Cut sheet as per drawing P3. Perform surface finishing with file P4. Level the surface with tri-square P5. Mark radius as per drawing P6. Develop radius as per drawing P7. Make the notch with round file P8. Perform finishing with sand paper |

Knowledge & Understanding

- Define basic measurement

- Describe basic measuring /Marking /cutting tools

- Describe clamping/holding methods

- Describe basic measurement

- Describe types of hacksaw frames

- Describe basic measuring Marking /cutting tools

- Describe clamping/holding methods

- Define methods and techniques of sawing.

- Define types of hammers

- Describe riveting methods

- Describe the drilling procedure

- Explain drilling machine

- Describe basic measurement

- Describe basic measuring /Marking /cutting tools

- Describe clamping/holding methods

- Describe use of round file

- Describe basic measurement

- Describe basic measuring /Marking /cutting tools

- Describe types of chisels

- Understanding of chiseling process

- Understanding of types of fits

- Describe clamping/holding methods

- Understanding of chiseling process

  - Describe use of bevel protector

- Knowledge of different kind of taps & die according to requirement

- Knowledge of calculation for drill size for internal threading

- Understanding proper use of hand shear

  - knowledge of flat lock seam  end in metal sheet working

- Describe clamping/holding methods

- Knowledge of inch system measurement

- Describe basic measurement

- Describe basic measuring /Marking /cutting tools

- Describe clamping/holding methods

- Knowledge of radius gauge

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | Work bench |
| 2 | Bench vice |
| 3 | Hammer |
| 4 | Tri-square |
| 5 | Hand hacksaw |
| 6 | Scriber |
| 7 | Vernier calliper |
| 8 | Flat File |
| 9 | Number/alphabet punch |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

### 0713E&E17 Perform Oxy Acetylene Welding

Overview:

This competency standard covers the skills and knowledge required to make flame for gas welding Carburizing flame, neutral flame, oxidizing flame, Pool making, Bead Making, Edge Joint, Open square butt joint (M.S Flat 3mm OR 5mm Thickness), make ‘V’ Groove

Butt joint (Flat Position), Corner joint, Open square brazing Butt joint (M.S Flat 5mm thick).

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Make flame for gas welding Carburizing flame Neutral flame Oxidizing flame | P1. Open gas cylinder with the help of cylinder key P2. Adjust pressure of both gas cylinders with the help of regulator P3. Open acetylene gas knob of welding torch P4. Ignite acetylene gas with help of spark lighter P5. Open oxygen gas knob of welding torch P6. Make carburizing flame by increasing acetylene gas quantity P7. Make neutral flame by adjusting both gases at same quantity P8. Make oxidizing flame by increasing oxygen gas quantity P9. Close valves of welding torch P10. Close both gas cylinders’ valves |
| CU2. Make Pool | P1. Open gas cylinder with the help of cylinder key P2. Adjust pressure of both gas cylinders with the help of regulator P3. Select the correct size of the nozzle P4. Set the both gas flame of welding torch as per standard P5. Make the pool as per standard Close welding torch valve P6. Close both gas cylinders’ valves P7. Clean with wire brush |
| CU3. Make Bead | P1. Open gas cylinder with the help of cylinder key P2. Adjust pressure of both gas cylinders with the help of regulator P3. Select the correct size of the nozzle P4. Set the both gas flame of welding torch as per standard P5. Draw the straight line on the job P6. Keep appropriate distance between end of the inner core and job surface P7. Establish the weld pool P8. Add filler rod to the pool P9. Slowly withdraw the flame near the left hand edge of the shield P10. Finish the job with wire brush |
| CU4. Make Edge Joint | P1. Perform marking as per drawing P2. Cut the metal sheet according to drawing using shearing machine P3. Straighten the material with help of hammer P4. File work if required P5. Open gas cylinder with the help of cylinder key P6. Adjust pressure of both gas cylinders with the help of regulator P7. Select the correct size of the nozzle P8. Set the both gas flame of welding torch as per standard P9. Arrange filler material as per requirement P10. Set both plates at edge joint position P11. Weld both plate using filler rod P12. Finish the job with wire brush |
| CU5. Make Open square butt joint (M.S Flat 3mm OR 5mm Thickness) | P1. Perform marking as per drawing P2. Cut the metal sheet according to drawing using shearing machine P3. Straighten the material with help of hammer P4. File work if required P5. Open gas cylinder with the help of cylinder key P6. Adjust pressure of both gas cylinders with the help of regulator P7. Select the correct size of the nozzle P8. Set the both gas flame of welding torch as per standard P9. Arrange filler material as per requirement P10. Set gap in both plates according to joint P11. Weld both plate using filler rod P12. Finish the job with wire brush |
| CU6. Make ‘V’ Groove butt joint ( Flat Position) | P1. Perform marking as per drawing P2. Cut the metal sheet according to drawing using shearing machine P3. Straighten the material with help of hammer P4. Open gas cylinder with the help of cylinder key P5. Adjust pressure of both gas cylinders with the help of regulator P6. Set the both gas flame of welding torch as per standard P7. Arrange filler material as per requirement P8. Prepare 45* angle on edge of metal plate according to drawing P9. Align edges of both plates according to drawing P10. Weld both plate using filler rod P11. Finish the job with wire brush |
| CU7. Make Corner joint | P1. Perform marking as per drawing P2. Cut the metal sheet according to drawing using shearing machine P3. Straighten the material with help of hammer P4. File work if required P5. Open gas cylinder with the help of cylinder key P6. Adjust pressure of both gas cylinders with the help of regulator P7. Select the correct size of the nozzle P8. Set the both gas flame of welding torch as per standard P9. Arrange filler material as per requirement P10. Set both plates at corner joint position P11. Weld both plate using filler rod P12. Finish the job with wire brush |
| CU8. Make Open square brazing Butt joint(M.S Flat 5mm thick) | P1. Perform marking as per drawing P2. Cut the metal sheet according to drawing using shearing machine P3. Straighten the material with help of hammer P4. File work if required P5. Open gas cylinder with the help of cylinder key P6. Adjust pressure of both gas cylinders with the help of regulator P7. Select the correct size of the nozzle P8. Set the both gas flame of welding torch as per standard P9. Arrange brazing rod and flux as per requirement P10. Set both plates at corner joint position P11. Weld both plate using filler rod P12. Finish the job with wire brush |

Knowledge & Understanding

- Knowledge about back fire arrestor

- Knowledge about flash back arrestor

- Describe the pressure and its units

- Explain metal properties

- Explain temperature and its units

- Knowledge of basic drawing

- Explain marking tools

- Explain the of marking methods

- Describe the filler rod

- Knowledge of basic drawing

- Explain marking tools

- Explain the of marking methods

- Describe the filler rod

- Explain kinds of joints

- Describe joint symbols

- Knowledge about edge joint

- Knowledge of basic drawing

- Explain marking tools

- Explain the of marking methods

- Describe the filler rod

- Explain kinds of joints

- Describe joint symbols

- Knowledge about edge joint

- Knowledge of shearing machine

- Knowledge of square butt joint

- Explain the of marking methods

- Describe the filler rod

- Explain kinds of joints

- Describe joint symbols

- Knowledge about ‘V’ groove butt joint

- Knowledge of grinding machine

- Kinds of grinding machines

- Explain the of marking methods

- Describe the filler rod

- Explain kinds of joints

- Describe joint symbols

- Knowledge about corner joint

- Knowledge of grinding machine

- Kinds of grinding machines

- Explain the of marking methods

- Describe the brazing filler rod

- Explain kinds of joints

- Describe joint symbols

- Knowledge about soldering, brazing and its flux

- Knowledge of grinding machine

- Kinds of grinding machines

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | Oxygen cylinder |
| 2 | Acetylene gas cylinder |
| 3 | Pressure regulators |
| 4 | Cylinder key |
| 5 | Welding torch |
| 6 | Rubber house pipe |
| 7 | Back fire arrester |
| 8 | Flash back arrester |
| 9 | Spark lighter |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

Make Open square brazing Butt joint (M.S Flat 5mm thick)

### 0713E&E18 Perform Wood Work

Overview:

This competency standard covers the skills and knowledge required to planning and squaring to dimensions, Sawing Exercise, Making Mortise and Tendon Joint, Making Dado Joint, Making Cross-lap Joint, making holes of different Diameters in Wood, Nailing and Wood Screwing Process, Making Middle Half Cross lap joint, Making Dovetail Joint.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Planning and Squaring to dimensions | P1. Select appropriate wooden piece according to drawing P2. Select marking tools P3. Select relevant vice for wood clamping P4. Ensure the work piece is clamped properly P5. Mark work pieces according to given dimensions in drawing P6. Perform planning operation using jack plane P7. Insure squaring of work piece by using tri square P8. Perform finishing with sand paper |
| CU2. Sawing Exercise | P1. Mark the job according to given drawing P2. Select appropriate blade according to job requirement P4. Ensure the work piece is clamped firmly and properly P5. Adopt methods and techniques for sawing that is appropriate to job requirement P6. Follow marked line during sawing to ensure accuracy. |
| CU3. Making Mortise and Tendon Joint | P1. Select appropriate wooden piece for mortise P2. Select appropriate wooden piece for Tendon P3. Select marking tools P4. Select relevant vice for wood clamping P5. Ensure the work piece is clamped properly P6. Mark work pieces according to given dimensions in drawing P7. Prepare Mortise piece using wooden saw P8. Prepare Tendon piece using Tendon machine or using wooden chisel P9. Check joint firm ability by clamping both work pieces P10. Perform finishing with sand paper |
| CU4. Making Dado Joint | P1. Mark the job according to given drawing P2. Select appropriate blade according to job requirement P4. Ensure the work piece is clamped firmly and properly P5. Mark width using the actual piece. P6. Make a "knife wall" with a chisel P7. Saw down the knife wall lines P8. Work the way across the waste P9. Fit and glue P10. Perform finishing with sand paper |
| CU5. Making Cross-lap Joint | P1. Select appropriate wooden piece for upper lap P2. Select appropriate wooden piece for lower lap P3. Select marking tools P4. Select relevant vice for wood clamping P5. Ensure the work piece is clamped properly P6. Mark work pieces according to given dimensions in drawing P7. Cut the notch using wood saw P8. Finish the notch using wood chisel P9. Check joint firm ability by clamping both work pieces P10. Perform finishing with sand paper |
| CU6. Making holes of different Diameters in Wood | P1. Select appropriate wooden piece according to drawing P2. Select marking tools P3. Select relevant machine vice for wood clamping P4. Ensure the work piece is clamped properly P5. Mark work piece according to given dimensions in drawing P6. Perform planning operation using jack plane if required P7. Select drills according to drawing P8. Perform drilling operation according to drawing P9. Check diameter using plug gauge P10. Perform finishing with sand paper |
| CU7. Nailing and Wood Screwing Process | P1. Select appropriate wooden piece according to drawing P2. Select marking tools P3. Select relevant vice for wood clamping P4. Ensure the work piece is clamped properly P5. Mark work piece according to given dimensions in drawing P6. Join two pieces together with metal nail using claw hammer P7. Join two pieces together with flaw head screw using flat screw driver P8. Join two pieces together with Philip head screw using Philip screw driver P9.Perform finishing with sand paper |
| CU8. Making Middle Half Cross lap joint | P1. Select appropriate wooden piece for upper lap P2. Select appropriate wooden piece for lower lap P3. Select marking tools P4. Select relevant vice for wood clamping P5. Ensure the work piece is clamped properly P6. Mark work pieces according to given dimensions in drawing P7. Cut the notch in middle of work pieces using circular saw P8. Finish the notch using wood file P9. Check joint firm ability by clamping both work pieces P10. Perform finishing with sand paper |
| CU9. Making Dovetail Joint | P1. Select appropriate wooden piece for Tail P2. Select appropriate wooden piece for Pin P3. Select marking tools P4. Select relevant vice for wood clamping P5. Ensure the work piece is clamped properly P6. Mark work pieces according to given dimensions in drawing P7. Prepare Tail piece using hand saw and wooden chisel P8. Prepare Pin piece using hand saw and wooden chisel P9. Check joint firm ability by clamping both work pieces P10. Perform finishing with sand paper |

Knowledge & Understanding

- Describe basic measurement

- Describe basic measuring /Marking /cutting tools

- Describe clamping/holding methods

- Knowledge of jackplane types

- Describe use of tri square

- Knowledge of squaring the work piece

- Describe basic measurement

- Describe types of wood working saw

- Describe basic measuring /Marking /cutting tools

- Describe clamping/holding methods

- Define methods and techniques of sawing.

- Describe basic measurement

- Describe basic measuring /Marking /cutting tools

- Describe clamping/holding methods

- Knowledge of wooden chisels

- Describe use of Tendon machine

- Knowledge of squaring the work piece

- Describe fits in joints

- Describe basic measurement

- Describe basic measuring /Marking /cutting tools

- Describe clamping/holding methods

- Knowledge of wooden chisels

- Describe application of Dado Joints

- Knowledge of glues use in wood work

- Describe fits in joints

- Describe basic measurement

- Describe clamping/holding methods

- Knowledge of wooden chisels

- Describe use of wood saw

- Describe fits in joints

- Describe basic measurement

- Describe basic measuring /Marking /cutting tools

- Describe clamping/holding methods

- Knowledge of Drill Machines

- Differentiate different types of drill

- Describe basic measurement

- Describe basic measuring /Marking /cutting tools

- Describe clamping/holding methods

- Knowledge of types of nails

- Knowledge of types of screws

- Differentiate between nail and screw

- Describe basic measurement

- Describe basic measuring /Marking /cutting tools

- Describe clamping/holding methods

- Knowledge of wooden files

- Describe use of circular saw

- Describe fits in joint

- Describe basic measurement

- Describe basic measuring /Marking /cutting tools

- Describe clamping/holding methods

- Knowledge of wooden chisels

- Describe application of Dove tail joint

- Describe fits in joints

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | Work bench |
| 2 | Punching tools |
| 3 | Vernier caliper |
| 4 | Scriber |
| 5 | Wood jackplane |
| 6 | Flat File |
| 7 | Tri-square |
| 8 | Wood Marking tools |
| 9 | Bench vice |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

### 0713E&E19 Perform Arc Welding

Overview:

This competency standard covers the skills and knowledge required to make Arc, Bead Making, Open Square Butt Joint (MS Flat 5mm Thick), V Groove Butt Joint, Lap Joint, Corner Joint Flat Position, Corner Joint Vertical Position, Spot Welding Practice(0.5mm MS Sheet), Seam Welding Practice(0.5mm MS Sheet).

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Make Arc | P1. Take M.S Flat as per drawing P2. Straiten it with the help of hammer and anvil P3. Place the work piece on welding table P4. Hold the electrode in electrode holder P5. Set the current on welding machine as per standard P6. Touch the electrode with base metal P7. Ensure that pre heating is accomplished by moving short arc rapidly back and forward P8. Remove the slag with chipping hammer clean with wire brush |
| CU2. Make Bead | P1. Take M.S Flat as per drawing P2. Straiten it with the help of hammer and anvil P3. Place the work piece on welding table P4. Hold the electrode in electrode holder P5. Set the current on welding machine as per standard P6. Position the electrode leaning about 20* from vertical in the direction of travel P7. Prepare the arc by touching the end of electrode with the base metal and withdrawing it to proper gap P8. Ensure the width and length of arc by watching the puddle P9. Remove the slag with chipping hammer P10. Clean with wire brush |
| CU3. Make Open Square Butt Joint (MS Flat 5mm Thick) | P1. Take M.S Flat as per drawing P2. Straiten it with the help of hammer and anvil P3. Place the work piece on welding table P4. Hold the electrode in electrode holder P5. Set the current on welding machine as per standard P6. Hold the electrode at right angle P7. Set the current at 120-125 ampere P8. Tack both pieces of M.S Flat P9. Prepare the arc by touching the end of electrode with the base metal and withdrawing it to proper gap P10. Lean the electrode at 70* to the base metal and complete the bead to form an open square butt joint P11. Remove the slag with chipping hammer clean with wire brush |
| CU4. Make V Groove Butt Joint | P1. Take M.S Flat as per drawing P2. Straiten it with the help of hammer and anvil P3. Place the work piece on welding table P4. Hold the electrode in electrode holder P5. Set the current on welding machine as per standard P6. Grind the work pieces on pedestal grinder as per drawing P7. Place both work pieces on welding table with equal distance and parallel to each other P8. Tack both work pieces of M.S Flat P9. Set the current to 200 amperes P10. Prepare an arc by touching the end of electrode with base metal P11. Complete the bead by back hand welding technique P12. Remove the slag with chipping hammer P13. Clean with wire brush |
| CU5. Make Lap Joint | P1. Take M.S Flat as per drawing P2. Straiten it with the help of hammer and anvil P3. Mark parallel line with the help of scriber for positioning of overlap the plate P4. Place the work piece on welding table P5. Place the Bottom piece on work table and place the Top plate along the marked line P6. Hold the electrode in electrode holder P7. Set the current on welding machine as per standard P8. Tack both work pieces of M.S Flat P9. Set the current to 200 amperes P10. Complete the bead by back hand welding technique P11. Remove the slag with chipping hammer P12. Clean with wire brush |
| CU5. Make Corner Joint Flat Position | P1. Take M.S Flat as per drawing P2. Straiten it with the help of hammer and anvil P3. Grind the work pieces on grinding machine to prepare the edges flat and parallel to each other P4. Place the Bottom piece on work table and place the Top plate at 90* to each other as per drawing P5. Hold the electrode in electrode holder P6. Set the current on welding machine as per standard P7. Tack both work pieces of M.S Flat P8. Lean the electrode and move along traverse direction P9. Remove the slag with chipping hammer P10. Clean with wire brush |
| CU6. Make Corner Joint Vertical Position | P1. Take M.S Flat as per drawing P2. Straiten it with the help of hammer and anvil P3. Grind the work pieces on grinding machine to prepare the edges flat and parallel to each other P4. Place the Bottom piece on work table and place the other piece vertically edge wise on it as per drawing P5. Hold the electrode in electrode holder P6. Set the current on welding machine as per standard P7. Check the squares of joint with try square. P8. Tack both work pieces of M.S Flat P9. Lean the electrode 20* downward and complete the joint by back hand technique P10. Remove the slag with chipping hammer P11. Clean with wire brush |
| CU7. Make Spot Welding Practice(0.5mm MS Sheet) | P1. Take M.S Sheet as per drawing P2. Straiten it with the help of rubber hammer and anvil P3. Mark parallel line with the help of scriber for positioning of overlap the plate P4. Grip the both part in Fixture P5. Switch on the spot welding machine and set the timer P6. Place the work piece on the lower electrode P7. Press down the pedal to make contact between the copper electrode to form spot weld for duration of 02 seconds P8. Perform this practice on the entire job as per drawing |
| CU8. Make Seam Welding Practice(0.5mm MS Sheet) | P1. Take M.S Sheet as per drawing P2. Straiten it with the help of rubber hammer and anvil P3. Check the squares of both work pieces P4. Mark parallel line with the help of scriber for positioning of overlap the plate P5. Grip the both part in Fixture P6. Switch on the seam welding machine and set the timer P7. Switch the motor to drive the electrode wheels P8. Place the work piece on the lower electrode P9. Press down the pedal to make contact between both electrode wheels P10. Perform this practice on the entire job as per drawing |

Knowledge & Understanding

- Knowledge about electricity, current, ampere, voltage, wattage etc.

- Knowledge offsetting the current on welding machine

- Understand types of electrodes

- Understand motion of electrodes

- Define temperature of arc

- Explain types of temperature

- Explain positions of electrode according to work

- Knowledge of setting the current on welding machine

- Describe Bead in Arc welding

- Explain importance of gap between electrode and base metal

- Explain positions of electrode according to work

- Knowledge of setting the current on welding machine

- Describe Bead in Arc welding

- Explain importance of gap between electrode and base metal

- Define Butt Joint

- Define Lap Joint

- Explain positions of electrode according to work

- Knowledge of setting the current on welding machine

- Describe Bead in Arc welding

- Explain importance of gap between electrode and base metal

- Define Butt Joint

- Explain use on pedestal grinder

- Identify the factors on which the current setting base upon

- Explain positions of electrode according to work

- Knowledge of setting the current on welding machine

- Describe motion of electrode in ARC welding

- Explain importance of gap between electrode and base metal

- Describe use of tri square

- Explain positions of electrode according to work

- Knowledge of setting the current on welding machine

- Describe motion of electrode in ARC welding

- Explain importance of gap between electrode and base metal

- Describe use of tri square

- Describe importance of cleanliness of surface to be welded

- Explain positions of electrode according to work

- Knowledge of setting the current on welding machine

- Describe motion of electrode in ARC welding

- Explain importance of gap between electrode and base metal

- Describe use of tri square

- Describe back hand welding technique

- Explain properties of copper electrode

- Knowledge of setting the current on spot welding machine

- Knowledge of setting the timer on spot welding machine

- Explain importance of pressure and timing between electrode and base metal

- Define conductivity

- Explain Resistant welding

- Explain properties of copper electrode

- Knowledge of setting the current on

- seam welding machine

- Knowledge of setting the timer on seam welding machine

- Explain the purpose of seam welding

- Describe types of electrodes used in seam welding

- Explain Resistant welding

- Importance of cooling for seam welding

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | Welding transformer |
| 2 | Welding rectifier |
| 3 | Extension leads |
| 4 | Chipping Hammer |
| 5 | Wire brush |
| 6 | Face shield |
| 7 | Electrode holder |
| 8 | Generator |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

Perform Seam Welding (0.5mm MS Sheet)