National Vocational Diploma Level 5 in Electrical Technology

(Electrical Supervisor)

Competency Standards (CS) Document

National Vocational and Technical Training Commission (NAVTTC)

Government of Pakistan

Table of Contents

Introduction	5

Purpose of the Qualification	6

Common Courses with respective levels	7

Levelling of Core Competencies of the Qualification along with the occupations	8

Packaging of Occupations	12

Mapping of the Qualifications	15

Date of Validation	16

Date of Review	16

Codes of Qualifications	16

Members of Qualifications Development Committee	17

Members of Qualification Validation Committee	18

Entry Requirements	18

Detail of Competency Standards	19

0713E&E1Develop and Analyse Electrical Circuit with MULTISM	19

0713E&E2 Develop and Analyze Electrical Circuit with PSPICES Software.	24

0713E&E3 Carry Out in Computer Electrical Planning and Estimation	28

0713E&E4 Carry Out Industrial Automation and PLC Installation	32

0713E&E5 Verify Basic Laws of Electrical Machines	36

0713E&E6 Analyze an Alternator	39

0713E&E7 Analyze Single Phase Motors.	42

0713E&E8 Analyze Special Purpose Motors.	44

0713E&E9 Analyze Three Phase Motors.	47

0713E&E10 Repairs/Maintenance and Service of Electric Motors and Generators	54

0713E&E11 Identify Basic Electronics Components	59

0713E&E12 Design a Rectifier Using Diode	62

0713E&E13Carry Out Diode Types and their Applications	65

0713E&E14 Implement Bipolar Junction Transistor (BJT) in Different Applications	67

0713E&E15 Implement Field Effect Transistor (FET) In Different Applications	71

0713E&E16 Apply (Uni Junction Transistor, Silicon Control Rectifier, Diac and Triac) in Various Application	75

0713E&E17 Design Operation Amplifier	78

0713E&E18Verify Truth Tables of Digital Gates	81

0713E&E19Construct & Verify Combinational Logic Circuit	84

0713E&E20 Construct and Verify Function of Flip Flops	88

0713E&E21 Use 555 IC as Multivibrator	91

0713E&E22 Construct Shift Registers and Counters with The Help of Flip Flops	93

0713E&E23 Implement Diode and Thyristor in Power Control Application.	95

0713E&E24 Implement Single Phase and Three Phase Inverter	98

0713E&E25 Control speed of DC/AC Motor	100

0713E&E26 Program PLC Using Ladder Logic	104

0713E&E27 Carry out Industrial Automation and PLC Installation	107

0713E&E28 Install PLC Software and Simulator	115

0713E&E29 Design and Test the PLC.	119

0713E&E30 Installation, Commissioning and Maintenance of PLC	123

0713E&E31 Identify the Parts of Analog & Digital Telephone Set & Verify their Function	127

0713E&E32 Demonstrate Modulation, Demodulation, Multiplexing & De Multiplexing of Information Signal	130

0713E&E33 Install Cord Less Telephone, PABX & Satellite Dish System.	134

0713E&E34 Install Power Supply	137

0713E&E35 Maintain Lead Acid Batteries and Implement Their Series Parallel Combination.	142

0713E&E36 Operate Hydro Power Plant	145

0713E&E37 Operate Thermal Power Plant	149

0713E&E38 Operate Diesel Power Plant	155

0713E&E39 Operate Nuclear Power Plant	160

0713E&E40 Design Solar System.	164

0713E&E41 Operate Solar Power Plant	168

0713E&E42 Operate Wind Power Plant	172

0713E&E43 Utilize Electrical Energy in Building	176

0713E&E44 Calculate Tariffs and Economics	179

0713E&E45 Carryout Transmission of Electrical Power	182

0713E&E46 Carryout Distribution of Electrical Power	185

0713E&E47 Carryout Protection of Distribution and Transmission System	189

0713E&E48 Investigate Micro Business Opportunities	193

0713E&E49 Develop a Micro Business Proposal	197

0713E&E50 Develop a Marketing Plan	200

0713E&E51 Develop and Review a Business Plan.	204

0713E&E52 Organize Finances for the Micro Business	207

0713E&E53 Manage Human Resources	210

0713E&E54 Market Products and Services	214

0713E&E55 Monitor and Review Business Performance	218

0713E&E56 Negotiate for Resolving Business Issues	221

0713E&E57 Manage Personal Finances	223

0713E&E58 Coordinate a Work Team	226

0713E&E159 Lead Small Teams	229

0713E&E60 Utilize Natural Energy in the Building	233

# Introduction

The Technical and Vocational is a profession that is increasingly getting attention in Pakistan, not only among the youth seeking to enter the industry but also among adults who wish to polish their skills to develop a career out of it. Scope and demand of Electrical engineers, supervisors, operator, and technician is very high in Pakistan due to fast growing economic zone like CPEC. There are more job opportunities for electrical technician and supervisors in Pakistan than any other field of engineering. Pakistan is still facing energy crises, for need for more power houses, renewable energy sources is being felt which will definitely result in more demand of electrical engineers.

CBT based level 1-5 course in Electrical Technology with updated concepts, not only allows students to equip with growing Industrial trends but will be beneficial in capturing the job market across the globe. The course is divided into sub fields like electronics, computer engineering, power engineering, telecommunications, control systems and microelectronics in respective levels. Many of these sub disciplines overlap with other engineering branches, spanning a huge number of specializations such as hardware engineering, power electronics, avionics, mechatronics and nanotechnology.

Pakistan can boost economic growth and job creation by overcoming inefficiencies in its power sector, “Reforms that address these distortions can make better use of existing facilities. These need to focus on eliminating waste, promoting the shift towards cleaner energy and attracting private investments.”

“Power sector reform should be a top priority, as few other reforms could yield economic gains of a similar magnitude so quickly, based on findings from this report,” says Fan Zhang, Senior Economist at the World Bank. “If well designed, these reforms will directly benefit the poor by increasing access, improving reliability, and reducing cost and pollution.”

# Purpose of the Qualification

Based upon this demand of industry these competency‐based qualifications for Electrical Technology are developed under National Vocational Qualification Framework (Level 1 to 5). The qualifications mainly cover competencies along with related knowledge and professional attitude which is essential for getting a job or self-employed.

The qualifications are also in line with the vision of Pakistan’s National Skills Strategy (NSS), National TVET Policy and National Vocational Qualification Framework (NVQF). This provides policy directions, support and an enabling environment to the public and private sectors to impart training for skills development to enhance social and economic profile. The National Competency Standards could be used as a referral document for the development of curricula to be used by training institutions.

The purpose of the training is to provide the basic skills to the trainees in the field of Electrical and convert it into value added product which is acceptable by International market reducing the line losses and fit-in a skilled graduate into National Vocational Qualification Framework for his / her vertical career progression and qualification equivalencies at par with acceptable international standards.

# Common Courses with respective levels

LEVEL 3

LEVEL 2

LEVEL 4

LEVEL 5

# Levelling of Core Competencies of the Qualification along with the occupations

| Competency Standards | Occupation | NVQF Level | Category | Estimated Contact Hours |  |  | Cr Hr |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Th | Pr | Total |  |
| Level 5 |  |  |  |  |  |  |  |
| Develop and Analyze Electrical Circuit with Multisim | Computer Applications for Electrical Technology | Level 5 | Technical | 9 | 12 | 21 | 2.1 |
| Develop and Analyze Electrical Circuit with PSPICE Software |  | Level 5 | Technical | 9 | 12 | 21 | 2.1 |
| Carry Out in Computer Electrical Planning and Estimation |  | Level 5 | Technical | 9 | 12 | 21 | 2.1 |
| Carry out Industrial Automation and PLC Installation |  | Level 5 | Technical | 9 | 12 | 21 | 2.1 |
| Verify Basic Laws of Electrical Machines | Motor Winding Supervisor | Level 4 | Technical | 9 | 12 | 21 | 2.1 |
| Analyze an Alternator. |  | Level 4 | Technical | 9 | 12 | 21 | 2.1 |
| Analyze Single Phase Motors |  | Level 4 | Technical | 9 | 12 | 21 | 2.1 |
| Analyze Special Purpose Motors |  | Level 4 | Technical | 9 | 12 | 21 | 2.1 |
| Analyze Three Phase Motors |  | Level 4 | Technical | 9 | 12 | 21 | 2.1 |
| Repair/ Maintenance and Service of Ac Electric Motors and Generators. |  | Level 4 | Technical | 7 | 12 | 19 | 1.9 |
| Identify Basic Electronics Components | Electronics Technician | Level 3 | Technical | 9 | 9 | 18 | 1.8 |
| Design a Rectifier using Diode |  | Level 4 | Technical | 9 | 9 | 18 | 1.8 |
| Carry out Diode types and their Applications |  | Level 3 | Technical | 9 | 9 | 18 | 1.8 |
| Implement Bipolar Junction Transistor (BJT) in Different Applications |  | Level 3 | Technical | 9 | 9 | 18 | 1.8 |
| Implement Field Effect Transistor (FET) in Different Applications |  | Level 3 | Technical | 9 | 9 | 18 | 1.8 |
| Implement Thyristor Family in Various Application |  | Level 3 | Technical | 9 | 9 | 18 | 1.8 |
| Design Operation Amplifier |  | Level 4 | Technical | 9 | 9 | 18 | 1.8 |
| Verify Truth Tables of Digital Gates. |  | Level 3 | Technical | 9 | 12 | 21 | 2.1 |
| Construct & Verify Combinational Logic Circuit. |  | Level 4 | Technical | 9 | 15 | 24 | 2.4 |
| Construct and Verify Function of Flip Flops. |  | Level 4 | Technical | 9 | 18 | 27 | 2.7 |
| Use 555 IC as Multi-vibrator. |  | Level 4 | Technical | 9 | 18 | 27 | 2.7 |
| Construct Shift Registers and Counters Used Flip Flops |  | Level 4 | Technical | 9 | 18 | 27 | 2.7 |
| Implement Diode and Thyristor in Power Control Application. | Industrial Supervisor | Level 5 | Technical | 9 | 18 | 27 | 2.7 |
| Implement Single Phase and Three Phase Inverter |  | Level 5 | Technical | 9 | 18 | 27 | 2.7 |
| Control Speed of Dc/Ac Motors. |  | Level 5 | Technical | 9 | 18 | 27 | 2.7 |
| Program PLC using Ladder Logic. |  | Level 5 | Technical | 9 | 18 | 27 | 2.7 |
| Carry out Industrial Automation and PLC Installation |  | Level 5 | Technical | 9 | 18 | 27 | 2.7 |
| Install PLC Software and Simulator |  | Level 5 | Technical | 6 | 12 | 18 | 1.8 |
| Design and Test the PLC |  | Level 5 | Technical | 9 | 21 | 30 | 3 |
| Installation, Commissioning and Maintenance of PLC. |  | Level 5 | Technical | 9 | 18 | 27 | 2.7 |
| Identify The Parts of Analog & Digital Telephone Set & Verify Their Function | Telecom Operator (Power) | Level 5 | Technical | 6 | 9 | 15 | 1.5 |
| Demonstrate Modulation, Demodulation, Multiplexing & Demulti-plexing. |  | Level 5 | Technical | 6 | 12 | 18 | 1.8 |
| Install Cord Less Telephone, PABX & Satellite Dish System. |  | Level 5 | Technical | 6 | 9 | 15 | 1.5 |
| Install Power Supply |  | Level 5 | Technical | 6 | 12 | 18 | 1.8 |
| Maintain Lead Acid Batteries and Implement Their Series Parallel Combination. |  | Level 5 | Technical | 6 | 6 | 12 | 1.2 |
| Operate Hydro Power Plant | Power Plant Operator | Level 5 | Technical | 8 | 15 | 23 | 2.3 |
| Operate Thermal Power Plant |  | Level 5 | Technical | 8 | 15 | 23 | 2.3 |
| Operate Diesel Power Plant |  | Level 5 | Technical | 8 | 15 | 23 | 2.3 |
| Operate Nuclear Power Plant |  | Level 5 | Technical | 8 | 15 | 23 | 2.3 |
| Design A Solar Power Plant |  | Level 5 | Technical | 8 | 15 | 23 | 2.3 |
| Operate Solar Power Plant |  | Level 5 | Technical | 8 | 15 | 23 | 2.3 |
| Operate Wind Power Plant |  | Level 5 | Technical | 8 | 15 | 23 | 2.3 |
| Calculate Tariffs and Economics |  | Level 5 | Technical | 8 | 12 | 20 | 2 |
| Utilize Electrical Energy in Building |  | Level 5 | Technical | 8 | 12 | 20 | 2 |
| Carryout Transmission of Electrical Power | Sr. Lineman | Level 5 | Technical | 8 | 9 | 17 | 1.7 |
| Carryout Distribution of Power System. |  | Level 5 | Technical | 8 | 9 | 17 | 1.7 |
| Carryout Protection of Distribution and Transmission System |  | Level 5 | Technical | 8 | 9 | 17 | 1.7 |
| Investigate Microbusiness Opportunities | Entrepreneurship | Level 5 | Generic | 6 | 9 | 15 | 1.5 |
| Develop A Micro Business Proposal |  | Level 5 | Generic | 6 | 9 | 15 | 1.5 |
| Develop A marketing Plan |  | Level 5 | Generic | 6 | 9 | 15 | 1.5 |
| Develop And Review A Business Plan |  | Level 5 | Generic | 6 | 9 | 15 | 1.5 |
| Organize Finances For The Micro Business |  | Level 5 | Generic | 6 | 9 | 15 | 1.5 |
| Manage Human Resources |  | Level 5 | Generic | 6 | 9 | 15 | 1.5 |
| Market Products And Services |  | Level 5 | Generic | 6 | 9 | 15 | 1.5 |
| Monitor And Review Business Performance |  | Level 5 | Generic | 6 | 9 | 15 | 1.5 |
| Negotiate For Resolving Business Issues |  | Level 5 | Generic | 6 | 9 | 15 | 1.5 |
| Manage Personal Finances |  | Level 5 | Generic | 6 | 9 | 15 | 1.5 |
| Coordinate A Work Team |  | Level 5 | Generic | 6 | 9 | 15 | 1.5 |
| Lead Small Teams |  | Level 5 | Generic | 6 | 9 | 15 | 1.5 |
| Utilize Natural Energy in Building | Green Skill | Level 5 | Technical | 8 | 12 | 20 | 2 |
| Total |  |  |  | 471 | 729 | 1200 | 120 |
| Percentage |  |  |  | 39.3 | 60.7 |  |  |

# Packaging of Occupations

The National Vocational Qualifications have been packaged as detailed below:

Level 5

(Electroplating Supervisor, Motor Winding Supervisor, Electronics Technician, Industrial Supervisor, Telecom Operator (Power), Power Plant Operator, Sr. Lineman, Digital Skills, Entrepreneur)

Computer Applications for Electrical Technology

- Perform Basic Mathematics Calculations in C++

- Perform Basic Circuit Analysis Calculations in C++

- Perform Electrical Analysis in C++

- Develop and Analyze Electrical Circuit with Multisim

- Develop and Analyze Electrical Circuit with PSPICE Software

- Carry Out in Computer Electrical Planning and Estimation

- Carry out Industrial Automation and PLC Installation

Motor Winding Supervisor

- Verify Basic Laws of Electrical Machines

- Analyze an Alternator.

- Analyze Single Phase Motors

- Analyze Special Purpose Motors

- Analyze Three Phase Motors

- Repair/ Maintenance and Service of Ac Electric Motors and Generators.

Electronics Technician

- Identify Basic Electronics Components

- Design A Rectifier Using Diode

- Carry Out Diode Application

- Implement Bipolar Junction Transistor (BJTs) In Different Applications

- Implement Field Effect Transistor (FETs) In Different Application

- Implement (Uni-Junction Transistor, Silicon Control Rectifier, DIAC and TRIAC) In Various Application.

- Design Operation Amplifier.

- Verify Truth Tables of Digital Gates.

- Construct & Verify Combinational Logic Circuit.

- Construct and Verify Function of Flip Flops.

- Use 555 IC as Multi-vibrator.

- Construct Shift Registers and Counters Used Flip Flops

Industrial Supervisor

- Implement Diode and Thyristor in Power Control Application.

- Implement Single Phase and Three Phase Inverter

- Control Speed of Dc/Ac Motors.

- Program PLC using Ladder Logic.

- Carry out Industrial Automation and PLC Installation

- Install PLC Software and Simulator

- Design and Test the PLC

- Installation, Commissioning and Maintenance of PLC.

Telecom Operator (Power)

- Identify The Parts of Analog & Digital Telephone Set & Verify Their Function

- Demonstrate Modulation, Demodulation, Multiplexing & Demulti-plexing.

- Install Cord Less Telephone, PABX & Satellite Dish System.

- Install Power Supply

- Maintain Lead Acid Batteries and Implement Their Series Parallel Combination.

Power Plant Operator

- Operate Hydro Power Plant

- Operate Thermal Power Plant

- Operate Diesel Power Plant

- Operate Nuclear Power Plant

- Design A Solar Power Plant

- Operate Solar Power Plant

- Operate Wind Power Plant

- Calculate Tariffs and Economics

- Utilize Electrical Energy in Building

- Utilize Natural Energy in Building

Sr. Lineman

- Carryout Transmission of Electrical Power

- Carryout Distribution of Power System.

- Carryout Protection of Distribution and Transmission System

Digital Skills

- Install Computer Operating Systems And Hardware

- Operate Word-Processing Applications

- Operate Spreadsheet Applications

- Operate Presentation Packages

- Perform  Writing And Editing Tasks

- Perform Computer Operations

- Use Computer Applications

- Create User Documentation

- Create Technical Documentation

- Create Basic Databases

- Operate Digital Media Technology

- Use Social Media Tools For Collaboration And Engagement

- E-Commerce

- Use Digital Devices

Entrepreneurship

- Investigate Microbusiness Opportunities

- Develop A Micro Business Proposal

- Develop A marketing Plan

- Develop And Review A Business Plan

- Organize Finances For The Micro Business

- Manage Human Resources

- Market Products And Services

- Monitor And Review Business Performance

- Negotiate For Resolving Business Issues\

- Manage Personal Finances

- Coordinate A Work Team

- Lead Small Teams

Green Skills

- Utilize Natural Energy in Building

# Mapping of the Qualifications

AC DC Machines…

Power Electronics…..

Analog Electronics

Digital Electronics

Wiring

Soft Skills

Digital Skills

Entrepreneurship

Microcontrollers

Analog Electronics

Digital Electronics

Process Variable Measurements….

# Date of Validation

The level 5 of National DAE qualification on Electrical has been validated by the Qualifications Validation Committee (QVC) members on 23-25 May, 2019 and will remain valid for ten years i.e 25 May, 2029

# Date of Review

The level 5 of National DAE qualification on Electrical has been validated by the Qualifications Validation Committee (QVC) members on 23-25 May, 2019 and shall be reviewed after three years i.e. 26 May, 2022

# Codes of Qualifications

The International Standard Classification of Education (ISCED) is a framework for assembling, compiling and analyzing cross-nationally comparable statistics on education and training. ISCED codes for these qualifications are assigned as follows:

| ISCED Classification for Electrical Technology level 5 |  |
| --- | --- |
| Code | Description |
| 0713E&E(1) | National Vocational Certificate Level 1 in Electrical Technology |
| 0713E&E(2) | National Vocational Certificate Level 2 in Electrical Technology |
| 0713E&E(3) | National Vocational Certificate Level 3 in Electrical Technology |
| 0713E&E(4) | National Vocational Certificate Level 4 in Electrical Technology |
| 0713E&E(5) | National Vocational Diploma Level 5 in Electrical Technology |

# Members of Qualifications Development Committee

The following members participated in the qualification development of this qualification:

| S# | Name | Designation | Organization |
| --- | --- | --- | --- |
| 1 | Aijaz Ahmed Zia | Design & Application Engineer | INTECH Process Automation. Lahore |
| 2 | Amir Amin | HOD Electrical, Research Associate LUMS | City Polytechnic |
| 3 | Muhammad Faheem Anjum | Chief Instructor Electrical | P-TEVTA |
| 4 | Muhammad Asad | Instructor Electrical | P-TEVTA |
| 5 | Tahreem Javeed | Lab Engineer | The University of Lahore |
| 6 | Huma Naeem | Research Associate | COMSATS Lahore |
| 7 | Engr.Inayat Ur Rehman | DACUM Facilitator | KPK-TEVTA |
| 8 | Muhammad Yasir | NAVTTC Coordinator | NAVTTC HQ Islamabad |

# Members of Qualification Validation Committee

The following members participated in the qualifications validation of this qualification:

| S# | Name | Designation | Organization |
| --- | --- | --- | --- |
| 1 | Israr Ahmaad | DD Academia | KP-TEVTA |
| 2 | Muhammad Yasir | NAVTTC Coordinator | NAVTTC HQ Islamabad |
| 3 | Sayeda Fatima Iqbal | System Analyst | PBTE Lahore |
| 4 | Danish Khan | Calibration Engineer | NPSL(PCSIR) Islamabad |
| 5 | Ghulam Rasool Maka | Vice Principal | S TEVTA |
| 6 | Muhammad Mehboob butt | Associate Professor | P TEVTA |
| 7 | Inayat Ur Rehman | EX Professor/ Ex-Director Academics | KP-TEVTA |
| 8 | Muhammad Asad | Instructor Electrical | P-TEVTA |

# Entry Requirements

The entry level for National Vocational Certificate Level 5 in Electrical Technology is:

- A person having National Vocational Certificate Level 4 in Electrical Technology

# Detail of Competency Standards

### 0713E&E1Develop and Analyse Electrical Circuit with MULTISM

Overview:

This competency standard covers the skills and knowledge required to identify computer software components. Install and operate MULTISM. Idenitify toolbox and libraries. Develop RC circuit and Develop RLC circuit identify computer requirements and assemble procedures and Use of computer software MULTISM for developing/analyzing electrical networks.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Identify computer software components | - Classify the software components of Computer. - Install the necessary plug-ins - Install computer software’s as for requirement. |
| CU2.Install and operate MULTISM | - Open the MULTISM Screen and its menu. - Select component of circuit diagrams. - Insert electric component. - Wiring schematic. - Simulating circuit. - Transfer to PCB layout. - Routing the board. - Analyze result by changing components. |
| CU3. Idenitify toolbox and libraries | - Open the MULTISM Screen and its menu. - Select component of circuit diagrams. - Study electric component simples. - Study wiring - Study tool box - Study PCB layout. - Study routing the board. |
| CU4.Develop RC circuit | P1. Open the MULTISM screen and its menu. P2. Select component of circuit diagrams. P3. Insert electric component. P4.Wiring schematic. P5. Transfer to PCB layout. P6.Analyze results for AC and DC. |
| CU5.Develop RLC circuit | P1. Open the MULTISM screen and its menu. P2. Select component of circuit diagrams. P3. Insert electric component. P4. Wiring schematic. P5. Transfer to PCB layout Analyze results for AC and DC. |
| CU6. construct active filter | P1. Open the MULTISM screen and its menu. P2. Select component of circuit diagrams. P3. Insert electric component. P4.Wiring schematic. P5. Transfer to PCB layout. P6.Analyze results follow pass and band pass filter. |
| CU7.construct passive filter | P1. Open the Multiscreen and its menu. P2. Select component of circuit diagrams. P3. Insert electric component. P4.Wiring schematic. P5.tranfer to PCB layout .analyze results for low, high, band and all pass. |
| CU8.Construct Amplifier frequency response | P1. Open the MULTISM and its menu. P2. Select component of circuit diagrams. P3. Insert electric component. P4.Wiring schematic. P5.Transfer to PCB layout Analyze frequency response. |
| CU9. Construct half wave rectifier | P1. Open the MULTISM screen and its menu. P2. Select component of circuit diagrams. P3. Insert electric component. P4.Wiring schematic. P5. Transfer to PCB layout analyze results |
| CU10.Construct full wave rectifier | P1. Open the MULTISM screen and its menu. P2. Select component of circuit diagrams. P3. Insert electric component. P4.Wiring schematic. P5. Transfer to PCB layout. Analyze results. |

Knowledge & Understanding

- Define Basic purpose of computer

- Describe basic software components of computer.

- Describe Input and Output devices.

- Describe basic input and output devices and their function.

- Describe basic issues/problems related to electric circuit.

- Describe basic hardware configuration required for software.

- Detail knowledge of circuit

- Diagram functions.

- Detail knowledge of electrical components.

- Describe basic input and output screen of software.

- Describe basic issues/problems related to software.

- Describe basic symbol for components.

- Understand basic input and output devices and their function.

- Understand basic issues/problems related to RC circuit.

- Understand basic hardware configuration required for software.

- Understand knowledge of circuit diagram functions.

- Understand and knowledge of basic input and output devices

- And their function.

- Understand and knowledge of basic issues/problems related to RLC circuit.

- Understand and knowledge of basic hardware configuration required for software.

- Understand and knowledge of circuit diagram functions

- Describe basic input and output devices and their function.

- Describe basic issues/problems related to active filter circuit.

- Describe basic hardware configuration required for software.

- describe knowledge of circuit diagram functions

- Describe basic input and output devices and their function.

- describe basic issues/problems

- Related to electric circuit.

- Describe basic hardware configuration required or software.

- Detail knowledge of circuit diagram functions.

- Describe basic input and output devices and their function.

- Describe basic issues/problems related to electric circuit.

- Describe basic hardware configuration required for software.

- Detail knowledge of circuit diagram functions

- Describe basic input and output devices and their function.

- Describe basic issues/problems related to rectifier circuit.

- Describe basic hardware configuration required for software.

- Describe knowledge of circuit

- diagram functions

- Describe basic input and output devices and their function.

- Describe basic issues/problems related to electric circuit.

- Describe basic hardware configuration required for software.

- Detail knowledge of circuit diagram functions.

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | Relevant Software Disks |
| 2 | Internet |
| 3 | Presentation manuals |
| 4 | Multimedia Screen |
| 5 | Multimedia |
| 6 | Printer |
| 7 | Power Cables |
| 8 | Hardware equipment |
| 9 | Monitor |
| 10 | Keyboard |
| 11 | CPU |
| 12 | Presentation manuals |
| 13 | Handouts Hardware |
| 14 | Equipment |
| 15 | Tool kits and Windows DVDs,6,10 etc. |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Installation techniques

### 0713E&E2 Develop and Analyze Electrical Circuit with PSPICES Software.

Overview:

This competency standard covers the skills and knowledge required to Install and operate PSPICE Develop filters and Develop Rectifier Identify computer requirements and assemble procedures and Use of computer software PSPICE for developing/analysing electrical networks.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1.Install and operate PSPICE | P1. Install the PSPICE from setup P2. Open the PSPICE Screen and its menu. P3. Select component of circuit diagrams. P4. Insert electric component. P5. Wiring schematic. Save file and Analysis setup. P6.AC sweep to change frequency. Analyze result by changing components. |
| CU2.Develop filters | - Open the PSPICE Screen and its menu. - Select component of circuit diagrams. - Insert electric component. - Wiring schematic. - Save file and Analysis setup. - AC sweep to change frequency. - Analyze result for low pass filter and high pass filter. |
| CU3.Develop rectifier | - Open the PSPICE Screen and its menu. - Select component of circuit diagrams. - Insert electric component. - Wiring schematic. - Save file and Analysis setup. - AC sweep to change frequency. - Analyze result for half and full rectifier. |
| CU4. Construct CE Amplifier | - Open the PSPICE Screen and its menu. - Select component of circuit diagrams. - Insert electric component. - Wiring schematic. - Save file and Analysis setup. - AC sweep to change frequency. - Analyze frequency response for CE Amplifier |
| CU5. Cconstruct CS amplifier | - Open the PSPICE Screen and its menu. - Select component of circuit diagrams. - Insert electric component. - Wiring schematic. - Save file and Analysis setup. - AC sweep to change frequency. - Analyze frequency response for CS amplifier. |
| CU6.Construct CC Amplifier | - Open the PSPICEScreen and its menu. - Select component of circuit diagrams. - Insert electric component. - Wiring schematic. - Save file and Analysis setup. - AC sweep to change frequency. - Analyze frequency response for CC amplifier. |
| CU7. Design WEIN Bridge Oscillator | P1. Open the PSPICE Screen and its menu. - Select component of circuit diagrams. - Insert electric component. - Wiring schematic. - Save file and Analysis setup. - AC sweep to change frequency. - Analyze the result at oscillator |

Knowledge & Understanding

- Describe basic input and output devices and their function.

- Describe basic issues/problems related to electric circuit.

- Describe basic hardware configuration required for software.

- Detail knowledge of circuit diagram functions.

- Detail knowledge of electrical components.

- Describe basic input and output devices and their function.

- Describe basic issues/problems related to filters circuit.

- Describe basic hardware configuration required for software.

- Detail knowledge of circuit diagram functions.

- Describe basic input and output devices and their function.

- Describe basic issues/problems related to rectifier’s circuit.

- Describe basic hardware configuration required for software.

- Describe knowledge of circuit diagram functions.

- Describe basic input and output devices and their function.

- Describe basic issues/problems related to amplifiers circuit.

- Describe basic hardware configuration required for software.

- describe knowledge of circuit diagram functions

- Describe basic input and output devices and their function.

- Describe basic issues/problems related to amplifiers circuit.

- Describe basic hardware configuration required for software.

- Describe knowledge of circuit diagram functions

- Describe basic input and output devices and their function.

- Describe basic issues/problems related to amplifiers circuit.

- Describe basic hardware configuration required for software.

- Describe knowledge of circuit diagram functions

- Describe basic input and output devices and their function.

- Describe basic issues/problems related to Oscillator.

- Describe basic hardware configuration required for

- Software.

- Describe knowledge of WEIN bridge oscillator diagram

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | Relevant Software Disks |
| 2 | Internet |
| 3 | Presentation manuals |
| 4 | Multimedia Screen |
| 5 | Multimedia |
| 6 | Printer |
| 7 | Power Cables |
| 8 | Hardware equipment |
| 9 | Monitor |
| 10 | Keyboard |
| 11 | CPU |
| 12 | Presentation manuals |
| 13 | Handouts Hardware |
| 14 | equipment |
| 15 | Tool kits and Windows DVDs,6,10 etc. |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

Develop the given circuit on PSPICE

### 0713E&E3 Carry Out in Computer Electrical Planning and Estimation

Overview:

This Competency Standard identifies the competencies required to. Analyse customer requirement and specification, Plan and estimate domestic electric work, Plan and estimate commercial   electric work and Plan and estimate industrial electric work.

| Competency Units | Performance Criteria |
| --- | --- |
| - .Analyze customer requirement and specification | - Draw the general value chain of the end user. - Highlight the various stages and set of activities in the value chain drawing - Enlist the electrical appliances/materials required in electrical development process - Identify critical stages in the development - Identify the safety aspect required in the critical stages of the development - Enlist the possible energy efficient appliance/devices and global trends in electrical works - Analyse the client requirement at broad level. - Generate a report of various stages of electrical works. - Proposed appliance/materials as per global trends and clarify the technical specification. |
| - Plan and estimate domestic electric work | - Plan the client requirement at broad level from the proposal. - Plan the electrical appliances/materials / used in different stages of the electrical development process - Plan and estimate electrical wiring - Estimate Installation and Material Cost - Estimate Power consumption for lighting and appliances - Plan a main and sub distribution board. - Ensure safety system. |
| - Plan and estimate commercial electric work. | - Analyze the client requirement at broad level from the proposal. - Plan the electrical equipment /appliances/materials / used in different stages of the commercial electrical work - Plan and estimate electrical wiring - Estimate Installation and Material Cost - Estimate Power consumption for centralize lighting, heating, cooling system and other appliances. - Plan a main and sub distribution board. - Ensure safety measures in development process. - Enlist the material required. |
| - Plan and estimate industrial electric work. | - Analyze the client requirement at broad level from the proposal. - Plan the electrical equipment /appliances/materials / used in different stages of the commercial electrical work - Plan and estimate electrical wiring - Estimate Installation and Material Cost - Estimate Power consumption for centralize lighting, heating, cooling system and other appliances. - Plan 3-phase line for heavy loads - Plan a separate control panel box for heavy machinery. - Plan a main and sub distribution board. - Design a safety system for protection of machinery and labor. - Assign color scheme for safety measures |

Knowledge & Understanding

- Define the term ‘value chain’ of industry.

- Define human machine interface (HMI)

- Define the Standard operating procedure (SOP) of the organization for control panel development process.

- How to prepare a General arrangement drawing?

- Define IEC standards

- Describes the standard of electrical components.

- Understanding and knowledge of equipment properties.

- Understanding and knowledge of input and output of tool of electronic.

- Describe the rules and regulation of work.

- Understanding and knowledge of Electrical Specifications component used.

- Describes the standard of electrical components.

- Understanding and knowledge of equipment properties.

- Understanding and knowledge of input and output of tool of electronic.

- Describe  the rules and regulation of work

- Describes the standard of electrical components.

- Understanding and knowledge of equipment properties.

- Understanding

- And

- Knowledge of input and output of tool of electronic.

- Describe the rules and regulation of work.

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | Site Visit |
| 2 | Reference Material |
| 3 | Clipboard |
| 4 | Calculation Sheet |
| 5 | Calculator |
| 6 | Psychometric Charts and Tables |
| 7 | Protective gear such as helmets, goggles, gloves, rubber shoes, etc. |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

Prepare an estimate for domestic living room

### 0713E&E4 Carry Out Industrial Automation and PLC Installation

Overview:

After this competency standard, the trainee will be able to get basic knowledge and competence to install PLC software and Simulator, Program a PLC using simulator and analyse user requirements and specifications.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Install PLC software and Simulator | - Install the PLC Programming - Software as per PLC manufacturer such as rest Logic 5000 and rest links classic etc. - Select the best and most frequently used Simulator for programming - Select the module - Input the generic value as per universal settings (check from internet/helped) - Start The Simulator. - Open PLC programming software |
| CU2. Program a PLC using simulator | P1. Open the programming software as per PLC manufacturer P2. Ensure that the Simulator is connected and is in ON condition P3. Create a basic ladder logic program for ON/OFF of a bulb using Examine ON and Examine OFF switch P4. Create a basic ladder logic program for Simple Start/Stop Ladder Logic Relay P5. Create a basic ladder logic program for Single Push Button On/Off Ladder Logic P6. Create a basic ladder logic program for with On Delay Timer P7. Create a basic ladder logic program for with Off Delay Timer P8. Create a basic ladder logic program for Traffic signal lights P9.Create a basic ladder logic program for Elevator Control. |
| CU3. Analyse user requirements and specifications | - Draw the general value chain of the end user industry - Highlight the set of activities that a firm operating in a specific industry performs in the value chain drawing - Enlist the equipment/gauges/sensors/actuators/transducers used in different stages of the process - Identify critical stages in the process - Identify the safety aspect required in the critical stages of the process - Analyse the possible automation in the existing processes and global trends in automation - Analyse the client requirement at broad level from the proposal - Generate a report of various industrial processes involved in industry - Collect the required specification of the equipment (if already prepared by the user) and clarify the technical specification. |

Knowledge & Understanding

- Describe the advantages of the PLC

- Describe the major components and

- Describe the applications of PLC.

- Describe the types of input and output signals.

- Define Piping and instrumentation diagram/drawing (P&ID)

- List down the PLC, DCS programming software

- List Down the SCADA, HMI development software

- Knowledge of Relevant documents and documentation procedures

- Define Ladder Logic

- Explain the Examine ON and Examine OFF switch

- How to use ladder logic to create basic programs?

- What is push button?

- Define the types of Timers.

- What is the working of traffic light?

- Describe the working of an elevator

- Define the term ‘value chain’ of industry.

- Define human machine interface (HMI)

- Define the Standard operating procedure (SOP) of the organization for control panel development process.

- How to prepare a General arrangement drawing?

- Define IEC standards

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | Site Visit |
| 2 | Reference Material |
| 3 | Clipboard |
| 4 | Calculation Sheet |
| 5 | Calculator |
| 6 | Psychometric Charts and Tables |
| 7 | Protective gear such as helmets, goggles, gloves, rubber shoes, etc. |
| 8 | PLC Software |
| 9 | Helping Manual |
| 10 | Simulator |
| 11 | User guide |
| 12 | Simulator |
| 13 | Page |
| 14 | Pen |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

Simulate the program

### 0713E&E5 Verify Basic Laws of Electrical Machines

Overview:

This competency standard covers the skills and knowledge required to verify Faraday’s law by moving permanent magnet inside the coil, verify Faraday’s law by moving coil near the magnet field, verify Faraday’s law using relative motion of coil and magnet, verify Faraday’s Law using simple loop generator, verify EMF through induction, verify Torque induce in a current carrying loop and verify Mutual induction.

| Competency Units | Performance Criteria |
| --- | --- |
| - Verify Faraday’s law by moving permanent magnet inside the coil. | - Construct a coil. - Connect Galvanometer with coil. - Move permanent magnet inside the coil fast and slow. - Record the effect of movement of magnet on reading of Galvanometer. - Hold the magnet inside the coil and do not move. - Record the effect on reading of Galvanometer again. |
| - Verify Faraday’s law by moving coil near the magnet field. | - Construct a coil. - Connect Galvanometer with coil. - Fix permanent magnet and move the coil fast and slow on it. - Record the effect of movement of coil on reading of Galvanometer. - Hold the coil near the magnetic field and do not move. - Record the effect on reading of Galvanometer. |
| - Verify Faraday’s law using relative motion of coil and magnet. | - Construct a coil. - Connect Galvanometer with coil. - Make relative motion of coil and magnet. - Record the effect of movement on reading of Galvanometer. |
| - Verify Faraday’s Law using simple loop generator. | - Identify single loop generator and its parts. - Select field winding. - Connect Galvanometer with single loop coil. - Rotate the single loop coil of generator in the field - Measure the voltage induced in loop generator. - Record the effect of movement on reading of Galvanometer. |
| - Verify Torque induce in a current carrying loop | - Construct a coil. - Apply DC voltage to this coil. - Place a current carrying loop in this coil. - Check the direction of force to verify the torque produce. |
| - Verify Mutual induction | - Construct 2 coils on two different legs of single core. - Apply AC voltage to the any one coil. - Check the volts induce in other coil by voltmeter. |

Knowledge & Understanding

- What is first law of Faraday?

- What is second law of Faraday?

- How can we find the direction of induced emf?

- What is the role of magnetic strength in Faraday’s Law?

- What is the role of conductor’s length or turns in Faraday’s Law?

- What is Lenz’s Law

- What is torque?

- Why force produce on current carrying conductor.

- What is core?

- What is induction?

- What is Faraday’s first law of Electro-Magnetic Induction?

- What is meaning of coupled coils.

- Why emf induced in 2nd coil, when voltage is applied to 1st coil.

- What is mutual induction?

- What is core?

- Which machine work on the principle of mutual induction

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | Coil |
| 2 | Galvanometer. |
| 3 | Magnet |
| 4 | Connecting leads |
| 5 | Transformer |
| 6 | Rheostat |
| 7 | Loop |
| 8 | Voltmeter |
| 9 | DC source |
| 10 | Coil |
| 11 | Connecting leads |
| 12 | Rheostat |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

### 0713E&E6 Analyze an Alternator

Overview:

This competency standard identifies the competencies required to measure the stator winding resistance with DC test, perform no load test, perform short circuit test, measure the efficiency of an alternator, measure the effect of field current on terminal voltage, measure the effect of speed changes on alternator frequency and active power and perform parallel operation of Alternators.

| Competency Units | Performance Criteria |
| --- | --- |
| - Measure the stator winding resistance with DC test. | - Connect DC Ampere meter in series with armature. - Apply DC voltage to any one of the other two phases. - Measure current and voltage. - Measure the resistance per phase by ohm’s law. |
| - Perform no load test. | - Make connection according to diagram. - Run the alternator with prime mover to its rated speed. - Supply the field current to make the terminal voltage to its rated value. - Measure Iron losses from watt meter. |
| - Perform short circuit test. | - Make connection according to diagram. - Run the alternator with prime mover to its rated speed. - Supply the field current to make the terminal voltage 10 to 15% to its rated value. - Measure the copper loss from the watt meter. |
| - Measure the efficiency of an alternator. | - Perform no load test to measure Iron losses. - Perform short circuit test to measure copper losses. - Apply formula to find efficiency. |
| - Measure the effect of field current on terminal voltage. | P1. Run the alternator with prime mover to its rated speed. P2. Regulate the field current from zero to high value to make the terminal voltage to its rated value P3. Note values of field current and respective terminal voltage P4. Draw no load curve between field current and terminal voltage. |
| - Measure the effect of speed changes on alternator frequency and active power. | - Run the alternator with prime mover to its rated speed. - Supply the field current to make the terminal voltage to its rated value. - Apply electrical load on alternator. - Measure the effects of load on alternator speed and frequency. - Regulate the prime mover speed from lower to its rated value. - Measure the effect of this change on alternator speed and frequency. |
| - Perform parallel operation of Alternators. | - Run the incoming alternator to its rated speed. - Supply the field current to make the terminal voltage to its rated value. - Use synchronous scope to synchronize alternator’s frequency and phase angle. - Check the phase sequence with dark lamp method. - Check the voltage of bus bar and incoming machine with volt meter. - Switch on the breaker after completing parallel operation. |

Knowledge and understanding

- How to connect DC source with three phase stator?

- How to calculate the resistance of winding by using meter reading.

- Which type of losses is found with no load test?

- What is iron loss?

- What is copper loss?

- Which types of losses are found with short circuit test?

- Which types of losses are found with no load test?

- Explain the effect of field current on terminal voltage.

- Explain the effect of rheostat resistance on terminal voltage.

- What is core saturation?

- Why the graph between field current and armature voltage is not a straight line

- How to change the RPM of the generator?

- Explain the conditions of paralleling of alternators.

- Why we connect alternators in parallel.

- Explain the working of synchronous scope.

- What is phase sequence?

- What is phase angle?

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | Alternator |
| 2 | Ampere meter |
| 3 | Voltmeter |
| 4 | Wattmeter |
| 5 | Frequency meter |
| 6 | DC source |
| 7 | Resistive load |
| 8 | Connecting wires |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

### 0713E&E7 Analyze Single Phase Motors.

Overview:

This competency standard identifies the competencies required to operate split phase single phase AC motor, operate capacitor start and Capacitor run single phase AC motor and operate shaded pole single phase motor.

| Competency Units |  | Performance Criteria |
| --- | --- | --- |
| - Operate split phase single phase AC motor. | - Make connections according to circuit diagram. - Apply rated voltage to the stator. - Measure RPM and direction of rotation. - Disconnect the auxiliary winding from centrifugal switch. - Check the effect of this change. |  |
| - Operate capacitor start and Capacitor run single phase AC motor. | - Make connections according to circuit diagram. - Apply rated voltage to the stator. - Measure the RPM. - Remove the starting capacitor with centrifugal switch. - Measure the RPM again. - Check the effect of starting capacitor on motor torque. |  |
| - Operate shaded pole single phase motor. | - Make connections according to circuit diagram. - Apply rated voltage to the stator. - Change the applied voltage to measure the effect on motor speed. |  |

Knowledge and understanding

- What is starting winding?

- What is running winding?

- How to rotate magnetic field in single phase motor.

- Why we connect capacitor with starting winding.

- Explain the effect of capacitor on starting torque.

- How to rotate magnetic field

In single phase motor

- Explain the working of shaded pole motor.

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | Single phase induction motor |
| 2 | Tachometer |
| 3 | Centrifugal switch |
| 4 | Connecting wires |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

### 0713E&E8 Analyze Special Purpose Motors.

Overview:

This competency standard identifies the competencies required to operate and speed control of AC series motor, operate miniature (reluctance and hysteresis) single phase AC motors, construct and operate stepper motor and construct and operate and develop control circuit with the help of servo motor.

| Competency Units | Performance Criteria |  |
| --- | --- | --- |
| - Operate and speed control of AC series motor. |  | - Connect the armature and field winding in series. - Apply rated voltage to the motor. - Measure the effect on motor speed by change in applied voltage. |
| - Operate miniature (reluctance and hysteresis) single phase AC motors. |  | - Connect the stator of motor with single phase supply. - Apply mechanical load by hand - Measure the effect of load change on motor speed. - Draw torque speed curves. |
| - Construct and operate stepper motor. |  | - Connect the stator of the motor with control circuit. - Adjust the control voltage of control unit. - Measure the RPM. - Readjust the control voltage of control unit. - Measure the RPM again. - Make comparison between Control voltage and RPM. |
| - Construct, Operate and develop control circuit of servo motor. |  | - Pair the servo motor with some encoder to provide position and speed feedback. - Compare the measured position to external input to the controller. - Check the effect of feedback on motor position. |

Knowledge and understanding

- Explain the working of AC series motor.

- Explain the effect of applied voltage on speed of the motor.

- Explain the working of reluctance motor.

- Explain the working of hysteresis motor.

- Explain the working of stepper motor.

- Explain the working of control unit for stepper motor.

- Explain the working of servo motor.

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | AC series motor |
| 2 | Tachometer |
| 3 | Connecting wires |
| 4 | Reluctance motor |
| 5 | Hysteresis motor |
| 6 | Tachometer |
| 7 | Connecting wires |
| 8 | Stepper motor |
| 9 | Control unit |
| 10 | Tachometer |
| 11 | Connecting wires |
| 12 | Servo motor |
| 13 | Control circuit |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

### 0713E&E9 Analyze Three Phase Motors.

Overview:

This competency standard identifies the competencies required to identify and operate 3 phase Squirrel cage motor, verify rotating magnetic field , measure the starting and running current of three phase induction motor, draw torque slip curve of three phase induction motor, determine the slip of 3 phase induction motor by stroboscope, start 3 phase Induction motor with direct on line, start 3 phase Induction motor with Auto transformer starter, start 3 phase Induction motor with Star Delta starter, make connection to reverse the direction of induction motor, determine the efficiency of three phase induction motor, control the speed of three phase induction motor by changing primary voltage, control the speed of three phase induction motor by rotor resistance control method, operate two motors in cascade, start the synchronous motor using external prime mover and identify the effect of field current on three phase synchronous motor.

| Competency Units | Performance Criteria |  |
| --- | --- | --- |
| - Identify and operate 3 phase Squirrel cage motor. |  | - Identify and select 3 phase squirrel cage motor. - Make connection as per diagram. - Connect the supply and interpret the result. |
| - Verify rotating magnetic field |  | - Connect the three terminals of stator winding with each other and with ground. - Apply three phase supply to the other terminals. - Place a compass between the stator and note the direction of rotating magnetic field. - Change the two phases with each other. - Note the direction of rotating magnetic field. |
| - Measure the starting and running current of three phase induction motor. |  | - Connect the stator winding in star. - Connect an ammeter in series with stator. - Note the ammeter reading at instant of just starting. - Note the reading of ammeter at running condition. |
| - Draw torque slip curve of three phase induction motor |  | - Connect three phase induction motor with electro dynamo meter with Belt. - Start the motor and run at rated speed - Measure the RPM. - Change the torque with electro dynamo meter and note the speed - Calculate slip with given values of speed. - Draw graph between slip and torque. |
| - Determine the slip of 3 phase induction motor by stroboscope |  | - Connect the stator of three phase induction motor in star - Apply single phase AC supply to the stroboscope. - Mark dot on motor shaft with marker. - Apply three phase supply to the motor - Note the RPM of the motor with the help of stroboscope. - Calculate slip. |
| - Start 3 phase Induction motor with direct on line. |  | - Draw line diagram of direct-on 3-phase motor Starter. - Identify and select 3 phase Induction motor. - Make Connection of motor with DOL as per diagram. - Switch on the 3 phase breaker and interpret the result. |
| - Start 3 phase Induction motor with Auto transformer starter. |  | - Connect power circuit with the help of diagram. - Connect control circuit with the help of diagram. - Apply rated voltage to the motor. - Gradually reduce the taping of auto transformer with the help of control circuit. |
| - Start 3 phase Induction motor with Star Delta starter. |  | P1: Draw line diagram of a starter-delta starter. P2: Identify and select 3 phase Induction motor. P3: Make Connection of motor with star delta starter as per diagram. P4: Switch On the 3 phase breaker and interpret the result. |
| - Make connection to reverse the direction of induction motor. |  | P1: Draw line diagram to reverse the direction of induction motor P2: Identify and select 3 phase Induction motor. P3: Make Connection of motor through 3 pole 2 way switch as per diagram. P4: Move the handle of 3Pole 2 Way switch from off position to position 1. P5: Move the handle of 3 Pole 2 Way switch from Position 1 to off position and wait for stopping of motor P6: Move the handle of 3Pole 2 way switch from off position to position 2 and interpret the result. |
| - Determine the efficiency of three phase induction motor |  | - Connect wattmeter, ammeter with the stator of three phase induction motor. - Run the motor at rated speed. - Measure the reading of wattmeter and voltmeter. - Stop the motor - Couple three phase induction motor and electro dynamo meter with belt. - Run the motor. - Supply single phase supply to the dynamo meter. - Adjust the supply of electro dynamo meter so that the maximum current passes through the motor. - Read the meters reading. - Calculate the efficiency of the motor by using these reading. |
| - Control the speed of three phase induction motor by changing primary voltage |  | - Connect the stator of the motor in star connection. - Insert rheostat in series with stator of the motor. - Set the rheostat resistance to its max value. - Run the motor - Change rheostat resistance and note the effect of speed changes. |
| - Control the speed of three phase induction motor by rotor resistance control method |  | - Select wound rotor induction motor. - Connect the stator of motor in star connection. - Connect a star connected starter with motor rotor. - Set rotor resistance to its max value. - Run the motor. - Change the rotor resistance and note the effect of speed change. |
| - Operate two motors in cascade |  | - Select two wound rotor induction motor. - Couple shaft of the both motors. - Connect the stator of motor 2 with the rotor of motor 1. - Connect the motor 2 rotor in star connection. - Apply three phase voltage to the stator of motor 1. - Control the speed of both motors by changing the rotor resistance of motor 2. |
| - Start the synchronous motor using external prime mover. |  | - Couple the shaft of the synchronous motor with external prime mover (DC Shunt motor) - Connect the stator winding of synchronous motor in star connection. - Apply rated voltage to the stator. - Run the prime mover, which results in locking of rotor with stator magnetic field |
| - Identify the effect of field current on three phase synchronous motor. |  | - Apply rated voltage to stator. - Excite the field winding. - Change the field winding voltage and measure the effect on Armature current. - Draw the V curve between Armature Current and Field current. |

Knowledge and understanding

- How many types of three phase induction motor with respect to the rotor?

- Explain the term rotating magnetic field

- How to change the direction of rotation of motor.

- Why we initially start the motor in star connection.

- Why motor take large current at the time of starting.

- What is back EMF?

- How to change the mechanical load on motor?

- How to measure the speed of DC motor?

- What is motor slip?

- Explain the effect of torque on motor slip.

- Explain the working of stroboscope.

- How to calculate the slip from motor speed.

- What in magnetic contactor.

- What is relay?

- Explain the working of DOL

- What is auto transformer?

- How to change the taping of auto transformer?

- Why we never start motor without load.

- Which type of connection is use at the starting of motor?

- Which type of connection is use at the running of motor?

- What is different between control circuit and power circuit

- What in magnetic contactor.

- How to reverse the direction of motor.

- Explain the connection of wattmeter.

- Explain the working of electro dynamo meter.

- What is iron loss?

- What is copper loss?

- Why we find copper loss at full load.

- Which loss having maximum value and why

- How many techniques are used to control the speed of three phase induction motor?

- What is rheostat?

- Explain the effect of voltage changes on speed of motor.

- Why we connect the stator of motor in star.

- How to insert the resistance in rotor circuit at the time of starting

- What is mean by cascade?

- Why we insert maximum resistance in rotor circuit at the time of starting.

- Write the name of starting method of synchronous motor.

- Why synchronous motor is not self-starting.

- Which type of supply voltage is applied to the synchronous motor rotor?

- How to change the field excitation of synchronous motor?

- Explain over and under excitation.

- Explain V curve of synchronous motor.

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | Three phase squirrel cage induction motor |
| 2 | Connecting wires |
| 3 | wires |
| 4 | meter |
| 5 | Belt |
| 6 | Tachometer |
| 7 | Ammeter |
| 8 | Connecting |
| 9 | wires |
| 10 | Tachometer |
| 11 | DOL starter |
| 12 | Auto transformer |
| 13 | Ammeter |
|  | Electro dynamo meter |
|  | Rotor start |
|  | DC shunt motor |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

### 0713E&E10 Repairs/Maintenance and Service of Electric Motors and Generators

Overview:

This Competency Standard identifies the competencies required to Diagnose Fault in Electric Motor, carry out Service/Repair in Electric Motor, carry out Preventive Service/ Maintenance of Generators and Diagnose and Repair Electric Generator carry out repair and maintenance of AC motors and generators in accordance with the manufacturer’s instructions. The underpinning knowledge regarding repair and maintenance of electrical motor & generators will be sufficient to provide the basis for your work.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Diagnose Fault in Electric Motor | - Identify the type of electric motor from data / name plate or manual and motor ratting. - Check the tripping of protective device using specified test instruments, while the motor is running with power supply connected with suitable control and protective device, the difficulty in starting/low RPM/ unusual noises/ excessive heat/ grounded winding etc. - Perform dismantling of three-phase motor - Inspect visual mechanical defect such as, loose or tight bearings, bent shaft, noisy running. - Check the motor by using specified test instruments to detect electrical defects such as loose/or burnt electrical connections, burnt windings, low insulation resistance. - Perform test to locate short circuit, circuit break, earth fault, wiring fault, bearing fault. |
| CU2. Carry out Service/Repair in Electric Motor | - Clean the parts of the motor with specified cleaning agents/tools & material - Repair the motor as per diagnosed mechanical faults - Check the windings insulation resistance with insulation tester - Rewind the motor as per requirement - Check the insulation resistance of repaired motor as per standard - Assemble the electric motor after performing internal tests/servicing/repairs according to manufacturer’s instructions - Carry out the No load and load tests as per standard practices |
| CU3. Carry out Preventive Service/ Maintenance of Generators | - Carry out routine maintenance of the generator as specified in the manufacturer’s manual - Check out the switch, timer, relay, protective device and wire connection - Update the maintenance/service records |
| CU4. Diagnose and Repair Electric Generator | - Check Load regulator of the generator and its control circuit. - Check control circuit of the auxiliary generator. - Check control circuit of the battery unit - Check circuit breaker and relay. - Dismantle the electric Generator for internal tests/servicing/repairs according to manufacturer’s instructions - Clean the parts of the generator with specified cleaning agents/tools & material - Find out the root cause of fault according to the signs of fault by applying the working principles of control equipment of generators: Speed controller, Magnetic field control circuit and equipment of the generator. - Repair the fault after finding out the faulty equipment or component - Check the windings insulation resistance with insulation tester as per standard - Repair / rewind the generator winding as per diagnosed fault. - Assemble the generator after performing internal tests/servicing/repairs according to manufacturer’s instructions - Carry out the No load and load tests as per standard practices |

Knowledge & Understanding

- Understand Type and Construction and working principal of Motors (Induction, Servo, Synchronous, DC motor DC Series Motor, DC shunt Motor, D.C. compound motor, Squirrel Cage Induction motor, Wound Rotor Induction motor, Synchronous motor, split phase motors (capacitor start and capacitor run motors), Universal motors, Synchronous motors.

- Understand working principal of

- electrical Generator, D.C. series generator, D.C. shunt generator, D.C. compound generator, 3 phase salient pole type 3 phase non salient pole type (Smooth Cylindrical type)

- Servicing of motors and standby generators

- Types of electrical measuring instruments used in testing electric motors & generators

- Fundamentals of electrical rotating machines

- Auto Transfer switch (ATS)/Auto Main Failure (AMF) module

- Maintenance procedures on standby generators

- Working principles of electric

- motors &generators

- Types of electrical control and protective devices

- Dis-assembling & re-assembling

- of electric motors

- Common faults found in electric motors/generators and troubleshooting methods

- Electrical legislation and regulations related to electric motors and allied motor control switchgear

- Preventive maintenance work on motors and stand by generators

- Record keeping and reporting

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | AC Motors |
| 2 | Ammeter |
| 3 | Battery Cleaning |
| 4 | Battery Charger |
| 5 | Kit |
| 6 | Bearing Puller |
| 7 | Bench Vice |
| 8 | Cable / Wire Gauge |
| 9 | Cable Cutter |
| 10 | Cable Knife |
| 11 | Cells tester |
| 12 | Chisel |
| 13 | Circuit Boards |
| 14 | clamp Meter |
| 15 | Earth Tester |
| 16 | Hammer |
| 17 | Safety Belt |
| 18 | RPM Meter |
| 19 | Philips Screw Drivers Set |
| 20 | Ring Spanner Set |
| 21 | Multi-meter |
| 22 | Lux Meter |
| 23 | Megger (Insulation Tester) |
| 24 | Micron Meter |
| 25 | OTDR Meter |
| 26 | Phase Sequence |
| 27 | Meter |
| 28 | Magnetic |
| 29 | Hacksaw |
| 30 | Safety Goggles |
| 31 | Safety Helmet |
| 32 | Safety Shoes |
| 33 | Set of Screw Drivers |
| 34 | Set of Nose Pliers |
| 35 | Thermometer |
| 36 | Splicing Machine |
| 37 | Test lamp |
| 38 | Soldering Lead |
| 39 | Soldering Iron |
| 40 | Gloves |
| 41 | Grinder |
| 42 | Generator |
| 43 | Filler gauge |
| 44 | Files (set) |
| 45 | Duct Rod |
| 46 | Disk Grinder |
| 47 | Combination Plier Set |
| 48 | Thimble Press Plier |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

:

- Perform insulation, frequency, no-load, load testing etc.

- Troubleshooting of starting system of AC motor & generator

- Replacement of capacitor and bearings of motor

### 0713E&E11 Identify Basic Electronics Components

Overview:

This competency standard covers the skills and knowledge required to Identify Various Diodes, Identify Resistors in circuit, Identify Capacitor in circuit, identify Inductor in circuit and Identify IC’s Packages. After this competency standard the candidate will be able to identify variety of basic electronic components and their usage in industry.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Identify Various Diodes | - Identify the Diodes - Identify its types & polarities - Draw Diode characteristics curves in forward and reverse Biased |
| CU2. Identify Resistors in circuit | - Identify Resistor & its types - Recognize Coding &Color coding of resistor - Design series & Parallel circuit of Resistor - Use formulas for Series & parallel circuit of resistors |
| CU3. Identify Capacitor in circuit | - Identify Capacitor & its types - Recognize Coding &rating of Capacitor - Design Parallel and series circuit of Capacitor - Use formulas for Series & parallel circuit of Capacitor |
| CU4. Identify Inductor in circuit | - Identify an Inductor - Recognize Coding & Rating of Inductor - Use formulas for Series & Parallel circuit of Inductor - Analyze Circuit of Inductor |
| CU5. Identify IC’s Packages. | P1. Identify IC Packages & types. P2. Apply the appropriate ICs Packages in circuit |

Knowledge & Understanding

- Describe the diodes, polarities & their applications in circuits

- Explain the uses of Multimeter & power Supply

- Explain the data sheets

- Explain Resistor & their applications in Parallel & Series circuits

- Explain the uses of Multimeter & power

- Supply

- Explain the data sheets

- Describe Capacitor& their applications in circuits

- Explain the uses of Multimeter & power Supply

- Explain the data sheets

- Describe the Inductor& their applications in circuits

- Explain the uses of Multimeter & power Supply

- Explain the data sheets

- Describe the basics of IC Packages

- Understand the data sheets

Tools & Equipment

| SN | Tools |
| --- | --- |
| 1 | Multimeter |
| 2 | Power supply |
| 3 | Trainer |
| 4 | Resistor |
| 5 | Inductor |
| 6 | ICs |
| 7 | Capacitor |
| 8 | Source of data sheets |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

### 0713E&E12 Design a Rectifier Using Diode

Overview:

This competency standard covers the skills and knowledge required to. This Identify The parameter of Diode and Draw the characteristic curve of Diode, Design Full Wave Rectifier (two diode rectifier) and Design Full Wave Rectifier using Diode Bridge competency standard will help the candidate in construction of Diode rectifier and its uses in industry.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Identify The parameter of Diode and Draw the characteristic curve of Diode | - Identify anode and cathode of diode. - Check the diode for specification (Current, Voltage, and power rating) using data sheet. - Connect the Diode in forward and Revers bias condition - Perform the forward and reveres biases operation - Monitor the Output waveform on oscilloscope - Draw the characteristic curves in forward and reverse Biased - Record the data. |
| CU2. Construct half wave and Full Wave center tapped Rectifier | - Identify the required components for Full Wave and half wave Rectifier circuits. - Construct circuit Diagram of half wave Rectifier - Construct circuit Diagram of Full Wave Rectifier - Connect the circuit with the AC supply - Observe the input and Output wave form on oscilloscope - Calculate the ripple Factor - Calculate output voltage using proper formulas |
| CU3. Construct Full Wave Rectifier using Diode Bridge | - Identify the required components for Full Wave Bridge Rectifier circuit. - Construct circuit Diagram of Full Wave Bridge Rectifier - Connect the circuit with the AC supply - Observe the input and Output wave form on oscilloscope - Calculate the ripple Factor - Calculate output voltage using proper formulas |

Knowledge and understanding

- Study the basic of diodes, & their applications in circuits

- Knowledge of basic electronics

- Understand Multimeter& power Supply

- Understand the data sheets

- Explain basics of diodes, AC & DC

- Voltages, transformers & Rectifiers.

- Explain the uses of multimeter

- Explain the uses oscilloscope and Power Supply

- Understand the data sheets

- Explain the basics of diodes, AC & DC voltages, Transformers & Rectifiers.

- Explain the uses of multimeter

- Explain the uses of oscilloscope and power Supply

Tools & Equipment

| SN | Tools |
| --- | --- |
| 1 | Multimeter |
| 2 | Power supply |
| 3 | Trainer |
| 4 | Diodes |
| 5 | Digital |
| 6 | Oscilloscope |
| 7 | Datasheets |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

. Construct Full Wave Rectifier using Diode Bridge

### 0713E&E13Carry Out Diode Types and their Applications

Overview:

This competency standard covers the skills and knowledge required toMake voltage regulator using Zener diode and. Make Seven Segment Using Light Emitting Diode After completing this competency standard, the student will be able to design a voltage regulator circuit using a Zener diode to maintain a constant DC output voltage across the load in spite of variations in the input voltage or changes in the load current.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1.Make voltage regulator using Zener diode | - Draw the voltage Regulator circuit - Select the Zener diode and components as per requirement for voltage regulator - Install the components for voltage regulator circuits. - Connect the circuit with DC supply. - Vary the input voltage and note down the effects on output. |
| CU2. Make Seven Segment display Using Light Emitting Diode | - Draw the Seven Segment Display Circuit - Select required components for seven segment display - Install the components for Seven Segment Display Circuit - Verify the numeric digits on Seven Segment Display by providing proper input to its terminal |

Knowledge & Understanding

- Learn basic knowledge offender Diode& its applications

- Learn knowledge of components

- Learn to use oscilloscope &power Supply

- Understand the data sheets

- Learn to Solder the Components

- Learn adequate

- knowledge of hand tools

- Learn basic knowledge of LED & its applications

- Learn knowledge of components

- Learn to use power Supply

- Understand the data sheets

- Learn to Solder the Components

Tools & Equipment

| SN | Tools |
| --- | --- |
| 1 | Oscilloscope |
| 2 | Zener diode |
| 3 | Resistor, |
| 4 | Variable DC power supply, Millimeter |
| 5 | Soldering iron |
| 6 | Connecting wire |
| 7 | Resistor |
| 8 | Variable DC power supply, Millimeter |
| 9 | Voltmeter |
| 10 | Light Emitting diode |
| 11 | Soldering iron |
| 12 | Connecting wire |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

Make Seven Segment display Using Light Emitting Diode

### 0713E&E14 Implement Bipolar Junction Transistor (BJT) in Different Applications

Overview:

This competency standard covers the skills and knowledge required to Perform the Biasing of Transistors, Implement Transistor as an amplifier using CB Configuration, Implement Transistor as an amplifier using CC Configuration, Implement Transistor as an amplifier using CE Configuration, Design the circuit of Class A Power Amplifier and Implement BJT as a switch After completion of this competency standard the student will be able to regulate the current or voltage flow and implement a switch for electronic signals.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Perform the Biasing of Transistors | P1. Identify the type of transistor. P2. Identify the base collector & Emitter of transistors. P3. Insert the transistor on bread board. P4. Perform the standard Biasing of PNP & NPN Transistor |
| CU2. Implement Transistor as an amplifier using CB Configuration. | - Draw the Circuit of amplifier using CB configuration of transistor - Select the components for CB configurations. - Insert the components for CB configuration amplifier. - Calculate the gain of transistor in CB modes. - Draw VI characteristics curve for CB configuration. |
| CU3. Implement Transistor as an amplifier using CC Configuration. | - Draw the Circuit of amplifier using CC configuration of transistor - Select the components for CC configurations. - Insert the components for CC configuration amplifier. - Calculate the gain of transistor in CC modes. - Draw VI characteristics curve for CC configuration. |
| CU4. Implement Transistor as an amplifier using CE Configuration. | - Draw the Circuit of amplifier using CE configuration of transistor - Select the components for CE configurations. - Insert the components for CE configuration amplifier. - Calculate the gain of transistor in CE modes. - Draw VI characteristics curve for CE configuration. |
| CU5. Construct the circuit of Class A Power Amplifier | - Identify the required components for Class A Power Amplifier. - Select the component for Class A Power Amplifier - Construct the circuit of class A power amplifier using PNP OR NPN transistor. - Analyze the different parameter of Class A Power Amplifier - Monitor the Output waveform on oscilloscope - Draw the characteristic curves of Class A Power Amplifier - Calculate the Voltage gain and Power Gain of Class A Power Amplifier - Generate the Lab report |
| CU6. Implement BJT as a switch. | - Draw transistor switching circuit. - Select the components for switching circuits - Insert the components on bread board. - Verify switching operation of transistor using LED. |

Knowledge &Understanding

- Learn basic concepts of transistor & Biasing

- Study semiconductor theory

- Learn Doping Procedure.

- Study the datasheet of transistor

- Learn basic concepts & working principles of transistor

- Study semiconductor theory

- Procedure.

- Learn basics of Coupling Capacitor.

- Learn the V-I Characteristics

- Learn basic concepts & working principles of transistor

- Study semiconductor theory

- Learn Doping Procedure.

- Learn basics of Coupling Capacitor.

- Learn the V-I Characteristics

- Learn basic concepts & working principles of transistor

- Study semiconductor theory

- Learn Doping Procedure.

- Learn basics of Coupling Capacitor.

- Learn the V-I Characteristics

- Study the basic of BJTs, & their applications in circuits

- Knowledge of basic electronics

- Understand Multimeter, Oscilloscope

- & power Supply

- Understand the data sheets

- Learn basic concepts of transistor

- Study semiconductor theory

- Procedure.

- Learn basics of Coupling Capacitor.

Tools & Equipment

| SN | Tools |
| --- | --- |
| 1 | Power |
| 2 | Oscilloscope |
| 3 | Trainer |
| 4 | Supplies |
| 5 | Resistors |
| 6 | Datasheets |
| 7 | Transistors |
| 8 | Trainer |
| 9 | MultiMate |
|  | Capacitors |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

Construct the circuit of Class A Power Amplifier

### 0713E&E15 Implement Field Effect Transistor (FET) In Different Applications

Overview:

This competency standard covers the skills and knowledge required to perform the Biasing of FET, Implement MOSFET as a switch, Draw the VI characteristics curves for FETs, Design the circuit of Common Drain (CD) Amplifier, Design the circuit of Common Gate (CG) Amplifier, design a switching Circuit Using MOSFET and Design a Low voltage transistor based regulated power supply. After completion of this competency standard the student will be able to implement the FET (Field Effect Transistor) to control the current flow through the device.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Perform the Biasing of FET | P1. Identify the type of FET. P2. Identify the Gate, Drain & Source of FET. P3. Insert the FET on bread board. P4. Perform the standard Biasing of (N-Channel, P-Channel) FET. P4. Measure the Gate-Source voltage (Veg’s) & Threshold Voltage (Vth) |
| CU2. Implement MOSFET as a switch. | - Draw switching circuit of MOSFET. - Select the components for switching circuits - Insert the components bread board. - Verify switching operation of MOSFET using LED. |
| CU3. Draw the VI characteristics curves for FETs | P1. Construct an amplifier circuit using FETs P2. Apply Vds & Vgs P3 Measure the drain current P4. Draw VI characteristic curves |
| CU4. Construct the circuit of Common Drain (CD) Amplifier | - Draw the Circuit of amplifier using CD configuration of FET - Select the components for CD configurations. - Insert the components for CD configuration amplifier On breadboard - Perform standard biasing for CD configuration - Apply the input signal and observe the output on oscilloscope. - Observe and Record the difference between input and output. |
| CU5. Construct the circuit of Common Gate (CG) amplifier | - Draw the Circuit of amplifier using CG configuration of FET - Select the components for CG configurations. - Install the components for CG configuration amplifier. - Perform standard biasing for CG configuration - Apply the input signal and observe the output on oscilloscope. - Observe and Record the difference between input and output. |
| CU6. Construct a Low voltage transistor based regulated power supply | - Draw the Schematic diagram of power supply - Select the components for power supply - Install the power supply circuit. - Connect the circuit with DC supply. - Vary the input voltage and note down the effects on output. |

Knowledge & Understanding

- Learn the basics of FET

- Learn the concept of FET Biasing.

- Learn the power rating of FET

- Study The datasheet of FET

- Learn the basics of FET

- Learn the concept of

- FET Biasing.

- Learn the power rating of FET

- Study The datasheet of FET

- Learn the Switching theory

- Learn the behavior of current and voltage in FET’s

- Learn biasing mechanism and basic formulae of FET’s

- Learn the vgs, VDs, Idss&Rds as per datasheet.

- Study the basic of FETs, & their applications in circuits

- Knowledge of basic

- electronics

- Understand Multimeter & power Supply

- Understand the data sheets

- Study the basic of FETs, & their applications in circuits

- Knowledge of basic electronics

- Understand Multimeter & power Supply

- Understand the data sheets

- Study the basic of FETs, & their applications in circuits

- Knowledge of basic electronics

- Understand Multimeter & power Supply

- Understand the data sheets

- Study the basic of BJTs, ICs & their applications in circuits

- Knowledge of basic electronics and amplifier

- Understand Multimeter, Oscilloscope

- & power Supply

- Understand the data sheets

Tools & Equipment

| SN | Tools |
| --- | --- |
| 1 | FET Transistors |
| 2 | Multimeter |
| 3 | Capacitors |
| 4 | Resistors |
| 5 | Power Supplies |
| 6 | Trainer |
| 7 | Simple FET |
| 8 | Digital |
| 9 | MOSFET |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

Construct the circuit of Common Gate (CG) amplifier

### 0713E&E16 Apply (Uni Junction Transistor, Silicon Control Rectifier, Diac and Triac) in Various Application

Overview:

This competency standard covers the skills and knowledge required to Implement the UJT in electronic circuits as switch, Implement the SCR in electronic circuits as switch and Construct the dimmer circuit using Diac & Triac. After the completion of this standard the candidate will be able to install Uni junction Transistor (UJT), Silicon-controlled rectifier (SCR) in power Control Application.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Construct relaxation oscillator using UJT | - Draw the Schematic diagram of relaxation oscillator using UJT. - Select the components for relaxation oscillator. - Construct the relaxation oscillator circuit on bread board. - Connect the circuit with DC supply. - Observe the output wave shape on oscilloscope. - Vary the value of input resistor and record the effect on output. |
| CU2. Construct switching circuit using SCR | - Draw the Schematic diagram of switching circuit using SCR. - Select the components for SCR switching circuit. - Construct the SCR switching circuit on bread board - Connect the circuit with DC supply. - Verify switching operation by triggering the SCR |
| CU3. Construct the dimmer circuit using DIAC & TRIAC | - Draw the Schematic diagram of dimmer circuit using TRIAC & DIAC. - Select the components for dimmer circuit. - Construct the dimmer circuit. - Connect the circuit with AC supply. - Vary the potentiometer and record the effect on load (Fan or Lamp) |

Knowledge & Understanding

- Learn the basics of UJT

- Learn the uses of oscilloscope and power Supply

- Understand the data sheets

- Learn adequate knowledge of hand tools

- Learn basics of SCR

- Understand the data sheets

- Learn adequate knowledge of hand tools

- Learn the uses of oscilloscope, power Supply& Multimeter

- Learn the basics of diac & triac

- Understand the data sheets

- Learn adequate knowledge of hand tools

Tools & Equipment

| SN | Tools |
| --- | --- |
| 1 | Oscilloscope |
| 2 | UJT, |
| 3 | Resistor |
| 4 | Variable DC power supply, |
| 5 | Multimeter |
| 6 | Soldering iron, wire |
| 7 | Breadboard or trainer |
| 8 | Supply |
| 9 | Trainer/Breadboard |
| 10 | Multimeter |
| 11 | SCR, Diodes, Resistors, Inductors, Capacitors & connecting wire |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

### 0713E&E17 Design Operation Amplifier

Overview:

This competency standard identifies the competencies required to Construct a Non-inverting amplifier using operational amplifier, construct an Inverting amplifier using and Construct a differentiator circuit using operational amplifier. Construct Operation Amplifier.

| Competency Units | Performance Criteria |
| --- | --- |
| - Construct Non-inverting amplifier using operational amplifier | - Draw the circuit of non-inverting amplifier using operational amplifier. - Select the components for non-inverting operational amplifier. - Insert the components of non-inverting operational amplifier. - Connect the circuit to DC supply. - Apply input voltage to the circuit and measure the output voltage. - Calculate the gain for non-inverting operational amplifier. |
| - Construct an Inverting amplifier using operational amplifier | - Identify the Operational amplifier and its terminals (Inverting, Non-inverting Inputs &Outputs) with the Help of Datasheet - Identify different parameters (Current, Voltage, and power rating) of Op-Amp using datasheet. - Draw the Schematic diagram of Inverting Op-Amp. - Select the components for Inverting Op-Amp. - Implement Non-Inverting Op-Amp circuit. - Perform the operations of Inverting Op-Amp circuit. - Measure the output frequency response & gain - Draw the characteristic curves of Op-Amp. - Generate the Output report |
| - Construct a differentiator circuit using operational amplifier | - Draw the Schematic diagram of differentiator circuit using Op-Amp. - Select the components for differentiator circuit. - Implement differentiator circuit. - Perform the operations of differentiator circuit. - Measure the output, frequency response & gain - Draw the characteristic curves of differentiator circuit. - Generate the Output report |

Knowledge and understanding

- Study the basics of Transistors, ICs, Capacitors, Resistors, Op-Amp & their applications in circuits

- Explain the basics of electronics & its applications

- Understand Multimeter, Oscilloscope

- & power Supply& their applications

- Understand the data sheets

- Study the basics of Transistors, ICs, Capacitors, Resistors, Op-Amp & their applications in circuits

- Explain the basics of electronics& its applications

- Understand Multimeter, Oscilloscope

- power Supply& their applications

- Understand the data sheets

- Study the basics of Transistors, ICs, Capacitors, Resistors, Op-Amp & their applications in

- circuits

- Explain the basics of electronics, Differentiator and their applications

- Understand Multimeter, Oscilloscope

- & power Supply& their applications

- Understand the data sheets

Tools & Equipment

| SN | Tools |
| --- | --- |
| 1 | Functions Generator |
| 2 | Datasheets |
| 3 | Digital Oscilloscope |
| 4 | Capacitors |
| 5 | Inductors |
| 6 | Op-Amp |
| 7 | Power supply |
| 8 | Trainer |
| 9 | Multimeter |
| 10 | Resistors |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

Construct an Inverting amplifier using operational amplifier

### 0713E&E18Verify Truth Tables of Digital Gates

Overview:

This competency standard covers the skills and knowledge required to Verify the truth table of AND gate, Verify the truth table of OR gate, Verify the truth table of NOT gate, Verify the truth table of NAND gate, Verify the truth table of NOR gate, Verify the truth table of XOR gate and Verify the truth table of XNOR gate Verify the truth table of AND gate, OR gate, NOT gate, NAND gate, NOR gate, XOR gate and XNOR gate

.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Verify the truth table of AND gate | - Identify the symbol of logic gate, IC & logic function. - Place (AND gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics ant input pins. - Record & verify the output result against each given input. |
| CU2. Verify the truth table of OR gate | - Identify the symbol of logic gate, IC & logic function. - Place (OR gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics ant input pins. - Record & verify the output result against each given input. |
| CU3. Verify the truth table of NOT gate | - Identify the symbol of logic gate, IC & logic function. - Place (NOT gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics ant input pins. - Record & verify the output result against each given input. |
| CU4. Verify the truth table of NAND gate | - Identify the symbol of logic gate, IC & logic function. - Place (NAND gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics ant input pins. - Record & verify the output result against each given input. |
| CU5. Verify the truth table of NOR gate | - Identify the symbol of logic gate, IC & logic function. - Place (NOR gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics ant input pins. - Record & verify the output result against each given input. |
| CU6. Verify the truth table of XOR gate | - Identify the symbol of logic gate, IC & logic function. - Place (XOR gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics ant input pins. - Record & verify the output result against each given input. |
| CU7. Verify the truth table of XNOR gate | - Identify the symbol of logic gate, IC & logic function. - Place (NOR gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics ant input pins. - Record & verify the output result against each given input. |

Knowledge & Understanding

- Study logic gates Logic gates. AND, OR, NAND, NOR, NOT, XOR and XNOR.

- Study the Boolean expression of AND, OR, NAND, NOR, NOT, XOR and XNOR, gate and its equivalent electrical circuit

- Define Universal gate and enlist its types.

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | AND gate (7408 2-input Quad) |
| 2 | OR gate (7432 2-input Quad) |
| 3 | NOT gate (7404 Hex) |
| 4 | NAND gate (7400 2-input Quad) |
| 5 | NOR gate (7402 2-input Quad) |
| 6 | X-OR gate (7486 2-input Quad) |
| 7 | X-NOR gate (74266 2-input Quad) |
| 8 | Bread board |
| 9 | DC supply (5 V) |
| 10 | LED |
| 11 | Connecting leads |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

Verify the truth table of XNOR gate

### 0713E&E19Construct & Verify Combinational Logic Circuit

Overview:

This competency standard covers the skills and knowledge required to, Apply Karnaugh mapping & Boolean algebra to simplify logic expressions, Construct & verify the truth table of Half adder, Construct & verify the truth table of Full adder, Construct & verify the truth table of Half Subtraction,   Operate seven segment display with seven segment decoder, Construct & verify the truth table of Full Subtraction, Verify Encoder, Verify Encoder and Verify multiplexer and DE- multiplexer Construct & verify the truth table of Half adder, full adder and Half Subtraction.

| Competency Units | Performance Criteria |
| --- | --- |
| - Apply Karnaugh mapping & Boolean algebra to simplify logic expressions | - Identify the SOP & POS - Apply Boolean algebra & Karnaugh mapping to simplify SOP & POS. - Construct logic circuits with simplified SOP & POS. |
| - Construct & verify the truth table of Half adder | - Place (AND gate IC) & (XOR gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics at input pins. - Record & verify the output result against each given input - Design, Construct, and test a half-adder circuit using one XOR gate and two NAND gates. |
| - Construct & verify the truth table of Full adder | - Place (AND gate IC) & (XOR gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics at input pins. - Record & verify the output result against each given input - Design, Construct, and test a full-adder circuit using two ICS, &7486 and &7400. |
| - Construct & verify the truth table of Half Subtraction | - Place (AND, NOT&XOR gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics at input pins. - Record & verify the output result against each given input |
| - Construct & verify the truth table of Full Subtraction | - Place (AND, NOT&XOR gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics at input pins. - Record & verify the output result against each given input. |
| - Verify Decoder | - Place (Decoder IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics at input pins. - Record & verify the output result against each given input. |
| - Operate seven segment display with seven segment decoder. | - Insert (7 segment decoder IC) and 7 segment display on bread board. - Identify the input, output, Vcc and ground pin. - Connect segment display with seven segment decoder input output pins. - Record & verify the output result against each given input. |
| - Verify Encoder | - Place (Encoder IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LED to the output pin of IC and apply different logics at input pins. - Record & verify the output result against each given input. |
| - Verify multiplexer and DE- multiplexer | - Implement following function with multiplexer F(ABC)=∑ (0,2,3,4,5,6): - Implement 4-to-1 mux and one 2-to-1 mux. - Implement 1-to-4 dmux using 1-to-2 dmux. |

Knowledge & Understanding

- Describe the laws and rules of Boolean algebra.

- Understanding of commutative. • And distributive expiration. That is, A • (B + C) = (A • B) + (A • C) and A + (B • C) = (A + B) • (A + C).

- Study the combinational logic circuit. (Half adder, Full adder, Half subtraction, Full subtraction, Binary Multiplier, Magnitude

- Comparator)

- Study the Product-of-Sums& SOP Simplification

- Knowledge of Don't-Care Conditions

- Understanding of Karnaugh Map of four Variable.

- Understanding of Decoders & Encoders&   Multiplexers.

- Knowledge of Pin configuration of iCs

- Knowledge of7 segment display.

- Explain pin 7 segment display and common cathode 7 segment display.

- Define limiting resistor

- Understanding how to implement functions using multiplexers.

- To study DE multiplexer

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | AND gate (7408 2-input Quad) |
| 2 | OR gate (7432 2-input Quad) |
| 3 | NOT gate (7404 Hex) |
| 4 | NAND gate (7400 2-input Quad) |
| 5 | NOR gate (7402 2-input Quad) |
| 6 | X-OR gate (7486 2-input Quad) |
| 7 | X-NOR gate (74266 2-input Quad) |
| 8 | Bread board |
| 9 | DC supply (5 V) |
| 10 | LED |
| 11 | Connecting leads |
| 12 | Bread board |
| 13 | Resistances (1K ohm) |
| 14 | Connecting leads |
| 15 | DC supply (5 V) |
| 16 | Mux KL-33006 block e |
| 17 | Seven segment display |
| 18 | 74LS47 IC |
| 19 | Mux KL-33006 block |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

Operate seven segment display with seven segment decoder.

### 0713E&E20 Construct and Verify Function of Flip Flops

Overview:

This competency standard covers the skills and knowledge required to Construct and verify the truth table of RS latch using NAND gate, Construct and verify the truth table of clocked RS latch using NAND gate, verify function of D flip flop, verify function of JK/T flip flop construct and verify the truth table of RS latch using NAND gate, clocked RS latch using NAND gate, D flip flop and JK flip flop.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Construct and verify the truth table of RS latch using NAND gate | - Place (NAND gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LEDs to outputs pins. - Apply different logic inputs to Record & verify the output result against each given input. |
| CU2. Construct and verify the truth table of clocked RS latch using NAND gat | - Place (NAND gate IC) on bread board. - Identify the input, output, Vcc and ground pin. - Connect LEDs to outputs pins. - Apply different logic inputs to Record & verify the output result against each given input. |
| CU3. Verify function of D flip flop. | - Insert (D flip flop) IC on bread board. - Identify the input, output, Vcc and ground pin. - Connect LEDs\ Scope to outputs pins. - Apply different logic inputs to Record & verify the output result against each given input. |
| CU4. Verify function of JK/T flip flop | - Insert 74112 (JK flip flop) IC on bread board. - Identify the input, output, Vcc and ground pin. - Connect LEDs\ Scope to outputs pins. - Apply different logic inputs to Record & verify the output result against each given input. |

Knowledge & Understanding

- Define Latch

- How many inputs are given to SR Latch?

- Which IC is used for NAND and NOR gate?

- What is clocked RS flip flop.

- What is difference between Latch and flip flop?

- Symbols for Combinational Elements (Symbols for Flip-Flops.)

- Define D flip flop.

- Define clock pulse.

- What is difference between synchronous & asynchronous input

- Define JK/T flip flop.

- Draw the symbol of JK flip flop

- Which IC is used for JK flip flop

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | AND gate (7408 2-input Quad) |
| 2 | OR gate (7432 2-input Quad) |
| 3 | NOT gate (7404 Hex) |
| 4 | NAND gate (7400 2-input Quad) |
| 5 | NOR gate (7402 2-input Quad) |
| 6 | X-OR gate (7486 2-input Quad) |
| 7 | X-NOR gate (74266 2-input Quad) |
| 8 | Digital clock |
| 9 | DC supply (5 V) |
| 10 | LED |
| 11 | Connecting leads |
| 12 | Bread board |
| 13 | JK Flip Flop (74112 Dual |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

Verify function of JK/T flip flop

### 0713E&E21 Use 555 IC as Multivibrator

Overview:

This competency standard covers the skills and knowledge required to.  Construct 555 IC as Actable Multivibrator, Construct 555 IC as Mono-stable Multivibrator, and Construct 555 IC as Bi-stable Multivibrator and verify its set and reset conduction. Construct 555 IC as Actable, nonstable & beatable Multivibrator and observe their outputs.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Construct 555 IC as Astable Multivibrator | - Draw circuit diagram for Astable Multivibrator - Place 555 IC on bread board/trainer - Make connection as per diagram. - Apply voltage to circuit. - Recod the output signal wave shape from oscilloscope. |
| CU2. Construct 555 IC as Mono-stable Multivibrator | - Draw circuit diagram for Mono-stable Multivibrator - Place 555 IC on bread board/trainer. - Make connection as per diagram. - Apply voltage to circuit and give triggering pulse at input pin. - Recode the output signal wave shape from oscilloscope. |
| CU3. Construct 555 IC as Bi-stable Multivibrator and verify its set and reset conduction | - Draw circuit diagram for Bi-stable Multivibrator - Place 555 IC on bread board/trainer. - Make connection as per diagram. - Apply voltage to circuit and give triggering pulse at input pin. - Recod the output signal wave shape from oscilloscope. |

Knowledge & Understanding

- Describe basic elements of 555 timer IC.

- Name pins of 555 timer IC.

- What is function of voltage control input?

- What is a stable Multivibrator?

- What is non stable Multivibrator?

- What is beatable Multivibrator

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | DC supply (5 V |
| 2 | Connecting leads |
| 3 | 555 Timer IC |
| 4 | Capacitor 0.1µF |
| 5 | Resistors 10 KΩ |
| 6 | dual trace Oscilloscope 0-20MHZ |
| 7 | Capacitor 0.01µF |
| 8 | Bread board |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

Construct 555 IC as Bi-stable Multivibrator and verify its set and reset conduction

### 0713E&E22 Construct Shift Registers and Counters with The Help of Flip Flops

Overview:

This competency standard covers the skills and knowledge required toConstruct a 4-bit shift register by Using Flip Flops, construct a 4-bit binary counter Using Flip Flops, Construct 4-bit synchronous Counter with D flip-Flops and Repair & Troubleshoot combinational logic circuits

| Competency Units | Performance Criteria |
| --- | --- |
| - Construct a 4 bit shift register by Using Flip Flops | - Draw circuit diagram 4-bit register. - Make connection of D-Flip Flop as per diagram to construct 4-bit shift register. - Apply data at the input of register and give clock pulse - Recod the output according to the input. |
| - Construct a4-bit binary counter Using Flip Flops | - Draw circuit diagram counter. - Make connection of JK-Flip Flop as per diagram to construct 4-bit binary counter. - Connect LEDs to the outputs pins. - Apply the clock pulse and record the output. |
| - Construct 4-bitsynchronous Counter with D flip-Flops | P1. Draw circuit diagram synchronous counter. P2. Make connection of JK-Flip Flop as per diagram to construct 4-bit synchronous counter. P3. Connect LEDs to the output pins. P4. Apply the clock pulse and record the output. |
| - Troubleshoot different combinational logic circuits. | - Identify faults in different combinational logic circuits IC’s. - Find the faults. - Troubleshoot the faults. |

Knowledge & Understanding

- Symbols for Combinational Elements (Symbols for Registers, Counters, RAM.)

- understand the function of registers, and counters in digital circuits

- Knowledge of basic computer memories and its types.

- Explain precautions when handling components.

- Demonstrate proper use of ESD equipment.

- Explain how to distinguish hardware from software issues

- Show how pulsars are used for digital signal tracing and how logic probes are used to verify states in digital equipment.

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | Software Simulator and IDE’s. |
| 2 | DLD trainer |
| 3 | Multimeter |
| 4 | IC’s and Components |
| 5 | Logic tester |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

Troubleshoot different combinational logic circuits

### 0713E&E23 Implement Diode and Thyristor in Power Control Application.

Overview:

This competency standard covers the skills and knowledge required toBuild forward bias circuit of diode and observe its behavior, Build reverse bias circuit of diode and observe its behavior, Use SCR to build Phase control rectifier, Build force commutated circuit for SCR and Build full wave converter and observe natural commutation. Build forward bias and reverse bias circuit of diode and observe its behavior.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Build forward bias circuit of diode and observe its behavior. | - Draw forward bias circuit diagram of diode. - Make connection as per diagram. - Connect volt meter parallel to diode and ampere meter in series - Turn on the supply and take readings of ampere meter as well as voltmeter. - Note the behavior of diode |
| CU2. Build reverse bias circuit of diode and observe its behavior. | - Draw reverse bias circuit diagram of diode. - Make connection as per diagram. - Connect volt meter parallel to diode and ampere meter in series - Turn on the supply and take readings of ampere meter as well as voltmeter. - Note the behavior of diode. |
| CU3. Use SCR to build Phase control rectifier. | - Draw circuit diagram of phase control rectifier. - Make connection as per diagram. - Switch on the power supply. - Vary the value of potentiometer to set the trigger level for the SCR - Recode the output signal wave shape from oscilloscope. |
| CU4. Build force commutated circuit for SCR. | - Draw circuit diagram of force commutated circuit for SCR. - Make connection as per diagram. - Switch on the power supply and apply triggering pulse at the gate of SCR. - Connect oscilloscope across the load resistor and record the output wave shape |
| CU5. Build full wave converter and observe natural commutation. | - Draw circuit diagram of full wave converter. - Make connection as per diagram. - Switch on the power supply and apply triggering pulse at the gate of SCR. - Connect oscilloscope across the load resistor and record the output wave shape |

Knowledge & Understanding

- Define forward biasing

- Define reverse biasing

- Define Reverse breakdown voltage

- Define junction potential

- Define SCR

- Describe the operation of SCR

- Define phase control rectifier

- Define triggering of SCR

- Describe the use of SCR

- Define VAK

- Define commutation and its types

- Define natural commutation.

- Define force commutation

- Name the methods of force commutation.

- Describe the use of force commutation

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | Power diode |
| 2 | Resistor of 1 K ohms |
| 3 | Volt meter |
| 4 | Ampere meter |
| 5 | DC supply |
| 6 | Connecting leads |
| 7 | Power supply |
| 8 | Centre tapped transformer |
| 9 | Bread Board |
| 10 | Oscilloscope |
| 11 | Connecting leads |
| 12 | Resistor of 10 K ohms |
| 13 | Capacitor 10uF |
| 14 | SCR |
| 15 | Connecting leads |
| 16 | Power supply |
| 17 | Bread Board |
| 18 | Oscilloscope Dual |
| 19 | Potentiometer 500 o |
| 20 | Resistor of 10 K & 100 K ohms |
| 21 | Inductor 2mH |
| 22 | Diode |
| 23 | SCR |
| 24 | Bread Board |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

Build full wave converter and observe natural commutation

### 0713E&E24 Implement Single Phase and Three Phase Inverter

Overview:

This competency standard covers the skills and knowledge required toConstruct single phase half bridge inverter and observe its output, construct single phase full bridge inverter and observe its output and Construct three phase bridge inverter and observe its output Construct single phase half bridge inverter, single phase full bridge, three phase bridge inverter and observe their outputs.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Construct single phase half bridge inverter and observe its output | - Draw circuit diagram of single-phase half bridge inverter. - Make connection as per diagram. - Apply triggering pulses to circuit with the help of signal generator. - Connect oscilloscope across the load resistor and record the output wave shape |
| CU2. Construct single phase full bridge inverter and observe its output | - Draw circuit diagram of single-phase full bridge inverter. - Make connection as per diagram. - Apply triggering pulses to circuit with the help of signal generator. - Connect oscilloscope across the load resistor and record the output wave shape |
| CU3. Construct three phase bridge inverter and observe its output | - Draw circuit diagram of three-phase bridge inverter. - Make connection as per diagram. - Apply output of controller IC to the gates of MOSFET’s for triggering pulses. - Connect oscilloscope across the load resistor and record the output wave shape |

Knowledge & Understanding

- Define Inverter

- Enlist different types of inverter

- Define half bridge inverter

- Define Full bridge inverter

- Define 3 phase bridge inverter

- Describe the use of function generator.

- Define use of controller IC

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | Power MOSFET IRF 840 |
| 2 | Power Diode 1N4007 |
| 3 | DC Source 12V |
| 4 | Resistor 10K |
| 5 | Function Generator |
| 6 | IC 7404 |
| 7 | Controller IC |
| 8 | Oscilloscope |
| 9 | Connecting leads |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

Construct three phase bridge inverter and observe its output

### 0713E&E25 Control speed of DC/AC Motor

Overview:

This competency standard covers the skills and knowledge required to.  Implement armature control method to control the speed of DC motor with 3 phase control bridge rectifier and Bridge rectifier, Implement Field control method to control the speed of DC motor with 3 phase bridge rectifier and control Bridge rectifier, Implement Field control method & armature control method to control the speed of DC motor with 3 phase bridge rectifier, Implement AC to AC converter to control the speed of 3 phase AC motor, Implement single phase cycle-converter with center tapped transformer and SCR’s and change the frequency of output AC signal and Implement synch’s to transmit torque Implement armature control method, field control method & both armature and field control methods to control the speed of DC motor with 3 phase control bridge rectifier and Bridge rectifier and implement AC to AC converter to control the speed of 3 phase AC motor.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Implement armature control method to control the speed of DC motor with 3 phase control bridge rectifier and Bridge rectifier. | - Draw circuit diagram of armature control method to control the speed of DC motor. - Make connection as per diagram. - Apply output of controller IC to the gates of all SCR’s - Now change the triggering of SCR’s and record the effect on speed of DC motor. |
| CU2. Implement Field control method to control the speed of DC motor with 3 phase bridge rectifier and control Bridge rectifier. | - Draw circuit diagram of field control method to control the speed of DC motor. - Make connection as per diagram. - Apply output of controller IC to the gates of all SCR’s - Now change the triggering of SCR’s and record the effect on speed of DC motor. |
| CU3. Implement Field control method & armature control method to control the speed of DC motor with 3 phase bridge rectifier. | - Draw circuit diagram of field control & armature control method to control the speed of DC motor. - Make connection as per diagram. - Apply output of controller IC to the gates of all SCR’s - Now change the triggering of SCR’s and record the effect on speed of DC motor. |
| CU4. Implement AC to AC converter to control the speed of 3 phase AC motor. | - Draw circuit diagram of AC to AC converter to control the speed of AC motor. - Make connection as per diagram. - Apply output of controller IC to the gates of all SCR’s - Now change the triggering of SCR’s and record the effect on speed of AC motor. |
| CU5. Implement single phase cycle-converter with center tapped transformer and SCR’s and change the frequency of output AC signal. | - Draw circuit diagram of single phase cycle-converter to produce variable frequency AC signal. - Make connection as per diagram. - Apply output of controller IC to the gates of all SCR’s - Apply Triggering to SCR’s 1 & 3 for 1st 25 cycles to get positive half cycle and apply triggering for remaining 25 half cycle to SCR’s 2 & 4 to get the negative half cycle with controller IC. - Connect the oscilloscope across the load and record the output |
| CU6. Implement synchros’s to transmit torque | - Place synchrony transmitter and synchrony receiver at two different places. - With help of cord establish connection between corresponding terminal of TX& Rx i.e. S1 to S1, S2 to S2 and S3 to S3 respectively. - Give single phase AC supply to the rotor of both TX and Rx. - Rotate the rotor of TX in step to 30 degrees and observe the new position of rotor of Rx. - Enter the input angular position & output angular position in the table. |

Knowledge & Understanding

- .Define DC motor

- Enlist parts of DC motor

- Enlist the methods to control the speed of DC motor.

- Describe armature control method.

- Describe field control method.

- What is range of firing angle of SCR’s

- Define use of controller IC.

- Define AC to AC conversion

- .Define cycle-converter.

- Enlist types of cycle-converter

- Define single phase cycle-converter.

- Define three phase cycle-converter.

- .  Define synchs.

- Define synchrony generator or TX

- Define synchrony receiver or Rx

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | Multimeter |
| 2 | Connecting leads |
| 3 | Synchrony Transmitter & Receiver |
| 4 | Supply |
| 5 | Power supply |
| 6 | Connecting leads |
| 7 | CRO |
| 8 | Firing Circuit |
| 9 | SCR’s |
| 10 | Power Electronics Trainer Kit |
| 11 | Power Diode 1N4007 |
| 12 | Controller IC |
| 13 | DC Motor |
| 14 | AC motor |
| 15 | Connecting leads |
| 16 | Three phase supply |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard

Implement armature control method to control the speed of DC motor with 3 phase control bridge rectifier and Bridge rectifier.

### 0713E&E26 Program PLC Using Ladder Logic

Overview:

This competency standard covers the skills and knowledge required to Install PLC software and Simulator and Program apply using simulator. After this competency standard, the trainee will be able to get basic knowledge and competence to program and understand ladder logic.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Install PLC software and Simulator | - Install the PLC Programming - Software as per PLC manufacturer such as PRs Logic 5000 and PRs links classic etc. - Select the best and most frequently used Simulator for programming - Select the module - Input the generic value as per universal settings (check from internet/helped) - Start The Simulator. - Open PLC programming software |

| CU2. Program a PLC using simulator | P1. Open the programming software as per PLC manufacturer P2. Ensure that the Simulator is connected and is in ON condition P3. Create a basic ladder logic program for ON/OFF of a bulb using Examine ON and Examine OFF switch P4. Create a basic ladder logic program for Simple Start/Stop Ladder Logic Relay P5. Create a basic ladder logic program for Single Push Button On/Off Ladder Logic P6. Create a basic ladder logic program for with On Delay Timer P7. Create a basic ladder logic program for with Off Delay Timer P8. Create a basic ladder logic program for Traffic signal lights P9.Create a basic ladder logic program for Elevator Control. |
| --- | --- |

Knowledge & Understanding

- Describe the advantages of the PLC

- Describe the major components and

- Describe the applications of PLC.

- Describe the types of input and output signals.

- Define Piping and instrumentation diagram/drawing (P&ID)

- List down the PLC, DCS programming software

- List Down the SCADA, HMI development software

- Knowledge of Relevant documents and documentation procedures

- Define Ladder Logic

- Explain the Examine ON and Examine OFF switch

- How to use ladder logic to create basic programs?

- What is push button?

- Define the types of Timers.

- What is the working of traffic light?

- Describe the working of an elevator.

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | PLC Software |
| 2 | Helping Manual |
| 3 | Simulator |
| 4 | User guide |
| 5 | Page |
| 6 | Pen |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

Program a PLC using simulator

### 0713E&E27 Carry out Industrial Automation and PLC Installation

Overview:

This competency standard covers the skills and knowledge required to Analyse user requirements and specification, prepare work plan, Design and program PLC, Test the PLC, ensure quality and productivity standards, Installation and Commissioning and Carry out maintenance of PLC. After this competency the candidate will be able to design, develop, install and maintain the automation and control system needed for Automation and Control.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1.Analyze user requirements and specifications for PLC installation | - Draw the general value chain of the end user industry - Highlight the set of activities that a firm operating in a specific industry performs in the value chain drawing - Enlist the equipment/gauges/sensors/actuators/transducers used in different stages of the process - Identify critical stages in the process - Identify the safety aspect required in the critical stages of the process - Analyse the possible automation in the existing processes and global trends in automation - Analyse the client requirement at broad level from the proposal - Generate a report of various industrial processes involved in industry - Collect the required specification of the equipment (if already prepared by the user) and clarify the technical specification. |
| CU2.Prepare work plan for PLC Installation | - Suggest globally practiced and accepted automation systems if the user is not aware of the technical specifications - List down the sub systems that are involved in the process - List down sensors and actuators requirement. - Collect information on process logic - Collect information for operator station screens - Decide on whether the system can be developed as per the user requirement - Support the project manager in calculating the time required for each stage to ensure completion of project - Assist in preparing the work plan with deliverables and timelines - Explain the expected output to the user - Calculate the number of days needed for commissioning of the panel at site - Summarize the user requirement. |
| CU3.Design and program PLC | - Develop PLC application as per user requirement by following the standard operating procedure (SOP) of the organization - Apply approved engineering concepts, processes and principles in developing the PLC application - Install organization approved software (system and application software) to develop the system - Identify the requirement of indications, switchgears and accessories - Develop the control circuit drawing - Prepare wiring plans - Integrate the main process system with the sub-systems as per the user requirement (e.g., using communication protocol) - Ensure that safety aspect of the process is captured in the design plan - P9. Program PLC as per FDF Program SCADA Application - PLC-SCADA Communication - P12. Create backup copies of all designs developed for control panel and store in a secure location - Prepare a product manual and store them for future references. |
| CU4. Test the PLC | - Locate field devices and their interface to PLC - Test the system in off line mode using simulator - Test the gauges independently for integration of main system with the sub-systems (if applicable) - Verify that the system conforms with all the user specifications during testing - Rework if there are any issues found and fix them - Send the test report for review to the customer - Perform Factory Acceptance Test (FAT) - Perform site acceptance test plan |
| CU5. Ensure quality and productivity standards | - Ensure timely delivery of the control panel design as per agreed timeline - Ensure that total cost and man hours spent is as per the budget planned - Ensure compliance with relevant regulations, standards and codes of practices - Ensure compliance of the application with manufacturing requirements and process capabilities analysis of the organization - Ensure that the design conforms with normal safety standards - Develop reliable panels so that the system does not fail during the usage |
| CU6.Installation and Commissioning | - Check availability of panel and tools required for installation - Check the internal panel wiring and ensure that it is in accordance with the design drawing - Carry out insulation check of internal panel wiring and devices within the panel - Check if batteries and chargers have been assembled in accordance with the manufacturers recommended procedures - Identify the conductors size and capacity for installation - Ensure that the panel is positioned as prescribed, following safety norms - Make connections to socket outlets, switches and protective conductors - Perform settings as per customer requirements on the equipment in each of the panels - Test all control system interlocks - Check each digital control point by comparing the command at the control panel and status of the device that it controls - Ensure that fuses, switches and other protective devices are labelled correctly - Follow the grounding and earthling procedures while commissioning - Put danger and warning notices, (if necessary) - Test continuity, insulation resistance, functions of all devices, etc., after completion of installation |
| CU7. Carry out maintenance of PLC | - Select and use required tools and equipment as per the job requirement - Identify the faults as per the LED indications on module - Identify different signal modules (DI, DO, AI, AO) and take necessary action as per job requirement - Identify & Check the terminals inside IO panels and take necessary action as per the job requirement - Check & test the condition of input power supply to IO panel, input/output power of SMPS and take necessary action as per job requirement - Check the conditions of breakers and fuses and take necessary action as per the job requirement |

Knowledge and understanding

- Define the term ‘value chain’ of industry.

- Define human machine interface (HMI)

- Define the Standard operating procedure (SOP) of the organization for control panel development process.

- How to prepare a General arrangement drawing?

- Define IEC standards

- Knowledge of Relevant documents and documentation procedures used in the process

- Define formula for Electrical load calculations

- Explain the Piping and instrumentation diagram/drawing (P&ID)

- Describe the Safety aspects to be inbuilt

- n the control system as per the process requirement

- Describe the advantages of the PLC

- Describe the major components and

- Describe the applications of PLC.

- Describe the types of input and output signals.

- Define Piping and instrumentation diagram/drawing (P&ID)

- How to prepare wiring diagram

- DCS programming software

- P8. List Down the SCADA, HMI development software

- P9. Knowledge of Relevant documents and documentation procedures used in the process

- What are the Testing process and parameters involved in the testing?

- Define the use of Emulator(Simulator)

- Panel?

- How to troubleshoot frequently occurring errors.

- Describe the Safety aspects to be inbuilt in the control panel system as per the process requirement

- Define IEC standards

- Define Relevant regulations, standards and codes of practice and their implications on the

- panel

- What are the safety norms in handling electrical/electronic components?

- Describe the Operation of PLCs, relays, contactors, circuit breakers, solenoids, actuators, controllers etc.

- Describe the various tools used during the installation process

- How to troubleshoot Frequently occurring errors, causes and preventive measures

- repairing/replacement of various components in a PLC

- What are safety precautions while working in an electronic assembly unit

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | Psychometrics Chart, Tables, and Other References |
| 2 | Charts |
| 3 | Psychometrics Chart, Tables, and Other References |
| 4 | Clipboard |
| 5 | MultiMate |
| 6 | Data loggers |
| 7 | Electronic Hygrometer |
| 8 | Tachometer |
| 9 | Anemometer |
| 10 | Site Visit |
| 11 | Budget Sheet |
| 12 | International Safety Standards |
| 13 | Protective gear such as helmets, goggles |
| 14 | Electronic Pressure Gauge |
| 15 | Power Supply |
| 16 | Simulating software(Emulator) |
| 17 | Connecting Cables |
| 18 | PLC Software |
| 19 | Protective gear such as helmets, goggles, gloves, rubber shoes, etc |
| 20 | Psychometric Charts and Tables |
| 21 | Calculator |
| 22 | Calculation Sheet |
| 23 | Clipboard |
| 24 | Reference Material |
| 25 | Sensors/Transducers |
| 26 | Anemometer |
| 27 | Tachometer |
| 28 | Electronic Hygrometer |
| 29 | Protective gear such as helmets, goggles, gloves |
| 30 | rubber shoes, etc. |
| 31 | Flashlight |
| 32 | Tool box |
| 33 | Calculator |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

Design and program PLC

### 0713E&E28 Install PLC Software and Simulator

Overview:

This competency standard covers the skills and knowledge required toinstall PLC software and Simulator, Program a PLC using simulator and analyse user requirements and specifications.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Install PLC software and Simulator | - Install the PLC Programming - Software as per PLC manufacturer such as Rs Logic 5000 and Rs links classic etc. - Select the best and most frequently used Simulator for programming - Select the module - Input the generic value as per universal settings (check from internet/helped) - Start The Simulator. - Open PLC programming software |
| CU2. Program a PLC using simulator | P1. Open the programming software as per PLC manufacturer P2. Ensure that the Simulator is connected and is in ON condition P3. Create a basic ladder logic program for ON/OFF of a bulb using Examine ON and Examine OFF switch P4. Create a basic ladder logic program for Simple Start/Stop Ladder Logic Relay P5. Create a basic ladder logic program for Single Push Button On/Off Ladder Logic P6. Create a basic ladder logic program for with On Delay Timer P7. Create a basic ladder logic program for with Off Delay Timer P8. Create a basic ladder logic program for Traffic signal lights P9.Create a basic ladder logic program for Elevator Control. |
| CU3. Analyse user requirements and specifications | - Draw the general value chain of the end user industry - Highlight the set of activities that a firm operating in a specific industry performs in the value chain drawing - Enlist the equipment/gauges/sensors/actuators/transducers used in different stages of the process - Identify critical stages in the process - Identify the safety aspect required in the critical stages of the process - Analyse the possible automation in the existing processes and global trends in automation - Analyse the client requirement at broad level from the proposal - Generate a report of various industrial processes involved in industry - Collect the required specification of the equipment (if already prepared by the user) and clarify the technical specification. |

Knowledge & Understanding

- Describe the advantages of the PLC

- Describe the major components and

- Describe the applications of PLC.

- Describe the types of input and output signals.

- Define Piping and instrumentation diagram/drawing (P&ID)

- List down the PLC

- DCS programming software

- List Down the SCADA, HMI development software

- Knowledge of Relevant documents and documentation procedures

- Define Ladder Logic

- Explain the Examine ON and Examine OFF switch

- How to use ladder logic to create basic programs?

- What is push button?

- Define the types of Timers.

- What is the working of traffic light?

- Describe the working of an elevator.

- Define the term ‘value chain’ of industry.

- Define human machine interface (HMI)

- Define the Standard operating procedure (SOP) of the organization for control panel development process.

- How to prepare a General arrangement drawing?

- Define IEC standards

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | PLC Software |
| 2 | Protective gear such as helmets, goggles, gloves, rubber shoes, etc |
| 3 | Psychometric Charts and Tables |
| 4 | Calculator |
| 5 | Calculation Sheet |
| 6 | Clipboard |
| 7 | Site Visit |
| 8 | Pen |
| 9 | Reference Material |
| 10 | Helping Manual |
| 11 | User guide |
| 12 | Simulator |
| 13 | Page |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

Program a PLC using simulator

### 0713E&E29 Design and Test the PLC.

Overview:

This competency standard covers the skills and knowledge required toPrepare work plan, design and program PLC and Test the PLC.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Prepare work plan | - Suggest globally practiced and accepted automation systems if the user is not aware of the technical specifications - List down the sub systems that are involved in the process - List down sensors and actuators requirement. - Collect information on process logic - Collect information for operator station screens - Decide on whether the system can be developed as per the user requirement - Support the project manager in calculating the time required for each stage to ensure completion of project - Assist in preparing the work plan with deliverables and timelines - Explain the expected output to the user - Calculate the number of days needed for commissioning of the panel at site - Summarize the user requirement. |
| CU2. Design and program PLC | - Develop PLC application as per user requirement by following the standard operating procedure (SOP) of the organization - Apply approved engineering concepts, processes and principles in developing the PLC application - Install organization approved software (system and application software) to develop the system - Identify the requirement of indications, switchgears and accessories - Develop the control circuit drawing - Prepare wiring plans - Integrate the main process system with the sub-systems as per the user requirement (e.g., using communication protocol) - Ensure that safety aspect of the process is captured in the design plan - P9. Program PLC as per FDF Program SCADA Application - PLC-SCADA Communication - P12. Create backup copies of all designs developed for control panel and store in a secure location - Prepare a product manual and store them for future references. |
| CU3. Test the PLC | - Locate field devices and their interface to PLC - Test the system in off line mode using simulator - Test the gauges independently for integration of main system with the sub-systems (if applicable) - Verify that the system conforms with all the user specifications during testing - Rework if there are any issues found and fix them - Send the test report for review to the customer - Perform Factory Acceptance Test (FAT) - Perform site acceptance test plan |

Knowledge and understanding

- Knowledge of Relevant documents and documentation procedures used in the process

- Define formula for Electrical load calculations

- Explain the Piping and instrumentation diagram/drawing (P&ID)

- Describe the Safety aspects to be inbuilt in the control system as per the process requirement

- Describe the advantages of the PLC

- Describe the major components and

- Describe the applications of PLC.

- Describe the types of input and output signals.

- Define Piping and instrumentation diagram/drawing (P&ID)

- How to prepare wiring diagram

- List down the PLC, DCS programming software

- P8. List Down the SCADA, HMI development software

- P9. Knowledge of Relevant documents and documentation procedures used in the process

- What are the Testing process and parameters involved in the testing

- Define the use of Emulator(Simulator)

- What are the Relevant regulations, standards and codes of practice and their implications on the panel

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | PLC Software |
| 2 | Protective gear such as helmets, goggles, gloves, rubber shoes, etc. |
| 3 | Psychometric Charts and Tables |
| 4 | Calculator |
| 5 | Calculation Sheet |
| 6 | Clipboard |
| 7 | Site Visit |
| 8 | Pen |
| 9 | Reference Material |
| 10 | Helping Manual |
| 11 | User guide |
| 12 | Simulator |
| 13 | Page |
| 14 | Reference Material |
| 15 | Psychometric Charts and Tables |
| 16 | Calculator |
| 17 | Clipboard |
| 18 | Protective gear such as helmets, goggles, gloves, rubber shoes etc. |
| 19 | Calculation Sheet |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

Test the PLC

### 0713E&E30 Installation, Commissioning and Maintenance of PLC

Overview:

This competency standard covers the skills and knowledge required toensure quality and productivity standards, installation and Commissioning and carry out maintenance of PLC.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Ensure quality and productivity standards | - Ensure timely delivery of the control panel design as per agreed timeline - Ensure that total cost and man hours spent is as per the budget planned - Ensure compliance with relevant regulations, standards and codes of practices - Ensure compliance of the application with manufacturing requirements and process capabilities analysis of the organization - Ensure that the design conforms with normal safety standards - Develop reliable panels so that the system does not fail during the usage |
| CU2. Installation and Commissioning | - Check availability of panel and tools required for installation - Check the internal panel wiring and ensure that it is in accordance with the design drawing - Carry out insulation check of internal panel wiring and devices within the panel - Check if batteries and chargers have been assembled in accordance with the manufacturers recommended procedures - Identify the conductors size and capacity for installation - Ensure that the panel is positioned as prescribed, following safety norms - Make connections to socket outlets, switches and protective conductors - Perform settings as per customer requirements on the equipment in each of the panels - Test all control system interlocks - Check each digital control point by comparing the command at the control panel and status of the device that it controls - Ensure that fuses, switches and other protective devices are labelled correctly - Follow the grounding and earthling procedures while commissioning - Put danger and warning notices, (if necessary) - Test continuity, insulation resistance, functions of all devices, etc., after completion of installation |
| CU3. Carry out maintenance of PLC | - Select and use required tools and equipment as per the job requirement - Identify the faults as per the LED indications on module - Identify different signal modules (DI, DO, AI, AO) and take necessary action as per job requirement - Identify & Check the terminals inside IO panels and take necessary action as per the job requirement - Check & test the condition of input power supply to IO panel, input/output power of SMPS and take necessary action as per job requirement - Check the conditions of breakers and fuses and take necessary action as per the job requirement |

Knowledge and understanding

- Describe the Safety aspects to be inbuilt in the control panel system as per the process requirement

- Define IEC standards

- Define Relevant regulations, standards and codes of practice and their implications on the panel

- What are the safety norms in handling electrical/electronic components?

- Describe the Operation of PLCs, relays, contactors, circuit breakers, solenoids, actuators, controllers etc.

- Describe the various tools used during the installation process

- How to troubleshoot Frequently occurring errors, causes and preventive measures

- Describe the procedure followed for repairing/replacement of various components in a PLC

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | Charts |
| 2 | Psychometrics Chart, Tables, and Other References |
| 3 | Anemometer |
| 4 | Tachometer |
| 5 | Data logger |
| 6 | Electronic Hygrometer |
| 7 | MultiMate |
| 8 | Electronic Pressure Gauge |
| 9 | Protective gear such as helmets, goggles, gloves, rubber shoes, etc. |
| 10 | International Safety Standards |
| 11 | Calculator |
| 12 | Clipboard |
| 13 | Flashlight |
| 14 | Tool box |
| 15 | Budget She |
| 16 | Psychometrics Chart, Tables, and Other References |
| 17 | Protective gear such as helmets, goggles, gloves, rubber shoes, etc. |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

Carry out maintenance of PLC

### 0713E&E31 Identify the Parts of Analog & Digital Telephone Set & Verify their Function

Overview:

This competency standard covers the skills and knowledge required toDismantle and identify the parts of automatic telephone set, Dismantle and identify the parts of digital telephone set, Demonstrate the operation of Telephone buzzer, Perform the operation of Transmitter & receiver, Identify the parts of rotary dial& verify its function and Identify the parts of digital dial& verify its function.

| Competency Units | Performance Criteria |
| --- | --- |
| CU.1 Dismantle and identify the parts of analog telephone set | - Select the automatic telephone set and establish a method for disassembling activity as per SOP. - Use standard tools described in user manual - Apply disassembling techniques - Organize the tag-identification of the parts/components/wires of telephone set. - Identify the parts (magneto bell, dial, antiskid tone circuit, transmitter, receiver, cradle switch etc.) - Check for the proper operation/ functionality - Apply assembling techniques. |
| CU2. Dismantle and identify the parts of digital telephone set | - Select the digital telephone set and establish a method for disassembling activity as per SOP. - Use standard tools described in user manual - Apply disassembling techniques - Organize the tag-identification of the parts/components/wires of telephone set. - Identify the parts (telephone buzzer, dial, Dial IC, speech IC transmitter, receiver, bridge rectifier, cradle switch etc.) - Check for the proper operation/ functionality - Apply assembling techniques. |
| CU3. Demonstrate the operation of Telephone buzzer | - Connect the telephone set to telephone line. - Dial the telephone No. which is connected to line, form any mobile/telephone. - Hear the ringing tone in mobile/telephone and note what happen in telephone set connected to line. |
| CU4. Perform the operation of Transmitter & receiver | - Connect the telephone set to telephone line. - Dial the telephone No. which is connected to line, form any mobile/telephone. - On hearing ringing bell, pic up hand set and speak on the lower portion of hand set (transmitter). - On replying form other side note the effect from upper portion of hand set (receiver). |
| CU5. Identify the parts of rotary dial& verify its function. | - Select automatic telephone set. - Identify the dial plate of dialer. - Identify the dial hole on dial plate. - Identify the finger stop. - Dial any number from rotary dial - Note the sound of pulsed on receiver and count them and compare them to dial number. |
| CU6. Identify the parts of digital dial& verify its function. | - Select digital telephone set. - Identify Key pad dialer on the telephone set. - Press different numbers from key pad dialer. - Note and compare the difference of sound effect in receiver against each dial number. |

Knowledge & Understanding

- Define telephone set.

- Define telephone transmitter and receiver.

- Define antiside tone circuit.

- Define function of telephone buzzer.

- Describe the function of balancing circuit.

- What is the function of cradle switch?

- Define telephone set.

- Define telephone transmitter and receiver.

- Define antiside tone circuit.

- Define function of telephone buzzer.

- Describe the function of balancing circuit.

- What is the function of cradle switch?

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | Screw drivers |
| 2 | tweezers |
| 3 | Wire Cutter |
| 4 | Test probes |
| 5 | Nose plier |
| 6 | Electrical test bench |
| 7 | Electrical test bench |
| 8 | Nose plier |
| 9 | Power source (AC/DC |
|  | Multimeter |
|  | Tweezers |
|  | Wire Cutter |
|  | Test probes |
|  | Power source (AC/DC) |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

Identify the parts of digital dial & verify its function.

### 0713E&E32 Demonstrate Modulation, Demodulation, Multiplexing & De Multiplexing of Information Signal

Overview:

This competency standard covers the skills and knowledge required toPerform Amplitude Modulation of Information Signal, Perform Demodulation of Modulated AM Signal, calculate band width and modulation index, Perform Frequency Modulation of Information Signal, Perform Demodulation of Modulated FM Signal, Perform Pulse Code Modulation (PCM) of an analog signal, Perform Time Division Multiplexing (TDM) and De-multiplexing and Perform Frequency Division Multiplexing and De-multiplexing

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Perform Amplitude Modulation of Information Signal | - Calibrate CRO. - Generate Low Frequency Message signal and observe on channel 1 of CRO. - Apply the Message and Carrier signals to the AM Modulator Section of the AM Trainer. - Connect the output of the modulator to channel 2 of CRO and observe the signal. |
| CU2. Perform Demodulation of Modulated AM Signal | - Calibrate CRO. - Generate AM Modulated signal and observe on channel 1 of CRO. - Apply the Modulated and Carrier signals to the AM De-Modulator Section of the AM De-Modulation Trainer. - Connect the output of the de-modulator to channel 2 of CRO and observe the signal. - Compare the Message signal to the Demodulated signal. |
| CU3. Calculate band width and modulation index | - Calibrate CRO. - Measure the frequency and voltage of Low frequency and carrier frequency signal with oscilloscope. - Calculate modulation by formula - Calculate bandwidth by formula Where as |
| CU4. Perform Frequency Modulation of Information Signal | - Calibrate CRO. - Generate Low Frequency Message signal and observe on channel 1 of CRO. - Apply the Message and Carrier signals to the FM Modulator Section of the FM Trainer. - Connect the output of the modulator to channel 2 of CRO and observe the signal. |
| CU5. Perform Demodulation of Modulated FM Signal | - Calibrate CRO. - Generate FM Modulated signal and observe on channel 1 of CRO. - Apply the Modulated and Carrier signals to the FM De-Modulator Section of the FM De-Modulation Trainer. - Connect the output of the de-modulator to channel 2 of CRO and observe the signal. - Compare the Message signal to the Demodulated signal. |
| CU6. Perform Pulse Code Modulation (PCM) of an analog signal | - Generate Sine wave signal and observe on channel 1 of CRO. - Apply the generated signal and Clock signal to the PCM Modulator using jumpers. - Observe the output on channel 2 of CRO and also note the pattern of blinking of output LEDs. |
| CU7. Perform Time Division Multiplexing (TDM) and De-multiplexing | - Generate multiple low frequency signals. - Apply the signals to TDM Multiplexer. - Observe the multiplexed output on channel 1 of CRO. - Apply to multiplexed output to the De-multiplexer. - Observe output for first message signal on channel 1 of CRO and compare the output signal with the relative Message signal by observing it on channel 2 of CRO. - Repeat the step 5 for all the DE multiplexed signals one by one. |
| CU8. Perform Frequency Division Multiplexing and De-multiplexing | - Generate multiple message signals. - Generate multiple carrier signals of different frequencies. - Apply each of the message and carrier signal pairs to their respective Balanced Modulators. - Combine the modulated signals using Adder and observe this FDM Multiplexed output on Channel 1 of CRO. - Apply the multiplexed signal to respective Band-Pass filters for each DE multiplexer section (or user). - Apply the output of each Filter to their respective Demodulators. - Observe the demodulated and hence FDM DE multiplexed signals for each user and compare them to their respective Transmitted signals by viewing both on CRO at the same time. |

Knowledge & Understanding

- Calibration of CRO.

- Working Principle of AM Modulator

- Working Principle of AM Demodulator.

- Define low frequency or information signal

- Define high frequency or carrier signal.

- Define modulation index

- Define bandwidth.

- Define USB “Upper side band”

- Define LSB “Lower side band

- Calibration of CRO.

- Working Principle of FM Modulator

- Working Principle of FM Demodulator.

- Sine wave and its different parameters.

- Calibrating and using CRO.

- Working principle of PCM

- Working principle of TDM.

- Calibrating and Using CRO

- Calibrating and Using CRO.

- Working principle of CRO.

- AM DSB and SSB

- AM demodulation

- Filtering

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | Oscilloscope with probes |
| 2 | Oscilloscope with probes |
| 3 | Frequency Modulation Trainer |
| 4 | FM Demodulation Trainers |
| 5 | Jumpers |
| 6 | FM Trainer |
| 7 | AM Trainer |
| 8 | Signal generator |
| 9 | Oscilloscope with probes |
| 10 | AM Demodulation Trainers |
| 11 | Jumpers |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

Calculate band width and modulation index

### 0713E&E33 Install Cord Less Telephone, PABX & Satellite Dish System.

Overview:

This competency standard covers the skills and knowledge required toInstall cord less telephone system, Install and Configure of Private Automatic Branch Exchange (PABX) and Install Home satellite dish system.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Install cord less telephone system. | - Chose proper location to install the base station - Connect the phone line to the base the base station of cord less telephone. - Connect the supply to the base station. - Pic-up the Mobile unit from cordless telephone and make a call to ensure the working functionality. |
| CU2. Install and Configure of Private Automatic Branch Exchange(PABX | - Lay 2-Pair Telephone drop wire around the premises. - Plug the telephone cord into your PBX console in an input that says, "Telephone Line." - Connect the other end into a wall jack. This allows your PBX system to send and receive phone calls. - Insert the PBX plug into the console. Plug the other end into the wall outlet. Wait for your PBX system to light up. This lets you know that you have properly installed the system. - Pick up the receiver to place a call. Dial the extension or phone number you would like to reach. Wait for the caller to answer on the other end. - Wait for the system to ring and light up, which means you have an incoming call. Pick up the receiver and answer the call. You can also press "Intercom" or "Hands Free" to answer the call |
| CU3. Install Home satellite dish system. | - Perform dish assembly as per SOP - Perform dish installation on roof top as per SOP. - Route coaxial cable and connect dish receiver to dish antenna. - Connect dish receiver output to TV receiver input. - Turn on the supply of both receivers, enter the frequency of required channel. - Adjust the antenna with the help of satellite locator chart till required channel is observed on TV screen. |

Knowledge & Understanding

- Function and operating procedure of cordless telephone.

- Installing RJ-11 Connectors on cable.

- Function and operating procedure

- of PABX

- Function & operating procedure of satellite

- dish system

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | Cordless telephone |
| 2 | Telephone wire |
| 3 | Supply |
| 4 | Installation kit. |
| 5 | Telephone Sets |
| 6 | PBX |
| 7 | Coaxial cable. |
| 8 | Dish antenna |
| 9 | Dish receiver |
| 10 | RJ-11 Connectors |
| 11 | Knife Installation kit |
| 12 | 2-Pair Cable |
| 13 | Crimping Tool |
| 14 | TV receiver |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

Install and Configure of Private Automatic Branch Exchange(PABX)

### 0713E&E34 Install Power Supply

Overview:

This competency standard covers the skills and knowledge required to Construct full wave rectifier/Bridge Rectifier, Construct filtered power supply, Construct Zener diode regulated power supply, Installation and Commissioning of Telecom Rectifier /UPS system, Installation of Valve Regulated Lead Acid Battery (VRLA) / Sealed Lead Acid Batteries (SLA) for Telecom Equipment, conduct different Tests of VRLA/SLA Battery and Perform Maintenance of Valve regulated lead Acid Battery

| Competency Units | Performance Criteria |
| --- | --- |
| CU.1 Construct full wave rectifier/Bridge Rectifier. | - Identify the Diodes and center tape Transformers, Resistors. - Draw the Circuit Diagram of Full wave rectifiers - Connect the components according to the diagram - Apply AC Input. - Check output with oscilloscope - Check Output with Multi Meter. - Draw out put wave form, note and observe the results |
| CU.2 Construct filtered power supply. | - Check the Diodes and Transformers, Resistors, capacitors. - Draw the Circuit Diagram of filtered power Supplies. - Connect the components according to the diagram. - Apply AC Input. - Check output with oscilloscope. - Check Output with Multi Meter. - Draw out put wave form, note and observe the results. |
| CU.3 Construct Zener diode regulated power supply | - Check and identify Zener Diodes, Diodes and Transformers, Resistors, capacitors. - Draw the Circuit Diagram of Zener Diode regulated power Supply. - Connect the components according to the diagram. - Apply AC Input. - Check output with oscilloscope - Check Output with Multi Meter. - Measure and Note the results |
| CU.4 Installation and Commissioning of Telecom Rectifier /UPS system. | - Installation of Rectifier/ UPS Cabinet as per Design. - Connect AC supply with Rectifiers from main distribution board (MDB). - Connect surge protection device (SPD) with rectifiers. - Connect with DC as per requirement. - Perform rectifier system commissioning. - Perform boost charging setting of batteries. - Perform AC over voltage setting. - Perform AC under voltage setting. - Perform DC under voltage setting. |
| CU.5 Installation of Valve Regulated Lead Acid Battery (VRLA) / Sealed Lead Acid Batteries (SLA) for Telecom Equipment | - Develop/Draw system schematic diagram which identifies how the individual batteries are to be placed on the rack, interconnected and numbered. - Install 3 tier rack for batteries. - Locating/Fixed the Batteries on the Racks - Prepare Battery Terminals. - Prepare and install the Inter tier, inter row and Inter Rack Connections. - Connect the Load/Charger with Batteries. |
| CU.6 Conduct different Tests of VRLA/SLA Battery. | - Perform High rate momentary load test. - Measurements of resistance/ conductance/impedance of battery. - Conduct Performance test of battery. |
| CU.7 Perform Maintenance Of Valve regulated lead Acid Battery | - Measure and record overall float voltage measured at the battery terminals. - Measure and record charge output current and voltage. - Measure and record ambient temperature of battery room or cabinet - Check condition of ventilation and monitoring equipment. - Visual inspection of cell/unit integrity for evidence of corrosion at terminals, connections, racks, or cabinets and area around batteries accessibilities - Check for excessive jar/cover distortion Measure and record DC float current (per string) using equipment - Measure and record cell/unit internal ohmic values. - Measure and record temperature of the negative terminal of each cell/unit of battery. - Measure and record voltage of each cell/unit Measure and record cell to cell and terminal connection detail resistance of entire battery. - Measure and record AC ripple current and/or voltage imposed on the battery |

Knowledge & Understanding

- Knowledge about Centre tape transformers

- Knowledge about working center tape transformer. Find input and output windings.

- Knowledge about Working of Full wave rectifier.

- Knowledge about Input and

- Output wave forms of full wave rectifier.

- Knowledge about Diodes, filters and transformer.

- Knowledge about working of diodes, filters and transformers.

- Knowledge about Use of resistor and capacitor as filter.

- Knowledge about Filter power supply working and applications.

- Knowledge about Ripple. Ripple factor.

- Knowledge about Input and output voltage levels. Wave forms.

- Knowledge about the difference between filter and simple power supply.

- Knowledge about the Working principle and use of Zener Diodes.

- Knowledge about Zener Diode Regulated Power Supplies.

- Regulation Methods

- Knowledge about Zener Regulated power supply working and applications.

- Knowledge about Input and output voltage levels.

- Knowledge about the Line regulation and load regulation

- Knowledge about Rectifier working.

- Knowledge about rectifier input and output power connections.

- Knowledge about SPD connection with rectifier.

- Knowledge about commissioning of rectifier system.

- Knowledge about boost charging, under and over AC voltage setting.

- Knowledge about Valve regulated batteries.

- Knowledge about installation of VRLA batteries

- Knowledge about battery connection

- Knowledge about Inter Row, Inter Rack, Inter battery String Connections.

- Knowledge about battery rack installation.

- Knowledge about the battery charger/ load.

- Knowledge about battery Performance test

- Knowledge about battery without load and on load test.

- Knowledge about Battery resistance/conductance/impedance.

- Knowledge about battery maintenance.

- Knowledge about ambient temperature, ventilation, float voltage, terminal corrosion, negative terminal temperature, battery resistance, Ripple current and voltage

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | Box end wrench, insulated |
| 2 | Electrical tool kit. |
| 3 | Multi Meter |
| 4 | Impedance Meter |
| 5 | Rubber gloves. |
| 6 | Battery lifting equipment (handles) and fork lift to lift pallets of batteries. |
| 7 | Box end wrench, insulated. |
| 8 | Torque wrench calibrated in inch-pounds, insulated. |
| 9 | Socket wrench, insulated. |
| 10 | Diodes |
| 11 | Transformers |
| 12 | Power Supply |
| 13 | Trainer |
| 14 | Digital Oscilloscope |
| 15 | Multi meter |
| 16 | Connecting wire |
| 17 | Zener Diodes |
| 18 | Power Supply |
| 19 | Diodes |
| 20 | Resistors |
| 21 | Transformer |
| 22 | Complete Electrical Toolkit |
| 23 | Toolkit of spanners and wrenches |
| 24 | Digital voltmeter. |
| 25 | RCL Meter |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

Perform Maintenance Of Valve regulated lead Acid Battery

### 0713E&E35 Maintain Lead Acid Batteries and Implement Their Series Parallel Combination.

Overview:

This competency standard covers the skills and knowledge required to construct series combination of batteries, Construct Parallel combination of batteries, Prepare Electrolyte for battery, check specific gravity of Electrolyte, determine internal resistance of cell, and Charge the battery with the help Charger.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Construct series combination of batteries. | - Select the specific quantity cells. - Connect them in series with proper polarity. - Measure the voltage of series combined cell. - Increase the number of cells and note the voltage. - Interpret the effect. |
| CU2. Construct Parallel combination of batteries. | - Select the specific quantity cells. - Connect them in parallel with proper polarity. - Measure the voltage of parallel combined cell. - Increase the number of cells and note the voltage. - Interpret the effect. |
| CU3. Prepare Electrolyte for battery | - Select the proper container & and use proper safety equipment to prepare electrolyte - Pour H2SO4 in the container as per specification. - Put the water as per specification into the container and slowly - Mix the mixture with wooden/glass rod so that two solutions mix completely. |
| CU4. Check specific gravity of Electrolyte. | - Open the vent plug of battery - Fill the hydrometer with electrolyte - Keep the hydrometer vertical and take reading form hydrometer |
| CU5. Determine internal resistance of cell | - Adjust the apparatus as per diagram. - Turn the switch off and take the reading of voltmeter and note its value as “E”. - Turn on the switch and again take the reading of voltmeter and note its value as “V”. - Note the reading of ammeter as well. - Use above measured values and find out internal resistance using appropriate formula. |
| CU6. Charge the battery with the help of Charger. | - Set proper voltage and current setting of charger. - Connect the battery at the output terminal of charger. - Turn on the supply and note the current reading of ammeter from charger. - Remain connect the battery with charger till the ammeter show minimum or zero reading. - Disconnect the battery and check its specific gravity with hydrometer |

Knowledge & Understanding

- Define series combination of cell

- Define parallel combination of cell

- Define Electrolyte.

- Define specific gravity.

- What is hydrometer describe its use.

- What is the value of specific gravity of lead acid battery

- Define internal resistance of cell.

- What is the difference between “E” and “V” of cell?

- Enlist the parts of a charger.

- Describe the function of charger.

- Why ammeter shows minimum or zero reading at full charging.

- What is the specific gravity of battery at full charge condition

Tools and Equipment

.

| SN | Tools |
| --- | --- |
| 1 | Connecting leads |
| 2 | Battery Charger |
| 3 | Ammeter |
| 4 | Battery |
| 5 | Voltmeter |
| 6 | Switch |
| 7 | Resistance |
| 8 | Ammeter |
| 9 | Cells |
| 10 | Container |
| 11 | Battery |
| 12 | Hydrometer |
| 13 | Water |
| 14 | Wooden/glass rod |
| 15 | Change over switch |
| 16 | Volt meter. |
| 17 | H2SO4 |
| 18 | Connecting leads. |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

Prepare Electrolyte for battery

### 0713E&E36 Operate Hydro Power Plant

Overview

This competency standard covers the skills and knowledge required to Inspect Hydropower Plant equipment, operate plant equipment, respond to equipment emergencies and alarms, and perform maintenance activities and Generate Report. Operate and monitor plant generating equipment and take appropriate actions to ensure safe, efficient and reliable operation of the Hydropower plant.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Inspect Hydropower Plant equipment. | - Adopt health and safety measures, as per standards. - Identify Generator, Turbine and other Auxiliary equipment. - Identify Control Panel. - Identify measurement Gauges (Rpm meter, Pressure Gauges, Temperature Gauges and flow valves). - Monitor readings of gauges and meters regularly - Perform calibration check for scale accuracy. P7. Carryout Generator inspection including its bearing and lubrication. P8. Ensure that all sensors operate correctly. P9. Ensure that all controller functions correctly. P10.Carryout Turbine functional checks and bearing lubrication inspection. P11.Perform Gearbox bearing inspection and analyze oil condition. P12.Examine the sounds, smells and vibrations of equipment. P13.Report Problem (If any) and take appropriate preventive measures. |
| CU2. Operate plant equipment. | - Follow safety measures, as per standards. - Performs pre start Inspections. - Turn on Power switch of the equipment safely. - Conduct post startup inspection. - Turn OFF Power switch of the equipment safely. - Analyze its smooth and efficient operation - Generate a report of switching operations and loads on generators, electrical lines and transformers. |
| CU3. Respond to equipment emergencies and alarms. | - Identify emergency alarms and switches. - Response to alarm in emergency. - Turn OFF power in emergency. - Take timely corrective measures. - Turn ON power when fault is removed. - Use of fire extinguisher and emergency exit. |
| CU4. Perform maintenance activities. | - Follow safety measures, as per standards - Investigate malfunction in accordance with company policies and procedures - Identify nature of fault by carrying out various tests. - Carryout Electrical inspection and testing. - Perform Sensor replacement and calibration - Perform Controller optimization / reprogramming. - Carryout Internal (endoscopic) inspections of turbines and gearboxes - Carryout Hydraulic accumulator testing and recharging. P9. Test the equipment after it’s repaired and ready to be placed back. P11.Generate a document and mention the fault and the measures taken, for future use. |
| CU5. Generate Report | - Follow safety measures, as per standards. - Generate Hydro system performance and operational reviews / optimization - Prepare Visit Report as per technical specifications of the plants |

Knowledge and understanding

- Define Hydropower Plant.

- Selection of site

- Types of hydropower plant

- Describe the schematics diagrams and symbols used in a power plant.

- Describe parts

- Describe other types of Power Plant.

- Compare Hydropower plant with other power plants.

- Describe Control Gauges, Pressure Gauges, Temperature Gauges and Electric Meters and their various measuring units

- Turbine and Gearbox.

- Basic Knowledge of personal and industrial safety standards.

- Describe the steps of generating a report

- Define SCADA.

- Describe pre start Inspections.

- Describe Switching procedure of the equipment.

- Knowledge of high voltage switching and use of hot sticks substation components, and circuit breakers.

- Describe Purpose of safety alarms and Switches

- Define the procedure followed in emergency to shut down power and save life

- Describe the purpose of fire extinguisher and emergency exist and its use.

- Define maintenance procedure.

- Describe the basic tools used for repairing.

- Describe the maintenance areas.

- Describe the procedure of testing the equipment.

- Knowledge of the operation and service/maintenance manuals.

- Define a visit report.

- Describe the procedures involved in preparing a report.

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | Visit to Hydropower Plant |
| 2 | Tool Box |
| 4 | Emergency Switch |
| 5 | Fire Extinguisher |
| 6 | Safety helmet, gloves, goggles and shoes |
| 8 | Notepad |
| 10 | Visit Report |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

Identify various plant equipment and define their purpose

### 0713E&E37 Operate Thermal Power Plant

Overview:

After this competency standard, the trainee will be able to Carry out General Inspection of the Thermal power plant, Operate Steam Turbine, Inspect and Monitor Boiler Operation, Inspect and Operate Ash handling plant, Inspect and Operate Coal Handling plant, Respond to equipment emergencies and alarms, Perform maintenance activities and Generate Report  operate and monitor plant generating equipment and take appropriate actions to ensure safe, efficient and reliable operation of the Thermal Power Plant.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1.Carry out General Inspection of the Thermal power plant | - Adopt health and safety measures, as per standards. - Identify boiler, turbine, alternator or generator, condenser and feed pump and other auxiliary equipment. - Recognize different major areas in thermal power plant. Boiler, Turbine, Ash handling, and Coal Handling - Identify Control Panel. - Identify measurement Gauges (Pressure Gauges, Temperature Gauges, vacuum gauge and flow gauge). - Monitor readings of gauges and meters regularly - Check the scale accuracy of meters. - Examine the sounds, smells and vibrations of equipment. - Report Problem (If any) and take appropriate preventive measures. |
| CU2. Operate Steam Turbine | - Implement the Controls for identified hazards and potential hazards in work area consistent with appropriate standards - Confirm the Availability of quality steam from upstream provider - Perform Pre-operational safety checks of steam turbine - Perform Start-up checks upon ancillary plant - Start the Steam turbine and placed on line safely, according to procedures, including performance of start-up checks - Monitor Steam turbine according to required procedures, including performing of operational checks and fault finding - Follow energy isolation procedures - Routine shutdown of steam turbine is performed according to operational and manufacturer s requirements and procedures, including performing shutdown checks - Generate a report of switching operations and loads on generators, electrical lines and transformers. |
| CU3.Inspect and Monitor Boiler Operation | - Follow safety measures, as per standards. - Check production requirements at start of shift and plan to carry out daily work activities following workplace health and safety and standard operating procedures (SOP) - Confirm operational status by inspection and routine observation - Adjust boiler controls to maintain operation within specifications - Monitor and record temperature controller display - Monitor and maintain steam pressure and steam distribution systems to meet production requirements - Complete routine preventative maintenance in line with maintenance schedules |
| CU4. Inspect and Operate Ash handling plant. | - Follow safety measures, as per standards. - Monitor vacuum levels, vacuum leakage, checking of hopper (empty, partially full, full) and clearing of choked. - Start and stop pumps, line up discharge lines, change over pumps - Operate pumps for series and parallel operations - Operate ash evacuation vacuum pumps - Operate dry ash loading plant for dust collection. - Handle operations of bottom ash handling equipment such as scrapper - Ensure smooth working of conveyor, clinker grinder, their line-up and change over identifying normal defects etc. - Continuously monitor and inspect ash handling equipment before and during operations |
| CU5.Inspect and Operate Coal Handling plant | - Follow safety measures, as per standards. - Operate the safety equipment/instruments installed in the unloading plant - Manage coal handling plant right from arrival of wagon, unloading and transportation to appropriate locations. - Ensure proper electrical operations of equipment such as conveyors, crushers, dozers and loaders - Operate coal sampler - Ensure removal of unwanted particulates from coal |
| CU6. Respond to equipment emergencies and alarms. | - Identify emergency alarms and switches. - Response to alarm in emergency. - Turn OFF power in emergency. - Take timely corrective measures. - Turn ON power when fault is removed. - Use of fire extinguisher and emergency exit. |
| CU7. Perform maintenance activities. | - Follow safety measures, as per standards - Investigate malfunction in accordance with company policies and procedures - Identify nature of fault. - Identify correct tool for repairing. - Inform higher authority for major fault (on time) - Test the equipment after it’s repaired and ready to be placed back. - Generate a document and mention the fault and the measures taken, for future use. |
| CU8. Generate Report | - Follow safety measures, as per standards. - Prepare Visit Report as per technical specifications of the plants |

Knowledge and understanding

- Define Thermal Power Plant.

- Describe other types of Power Plant.

- Compare Thermal Power Plant with other generation plants.

- Describe Control Gauges, Pressure Gauges, Temperature Gauges and Electric Meters.

- Define SCADA.

- Describe the schematics diagrams and symbols used in a power plant.

- Selection of site

- Basic Knowledge of personal and industrial safety standards

- Define Steam Turbine.

- Describe pre start Inspections.

- Describe Switching procedure of the equipment.

- Knowledge of upstream and downstream.

- Knowledge of high voltage switching and use of hot sticks substation components, and circuit breakers.

- Describe the Boiler Operation.

- What are the major Inspections being carried out in a Boiler Operation?

- Describe the Occupational Health and Safety standards.

- Describe the types of maintenances.

- What parameters are required for the smooth operation of the plant?

- Describe the Ash Handling Plant.

- What are the major operations being carried out in an ash handling plant?

- What are the inspection methods in an ash handling plant?

- Describe the Coal Handling Plant.

- What are the major operations being carried out in a coal handling plant?

- What are the inspection methods in a coal handling plant?

- What parameters are required for the smooth operation of the plant?

- Describe Purpose of safety alarms and Switches

- Define the procedure followed in emergency to shut down power and save life

- Describe the purpose of fire extinguisher and emergency exist and its use

- Define maintenance procedure.

- Describe the basic tools used for repairing.

- Describe the maintenance areas.

- Define a visit report.

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | Visit to Thermal Power Plant |
| 2 | Fire Extinguisher |
| 3 | Emergency Switch |
| 4 | Visit to Ash handling plant |
| 5 | Safety helmet, gloves, goggles and shoes |
| 6 | Tool Box |
| 7 | Notepad |
| 8 | Report |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

Identify various plant equipment and define their purpose

### 0713E&E38 Operate Diesel Power Plant

Overview:

This competency standard covers the skills and knowledge required to inspect plant equipment, operate plant equipment, respond to equipment emergencies and alarms, and perform maintenance activities and Generate Report. Operate and monitor plant generating equipment and take appropriate actions to ensure safe, efficient and reliable operation of the Diesel power plant.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Inspect plant equipment. | - Adopt health and safety measures, as per standards. - Identify Generator, Boiler, and Turbine and other Auxiliary equipment. - Identify Control Panel. - Identify measurement Gauges (Pressure Gauges, Temperature Gauges and flow valves). - Ensure that the fuel oil system is activated - Monitor and maintain the pre heating and pressure as per requirement. - Ensure that the lube oil system is activated and is operational. - Make sure that the pre lube pump has been in operation for at least 15 min’s. - Activate the engine air system is activated. - Ensure that the charge air system and exhaust gas system are activated - Monitor and maintain the temperature of the cooling system. - Examine the sounds, smells and vibrations of equipment. - Report Problem (If any) and take appropriate preventive measures. |
| CU2. Operate plant equipment. | - Follow safety measures, as per standards. - Ensure all the responsibilities related to start and stop of the plant. - Communicate with relevant authority for regulating power evacuation to the grid. - Co-ordinate with local 220kv /110 kv substations for load monitoring. - Monitor plant parameters and take corrective action. - Ensure data logging and operational schedules. - Assist the HOD in maintaining the all relevant documents - Analyze its smooth and efficient operation - Generate a report of switching operations and loads on generators, electrical lines and transformers. - Start and stop the equipment as per the instruction of control room. - Monitor the parameters periodically and observe gauges and leakages. |
| CU3. Respond to equipment emergencies and alarms. | - Identify emergency alarms and switches. - Response to alarm in emergency. - Turn OFF power in emergency. - Take timely corrective measures. - Turn ON power when fault is removed. - Use of fire extinguisher and emergency exit. |
| CU4. Perform maintenance activities. | - Follow safety measures, as per standards - Carryout scheduled operational/ maintenance activities as per the instruction - Investigate malfunction in accordance with company policies and procedures - Identify nature of fault. - Identify correct tool for repairing. - Inform higher authority for major fault (on time) - Test the equipment after it’s repaired and ready to be placed back. - Generate a document and mention the fault and the measures taken, for future use. |
| CU5. Generate Report | - Follow safety measures, as per standards. - Prepare Visit Report as per technical specifications of the plants |

Knowledge and understanding

- Define Diesel Power Plant.

- Describe other types of Power Plant.

- Compare Diesel power plants with other generation plants.

- Describe Control Gauges, Pressure Gauges, Temperature Gauges and Electric Meters.

- Describe the schematics diagrams and symbols used in a power plant.

- .Basic Knowledge of personal and industrial safety standards

- Describe pre start Inspections.

- Describe Switching procedure of the equipment.

- Knowledge of schematics and diagrams and symbols.

- Knowledge of high voltage switching and use of hot sticks substation components, and circuit breakers.

- Basic Knowledge of personal and industrial safety standards.

- Describe Purpose of safety alarms and Switches

- Define the procedure followed in emergency to shut down power and save life

- Describe the purpose of fire extinguisher and emergency exist and its use.

- Define maintenance procedure.

- Describe the basic tools used for repairing.

- Describe the maintenance areas.

- Describe the procedure of testing the equipment.

- Define a visit report

.

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | Visit to Thermal Power Plant |
| 2 | Fire Extinguisher |
| 3 | Emergency Switch |
| 4 | Visit to Ash handling plant |
| 5 | Safety helmet, gloves, goggles and shoes |
| 6 | Tool Box |
| 7 | Visit to Power Plant. |
| 8 | Notepad |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

Identify various plant equipment and define their purpose

### 0713E&E39 Operate Nuclear Power Plant

Overview:

After this competency standard, the trainee will be able to operate and monitor plant generating equipment and take appropriate actions to ensure safe, efficient and reliable operation of the Nuclear Power Plant.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Carry out General Inspection of the equipment in Nuclear Power Plant | - Adopt health and safety measures, as per standards. - Identify Nuclear reactor, Steam turbine, Generator, Cooling system, Safety valves, Main condenser, Feed water pump, Emergency power supply - Recognize different major areas in nuclear power plant(Reactor vessels, Turbine/Generator, fuel waste handling, Cooling tower) - Identify Control Panel. - Identify measurement Gauges (Pressure Gauges, Temperature Gauges, vacuum gauge and flow gauge). - Monitor readings of gauges and meters regularly - Examine the sounds, smells and vibrations of equipment. - Report Problem (If any) and take appropriate preventive measures. |
| CU2. Operate Nuclear Reactor | - Ensure the continuous supply of the fuel in Control reactor - Confirm the Availability of quality steam from upstream provider - Perform Pre-operational safety checks of steam turbine - Perform Start-up checks upon ancillary plant - Start the Steam turbine and placed on line safely, according to procedures, including performance of start-up checks - Monitor the equipment according to required procedures, including performing of operational checks and fault finding P9.Generate a report of switching operations and loads on generators, electrical lines and transformers. |
| CU3.Inspect and Monitor Water Cooling System in Nuclear Power Plant | - Follow safety measures, as per standards. - Confirm operational status by inspection and routine observation - Monitor and record temperature/pressure controller display - Monitor and maintain steam pressure and steam distribution systems to meet production requirements - Ensure proper cooling operation. - Complete routine preventative maintenance in line with maintenance schedules |
| CU4. Inspect Emergency power supply | - Follow safety measures, as per standards. - Inspect the Emergency transformers and generators - Ensure at least two distinct sources of offsite power for redundancy - Generate a report of backup emergency supply. |
| CU5. Generate Report | - Follow safety measures, as per standards. - Prepare Visit Report as per technical specifications of the plants |

Knowledge and understanding

- Define Nuclear Power Plant.

- Selection of site

- Schematic diagram

- Reactor and other main parts

- Discuss the fuels used in Nuclear power plant and its half-life.

- Describe chain reactions.

- Describe other types of Power Plant.

- Compare Nuclear Power Plant with other generation plants.

- Waste handling, cooling tower.

- Describe pre start Inspections.

- Describe Switching procedure of the equipment.

- Knowledge of upstream and downstream.

- Knowledge of high voltage switching and use of hot sticks substation components, and circuit breakers.

- personal and industrial safety standards

- Describe the Water Cooling System.

- What are the major Inspections being carried out in a Water Cooling System?

- Describe the Occupational Health and Safety standards.

- What parameters are required for the smooth operation for the cooling system?

- Describe the  Emergency power

- supply

- What are the inspection methods in Emergency power supply

- What parameters are required for the smooth operation of the plant

- Define a visit report.

- Describe the procedures involved in preparing a report

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | Visit to Nuclear Power Plant |
| 2 | Fire Extinguisher |
| 3 | Emergency Switch |
| 4 | Visit to Ash handling plant |
| 5 | Safety helmet, gloves, goggles and shoes |
| 6 | Tool Box |
| 7 | Notepad |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

Identify various plant equipment and define their purpose

### 0713E&E40 Design Solar System.

Overview:

This competency standard covers the skills and knowledge required toEnsure Personal safety, calculate load for solar PV system design, Plan for installation of solar panel, Install the solar panel, Install the solar panel, perform testing and Diagnose and remove the fault. Carry out electrical operations, including installation and commissioning of off-grid solar PV systems. In addition, they will be able to: Plan and prepare for installation; mount a photovoltaic array on a roof; install and commission solar photovoltaic array; carry out maintenance on off-grid solar PV system; and complete work.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1.Ensure Personal safety | - Arrange PPEs as per requirements - Wear proper PPE as per nature of job - Store PPEs at appropriate place after use - Ensure availability of first aid box |
| CU2.Calculate load for solar PV system design | - Enlist the No. of appliances to be operated - Calculate the PV system load - Record the PV system load. |
| CU3.Plan for installation of solar panel | - Identify and obtain safety and other regulatory requirements for installation - Choose suitable location for the PV array and other components - Obtain tools, equipment and testing devices - Specify installation requirements for all system components to ensure correct operation, long life, safety and ease of Maintenance |
| CU4. Install the solar panel frame | - Interpret and confirm installation in terms of roof construction and suitable mounting methods - Choose appropriate array frame for the roof type and determine and set correct tilt angle adjustments - Ensure the frame is properly fitted, eliminating vibration in the frame - Place flashing and other waterproofing measures. |
| CU5.Install the solar panel | - Install system components in suitable location - Connect PV panels and electrical components and minimize cable route length to minimize power loss - Perform Earthling - Make connections of wires - Confirm installation |
| CU6.Perform testing | - Test and adjust component and/or parts - Confirm operation of PV system - Check the voltage and current at receiver end. |
| CU7.Diagnose and remove the fault | - Check invertor for fault code - Verify safety circuits - Check status of wiring - Verify status of battery - Check status of PV Panels - Identify nature of the fault (if any) - Remove the fault |

Knowledge and understanding

- How to wear Personal Protective Equipment (PPE

- How to measure voltage and current by digital multi-meter?

- Basic knowledge of load calculation

- Basic knowledge of AC/DC operations

- Safety requirements; Specifications; Hazard identification

- Factors influencing the efficiency of solar panels; Physical structure

- Summer and winter requirements

- Aesthetical aspects

- Roof constructions

- Azimuth inclination

- Relation of sun rays to a tilted surface

- Fixing methods and waterproofing measure

- Installation requirements

- Series and parallel circuit setup; Cable sizing; Forward and reverse diodes

- Requirements for properly bonded earthling

- Supervisor and/or client communication

- How to connect proper wire by connector.

- Functional tests and adjustments of instrument.

- Basic knowledge and

- calculation of open circuit voltage, on load voltage, short circuit current, maximum current load; Basic function of relay change over

- Basic operation of PV system and settings to adjust performance

- How to diagnose the fault?

- How to check wiring status?

- Method of verifying the status of battery

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | First Aid box |
| 2 | Bench vice |
| 3 | Hand drill machine |
| 4 | Battery charger |
| 5 | Ceiling |
| 6 | Batteries |
| 7 | AVO meter analog |
| 8 | AVO meter digital |
| 9 | Amp meter |
| 10 | Multi-meter |
| 11 | Personal protective equipment |
| 12 | First aid box |
| 14 | Calculator |
| 15 | Blank paper |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

Install Solar Panel

### 0713E&E41 Operate Solar Power Plant

Overview:

This competency standard covers the skills and knowledge required toCarry out General Inspection of the power plant, Operate Solar plant equipment, respond to equipment emergencies and alarms, perform maintenance activities and Generate Report. Operate and monitor plant generating equipment and take appropriate actions to ensure safe, efficient and reliable operation of the Solar Power Plant.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Carry out General Inspection of the power plant | - Adopt health and safety measures, as per standards. - Identify PV Cells, Inverter Batteries, Circuits and other Auxiliary equipment. - Ensure the PV cells are clean and placed at a maximum absorption position. - Identify Control and Switching Panel. - Identify voltage meter, load indicator, battery gravity indicator, charging/discharging meter. - Monitor readings of indicator gauges and meters regularly - Perform calibration check for scale accuracy. - Examine the sounds, smells and vibrations of equipment. - Report Problem (If any) and take appropriate preventive measures. |
| CU2. Operate Solar plant equipment. | - Follow safety measures, as per standards. - Perform switching as per requirement - Carry out shifting of the load according to the load management requirement at the time of peak load and base load. - Switch to alternative source during unfavorable weather conditions. - Analyze its smooth and efficient operation - Generate a report of switching operations and loads on generators, electrical lines and transformers. |
| CU3. Respond to equipment emergencies and alarms. | - Identify emergency alarms and switches. - Response to alarm in emergency. - Turn OFF power in emergency. - Take timely corrective measures. - Turn ON power when fault is removed. - Use of fire extinguisher and emergency exit. |
| CU4. Perform maintenance activities. | - Follow safety measures, as per standards - Investigate malfunction in accordance with company policies and procedures - Identify nature of fault. - Check the charging and gravity level of the battery - Ensure there is no breakage in the circuit. - Ensure that the PV cells are clean. - Inform higher authority for major fault (on time) - Test the equipment after it’s repaired and ready to be placed back. - Generate a document and mention the fault and the measures taken, for future use. |
| CU5. Generate Report | - Follow safety measures, as per standards. - Prepare Visit Report as per technical specifications of the plants |

Knowledge and understanding

- Define Solar Power Plant.

- Describe other types of Power Plant.

- Describe the role of PV Cells, Inverter Batteries, Circuit

- Describe the schematics diagrams and symbols used in a power plant.

- Basic Knowledge of personal and industrial safety standards.

- Describe pre start Inspections.

- Describe Switching procedure of the load.

- Knowledge of schematics and diagrams and symbols.

- Basic Knowledge of personal and industrial safety standards.

- Describe Purpose of safety alarms and Switches

- Life & efficiency

- Describe the purpose of fire extinguisher and emergency exist and its use.

- Define maintenance procedure.

- Describe the basic tools used for repairing.

- Describe the maintenance areas.

- Describe the procedure of testing the equipment.

- Define a visit report.

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | Visit to solar Power Plant |
| 2 | Fire Extinguisher |
| 3 | Emergency Switch |
| 4 | Safety helmet, gloves, goggles and shoes |
| 5 | Tool Box |
| 6 | Notepad |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

Identify various plant equipment and define their purpose

### 0713E&E42 Operate Wind Power Plant

Overview:

This competency standard covers the skills and knowledge required to inspect plant equipment, operate plant equipment, respond to equipment emergencies and alarms, perform maintenance activities and Generate Report. Operate and monitor plant generating equipment and take appropriate actions to ensure safe, efficient and reliable operation of the Wind power plant.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Inspect plant equipment. | - Adopt health and safety measures, as per standards. - Identify Generator, Turbine, Blades, Gearbox, transformer, transmission lines and other Auxiliary equipment. - Identify the type of Generator. - Monitor the speed of turbine at regular intervals. - Identify Control Panel and remote monitoring system. - Monitor readings of gauges and meters regularly - Perform calibration check for scale accuracy. - Examine the sounds, smells and vibrations of equipment. - Report Problem (If any) and take appropriate preventive measures. |
| CU2. Operate plant equipment. | - Follow safety measures, as per standards. - Perform switching as per requirement - Carry out shifting of the load according to the load management requirement at the time of peak load and base load. - Switch to alternative source during unfavorable weather conditions. - Analyze its smooth and efficient operation - Generate a report of switching operations and loads on generators, electrical lines and transformers. |
| CU3. Respond to equipment emergencies and alarms. | - Identify emergency alarms and switches. - Response to alarm in emergency. - Turn OFF power in emergency. - Take timely corrective measures. - Turn ON power when fault is removed. - Use of fire extinguisher and emergency exit. |
| CU4. Perform maintenance activities. | - Follow safety measures, as per standards - Investigate malfunction in accordance with company policies and procedures - Inspect the wind turbine periodically. - Monitor through SCADA and trace the fault. - Check and remove Operational errors (such as yaw error, pitching errors, excessive vibrations etc.) - Check and remove human based error - Carry out Routine checkups - Carry out Periodic Maintenance - Perform higher voltage equipment maintenance. - Determine the nature of the fault. - Identify correct tool for repairing. - Inform higher authority for major fault (on time) - Test the equipment after it’s repaired and ready to be placed back. - Generate a document and mention the fault and the measures taken, for future use. |
| CU5. Generate Report | - Follow safety measures, as per standards. - Prepare Visit Report as per technical specifications of the plants |

Knowledge and understanding

- Define Wind Power Plant.

- Describe types of Power Plant.

- Compare the Wind power plant with other generation plants

- Describe the role of Generator, Turbine, Blades, Gearbox, transformer, transmission lines and other Auxiliary equipment.

- Describe the schematics diagrams and symbols used in a power plant.

- Basic Knowledge of personal and industrial safety standards

- Define SCADA

- Describe pre start Inspections.

- Describe Switching procedure of the equipment.

- Knowledge of schematics and diagrams and symbols.

- Knowledge of high voltage switching and use of hot sticks substation components, and circuit breakers.

- Describe Purpose of safety alarms and Switches

- Define the procedure followed in emergency to shut down power and save life

- Describe the purpose of fire extinguisher and emergency exist and its use.

- Define maintenance procedure.

- Describe the basic tools used for repairing.

- Describe the maintenance areas.

- Describe the procedure of testing the equipment.

- Define a visit report.

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | Visit to wind Power Plant |
| 2 | Fire Extinguisher |
| 3 | Emergency Switch |
| 4 | Safety helmet, gloves, goggles and shoes |
| 5 | Tool Box |
| 6 | Notepad |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

Identify various plant equipment and define their purpose

### 0713E&E43 Utilize Electrical Energy in Building

Overview:

This competency standard covers the skills and knowledge required to specify the requirements and characteristics of electrical heating systems specify the requirements and characteristics of lighting systems and specifies the requirements and characteristics of ventilation and air-conditioning systems. Gain knowledge and develop competence related to the specification of services required for the utilization of electrical energy in buildings. The unit provides candidates with the opportunity to understand the concepts of heating, lighting, ventilation and air-conditioning systems and to develop the skills necessary to solve and design problems on these systems.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Specify the requirements and characteristics of electrical heating systems. | - Calculate specific heat capacity. - Calculate conduction heat loss through building - Calculate radiation heat loss through building - Choose the appliance with the ratings based on calculations. |
| CU2. Specify the requirements and characteristics of lighting systems. | - Calculate room index. - Calculate maintenance factor - Calculate Utilization Factor - Calculate space to height ratio - Select the quantity of illumination sources and their power accordingly. |
| CU3.Specify the requirements and characteristics of ventilation and air-conditioning systems. | - Determine thermal comfort level, using psychometric charts - Calculate heat loss through building - Identify the type of ventilation system (Natural or Mechanical) - Select ventilation and air-conditioning system according to given specifications |

Knowledge and understanding

- Describe the term ‘heat transfer’

- Describe conduction, convection and radiation

- Define Formula for calculating loss through conduction, convection and radiation

- Understanding of basic characteristics of space heating appliances

- Selection of space heating appliances to meet specific application

- Understands basic formula of room index (R.I) =(l*b)/hawk(l+b)

- M.F = (lumen output of lamp after sometimes)/ (lumen output of lamp after new)

  - Typical used values:

  - 0.8 – For offices/classroom

  - 0.7 – For clean Industry

  - 0.6 – For dirty Industry

- Describe Utilization factor (i.e. the ratio of effective luminous flux to the total luminous flux of light sources)

- Understands that height ration is the ratio of distance between adjacent luminaires (center to center) to their height above the working plane.

- Define Power.

- Define Thermal Comfort level

- Knowledge of reading psychometric chart

- Describe Reasons for heat loss in buildings

- Natural and mechanical ventilation systems

- Differentiate between ventilation and air-conditioning

- Define Humidifiers and de-humidifiers

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | Wiring |
| 2 | Psychometric chart |
| 3 | Calculator |
| 4 | Calculation Sheet |
| 5 | Visit to the site of installation (or reference location) |
| 6 | Measuring tape |
| 7 | Dimensions of the premises(already calculated by authorized person) |
| 8 | Owner manual of the appliance |
| 9 | Digital Thermometer |
| 10 | Lamps |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

Specify the requirements and characteristics of lighting systems.

### 0713E&E44 Calculate Tariffs and Economics

Overview:

This competency standard covers the skills and knowledge required to Calculate Tariff for Hyde Power Plant, Calculate Tariff for Solar Power Plant, Calculate Tariff for Thermal Power Plant and Comparative Analysis. After this competency standard, the trainee will be able to gain knowledge required to calculate tariffs of various power plants, their economy and merits/demerits.

| Competency Units | Performance Criteria |
| --- | --- |
| - Calculate Tariff for Hydro Power Plant | - Determine factors influencing cost (Types of Load, Maximum demand, the power factor of the load as per Go tariff regulation). - Check the latest Tariff (per unit cost), imposed by regulatory authority - Calculate per unit cost of hydro power plant. |
| - Calculate Tariff for Solar Power Plant | - Determine the efficiency of PV cell - Calculate the per unit cost of Generation (neglecting losses) |
| - Calculate Tariff for Thermal Power Plant | - Calculate the running cost of the plant - Calculate the operational cost of the plant - Calculate the per unit cost of Generation |
| - Comparative Analysis | - Compare the efficiency of Hydel, Solar and Thermal Power Plant - Compare the minimum and maximum generation capacity of Hydro, Solar and Thermal Power Plant - Compare the per unit generation cost of Hydel, Solar and Thermal Power Plant - Analyze sustainable and environment-friendly power generation system. |

Knowledge and understanding

- Knowledge of searching the required statistics from the official reports.

- Describe power factor, types of load, maximum demand

- Describe Power Factor Improvement Methods

- Define Merits and Demerits of Hydro Power Plant

- Determine the factors effecting efficiency of PV Cell (15-19%)

- Performance ratio= Real energy yield/theoretical energy yield (in %)

- Real output=kWp*number of peak un hours*performance ratio=kWh

- Basic Power(P)=VI

- Formula for finding per unit cost

- Define the cost of operations in running cost of power plant

- thermal power plant

- Define the payback period

- Formula for finding per unit cost

- Describe the merits and demerits of thermal power plant

- Understands the term comparative analysis

- Knowledge of construction of flow charts and tables for analysis

- Efficient in calculations

- Describe the procedure of preparing a report based on final conclusion.

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | Internet |
| 2 | Calculator |
| 3 | Calculation Sheet |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

Perform Comparative Analysis

### 0713E&E45 Carryout Transmission of Electrical Power

Overview:

This Competency standard deals with the skills and knowledge required to select the conductor for transmission line, sort different effects on Transmission lines during conduction, select the right insulator from different types according to the requirement, make foundation and erect poles and towers, perform stringing work and sag of transmission lines, install lightning arrester in a transmission system as per requirement.

| Competency Units | Performance Criteria |
| --- | --- |
| - Select the Conductor for Transmission line. | - Select the appropriate voltage level for transmission. - Select the conductor in accordance with current (A) value. |
| - Sort different effects on transmission lines during conduction | - Verify Ferranti effect, Skin effect and corona effect. - Apply method to correct losses. |
| - Select the right insulator from different types according to the requirement | - Select appropriate insulator according to utility. - Separate healthy, punctured and broken disk. - Calculate number of disks according to voltage level of transmission lines |
| - Make Foundation and Erect poles and towers | - Select the foundation type according to the nature of soil. - Select the foundation type according to ground clearance and circuit requirement. - Supervise foundation and erection of tower. |
| - Perform stringing work and sag of Transmission lines | - Calculate sag and bring it in accordance with clearance standards of WPADA/NTDCL. - Ensure to remove effects of wind and ice on Transmission Line. - Adjust sag according to present temperature. |
| - Install lightning arrester in a transmission system as per requirement | - Select appropriate Lightening Arrestors for Substation/Grid station with respect to the current. - Install the lightning arrestor. - Carryout connections for grounding. |

Knowledge & Understanding

- Describe the Current carrying capacity of different conductors

- Describe  the benefits/cost  of high voltage  Transmission

- Define Ferranti effect

- Define Skin effect

- Define corona effect

- Define the types of insulator

- Define suspension type and string

- Define Cree page  distance

- Define tower and pole type

- Define Foundation type

- Describe tower assembly

- Elaborate Standards for erection and foundation

- Define hazards and safety codes.

- Describe Sag calculation

- Explain Effect of weather condition on sag

- Explain Effect of sag on insulator assembly

- Define Lightning arrester with respect its types according to the

- Requirements.

- Define usage of Lightning Arrester

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | Lightning Arrester |
| 2 | Chain block |
| 3 | Temperature gage |
| 4 | Safety material |
| 5 | Theodolite |
| 6 | Torque |
| 7 | Dipole |
| 8 | Transmission system |
| 9 | Conductors. |
|  | Insulators. |
|  | Poles |
|  | Crane |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

As Instructed by the Teacher, in accordance with the available conditions

### 0713E&E46 Carryout Distribution of Electrical Power

Overview:

This Competency standard deals with the skills and knowledge required to select he conductor for distribution of power, Improve the Power factor, Separate the correct insulator from different types according to the requirement, Make Foundation and Erect poles and structures, Supervise construction of Trenches Cable try and select appropriate cable, Perform stringing work and sag of distribution lines, Measure performance of distribution lines and take steps to improve Primary and secondary distribution, Install the equipment for  grounding  of Substation/Transmission lines, Select and justify the use of reactor in a system, Install Lightning Arrester in a Distribution system as per requirement, Install Lightning Arrester in a Distribution system as per requirement.

| Competency Units | Performance Criteria |
| --- | --- |
| - Select the Conductor for Distribution of power. | P1. Select the appropriate voltage level for Distribution. P2.Select the conductor in accordance with current (A) value. |
| - Improve the Power factor | - Check the power factor. - Measure the difference between apparent power and real power. - Apply the appropriate method to improve power factor. |
| - Select the correct insulator from different types according to the requirement | - Select appropriate insulator according to utility. - Separate healthy, broken and punctured disk. - Calculate number of disks according to voltage level of transmission lines |
| - Make Foundation and Erect poles and structures | - Select foundation type according to the nature of soil. - Select the type according to ground clearance and circuit requirement. - Supervise foundation and erection of structures |
| - Supervise construction of Trenches ,Cable try and select appropriate cable | - Choose the right cable among belted, screamed and pressure cable - Select and supervise appropriate trench among direct laying and draw solid system - Select appropriate Cable tray |
| - Perform stringing work and sag of distribution lines | - Calculate sag and bring it in accordance with clearance standards of WPADA/NTDCL - Calculate effect of wind and ice on Distribution Line - Adjust sag according to current temperature |
| - Measure performance of distribution lines and take steps to improve Primary and secondary distribution. | - Calculate the power losses. - The voltage drops on a reaction of line - Identify different methods to regulate the voltage and minimize the losses. |
| - Install the equipment for grounding of Substation/Transmission lines | - Carryout different types of grounding (Resistance, Reactance, solid) - Supervise the grounding in accordance with WAPDA Standards - Select appropriate copper conductor depending on the capacity of grid - Test the resistance of ground using megger meter. - Carry out the jointing of copper conductor |
| - Select and justify the use of reactor in a system | - Select the appropriate reactor among the generator reactor, feeder reactor, bas bar reactor depending on the short circuit current in a system - Carryout the appropriate location for installation of reactor |
| - Install Lightning Arrester in a Distribution system as per requirement | - Select appropriate LA for Substation/Grid station with respect to the current - Carryout line connection |

Knowledge & Understanding

- Describe the Current carrying capacity of different conductors.

- Explain  the benefits/cost  of high voltage  Transmission

- Define the power Factor

- Define the types of insulator

- Define suspension type and string.

- Define Cree page distance.

- Define tower and pole type

- Define Foundation type

- Describe structure assembly

- Understand Standards

- Define hazards and follow safety codes

- Explain Cable sizing

- Describe Cable tray types

- Describe Trenches

- standards

- Explain Importance of insulation material for selection of appropriate cables

- Calculate Sag.

- Explain effect of weather condition on sag

- Explain effect of sag on insulator assembly

- Describe voltage drop, voltage regulation and power loses in a distribution system

- Explain the reasons of voltage drop on a line.

- Explain Concepts of grounding

- Describe Necessity of grounding

- Describe Types of grounding

- Describe Resistance Requirement for grounding

- Explain WAPDA standards

- Explain types and working principal of reactors.

- Explain Advantages and disadvantages of different types of reactors.

- Define Lightning arrester with respect it’s types according to the requirements

- Define usage of Lightning Arrester

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | Distribution system |
| 2 | Power factor improvement panel |
| 3 | Voltmeter |
| 4 | Temperature gage |
| 5 | Chain block |
| 6 | Theodolite |
| 7 | Magger |
| 8 | Megger |
| 9 | Insulation tester |
| 10 | Earthling equipment |
| 11 | Insulators |
| 12 | Poles |
| 13 | Crane |
| 14 | Safety material |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

As Instructed by the Teacher, in accordance with the available conditions

### 0713E&E47 Carryout Protection of Distribution and Transmission System

Overview:

This Competency standard deals with the skills and knowledge required to Install the equipment for grounding of Substation/Transmission lines, Categorize and rectify the symmetrical and asymmetrical faults, Install Circuit breaker in a distribution system (Substation/Grid station), Install an isolator at grid station/substation, install appropriate relays for the protection of power system, Carry out Bus Bar protection, Carry out time graded protection of feeder, Install a fuse in a circuit (Transformer and Distribution panels), Supervise Protection of power transformer, Carryout protection arrangement for alternator.

| Competency Units | Performance Criteria |
| --- | --- |
| - Categorize and rectify the symmetrical and asymmetrical faults | - Identify different type of faults on power system and categorize them in symmetrical and asymmetrical - Carryout remedial measure to rectify the faults - Plan to avoid faults in future |
| - Install Circuit breaker in a distribution system (Substation/Grid station) | - Select appropriate C.B for a substation or grid station among oil, gas, air, vacuum circuit breaker - Purpose required for a grid station/ substation. - Carryout connection of breaker. |
| - Install an isolator at grid station/sub station | - Select an appropriate type of isolator - Supervise connections according to the arrangement of an isolator - Trace the appropriate position of an isolator from single line diagram |
| - Install appropriate relays for the protection of power system | - Make selection of requisite proportion among different relays available - Make connection of respective relays (Current relay, distance relay, impedance relay etc.) |
| - Carry out Bus Bar protection | - Supervise arrangement for frame leakage protection. - Supervise arrangement for circuiting current protection |
| - Carry out time graded protection of feeder | - Make settings for time graded protections. - Make a system to isolate smaller part on a feeder when fault arise - Ensure to minimize the losses arising due to the fault by biasing appropriate section of timing and isolation type. |
| - Install a fuse in a circuit (Transformer and Distribution panels) | - Select among circuit breaker and fuse provided given circumstances - Calculate the current carrying capacity of fuse - Install fuse according to rating for 25KVA, 50KVA, 100KVA… Transformer - Carryout fuse arrangement for housing building after load |
| - Supervise Protection of power transformer | - Identify the need for protection of transformer. - Calculate the phase displacement in a transformer and identify the connection type’s i.e. star-star-star - Star-delta-delta - Delta-Delta-star |
| - Carryout protection arrangement for alternator | - Carry out protection against stator faults, balanced earthling fault protection and stator in-turn protection. - Select appropriate rating and settings for protections. |

Knowledge & Understanding

- Explain Type of faults in power system

- Explain the problems arising due to short circuit on power line

- Describe Necessity of Circuit breaker

- Explain Working principal for fault.

- Explain different types of circuit breaker

- Define different types of relays

- Define the working principal of relays of different type respectively

- Explain the purpose of relays and the need of relays

- Describe the Selection criteria.

- Describe working of frame protection

- Explain concept of time graded protection.

- Define Buss Bar protection

- Describe rating of circuit breaker

- Explain Using principal of an isolator

- Describe Usage of isolator

- Describe Types of isolator

- Describe working principal

- Explain rating parameters

- Explain fuse materials

- Define types of fuse

- Define advantages and disadvantages.

- Define faults on transformer

- Describe Merz-price system

- Explain protection for different types of connection.

- Explain Alternators faults

- Describe Protection against stator faults

- Explain Balanced earth fault protection

- Explain stator in-turn protection

Tools and Equipment

- Relays

- Insulator

- Circuit breakers

- Protection equipment

- Fuse

- Transformer

- Protection equipment of transformer.

- Protection equipment of alternator

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard

As Instructed by the Teacher, in accordance with the available conditions

### 0713E&E48 Investigate Micro Business Opportunities

Overview:

This competency describes the performance outcomes, skills and knowledge required to develop business ideas, and to investigate market needs and factors affecting potential markets.

| Competency Unit | Performance Criteria |
| --- | --- |
| - Describe business ideas | P1. Gather information for business ideas from appropriate sources P2. List details of business ideas and opportunities P3. Research alternative business ideas in light of the resources available P4. Specify and list products and services to match business ideas P5. .Identify and research potential customer information for business ideas P6. Identify and take into account financial, business and technical skills available when researching business opportunities |
| - Identify market needs | P1. Collect information regarding market size and potential from appropriate sources P2. Investigate market trends and developments to identify market needs relative to business ideas P3. Gather market information from primary and secondary sources to identify possible market needs in relation to business ideas P4. Identify ethical and cultural requirements of the market and their impact on business ideas P5. Identify new and emerging markets and document their features P6. Identify and organise information on expected market growth or decline and associated risk factors |
| - Investigate factors affecting the market | P1. Identify projected changes in population, economic activity and the labour force that may affect business ideas P2. Identify movements in prices and projected changes in availability of resources P3. Review trends and developments and identify their potential impact on business ideas |

Knowledge and understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Define entrepreneurship.

- Explain the concept of entrepreneurship

- Explain the various types of enterprise that exist in the community

- Identify and interpret the terms and elements involved in the concept of enterprise

- Appreciate that the advancement of individual and society in general when entrepreneurship is adopted

- Explain various motivational factors that entrepreneurs possess and utilize.

- Exhibit the skills needed to assess and evaluate a risk

- Describe the outline of small enterprise

- Describe the creativity and innovation

- Apply the techniques for developing creative abilities

- Explain the resources of business idea

- Explain the collective and creative thinking

- Explain how to generate a business idea

- Appreciate the importance of, and possess techniques for identifying and assessing business opportunities.

- Identify the various entrepreneurial characteristics

- Access personal potential for becoming future entrepreneurs.

- Identify leadership qualities which are essential to the success of entrepreneurs

- Identify self- management skills and how they are important to be enterprising

- Apply a rational approach to make personal and business decisions

- Explain the steps for decision making and rating of decision making skills

- Apply the rules of negotiation for resolving business issues

Tools and Equipment

| S No. | Tools |
| --- | --- |
| 1 | Calculator |
| 2 | Ruler |
| 3 | Papers and Pencil |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

Evidence of the following is essential:

Thorough investigation of business opportunities and ideas

Clearly identified products/services and customer information for each business idea

Thorough collection and analysis of market information and associated factors relating to business ideas

Knowledge of ethical and cultural requirements.

### 0713E&E49 Develop a Micro Business Proposal

Overview:

This competency describes the performance outcomes, skills and knowledge required to develop an identified business idea, to research the feasibility of the business opportunity and to present a business idea in formats that suit a range of stakeholders

| Competency Unit | Performance Criteria |
| --- | --- |
| CU1.Evaluate business opportunities | P1. Identify and research key factors that influence viability of business ideas P2. Analyse business ideas in terms of personal or family needs and commitments P3. Evaluate impacts of emerging or changing technology, including e-commerce, on the business P4. Determine viability of business opportunity in line with perceived risks, resources available, financial returns and other outcomes sought P5. Assess and match personal skills/attributes against those perceived as necessary for a particular business opportunity P6. Identify and assess business risks according to resources available and personal preferences |
| CU2.Detail the business idea | P1. .Develop an accurate description of the business idea for key stakeholders P2. Develop an accurate summary of the major products and/or services required to suit personal needs and requirement |
| CU3.Prepare the business Overview to suit different stakeholders | P1. Present an accurate list of key stakeholders and their information requirements P2. Determine an acceptable method of presentation of information for each stakeholder P3. Provide accurate customised information to target audiences |

Knowledge and understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- State and local government legislative requirements relating to business operation, especially in regard to occupational health and safety (OHS) and environmental issues,

- Income and expenditure costing

- Principles of risk assessment relevant to the business opportunity

Tools and Equipment

| S No. | Tools |
| --- | --- |
| 1 | Calculator |
| 2 | Ruler |
| 3 | Papers and Pencil |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

Evidence of the following is essential:

- accurate and complete outline of the business idea that considers the major elements of:

  - products/services

  - customers

  - operations and processes

  - income and expenditure

  - resources

  - marketing

  - location

### 0713E&E50 Develop a Marketing Plan

Overview:

This competency describes the performance outcomes, skills and knowledge required to research, develop and present a marketing plan for an entrepreneurship business

| Competency Unit | Performance Criteria |
| --- | --- |
| - Devise marketing strategies | P1.Evaluate marketing opportunity options that address organisational objectives, and evaluate their risks and returns in the selection process P2.Develop marketing strategies that address strengths and opportunities within the organisation's projected capabilities and resources P3. Develop strategies which increase resources or organisational expertise where gaps exist between current capability and marketing objectives P4.Develop feasible marketing strategies and communicate reasons that justifies their selection P5. Ensure strategies align with organisation's strategic direction P6.Develop a marketing performance review strategy, incorporating appropriate marketing metrics to review of organisational performance against marketing objectives |
| - Plan marketing tactics | P1. Detail tactics to implement each marketing strategy in terms of scheduling, costing, accountabilities and persons responsible P2. Identify coordination and monitoring mechanisms for scheduled activities P3.Ensure tactics are achievable within organisation's projected capabilities and budget P4. Ensure tactics meeting legal and ethical requirements P5.Ensure tactics provide for ongoing review of performance against objectives and budgets, and allow marketing targets to be adjusted if necessary |
| - Prepare and present a marketing plan | P1.Ensure marketing plan meets organisational, as well as marketing, objectives and incorporates marketing approaches and a strategic marketing mix P2. Ensure marketing plan contains a rationale for objectives and information that supports the choice of strategies and tactics P3. Present marketing plan for approval in the required format and timeframe P4. Adjust marketing plan in response to feedback from key stakeholders and disseminate for implementation within the required timeframe |

Knowledge and understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of: Culturally appropriate communication skills to relate to people from diverse backgrounds and people with diverse abilities

- Describe the market & marketing

- Differentiate between sellers and buyers’ market

- Describe the five ‘w’ of market

- Explain the procedure for assessing the market size and demand

- Explain the major factors to be considered when selecting a location for a business

- Describe the basic types of business ownership and the limitation of each

- Explain the computation of initial and working capital needed to start an enterprise

- Identify the advantages and disadvantages of using various sources of capital to start an enterprise

- Explain the component of cost of product

- Explain the breakeven analysis for a new business

- Calculate the breakeven point for various new business

Tools and Equipment

| S No. | Tools |
| --- | --- |
| 1 | Calculator |
| 2 | Ruler |
| 3 | Papers and Pencil |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard

Devising, documenting and presenting a marketing plan

Detailing approaches and

The marketing mix to achieve organisational marketing objectives.

### 0713E&E51 Develop and Review a Business Plan.

Overview:

This competency standard covers the process of developing and reviewing business for a small business enterprise. It requires the application of knowledge and skills to determine the scope of the business plan, prepare a business plan, determine goals, trial systems, and document, monitor and review the business plan.

| Competency Unit | Performance Criteria |
| --- | --- |
| - Determine scope of business | - Determine scope of the business plan and associated systems is determined in consultation with specialist personnel. - Access accurate information for inform business plan development - Account for and incorporate trends and seasonal variations into the business plan. - Account for strategic goals, targets and directions of the enterprise in the development of the business plan - Comply Legal obligations in developing the business plan. |
| - Prepare business plan | - Develop operational goals and targets to meet the enterprise strategic plan. - Identify and incorporate supply chains into the business plan. - Identify risk management needs are within the business plan. - Incorporate trial systems in order to test budgetary impact and operational potential prior to full implementation of the business plan. - Set clear and measureable indicators of operational performance to allow for realistic analysis of performance. |
| - Document and review business plan | - Include fiscal and operational systems that enhance performance management and suit enterprise requirements. - Incorporate resource considerations the business plan. - Document accurately and clearly communicate business Plan to all relevant parties. - Monitor to identify strengths, weaknesses and areas for improvement performance against the business plan - Make recommendations to improve the business plan and associated systems as required. |

Knowledge and understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Appreciate the importance of business plan

- Explain the process of writing a business plan

- Develop feasibility for a business idea

- Realize the problem that may be encountered when starting a small business/Enterprise

- Develop a business plan for a small business on the standard format

- Evaluate the business plan in a real market satiation

- The knowledge requirements for this competency standard are listed below:

- budgeting

- forecasting

- operational systems

- relevant industrial awards and agreements

- communication techniques

- logical and analytic methods

- profit and loss and cash flow systems

- working knowledge of environmental, OHS, industrial relations, taxation, corporate and industry legislation as they relate to the enterprise

Tools and Equipment

| S No. | Tools |
| --- | --- |
| 1 | Construction Lab Tools |
| 2 | Rule,tape, square, hammer, hand saw, hand plane, chisel, shovel, wheelbarrow, sledge hammer, pick, mattock andcrowbar andpinchbarfor given tasks. |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard.

The skills and knowledge required to develop and review a business plan must be transferable to a different work environment.  For example, if competence is demonstrated in developing a business plan for a small enterprise, it must also be evident in reviewing a business plan in medium or large enterprise environment.

### 0713E&E52 Organize Finances for the Micro Business

Overview:

This competency standard describes the performance outcomes, skills and knowledge required to investigate the financial capacity to enter into a micro business, to determine the projected cash flow, to source finances and to monitor the profitability of the business.

| Competency Unit | Performance Criteria |
| --- | --- |
| - Ascertain own financial position and the ability to provide capital/equity for the business | P1. Realistically detail personal, family or community financial situation in terms of funds available and commitments already incurred P2. Determine equity finance and assets available for micro business from personal, family or community sources |
| - Determine projected cash flow for the business | P1. Determine the level of forecast business activity over a year and the business mix P2. Estimate establishment costs for the business and repayment schedule for borrowings P3. Calculate the monthly variable and fixed costs needed to conduct business activity over a year P4. Estimate personal drawings needed to be taken from the business P5.Estimate the monthly income generated by the business for a year based on price per unit item or hourly charge rate for labor P6. Develop a cash flow budget for the first year of business operation P7. Seek professional advice to estimate goods and services tax and operating finance required for the business |
| - Source the required funds to establish the business | P1. Estimate required funding to establish and run the business based on expected sales and activity levels, available finances and commitments P2. Investigate methods of accessing alternative sources of finance P3. Identify strategies for meeting financial obligations P4. Implement plans to access available funds as required |
| - Monitor profitability of the business | P1. Maintain and review monthly expenditure and income records P2. Compare equity at beginning and end of a year to estimate business performance P3. Assess the financial viability of the business after a year of operation P4. Seek professional advice on depreciation, insurance and tax implications of the business |

Knowledge and understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Basic budgeting

- basic costing for the business

- financial commitments and requirements

- financial reports and terminology

- methods and relative costs of obtaining finance

- own financial position

- Sources of advice and assistance.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard

- investigation of projected cash flow for the business

- estimation of the funding needed to establish and operate the business

- assessment of the financial viability of the business

- Knowledge of basic budgeting.

### 0713E&E53 Manage Human Resources

Overview:

This competency standard covers the skills and knowledge required to manage human resources and to manage and develop human resources to achieve organization’s operational objectives

| Competency Unit | Performance Criteria |
| --- | --- |
| - Lead and motivate people | P1.Establish goals for people and teams to optimise achievement in work tasks. P2.Take into account the capabilities of people and teams. P3. Provide advice and support sensitive to the individual's needs to people in the performance of their duties. P4. Undertake activities to achieve commitment to common goals. P5. Recognise and encourage initiative and innovation P6. Recognise and communicate achievements within the organisation. |
| - Undertake human resource planning | P1.Determine human resource needs within the anticipated operational needs and allocated budget. P2. Analyse alternatives to staffing levels which clearly demonstrate returns to the organisation. P3. Develop contingency plans for staffing which meet key provisions of the human resources plan. P4. Compare existing competencies of staff with the needs of the work group. P5. Plan staffing levels and negotiate with stakeholders within the organisational framework to achieve maximum efficiency of operations. |
| - Develop and facilitate performance | P1.Negotiate Performance Criteria individuals, teams and work groups. P2. Review Performance Criteria as circumstances change. P3. Conduct performance appraisal based on clearly established and agreed Performance Criteria. P4.Identify and propose the total performance development system strategies to rectify performance shortfalls and recognise success. P5. Address performance problems confidentially and in a constructive and timely manner, in line with relevant organisational procedures. P6. Make selections, transfers and promotions in accordance with organisation policies and supported with documented information. P7. Develop and implement mechanisms for the identification of human resource development needs within the work group taking account of the strategic plan for the organisation. |
| - Facilitate training, education and development opportunities | P1.Make information on planned training events widely available throughout the organisation. P2. Include training, education and development plans as part of individual/team performance plans. P3. Facilitate individual/team access to, and participation in, training, education and development opportunities. P4.Contribute coaching and mentoring effectively to the training, education and development of personnel in an environment of change. P5.Enhance training, education and development opportunities of individual, team and organisational performance. P6. Create workplace environment is which facilitates training, education and development |

Knowledge and understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Describe the hiring method/Procedures

- Describe the term & conditions of services and job description for various employments

- Describe the characteristics of successful sales personals

- communication principles

- conflict resolution principles and practice

- Equal Employment Opportunity

- grievance procedures

- interpersonal relations

- leadership theory and principles

- management principles and practice

- Occupational Health and Safety

- training and education principles

- training need analysis

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard

It is essential that competence be demonstrated in the application of human resource management in a wide range of contexts in achieving the organisation's objectives.

- Evidence should be gathered over a period of time in a range of actual or simulated workplace environments.

- Evidence of competent performance should be obtained by observing an individual in a management role within the workplace or exercise or operational environment. Knowledge may be assessed through written assignments, project reports, debriefings and action learning projects.

### 0713E&E54 Market Products and Services

Overview:

This competency standard covers the skills and knowledge required to market products and services

| Competency Unit | Performance Criteria |
| --- | --- |
| - Analyze market information | P1. Identify, research and analyse existing or new markets for existing or new products or services using techniques to ensure reliable data P2. Analyse past trends and developments to determine market variability and associated risks P3. Develop gross margin budgets to account for market variability P4.Identify and evaluate competing products to determine strengths and weaknesses of own products P5. Monitor market environment to ensure information is current and reliable P6. Identify the legal, ethical and environmental constraints of the markets and their effect on the enterprise P7. Identify product specifications that suit market requirements and price advantage at the time P8. Present clear and concise information to the enterprise management team. |
| - Identify and evaluate factors to include in a marketing plan | P1. Identify and evaluate production processes to ensure required product specifications are met P2. Identify and assess alternative selling strategies and techniques to identify marketing targets and methods P3. Identify and assess distribution channels and their role in your marketing strategies P4. Ensure the data used is reliable and the market environment and trends are substantiated P5. Evaluate the role of marketing professionals in providing advice |
| - Develop a marketing plan for your products and services | P1. Establish marketing objectives based on current and potential product specifications P2.Select appropriate production processes to ensure product specifications are met P3. Select selling strategies to ensure required prices are achieved P4. Select appropriate distribution channel options to ensure access to target markets is achieved efficiently and appropriately P5.Establish time-frames for production, distribution and selling activities P6.Develop a gross margin budget to demonstrate the cost effectiveness of the marketing plan P7.Develop partial gross margin budgets to account for market variability |
| - Determine promotional strategies | P1. Prepare and record detailed plans for promotional activities P2. Outline objectives, level of exposure and available markets P3. Ensure strategies take account of time management and scheduling issues, and resource constraints P4. Create promotional materials that enhance the product and commercial presentation P5.Record and communicate priorities, responsibilities, timelines and budgets for promotional activities. |
| - Implement marketing activities | P1. Schedule planned marketing activities within appropriate timeframes P2. Develop measurable performance targets that meet business plan objectives P3. Organise distribution channels and ensure product and service information is accurate and readily available to clients P4. Implement marketing activities within budgetary constraints to meet legal, ethical and enterprise requirements |
| - Evaluate marketing performance. | P1. Review the established marketing objectives to ensure they remain viable P2. Make an objective assessment of the marketing plan and its implementation by a comparison of valid and reliable data against the established objectives P3.Assess product, pricing and distribution policies in relation to market changes, marketing objectives and enterprise requirements P4. Identify areas of positive marketing performance and take corrective action to remedy poor marketing performance areas P5. Document and distribute information for continual analysis and effective planning management |

Knowledge and understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Describe the life cycle of product

- Identify the various ways of selecting suppliers,

- Explain the inventory management of stock, raw material and finished goods etc.

- Appreciate the importance of financial record keeping in a small business

- Explain techniques to keep cost as low as possible

- Develop balance sheet for a small enterprise

- Explain the operating cycle concept

- Explain the income tax computation procedure for a small business

- Explain the basic scheme of sales tax

- Explain the assessment procedure for returns and filling of returns.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard

- Handle and set Lightening for Current affair program

- identify the marketable features of the product and potential markets

- develop a range of marketing alternatives

- collect and analyses data to assess alternatives in a marketing plan

- evaluate performance targets and recommend modifications or improvements

- implement and evaluate a marketing plan

- Plan to manage promotional activities.

### 0713E&E55 Monitor and Review Business Performance

Overview:

This competency standard covers the skills and knowledge required tomonitor and review business performance

| Competency Unit | Performance Criteria |
| --- | --- |
| - Evaluate commercial performance | P1. Gather and analyse data relating to enterprise performance to identify historical and current performance. P2. Review and analyse operational structures to determine the suitability of organisational processes to enterprise objectives. P3. Evaluate enterprise strengths and weaknesses against market conditions to determine current and future capacities. P4. Evaluate enterprise objectives are to identify variations and scope for future development. |
| - Allocate and co-ordinate business resources | P1. Identify and communicate roles and responsibilities of personnel. P2. Identify resource requirements for enterprise and cost them using standard financial analysis techniques. P3. Calculate costs of ensuring sustainability of enterprise operations and factor into business planning for the enterprise. |
| - Identify performance requirements | - Develop realistic performance indicators within available timeframes and resources - Identify and minimize factors inhibiting performance against objectives. - Monitor and assess market conditions based on relevant data. - Prepare and incorporate strategies and programs to promote the sustainability of operations into enterprise procedures. |
| - Review business performance | - Review regularly enterprise operations to identify opportunities for improvements in performance. - Monitor and anticipate impact of natural conditions on enterprise to assess sustainability of resource use. - Compare costs and estimates with resource allocation. - Determine operational plans to determine schedule of activities |

Knowledge and understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- rates of return for products and/or services

- financial analysis techniques

- structure and operation of small businesses relevant State/Territory Occupational Health and Safety (OHS)

- legislative requirements

- environmental conditions, positive environmental practices and negative impact minimisation measures

- human resource requirements for the enterprise

- transport requirements for the enterprise

- Enterprise/property improvement requirements.

- market performance in commodities

- statutory marketing requirements

Critical Evidence(s) Required

- The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard

review operational structures to determine effectiveness

identify available resources to assess capacity

develop realistic performance indicators

review enterprise operations against performance indicators

Plan to improve business performance by addressing results of review.

### 0713E&E56 Negotiate for Resolving Business Issues

Overview:

This competency standard covers the skills and knowledge required to negotiate for resolving business issues

| Competency Unit | Performance Criteria |
| --- | --- |
| - Develop and implement issue resolution and grievance procedures | P1. Establish problem solving/issue resolution procedures within legislative requirements and organisation's guidelines. P2. Manage grievances and complaints are in a timely and caring way to optimise likelihood of a favourable outcome for all parties and in line with organisational objectives and procedures. P3.Document and communicate individual's rights and obligations under industrial awards/agreements and legislation are in a clear and concise manner and in appropriate language. P4. Conduct hearings, interviews and meetings within the principles of industrial democracy and participative, consultative processes. P5. Identify and invite all relevant industrial parties to participate in the resolution process. |
| - Manage disciplinary matters | P1.Implement disciplinary matters in accordance with organisation's procedures. P2. Conduct investigations in a caring and confidential manner to maintain performance and morale. P3. Provide feedback promptly. P4.Institute appeals processes in accordance with organisational procedures. P5. Recognize and recommend deficiencies in procedures for changes made |

Knowledge and understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Communication Skills

- Interpersonal skills

- Business letter writing

- Forecasting techniques

### 0713E&E57 Manage Personal Finances

Overview:

This unit of competency describes the outcomes required to develop, implement and monitor a personal budget in order to plan regular savings and manage debt effectively.

| Unit of Competency | Performance Criteria |
| --- | --- |
| - Develop a personal budget | - Calculate current living expenses using available information to prepare a personal budget. - Keep a record of all income and expenses for a short period of time to help estimate ongoing expenses. - Subtract total expenses from total income to determine a surplus or deficit budget for the specified period. - Find reasons for a deficit budget and ways to reduce expenditure identified. - Identify ways to increase income, if possible |
| - Develop longer term personal budget | - Analyze income and expenditure and set longer term personal, work and financial goals. - Develop a longer-term budget based on the outcomes of short-term budgeting, and adjust to meet living, work and future career requirements. - Identify obstacles that might affect finances such as job loss, sickness or unexpected expenses contingency savings - Formulate a regular savings plan based on budget, using secure savings products and services. - Monitor expenditure against budget and identify areas of possible expenditure saving |
| - Identify ways to maximize future finances | - Determine sources and ways to maximize personal income, including from work, investments or available government payments/allowances. - Get further education or training to maintain or improve future income. - Identify the need for debt to finance living and other expenses, and determine the appropriate levels of debt and repayment. - Consolidate existing debt, where possible, to minimize interest costs and fees. - Seek professional money management services, where available, to ensure financial plans are effective and achievable. |

Knowledge and understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- .Explain the abilities to plan and organize to keep records and monitor a personal budget

- .Describe abilities to set and review goals

- Explain basic financial management and record keeping to enable development and management of a personal budget

- Describe benefits of financial goal setting and personal budgeting to enable effective management of personal finances

- Outline numeracy skills to compare income and expenditure

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard

A person who demonstrates competency in this unit must be able to provide evidence of the ability to manage personal finances. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

- develop a personal budget based on analysis of expenditure and income;

- formulate goals and identify financial contingency plans; and

- Monitor expenditure for a period of up to 2 weeks

### 0713E&E58 Coordinate a Work Team

Overview:

This unit is concerned with the competencies required to achieve operational outcomes and effective working relationships through managing and developing individuals and teams.

| Unit of Competency | Performance Criteria |
| --- | --- |
| - Develop and maintain a cooperative work group | - Work contributions and suggestions from staff are - continually sought and encouraged - Contributions to work group operations are acknowledged and suggestions are dealt with constructively - Develop staff skills according to work requirements - Implement new work practices - Address conflict between staff members in accordance with current personnel practices. |
| - Communicate objectives and required standards | - Inform the staff of the objectives and standards - required - Commit to objectives and standards - Practices of safe, fair and participative work principals are and promote to staff |
| Provide feedback on performance | - Give constructive feedback on all aspects of work performance provided to individuals and team - Access and address performance in a fair and timely manner in accordance with relevant guidelines, procedures and natural justice |
| - Support and participate in development activities | - Assess training needs of all staff, implemented and promoted - Devise an action plan to meet individual and group training - and development needs is collaboratively developed, - agreed to and implemented - Identify specific training needs of individuals - Encourage staff in applying skills and knowledge in the workplace - Provide training to the required standard on the job - Support and encourage staff to attend training courses and to take up other development opportunities. |
| - Provide leadership. direction and guidance to the work group | - Link between the function of the group and the - goals of the organization - Participate in decision making routinely to develop, implement and review work of the group and to allocate responsibilities where appropriate - Give opportunities and encouragement to others to develop new and innovative work practices and strategies - Identify conflict and resolve with minimum disruption to work group function - Provide staff with the support and supervision necessary to perform work safely and without risk to health - Allocate tasks within the competence of staff and support with appropriate authority, autonomy and training - Supervise appropriately the changing priorities and situations and takes into account the different needs of individuals and the requirements of the task |

Knowledge and understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Explain the principles of effective team operation

- Explain the principles of human resource management

- Describe the training delivery processes in the workplace

- Outline the industry assessment guidelines

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard

A person who demonstrates competency in this unit must be able to provide evidence of the ability to coordinate a work team. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Evidence of the following is essential:

- assessing and evaluating skills

- working effectively in a team environment

- achievement of work outcomes

### 0713E&E159 Lead Small Teams

Overview:

This unit describes the outcomes required to lead small teams including setting and maintaining team and individual performance standards

.

| Unit of Competency | Performance Criteria |
| --- | --- |
| - Facilitate team Development | - Identify work requirements, standards and purpose to team members. - Assist team to develop objectives, targets and key performance indicators relevant its purpose and workplace goals. - Allocate duties regard to the skills required to properly - undertake the assigned task and according to company policy - Identify roles, responsibilities and expectations of each team member - Disseminate and discuss performance expectations to individual team members. |
| - Motivate and build the Team | - Develop positive and constructive relationships with and between team members - Facilitate team communication processes - Involve team members in the process of examining risks and options and making decisions, to ensure acceptance and support. - Encourage individual and team efforts and contributions - Strengths and weaknesses of team members are determined and sharing of work tasks is promoted to up skill team members. - Recognize team members’ queries and discuss and deal with it. |
| - Facilitate and monitor team effectiveness | - Monitor the implementation of work plan and team and individual performance against agreed strategies, targets and standards, according to workplace policies and procedures. - Monitor performance against defined Performance Criteria and/or assignment instructions and corrective action taken if required. - Support team in identifying and resolving problems that may impede performance and to suggest improvements in team Performance. - Consult team members in any review and revision of team objectives and goals. - Address performance issues which cannot be rectified within the team to appropriate personnel according to employer policy. - Refer concerns of a team and individual are referred to next level of management or appropriate specialist and conduct negotiations on their behalf. - Keep team members inform of any changes in the priority allocated to assignments, or tasks which might impact on client/customer needs and satisfaction. - Monitor team operations to ensure that internal or external employer/client needs and requirements are met. - Provide follow-up communication on all issues affecting the team - Conduct team meetings to review work operations and address issues according to workplace policies and procedures. - Support team in identifying and resolving problems that may impede performance and to suggest improvements in team performance. - Consult team members in any review and revision of team objectives and goals. - Raise any inappropriate values and standards exhibited in the workplace with the person concerned. |

Knowledge and understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Explain conflict resolution techniques

- Explain management styles

- Describe methods of monitoring performance

- Outline the relevant legal requirements

- Outline strategies for dealing effectively with team member complaints or grievances and

- Explain team dynamics and facilitation processes

- Describe communication skills

- State workplace policies and procedures

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard

A person who demonstrates competency in this unit must be able to provide evidence of the ability to lead small teams. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Evidence of the following is essential:

- demonstrate the ability to build positive team spirit and effectively manage overall team

- performance within a workplace context;

- demonstrate the ability to coordinate a work team in a range of contexts or occasions; and

- Show knowledge of leadership, motivation and teamwork principles.

Green Skill

### 0713E&E60 Utilize Natural Energy in the Building

Overview:

This competency standard covers the skills and knowledge required toDesign Natural lightning system, Design Natural Cooling and ventilation system and Design Natural heating system after this competency standard, the trainee will be able to develop knowledge and competence to utilize natural energy in efficient way for lighting, heating and cooling system. The unit provides candidates with the opportunity to create an eco-friendly building that not only benefits the consumer but helps in balancing the load on the national grid during power crises

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Design Natural lightning system. | - Determine the direction of Sunrise and Sunset. - Choose the color scheme of the interior - Select the quality of glass for windows - Design a Sun tunnel (skylight) - Calculate the size of skylight for ceilings. - Select the reflectors wisely. |
| CU2. Design Natural Cooling and ventilation system | - Determine the direction of Air - Select the type of paint used for external walls and roof. - Identify the type of coating used for tilting glass of window. - Select the type of shades required to block heat - . Locate the position for natural vent based on the phenomenon of conduction and convection - . Identify the location for construction of water tanks - . Determine the excessive heat producing sources - Select energy efficient devices for heating, lighting, cooling and home appliances. - Choose the Sun vent according to requirement of the premises. - Choose the cooling system from variety of Solar Powered Inverter AC/heating system. |
| CU3. Design Natural heating system | - Select the type of solar cell(heating) - Choose the angle of placing solar cell. - Identify the location for installation of solar tank. - Design a solar heating system. - Identify the location for Boiler. - Make suitable connections based on drawings. - Identify the place for blocking air inside room(airtight) |

Knowledge and understanding

- Knowledge of using earth compass

- Describe the heat conduction and radiation properties of different colors.

- Describe the phenomenon of Skylight

- Describe the procedure adopted for the design of Sun Tunnel

- Define the properties of reflection Glass.

- Knowledge of using wind vane.

- Describe the properties of paints with respect to its heat conduction

- Define Geo-Thermal Energy

- Describe Reflective window coatings

- Describe the types of shades (indoor and outdoor)

- What are the advantages of using underground tanks over sun exposed tanks?

- Describe the advantage of LED over tungsten bulb

- Describe the phenomenon of Sun vent

- Define the natural heat sources.

- Define fossil fuels

- Explain the terms Solar Energy, Geothermal energy, biomass, fossil fuels.

- Explain ‘Greenhouse’ effect.

- Knowledge of drawing of the solar heating system.

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | Solar Cell |
| 2 | Boiler |
| 3 | Insulation film |
| 4 | Wind vane |
| 5 | Reflection glass |
| 6 | Skylight |
| 7 | Pipe like steel structure for construction of sun tunnel |
| 8 | Paint |
| 9 | Solar plate(Small) |
| 10 | Earth Compass |
| 11 | Glass for windows and entry sources |
| 12 | Sun Shades |
| 13 | Drawing sheet |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard

Design Natural lightning system