|  | Assessment Evidence Guide For National Vocational Certificate in Electric Vehicle (EV) Assistant Technician L-2 Formative Assessment (0716MSA1911. Maintain 4-Wheeler EVs) |
| --- | --- |
|  | National Vocational & Technical Training Commission |

Section A: Instruction Sheet for the Candidate

| Qualification | Electric Vehicle (EV) Assistant Technician L-2 |
| --- | --- |
| Competency Standard(s) | 0716MSA1911. Maintain 4-Wheeler EVs |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name_______________________________________________________ Registration Number_________________________________________ |

Section A1: Guidance for Candidate

| General Guidance | To meet this standard, you are required to - Maintain Battery Pack and Charging System of 4-Wheel - Maintain Electric Motor and drivetrain System of 4-Wheel - Identify HVAC of 4-Wheel - Inspect Suspension, steering, Brake, and Tire of 4-Wheeler - Interpret fault codes on cluster of 4-Wheeler Activity: Perform complete Diagnostic and Maintenance check of a 4-Wheeler EV |
| --- | --- |
| Time | 04 Hours |
| Minimum Evidence Required | During a practical assessment, under observation by an assessor, the candidate (s) is/are required to: - Measure battery pack voltage using approved diagnostic equipment. - Inspect charging inlet for damage, contamination, and loose fittings. - Check battery terminals for corrosion and secure connections. - Check traction motor for abnormal noise, vibration, and overheating. - Inspect reduction gearbox for lubrication level and mechanical wear. - Check drive shafts for alignment and physical damage. - Test accelerator system under controlled operating conditions. - Identify HVAC components and clarify control functions. - Check brake pads for wear and replacement requirements. - Perform diagnostic scan to identify and interpret fault codes. |

Section A2: Self-Assessment Checklist

| Assessment Task | Activity: - Perform complete Diagnostic and Maintenance check of a 4-Wheeler EV |
| --- | --- |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Measure battery pack voltage using approved diagnostic equipment. |  |  |
| - Inspect charging inlet for damage, contamination, and loose fittings. |  |  |
| - Check battery terminals for corrosion and secure connections. |  |  |
| - Check traction motor for abnormal noise, vibration, and overheating. |  |  |
| - Inspect reduction gearbox for lubrication level and mechanical wear. |  |  |
| - Check drive shafts for alignment and physical damage. |  |  |
| - Test accelerator system under controlled operating conditions. |  |  |
| - Identify HVAC components and clarify control functions. |  |  |
| - Check brake pads for wear and replacement requirements. |  |  |
| - Perform diagnostic scan to identify and interpret fault codes. |  |  |

Candidate’s Signature__________________ Assessor’s Signature__________________

Date: _______________________________

Section B: Assessors Judgment Guide

| Qualification | Electric Vehicle (EV) Assistant Technician L-2 |
| --- | --- |
| Competency Standard(s) | 0716MSA1911. Maintain 4-Wheeler EVs |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: __________________________________________________ Registration Number: ________________________ Signature: __________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor_______________________________________ Assessor’s code: __________________ Signature: _______________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  | X |  | X |  |  |
| Knowledge Assessment | X | X |  |  |  |  |  |
| Other Requirement |  |  |  | X |  |  |  |

Section B1: Observation Checklist

| Assessment Task |  | Activity: Perform complete Diagnostic and Maintenance check of a 4-Wheeler EV |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Measured battery pack voltage using approved diagnostic equipment. |  |  |  |  |  |
|  | Inspected charging inlet for damage, contamination, and loose fittings. |  |  |  |  |  |
|  | Checked battery terminals for corrosion and secure connections. |  |  |  |  |  |
|  | Checked traction motor for abnormal noise, vibration, and overheating. |  |  |  |  |  |
|  | Inspected reduction gearbox for lubrication level and mechanical wear. |  |  |  |  |  |
|  | Checked drive shafts for alignment and physical damage. |  |  |  |  |  |
|  | Tested accelerator system under controlled operating conditions. |  |  |  |  |  |
|  | Identified HVAC components and clarified control functions. |  |  |  |  |  |
|  | Checked brake pads for wear and replacement requirements. |  |  |  |  |  |
|  | Performed diagnostic scan to identify and interpret fault codes. |  |  |  |  |  |
| Competent |  |  | Not yet Competent |  |  |  |  |

Section B2: Knowledge Assessment

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
| 1 | What is a traction battery? |  |  |
| 2 | What is the function of HVAC in EVs? |  |  |
| 3 | Why is wheel alignment important? |  |  |
| 4 | What is ABS? |  |  |
| 5 | Why are airbags important? |  |  |
| 6 | What is preventive maintenance? |  |  |
| 7 | Why should brake fluid be checked? |  |  |
| 8 | What is the purpose of a cabin battery? |  |  |
| 9 | Why is software diagnostics important? |  |  |
| 10 | What does the dashboard warning system indicate? |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature ___________________