|  | Assessment Evidence Guide For National Diploma Certificate in Electric Vehicle (EV) Supervisor L-4 Formative Assessment (0716MSA218. Conduct ADAS & Autonomous Sensor Integration) |
| --- | --- |
|  | National Vocational & Technical Training Commission |

Section A: Instruction Sheet for the Candidate

| Qualification | Electric Vehicle (EV) Supervisor L-4 |
| --- | --- |
| Competency Standard(s) | 0716MSA218. Conduct ADAS & Autonomous Sensor Integration |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name_______________________________________________________ Registration Number_________________________________________ |

Section A1: Guidance for Candidate

| General Guidance | To meet this standard, you are required to - Calibrate Forward-Facing Long-Range Radar (LRR) - Align Blind Spot Monitoring (BSM) Sensors - Program Front View Camera (FVC) Module - Troubleshoot Ultrasonic Park Assist Sensors. - Synchronize LiDAR and Camera Data Streams Activity: A post collision vehicle’s bumper cover and right-side rear bumper section were repaired. Right-side LiDAR and front camera module were replaced. Perform complete calibration, alignment, programming, troubleshooting, and synchronization procedures, ensuring the vehicle is safe and all features function correctly. |
| --- | --- |
| Time | 04 Hours |
| Minimum Evidence Required | During a practical assessment, under observation by an assessor, the candidate (s) is/are required to: - Verify that vehicle is on a level surface and at correct ride height before any sensor alignment - Use OEM-specified target placement (distance, height, and angle) for LRR calibration without guessing - Position the LRR calibration target at exact manufacturer-specified distance and height, connect scan tool, perform static or dynamic calibration, verify DTC clears and radar tracks objects correctly. - Use insulated tools and HV safety procedures while working near HV components during BSM alignment. - Measure vehicle ride height for alignment of BSM sensors, adjust sensor brackets to specified horizontal and vertical angles, use alignment tool or scan tool live data to confirm target detection at rear corners. - Connect FVC module to OEM scan tool, input Vehicle Identification Number (VIN) and program the module, execute surround view or lane departure calibration using checkerboard pattern or road markings, verify camera live view is clear and correctly oriented. - Identify faulty park assist sensor via DTC, perform voltage/signal test at connector, replace sensor, perform system auto-learn or manual teach-in, verify audible and visual park assist function. - Perform a functional test (e.g., walking behind vehicle) to confirm ultrasonic sensor detection after replacement |

Section A2: Self-Assessment Checklist

| Assessment Task | Activity: - A post collision vehicle’s bumper cover and right-side rear bumper section were repaired. Right-side LiDAR and front camera module were replaced. Perform complete calibration, alignment, programming, troubleshooting, and synchronization procedures, ensuring the vehicle is safe and all features function correctly. |
| --- | --- |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Verify that vehicle is on a level surface and at correct ride height before any sensor alignment |  |  |
| - Use OEM-specified target placement (distance, height, and angle) for LRR calibration without guessing |  |  |
| - Position the LRR calibration target at exact manufacturer-specified distance and height, connect scan tool, perform static or dynamic calibration, verify DTC clears and radar tracks objects correctly. |  |  |
| - Use insulated tools and HV safety procedures while working near HV components during BSM alignment. |  |  |
| - Measure vehicle ride height for alignment of BSM sensors, adjust sensor brackets to specified horizontal and vertical angles, use alignment tool or scan tool live data to confirm target detection at rear corners. |  |  |
| - Connect FVC module to OEM scan tool, input Vehicle Identification Number (VIN) and program the module, execute surround view or lane departure calibration using checkerboard pattern or road markings, verify camera live view is clear and correctly oriented. |  |  |
| - Identify faulty park assist sensor via DTC, perform voltage/signal test at connector, replace sensors, perform system auto-learn or manual teach-in, verify audible and visual park assist function. |  |  |
| - Perform a functional test (e.g., walking behind vehicle) to confirm ultrasonic sensor detection after replacement |  |  |

Candidate’s Signature__________________ Assessor’s Signature__________________

Date: _______________________________

Section B: Assessors Judgment Guide

| Qualification | Electric Vehicle (EV) Supervisor L-4 |
| --- | --- |
| Competency Standard(s) | 0716MSA218. Conduct ADAS & Autonomous Sensor Integration |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: __________________________________________________ Registration Number: ________________________ Signature: __________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor_______________________________________ Assessor’s code: __________________ Signature: _______________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  | X |  | X |  |  |
| Knowledge Assessment | X | X |  |  |  |  |  |
| Other Requirement |  |  |  | X |  |  |  |

Section B1: Observation Checklist

| Assessment Task |  | Activity: A post collision vehicle’s bumper cover and right-side rear bumper section were repaired. Right-side LiDAR and front camera module were replaced. Perform complete calibration, alignment, programming, troubleshooting, and synchronization procedures, ensuring the vehicle is safe and all features function correctly. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Verify that vehicle is on a level surface and at correct ride height before any sensor alignment |  |  |  |  |  |
|  | Use OEM-specified target placement (distance, height, and angle) for LRR calibration without guessing |  |  |  |  |  |
|  | Position the LRR calibration target at exact manufacturer-specified distance and height, connect scan tool, performed static or dynamic calibration, verified DTC clears and radar tracks objects correctly. |  |  |  |  |  |
|  | Used insulated tools and HV safety procedures while working near HV components during BSM alignment. |  |  |  |  |  |
|  | Measure vehicle ride height for alignment of BSM sensors, adjusted sensor brackets to specified horizontal and vertical angles, used alignment tool or scan tool live data to confirm target detection at rear corners. |  |  |  |  |  |
|  | Connect FVC module to OEM scan tool, input Vehicle Identification Number (VIN) and programmed the module, executed surround view or lane departure calibration using checkerboard pattern or road markings, verified that camera live view is clear and correctly oriented. |  |  |  |  |  |
|  | Identify faulty park assist sensor via DTC, perform voltage/signal test at connector, replaced sensor, perform system auto-learn or manual teach-in, verified audible and visual park assist function. |  |  |  |  |  |
|  | Perform a functional test (e.g., walking behind vehicle) to confirm ultrasonic sensor detection after replacement |  |  |  |  |  |
| Competent |  |  | Not yet Competent |  |  |  |  |

Section B2: Knowledge Assessment

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
| 1 | What could happen if the BSM) sensor is aligned with the correct horizontal angle but the wrong vertical angle? |  |  |
| 2 | When programming the FVC module, why is it necessary to enter the exact VIN rather than using a generic firmware file? |  |  |
| 3 | How can you distinguish between a faulty ultrasonic sensor and a wiring problem without removing the bumper? |  |  |
| 4 | If the LiDAR and camera timestamps differ by more than 5 milliseconds, what symptom might appear in the fused data output? |  |  |
| 5 | If a front camera module loses Ethernet communication but radar on the same bus works, how would you isolate the fault? |  |  |
| 6 | Why is it unsafe to use a generic radar calibration target instead of the OEM-specified target for a specific vehicle model? |  |  |
| 7 | What symptom indicates a missing or damaged termination resistor in case of CAN bus diagnostics for ADAS? |  |  |
| 8 | What minimum conditions must be met for dynamic radar verification? |  |  |
| 9 | Why the static LRR calibration fail even with a correctly positioned target? |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature ___________________