|  | Assessment Evidence Guide For National Diploma Certificate in Electric Vehicle (EV) Supervisor L-4 Formative Assessment (0716MSA219. Perform HV Safety Isolation & Lockout-Tagout (LOTO)) |
| --- | --- |
|  | National Vocational & Technical Training Commission |

Section A: Instruction Sheet for the Candidate

| Qualification | Electric Vehicle (EV) Supervisor L-4 |
| --- | --- |
| Competency Standard(s) | 0716MSA219. Perform HV Safety Isolation & Lockout-Tagout (LOTO) |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name_______________________________________________________ Registration Number_________________________________________ |

Section A1: Guidance for Candidate

| General Guidance | To meet this standard, you are required to - Establish High-Voltage Exclusion Zone - Remove Switch by Manual Service Disconnect (MSD) - Implement Lockout-Tagout (LOTO) Techniques - Verify Absence of High Voltage (Live-Dead-Live Test) - Discharge Residual High-Voltage Capacitors Activity: EV presented with confirmed high-voltage inverter failure. The vehicle’s HV battery is rated at 400V DC. Perform complete HV safety isolation, including establishing an exclusion zone, removing the Manual Service Disconnect (MSD), applying Lockout-Tagout (LOTO), verifying absence of voltage using Live-Dead-Live test, and discharging residual HV capacitors. |
| --- | --- |
| Time | 04 Hours |
| Minimum Evidence Required | During a practical assessment, under observation by an assessor, the candidate (s) is/are required to: - Place safety fences, warning signs, chains, and cones at least 1 meter around the vehicle and ensure no unauthorized access - Confirm the availability and condition of HV-rated gloves, HV-rated non-conductive footwear and safety hook - Remove all metal jewelry, flammable materials, and place non-conductive work mat - Engage parking brake, transmission in Park/Neutral, Turn OFF ignition, key/fob removed and store - Disconnect EVSE from charge port and check charge port light - Disconnect 12V battery negative terminal - Wear insulated gloves (1000V, Class 0+) and face shield before touching MSD - Locate and remove MSD using insulated puller tool or OEM-specified method - Store removed MSD away from vehicle - Attach personal LOTO lock to MSD - Attach LOTO tag with name, date, and work order - Select CAT III-1000V rated multimeter and Inspect test leads for damage - Perform Live-Dead-Live Test in correct sequence - Identify capacitor discharge points and perform active/passive discharge - Confirm voltage less than 60V DC - Perform repeat Live-Dead-Live test on capacitor terminals to confirm 0V before servicing - Ground the circuit using a grounding rod - Re-confirm HV system de-energization post-work - Inspect HV components for damage - Document MSD removal, LOTO, discharge, and all verification steps in vehicle service record |

Section A2: Self-Assessment Checklist

| Assessment Task | Activity: - EV presented with confirmed high-voltage inverter failure. The vehicle’s HV battery is rated at 400V DC. Perform complete HV safety isolation, including establishing an exclusion zone, removing the Manual Service Disconnect (MSD), applying Lockout-Tagout (LOTO), verifying absence of voltage using Live-Dead-Live test, and discharging residual HV capacitors. |
| --- | --- |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Place safety fences, warning signs, chains, and cones at least 1 meter around the vehicle and ensure no unauthorized access |  |  |
| - Confirm the availability and condition of HV-rated gloves, HV-rated non-conductive footwear and safety hook |  |  |
| - Remove all metal jewelry, flammable materials, and place non-conductive work mat |  |  |
| - Engage parking brake, transmission in Park/Neutral, Turn OFF ignition, key/fob removed and store |  |  |
| - Disconnect EVSE from charge port and check charge port light |  |  |
| - Disconnect 12V battery negative terminal |  |  |
| - Wear insulated gloves (1000V, Class 0+) and face shield before touching MSD |  |  |
| - Locate and remove MSD using insulated puller tool or OEM-specified method |  |  |
| - Store removed MSD away from vehicle |  |  |
| - Attach personal LOTO lock to MSD |  |  |
| - Attach LOTO tag with name, date, and work order |  |  |
| - Select CAT III-1000V rated multimeter and Inspect test leads for damage |  |  |
| - Perform Live-Dead-Live Test in correct sequence |  |  |
| - Identify capacitor discharge points and perform active/passive discharge |  |  |
| - Confirm voltage less than 60V DC |  |  |
| - Perform repeat Live-Dead-Live test on capacitor terminals to confirm 0V before servicing |  |  |
| - Ground the circuit using a grounding rod |  |  |
| - Re-confirm HV system de-energization |  |  |
| - Inspect HV components for damage |  |  |
| - Document MSD removal, LOTO, discharge, and all verification steps in vehicle service record |  |  |

Candidate’s Signature__________________ Assessor’s Signature__________________

Date: _______________________________

Section B: Assessors Judgment Guide

| Qualification | Electric Vehicle (EV) Supervisor L-4 |
| --- | --- |
| Competency Standard(s) | 0716MSA219. Perform HV Safety Isolation & Lockout-Tagout (LOTO) |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: __________________________________________________ Registration Number: ________________________ Signature: __________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor_______________________________________ Assessor’s code: __________________ Signature: _______________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  | X |  | X |  |  |
| Knowledge Assessment | X | X |  |  |  |  |  |
| Other Requirement |  |  |  | X |  |  |  |

Section B1: Observation Checklist

| Assessment Task |  | Activity: EV presented with confirmed high-voltage inverter failure. The vehicle’s HV battery is rated at 400V DC. Perform complete HV safety isolation, including establishing an exclusion zone, removing the Manual Service Disconnect (MSD), applying Lockout-Tagout (LOTO), verifying absence of voltage using Live-Dead-Live test, and discharging residual HV capacitors. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrate the following: |  |  |  | Yes | No | Remarks |
|  | Place safety fences, warning signs, chains, and cones at least 1 meter around the vehicle and ensure no unauthorized access |  |  |  |  |  |
|  | Confirm the availability and condition of HV-rated gloves, HV-rated non-conductive footwear and safety hook |  |  |  |  |  |
|  | Remove all metal jewelry, flammable materials, and place non-conductive work mat |  |  |  |  |  |
|  | Engage parking brake, transmission in Park/Neutral, Turn OFF ignition, key/fob removed and store |  |  |  |  |  |
|  | Disconnect EVSE from charge port and check charge port light |  |  |  |  |  |
|  | Disconnect 12V battery negative terminal |  |  |  |  |  |
|  | Wear insulated gloves (1000V, Class 0+) and face shield before touching MSD |  |  |  |  |  |
|  | Locate and remove MSD using insulated puller tool or OEM-specified method |  |  |  |  |  |
|  | Store removed MSD away from vehicle |  |  |  |  |  |
|  | Attach personal LOTO lock to MSD |  |  |  |  |  |
|  | Attach LOTO tag with name, date, and work order |  |  |  |  |  |
|  | Select CAT III-1000V rated multimeter and Inspect test leads for damage |  |  |  |  |  |
|  | Perform Live-Dead-Live Test in correct sequence |  |  |  |  |  |
|  | Identify capacitor discharge points and perform active/passive discharge |  |  |  |  |  |
|  | Confirm voltage less than 60V DC |  |  |  |  |  |
|  | Perform repeat Live-Dead-Live test on capacitor terminals to confirm 0V before servicing |  |  |  |  |  |
|  | Ground the circuit using a grounding rod |  |  |  |  |  |
|  | Re-confirm HV system de-energization post-work |  |  |  |  |  |
|  | Inspect HV components for damage |  |  |  |  |  |
|  | Document MSD removal, LOTO, discharge, and all verification steps in vehicle service record |  |  |  |  |  |
| Competent |  |  | Not yet Competent |  |  |  |  |

Section B2: Knowledge Assessment

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
| 1 | What are the three general safety precautions in an EV workshop environment? |  |  |
| 2 | How do you inspect a CAT III 1000V multimeter and its test leads to ensure they are safe for HV testing? |  |  |
| 3 | What is poor risk management? |  |  |
| 4 | What are the correct sequence of de-energization steps from vehicle entry to HV system shutdown? |  |  |
| 5 | Why it’s mandatory to follow manufacturer’s manual when removing the Manual Service Disconnect (MSD)? |  |  |
| 6 | After MSD removal and capacitor discharge, what reading is considered safe? |  |  |
| 7 | What information must be documented in the vehicle service record after MSD removal and HV isolation? |  |  |
| 8 | How you would isolate an EV’s HV system and apply Lockout- Tagout (LOTO) after MSD removal? |  |  |
| 9 | Describe three safe HV work practices that must be followed when probing live or recently de-energized HV connectors. |  |  |
| 10 | How do you verify that your test meter is functioning correctly before and after measuring zero voltage on an HV circuit? |  |  |
| 11 | What is the internationally accepted safety limit for capacitor voltage before touching exposed HV terminals, and why is this limit chosen? |  |  |
| 12 | What is the correct procedure to discharge a capacitor bank as per the manufacturer’s manual, and what tool is typically used? |  |  |
| 13 | What PPE is required when working inside an HV exclusion vehicle entry to HV system shutdown? |  |  |
| 14 | Why must you wait for the manufacturer-specified discharge time after MSD removal, and what happens if you skip this wait? |  |  |
| 15 | During zero-voltage testing, you measure 45V DC on a nominal 400V system. Is this safe to touch? Why or why not? |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature ___________________