National Certificate of Electric Vehicle (EV) Assistant Technician  Level-2

Curriculum

National Vocational and Technical Training Commission (NAVTTC)

Government of Pakistan

FOREWORD

National Vocational and Technical Training Commission (NAVTTC) extends its gratitude and appreciation to representatives of business, industry, academia, government agencies, provincial TEVTAs, sector skill councils and trade associations who spared time and extended their expertise for the development of National Diploma Qualification for Electric Vehicle (EV) Assistant Technician Level-2. This work would not have been possible without the technical support of the experts.

It may not be out of place to mention here that all the experts of Industry, Academia and TVET experts of TEVTAs, BTEs and PVTC work diligently for making this qualification worthy and error free for which all credit goes to them. However, NAVTTC accepts the responsibility of all the errors and omissions still prevailing in the Qualification document.

It is also noteworthy that development of Skill Standards is a dynamic and ongoing process, and the developed competency standards needs periodic review and updating owing to the constant technological advancements, development in scientific knowledge, and growing experience of implementation at the grass root level as well as the demand of industry. The NAVTTC, Government of Pakistan will ensure to keep the qualifications abreast with the changing demands of both national and international job markets.

Executive Director (NAVTTC)

Table of Contents

1.	Introduction	5

1.1. Date of Development	5

1.2. Date of Review and Validation	5

1.3. Entry Requirement	5

1.4. Minimum qualification for teachers	5

1.5. Medium of instruction	5

1.6. Duration of Course	5

2.	Scheme of Studies	6

3.	Overview of the Curriculum for Electric Vehicle (EV) Assistant Technician Level-2	7

4.	Detail of Modules.	9

0716MSA191	Perform Basic Computer Operations	9

0716MSA192	Maintain Health, Safety and Environment.	10

0716MSA193	Perform Basic Electrical Measurements	15

0716MSA194	Maintain EV Battery System	17

0716MSA195	Perform Battery Charging	21

0716MSA196	Maintain EVs Motor Drive and Brake System	25

0716MSA197	Maintain EV Controller and Power Electronics	29

0716MSA198	Maintain EVs Thermal and Cooling System	32

0716MSA199	Maintain 2-Wheeler EVs	35

0716MSA1910	Maintain 3-Wheeler EVs	37

0716MSA1911	Maintain 4-Wheeler EVs	40

5.	List of Machines, Tools and Equipment	43

6.	List of Consumables	47

7.	Members of Qualification Development Committee	52

# Introduction

An Electric Vehicle (EV) is any vehicle that uses one or more electric motors for propulsion, drawing power from an energy storage system, most commonly a rechargeable battery. Unlike traditional internal combustion engine (ICE) vehicles that burn fossil fuels like petrol or diesel, EVs use electricity as their primary energy source. Therefore, industry expectations for skilled workforce are also dynamic which can only be managed through setting relevant Modules in collaboration with the leading industries. Being aware of this fact, National Vocational & Technical Training Commission (NAVTTC) developed Modules for Electric Vehicle (EV) under National Vocational Qualifications Framework (NVQF). These competency standards have been developed by a Qualifications Development Committee (QDC) and validated by the Qualifications Validation Committee (QVC) having representation from the Auto Mobile industry of the country.

| ## 1.1. Date of Development | May, 2026 |
| --- | --- |
| ## 1.2. Date of Validation/NQAC | June 18, 2026 |
| ## 1.3. Entry Requirement | Middle or Equivalent |
| ## 1.4. Minimum qualification for teachers | - BSc Engg. /B.E./ B-Tech (4 Years) (Automotive/Mechanical/Electrical/Electronics) with 2 years’ experience in the field of Electric Vehicles. OR - DAE/ DAE Level 5 (Automotive/Mechanical/Electrical/Electronics) with 3 years’ experience in the field of Electric Vehicles. |
| ## 1.5. Medium of instruction | Urdu, local language, English |
| ## 1.6. Duration of Course | 6 Months |

# Scheme of Studies

| Code | Competency Standards | Category | Level | Th | Pr | Total | Cr.H |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Perform Basic Computer Operations | Generic | 2 | 15 | 24 | 39 | 2.7 |
|  | Maintain Health, Safety and Environment | Functional | 2 | 15 | 24 | 39 | 2.7 |
|  | Perform Basic Electrical Measurements | Technical | 2 | 7 | 21 | 28 | 1.8 |
|  | Maintain EV Battery System | Technical | 2 | 7 | 21 | 28 | 1.8 |
|  | Perform Battery Charging | Technical | 2 | 8 | 24 | 32 | 2.0 |
|  | Maintain EVs Motor Drive and Brake system | Technical | 2 | 9 | 24 | 33 | 2.1 |
|  | Maintain EV Controller and Power Electronics | Technical | 2 | 10 | 30 | 40 | 2.5 |
|  | Maintain EVs Thermal and Cooling System | Technical | 2 | 10 | 30 | 40 | 2.5 |
|  | Maintain 2-Wheeler EVs | Technical | 2 | 10 | 30 | 40 | 2.5 |
|  | Maintain 3-Wheeler EVs | Technical | 2 | 10 | 30 | 40 | 2.5 |
|  | Maintain 4-Wheeler EVs | Technical | 2 | 10 | 30 | 40 | 2.5 |
| On Job Training (OJT) |  |  |  | 0 | 180 | 180 | 4.5 |
| Total |  |  |  | 111 | 468 | 579 | 30.0 |

Daily Contact Hours (For Reference)

| Daily Contact Hours adopt by Institute | Duration | Note: - For institute offering 6 days in a week. The Weekend institute may use formula: Credit hours/Daily hours = Duration in Months |
| --- | --- | --- |
| 4 | 11 months |  |
| 5 | 09 months |  |
| 6 | 08 months |  |
| 8 | 06 months |  |

# Overview of the Curriculum for Electric Vehicle (EV) Assistant Technician Level-2

| Module Title | Learning Units | Theory hours | Workplace hours | Timeframe of modules |
| --- | --- | --- | --- | --- |
| Perform Basic Computer Operations | - Operate MS word . - Operate MS Excel . - Operate MS Power Point . - Perform Browsing . - Print documents | 15 | 24 | 39 |
| Maintain Health, Safety and Environment | - Follow Health and Safety protocols . - Maintain fire safety measures . - Identify HV risks in 2W to 4W EVs . - Handle hazardous materials safely . - Follow emergency measures - Maintain PPEs for EV works . - Apply waste disposal system . - Maintain personal hygiene and discipline | 15 | 24 | 39 |
| Perform Basic Electrical Measurements | - Operate electrical measuring instruments . - Measure electrical Quantities along with its units. - Construct simple series and parallel circuit . - Detect high & Low voltage components in EV . - Detect AC/DC wire cables & Insolation resistance - Diagnose basic wiring faults | 7 | 21 | 28 |
| Maintain EV Battery System | - Adopt Safety Measures Before Handling EV Battery - Identify Basic EV Types (BEV, HEV, PHEV, REEV) - Label various types of EV Battery - Identify Battery Pack of 2 to 4-wheeler EVs - Identify Battery Terminals and Connectors - Secure Battery Mounting - Measure Battery Voltage with Multimeter | 7 | 21 | 28 |
| Perform EV Battery Charging | - Identify Charging Station Components (Connector, Display, Cable) - Identify EV Charging Port - Connect & Disconnect Charging Cable Safely - Identify EV AC Charging and DC Fast Charging - Identify Charging Status Indicators in 2-Wheeler to 4-Wheeler EV - Identify Charging System of 2 to 4-Wheeler EVs - Detect Charging Faults in EVs - Keep Charging Records | 8 | 24 | 32 |
| Maintain EVs Motor Drive and Brake system | - Identify Components of Motor Drive System - Detect hub motor in 02 to 03-Wheeler EVs - Identify Traction Motor in 04-Wheeler EVs - Detect Signs of Motor Fault - Identify basic regenerative system - Identify Braking System in 2-Wheeler and 4-Wheeler EVs | 9 | 24 | 33 |
| Maintain EV Controller and Power Electronics | - Measure Power Electronics Fundamentals - Apply Motor Controlling Techniques - Implement EV Controller Policy - Identify Power Electronic Components - Identify EV Controller of 2-4-wheeler - Interpret accelerator signals in EV system | 10 | 30 | 40 |
| Maintain EVs Thermal and Cooling System | - Manage Thermal Systems in EV - Identify EV cooling system and Types - Identify components of cooling system - Identify liquid cooling system - Identify Air Cooling System of HVAC - Identify EV overheating signs | 10 | 30 | 40 |
| Maintain 2-Wheeler EVs | - Check Battery Pack and Charging System - Maintain Electric Motor and Drivetrain System - Service Thermal Management and Cooling System - Inspect Braking, Suspension, and Tire System - Perform Electrical Safety and Software Maintenance | 10 | 30 | 40 |
| Maintain 3-Wheeler EVs | - Check Battery and Charging System of 3-Wheeler - Maintain Motor and Powertrain System - Maintain Brake and Suspension System of 3-Wheeler - Check Body, Tires, and Chassis Components of 3-Wheeler - Interpret faults signs on cluster of 3-Wheeler | 10 | 30 | 40 |
| Maintain 4-Wheeler EVs | - Maintain Battery Pack and Charging System of 4-Wheel - Maintain Electric Motor and drivetrain System of 4-Wheel - Identify HVAC of 4-Wheel - Inspect Suspension, steering, Brake, and Tire of 4-Wheel - Interpret fault codes on cluster of 4-Wheel | 10 | 30 | 40 |
|  | On Job Training (OJT) | 0 | 180 | 180 |
| Total |  | 111 | 468 | 579 |

# Detail of Modules.

- Perform Basic Computer Operations

Objective: This module covers the skills and knowledge required to Operate MS word, Operate MS Excel, Operate MS Power Point, Perform Browsing and Print Document.

Theory: 15 Hours						Practice: 24 Hours`	 				Duration: 39 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Operate MS word | - Perform Microsoft basic commands in MS word - Open File - Format a file - Font (Type/size/bold/Italic) - Header Footer - Page number - Insert pics / table/hyperlink - Save a File - Save a folder | - Computer and Basic parts of computers - Operating System & its types - Importance and uses of MS Word Practical Activity - Typing Practice - Tab knowledge (Draw tables, formatting, Design, Layout Documents) | Theory-03Hrs Practical-06Hrs Total- 09Hrs |  | Class Room / Lab |
| Operate MS Excel | - Perform basic commands in Microsoft MS Excel - Open a worksheet - Sum functions - IF functions - Basic calculations - Table and graphs - Save a worksheet/folder | - Importance and uses of MS Excel Practical Activity - Mark sheet Design - Calculation formulas - Make table with all formulas master sheet | Theory-03Hrs Practical-09Hrs Total- 12Hrs |  | Class Room / Lab |
| Operate MS Power Point | - Prepare Microsoft power point presentation with basic commands - Make a power point file - Insert pics/table/hyperlink - Design a theme for slides - Save a power point file | - Importance and uses of MS Power Point Practical activity: - Design sheets - Insert and Delete Sheet - Design profile for organization and make PPT - Make PPT with proper video and theme. | Theory-03Hrs Practical-03Hrs Total- 06Hrs |  | Class Room / Lab |
| Perform Browsing | - Perform browsing on the internet as per needs - Perform search online on new trends in the market with the help of internet. - Creating Email Account - Compose Email | - Importance of Internet. - Use of various search engines like Google, U-tube etc. Practical activity: - Internet browsing methodology - Make Account on social media and create ID’S & compose email | Theory-03Hrs Practical-03Hrs Total- 06Hrs |  | Class Room / Lab |
| Print Document | - Select Printer - Select page setup - Print relevant pages | - Printing Procedure. Practical activity: - Print Document | Theory-03Hrs Practical-03Hrs Total- 06Hrs |  | Lab |

- Maintain Health, Safety and Environment.

Objective:  This module covers the knowledge and  skills required to Follow health and safety protocols, Maintain fire safety measures, Identify HV risks in 2W to 4W EVs, Handle hazardous materials Safely, Follow emergency measures, Maintain PPEs for EV works, Apply waste disposal system and Maintain personal hygiene and discipline

Theory: 10 Hours	Practice:  30 Hours 					Duration: 40 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Follow health and safety protocols | - Identify hazards at workplace during daily inspection - Wear PPE according to safety guidelines - Report incidents to supervisor immediately - Place warning signs near hazardous areas properly - Maintain safety records as per SOP | - Workplace Hazard Identification - Personal Protective Equipment (PPE) - Incident Reporting Procedures - Safety Signs and Hazard Warnings - Safety Documentation and SOP Compliance Practical Activity Perform Workplace Hazard Identification and Safety Compliance | Theory-01Hrs Practical-03Hrs Total- 04Hrs | - PPE kit (helmet, gloves, mask, safety shoes) - Workplace inspection checklist - Safety signs and warning labels - Hazard reporting form - Safety logbook - Pen and clipboard - Flashlight or inspection lamp - Barricade tape or safety cones - First aid box - Fire extinguisher | Classroom and workshop |
| Maintain fire safety measures | - Check fire extinguishers for validity regularly - Place firefighting equipment at designated locations - Inspect emergency exits for accessibility daily - Report faulty safety equipment to supervisor promptly - Conduct basic fire drills under supervision | - Fire Extinguisher Inspection - Firefighting Equipment Placement - Emergency Exit Inspection - Faulty Safety Equipment Reporting - Basic Fire Drill Practices Practical Activity Perform Fire Safety Inspection and Emergency Drill | Theory-02Hrs Practical-06Hrs Total- 08Hrs | - Fire extinguishers - Fire alarm system - Fire safety signs and labels - Emergency evacuation map - Inspection checklist | Classroom and workshop |
| Identify HV risks in 2W to 4W EVs | - Inspect high-voltage components as per safety guidelines. - Detect exposed cables and connectors during visual safety checks. - Assess battery pack condition for leakage, swelling, or damage. - Verify isolation status before starting maintenance activities. - Report identified hazards to supervisor as per SOP | - High-Voltage Component Inspection - Cable and Connector Safety Checks - Battery Pack Condition Assessment - Isolation Verification Procedures - Hazard Reporting as per SOP Practical Activity Perform High-Voltage Component and Battery Safety Inspection | Theory-01Hrs Practical-03Hrs Total- 04Hrs | - Insulated gloves - Safety helmet - Face shield - Safety shoes - Insulated tools - High-voltage warning signs - Battery pack or training model - Inspection checklist - Hazard reporting form - Lockout/Tagout (LOTO) kit - Safety barriers or cones |  |
| Handle hazardous materials Safely | - Store chemicals in labelled containers properly - Dispose waste according to environmental regulations - Handle battery chemicals Safely Using gloves - Clean spills using appropriate materials safely - Report leakage incidents immediately | - Chemical Storage Procedures - Waste Disposal Regulations - Safe Handling of Battery Chemicals - Spill Cleaning Procedures - Leakage Incident Reporting Practical Activity Perform Chemical Storage and Spill Handling Procedures | Theory-02Hrs Practical-06Hrs Total- 08Hrs | - Labelled chemical containers - Safety gloves - Safety goggles - Face mask - Protective apron - Spill control kit - Absorbent pads or sand - Waste disposal bins - Hazard warning signs - Cleaning cloths and brushes - Battery chemical handling tools - Incident reporting form - First aid box |  |
| Follow emergency measures | - Identify emergency exits during shift start - Inform emergency services during incidents quickly - Assist evacuation during emergency situations - Follow supervisor instructions during crises - Maintain calm behavior during emergencies | - Emergency Exit Identification - Emergency Communication Procedures - Evacuation Assistance Practices - Crisis Response Instructions - Emergency Behavior and Discipline Practical Activity Perform Emergency Response and Evacuation Procedures | Theory-01Hrs Practical-03Hrs Total- 04Hrs | - Emergency evacuation map - Emergency alarm or whistle - Communication device (phone/intercom) - Exit signs and safety indicators - PPE kit - Safety cones or warning tape - Flashlight or emergency light - Incident reporting form - Attendance or evacuation checklist | Classroom and workshop |
| Maintain PPEs for EV works | - Dress personal protective equipment before each work assignment. - Clean insulated gloves and face shields with approved materials. - Store protective equipment in designated dry storage areas. - Replace damaged PPEs immediately according to safety standards. - Record PPE inspection details in maintenance logbooks regularly. | - PPE Wearing Procedures - Cleaning of Protective Equipment - PPE Storage Requirements - Damaged PPE Replacement - PPE Inspection Recordkeeping Practical Activity Perform PPE Wearing, Inspection, and Maintenance Procedures | Theory-01Hrs Practical-03Hrs Total- 04Hrs | - Safety helmet - Safety gloves - Safety goggles - Face mask - Safety shoes - Reflective vest or protective clothing - Cleaning cloths - Approved cleaning solution - PPE storage cabinet or rack - Inspection checklist - PPE maintenance logbook - Damaged PPE samples - Waste disposal bin | Classroom and workshop |
| Apply waste disposal system | - Segregate waste materials into categories according to disposal policy - Collect hazardous waste through labelled containers safely - Store recyclable materials in designated area properly - Transport waste to disposal points following safety measures - Record waste disposal activities in logbook as per SOP | - Waste Segregation Methods - Hazardous Waste Collection - Recyclable Material Storage - Safe Waste Transportation - Waste Disposal Recordkeeping Practical Activity Perform Waste Segregation and Disposal Procedures | Theory-01Hrs Practical-03Hrs Total- 04Hrs | - Labelled waste bins - Hazardous waste containers - PPE kit (gloves, mask, apron, safety shoes) - Trolley or waste transport cart - Recyclable material storage containers - Waste segregation labels - Cleaning materials |  |
| Maintain personal hygiene and discipline | - Wear clean uniform during duty hours as per workplace rules - Wear protective equipment properly according to task requirements - Wash hands before and after work with hygiene standards - Follow workplace discipline policies during working hours - Report health issues to supervisor as per SOP | - Workplace Uniform Requirements - Proper Use of Protective Equipment - Personal Hygiene Practices - Workplace Discipline Procedures - Health Issue Reporting Procedures Practical Activity Perform Workplace Health, Safety, and Environment Inspection | Theory-01Hrs Practical-03Hrs Total- 04Hrs | - PPE kit (helmet, gloves, mask, safety shoes) - Workplace inspection checklist - Safety warning signs - Fire extinguisher - Inspection logbook - Pen/clipboard - Hazard reporting form - Flashlight or inspection lamp | Classroom and workshop |

- Perform Basic Electrical Measurements

Objective:  This module covers the knowledge and  skills required to to Operate electrical measuring instruments, Measure electrical Quantities along with its units, Construct simple series and parallel circuit, Detect high & Low voltage components in EV, Detect AC/DC wire cables & Insolation resistance and Diagnose basic wiring faults

Theory: 07Hours	Practice:  21Hours 					Duration:  28Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Operate electrical measuring instruments | - Set multimeter for voltage, current, and resistance measurement. - Apply clamp meter for non-contact current testing. - Select insulation tester according to insulation resistance requirements. - Separate oscilloscope features for waveform analysis purposes. - Verify instrument condition before performing electrical measurements. | - Multimeter Settings for Electrical Measurements - Using Clamp Meter for Non-Contact Current Testing - Oscilloscope Features for Waveform Analysis Practical Activity Using multimeter for performing voltage, current, and resistance measurements | Theory-01Hrs Practical-03Hrs Total- 04Hrs | - Electrical Wires - Resistors - Insulating Tape - Circuit Component | Classroom and workshop |
| Measure electrical Quantities along with its units. | - Measure voltage values in volts with meter settings. - Measure current flow in amperes through correct circuit connection. - Measure resistance values in ohms with resistance mode. - Record power readings in watts according to test results. - Confirm frequency values in hertz during AC supply checks. | - Voltage, Current, and Resistance measurement - Power and Frequency Measurement - Recording Electrical Quantities and Units Practical Activity Perform and measure Electrical Quantities Along with Its Units. | Theory-01Hrs Practical-03Hrs Total- 04Hrs | - Electrical Wires - Resistors - Insulating Tape | workshop |
| Construct simple series and parallel circuit | - Assemble series circuit components according to circuit diagram. - Connect parallel circuit branches following electrical design. - Install resistors and loads with proper wiring methods. - Test circuit continuity before applying power supply. - Verify circuit operation under safe working conditions. | - Series Circuit Assembly - Parallel Circuit Connection - Circuit Continuity and Verifying Circuit Operation Practical Activity Perform Construction and Testing of Series and Parallel Circuits. | Theory-02Hrs Practical-06Hrs Total- 08Hrs | - Electrical Wires - Resistors - Insulating Tape - Circuit Component | Classroom and workshop |
| Detect high & Low voltage components in EV | - Identify high-voltage battery components through EV system layout. - Locate low-voltage accessories with vehicle wiring diagram. - Separate HV cables by color coding standards. - Recognize DC-DC converter during component inspection. - Verify component voltage classification according to manufacturer specifications. | - Identification of High-Voltage Battery Components - Low-Voltage Accessory Location in EV Wiring - HV Cable Color Coding and Voltage Classification Practical Activity Perform Identification and Verification of High & Low Voltage Components in EV Systems | Theory-01Hrs Practical-3Hrs Total- 04Hrs | - Insulation Tape - Circuit Testing Fuses - Testing Wire - Component Identification Tags | workshop |
| Detect AC/DC wire cables & Insolation resistance | - Identify AC and DC cables through insulation markings. - Check cable insulation condition before electrical testing. - Measure insulation resistance with approved insulation tester. - Identify damaged cables through resistance test results. - Record cable specifications according to SOP | - Identification of AC and DC Cables by Insulation Markings - Insulation Condition Before Testing - Insulation Resistance Measurement and Damage Identification Practical Activity Identify the AC/DC cable insulation, check its condition, and test its functionality. | Theory-01Hrs Practical-03Hrs Total- 04Hrs | - Testing Wire - Component Identification Tags - Resistance Test Standards sheets | workshop |
| Diagnose basic wiring faults | - Check wiring connections for looseness or damage. - Detect short circuits with continuity testing methods. - Identify open circuits through resistance measurement. - Trace faulty wiring paths with diagnostic tools. - Report wiring defects according to maintenance procedures. | - Wiring Connection Check for Looseness - Short Circuit Detection Using Continuity Testing - Open Circuit Identification and Fault Reporting Practical Activity Diagnose and Report Basic Wiring Faults. | Theory-01Hrs Practical-03Hrs Total- 04Hrs |  | Classroom and workshop |

- Maintain EV Battery System

Objective:  This module covers the knowledge and skills required to Adopt Safety Measures Before Handling EV Battery, Identify Basic EV Types (BEV, HEV, PHEV, REEV), Label various types of EV Battery, Identify Battery Pack of 2 to 4-wheeler EVs, Identify Battery Terminals and Connectors, Secure Battery Mounting and Measure Battery Voltage with Multimeter

Theory: 07Hours	Practice:  21Hours 					Duration:  28Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Adopt Safety Measures Before Handling EV Battery | - Wear insulated PPE before touching battery components. - Disconnect power source with approved isolation Techniques - Check work area for electrical and fire hazards. - Apply insulated tools during battery handling operations. - Place warning signs around EV maintenance zone. | - Importance of Isolation. Before EV Battery Work. - Safe Battery Handling procedure. Practical Activity Demonstrate safety steps in the correct order before starting EV battery work. | Theory-01Hrs Practical-03Hrs Total- 04Hrs | - Insulated PPE - Fire extinguisher - Insulated tools. - warning signs. | Classroom and workshop |
| Identify Basic EV Types (BEV, HEV, PHEV, REEV) | - Separate EV types based on powertrain design. - Identify battery-only system in BEV models. - Recognize hybrid engine-battery system in HEV models. - Identify plug-in charging system in PHEV models. - Categorize vehicle type with manufacturer specifications - Identify Major Components of REEV | - EV Powertrain Identification. - EV System Recognition & Classification. Practical Activity Perform plug-in charging. | Theory-01Hrs Practical-03Hrs Total- 04Hrs | - BEV – Battery-only - HEV – Engine + battery - PHEV – Plug-in hybrid - REEV – Range extender | Classroom and workshop |
| Label various types of EV Battery | - Recognize lithium-ion battery by high energy density. - Identify NiMH battery through hybrid vehicle application. - Separate lead-acid battery by low-cost design. - Compare battery types using voltage and capacity ratings. - Label battery chemistry according to manufacturer markings | - Battery Types - Compare & Classify Battery Systems Practical Activity Identify and match each battery type with its key features and labels. | Theory-01Hrs Practical-03Hrs Total- 04Hrs | - Lithium‑ion, High energy - NiMH, Hybrid use - Lead‑acid battery | Classroom and workshop |
| Identify Battery Pack of 2 to 4-Wheeler EVs | - Locate battery pack position in 2-wheeler EVs. - Locate battery pack position in 4-wheeler EVs. - Identify battery pack casing for physical protection. - Recognize battery module arrangement within pack. - Verify pack type using vehicle service manual | - Locate Battery Pack - Identify Battery Structure Practical Activity Locate EV battery pack position, casing, and module layout. | Theory-01Hrs Practical-03Hrs Total- 04Hrs | - BEV – Battery-only - HEV – Engine + battery - PHEV – Plug-in hybrid - REEV – Range extender - EV Bike/Scooter | Classroom and workshop |
| Identify Battery Terminals and Connectors | - Locate positive terminal using polarity marking. - Locate negative terminal using polarity marking. - Identify high-voltage connectors by color coding. - Check terminal condition for corrosion signs. - Verify connector locking mechanism for secure fitment | - Terminals & Connectors - High-voltage color coding. Practical Activity Inspect terminals, connectors, corrosion, and locking by using markings and visual inspection. | Theory-01Hrs Practical-03Hrs Total- 04Hrs | - Red mark for Positive (+) - Black mark for Negative (−) - Orange mark for HV Connectors or high‑voltage - Contact Cleaner | Classroom and workshop |
| Secure Battery Mounting | - Check battery brackets for secure attachment. - Tighten mounting bolts according to torque specification. - Check battery alignment within compartment frame. - Verify vibration pads for proper installation. - Confirm battery pack stability. | - Battery Mounting - Battery Pack Practical Activity Perform a secure battery pack by checking brackets and torque bolts. | Theory-01Hrs Practical-03Hrs Total- 04Hrs | - Insulated PPE - Insulated tools. | Classroom and workshop |
| Measure Battery Voltage with Multimeter | - Set multimeter to correct DC voltage range. - Connect test probes to battery terminals safely. - Measure battery voltage under no-load condition. - Record voltage reading according to specification. - Compare measured voltage with manufacturer standard. | - Battery Voltage by types. - Record & Compare Voltage Practical Activity Set multimeter and measure the battery voltage. | Theory-01Hrs Practical-03Hrs Total- 04Hrs | - Insulated PPE - Insulated multimeter. - Data record sheet. | Classroom and workshop |

- Perform EV Battery Charging

Objective:  This module covers the knowledge and  skills required to Identify Charging Station Components (Connector, Display, Cable), Identify EV Charging Port, Connect & Disconnect Charging Cable Safely, Identify EV AC Charging and DC Fast Charging, Identify Charging Status Indicators in 2-Wheeler to 4-Wheeler EV, Identify Charging System of 2 to 4-Wheeler EVs, Detect Charging Faults in EVs and Keep Charging Records

Theory: Hours: 08				Practice:  Hours: 24					Duration:  Hours:32

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Identify Charging Station Components (Connector, Display, Cable) | - Identify charging connector by plug design type. - Recognize charging display for operational information. - Check charging cable for insulation condition. - Identify emergency stop button on charging station. - Verify charging station components using user manual | - Charging Connectors by Plug Design Type. - Charging Display Operational Information - Charging Cable Inspection - Locating the Emergency Stop Button Practical Activity Inspect charging station components with OEM. | Theory-01Hrs Practical-03Hrs Total- 04Hrs | - EV charging station - Training unit - Different charging connector types - Charging cables - (PPE) - Inspection checklist - Marker and notepad | Classroom and workshop |
| Identify EV Charging Port | - Locate charging port on 2-wheeler EV body. - Locate charging port on 4-wheeler EV body. - Identify charging port type by connector shape. - Check charging port cover for physical damage. - Verify charging port compatibility with charger type | - Information Charging Ports on 2- 4Wheeler EVs - Locating Charging Ports on 2-4Wheeler EVs - Type of Charging Ports - Inspecting Charging Port Practical Activity Inspect EV charging ports for physical damage using manufacturer guidelines. | Theory-01Hrs Practical-03Hrs Total- 04Hrs | - Training units - EV chargers’ types - Inspection checklist - (PPE) - Flashlight - Notepad and marker | Classroom and workshop |
| Connect & Disconnect Charging Cable Safely | - Switch off vehicle before connecting charger. - Insert charging connector into port securely. - Confirm charging initiation through system indicator. - Disconnect charging cable after charging completion. - Store charging cable properly after use. | - Switching Of Charging System - Connecting the Charging Connector - Charging System Indicators - Disconnecting the Charging Cable - Storage of Charging Cable Practical Activity Perform connect and disconnect the charging cable securely | Theory-01Hrs Practical-03Hrs Total- 04Hrs | - Training unit) - EV charging cable - User manual - (PPE) - Inspection checklist t - Cable storage rack - Notepad and pen | Classroom and workshop |
| Identify EV AC Charging and DC Fast Charging | - Recognize AC charging by onboard charger practice. - Identify DC fast charging by direct battery supply. - Separate charging speed between AC and DC systems. - Verify charging method through charger specification. - Select appropriate charging mode for vehicle type. | - AC Charging On-board Charger Operation - DC Fast Charging - Comparing Charging Speeds of AC and DC Systems - Charging Methods Using Charger Specifications Practical Activity Perform charging method through charger specification | Theory-01Hrs Practical-03Hrs Total- 04Hrs | - Training unit - AC charging station - DC fast charging unit - Charger specification sheets - Comparison chart - PPE - Notepad and pen - SOP for charging modes | Classroom and workshop |
| Identify Charging Status Indicators in 2-Wheeler to 4-Wheeler EV | - Detect charging light color during charging process. - Identify full-charge indicator on dashboard display. - Recognize fault warning through error symbols. - Monitor charging progress with battery percentage display. - Verify charging status through vehicle manual guide. | - Detecting Charging Indicator - Full-Charge Indicators on Dashboard Display - Using of Charging Status Guidelines Practical Activity Monitor charging progress with battery percentage display. | Theory-01Hrs Practical-03Hrs Total- 04Hrs | - Training unit - AC charging station - DC fast charging unit - Charger specification sheets - Comparison chart - PPE - Notepad and pen - SOP for charging modes | Classroom and workshop |
| Identify Charging System of 2 to 4-Wheeler EVs | - Identify onboard charger in 2-wheeler EV system. - Identify charging controller in 4-wheeler EV system. - Locate charging inlet in vehicle structure. - Recognize battery management interface during charging. - Verify charging system components using technical manual | - On-board Charger in 2-Wheeler EV Systems - Charging Controller in 4-Wheeler EV Systems - Charging System Components Using Technical Manuals Practical Activity Inspect charging inlet in vehicle structure. | Theory-01Hrs Practical-03Hrs Total- 04Hrs | - Training unit - AC and DC charging station - Charger specification sheets - Comparison chart - PPE - Notepad and pen - SOP for charging modes | Classroom and workshop |
| Detect Charging Faults in EVs | - Check charger connection for loose fitment. - Identify charging interruption through warning alerts. - Check cable condition for physical damage. - Detect charging port faults using visual inspection. - Report charging system errors according to procedure | - Charger Connection for Loose Fitment - Charging Interruptions Through Warning Alerts - Cable Condition for Physical Damage - Reporting Charging System Errors Practical Activity Inspect EV charging setup by checking connector fitment, cable and charging port condition | Theory-01Hrs Practical-03Hrs Total- 04Hrs | - Training unit - EV charging station - Charging cables and connectors - Warning indicators display - Inspection checklist - PPE - Flashlight, - Notepad and pen | Classroom and workshop |
| Keep Charging Records | - Record charging date in maintenance logbook. - Note charging duration for battery monitoring. - Document charging type used for each session. - Record battery status before and after charging. - Maintain charging history for service reference. | - Charging Maintenance Logbook - Documenting Charging Type Used in Each Session - Recording Battery Status Before and After Charging - Maintaining Charging History for Service Reference Practical Activity Record charging history for service reference. | Theory-01Hrs Practical-03Hrs Total- 04Hrs | - Maintenance logbook - Electric vehicle (2-wheeler or 4-wheeler training unit) - EV charging station - Pen, marker, or tablet for data entry | Classroom and workshop |

- Maintain EVs Motor Drive and Brake System

Objective:  This module covers the knowledge and skills required to Identify Components of Motor Drive System, Detect hub motor in 02 to 03-Wheeler EVs, Identify Traction Motor in 04-Wheeler EVs, Detect Signs of Motor Fault, Identify basic regenerative system and Identify Braking System in 2-Wheeler and 4-Wheeler EVs

Theory: 09Hours	Practice: 24Hours 			Duration: 33Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Identify Components of Motor Drive System | - Identify electric motor within drive assembly. - Locate motor controller in drive system. - Identify inverter for power conversion function. - Identify transmission linkage in drivetrain layout. - Verify drive system components using service manual | - EV Drivetrain Architecture for Light Vehicles - Layout Types: Hub Motor, Mid-Drive motor, Axle-mounted motor - Transmission Linkage in Drive Trains - Electric Motor Types and Identification - Motor Controller and Inverter: Function, Appearance, Location and Difference - Service Manual Interpretation - Safety Precautions Practical Activity Identify drive system components using OEM service manual | Theory-02Hrs Practical-3Hrs Total- 5Hrs | - Cleaning spray - Electrical insulation tape - Heat shrink tubing - Cable ties - Hydraulic seals and O-rings - Shop rags / wiping cloths | Classroom and workshop |
| Detect hub motor in 02 to 03-Wheeler EVs | - Identify hub motor in 2-3-wheeler EVs. - Locate hub motor in 2-3-wheeler EVs - Check motor casing for identification markings - Separate motor types by construction design. - Inspect motor label for technical specifications. | - Hub Motor - Motor Classification based on construction design - Motor Label Interpretation - Categorization of motor application based on vehicle type Practical Activity - Identify the motor, its type based on construction and application - Write its technical specifications from the label | Theory-01Hrs Practical-03Hrs Total- 04Hrs | - Copper grease - Grease for bearings | Classroom and workshop |
| Identify Traction Motor in 04-Wheeler EVs | - Identify traction motor in 4-wheeler EVs - Locate traction motor in 4-wheeler - Check motor casing for identification markings - Inspect motor label for technical specifications - Follow basic safety while identifying motors | - Traction Motor - Motor construction, external identification and types - Comparison of hub and traction motor - Motor Label interpretation Practical Activity Locate and identify the motor type | Theory-01Hrs Practical-03Hrs Total- 04Hrs | - Electrical insulation tape - Heat shrink tubing - Cable ties - Copper grease - Grease for bearings - Hydraulic seals and O-rings - Shop rags / wiping cloths | Classroom and workshop |
| Detect Signs of Motor Fault | - Identify abnormal sound from motor - Observe loss of vehicle performance - Detect overheating of motor - Identify unusual electrical signs - Take initial action on motor fault signs | - Normal and abnormal motor sounds - Types and causes of abnormal sounds - Loss of vehicle performance: Signs and Causes - Methods to detect overheating of motor - Observation methods to spot unusual electrical signs - Emergency actions on the detection of motor fault Practical Activity Identify electrical fault in the 2-4 W EV motor and perform the initial action | Theory-01Hrs Practical-03Hrs Total- 04Hrs | - Electrical insulation tape - Heat shrink tubing - Cable ties - Copper grease - Grease for bearings - Hydraulic seals and O-rings - Shop rags / wiping cloths | Classroom and workshop |
| Identify basic regenerative system | - Identify regenerative braking components - Describe regenerative braking function - Observe energy recovery process - Differentiate regenerative and conventional braking - Check regenerative system operation | - Regenerative Braking System - Component Identification & Location - Functional Description & Energy Recovery Process Practical Activity Verify the performance of energy recovery process of the regenerative braking system | Theory-02Hrs Practical-06Hrs Total- 08Hrs | - Cleaning spray for brake parts - Electrical insulation tape - Heat shrink tubing - Cable ties - Copper grease - Grease for bearings - Brake cleaner spray - Hydraulic seals and O-rings - Shop rags / wiping cloths | Classroom and workshop |
| Identify Braking System in 2-Wheeler and 4-Wheeler EVs | - Identify disc brake system in EV models. - Identify drum brake system in applicable vehicles. - Identify regenerative braking system function. - Locate brake pedal or lever mechanism. - Verify braking system type with vehicle manual. | - Comparison of conventional and regenerative braking system - Identification methods of disc and drum brake system - Brake pedal and lever mechanism - Interpretation of OEM manual for braking system specifications Practical Activity Identify the braking system based on components in the provided vehicle. | Theory-02Hrs Practical-06Hrs Total- 08Hrs | - Brake fluid (DOT 3 / DOT 4 as per vehicle) - Cleaning spray for brake parts - Shop rags / wiping cloths - Hydraulic seals and O-rings - Grease for bearings | Classroom and workshop |

- Maintain EV Controller and Power Electronics

Objective: This module covers the knowledge and skills required to Measure Power Electronics Fundamentals, Apply Motor Controlling Techniques, Implement EV Controller Policy, Identify Power Electronic Components, Identify EV Controller of 2-4-wheeler and Interpret accelerator signals in EV system

Theory: 10 Hours					Practice: 30 Hours						Duration: 40 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Measure Power Electronics Fundamentals | - Identify basic components - Clarify working of components - Perform basic power conversion - Identify losses and heating - Recognize switching operation | - Basic power electronic components in EVs - Switching operation in power devices - Heat generation in electronic circuits - Power losses in EV systems Practical Activity Perform power conversion and switching operation in a simple EV circuit. | Theory-02Hrs Practical-03Hrs Total- 05Hrs | - Electrical insulation tape - Electrical wires - Wire connectors - Heat shrink tubing | Classroom and workshop |
| Apply Motor Controlling Techniques | - Recognize motor control - Identify motor types - Recognize basic motor operation - Recognize commutation - Control motor direction - Identify controller role | - Motor types used in EV systems - Techniques of Motor rotation, speed control - Commutation and Controller role - Techniques of Controller Switching sequence Practical Activity Operate a BLDC motor in forward and reverse direction. | Theory-02Hrs Practical-06Hrs Total- 08Hrs | - Electrical insulation tape - Electrical wires - Wire connectors - Crimp terminals | Classroom and workshop |

| Implement EV Controller Policy | - Identify system inputs and outputs - Clarify controller working - Integrate sensors correctly - Implement protection logic - Recognize communication system | - Controller input and output signals - Sensor interfacing methods - Protection logic in controllers - Communication between EV modules - Signal processing in EV controller Practical Activity Connect a sensor to a controller and observe inputs outputs | Theory-02Hrs Practical-06Hrs Total- 08Hrs | - Electrical wires - Wire connectors - Crimp terminals - Heat shrink tubing - Solder wire | Classroom and workshop |
| --- | --- | --- | --- | --- | --- |
| Identify Power Electronic Components | - Correct identification of components - Recognize basic function - Identify power devices - Recognize circuit roles - Apply Safe handling Techniques | - Identification of power devices - Terminal and polarity recognition - Function of electronic components - Circuit placement of components - Safe handling of electronic devices Practical Activity Identify power electronic components on a circuit board. | Theory-01Hrs Practical-03Hrs Total- 04Hrs | - Electrical insulation tape - Wire connectors | Classroom and workshop |
| Identify EV Controller of 2-4-wheeler | - Measure input voltage with calibrated testing equipment. - Verify output signals according to system specifications. - Check continuity of controller circuits before energizing. - Assess sensor connections for correct electrical response. - Compare test readings with manufacturer reference values | - Input voltage and output signals measurement - Circuit continuity identification - Sensor response verification - Comparison with standard values Practical Activity Measure controller voltage and verify output signals. | Theory-01Hrs Practical-06Hrs Total- 07Hrs | - Electrical wires - Wire connectors - Crimp terminals - Electrical insulation tape | Workshop |
| Interpret accelerator signals in EV system | - Identify accelerator pedal sensor in EV system as input device - Recognize accelerator position signal sent to controller as voltage or digital signal - Identify how throttle signal controls motor speed through controller - Check basic wiring and connector of accelerator signal circuit - Identify safety function that limits power if accelerator signal is abnormal | - Accelerator pedal sensor identification - Voltage and digital signal recognition - Throttle signal operation - Wiring and connector checking - Safety response during signal fault Practical Activity Test accelerator signal - operation using diagnostic equipment. | Theory-02Hrs Practical-06Hrs Total- 08Hrs | - Electrical wires - Wire connectors - Electrical insulation tape | Workshop |

- Maintain EVs Thermal and Cooling System

Objective:  This module covers the knowledge and  skills required to to Manage Thermal Systems in EV, Identify EV cooling system and Types, Identify components of cooling system, Identify liquid cooling system, Identify Air Cooling System of HVAC and Identify EV overheating signs

Theory: 10 Hours	Practice:  30 Hours 					Duration: 40 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Manage Thermal Systems in EV | - Heat Source Identification - Temperature Monitoring and Sensors - Cooling Method Selection - Thermal Protection Implementation - Thermal Performance Check - Identify cause of overheating | - Heat-generating components in EV system. - Operation of cooling and thermal protection system. - Causes of overheating in EV system. Practical Activity Inspect EV Thermal Management and Overheating Protection System | Theory-02Hrs Practical-06Hrs Total- 08Hrs | - Coolant - Distilled water - Electrical insulation tape - Cable ties - Cleaning cloth or rags - Sealant or gasket paste - Contact cleaner spray - Rust remover spray | Classroom and workshop |
| Identify EV cooling system and Types | - Purpose of EV cooling system - Identify types of cooling systems in EV - Identify cooling of battery pack - Inspection cooling of motor and inverter - Follow basic safety during inspection | - Purpose of EV cooling system. - Types of EV cooling systems. - Cooling system of battery pack, motor, and inverter. - Safety procedures during cooling system inspection. Practical Activity Inspect EV Cooling Systems and Safety Procedures | Theory-02Hrs Practical-06Hrs Total- 08Hrs | - Coolant - Distilled water - Electrical insulation tape - Cable ties - Cleaning cloth or rags - Sealant or gasket paste - Contact cleaner spray - Rust remover spray | Classroom and workshop |
| Identify components of cooling system | - Identify radiator in cooling system - Purpose of cooling fan - Purpose of coolant pump - Identify coolant reservoir - Identify hoses and pipes | - Radiator in EV cooling system. - Function of cooling fan and coolant pump. - Identification of Coolant reservoir, hoses, and pipes. Practical Activity Inspect Components of EV Cooling System | Theory-02Hrs Practical-06Hrs Total- 08Hrs | - Coolant - Distilled water - Electrical insulation tape - Cable ties - Cleaning cloth or rags - Sealant or gasket paste - Contact cleaner spray - Rust remover spray | Classroom and workshop |
| Identify liquid cooling system | - Purpose of liquid cooling system - Identify coolant in system - Describe main parts of system - Describe coolant flow path | - Purpose of liquid cooling system. - Identification of coolant and main parts of system. - Coolant flow path in EV cooling system. Practical Activity Check for leaks, damage, or blockage | Theory-02Hrs Practical-06Hrs Total- 08Hrs | - Coolant - Distilled water - Electrical insulation tape - Cable ties - Cleaning cloth or rags - Sealant or gasket paste - Contact cleaner spray - Rust remover spray | Classroom and workshop |
| Identify Air Cooling System of HVAC | - Function air cooling in HVAC system - Identify airflow path in air cooling - Role of blower fan in air cooling - Identify temperature control in air cooling - Check basic faults in air cooling system | - Function of air cooling in HVAC system. - Identification of airflow path and blower fan operation. - Basic faults in air cooling system. Practical Activity Inspect Air Cooling and HVAC System Components |  |  |  |
| Identify EV overheating signs | - Check temperature warning light on dashboard. - Check unusual noisy component of EV cooling system. - Notice reduced vehicle performance or power drop. - Check unusual smell /smoke in EV component. | - Understating of temperature indication - Identification of unusual noise in EV cooling system. - Introduction to vehicle performance and overheating signs. Practical Activity Inspect EV Cooling System Warning Signs and Fault Symptoms | Theory-02Hrs Practical-06Hrs Total- 08Hrs | - Coolant - Distilled water - Electrical insulation tape - Cable ties - Cleaning cloth or rags - Sealant or gasket paste - Contact cleaner spray - Rust remover spray | Classroom and workshop |

- Maintain 2-Wheeler EVs

Objective:  This module covers the knowledge and  skills required to Check Battery Pack and Charging System, Maintain Electric Motor and Drivetrain System, Service Thermal Management and Cooling System, Inspect Braking, Suspension, and Tire System and Perform Electrical Safety and Software Maintenance

Theory: 10 Hours	Practice: 30 Hours 				Duration: 40 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Check Battery Pack and Charging System | - Check battery pack voltage with approved diagnostic equipment before maintenance. - Check charging inlet for damage, contamination, or loose fittings carefully. - Check battery terminals for corrosion and secure electrical connections thoroughly. - Test onboard charger under specified charging conditions accurately. - Monitor battery temperature during charging and operation cycles regularly. | - Pre maintenance of EV battery system - Inspection of battery connections and charging system Practical Activity - Perform EV battery system inspection before maintenance | Theory-02Hrs Practical-06Hrs Total- 08Hrs | - Electrical insulation tape - Cable ties - Contact cleaner spray - Contact cleaner spray - Grease or lubricant - Valve caps - Tire patch kit - Air valve cores | Classroom and workshop |
| Maintain Electric Motor and Drivetrain System | - Check traction motor for abnormal noise, vibration, or overheating routinely. - Check reduction gearbox for lubrication level and mechanical wear carefully. - Check drive shafts for alignment and physical damage thoroughly. - Test accelerator system under controlled operating conditions safely. - Verify motor controller with diagnostic software for optimal power delivery | - Inspection of traction motor system - Inspection of drivetrain components and control system Practical Activity Check motor for abnormal sound, vibration, and heat | Theory-02Hrs Practical-06Hrs Total- 08Hrs | - Electrical insulation tape - Cable ties - Contact cleaner spray - Contact cleaner spray - Grease or lubricant - Valve caps - Tire patch kit - Air valve cores | Classroom and workshop |
| Service Thermal Management and Cooling System | - Check coolant level within specified range for thermal system efficiency. - Check coolant hoses for leaks, cracks, or wear carefully. - Check radiator assembly for blockage or physical damage thoroughly. - Test cooling pump under operational conditions for proper circulation. - Verify temperature sensors with diagnostic tools for accurate readings | - Coolant level and system condition - Inspection of cooling components Practical Activity Check radiator assembly for blockage or physical damage thoroughly | Theory-02Hrs Practical-06Hrs Total- 08Hrs | - Electrical insulation tape - Cable ties - Contact cleaner spray - Contact cleaner spray - Grease or lubricant - Valve caps - Tire patch kit - Air valve cores | Classroom and workshop |
| Inspect Braking, Suspension, and Tire System | - Check brake pads for wear and replacement requirements periodically. - Check regenerative braking for effective energy recovery performance accurately. - Check suspension components for leaks or mechanical damage carefully. - Check tire pressure according to manufacturer load specifications regularly. - Adjust wheel alignment with service equipment precisely. | - Introduction to suspension and tires alignment - Inspection of brake and energy recovery system Practical Activity Inspect EV chassis and braking system in workshop | Theory-02Hrs Practical-06Hrs Total- 08Hrs | - Electrical insulation tape - Cable ties - Contact cleaner spray - Contact cleaner spray - Grease or lubricant - Valve caps - Tire patch kit - Air valve cores | Classroom and workshop |
| Perform Electrical Safety and Software Maintenance | - Check high-voltage cables for insulation integrity and secure routing thoroughly. - Test safety interlocks for proper high-voltage isolation reliably. - Update vehicle software with authorized service platforms correctly. - Verify dashboard indicators for accurate system status communication properly. - Document maintenance records according to workshop and manufacturer standards. | - Introduction of high voltage safety and wiring condition - System control and vehicle communication Practical Activity Check high voltage cables for damage and secure routing | Theory-02Hrs Practical-06Hrs Total- 08Hrs | - Electrical insulation tape - Cable ties - Contact cleaner spray - Contact cleaner spray - Grease or lubricant - Valve caps - Tire patch kit - Air valve cores | Classroom and workshop |

- Maintain 3-Wheeler EVs

Objective:  This module covers the knowledge and  skills required  to Check Battery and Charging System of 3-Wheeler, Maintain Motor and Powertrain System, Maintain Brake and Suspension System of 3-Wheeler, Check Body, Tires, and Chassis Components of 3-Wheeler and Interpret faults signs on cluster of 3-Wheeler

Theory: 10 Hours	Practice:  30 Hours 				Duration:  40 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Check Battery and Charging System | - Check battery voltage with calibrated diagnostic equipment before operation. - Check charging socket for cleanliness, damage, or loose connections carefully. - Check battery terminals for corrosion and secure attachment thoroughly. - Test charger performance under standard charging conditions accurately. - Monitor battery temperature during charging cycles for safety compliance | - Check battery voltage before operation - Inspect charging interface and battery connections Practical Activity Check battery terminals for corrosion and secure fitting | Theory-02Hrs Practical-06Hrs Total- 08Hrs | - Electrical insulation tape - Cable ties - Contact cleaner spray - Cleaning cloth or waste rags - Grease or lubricant - Brake fluid - Coolant - Tire soap or cleaning solution | Classroom and workshop |
| Maintain Motor and Powertrain System | - Check traction motor for overheating, noise, or vibration regularly. - Check drive axle for alignment and wear during maintenance procedures. - Lubricate transmission components according to manufacturer specifications properly. - Test accelerator response under controlled operating conditions safely. - Verify motor controller using diagnostic software for efficient performance | - Inspect traction motor condition - Inspect drivetrain components and lubrication Practical Activity Check traction motor for abnormal noise, heat, and vibration | Theory-02Hrs Practical-06Hrs Total- 08Hrs | - Electrical insulation tape - Cable ties - Contact cleaner spray - Cleaning cloth or waste rags - Grease or lubricant - Brake fluid - Coolant - Tire soap or cleaning solution | Classroom and workshop |
| Maintain Brake and Suspension System of 3-W | - Check brake linings for wear and serviceability periodically. - Check brake fluid within specified levels for hydraulic systems. - Test braking efficiency under load conditions for operational safety. - Check suspension assemblies for leaks, wear, or mechanical damage carefully. - Adjust steering linkage according to service manual standards precisely | - Inspect braking system condition - Test braking and suspension performance Practical Activity Adjust steering linkage using service manual specifications | Theory-02Hrs Practical-06Hrs Total- 08Hrs | - Electrical insulation tape - Cable ties - Contact cleaner spray - Cleaning cloth or waste rags - Grease or lubricant - Brake fluid - Coolant - Tire soap or cleaning solution | Classroom and workshop |
| Check Body, Tires, and Chassis Components | - Check tire pressure according to recommended load specifications regularly. - Check tire condition for tread wear and sidewall damage thoroughly. - Check chassis frame for cracks, rust, or structural weakness carefully. - Tighten wheel fasteners with approved torque settings accurately. - Clean vehicle body to maintain operational appearance and prevent corrosion | - Maintain tire condition and pressure - Inspect chassis and structural condition Practical Activity Check tire pressure using gauge as per load requirement | Theory-02Hrs Practical-06Hrs Total- 08Hrs | - Electrical insulation tape - Cable ties - Contact cleaner spray - Cleaning cloth or waste rags - Grease or lubricant - Brake fluid - Coolant - Tire soap or cleaning solution | Classroom and workshop |
| Interpret faults signs on cluster of 3-W | - Identify common warning lights on 3-wheeler cluster. - Recognize color signals on cluster. - Interpret basic symbols and indicators. - Detect abnormal readings on gauges. - Document takes initial action based on fault signs. | - Identify and recognize cluster indications - Interpret symbols and monitor readings - Record and respond to fault signs Practical Activity inspect a 3-wheeler instrument cluster and identify fault signs | Theory-02Hrs Practical-06Hrs Total- 08Hrs | - Electrical insulation tape - Cable ties - Contact cleaner spray - Cleaning cloth or waste rags - Grease or lubricant - Brake fluid - Coolant - Tire soap or cleaning solution | Classroom and workshop |

- Maintain 4-Wheeler EVs

Objective: This module covers the knowledge and  skills required to Maintain Battery Pack and Charging System of 4-Wheeler, Maintain Electric Motor and Drivetrain System of 4-Wheeler, Identify HVAC of 4-Wheeler, Inspect Suspension, steering, Brake, and Tire of 4-Wheeler and Interpret fault codes on cluster of 4-Wheeler

Theory: 10 Hours							Practice: 30 Hours 						Duration: 40 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Maintain Battery Pack and Charging System of 4-W | - Check battery pack voltage with approved diagnostic equipment before maintenance. - Check charging inlet for damage, contamination, or loose fittings carefully. - Check battery terminals for corrosion and secure electrical connections thoroughly. - Test onboard charger under specified charging conditions accurately. - Monitor battery temperature during charging and operation cycles regularly. | - Importance of Battery Pack Voltage Inspection - Procedure for Battery Pack Voltage Testing - Charging Inlet Inspection Methods - Battery Terminal Condition Assessment Practical Activity - Perform battery pack voltage testing, inspect charging inlet condition, assess battery terminal connections, and test onboard charger operation | Theory-02Hrs Practical-06Hrs Total- 08Hrs | - Diagnostic scan tool printout paper - Electrical contact cleaner - Isopropyl alcohol wipes - Cleaning cloth / rags - Marker pens - Battery terminal cleaning brush | Classroom and workshop |
| Maintain Electric Motor and Drivetrain System of 4-W | - Check traction motor for abnormal noise, vibration, or overheating routinely. - Check reduction gearbox for lubrication level and mechanical wear carefully. - check drive shafts for alignment and physical damage thoroughly. - Test accelerator system under controlled operating conditions safely. - Verify motor controller with diagnostic software for optimal power delivery. | - Importance of Traction Motor Inspection - Procedure for Reduction of Gearbox Inspection - Drive Shaft Condition Assessment Techniques - Accelerator System Testing Methods - Motor Controller Diagnostic Procedures Practical Activity Inspect traction motor, gearbox, drive shafts, accelerator response, and motor controller using basic visual checks and simple diagnostic scan tool readings. | Theory-02Hrs Practical-06Hrs Total- 08Hrs | - Diagnostic scan tool printout paper - Electrical contact cleaner - Penetrating oil spray - Cleaning cloth / rags - Marker pens | Classroom and workshop |
| Identify HVAC of 4-W | - Identify the main parts of a 4-wheeler HVAC system. - Clarify basic working of HVAC in a vehicle. - Locate HVAC components in a 4-wheeler. - Identify control functions of HVAC system. - Follow basic safety while working near HVAC system. | - Basic Working Principle of Automotive HVAC System - Identification and Location of HVAC Components - HVAC Control Functions and Operations Practical Activity Inspect and identify main HVAC components in a 4-wheeler, demonstrate basic airflow and cooling operation and locate control units | Theory-02Hrs Practical-06Hrs Total- 08Hrs | - Isopropyl alcohol wipes - Cleaning cloth / rags - Electrical contact cleaner - Marking chalk | Classroom and workshop |
| Inspect Suspension, steering, Brake, and Tire of 4-W | - Check brake pads for wear and replacement requirements periodically. - Check regenerative braking for effective energy recovery performance accurately. - Check suspension components for leaks or mechanical damage carefully. - Check tire pressure according to manufacturer load specifications regularly. - Adjust wheel alignment with service equipment precisely | - Importance of Brake Pad Inspection - Regenerative Braking Performance Evaluation - Suspension System Condition Assessment - Tire Pressure Checking Procedures - Wheel Alignment Adjustment Methods Practical Activity Inspect brake pads, evaluate regenerative braking performance, check suspension condition, verify tire pressure, and perform wheel alignment using service equipment. | Theory-02Hrs Practical-06Hrs Total- 08Hrs | - Brake cleaner spray - Penetrating oil spray - Air compressor supply (compressed air) - Valve caps - Alignment adjustment shims - Cleaning cloth / rags - Marking chalk | Classroom and workshop |
| Interpret fault codes on cluster of 4-W | - Identify warning lights on the vehicle cluster. - Interpret basic fault symbols and indicators. - Locate fault messages on digital instrument cluster. - Apply basic scan tool to read fault codes. - Identify initial action based on fault indication. | - Recognition of Vehicle Warning Lights - Fault Symbols and Indicators - Reading of Digital Cluster Fault Messages - Retrieval of Fault Codes - Response Actions for Fault Conditions Practical Activity Inspect vehicle cluster warning lights, interpret fault symbols and digital messages, retrieve fault codes using scan tool | Theory-02Hrs Practical-06Hrs Total- 08Hrs | - Diagnostic scan tool printout paper - Fault code report sheets - Instruction reference sheets (warning lights chart) - Cleaning cloth / rags - Marker pens | Classroom and workshop |

# List of Machines, Tools and Equipment (for 25 Trainees)

Electric Vehicle (EV) Specifications

- 2-Wheeler EV	1 No.

- 3-Wheeler EV	1 No.

- 4-Wheeler EV	1 No.

| Specification | 2-Wheeler EV | 3-Wheeler EV | 4-Wheeler EV |
| --- | --- | --- | --- |
| Vehicle Type | Electric Motorcycle/ Scooter | Electric Rickshaw/ Cargo Loader | Electric Car/SUV/Bus |
| Number of Wheels | 2 | 3 | 4 |
| Seating Capacity | 1–2 | 3–6 | 4–7 (cars) |
| Motor Type | BLDC Hub or Mid-drive | BLDC/PMSM | PMSM or Induction Motor |
| Motor Power | 0.5–8 kW | 1–12 kW | 30–250+ kW |
| Maximum Speed | 25–120 km/h | 25–60 km/h | 100–220 km/h |
| Driving Range | 50–180 km | 70–180 km | 200–700 km |
| Charging Time | 3–6 hours | 4–8 hours | 30 minutes (DC Fast) to 12 hours (AC) |
| Transmission | Single Speed | Single Speed | Single Speed Reduction Gear |
| Regenerative Braking | Optional | Optional | Standard |
| Cooling System | Air cooled | Air/Liquid cooled | Liquid cooled |

| Name of Trade |  | 0716MSA19. Electric Vehicle (EV) Assistant Technician Level-2 |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Duration of the Course |  |  | 6 Months |  |  |
| S. No | Tools and Equipment |  |  | Quantity |
| 1 | Screwdriver set |  |  | 5Sets |
| 2 | Flat screwdriver |  |  | 5 Sets |
| 3 | Phillips screwdriver |  |  | 5 Sets |
| 4 | Insulated screwdriver |  |  | 5 Sets |
| 5 | Combination plier |  |  | 5 Sets |
| 6 | Wire stripper |  |  | 5 Packets |
| 7 | Long nose plier |  |  | 5 Pcs |
| 8 | Side cutter |  |  | 5 Pcs |
| 9 | Crimping tool |  |  | 5 Pcs |
| 10 | Adjustable spanner |  |  | 5 pcs |
| 11 | Ring spanner sets |  |  | 5Sets |
| 12 | Open end spanner |  |  | 5 Sets |
| 13 | Socket set |  |  | 5 Sets |
| 14 | Ratchet handle |  |  | 5 pcs |
| 15 | Allen key set |  |  | 5 Sets |
| 16 | Torque wrench |  |  | 5 Pcs |
| 17 | Hammer |  |  | 3 Pcs |
| 18 | Rubber mallet |  |  | 5 Pcs |
| 19 | Punch set |  |  | 5 Sets |
| 20 | Chisel |  |  | 5 Sets |
| 21 | Bench vice |  |  | 5 Sets |
| 22 | Hand drill |  |  | 5 Pcs |
| 23 | Drill bits |  |  | 5 Kits |
| 24 | Hacksaw |  |  | 5 Pcs |
| 25 | File set |  |  | 5 Sets |
| 26 | Measuring tape |  |  | 5 Pcs |
| 27 | Brake puller (Disc and Durm) |  |  | 2 Pcs |
| 28 | Steel ruler |  |  | 5 Pcs |
| 29 | Vernier caliper |  |  | 5 Pcs |
| 30 | Multimeter |  |  | 5 Sets |
| 31 | Clamp meter |  |  | 5 Pcs |
| 32 | Test lamp |  |  | 5 Pcs |
| 33 | Continuity tester |  |  | 5 Pcs |
| 34 | Insulation tester |  |  | 10 Pcs |
| 35 | Battery tester |  |  | 5 Sets |
| 36 | Oscilloscope basic |  |  | 2 Pcs |
| 37 | Temperature gun |  |  | 5 Pcs |
| 38 | Test light |  |  | 5 Pcs |
| 39 | Fuse puller |  |  | 5 Pcs |
| 40 | Trim removal tool Kits |  |  | 5 Pcs |
| 41 | Inspection mirror |  |  | 5 Pcs |
| 42 | Magnetic pickup tool |  |  | 5 Pcs |
| 43 | Flashlight, rechargeable |  |  | 5 Pcs |
| 44 | Creeper seat |  |  | 5 Pcs |
| 45 | Parts tray, magnetic |  |  | 15 Pcs |
| 46 | DC power supply |  |  | 5 Pcs |
| 47 | Insulation resistance tester |  |  | 5 Pcs |
| 48 | Torque wrench Big Size |  |  | 5 Pcs |
| 49 | Clamp meter (DC) |  |  | 5 Pcs |
| 50 | Test leads replacement |  |  | 2 Kits |
| 51 | Extension lead |  |  | 5 Pcs |
| 52 | Cable reel |  |  | 5 Pcs |
| 53 | Soldering iron |  |  | 5 Pcs |
| 54 | DE soldering pump |  |  | As per Uses |
| 55 | Heat gun |  |  | 5 Pcs |
| 56 | Utility knife |  |  | 5 Pcs |
| 57 | Electrical tape |  |  | 3 Role |
| 58 | Charging cable |  |  | 2 Sets |
| 59 | Battery charger |  |  | 2 Pcs |
| 60 | Portable EV charger |  |  | 2 pcs |
| 61 | EV training board |  |  | 2 Pcs |
| 62 | Motor controller trainer |  |  | 2 |
| 63 | BLDC motor model |  |  | 2 Station |
| 64 | Hub motor model |  |  | 2 Station |
| 65 | EV battery pack model |  |  | 2 |
| 66 | Lithium-ion cell sample |  |  | 5 Sets |
| 67 | Battery management system trainer |  |  | 2 |
| 68 | Accelerator throttle |  |  | 2 |
| 69 | Brake switch sensor |  |  | 5 Pcs |
| 70 | Temperature sensor |  |  | 5 Pcs |
| 71 | DC motor |  |  | 2 Pcs |
| 72 | AC motor model |  |  | 2 Pcs |
| 73 | Inverter trainer |  |  | 2 |
| 74 | Converter trainer |  |  | 2 |
| 75 | Wiring harness |  |  | 2 Sets |
| 76 | Connector set |  |  | 5 |
| 77 | Electrical panel board |  |  | 5 |
| 78 | Junction box |  |  | 2 Station |
| 79 | Circuit breaker |  |  | 10 Pcs |
| 80 | Insulated gloves |  |  | 15 Packets |
| 81 | Safety shoes |  |  | 25 Packets |
| 82 | Fire extinguisher |  |  | 3 Pcs |
| 83 | Lockout tag |  |  | 2 Sets |
| 84 | Stethoscope |  |  | 5 Pcs |
| 85 | Cable cutter |  |  | 5 Pcs |
| 86 | Fire suppression system (lithium battery specific) |  |  | 2 |
| 87 | Explosion proof storage cabinet (damaged battery) |  |  | 1 |
| 88 | Air blower |  |  | 2 |
| 89 | Laptop for diagnostics |  |  | 2 System |
| 90 | Diagnostic scanner basic |  |  | 2 System |
| 91 | HV voltage detector |  |  | 5 Pcs |
| 92 | Ventilation system |  |  | 2 |
| 93 | AVC Gas Recycling Machine |  |  | 1 |
| 94 | Soldering Iron Kit |  |  | 3 Kits |
| 95 | Bearing puller |  |  | 3 Sets |
| 96 | Battery Pack Analyzer |  |  | 5 Pcs |
| 97 | Hydraulic jack |  |  | 5 Pcs |
| 98 | Jack stand |  |  | 20 Pcs |
| 99 | Wheel spanner |  |  | 5 Pcs |
| 100 | Air compressor |  |  | 1 Pcs |

# List of Consumables (for 25 Trainees)

Battery Specifications

- 2-Wheeler EV	1 No.

- 3-Wheeler EV	1 No.

- 4-Wheeler EV	1 No.

| Parameter | Typical Specification |  |  |
| --- | --- | --- | --- |
|  | 2-Wheeler EV | 3-Wheeler EV | 4-Wheeler EV |
| Battery Chemistry | Lithium-ion (LFP/NMC) | Lithium-ion or LFP | Lithium-ion (NMC/LFP) |
| Battery Voltage | 48V, 60V, 72V | 48V, 60V, 72V, 96V | 200V–800V |
| Battery Capacity | 20–80 Ah | 100–250 Ah | 100–250 Ah |
| Energy Capacity | 1–5 kWh | 5–15 kWh | 20–120 kWh |
| Battery Type | Mostly Low Voltage | Low Voltage (48–72V) | High Voltage (200–800V |
| BMS | Yes | Yes | Advanced BMS |
| Charger | 48V/60V/72V Smart Charger | AC Charger | DC CCS/NACS (Fast Charging) |
| Cooling | Air cooling | Air cooling | Liquid Cooling |

| S. No | Consumables | Quantity |
| --- | --- | --- |
| 1 | Electrical insulation tape | 5 Packs |
| 2 | Cable ties | 2 Packs |
| 3 | Cleaning cloth | 25 Pcs |
| 4 | Marker and labels | 40 Pcs |
| 5 | Disposable gloves | 10 Sets |
| 6 | Sandpaper | 15 Pcs |
| 7 | Wire connectors | 2 Packs |
| 8 | Fuse wire | 50 Pcs |
| 9 | Lubricant spray | 5 Pcs |
| 10 | Battery terminal grease | 2 Pcs |
| 11 | Distilled water | 10 Liters |
| 12 | Notebook and recording sheets | 20 pcs |
| 13 | Masking tape | 20 pcs |
| 14 | Cleaning brush | 5 Pcs |
| 15 | Cotton waste | 10 Pack |
| 16 | PVC tape | 10 Pcs |
| 17 | Safety gloves | 50 Pcs |
| 18 | Face masks | 100 Pieces |
| 19 | Safety glasses wipes | 2 Bulk Packs |
| 20 | Hand cleaner | 3 Bottles |
| 21 | Disposable gloves | 20 Pcs |
| 22 | Permanent markers | 15 Pieces |
| 23 | Whiteboard markers | 1 dozen |
| 24 | Labels and Tag stickers | - Pcs |
| 25 | Recording sheets and Inspection sheets | 50 Pcs |
| 26 | Log book | 10 Pcs |
| 27 | Pens, Pencils, Erasers | As Required |
| 28 | Sharpeners | As Required |
| 29 | Printing paper and Clip boards | As Required |
| 30 | Carbon paper | As Required |
| 31 | Wire brush | 5 Pcs |
| 32 | Lubricant spray | 5 Rolls |
| 33 | Rust remover spray | 5 Cans |
| 34 | Degreaser | 5 Pcs |
| 35 | Glass cleaner | 3 Cans |
| 36 | Distilled water | 10 Liters |
| 37 | Battery water | As required |
| 38 | Coolant liquid | 15 Liters |
| 39 | Grease | 3 Pcs |
| 40 | Contact cleaner spray | 5 Pcs |
| 41 | Condenser cleaning solvent | 15 Liters |
| 42 | Silicone sealant | 2 Bottles |
| 43 | Adhesive glue | 5 Bottles |
| 44 | Epoxy adhesive | 5 Pcs |
| 45 | Thread seal tape | 5 pcs |
| 46 | Wire connectors | 5 Sets |
| 47 | Cable lugs | 5 Pcs |
| 48 | Crimp terminals | 10 Pcs |
| 49 | Ring terminals | 10 Pcs |
| 50 | Spade terminals | 5 Pcs |
| 51 | Heat shrink sleeves | 5 Packets |
| 52 | Ferrules | 5 Pcs |
| 53 | Fuse wire | 5 Role |
| 54 | Glass fuses | 15 Pcs |
| 55 | Electrical sleeves | 5 Role |
| 56 | Plastic covers | 5 Role |
| 57 | Storage boxes | 5 Pcs |
| 58 | Water bottle | 5 Sets |
| 59 | Mop cloth | 5 Sets |
| 60 | Floor cleaner | 5 Pcs |
| 61 | Dustpan bags | 5 Pcs |
| 62 | Waste bags | 10 Packets |
| 63 | Measuring tape stickers | 25 Pcs |
| 64 | Warning labels | 25 Pcs |
| 65 | Danger tags | 15 Pcs |
| 66 | Reflective tape | 15 Pcs |
| 67 | Cable markers | 15 Pcs |
| 68 | Battery labels | 20 Rolls |
| 69 | Microfiber cloth | 5 Packs |
| 70 | O rings | 10 Packs |
| 71 | Rubber seals | 2 Boxes |
| 72 | Foam tape | 15 Role |
| 73 | Electrical sleeves | 5 Sets |
| 74 | Hose clamps | 20 Pcs |
| 75 | Nuts, Bolts, Washers, Screws | 200 Pcs |
| 76 | Self-tapping screws | 2 Box |
| 77 | Rivets | Assorted kit |
| 78 | Battery Storage Containers | 5 Pcs |
| 79 | Straps / Securing Materials | 5 Sets |
| 80 | Hazard Labels (UN Markings) | 25 Pcs |
| 81 | Adhesive-lined Heat Shrink Tubing | 3 Role |
| 82 | Contact Cleaner Spray | 3 cans |
| 83 | Cleaning wipes for tools and PPE | 10 packs |
| 84 | Battery terminal covers and caps | 10 Sets |
| 85 | Warning labels and HV safety stickers | 25 Pcs |
| 86 | Absorbent pads for electrolyte leakage | 5 Packs |
| 87 | Cleaning solvent for workshop equipment | 10 liters |
| 88 | Refrigerant filter drier | 5 Pcs |
| 89 | Insulation tape (HVAC grade) | 50 Rolls |
| 90 | UV leak detection dye | 10 bottles |

# Members of Qualification Development Committee

The Qualifications Development Committee consisted of following members

| Sr.# | Name | Designation | Department |
| --- | --- | --- | --- |
|  | Inayat ur Rehman | DACUM Expert | KPK-TEVTA |
|  | Mohammad Ishaq | Deputy Director | NAVTTC |
|  | Shehzad Ahmed | Director | NAVTTC |
|  | Abdul Hafeez Abbasi | Director Operations | S-TEVTA |
|  | Mohammad Yousif Baloch | Director Academic | S-TEVTA |
|  | Syed Sultan Shah | Field Officer | Ministry of Federal Education and Professional Training |
|  | Rehman Ali | Sr: Executive | HYNDAI/ Nishat Motor Lahore |
|  | Syed Umar Azhar Bukhari | EV Technician | Yutong EV Buses R. Y. Khan |
|  | Shabaz Haider | Assistant Director | KP-TEVTA |
|  | Mohammad Zulqarnain | EV Technician | BYD Bahawalpur |
|  | Mohammad Zeeshan | Maintenance Supervisor | MG Service Center Faisalabad |
|  | Engr Zohaib Afzal | Head of R& D | Pak zone Lahore |
|  | Engr Saqib Abdullah | GM Plant | Jolta Electric Lahore |
|  | Zahid Razzaq | MM Tech | WESCON, Engg. Multan |
|  | Ijaz Hamid | Ex- Chief Instructor | P-TEVTA |
|  | Dr. Maliha Bakhs | Asst: Professor | UET Lahore |
|  | Abdul Haseeb Imtiaz | Service Engineer | KTS, Green Buses. Bahawalpur |
|  | Mian Abdul Atique | Service Manager | HONDA Center. Lahore |
|  | Muhammad Naveed | CEO | Vision Plus Automotive. Faisalabad |
|  | Muzammal Hussain | Assistant Director | P-TEVTA. R. Yar Khan |
|  | Mohid Ali | Service Advisor | Max Power. Lahore |
|  | Abdul Mannan | Instructor | P-TEVTA. Gujrat |
|  | Aftab Hussain Soomro | Program Coordinator TEVT | RDF. Sindh |