National Diploma Certificate of Electric Vehicle (EV) Supervisor Level-4

Curriculum

National Vocational and Technical Training Commission (NAVTTC)

Government of Pakistan

FOREWORD

National Vocational and Technical Training Commission (NAVTTC) extends its gratitude and appreciation to representatives of business, industry, academia, government agencies, provincial TEVTAs, sector skill councils and trade associations who spared time and extended their expertise for the development of National Diploma Qualification for Electric Vehicle (EV) Supervisor Level-4. This work would not have been possible without the technical support of the experts.

It may not be out of place to mention here that all the experts of Industry, Academia and TVET experts of TEVTAs, BTEs and PVTC work diligently for making this qualification worthy and error free for which all credit goes to them. However, NAVTTC accepts the responsibility of all the errors and omissions still prevailing in the Qualification document.

It is also noteworthy that development of Skill Standards is a dynamic and ongoing process, and the developed competency standards needs periodic review and updating owing to the constant technological advancements, development in scientific knowledge, and growing experience of implementation at the grass root level as well as the demand of industry. The NAVTTC, Government of Pakistan will ensure to keep the qualifications abreast with the changing demands of both national and international job markets.

Executive Director (NAVTTC)

Table of Contents

1.	Introduction	4

1.1. Date of Development	4

1.2. Date of Validation/NQRC	4

1.3. Entry Requirement	4

1.4. Minimum qualification for teachers	4

1.5. Medium of instruction	4

1.6. Duration of Course	4

2.	Scheme of Studies	6

3.	Overview of the Curriculum for Electric Vehicle (EV) Supervisor Level-4	7

4.	Detail of Modules.	10

0716MSA211	Supervise Workshop Safety	10

0716MSA212	Diagnose High-Voltage (HV) System	12

0716MSA213	Diagnose and repair Battery system	15

0716MSA214	Maintain EV HVAC system	18

0716MSA215	Maintain EV Inverter & Converter system	21

0716MSA216	Service Electric Motor/Drive Unit	23

0716MSA217	Analysis CAN Bus & Network Communication Practices	26

0716MSA218	Conduct ADAS & Autonomous Sensor Integration	30

0716MSA219	Perform HV Safety Isolation & Lockout-Tagout (LOTO)	37

0716MSA2110	Repair On-Board Charging System (OBC)	42

0716MSA2111	Perform Sensor and actuator Calibration	47

0716MSA2112	Perform HV Component Sealing and IP Rating Validation	57

0716MSA2113	Maintain Regenerative Braking System	64

0716MSA2114	Test Insulation Resistance (Megohmmeter)	69

0716MSA2115	Maintain Technical Documentation & Compliance Reporting	72

0716MSA2116	Perform HV Cabling and Connector Specialized Crimping	80

5.	List of Machines,Tools and Equipment	88

6.	List of Consumables	92

7.	Members of Qualification Development Committee	96

# Introduction

An Electric Vehicle (EV) is any vehicle that uses one or more electric motors for propulsion, drawing power from an energy storage system, most commonly a rechargeable battery. Unlike traditional internal combustion engine (ICE) vehicles that burn fossil fuels like petrol or diesel, EVs use electricity as their primary energy source. Therefore, industry expectations for skilled workforce are also dynamic which can only be managed through setting relevant Modules in collaboration with the leading industries. Being aware of this fact, National Vocational & Technical Training Commission (NAVTTC) developed Modules for Electric Vehicle (EV) under National Vocational Qualifications Framework (NVQF). These competency standards have been developed by a Qualifications Development Committee (QDC) and validated by the Qualifications Validation Committee (QVC) having representation from the Auto Mobile industry of the country.

| ## 1.1. Date of Development | April, 2026 |
| --- | --- |
| ## 1.2. Date of Validation/NQAC | June 18, 2026 |
| ## 1.3. Entry Requirement | Electric Vehicle (EV) Technician Level-3 |
| ## 1.4. Minimum qualification for teachers | - BSc Engg. /B.E./ B-Tech (4 Years) (Automotive/Mechanical/Electrical/Electronics) with 2 years’ experience in the field of Electric Vehicles. OR - DAE / DAE Level 5 (Automotive/Mechanical/Electrical/Electronics) with 3 years’ experience in the field of Electric Vehicles. |
| ## 1.5. Medium of instruction | Urdu, local language, English |
| ## 1.6. Duration of Course | 1 Year |

# Scheme of Studies

| Module Code | Name of Module | Category | NVQF Level | Contact Hours |  |  |  |  |  | Credit Hours |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Th. |  | Pr. |  | Total |  |  |  |
|  | Supervise Workshop Safety | Generic | 4 | 9 |  | 30 |  | 39 |  | 2.4 |  |
|  | Diagnose High-Voltage (HV) System | Technical | 4 | 14 |  | 36 |  | 50 |  | 3.2 |  |
|  | Diagnose and repair Battery system | Technical | 4 | 12 |  | 36 |  | 48 |  | 3.0 |  |
|  | Maintain EV HVAC system | Technical | 4 | 12 |  | 33 |  | 45 |  | 2.9 |  |
|  | Maintain Inverter & Converter system | Technical | 4 | 12 |  | 33 |  | 45 |  | 2.9 |  |
|  | Service Electric Motor/Drive Unit | Technical | 4 | 12 |  | 33 |  | 45 |  | 2.9 |  |
|  | Analysis CAN Bus & Network Communication | Technical | 4 | 14 |  | 33 |  | 47 |  | 3.1 |  |
|  | Conduct ADAS & Autonomous Sensor Integration | Technical | 4 | 14 |  | 33 |  | 47 |  | 3.1 |  |
|  | Perform HV Safety Isolation & Lockout-Tagout (LOTO) | Technical | 4 | 12 |  | 33 |  | 45 |  | 2.9 |  |
|  | Repair On-Board Charging System (OBC) | Technical | 4 | 10 |  | 33 |  | 43 |  | 2.7 |  |
|  | Perform Sensor and actuator Calibration | Technical | 4 | 10 |  | 33 |  | 43 |  | 2.7 |  |
|  | Perform HV Component Sealing & IP Rating Validation | Technical | 4 | 10 |  | 33 |  | 43 |  | 2.7 |  |
|  | Maintain Regenerative Braking System | Technical | 4 | 9 |  | 33 |  | 42 |  | 2.6 |  |
|  | Test Insulation Resistance (Megohmmeter) | Technical | 4 | 9 |  | 33 |  | 42 |  | 2.6 |  |
|  | Maintain Technical Documentation & Compliance Reporting | Generic | 4 | 9 |  | 30 |  | 39 |  | 2.4 |  |
|  | Perform HV Cabling & Connector Specialized Crimping | Technical | 4 | 9 |  | 30 |  | 39 |  | 2.4 |  |
| On Job Training (OJT) |  |  |  |  | 0 |  | 240 |  | 240 |  | 6 |  |
| Total |  |  |  |  | 177 |  | 765 |  | 942 |  | 50 |  |

Daily Contact Hours (For Reference)

| Daily Contact Hours adopt by Institute | Duration | Note: - For institute offering 6 days in a week. The Weekend institute may use formula: Credit hours/Daily hours = Duration in Months |
| --- | --- | --- |
| 4 | 11 months |  |
| 5 | 09 months |  |
| 6 | 08 months |  |
| 8 | 06 months |  |

# Overview of the Curriculum for Electric Vehicle (EV) Supervisor Level-4

| Module Title | Learning Units | Theory hours | Workplace hours | Timeframe of modules |
| --- | --- | --- | --- | --- |
| Supervise Workshop Safety | - Conduct daily safety briefing on High-Voltage (HV) - Inspect Technician Personal Protective Equipment (PPE) - Supervise Initial High-Voltage De- Energization Process - Manage High-Voltage Battery "Quarantine Area" - Perform Emergency Response Drill for EV fires | 9 | 30 | 39 |
| Diagnose High-Voltage (HV) System | - Perform Bidirectional Component Test through Scan Tool - Measure Live Sensor Data for Low Voltage System - Measure Live Sensor Data for High Voltage System - Perform Isolation & Verification of HV System - Perform Insulation Resistance Test on HV Cables - Record CAN-Bus Communication Signals with Oscilloscope | 14 | 36 | 50 |
| Diagnose and repair Battery system | - Inspect Battery Management System (BMS) - Balance Individual Cell Voltages within the Battery Pack - Replace / Repair Internal High-Voltage Contactors, Fuses, and Relays - Inspect Internal Thermal Management Plates for Leaks - Reseal Battery Enclosure with OEM-Approved Silicone - Isolate and Replace Faulty Battery Module | 12 | 36 | 48 |
| Maintain EV HVAC system | - Locate HVAC components - Diagnose faults in HVAC system of EV through scanner - Perform Service of HVAC Condenser - Repair/Replace faulty components of compressor - Test climate control sensors - Calibrate electronic expansion valves (EXV) | 12 | 33 | 45 |
| Maintain EV Inverter & Converter system | - Locate Fundamental Architecture of EV Inverter/Converter System - Diagnose Faults in EV Inverter & Converter System - Analyze Converter/Inverter Modules Data - Test Actuators of Inverter/Converter System - Inspect Wiring Harness of Inverter/Converter System | 12 | 33 | 45 |
| Service Electric Motor/Drive Unit | - Test Insulation Resistance of Stator - Test motor stator for phase-to-phase continuity - Inspect Internal Rotor Position Sensor (Resolver) - Inspect Bearings of High-Speed Drive Unit - Maintain Reduction Gear, Differential & Drive Axle Assemblies | 12 | 33 | 45 |
| Analysis Controller Area Network (CAN) Bus & Network Communication | - Analyze CAN-Bus Communication Signals through Oscilloscope - Measure Terminating Resistance of CAN-Bus Network - Detect Malfunctioning ECU Node by Bus Isolation - Repair Damaged Twisted-Pair Communication Wiring - Analyze Gateway Module Data Routing Efficiency | 14 | 33 | 47 |
| Conduct Advanced Driver Assistance Systems (ADAS) & Autonomous Sensor Integration | - Calibrate Forward-Facing Long-Range Radar (LRR) - Align Blind Spot Monitoring (BSM) Sensors - Program Front View Camera (FVC) Module - Troubleshoot Ultrasonic Park Assist Sensors - Synchronize LiDAR and Camera Data Streams | 14 | 33 | 47 |
| Perform HV Safety Isolation & Lockout-Tagout (LOTO) | - Establish High-Voltage Exclusion Zone - Remove Switch by Manual Service Disconnect (MSD) - Implement Lockout-Tagout (LOTO) Techniques - Verify Absence of High Voltage (Live-Dead-Live Test) - Discharge Residual High-Voltage Capacitors | 12 | 33 | 45 |
| Repair On-Board Charging System (OBC) | - Analyze AC to DC conversion failures. - Replace defective On-Board Charger (OBC) unit - Test Charging Connector proximity and pilot signals - Inspect charging port high-voltage pins for arcing - Configure Charging Control Module (CCM) software | 10 | 33 | 43 |
| Perform Sensor and actuator Calibration | - Flash Vehicle Control Unit (VCU) with updated firmware - Calibrate Accelerator Pedal Position Sensor (APPS) - Perform zero-point calibration of steering angle sensor (SAS) - Perform BMS State-of-Health (SOH) recalibration - Synchronize motor rotor position sensor (Resolver) | 10 | 33 | 43 |
| Perform HV Component Sealing & Ingress Protection (IP) Rating Validation | - Perform vacuum decay test on battery enclosure - Perform vacuum decay test on e-drive system - Inspect high-voltage connector O-rings for degradation - Apply specialized mastic sealant to battery tray | 10 | 33 | 43 |
| Maintain Regenerative Braking System | - Calibrate Electronic Brake Pedal Position Sensor (BPPS) - Evaluate Blending Efficiency Between Friction and Regen - Test Motor-Generator Torque Response During Coasting - Inspect Regenerative Braking Cooling requirements - Update ABS/ESC Module for Optimized Energy Recovery | 9 | 33 | 42 |
| Test Insulation Resistance (Megohmmeter) | - Calibrate Insulation Tester for High-Voltage Diagnostics - Test internal insulation of HV battery pack - Measure Motor Stator Winding-To-Case Resistance - Conduct Chassis-To-HV-Component Isolation Check - Evaluate High-Voltage Cable Shielding Integrity | 9 | 33 | 42 |
| Maintain Technical Documentation & Compliance Reporting | - Generate a comprehensive High-Voltage (HV) Battery health report - Record isolation test results for safety compliance - Document firmware version history and update logs - Prepare a Root Cause Analysis (RCA) for HV component failures - Maintain workshop hazardous material (Battery) disposal log - Maintain record on repair order | 9 | 30 | 39 |
| Perform HV Cabling & Connector Specialized Crimping | - Strip high-voltage orange insulation layers - Perform high-pressure hexagonal crimp - Perform terminal pull-test for mechanical strength - Apply adhesive-lined heat shrink tubing - Perform continuity Test of HV connector interlock (HVIL) | 9 | 30 | 39 |
|  | On Job Training (OJT) | 0 | 240 | 240 |
| Total |  | 177 | 765 | 942 |

# Detail of Modules.

## Supervise Workshop Safety

Objective:  This module covers the knowledge and  skills required to Conduct daily safety briefing on High-Voltage (HV), Inspect Technician Personal Protective Equipment (PPE), Supervise Initial High-Voltage De- Energization Process, Manage High-Voltage Battery "Quarantine Area" and Perform Emergency Response Drill for EV fires

Theory: 09 Hours				Practice:  30 Hours 					Duration:  39 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Conduct daily safety briefing on High-Voltage (HV) | - Conduct safety briefing at the start of each shift - Identify High-Voltage hazards to all technicians - Assign safety roles to team members - Communicate emergency procedures to all staff - Record briefing details in logbook | - Safety briefing for health safety environment (HSE) - Identification of HV hazard - Importance of Emergency communication - Procedures of record keeping Practical Activity Conduct High Voltage Safety Briefing | Theory-02Hrs Practical-06Hrs Total- 8Hrs | - safety briefing - Records/logbook entries - Paper - PPE items - Cleaning wipes | Classroom and workshop |
| Inspect Technician Personal Protective Equipment (PPE) | - Identify required PPE for each task - Inspect PPE of technicians before work - Replace defective PPE immediately - Monitor PPE usage during work - Document PPE audit results | - Introduction to PPE’s - Inspection techniques of PPE’s - Audit of PPE’s and documentation Practical Activity Audit of Personal Protective Equipment (PPE’s) | Theory-1Hrs Practical-6Hrs Total- 7Hrs | - PPE audit checklist - Pens, markers - Inspection tags - Disposable PPE | Classroom and workshop |
| Supervise Initial High-Voltage De- Energization Process | - Confirm EV system shutdown status - Apply Implement Lockout Tagout procedure for safety - Confirm absence of voltage - Confirm High-Voltage battery isolation - Record de-energization steps | - Techniques for EV system status - high voltage battery isolation procedures - Lockout/Tagout (LOTO) procedures - Documentation of de-energization steps Practical Activity Perform Lockout/Tagout (LOTO) for safe isolation | Theory-02Hrs Practical-06Hrs Total- 08Hrs | - verification checklist - Lockout/Tagout - isolation verification tools - De-energization log sheet | Classroom and workshop |
| Manage High-Voltage Battery "Quarantine Area" | - Mark quarantine area boundaries - Store damaged High-Voltage batteries safely - Maintain fire safety equipment in area - Monitor safety conditions continuously - Label quarantined batteries clearly | - Selection of Quarantine EV service bay - Handling of Damaged high-voltage batteries - Maintain fire safety equipment - Labeling of quarantined batteries Practical Activity Preform quarantine and Safety Management | Theory-02Hrs Practical-06Hrs Total- 08Hrs | - Barrier tape - Safety cons - Hazard stickers - Fire extinguisher tags - Marking chalk | Classroom and workshop |
| Perform Emergency Response Drill for EV fires | - Plan emergency response drill - Train personnel on fire response - Demonstrate fire suppression techniques - Conduct evacuation drill - Evaluate drill performance | - Planning for Emergency Drill - Evaluation of Emergency Response - Response to EV Fire Fighting Practical Activity Preform emergency Response and Fire Fighting Drill | Theory-02Hrs Practical-6Hrs Total- 08Hrs | - Fire extinguishers - Safety cones - Barrier tape - Evacuation floor plans - report Incident forms - First aid supplies - Warning labels | Classroom and workshop |

## Diagnose High-Voltage (HV) System

Objective:	This module covers the knowledge and skills required to Perform Bidirectional Component Test through Scan Tool, Measure Live Sensor Data for Low Voltage System, Measure Live Sensor Data for High Voltage System, Perform Isolation & Verification of HV System, Perform Insulation Resistance Test on HV Cables and Record CAN-Bus Communication Signals with Oscilloscope

Theory: 14 Hours				Practice:  36 Hours 					Duration:  50 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Perform Bidirectional Component Test through Scan Tool | - Identify vehicle configuration system and software version prior to testing - Start communication between scan tool and vehicle control modules - Select relevant module and execute bidirectional actuation tests as per OEM procedure. - Operate actuators/components and monitor/ record their response. - Compare actual actuator values with standard values of service Manual. - Identify abnormal behavior and record any DTCs (if appearing). - Confirm component functionality or diagnose fault based on test results - Record and document test outcomes. | - Pre-Diagnostic Setup and System Verification. - Bidirectional Control and Actuator Testing via Scan Tool - OEM scan tool operation and bidirectional control strategies - Analysis, Fault Identification, and Documentation Practical Activity Diagnose abnormal behavior of HV System Components with OEM Scan Tool | Theory-03Hrs Practical-06Hrs Total-09Hrs | Latest software | Class Room and workshop |
| Measure Live Sensor Data for Low Voltage System | - Access and monitor live data parameters for Low Voltage sensors using diagnostic equipment. - Measure and interpret sensor outputs against OEM reference values - Detect signal deviations, fluctuations, or inconsistencies - Compare multiple sensor inputs for accurate system diagnosis - Identify sensor, wiring, and ECU-related faults. | - Working Principle of (LV & HV) Sensor. - LV & HV Sensor Signal characteristics. - Live Sensor Data Monitoring Process. - Measurement and Interpretation of Sensor Signal. - Techniques for Diagnosis and Rectification of Faults. Practical Activity Analyze Live Sensor Data of LV System and Diagnose Fault. | Theory-02Hrs Practical-06Hrs Total-08Hrs | Job card / diagnostic report sheet | Class Room and workshop |
| Measure Live Sensor Data for High Voltage System | - Confirm HV safety compliance (de-energized vehicle) before accessing system data. - Access and monitor live data parameters for High Voltage components, sensors and control units using diagnostic equipment. - Compare data with OEM thresholds and specifications - Identify and monitor abnormal patterns or values. - Analyze system behavior to determine root cause - Confirm diagnostic conclusion with supporting data | - Working Principles of HV system. - Safety Compliance during Live Data Access. - HV Data Interpretation and Standards Comparison. - Pattern Analysis of HV Sensor Signal. Practical Activity Analysis HV System Live Data. | Theory-02Hrs Practical-06Hrs Total-08Hrs |  | Class Room and workshop |
| Perform Isolation & Verification of HV System | - Identify HV system layout and isolation points - Apply Megaohm meter to test isolation of HV system. - Discharge capacitors following OEM specified procedures - Confirm zero-voltage condition at all required points - Recheck isolation status prior to system handling. - Document isolation and verification process | - HV safety standards and PPEs. - HV System Layout Identification, Safety Isolation and Preparation. - HV Testing and Capacitor Discharge Procedures. - Zero-Voltage Verification, Reconfirmation and Documentation. Practical Activity Isolate High-Voltage System, Check Insulation, and Verify Zero-Voltage. | Theory-03Hrs Practical-06Hrs Total-09Hrs |  | Class Room and workshop |
| Perform Insulation Resistance Test on HV Cables | - Select appropriate insulation resistance testing equipment like Mega Ohm Meter. - Prepare system ensuring complete HV isolation - Apply specified test voltage according to OEM standards - Measure insulation resistance across cables and components - Compare readings with acceptable limits - Identify insulation degradation or leakage paths - Confirm results through repeat or comparative testing | - Selection of Equipment for HV System Preparation. - Measurement of Insulation Resistance and Testing Procedures. - Fault Identification, and Result Validation. - Insulation resistance testing techniques and evaluation criteria. Practical Activity Test Insulation Resistance of HV Cabling in EV System. | Theory-02Hrs Practical-06Hrs Total-08Hrs | Lockout/Tagout (LOTO) kit | Class Room and workshop |
| Record CAN-Bus Communication Signals with Oscilloscope | - Identify CAN network structure and access points - Connect oscilloscope probes correctly to CAN High and CAN Low lines - Capture waveform signals under operating conditions - Measure voltage levels and analyze signal patterns - Identify abnormalities such as noise, distortion, or signal loss - Diagnose communication faults (open, short, termination issues) - Confirm findings by cross-checking with system behavior - Document waveform analysis and diagnostic results | - CAN Network Structure Identification and Access Point Selection - Oscilloscope Setup and CAN Signal Capture - CAN Signal Analysis, Fault Diagnosis, and Validation Practical Activity Analyze CAN Network Signal and Diagnose Fault with Oscilloscope. | Theory-02Hrs Practical-06Hrs Total-08Hrs |  | Class Room and workshop |

## Diagnose and repair Battery system

Objective: This module covers the knowledge and  skills required to Inspect Battery Management System (BMS), Balance Individual Cell Voltages within the Battery Pack, Replace / Repair Internal High-Voltage Contactors, Fuses, and Relays, Inspect Internal Thermal Management Plates for Leaks, Reseal Battery Enclosure with OEM-Approved Silicone and Isolate and Replace Faulty Battery Module

Theory: 12 Hours				Practice:  36 Hours 				Duration:  48 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Inspect Battery Management System (BMS) | - Identify vehicle configuration and access BMS data using OEM approved tools - Verify communication between BMS and vehicle control systems - Monitor key parameters including cell voltage, temperature, SOC, and SOH - Detect abnormal readings and fault codes - Compare system data with OEM specifications and limits - Analyze BMS behavior to identify faults - Confirm faults through cross checking of system data - Record and document inspection results as per OEM standards | - Battery System Diagnosis and Fault Identification. - Battery Components Inspection and Testing. - Techniques for Repair/Replace of Components. Practical Activity Perform BMS diagnosis of EV battery system. | Theory-02Hrs Practical-06Hrs Total- 08Hrs | - Job card - Cleaning materials - Disposable wipes - Cleaning cloths - Paper | Class Room and workshop |
| Balance Individual Cell Voltages within the Battery Pack | - Access cell voltage data using OEM diagnostic system - Identify imbalance between battery cells - Perform cell balancing using OEM approved method - Monitor balancing process and track progress - Verify uniform cell voltage after balancing - Detect cells that do not respond to balancing - Confirm system stability after balancing process - Record and document balancing results as per standards | - Cell Voltage Analysis and Balancing - Techniques to access cell voltage data. - Measurement of voltage balancing. Practical Activity Perform and verify EV battery cell voltage balancing with OEM standard. | Theory-02Hrs Practical-06Hrs Total- 08Hrs | - Cleaning wipes - Disposable gloves - Paper - Cable tie - Marking tags |  |
| Replace / Repair Internal High-Voltage Contactors, Fuses, and Relays | - Identify faulty high voltage components using diagnostic methods - Ensure complete high voltage isolation before handling components - Access battery pack and locate defective components - Remove, repair, or replace contactors, fuses, and relays - Inspect connections and ensure correct installation - Reassemble battery system as per OEM guidelines - Verify proper functionality after component replacement | - Identification of HV faulty parts. - Techniques to check connections. - Reassembling of battery system as per OEM guidelines. Practical Activity Diagnose and replace faulty HV components in EV battery system. | Theory-02Hrs Practical-06Hrs Total- 08Hrs | - Cleaning wipes - Disposable gloves - Paper - Cable tie - Marking tags | Class Room and workshop |
| Inspect Internal Thermal Management Plates for Leaks | - Access battery thermal management system components - Inspect cooling plates and channels for leakage - Identify signs of coolant loss or contamination - Check pressure or flow where applicable - Determine source and severity of leakage - Assess impact of leakage on battery performance and safety | - Introduction to Battery Thermal Management System. - Inspection of Cooling System. - Identification of coolant leakage. - Impact of leakage and risk assessment. Practical Activity Inspect Leakage of Battery Thermal Management System | Theory-02Hrs Practical-06Hrs Total- 08Hrs | - Coolant, - Distilled water - Cleaning solvent - Cleaning wipes - Disposable gloves - Paper - Cable tie - Marking tags | Class Room and workshop |
| Reseal Battery Enclosure with OEM-Approved Silicone | - Prepare battery enclosure surface for resealing - Remove old sealing material without damaging components - Apply OEM approved sealant as per specification - Ensure uniform sealant application and proper sealing - Reassemble enclosure using correct torque procedures - Verify sealing integrity through visual and functional checks - Confirm no internal contamination inside enclosure - Record and document resealing process as per standards | - Surface preparation to apply OEM approved sealant. - Reassembling enclosure with correct torque. - Techniques to check sealing integrity. Practical Activity Reseal Battery Enclosure with OEM Approved Silicone | Theory-02Hrs Practical-06Hrs Total- 08Hrs | - Sheets - Cleaning wipes - Disposable gloves - Paper labels - Cable ties | Class Room and workshop |
| Isolate and Replace Faulty Battery Module | - Identify faulty battery module using diagnostic data - Perform complete high voltage system isolation - Access battery pack and locate defective module - Remove faulty module safely as per OEM procedures - Install replacement module with correct alignment and connections - Reassemble battery pack and restore system integrity - Conduct post replacement system checks and verification - Record replacement details and test results as per standards | - Fault Identification and Safe Module Replacement - System Reassembling, Testing and Documentation Practical Activity Perform high voltage battery system isolation. | Theory-02Hrs Practical-06Hrs Total- 08Hrs | - Sheets - Cleaning wipes - Disposable gloves - Marking labels - Cable ties |  |

## Maintain EV HVAC system

Objective: This module covers the knowledge and  skills required to Locate HVAC components, Diagnose faults in HVAC system of EV through scanner, Perform Service of HVAC Condenser, Repair/Replace faulty components of compressor, Test climate control sensors and Calibrate electronic expansion valves (EXV)

Theory: 12 Hours							Practice:  33 Hours 						    Duration:  45 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Locate HVAC components | - Identify HVAC components - Confirm component locations - Inspect component condition - Trace system connections - Confirm components identification | - Identification of EV HVAC components - Location of components and system connections - Inspection techniques of HVAC components Practical Activity Identify EV HVAC Components | Theory-2Hrs Practical-3Hrs Total- 5Hrs | Cleaning cloths Cleaner spray Cable ties Cleaning brushes Disposable gloves Checklist sheets Gloves (disposable) | Classroom and workshop |
| Diagnose faults in HVAC system of EV through scanner | - Connect diagnostic scanner - Retrieve diagnostic codes - Interpret diagnostic data - Monitor system parameters - Document fault information | - Introduction to diagnostic scanner - Trouble codes and data - Importance of system parameters Practical Activity Diagnose EV HVAC Faults Through Scanner | Theory-2Hrs Practical-6Hrs Total- 8Hrs | Electrical contact cleaner Fault report forms Insulation tape Battery terminal grease Cleaning cloth Disposable gloves | Classroom and workshop |
| Perform Service of HVAC Condenser | - Clean condenser surface - Inspect condenser condition - Straighten condenser fins - Check airflow passage - Secure condenser assembly | - Inspection techniques of condenser surface leakage - Importance of condenser- airflow and metal fins - Fixing techniques of condenser assembly Practical Activity Service EV HVAC Condenser | Theory-2Hrs Practical-6Hrs Total- 8Hrs | Cleaning brush Compressed air Refrigerant Compressor oil Replacement O-rings and seals Nitrogen gas for pressure testing | Classroom and workshop |
| Repair/Replace faulty components of compressor | - Diagnose compressor faults - Inspect compressor assembly - Repair compressor components - Replace faulty components of compressor - Test compressor efficiency | - Diagnosis techniques of compressor faults - Testing procedures for compressor efficiency and performance - Compressor components Repair and replacement procedures Practical Activity Repair/Replace EV Compressor Components | Theory-2Hrs Practical-6Hrs Total- 8Hrs | Refrigerant Compressor oil O-rings and sealing gaskets Cleaning solvent Insulation tape Cable ties Inspection checklist sheet | Classroom and workshop |
| Test climate control sensors | - Identify system sensors - Measure sensor signals - Assess sensor readings - Detect sensor faults - Confirm sensor performance | - Working principles of climate control system - Importance of sensor signals and reading - Detection techniques of sensor faults and abnormal conditions Practical Activity Test Climate Control Sensors in EV HVAC System | Theory-2Hrs Practical-6Hrs Total- 8Hrs | Cleaning cloth Electrical contact cleaner Grease Insulation tape Cable ties Marker labels | Classroom and workshop |
| Calibrate electronic expansion valves (EXV) | - Inspect EXV operation - Calibrate EXV settings - Monitor Valve Responses - Confirm refrigerant control - Confirm system operation | - Introduction to electronic expansion valve (EXV) - Inspection techniques of electronic expansion valve (EXV) - Calibration of electronic expansion valve (EXV) Practical Activity Calibrate Electronic Expansion Valve (EXV) in EV HVAC System | Theory-2Hrs Practical-6Hrs Total- 8Hrs | Refrigerant Compressor oil Cleaning solvent O-ring seals Thread sealant Electrical contact cleaner | Classroom and workshop |

## Maintain EV Inverter & Converter system

Objective: This module covers the knowledge and  skills required to Locate Fundamental Architecture of EV Inverter/Converter System, Diagnose Faults in EV Inverter & Converter System, Analyze Converter/Inverter Modules Data, Test Actuators of Inverter/Converter System and Inspect Wiring Harness of Inverter/Converter System

Theory: 12 Hours				Practice:  33 Hours 					Duration:  45 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Locate Fundamental Architecture of EV Inverter/Converter System | - Check Safety & EMI Shielding Architecture - Detect System Layout & Physical Integration - Locate Inverter Power Stage Components - Locate AC/DC Converter - Locate DC/AC Inverter - Detect DC-DC Converter (Step-Down) Architecture - Record Thermal Management Architecture | - Introduction to Inverter/Converter System - Principle of Power Conversion (AC/DC, DC/AC, DC/DC) - System Integration and Configuration with Battery and Motor - EMI Shielding in EV Systems - Thermal Management system Practical Activity: Analyze power conversions with diagnostic tool | Theory-02Hrs Practical-06Hrs Total- 08Hrs | - Electrical Contact Cleaner - Cotton Swabs/ - Wipes - Nitrile Gloves | Class Room and workshop |
| Diagnose Faults in EV Inverter & Converter System | - Conduct Preliminary Diagnostic Scanning, DTC Analysis and Freeze frame data, - Diagnose Trouble Code (DTC) as per repair Manual - Conduct High-Voltage Isolation (Insulation) Testing - Analyze sensor, actuator and Switching Signals - Assess Thermal Performance via Infrared (IR) Thermography - Diagnose DC-DC Converter Output and Efficiency | - Scanning Tools for System Diagnosis - Inspection Techniques of Thermal Faults Detection - Analyze (DC-DC) Converter Output - Procedure to Diagnose Trouble Codes (DTCs) Practical Activity: Retrieve and Rectify DTCs with scanning tool | Theory-03Hrs Practical-09Hrs Total- 12Hrs | - Dielectric Grease - Cotton Swabs - Nitrile Gloves - Electric Contact Cleaner | Workshop |
| Analyze Converter/Inverter Modules Data | - Analyze Live Data Streams via Diagnostic Interface as per manual - Compare DC-Bus Voltage Stability Against Battery Management (BMS) Data - Assess sensors Switching Frequencies and Phase Currents - Cross-Reference Thermal Sensor Data with Cooling System Status - Validate DC-DC Conversion Efficiency and Low-Voltage Output Logic | - Live Data analysis with Scanner - DC Bus Voltage analysis - Sensor Switching techniques - Importance of Phase Current - Power Conversion Efficiency - Temperature Sensing and Thermal Feedback Practical Activity: Compare BMS data with DC bus voltage | Theory-03Hrs Practical-06Hrs Total- 09Hrs | - Nitrile Gloves - Cotton Swabs | Class Room and workshop |
| Test Actuators of Inverter/Converter System | - Conduct actuation test using diagnostic tools - Cycle the Low-Temperature Coolant Pump (PWM). - Actuate the High-Voltage System Main Relays (SMR). - Test Power Electronics Cooling Fans | - Actuators in Inverter Management - Actuators in Thermal Management - Coolant Pumps and Cooling Fans - High-Voltage Relays - PWM Control Signals Practical Activity: Perform actuation test with diagnostic tools | Theory-02Hrs Practical-06Hrs Total- 08Hrs | - EV Specific Coolant - Dielectric Grease - Electrical Contact Cleaner - Nitrile Gloves | Workshop |
| Inspect Wiring Harness of Inverter/Converter System | - Tactile Check for Connector Seating and Lock Engagement - Conduct Visual inspection of High-Voltage Insulation - Check the High-Voltage Interlock Loop (HVIL) Pilot Circuit - Inspect the resistance values of high/low-voltage circuits wiring. - Inspect Low-Voltage Signal Wire "Bend Radii" and Stress Point - Conduct Physical Probe of Connector Seals for Moisture Ingress - Repair or replace faulty wiring and relevant components - Test wiring after repair work | - Structure and Routing of Wiring Harness - High-Voltage Insulation Standards - High-Voltage Interlock Loop (HVIL) - Continuity and Resistance Concepts - Causes of wiring Damage - Connector Types and Locking Mechanism Practical Activity: Identify and Repair damaged wiring and retest | Theory-02Hrs Practical-06Hrs Total- 08Hrs | - Orange Shielded Cables - Terminal Pins/Sockets - Heat Shrinking Tubing - High Temp Tape - Cable Ties - Dielectric Grease - Electrical Contact Cleaner - Distilled Water - Nitrile Gloves | Workshop |

## Service Electric Motor/Drive Unit

Objective: This module covers the knowledge and  skills required to Test Insulation Resistance of Stator, Test motor stator for phase-to-phase continuity, Inspect Internal Rotor Position Sensor (Resolver), Inspect Bearings of High-Speed Drive Unit and Maintain Reduction Gear, Differential & Drive Axle Assemblies

Theory: 12 Hours								Practice:  33 Hours 					    Duration:  45 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Test Insulation Resistance of Stator | - Confirm readiness of HV system after proper isolation. - Confirm availability of non-conductive work mat. - Confirm availability of HV-specific tools and equipment. - Select appropriate insulation resistance tester (Megger) - Measure insulation resistance between U, V, W phases and ground. - Apply correct test voltage as per OEM standards (500V/1000V DC). - Interpret insulation values against specified limits. - Identify insulation degradation or dielectric breakdown. - Detect moisture ingress or coolant contamination Clear workspace of any flammable materials - Compare readings under different environmental conditions. - Confirm repeatability and accuracy of measurements - Record results and conclude system health. | - HV system isolation and readiness - Selection of specific insulation tester - Insulation testing with proper voltage - Identification of insulation faults - Comparison of resistance value with OEM standard - Results and system condition assessment Practical Activity Perform HV system Isolation, insulation test, analyze and record results | Theory-4Hrs Practical-06Hrs Total- 10Hrs | Non-conductive work mat Test record sheet Safety isolation tags (LOTO) Clean, dry, flammable-free workspace materials | Classroom and workshop |
| Test motor stator for phase-to-phase continuity | - Confirm milli-ohm meter calibrated for low resistance measurement. - Measure resistance between U–V, V–W, and U–W. - Confirm symmetry across all phases. - Identify open circuit conditions. - Detect short circuit or partial winding faults. - Analyze imbalance indicating internal damage. - Inspect terminal connections and contact points. - Compare readings with diagnostic trouble codes (DTCs) & Compare with OEM reference values. | - Calibration of Milli-ohm meter - Measurements of phase resistance - Diagnostic trouble codes (DTCs) and Comparison with OEM reference values Practical Activity Measure phase resistance, check symmetry, detect faults, and compare them with standards. | Theory-02Hrs Practical-06Hrs Total- 08Hrs | OEM reference resistance values/specifications Diagnostic trouble codes (DTC) reference sheet logbook Clean terminal connection points / terminals Safety isolation tags (LOTO) | Classroom and workshop |
| Inspect Internal Rotor Position Sensor (Resolver) | - Inspect wiring harness and shielding condition. - Confirm excitation voltage supply. - Measure resolver output signals - Analyze waveform using oscilloscope - Detect signal distortion or phase shift - Confirm synchronization with control unit - Diagnose resolver-related DTCs. - Perform calibration/relearn procedure | - Wiring condition inspection - Resolver signal measurement - Waveform analysis with oscilloscope - Calibration procedure of rotor position sensor Practical Activity Perform resolver signal testing, fault identification, and calibration. | Theory-02Hrs Practical-06Hrs Total- 08Hrs | Resolver wiring harness OEM reference values / specifications Diagnostic trouble codes (DTC) Test record sheet / logbook Calibration / relearn procedure data sheet | Classroom and workshop |
| Inspect Bearings of High-Speed Drive Unit | - Conduct operational noise analysis under varying conditions - Identify abnormal sounds (whining, grinding) - Perform vibration/NVH analysis. - Apply diagnostic tools (stethoscope/NVH analyzer). - Inspect lubrication condition and contamination. - Measure axial and radial play. - Identify overheating or discoloration on the bearing and motor housing. - Assess impact on performance and efficiency. - Regulate replacement criteria. | - Identification of abnormal noise and vibration - Fault detection with diagnostic tools - Inspection of lubrication and condition - Detection of wear or overheating - Assessment of bearing condition and replacement Practical Activity Inspect overheating or discoloration on the bearing and motor housing. | Theory-02Hrs Practical-06Hrs Total- 08Hrs | Clean Lubricant OEM specifications sheet Test record sheet Safety tags (LOTO) Cleaning cloths | Classroom and workshop |
| Maintain Reduction Gear, Differential & Drive Axle Assemblies | - Inspect gear teeth for wear, pitting, or scoring - Check gear oil level and condition - Identify contamination or metal particles - Assess differential operation - Inspect axle shafts and CV joints - Diagnose abnormal noise during operation - Measure backlash and gear clearances - Inspect seals and leakage points - Confirm smooth torque transmission. | - Inspection of gear teeth condition - Checking the gear oil level and contamination - Types of noises of drivetrain - leakage Inspection and causes Practical Activity Inspect drive train system | Theory-02Hrs Practical-09Hrs Total- 11Hrs | Gear oil (standard/clean sample) OEM specifications sheet Test record sheet Cleaning clothes Safety tags (LOTO) | Classroom and workshop |

## Analysis Controller Area Network (CAN) Bus & Network Communication Practices

Objective: This module covers the knowledge and  skills required to Analyze CAN-Bus Communication Signals through Oscilloscope, Measure Terminating Resistance of CAN-Bus Network, Detect Malfunctioning ECU Node by Bus Isolation, Repair Damaged Twisted-Pair Communication Wiring and Analyze Gateway Module Data Routing Efficiency

Theory: 14 Hours								Practice:  33 Hours 					   Duration:  47 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Analyze CAN-Bus Communication Signals through Oscilloscope | - Identify CAN system issues with Oscilloscopes for CAN-Bus decode functions. - Set the oscilloscope to CAN-Bus protocol decoding mode. - Connect probes to CAN-H and CAN-L lines (differential signal) - Check CAN-H and CAN-L signal voltage levels (as per standard guidelines) - Assess CAN Dominant (0) and recessive (1) Values - Assess CAN graph Bit timing and baud rate - Identify CAN-H and CAN-L Error frames or corrupted data | - Introduction to network and CAN Communication - Analysis of automotive CAN Network Protocol Decoding and waveform - Identification of CAN-H and CAN-L Error frames Practical Activity Perform CAN-Bus Signal Decode and Fault Analysis with Oscilloscope | Theory-3Hrs Practical-6Hrs Total- 9Hrs | Inspection Sheets. | Classroom and workshop |
| Measure Terminating Resistance of CAN-Bus Network | - Disconnect power from the Vehicle network. - Set the multimeter to the resistance mode (Ohms/Ω) ideally on the 200Ω or 600Ω range (if auto-ranging not available) - Identify CAN-H and CAN-L pins at DLC connector for measuring termination resistance - Measure the CAN-H and CAN-L terminating resistance values. - Assess standard termination resistance values of CAN-H and CAN-L as per repair manual guidelines. | - Fundamentals of CAN-Bus Network Topology and Termination Requirements - Testing CAN-Bus Termination Resistance and Physical Layer Specifications - Standard termination resistance values of CAN-H and CAN-L as per manual Practical Activity Perform CAN-Bus Termination Resistance with Digital Multimeter and OEM Manual | Theory-3Hrs Practical-6Hrs Total- 9Hrs | Inspection Sheets. | Classroom and workshop |
| Detect Malfunctioning ECU Node by Bus Isolation | - Connect a Vehicle diagnostic tool to the vehicle's OBD Port. - Perform DTC interrogation on the malfunctioning ECU using the diagnostic Tool - Conduct a comprehensive diagnostic scan to inspect all effected ECUs within the vehicular network topology. - Access the live data stream or parameter list on the diagnostic Tool. - Perform an in-depth analysis of the DTC freeze frame data set. - Confirm the signal voltage parameters in accordance with the prescribed DTC remediation procedure. - Disconnect ECUs individually from the CAN-Bus network to facilitate fault isolation and identification. in case completely BUS failure. - Check network communication after each disconnection. - Identify aberrant ECU, compromised wiring segment, or defective connector precipitating the CAN-Bus network outage. | - Diagnostic Systematic CAN-Bus Network and Fault Isolation - CAN-Bus Network Failure Modes and Diagnostic Classifications - Live data parameters. - Network topology - Divide and isolation of fault - Precautions for Vehicle Wiring Harnesses, Modules and Actuators Practical Activity Perform CAN-Bus Network Fault Diagnosis and ECU Isolation with scan tool | Theory-4Hrs Practical-6Hrs Total- 10Hrs | Inspection Sheets. | Classroom and workshop |
| Repair Damaged Twisted-Pair Communication Wiring | - Check relevant repair manual or electrical wiring diagram to Identify the defective area. - Analysis and modify defective ECU component, wiring segment, or connectors with precision tools to extract defects. - Perform a multimeter-based resistance measurement across the wiring or joint connector to detect potential open-circuit discontinuities or short-circuit anomalies. - Identify and isolate the defected area - Strip back damaged insulation and trim wires. - Apply solder or crimp connectors to establish robust, low-resistance joints, ensuring mechanical integrity and reliable electrical continuity. - Re-twist the wires, maintaining the original twist rate if feasible, to restore the twisted pair's electromagnetic interference (EMI) rejection properties. - Apply heat shrink tubing or wrap with electrical tape to provide insulation, moisture resistance, and mechanical protection to the repaired joint - Conduct a comprehensive wiring validation to confirm conductor continuity, inter-wire and ground-plane isolation, and adherence to termination and shielding protocols (if applicable) | - Introduction to Wiring diagram - Precautions for Vehicle Wiring Harnesses - Twisted pair principles - Techniques of wiring Harness, Fault Diagnosis and CAN Twisted Pair Repair. - Techniques for Multimeter-Based Diagnosis of Electrical Circuits. - Post-repair validation - Wastage Disposal for Environmental Protection Practical Activity Check CAN-Bus Wiring Fault Isolation and OEM-Standard Repair of Twisted Pair Circuits. | Theory-4Hrs Practical-6Hrs Total- 10Hrs | Measuring Pins. Insulation Tape. Wiring Contact Cleaner. | Classroom and workshop |
| Analyse Gateway Module Data Routing Efficiency | - Consult the repair manual to identify the Gateway Module's location, pinout, and communication protocols, facilitating targeted diagnostic access - Connect the diagnostic Tool to the OBD-II port and establish communication with all Electronic Control Units (ECUs) via the Gateway Module. - Conduct a thorough diagnostic scan via the Gateway Module to retrieve fault codes and live data from affected ECUs, analyzing the vehicle's network topology for anomalies - Check message prioritization (like safety-critical vs. comfort systems) - Perform assessment of the Gateway Module's Techniques including Power Management, Wake-up and Sleep Modes, ECUs Firmware Updates, and ECUs Communication to assess system operation. | - Gateway Module Architecture - Gateway Module role and function - Common Gateway DTCs - Multi-Network Diagnostics in Modern Vehicles - Gateway Module Message prioritization Practical Activity Analyze Gateway Module Network and Multi-Domain Communication | Theory-3Hrs Practical-6Hrs Total- 9Hrs | Inspection Sheets. | Classroom and workshop |

## Conduct Advanced Driver Assistance Systems (ADAS) & Autonomous Sensor Integration

Objective: This module covers the knowledge and  skills required to Calibrate Forward-Facing Long-Range Radar (LRR), Align Blind Spot Monitoring (BSM) Sensors, Program Front View Camera (FVC) Module, Troubleshoot Ultrasonic Park Assist Sensors and Synchronize LiDAR and Camera Data Streams.

Theory: 14 Hours				Practice: 33 Hours 					Duration: 47 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Calibrate Forward-Facing Long-Range Radar (LRR) | - Position the OEM radar target board at the correct distance, offset, and height per the calibration procedure. - Confirm target board alignment using a calibrated measurement reference system before initiating the calibration routine. - Connect the OEM diagnostic platform to the vehicle CAN/Ethernet interface and navigate to the LRR calibration function. - Confirm all pre-calibration conditions (ride height, steering angle at zero, tyre pressures) are within OEM specification before starting the static calibration sequence. - Execute the LRR static calibration sequence to completion and confirm azimuth and elevation offsets are within OEM acceptance thresholds. - Conduct a controlled road test at OEM-specified speed to confirm the radar correctly detects and tracks targets within the specified range and angular window. - Log dynamic road test results using the diagnostic platform's data recording function and sign the road test record. - Clear all LRR-related DTCs and verify zero active codes on a final diagnostic scan. - Document all calibration parameter values on the job card and obtain assessor sign-off before returning the vehicle to service. | - Introduction to Advanced Driver Assistance System (ADAS) - Autonomous Sensor Integration and verification - Radio frequency (RF) radar operation - CAN bus and Ethernet/DoIP communication protocols - Long Range Radar (LRR) calibration procedures for multiple OEM - LRR Diagnostic Trouble Codes (DTCs) and rectification methods - Dynamic radar verification principles - ISO 26262 Functional Safety and ISO/SAE 21434 Cybersecurity awareness in context of ADAS sensor programming and fusion system commissioning Practical Activity Perform ADAS Diagnostic scanning and calibrate it. | Theory-03Hrs Practical-09Hrs Total-12Hrs | - Updated OEM software - Floor marking tape - Chalks / markers for marking reference points - Zip ties - Calibration checklists - Paper sheets | Classroom and workshop |
| Align Blind Spot Monitoring (BSM) Sensors | - Retrieve all active and stored BSM DTCs using the OEM diagnostic platform. - Inspect BSM sensor housings and mounting brackets for physical damage, corrosion, and deformation, and verify sensor faces are clean and unobstructed. - Confirm mounting fastener torque meets OEM specification using a calibrated torque wrench. - Input all required BSM module configuration parameters (vehicle width, sensor variant, tow package status) via the diagnostic platform before initiating the alignment routine. - Run the BSM alignment calibration sequence and confirm left and right detection-zone angles are within OEM symmetry tolerance. - Adjust sensor mounting brackets per OEM guidance where angle values exceed tolerance and re-run the alignment sequence to confirm acceptance. - Pass a reference obstacle alongside the vehicle at OEM-specified speed and lateral distance, confirming both BSM sensors activate the warning indicator within the correct detection zone. - Clear all BSM DTCs and confirm zero active codes on a final diagnostic scan. - Record alignment results and sensor IDs on the completed job card. | - Operating principles of microwave/radar-based Blind Spot Monitoring (BSM) sensors - BSM system architecture and fault classification - BSM system alignment and calibration procedure Practical Activity Diagnose and calibrate, the BSM system of a vehicle | Theory-03Hrs Practical-06Hrs Total-09Hrs | - Updated software - Cable Ties - marking pens/chalks - electronic cleaner - cleaning cloth - Floor marking Tape - Paper sheets | Classroom and workshop |
| Program Front View Camera (FVC) Module | - Confirm FVC module part number and hardware version compatibility with the vehicle ADAS network by cross-referencing the module label against OEM fitment data. - Confirm battery voltage meets OEM minimum (≥12.5 V) using a battery support unit before starting any programming operation. - Create a stable DoIP/Ethernet or CAN programming connection and initiate the VIN-writing and variant coding routine. - Enter variant coding parameters (camera type, lane width variant, market region) per the OEM coding guide without interrupting the programming cycle. - Run the FVC intrinsic calibration routine using the OEM camera target board. - Confirm image quality metrics (sharpness index, distortion coefficient) are within OEM acceptance limits from the intrinsic calibration output. - Position the OEM extrinsic calibration target at the specified longitudinal distance and lateral offset per the calibration procedure. - Run the pitch/roll/yaw extrinsic calibration and confirm the coordinate transform values are accepted and stored in non-volatile memory. - Conduct an LDW/LKA practical check confirming the FVC detects lane boundaries at OEM-specified distances and triggers outputs within the required latency. - Apply the OEM diagnostic flowchart to a simulated FVC fault (coding mismatch, image quality error, or bus dropout) and identify the root cause. - Document the corrective action on the fault analysis form and justify the diagnosis verbally to the assessor. | - Operation of Lane Departure Warning (LDW) and Lane Keeping Assist (LKA) function - FVC module architecture and ADAS Integration - Camera internal and external calibration Practical Activity Program and calibrate FVC module with special service tools (SST) and test LDW/LKA functionality | Theory-03Hrs Practical-06Hrs Total-09Hrs | - Updated software - Lens cleaning wipes - Alignment tape - Markers / chalks - Battery jaw clamps - Equipment covers - Paper sheets | Classroom and workshop |
| Troubleshoot Ultrasonic Park Assist Sensors | - Retrieve all active and historical park assist system DTCs using the OEM diagnostic platform - Locate the implicated sensor position(s) on the OEM wiring diagram sensor map. - Identify each fault as an open circuit, short circuit, internal sensor failure, or configuration error before commencing physical inspection. - Inspect sensor housings, wiring connectors, and mounting grommets for impact damage, moisture ingress, corroded pins, and correct seating depth. - Measure supply voltage and ground continuity at each suspect sensor connector using a calibrated DMM. - Activate individual sensor transmits pulses via the diagnostic platform and capture the echo return waveform for each suspect sensor. - Compare captured wave form frequency, amplitude, and echo delay against OEM acoustic performance specification values. - Install the correct OEM replacement sensor flush to the bumper skin within the specified protrusion tolerance with the connector fully locked and grommet fully sealed. - Run a post-replacement park assist system self-test using the diagnostic platform and verify the replaced sensor reports within the OEM-specified signal quality range. - Place a flat-face test target at OEM-defined detection zone distances and confirm correct audible and visual alert activation at each zone. - Separate a multi-sensor simultaneous fault scenario between individual sensor failures and a systemic cause (ECU supply, CAN error, or shared ground) using structured diagnostic reasoning. | - Principles of ultrasonic sensors - Park assists system architecture - Sensor wiring circuit topology - System fault analysis and diagnostic methodology Practical Activity Identify each fault as an open circuit, short circuit, internal sensor failure, or configuration error | Theory-02Hrs Practical-06Hrs Total-08Hrs | - Updated Software - Replacement parking sensors - Sensor mounting grommets / seals - Electrical terminals / connector repair kits - Heat shrink tubing - Electrical insulation tape - Zip ties - Protective covers - Marker pen / chalk - Sensor test and verification checklist / report sheets - Dielectric grease - Cleaning wipes/cloth - Electronic contact cleaner | Classroom and workshop |
| Synchronize LiDAR and Camera Data Streams | - Confirm both the LiDAR and FVC are outputting PPS/PTP synchronization pulses at OEM-specified frequencies using a dual-channel oscilloscope. - Record the measured phase offset between the two sensor synchronization pulses. - Enter the OEM LiDAR-to-camera extrinsic transform parameters (rotation matrix and translation vector) into the sensor fusion ECU via the diagnostic platform. - confirm correct storage of the entered extrinsic transform values with a readback confirmation from the module. - Place a retroreflective calibration target at a known, measured position in front of the vehicle. - Capture simultaneous LiDAR point-cloud and camera image frames of the target. - Evaluate the target centroid spatial overlap in both data streams using OEM validation software and confirm co-registration error is within OEM tolerance. - Monitor live fused sensor output during a controlled low-speed pass (≤10 km/h) past a static obstacle using the OEM engineering tool. - Confirm LiDAR and camera detections are temporally aligned within OEM latency tolerance with no ghost objects or missed detections. - Analyze sensor logs to identify the specific parameter or hardware issue causing a synchronization fault (PTP drift, dropped LiDAR frames, or camera frame-rate mismatch). - Document the root cause and corrective action recommendation on the prescribed fault report. - Perform a final ADAS integration scan confirming ACC, AEB, and all fusion-dependent functions are within OEM operational specification. Operate connector lock/unlock mechanism to verify change in HVIL circuit status. - Compare measured results with OEM specifications and determine Pass/Fail status. - Record test results (connector ID, readings, operator details), capture evidence if required, and store records in a traceable system | - LiDAR working principles - Camera-LiDAR sensor fusion architecture - Hardware time synchronization methods - External calibration concepts - Co-registration test methodology Practical Activity Evaluate the target centroid spatial overlap in both data streams with software and check co-registration error | Theory-03Hrs Practical-06Hrs Total-09Hrs | - Updated software - Retroreflective calibration target material / panel - Marker tape - Floor marking / alignment tape - Chalk / marking pen - Zip ties - Electrical insulation tape - Protective covers - Calibration and validation checklists - Paper sheets - Cleaning wipes - Camera lens cleaner - Microfiber cleaning cloths - Labels / identification tags | Classroom |

## Perform HV Safety Isolation & Lockout-Tagout (LOTO)

Objective: This module covers the knowledge and skills required to Establish a High-Voltage Exclusion Zone, Remove Switch by Manual Service Disconnect (MSD), Implement Lockout-Tagout (LOTO) Techniques, Verify Absence of High Voltage (Live-Dead-Live Test) and Discharge Residual High-Voltage Capacitors

Theory: 12 Hours				Practice:  33 Hours 					Duration:  45 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Establish High-Voltage Exclusion Zone | - Allocate designated EV bays and ensure clear marking as per handling of High Voltage Standard Operating Procedures (SOP) during training sessions. - Confirm safety fences are in place and secure before starting work on the EV vehicle to isolate the area and prevent unauthorized access. - Confirm HV-rated gloves are available and in good condition before starting work. - Confirm HV-rated non-conductive footwear is available and in good condition before starting work. - Remove any metal jewelry or accessories - Confirm HV system is de-energized - Confirm voltage meter with a High-Voltage probe (like Fluke HV probe) is available to verify circuit de-energization. - Confirm availability of HV warning signs or labels - Clear workspace of any flammable materials - Confirm good ventilation - Confirm availability of non-conductive work mat. - Installed Lockout/Tagout (LTO) Procedure protocols SOP - Confirm availability of HV-specific tools and equipment - Display a detailed buddy system protocol in the working bay, outlining specific guidelines for HV work, including communication procedures and emergency response steps, to ensure safe working practices. - Confirm Safety hook is readily available in case of emergency. | - Safety precaution during working on EV. - Inspection of HV tools & equipment. - Importance of EV high-voltage risks management. - De-energization procedures. Practical Activity - Prepare an EV high-voltage bay apply safety procedure and de-energize the HV system as per SOPs. | Theory-2Hrs Practical-6Hrs Total- 8Hrs | HV‑rated PPE Warning and HV signage OEM service manual LOTO kit Insulated HV tools HV‑rated multimeter | Class Room and workshop |
| Remove Switch by Manual Service Disconnect (MSD) | - Confirm vehicle is secured by engaging the parking brake and shifting into park (automatic) or neutral (manual) gear before commencing work. - Perform de-energization of the HV system. - Confirm HV system de-energization and confirm absence of voltage (EOV) before commencing work. - Locate the Manual Service Disconnect (MSD) switch, typically situated adjacent to the high-voltage battery pack or HV connector assembly. - Remove the MSD in accordance with OEM-specified procedures and guidelines. - Re-confirm HV system de-energization via voltage detection post-work. - inspect HV system components for damage or wear post-work. - Document the MSD removal procedure and note any observations in the vehicle service record. | - De-energizing through Manual Service Disconnection (MSD) the HV system as per manual. - Confirmation of zero voltage after HV isolation. - Documentation of MSD removal, HV isolation, and test results. Practical Activity Secure vehicle, isolate HV via MSD, verify zero voltage. | Theory-2Hrs Practical-6Hrs Total- 8Hrs | HV‑rated PPE Wheel chocks / parking brake OEM service manual MSD removal tool (if required) Lockout/Tagout (LOTO) kit HV‑rated multimeter and test leads Inspection checklist / service record form | Class Room and workshop |
| Implement Lockout-Tagout (LOTO) Techniques | - Check specific manufacturer's service manual for the EV model. - Put on insulating gloves (rated for 1000V, Class 0 or higher) and leather protectors. - Put cones, warning tape, and signs to designate a high-voltage work area. - Remove watches, rings, and jewelry to prevent short circuits - Turn off the ignition, turn off the HV system, and remove the key/proximity fob (store it at least 15 feet away). - Disconnect the negative terminal of the auxiliary 12V battery to prevent accidental HV relay activation. - Locate the HV Service Disconnect (Refer service manual). - Unlock and remove the orange plug as per service manual instructions. - Secure the removed Service Plug inside a lockbox or with a specialized LOTO cover over the plug socket. - Apply your personal padlock and tag to the lockbox, ensuring it cannot be re-energized by others - Wait for the required period as per given instructions in service manual (typically 5–10 minutes) for inverter capacitors to discharge. - Perform a voltage verification test using a voltmeter: first, test the meter on a known live source to confirm functionality; then, measure voltage at the HV battery terminals. - Measure voltage between HV positive and ground, and HV negative and ground | - Introduction to PPE and warning controls in the HV work area. - HV isolation and service disconnect as per manual manuals. - Isolation EV HV system shutdown and LOTO. - HV de‑energization after the required discharge time. Practical Activity - Prepare HV work area, isolate HV system, allow discharge, and verify zero voltage. | Theory-03Hrs Practical-06Hrs Total- 09Hrs | HV‑rated PPE. Warning cones and HV signage. Service/repair manual. Manual Service Disconnect (MSD) access tools. Lockout/Tagout (LOTO) kit. HV‑rated multimeter and test leads. | Class Room and workshop |
| Verify Absence of High Voltage (Live-Dead-Live Test) | - Test your meter on a known live source (like the 12V auxiliary battery or a dedicated proving unit) to confirm the meter and leads are functioning correctly. - Test the target HV terminals where you intend to work. A reading of 0V confirms the system is powered down. - Perform a post-test verification on the known live source to validate meter functionality. - Keep one hand in your pocket when working with high voltage components to minimize risk of cardiac shock via a hand-to-hand path. - Inspect test leads for damage (cracks, burns, exposed wire). - Perform a continuity check by touching leads together on the Ω setting; expected reading is approximately 0 Ω. An OL (open loop) indication signifies faulty leads. - Perform an air-test on insulated gloves before use: trap air inside, roll the cuff, and inspect for leaks. | - Introduction to Safe HV work practices. - HV de‑energization confirmation by zero‑voltage testing. - Verification of live source with test meter. Practical Activity - Verify PPE and equipment, follow safe HV practices, and confirm zero HV voltage. | Theory-3Hrs Practical-9Hrs Total- 12Hrs | HV‑rated PPE. HV‑rated test meter and leads. Service/repair manual. Insulated HV hand tools. | Class Room and workshop |
| Discharge Residual High-Voltage Capacitors | - Check the repair manual to identify the capacitor bank and discharge points - Connect the discharge tool to the capacitor terminals - Monitor voltage drops; wait until <60V or manufacturer's threshold. - Confirm 0 V with a multimeter before servicing | - Discharging of capacitor bank as per manual. - Safety limit of Capacitor voltage. Practical Activity - Perform discharge capacitors as per manual and verify zero voltage. | Theory-2Hrs Practical-6Hrs Total- 8Hrs | HV‑rated PPE. Service/repair manual. Capacitor discharge tool. Lockout/Tagout (LOTO) kit. HV‑rated multimeter and test leads. | Class Room |

## Repair On-Board Charging System (OBC)

Objective: This module covers the knowledge and  skills required to Analyse AC to DC conversion failures, Replace defective On-Board Charger (OBC) unit, Test Charging Connector proximity and pilot signals, Inspect charging port high-voltage pins for arcing and Configure Charging Control Module (CCM) software

Theory: 10 Hours				Practice:  33 Hours 					Duration:  43 Hours

| Learning Unit | Learning outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Analyse AC to DC conversion failures. | - Analyze charging complaints and identify possible causes. - Allocate designated EV bays and ensure clear marking as per handling of High Voltage Standard Operating Procedures (SOP) during training sessions. - Confirm safety fences are in place and secure before starting work on the EV vehicle to isolate the area and prevent unauthorized access. - Analyse the Diagnostic Trouble Codes (DTCs). - Confirm and check fuses, relays, and proper connections of Power, Neutral, and Earth (L, N & Ground). - Confirm and analyze the AC input supply. - Confirm and analyze the DC output parameters. - Assess thermal behavior and inspect power electronics, including the AC/DC converter circuit (rectifier circuit, capacitors, and PCB-circuit board). - Confirm the AC/DC integrated charging socket wiring harness assembly and check for interlock or short circuit failures. - Compare readings under different environmental conditions Compare measured values with OEM specifications and confirm the root cause | - Electrical supply parameters & charging faults - AC to DC conversion fault diagnosis - Root cause specification and comparison - Components of AC input, DC output, and converter Practical Activity Perform thermal behavior and inspect power electronics, including the AC/DC converter circuit | Theory-2Hrs Practical-6Hrs Total- 8Hrs | - Cleaning cloths - Electrical insulation tape - Cable tags - Marker pens - Disposable gloves - Fault report - Printed OEM specification sheets - Fuse wire | Class Room/ workshop |
| Replace defective On-Board Charger (OBC) unit | - Retrieve and analyze Diagnostic Trouble Codes (DTCs) related to the OBC system - Apply High Voltage (HV) safety measures, including use of PPE and proper isolation. - Power off the vehicle and disconnect the 12V auxiliary battery - Disconnect High Voltage system and wait for the specified capacitor discharge time. - Inspect OBC wiring harness, connectors, and cooling lines for any damage or leakage. - Remove safely the defective OBC unit as per OEM service manual. - Install the new OBC unit and ensure proper mounting and torque specifications. - Reconnect all electrical connectors and cooling pipes securely. - Refill coolant (if applicable) and ensure no leakage in the system. - Clear all fault codes with a diagnostic scanner and perform system reset. - Perform practical verification by testing AC charging operation and confirming normal system performance. | - Knowledge of connector, cooling line, and system reconnection - HV system isolation and shutdown requirements - OBC faults removal and replacement techniques - Importance of charging verification after OBC replacement Practical Activity Replace defective On-Board Charger (OBC) unit. | Theory-2Hrs Practical-6Hrs Total- 8Hrs | - Coolant fluid - Cleaning cloths - Electrical insulation tape - Cable tags - Marker pens - Disposable gloves - Job card | Class Room and workshop |
| Test Charging Connector proximity and pilot signals | - Analyze Possible Causes of Charging Failure and component identification. - Apply High Voltage (HV) safety measures and confirm proper PPE usage. - Inspect charging connector physically for damage, dirt, or bent pins. - Confirm proper connection between the charging gun and vehicle inlet. - Check Proximity (PP) signal to confirm correct cable connection and locking status. - Measure and analyze the Pilot (CP) signal waveform and voltage levels. - Confirm communication between EV and charger through pilot signal response. - Inspect wiring harness and connector pins for continuity, short circuit, or open circuit. - Compare measured PP and CP values with OEM specifications. - Identify faults such as signal interruption, incorrect voltage, or communication failure. - Confirm proper charging initiation and termination through signal validation. | - Charging failure causes and safety requirements - Charging connector condition and connection status - Analys’s proximity and pilot signal values - Signal faults and OEM specification comparison Practical Activity Inspect wiring harness and connector pins for continuity, short circuit or open circuit | Theory-2Hrs Practical-6Hrs Total- 8Hrs | - Electrical contact cleaner - Disposable gloves - fault report forms - Insulation tape | Class Room and workshop |
| Inspect charging port high-voltage pins for arcing | - Confirm the vehicle is powered off and the HV system is isolated before inspection. - Inspect charging port HV pins for signs of arcing, such as burn marks, discoloration, or pitting. - Check melted insulation, carbon deposits, or damaged pin surfaces. - Inspect connector alignment and ensure pins are not bent, loose, or recessed. - Confirm cleanliness of the charging port (no dust, moisture, or foreign particles). - Check overheating signs around the connector housing. - Inspect mating surface of charging gun/connector for similar damage. - Compare findings with OEM standards and allowable limits. - Identify root cause (poor contact, loose connection, contamination) | - Types of faults in HV charging ports - Arcing signs on HV connector pins and surfaces - Connector alignment, cleanliness, and overheating signs - Root cause finding with OEM limits Practical Activity Inspect the EV charging port high-voltage pins for arcing and confirm the fault condition. | Theory-2Hrs Practical-9Hrs Total- 11Hrs | - Electrical contact cleaner - Disposable gloves - Job sheet/fault report forms - Digital multimeter - Hand tools set | Class Room and workshop |
| Configure Charging Control Module (CCM) software | - Confirm requirement for CCM configuration (software update, replacement, or fault rectification). - Connect approved diagnostic tool and ensure stable communication with the vehicle. - Confirm vehicle battery voltage (12V & HV) is within specified charging range before programming. - Confirm stable power supply (support charger) to avoid interruption during software configuration. - Retrieve and record existing software version and configuration data. - Download and select the correct OEM-approved CCM software version. - Perform CCM software configuration/programming as per OEM procedure. - Monitor programming process and confirm no interruption or communication loss. - Clear all Diagnostic Trouble Codes (DTCs) after programming, - Perform system reset and reinitialization of the CCM. - Confirm proper communication between CCM and other vehicle systems. - Perform practical test (AC/DC charging) to confirm correct operation. - Compare system parameters with OEM specifications. | - Importance of system communication and charging operation verification - Procedure of CCM software configuration and programming - Techniques of diagnostic connection, reset and DTC clearing Practical Activity Perform CCM software configuration/programming communication and charging operation with OEM standards | Theory-2Hrs Practical-6Hrs Total- 8Hrs | - Job sheet - Marker pens - Cable tags - PPE kit | Class Room/Workshop |

## Perform Sensor and actuator Calibration

Objective: This module covers the knowledge and  skills required to Flash Vehicle Control Unit (VCU) with updated firmware, Calibrate Accelerator Pedal Position Sensor (APPS), Perform zero-point calibration of steering angle sensor (SAS), Perform BMS State-of-Health (SOH) recalibration and Synchronize motor rotor position sensor (Resolver)

Theory: 10 Hours				Practice:  33 Hours 					Duration:  43 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Flash Vehicle Control Unit (VCU) with updated firmware | - Collect vehicle details, VCU specifications, VIN, and OEM firmware update requirements. - Review technical documentation to confirm correct firmware version, compatibility, and update procedure. - Detect availability and calibration/status of diagnostic tools, flashing device, and stable power supply. - Connect an external power source or battery support unit to maintain stable voltage during flashing. - Power down the vehicle as required and prepare the system for firmware update mode. - Apply safety precautions and wear appropriate PPE as per workshop standards. - Connect the diagnostic/flashing tool to the vehicle communication interface (OBD/VCU port). - Establish secure communication between the tool and the Vehicle Control Unit (VCU). - Backup existing firmware and record current version details before initiating the update. - Select and load the correct firmware file from OEM-approved source. - Initiate the flashing process following OEM sequence and monitor progress continuously. - Maintain stable connection and power supply throughout the flashing process to avoid interruption. - Confirm successful installation to update firmware version in the system. - Perform post-flash diagnostics, clear DTCs, and conduct functional tests to validate system operation. - Record updated details (firmware version, date/time, tool used, operator details), capture logs, and store records in a traceable system. | - Introduction to VIN, OEM and its importance - Firmware software updating procedure and its types - Firmware file handling and verification - Function of VCU - VCU flashing technique - Safety compliance while flashing and updating Practical Activity Install the updated firmware software in vehicle VCU | Theory-2Hrs Practical-6Hrs Total- 8Hrs | Camera / Mobile Device Printer/Scanner Tool Calibration Certificates Software License & Update Access Firmware Repository Access (OEM) Digital Record/PLM System Access | Class Room and workshop |
| Calibrate Accelerator Pedal Position Sensor (APPS) | - Collect vehicle details, VIN, APPS specifications, and OEM calibration procedures. - Review technical documentation to identify calibration parameters, voltage ranges, and acceptance limits. - Confirm availability and calibration status of diagnostic scanner and measurement tools. - Power down the vehicle if required and prepare the system for calibration mode. - Apply safety precautions and wear appropriate PPE as per workshop standards. - Connect the diagnostic scanner to the vehicle and establish communication with the control unit. - Inspect the accelerator pedal assembly and sensor for physical damage, loose connections, or contamination. - Turn ignition ON (without starting the vehicle) as per OEM procedure. - Access APPS calibration or relearn function through the diagnostic tool. - Perform initial (zero/throttle closed) position calibration following OEM instructions. - Depress the accelerator pedal gradually to full position and perform full-scale calibration. - Monitor real-time sensor signals (voltage/percentage) for smooth and linear response. - Compare measured values with OEM specifications and confirm calibration within tolerance. - Perform functional test by operating the pedal and verifying correct system response. - Record calibration data (values, tool used, operator details), capture diagnostic logs if required, and store records in a traceable system. | - Working principal APPS - Function of Accelerator Pedal Position Sensor (APPS) - APPS specification and vehicle detail - Calibration and parameters of APPS - Calibration techniques of APPS - Diagnosing procedure for checking pedal position sensor - Comparison of recorded calibration data with OEM standard value - Safety compliance while working with APPS Practical Activity Scan and calibrate the faulty Accelerator Pedal Position Sensor through diagnostic tools | Theory-2Hrs Practical-6Hrs Total- 8Hrs | Camera / Mobile Device Printer/Scanner Tool Calibration Certificates Software License & Update Access Digital Record/PLM System Access Cable Ties & Harness Clips Cleaning Wipes / Lint-Free Cloth | Class Room and workshop |
| Perform zero-point calibration of steering angle sensor (SAS) | - Collect vehicle details, VIN, steering system specifications, and OEM SAS calibration procedures. - Review technical documentation to identify zero-point calibration requirements and acceptance criteria. - Confirm availability and calibration status of diagnostic scanner and required tools. - Position the vehicle on a level surface with wheels aligned straight ahead. - Power down the system if required and prepare the vehicle for calibration mode. - Apply safety precautions and wear appropriate PPE as per workshop standards. - Connect the diagnostic scanner to the vehicle and establish communication with the control module. - Inspect steering components and sensor connections for damage, misalignment, or faults. - Turn ignition ON (engine OFF) as per OEM procedure. - Access steering angle sensor (SAS) calibration or zero-point reset function via the diagnostic tool. - Execute zero-point calibration while maintaining the steering wheel in the straight-ahead position. - Monitor calibration process and confirm successful completion through the diagnostic tool. - Confirm sensor output values (angle reading near zero) and compare with OEM specifications. - Perform practical checks by turning the steering wheel left and right to confirm accurate response. - Record calibration results (values, tool used, operator details), capture diagnostic logs if required, and store records in a traceable system. | - Working principal and function of Steering Angle Sensor (SAS) - Inspection techniques of steering components and connection - Calibration techniques of SAS - Handling the recorded calibration data - Safety compliance while working in steering system Practical Activity - Inspect the steering system for faulty Steering Angel Sensor - Change the faulty steering angle sensor and monitor calibration process through the diagnostic tool | Theory-2Hrs Practical-6Hrs Total- 8Hrs | Camera / Mobile Device Printer/Scanner Tool Calibration Certificates Software License & Update Access Digital Record/PLM System Access Cable Ties & Harness Clips Cleaning Wipes / Lint-Free Cloth | Class Room and workshop |
| Perform BMS State-of-Health (SOH) recalibration | - Collect vehicle details, VIN, battery specifications, and OEM BMS SOH recalibration procedures. - Review technical documentation to identify recalibration method, required conditions, and acceptance criteria. - Confirm availability and calibration status of diagnostic tools, battery cycler and data logging equipment. - Connect the vehicle to a stable power supply or charging system as specified by OEM. - Apply safety precautions and wear appropriate PPE for high-voltage systems. - Connect the diagnostic scanner to the vehicle and establish communication with the Battery Management System (BMS). - Retrieve and record initial battery data including SOH, SoC, voltage, current, temperature, and DTCs. - Inspect battery system status and confirm no active faults that may affect recalibration. - Initiate BMS SOH recalibration function or guided routine through the diagnostic tool. - Perform controlled charge and/or discharge cycle as required by OEM procedure. - Monitor real-time battery parameters during the recalibration process for stability and safety. - Allow the BMS to relearn capacity and update SOH values based on measured data. - Confirm updated SOH value and compare with expected range or OEM specifications. - Perform post-recalibration checks, clear DTCs if necessary, and confirm normal system operation. - Record recalibration details (initial/final SOH, parameters, tool used, operator details), capture logs, and store records in a traceable system. | - Function and working of BMS - Introduction to state of health and state of charge (SOH,SOC) - Scanning procedure of BMS recalibration - Process of recalibration BMS - SOPs for battery management system Practical Activity Perform State-of-Health (SOH) recalibration with diagnostic tools and verify battery performance | Theory-2Hrs Practical-9Hrs Total- 11Hrs | Isopropyl Alcohol Dielectric Grease Camera / Mobile Device Printer/Scanner Tool Calibration Certificates Software License & Update Access Digital Record/PLM System Access Cable Ties & Harness Clips Cleaning Wipes / Lint-Free Cloth | Class Room and workshop |
| Synchronize motor rotor position sensor (Resolver) | - Collect vehicle details, VIN, motor specifications, and OEM resolver synchronization procedures. - Review technical documentation to identify resolver calibration method, reference angles, and acceptance criteria. - Confirm availability and calibration status of diagnostic tools, oscilloscope (if required), and interface equipment. - Connect the vehicle to a stable power supply or auxiliary support unit as specified by OEM. - Power down the system where required and prepare the vehicle for safe calibration mode. - Apply safety precautions and wear appropriate PPE for high-voltage systems. - Connect the diagnostic tool to the vehicle and establish communication with the motor control unit (MCU/inverter). - Inspect resolver wiring, connectors, and shielding for damage, looseness, or interference. - Check for existing fault codes related to resolver or motor position and record them. - Clear relevant DTCs if required and confirm system readiness for synchronization. - Access resolver calibration or synchronization function through the diagnostic tool. - Align the motor rotor to the reference position as specified by OEM procedure. - Initiate resolver offset learning or synchronization routine via the control unit. - Monitor resolver signals (sine/cosine outputs) and verify correct phase alignment and signal integrity. - Confirm calculated rotor position matches expected reference within OEM tolerance limits. - Perform functional test by rotating the motor (or simulation) to verify accurate position tracking. - Confirm system performance, confirm no fault codes are present, and verify stable operation. - Record synchronization data (offset values, tool used, operator details), capture diagnostic logs, and store records in a traceable system. | - Method of rotor positioning - Power down procedure - Inspection of resolver wiring - Importance of rotor position values - Scanning procedure for fault detections and rectification Practical Activity Perform resolver synchronization/calibration and verify accurate rotor position | Theory-2Hrs Practical-6Hrs Total- 8Hrs | Isopropyl Alcohol Dielectric Grease Camera / Mobile Device Printer/Scanner Tool Calibration Certificates Software License & Update Access Digital Record/PLM System Access Cable Ties & Harness Clips Cleaning Wipes / Lint-Free Cloth | Class Room and workshop |

## Perform HV Component Sealing and Ingress Protection (IP) Rating Validation

Objective: This module covers the knowledge and  skills required to Perform vacuum decay test on battery enclosure, Perform vacuum decay test on e-drive system, Inspect high-voltage connector O-rings for degradation and Apply specialized mastic sealant to battery tray

Theory: 10 Hours				Practice:  33 Hours 					Duration:  43 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Perform vacuum decay test on battery enclosure | - Collect vehicle details, battery enclosure specifications, and OEM vacuum decay test procedures. - Review technical documentation to identify test pressure limits, decay thresholds, duration, and acceptance criteria. - Confirm availability and calibration status of vacuum decay tester, pressure sensors, and data acquisition system. - Inspect test setup components including hoses, fittings, seals, and adapters for integrity and compatibility. - Power down the vehicle, isolate the HV battery system, and secure the work area. - Apply safety precautions and wear appropriate PPE for high-voltage and pressure testing. - Clean the battery enclosure surface and sealing interfaces to remove contaminants. - Inspect enclosure, gaskets, and sealing areas for visible damage, deformation, or leakage paths. - Seal all openings and connect the vacuum decay test equipment to the designated test port. - Evacuate air from the enclosure to reach the specified vacuum level as per OEM requirements. - Stabilize the system for the defined settling time to eliminate transient pressure variations. - Isolate the enclosure from the vacuum source and initiate the decay measurement phase. - Monitor and record pressure change over the specified test duration using calibrated sensors. - Calculate vacuum decay rate and compare with defined leak rate or pressure loss limits. - Identify potential leakage if pressure decay exceeds acceptable thresholds. - Perform secondary inspection or leak localization (if required) using approved methods. - Regulate Pass/Fail status based on OEM specifications and document any abnormalities. - Record complete test data (initial/final pressure, decay rate, duration, equipment used, operator details), capture logs/graphs, and store records in a traceable system | - Vacuum decay test requirements - Verification of test equipment readiness - Inspection of test setup components - Implementation of safety and isolation procedures - Preparation of battery enclosure for testing - Specification Review and Test Planning for battery enclosure - Establishing vacuum conditions - Stabilizing and initiating decay test - Monitoring and evaluating pressure decay Practical Activity Check sealing integrity of battery enclosure and detect leakage with vacuum decay method | Theory-02Hrs Practical-06Hrs Total-08Hrs | - Documentation Material - Seals - Electrical Cleaning Solvent - Gaskets - Lint Free Wipes/Cloth | Class Room and workshop |
| Perform vacuum decay test on e-drive system | - Collect vehicle details, e-drive system specifications, and OEM vacuum decay test procedures. - Review technical documentation to identify test pressure limits, decay thresholds, duration, and acceptance criteria. - Confirm availability and calibration status of vacuum decay tester, pressure sensors, and data logging system. - Inspect test equipment, hoses, fittings, and adapters for integrity and compatibility with the e-drive system. - Power down the vehicle, isolate the HV system, and secure the work area. - Apply safety precautions and wear appropriate PPE for high-voltage and pressure testing. - Clean external surfaces and sealing interfaces of the e-drive system to remove dirt, oil, or contaminants. - Inspect housing, seals, gaskets, and connectors for visible damage, wear, or potential leakage points. - Seal all openings and connect the vacuum decay tester to the designated test port of the e-drive system. - Evacuate air from the system to achieve the specified vacuum level as per OEM requirements. - Stabilize the system for the defined settling time to eliminate transient pressure fluctuations. - Isolate the system from the vacuum source and initiate the decay measurement phase. - Monitor and record pressure variation over the specified test duration using calibrated instruments. - Calculate vacuum decay rate and compare results with OEM-defined leakage limits. - Identify and document any excessive pressure loss indicating potential leakage. - Perform secondary inspection or leak localization procedures if leakage is detected. - Regulate Pass/Fail status based on comparison with OEM specifications and standards. - Record complete test data (initial/final pressure, decay rate, duration, equipment used, operator details), capture logs/graphs, and store records in a traceable system. | - Test Planning of E-Drive system - Equipment preparation for E-Drive setup - Techniques for Vacuum Decay Test Execution on e-drive system - Analysis, Leak Detection on e-drive system and Documentation Practical Activity Perform vacuum decay test on EV battery enclosure | Theory-03Hrs Practical-09Hrs Total- 12Hrs | - Documentation Material - Cooling Line - Oil Circuits - Housings - Lint Free Wipes/Cloth | Class Room and workshop |
| Inspect high-voltage connector O-rings for degradation | - Identify the high-voltage connector location and verify system isolation before inspection. - Disconnect power sources and confirm zero voltage using appropriate testing equipment. - Wear required personal protective equipment (PPE) such as insulated gloves and safety goggles. - Access the connector housing carefully to avoid damaging surrounding components. - Remove the connector following manufacturer-recommended procedures. - Locate the O-rings within the connector assembly. - Clean the O-rings and surrounding surfaces using approved, non-reactive cleaning agents. - Inspect the O-rings for cracks, cuts, or surface irregularities. - Check for signs of hardening, brittleness, or loss of elasticity. - Inspect the O-rings for swelling, deformation, or compression set. - Inspect for chemical degradation such as discoloration or residue buildup. - Measure O-ring dimensions and compare them with manufacturer specifications. - Confirm proper seating and alignment of O-rings within their grooves. - Assess environmental factors such as heat exposure, moisture, or contamination. - Adjust whether O-rings meet operational standards or require replacement. - Replace degraded O-rings with approved, specification-compliant components if necessary. - Reassemble the connector and ensure proper sealing and secure fitment. - Document findings, actions taken, and final condition in the assessment report. | - Safety isolation and verification - Accessing of disassemble connector assembly - Identification and preparation of O ring components - Physical condition of O rings - Evaluation of deformation and chemical effects - Dimensional and installation accuracy - Assessment of environmental impact factors - Serviceability and replacement Practical Activity Check and identify sealing integrity of O rings damage, wear, or degradation. | Theory-02Hrs Practical-09Hrs Total- 11Hrs | - O-rings - Compressed Air - Moisture barrier Spray - Lint -Free Wipes/Swab | Class Room and workshop |
| Apply specialized mastic sealant to battery tray | - Review manufacturer specifications and safety guidelines for applying mastic sealant to the battery tray. - Isolate the battery system and disconnect all power sources before starting the task. - Wear personal protective equipment (PPE) including gloves, goggles, and protective clothing. - Inspect the battery tray for dirt, corrosion, moisture, or existing sealant residue. - Clean the battery tray surface using approved cleaning agents to ensure proper adhesion. - Remove old or degraded sealant completely using suitable tools without damaging the tray. - Dry the battery tray thoroughly to eliminate any moisture. - Inspect the tray for cracks, deformation, or structural damage prior to sealant application. - Select the correct specialized mastic sealant as per manufacturer recommendations. - Prepare the sealant according to product instructions (mixing or conditioning if required). - Load the sealant into an appropriate application tool such as a caulking gun. - Apply the mastic sealant evenly along designated seams and joints of the battery tray. - Confirm continuous and uniform bead formation to avoid gaps or air pockets. - Smooth and press the sealant into place to enhance adhesion and sealing effectiveness. - Inspect the applied sealant for consistency, coverage, and proper thickness. - Allow the sealant to cure as per specified curing time and environmental conditions. - Reassemble any removed components and reconnect the battery system safely. - Document the application process, materials used, curing time, and final inspection results in the assessment report. | - Selection and preparation of sealant material - Application of sealant with correct technique - Controlling curing process and sealing quality - Reassembling system and complete documentation - Inspection of battery tray condition - Isolation of battery system and ensure safe work condition Practical Activity Inspect, prepare, and apply mastic sealant on battery tray seams | Theory-03Hrs Practical-09Hrs Total- 12Hrs | - Documentation Material - Specialized Mastic Sealant - Watertight Bond - Lint Free Wipes/Cloth | Class Room and workshop |

## Maintain Regenerative Braking System

Objective: This module covers the knowledge and  skills required to Calibrate Electronic Brake Pedal Position Sensor (BPPS), Evaluate Blending Efficiency Between Friction and Regen, Test Motor-Generator Torque Response During Coasting, Inspect Regenerative Braking Cooling requirements and Update ABS/ESC Module for Optimized Energy Recovery

Theory: 09 Hours				Practice:  33 Hours 					Duration:  42 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Calibrate Electronic Brake Pedal Position Sensor (BPPS) | - Connect the OEM diagnostic platform to the vehicle CAN interface and navigate to the BPPS calibration function. - Check brake pedal is at the fully released rest position and confirm zero-point reading matches OEM specification before initializing calibration. - Apply a series of pedal inputs at OEM-defined incremental positions and record the corresponding sensor output voltage or signal values. - Compare recorded BPPS output values against the OEM calibration curve and confirm all data points fall within the specified tolerance band. - Perform a full-stroke pedal test and verify the BPPS signal transitions smoothly from zero to maximum without dropout, noise, or non-linearity. - Clear all BPPS-related DTCs and confirm zero active codes on a final diagnostic scan. - Document calibration parameter values and sensor ID on the job card and obtain assessor sign-off. | - Principle and Function of brake pedal position sensor - CAN Interface Techniques and OEM Comparison. - Procedure of Pedal Signal Monitoring and Recording - Calibration Procedure of BPPS (Brake Pedal Position Sensor) - BPPS Parameters as per OEM. - Diagnostic Trouble Codes (DTCs) related to BPPS Practical Activity: Perform full-stroke pedal test and verify the BPPS signal transitions . | Theory-01Hrs Practical-06Hrs Total- 07Hrs | - OEM diagnostic scanner - Multimeter - Wiring diagram - Data recording sheet or laptop - Basic hand tools - PPE, gloves, safety shoes | Class Room and workshop |
| Evaluate Blending Efficiency Between Friction and Regen | - Retrieve and record all active and stored DTCs related to the brake blending control module using the OEM diagnostic platform. - Access the live data stream and monitor regen torque demand, friction brake pressure, and vehicle deceleration simultaneously during a controlled low-speed stop. - Analyze the regen-to-friction brake contribution ratio from the captured data and compare it against the OEM-specified blending efficiency target. - Conduct a graduated brake application test at three speed thresholds defined by OEM and verifies the blend transition point occurs within the OEM-specified speed window. - Identify any blending inefficiency from the test data and document the root cause and corrective action on the fault analysis form. - Clear resolved DTCs and record blending efficiency test results on the completed job card. | - Principle of Brake Blending - Monitoring Blending Performance - Efficiency Evaluation Techniques - Fault Identification and Improvement Practical Activity: Conduct a graduated brake application test at three speed thresholds | Theory-2Hrs Practical-6Hrs Total- 8Hrs | - OEM diagnostic scanner - Data logging tool or laptop - Multimeter - Test vehicle with regen system - Safe test track or controlled road - PPE | Class Room and workshop |
| Test Motor-Generator Torque Response During Coasting | - Configure the OEM diagnostic platform to monitor motor-generator torque output, rotational speed (RPM), and DC bus voltage simultaneously in real time. - Conduct a coasting test by releasing the accelerator at OEM-specified vehicle speed and record the initial torque response of the motor-generator. - Measure the torque rise time from zero to OEM-specified peak regen torque and confirm it is within the OEM-defined response time tolerance. - Confirm DC bus voltage rises within the OEM-acceptable range during the coasting regen event without triggering over voltage protection. - Repeat coasting test at two additional speed thresholds specified by OEM and record torque response data for each. - Compare all recorded torque response data against OEM performance benchmarks and identify any deviation outside tolerance. - Document test findings and any identified deviations on the job card and present results to the assessor. | - Configuration and management of real time diagnostic monitoring - Techniques to monitor key system parameters Practical Activity: Measures the torque rise time from zero to OEM-specified | Theory-02Hrs Practical-06Hrs Total- 08Hrs | - Disposable gloves - Cleaning cloths - Workshop wips - Electrical contact cleaner - Dielectric grease - OEM diagnostic scanner - Data logging tool or laptop - Test vehicle with motor generator system - Safe test road or track - PPE | Class Room and workshop |
| Inspect Regenerative Braking Cooling requirements | - Retrieve all active DTCs related to the motor-generator and power electronics cooling circuit using the OEM diagnostic platform. - Inspect cooling circuit hoses, fittings, radiator, and pump for leakage, damage, or blockage. - Check coolant level and condition against OEM specification and record the result - Monitor motor-generator coolant inlet and outlet temperatures using the diagnostic platform during a simulated regen load event and confirm the delta-T is within OEM limits. - Check coolant pump operation by confirming OEM-specified flow rate or pump duty cycle is achieved at regen load conditions. - Inspect power electronics (inverter and motor controller) heatsink for contamination, corrosion, or fin damage. - Document all cooling system inspection findings and recommended maintenance actions on the job card. | - Purpose of Regenerative Braking Cooling - Components of Cooling System - Cooling Performance Monitoring - Fault Identification in Cooling System Practical Activity: Perform the following test of cooling system; - Pressure test - Electrolysis test - Thermal test - Cooling pump operation test | Theory-2Hrs Practical-9Hrs Total- 11Hrs | - OEM diagnostic scanner - Infrared thermometer or thermal camera - Coolant pressure tester - Multimeter - Basic hand tools - PPE | Class Room and workshop |
| Update ABS/ESC Module for Optimized Energy Recovery | - Confirm ABS/ESC module hardware version and software version compatibility with the target calibration file using OEM fitment data. - Connect a battery support unit and verify stable supply voltage (≥12.5 V) before initiating any module programming or calibration write operation. - Create a DoIP or CAN programming connection to the ABS/ESC module and initiate the OEM-specified software update sequence without interrupting the cycle. - Perform a wheel speed sensor plausibility check by rolling the vehicle at low speed and confirming all four-wheel speed signals are present and within tolerance on the diagnostic platform. - Conduct a controlled ABS stop and verify regen is correctly cut off at the OEM-defined wheel slip threshold, with friction brakes assuming full authority. - Clear all ABS/ESC-related DTCs, confirm zero active codes on a final scan, and document all programmed parameter values on the job card before assessor sign-off. | - Working principle of ABS and ESC. - Role of ABS and ESC in Energy Recovery. - Verify ABS/ESC Module Compatibility Before Update - Ensure Stable Power Supply for Programming - Scanning procedure of ABS/ESC and record/retrieval initial data - Analysis of recorded data with OEM standard. - Post Update Testing and System Validation Practical Activity: Evaluate braking efficiency and regenerative energy recovery | Theory-2Hrs Practical-6Hrs Total- 8Hrs | - OEM diagnostic and programming tool - Battery support unit (stable voltage supply) - DoIP or CAN interface cable - Test vehicle with ABS and regenerative braking system - Wheel speed monitoring tool - PPE | Class Room |

## Test Insulation Resistance (Megohmmeter)

Objective: This module covers the knowledge and skills required to Calibrate Insulation Tester for High-Voltage Diagnostics, Test internal insulation of HV battery pack, Measure Motor Stator Winding-To-Case Resistance, Conduct Chassis-To-HV-Component Isolation Check and Evaluate High-Voltage Cable Shielding Integrity

Theory: 09 Hours							Practice:  33 Hours 					                Duration: 42 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Calibrate Insulation Tester for High-Voltage Diagnostics | - Select appropriate insulation tester for EV high-voltage system - Inspect insulation tester for physical and functional condition - Set voltage range on megohmmeter according to system requirement - Confirm calibration status of insulation tester before use - Record calibration checks results in test log | - Introduction to insulation tester (megohmmeter) - Selection criteria for insulation tester - Techniques for tester condition of insulation - Calibration procedure of insulation tester Practical Activity Calibrate insulation tester for EV high-voltage diagnostics | Theory-1Hrs Practical-6Hrs Total- 7Hrs | - Cleaning cloth - Isopropyl alcohol wipes - Insulating tape - Cable ties - Disposable gloves | Classroom and workshop |
| Test internal insulation of HV battery pack | - Isolate HV battery pack from vehicle system - Connect insulation tester to battery terminals correctly - Apply test voltage to measure internal insulation resistance - Record insulation resistance values from battery pack - Compare results with acceptable safety limits | - Introduction to HV insulation resistance testing - Isolation procedures for HV battery pack - Megohmmeter operation and test connection procedures - Procedure to test voltage for insulation resistance Practical Activity Test HV Battery Pack and Insulation | Theory-2Hrs Practical-6Hrs Total- 8Hrs | Cleaning cloth Isopropyl alcohol wipes Cable ties Insulating tape Disposable gloves Battery terminal grease Contact cleaner spray | Classroom and workshop |
| Measure Motor Stator Winding-To-Case Resistance | - Isolate electric traction motor from power source - Connect megohmmeter between stator windings and motor casing - Confirm specified test voltage for insulation measurement - Measure and record winding-to-case resistance values - Evaluate insulation condition based on measured results | - Procedure for Isolation of electric motor from power source - Megohmmeter connection between stator windings and motor casing - Voltage selection for insulation measurement - Importance of winding-to-case resistance values - Evaluation techniques for motor insulation conditions Practical Activity Test Motor Stator Winding-to-Case Resistance with Megohmmeter | Theory-2Hrs Practical-6Hrs Total- 8Hrs | Insulation cleaning solvent Electrical insulating tape Contact cleaner spray Disposable gloves Absorbent wipes | Classroom and workshop |
| Conduct Chassis-To-HV-Component Isolation Check | - Disconnect HV system from vehicle chassis ground - Connect insulation tester between chassis and HV components - Apply test voltage to detect leakage paths - Measure insulation resistance between chassis and HV system - Record and assess isolation integrity results | - Isolation testing procedures for Chassis-to-HV component - Isolating techniques for HV system from vehicle chassis ground - Insulation tester connection between chassis and HV components - Voltage leakage detection - Resistance measurement of insulation between chassis and HV system Practical Activity Perform Isolation Check of Chassis-to-HV Components | Theory-2Hrs Practical-9Hrs Total- 11Hrs | Cleaning wipes Electrical contact cleaner Insulating tape Disposable gloves Battery terminal protector spray Record sheets | Classroom and workshop |
| Evaluate High-Voltage Cable Shielding Integrity | - Inspect HV cables for physical damage and wear Execute Chassis-to-HV Component Isolation Check - Connect insulation tester between cable shield and conductor - Apply test voltage for shielding integrity check - Measure insulation resistance of cable shielding system - Record results and determine cable safety condition | - Techniques for visual inspection of HV cables for physical damage and wear - Importance of insulation tester connection between cable shield and conductor - Voltage testing for shielding integrity Practical Activity Perform HV Cable Shielding Integrity Test | Theory-2Hrs Practical-6Hrs Total- 8Hrs | insulation tape cleaning solvent heat shrink tubing contact cleaner marking labels abrasive paper grease sealant compound | Classroom and workshop |

## Maintain Technical Documentation & Compliance Reporting

Objective: This module covers the knowledge and  skills required to Generate a comprehensive High-Voltage (HV) Battery health report, Record isolation test results for safety compliance, Document firmware version history and update logs, Prepare a Root Cause Analysis (RCA) for HV component failures, Maintain workshop hazardous material (Battery) disposal log and Maintain record on repair order

Theory: 09 Hours				Practice:  30 Hours 					Duration:  39 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Generate a comprehensive High-Voltage (HV) Battery health report | - Collect vehicle details, previous maintenance records, and OEM specifications. - Dress Personal Protective Equipment (PPE) - Calibrate diagnostic tools - Inspect HV battery components - Record the inspection observations in assessment/ inspection form. - Capture photographic/Video evidence of any damage or abnormalities. - Connect diagnostic scanner to the vehicle for communication. - Retrieve and record all Diagnostic Trouble Codes (DTCs). - Record real-time battery parameters (SoC, SoH, voltage, current). - Conduct charge and discharge performance tests. - Record system behavior, responses, and any deviations during testing. - Test safety systems (HVIL, insulation, emergency shutdown). - Record measured values and indicate Pass/Fail status. - Compare results with OEM specifications and applicable standards. - Identify deviations, severity, and recommend corrective/preventive actions. - Review findings, complete overall assessment, sign/date the report, and store/share in digital compliance system. | - Vehicle Data Collection - Documentation for HV Battery Inspection, Diagnostic, testing and evaluation - Reporting and Compliance Documentation Practical Activity Generate a comprehensive High-Voltage (HV) Battery health report with the standard of OEM. | Theory-01Hrs Practical-03Hrs Total- 04Hrs | - Coolant - Distilled water - Coolant test strips - leak detection dye - Absorbent pads - Cleaning cloths - Cleaning spray - Disposable nitrile gloves - Inspection tags and labels - Permanent marker | Class Room and workshop |
| Record isolation test results for safety compliance | - Check vehicle is powered down, ignition OFF, key removed, and secured. - Apply Lock-Out/Tag-Out (LOTO) Techniques to prevent accidental energization. - Wear Personal Protective Equipment (PPE) during high-voltage operations. - Verify insulation tester (me ohmmeter) is calibrated and certified for use. - Record vehicle and battery identification details (VIN, model, battery type). - Document ambient conditions (temperature, humidity) and test setup parameters. - Identify and record test equipment details (device name, model, calibration date, test voltage level). - Disconnect the HV battery system as per OEM Techniques for safe isolation. - Measure insulation resistance between, HV positive (+) and chassis ground & HV negative (–) and chassis ground - Apply specified test voltage and maintain for OEM-defined duration. - Record all measured values immediately and accurately. - Evaluate results against OEM specifications and valid safety standards; mark Pass/Fail. - Identify and document abnormalities (low resistance, fluctuating readings) and affected components. - Assign severity level and recommend corrective actions; attach supporting evidence (logs, photos, previous records). - Review compliance status, include inspector details and approvals, and store records in a controlled digital system with audit traceability | - Documentation of LOTO procedures. - Documentation for ambient conditions and test setup parameters. - Insulation testing Procedure and recording of all measured values - Result Evaluation and Documentation. Practical Activity Measure and record insulation Resistance between HV positive and chassis ground, and HV Negative and chassis ground. | Theory-01Hrs Practical-03Hrs Total- 04Hrs | - Insulation test - leads and probes - Cleaning wipes - Labels and - LOTO tags - Markers - Adhesive tapes - Cable ties - Disposable gloves | Class Room and workshop |
| Document firmware version history and update logs | - Collect system identification details (vehicle model, ECU/module name, VIN, software platform). - Retrieve current firmware version from the system using approved diagnostic tools. - Verify firmware authenticity and compatibility with OEM specifications. - Access historical firmware records from maintenance logs or digital systems. - Record previous firmware versions with corresponding release dates. - Document resolution of each firmware updates (bug fix, performance improvement, security patch, feature update). - Record update source (OEM release, authorized service center, OTA update). - Document update method used (wired flashing, OTA, diagnostic tool-based update). - Record pre-update system condition (fault codes, system status, battery level). - Document update execution details (date, time, duration, technician/authority). - Record post-update firmware version and confirm successful installation. - Perform practical validation tests and record system performance after update. - Log any errors, failures, or rollback actions during or after update. - Compare firmware versions with OEM latest release and identify gaps or non-compliance. - Review update history, include approvals (signature/date), and store records in a controlled digital system with traceability. | - System Identification and Data Collection - Firmware Version-Retrieval and Verification - Historical Firmware Record Review - Firmware Update Classification - Documentation of Source Method - Validation, Error Logging, and Compliance Review Practical Activity Record firmware versions and update activity for system tracking, troubleshooting, and compliance. | Theory-01Hrs Practical-06Hrs Total- 07Hrs | - Update logs - Record sheets - Data entry forms - inspection checklists - Labels for firmware version - Cleaning wipes - Disposable gloves - USB drives | Class Room and workshop |
| Prepare a Root Cause Analysis (RCA) for HV component failures | - Collect system and vehicle details (vehicle model, VIN, HV component name, operating conditions). - Follow safety precautions (PPE, HV isolation, Lock-Out/Tag-Out procedures) before inspection. - Identify and document the problem statement clearly (failure type, symptoms, occurrence conditions). - Gather initial evidence including fault codes (DTCs), warning indicators, and system logs. - Review historical data (maintenance records, previous failures, firmware updates, usage patterns). - inspect HV components (battery, cables, connectors, insulation, cooling system). - Conduct diagnostic tests (voltage, current, insulation resistance, thermal readings). - Record all observations, measured values, and deviations from OEM specifications. - Identify causes with designed Techniques (Like 5 Whys, Fishbone/ Ishikawa analysis). - Analyze contributing factors (environmental, electrical, mechanical, software-related issues). - Isolate the root cause by validating suspected causes through testing or evidence correlation. - Assess severity and impact of the failure (safety risk, performance loss, system reliability). - Recommend corrective actions (repair, replacement, recalibration, software update). - Propose preventive actions to avoid recurrence (design improvement, maintenance changes, monitoring). - Document RCA findings, include supporting evidence (photos, logs), obtain approvals, and store report in a traceable system. | - System Data Collection and Safety Preparation. - Problem Identification and Recording evidences - Root Cause Analysis and Validation. - Documentation of Corrective and Preventive Actions . Practical Activity Identify the real cause of HV system failure and document corrective action to prevent repeat issues. | Theory-02Hrs Practical-06Hrs Total- 08Hrs | - Inspection forms - RC worksheet - Printed checklists - Marking labels - Evidence tags - Cleaning wipes - Marker pens - Disposable gloves - Camera print sheets - Diagnostic report printouts - Adhesive tapes | Class Room and workshop |
| Maintain workshop hazardous material (Battery) disposal log | - Identify and categorize the battery as hazardous material (like lithium-ion HV battery, damaged/defective unit). - Collect and record battery details (type, capacity, serial number, vehicle VIN, and condition). - Follow safety precautions (PPE, insulated tools, and fire protection measures) are in place. - Isolate and store the battery in a designated hazardous material area as per safety guidelines. - Inspect the battery for damage, leakage, swelling, or thermal risk and document condition. - Label the battery clearly with hazard warnings and identification tags. - Record date of removal and reason for disposal (end-of-life, damage, failure, recall). - Document temporary storage conditions (location, temperature control, containment measures). - Select an authorized hazardous waste disposal/recycling vendor as per regulations. - Prepare required documentation (waste manifest, transport permits, compliance forms). - Record transportation details (date, transporter name, method, tracking reference). - Follow safe packaging and handling during dispatch (insulated, sealed, compliant with transport standards). - Obtain disposal/recycling confirmation certificate from authorized facility. - Record final disposal method (recycling, treatment, destruction) and compliance status. - Maintain complete disposal log with signatures, approvals, and store records in a traceable system for audits. | - Hazard Identification and Battery Documentation - Battery Data Recording. - Safety Preparation and Compliance. - Condition Inspection and Documentation. - Safety Handling and Storage Control. - Labeling and Traceability. - Disposal Planning and Transportation - Final Disposal and Record Management. Practical Activity Track safe handling, storage, and disposal of HV and conventional batteries. | Theory-02Hrs Practical-06Hrs Total- 08Hrs | - PPE kit - Insulated tools - Battery inspection checklist • Labels and hazard tags - Storage container or isolation box - Fire extinguisher - Repair or disposal log form - Disposal log register - Waste manifest forms - Transport approval documents - PPE kit - Packaging materials - Labels and sealing materials | Class Room and workshop |
| Maintain record on repair order | - Receive and register the vehicle with basic details (customer name, vehicle model, VIN, registration number). - Record customer complaint and service request clearly and accurately. - Assign a unique Repair Order (RO) number for traceability. - Document initial vehicle condition (mileage, battery level, visible damages). - Follow safety checks before work begins (HV isolation if required, PPE compliance). - Allocate job to technician and record technician details. - Perform diagnostic inspection and record findings (fault codes, test results). - Prepare detailed job description including required repairs, parts, and labor. - Obtain customer approval for estimated cost, scope of work, and timeline. - Record all repair activities carried out (step-by-step work performed). - Document parts used (part number, quantity, source, warranty details). - Conduct post-repair testing and quality checks; record results. - Update final job status, total cost, and completion time. - Obtain final customer acknowledgment/signature upon vehicle handover. - Store repair order record in digital/physical system with proper documentation for future reference and audits. | - Vehicle Reception and Registration - Customer Complaint Handling - Initial Inspection and Safety Compliance - Job Allocation and Workflow Control - Diagnostic and Fault Recording - Repair Planning and Approval - Repair Documentation and Final Closure Practical Activity Maintain and Complete Repair Order Documentation for EV Vehicle | Theory-02Hrs Practical-06Hrs Total- 08Hrs | - Job cards - Ink pens and markers - Printer paper - Toner or ink cartridges - Label stickers - Cleaning wipes - Adhesive notes - File folders - workshop software - Diagnostic scanner - PPE kit - Pen, tablet, or computer system | Workshop and Classroom |

## Perform HV Cabling and Connector Specialized Crimping

Objective: This module covers the knowledge and  skills required to Strip high-voltage orange insulation layers, Perform high-pressure hexagonal crimp, Perform terminal pull-test for mechanical strength, Apply adhesive-lined heat shrink tubing and Perform continuity Test of HV connector interlock (HVIL)

Theory: 09 Hours				Practice:  30 Hours 					Duration:  39 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Strip high-voltage orange insulation layers | - Collect vehicle details, HV cable specifications, and OEM stripping guidelines. - Review previous maintenance records and identify HV circuits to be worked on. - Verify availability and calibration of all required tools, PPE, and consumables. - Power down the vehicle, switch off ignition, and remove the key. - Apply Lock-Out/Tag-Out (LOTO) Techniques to prevent accidental energization. - Wear appropriate PPE for HV systems (insulated gloves, sleeves, face shield, protective clothing). - Isolate the HV battery or system as per OEM procedures. - Prepare an insulated work surface and remove conductive objects from the area. - Inspect the HV cable for external damage or prior tampering. - Mark the cable section according to OEM stripping length specifications. - Strip the orange insulation layer using approved HV stripping tools without damaging the conductor. - Verify the exposed conductor length and remove any insulation residues. - Inspect the stripped cable for nicks, cuts, or uneven surfaces. - Record observations, cable ID, vehicle VIN, and operator details in the inspection form. - Capture photographs of the stripped cable sections as supporting evidence. | - Selection of the tools and equipment - Safety and isolation procedures for vehicle repairing - Personal protection and workplace environment setup - Technique’s for HV cable handling and stripping - Inspection and job documentation management Practical activity Inspect the HV cable for external damage and perform stripping | Theory-02Hrs Practical-06Hrs Total- 08Hrs | - Lint free wipes/clothes - Cleaning solvent - Markers - Labeling tags - Cutters - Heat shrinks tubes - Cable ties - Inspection forms | Class Room and workshop |
| Perform high-pressure hexagonal crimp | - Collect vehicle details, cable specifications, terminal type, and OEM crimping requirements. - Review technical manuals to confirm correct crimping dimensions and standards. - Verify availability and calibration status of crimping tools, dies, and measuring instruments. - Power down the system, isolate the HV circuit, and remove ignition key. - Apply Lock-Out/Tag-Out (LOTO) procedures to prevent accidental energization. - Wear appropriate PPE including insulated gloves, eye protection, and protective clothing. - Inspect the cable and terminal lug for damage, corrosion, or contamination. - Strip the cable insulation to the specified length using approved stripping tools. - Clean the conductor strands and remove any debris or oxidation. - Insert the conductor fully into the hexagonal terminal lug as per specification. - Apply correct die size and position the terminal in the crimping tool. - Execute the high-pressure hexagonal crimp using the approved crimping tool. - Inspect the crimped joint for proper shape, uniform compression, and absence of defects. - Measure crimp dimensions and perform pull test (if required) to verify mechanical strength. - Record crimping details (cable ID, tool used, measurements, operator details), capture photographic evidence, and store records in a traceable system. | - Data collection and specification review of the cable - Tooling and equipment preparation of the job - System shutdown and safety control for the task - Techniques for Cable and terminal Crimping - Inspection testing and documentation Practical Activity Perform crimping and inspect joint for proper shape, uniform compression | Theory-02Hrs Practical-06Hrs Total- 08Hrs | - Lint free wipes/clothes - Terminal lugs/connectors - Anti -oxidation compound - Cleaning solvent - Markers - Labeling tags - Heat shrinks tubes - Cable ties - Inspection forms | Class Room and workshop |
| Perform terminal pull-test for mechanical strength | - Collect vehicle details, cable/terminal specifications, and OEM pull-test requirements. - Review technical standards to determine required pull force and acceptance criteria. - Verify availability and calibration status of pull-test equipment (tensile tester/force gauge). - Power down the system, isolate the HV circuit, and remove the ignition key. - Apply Lock-Out/Tag-Out (LOTO) Techniques to prevent accidental energization. - Wear appropriate PPE including insulated gloves, eye protection, and protective clothing. - Inspect the crimped terminal and cable assembly for visible defects or damage. - Mount the cable and terminal securely in the pull-test fixture according to test setup guidelines. - Align the cable and terminal along the axis of the pulling force to avoid side loading. - Apply tensile force gradually using the pull-test equipment. - Increase the force up to the specified limit or until failure occurs as per test requirement. - Detect and record the maximum force applied and the mode of failure (if any). - Compare measured force with OEM specifications and determine Pass/Fail status. - Document any abnormalities such as slippage, conductor breakage, or improper crimping. - Record test results (force values, cable ID, tool used, operator details), capture photographic evidence, and store records in a traceable system. | - Knowledge about pull test as per OEM standard - Preparation of equipment and calibration process - Pre-test inspection and setup techniques - Technique for pull test - Pull test evaluation and documentation Practical Activity Perform terminal pull test, observe and document the process | Theory-02Hrs Practical-06Hrs Total- 08Hrs | - Lint free wipes/clothes - Cleaning solvent - Markers - Labeling tags - Inspection forms | Class Room and workshop |
| Apply adhesive-lined heat shrink tubing | - Identify vehicle details, cable specifications, tubing size, and OEM requirements for heat shrink application. - Review technical guidelines to confirm correct tubing type (adhesive-lined), shrink ratio, and temperature range. - Arrange availability and condition of heat gun, temperature control device, and required consumables. - Power down the system, isolate the HV circuit, and remove the ignition key. - Apply Lock-Out/Tag-Out (LOTO) Techniques to prevent accidental energization. - Wear appropriate PPE including heat-resistant gloves, eye protection, and protective clothing. - Inspect the cable and connection area for cleanliness, damage, or contamination. - Clean the cable surface using approved cleaning agents to remove dirt, oil, or moisture. - Select the correct size adhesive-lined heat shrink tubing based on cable diameter and specification. - Position the heat shrink tubing over the joint or exposed area with proper coverage length. - Apply controlled heat evenly using a heat gun while rotating the cable for uniform shrinkage. - Detect tubing contraction and adhesive flow to form a tight seal around the cable. - Allow the tubing to cool naturally without disturbance. - Inspect the finished application for uniform shrinkage, proper sealing, and absence of gaps or overheating marks. - Record application details (cable ID, tubing type, temperature used, operator details), capture photographic evidence, and store records in a traceable system. | - Selection procedure for materials - Surface preparation and inspection - Heat shrink application technique - Inspection of heat shrink application and cooling period Practical Activity - Perform adhesive line shrink tubing, surface inspection and documentation | Theory-01Hrs Practical-06Hrs Total- 07Hrs | - Adhesive lined heat shrink tubes - Lint free wipes/clothes - Cleaning solvent - Protective covering tapes - Cable ties - Documentation material | Class Room and workshop |
| Perform continuity Test of HV connector interlock (HVIL) | - Collect vehicle details, HVIL circuit specifications, and OEM testing requirements. - Review technical documentation to identify HVIL circuit path, connectors, and expected continuity values. - Arrange availability and calibration status of diagnostic tools and multimeter (continuity tester). - Power down the vehicle, switch off ignition, and remove the key. - Apply Lock-Out/Tag-Out (LOTO) Techniques to prevent accidental energization. - Wear appropriate PPE including insulated gloves, eye protection, and protective clothing. - Isolate the HV system as per OEM Techniques before accessing connectors. - Locate HV connectors and identify HVIL pins using wiring diagrams. - Inspect connectors and wiring for damage, corrosion, or loose connections. - Set the multimeter to continuity or resistance measurement mode. - Connect test probes to HVIL circuit terminals across the connector. - Measure continuity and record resistance value or audible indication. - Operate connector lock/unlock mechanism to verify change in HVIL circuit status. - Compare measured results with OEM specifications and determine Pass/Fail status. - Record test results (connector ID, readings, operator details), capture evidence if required, and store records in a traceable system | - Identification of various circuits - Safety procedures for HV isolation - Identification of HVIL connector and inspection - Function of HVIL circuit - Continuity testing techniques - Evaluation of Result for continuity test and documentation process Practical Activity Perform continuity test of HV connectors | Theory-02Hrs Practical-06Hrs Total- 08Hrs | - Lint free wipes/clothes - Contact cleaner spray - Dielectric grease - Marking and labeling material - Cable ties - Documentation material | Class Room |

# List of Machines, Tools and Equipment (for 25 Trainees)

Electric Vehicle (EV) Specifications

- 2-Wheeler EV	1 No.

- 3-Wheeler EV	1 No.

- 4-Wheeler EV	1 No.

| Specification | 2-Wheeler EV | 3-Wheeler EV | 4-Wheeler EV |
| --- | --- | --- | --- |
| Vehicle Type | Electric Motorcycle/ Scooter | Electric Rickshaw/ Cargo Loader | Electric Car/SUV/Bus |
| Number of Wheels | 2 | 3 | 4 |
| Seating Capacity | 1–2 | 3–6 | 4–7 (cars) |
| Motor Type | BLDC Hub or Mid-drive | BLDC/PMSM | PMSM or Induction Motor |
| Motor Power | 0.5–8 kW | 1–12 kW | 30–250+ kW |
| Maximum Speed | 25–120 km/h | 25–60 km/h | 100–220 km/h |
| Driving Range | 50–180 km | 70–180 km | 200–700 km |
| Charging Time | 3–6 hours | 4–8 hours | 30 minutes (DC Fast) to 12 hours (AC) |
| Transmission | Single Speed | Single Speed | Single Speed Reduction Gear |
| Regenerative Braking | Optional | Optional | Standard |
| Cooling System | Air cooled | Air/Liquid cooled | Liquid cooled |

| Name of Trade |  | 0716MSA21. Electric Vehicle (EV) Supervisor Level-4 |  |
| --- | --- | --- | --- |
| Duration of the Course |  | 1 Year |  |
| S. No | Tools and Equipment |  | Quantity |
| 1 | Basic hand tools set |  | 5Sets |
| 2 | Ball Rubber Peen Hammer |  | 5 |
| 3 | Digital multimeter low range |  | 5 |
| 4 | Digital multimeter High range |  | 5 |
| 5 | Insulated gloves Class 0 |  | 5Sets |
| 6 | Safety goggles aur face shield |  | 100 |
| 7 | Trim removal tools |  | 5 |
| 8 | OBD scanner (basic) |  | 5 |
| 9 | Insulated mat |  | 5Sets |
| 10 | Lockout Tagout kit (LOTO) |  | 5 |
| 11 | Warning signs and cones |  | 5Sets |
| 12 | Advanced EV diagnostic scanner |  | 5 |
| 13 | Insulated tool kit (1000V rated) |  | 5 |
| 14 | Insulation tester (Megger) |  | 5 |
| 15 | Clamp meter (DC high current) |  | 5 |
| 16 | Battery lifting tools |  | 5 |
| 17 | Battery Tester Heavy Load |  | 3 |
| 18 | HV disconnect tools |  | 5 |
| 19 | Thermal camera |  | 5 |
| 20 | Continuity tester |  | 5 |
| 21 | PPE kit |  | 5Sets |
| 22 | EV safety barriers and chains |  | 5 |
| 23 | Complete EV insulated master tool kit |  | 5 Kits |
| 24 | High voltage insulation tester (CAT III / IV) |  | 5 |
| 25 | Battery module tester |  | 5 |
| 26 | Cell balancing equipment |  | 5 |
| 27 | Oscilloscope (signal analysis) |  | 2 |
| 28 | Advanced thermal imaging camera |  | 5 |
| 29 | HV rescue hook |  | 5 |
| 30 | Arc flash suit (full body PPE) |  | 5Sets |
| 31 | Battery pack lifting table |  | 5 |
| 32 | Diagnostic software with OEM access |  | 3 |
| 33 | Data logging tools |  | 5 |
| 34 | Electrical insulation tape |  | 10 |
| 35 | Allen key set, hex keys |  | 5 Sets |
| 36 | Wire stripper |  | 10 |
| 37 | Crimping tool |  | 5 |
| 38 | Test light |  | 5 |
| 39 | Fuse puller |  | 5 |
| 40 | Trim removal tool Kits |  | 5 |
| 41 | Inspection mirror |  | 5 |
| 42 | Magnetic pickup tool |  | 5 |
| 43 | Flashlight, rechargeable |  | 5 |
| 44 | Creeper seat |  | 5Sets |
| 45 | Parts tray, magnetic |  | 5 |
| 46 | Insulated screwdrivers |  | 5 Sets |
| 47 | Insulation resistance tester |  | 5 |
| 48 | Torque wrench |  | 5 |
| 49 | Clamp meter (DC) |  | As Per Uses |
| 50 | Test leads replacement |  | 2 Kits |
| 51 | Absorbent pads, spill kit |  | 2 Kits |
| 52 | Thermal paste |  | 2 |
| 53 | Cell insulation sheets |  | 5 |
| 54 | Bus bar insulation covers |  | As per Uses |
| 55 | HV cables and lugs |  | 5 Sets |
| 56 | Coolant fluid specific type |  | 4 Liters |
| 57 | Adhesive foam seals |  | 3 Kits |
| 58 | Fire resistant blankets |  | 2 |
| 59 | Hazardous waste bags |  | 4 |
| 60 | Electrolyte neutralizer |  | 4 |
| 61 | Vehicle lift, 2 post or 4 post |  | 2 |
| 62 | Tool storage cabinet/Trolley |  | 5 |
| 63 | Battery charging station 12 Volts |  | 2 Station |
| 64 | Battery charging station, HV systems |  | 2 Station |
| 65 | Parts washer machine |  | 1 |
| 66 | Tire inflator pressure unit |  | 1 |
| 67 | Oil drain unit |  | 1 |
| 68 | Panel clip pliers |  | 5 |
| 69 | Lighting system |  | 1 |
| 70 | Fire extinguisher, Class ABC |  | 4 |
| 71 | HV isolation station (lockout tagout panel) |  | 2 Station |
| 72 | Safety barrier system (cones, chains, retractable barriers) |  | 2 |
| 73 | Insulated rubber flooring |  | 4 |
| 74 | Battery lifting table (controlled height adjustment) |  | 3 |
| 75 | Thermal monitoring station (overheating detection) |  | 2 Station |
| 76 | Battery pack lifting machine |  | 1 |
| 77 | HV battery test bench |  | 2 |
| 78 | Cell balancing station |  | 2 Station |
| 79 | Battery charge discharge machine |  | 2 |
| 80 | Cleaning spray, contact cleaner |  | 5 |
| 81 | Safety Helmet |  | 25 |
| 82 | Voltage detector pen |  | 5 |
| 83 | Insulated pliers |  | 5 Sets |
| 84 | Stethoscope |  | 5 |
| 85 | Cable cutter |  | 5 |
| 86 | Fire suppression system (lithium battery specific) |  | 2 |
| 87 | Explosion proof storage cabinet (damaged battery) |  | 1 |
| 88 | HV rescue equipment station |  | 2 Station |
| 89 | Data logging and monitoring system |  | 1 System |
| 90 | Clean room (sealed battery room setup) |  | 1 Room |
| 91 | HV voltage detector |  | 5 |
| 92 | Ventilation system |  | 5 |
| 93 | AVC Gas Recycling Machine |  | 1 |
| 94 | Soldering Iron Kit |  | 3 Kits |
| 95 | Heat Gun |  | 3 |
| 96 | Battery Pack Analyzer |  | 5 |
| 97 | Laptop with Diagnostic Software |  | 2 |
| 98 | Thermal Imaging camera |  | 2 |
| 99 | Battery Cycler Machine |  | 2 |
| 100 | Measuring Taps |  | 5 |

# List of Consumables (for 25 Trainees)

Battery Specifications

- 2-Wheeler EV	1 No.

- 3-Wheeler EV	1 No.

- 4-Wheeler EV	1 No.

| Parameter | Typical Specification |  |  |
| --- | --- | --- | --- |
|  | 2-Wheeler EV | 3-Wheeler EV | 4-Wheeler EV |
| Battery Chemistry | Lithium-ion (LFP/NMC) | Lithium-ion or LFP | Lithium-ion (NMC/LFP) |
| Battery Voltage | 48V, 60V, 72V | 48V, 60V, 72V, 96V | 200V–800V |
| Battery Capacity | 20–80 Ah | 100–250 Ah | 100–250 Ah |
| Energy Capacity | 1–5 kWh | 5–15 kWh | 20–120 kWh |
| Battery Type | Mostly Low Voltage | Low Voltage (48–72V) | High Voltage (200–800V |
| BMS | Yes | Yes | Advanced BMS |
| Charger | 48V/60V/72V Smart Charger | AC Charger | DC CCS/NACS (Fast Charging) |
| Cooling | Air cooling | Air cooling | Liquid Cooling |

| S. No | Consumables | Quantity |
| --- | --- | --- |
|  | Electrical Contact Cleaner | 15 cans |
|  | Cable Ties | 25 pcs |
|  | Marker Pens / Labels | 25 pcs |
|  | Cleaning Cloths | 40 pcs |
|  | Probe Tips Replacement | 20 sets |
|  | Fuse Assortment (Practice) | 25 pcs |
|  | OEM Silicone Sealant | 20 tubes |
|  | Replacement Fuses | 50 pcs |
|  | Replacement Relays | 20 pcs |
|  | Gaskets / Sealing Kits | 20 sets |
|  | Battery Coolant | 20 liters |
|  | Abrasive Pads | 20 pcs |
|  | Cleaning Brushes | 20 pcs |
|  | EV-Specific Coolant | 20 Liters |
|  | Dielectric Grease | 10 Tubes (3oz) |
|  | Orange Shielded Cable | 10 Meters |
|  | High-Temp Loom/Tape | 10 Rolls |
|  | Terminal Pins/Sockets | 200 Pieces |
|  | Cotton Swabs/Lint-free Wipes | 2 Bulk Packs |
|  | Glove Dusting Powder | 3 Bottles |
|  | Distilled Water | 2 Gallons |
|  | Nylon Cable Ties | 500 Pieces |
|  | Heat shrink Tubes | 3 dozen |
|  | Insulated Tap | 3 dozen |
|  | Thermal paste (for cooling systems) | 10 |
|  | WD-40 | 12 |
|  | E-drive assembly lubricating oil | As required |
|  | Label makers and labels (for wiring and components) | 12 |
|  | Wire connectors (various sizes) | As required |
|  | Fuses for different rating (Blade fuses, Bolt-down fuses) | As required |
|  | HV Cables (Training/Practice) | Bulk |
|  | Terminal Lugs (Various Sizes) | Assorted |
|  | Insulation Tape (HV Rated) | 5 rolls |
|  | Cable Marking Labels/Tags | Bulk |
|  | Calibration floor marking tape | 2 bottles |
|  | Printed job cards | 3 cans |
|  | Wire Brushes (Soft) | 5 |
|  | Inspection/Assessment Forms | As required |
|  | Clipboards / Tablets | 5 |
|  | Fault analysis forms | 3 |
|  | Anti-static bags | 20 |
|  | Lint-Free Cloth | Bulk |
|  | Isopropyl Alcohol | 2 bottles |
|  | Cable Ties & Harness Clips | Bulk |
|  | Camera / Mobile Device | 3 |
|  | Firmware Repository Access (OEM) | 1 |
|  | Digital Record/PLM System Access | 1 |
|  | Specialized Mastic Sealant | 10 tubes |
|  | Replacement O-Rings (Assorted Kit) | 5 sets |
|  | Sealant Remover Chemical | 5 |
|  | Leak Detection Spray | 5 |
|  | Marking Chalk / Marker | 5 |
|  | First Aid Kit | 5 |
|  | Emergency Stop Switch Access | 5 |
|  | Warning Signage (HV Area) | 10 |
|  | Spill Kit (Chemical/Sealant) | 5 |
|  | Test Report Sheets / Logbooks | 5 sets |
|  | Calibration Certificates (Copies) | 5 sets |
|  | OEM Technical Manuals | 5 |
|  | Clipboard & Pen | 5 |
|  | Waste Disposal Bins (Hazardous/Non-Hazardous) | 10 |
|  | Cable Management Ties | 5 |
|  | Labels & Tags | 5 |
|  | Brake fluid (DOT 4) | 5 liters |
|  | Coolant | 6 liters |
|  | Connector cleaning pins / pin removal kit | 20 |
|  | High-voltage insulating tape | 20 rolls |
|  | Cable ties (assorted sizes) | 5 packs |
|  | Cleaning wipes for terminals and connectors | 10 packs |
|  | Disposable nitrile gloves | 5 boxes |
|  | HV warning labels / isolation tags | 100 pieces |
|  | Terminal protective caps and covers | 25 sets |
|  | Absorbent pads for moisture control | 10 packs |
|  | Printed test record sheets and log forms | 200 sheets |
|  | Thermal Paste | 2 tubes |
|  | Fasteners (Nuts/Bolts) | Assorted kit |
|  | Battery Storage Containers | 2 |
|  | Insulated Packaging Material | Bulk |
|  | Waste Manifest Forms | As required |
|  | Transport Documents | As required |
|  | Pallets / Containment Boxes | 2 |
|  | Straps / Securing Materials | 4 sets |
|  | Hazard Labels (UN Markings) | Bulk |
|  | Adhesive-lined Heat Shrink Tubing | Assorted |
|  | Contact Cleaner Spray | 3 cans |
|  | Cleaning wipes for tools and PPE | 10 packs |
|  | Battery terminal covers and caps | 25 sets |
|  | Warning labels and HV safety stickers | 100 pieces |
|  | Absorbent pads for electrolyte leakage | 10 packs |
|  | Cleaning solvent for workshop equipment | 10 liters |
|  | Permanent marker pens for labeling | 25 pieces |
|  | Printed safety briefing sheets and log forms | 200 sheets |
|  | Coolant (as per OEM specification) | As required |
|  | Contact cleaners | 12 |
|  | Refrigerant gas (R134a / R1234yf) | 10 cylinders |
|  | Compressor oil (PAG / POE oil) | 5 liters |
|  | O-rings and sealing gaskets | 100 pcs |
|  | Refrigerant filter drier | 25 pcs |
|  | Nitrogen gas cylinder (leak testing) | 3 cylinders |
|  | Condenser cleaning solvent | 10 liters |
|  | Cable ties (nylon) | 500 pcs |
|  | Insulation tape (HVAC grade) | 50 rolls |
|  | UV leak detection dye | 10 bottles |

# Members of Qualification Development Committee

The Qualifications Development Committee consisted of following members:

| S. # | Name | Designation | Organization |
| --- | --- | --- | --- |
|  | Engr. Sanaullah Khan | Lecturer | KP TEVTA |
|  | Engr. Sadam Hussain | Instructor | KP TEVTA |
|  | Rehman Ali | Sr: Executive | HYNDAI/ Nishat Motor |
|  | Syed Umar Azhar Bukhari | EV Technician | Yutong EV Buses |
|  | Mohammad Yousif Baloch | Director | S-TEVTA |
|  | Mohammad Zulqarnain | EV Technician | BYD Bahawalpur |
|  | Mohammad Zeeshan | Maintenance Supervisor | MG Service Center |
|  | Engr Zohaib Afzal | Head of R& D | Pakzon |
|  | Engr Saqib Abdullah | GM Plant | Jolta Electric |
|  | Zahid Razzaq | M. MTech | WESCON, Engineering |
|  | Farhan Ahmed Durrani | Head of CED | Lucky Motors Company |
|  | Ijaz Hamid | Ex- Chief Instructor | P-TEVTA |
|  | Dr Maliha Bakhs | Asst: Professor | UET Lahore |
|  | Mian Abdul Atique | Service Manager | HONDA Center Lahore |
|  | Muzammal Hussain | Assistant Director | P-TEVTA |
|  | Mohid Ali | Service Advisor | Max Power Lahore |
|  | Akmal Khan | Project Coordinator | WHH |
|  | Aftab Hussain Soomro | Program Coordinator TEVT | RDF |
|  | Inayat ur Rehman | DACUM Expert | Ex KP-TEVTA |
|  | Muhammad Ishaq | Deputy Director | NAVTTC Islamabad |
|  | Shehzad Ahmed | Director NAVTTC | Academia |