|  | Assessment Evidence Guide For Electric Vehicle (EV) Technician Level-3 Summative Assessment Activity-4 |
| --- | --- |
|  | Date 1-5 December, 2025 Lahore |
|  | National Vocational & Technical Training Commission |

Section A: Instruction Sheet for the Candidate

| Qualification | Electrical Vehicle (EV) Technician Level-3 |
| --- | --- |
| Competency Standards | - Demonstrate Communication Skills - Perform Occupational Health and Safety Practices - Interpret Basic Electrical Principles - Maintain Electric vehicle (EV) Battery System - Maintain Electric vehicle (EV) Motor - Maintain Electric vehicle (EV) Control System - Perform Electric vehicle (EV) Charging System - Maintain Electric vehicle (EV) Cooling System - Maintain Electric vehicle (EV) Transmission and Drive - Maintain Electric vehicle (EV) Brake System |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name_______________________________________________________ Registration Number_________________________________________________ |

Section A1: Guidance for Candidate

| Guidance for Candidate | To meet this standard, you are required to complete the following within 04 hrs. time frame (for practical demonstration & assessment): Activity-1: Maintain an Electric Vehicle EV battery system and its integration with the vehicle control system adhering to health and safety protocols and maintenance according to standard manufacturer’s defined procedures. |
| --- | --- |
| Time: 04 Hrs | During a practical assessment, under observation by an assessor, you are required to demonstrate the following criteria: |
| Minimum Evidence Required | - Follow high-voltage safety protocols, - Wear PPEs, and isolate motor power prior to maintenance. - Identify key EV control system components and their functions. - Identify High voltage (HV) system interactions with vehicle controls, - Depower the system to start the work and wait for the specified depowering time. - Isolate RESS supply, stabilizing systems. - Inspect control units (VCU, MCU, inverters), wiring harnesses, connectors, fuses, and relays for damage, corrosion, or loose connections. - Conduct diagnostic tests to detect and identify EV control system faults. - Interpret diagnostic trouble codes (DTCs) and warning indicators for VCU, MCU, BMS, and charging controller using a diagnostic scanner. - Apply systematic troubleshooting to isolate faulty subsystems such as VCU, BMS, MCU, inverter, or charger controller. - Verify wiring and connectors - Perform the software updates and the clean modules. - Repair or replace defective components based on diagnostic findings. - Perform functional and communication tests to confirm fault resolution and system performance. - Document replaced components. |

Section A2: Self-Assessment Checklist

| Assessment Task | Activity-1: Perform maintenance of an Electric Vehicle (EV) battery system and its integration with the vehicle control system adhering to health and safety protocols and maintenance according to standard manufacturer’s defined procedures. |
| --- | --- |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Record vehicle make, model, year, and VIN (Vehicle Identification Number) accurately. |  |  |
| - Estimate the time required for each service accurately based on industry standards. |  |  |
| - Verify all parts and materials required for the job based on the service requested. |  |  |
| - Identify major EV control units such as VCU, MCU, BMS, inverter, and charging controller |  |  |
| - Identify the main functions of the motor controller such as torque control, speed regulation, direction control, and regenerative braking. |  |  |
| - Identify difference between motor control methods including Field-Oriented Control (FOC), Direct Torque Control (DTC), and Pulse Width Modulation (PWM). |  |  |
| - Apply high-voltage safety procedures including isolation, PPE usage, and lock-out/tag-out. |  |  |
| - Consult the OEM service manual, battery specifications, and safety requirements before testing. |  |  |
| - Identify symptoms of faults in the EV control system using inspection and diagnostic tools. |  |  |
| - Check connections of terminals, busbars, and harnesses, and measure voltage, current, and SOC using diagnostic tools. |  |  |
| - Check and Interpret BMS data for cell voltage, SOC, SOH, and imbalance, and perform internal resistance or health checks with analyzers. |  |  |
| - Check battery temperature and cooling system. |  |  |
| - Test under load to assess voltage drop, current delivery, and performance against manufacturer specifications |  |  |
| - Diagnose with diagnostic trouble codes (DTCs) and warning indicators related to VCU, MCU, BMS and Charging controller operation using a diagnostic scanner. |  |  |
| - Trace data and signal flow between subsystems through CAN bus or other protocols. |  |  |
| - Check abnormalities such as warning messages, vibrations, or performance drops. |  |  |
| - Verify faults against manufacturer specifications, diagnostic codes, and performance data. |  |  |
| - Perform systematic troubleshooting techniques to isolate faulty subsystems such as VCU, BMS, MCU, inverter, or charger controller |  |  |
| - Repair minor faults in components such as wiring, connectors, or terminals following OEM guidelines |  |  |
| - Replace defective components with OEM-approved or equivalent spare parts if repair is not feasible. |  |  |
| - Perform functional and communication tests to confirm rectified faults and restored performance. |  |  |
| - Operate the EV under different driving conditions to evaluate control system responses. |  |  |
| - Monitor VCU, MCU, regenerative braking, and communication data during the drive. |  |  |
| - Document test drive results, diagnostic data, and corrective actions in service records. |  |  |

Candidate’s Signature__________________ Assessor’s Signature__________________

Date: _______________________________

Section B : Assessors Judgment Guide 1

| Qualification | Electrical Vehicle (EV) Technician Level-3 |
| --- | --- |
| Competency Standards | - Demonstrate Communication Skills - Perform Occupational Health and Safety Practices - Interpret Basic Electrical Principles - Maintain Electric vehicle (EV) Battery System - Maintain Electric vehicle (EV) Motor - Maintain Electric vehicle (EV) Control System - Perform Electric vehicle (EV) Charging System - Maintain Electric vehicle (EV) Cooling System - Maintain Electric vehicle (EV) Transmission and Drive - Maintain Electric vehicle (EV) Brake System |
| Candidate Details | Name: ______________________________________________________________ Registration Number: ________________________ Signature: _________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________________________________ Assessor’s code: ___________________ Signature: _____________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Section B1: Observation Checklist

| Assessment Task |  | Activity-1: Perform inspection, diagnosis procedure and maintenance of an Electric Vehicle EV battery system and its integration with the vehicle control system adhering to health and safety protocols and maintenance according to standard manufacturer’s defined procedures. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Record vehicle make, model, year, and VIN (Vehicle Identification Number) accurately. |  |  |  |  |  |
|  | Estimate the time required for each service accurately based on industry standards. |  |  |  |  |  |
|  | Verify required parts and materials required for the task. |  |  |  |  |  |
|  | Identify major EV control units such as VCU, MCU, BMS, inverter, and charging controller |  |  |  |  |  |
|  | Identify the main functions of the motor controller such as torque control, speed regulation, direction control, and regenerative braking. |  |  |  |  |  |
|  | Identify difference between motor control methods including Field-Oriented Control (FOC), Direct Torque Control (DTC), and Pulse Width Modulation (PWM). |  |  |  |  |  |
|  | Apply high-voltage safety procedures including isolation, PPE usage, and lock-out/tag-out. |  |  |  |  |  |
|  | Consult the OEM service manual, battery specifications, and safety requirements before testing. |  |  |  |  |  |
|  | Detect symptoms of faults in the EV control system using inspection and diagnostic tools. |  |  |  |  |  |
|  | Check connections of terminals, busbars, and harnesses, and measure voltage, current, and SOC using diagnostic tools. |  |  |  |  |  |
|  | Check and Interpret BMS data for cell voltage, SOC, SOH, and imbalance, and perform internal resistance or health checks with analyzers. |  |  |  |  |  |
|  | Check battery temperature and cooling system. |  |  |  |  |  |
|  | Test under load to assess voltage drop, current delivery, and performance against manufacturer specifications |  |  |  |  |  |
|  | Diagnose with diagnostic trouble codes (DTCs) and warning indicators related to VCU, MCU, BMS and Charging controller operation using a diagnostic scanner. |  |  |  |  |  |
|  | Trace data and signal flow between subsystems through CAN bus or other protocols. |  |  |  |  |  |
|  | Check abnormalities such as warning messages, vibrations, or performance drops. |  |  |  |  |  |
|  | Verify faults against manufacturer specifications, diagnostic codes, and performance data. |  |  |  |  |  |
|  | Perform systematic troubleshooting techniques to isolate faulty subsystems such as VCU, BMS, MCU, inverter, or charger controller |  |  |  |  |  |
|  | Repair minor faults in components such as wiring, connectors, or terminals following OEM guidelines |  |  |  |  |  |
|  | Replace defective components with OEM-approved or equivalent spare parts if repair is not feasible. |  |  |  |  |  |
|  | Perform functional and communication tests to confirm rectified faults and restored performance. |  |  |  |  |  |
|  | Operate the EV under different driving conditions to evaluate control system responses. |  |  |  |  |  |
|  | Monitor VCU, MCU, regenerative braking, and communication data during the drive. |  |  |  |  |  |
|  | Document test drive results, diagnostic data, and corrective actions in service records. |  |  |  |  |  |
| Competent |  |  | Not Competent |  |  |  |

Section B2: Knowledge Assessment

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
| 1 | What is the standard voltage range for high-voltage EV battery systems? |  |  |
|  | The standard voltage range for the EV battery system ranges from 200V – 800V, depending on the vehicle design and make specifications. |  |  |
| 2 | State the recommended PPE for handling high-voltage components. |  |  |
|  | - Insulated gloves (Class 0 or Higher) - Face Shield - Safety googles - Arc-rated clothing - Insulated tools |  |  |
| 3 | What is the typical SOC range considered safe for battery maintenance? |  |  |
|  | General recommendations for safe range of state of charge (SOC) is below 30%. |  |  |
| 4 | List the correct sequence of steps for safely isolating a high-voltage EV battery system. |  |  |
|  | 1.Apply parking brake and place in Park. 2. Remove ignition key/key fob (place >5 meters away). 3. Disconnect 12V battery (negative first, then positive). 4. Remove HV service plug (orange handle). 6. Wait 5-10 minutes for capacitor discharge. 7. Verify voltage <5V at three test points (HV+ to HV-, HV+ to ground, HV- to ground). 8. Apply lockout/tagout devices. |  |  |
| 5 | After removing the HV service plug and waiting for capacitor discharge, State the maximum acceptable voltage for each. |  |  |
|  | - HV+ busbar to HV- busbar - Reading <5V DC - HV+ busbar to chassis ground - Reading <5V DC - HV- busbar to chassis ground - Reading <5V DC |  |  |
| 6 | State the primary functions of the Battery Management System (BMS) in an electric vehicle. |  |  |
|  | - Cell voltage monitoring - State of Charge (SOC) estimation - State of Health (SOH) Assessment - Cell balancing - Thermal Management Control - Protection and Safety functions |  |  |
| 7 | What is the acceptable cell voltage imbalance threshold in most EV battery systems? |  |  |
|  | The acceptable cell voltage imbalance is typically 30-50 mV maximum for most EV systems, with many manufacturers requiring <50 mV for warranty coverage. A 120 mV imbalance is NOT acceptable and indicates significant cell degradation or BMS malfunction |  |  |
| 8 | State the maximum allowable voltage drop during a load test. |  |  |
|  | Not more than 10% of nominal voltage under load. |  |  |
| 9 | What is the maximum permissible internal resistance for a healthy battery cell? |  |  |
|  | Less than 5 mΩ, it varies by cell type and OEM specification. |  |  |
| 10 | State the optimal temperature range |  |  |
|  | Optimal Temperature Range: 20-35°C (68-95°F) for lithium-ion batteries. |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature ___________________