National Curriculum for Electric Vehicles (EV)Technician, Level-3

National Vocational and Technical Training Commission (NAVTTC), Government of Pakistan

FOREWORD

National Vocational and Technical Training Commission (NAVTTC) extends its gratitude and appreciation to representatives of business, industry, academia, government agencies, provincial TEVTAs, sector skill councils and trade associations who spared time and extended their expertise for the development of National Vocational Qualification for Electrical Vehicle (EV)Technician Level-3. This work would not have been possible without the technical support of the experts.

It may not be out of place to mention here that all the experts of Industry, Academia and TVET experts of TEVTAs, BTEs and PVTC work diligently for making this qualification worthy and error free for which all credit goes to them. However, NAVTTC accepts the responsibility of all the errors and omissions still prevailing in the Qualification document.

It is also noteworthy that development of Skill Standards is a dynamic and ongoing process, and the developed competency standards needs periodic review and updating owing to the constant technological advancements, development in scientific knowledge, and growing experience of implementation at the grass root level as well as the demand of industry. The NAVTTC, Government of Pakistan will ensure to keep the qualifications abreast with the changing demands of both national and international job markets.

Executive Director (NAVTTC)

Table of Contents

1.	Introduction	4

1.1 Date of Development	5

1.2.Date of Review and Validation	5

1.3. Entry Requirement	5

1.4. Minimum Qualification for Teachers	5

1.5. Medium of Instruction	5

1.6. Duration of Course	5

2.	Scheme of Studies	6

3.	Detail of Modules	9

3.1 0716MSA1901 Demonstrate Communication Skills	9

3.2 0716MSA1902 Perform Occupational Health and Safety Practices	12

3.3 0716MSA1903 Interpret Basic Electrical Principles	16

3.4 0716MSA1904 Maintain Electric Vehicle (EV) Battery System	20

3.5 0716MSA1905 Maintain Electric Vehicle (EV) Motor	25

3.6 0716MSA1906 Maintain Electric Vehicle (EV)Control System	29

3.7 0716MSA1907 Perform Electric Vehicle (EV) Charging System	38

3.8 0716MSA1908 Maintain Electric Vehicle (EV) Cooling System	43

3.9 0716MSA1909 Maintain Electric Vehicle (EV) Transmission and Drive	49

3.10.0716MSA1910 Maintain Electric Vehicle (EV) Brake System	55

4-	List of Tools and Equipment	60

5- List 0f Consumable Materials.	62

6- Members of Qualification Development & Review Committee	65

# Introduction

This training course is designed for the occupation of ‘Electrical Vehicle (EV)Technician Level-3’ in the Automobile Sector. The Electrical Vehicle (EV)Technician is responsible for Maintaining Brake, Motors. Cooling, Control, charging and EV Transmission and Drive Systems adhering to Occupational Health and Safety (OHS) regulations.

Automobile Technology is the branch of engineering which deals with everything about Automobiles and practices to propel them. Automobile can also be defined as a vehicle which can move by itself, for example Car, jeep, bus, truck, scooter, etc. Therefore, industry expectations for skilled workforce are also dynamic which can only be managed through setting relevant competency standards in collaboration with the leading industries. Electrical Vehicle (EV)Technician play a crucial role in Automobile industry so being cognizant of the fact, and growing demand for skilled professionals in this field, NAVTTC, in collaboration with GIZ, developed competency standards for Automobile Technology under National Vocational Qualifications Framework (NVQF) having representation from the Auto Mobile industry of the country. These competency standards are as per essence of the Competency-Based Training and Assessment (CBT&A) programs, ensuring the development of a competent workforce aligned with industry needs.

| ## 1.1 Date of Development | 29 Sept.to 03 October 2025 |
| --- | --- |
| - Date of Review and Validation |  |
| ## 1.3. Entry Requirement | - Electrical Vehicle Technician Level-2 - Automobile Mechanic Level-2 - Auto Electrician Level-2 - Matric (Science/Arts) |
| ## 1.4. Minimum Qualification for Teachers | - DAE in Automobile/ Mechanical Technology with 02 Years Relevant Experience - G-II in Auto Mechanic/ Auto Electrician/Auto & Farm Machinery with 05 Years’ Experience |
| ## 1.5. Medium of Instruction | Urdu, local language, English |
| ## 1.6. Duration of Course | 6-Months |

Overall Objectives of Training Program

The Electrical Vehicle Technician qualification of Level-3 consists of 20 % Theory and 80 % Practical and having the following Modules.

- 0716MSA1901 Demonstrate communication skills.

- 0716MSA1902 Perform occupational health and safety practices.

- 0716MSA1903 Interpret Basic Electrical Principles.

- 0716MSA1904 Maintain Electric vehicle (EV) battery system.

- 0716MSA1905 Maintain Electric vehicle (EV) motor.

- 0716MSA1906 Maintain Electric vehicle (EV) control system.

- 0716MSA1907 Perform Electric vehicle (EV) charging system.

- 0716MSA1908 Maintain Electric vehicle (EV) cooling system.

- 0716MSA1909 Maintain Electric vehicle (EV) transmission and drive.

- 0716MSA1910 Maintain Electric vehicle (EV) braking system.

# Scheme of Studies

| Code | Competency Standards | Category | Level | Th | Pr | Total | Cr.H |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0716MSA1901 | Demonstrate Communication Skills | Generic | 3 | 5 | 20 | 25 | 1.5 |
| 0716MSA1902 | Perform Occupational Health and Safety Practices | Functional | 3 | 5 | 20 | 25 | 1.5 |
| 0716MSA1903 | Interpret Basic Electrical Principles | Technical | 3 | 6 | 18 | 24 | 1.5 |
| 0716MSA1904 | Maintain Electric vehicle (EV) Battery System | Technical | 3 | 15 | 40 | 55 | 3.5 |
| 0716MSA1905 | Maintain Electric vehicle (EV) Motor | Technical | 3 | 10 | 30 | 40 | 2.5 |
| 0716MSA1906 | Maintain Electric vehicle (EV) Control System | Technical | 3 | 10 | 30 | 40 | 2.5 |
| 0716MSA1907 | Perform Electric vehicle (EV) Charging System | Technical | 3 | 10 | 30 | 40 | 2.5 |
| 0716MSA1908 | Maintain Electric vehicle (EV) Cooling System | Technical | 3 | 5 | 20 | 25 | 1.5 |
| 0716MSA1909 | Maintain Electric vehicle (EV) Transmission and Drive | Technical | 3 | 15 | 40 | 55 | 3.5 |
| 0716MSA1910 | Maintain Electric vehicle (EV) Brake System | Technical | 3 | 15 | 40 | 55 | 3.5 |
| On Job Training |  |  |  | 0 | 240 | 240 | 6 |
| Total |  |  |  | 96 | 528 | 624 | 30 |

Overview of the Curriculum for Electrical Vehicle (EV)Technician Level-03

| Module Title | Learning Units | Theory hours | Workplace hours | Timeframe of Modules |
| --- | --- | --- | --- | --- |
| 0716MSA1901 Demonstrate Communication Skills | - Work in Team - Deal with Clients - Demonstrate Basic IT Skills - Develop Basic Functional English - Create a Job card | 5 | 20 | 25 |
| 0716MSA1902 Perform Occupational health and safety practices | - Identify potential Hazards at workplace - Maintain Personal Protective Equipment (PPE) - Maintain Tools and Equipment - Maintain Fire Extinguisher - Maintain First aid Box - Provide Safeguarding to Machines - Prepare for Emergencies - Respond to Emergencies - Monitor Activities of People, Vehicles, And Other Equipment In Workplace - Investigate Incident at Workplace | 5 | 20 | 25 |
| 0716MSA1903 Interpret Basic Electrical Principles | - Carry Out Visual Inspection of The EV Circuit - Operate the Safety Circuits - Select the required Cables and Wires - Perform Connection of Electrical Circuits | 6 | 18 | 24 |
| 0716MSA1904 Maintain Electric vehicle (EV) Battery System | - Identify Different Types of Batteries - Check EV Battery Performance - Clean the Terminal for any Contamination - Perform Tightening of Connectors - Repair/Replace the Faulty Component of The EV Battery | 15 | 40 | 55 |
| 0716MSA1905 Maintain Electric vehicle (EV) Motor | - Identify the Types of EV Motors - Perform Visual Inspection of The EV Motor - Analyse the Working of EV Motors - Replace the Faulty EV Motor | 10 | 30 | 40 |
| 0716MSA1906 Maintain Electric vehicle (EV) Control System | - Identify the Fundamentals of EV Control Systems - Check Functions of the Vehicle Control Unit - Demonstrate the Role of Drive Motor Controller - Analyse the Process of Regenerative Braking - Fix the fault in the EV Control System - Repair/Replace the Faulty Components - Carryy Out A Test Drive of EV Controls System | 10 | 30 | 40 |
| 0716MSA1907 Perform Electric vehicle (EV) Charging System | - Differentiate Between Types of EV Chargers - Identify Components of EV Charging System - Calculate the Charging Time of EV Battery - Repair/Replace the Faulty Components of The EV Charging System. | 10 | 30 | 40 |
| 0716MSA1908 Maintain Electric vehicle (EV) Cooling System | - Identify the Components of EV Cooling Systems - Check EV Cooling Systems for Fault Detection - Repair/Replace the Faulty Components of The EV Cooling Systems - Replace the Coolant in The EV Cooling System - Evaluate the Performance of EV Cooling system | 5 | 20 | 25 |
| 0716MSA1909 Maintain Electric vehicle (EV) Transmission and Drive | - Identify the Types of EV Transmission and Drive System - Identify the Components of EV Transmissions and Drive System - Demonstrate the Function of Inverter in EV System - Demonstrate the Role of Controller Area Network (CAN) in EVs. - Repair/Replace Faulty Components of The EV Transmission and Drive System - Evaluate the Performance of EV Transmission and Drive System | 15 | 40 | 55 |
| 0716MSA1910 Maintain Electric vehicle (EV) Brake System | - Inspect the Brakee System for Fault Detection Through OBD-II Scanner - Diagnose the Faults of the Anti-Lock Braking System - Perform Servicing/Repair of The Hydraulic Brake system. - Replace the Faulty Component of The Anti-Lock Braking system. - Replace the Faulty Component of The Electronic Parking Brake - Perform Test Drive of The Brake System | 15 | 40 | 55 |
| On Job Training (OJT) |  | 00 | 00 | 240 |
| Total |  | 96 | 528 | 624 |

# Detail of Modules

## 3.1 0716MSA1901 Demonstrate Communication Skills

Objective: This module covers the under-pinning knowledge and skills required to Demonstrate communication skills at workplace in accordance with the organization guidelines and procedures. You are expected to work in a team to achieve common organizational goals and avoid conflicts. After completing this Module, the student/Trainee will be able to achieve all the learning outcomes mentioned in this Module in accordance with the essence of the Competency-Based Training & Assessment System.

Duration: 24 Hours			Theory: 06 Hours				Practice: 18 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Work in Team | - Treat team members with respect and maintain positive relationship to achieve common organizational goals. - Listen to instructions carefully and fully comply with them - Provide work related information to team members and identify interrelated work activities to avoid confusion - Adopt communication skills appropriate to work activities and company procedures - Identify problems and resolve them through discussion and mutual agreement | - Respect and positive workplace relationships - Following and complying with instructions - Sharing information and coordinating interrelated activities - Workplace communication skills - Team problem-solving and conflict resolution Activity: Trainee is required to participate in a group-based workplace task, demonstrate respect and cooperation, follow given instructions, share and coordinate information with team members, apply appropriate communication skills, and resolve a simple issue through discussion and agreement | Theory-01Hrs Practice-03Hrs Total- 04Hrs | Consumables - Activity sheets - Case study scenarios Non-Consumables - Whiteboard/ Flipchart - Workplace guidelines | - Classroom - Lab |
| Deal with Clients | - Collect and confirm work requirements from clients using appropriate communication procedures. - Provide clear information to clients about work requirements including costs and time needed to accomplish the task. - Negotiate with clients regarding wages, time, labor requirements etc. | - Communication Procedures procedure with client. - Work Requirement Confirmation Techniques - Costs, Time, and Service Details - Negotiation Skills for Wages, Time, and Labor - Professional Management of client. Activity: Trainee is required to role-play a client interaction: collect and confirm a client’s work request, explain estimated time and cost, and negotiate a service agreement while maintaining professionalism. | Theory-02Hrs Practice-06Hrs Total- 08Hrs | Consumables - Case study scenarios - Sample client request forms Non-Consumables - Whiteboard/ Flipchart - Communication guidelines | - Classroom - Lab |
| Demonstrate Basic IT Skills | - Create folders and files and learn major commands of operating system/windows. - Type text and use major commands such as printing, editing, creating tables, header, footer, footnotes, table of contents and page number. - Make the document as per work specifications and client requirements. - Generate reports for clients as required using appropriate computer applications. - Use internet for sending/receiving emails and connecting through social or other media. | - File and Folder Management in Operating Systems - Word Processing Basics (Typing, Editing, Formatting, TOC, Footnotes, Page Setup) - Document Preparation as per Work Specifications - Report Generation using Computer Applications - Internet and Email Communication Skills - Social Media and Online Communication Tools Activity: Trainee is required to create a structured project folder on the computer, draft a formatted document with tables, header, footer, and TOC, save it in the folder, and email it to the instructor as a client report. | Theory-01Hrs Practice-03Hrs Total- 04Hrs | Consumables - Sample work specification sheets - Report templates Non-Consumables - Computers with Windows OS - MS Office / Libre Office software - Internet access | - Classroom - Lab |
| Develop Basic Functional English | - Develop Listening and Comprehension Skills. - Use common greetings, farewells, and polite expressions correctly. - Form simple sentences to express needs, wants, and basic information. - Pronounce common words clearly and intelligibly. - Read and comprehends simple texts such as signs, notices, and short paragraphs. - Identify key information and main ideas in basic written materials. - Construct basic sentences with correct word order and basic grammar and use appropriate vocabulary for everyday topics. | - Listening and Comprehension Practice - Common Greetings and Polite Expressions - Sentence Formation for Everyday Communication - Pronunciation of Common Words - Reading and Understanding Simple Texts (Signs, Notices, Short Passages) - Identifying Main Ideas in Written Materials - Basic Grammar, Word Order, and Vocabulary Use | Theory-01Hrs Practice-03Hrs Total- 04Hrs | Consumables - Worksheets with simple texts, signs, and notices - Audio recordings of conversations - Practice handouts Non-Consumables - Multimedia projector / Speakers - Whiteboard / Markers | - Classroom - Lab |
| Create a Job card | - Gather essential customer details, including name, contact information, and vehicle identification. - Record vehicle make, model, year, and VIN (Vehicle Identification Number) accurately. - List specific services requested by the customer, including any symptoms or issues reported. - Estimate the time required for each service accurately based on industry standards and previous experience. - Break down the job into individual tasks and assigns realistic timeframes for each. - Identify all parts and materials required for the job based on the service requested. - Confirm with the customer any special requests or additional notes before commencing work. | - Collection of Customer Information - Vehicle Details and VIN Recording - Service Requests and Complaint Logging - Time Estimation for Services - Job Breakdown and Task Scheduling - Identification of Required Parts and Materials Activity: - Trainee is required to create a complete Job Card for a given vehicle scenario. This includes collecting customer details, recording vehicle data, listing reported issues, estimating service times, breaking down tasks, identifying required parts, and finalizing with customer confirmation. | Theory-01Hrs Practice-03Hrs Total- 04Hrs | Consumables - Job Card Forms / Templates (printed or digital) - Sample Vehicle Service Requests Non-Consumables - Computer with Job Card Management Software (or printed forms) - Reference manuals / service time guides | - Classroom - Lab |

## 3.2 0716MSA1902 Perform Occupational Health and Safety Practices

Objective: This module encompasses the essential knowledge and practical skills required to enable trainees to identify, assess, and control workplace hazards in accordance with Occupational Health and Safety (OHS) standards. It focuses on ensuring the proper use, maintenance, and storage of personal protective equipment (PPE), tools, and safety devices to maintain a safe and efficient work environment. Additionally, it equips trainees with the competence to effectively respond to emergencies, monitor workplace activities, and investigate incidents to prevent recurrence and promote a culture of safety. After completing this Module, the student/Trainee will be able to achieve all the learning outcomes mentioned in this Module in accordance with the essence of the Competency-Based Training & Assessment System.

Theory: 05 Hours				Practice:  20 Hours 					Duration:  25 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Identify Potential hazards at workplace | - Inspect workplace for potential hazards - Categorize potential hazard at workplace - Identify safety signs and symbols - Control the potential hazard | - Identification of Work place hazards in accordance with OHS standards - OHS signs and symbols at work place - Control techniques of potential hazards Practical Activity Perform visit of the lab workshop and identify the potential hazard to ensure health and safety standards | Theory-30mints Practical-01Hrs Total- 01;30Hrs | - PPE - Other Safety Equipment - Firefighting Equipment - Safety Charts - Hazard Sample or Visual Aids - Learning Aids | Class Room and workshop |
| LU2 Maintain personal protective equipment (PPE) | - Arrange PPEs as per requirement - Check PPE’s for safe and smooth functioning - Wear proper PPE as per nature of job - Clean PPEs - Store PPE at their designated place after use | - Identification and use of personal protective equipment (PPE’s) - Maintenance of personal protective equipment (PPE’s) | Theory -30mints Practical-02Hrs Total- 02;30Hrs | - PPE - Other Safety Equipment - Firefighting Equipment - Safety Charts | Class Room and workshop |
| LU3. Maintain Tools and Equipment | - Identify the required tools and equipment - Check insulation of tools and equipment - Clean tools and equipment safely - Store tools and equipment at their designated place | - identification of tools equipment - Checking, cleaning and storing techniques of tools and equipment’s insulation of tools Practical Activity Perform inspection, cleaning and storing of the tools/equipment | Theory-30mints Practical-02Hrs Total- 02;30Hrs | - PPE - Other Safety Equipment - Firefighting Equipment - Safety Charts | Class Room and workshop |
| LU4. Maintain Fire Extinguisher | - Check the fire extinguisher for expiry of date and pressure gauge - Install the fire extinguisher - Operate the fire extinguisher - Replace fire extinguisher - . | - Types of fire extinguisher and inspection techniques for checking expiry date and pressure gauge - Installation and operation procedure of fire extinguisher Practical - Perform visual inspection fire extinguisher for expiry date/gauge and operate the safety equipment | Theory-30mints Practical-1:45Hrs Total- 02:15Hrs | - Fire Extinguisher Units - PPE - Other Safety Equipment | Class Room and workshop |
| LU5. Maintain First aid Box | - Check availability of first aid box - Check first aid box for requisite emergency medicines - Check expiry date of medicines - Perform first aid against electric shock - Provide first aid bandages against minor injuries | - Maintenance procedure of first aid box. - Techniques for provision of First Aid in case of injuries/shock Practical - Carryout rehearsal of CPR activity | Theory-30mints Practical-1;45Hrs Total- 02;15Hrs | - First aid box - PPE - Other Safety Equipment | Class Room |
| LU6. Provide Safeguarding of Machines | - Check safety guards of machine - Check brake of machines - Check controlling devices of machine - Perform test operation on machine | - Procedure for ensuring safety of machines. - sustainable materials for machine safe guarding like recycled plastics - Preventive maintenance Practical - Perform inspection of safe guards and carry out preventive maintenance of machines | Theory-30mints Practical-02Hrs Total- 02;30Hrs | - PPE - Other Safety - Equipment - Safety Equipment’s - Physical Guard &Barriers - Safety Devices - Training & Documentation |  |
| LU7. Prepare for Emergencies | - Participate an emergency response training - Response to emergency situation - Perform CPR for cardio respiratory resuscitation | - Understanding of emergency situations - Techniques for handling emergency situations Practical - Handle emergency situation in case of minor injury | Theory-30mints Practical-02Hrs Total- 02;45Hrs | - Emergency plan - Emergency Contact List - First Aid Kit - Fire Extinguishers - PPE - Other Safety - Equipment |  |
| LU8. Respond to Emergencies | - Follow emergency plan - Communicate instructions - Assess risk and determine course of action - Operate emergency equipment and supplies | - Understanding of emergency plan - Risk assessment and reduction techniques - Operation techniques of emergency tools/equipment Practical Follow emergency plan in case of fire | Theory 30mint-Hrs Practical-02Hrs Total- 03Hrs | - First Aid Equipment’s - Fire Safety Equipment - PPE - Other Safety Equipment - Firefighting Equipment |  |
| LU9 Monitor activities of people, vehicles, and other equipment in area | - Identify different types of movement in work area - Respond to signals or traffic control person - Communicate with site person | - Monitoring techniques of movements in work area - Understanding of safety and communication signals in the work area Practical Perform monitoring of movements in work place and follow communication signals | Theory-30mint Practical-03Hrs Total- 3;30Hrs | - Observation report forms - Cctv cameras - PPE (reflective jacket, safety helmet gloves etc.) - Computer with monitoring software - Traffic & Vehicle Control Tools - First aid Kit |  |
| LU10. Investigate Incident At Workplace | - Identify incidents causes - Collect relevant data for evidences - Analyze the accident and plan a control measure - Implement the plan | - Sources of reliable information - Types incidents and collection evidences - Data collection and implementation of control plan Practical Identify the type of incident and recommend measures | Theory-30mintHrs Practical-02Hrs Total- 2;30Hrs | - Documentation & Recording Tolls - Evidence Collection Materials - PPE - Other Safety Equipment - Interview & Co |  |

## 3.3 0716MSA1903 Interpret Basic Electrical Principles

Objective This Module covers the knowledge and skills required to Interpret Electrical Basic Principles imperative for Selection of the required Cables and Wires, operation of safety circuits and carrying out connections of electrical circuits. After completing this Module, the student/Trainee will be able to achieve all the learning outcomes mentioned in this Module in accordance with the essence of the Competency-Based Training & Assessment System.

Theory: 06 Hours				Practice:  018 Hours 					Duration:  24 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Carry Out Visual Inspection of The EV Circuit. | - Turn off the vehicle, isolate it from high voltage, and wear proper PPE. - Inspect harnesses, cables, and connectors for cracks, cuts, burns, wear, or other physical damage. - Verify connectors are secure, free from corrosion or moisture, and confirm no loose or exposed HV lines. - Check high-voltage cables for correct color coding, proper routing, and no excessive bending, pinching, or rubbing. - Look for discoloration, melting, burn marks, or unusual smells that indicate overheating. - Inspect fuses, relays, junction boxes, control units, and enclosures for damage, missing bolts, clips, or covers. | - Procedure for Turning off, isolation form HV and wearing of PPE before inspection. - Inspection techniques of cables, harnesses, and connectors for damage or wear. - Verification of secure connections, proper routing, and no HV exposure. - Checking of fuses, relays, and control units for damage or missing parts. Practical Activity - Perform visual inspection of the EV to check harnesses, cables, and connectors for cracks, cuts, burns, or wear. | Theory-1:30Hrs Practical-4Hrs Total- 5:30Hrs | - Flashlight or inspection light - Screwdrivers and pliers - Wrenches and socket set - Magnifying glass (optional) - Multimeter (for basic continuity checks) - Materials: - Service manual - Cleaning cloths - Safety gloves and glasses - Notepad for recording observations | Class Room and workshop |
| - Operate the Safety Circuits | - Follow the manufacturer’s operation manual and EV safety guidelines. - Verify installation and accessibility of safety switches, relays, and emergency stop devices. - Confirm circuit protection devices, including fuses, breakers, and disconnects, are functional. - Check high-voltage interlock loop (HVIL), emergency shut-off, and warning indicators for correct operation. - Monitor voltage, current, and communication signals such as CAN bus and sensors during activation and deactivation. - Ensure the safety circuit meets OEM specifications and regulatory requirements, and cannot energize without engagement. - Reset all safety switches and devices to their default positions. | - Safety guidelines and manufacturer instructions. - Verification of Installed safety and protection devices. - Monitoring of HVIL, emergency stops, and system signals during operation. - Ensure circuits meet OEM standards and reset devices after testing. Practical Activity - Perform testing of safety switches, relays, fuses, breakers, and emergency stop devices. - Monitor voltage, current, and CAN bus signals during activation/deactivation. | Theory-1:15Hrs Practical-4:00Hrs Total- 5:15Hrs | - Insulated gloves - Safety goggles - Insulated tools set - Multimeter - Test lamp - Circuit breaker lockout/ tag out kit - Warning signs/labels - EV service manual / wiring diagram | Class Room and workshop |
| - Select the required Cables and Wires. | - Interpret manufacturer specifications and industry standards (IEC, SAE, ISO) for correct cable and wire selection. - Review circuit diagrams and load requirements to determine appropriate cable size and current carrying capacity. - Match wire gauge with current rating and ensure safety margin above maximum load. - Verify cable temperature rating and insulation type (PVC, XLPE, silicone) for heat, chemical, and mechanical resistance. - Use shielded or twisted cables where EMI protection is required, and confirm double insulation for HV circuits. - Ensure correct color coding, such as orange for HV and blue/black for LV. - Match cables with suitable connectors, terminals, crimps, and HVIL system compatibility. - Label cables for proper identification before installation. | - Interpretation of standards and specifications for cable selection. - Cable size, current rating, and insulation type. - Proper shielding, color coding, and HVIL compatibility. - Labeling and verification of cables with correct connectors and terminals. Practical Activity - Perform the selection , labeling and installation of cables with correct connectors, terminals, and routing. - Verify compliance with current rating, insulation, and HVIL requirements. | Theory1:45Hrs Practical-5Hrs Total- 0Hrs | - Different types of automotive cables (low voltage, high voltage, signal wires) - Wire gauges (various sizes) - Cable lugs and terminals - Insulation tape - Heat shrink tubing - Wire stripper and cutter - Crimping tool - EV wiring diagram / service manual | Class Room and workshop |
| - Perform Connection of Electrical Circuits | - Isolate circuits from the power supply, wear PPE, and review circuit diagrams before starting work. - Select cables, wires, and connectors as per specifications for voltage, current, and polarity. - Cut, strip, and prepare wires to required length without damaging conductors. - Fit terminals, lugs, or crimps using proper tools and make tight, corrosion-free connections with correct torque. - Connect wires according to circuit design, ensuring correct polarity, sequence, and routing. Avoid loose, exposed, or twisted joints. - Insulate joints with heat-shrink, tape, or sleeves, then route and secure wires to prevent vibration, sharp edge damage, or overheating. - Test continuity, resistance, and HV interlock integrity using a multimeter or insulation tester, confirming no short circuits or open connections. - Reconnect power supply safely and test circuit functionality against design specifications, checking indicator lights, relays, and fuses. | - Safety procedures and review of circuit diagrams before work. - Preparing and connecting wires with correct terminals, polarity, and torque. - Insulation, securing and routing of wires to prevent damage or shorts. - Testing continuity, HV interlock, and circuit functionality after installation. Practical Activity: - Cut, strip, and connect wires using correct terminals and torque and - Test continuity, HV interlock, and circuit operation safely. | Theory1:30Hrs Practical-5:00Hrs Total- 0Hrs | - Electrical cables/wires - Connectors and terminals - Insulation tape - Heat shrink tubing - Wire stripper and cutter - Crimping tool - Multimeter - Screwdrivers (insulated) - EV wiring diagram / service manual | Class Room and workshop |

## 3.4 0716MSA1904 Maintain Electric Vehicle (EV) Battery System

Objective: This module is designed to provide skills and knowledge for maintain an EV battery system, practice smart charging by keeping the charge between 20% and 80% for daily use, minimizing rapid charging, and avoiding extreme temperatures by charging in moderate conditions. Also, adopt a smoother driving style by minimizing rapid acceleration and braking, and use regenerative braking to reduce battery strain. Regular servicing, following manufacturer guidelines, and using pre-conditioning features when available can also help prolong battery life and performance. After completing this Module, the student/Trainee will be able to achieve all the learning outcomes mentioned in this Module in accordance with the essence of the Competency-Based Training & Assessment System.

Theory: 15 Hours				Practice:  40 Hours 					Duration:  60 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Identify different Types of Batteries | - Wear appropriate PPE and follow OEM and workshop safety guidelines when handling batteries. - Ensure proper ventilation during inspection of lead-acid or lithium batteries. - Verify and distinguish between high-voltage batteries (orange/warning labels) and 12V auxiliary batteries. - Identify SLI batteries (starting, lighting, ignition) versus traction or propulsion batteries in EVs. - Differentiate between primary (non-rechargeable) and secondary (rechargeable) batteries. - Recognize battery applications in EV/HEV systems, including HV traction, 12V auxiliary, and support batteries. - Identify battery types by construction and chemistry, including Lead-Acid (Flooded, AGM, Gel), NiMH, Lithium-Ion variants (LFP, NMC, NCA), solid-state, alkaline, and other specialized types. | - Introduction to Personal Protective Equipment’s (PPE’s) - Function, types and application of battery - Working principal and construction of battery - High vs. low voltage batteries charging/discharging indication and its technical parameter - High/low voltage battery in EV car Practical Activity: - Identify different types of batteries and demonstrate applications of batteries in EV | Theory 3Hrs Practical 6 Hrs Total- 09 Hrs | - PPE’s (High voltage insulated gloves, fire resistant Apron, Goggles, Safety helmet, Safety Shoes) - Service manual and guidelines - High voltage battery batteries - Low voltage battery | Class Room and workshop |
| LU2. Check EV Battery Performance | - Review the OEM service manual, battery specifications, and safety requirements before testing. - Select and calibrate appropriate test equipment such as a multimeter, insulation resistance tester, scan tool, or battery analyzer. - Identify the battery type, voltage, capacity, and chemistry, then inspect for physical damage, leakage, corrosion, or swelling. - Verify proper connections of terminals, busbars, and harnesses, and measure voltage, current, and SOC using diagnostic tools. - Interpret BMS data for cell voltage, SOC, SOH, and imbalance, and perform internal resistance or health checks with analyzers - Check battery temperature and cooling system, then test under load to assess voltage drop, current delivery, and performance against manufacturer specifications. - Identify faulty cells or modules, and document test results, observations, and recommendations for repair, replacement, or maintenance. | - interpretation of service manual - Calibration and use of testing/analyzing equipment (DDM, Scanner, battery tester) - Types and specification of EV batteries - Inspection techniques of high voltage battery - knowledge of high voltage battery Input/output voltage, current state (SOC), battery health and temperature - Function of Battery management system (BMS) of EV - Scanning procedure of Battery management system (BMS) of EV Practical Activity - Perform battery health test and state of charge (SOC) | - Theory 3 Hrs - Practical-7.5 Hrs - Total- 10.5 Hrs | - PPE’s (High voltage insulated gloves, fire resistant Apron, Goggles, Safety helmet, Safety Shoes) - Service manual and guidelines - High voltage battery batteries, - Low voltage battery - Battery tester, digital multimeter (DMM), Scanning tools, - EV car | Class Room and workshop |
| LU3. Clean the Terminal for any Contamination | - Follow manufacturer’s instructions for approved cleaning agents and inspect for contamination, corrosion, dust, grease, oxidation, or physical damage. - Verify insulation and sealing integrity, then neutralize acid corrosion on lead-acid terminals using baking soda solution. - Clean HV connectors with dielectric/contact cleaner only, and wipe all surfaces with a lint-free cloth until clean. - Apply petroleum jelly or anti-corrosion spray on 12V terminals, and reseal HV connectors with proper gaskets, covers, and insulation. - Ensure terminals are dry, free of particles, and reconnect in the correct sequence with tight, vibration-resistant connections. - Test continuity and voltage after cleaning, and confirm no abnormal heating or sparking during operation - Verify no abnormal heating or sparking at terminals after operation. | - Interpretation of service manual for cleaning of battery terminals - Knowledge of cleaning agents and cleaning techniques of high voltage battery terminal - Procedure of HV battery terminal connections and checking - Identification of physical damages (cracks, burns, pitting) - Practical Activity - Perform cleaning of HV battery terminals for smooth functioning | Theory-3 Hrs Practical-7.5 Hrs Total- 10.5 Hrs | - PPE’s (High voltage insulated gloves, fire resistant Apron, Goggles, Safety helmet, Safety Shoes) - Service manual and guidelines - High voltage battery batteries, - Low voltage battery - Cleaning agents, Yellow cloths/duster - Battery tester, digital multimeter (DMM), Scanning tools, - EV car | Class Room and workshop |
| LU4. Perform the tightening of connectors | - Review OEM specifications for torque values and select proper insulated tools such as torque wrenches, spanners, or sockets. - Inspect connectors, terminals, pins, and sockets for contamination, corrosion, or damage, and replace faulty parts if needed. - Ensure connectors are properly seated, aligned, and that insulation, boots, or protective covers are intact. - Tighten connectors to specified torque using insulated tools, avoiding under-tightening or over-tightening. - Apply dielectric grease or anti-corrosion compound where recommended. - Perform continuity or voltage drop tests to confirm good electrical contact. - Inspect for looseness, misalignment, or abnormal heating and sparking during and after system re-energization. | - Interpretation of service manual for tightening (torque value) of battery terminals/connectors - Selection of proper tools for terminals tightening - Inspecting techniques of connector and terminal for contamination, corrosion, or damage before tightening - Checking procedure for the connectors that are properly seated and aligned - Checking techniques of continuity and voltage at the terminal after tightening - Practical Activity - Perform tightening of connectors as per OEM | Theory-03Hrs Practical-7.5 Hrs Total- 10.5 Hrs | - PPE’s (High voltage insulated gloves, fire resistant Apron, Goggles, Safety helmet, Safety Shoes) - Service manual and guidelines - High voltage battery batteries, - Low voltage battery - Combination pliers, spanner set, Philips screw driver, flat screw driver - Battery tester, digital multimeter (DMM), scanning tools, | Class Room and workshop |
| LU5. Repair/Replace the Faulty Component of The EV Battery | - Wear full PPE and discharge the battery safely according to OEM guidelines - Use OEM diagnostic tools or BMS to identify faults in the battery system. - Inspect cells, modules, connectors, bus-bars, fuses, and cooling systems for defects. - Measure voltage, resistance, and temperature of cells or modules to confirm faults. - Select proper insulated tools, lifting devices, and gather correct replacement components as per OEM specifications. - Disconnect harnesses, bus-bars, and connectors, then remove the faulty part without damaging adjacent components. - Position and install the replacement part, reconnecting terminals, bus-bars, and cooling connections with specified torque. - Apply insulation, seals, or dielectric grease if required, restore HVIL, and reconnect cooling and ventilation systems. - Reconnect power supply, perform diagnostics to clear fault codes, check SOC, SOH, and cell balancing, and monitor for heating, noise, or voltage drops. | - Checking techniques of Battery management system (BMS) - Rectification techniques regarding damage/bad battery components - Procedure for re-assembling/connecting and insulating of the battery parts/terminals/connectors - Fitting and installation techniques of EV batteries - Practical Activity - Repair/replace the HV battery according to BMS | Theory 3 Hrs Practical-12 Hrs Total- 15 Hrs | - Insulated screwdrivers and pliers - Socket and wrench set - Torque wrench - Multimeter and clamp meter - Battery diagnostic tool - Safety gloves and high-voltage rated tools - Lifting equipment (if needed) - Replacement battery module or cells - Service manual - Electrical contact cleaner - Insulated mats - Safety glasses and protective clothing - Rags and cleaning cloths | Class Room And workshop |

## 3.5 0716MSA1905 Maintain Electric Vehicle (EV) Motor

Objective: This module is designed to provide skills and knowledge to repair and maintain an EV motor, follow your vehicle's specific service schedule, ensure the motor's cooling system is functioning correctly by checking coolant levels and avoiding overheating, and inspect for any damage or debris that may have accumulated. While electric motors require less maintenance than traditional engines, regular professional servicing, checking fluids, and keeping software updated are crucial for optimal performance and longevity.

Theory: 10 Hours				Practice:  30 Hours 					Duration:  40 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Identify the types of EV Motors | - Identify the common types of EV motors such as DC Brushed, BLDC, PMSM, Induction Motor, and Switched Reluctance Motors - Differentiate motors based on construction, including rotor, stator, magnets, and windings. - Classify motors by supply type (AC or DC) and control requirements. - Recognize operating characteristics like torque, efficiency, speed range, and cooling needs, along with advantages and disadvantages. - Identify typical EV applications of each motor type, and compare performance parameters such as efficiency, torque density, cost, and reliability. - Verify motor type against manufacturer specifications or datasheets and document findings with correct naming, classification, and applications | - Identification of common EV motor - Types and construction of EV motor. - Classification of motors by supply and control type. - Operating characteristics and pros/cons. - Application and, performance - Practical Activity - Differentiate between motors based on construction (rotor, stator, magnets, and windings). and Classify various motors by supply type (AC or DC) | Theory 02 Hrs Practical06 Hrs Total- 08 Hrs | - Multimeter - Clamp meter - Insulation tester - Tachometer - Oscilloscope - Basic hand tools (screwdrivers, pliers, wrenches) - Service manual - Motor datasheets - Manufacturer specifications - Notepad for observations - Safety gloves and glasses | Class Room and workshop |
| - Perform Visual Inspection of The EV Motor | - Wear PPE and follow high-voltage safety protocols, then power down and isolate the EV before inspection. - Inspect the external motor housing for cracks, corrosion, or other physical damage. - Check cooling system components such as pipes, fans, and radiators for leakage, blockage, or damage. - Examine electrical connectors, terminals, and wiring harnesses for overheating, insulation damage, wear, or looseness, and verify proper grounding and shielding. - Inspect motor mountings, seals, gaskets, and insulation for deterioration, misalignment, or oil/coolant ingress. - Check for dirt, dust, or foreign particles that may affect motor performance or cooling, and confirm motor labels and ratings are intact. - Document inspection findings with photos and notes, and report issues in service records for diagnosis or repair. | - HV safety procedure and isolation of EV before inspection. - Inspection techniques of motor housing, mounts, seals, and cooling system. - Checking procedure of cooling system components such as pipes, fans, and radiators for leakage, blockage, or damage. - Checking wiring, connectors, and grounding for damage. - Recording findings, confirm labels, and report issues. Practical Activity: Perform visual inspection of EV motor to check mountings, insulations, fitting, vibration, noises, connections, sealing, gasket, cracks, misalignment and note down your observation | Theory 02 Hrs Practical06 Hrs Total- 15 Hrs | - Flashlight or inspection light - •Screwdrivers and pliers - Wrenches and socket set - Magnifying glass (optional) - Multimeter (for basic checks) - Service manual - Cleaning cloths - Safety gloves and glasses - Notepad for recording observations | Class Room and workshop |
| - Analyze the Working of EV Motors | - Identify the fundamental principle of EV motors (electromagnetic induction or interaction of magnetic fields). - Demonstrate the conversion of electrical energy into mechanical energy in different motor types. - Differentiate the working of: - DC Motors (BDC, BLDC) – commutation, torque generation. - AC Motors (PMSM, Induction Motor, SRM) – rotating magnetic field, synchronous vs asynchronous operation. - Demonstrate the role of stator (produces rotating field) and rotor (produces torque). - Analyse torque–speed characteristics of each motor type. - Relate motor performance to EV applications (e.g., PMSM for high efficiency, Induction motor for robustness, BLDC for two-wheelers). - Demonstrate the effect of motor control (inverter & controller signals) on speed, torque, and regenerative braking. - Evaluate efficiency, heat generation, and cooling requirements during operation. | - Function and working principal of EV motors - Construction and types of EV motors - Torque, Efficiency and speed/RPM of EV motors - Technical parameter and Control strategy/drive mode of EV motors - Rotation angle sensor of EV motor - EV Motor controller - Motor communication with controller (CAN/ECU) Practical Activity: Check EV motor parameters i.e. input/out voltage, speed/RPM, torque etc. and compare with the standards OEM | Theory 03 Hrs Practical09 Hrs Total- 12 Hrs | - Multimeter - Clamp meter - Oscilloscope - Tachometer - Diagnostic scanner or laptop - Basic hand tools (screwdrivers, pliers, wrenches) Materials: - Motor datasheets and service manual - Safety gloves and glasses - Insulated tools - Notepad for observations and measurements | Class Room and workshop |
| - Replace the Faulty EV Motor | - Follow high-voltage safety procedures, including battery isolation, lock-out/tag-out, and PPE use. - Confirm motor fault through diagnostics, test results, and manufacturer specifications before replacement. - Select correct tools, lifting devices, and replacement motor as per OEM guidelines. - Disconnect the faulty motor from power supply, controller, cooling lines, and mechanical drive, then remove it carefully without damaging nearby components. - Inspect wiring harnesses, connectors, mounts, and cooling pipes during removal, then install the new motor with correct alignment and torque values. - Reconnect electrical, mechanical, and cooling connections, perform insulation resistance and continuity tests, and check coolant or lubrication lines for leaks. - Run a test operation to confirm smooth motor performance, communication with the controller, and compliance with OEM specifications. - Dispose of the faulty motor according to safety and environmental regulations, and document replacement steps, test results, and service updates. | - Interpretation of service manual for EV motor inspection/standards/parameters - Inspection techniques for fault detection of EV motor - Removal procedure for faulty motors - installation techniques of new motor as per manual instructions - Inspection techniques of EV motor operation after installation by using scanning tools - Reconnection techniques of electrical, mechanical, and cooling connections. - Importance of insulation resistance and conduct of continuity tests, - Checking of coolant or lubrication lines for leaks. - Practical Activity - Perform inspection of the EV motor for fault detection. Remove the faulty motor, install the new EV motor, perform connections of the EV motor and carry out the run test for smooth functioning of EV motor adhering to safety protocol | Theory 03 Hrs Practical09 Hrs Total- 12 Hrs | - Socket and wrench set - Screwdrivers and pliers - Torque wrench - Jack and jack stands or lifting hoist - Multimeter - Insulated hand tools - Diagnostic scanner or laptop - Replacement EV motor - Service manual - Electrical contact cleaner - Lubricant (if required) - Safety gloves and glasses - Rags and cleaning cloths EV motors | Class Room and workshop |

## 3.6 0716MSA1906 Maintain Electric Vehicle (EV)Control System

Objective: This module covers the knowledge and skills required to maintain an electric vehicle's (EV) control system by installing regular software updates to enhance efficiency and performance, ensuring the controller is waterproof and free from dust, conducting routine diagnostic checks on the battery and motor to detect and address issues early, preventing battery overheating or freezing, and practicing gentle braking to reduce strain on system components.

Theory: 10 Hours				Practice:  30 Hours 					Duration:  40 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Identify the Fundamentals Of EV Control Systems | - Interpret the purpose of EV control systems in managing power flow, safety, and vehicle operation. - Identify major EV control units such as VCU, MCU, BMS, inverter, and charging controller - Differentiate between hardware components and software functions in EV control systems. - Interpret the role of sensors in providing feedback for voltage, current, speed, torque, and temperature. - Trace data and signal flow between subsystems through CAN bus or other protocols. - Identify faults that occur during interaction of EV control subsystems. - Compare EV control systems with conventional ICE vehicle control systems. - Illustrate the architecture of an EV control system through diagrams or schematics | - Purpose of EV control systems in managing power flow, safety, and vehicle operation. - Identification of major EV control units such as VCU, MCU, BMS, inverter, and charging controller - Diagnostic tools to read voltage, current, speed, torque, and temperature sensors. Recording of real-time values and compare with expected limits. - Tracing signals between VCU, MCU, BMS, and inverter via CAN bus. Map communication flow and potential fault points. - Compilation EV control systems with ICE vehicle control systems. Simulate faults and observe system response. Practical Activity: - Identify major EV control units such as VCU, MCU, BMS, inverter, and charging controller and Trace data and signal flow between subsystems through CAN bus or other protocols. | Theory-1.5Hrs Practical-3Hrs Total- 4.5Hrs | - Basic hand Tool kit - Multimeter - CAN Bus Analyzer / Logger - Diagnostic Scanner / ECU Reader - Battery Simulator/ - Test Bench - Insulation Resistance Tester (Megger) - Safety Equipment (PPE) - Insulation Tape / Heat Shrink Tubing - Old Connectors and Terminals - Thermal Paste / Heat Sink Compound - Replacement Sensors - Cable Ties and Clips - Lubricants (non-conductive | Class Room and workshop |
| - Check Function of The Vehicle Control Unit | - Identify the Vehicle Control Unit (VCU) as the central controller coordinating powertrain, battery, charging, and safety systems. - Identify the major functions of the VCU such as torque management, drive mode selection, regenerative braking control, and subsystem communication. - Inspect the physical condition of VCU connectors, wiring harness, and mounting for possible faults or damages. - Interpret diagnostic trouble codes (DTCs) and warning indicators related to VCU operation using a diagnostic scanner. - Test VCU input signals (accelerator, brake, gear selector) and output commands (motor torque, regenerative braking). - Verify safety functions managed by the VCU including emergency shutdown, interlocks, and isolation monitoring. - Compare observed VCU performance with manufacturer specifications and document test results. | - Identification of VCU and its role in controlling powertrain, battery, charging, and safety systems. - Inspection VCU connectors, wiring, and mounting for faults. - Testing input signals (accelerator, brake, gear selector) and output commands (motor torque, regenerative braking). - Verification of safety functions and comparison of VCU performance with manufacturer specifications. - Identification of diagnostic trouble codes (DTCs) and warning indicators related to VCU operation using a diagnostic scanner. - Testing procedure of VCU input signals (accelerator, brake, gear selector) and output commands (motor torque, regenerative braking). Practical Activity: - Inspect the VCU’s condition, verify CAN communication, interpret diagnostic codes, test input/output signals, check safety functions and firmware, compare performance to specs, document results. | Theory-01Hrs Practical-03Hrs Total- 04Hrs | •Basic hand Tool kit •Multimeter CAN Bus Analyzer / Logger Diagnostic Scanner / ECU Reader Battery Simulator/ Test Bench Insulation Resistance Tester (Megger) Safety Equipment (PPE) Insulation Tape / Heat Shrink Tubing • Old Connectors and Terminals • Thermal Paste / Heat Sink Compound • Replacement Sensors • Cable Ties and Clips • Lubricants (non-conductive | Class Room and workshop |
| - Demonstrate the Role Of Drive Motor Controller | - Identify the drive motor controller as the interface between the Vehicle Control Unit (VCU) and the electric motor. - Recognize the main functions of the motor controller such as torque control, speed regulation, direction control, and regenerative braking. - Demonstrate how the controller regulates voltage, current, and frequency to convert VCU commands into motor operation. - Interpret signals from sensors like rotor position, speed, current, and temperature used for feedback control. - Differentiate between motor controls methods including Field-Oriented Control (FOC), Direct Torque Control (DTC), and Pulse Width Modulation (PWM). - Illustrate the role of the motor controller in energy efficiency and smooth power delivery in EVs. - Trace the process of regenerative braking and explain how the motor controller assists in energy recovery. - Verify motor controller operation using diagnostic tools and confirm correct communication with the VCU and inverter | - Identification face role of motor controller between VCU and motor. - Functions of motor controller: torque, speed, direction, regen braking. - Conversion of VCU commands to motor operation. - Inspection of regenerative braking process and energy recovery. - Operation checks via diagnostics and ECU data & communication with inverter and VCU via CAN. Practical Activity: - Demonstrate how a motor drive converts VCU commands to motor operation, interpret sensor feedback, differentiate control methods, trace regenerative braking, verify CAN communication and document the test results adhering to safety procedures. | Theory-1.5Hrs Practical-03Hrs Total- 4.5Hrs | - Basic hand Tool kit - Digital Multimeter - Oscilloscope - CAN Bus Analyzer / Logger - Diagnostic Scanner / ECU Reader - Thermal Imaging Camera - Data Logger - PPE (Insulated gloves, face shield, dielectric mats - Electrical Tape / Heat Shrink - Cable Ties / Clips - Replacement Connectors/ - Terminals - Thermal Paste - Tags & Labels | Class Room and workshop |
| - Analyze the Process Of Regenerative Braking | - Identify the basic purpose of regenerative braking in recovering kinetic energy and converting it into electrical energy. - Recognize the roles of key components in regenerative braking, including the motor, inverter, controller, BMS, and VCU. - Trace the energy flow during braking from wheels to motor (generator mode), inverter, and battery. - Differentiate regenerative braking from conventional friction braking in terms of operation and energy use. - Analyze factors affecting regenerative braking efficiency such as state of charge, battery temperature, road conditions, and speed. - Interpret ECU or CAN bus data related to regenerative braking performance. - Evaluate the contribution of regenerative braking to extended driving range and reduced brake wear. - Verify smooth blending between regenerative braking and friction braking in line with manufacturer specifications. | - Concept of regenerative braking and energy recovery. - Major components of regenerative braking: motor, inverter, motor controller, BMS, VCU. - Energy flow during braking: wheels → motor → inverter → battery. - Difference between Regenerative & conventional friction braking. Practical Activity: - Identify components and energy flow in regenerative braking, compare with friction braking and analyze efficiency factor. - Interpret EV diagnostic data, verify system performance during drive mode transitions, apply high-voltage safety procedures, and document all faults and findings in a test report. | Theory-1.5Hrs Practical-03Hrs Total- 4.5Hrs | - Basic hand Tool kit - Digital Multimeter - Oscilloscope - CAN Bus Analyzer / Logger - Diagnostic Scanner / ECU Reader - Thermal Imaging Camera - Brake Pad Depth Gauge / Caliper Tool - PPE (Insulated gloves, face shield, dielectric mats) Cable Ties / Clips - Electrical Tape / Heat Shrink - Test Drive Forms / Print Logs - Tags & Safety Labels | Class Room and workshop |
| - Fix the Fault In The EV Control System | - Identify symptoms of faults in the EV control system using inspection and diagnostic tools. - Verify faults against manufacturer specifications, diagnostic codes, and performance data. - Apply systematic troubleshooting techniques to isolate faulty subsystems such as VCU, BMS, MCU, inverter, or charger controller. - Analyze CAN bus communication to detect interruptions or incorrect signals. - Repair or replace faulty wiring, connectors, sensors, or control units following OEM standards. - Perform functional and communication tests to confirm rectified faults and restored performance. - Document faults, corrective actions, replaced parts, and final test results in maintenance records. | - Inspection of EV controls system components (VCU, BMS, MCU, inverter, charger controller) for visible issues. - Analyzing CAN bus communication for interruptions or incorrect signals. - Connections, wiring integrity, and sensor alignment. - Identification of faulty wiring, connectors, sensors, or control units. Practical Activity: - Inspect EV control system for faults Detection using diagnostic codes, analyze CAN bus data. Carry out repair / replacement of the faulty components and verify system for smooth functioning. - Preform visual inspection of wiring harnesses, connectors, sensors, and controllers for damage | Theory-1.5Hrs Practical-6Hrs Total- 7.5Hrs | - Basic hand Tool kit - Multimeter - Oscilloscope - CAN Bus Analyzer / Logger - Thermal Camera - Voltage Tester - PPE (Insulated gloves, face shield, dielectric mats) - Electrical tape - Heat shrink tubing - Contact cleaner - Replacement wires, terminals, fuses - Replacement sensors or modules - Safety tags and labels | Class Room and workshop |
| - Repair/Replace the Faulty Components | - Identify the faulty component(s) in the EV system through inspection, diagnostic tools, and test procedures. - Verify the fault using manufacturer specifications, diagnostic codes, and performance data. - Select appropriate tools, equipment, and service manuals for the repair/replacement task. - Isolate high-voltage circuits and apply all safety procedures (PPE, insulated tools, lock-out/tag-out). - Remove the faulty component carefully without causing damage to surrounding parts. - Repair the component if possible (e.g., wiring, connectors, minor parts) following OEM standards. - Replace the component if repair is not feasible, using OEM-approved or equivalent spare parts. - Reinstall the repaired/replaced component securely, ensuring proper connections and torque values. - Reconnect electrical/hydraulic/pneumatic systems as required and check for leaks, loose fittings, or improper alignment. - Perform functional tests to ensure the repaired/replaced component operates correctly. - Verify that the overall system performance is restored to manufacturer specifications. - Dispose of damaged or replaced parts in compliance with environmental and safety regulations. - Document all repair/replacement activities, parts used, and final performance results in service records. | - Detection of faulty component identification using inspection, diagnostics, and test procedures. - Fault verification through manufacturer specifications, DTCs, and performance data. - High-voltage circuit isolation and application of safety procedures (PPE, insulated tools, lock-out/tag-out. - Component repair methods as per OEM standards (wiring, connectors, minor parts). - Secure reinstallation with correct connections and torque values. - System reconnection checks for leaks, loose fittings, and alignment. - Functional testing of repaired or replaced components. - System performance verification against manufacturer specifications. Practical Activity: - Identify and verify faulty EV components, isolate high-voltage circuits safely. - Repair and replace faulty parts with OEM standards. - Perform functional tests to ensure the repaired/replaced component operates correctly. | Theory-1.5Hrs Practical-06Hrs Total- 7.5Hrs | Consumable: - Basic hand Tool kit - Multimeter - Oscilloscope - CAN Bus Analyzer / Logger - Diagnostic Scanner - Heat Gun - Test Probes / Connector Tools - Inspection Mirror / Flashlight - PPE (Insulated gloves, face shield, dielectric mats) Non-Consumable: - Contact Cleaner - Heat Shrink Tubing - Electrical Tape - Wire Terminals / Pins - Warning Labels / Safety Tags - OEM Replacement Parts (sensor, ECU, wiring) - Waste Labels | Class Room and workshop |
| - Carrie Out A Test Drive of EV Controls System. | - Plan and prepare a test drive according to OEM safety and operational guidelines. - Inspect the EV before the test drive, checking SOC, tire pressure, warning lights, and fault codes. - Operate the EV under different driving conditions to evaluate control system responses. - Monitor VCU, MCU, regenerative braking, and communication data during the drive. - Identify abnormalities such as warning messages, vibrations, or performance drops. - Document test drive results, diagnostic data, and corrective actions in service records. | - OEM guidelines for Preparation and inspection of EV. - Conduct of test drive under varied conditions. - Monitoring of system data and performance. - Record findings and suggest corrections - Practical Evidence: - Demonstrate the OEM test drive procedures and safety checks. - Perform EV pre-drive inspection and test drive. | Theory-1.5Hrs Practical-6Hrs Total- 7.5Hrs | - OBD-II scanner or diagnostic tablet - Multimeter - Tire pressure gauge - Stopwatch or timer - Laptop for data logging - Basic hand tools (screwdrivers, wrenches, pliers) - Service manual - Vehicle inspection checklist - Safety gloves and glasses - Notepad for observations - Seatbelts and safety cones for test area | Class Room and workshop |

## 3.7 0716MSA1907 Perform Electric Vehicle (EV) Charging System

Objective: This module covers the knowledge and  skills required to service and repair EV charging systems, connect the appropriate cable from a charging station or home charger to the EV charging port, apply the correct charging level (Level 1, Level 2, or Level 3) as required, understand the conversion of AC grid power to DC for battery charging, monitor the charging process through the vehicle interface or cloud-based systems, and complete charging when the battery reaches the required level. .

Theory: 10 Hours				Practice:  30 Hours 					Duration:  40 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Differentiate Between Types of EV Chargers | - Identify the different levels of EV chargers (Level 1, Level 2, and DC Fast Charging) along with their voltage, power, and charging time. - Differentiate between AC and DC charging systems in terms of functionality, efficiency, and application. - Identify international connector/plug standards (Type 1, Type 2, CHAdeMO, CCS, GB/T, Tesla) and their regional usage. - Classify chargers based on installation location (home, workplace, public, and highway fast charging). - Compare the advantages, limitations, and suitable applications of each charger type. - Demonstrate knowledge of safety considerations, compatibility issues, and charging infrastructure requirements. - Illustrate differences using real-world examples (e.g., slow home charging vs highway ultra-fast charging). - Identify key cooling system components, including the radiator, water pump, thermostat, cooling fan, hoses, and coolant reservoir. | - Identification EV charger levels (Level 1, Level 2, DC Fast Charging) with voltage, power, and charging time. - Differentiation of AC and DC charging systems by functionality, efficiency, and application. - Identification of international connector standards (Type 1, Type 2, CHAdeMO, CCS, GB/T, Tesla) and their regional usage. - Classification of chargers by installation location: home, workplace, public, and highway fast charging. - Comparison, advantages, limitations, and applications of each charger type, including safety, compatibility, and infrastructure requirements. Practical Activity: - Identify and compare charger levels (Level 1, Level 2, DC Fast) through observation and connection with EVs - Identify connector types (Type 1, Type 2, CHAdeMO, CCS, GB/T, Tesla) and note their regional usage. | Theory-02 Hrs. Practical-06 Hrs. Tof otal- 08 Hrs. | - Multimeter - Clamp meter - Insulation tester - Portable EVSE tester - Diagnostic scanner - Basic hand tools (screwdrivers, pliers, wrenches) - Service manual - Charging cables and connectors (Type 1, Type 2, CCS, CHAdeMO) - Safety gloves and glasses - Insulated mats - Notepad for readings and observations | Class Room and workshop |
| - Identify Components of EV Charging System | - Identify the main external charging components (charging gun/connector, charging cable, charging port/inlet). - Identify the types of charging connectors (Type 1, Type 2, CHAdeMO, CCS, GB/T, Tesla). - Differentiate between AC charging equipment (on-board charger, charging station) and DC fast charging equipment (off-board charger, DC supply unit). - Locate vehicle-side charging components (on-board charger, charge port, BMS, VCU integration). - Identify protective components (fuses, relays, circuit breakers, contactors, insulation monitors, earthing). - Identify cooling and thermal management parts for high-power charging (cooling fans, liquid-cooled cables, heat exchangers). - Identify communication interfaces (CAN, PLC communication between EVSE and vehicle). - Interpret the role of the Battery Management System (BMS) in charge monitoring, balancing, and protection | - Main - External charging component including gun, cable, and port. - Charging connector types such as Type 1, Type 2, CHAdeMO, CCS, GB/T, and Tesla. - Differentiation between AC charging equipment and DC fast charging equipment. - Identification of cooling and thermal management parts used in high-power charging. - Interpretation of communication interfaces and BMS role in monitoring, balancing, protection, and safety interlocks... Practical Activity: - Identify connector types such as Type 1, Type 2, CHAdeMO, and CCS. Examine protective devices like fuses, relays, breakers, and contactors. - Check cooling parts such as fans, liquid-cooled cables, and heat exchangers. | Theory-03Hrs Practical-08Hrs Total- 11Hrs | - Multimeter - Clamp meter - Insulation tester - Diagnostic scanner - Basic hand tools (screwdrivers, pliers, wrenches) - Materials: - Service manual - EV charging circuit diagram - Safety gloves and glasses - Insulated tools - Notepad for component labeling and notes | Class Room and workshop |
| - Calculate the Charging Time of EV Battery. | - Identify the EV battery specifications (capacity in kWh, State of Charge (SOC), and usable capacity). - Determine the charging power available (AC Level 1, AC Level 2, DC fast charging, kW rating of charger).. - Differentiate charging times for AC slow charging vs DC fast charging. - Estimate partial charging times (e.g., 20% to 80% SOC) commonly used in EV operations. - Verify results by cross-checking with manufacturer-provided charging time data. - Interpret variations in charging time caused by factors like battery temperature, charger limitations, and grid voltage. - Document calculated charging times for different scenarios (slow charge, fast charge, full charge, partial charge. | - EV battery specifications including capacity in kWh, State of Charge (SOC), and usable capacity. - Determination of available Charging power for AC Level 1, AC Level 2, and DC fast charging, including charger kW rating. - Differentiation of charging times between AC slow charging and DC fast charging. - Estimation of partial charging times (e.g., 20% to 80% SOC) and verify against manufacturer-provided data. - Recording the charging time changes from battery temperature, charger limits, and voltage. Practical Activity: - Determine available charging power for AC Level 1, AC Level 2, and DC fast charging, including charger kW rating. - Monitor battery temperature, charger output, and grid voltage during charging. | Theory-02Hrs Practical-08Hrs Total- 10Hrs | - Multimeter - Clamp meter - Stopwatch or timer - Calculator or diagnostic software - Basic hand tools (if needed) - Materials: - Service manual - EV battery and charger specifications sheet - Safety gloves and glasses - Notepad for calculations and readings | Class Room and workshop |
| - Repair/Replace The Faulty Components Of The EV Charging System. | - Inspect the EV charging system for fault detection - Identify faulty components in the EV charging system using diagnostic tools, and error codes. - Verify the fault by comparing test results with manufacturer’s specifications... - Remove the defective component carefully without causing damage to wiring, connectors, or surrounding parts. - Repair minor faults (e.g., wiring damage, connector cleaning, fuse replacement) where possible. - Replace non-repairable components (e.g., charging inlet, on-board charger, relays, fuses, contactors, cooling fan). - Reinstall the repaired/replaced part securely and connect all wiring with correct polarity and torque values. - Check insulation resistance, continuity, and grounding after repair/replacement. - Perform a test charge to ensure the system functions correctly and safely. | - Inspection and fault detection in EV charging systems using diagnostic tools and error codes. - Inspection the EV charging system and identify faulty components using diagnostic tools and error codes. - Verifying faults by comparing test results with manufacturer specifications. - Repairing minor faults or replace parts like inlet, charger, relays, or fuses. - Reinstallation of parts securely, - Verification of wiring, polarity, torque, and check insulation. Practical Activity: - Inspect EV charging systems, detect faults using diagnostic tools, and verify errors with OEM specifications. - Perform test charging system after repair and document results, following safety rules. | Theory-03Hrs Practical-8Hrs Total- 11Hrs | - Multimeter - Insulation resistance tester - Clamp meter - Torque wrench - Screwdrivers and pliers - Socket and wrench set - Diagnostic scanner - Replacement components (charging inlet, fuses, relays, cooling fan, onboard charger) - Service manual - Insulated tools and mats - Electrical contact cleaner - Safety gloves and glasses - Cleaning cloths | Class Room and workshop |

## 3.8 0716MSA1908 Maintain Electric Vehicle (EV) Cooling System

Objective: This module covers the knowledge and skills required to maintain an electric vehicle's cooling system, perform regular inspections for leaks and damage, follow the manufacturer's schedule for coolant flushes and replacements, and keep the radiator and Air ducts clear of debris. For liquid-cooled systems, this means checking the coolant's condition and levels, while Air-cooled systems require ensuring proper airflow through clear ducts. Promptly address any warning signs like unusual noises or smells, and for optimal performance, park in shaded areas to keep the battery cool.

Theory: 05 Hours				Practice:  20 Hours 					Duration:  25 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Identify the Components of EV Cooling Systems | - Identify major cooling system components (radiator, coolant pump, reservoir tank, cooling fans, heat exchangers). - Identify cooling lines, hoses, and valves that circulate coolant within the system. - Differentiate between cooling requirements of battery pack, inverter, power electronics, and electric motor. - Identify sensors and control units (temperature sensors, coolant flow sensors, ECU for thermal management). - Interpret the function of auxiliary systems (liquid cooling plates, chiller, heater, expansion tank). - Classify cooling methods used in EVs (air cooling, liquid cooling, refrigerant-based cooling). - Verify the location and connection of vehicle-side vs charger-side cooling system components (in case of DC fast charging). - Demonstrate knowledge of safety precautions while inspecting cooling systems (handling coolant, checking for leaks, pressure release) | - Identification of cooling parts and system needs for battery, inverter, motor, and sensors - Function of auxiliary systems: liquid cooling plates, chiller, heater, expansion tank. - Classification of cooling types: air, liquid, refrigerant, and locate system components. - Safety steps when handling coolant, checking leaks, and releasing pressure. Practical Activity: - Identify sensors and control units for temperature and coolant flow. - Measure coolant temperature at battery pack, inverter, and motor | Theory-0.5Hrs Practical-03Hrs Total- 3.5Hrs | - Wrenches and screwdrivers - Pliers - Coolant pressure tester - Multimeter - Infrared thermometer - Jack and stands - Coolant - Service manual - Replacement hoses or clamps - Cleaning cloths - Safety gloves and glasses Inspection light | Class Room and workshop |
| - Check EV Cooling Systems for Fault Detection | - Identify standard operating parameters of the EV cooling system (temperature range, coolant level, pressure, flow rate). - Inspect cooling system components (radiator, hoses, reservoir, cooling pump, fans, and connectors) for leaks, wear, or damage. - Check coolant quality (level, color, contamination, concentration) and ensure it meets manufacturer’s specifications. - Test operation of coolant pump, fans, and valves using diagnostic tools. - Measure coolant flow and system pressure with appropriate gauges/sensors. - Interpret diagnostic trouble codes (DTCs) and warning indicators from the thermal management ECU. - Detect abnormal noises, vibrations, or overheating signs that indicate cooling faults. - Differentiate between electrical faults (fan motor, pump motor, sensor malfunction) and mechanical faults (leaks, blockages, damaged hoses).. | - EV cooling system standards like temperature, coolant level, pressure, and flow. - Inspection of radiator, hoses, pump, fans, and connectors for leaks or damage. - Checking coolant quality for level, color,ete. - Testing pump, fans, and valves, and measure coolant flow and pressure. - Interpretation of diagnostic trouble codes (DTCs), noises or overheating for fault find. - Practical Activity - Detect abnormal noises, vibrations, or overheating signs that indicate cooling faults and apply systematic troubleshooting techniques to locate root causes of faults. | Theory-01Hrs Practical-03Hrs Total- 04Hrs | - Diagnostic scanner - Multimeter - Infrared thermometer - Coolant pressure tester - Flow meter - Basic hand tools (wrenches, pliers, screwdrivers) - Service manual - Coolant - Replacement hoses or clamps - Cleaning cloths - Safety gloves and glasses | Class Room and workshop |
| - Repair/Replace the Faulty Components of The EV Cooling Systems | - Identify faulty cooling system components (radiator, pump, hoses, reservoir, fans, sensors, valves) through inspection and diagnostics. - Interpret diagnostic codes, warning lights, and ECU data related to thermal management faults. - Follow standard operating procedures (SOPs) and manufacturer’s guidelines for repair/replacement. - Select correct tools, equipment, and replacement parts suitable for the specific EV model. - Demonstrate safe handling of coolant, high-voltage connections, and thermal management electronics. - Repair minor faults (tightening connections, fixing leaks, cleaning blockages, sensor recalibration). - Replace major defective components (coolant pump, radiator, cooling fan, expansion tank, control unit, hoses). - Refill & bleed the cooling system with recommended coolant to remove air pockets and ensure proper circulation. - Test the cooling system operation after repair/replacement (temperature stability, pressure, flow, leak check). | - Identification of faulty cooling parts like radiator, pump, hoses, fans, and sensors. - Interpretation of diagnostic codes, warning lights, and ECU data related to thermal management faults. - SOPs and manufacturer guidelines for proper repair or replacement. - Fixing minor faults or replaces major parts like pump, radiator, or fans. - Conduct of test for stable temperature, pressure, and leaks after Refilling and bleed the system. Practical Activity - Diagnose, repair, or replace faulty cooling parts, refill coolant, and test system safely. | Theory-01Hrs Practical-04Hrs Total- 05Hrs | - Wrenches and sockets - Screwdrivers and pliers - Coolant pressure tester - Multimeter - Drain pan and funnel - Jack and stands - Torque wrench - Recommended coolant - Replacement parts (pump, hoses, radiator, fans, sensors) - Service manual - Cleaning cloths - Safety gloves and glasses | Class Room and workshop |
| - Replace the Coolant in The EV Cooling System. | - Drain the old coolant completely from the system (radiator, reservoir, hoses, cooling plates). - Inspect drained coolant for contamination, debris, or discoloration to detect underlying faults. - Flush the system (if required) to remove residues and contaminants before refilling. - Refill the system with the correct coolant type, concentration, and volume as per specifications. - Bleed the system to remove trapped air and ensure smooth coolant circulation. - Test the system operation (coolant flow, pressure, pump and fan function, temperature stability). - Check for leaks, abnormal noises, or warning lights after replacement. - Record coolant replacement procedure, coolant type/quantity used, and test results in maintenance logs. | - Techniques for Draining old coolant completely from the radiator, reservoir, hoses, and cooling plates. - Inspecting drained coolant for contamination, debris, or discoloration, and flush the system if needed. - Refilling with correct coolant type, concentration, and volume, then bleed the system to remove trapped air. - Testing the cooling system flow, pressure, pump, fans, temperature, and record results. Practical Activity - Drain the old coolant, hoses and connections, refill with recommended coolant, bleed air from the system, and verify proper operation while following safety procedures. | Theory-01Hrs Practical-03Hrs Total- 04Hrs | - Wrenches and screwdrivers - Drain pan and funnel - Coolant pressure tester - Jack and stands - Torque wrench - Pliers - Recommended coolant - Distilled water (if required) - Replacement hoses or clamps - Cleaning cloths - Service manual - Safety gloves and glasses | Class Room and workshop |
| - Evaluate the Performance Of EV Cooling System | - Identify standard performance parameters of the EV cooling system (coolant temperature range, flow rate, system pressure, fan speed, pump efficiency). - Inspect the coolant level, quality, and circulation to ensure compliance with manufacturer’s specifications. - Measure system temperature at key points (battery pack, inverter, motor, radiator) using sensors or diagnostic tools. - Test operation of the coolant pump, cooling fans, valves, and radiator under different load/operating conditions. - Interpret diagnostic trouble codes (DTCs), warning lights, and ECU/thermal management data. - Compare actual system performance with recommended specifications and thresholds. - Evaluate efficiency of heat dissipation under normal, partial load, and high-load conditions. - Detect abnormalities (overheating, insufficient cooling, air lock, weak pump, blocked radiator/hoses). | - Identification of standard performance parameters of the EV cooling system, including coolant temperature range, flow rate, system pressure, fan speed, and pump efficiency. - Inspection coolant level, quality, and circulation to ensure compliance with manufacturer specifications. - Measuring system temperature at key points (battery pack, inverter, motor, radiator) and test operation of pump, fans, valves, and radiator under different loads. - Interpreting diagnostic trouble codes, warning lights, and ECU/thermal management data, and compare actual performance with recommended specifications. - Evaluating heat dissipation efficiency and detect abnormalities such as overheating, insufficient cooling, air locks, weak pump, or blocked radiator/hoses. Practical Activity - Check and identify the cooling performance, coolant level, quality, and flow per specifications. | Theory-01Hrs Practical-03Hrs Total- 04Hrs | - Diagnostic scanner - Multimeter - Infrared thermometer - Coolant pressure tester - Multimeter - Basic hand tools (wrenches, pliers, screwdrivers) - Service manual - Recommended coolant - Replacement sensors or hoses (if needed) - Cleaning cloths - Safety gloves and glasses |  |

## 3.9 0716MSA1909 Maintain Electric Vehicle (EV) Transmission and Drive

Objective: This module covers the knowledge and  skills required to repair and maintain EV transmission and drive systems, follow manufacturer-recommended maintenance schedules, check and replace essential fluids such as transmission fluid, brake fluid, and battery coolant, perform routine tire inspections for wear and proper inflation, and inspect the brake system to ensure safety, efficiency, and overall performance of the EV.

Theory: 15 Hours				Practice:  40 Hours 					Duration:  55 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Identify the Types Of EV Transmission And Drive system | - Identify the main components of an EV transmission system (single-speed gearbox, reduction gear, differential, shafts, and bearings). - Recognize drive system components (electric motor, inverter, half-shafts, constant velocity (CV) joints, drive axles). - Differentiate between EV drive types (Front-Wheel Drive, Rear-Wheel Drive, All-Wheel Drive, In-Wheel Drive). - Classify transmission arrangements used in EVs (direct drive, single-speed reduction, multi-speed transmission in advanced EVs). - Locate electronic control units (Motor Control Unit, Transmission Control Module, sensors for torque, speed, and position). - Identify cooling and lubrication provisions for the transmission and drive system. | - Identification and location of the main components of an EV transmission and drive system. - Differentiation between EV drive types and transmission arrangements. - Identification of electronic control units and associated sensors in EV transmission systems. - Cooling and lubrication provisions for the EV transmission and drive system. Practical Activity: - Inspect and identify the main components of an EV transmission and drive system, including gearbox, differential, shafts, bearings, electric motor, inverter, half-shafts, CV joints, and drive axles. | Theory-02Hrs Practical-07Hrs Total- 09Hrs | - Vehicle service manual - Inspection tools (wrenches, screwdrivers, pliers) - Multimeter - Scan tool or diagnostic tablet - Jack and jack stand - EV drivetrain diagrams - Manufacturer specifications - Safety gloves and glasses - Marker and notepad for | Class Room and workshop |
| - Identify the Components of EV Transmissions and Drive system | - Identify the major components of an EV transmission, including gearbox, reduction gear, and differential. - Recognize drive system parts such as shafts, bearings, CV joints, and motor mounts - Interpret the role of the electric motor and inverter within the drive system. - Differentiate between mechanical and electrical elements of the transmission and drive. - Inspect components visually for wear, alignment, and secure connections. - Relate each component to its function in power transmission and vehicle movement. - Verify component identification with manufacturer diagrams, manuals, and specifications. | - Major components of an EV transmission and drive system. - Drive system parts and differentiate between mechanical and electrical elements. - Interpreting the role of each component in power transmission and vehicle movement. - Practical activity: - Use manufacturer diagrams, manuals, and specifications to verify correct identification and arrangement of transmission and drive system components. | Theory-03Hrs Practical-06Hrs Total- 09Hrs | - Basic hand tools (wrenches, screwdrivers, pliers) - Multimeter - Torque wrench - Jack and jack stands - Scan tool or diagnostic tablet - EV transmission and drive diagrams - Manufacturer’s service manual - Safety gloves and glasses - Marker and notepad for labeling and notes Socket and spanner set | Class Room and workshop |
| - Demonstrate Function Of The Inverter In EV System | - Identify the inverter as a key component of the EV power electronics system. - Recognize the main function of the inverter: converting DC power from the battery into AC power for the electric motor. - Demonstrate how the inverter controls motor speed and torque by varying voltage and frequency. - Interpret the inverter’s role in regenerative braking by converting AC from the motor back into DC - Differentiate between types of inverters used in EVs, such as voltage source inverters and current source inverters. - Monitor inverter performance using diagnostic tools and Interpret operational data. - Apply safety measures and manufacturer guidelines while handling or testing the inverter. | - Role of Inverter as a key component in the EV power electronics system. - Mechanism of inverter to controls motor operation and supports regenerative braking. - Types of inverters and their roles in EVs. - Monitoring and interpret inverter performance using diagnostic tools, following safety guidelines. Practical Activity: - Demonstrate how it converts DC from the battery into AC for the motor. - Use diagnostic tools to monitor inverter performance and interpret operational data. | Theory-02rs Practical-07Hrs Total- 09Hrs | - Multimeter - Oscilloscope - OBD-II scanner or diagnostic tablet - Insulation resistance tester - Basic hand tools (wrenches, screwdrivers, pliers) - EV inverter circuit diagram - Service manual - High-voltage safety gloves - Insulated mats and tools - Safety glasses - Notepad for readings and observations | Class Room and workshop |
| - Demonstrate the Role of Controller Area Network (CAN) in EVs | - Identify the purpose of the CAN bus in EVs for communication between electronic control units (ECUs). - Demonstrate how the CAN protocol transfers real-time data such as speed, torque, voltage, and temperature. - Demonstrate the process of connecting diagnostic tools to access CAN bus signals. - Interpret CAN messages and fault codes related to EV subsystems. - Evaluate the role of CAN in coordinating VCU, MCU, BMS, inverter, and other components. - Detect communication errors such as data loss, latency, or mismatched signals. - Apply safety and manufacturer guidelines while monitoring or troubleshooting CAN communication | - CAN bus as the primary communication system in EVs. - Purpose and function of the CAN bus in EVs for ECU communication. - Real-time data transfer between components by CAN protocol. - Demonstrating accessing, interpreting, and troubleshooting CAN bus signals and messages. - Evaluation of the role of CAN in coordinating EV subsystems while applying safety guidelines. Practical Activity: - Connect diagnostic tools to the CAN bus to access real-time signals. - Interpret CAN messages and fault codes related to VCU, MCU, BMS, inverter, and other EV subsystems | Theory-02Hrs Practical-06Hrs Total- 08Hrs | - CAN analyzer or scan tool - Digital multimeter - Oscilloscope - Laptop with diagnostic software - Basic hand tools (wrenches, screwdrivers, pliers) - CAN wiring diagram - Service manual - Safety gloves and glasses - Insulated tools - Notepad for data logging and analysis | Class Room and workshop |
| - Repair/Replace Faulty Components of EV Transmission and Drive | - Identify faulty components in the EV transmission and drive system through inspection and testing. - Interpret fault codes, diagnostic data, and performance symptoms such as noise, vibration, and overheating. - Select and use appropriate diagnostic tools, measuring instruments, and service equipment. - Follow manufacturer’s service manuals and standard operating procedures during repair or replacement. - Apply proper safety practices including high-voltage isolation, lockout/tag out, and PPE usage. - Remove, repair, or replace defective components such as gearbox, reduction gear, differential, shafts, CV joints, and bearings. - Reassemble and align transmission and drive system components according to OEM specifications. - Validate system performance after repair by conducting functional tests and comparing with manufacturer standards | - Identification of faulty components in the EV transmission and drive system using inspection, testing, and diagnostic data. - Identification of fault codes, performance symptoms, and selection of appropriate tools for diagnosis. - Manufacturer procedures and safety practices during repair or replacement of transmission and drive components. - Removal, repair, or replacement techniques of defective components such as gearbox, reduction gear, differential, shafts, CV joints, and bearings. - Reassembling, aligning, and validation of system performance through functional tests against OEM standards. Practical Activity: - Inspect and identify faulty components such as gearbox, reduction gear, differential, shafts, CV joints, and bearings. - Remove, repair, or replace defective components following manufacturer procedures and safety guidelines. | Theory-03Hrs Practical-07Hrs Total- 10Hrs | - Diagnostic scanner - Multimeter - Torque wrench - Socket and wrench set - Screwdrivers and pliers - Jack and stands - Transmission jack or hoist - Replacement parts (motor, gearbox, coupler) - Service manual - Lubricant or transmission oil - Electrical contact cleaner - Gloves, glasses, rags | Classroom and workshop |
| - Evaluate the Performance Of EV Transmission And Drive | - Identify performance parameters of EV transmission and drive systems such as efficiency, torque, noise, and vibration. - Select and use appropriate diagnostic tools, sensors, and measuring instruments. - Collect and interpret data from test runs and onboard diagnostic systems. - Compare measured values with manufacturer’s performance specifications. - Detect abnormalities including power loss, overheating, leakage, or unusual sounds. - Apply safe testing procedures while handling high-voltage and rotating components. | - Performance parameters of EV transmission and drive systems. - Selection and use of appropriate diagnostic tools, sensors, and measuring instruments. - Interpretation and Collection and data from test runs and onboard diagnostic systems. - Comparison of measured performance values with manufacturer specifications. - Detestation abnormalities such as power loss, overheating, leakage, or unusual sounds while applying safe testing procedures Practical Activity - Measure torque, efficiency, noise, and vibration using sensors and diagnostic tools. - Observe for abnormalities such as power loss, overheating, leakage, or unusual sounds. | Theory-03Hrs Practical-07Hrs Total- 10Hrs | - EV training vehicle or test vehicle - Diagnostic scanner or ECU reading tool - Wiring diagrams and powertrain layout charts - Service manuals and technical literature - Multimeter - Basic hand tools and Safety hand tools kits - Torque meter or dynamometer (if available) - PPE (insulated gloves, goggles, safety shoes) - Maintenance or training log sheets for recording observation | Class room and Workshop |

## 3.10.0716MSA1910 Maintain Electric Vehicle (EV) Brake System

Objective: This Module covers the skills and knowledge to maintain the EV's Brake System. This includes Inspection of  the Braking System For Fault Detection Through OBD-II Scanner for, Diagnose the Faults of the Anti-Lock Braking System, Perform Servicing/Repair Of The Hydraulic Brake System and Replace the faulty component of the Anti-Lock Braking System and electronic parking brakes .Always follow  manufacturer's maintenance schedule, but in general, have your brakes inspected regularly to check for rust due to infrequent use and to inspect brake pads and rotors. Change the brake fluid every two years, keep the moving parts clean and lubricated, and address any issues promptly to prevent costly repairs.

Theory: 15 Hours				Practice:  40 Hours 					Duration:  55 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Inspect The Braking System For Fault Detection Through OBD-II Scanner | - Identify braking system components monitored by the OBD-II scanner such as ABS, sensors, and actuators. - Connect the OBD-II scanner correctly and establish communication with the vehicle ECU - Retrieve and interpret fault codes related to the braking system. - Analyze live data parameters such as wheel speed, brake pressure, and ABS signals - Compare diagnostic results with manufacturer’s specifications to confirm faults. - Document inspection findings and recommend suitable repair or replacement actions. | - Identify braking components monitored by OBD-II. - Connect and communicate with vehicle ECU. - Retrieving and analyzing braking fault codes and live data. - Comparison of results with specs and recommend repairs. Practical Activity: - Perform visual inspection of ABS components, wiring, and connections. Sensors and actuators monitored and diagnostic with the scanner. | Theory-2Hrs Practical-6Hrs Total- 8Hrs | - OBD-II scanner - Diagnostic laptop or tablet - Multimeter - Basic hand tools (wrenches, screwdrivers, pliers) - Torque wrench Manufacturer’s service manual - Vehicle wiring diagram - Electrical contact cleaner | Class Room and workshop |
| LU2. Diagnose the Faults of the Anti-Lock Braking System | - Identify the major components of ABS (sensors, ECU, hydraulic unit, wiring). - Interpret ABS warning lights and diagnostic trouble codes using OBD-II scanner. - Inspect wheel speed sensors for alignment, resistance, and signal output. - Check wiring harnesses and connectors for continuity, short circuits, or damage. - Test the ABS hydraulic control unit for valve and pump operation. - Verify system performance through road testing under different braking conditions. - Apply manufacturer service data and standard procedures for accurate fault diagnosis. | - Identification of braking system components monitored by OBD-II. - Connection of OBD-II scanner with vehicle ECU. - Verification of ABS signals and brake pressure during diagnostics. - Interpretation of wheel speed sensor data for fault detection. Practical Activity: - Connect the OBD-II scanner to the vehicle ECU and identify ABS, sensors, and actuators monitored by the scanner. - Inspect the braking system physically, including ABS components and wiring. | Theory-2Hrs Practical-6Hrs Total- 8Hrs | - OBD-II scanner - Digital multimeter - Brake pressure gauge - Wheel speed sensor tester - Hydraulic line tester - Basic hand tools (wrenches, pliers, screwdrivers): - Manufacturer’s ABS wiring diagram - Service manual - Brake fluid | Class Room and workshop |
| LU3. Perform Servicing/Repair of The Hydraulic Brake System. | - Identify the components of the hydraulic brake system (master cylinder, brake lines, hoses, calipers, wheel cylinders, brake fluid, and reservoir). - Inspect brake components for wear, leakage, corrosion, or damage. - Test brake fluid level, quality, and contamination. - Perform brake bleeding to remove air from the hydraulic system. - Replace or service faulty components such as hoses, cylinders, or calipers. - Reassemble and refill the system with correct brake fluid type. - Verify brake system performance through functional and road testing. | - Components of the hydraulic brake system. - Inspection of brake components for wear, leakage, or damage. - Replacing or servicing faulty brake components. - Testing of brake fluid level, quality, and contamination. - Brake bleeding to remove air from the - Reassembling and refilling of the system with correct brake fluid type. Practical Activity: - Perform brake bleeding to remove air from the system. - Reassemble and refill the system with correct brake fluid type. | Theory-2Hrs Practical-7Hrs Total- 9Hrs | - Brake line wrench - Screwdrivers and pliers - Torque wrench - Brake piston compressor - Jack and jack stand - Drain pan - Brake fluid - Replacement brake pads or shoes - Brake hoses or lines (if damaged) - Seals and gaskets - Cleaning solvent - Rags and gloves - Service manual | Class Room and workshop |
| LU4. Replace the faulty component of the Anti-Lock Braking System. | - Identify faulty ABS components through diagnostic tools and fault codes. - Inspect sensors, wiring, ECU, and hydraulic unit for defects. - Apply safety measures before removing faulty ABS components. - Replace defective ABS components following manufacturer procedures. - Reconnect wiring, sensors, and hydraulic lines securely. - Test ABS function through diagnostic tools and road performance checks. | - Identification and inspection of faulty ABS components, sensors, wiring, ECU, and hydraulic unit - Replacement of defective ABS components following procedures. - Reconnecting system components and test ABS function using diagnostic tools and road checks. Practical Activity: - Test ABS function using diagnostic tools and perform road performance checks. | Theory-3Hrs Practical-7Hrs Total- 10Hrs | - OBD-II scanner - • Digital multimeter - Socket and wrench set - Screwdrivers and pliers - Torque wrench - Jack and jack stand - Brake bleeder kit - Replacement ABS component (sensor, pump, or module | Class Room and workshop |
| LU5. Replace the faulty component of the Electronic parking brake | - Identify the components of the electronic parking brake system. - Interpret EPB fault codes using diagnostic tools. - Inspect wiring harnesses, connectors, and actuators for faults. - Check the condition and function of EPB control module. - Safely remove faulty EPB components following manufacturer guidelines. - Install new or repaired components with correct calibration. - Perform system reset or reprogramming using diagnostic software. - Verify EPB performance through functional and safety tests. | - Components and wiring of the electronic parking brake (EPB) system. - Identification and interpretation of EPB fault codes using diagnostic tools. - Identification of faulty EPB actuators, modules, and connectors, and follow safe removal procedures. - Procedures to install, calibrate, and verify EPB performance using diagnostic software. Practical Activity: - Check and identify faulty EPB actuators, control modules, and connectors, and remove them safely. | Theory-3Hrs Practical-7Hrs Total- 10Hrs | - OBD-II scanner - Digital multimeter - Socket and wrench set - Screwdrivers and pliers - Torque wrench - Jack and jack stands - Brake caliper tool - Brake fluid (if hydraulic type) - Electrical contact cleaner - Service manual - Rags and gloves - Safety glasses |  |
| LU6. Perform test drive of the Brake System | - Check brake response time and pedal feel during operation. - Evaluate braking distance under normal and emergency conditions. - Observe for abnormal noises, vibrations, or pulling during braking. - Test ABS and electronic brake functions during controlled braking. - Record and compare results with manufacturer specifications. - Confirm overall brake system safety and reliability after testing. | - Evaluation of brake response time. - Recognition of abnormal noises, vibrations, or pulling during braking. - Testing of ABS and electronic brake functions. - Recording procedure of results and confirmation of overall brake system safety. Practical Activity: - Observe for abnormal noises, vibrations, or pulling during braking during test the electronic brake functions using controlled braking procedures. | Theory-2Hrs Practical-7Hrs Total- 10Hrs | - OBD-II scanner - Diagnostic laptop or tablet - Tire pressure gauge - Stopwatch - Notepad and pen - Service manual - Vehicle inspection checklist - Safety cones (for test area) - Personal protective equipment (gloves, safety glasses, seatbelt) - Assistant or observer (for data recording) | Class Room and workshop |

- List of Tools and Equipment

| S# | Tools and Equipment | Quantity |
| --- | --- | --- |
| 1 | Work Shop Basic Tool | 5 |
| 2 | Socket sets (metric & standard ) & Combination wrenches | 5 |
| 3 | Screwdrivers (flat, Phillips, Torx) | 15 sets each |
| 4 | Pliers set (needle nose, slip joint, locking | 15 Sets |
| 5 | Rubber/soft-faced mallet | 15 |
| 6 | Torque wrench | 15 |
| 7 | Pry bars and panel removal tools | 10 |
| 8 | Oil and fluid drain pans | 5 |
| 9 | Hydraulic 2 Post Lift | 3 |
| 10 | Jack stands | 20 |
| 11 | Hydraulic jack 5 Ton | 5 |
| 12 | Engine Hoist | 5 |
| 13 | Insulated hand tools (rated 1000V, VDE certified): | 5 |
| 14 | High Voltage (HV) safety tools: | 5 |
| 15 | HV insulating gloves (Class 0/00, tested) | 5 |
| 16 | Leather protectors for HV gloves | 5 Set |
| 17 | Insulation mats | 5 Sets |
| 18 | Safety helmet with face shield | 5 |
| 19 | Insulation resistance tester (mega ohmmeter) | 5 |
| 20 | Digital Multi-meter (True RMS, CAT III/IV rated, 1000V min) | 5 sets each |
| 21 | High Voltage probes for safe measurement | 5 sets each |
| 22 | OEM or universal scan tool/diagnostic computer with HEV/PHEV support | 5 |
| 23 | OBD-II scanner (advanced with hybrid module access) | 5 |
| 24 | Laptops | 3 |
| 25 | Oscilloscope (for signal and waveform analysis | 2 |
| 26 | Battery lifting equipment (insulated lifting straps, hoist if needed) | 3 |
| 27 | Coolant evacuation and refill kit (for battery cooling system) | 5 |
| 28 | Thermal imaging camera (for detecting overheating cells/modules) | 5 |
| 29 | Alignment and puller sets (for couplings, bearings, pulleys) | 5 |
| 30 | Dial gauge (for backlash/run-out measurement) | 5 |
| 31 | Arc flash protective clothing (FR-rated suit if working on live HV parts) | 5 |
| 32 | Safety shoes (EH-rated, anti-static) | 25 Sets |
| 33 | Safety goggles/face shield | 25 Sets |
| 34 | Ear protection (if using power tools) | 25 Sets |
| 35 | Lockout/Tag out (LOTO) kit for HV systems | 5 |
| 36 | Insulated covers for HV connectors/cables | 5 |
| 37 | Spill kit (for coolant or electrolyte leakage) | 5 |
| 38 | Fire extinguisher (Class C for electrical fires) | 5 |

# 5- List 0f Consumable Materials.

| S. No | Consumables | Quantity |
| --- | --- | --- |
| 1 | Insulation tape (PVC, HV grade) | 2 Sets |
| 2 | Heat shrink tubing | 4Sets |
| 3 | Coolant fluid (EV-specific) | 4 Ltr. |
| 4 | Electrical connectors, terminals and Fuses | 4 Kits |
| 5 | Lubricants (grease, anti-seize, thread locker) | 2/Each |
| 6 | Cleaning solvents (HV-safe, non-conductive) | 2 litr. |
| 7 | Hose clamps & seals | 5/Each |
| 8 | Electrical Vehicle Battery Controller, | 3 |
| 9 | Solder wire & flux | 15 Foots |
| 10 | Transmission element | 10 |
| 11 | Fuse Different Values | 15L |
| 12 | Distilled water | 20 |
| 13 | EV Battery Cooling Filters (for battery air intake ducts) | 5 |
| 14 | Kerosene oil | 20L |
| 15 | Inverter & Battery Coolant (special hybrid coolant, e.g., Toyota SLLC, Honda Type-2) | 3 / Each |
| 16 | Transmission Fluid (CVT, ATF, Hybrid fluid) | 20 L |
| 17 | Sealing gaskets & O-rings (battery pack, inverter housing, cooling system | 3 Kits |
| 19 | Nuts, bolts, washers (assorted sizes) and Cable ties (zip ties) | 2 Kits |
| 20 | Face masks | 50 |
| 21 | Cleaning cloths & wipes | 10 Kg |
| 22 | Rubber gloves (disposable) | 50 |
| 23 | Brake fluid (DOT 3 / DOT 4, depending on OEM) | 5/Each |
| 24 | Marker pens & labels | 10/Each |
| 25 | Log sheets / checklists | 10 Sets |
| 26 | Coolant (engine cooling loop & inverter cooling loop may differ) | 4 L |
| 27 | Refrigerant (R134a / R1234yf depending on vehicle) | 8 kg |
| 28 | Small relays (training use) | 10 |
| 29 | Resistors, capacitors (assorted) | 10 |
| 30 | Silicon tube | 5 |
| 31 | 12V auxiliary battery (AGM or Li-ion depending on model | 1 |
| 32 | Thermostat valve | 1 |
| 33 | High-voltage warning labels/stickers | 10 |
| 34 | Radiator Fan motor | 2 |
| 35 | Hose clips, zip-lock bags | 2 |
| 36 | Copper lugs & crimp sleeves | 2+2 |
| 37 | ABS wheel sensor | 4 |

# 6- Members of Qualification Development & Review Committee

The following members participated in the qualification development and review process:

| S. # | Name |  | Designation |  |  | Organization |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Engr. Abdul Maqsood |  | DACUM Facilitator |  |  | Ex- Principal, KP-TEVTA |
| 2 | Mr. Muzammal Hussain |  | Auto Instructor |  |  | (P-TEVTA) Rahim Yar Khan |
| 3 | Mr. Mian Abdul Atique |  | HEV & EV Technician Expert, |  |  | Honda Cars Pvt. |
| 4 | Mr. Sajjas Ali Miraj |  | Auto Instructor |  |  | GTTI, Depalpur |
| 5 | Mr. M. Arham Mehfooz |  | Electrical Data Engineer |  |  | Vision Developers, Lahore |
| 6 | Mr. Khawaja Muhammad Umar |  | Tech Manager, Resources |  |  | Lahore |
| 7 | Mr. Saddam Hussain |  | Trade Instructor |  |  | GCT-Tangi Charsada KPK=TEVTA |
| 8 | Mr. Altaf Khan |  | Lecturer Auto |  |  | GCT,Peshawar |
| 9 | Mr, Muhammad Boota Saleem |  | Auto Insructor |  |  | VTI-Okara (PVTC) |
| 10 | Miss.Areej |  | Assistant Director |  |  | NAVTTC H/Q’s, Islamabad |
| Qualification Review Committee |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |