National vocations certificate Level -3 in “Electronics Technology (Electronics Technician)”

National Vocational and Technical Training Commission (NAVTTC),

Government of Pakistan

# ACKNOWLEDGEMENTS

National Vocational and Technical Training Commission (NAVTTC) extends its gratitude and appreciation to many representatives of business, industry, academia, government agencies, Provincial TEVTAs, Sector Skill Councils and trade associations who speared their time and expertise to the development and validation of these National Vocational Qualifications (Competency Standards, Curricula, Assessments Packs and related material). This work would not have been possible without the financial and technical support of the TVET Sector Support Programme co-funded by European Union, Germans and Norwegian and German Governments implemented by GIZ Pakistan. NAVTTC is especially indebted to Dr. Muqeem ul Islam, who lead the project from the front. The core team was comprised on:

- Dr. Muqeem ul Islam, Director General (Skills Standards and Curricula) NAVTTC

- Mr. Muhammad Naeem Akhtar, Senior Technical Advisor TSSP-GIZ,

- Mr. Muhammad Yasir, Deputy Director (SS&C Wing) NAVTTC

- Mr. Muhammad Ishaq, Deputy Director (SS&C Wing) NAVTTC

- Mr. Muhammad Fayaz Soomro, Deputy Director (SS&C Wing) NAVTTC

NAVTTC team under the leadership of Dr. Muqeem ul Islam initiated development of CBT & A based qualifications of diploma level-3 as a reform project of TVET sector in November 2018 and completed 27 NVQF diplomas of Level-5 in September, 2019. It seems worth highlighting that during this endeavor apart from developing competency standards/curricula in conventional trades new dimensions containing high-tech trades in TVET sector in the context of generation IR 4.0 trades have also been developed which inter alia includes Robotics, Mechatronics, artificial intelligence, industrial automation, instrumentation and process control. Moreover, trades like entrepreneurship, green/environmental skills and variety of soft/digital skill have also been developed to equipped the Pakistani youth with skills set as per requirement of the global trends. These skills have been made integral part of all the 27 diplomas.

Nobody has been more important in the pursuit of this project than Dr. Nasir Khan, Executive Director, NAVTTC, whose patronage and support remain there throughout the development process and lastly to thanks specially to Syed Javed Hassan Chairman NAVTTC and Raja Saad Khan, Deputy Team Lead TSSP-GIZ who made it happened in this challenging time.

CONTENTS

Contents

1.	INTRODUCTION	6

2.	PURPOSE OF THE QUALIFICATION	7

3.	COMMON COURSES WITH RESPECTIVE LEVELS	7

4.	MAPPING OF THE QUALIFICATION	9

5.	DATE OF VALIDATION	10

6.	DATE OF REVIEW	10

7.	CODE OF QUALIFICATION	10

8.	QUALIFICATIONS DEVELOPMENT COMMITTEE	10

9.	QUALIFICATION VALIDATION COMMITTEE	12

10.	ENTRY REQUIREMENTS	13

11.	OCCUPATION PROFILE LEVEL PACKAGING CHART	13

12.	PACKAGING OF QUALIFICATION	14

13.	SUMMARY OF COMPETENCY STANDARDS	16

Adopt Safety Regulations, Labour Protection Laws, Environmental Protection Laws at Workplace	16

Create User Documentation	20

Create Technical Documentation	21

Create Basic Databases	23

Operate Digital Media Technology	25

Use Social Media Tools for Collaboration and Engagement	26

Use Digital Devices	27

Carryout Basic Programming	30

Perform Basic Mathematical Calculations in C++	31

Identify Basic Electronics Components	34

Use digital instruments to test the logic signal	36

Use different instruments to measure the parameters of electrical signals &components	37

Convert galvanometer into ammeter, voltmeter, Ohm meter & calibrate them	40

Measure Electrical power, Energy & Power Factor	43

Design a Rectifier using Diode	46

Implement Bipolar Junction Transistor (BJT) in Different Applications	47

Implement Field Effect Transistor (FET) in Different Applications	49

Carry out thyristor Implementation (Uni Junction Transistor, Silicon Control Rectifier, Diac and Triac) in electronics circuits.	51

Design & Assemble a Printed Circuit Board	53

Verify Ohm’s Law & Kirchhoff’s Law by `implementing Series & Parallel circuits	56

Implement Electromagnet to observe various effects & verify Faraday’s law	58

Measure different parameters of AC signal & Components	61

Install Single Phase Complex Electrical Wiring	63

Install Three Phase Complex Electrical Wiring	65

Overhaul the Lead Acid Batteries & implement their series & Parallel circuits	68

# INTRODUCTION

The Technical and Vocational is a profession that is increasingly getting attention in Pakistan, not only among the youth seeking to enter the industry but also among adults who wish to polish their skills to develop a career out of it. Scope and demand of Electronics engineers, Supervisors, Operators and Technicians is now increasing day by day in the industry. This qualification is designed so as to inculcate the competencies of basic design and construction of electronic circuits, identification and implementation of certain electronic components, linkage between the components and their industrial workability. The fundamental concepts of digital systems, the practical aspects of digital system design will be covered using hardware which is widely used in industry. Students will gain hands on experience in designing, Servicing and implementing a number of digital systems.

Electronics Diploma is further sub-divided into many other fields including electronics, computer engineering, power engineering, telecommunications, control systems, and microelectronics. Many of these sub disciplines overlap with other engineering branches, spanning a huge number of specializations such as hardware engineering, power electronics, avionics, mechatronics and nanotechnology.

The National Skills Strategy (NSS) aims at achieving a paradigm shift from time-bound   and supply led to competency-based and demand driven training in Pakistan. Competency-based training approach focuses on the demonstration of actual skills required in the workplace/industry. To achieve this strategy, NAVTTC in collaboration with GIZ is involved in the development process of CBT program to ensure competent skilled labor in demand driven vocational trades.

- National Vocational Qualification Framework (NVQF) as designed in consultation with the stakeholders including academia, researchers, industry, chambers and TEVTAs. Its aim is to identify knowledge & skills required for a qualification, determine equivalence, provide guidelines for Recognition of Prior Learning (RPL) and assure quality of training. NVQF designed for the Vertical and horizontal progression for learners, recognition of qualification, meeting with the national and international standards, facilitate conversion of informal training to formal training through RPL, improvement in quality of training and increased options for learners for selecting training programs in different trades. Competency Standards are performance specification that identify the knowledge and competencies an individual need to succeed in the workplace.

# PURPOSE OF THE QUALIFICATION

This qualification will equip the students with knowledge of how the industry works, as well as giving them the technological knowledge and technical skills which are needed to design, assess and improve electrical and electronic systems.

This qualification will help candidate to design and construct the electronic circuits, identify and implement certain electronic components, elaborate linkage between the components and their industrial workability & maintenance of various electronic devices.

# COMMON COURSES WITH RESPECTIVE LEVELS

LEVEL 3

LEVEL 2

LEVEL 4

LEVEL 5

# MAPPING OF THE QUALIFICATION

Power Electronics…..

Analog Electronics

Digital Electronics

Wiring

AC DC Machines…

Process Variable Measurements….

Microcontrollers

Analog Electronics

Digital Electronics

Soft Skills

Digital Skills

Entrepreneurship

# DATE OF VALIDATION

These national qualifications have been validated by the Qualification Development Committee (QVC) on 27-29 May, 2019 in Lahore and will remain valid till 29-May-2029.

# DATE OF REVIEW

These national qualifications shall be reviewed by the validation committee on 30-May-2022.

# CODE OF QUALIFICATION

The International Standard Classification of Education (ISCED) is a framework for assembling, compiling and analyzing cross-nationally comparable statistics on education and training. ISCED codes for these qualifications are assigned as follows:

| ISCED Classification for Electrical Technology level 5 |  |
| --- | --- |
| Code | Description |
| 0714E&A(1) | 1st Level D.A. E National Certificate of level-5, in “ Electronics Technology” |
| 0714E&A(2) | 2nd Level D.A. E National Certificate of level-5, in “Electronics Technology” |
| 0714E&A(3) | 3rd Level D.A. E National Certificate of level-5, in “Electronics Technology” |
| 0714E&A(4) | 4th Level D.A. E National Certificate of level-5, in “Electronics Technology” |
| 0714E&A(5) | 5th Level D.A. E National Certificate of level-5, in “Electronics Technology” |

# QUALIFICATIONS DEVELOPMENT COMMITTEE

The Qualifications Development Committee consisted of following members

| Name & Designation | Organization |
| --- | --- |
| Mr. Muhammad Yasir, AD NAVTTC | NAVTTC |
| Mr.Ayub Nasir | DACCUM Facilitator |
| Mr. Raheel Ahmed, Lecturer | COMSATS University Islamabad |
| Mr. Muhammad Aleem, Researcher | PAEC, DG Khan |
| Mr. Noman Khan, Senior Engr | COMCEPT Electronics |
| Mr. Adeel Iqbal, Lecturer | COMSATS University Islamabad |
| Mr. Tahir Muhammad Khan, Design Engineer | CARE , Islamabad |
| Mr. Hamza Ikhlaq, Lecturer | ISRA University, Islamabad |
| Mr. Imran Ahmed, Instructor | GCT Rawalpindi |
| Mr. Mudassar Ali, Instructor | Swedish Group of Colleges |
| Mr. SardarHussain, Instructor PLC | PITAC, Peshawar |
| Mr. Abdul BasitTaj, Instructor | NIE Islamabad |
| Mr. ShehzadRaza, Lab Designer | NSETS Pvt. LTD |
| Ms.Siqa,Instructor | P-TEVTA |
| Mr.AliAnwer Shah, assistant Professor | S-TEVTA |

# QUALIFICATION VALIDATION COMMITTEE

The following members participated in the qualification’s development workshop 27-29 May 2019 in Lahore.

| Name & Designation | Organization |
| --- | --- |
| Ms. Bushra Asghar DACCUM Facilitator | P-TEVTA |
| M. Hamza Akhlaq, Lecturer | Isra University, Islamabad. |
| Eng. Syed Ali Anwar Shah, Assistant Professor | S-TEVTA |
| Mst.Saiqa Ijaz, Instructor (GCT) | P-TEVTA |
| F.M.Kayfi, Chairman | PBTE |
| Fazal-E-Elahi,Asst.prof. | KP-TEVTA |
| Amir Amin, HOD Electrical. | City Poly Tech. Inst. |
| Aamir Ali Khan, Asst. Director, NAVTTC | NAVTTC |
| Muhammad Yasir, Asst. Director, NAVTTC | NAVTTC |

# ENTRY REQUIREMENTS

Entry requirements of this qualification are Matric Science or level 4 or matric equivalent

# OCCUPATION PROFILE LEVEL PACKAGING CHART

# PACKAGING OF QUALIFICATION

| Competency Standards | NVQF Level | Category | Estimated Contact Hours |  |  | Credit Hours |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Theory | Practical | Total |  |
| National Vocational Certificate Level 3 in Electronics Technology (Technician) |  |  |  |  |  |  |
| Adopt Safety Regulations, Labour Protection Laws, Environmental Protection Laws at Workplace | Level 3 | Functional | 3 | 12 | 15 | 1.5 |
| Create User Documentation | Level 3 | Generic | 4 | 6 | 10 | 1 |
| Create Technical Documentation | Level 3 | Generic | 2 | 9 | 11 | 1.1 |
| Create Basic Databases | Level 3 | Generic | 2 | 12 | 14 | 1.4 |
| Operate Digital Media Technology | Level 3 | Generic | 2 | 6 | 8 | 0.8 |
| Use Social Media Tools For Collaboration And Engagement | Level 3 | Generic | 4 | 6 | 10 | 1 |
| Use Digital Devices | Level 3 | Functional | 3 | 9 | 12 | 1.2 |
| Carryout Basic Programming | Level 3 | Technical | 2 | 18 | 20 | 2 |
| Perform basic Mathematical calculations in C++ | Level 3 | Technical | 6 | 24 | 30 | 3 |
| Identify Basic Electronics Components | Level 3 | Technical | 4 | 12 | 16 | 1.6 |
| Use digital instruments to test the logic signal | Level 3 | Technical | 5 | 12 | 17 | 1.7 |
| Use different instruments to measure the parameters of electrical signals & components | Level 3 | Technical | 5 | 12 | 17 | 1.7 |
| Convert galvanometer into ammeter, voltmeter, Ohm meter & calibrate them | Level 3 | Technical | 3 | 15 | 18 | 1.8 |
| Measure Electrical power, Energy & Power Factor | Level 3 | Technical | 4 | 18 | 22 | 2.2 |
| Carry out Diode Application | Level 3 | Technical | 4 | 18 | 22 | 2.2 |
| Design a Rectifier using Diode | Level 3 | Technical | 6 | 24 | 30 | 3 |
| Implement Bipolar Junction Transistor (BJT) in Different Applications | Level 3 | Technical | 4 | 21 | 25 | 2.5 |
| Implement Field Effect Transistor (FET) in Different Applications | Level 3 | Technical | 2 | 21 | 23 | 2.3 |
| Implement Thyristor Family in Various Application | Level 3 | Technical | 5 | 18 | 23 | 2.3 |
| Applications of Operation Amplifier | Level 3 | Technical | 4 | 21 | 25 | 2.5 |
| Design & Assemble the Printed Circuit Board | Level 3 | Technical | 6 | 30 | 36 | 3.6 |
| Verify Ohm’s law & Kirchhoff’s law by implementing their series and parallel circuits | Level 3 | Technical | 5 | 30 | 35 | 3.5 |
| Implement Electromagnet to observe various effects & verify Faraday’s law | Level 3 | Technical | 4 | 24 | 28 | 2.8 |
| Measure different parameters of AC signal & Components | Level 3 | Technical | 3 | 18 | 21 | 2.1 |
| Install Single Phase Complex Electrical Wiring | Level 3 | Technical | 8 | 27 | 35 | 3.5 |
| Install Three Phase Complex Electrical Wiring | Level 3 | Technical | 6 | 30 | 36 | 3.6 |
| Overhaul the Lead acid batteries & Implement their series & Parallel circuits | Level 3 | Technical | 5 | 36 | 41 | 4.1 |
| Total |  |  | 111 | 489 | 600 | 60 |

# SUMMARY OF COMPETENCY STANDARDS

### Adopt Safety Regulations, Labour Protection Laws, Environmental Protection Laws at Workplace

Overview :

After this Competency Standard, the Trainee will be able to develop skill and competence required to maintain Occupational Safety, Health and Environment at the workplace according to the National and International Protection Agencies Standards and take remedial measures for Personal, Occupational and Environmental Protection. The Trainee will be able to deal with the emergencies in a professional manner, thus minimizing the losses and providing a safe and healthy working environment.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1: Implement International Safety Standards in your work environment | - Recognize Electrical Safety hazards as per International Electro- Technical Commission (IEC) Standards - Determine Environmental Pollution risk factors as per Protection Agency (EPA) standards - Identify Electrical Safety Hazards as per Institute of Electrical and Electronics Engineers (IEE) standards - Categorize the Electrical Safety Hazards as per Electrical Safety Foundation International(ESFI) standards - Identify Labor Protection Laws as per International Labor Organization(ILO) rules - Identify the steps to minimize the Electrical hazards and Environmental Pollution. - Prepare a report for all the above activity. |
| CU2: Implement National Safety Standards in your work environment | - Identify Factory associated hazard as per Chapter 3 of Factories Act, 1934 - Determine Environmental Pollution factors as per Pakistan Environmental Protection Act, 1997 - Recognize the Labor protection laws as per Labor Protection Policy 2006 - Identify the workplace hazards as per Occupational health and safety (OHS) standards - Identify the steps to minimize the Electrical hazards, Environmental Pollution and Labor Safety - Prepare a report for all the above activity. |
| CU3: Implement International and National Labor Protection Laws | - Identify Labor Protection Laws as per International Labor Organization(ILO) rules - Recognize the Labor protection laws as per Labor Protection Policy 2006 - Identify the Bonded Labor and Child Labor policy. - Determine the leaves policy and compensation policy for the Labor. - Recognize the minimum wage for the Labor - Identify the remedial steps for protection and prosperity of Labor. - Prepare a report for all the above activity. |
| CU4: Implement National and International Environmental protection laws | - Determine Environmental Pollution risk factors as per Protection Agency (EPA) standards - Identify the steps to minimize the Electrical hazards and Environmental Pollution. - Determine Environmental Pollution factors as per Pakistan Environmental Protection Act, 1997 - Identify the requirements for Initial Environmental Examination (IEE) - Identify the requirements for Environmental Impact Assessment (EIA) - Prepare a report for all the above activity. |
| CU5: Prepare for emergencies | - Take emergency response training - Ensure practice of emergency exercises - Ensure the availability of first aid box and fire extinguisher - Check the expiry of medicines and fire extinguishers - Check the emergency alarms - Ensure regular practice of gathering the workers in assembly area during the emergency. |
| CU6: Respond to emergencies | - Follow emergency plan - Communicate instructions to co workers - Assess risk and determine course of action - Operate emergency equipment and supplies - Ensure that the ambulance and fire brigade is at stand by (for major emergency) |
| CU7: Adopt company policies and procedures | - Ensure company’s safety policy - Adopt company safety procedure - Advocate worker with company safety policy - Implement Safety sign board as per standard |

Knowledge & Understanding

- Factors affecting Health & Safety in the workplace.

- Personal Protective Equipment (PPE)

- First-Aid-Box.

- Emergency medicines and expiry

- Methods of treatment against electric shock

- Methods of treatment against minor injuries

- Types of Fire Extinguisher

- Uses of Fire Extinguisher

- IEC/EPA/IEE/ESFI Standards

- Factories Act 1934

- Pakistan Environmental Protection Act, 1997

- Occupational health and safety (OHS) standards

- Labor Protection Policy 2006

- Company policies and procedures

- Understand various safe guards

- Safety measures

- Work permit/no objection certificate(NOC)

- Types of work site Hazards

- Hazardous chemical control procedures

- Methods of first aid cardio respiratory Procedure

- Types of emergencies

- Response various types of emergencies

- Emergency equipment, supplies and their operation

- Methods of communication during emergency

- Factors affecting Health & Safety in the workplace.

- Personal Protective Equipment (PPE)

- First-Aid-Box.

- Emergency medicines and expiry

- Methods of treatment against electric shock

- Methods of treatment against minor injuries

- Types of Fire Extinguisher

- Uses of Fire Extinguisher

- IEC/EPA/IEE/ESFI Standards

- Factories Act 1934

- Pakistan Environmental Protection Act, 1997

- Occupational health and safety (OHS) standards

- Labor Protection Policy 2006

- Factors affecting Health & Safety in the workplace.

- Personal Protective Equipment (PPE)

- First-Aid-Box.

- Emergency medicines and expiry

- Methods of treatment against electric shock

- Methods of treatment against minor injuries

- Types of Fire Extinguisher

- Uses of Fire Extinguisher

- IEE/EIA Standards

- Factories Act 1934

- Pakistan Environmental Protection Act, 1997

- Occupational health and safety

- OHS) standards

- Labor Protection Policy 2006

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | Respiratory mask |
| 2 | First Aid Box |
| 3 | First Aid Kit |
| 4 | Stretcher |
| 5 | Fire Buckets |
| 6 | Fire Extinguisher |
| 7 | Emergency Alarm/Bell |
| 8 | Fall Protection Plan |
| 9 | International Safety Standards Manual |
| 10 | Emergency response Plan |
| 11 | National Safety Standards Manual |
| 12 | Plan |
| 13 | National Safety Standards Manual |
| 14 | WHMIS Handbook |
| 15 | International Safety Standards Manual |
| 16 | Safety shoes |
| 17 | Hearing protection |
| 18 | Safety goggles |
| 19 | Emergency response |
| 20 | Emergency Alarm/Bell |
| 21 | Fire Extinguisher |
| 22 | Fire Buckets |
| 23 | Stretcher |
| 24 | First Aid Box |
| 25 | Respiratory mask |
| 26 | First Aid Kit |
| 27 | Protection suite |
| 28 | Protection suite |
| 29 | Hearing protection |
| 30 | WHMIS Handbook |
| 31 | National Safety Standards Manual |
| 32 | International Safety Standards Manual |
| 33 | Fall Protection Plan |
| 34 | Emergency response Plan |
| 35 | Emergency Alarm/Bell |
| 36 | Fire Extinguisher |
| 37 | Fire Buckets |
| 38 | Stretcher |
| 39 | First Aid Kit |
| 40 | Respiratory mask |
| 41 | First Aid Box |
| 42 | Safety shoes |
| 43 | Safety goggles |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Types of hazards that are most likely to cause harm to health and safety

- Health and safety precautions

- Health and safety signs and symbols

- Techniques and methods to identify the risks of hazards at workplace

- Dealing with hazards to avoid any accident or injury

- Safety reporting procedures and documentation

- Describe fire-fighting methods

- Demonstrate use of appropriate Personal Protective Equipment (PPE)

### Create User Documentation

Overview :

This unit describes the performance outcomes, skills and knowledge required to create user documentation that is clear to the target audience and easy to navigate.

| Unit of Competency | Performance Criteria |
| --- | --- |
| Determine documentation standards and requirements | - Determine documentation requirements - Investigate documentation and industry standards for requirements and determine appropriate application to user documentation - Design documentation templates using appropriate software and obtain approval from appropriate person |
| Produce user documentation | - Conduct a review of the subject system, program, network or application in order to understand its functionality - Gather existing technical, design or user specifications and supporting documentation - Create user documentation based on template to record the operation of the subject system, program, network or application |
| Review and obtain sign-off | - Submit user documentation to target audience for review - Gather and analyze feedback - Make changes to user documentation - Submit user documentation to appropriate person for approval |

Knowledge and understanding

- Content features, including clarity and readability

- Document design, web design and usability

- Functions and features of templates and style guides

- Instructional design principles

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to create user documentation that:

- Meets business requirements

- Caters for a diverse readership

- Is clear to the target audience

- Is easy to navigate.

### Create Technical Documentation

Overview :

This unit describes the performance outcomes, skills and knowledge required to create technical documentation that is clear to the target audience and easy to navigate.

| Unit of Competency | Performance Criteria |
| --- | --- |
| Identify and analyze documentation requirements and client needs | - Consult with client to identify documentation requirements - Interpret and evaluate documentation requirements and confirm details with client - Investigate industry and documentation standards for requirements - Define and document the scope of work to be produced - Consult with client to validate and confirm the scope of work |
| Design documentation | - Identify information requirements with reference to layout and document structure - Create document templates and style guides consistent with information requirements - Conduct a review of the system in order to understand its functionality - Extract content that meets information requirements according to copyright restrictions - Develop the structure of the technical documentation giving focus to the flow of information, style, tone and content format - Validate the technical documentation structure with the client |
| Develop documentation | - Write technical documentation based on the template and scope of work using the information gathered - Translate technical terminology into plain English where appropriate - Apply content format and style according to documentation standards and templates |
| Evaluate and edit documentation | - Submit technical documentation to appropriate person for review Gather and analyze feedback - Incorporate alterations into the technical documentation - Edit the technical documentation for technical and grammatical accuracy |
| Prepare documentation for publication | - Check that the completed technical documentation meets client requirements and scope of work - Submit the technical documentation to appropriate person for approval - Prepare the technical documentation for publication and distribution using appropriate channels |

Knowledge and understanding

- Content features, such as clarity and readability

- Document design, web design and usability

- Functions and features of templates and style guides

- Instructional design principles

- Organizational policies, procedures and standards that cover document design.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

- Establish customer needs

- Design and develop technical documentation, such as system, procedures, training material and user guides, incorporating appropriate standards

- Update document with client feedback

- Prepare documentation for publication.

### Create Basic Databases

Overview :

This unit describes the skills and knowledge required to design, develop and test a database in order to meet a specification. It applies to individuals who may be either database, or web designers, required to create a simple database to store information for an online application, using a simple entity relational database.

| Unit of Competency | Performance Criteria |
| --- | --- |
| Analyze the requirements for the database | - Determine the information that the database is required to hold - Develop a written requirement report for the functionality of the database - Complete the documentation, and submit it to the appropriate person for approval |
| Use data modeling to design the database to suit requirements | - Design an entity-relationship (ER) diagram to model the relationships between the entities and the attributes that the database will hold - Develop primary and foreign keys to link the entities - Develop a data dictionary - Complete the documentation, and submit it to the appropriate person for approval |
| Create a database on a web or database server | - Use the appropriate language on a web or database server to create one or more databases - Use the appropriate language on a web or database server to create tables - Populate the database fields |
| Test the database and debug | - Test the database on the web or database server - Ensure that the information represented matches the requirements |

Knowledge and understanding

- outline the principles of open platforms, including browsers and databases

- list the processes associated with the creation of entities, attributes, and I populating fields, using both software solutions and script- based input

- describe data-modeling techniques to design a database

- outline the steps in database design, modeling and implementation

- describe the internet operation related to web servers and clients

- identify the naming conventions appropriate to database design

- identify security restrictions on servers, incorporating some theoretical concepts

- Describe best practice communication, and accessibility, for audiences with special needs.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

- research client requirements for a database solution

- design a database that meets client requirements

- create a database on a web hosting service or server to meet client requirements by a due date

- Test and debug the database.

### Operate Digital Media Technology

Overview :

This unit describes the performance outcomes, skills and knowledge required to identify, select and use a digital media package and supporting technologies.

| Unit of Competency | Performance Criteria |
| --- | --- |
| Use appropriate OHS office work practices | - Use safe work practices to ensure ergonomic, work organization, energy and resource conservation requirements are addressed - Use wrist rests and document holders where appropriate - Use monitor anti-glare and radiation reduction screens where appropriate |
| Identify and select appropriate digital media package | - identify the basic requirements of a design brief, including user environment - Research and review suitable available digital media packages - Select an appropriate digital media package to meet design brief requirements |
| Use digital media package | - Procure or create suitable data to meet requirements of the brief - Manipulate data using digital media package tools - Ensure naming and storing of documents in appropriate file format in directories or folders |
| Review digital media design | - Evaluate design for creative, dramatic and technical quality, file size, and suitability to meet the brief - Test and run any incorporated graphics, video or sound as part of a digital media presentation and present designs in the appropriate format - Review final product against design brief |

Knowledge and understanding

- Basic principles of visual design

- Functions and features of digital media packages and technologies

- Graphic design and stylistic language conventions

- OHS principles and responsibilities for ergonomics, such as work periods and breaks

- Principles of digital imaging and file formats, video and sound file formats, file management and transfer systems

- Vendor product directions in digital media hardware and software

- Visualization and interpreting creative information, scripts (text) and images

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence of the ability to:

- Identify basic requirements of a design brief

- Use digital media package to meet organizational requirements

- Use OHS principles and responsibilities for ergonomics, such as work periods and breaks

- Use help manuals and online help when appropriate

- Use digital media technologies to support design brief requirements.

### Use Social Media Tools for Collaboration and Engagement

Overview :

This unit describes the performance outcomes, skills and knowledge required to establish a social networking presence using social media tools and applications. The unit specifically identifies the requirement to review, compare and use different types of social networking tools and applications.

| Unit of Competency | Performance Criteria |
| --- | --- |
| Describe different types of social media tools and applications | - Explain characteristics of the term social media - Identify different types of social-media tools and applications - Illustrate some of the issues associated with the use of social media tools and applications |
| Compare different types of social media tools and applications | - Select one social media type for review - Review most popular tools and applications within that social media type - Itemize benefits across a range of the most popular tools and applications - Select most appropriate social media tool or application |
| Set up and use popular social media tools and applications | - Identify social media tools and applications for possible implementation - Initiate preferred social media tools and applications for use - Establish social media interface using text and file content - nitiate social networking interaction - Test and evaluate tools and applications for ease of use - Present findings |

Knowledge and understanding

- Basic technical terminology in relation to social networking and social media applications and tools

- Basic knowledge of uploading images, text files, pdf files, audio files, video files and link associated files

- Features and functions of social media applications

- Import and export software functions

- Linking documents

- OHS principles and responsibilities for ergonomics, including work periods and breaks

- Tagging to facilitate collaborative folksonomy

- Social media applications and procedures for connecting to social networking sites

- Use of input and output devices

- Use of RSS feeds to connect a social network.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

- Establish customer needs

- Design and develop technical documentation, such as system, procedures, training material and user guides, incorporating appropriate standards

- Update document with client feedback

Prepare documentation for publication.

### Use Digital Devices

Overview :

This unit describes the skills and knowledge required to use a range of digital devices, such as a digital camera, video camera, or personal digital assistant (PDA) device. It applies to individuals who require entry level information and communications technology (ICT) knowledge and literacy skills to support their work in a home office or small office environment.

| Unit of Competency | Performance Criteria |
| --- | --- |
| Prepare to use the digital device | - Review the instruction manual and ensure identified components are available - Identify the physical components of the digital device - Turn on and follow access procedures to activate the digital device - Alter the digital device settings to best suit intended use - 1Configure power management settings where appropriate to minimize power consumption, as an environmentally sustainable measure |
| Set up and use the digital device | - Identify and set the basic operating, security and menu settings - Navigate and manipulate the screen environment - Customize screen icons and access to applications where applicable - Use the digital device, and save and edit output where applicable - Identify more advanced features available and use as required |
| Access and use basic connectivity devices | - Connect to external digital devices, such as computer devices or storage devices, to retrieve, copy, move and save information - Check physical connectivity of computer devices or storage devices to ensure operation and performance - Connect to a printer either through a computer device or directly, and use printer settings and print data - Access audio-visual devices to view and play a multimedia file |
| Shut down digital device | - Save current work and back up important data - Close open programs on the digital device and any computer device or storage device - Shut down digital devices, according to manufacturer instructions |

Knowledge and understanding

- outline the capabilities and connectivity requirements of relevant:

  - audio-visual devices

  - peripheral devices

  - storage devices

- list basic security functions

- explain basic software operation and associated applications

- explain digital device functions

- Explain digital device settings.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

- turn on a digital device

- use the menu features and navigate a graphical user interface

- use device features to perform tasks

- Save the results of work.

### Carryout Basic Programming

Overview:

This Competency standard deals with the skills and knowledge required to write a program to perform arithmetic operations, write a program in C++ to preparing logical operation, and write a program to sort a string of numbers.

| Competency Units | Performance Criteria |
| --- | --- |
| 1.Write a program to perform arithmetic operations | - Install eclipse or other IDE with C++ - Design a program that can take inputs from users and perform arithmetic operations like a calculator - Run the program and verify the results |
| 2.Write a program in C++ to preparing logical operation | - Design a program that takes two numbers and decides which is bigger and which is smaller - Design a program that takes number of lights, fans in a house and calculate load |
| 3.Design a program to sort a string of numbers | - Write a script to take 10 numbers as input and display - Design a program to sort numbers in increasing order - Run the program and verify the results |

Knowledge & understanding

- Define Arithmetic operations

- Arithmetic and logical operations in C++

- Define Loop operations in C++

Tools and Equipment required

The tools and equipment required for this competency standard are given below:

| Sr.No. | Name |
| --- | --- |
| 1 | Turbo C++ Software |
| 2 | PC |

Critical Evidence(s) Required

The candidates need to produce following critical evidence(s) in order to be competent in this competency standard:

  - Design a program to sort a string of numbers

  - Write program to perform add two numbers

### Perform Basic Mathematical Calculations in C++

Overview:

This competency standard covers the skills and knowledge required for Application for the mathematic in C++ language

| Competency Units | Performance Criteria |
| --- | --- |
| - Generate Addition & Subtraction Program | - Open turbo C++ software - Create new file - Write a program for addition. - Save and run the program - Identify the error in compiler - Remove the error if required |
| - Generate Multiplication & Division Program | - Open turbo C++ software - Create new file - write a program for multiplication - Save and run the program - Identify the error in compiler - Remove the error if required |
| - Generate Program For Power Calculations | - Open turbo C++ software - Create new file - Write a program for division. - Save and run the program - Identify the error in compiler - Remove the error if required |
| - Generate Program For Roots Calculations | - Open turbo C++ software - Create new file - Write a program for roots calculation. - Save and run the program - Identify the error in compiler - Remove the error if required |
| - Generate Program For Exponential Calculations | - Open turbo C++ software - Create new file - Write a program for roots calculation. - Save and run the program - Identify the error in compiler - Remove the error if required |
| - Generate Trigonometric Function Program | - Open turbo C++ software - Create new file - Write a program for roots calculation. - Save and run the program - Identify the error in compiler - Remove the error if required |
| - Generate Inverse Trigonometric Function Program | - Open turbo C++ software - Create new file - Write a program for roots calculation. - Save and run the program - Identify the error in compiler - Remove the error if required |
| - Generate The Program and Calculate Impedance In Polar | - Open turbo C++ software - Create new file - Write a program for Impedance in polar. - Save and run the program. - Identify the error in compiler - Remove the error if required |
| - Generate the Program And Calculate Impedance In Rectangular | - Open turbo C++ software - Create new file - Write a program for Impedance in rectangular form - Save and run the program - Identify the error in compiler - Remove the error if required |
| - Generate the Program And - Calculate Impedance In Polar | - Open turbo C++ software - Create new file - Write a program for Impedance in rectangular form - Save and run the program - Identify the error in compiler - Remove the error if required |
| - Generate the Program And Calculate Impedance In Rectangular | - Open turbo C++ software - Create new file - Write a program for Impedance in rectangular form - Save and run the program - Identify the error in compiler - Remove the error if required |
| - Generate Rectangular To Polar Form Conversion program | - Open turbo C++ software - Create new file - Write a program for Impedance in rectangular form - Save and run the program - Identify the error in compiler - Remove the error if required |
| - Generate Polar Form To Rectangular Form program | - Open turbo C++ software - Create new file - Write a Program for conversion polar to rectangular form function. - Save and run the program - Identify the error in compiler - Remove the error if required |

Knowledge & Understanding

- Define C++ language

- Describe basic purpose of C++ language.

- Describe Data Types

- Describe operational procedure of C++.

- Describe complex number, its rectangular and polar form.

- Describe formula for conversion of rectangular to polar form.

Tools and Equipment Required

Tools and equipment required for this competency standard are given below:

| Sr.No. | Name |
| --- | --- |
| 1 | Power Cables |
| 2 | Keyboard, |
| 3 | CPU |
| 4 | Monitor |
| 5 | Printer |
| 6 | Multimedia |
| 7 | Multimedia Screen |
| 8 | Internet |
| 9 | Relevant Software Disks. |
| 10 | Presentation manuals |
| 11 | Handouts |
| 12 | Hardware equipment |

Critical Evidence(s) Required

The candidates need to produce following critical evidence(s) in order to be competent in this competency standard:

- How an existing C++ program works

- discover errors in a C++ program and describe how to fix them

- critique a C++ program and describe ways to improve it write code of calculator

### Identify Basic Electronics Components

Overview:

After this competency standard the candidate will be able to identify variety of basic electronic components and their usage in industry.

| Competency Units | Performance Criteria |
| --- | --- |
| 1. Identify Various Diodes | - Identify the Diodes - Identify its types & polarities - Draw Diode characteristics curves in forward and reverse Biased |
| 2. Identify Resistors in circuit | - Identify Resistor & its types - Recognize Coding &Color coding of resistor - Design series & Parallel circuit of Resistor - Use formulas to calculate the resistance in Series & parallel circuit |
| 3. Identify Capacitor in circuit | - Identify Capacitor & its types - Recognize Coding &rating of Capacitor - Design Parallel and series circuit of Capacitor - Use formulas to calculate the capacitance in Series & parallel circuit |
| 4. Identify Inductor in circuit | - Identify an Inductor - Recognize Coding & Rating of Inductor - Use formulas to calculate the inductance in Series & parallel circuit, |
| 5. Identify IC’s Packages. | - Identify IC Packages & types. - Apply the appropriate ICs Packages in circuit |

Critical Evidence(s) Required

The candidates need to produce following critical evidence(s) in order to be competent in this competency standard:

Evidence of the following is essential:

- Identify the anode and cathode of diode.

- Calculate the resistance of resistor having color (Red, Green, Blue& Gold).

- Enlist types of capacitor

- Identify the pin no. of an IC

Knowledge & Understanding

- Describe the diodes, polarities & their applications in circuits

- Explain the uses of multi-meter& power Supply

- Explain the data sheets

- Explain Resistor & their applications in Parallel & Series circuits

- Describe Capacitor & their applications in circuits

- Describe the Inductor& their applications in circuits

- Understand the data sheets

- Describe the basics of IC Packages

Tools and Equipment required

The tools and equipment required for this competency standard are given below:

| Sr.No. | Name |
| --- | --- |
| 1 | Multi-meter |
| 2 | Power supply |
| 3 | Trainer |
| 4 | Resistor |
| 5 | Capacitor |
| 6 | Inductor |
| 7 | ICs |
| 8 | Source of data sheets |

### Use digital instruments to test the logic signal

Overview:

This competency standard covers the skills and knowledge required toUse logic pulser and logic probe to test the logic output of digital gates, Use clip to test the logic output of digital gates.

| Competency Units | Performance Criteria |
| --- | --- |
| Test the logic output of digital gates using logic pulser and logic probe | - Place digital gate IC on bread board. - Connect IC with Vcc and Ground. - Give logic pulses at the input of digital gate with logic pulser. - Connect logic probe at the output terminal of gate and record the output. |
| Test the logic output of digital gates using clip. | - Place digital gate IC on bread board. - Connect IC with Vcc and Ground. - Connect logic Clip with pins of digital gate IC. - Give logic pulses at the input of digital gate with logic pulser. - Record the output form logic clip against each given input. |

Knowledge & Understanding

- Describe logic pulser.

- Describe the function of logic probe.

- Define logic clip.

Critical Evidence(s) Required

The candidates need to produce following critical evidence(s) in order to be competent in this competency standard:

Evidence of the following is essential:

- Generate 15 KHz, square wave signal, using signal generator.

- Measure voltage, frequency and time period of a signal using oscilloscope.

- Find out the rms& average value of AC signal using oscilloscope

Tools and Equipment required

The tools and equipment required for this competency standard are given below:

| Sr. No | Items |
| --- | --- |
| 1. | Digital gate IC |
| 2. | Bread board |
| 3. | Logic Probe |
| 4. | Logic pulser |
| 5. | Logic clip |
| 6. | Connecting leads |

### Use different instruments to measure the parameters of electrical signals &components

Overview:

This competency standard covers the skills and knowledge required to Measure AC voltage, current, frequency, time period with oscilloscope, RMS value and average value of AC signal with oscilloscope, Use frequency counter to measure the frequency, Use VU meter to measure sound intensity, Use XY recorder to record the signal on paper.

| Competency Units |  | Performance Criteria |
| --- | --- | --- |
| Measure frequency with the help of frequency counter. | - Generate sine or square wave with signal generator. - Connect the output of signal generator to the input of frequency meter. - Note the reading form display |  |
| Measure sound intensity by Using VU meter | - Connect vu meter directly to 600 ohm line as per specification. - Give audio electrical signal to the input of 600 ohm line. - Take reading form vu meter. |  |
| Record the signal on paper Using XY recorder. | - Turn on the XY recorder and make proper adjustment to record the electrical signal. - Generate the signal form signal generator. - Apply the output of signal generator to its x and y input of recorder. - Take the pattern of given signal on the paper. |  |
| Measure unknown resistance using wheat stone bridge | - Select the unknown resistor Rx - Connect Rx in Wheatstone - bridge. - Balance the wheat stone bridge. - Calculate unknown resistance Rx using formula. |  |
| Measure unknown C & L. Using universal bridge | - Select the unknown Cx/Lx - Connect the Cx/Lx in the universal bridge. - Balance the universal bridge. - Calculate unknown Cx/Lx using formula. |  |
| Measure unknown R,L& C Using LCR meter | - Select R/L/C to measure its value. - Select the proper control and range of LCR meter for R/L/C measurement. - Connect the leads of LCR meter across the unknown R/L/C - Record the value of R/L/C from display. |  |
| Measure Q of coil Using Q meter | - Select the coil whose quality factor has to measure. - Turn on the supply of Q meter and Connect the coil with Q meter - Change the capacitance of Q meter form its control till voltmeter shows maximum reading. - Take reading of electronic voltmeter and thermal volt meter in Q meter & find out the Q value of coil by taking ration of both meter. |  |
| Measure AC current Using Clamp on AC . | - Select the proper load for the measurement of current - Select the proper control and scale of clamp on AC meter. - Give supply to the load and insert one wire of load in the mouth of clamp on AC meter by use of pushing clip. - Record the value form display. |  |
| Measure High voltage with high voltage probe. | - Select voltage control of multimeter - Connect positive lead of multimeter with high voltage probe and negative to ground. - Turn on the supply of TV set and connect the high voltage probe to the anode of CRT. - Take the reading of voltmeter and multiply it by multiplying factor of high voltage probe and get the actual value of high voltage. |  |
| Measure power using watt meter | - Select a proper load to calculate the power - Connect watt meter with it. - Give supply and measure the reading of power. |  |

Knowledge & Understanding

- Explain different controls of frequency meter.

- Describe working principle of frequency meter.

- Define is Schmitt trigger and describe its use.

- Define VU meter.

- Describe the unit of sound intensity

- Define XY recorder.

- Define working principle of XY recorder.

- Define recording type instrument.

- Describe and draw the wheat stone bridge circuit

- Working principle of Wheat stone Bridge circuit

- Describe and draw the universal bridge

- Working principle of universal Bridge.

- Describe the balance condition of bridge.

- Describe purpose of LCR meter.

- Define resistance, capacitance and inductance.

- Describe working principle of LCR meter

- Describe Q meter.

- Explain the Q of a coil

- Define thermal voltmeter

- Describe Clamp on AC meter.

- Describe working principle of clamp on AC meter.

- Describe different controls of clamp on AC meter.

- Describe the nominal range clamp on AC meter.

- Describe High voltage probe.

- Describe working principle of High voltage probe.

- What is multiplying factor of high voltage probe.

- Describe Watt meter

- Describe working principle of watt meter.

- Define current coil & voltage coil.

Critical Evidence(s) Required

The candidates need to produce following critical evidence(s) in order to be competent in this competency standard:

Evidence of the following is essential:

- Make connection diagram of VU meter to measure sound intensity.

- Draw wave shape of electrical signal using XY recorder.

- Measure value of unknown resistor using wheat stone bridge.

- Enlist the parameters which can be measured by universal bridge.

- Measure the Q value of a coil by using Q-meter.

- Measure AC current of a load using clamp on AC meter.

- Make connection of watt meter with load to measure its power.

Tools and Equipment required

The tools and equipment required for this competency standard are given below:

| Sr. No | Items |
| --- | --- |
| 1. | Power supply |
| 2. | Signal generator |
| 3. | Oscilloscope |
| 4. | Connecting leads |
| 5. | UV meter |
| 6. | X ray recorder |
| 8. | Universal bridge |
| 9. | Wheat stone bridge circuit |
| 10. | Resistors |
| 11. | Inductors |
| 12. | Capacitors |
| 13. | LCR meter |
| 14. | Q meter |
| 15. | Coil |
| 16. | Voltmeter |
| 17. | Clamp on AC meter |
| 18. | Load |
| 19. | High Voltage probe |
| 20. | Watt meter |
| 21. | Proper AC load |

### Convert galvanometer into ammeter, voltmeter, Ohm meter & calibrate them

Overview:

This competency standard covers the skills and knowledge required to Dismantle and identify the parts of PMMC, Convert milli ammeter into ampere meter with shunt, Convert ammeter meter into voltmeter with multiplier, Convert ammeter into ohm meter.

| Competency Units | Performance Criteria |
| --- | --- |
| Disassemble and identify the parts of PMMC. | - Select the PMMC and establish a method for disassembling activity as per SOP. - Use standard tools described in user manual - Apply disassembling techniques - Organize the tag-identification of the parts/components/wires of instruments. - Identify the parts (coil, needle, scale, moving iron, balance and control weight, scale, air damping chamber, permanent magnet etc ) - Check for the proper operation/ functionality - Apply assembling techniques. |
| Convert milli ammeter into ampere meter with shunt. | - Identify the values of Im&Rm from Galvano meter. - Use the values of Im, Rm& IT (total current) for which meter has to design, in shunt resistance (Rsh) formula to calculate value of shunt resistor. - Connect the calculated shunt resistor in parallel to the Galvano meter. - Apply desired current to modified Galvano meter. - Record full scale movement of needle on the Galvanometer scale |
| Convert ammeter meter into voltmeter with multiplier. | - Identify the values of Im&Rm form ammeter. - Calculate value of multiplier resistor by Using the values of Im, Rm& VT (total voltage) for which meter must design, in multiplier resistance (Rse) formula - Connect the calculated multiplier resistor in series to the ammeter. - Apply desired voltage to the modified ammeter. - Record full scale movement of needle on the ammeter. |
| Convert ammeter into ohm meter. | - Get the full-scale deflection by using DC battery and series resistance - Connect the rheostat parallel to galvanometer for zero adjustment. - Short the connecting lead and verify the full-scale deflection. - Connect the leads across the known value resistor and record the needle deflection according to their values. |
| Calibrate voltmeter. | - Check the voltmeter physically for any abnormality. - If it is a digital voltmeter check its power On / Off system and necessary display. And if it is an Analog voltmeter carry out its physical checking of its electro dynamic or needle movement. - Select relevant appropriate reference voltage standard / source against which the voltmeter is to be calibrated. - Give proper warm up time / stabilization time to voltage meter before putting into the calibration setup. - Connect the voltage meter in the test / calibration set up / circuit according to the requirement of method selected or to the instructions given in its manual. - Select the desired ranges on the voltage meter and apply appropriate magnitude of voltage according to the range selected for calibration from the calibrator or the reference source. - Note the readings observed on the voltmeter. - Keep on increasing the applied magnitude of voltage from the source and note the corresponding readings given by the voltmeter. - Repeat every observation at least three times for each value of parameter selected. - Find standard deviation and uncertainty. - Prepare calibration report. |
| Calibrate ammeter | - Check the Instrument physically for any damage or abnormality. - Switch on the instrument and give proper time to stabilize the instrument. - Connect the two terminals of the voltage source to either side of the 1 k Ohm resistor. - Connect the two terminals of the ammeter in series to the resistor. - Switch on the voltage supply, and set it to 1 V. - Calculate the expected value of current using Ohm's law. Ohm's law states V=IR, where V is the voltage, I is the current and R is the resistance - Repeat every observation at least three times for each value of parameter selected. - Find standard deviation and uncertainty. - Prepare calibration report. |

Knowledge & Understanding

- Explain Electrical symbols.

- Explain law of electro magnetism.

- Describe the instrument’s functional parameter.

- Explain the various parts and components of the instrument.

- Explain Tagging techniques of connections

- Describe assembling & disassembling technique

- Define shunt.

- Define multiplier

- Describe the formula of shunt & multiplier.

- Define full scale current.

- Define Im, Rsh, Rse.

- Knowledge of Calibration.

- Working of precision digital multimeter.

- Working of voltmeter.

- Working of ammeter.

- Mathematical knowledge of calculating standard deviation,

- Average Uncertainty A and Uncertainty B

Critical Evidence(s) Required

The candidates need to produce following critical evidence(s) in order to be competent in this competency standard:

Evidence of the following is essential:

- Identify different parts of PMMC.

- Convert milli ammeter into voltmeter.

- Convert milli ammeter into ampere meter.

- Draw circuit diagram of series ohm meter using galvanometer.

- Draw circuit diagram of shunt ohm meter using galvanometer.

- Enlist methods of calibration for voltmeter &ampere meter

Tools and Equipment required

The tools and equipment required for this competency standard are given below:

| Sr. No | Items |
| --- | --- |
|  | Ammeter |
|  | Voltmeter |
|  | Electrical test bench |
|  | Power source (AC/DC) |
|  | Multimeter |
|  | Test probes |
|  | Magnetic/ compass |
|  | Static hand gloves |
|  | Screw drivers |
|  | Tweezer |
|  | Wire cutter |
|  | Nose plier |
|  | Galvanometer |
|  | DC power supply |
|  | Connecting leads |
|  | Resistance decade box |
|  | Plier |

### Measure Electrical power, Energy & Power Factor

Overview:

This competency standard covers the skills and knowledge required to Measure power using Volt-Ampere meter method, watt meter& Measure consumed energy with Energy meter

| Competency Units | Performance Criteria |
| --- | --- |
| - Measure power using Volt-Ampere meter method | - Select a proper load to calculate the power and give supply to it. - Measure the voltage and current by using voltmeter and ampere meter. - Find power using power formula. |
| - Measure power using watt meter | - Select a proper load to calculate the power and connect watt meter with it. - Give supply and measure the reading of power. |
| - Measure consumed energy with Energy meter | - Connect phase line of AC supply to the left side terminal of energy meter. - Connect neutral line of AC supply to the next input terminal of energy meter. - Connect any AC load to the output terminals of energy meter. - Give supply and take reading of energy in terms of unit after few minutes form display. |
| - Measure power factor with voltmeter, ampere meter and watt meter | - Connect voltmeter & ampere meter with inductive load (Motor) - Connect watt meter with the load also. - Give AC supply and take the readings of volt, ampere and watt meter. - Utilize the above readings and calculate power factor using power formula P=VI |
| - Measure power factor with power factor meter | - Connect current coil of power factor meter in series to the load. - Connect voltage coil of power factor meter in parallel to the load. - Give supply and note the value of power factor form power factor meter. |
| - Improve power factor & measure it with power factor meter | - Connect ampere meter as well as current coil of power factor meter in series with load - Connect voltage coil of power factor meter in parallel to the load. - Connect a capacitor bank parallel to load. - Select the small value capacitor with the help of selector switch and give supply and note the value of power factor form power factor meter and current reading form ampere meter. - Now select the higher value of capacitor form capacitor bank with the help of selector switch and note its effect on power factor and load current. |

Knowledge and Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Formula of D.C power

- measurement of power of any circuit with any meter

- difference between electrical and mechanical power

- C.C (current coil) & P.C (potential Coil)internal resistance of cell

- diffrence between power & Energy

- B.O.T

- Revolutions of energymeter

- loading Rheostat

- power factor

- the reasons for power factor lagging

- way to improve power factor of an A.C inductive circuit

- uses of wattmeter

- leading power factor

- way to improve power factor of an A.C inductive circuit with capacitor

- inductive load

- effect of improved power factor on load current

Tools and Equipment required

The tools and equipment required for this competency standard are given below:

| Sr.No. | Name |
| --- | --- |
| 1 | Wattmeter |
| 2 | Ammeter |
| 3 | Voltmeter |
| 4 | Connecting Leads |
| 5 | Power Supply |
| 6 | Screw driver & plier |
| 7 | Single Phase Load or 100 watt lamp. |
| 8 | Single phase energy meter with connecting lead |
| 9 | Rheostat load 2 Kw. 220/230 volts A.C |
| 10 | Power Supply with switch & fuse protection (0 -220 V.ac). Single phase Wattmeter. 0-500 watt. |
| 11 | Single phase inductive load. Any available load at your lab (up to 100 watt) |
| 12 | Capacitor bank |
| 13 | Single phase power factor meter |
| 14 | Connecting leads. As required. |

Critical Evidence(s) Required

The candidates need to produce following critical evidence(s) in order to be competent in this competency standard:

Evidence of the following is essential:

- Measure power of load using voltmeter and ampere meter.

- Make connection of watt meter to measure the power of a load.

- Make connection of energy meter to measure energy consumption of load.

- Connect power factor meter with load to measure its power factor.

- Connect the capacitor with inductive load to improve its power factor.

### Design a Rectifier using Diode

Overview:

This competency standard will help the candidate in construction of Diode rectifier and its uses in industry.

| Competency Units | Performance Criteria |
| --- | --- |
| 1. Construct half wave Rectifier | - Identify the required components for half wave Rectifier circuits. - Construct circuit Diagram of half wave Rectifier - Connect the circuit with the AC supply - Observe the input and Output wave form on oscilloscope - Calculate the ripple Factor - Calculate output voltage using proper formulas |
| 2. Construct Full Wave Rectifier (two diode rectifier) | - Identify the required components for Full Wave and half wave Rectifier circuits. - Construct circuit Diagram of half wave Rectifier - Construct circuit Diagram of Full Wave Rectifier - Connect the circuit with the AC supply - Observe the input and Output wave form on oscilloscope - Calculate the ripple Factor - Calculate output voltage using proper formulas |
| 3. Construct Full Wave Rectifier using Diode Bridge | - Identify the required components for Full Wave Bridge Rectifier circuit. - Construct circuit Diagram of Full Wave Bridge Rectifier - Connect the circuit with the AC supply - Observe the input and Output wave form on oscilloscope - Calculate the ripple Factor - Calculate output voltage using proper formulas |

Knowledge and understanding

- Explain the basics of diodes, AC &DC voltages For Rectifiers.

- Explain the uses of multi-meter

- Understand the data sheets

- Explain the uses oscilloscope

- Explain the uses of power Supply

- Explain the uses of oscilloscope and power Supply

Critical Evidence(s) Required

The candidates need to produce following critical evidence(s) in order to be competent in this competency standard:

Evidence of the following is essential:

- Enlist types of rectifiers circuits.

- Draw the circuit diagram of full wave bridge rectifier.

- Draw input & output wave form of half wave & full wave rectifier.

Tools and Equipment required

The tools and equipment required for this competency standard are given below:

| Sr.No. | Name |
| --- | --- |
| 1 | Multi-meter |
| 2 | Power Supply |
| 3 | Trainer |
| 4 | Oscilloscope |
| 5 | Diodes and Resistors |
| 6 | Source of Data Sheets |
| 7 | Functions Generator |
| 8 | Center taped Transformer |

### Implement Bipolar Junction Transistor (BJT) in Different Applications

Overview:

After completion of this competency standard the student will be able to regulate the current or voltage flow and implement a switch for electronic signals.

| Competency Units | Performance Criteria |
| --- | --- |
| Perform the Biasing of Transistors | P1. Identify the type of transistor. P2. Identify the base collector & Emitter of transistors. P3. Insert the transistor on bread board. P4. Perform the standard Biasing of PNP & NPN Transistor |
| Implement Transistor as an amplifier using CB Configuration. | - Draw the Circuit of amplifier using CB configuration of transistor - Select the components for CB configurations. - Insert the components for CB configuration amplifier. - Calculate the gain of transistor in CB modes. - Draw VI characteristics curve for CB configuration. |
| Implement Transistor as an amplifier using CC Configuration. | P1. Draw the Circuit of amplifier using CC configuration of transistor P2. Select the components for CC configurations. P3. Insert the components for CC configuration amplifier. P4. Calculate the gain of transistor in CC modes. P5. Draw VI characteristics curve for CC configuration |
| Implement Transistor as an amplifier using CE Configuration. | - Draw the Circuit of amplifier using CE configuration of transistor - Select the components for CE configurations. - Insert the components for CE configuration amplifier. - Calculate the gain of transistor in CE modes. |
| Implement BJT as a switch. | - Draw transistor switching circuit. - Select the components for switching circuits - Insert the components on bread board. - Verify switching operation of transistor using LED. |

Knowledge &Understanding

- Learn basic concepts of transistor & Biasing

- Study semiconductor theory

- Learn Doping Procedure.

- Study the datasheet of transistor

- Learn basic concepts & working principles of transistor

- Learn basics of Coupling Capacitor.

- Learn the V-I Characteristics

Critical Evidence(s) Required

The candidates need to produce following critical evidence(s) in order to be competent in this competency standard:

Evidence of the following is essential:

- Draw connection for proper biasing of transistor.

- Identify the base, collector and emitter terminals of a transistor.

- Enlist types of transistor.

- Enlist the types of amplifiers according to configuration of transistor.

- Implement transistor as a switch.

Tools and Equipment required

The tools and equipment required for this competency standard are given below:

| Sr.No. | Name |
| --- | --- |
| 1 | Transistors |
| 2 | Trainer |
| 3 | Multi-meter |
| 4 | Capacitors |
| 5 | Resistors |
| 6 | Power Supplies |
| 7 | Oscilloscope |

### Implement Field Effect Transistor (FET) in Different Applications

Overview:

After completion of this competency standard the student will be able to implement the FET (Field Effect Transistor) to control the current flow through the device.

| Competency Units | Performance Criteria |
| --- | --- |
| 1. Perform the Biasing of FET | P1. Identify the type of FET. P2. Identify the Gate, Drain & Source of FET. P3. Insert the FET on bread board. P4. Perform the standard Biasing of (N-Channel, P-Channel) FET. P4. Measure the Gate-Source voltage (Veg’s) & Threshold Voltage (Vth) |
| 2. Implement MOSFET as a switch. | - Draw switching circuit of MOSFET. - Select the components for switching circuits - Insert the components bread board. - Verify switching operation of MOSFET using LED. |
| 3. Draw the VI characteristics curves for FETs | P1. Construct an amplifier circuit using FETs P2. Apply Vds & Vgs P3 Measure the drain current P4. Draw VI characteristic curves |

Knowledge & understanding

- Learn the basics of FET

- Learn the concept of FET Biasing.

- Learn the power rating of FET

- Study The datasheet of FET

- Learn the Switching theory

- Learn the behavior of current and voltage in FET’s

- Learn biasing mechanism and basic formulae of FET’s

- Learn the vgs, vds, Idss&Rds as per datasheet.

Critical Evidence(s) Required

The candidates need to produce following critical evidence(s) in order to be competent in this competency standard:

Evidence of the following is essential:

- Draw connection for proper biasing of FET

- Identify the source, drain and gate terminals of a FET.

- Enlist types of FET

- Differentiate between JFET and MOSFET.

- Implement MOSFET as a switch.

Tools and Equipment required

The tools and equipment required for this competency standard are given below:

| Sr.No. | Name |
| --- | --- |
| 1 | FET Transistors |
| 2 | Multi-meter |
| 3 | Capacitors |
| 4 | Resistors |
| 5 | Power Supplies |
| 6 | Trainer |
| 7 | Oscilloscope |
| 8 | FET Transistors |
| 9 | Trainer |
| 10 | Oscilloscope |
| 11 | Simple FET |
| 12 | MOSFET |
| 13 | FET Trainer |

### Carry out thyristor Implementation (Uni Junction Transistor, Silicon Control Rectifier, Diac and Triac) in electronics circuits.

Overview:

After the completion of this standard the candidate will be able to install Uni junction Transistor (UJT), Silicon-controlled rectifier (SCR) in power Control Application.

| Competency Units | Performance Criteria |
| --- | --- |
| 1. Construct relaxation oscillator using UJT | - Draw the Schematic diagram of relaxation oscillator using UJT. - Select the components for relaxation oscillator. - Construct the relaxation oscillator circuit on bread board. - Connect the circuit with DC supply. - Observe the output wave shape on oscilloscope. - Vary the value of input resistor and record the effect on output. |
| 2. Implement the SCR in electronic circuits as switch | - Identify the SCR terminals - Draw the circuit of switch using SCR. - Select the components for SCR switching circuits. - Construct the SCR switching circuit. - Apply the trigger Pulse and Check out the desired outputs |
| 3. Construct the dimmer circuit using Diac & Triac. | P1. Draw the Schematic diagram of dimmer circuit using TRIAC & DIAC. P2. Select the components for dimmer circuit. P3. Construct the dimmer circuit. P4. Connect the circuit with AC supply. P5. Vary the potentiometer and record the effect on load (Fan or Lamp) |

Knowledge & understanding

- Learn the basics of UJT

- Understand the data sheets

- Learn adequate knowledge of hand tools

- Learn basics of SCR

- Learn the uses of oscilloscope, power Supply& Multi-meter

- Learn the basics of diac&triac

Critical Evidence(s) Required

The candidates need to produce following critical evidence(s) in order to be competent in this competency standard:

Evidence of the following is essential:

- Draw schematic diagram of relaxation oscillator using UJT.

- Identify the anode, cathode and gate terminals of a SCR.

- Differentiate between DIAC and TRIAC

- Draw schematic diagram of SCR as phase control rectifier.

- Draw characteristic cure of SCR.

Tools and Equipment required

The tools and equipment required for this competency standard are given below:

| Sr.No. | Name |
| --- | --- |
| 1 | Oscilloscope |
| 2 | UJT, |
| 3 | Resistor, |
| 4 | Variable DC power supply, |
| 5 | Multi-meter |
| 6 | Soldering iron, wire |
| 7 | Breadboard or trainer |
| 8 | SCR, Diodes, Resistors, Inductors, Capacitors & connecting wire |
| 9 | Trainer / Breadboard |
| 10 | Diac, Triac and resistors |
| 11 | Diode, Capacitor |

### Design & Assemble a Printed Circuit Board

Overview:

After this competency standard, the trainee will be able to gain knowledge and develop competence required to design and assemble a Printed Circuit Board (PCB).

| Competency Units | Performance Criteria |
| --- | --- |
| Prepare the design for PCB | - Recognize the occupational health and safety (OHS) rules and establish risk control measures - Choose the most appropriate method from Iron on Glossy paper method, Circuit by hand method and Laser cutting edge etching method. - Select tool and material required during various steps, involved in manufacturing - Prepare a time and cost estimation draft for the PCB. |
| Prepare a schematic diagram and PCB layout | - Select the PCB design software and its version - Discover the toolbar and libraries. - Draw a schematic diagram of the circuit. - Perform Routing. - Convert the schematic diagram into PCB layout diagram using software. |
| Print the PCB layout. | - Select the appropriate printer - Choose the suitable paper - Choose the proper ink for the print - Adjust the size of paper in printer - Send ‘Print’ command |
| Transfer Circuit to the PCB. | - Cut the copper board according to the size of the layout using a hacksaw or a cutter - Gently rub the copper side of PCB using sponge in order to remove the top oxide layer of copper as well as the photo resist layer by gentle rub - Adjust the printed paper on the board and use tape to align it properly - Apply moderate heat for few minutes using laundry iron to transfer ink from the paper to PCB board. - Soak it into the water until paper gets soggy and then gently remove the paper. |
| Perform Etching and cleaning | - Put on rubber or plastic gloves. - Place some newspaper on the bottom so the etching solution does not spoil your floor. - Take a plastic box and fill it up with some water. - Dissolve 2-3 teaspoons of ferric chloride powder in the water. - Dip the PCB into the etching solution (Ferric chloride solution, FeCl3) for approximately 30 minutes. - Use thinner (nail polish remover) on a pinch of cotton wool to remove tonner ink. - Dry the plate using clean cloth |
| Drill and Solder the components | - Select the mini drill. - Perform drilling on the copper side according to the layout - Properly rinse with water and dry the plate to remove unwanted particles - Select solder and perform soldering of the components - Perform Testing(If required) |

Knowledge and understanding

- Describe the occupational health and safety (OHS) rules

- Differentiate between Glossy paper method, Circuit by hand on PCB and Laser cutting edge etching.

- Which method is an industrial method?

- Describe the various steps involved in manufacturing and the tools and materials required.

- How to prepare cost and time estimation draft for the PCB

- Enlist the PCB design software and their latest versions

- Explain the tools and their functions in a toolbar

- Describe the Electronic Components and Sub-Assemblies required in circuit design

- Define ‘Routing’.

- How to prepare a schematic drawing and convert it into a PCB layout

- Explain the selection of printers

- Explain the usage of black ink

- Define the side of the paper, suitable for prints

- Explain the printing of your circuit

- Explain the steps involved in cutting a PCB board and removing oxide layer of copper

- What are the different temperature ranges for different papers?

- How to remove paper from the board?

- Describe the safety measures.

- Describe Etching?

- Why is there need for removing excessive copper?

- Which solution is used for Etching?

- What are the advantages of using FeCLl3?

- What are advantages of using thinner?

- What safety measures should be followed?

- Explain the uses of mini drill

- Why is drilling done on the copper side?

- Explain the size selection for solder

- Describe the post assembly Tests.

Critical Evidence(s) Required

The candidates need to produce following critical evidence(s) in order to be competent in this competency standard:

- PCB of charging circuit

Tools and Equipment required

The tools and equipment required for this competency standard are given below:

| Sr.No. | Name |
| --- | --- |
| 1 | PCB Board |
| 2 | Etching Solution (Ferric Chloride) |
| 3 | Fine Tipped Marker |
| 4 | Ruler (optional) |
| 5 | Magazine Paper |
| 6 | Glossy Paper |
| 7 | Plastic Straws |
| 8 | Small Piece of Cloth |
| 9 | Sanding Paper |
| 10 | Mini Drill (Dremel) |
| 11 | Flat Iron |
| 12 | Laser Printer / Photocopying Machine |
| 13 | Latex Gloves /Eye Protection |
| 14 | Altium Designer, OrCAD, Pads, KiCad, Eagle |

### Verify Ohm’s Law & Kirchhoff’s Law by `implementing Series & Parallel circuits

Overview:

This competency standard covers the skills and knowledge required to Make series circuit and measure voltage and verify KVL, make parallel circuit and measure current and verify KCL, verify resistance of a resistor,find unknown value of ohm’s law.

| Competency Units | Performance Criteria |
| --- | --- |
| - Make series circuit and measure voltage and verify KVL | - Construct series circuit with different resistors on bread board and give supply voltage. - Measure the voltage across each resistor. - Note the value of voltage against each resistor. - Sum the voltage of each resistor and verify is it equal to applied voltage. |
| - Make parallel circuit, measure current and verify KCL | - Construct parallel circuit with different resistors on bread board. - Measure the current of each resistor as well as total current. - Note the value of current against each resistor. - Sum the reading of ampere meters & verify |
| - Verify resistance of a resistor | - Select the 4 color band resistor. - Determine the value of resistor using color code. - Connect ohm meter across the resistor. - Note reading form ohm meter. - Compare both reading. |
| - Find unknown value of ohms law | - Construct the complex circuit using different resistors. - Find I, V with proper meter and determine R using ohms law - Find I, R with proper meter and determine V using ohm law - Find R,V with proper meter and determine I by ohm law |

Knowledge and Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Characteristics of series circuit

- Use of volt meter

- Characteristics of parallel circuit

- Use of ampere meter

- Color code and its use.

- Purpose of ohm meter

- Current, voltage and resistor interact with one another

- Purpose of ohm meter

- Kirchhoff’s first law.

- Kirchhoff’s second law.

Tools and Equipment required

The tools and equipment required for this competency standard are given below:

| Sr.No. | Name |
| --- | --- |
| 1 | Ammeter |
| 2 | Ohmmeter |
| 3 | Multimeter |
| 4 | Resistors of different values |
| 5 | DC battery |
| 6 | Small pieces of copper wire |

Critical Evidence(s) Required

The candidates need to produce following critical evidence(s) in order to be competent in this competency standard:

- Evidence of the following is essential:

- Implement series circuit and verify KVL.

- Implement parallel circuit and verify KCL.

- Find the value of resistance having colors Red, Green, Blue and Golden.

### Implement Electromagnet to observe various effects & verify Faraday’s law

Overview:

This competency standard covers the skills and knowledge required to implement the electromagnet, observe its various characteristics and verify the Faraday’s law.

| Competency Units | Performance Criteria |
| --- | --- |
| - Construct Electromagnet | - Take iron nail (approximately 3 inches in length) as iron core and make 30 to 40 turns of thin coated copper wire to form a coil. - Connect dry cell battery with coil wound on the iron nail. - Bring iron nail near the iron pieces and record the observation |
| - Implement circuit to determine the effect on current carrying conductor in magnetic field. | - Take copper rod (5cm in length) and connect wires across it. - Give DC supply to copper rod through rheostat. - Place current carrying copper rod inside the horse shoe magnet. |
| - Determine the effect on conductor by varying the current with the help of rheostat. | - Reduce the rheostat resistance - Record the effect on copper rod. - Increase the rheostat resistance - Record the effect on copper rod. |
| - Plot magnetic lines of forces of bar magnet | - Place a bar magnet on paper and outline its boundary with the help of lead pencil. - Place a compass needle at one side the magnet. - Mark points on paper where the compass needle stop. - Repeat the same procedure till compass reach at the other end of magnet. - Change the position of compass needle near the magnetic pole and repeat the procedure for P3 to P4 |
| - Verify Faradays law by moving magnet inside the coil. | - Construct a coil with hollow iron cylinder (approximately 3 inches in length 1.5 inch in diameter.) and make 150 to 200 turns on it. - Connect Galvano meter with coil. - Move permanent magnet inside the coil, vary its speed and record the effect on reading of Galvanometer. - Hold the magnet inside the coil and do not move, now record the effect on reading of Galvano meter. |
| - Verify Faradays law by moving coil near the magnet field. | - Construct a coil with hollow iron cylinder (approximately 3 inches in length 1.5 inch in diameter.) and make 150 to 200 turns on it. - Connect Galvano meter with coil. - Fix permanent magnet and move the coil fast and slow on it and record the effect on reading of Galvano meter. - Hold the coil near the magnetic field do not move, now record the effect on reading of Galvano meter. |
| - Verify emf through induction. | - Take step down transformer and connect its secondary with a Galvano meter and primary winding with a DC battery through a rheostat. - Continuously variate the rheostat and observe the reading on the Galvano meter |

Knowledge and Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Magnet, magnetism & electromagnet

- The functions of iron core

- Strong magnet

- The movement of current carrying conductor which is placed in magnetic field

- Fleming’s left-hand rule

- Magnetic lines of force.

- Magnetic field.

- Travel of magnetic lines of force with respect to each other.

- The first & 2nd law of michael faraday

- The role of magnetic strength in faraday’s law

- The role of conductor’s length or turns in faraday’s law

- Lenz’s law

- Induction

- Faraday’s first law of electro-magnetic induction

- Coupled coils

- The induction of emf in 2nd coil, when voltage is applied to 1st coil

- Mutual induction core

Tools and Equipment required

The tools and equipment required for this competency standard are given below:

| Sr.No. | Name |
| --- | --- |
| 1 | Iron nail as core |
| 2 | Thin coated copper wire |
| 3 | Current carrying conductor |
| 4 | Horse shoe magnet |
| 5 | Power Supply |
| 6 | Compass needle |
| 7 | Bar Magnet |
| 8 | Paper |
| 9 | Lead pencil |
| 10 | Coil |
| 11 | Galvanometer |
| 12 | Magnet |
| 13 | Connecting leads |
| 14 | Transformer |

Critical Evidence(s) Required

The candidates need to produce following critical evidence(s) in order to be competent in this competency standard:

Evidence of the following is essential:

- Construct an electromagnet and verify its operation.

- Plot magnetic field of around the magnet using compass.

- Verify faradays law using magnet and coil.

- Implement a circuit to verify emf through induction

### Measure different parameters of AC signal & Components

Overview:

This competency standard covers the skills and knowledge required to Measure AC voltage, current, frequency, time period with oscilloscope, rms value and average value of AC signal with oscilloscope

| Competency Units | Performance Criteria |
| --- | --- |
| - Measure AC voltage with oscilloscope | - Select a proper volt/division and give AC voltage to oscilloscope. - Read number of divisions between the +ve and -ve peaks of the signal. - Apply the formula to calculate the actual amplitude of AC voltage. |
| - Measure current with oscilloscope | - Select the proper load to measure the current. - Connect the low value of resistance in series with the load. - Connect the oscilloscope across low value of resistor and measure the voltage. - Apply ohms law by using value of low value of resistor and voltage across it and find out current. |
| - Measure time period of AC signal with oscilloscope | - Select a proper time/division and give AC voltage to oscilloscope. - Calculate number of divisions between the one AC cycle of the AC signal. - Apply the formula to calculate the time period of AC voltage. |
| - Measure frequency of AC signal with oscilloscope | - Select a proper time/division and give AC voltage to oscilloscope. - Calculate number of divisions between the one AC cycle of the signal. - Apply the formula to calculate time period - Apply the formula to calculate the actual frequency of AC voltage |
| - Measure rms value of AC signal with oscilloscope | - Select a proper volt/division and give AC voltage to oscilloscope. - Read number of divisions between the +ve and -ve peaks of the signal. - Apply the formula to calculate Vp-p of AC voltage. - Calculate peak value form Vp-p - Calculate rms value form peak voltage of AC |
| - Use LCR meter to measure unknown Resistor. | - Select Resistor to measure its value. - Select the proper control and range of LCR meter for resistance measurement. - Connect the leads of LCR meter across the unknown Resistor. - Record the value of Resistance from display. |
| - Use LCR meter to measure unknown Capacitance. | - Select capacitor to measure its value. - Select the proper control and range of LCR meter for capacitance measurement. - Connect the leads of LCR meter across the unknown capacitor. - Record the value of capacitance from display. |
| - Use LCR meter to measure unknown Inductance | - Select Inductor to measure its value. - Select the proper control and range of LCR meter for inductance measurement. - Connect the leads of LCR meter across the unknown Inductor. - Record the value of Inductance from display. |

Knowledge and Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Different controls of an oscilloscope

- Volt/division control.

- time/division control

- trigger, channel, frequency, time period & RMS value

- average value of ac signal.

- purpose of LCR meter.

- resistance, capacitance and inductance

- working principle of LCR meter

Tools and Equipment required

The tools and equipment required for this competency standard are given below:

| Sr.No. | Name |
| --- | --- |
| 1 | Cathode-ray oscilloscope |
| 2 | Multimeter |
| 3 | Oscillator |
| 4 | Signal generator |
| 5 | Power supply |
| 6 | LCR meter |
| 7 | Resistors |
| 8 | Inductors |
| 9 | Capacitors |
| 10 | Connecting leads |

Critical Evidence(s) Required

The candidates need to produce following critical evidence(s) in order to be competent in this competency standard:

Evidence of the following is essential:

- Measure amplitude of AC voltage with the help of oscilloscope.

- Measure frequency of AC signal using oscilloscope.

- Measure time period of AC signal using oscilloscope.

- Use LCR meter to measure Inductance, capacitance and resistance.

### Install Single Phase Complex Electrical Wiring

Overview:

This Competency Standard covers the skills and knowledge required to Install kitchen circuit, Install Drawing room circuit, Install Sleeping room circuit, Install hall wiring, Install impulse switch, Install Bell indicator circuit, Install timer/time switch circuit, Prepare fluorescent tube circuit and install

| Competency Units | Performance Criteria |
| --- | --- |
| 1. Install Bell indicator circuit. | - Draw wiring diagram of single pole switch circuit. - Draw Current path diagram of single pole switch circuit. - Draw layout diagram of single pole switch circuit. - Mark on working board according to layout diagram. - Install accessories according to layout diagram. - Lay wires in duct/pipe according to layout diagram. - Make connections according to wiring diagram. - Check the circuit before connect the main supply. - Make connection with main supply. - Check the function of circuit after connect the main supply |
| Prepare fluorescent tube circuit and install. | - Draw wiring diagram of single pole switch circuit. - Draw Current path diagram of single pole switch circuit. - Draw layout diagram of single pole switch circuit. - Mark on working board according to layout diagram. - Install accessories according to layout diagram. - Lay wires in duct/pipe according to layout diagram. - Make connections according to wiring diagram. - Check the circuit before connect the main supply. - Make connection with main supply. - Check the function of circuit after connect the main supply |

Knowledge & Understanding

- Define single pole switch.

- What is lamp?

- Explain single pole switch circuit and its use.

- Define current, voltage, power and resistance.

- Describe socket.

- Define series circuit.

- Define parallel circuit.

- Define impulse switch.

- Define power plug.

- Explain bell circuit.

- Explain fluorescent tube circuit

Critical Evidence(s) Required

The candidates need to produce following critical evidence(s) in order to be competent in this competency standard:

Evidence of the following is essential:

- Make circuit connection for door bell.

- Make connection to install fluorescent tube light.

Tools and Equipment required

The tools and equipment required for this competency standard are given below:

| Sr.No. | Name |
| --- | --- |
| 1 | Electrician Tool kit. |
| 2 | Single pole switch. |
| 3 | Lamp holder |
| 4 | Lamp. |
| 5 | Wooden/PVC board. |
| 6 | PVC Pipe/Duct. |
| 7 | PVC clamp. |
| 8 | Screw |
| 9 | PVC wire according to load. |
| 10 | Circuit breaker |
| 11 | AVO meter. |
| 12 | Test Indicator. |
| 13 | Series board. |
| 15 | Test Indicator. |
| 16 | Fluorescent Tube. |

### Install Three Phase Complex Electrical Wiring

Overview:

This Competency Standard covers the skills and knowledge required to Make and Install 3 phase motor connection Reverse/ forward by Magnetic contactor, Make and Install 3 phase motor connection Reverse/ forward by Magnetic contactor with indicator, Make and Install 3 phase motor connection Star/Delta (Manual) by Magnetic contactor, Make and Install 3 phase motor connection Star/Delta (Manual) by Magnetic contactor with indicator, Make and Install 3 phase motor connection Star/Delta (Auto) by Magnetic contactor, Make and Install 3 phase motor connection Star/Delta (Auto) by Magnetic contactor with indicator, Make and Install 3 phase motor connection 2 speed by Magnetic contactor, Make and Install 3 phase motor connection 2 speed by Magnetic contactor with indicator.

| Competency Units | Performance Criteria |
| --- | --- |
| Make and Install 3 phase motor connection Reverse/ forward by Magnetic contactor. | - Draw power diagram of circuit. - Draw control diagram of circuit. - Draw installation diagram of circuit. - Mark on exercise board according to installation/layout diagram. - Install accessories according to layout diagram. - Lay wires in duct/pipe according to layout diagram. - Make connections according to wiring diagram. - Check the circuit before connect the main supply. - Make connection with main supply. - Check the function of circuit after connect the main supply |
| Make and Install 3 phase motor connection Reverse/ forward by Magnetic contactor with indicator. | - Draw power diagram of circuit. - Draw control diagram of circuit. - Draw installation diagram of circuit. - Mark on exercise board according to installation/layout diagram. - Install accessories according to layout diagram. - Lay wires in duct/pipe according to layout diagram. - Make connections according to wiring diagram. - Check the circuit before connect the main supply. - Make connection with main supply. - Check the function of circuit after connect the main supply |
| Make and Install 3 phase motor connection Star/Delta (Manual) by Magnetic contactor. | - Draw power diagram of circuit. - Draw control diagram of circuit. - Draw installation diagram of circuit. - Mark on exercise board according to installation/layout diagram. - Install accessories according to layout diagram. - Lay wires in duct/pipe according to layout diagram. - Make connections according to wiring diagram. - Check the circuit before connect the main supply. - Make connection with main supply. - Check the function of circuit after connect the main supply |
| Make and Install 3 phase motor connection Star/Delta (Manual) by Magnetic contactor with indicator. | - Draw power diagram of circuit. - Draw control diagram of circuit. - Draw installation diagram of circuit. - Mark on exercise board according to installation/layout diagram. - Install accessories according to layout diagram. - Lay wires in duct/pipe according to layout diagram. - Make connections according to wiring diagram. - Check the circuit before connect the main supply. - Make connection with main supply. - Check the function of circuit after connect the main supply |
| Make and Install 3 phase motor connection Star/Delta (Auto) by Magnetic contactor. | - Draw power diagram of circuit. - Draw control diagram of circuit. - Draw installation diagram of circuit. - Mark on exercise board according to installation/layout diagram. - Install accessories according to layout diagram. - Lay wires in duct/pipe according to layout diagram. - Make connections according to wiring diagram. - Check the circuit before connect the main supply. - Make connection with main supply. - Check the function of circuit after connect the main supply |
| Make and Install 3 phase motor connection Star/Delta (Auto) by Magnetic contactor with indicator | - Draw power diagram of circuit. - Draw control diagram of circuit. - Draw installation diagram of circuit. - Mark on exercise board according to installation/layout diagram. - Install accessories according to layout diagram. - Lay wires in duct/pipe according to layout diagram. - Make connections according to wiring diagram. - Check the circuit before connect the main supply. - Make connection with main supply. - Check the function of circuit after connect the main supply |
| Make and Install 3 phase motor connection 2 speed by Magnetic contactor. | - Draw power diagram of circuit. - Draw control diagram of circuit. - Draw installation diagram of circuit. - Mark on exercise board according to installation/layout diagram. - Install accessories according to layout diagram. - Lay wires in duct/pipe according to layout diagram. - Make connections according to wiring diagram. - Check the circuit before connect the main supply. - Make connection with main supply. - Check the function of circuit after connect the main supply |
| Make and Install 3 phase motor connection 2 speed by Magnetic contactor with indicator | - Draw power diagram of circuit. - Draw control diagram of circuit. - Draw installation diagram of circuit. - Mark on exercise board according to installation/layout diagram. - Install accessories according to layout diagram. - Lay wires in duct/pipe according to layout diagram. - Make connections according to wiring diagram. - Check the circuit before connect the main supply. - Make connection with main supply. - Check the function of circuit after connect the main supply |

Knowledge & Understanding

- Define three phase supply.

- Describe three phase motor.

- Describe types of three phase motor.

- Define power diagram.

- Define installation diagram.

- Describe Control Diagram.

- Describe types of three phase motor connection.

- Describe Star/Delta connection.

- Describe the purpose of indicator.

- Explain reverse/forward circuit and its uses.

- Define star/delta circuit.

- Compare star/delta circuit.

- Describe purpose and uses of star/delta circuit.

- Describe the function of timer.

- Define timer.

- Describe 2 speed motor connection and its uses.

- Describe the purpose of 2 speed motor circuit.

Critical Evidence(s) Required

The candidates need to produce following critical evidence(s) in order to be competent in this competency standard:

Evidence of the following is essential:

- Make DOL starter connection with three phase motor.

- Make connection of automatic star-delta starter with three phase motor.

- Make manual star delta starter connection with three phase motor.

Tools and Equipment required

The tools and equipment required for this competency standard are given below:

| Sr.No. | Name |
| --- | --- |
| 1 | Electrician Tool kit |
| 2 | Magnetic Contactor. |
| 3 | Three phase induction motor. |
| 4 | Fuse. |
| 5 | PVC Board. |
| 6 | PVC Wire. |
| 7 | PVC Connector Bar |
| 8 | Screw. |
| 9 | Push button. |
| 10 | Over load relay. |
| 11 | Three phase induction motor |
| 15 | PVC Pipe/Duct. |
| 17 | Push button. |
| 20 | Indicator |
| 21 | PVC Board. |

### Overhaul the Lead Acid Batteries & implement their series & Parallel circuits

Overview:

This competency standard covers the skills and knowledge required to overhaul the lead acid batteries, preparation of their electrolyte, servicing & implementation of lead acid batteries in series & parallel circuit.

| Competency Units | Performance Criteria |
| --- | --- |
| - Construct series combination of batteries. | - Select the specific quantity cells. - Connect them in series with proper polarity. - Measure the voltage of series combined cell. - Increase the number of cells and note the voltage. - Interpret the effect. |
| - Construct series combination of batteries | - Select the specific quantity cells. - Connect them in parallel with proper polarity. - Measure the voltage of parallel combined cell. - Increase the number of cells and note the voltage. - Interpret the effect. |
| - Prepare Electrolyte for battery | - Select the proper container & and use proper safety equipment to prepare electrolyte - Pour H2SO4 in the container as per specification. - Put the water as per specification into the container and slowly - Mix the mixture with wooden/glass rod so that two solutions mix completely. |
| - Check specific gravity of Electrolyte. | - Open the vent plug of battery - Fill the hydrometer with electrolyte - Keep the hydrometer vertical and take reading form hydrometer |
| - Determine internal resistance of cell | - Adjust the apparatus as per diagram. - Turn the switch off and take the reading of voltmeter and note its value as “E”. - Turn on the switch and again take the reading of voltmeter and note its value as “V”. - Note the reading of ammeter as well. - Use above measured values and find out internal resistance using appropriate formula. |
| - Charge the battery with appropriate voltage. | - Set proper voltage and current setting of charger. - Connect the battery at the output terminal of charger. - Turn on the supply and note the current reading of ammeter from charger. - Remain connect the battery with charger till the ammeter show minimum or zero reading. - Disconnect the battery and check its specific gravity with hydrometer. |

Knowledge and Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- series combination of cell

- parallel combination of cell

- Electrolyte

- specific gravity

- hydrometer and its uses

- value of specific gravity of lead acid battery

- internal resistance of cell

- difference between “E” and “V” of cell

- parts of a charger

- function of charger

- minimum or zero reading OF Ammeter at full charging

- specific gravity of battery at full charge condition

Tools and Equipment required

The tools and equipment required for this competency standard are given below:

| Sr.No. | Name |
| --- | --- |
| 1 | Cells |
| 2 | Connecting leads |
| 3 | Voltmeter |
| 4 | Change over switch |
| 5 | H2SO4 |
| 6 | Water |
| 7 | Container |
| 8 | Hydrometer |
| 9 | Wooden/ Glass Rod |

Critical Evidence(s) Required

The candidates need to produce following critical evidence(s) in order to be competent in this competency standard:

- Implement batteries battery connection to get more voltage.

- Implement batteries battery connection to get more current.

- Prepare electrolyte having specific gravity of 1.2 to 1.23.

- Check and note specific gravity of electrolyte of using hydrometer.

- Write the steps to determine the internal resistance of cell.

- Implement connection of charger to charge two batteries in series and set proper voltage to charge them.

# Islamabad 31st May, 2019

# NOTIFICATION

# No. F. 5(13)/2018-DD (TE):     In pursuance of sub-section (d) of section-6" Functions of the Commission" National Vocational & Technical Training Commission (NAVTTC) Act-2011, NAVTTC is pleased  to approve and notify following qualifications in twenty (20) trades for Level 1-5 under National Vocational Qualification Framework (NVQF), which have been developed in compatibility with latest global trends in the fields and fulfilling requirements  of competency based training and assessment (CBT&A) system. The qualifications have been developed and validated in collaboration with TEVTAs, QABs, industry and other relevant stakeholders: -

| # S# | # National Vocational Qualifications |
| --- | --- |
|  | # National Qualification Level-5 diploma in Automobile Technology |
|  | National Qualification Level-5 diploma in Civil Technology |
|  | National Qualification Level-5 diploma in Construction Technology |
|  | National Qualification Level-5 diploma in Information & Commutation Technology (ICT) |
|  | National Qualification Level-5 diploma in Garment Manufacturing Technology |
|  | National Qualification Level-5 diploma in Electrical Technology |
|  | National Qualification Level-5 diploma in Electronics Technology |
|  | National Qualification Level-5 diploma in Instrumentation Technology |
|  | National Qualification Level-5 diploma in Computer Aided Design & Manufacturing (CAD /CAM) |
|  | National Qualification Level-5 diploma in Mechanical Technology |
|  | National Qualification Level-5 diploma in Graphics Designing |
|  | National Qualification Level-5 diploma in Heating, Ventilation, Air-conditioning & Refrigeration (HVACR) Technology |
|  | National Qualification Level-5 diploma in Media Production |
|  | National Qualification Level-5 diploma in Hotel Management |
|  | National Qualification Level-5 diploma in Professional Chef |
|  | National Qualification Level-5 diploma in Tourism Management |
|  | National Qualification Level-5 diploma in Hair & Beauty Services |
|  | National Qualification Level-5 diploma in Fashion Designing |
|  | National Qualification Level-5 diploma in Ceramics Technology |
|  | # National Qualification Level-5 diploma in Telecom Technology |

# 2.	All the TVET related institutions / organizations are required to implement aforementioned qualifications so that a uniform and standardized TVET qualification system is established in Pakistan and efforts are made for international equivalence / recognition of these qualifications.

# 3.	Competency Standards of the above enlisted qualifications can be accessed at NAVTTC's website (www.navttc.org).

# (Muqeem Islam)

# Director General (Skill Standards & Curricula)

# Phone: 051-9215385

# Distribution:

- Federal Secretary, Ministry of Federal Education & Professional Training, Govt of Pakistan

- Federal Secretary, Ministry of Overseas Pakistanis and Human Resource Development, Govt of Pakistan, Islamabad

- Federal Secretary, Ministry of Industry and Production, Govt of Pakistan, Islamabad

- Federal Secretary, Ministry of Textile Industry, Govt of Pakistan, Islamabad

- Federal Secretary, Ministry of Commerce, Govt of Pakistan, Islamabad

- Federal Secretary, Ministry of Railway, Govt of Pakistan, Islamabad

- Federal Secretary, Ministry of Climate Change, Govt of Pakistan, Islamabad

- Federal Secretary, Ministry of Religious Affairs, Govt of Pakistan, Islamabad

- Federal Secretary, Ministry of Communication, Govt of Pakistan, Islamabad

- Federal Secretary, Ministry of Aviation Division, Govt of Pakistan, Islamabad

- Federal Secretary, Ministry of Science & Technology, Govt of Pakistan, Islamabad

- Chairperson, Punjab Technical Education and Vocational Training Authority (P-TEVTA), Lahore

- Managing Director,  Khyber Pakhtunkhwa Technical Education and Vocational Training Authority (KP-TEVTA),

- Managing Director,  Sindh Technical Education and Vocational Training Authority (S-TEVTA), Karachi

- Chairman, Azad Jammu & Kashmir, Technical Education and Vocational Training Authority (AJ&K TEVTA), Muzafarabad

- Director TVET Cell, Gilgit Baltistan, Gilgit

- Director General, Punjab Vocational Training Council (PVTC), Punjab

- Managing Director,  Technology Upgradation and Skill Development Company (TUSDEC) Lahore

- Project Director, Punjab Skill Development Program (PSDP) Lahore

- CEO, Punjab Skill Development Fund, Lahore

- Rector, UNTECH University Islamabad

- National Deputy Leader, GIZ Islamabad

- PS to Minister of Federal Education & Professional Training, Govt of Pakistan

- PS to Special Adviser to the Prime Minister on Youth Affairs, Prime Minister's Office, Islamabad

- Chairperson, Federal of Pakistan Chamber of Commerce and Industry (FPCCI), Karachi

- Conveyor, Sector Skills Council (Textile/ Construction/ Renewable Energy/ Hospitality and Tourism)

- Director Technical Education and Vocational Training Authorities (TEVTA), Balochistan

- Chairman, Pakistan Tourism Development Corporation, Lahore

- Chairman, PCSIR Headquarters, Islamabad

- Director General, Pakistan Forest Institute, Peshawar

- Chairman, Wafaq ul Madaris, Multan

- Director General, Staff Welfare, Islamabad

- Director General, NISTE Capital Administration and Development Division, Islamabad

- Director General, National Training Bureau, Islamabad

- Chairmen, Provincial Technical Education Boards

- Chairmen, Provincial Trade Testing Boards

- Secretary, IBCC, Islamabad: with the request that National qualifications of Level 5 diploma in the aforementioned trades may be considered equivalent to Diploma of Associate Engineer/HSSC after inclusion of compulsory courses in the light of IBCC general requirement.

# Copy for information to: -

- DG (P&D)/(A&F)/ (A&C) (S&C) NAVTTC

- Director General(s), NAVTTC Regional Office(s).

- Sr. Technical Advisor, TSSP-GIZ

- Staff Officer to Chairman, NAVTTC

- PS to Executive Director, NAVTTC Islamabad

- Concerned File/ Office Copy