|  | # Assessment Evidence Guide # National Vocational Certificate Level-4 # in # “Fashion Design” # Associate Fashion Designer (Summative Assessment) |
| --- | --- |
|  | 13-17 Oct 2020 |
|  | National Vocational & Technical Training Commission |

| Title of Qualification: National Vocational Certificate Level 4, in (Fashion Design “Associate Fashion Designer” |  | CS Code: | Level:04 | Version:01 |
| --- | --- | --- | --- | --- |
| Competency Standard Title: - Establish and maintain the occupational Health and safety system - Perform Advance communication - Analyze Workplace Policies and Procedures - Perform Textile Basic Surface Techniques - Perform Surface Embellishment Techniques - Perform Basic Draping - Perform Fashion Drawing and Illustration - Perform Garment Stitching - Create Garment Design - Perform CAD for Textiles - Promote Market and Merchandising |  | Assessment Date (DD/MM/YY): Assessment Time: 30 min |  |  |
| Guidance for Candidate | To complete your assessment for this Competency Standard, you need to answer the questions on the following pages successfully. |  |  |  |

Assessors Guide (to be completed by the Assessor and signed both by the assessor and the candidate after the assessment)

| Candidate Details | Name: Registration/Roll Number: Candidate Signature: |
| --- | --- |
| Written Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor: Assessor’s code: Signature of the Assessor: |

| Title of Qualification: National Vocational Certificate Level 4, in (Fashion Design) “Associate Fashion Designer” | CS Code: | Level:04 | Version:01 |
| --- | --- | --- | --- |
| Competency Standard Title: - Establish and maintain the occupational Health and safety system - Perform Advance communication - Analyze Workplace Policies and Procedures - Perform Textile Basic Surface Techniques - Perform Surface Embellishment Techniques - Perform Basic Draping - Perform Fashion Drawing and Illustration - Perform Garment Stitching - Create Garment Design - Perform CAD for Textiles - Promote Market and Merchandising | Assessment Date (DD/MM/YY): Assessment Time: 30 min |  |  |

WRITTEN ASSESSMENT

| Question | Candidate’s answer |
| --- | --- |
| - What is the difference between domestic sewing machine and industrial sewing machine? | Industrial sewing machines differentiate themselves from home sewing machines by varieties of bed and feed type, computerized functions and the possibility of integrating the work aids. There is difference of oiling, speed/RPM. |
| - What is the ‘Feed’ of Machine? | Feed is the rate at which the cutting tool crosses the work piece. |
| - What is the difference between Dabka, Kora and Nakshi? | Dabka is a light weight coiled wire which is soft, flexible, and light both in weight and colour. A heavier form of dabka known as kora. Nakshi is a flat metal wire coiled in angular way similar to dabka except that it is thicker. |
| - What is meant by CAD? | CAD means Computer-aided design. It is the use of computers (or workstations) to aid in the creation, modification, analysis, or optimization of a design. CAD software is used to increase the productivity of the designer, improve the quality of design |
| - What is mood board? | Mood boards are a great reference point for starting your surface pattern designs. They enable you to visually communicate your ideas and allow you to collate all your inspiration together in one place. There are no rules for creating mood boards but they should convey your theme, colour and motif direction. |
| - What is Fashion figure/ Croquis? | Fashion figure/croquis means “sketch” in French. In the world of fashion design, a croquis is a quick sketch of a fashion figure. The word can also refer to the practice of drawing live models. Fashion figure/Croquis drawings are minimalist in style and serve as a blank canvas for drawing clothing. |
| - What is monochrome tone? | Monochrome colors are all the colors (tints, tones, and shades) of a single hue. Monochrome color schemes are derived from a single base hue. |
| - Differentiate between Plain weave and Satin weave. | A common and basic style of weave in which the weft alternates over and under the warp is called a plain weave whereas, A method of weaving fabric in which either the warp or the weft predominates on the surface is called Satin weave. |
| - What is knitting? | Forming and interlacing of loops using needles according to a prescribed manner or pattern, either by machine or by hand is called Knitting. |
| - What is the difference between Primary Research and Secondary Research? | Primary research is one that involves the gathering of fresh data, i.e. when data about a particular subject is collected for the first time whereas, Secondary research is a research method which involves the use of data, already collected through primary research. The main difference between primary and secondary research lies in the fact that whether the research is conducted previously or not. |
| - What is Trend forecast? | Trend Forecasting is the process of researching and formulating predictions on consumer’s future buying habits. By identifying the source, tracing the evolution, and recognizing patterns of trends, forecasters are able to provide designers and brands with a ‘vision’ of the future |
| - Difference between flat pattern and draping. | The draping pattern design is the art of wrapping a fabric around a mannequin and fastening it into a particular shape with the help of pins and thread in a three dimensional form. The flat-pattern method is where the entire pattern is drafted on a flat surface from measurements, using rulers, curves and straight-edges in 2D. |
| - Define grain line. | It’s actually the weave of the fabric in which direction the thread of the fabric is running. Cross grains are the threads running the in width of the fabric from one selvage to another. |
| - List any three variations of Bodice block. | Suspended panel bodice, straight yoke with bodice, prince’s panel bodice, round yoke with gathers. |
| - What is finishing in garment? | In garments industry finishing section involves with garments washing, checking, final inspection, pressing packing etc. |