Instruction Sheet for the Candidate

| Qualification | AC Machines (Transformer) |
| --- | --- |
| Competency Standard | Operate Auto Transformer |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within time frame (for practical demonstration & assessment): - Identify the connection of auto transformer - Calculate Voltage ratio of Auto Transformer. - Calculate Current ratio of Auto Transformer. |
| Time: 03 Hrs. | During a practical assessment, under observation by an assessor, you are required to |
| Minimum Evidence Required | Identify the connection of auto transformer - Connect voltmeters to the both side of the transformer. - Apply rated voltage to the primary side. - Identify the lower voltage and higher voltage side with the help of volt meter reading. - Identify step up and step down transformer. Calculate Voltage ratio of Auto Transformer. - Connect voltmeters to the both side of the transformer. - Apply rated voltage to the primary side. - Read the voltmeters reading. - Calculate voltage ratio with the help of voltmeters reading. Calculate Current ratio of Auto Transformer. - Connect ammeters to the both side of the transformer. - Connect a resistive load to secondary side of the transformer. - Apply rated voltage to the primary side. - Read the ammeters reading. - Calculate current ratio with the help of ammeters reading. |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | AC Machines (Transformer) |
| Competency Standard | Operate Auto Transformer |
| Purpose of Assessment | Formative Assessment |
| Assessment Task | - Identify the connection of auto transformer - Calculate Voltage ratio of Auto Transformer. - Calculate Current ratio of Auto Transformer. |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Connect voltmeters to the both side of the transformer. |  |  |
| - Apply rated voltage to the primary side. |  |  |
| - Identify the lower voltage and higher voltage side with the help of volt meter reading. |  |  |
| - Identify step up and step down transformer. |  |  |
| - Connect voltmeters to the both side of the transformer. |  |  |
| - Apply rated voltage to the primary side. |  |  |
| - Read the voltmeters reading. |  |  |
| - Calculate voltage ratio with the help of voltmeters reading. |  |  |
| - Connect ammeters to the both side of the transformer. |  |  |
| - Connect a resistive load to secondary side of the transformer. |  |  |
| - Apply rated voltage to the primary side. |  |  |
| - Read the ammeters reading. |  |  |
| - Calculate current ratio with the help of ammeters reading. |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | AC Machines (Transformer) |
| --- | --- |
| Competency Standard | Operate Auto Transformer |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________ Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________________________________ Assessor’s code:____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Identify the connection of auto transformer - Calculate Voltage ratio of Auto Transformer. - Calculate Current ratio of Auto Transformer. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Connected voltmeters to the both side of the transformer. |  |  |  |  |  |
|  | Applied rated voltage to the primary side. |  |  |  |  |  |
|  | Identified the lower voltage and higher voltage side with the help of volt meter reading. |  |  |  |  |  |
|  | Identified step up and step down transformer. |  |  |  |  |  |
|  | Connected voltmeters to the both side of the transformer. |  |  |  |  |  |
|  | Applied rated voltage to the primary side. |  |  |  |  |  |
|  | Read the voltmeters reading. |  |  |  |  |  |
|  | Calculated voltage ratio with the help of voltmeters reading. |  |  |  |  |  |
|  | Connected ammeters to the both side of the transformer. |  |  |  |  |  |
|  | Connected a resistive load to secondary side of the transformer. |  |  |  |  |  |
|  | Applied rated voltage to the primary side. |  |  |  |  |  |
|  | Read the ammeters reading. |  |  |  |  |  |
|  | Calculated current ratio with the help of ammeters reading. |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | AC Machines (Transformer) |
| --- | --- |
| Competency Standard | Operate Auto Transformer |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name:______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature:___________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor:________________________________________________ Assessor’s code:____________________________________________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | What is the different between auto transformer and ordinary transformer? |  |  |
|  | How to identify step up and step down transformer? |  |  |
|  | Explain voltage ratio of auto transformer. |  |  |
|  | Explain current ratio of auto transformer. |  |  |
|  | Explain the connection of ammeter and volt meter. |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature _________________