Instruction Sheet for the Candidate

| Qualification | AC Machines (Transformer) |
| --- | --- |
| Competency Standard | Operate Three Phase Transformer |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within time frame (for practical demonstration & assessment): - Make Three Phase Transformer Winding. - Perform Star To Star Connection Of Three Phase Transformer. - Perform Star To Delta Connection Of Three Phase Transformer. - Perform Delta To Delta Connection Of Three Phase Transformer - Perform Delta To Star Connection Of Three Phase Transformer - Connect Two Single Phase Transformers In Open Delta. - Perform Parallel operation of 3 phase transformers. |
| Time: 05 Hrs. | During a practical assessment, under observation by an assessor, you are required to |
| Minimum Evidence Required | Make Three Phase Transformer Winding - Design the core of transformer according to the transformer rating. - Select the connection of primary and secondary. - Select the wire for primary and secondary winding. - Perform winding according to require turns with the help of winding machine. - Cover the winding with insulating paper. - Adjust the core pieces into the winding. - Remove the insulation of enameled wire. - Cover the ends of wire with sleeve. - Connect the winding terminals in star or delta. - Perform continuity test with ohm meter. Perform Star To Star Connection Of Three Phase Transformer - Select three same rated single phase transformers. - Mark the polarity of the winding terminals. - Connect primary terminals and secondary terminals in star. - Connect voltmeters with primary and secondary. - Apply rated voltage to the primary. - Calculate and Verify line voltage and phase voltage Perform Star To Delta Connection Of Three Phase Transformer - Select three same rated single phase transformers. - Mark the polarity of the winding terminals. - Connect primary terminals in star and secondary terminals in delta. - Connect voltmeters with primary and secondary. - Apply rated voltage to the primary. - Calculate and Verify line voltage and phase voltage Perform Delta To Delta Connection Of Three Phase Transformer - Select three same rated single phase transformers. - Mark the polarity of the winding terminals. - Connect primary terminals and secondary terminals in delta. - Connect voltmeters with primary and secondary. - Apply rated voltage to the primary. - Calculate and Verify line voltage and phase voltage Perform Delta To Star Connection Of Three Phase Transformer - Select three same rated single phase transformers. - Mark the polarity of the winding terminals. - Connect primary terminals in Delta and secondary terminals in Star. - Connect voltmeters with primary and secondary. - Apply rated voltage to the primary. - Calculate and Verify line voltage and phase voltage. Connect Two Single Phase Transformers In Open Delta. - Select two same rated single phase transformers. - Mark the polarity of the winding terminals. - Connect opposite terminals of primary winding of both transformers - Take out connected terminals and other two terminals. - Repeat 35 to 38 for secondary side of transformer. - Connect voltmeters with primary and secondary sides of transformers. - Apply rated voltage to the primary. - Calculate and Verify line voltage and phase voltage. Perform Parallel operation of 3 phase transformers. - Identify two three phase transformer having same rated voltage, frequency and transformation ratio. - Connect the primary of each transformer in star connection. - Connect the secondary of each transformer in Delta connection. - Connect secondary sides of both transformers in parallel. - Connect secondary sides of both transformers with load bus bar having indicator between them. - Connect the primary side of both transformers in parallel. - Connect primary side of transformer with generation bus bar. - Adjust the series resistance and series inductance to off the indicator. |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | AC Machines (Transformer) |
| Competency Standard | Operate Three Phase Transformer |
| Purpose of Assessment | Formative Assessment |
| Assessment Task | - Make Three Phase Transformer Winding. - Perform Star To Star Connection Of Three Phase Transformer. - Perform Star To Delta Connection Of Three Phase Transformer. - Perform Delta To Delta Connection Of Three Phase Transformer - Perform Delta To Star Connection Of Three Phase Transformer - Connect Two Single Phase Transformers In Open Delta. - Perform Parallel operation of 3 phase transformers. |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Design the core of transformer according to the transformer rating. |  |  |
| - Select the connection of primary and secondary. |  |  |
| - Select the wire for primary and secondary winding. |  |  |
| - Perform winding according to require turns with the help of winding machine. |  |  |
| - Cover the winding with insulating paper. |  |  |
| - Adjust the core pieces into the winding. |  |  |
| - Remove the insulation of enameled wire. |  |  |
| - Cover the ends of wire with sleeve. |  |  |
| - Connect the winding terminals in star or delta. |  |  |
| - Perform continuity test with ohm meter. |  |  |
| - Select three same rated single phase transformers. |  |  |
| - Mark the polarity of the winding terminals. |  |  |
| - Connect primary terminals and secondary terminals in star. |  |  |
| - Connect voltmeters with primary and secondary. |  |  |
| - Apply rated voltage to the primary. |  |  |
| - Calculate and Verify line voltage and phase voltage |  |  |
| - Select three same rated single phase transformers. |  |  |
| - Mark the polarity of the winding terminals. |  |  |
| - Connect primary terminals in star and secondary terminals in delta. |  |  |
| - Connect voltmeters with primary and secondary. |  |  |
| - Apply rated voltage to the primary. |  |  |
| - Calculate and Verify line voltage and phase voltage |  |  |
| - Select three same rated single phase transformers. |  |  |
| - Mark the polarity of the winding terminals. |  |  |
| - Connect primary terminals and secondary terminals in delta. |  |  |
| - Connect voltmeters with primary and secondary. |  |  |
| - Apply rated voltage to the primary. |  |  |
| - Calculate and Verify line voltage and phase voltage |  |  |
| - Select three same rated single phase transformers. |  |  |
| - Mark the polarity of the winding terminals. |  |  |
| - Connect primary terminals in Delta and secondary terminals in Star. |  |  |
| - Connect voltmeters with primary and secondary. |  |  |
| - Apply rated voltage to the primary. |  |  |
| - Calculate and Verify line voltage and phase voltage. |  |  |
| - Select two same rated single phase transformers. |  |  |
| - Mark the polarity of the winding terminals. |  |  |
| - Connect opposite terminals of primary winding of both transformers |  |  |
| - Take out connected terminals and other two terminals. |  |  |
| - Repeat 35 to 38 for secondary side of transformer. |  |  |
| - Connect voltmeters with primary and secondary sides of transformers. |  |  |
| - Apply rated voltage to the primary. |  |  |
| - Calculate and Verify line voltage and phase voltage. |  |  |
| - Identify two three phase transformer having same rated voltage, frequency and transformation ratio. |  |  |
| - Connect the primary of each transformer in star connection. |  |  |
| - Connect the secondary of each transformer in Delta connection. |  |  |
| - Connect secondary sides of both transformers in parallel. |  |  |
| - Connect secondary sides of both transformers with load bus bar having indicator between them. |  |  |
| - Connect the primary side of both transformers in parallel. |  |  |
| - Connect primary side of transformer with generation bus bar. |  |  |
| - Adjust the series resistance and series inductance to off the indicator. |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | AC Machines (Transformer) |
| --- | --- |
| Competency Standard | Operate Three Phase Transformer |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________ Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________________________________ Assessor’s code:____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Make Three Phase Transformer Winding. - Perform Star To Star Connection Of Three Phase Transformer. - Perform Star To Delta Connection Of Three Phase Transformer. - Perform Delta To Delta Connection Of Three Phase Transformer - Perform Delta To Star Connection Of Three Phase Transformer - Connect Two Single Phase Transformers In Open Delta. - Perform Parallel operation of 3 phase transformers. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Designed the core of transformer according to the transformer rating. |  |  |  |  |  |
|  | Selected the connection of primary and secondary. |  |  |  |  |  |
|  | Selected the wire for primary and secondary winding. |  |  |  |  |  |
|  | Performed winding according to require turns with the help of winding machine. |  |  |  |  |  |
|  | Covered the winding with insulating paper. |  |  |  |  |  |
|  | Adjusted the core pieces into the winding. |  |  |  |  |  |
|  | Removed the insulation of enameled wire. |  |  |  |  |  |
|  | Covered the ends of wire with sleeve. |  |  |  |  |  |
|  | Connected the winding terminals in star or delta. |  |  |  |  |  |
|  | Performed continuity test with ohm meter. |  |  |  |  |  |
|  | Selected three same rated single phase transformers. |  |  |  |  |  |
|  | Marked the polarity of the winding terminals. |  |  |  |  |  |
|  | Connected primary terminals and secondary terminals in star. |  |  |  |  |  |
|  | Connected voltmeters with primary and secondary. |  |  |  |  |  |
|  | Applied rated voltage to the primary. |  |  |  |  |  |
|  | Calculated and Verify line voltage and phase voltage |  |  |  |  |  |
|  | Selected three same rated single phase transformers. |  |  |  |  |  |
|  | Marked the polarity of the winding terminals. |  |  |  |  |  |
|  | Connected primary terminals in star and secondary terminals in delta. |  |  |  |  |  |
|  | Connected voltmeters with primary and secondary. |  |  |  |  |  |
|  | Applied rated voltage to the primary. |  |  |  |  |  |
|  | Calculated and Verify line voltage and phase voltage |  |  |  |  |  |
|  | Selected three same rated single phase transformers. |  |  |  |  |  |
|  | Marked the polarity of the winding terminals. |  |  |  |  |  |
|  | Connected primary terminals and secondary terminals in delta. |  |  |  |  |  |
|  | Connected voltmeters with primary and secondary. |  |  |  |  |  |
|  | Applied rated voltage to the primary. |  |  |  |  |  |
|  | Calculated and Verify line voltage and phase voltage |  |  |  |  |  |
|  | Selected three same rated single phase transformers. |  |  |  |  |  |
|  | Marked the polarity of the winding terminals. |  |  |  |  |  |
|  | Connected primary terminals in Delta and secondary terminals in Star. |  |  |  |  |  |
|  | Connected voltmeters with primary and secondary. |  |  |  |  |  |
|  | Applied rated voltage to the primary. |  |  |  |  |  |
|  | Calculated and Verify line voltage and phase voltage. |  |  |  |  |  |
|  | Selected two same rated single phase transformers. |  |  |  |  |  |
|  | Marked the polarity of the winding terminals. |  |  |  |  |  |
|  | Connected opposite terminals of primary winding of both transformers |  |  |  |  |  |
|  | Took out connected terminals and other two terminals. |  |  |  |  |  |
|  | Repeated 35 to 38 for secondary side of transformer. |  |  |  |  |  |
|  | Connected voltmeters with primary and secondary sides of transformers. |  |  |  |  |  |
|  | Applied rated voltage to the primary. |  |  |  |  |  |
|  | Calculated and Verify line voltage and phase voltage. |  |  |  |  |  |
|  | Identified two three phase transformer having same rated voltage, frequency and transformation ratio. |  |  |  |  |  |
|  | Connected the primary of each transformer in star connection. |  |  |  |  |  |
|  | Connected the secondary of each transformer in Delta connection. |  |  |  |  |  |
|  | Connected secondary sides of both transformers in parallel. |  |  |  |  |  |
|  | Connected secondary sides of both transformers with load bus bar having indicator between them. |  |  |  |  |  |
|  | Connected the primary side of both transformers in parallel. |  |  |  |  |  |
|  | Connected primary side of transformer with generation bus bar. |  |  |  |  |  |
|  | Adjusted the series resistance and series inductance to off the indicator. |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | AC Machines (Transformer) |
| --- | --- |
| Competency Standard | Operate Three Phase Transformer |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name:______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature:___________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor:________________________________________________ Assessor’s code:____________________________________________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | How to select the wire for primary and secondary winding? |  |  |
|  | Explain winding procedure. |  |  |
|  | What is star connection? |  |  |
|  | Explain the relation between line voltage and phase voltage in star connection. |  |  |
|  | Describe the conditions for paralleling of the transformer |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature _________________