Instruction Sheet for the Candidate

| Qualification | Telecom operator (Power) |
| --- | --- |
| Competency Standard | Demonstrate Modulation, Demodulation, Multiplexing & Demulti-plexing. |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within 04 Hrs. time frame (for practical demonstration & assessment): - Perform Amplitude Modulation of Information Signal - Perform Demodulation of Modulated AM Signal - Calculate band width and modulation index - Perform Frequency Modulation of Information Signal - Perform Demodulation of Modulated FM Signal - Perform Pulse Code Modulation (PCM) of an analog signal - Perform Time Division Multiplexing (TDM) and De-multiplexing - Perform Frequency Division Multiplexing and De-multiplexing |
| Time: 04 Hrs. | During a practical assessment, under observation by an assessor, you are required to |
| Minimum Evidence Required | Perform Amplitude Modulation of Information Signal - Calibrate CRO. - Generate Low Frequency Message signal and observe on channel 1 of CRO. - Apply the Message and Carrier signals to the AM Modulator Section of the AM Trainer. - Connect the output of the modulator to channel 2 of CRO and observe the signal. Perform Demodulation of Modulated AM Signal - Calibrate CRO. - Generate AM Modulated signal and observe on channel 1 of CRO. - Apply the Modulated and Carrier signals to the AM De-Modulator Section of the AM De-Modulation Trainer. - Connect the output of the de-modulator to channel 2 of CRO and observe the signal. - Compare the Message signal to the Demodulated signal. Calculate band width and modulation index - Calibrate CRO. - Measure the frequency and voltage of Low frequency and carrier frequency signal with oscilloscope. - Calculate modulation by formula - Calculate bandwidth by formula Where as Perform Frequency Modulation of Information Signal - Calibrate CRO. - Generate Low Frequency Message signal and observe on channel 1 of CRO. - Apply the Message and Carrier signals to the FM Modulator Section of the FM Trainer. - Connect the output of the modulator to channel 2 of CRO and observe the signal. Perform Demodulation of Modulated FM Signal - Calibrate CRO. - Generate FM Modulated signal and observe on channel 1 of CRO. - Apply the Modulated and Carrier signals to the FM De-Modulator Section of the FM De-Modulation Trainer. - Connect the output of the de-modulator to channel 2 of CRO and observe the signal. - Compare the Message signal to the Demodulated signal. Perform Pulse Code Modulation (PCM) of an analog signal - Generate Sine wave signal and observe on channel 1 of CRO. - Apply the generated signal and Clock signal to the PCM Modulator using jumpers. - Observe the output on channel 2 of CRO and also note the pattern of blinking of output LEDs. Perform Time Division Multiplexing (TDM) and De-multiplexing - Generate multiple low frequency signals. - Apply the signals to TDM Multiplexer. - Observe the multiplexed output on channel 1 of CRO. - Apply to multiplexed output to the De-multiplexer. - Observe output for first message signal on channel 1 of CRO and compare the output signal with the relative Message signal by observing it on channel 2 of CRO. - Repeat the step 5 for all the DE multiplexed signals one by one Perform Frequency Division Multiplexing and De-multiplexing - Generate multiple message signals. - Generate multiple carrier signals of different frequencies. - Apply each of the message and carrier signal pairs to their respective Balanced Modulators. - Combine the modulated signals using Adder and observe this FDM Multiplexed output on Channel 1 of CRO. - Apply the multiplexed signal to respective Band-Pass filters for each DE multiplexer section (or user). - Apply the output of each Filter to their respective Demodulators. - Observe the demodulated and hence FDM DE multiplexed signals for each user and compare them to their respective Transmitted signals by viewing both on CRO at the same time. |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | Telecom operator (Power) |
| Competency Standard | Demonstrate Modulation, Demodulation, Multiplexing & Demulti-plexing. |
| Purpose of Assessment | Formative Assessment |
| Assessment Task |  |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Generate Low Frequency Message signal and observe on channel 1 of CRO. |  |  |
| - Apply the Message and Carrier signals to the AM Modulator Section of the AM Trainer. |  |  |
| - Connect the output of the modulator to channel 2 of CRO and observe the signal. |  |  |
| - Generate AM Modulated signal and observe on channel 1 of CRO. |  |  |
| - Apply the Modulated and Carrier signals to the AM De-Modulator Section of the AM De-Modulation Trainer. |  |  |
| - Connect the output of the de-modulator to channel 2 of CRO and observe the signal. |  |  |
| - Compare the Message signal to the Demodulated signal. |  |  |
| - Measure the frequency and voltage of Low frequency and carrier frequency signal with oscilloscope. |  |  |
| - Calculate modulation by formula |  |  |
| - Calculate bandwidth by formula Where as |  |  |
| - Generate Low Frequency Message signal and observe on channel 1 of CRO. |  |  |
| - Apply the Message and Carrier signals to the FM Modulator Section of the FM Trainer. |  |  |
| - Connect the output of the modulator to channel 2 of CRO and observe the signal. |  |  |
| - Generate FM Modulated signal and observe on channel 1 of CRO. |  |  |
| - Apply the Modulated and Carrier signals to the FM De-Modulator Section of the FM De-Modulation Trainer. |  |  |
| - Connect the output of the de-modulator to channel 2 of CRO and observe the signal. |  |  |
| - Compare the Message signal to the Demodulated signal. |  |  |
| - Generate Sine wave signal and observe on channel 1 of CRO. |  |  |
| - Apply the generated signal and Clock signal to the PCM Modulator using jumpers. |  |  |
| - Observe the output on channel 2 of CRO and also note the pattern of blinking of output LEDs. |  |  |
| - Generate multiple low frequency signals. |  |  |
| - Apply the signals to TDM Multiplexer. |  |  |
| - Observe the multiplexed output on channel 1 of CRO. |  |  |
| - Apply to multiplexed output to the De-multiplexer. |  |  |
| - Observe output for first message signal on channel 1 of CRO and compare the output signal with the relative Message signal by observing it on channel 2 of CRO. |  |  |
| - Repeat the step 5 for all the DE multiplexed signals one by one |  |  |
| - Generate multiple message signals. |  |  |
| - Generate multiple carrier signals of different frequencies. |  |  |
| - Apply each of the message and carrier signal pairs to their respective Balanced Modulators. |  |  |
| - Combine the modulated signals using Adder and observe this FDM Multiplexed output on Channel 1 of CRO. |  |  |
| - Apply the multiplexed signal to respective Band-Pass filters for each DE multiplexer section (or user). |  |  |
| - Apply the output of each Filter to their respective Demodulators. |  |  |
| - Observe the demodulated and hence FDM DE multiplexed signals for each user and compare them to their respective Transmitted signals by viewing both on CRO at the same time. |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | Telecom operator (Power) |
| --- | --- |
| Competency Standard | Demonstrate Modulation, Demodulation, Multiplexing & Demulti-plexing. |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________ Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________________________________ Assessor’s code: ____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Generate Low Frequency Message signal and observe on channel 1 of CRO. |  |  |  |  |  |
|  | Apply the Message and Carrier signals to the AM Modulator Section of the AM Trainer. |  |  |  |  |  |
|  | Connect the output of the modulator to channel 2 of CRO and observe the signal. |  |  |  |  |  |
|  | Generate AM Modulated signal and observe on channel 1 of CRO. |  |  |  |  |  |
|  | Apply the Modulated and Carrier signals to the AM De-Modulator Section of the AM De-Modulation Trainer. |  |  |  |  |  |
|  | Connect the output of the de-modulator to channel 2 of CRO and observe the signal. |  |  |  |  |  |
|  | Compare the Message signal to the Demodulated signal. |  |  |  |  |  |
|  | Measure the frequency and voltage of Low frequency and carrier frequency signal with oscilloscope. |  |  |  |  |  |
|  | Calculate modulation by formula |  |  |  |  |  |
|  | Calculate bandwidth by formula Where as |  |  |  |  |  |
|  | Generate Low Frequency Message signal and observe on channel 1 of CRO. |  |  |  |  |  |
|  | Apply the Message and Carrier signals to the FM Modulator Section of the FM Trainer. |  |  |  |  |  |
|  | Connect the output of the modulator to channel 2 of CRO and observe the signal. |  |  |  |  |  |
|  | Generate FM Modulated signal and observe on channel 1 of CRO. |  |  |  |  |  |
|  | Apply the Modulated and Carrier signals to the FM De-Modulator Section of the FM De-Modulation Trainer. |  |  |  |  |  |
|  | Connect the output of the de-modulator to channel 2 of CRO and observe the signal. |  |  |  |  |  |
|  | Compare the Message signal to the Demodulated signal. |  |  |  |  |  |
|  | Generate Sine wave signal and observe on channel 1 of CRO. |  |  |  |  |  |
|  | Apply the generated signal and Clock signal to the PCM Modulator using jumpers. |  |  |  |  |  |
|  | Observe the output on channel 2 of CRO and also note the pattern of blinking of output LEDs. |  |  |  |  |  |
|  | Generate multiple low frequency signals. |  |  |  |  |  |
|  | Apply the signals to TDM Multiplexer. |  |  |  |  |  |
|  | Observe the multiplexed output on channel 1 of CRO. |  |  |  |  |  |
|  | Apply to multiplexed output to the De-multiplexer. |  |  |  |  |  |
|  | Observe output for first message signal on channel 1 of CRO and compare the output signal with the relative Message signal by observing it on channel 2 of CRO. |  |  |  |  |  |
|  | Repeat the step 5 for all the DE multiplexed signals one by one |  |  |  |  |  |
|  | Generate multiple message signals. |  |  |  |  |  |
|  | Generate multiple carrier signals of different frequencies. |  |  |  |  |  |
|  | Apply each of the message and carrier signal pairs to their respective Balanced Modulators. |  |  |  |  |  |
|  | Combine the modulated signals using Adder and observe this FDM Multiplexed output on Channel 1 of CRO. |  |  |  |  |  |
|  | Apply the multiplexed signal to respective Band-Pass filters for each DE multiplexer section (or user). |  |  |  |  |  |
|  | Apply the output of each Filter to their respective Demodulators. |  |  |  |  |  |
|  | Observe the demodulated and hence FDM DE multiplexed signals for each user and compare them to their respective Transmitted signals by viewing both on CRO at the same time. |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | Telecom operator (Power) |
| --- | --- |
| Competency Standard | Demonstrate Modulation, Demodulation, Multiplexing & Demultiplexing. |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature: ___________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor: ________________________________________________ Assessor’s code: ____________________________________________________ Signature of the Assessor: _______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Define modulation. |  |  |
|  | Enlist types of modulation. |  |  |
|  | Define amplitude modulation. |  |  |
|  | Define side band. |  |  |
|  | Define bandwidth. |  |  |
|  | Define modulation index. |  |  |
|  | Define frequency modulation. |  |  |
|  | Define demodulation. |  |  |
|  | Define multiplexing & demultiplexing. |  |  |
|  | Define frequency division multiplexing. |  |  |
|  | Define time division multiplexing. |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature _________________