Instruction Sheet for the Candidate

| Qualification | Telecom operator (Power) |
| --- | --- |
| Competency Standard | Install Cord Less Telephone, PABX & Satellite Dish System. |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within 04 Hrs. time frame (for practical demonstration & assessment): - Install cord less telephone system. - Install and Configure of Private Automatic Branch Exchange (PABX) - Install Home satellite dish system. |
| Time: 04 Hrs. | During a practical assessment, under observation by an assessor, you are required to |
| Minimum Evidence Required | Install cord less telephone system - Chose proper location to install the base station - Connect the phone line to the base the base station of cord less telephone. - Connect the supply to the base station. - Pic-up the Mobile unit from cordless telephone and make a call to ensure the working functionality. Install and Configure of Private Automatic Branch Exchange(PABX - Lay 2-Pair Telephone drop wire around the premises. - Plug the telephone cord into your PBX console in an input that says, "Telephone Line." - Connect the other end into a wall jack. This allows your PBX system to send and receive phone calls. - Insert the PBX plug into the console. Plug the other end into the wall outlet. Wait for your PBX system to light up. This lets you know that you have properly installed the system. - Pick up the receiver to place a call. Dial the extension or phone number you would like to reach. Wait for the caller to answer on the other end. - Wait for the system to ring and light up, which means you have an incoming call. Pick up the receiver and answer the call. You can also press "Intercom" or "Hands Free" to answer the call Install Home satellite dish system. - Perform dish assembly as per SOP - Perform dish installation on roof top as per SOP. - Route coaxial cable and connect dish receiver to dish antenna. - Connect dish receiver output to TV receiver input. - Turn on the supply of both receivers, enter the frequency of required channel. - Adjust the antenna with the help of satellite locator chart till required channel is observed on TV screen |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | Telecom operator (Power) |
| Competency Standard | Install Cord Less Telephone, PABX & Satellite Dish System. |
| Purpose of Assessment | Formative Assessment |
| Assessment Task |  |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Chose proper location to install the base station |  |  |
| - Connect the phone line to the base the base station of cord less telephone. |  |  |
| - Connect the supply to the base station. |  |  |
| - Pic-up the Mobile unit from cordless telephone and make a call to ensure the working functionality. |  |  |
| - Lay 2-Pair Telephone drop wire around the premises. |  |  |
| - Plug the telephone cord into your PBX console in an input that says, "Telephone Line." |  |  |
| - Connect the other end into a wall jack. This allows your PBX system to send and receive phone calls. |  |  |
| - Insert the PBX plug into the console. Plug the other end into the wall outlet. Wait for your PBX system to light up. This lets you know that you have properly installed the system. |  |  |
| - Pick up the receiver to place a call. Dial the extension or phone number you would like to reach. Wait for the caller to answer on the other end. |  |  |
| - Wait for the system to ring and light up, which means you have an incoming call. Pick up the receiver and answer the call. You can also press "Intercom" or "Hands Free" to answer the call |  |  |
| - Perform dish assembly as per SOP |  |  |
| - Perform dish installation on roof top as per SOP. |  |  |
| - Route coaxial cable and connect dish receiver to dish antenna. |  |  |
| - Connect dish receiver output to TV receiver input. |  |  |
| - Turn on the supply of both receivers, enter the frequency of required channel. |  |  |
| - Adjust the antenna with the help of satellite locator chart till required channel is observed on TV screen |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | Telecom operator (Power) |
| --- | --- |
| Competency Standard | Install Cord Less Telephone, PABX & Satellite Dish System. |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________ Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________________________________ Assessor’s code: ____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Chose proper location to install the base station |  |  |  |  |  |
|  | Connect the phone line to the base the base station of cord less telephone. |  |  |  |  |  |
|  | Connect the supply to the base station. |  |  |  |  |  |
|  | Pic-up the Mobile unit from cordless telephone and make a call to ensure the working functionality. |  |  |  |  |  |
|  | Lay 2-Pair Telephone drop wire around the premises. |  |  |  |  |  |
|  | Plug the telephone cord into your PBX console in an input that says, "Telephone Line." |  |  |  |  |  |
|  | Connect the other end into a wall jack. This allows your PBX system to send and receive phone calls. |  |  |  |  |  |
|  | Insert the PBX plug into the console. Plug the other end into the wall outlet. Wait for your PBX system to light up. This lets you know that you have properly installed the system. |  |  |  |  |  |
|  | Pick up the receiver to place a call. Dial the extension or phone number you would like to reach. Wait for the caller to answer on the other end. |  |  |  |  |  |
|  | Wait for the system to ring and light up, which means you have an incoming call. Pick up the receiver and answer the call. You can also press "Intercom" or "Hands Free" to answer the call |  |  |  |  |  |
|  | Perform dish assembly as per SOP |  |  |  |  |  |
|  | Perform dish installation on roof top as per SOP. |  |  |  |  |  |
|  | Route coaxial cable and connect dish receiver to dish antenna. |  |  |  |  |  |
|  | Connect dish receiver output to TV receiver input. |  |  |  |  |  |
|  | Turn on the supply of both receivers, enter the frequency of required channel. |  |  |  |  |  |
|  | Adjust the antenna with the help of satellite locator chart till required channel is observed on TV screen |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | Telecom operator (Power) |
| --- | --- |
| Competency Standard | Install Cord Less Telephone, PABX & Satellite Dish System. |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature: ___________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor: ________________________________________________ Assessor’s code: ____________________________________________________ Signature of the Assessor: _______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | What are basic units of cord less telephone set? |  |  |
|  | What is base unit? |  |  |
|  | What is mobile unit? |  |  |
|  | What does PABX stands for? |  |  |
|  | What are different functional of PABX? |  |  |
|  | What is meant by parabola? |  |  |
|  | Differentiate between directional and omni-directional antenna. |  |  |
|  | Which type of antenna is used for home satellite dish system.? |  |  |
|  | what do you meant by polarization? |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature _________________