Instruction Sheet for the Candidate

| Qualification | DC machines (Motor and Generator) |
| --- | --- |
| Competency Standard | Analyze DC Generators |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within time frame (for practical demonstration & assessment): - Calculate different terms used in armature winding of DC machines - Develop Single Layer Lap Winding - Develop Double Layer Wave Winding - Measure the resistance of field winding, armature winding and terminal polarity - Identify DC shunt generator - Identify DC series generator - Identify DC compound generator |
| Time: 05 Hrs. | During a practical assessment, under observation by an assessor, you are required to |
| Minimum Evidence Required | Calculate different terms used in armature winding of DC machines - Count the number of slots (S) of armature. - Calculate pole pitch (Y). - Calculate Coil span/pitch. - Calculate Back pitch (Yb) - Calculate Front pitch (Yf) - Calculate Resultant Pitch (YR) - Calculate commentator pitch (Yc) Develop Single Layer Lap Winding - Count the number of slot in armature core and numbering on it. - Calculate total number of conductors (Z) - Calculate average pitch, back pitch and commentator pitch. - Start winding from slot no 1. - Add the value of back pitch in conductor. - Connect conductor 1 from back side to the added value. - Subtract the value of back pitch from the added value. - Connect added value from front side to subtracted value. - Continue this process till completion of all conductors. - Connect armature segment with armature conductor. - Insert brushes in proper position. Develop Double Layer Wave Winding - Count the number of slot in armature core and numbering on it. - Calculate total number of conductors (Z) - Calculate average pitch, back pitch and commentator pitch. - Start winding from slot no 1. - Add the value of back pitch in conductor. - Connect conductor 1 from back side to the added value. - Add the value of front pitch in previous added value. - Continue this process till completion of all conductors. - Connect armature segment with armature conductor. - Insert brushes in proper position. Measure the resistance of field winding, armature winding and terminal polarity - Open the terminal box. - Connect the ohm meter with the terminal of both winding. - Read down ohm meter reading. - Connect the probe of galvanometer with any terminal and touch other probe to other terminals one by one. - Rotate the generator slowly by hand. - Touch the probe of galvanometer with each winding. - Identify armature with the deflection of galvanometer pointer. - Mark the polarity of terminals with the deflection of galvanometer. Identify DC shunt generator - Measure the terminal resistance of generator with ohm meter. - Identify DC shunt generator from measured value of resistance. - Disconnect both windings. - Identify the shunt field winding of shunt generator. - Identify armature of shunt generator. - Measure the resistance of both winding separately. - Record the resistance of armature winding and field winding. - Compare the resistance of armature winding and field winding to identify the difference between them. Identify DC series generator - Measure the terminal resistance of generator with ohm meter. - Identify DC series generator from measured value of resistance. - Disconnect both windings. - Identify the series field winding of series generator. - Identify armature of series generator. - Measure the resistance of both winding separately. - Record the resistance of armature winding and field winding. - Compare the resistance of armature winding and field winding to identify the difference between them. Identify DC compound generator - Measure the terminal resistance of generator with ohm meter. - Identify DC Compound generator from measured value of resistance. - Disconnect both windings. - Identify the series field winding of generator. - Identify the shunt field winding of generator. - Identify armature of shunt generator. - Measure and record the resistance of both winding separately. - Measure and Record the resistance of armature winding. - Compare the resistance of armature winding, series field winding and shunt field winding to identify the difference between them. |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | DC machines (Motor and Generator) |
| Competency Standard | Analyze DC Generators |
| Purpose of Assessment | Formative Assessment |
| Assessment Task | - Calculate different terms used in armature winding of DC machines - Develop Single Layer Lap Winding - Develop Double Layer Wave Winding - Measure the resistance of field winding, armature winding and terminal polarity - Identify DC shunt generator - Identify DC series generator - Identify DC compound generator |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Count the number of slots (S) of armature. |  |  |
| - Calculate pole pitch (Y). |  |  |
| - Calculate Coil span/pitch. |  |  |
| - Calculate Back pitch (Yb) |  |  |
| - Calculate Front pitch (Yf) |  |  |
| - Calculate Resultant Pitch (YR) |  |  |
| - Calculate commentator pitch (Yc) |  |  |
| - Count the number of slot in armature core and numbering on it. |  |  |
| - Calculate total number of conductors (Z) |  |  |
| - Calculate average pitch, back pitch and commentator pitch. |  |  |
| - Start winding from slot no 1. |  |  |
| - Add the value of back pitch in conductor. |  |  |
| - Connect conductor 1 from back side to the added value. |  |  |
| - Subtract the value of back pitch from the added value. |  |  |
| - Connect added value from front side to subtracted value. |  |  |
| - Continue this process till completion of all conductors. |  |  |
| - Connect armature segment with armature conductor. |  |  |
| - Insert brushes in proper position. |  |  |
| - Count the number of slot in armature core and numbering on it. |  |  |
| - Calculate total number of conductors (Z) |  |  |
| - Calculate average pitch, back pitch and commentator pitch. |  |  |
| - Start winding from slot no 1. |  |  |
| - Add the value of back pitch in conductor. |  |  |
| - Connect conductor 1 from back side to the added value. |  |  |
| - Add the value of front pitch in previous added value. |  |  |
| - Continue this process till completion of all conductors. |  |  |
| - Connect armature segment with armature conductor. |  |  |
| - Insert brushes in proper position. |  |  |
| - Open the terminal box. |  |  |
| - Connect the ohm meter with the terminal of both winding. |  |  |
| - Read down ohm meter reading. |  |  |
| - Connect the probe of galvanometer with any terminal and touch other probe to other terminals one by one. |  |  |
| - Rotate the generator slowly by hand. |  |  |
| - Touch the probe of galvanometer with each winding. |  |  |
| - Identify armature with the deflection of galvanometer pointer. |  |  |
| - Mark the polarity of terminals with the deflection of galvanometer. |  |  |
| - Measure the terminal resistance of generator with ohm meter. |  |  |
| - Identify DC shunt generator from measured value of resistance. |  |  |
| - Disconnect both windings. |  |  |
| - Identify the shunt field winding of shunt generator. |  |  |
| - Identify armature of shunt generator. |  |  |
| - Measure the resistance of both winding separately. |  |  |
| - Record the resistance of armature winding and field winding. |  |  |
| - Compare the resistance of armature winding and field winding to identify the difference between them. |  |  |
| - Measure the terminal resistance of generator with ohm meter. |  |  |
| - Identify DC series generator from measured value of resistance. |  |  |
| - Disconnect both windings. |  |  |
| - Identify the series field winding of series generator. |  |  |
| - Identify armature of series generator. |  |  |
| - Measure the resistance of both winding separately. |  |  |
| - Record the resistance of armature winding and field winding. |  |  |
| - Compare the resistance of armature winding and field winding to identify the difference between them. |  |  |
| - Measure the terminal resistance of generator with ohm meter. |  |  |
| - Identify DC Compound generator from measured value of resistance. |  |  |
| - Disconnect both windings. |  |  |
| - Identify the series field winding of generator. |  |  |
| - Identify the shunt field winding of generator. |  |  |
| - Identify armature of shunt generator. |  |  |
| - Measure and record the resistance of both winding separately. |  |  |
| - Measure and Record the resistance of armature winding. |  |  |
| - Compare the resistance of armature winding, series field winding and shunt field winding to identify the difference between them. |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | DC machines (Motor and Generator) |
| --- | --- |
| Competency Standard | Analyze DC Generators |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________ Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________________________________ Assessor’s code:____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | 1.Calculate different terms used in armature winding of DC machines 2.Develop Single Layer Lap Winding 3.Develop Double Layer Wave Winding 4.Measure the resistance of field winding, armature winding and terminal polarity 5.Identify DC shunt generator 6.Identify DC series generator 7.Identify DC compound generator |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Counted the number of slots (S) of armature. |  |  |  |  |  |
|  | Calculated pole pitch (Y). |  |  |  |  |  |
|  | Calculated Coil span/pitch. |  |  |  |  |  |
|  | Calculated Back pitch (Yb) |  |  |  |  |  |
|  | Calculated Front pitch (Yf) |  |  |  |  |  |
|  | Calculated Resultant Pitch (YR) |  |  |  |  |  |
|  | Calculated commentator pitch (Yc) |  |  |  |  |  |
|  | Counted the number of slot in armature core and numbering on it. |  |  |  |  |  |
|  | Calculated total number of conductors (Z) |  |  |  |  |  |
|  | Calculated average pitch, back pitch and commentator pitch. |  |  |  |  |  |
|  | Started winding from slot no 1. |  |  |  |  |  |
|  | Added the value of back pitch in conductor. |  |  |  |  |  |
|  | Connected conductor 1 from back side to the added value. |  |  |  |  |  |
|  | Subtracted the value of back pitch from the added value. |  |  |  |  |  |
|  | Connected added value from front side to subtracted value. |  |  |  |  |  |
|  | Continued this process till completion of all conductors. |  |  |  |  |  |
|  | Connected armature segment with armature conductor. |  |  |  |  |  |
|  | Inserted brushes in proper position. |  |  |  |  |  |
|  | Counted the number of slot in armature core and numbering on it. |  |  |  |  |  |
|  | Calculated total number of conductors (Z) |  |  |  |  |  |
|  | Calculated average pitch, back pitch and commentator pitch. |  |  |  |  |  |
|  | Started winding from slot no 1. |  |  |  |  |  |
|  | Added the value of back pitch in conductor. |  |  |  |  |  |
|  | Connected conductor 1 from back side to the added value. |  |  |  |  |  |
|  | Added the value of front pitch in previous added value. |  |  |  |  |  |
|  | Continued this process till completion of all conductors. |  |  |  |  |  |
|  | Connected armature segment with armature conductor. |  |  |  |  |  |
|  | Inserted brushes in proper position. |  |  |  |  |  |
|  | Opened the terminal box. |  |  |  |  |  |
|  | Connected the ohm meter with the terminal of both winding. |  |  |  |  |  |
|  | Read down ohm meter reading. |  |  |  |  |  |
|  | Connected the probe of galvanometer with any terminal and touch other probe to other terminals one by one. |  |  |  |  |  |
|  | Rotated the generator slowly by hand. |  |  |  |  |  |
|  | Touched the probe of galvanometer with each winding. |  |  |  |  |  |
|  | Identified armature with the deflection of galvanometer pointer. |  |  |  |  |  |
|  | Marked the polarity of terminals with the deflection of galvanometer. |  |  |  |  |  |
|  | Measured the terminal resistance of generator with ohm meter. |  |  |  |  |  |
|  | Identified DC shunt generator from measured value of resistance. |  |  |  |  |  |
|  | Disconnected both windings. |  |  |  |  |  |
|  | Identified the shunt field winding of shunt generator. |  |  |  |  |  |
|  | Identified armature of shunt generator. |  |  |  |  |  |
|  | Measured the resistance of both winding separately. |  |  |  |  |  |
|  | Recorded the resistance of armature winding and field winding. |  |  |  |  |  |
|  | Compared the resistance of armature winding and field winding to identify the difference between them. |  |  |  |  |  |
|  | Measured the terminal resistance of generator with ohm meter. |  |  |  |  |  |
|  | Identified DC series generator from measured value of resistance. |  |  |  |  |  |
|  | Disconnected both windings. |  |  |  |  |  |
|  | Identified the series field winding of series generator. |  |  |  |  |  |
|  | Identified armature of series generator. |  |  |  |  |  |
|  | Measured the resistance of both winding separately. |  |  |  |  |  |
|  | Recorded the resistance of armature winding and field winding. |  |  |  |  |  |
|  | Compared the resistance of armature winding and field winding to identify the difference between them. |  |  |  |  |  |
|  | Measured the terminal resistance of generator with ohm meter. |  |  |  |  |  |
|  | Identified DC Compound generator from measured value of resistance. |  |  |  |  |  |
|  | Disconnected both windings. |  |  |  |  |  |
|  | Identified the series field winding of generator. |  |  |  |  |  |
|  | Identified the shunt field winding of generator. |  |  |  |  |  |
|  | Identified armature of shunt generator. |  |  |  |  |  |
|  | Measured and record the resistance of both winding separately. |  |  |  |  |  |
|  | Measured and Record the resistance of armature winding. |  |  |  |  |  |
|  | Compared the resistance of armature winding, series field winding and shunt field winding to identify the difference between them. |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | DC machines (Motor and Generator) |
| --- | --- |
| Competency Standard | Analyze DC Generators |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name:______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature:___________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor:________________________________________________ Assessor’s code:____________________________________________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | What is pole pitch (Y)? |  |  |
|  | How many parallel paths in lap winding? |  |  |
|  | What is mean by double layer winding? |  |  |
|  | What is different between field winding and armature winding? |  |  |
|  | Explain the technique to identify the terminal polarity |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature _________________