Instruction Sheet for the Candidate

| Qualification | DC machines (Motor and Generator) |
| --- | --- |
| Competency Standard | Repair Maintenance and Service of DC Motors and DC Generators |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within time frame (for practical demonstration & assessment): - Diagnose Fault in DC Motor - Carry out Service/Repair in Electric Motor - Carry out Preventive Service/ Maintenance of Generators - Diagnose and Repair Electric Generator |
| Time: 04 Hrs. | During a practical assessment, under observation by an assessor, you are required to |
| Minimum Evidence Required | Diagnose Fault in DC Motor - Identify the type of electric motor from data / name plate or manual and motor ratting. - Perform Methods of dismantling and assembling DC motor - Inspect visual mechanical defect such as, tight bearings, bent shaft, noisy running, short circuit, and temperatures. - Check the motor by using specified test instruments to detect electrical defects such as loose/or burnt electrical connections, defective capacitors, burnt windings, low insulation resistance. - Test short circuit, circuit break, earth fault, wiring fault, bearing fault. - Perform replacing of bearing - Identify the faults of a DC motor and repair them according to procedures. - Rewind a DC motor - Check the tripping of protective device using specified test instruments, while the motor is running with power supply connected with suitable control and protective device, the difficulty in starting/low RPM/ unusual noises/ excessive heat/ /grounded winding etc. Carry out Service/Repair in Electric Motor - Repair the motor as per diagnosed fault - Assemble the electric motor for internal tests/servicing/repairs according to manufacturer’s instructions - Clean the parts of the motor with specified cleaning agents/tools & material - Check the windings insulation resistance with insulation tester - Rewind the motor as per requirement - Check the insulation resistance of repaired motor as per standard - Carry out the No load and load tests as per standard practices Carry out Preventive Service/ Maintenance of Generators - Carry out routine maintenance of the generator as specified in the manufacturer’s manual - Check out the switch/ timer, relay, protective device and wire connection - Update the maintenance/service records Diagnose and Repair Electric Generator - Dismantle the electric Generator for internal tests/servicing/repairs according to manufacturer’s instructions - Clean the parts of the generator with specified cleaning agents/tools & material - Check the control equipment of generators. - Find out the root cause of fault according to the signs of fault by applying the working principles of control equipment of generators: Speed controller, Magnetic field control circuit and equipment of the generator. - Check Load regulator of the generator and its control circuit. - Control circuit of the auxiliary generator. - Check Control circuit of the battery unit - Check Circuit breaker and relay. - Repair the fault after finding out the faulty equipment or component - Check the windings insulation resistance with insulation tester as per standard - Repair the generator as per diagnosed fault. - Carry out the No load and load tests as per standard practices |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | DC machines (Motor and Generator) |
| Competency Standard | Repair Maintenance and Service of DC Motors and DC Generators |
| Purpose of Assessment | Formative Assessment |
| Assessment Task | - Diagnose Fault in DC Motor - Carry out Service/Repair in Electric Motor - Carry out Preventive Service/ Maintenance of Generators - Diagnose and Repair Electric Generator |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Identify the type of electric motor from data / name plate or manual and motor ratting. |  |  |
| - Perform Methods of dismantling and assembling DC motor |  |  |
| - Inspect visual mechanical defect such as, tight bearings, bent shaft, noisy running, short circuit, and temperatures. |  |  |
| - Check the motor by using specified test instruments to detect electrical defects such as loose/or burnt electrical connections, defective capacitors, burnt windings, low insulation resistance. |  |  |
| - Test short circuit, circuit break, earth fault, wiring fault, bearing fault. |  |  |
| - Perform replacing of bearing |  |  |
| - Identify the faults of a DC motor and repair them according to procedures. |  |  |
| - Rewind a DC motor |  |  |
| - Check the tripping of protective device using specified test instruments, while the motor is running with power supply connected with suitable control and protective device, the difficulty in starting/low RPM/ unusual noises/ excessive heat/ / grounded winding etc. |  |  |
| - Repair the motor as per diagnosed fault |  |  |
| - Assemble the electric motor for internal tests/servicing/repairs according to manufacturer’s instructions |  |  |
| - Clean the parts of the motor with specified cleaning agents/tools & material |  |  |
| - Check the windings insulation resistance with insulation tester |  |  |
| - Rewind the motor as per requirement |  |  |
| - Check the insulation resistance of repaired motor as per standard |  |  |
| - Carry out the No load and load tests as per standard practices |  |  |
| - Carry out routine maintenance of the generator as specified in the manufacturer’s manual |  |  |
| - Check out the switch/ timer, relay, protective device and wire connection |  |  |
| - Update the maintenance/service records |  |  |
| - Dismantle the electric Generator for internal tests/servicing/repairs according to manufacturer’s instructions |  |  |
| - Clean the parts of the generator with specified cleaning agents/tools & material |  |  |
| - Check the control equipment of generators. |  |  |
| - Find out the root cause of fault according to the signs of fault by applying the working principles of control equipment of generators: Speed controller, Magnetic field control circuit and equipment of the generator. |  |  |
| - Check Load regulator of the generator and its control circuit. |  |  |
| - Control circuit of the auxiliary generator. |  |  |
| - Check Control circuit of the battery unit |  |  |
| - Check Circuit breaker and relay. |  |  |
| - Repair the fault after finding out the faulty equipment or component |  |  |
| - Check the windings insulation resistance with insulation tester as per standard |  |  |
| - Repair the generator as per diagnosed fault. |  |  |
| - Carry out the No load and load tests as per standard practices |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | DC machines (Motor and Generator) |
| --- | --- |
| Competency Standard | Repair Maintenance and Service of DC Motors and DC Generators |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________ Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________________________________ Assessor’s code:____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Diagnose Fault in DC Motor - Carry out Service/Repair in Electric Motor - Carry out Preventive Service/ Maintenance of Generators - Diagnose and Repair Electric Generator |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Identified the type of electric motor from data / name plate or manual and motor ratting. |  |  |  |  |  |
|  | Performed Methods of dismantling and assembling DC motor |  |  |  |  |  |
|  | Inspected visual mechanical defect such as, tight bearings, bent shaft, noisy running, short circuit, and temperatures. |  |  |  |  |  |
|  | Checked the motor by using specified test instruments to detect electrical defects such as loose/or burnt electrical connections, defective capacitors, burnt windings, low insulation resistance. |  |  |  |  |  |
|  | Tested short circuit, circuit break, earth fault, wiring fault, bearing fault. |  |  |  |  |  |
|  | Performed replacing of bearing |  |  |  |  |  |
|  | Identified the faults of a DC motor and repair them according to procedures. |  |  |  |  |  |
|  | Rewound a DC motor |  |  |  |  |  |
|  | Checked the tripping of protective device using specified test instruments, while the motor is running with power supply connected with suitable control and protective device, the difficulty in starting/low RPM/ unusual noises/ excessive heat/ / grounded winding etc. |  |  |  |  |  |
|  | Repaired the motor as per diagnosed fault |  |  |  |  |  |
|  | Assembled the electric motor for internal tests/servicing/repairs according to manufacturer’s instructions |  |  |  |  |  |
|  | Cleaned the parts of the motor with specified cleaning agents/tools & material |  |  |  |  |  |
|  | Checked the windings insulation resistance with insulation tester |  |  |  |  |  |
|  | Rewound the motor as per requirement |  |  |  |  |  |
|  | Checked the insulation resistance of repaired motor as per standard |  |  |  |  |  |
|  | Carry out the No load and load tests as per standard practices |  |  |  |  |  |
|  | Carry out routine maintenance of the generator as specified in the manufacturer’s manual |  |  |  |  |  |
|  | Checked out the switch/ timer, relay, protective device and wire connection |  |  |  |  |  |
|  | Updated the maintenance/service records |  |  |  |  |  |
|  | Dismantled the electric Generator for internal tests/servicing/repairs according to manufacturer’s instructions |  |  |  |  |  |
|  | Cleaned the parts of the generator with specified cleaning agents/tools & material |  |  |  |  |  |
|  | Checked the control equipment of generators. |  |  |  |  |  |
|  | Find out the root cause of fault according to the signs of fault by applying the working principles of control equipment of generators, Speed controller, Magnetic field control circuit and equipment of the generator. |  |  |  |  |  |
|  | Checked Load regulator of the generator and its control circuit. |  |  |  |  |  |
|  | Checked control circuit of the auxiliary generator. |  |  |  |  |  |
|  | Checked Control circuit of the battery unit |  |  |  |  |  |
|  | Checked Circuit breaker and relay. |  |  |  |  |  |
|  | Repaired the fault after finding out the faulty equipment or component |  |  |  |  |  |
|  | Checked the windings insulation resistance with insulation tester as per standard |  |  |  |  |  |
|  | Repaired the generator as per diagnosed fault. |  |  |  |  |  |
|  | Carry out the No load and load tests as per standard practices |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | DC machines (Motor and Generator) |
| --- | --- |
| Competency Standard | Repair Maintenance and Service of DC Motors and DC Generators |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name:______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature:___________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor:________________________________________________ Assessor’s code:____________________________________________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | How to find the short circuit fault in DC motors? |  |  |
|  | What is preventive maintenance? |  |  |
|  | Discuss the type of maintenance. |  |  |
|  | Discuss Common faults found in electric motors/generators and troubleshooting methods |  |  |
|  | Explain the steps of preventive maintenance work on motors and stand by generators |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature _________________