National Vocational Certificate Level 4 in Electrical Technology

(Industrial Electrician)

Competency Standards (CS) Document

National Vocational and Technical Training Commission (NAVTTC)

Government of Pakistan

Table of Contents

Introduction	4

Purpose of the Qualification	5

Common Courses with respective levels	6

Levelling of Core Competencies of the Qualification along with the occupations	7

Packaging of Occupations	10

Mapping of the Qualifications	12

Date of Validation	13

Date of Review	13

Codes of Qualifications	13

Members of Qualifications Development Committee	14

Members of Qualification Validation Committee	15

Entry Requirements	15

Detail of Competency Standards	16

0713E&E1 Select Computer Specification and Work with Windows	16

0713E&E2 Process Data with MS Office	19

0713E&E3 Carryout Basic Programming	22

0713E&E4 Perform Internet Browsing	24

0713E&E5 Perform the Different Engineering Curves Used in Various Mechanism	26

0713E&E6 Perform Multi-View Drawings	31

0713E&E7 Draw Production Drawing	34

0713E&E8 Design and Simulate the Electronic Circuit by Using Computer Aided Design (CAD) Software	37

0713E&E9 Verify Basic Laws of Electrical Machines	41

0713E&E10 Analyze Dc Generators	44

0713E&E11 Perform Tests on DC Generators	50

0713E&E12 Analyze Dc Motors.	55

0713E&E13 Perform Tests on DC Motors.	58

0713E&E14 Repair/ Maintenance and Service of DC Motors and DC Generators.	64

0713E&E15 Verify Basic Laws of Electrical Machines	68

0713E&E16 Operate Single Phase Transformer	71

0713E&E17 Perform Tests on Single Phase Transformer	75

0713E&E18 Operate Auto Transformer	78

0713E&E19 Perform Tests on Auto Transformer.	80

0713E&E20 Operate Three Phase Transformer	82

0713E&E21 Identify Vector Group of Three Phase Transformer	86

0713E&E22 Repair/ Maintenance and Service of Transformer	89

0713E&E23 Maintain Tools & Equipment for Three Phase	92

0713E&E24 Make Cable/Wire Joints for Three Phase	95

0713E&E25 Install Three Phase Complex Electrical Wiring	98

0713E&E26 Perform Testing of Electrical Three Phase Wiring	109

0713E&E27 Diagnose Electrical Fault in Generator	114

0713E&E28 Repair/ Maintenance of Electrical Installations for Three Phase	118

0713E&E29 Carry Out Manual Electrical Planning and Estimation	122

0713E&E30 Develop Workplace Policy and Procedures for Sustainability	126

0713E&E31 Maintain Professionalism in the Workplace	130

0713E&E32 Manage Personal Work Priorities and Professional Development	133

0713E&E33 Manage Workforce Planning	136

0713E&E34 Undertake Project Work	140

0713E&E35 Prepare and Implement Negotiation	144

0713E&E36 Manage Meetings.	148

0713E&E37 Organize Schedules	151

0713E&E38 Identify and Communicate Trends in Career Development	153

0713E&E39 Apply Specialist Interpersonal and Counselling Interview Skills	157

# Introduction

The Technical and Vocational is a profession that is increasingly getting attention in Pakistan, not only among the youth seeking to enter the industry but also among adults who wish to polish their skills to develop a career out of it. Scope and demand of Electrical engineers, supervisors, operator, and technician is very high in Pakistan due to fast growing economic zone like CPEC. There are more job opportunities for electrical technician and supervisors in Pakistan than any other field of engineering. Pakistan is still facing energy crises, for need for more power houses, renewable energy sources is being felt which will definitely result in more demand of electrical engineers.

CBT based level 1-5 course in Electrical Technology with updated concepts, not only allows students to equip with growing Industrial trends but will be beneficial in capturing the job market across the globe. The course is divided into sub fields like electronics, computer engineering, power engineering, telecommunications, control systems and microelectronics in respective levels. Many of these sub disciplines overlap with other engineering branches, spanning a huge number of specializations such as hardware engineering, power electronics, avionics, mechatronics and nanotechnology.

Pakistan can boost economic growth and job creation by overcoming inefficiencies in its power sector, “Reforms that address these distortions can make better use of existing facilities. These need to focus on eliminating waste, promoting the shift towards cleaner energy and attracting private investments.”

“Power sector reform should be a top priority, as few other reforms could yield economic gains of a similar magnitude so quickly, based on findings from this report,” says Fan Zhang, Senior Economist at the World Bank. “If well designed, these reforms will directly benefit the poor by increasing access, improving reliability, and reducing cost and pollution.”

# Purpose of the Qualification

Based upon this demand of industry these competency‐based qualifications for Electrical Technology are developed under National Vocational Qualification Framework (Level 1 to 5). The qualifications mainly cover competencies along with related knowledge and professional attitude which is essential for getting a job or self-employed.

The qualifications are also in line with the vision of Pakistan’s National Skills Strategy (NSS), National TVET Policy and National Vocational Qualification Framework (NVQF). This provides policy directions, support and an enabling environment to the public and private sectors to impart training for skills development to enhance social and economic profile. The National Competency Standards could be used as a referral document for the development of curricula to be used by training institutions.

The purpose of the training is to provide the basic skills to the trainees in the field of Electrical and convert it into value added product which is acceptable by International market reducing the line losses and fit-in a skilled graduate into National Vocational Qualification Framework for his / her vertical career progression and qualification equivalencies at par with acceptable international standards.

# Common Courses with respective levels

LEVEL 3

LEVEL 2

LEVEL 4

LEVEL 5

# Levelling of Core Competencies of the Qualification along with the occupations

| Competency Standards | Occupation | NVQF Level | Category | Estimated Contact Hours |  |  | Cr Hr |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Th | Pr | Total |  |
| Level 4 |  |  |  |  |  |  |  |
| Select Computer Specification and Work with Windows. | Computer Operator (Electrical) | Level 4 | Technical | 6 | 21 | 27 | 2.7 |
| Process Data with MS Office. |  | Level 4 | Technical | 6 | 21 | 27 | 2.7 |
| Carryout Basic Programming |  | Level 4 | Technical | 6 | 21 | 27 | 2.7 |
| Perform Internet Browsing |  | Level 4 | Technical | 6 | 18 | 24 | 2.4 |
| Perform The Different Engineering Curves Used in Various Mechanisms. |  | Level 4 | Technical | 6 | 24 | 30 | 3 |
| Perform Multi-View Drawings. |  | Level 4 | Technical | 6 | 24 | 30 | 3 |
| Draw Production Drawing. |  | Level 4 | Technical | 6 | 18 | 24 | 2.4 |
| Design and Simulate the Electronic Circuit by Using Computer-Aided Design (Cad) Software |  | Level 4 | Technical | 3 | 30 | 33 | 3.3 |
| Verify Basic Laws of Electrical Machines | DC Machine Operator | Level 4 | Technical | 6 | 21 | 27 | 2.7 |
| Analyze Dc Generators |  | Level 4 | Technical | 6 | 21 | 27 | 2.7 |
| Perform Tests On Dc Generators |  | Level 4 | Technical | 6 | 24 | 30 | 3 |
| Analyze Dc Motors |  | Level 4 | Technical | 6 | 21 | 27 | 2.7 |
| Perform Tests On Dc Motors |  | Level 4 | Technical | 6 | 24 | 30 | 3 |
| Repair/ Maintenance and Service of Dc Motors and Dc Generators. |  | Level 4 | Technical | 6 | 24 | 30 | 3 |
| Verify Basic Laws of Electrical Machines | Transformer Winding Supervisor | Level 4 | Technical | 6 | 18 | 24 | 2.4 |
| Operate Single Phase Transformer |  | Level 4 | Technical | 6 | 18 | 24 | 2.4 |
| Perform Tests On Single Phase Transformer. |  | Level 4 | Technical | 6 | 24 | 30 | 3 |
| Operate Auto Transformer. |  | Level 4 | Technical | 3 | 24 | 27 | 2.7 |
| Perform Tests On Auto Transformer. |  | Level 4 | Technical | 6 | 30 | 36 | 3.6 |
| Operate Three Phase Transformer. |  | Level 4 | Technical | 6 | 27 | 33 | 3.3 |
| Identify Vector Group of Three Phase Transformer |  | Level 4 | Technical | 6 | 24 | 30 | 3 |
| Repair/ Maintenance and Service of Transformer |  | Level 4 | Technical | 6 | 24 | 30 | 3 |
| Maintain Tools & Equipment. | Industrial Electrician | Level 2 | Technical | 6 | 21 | 27 | 2.7 |
| Make Cable/Wire Joints for three phase |  | Level 2 | Technical | 6 | 24 | 30 | 3 |
| Prepare and Install Distribution Boards for three phase |  | Level 2 | Technical | 6 | 27 | 33 | 3.3 |
| Install Three Phase Electrical Wiring |  | Level 3 | Technical | 6 | 24 | 30 | 3 |
| Install Three Phase Complex Electrical Wiring |  | Level 3 | Technical | 6 | 27 | 33 | 3.3 |
| Perform Testing of Electrical Three Phase Wiring |  | Level 3 | Technical | 6 | 24 | 30 | 3 |
| Diagnose Electrical Fault in Generator |  | Level 3 | Technical | 6 | 24 | 30 | 3 |
| Repair/ Maintenance of Electrical Installations |  | Level 3 | Technical | 3 | 27 | 30 | 3 |
| Carry Out Manual Electrical Planning and Estimation |  | Level 3 | Technical | 9 | 24 | 33 | 3.3 |
| Develop Workplace Policy and Procedures for Sustainability | Soft Skills | Level 4 | Generic | 6 | 27 | 33 | 3.3 |
| Maintain Professionalism in the Workplace |  | Level 4 | Generic | 6 | 24 | 30 | 3 |
| Manage Personal Work Priorities and Professional Development |  | Level 4 | Generic | 6 | 24 | 30 | 3 |
| Manage Workforce Planning |  | Level 4 | Generic | 6 | 27 | 33 | 3.3 |
| Undertake Project Work |  | Level 4 | Generic | 6 | 30 | 36 | 3.6 |
| Prepare and Implement Negotiation |  | Level 4 | Generic | 6 | 24 | 30 | 3 |
| Manage Meetings |  | Level 4 | Generic | 6 | 15 | 21 | 2.1 |
| Organize Schedules |  | Level 4 | Generic | 6 | 15 | 21 | 2.1 |
| Identify and Communicate Trends in Career Development |  | Level 4 | Generic | 6 | 24 | 30 | 3 |
| Apply Specialist Interpersonal and Counseling Interview Skills |  | Level 4 | Generic | 6 | 27 | 33 | 3.3 |
| Total |  |  |  | 240 | 960 | 1200 | 120 |
| Percentage |  |  |  | 20 | 80 |  |  |

# Packaging of Occupations

The National Vocational Qualifications have been packaged as detailed below:

Level 4

(Computer Operator-Electrical, DC Machine Operator, Transformer Winding Supervisor, Soft Skills)

Industrial Electrician

- Maintain Tools & Equipment.

- Make Cable/Wire Joints

- Prepare and Install Distribution Boards.

- Install Three Phase Electrical Wiring

- Install Three Phase Complex Electrical Wiring

- Perform Testing of Electrical Wiring

- Diagnose Electrical Fault in Generator

- Repair/ Maintenance of Electrical Installations

- Carry Out Manual Electrical Planning and Estimation

Computer Operator-Electrical

- Select Computer Specification and Work with Windows.

- Process Data with MS Office.

- Carryout Basic Programming

- Explore Electrical

- Perform The Different Engineering Curves Used in Various Mechanism.

- Perform Multi-View Drawings.

- Draw Production Drawing.

- Design and Simulate the Electronic Circuit by Using Computer-Aided Design (Cad) Software

DC Machine Operator

- Verify Basic Laws of Electrical Machines

- Analyze Dc Generators

- Perform Tests On Dc Generators

- Analyze Dc Motors

- Perform Tests On Dc Motors

- Repair/ Maintenance and Service of Dc Motors and Dc Generators.

Transformer Winding Supervisor

- Verify Basic Laws of Electrical Machines

- Operate Single Phase Transformer

- Perform Tests On Single Phase Transformer.

- Operate Auto Transformer.

- Perform Tests On Auto Transformer.

- Operate Three Phase Transformer.

- Identify Vector Group of Three Phase Transformer

- Repair/ Maintenance and Service of Transformer

Soft Skills

- Develop Workplace Policy and Procedures for Sustainability

- Maintain Professionalism in the Workplace

- Manage Personal Work Priorities and Professional Development

- Manage Workforce Planning

- Undertake Project Work

- Prepare and Implement Negotiation

- Manage Meetings

- Organize Schedules

- Identify and Communicate Trends in Career Development

- Apply Specialist Interpersonal and Counseling Interview Skills

# Mapping of the Qualifications

AC DC Machines…

Power Electronics…..

Analog Electronics

Digital Electronics

Wiring

Soft Skills

Digital Skills

Entrepreneurship

Microcontrollers

Analog Electronics

Digital Electronics

Process Variable Measurements….

# Date of Validation

The level 5 of National DAE qualification on Electrical has been validated by the Qualifications Validation Committee (QVC) members on 23-25 May, 2019 and will remain valid for ten years i.e 25 May, 2029

# Date of Review

The level 5 of National DAE qualification on Electrical has been validated by the Qualifications Validation Committee (QVC) members on 23-25 May, 2019 and shall be reviewed after three years i.e. 26 May, 2022

# Codes of Qualifications

The International Standard Classification of Education (ISCED) is a framework for assembling, compiling and analyzing cross-nationally comparable statistics on education and training. ISCED codes for these qualifications are assigned as follows:

| ISCED Classification for Electrical Technology level 5 |  |
| --- | --- |
| Code | Description |
| 0713E&E(1) | National Vocational Certificate Level 1 in Electrical Technology |
| 0713E&E(2) | National Vocational Certificate Level 2 in Electrical Technology |
| 0713E&E(3) | National Vocational Certificate Level 3 in Electrical Technology |
| 0713E&E(4) | National Vocational Certificate Level 4 in Electrical Technology |
| 0713E&E(5) | National Vocational Diploma Level 5 in Electrical Technology |

# Members of Qualifications Development Committee

The following members participated in the qualification development of this qualification:

| S# | Name | Designation | Organization |
| --- | --- | --- | --- |
| 1 | Aijaz Ahmed Zia | Design & Application Engineer | INTECH Process Automation. Lahore |
| 2 | Amir Amin | HOD Electrical, Research Associate LUMS | City Polytechnic |
| 3 | Muhammad Faheem Anjum | Chief Instructor Electrical | P-TEVTA |
| 4 | Muhammad Asad | Instructor Electrical | P-TEVTA |
| 5 | Tahreem Javeed | Lab Engineer | The University of Lahore |
| 6 | Huma Naeem | Research Associate | COMSATS Lahore |
| 7 | Engr.Inayat Ur Rehman | DACUM Facilitator | KPK-TEVTA |
| 8 | Muhammad Yasir | NAVTTC Coordinator | NAVTTC HQ Islamabad |

# Members of Qualification Validation Committee

The following members participated in the qualifications validation of this qualification:

| S# | Name | Designation | Organization |
| --- | --- | --- | --- |
| 1 | Israr Ahmaad | DD Academia | KP-TEVTA |
| 2 | Muhammad Yasir | NAVTTC Coordinator | NAVTTC HQ Islamabad |
| 3 | Sayeda Fatima Iqbal | System Analyst | PBTE Lahore |
| 4 | Danish Khan | Calibration Engineer | NPSL(PCSIR) Islamabad |
| 5 | Ghulam Rasool Maka | Vice Principal | S TEVTA |
| 6 | Muhammad Mehboob butt | Associate Professor | P TEVTA |
| 7 | Inayat Ur Rehman | EX Professor/ Ex-Director Academics | KP-TEVTA |
| 8 | Muhammad Asad | Instructor Electrical | P-TEVTA |

# Entry Requirements

The entry level for National Vocational Certificate Level 4 in Electrical Technology is:

- A person having National Vocational Certificate Level 3 in Electrical Technology

# Detail of Competency Standards

### 0713E&E1 Select Computer Specification and Work with Windows

Overview:

This Competency standard deals with the skills and knowledge required to identify computer requirements and assembles procedures, Make a set of requirements for a personal computer, Use DOS to write a script, Work on windows, Environment and Install the required operating System with Device drivers.

| Competency Units | Performance Criteria |
| --- | --- |
| - Identify computer requirements and assemble procedures | - Select the hardware components of Computer. - Install the necessary plug-ins - Install the required computer software’s for operations of peripherals |
| - Make a set of requirements for a personal computer | - Arrange processer, RAM, Hard drive, Graphic card according to the requirement. - Identify requirements for a computer to run windows MS Office and Eclipse IDE |
| - Work on windows Environment and Install the required operating System with Device drivers. | - Install and execute test of required Application Software’s. - Make the Ghost of Hard Disk / Partitions. - Apply the appropriate operation and execution of system as per standard - Perform loading and shutdown of operating system. - Create items (icons, shortcut, folders etc) and modifying taskbar. - Change the wallpaper, screensaver, and resolution. - Check the control panel items (add/remove, time and date, mouse, and create user account.) |

Knowledge & Understanding

- Define Basic purpose of computer

- Describe basic components of computer.

- Describe relevant software according to your work.

- Describe Input and Output devices

- Explain different parts of a functions on present computer

- Explain specification of processor RAM, Hard drive and Graphic cards

- Explain DOS system

- Explain Commands used in Dos system

- Explain type of Installation, Testing and inspection of operating system and their applications

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | Power Cables, |
| 2 | Presentation manuals, Handouts Hardware, equipment |
| 3 | Tool kits and Windows |
| 4 | Relevant Software Disks |
| 5 | Internet |
| 6 | Keyboard, |
| 7 | Printer |
| 8 | CPU |
| 9 | Monitor |
| 10 | Multimedia |
| 11 | Multimedia Screen |
| 12 | DVDs,6,10 etc. |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Arrange processer, RAM, Hard drive, Graphic card according to the requirement

### 0713E&E2 Process Data with MS Office

Overview:

These Competency standard deals with the skills and knowledge required to Use MS Office (MS-Word), Use MS Office (MS-Excel), and useMS-Office (MS-Power Point).

| Competency Units | Performance Criteria |
| --- | --- |
| - Use MS OFFICE (MS-WORD) | - Open the MS Word Screen and its menu. - Create a new document, save it and re-open it from the location and apply spell check & grammar. - Perform Page Formatting (Borders, Character Spacing, Paragraph, Bullets & Numberings and Fonts). - Apply different tool bars like standard, format& drawing tool bars. - Insert pictures, clipart, and shapes. - Use header and footer. - Insert table and also format of table. - Set the page margins, and printing documents. - Implement a lab report in MS word and insert table for measurement |
| - Use MS OFFICE (MS-EXCEL) | - Open the MS EXCEL Screen and its menu. - Create a new excel sheet, save and re-open it from the location and spell check. - Insert and delete row and columns (format of cell). - Apply Merge cell and wrap text command. - Change text size and formats - Check print previews and page setup - Set the page margins, and printing documents. - Make a lab report in MS word - Enter data and Apply formulas in worksheet(Add, Subtract, Multiplying, and Divide & Average) |
| - Use MS- OFFICE (MS-POWER POINT) | - Open PPT - Select page theme and style - Make a presentation on Mechanical technology. - Select Edit & format a text box - Insert pictures & colors to a slide - Make slide show - Apply Hide and unhide a slide - Insert and delete new slide - Insert table and video in slide. - Use different style of Writing. |

Knowledge & Understanding

- Define basic input.

- Explain of basic issues/problems related to rectification of computer operations.

- Explain basic hardware configuration required for software.

- Describe detail knowledge of MS Office functions.

- Explain basic input and out devices and their use.

- Explain knowledge of basic issues/problems related to rectification of computer

- Operations.

- Explain basic hardware configuration required for software.

- Explain detail knowledge of MS Excel functions.

- Explain about the usage of Ready-made templates.

- Describe advance features

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | MS Office |
| 2 | PC |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Create a report using word

### 0713E&E3 Carryout Basic Programming

Overview:

This Competency standard deals with the skills and knowledge required to write a program to perform arithmetic operations, write a program in C++ to preparing logical operation, and write a program to sort a string of numbers.

| Competency Units | Performance Criteria |
| --- | --- |
| - Make a program to perform arithmetic operations | - Install eclipse or other IDE with C++ - Make a program that can take inputs from users and perform arithmetic operations like a calculator - Run the program and verify the results |
| - Make a program in C++ to preparing logical operation | - Make a program that takes two numbers and decides which is bigger and which is smaller - Make a program that takes number of lights, fans in a house and calculate load |
| - Make a program to sort a string of numbers | - Make a script to take 10 numbers as input and display - Make a program to sort numbers in increasing order - Run the program and verify the results |

Knowledge & Understanding

- Define Arithmetic operations

- Explain Arithmetic and logical operations in C++

- Define Loop operations in C++

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | Computer |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Create a program in C++ for loop

### 0713E&E4 Perform Internet Browsing

Overview:

This Competency standard deals with the skills and knowledge required to draw pin diagram of electronic IC using of search engine, Browse data sheets and symbols, Browse manuals

| Competency Units | Performance Criteria |
| --- | --- |
| - Draw pin diagram of electronic IC using of search engine | - Open the Internet browser. - Identify various search engines - Open google.com and carry out searching related to course - Open Yahoo.com and carry out searching related to course - Open Ask.com and carry out searching related to course - Open Wikipedia.com and carry out searching related to course - Identify best search engine |
| CU2.Browse data sheets and symbols | - Search data sheet of various electronic components from the Internet - Search various symbols of electrical components - Search the Electric design software - Prepare a report for the best software and their application |
| CU3.Browse manuals | - Search various manuals available on the internet related to course - Read the key terms - Clear your concepts from the manual |

Knowledge & Understanding

- Define search engine

- Describe different types of search engine.

- Determine frequently use search engine.

- Define different parameters

- Define data sheets.

- Determine symbols

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | Computer |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Elaborate Datasheet.

### 0713E&E5 Perform the Different Engineering Curves Used in Various Mechanism

Overview:

This competency standard covers the skills and knowledge required to Construct inscribe and circumscribe figures, Construct a pentagon, Hexagon and Octagon by circumscribe method, Construct a pentagon, Hexagon and Octagon by inscribe method, Construct a Tangents of circles (Inside & Outside)When the centre of the given circle is known and when the circle of centre is not known, Construct an Ellipse by Concentric Circle Method, Rectangle Method, Oblong Method, Arcs of Circle Method, Rhombus Method and Basic Locus Method, Construct a parabola curve by Rectangle Method, Method of Tangents( Triangle Method) and Basic Locus Method, Construct a hyperbola curve, Construct a Archimedean Spiral curve, Construct a involutes curve of square rectangle hexagon and circle and Construct of cycloid, epicycloids, and hypocycloid.

| Competency Units | Performance Criteria |
| --- | --- |
| - Construct inscribe and circumscribe figures. | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw square, triangle and hexagon according to dimension. |
| - Construct a pentagon, Hexagon and Octagon by circumscribe method. | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw pentagon, Hexagon and Octagon . |
| - Construct a pentagon, Hexagon and Octagon by inscribe method | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw pentagon, Hexagon and Octagon. |
| - Construct a Tangents of circles (Inside &Outside)When the centre of the given circle is known and when the circle of centre is not known. | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw Tangents of circles |
| - Construct an Ellipse by Concentric Circle Method, Rectangle Method, and Oblong Method, Arcs of Circle Method, Rhombus Method and Basic Locus Method. | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw an Ellipse. |
| - Construct a parabola curve by Rectangle Method, Method of Tangents( Triangle Method) and Basic Locus Method | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw a parabola curve. |
| - Construct a hyperbola curve. | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw a hyperbola curve. |
| - Construct a Archimedean Spiral curve | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw spiral curve. |
| - Construct a involutes curve of square rectangle hexagon and circle. | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw involute curve. |
| - Construct of cycloid, epicycloids, and hypocycloid. | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in different equal parts. - Draw the generating circle and the base line equal to the circumference of the generating circle - Divide the circle and the base line in to equal number of parts - Draw by completing the cycloid. |

Knowledge & Understanding

- Describe Techniques of sketching straight lines in different directions

- Define Triangles, Quadrilateral and Polygons

- Describe circular arc using different line method

- Describe circular arc

- Types of Geometric Shape

- Describe Two-dimensional shapes

- Describe Three-dimensional shapes

- Describe Types of Geometric Shape

- Describe Two-dimensional shapes

- Describe Three-dimensional shapes

- Describe Types of Geometric Shape

- Define Regular Polyhedrons

- Describe Methods of drawing Tangents & Normal

- Define ellipse

- Describe different methods of sketching ellipse

- Describe parabola

- Describe different methods of parabola

- Describe hyperbola curve

- Describe different methods of hyperbola curve

- Describe spiral curve

- Describe involute curve

- Describe cycloid

- Describe epicycloids

- Describe hypocycloid

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | Graph and drawing sheet |
| 2 | Drawing Board/Table |
| 3 | Tea-Square |
| 4 | Set Square |
| 5 | Temple |
| 6 | Geometry Box |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Construct a Tangents of circles

### 0713E&E6 Perform Multi-View Drawings

Overview:

This competency standard covers the skills and knowledge required to Sketch Orthographic projection 1st angle, Sketch Orthographic projection 3rd angle, Sketch Oblique Drawing, Construct multi view drawing of Simple Bearing, Construct multi view drawing of Open Bearing, Sketch prism, Sketch cone and Draw pyramid.

| Competency Units | Performance Criteria |
| --- | --- |
| - Sketch Orthographic projection 1st angle. | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in equal parts. - Draw plan view - Draw front view - Draw side view |
| - .Sketch Orthographic projection 3rd angle. | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in equal parts. - Draw plan view - Draw front view - Draw side view |
| - Sketch Oblique Drawing | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in equal parts - Draw the front or side view of the object. |
| - .Construct multi view drawing of Simple Bearing. | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in equal parts. - Draw plan view of simple bearing - Draw front view of simple bearing - Draw side view of simple bearing |
| - .Construct multi view drawing of Open Bearing. | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in equal parts. - Draw plan view of open bearing - Draw front view of open bearing - Draw side view of open bearing |
| - .Sketch prism | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in equal parts. - Sketch prism |
| - .Sketch cone | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in equal parts. - Draw horizontal oval - Draw the two sides of a triangle which meets at a common point |
| - .Draw pyramid | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Divide the sheets in equal parts. - Sketch pyramid |

.

Knowledge & Understanding

- Define Orthographic projection 1st angle.

- Define Orthographic projection 3rd angle.

- Define Oblique Drawing.

- Describe Multi view drawing of Simple Bearing.

- Define Prism, Cone and pyramid

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | Graph and drawing sheet. |
| 2 | Drawing Board/Table. |
| 3 | Tea-Square |
| 4 | Set Square. |
| 5 | Temples. |
| 6 | Geometry Box |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Draw Boundaries lines as per standards

### 0713E&E7 Draw Production Drawing

Overview:

This competency standard covers the skills and knowledge required to draw the assembly drawing of Boiler Stop Valve, draw the views of Connecting Rod in detail and assembly drawing, draw the assembly drawing of Screw Jack and draw the assembly of Tail Stock in full section showing its parts.

| Competency Units | Performance Criteria |
| --- | --- |
| - Draw the assembly drawing of Boiler Stop Valve | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Sketch the 1st angle orthographic projection views - Sketch the 3rd angle orthographic projection views - Sketch Assembly of boiler stop valve |
| CU2.Draw the views of Connecting Rod in detail and assembly drawing | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Sketch the 1st angle orthographic projection views - Sketch the 3rd angle orthographic projection views - Sketch Assembly of connecting rod |
| CU3. Draw the assembly drawing of Screw Jack | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Sketch the 1st angle orthographic projection views - Sketch the 3rd angle orthographic projection views - Sketch Assembly of screw jack |
| CU4. Draw the assembly of Tail Stock in full section showing its parts | - Prepare Drawing sheet. - Select the tools. - Draw Boundaries lines as per standards. - Make title bar - Sketch the 1st angle orthographic projection views - Sketch the 3rd angle orthographic projection views - Sketch Assembly of tail stock |

Knowledge & Understanding

- Identify the parts and material of Boiler Stop Valve

- Explain the parts detail of Screw Jack

- Identify parts of Tail Stock

- Describe the parts of connecting rod

- Describe the parts of boiler stop valve

- Describe the parts of screw jack

- Describe the parts of Tail Stock

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | Plotter |
| 2 | Multimedia Projector |
| 3 | Computer |
| 4 | Scanner |
| 5 | Printer |
| 6 | Computer |
| 7 | Drafting Table |
| 8 | Board |
| 9 | Portable Drawing |
| 10 | Clutch Pencil |
| 11 | Templates |
| 12 | French Curves |
| 13 | Lead Pencil |
| 14 | Set Square |
| 15 | Tee Square |
| 16 | Instruments Box |
| 17 | Erasing Machine (Manual) |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Sketch the 3rd angle orthographic projection views

### 0713E&E8 Design and Simulate the Electronic Circuit by Using Computer Aided Design (CAD) Software

Overview:

This competency standard covers the skills and knowledge required to. Analyse customer requirement and specification. Plan and estimate domestic electric work. Plan and estimate commercial   electric work and Plan and estimate industrial electric work.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1.Analyze customer requirement and specification | - Draw the general value chain of the end user. - Highlight the various stages and set of activities in the value chain drawing - Enlist the electrical appliances/materials required in electrical development process - Identify critical stages in the development - Identify the safety aspect required in the critical stages of the development - Enlist the possible energy efficient appliance/devices and global trends in electrical works - Analyse the client requirement at broad level. - Generate a report of various stages of electrical works. - Proposed appliance/materials as per global trends and clarify the technical specification. |
| CU2. Plan and estimate domestic electric work | - Plan the client requirement at broad level from the proposal. - Plan the electrical appliances/materials / used in different stages of the electrical development process - Plan and estimate electrical wiring - Estimate Installation and Material Cost - Estimate Power consumption for lighting and appliances - Plan a main and sub distribution board. - Ensure safety system. |
| CU2. Plan and estimate commercial electric work. | - Analyze the client requirement at broad level from the proposal. - Plan the electrical equipment /appliances/materials / used in different stages of the commercial electrical work - Plan and estimate electrical wiring - Estimate Installation and Material Cost - Estimate Power consumption for centralize lighting, heating, cooling system and other appliances. - Plan a main and sub distribution board. - Ensure safety measures in development process. - Enlist the material required |
| CU3. Plan and estimate industrial electric work. | - Analyze the client requirement at broad level from the proposal. - Plan the electrical equipment /appliances/materials / used in different stages of the commercial electrical work - Plan and estimate electrical wiring - Estimate Installation and Material Cost - Estimate Power consumption for centralize lighting, heating, cooling system and other appliances. - Plan 3-phase line for heavy loads - Plan a separate control panel box for heavy machinery. - Plan a main and sub distribution board. - Design a safety system for protection of machinery and labor. - Assign color scheme for safety measures |

Knowledge & Understanding

- Define the term ‘value chain’ of industry.

- Define human machine interface (HMI)

- Define the Standard operating procedure (SOP) of the organization for control panel development process.

- How to prepare a General arrangement drawing?

- Define IEC standards

- Describes the standard of electrical components.

- Understanding and knowledge of equipment properties.

- Understanding and knowledge of input and output of tool of electronic.

- Describe the rules and regulation of work.

- Understanding and knowledge of Electrical Specifications component used.

- Describes the standard of electrical components.

- Understanding and knowledge of equipment properties.

- Understanding and knowledge of input and output of tool of electronic.

- Describe  the rules and regulation of work

- Describes the standard of electrical components.

- Understanding and knowledge of equipment properties.

- Understanding and knowledge of input and output of tool of electronic.

- Describe the rules and regulation of work

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | Site Visit |
| 2 | Protective gear such as helmets, goggles, gloves, rubber shoes, etc. |
| 3 | Reference Material |
| 4 | Clipboard |
| 5 | Calculation Sheet |
| 6 | Calculator |
| 7 | Psychometric Charts and Tables |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

Installation techniques

### 0713E&E9 Verify Basic Laws of Electrical Machines

Overview:

This competency standard covers the skills and knowledge required to verify Faraday’s law by moving permanent magnet inside the coil, verify Faraday’s law by moving coil near the magnet field, verify Faraday’s law using relative motion of coil and magnet, verify Faraday’s Law using simple loop generator, verify EMF through induction, verify Torque induce in a current carrying loop and verify Mutual induction.

| Competency Units | Performance Criteria |
| --- | --- |
| - Verify Faraday’s law by moving permanent magnet inside the coil. | - Construct a coil. - Connect Galvanometer with coil. - Move permanent magnet inside the coil fast and slow. - Record the effect of movement of magnet on reading of Galvanometer. - Hold the magnet inside the coil and do not move. - Record the effect on reading of Galvanometer again. |
| - Verify Faraday’s law by moving coil near the magnet field. | - Construct a coil. - Connect Galvanometer with coil. - Fix permanent magnet and move the coil fast and slow on it. - Record the effect of movement of coil on reading of Galvanometer. - Hold the coil near the magnetic field and do not move. - Record the effect on reading of Galvanometer. |
| - Verify Faraday’s law using relative motion of coil and magnet. | - Construct a coil. - Connect Galvanometer with coil. - Make relative motion of coil and magnet. - Record the effect of movement on reading of Galvanometer. |
| - Verify Faraday’s Law using simple loop generator. | - Identify single loop generator and its parts. - Select field winding. - Connect Galvanometer with single loop coil. - Rotate the single loop coil of generator in the field - Measure the voltage induce in loop generator. - Record the effect of movement on reading of Galvanometer. |
| - Verify Torque induce in a current carrying loop | - Construct a coil. - Apply DC voltage to this coil. - Place a current carrying loop in this coil. - Check the direction of force to verify the torque produce. |
| - Verify Mutual induction | - Construct 2 coils on two different legs of single core. - Apply AC voltage to the any one coil. - Check the volts induce in other coil by voltmeter. |

Knowledge & Understanding

- What is Faraday’s first law of Electro-Magnetic Induction?

- What is second law of Faraday?

- How can we find the direction of induced EMF?

- What is the role of magnetic strength in Faraday’s Law?

- What is the role of conductor’s length or turns in Faraday’s Law?

- What is Lenz’s Law

- What is torque?

- Why force produce on current carrying conductor?

- What is core?

- What is induction?

- What is meaning of coupled coils?

- Why EMF induced in 2nd coil, when voltage is applied to 1st coil?

- What is mutual induction?

- Which machine work on the principle of mutual induction?

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | Coil |
| 2 | Galvanometer. |
| 3 | Voltmeter |
| 4 | Transformer |
| 5 | Connecting leads. |
| 6 | Magnet. |
| 9 | DC source |
| 10 | Rheostat |
| 12 | Loop |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

Verify Faraday’s law by moving coil near the magnet field.

### 0713E&E10 Analyze Dc Generators

Overview:

This competency standard identifies the competencies required to calculate different terms used in armature winding of DC machines, develop Single Layer Lap Winding, develop Double Layer Wave Winding, measure the resistance of field winding, armature winding and terminal polarity, identify DC shunt generator, identify DC series generator and identify DC compound generator.

| Competency Units | Performance Criteria |
| --- | --- |
| - Calculate different terms used in armature winding of DC machines | - Count the number of slots (S) of armature. - Calculate pole pitch (Y). - Calculate Coil span/pitch. - Calculate Back pitch (Yb) - Calculate Front pitch (Yf) - Calculate Resultant Pitch (YR) - Calculate commentator pitch (Yc) |
| - Develop Single Layer Lap Winding | - Count the number of slot in armature core and numbering on it. - Calculate total number of conductors (Z) - Calculate average pitch, back pitch and commentator pitch. - Start winding from slot no 1. - Add the value of back pitch in conductor. - Connect conductor 1 from back side to the added value. - Subtract the value of back pitch from the added value. - Connect added value from front side to subtracted value. - Continue this process till completion of all conductors. - Connect armature segment with armature conductor. - Insert brushes in proper position. |
| - Develop Double Layer Wave Winding | - Count the number of slot in armature core and numbering on it. - Calculate total number of conductors (Z) - Calculate average pitch, back pitch and commentator pitch. - Start winding from slot no 1. - Add the value of back pitch in conductor. - Connect conductor 1 from back side to the added value. - Add the value of front pitch in previous added value. - Continue this process till completion of all conductors. - Connect armature segment with armature conductor. - Insert brushes in proper position. |
| - Measure the resistance of field winding, armature winding and terminal polarity | - Open the terminal box. - Connect the ohm meter with the terminal of both winding. - Read down ohm meter reading. - Connect the probe of galvanometer with any terminal and touch other probe to other terminals one by one. - Rotate the generator slowly by hand. - Touch the probe of galvanometer with each winding. - Identify armature with the deflection of galvanometer pointer. - Mark the polarity of terminals with the deflection of galvanometer. |
| - Identify DC shunt generator | - Measure the terminal resistance of generator with ohm meter. - Identify DC shunt generator from measured value of resistance. - Disconnect both windings. - Identify the shunt field winding of shunt generator. - Identify armature of shunt generator. - Measure the resistance of both winding separately. - Record the resistance of armature winding and field winding. - Compare the resistance of armature winding and field winding to identify the difference between them. |
| - Identify DC series generator | - Measure the terminal resistance of generator with ohm meter. - Identify DC series generator from measured value of resistance. - Disconnect both windings. - Identify the series field winding of series generator. - Identify armature of series generator. - Measure the resistance of both winding separately. - Record the resistance of armature winding and field winding. - Compare the resistance of armature winding and field winding to identify the difference between them. |
| - Identify DC compound generator | - Measure the terminal resistance of generator with ohm meter. - Identify DC Compound generator from measured value of resistance. - Disconnect both windings. - Identify the series field winding of generator. - Identify the shunt field winding of generator. - Identify armature of shunt generator. - Measure and record the resistance of both winding separately. - Measure and Record the resistance of armature winding. - Compare the resistance of armature winding, series field winding and shunt field winding to identify the difference between them. |

Knowledge and understanding

- Explain the term armature slots.

- What is pole pitch (Y)?

- What is coil span/pitch?

- What is Back pitch (Yb)

- What is Front pitch (Yf)

- What is Resultant Pitch (YR)

- What is commutation pitch (Yc)

- What is lap winding?

- How many parallel paths in lap winding?

- What is mean by single layer winding?

- What is wave winding?

- How many parallel paths in wave winding.

- What is mean by double layer winding?

- How to read the scale of galvanometer?

- What is difference between field winding and armature winding?

- Which winding having low resistance and why?

- Explain the technique to identify the terminal polarity

- How many winding in shunt generator?

- How to disconnect both winding of shunt generator

- How many winding in series generator?

- How to disconnect both winding of series generator.

- Which conditions you follow at the time of reconnection of both winding.

- Explain faraday’s law.

- Explain rules of generators.

- Define the Type of DC machines

- Explain the principle of simple loop generator.

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | Connecting wires |
| 2 | Galvanometer |
| 3 | Ohm meter |
| 4 | DC machines |
| 18 | Soldering wire |
| 19 | Soldering iron |
| 20 | Sleeve |
| 22 | Winding machine |
| 23 | Insolating paper |
| 24 | Carbon brushes |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

Measure the resistance of field winding, armature winding and terminal polarity.

### 0713E&E11 Perform Tests on DC Generators

Overview

This competency standard identifies the competencies required to Plot the Open circuit characteristics of a Separately excited D.C  generator, plot the Open circuit characteristics of a D.C shunt generator, plot the load characteristics of D.C shunt generator, plot the load characteristics of D.C series generator, plot the load characteristics of D.C compound generator for under, flat and over compounding, perform parallel operation of two DC shunt generators and determine copper, iron & friction losses of shunt generator by actual loading.

| Competency Units | Performance Criteria |
| --- | --- |
| - Plot the Open circuit characteristics of a Separately excited D.C generator | - Connect DC supply to field coil through rheostat and ampere meter. - Rotate armature at constant speed. - Connect voltmeter across armature terminal. - Decrease the resistance of rheostat in steps and measure and record the relevant output voltage of armature and field current. - Repeat the P2 to P4 till voltage of armature reach at its saturation point. - Plot the graph between field current and armature voltage from the recorded values. |
| - Plot the Open circuit characteristics of a D.C shunt generator | - Connect DC supply to field coil through rheostat and ampere meter. - Rotate armature at constant speed. - Connect voltmeter across armature terminal. - Decrease the resistance of rheostat in steps and measure and record the relevant output voltage of armature and field current. - Repeat the P2 to P4 till voltage of armature reach at its saturation point. - Plot the graph between field current and armature voltage from the recorded values. |
| - Plot the load characteristics of D.C shunt generator. | - Select DC shunt generator and connect variable load. - Connect ampere meter and voltmeter across the load. - Rotate armature at constant speed and note the readings of ampere meter and voltmeter. - Increase the load in steps till specific reduce in terminal voltage observed. - Plot the graph between load current and terminal voltage from the recorded values. |
| - Plot the load characteristics of D.C series generator. | - Select DC series generator and connect variable load. - Connect ampere meter and voltmeter across the load. - Rotate armature at constant speed and note the readings of ampere meter and voltmeter. - Increase the load in steps till specific reduce in terminal voltage observed. - Plot the graph between load current and terminal voltage from the recorded values. |
| - Plot the load characteristics of D.C compound generator for under, flat and over compounding | - Select DC compound generator and connect variable load. - Connect ampere meter and voltmeter across the load. - Rotate armature at constant speed and note the readings of ampere meter and voltmeter. - Adjust field diverter on zero ohm. - Increase the load in steps till specific reduce in terminal voltage observed. - Plot the graph between load current and terminal voltage from the recorded values. - Such off the load. - Adjust the field diverter at such position that the terminal voltages are equal to no load voltages. - Increase the load in steps till specific reduce in terminal voltage observed. - Plot the graph between load current and terminal voltage from the recorded values. - Such off the load. - Adjust the field diverter at such position that the terminal voltages are greater than no load voltages. - Increase the load in steps till specific reduce in terminal voltage observed. - Plot the graph between load current and terminal voltage from the recorded values. |
| - Perform parallel operation of two DC shunt generators. | - Connect the circuit according to the circuit diagram. - Place all switches in off position. - Run generator 1 at rated speed. - Reduce field resistance of generator 1 - Connect generator with bus bar after attaining rated voltage. - Run generator 2 with prime mover. - Reduce the field resistance of generator 2 to attain its rated voltage. - Close the switches for parallel operation. - Calculate and record the reading of voltage and current on different loads. |
| - Determine copper, iron & friction losses of shunt generator by actual loading. | - Connect ammeter in series with both winding. - Connect voltmeter in parallel with both winding. - Apply 10-20 DC volt. - Record the readings of voltmeter and ammeter. - Calculate Ra and Rsh. - Run the generator at rated speed with prime mover. - Turn on the load switch and record the meter reading. - Calculate input power and output power. - Calculate total losses (Copper loss, Friction loss, Iron loss) |

Knowledge and understanding

- What is different between shunt and separately excited generator?

- What is rheostat?

- How to use rheostat with field winding?

- Explain the effect of rheostat resistance on terminal voltage.

- What is core saturation?

- Why the graph between field current and armature voltage is not a straight line?

- What is armature reaction?

- Explain the reason of voltage reduce due to increase in load.

- How to compensate armature reaction?

- How to connect field and armature winding?

- How to connect field and armature winding in compound generator?

- Explain under compounding.

- Explain flat compounding.

- Explain over compounding.

- Explain the conditions of paralleling of DC generators.

- How to connect field and armature winding in shunt generator?

- Why we connect DC generators in parallel.

- What is iron loss?

- What is copper loss?

- Why we find copper loss at full load.

- Which loss having maximum value and why?

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | DC generators |
| 2 | Ampere meters |
| 3 | Voltmeters |
| 4 | Connecting wires |
| 5 | Rheostat |
| 6 | Tachometer |
| 7 | DC source |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

Determine copper, iron & friction losses of shunt generator by actual loading.

### 0713E&E12 Analyze Dc Motors.

Overview

This competency standard identifies the competencies required to identify DC shunt motor, identify DC series motor and identify DC compound motor.

| Competency Units | Performance Criteria |
| --- | --- |
| - Identify DC shunt motor | - Measure the terminal resistance of motors with ohm meter. - Identify DC shunt motor from measured value of resistance. - Disconnect both windings. - Identify field winding of shunt motor. - Identify armature of shunt motor. - Measure the resistance of both winding separately. - Record the resistance of armature winding and field winding. - Compare the resistance of armature winding and field winding to identify the difference between them. |
| - Identify DC series motor | - Measure the terminal resistance of DC motors with ohm meter. - Identify DC series motor from measured value of resistance. - Disconnect both windings. - Identify the series field winding of series motor. - Identify armature of series motor. - Measure the resistance of both winding separately. - Record the resistance of armature winding and field winding. - Compare the resistance of armature winding and field winding to identify the difference between them. |
| - Identify DC compound motor | - Measure the terminal resistance of DC motor with ohm meter. - Identify DC Compound motor from measured value of resistance. - Disconnect both windings. - Identify the series field winding of motor. - Identify the shunt field winding of motor. - Identify armature of compound motor. - Measure and record the resistance of both winding separately. - Record the resistance of armature winding. - Compare the resistance of armature winding, series field winding and shunt field winding to identify the difference between them. |

Knowledge and understanding

- How many winding in shunt motor?

- Explain the technique to identify the terminal polarity.

- How to disconnect both winding of shunt motor?

- How many winding in series motor?

- What is different between field winding and armature winding?

- Which winding having low resistance and why?

- How to disconnect both winding of series generator.

- Which condition you follow at the time of reconnection of both winding?

- Define the Type of DC machines

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | DC compound motor |
| 2 | Ohm meter |
| 3 | Connecting wires |
| 6 | DC series motor |
| 7 | DC shunt motor |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

Identify DC compound motor

### 0713E&E13 Perform Tests on DC Motors.

Overview

This competency standard identifies the competencies required to Plot the load characteristics of D.C shunt motor, plot the load characteristics of D.C series motor, plot the load characteristics of D.C compound motor, control the speed of DC series motor, control the speed of DC shunt motor, start  DC shunt motor with 3 point starter, identify the effect of back EMF in DC motor, calculate BHP of motor by Brake test, calculate efficiency of the motor by Swinburne Test and perform Regenerative / Hopkinson/ Back to back Test.

| Competency Units | Performance Criteria |
| --- | --- |
| - Plot the load characteristics of D.C shunt motor | - Couple electro dynamo meter to the DC motor with belt - Connect shunt motor according to the diagram. - Set the dynamo meter control knob in such position to produce a minimum starting load. - Turn on the power supply - Run the motor at rated RPM. - Apply a load to DC motor by varying the dynamo meter control knob. - Measure and calculate the current, speed and torque. - Plot the load characteristics on DC shunt motor.. |
| - Plot the load characteristics of D.C series motor. | - Select DC series motor and achieve connection as per diagram. - Turn on DC power supply and increase the load to full value. - Record the value of speed and current on each load. - Draw the graph between load current and speed |
| - Plot the load characteristics of D.C compound motor | - Couple electro dynamo meter to the DC motor with belt - Connect compound motor according to the diagram. - Set the dynamo meter control knob in such position to produce a minimum starting load. - Turn on the power supply - Run the motor at rated RPM. - Apply a load to DC motor by varying the dynamo meter control knob. - Measure and calculate the current, speed and torque. - Plot the load characteristics on DC compound motor.. |
| - Control the speed of DC series motor. | - Make the connection as per circuit diagram. - Insert external resistances in series to armature and field coil. - Switch on the supply and increase the voltage gradually to its rated voltage. - Record speed at different supply voltages. - Make graph between speed and applied voltage. |
| - Control the speed of DC shunt motor. | - Make the connection as per circuit diagram. - Switch on the supply and increase the voltage gradually to its rated voltage. - Record speed at different field current. - Make graph between speed and field current - Fixed the field current and very armature current - Record the speed at different armature current. - Make graph between speed and armature current |
| - Start DC shunt motor with 3 point starter. | - Make the connection as per circuit diagram. - Switch on the supply and move starter handle from off position to position 1. - Move the starter arm from position 1 to position 2 and continue this process till starter arm reach at its extreme position. |
| - Identify the effect of back EMF in DC motor. | - Select DC series motor and make connection as per diagram. - Turn on DC power supply and increase the load 0 to full value. - Record the value of speed and current on each load. - Calculate back EMF by using the recoded values and appropriate formula. - Make graph between speed and back EMF and interpret the result. |
| - Calculate Brake Horse Power (BHP) of motor by Brake test. | - Connect the spring balance with one end of the rope. - Connect other end of rope with weight (W1). - Measure the radius of the pulley. - Hang the weight on the pulley of the motor and fix the spring balance end with earth. - Run the motor. - Measured the RPM of the motor. - Adjust the weight to pass full load current from motor. - Calculate spring balance weight and hanged weight (W1) - Calculate shaft torque. - Calculate BHP of motor by shaft torque. |
| - Calculate efficiency of the motor by Swinburne Test | - Perform DC test to find the value of armature resistance and field resistance - Run the motor at rated voltage without load. - Adjust the value of shunt regulator to attain rated speed. - Measure Field current and No load current with ampere meter. - Calculate total copper losses and iron losses. - Calculate input and output power. - Calculate efficiency from calculated power. |
| - Perform Regenerative / Hopkinson/ Back to back Test | - Identify two shunt machines. - Couple the shafts of both machines. - Connect the machines electrically in such a way that one machine work as a motor and other as a generator. - Connect external power supply to the machines to overcome the machine losses. - Run the both machine at no load. - Reduce the field excitation of one machine which work as a motor. - Increase the field excitation of one machine which work as a generator. - Adjust the field winding so that voltage of generator becomes equal to the applied voltage which result the machines runs in parallel - Calculate the efficiency by using reading of voltmeter and ammeter. |

Knowledge and understanding

- How many winding in compound generator?

- Why series winding having low turns.

- Explain the effect of field current changes on motor speed.

- Explain the effect of load changes on motor speed.

- Why we never start DC series motor without load.

- Why series winding having low turns and shunt winding having high number of turns?

- What is the different between shunt, series and compound motor load characteristics?

- Explain speed control of DC motor with applied voltage.

- Why we control the speed of DC motor.

- How many techniques to control the speed of DC shunt motor?

- What is 3-point starter?

- Why we never start DC shunt motor without starter.

- Explain generator action in motor.

- What is back EMF?

- Explain the effect of motor speed on back EMF

- How to measure the RPM of the motor?

- What is shaft torque?

- How to calculate BHP from shaft torque?

- What is iron loss?

- What is copper loss?

- Why we find copper loss at full load.

- Which loss having maximum value and why.

- Why we connect external source with motor generator set.

- What is mean by field excitation?

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | DC shunt motor |
| 2 | Tachometer |
| 3 | Voltmeters |
| 4 | Ammeters |
| 5 | Connecting wire |
| 6 | DC shunt generator |
| 12 | Spring balance |
| 17 | Rope |
| 22 | DC series motor |
| 25 | Belt |
| 26 | Electro dynamo meter |
| 36 | Ohm mete |
| 38 | DC compound motor |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

Calculate efficiency of the motor by Swinburne Test.

### 0713E&E14 Repair/ Maintenance and Service of DC Motors and DC Generators.

Overview

This Competency Standard identifies the competencies required to. Diagnose Fault in DC Motor, carry out Service/Repair in Electric Motor, and carry out Preventive Service / Maintenance of Generators and Diagnose and Repair Electric Generator carry out repair and maintenance of DC motors and generators in accordance with the manufacturer’s instructions. The underpinning knowledge regarding repair and maintenance of electrical DC motor & generators will be sufficient to provide the basis for your work.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1. Diagnose Fault in DC Motor | - Identify the type of electric motor from data / name plate or manual and motor ratting. - Perform Methods of dismantling and assembling DC motor - Inspect visual mechanical defect such as, tight bearings, bent shaft, noisy running, short circuit, and temperatures. - Check the motor by using specified test instruments to detect electrical defects such as loose/or burnt electrical connections, defective capacitors, burnt windings, low insulation resistance. - Test short circuit, circuit break, earth fault, wiring fault, bearing fault. - Perform replacing of bearing - Identify the faults of a DC motor and repair them according to procedures. - Rewind a DC motor - Check the tripping of protective device using specified test instruments, while the motor is running with power supply connected with suitable control and protective device, the difficulty in starting/low RPM/ unusual noises/ excessive heat/ / grounded winding etc. |
| CU2. Carry out Service/Repair in Electric Motor | - Repair the motor as per diagnosed fault - Assemble the electric motor for internal tests/servicing/repairs according to manufacturer’s instructions - Clean the parts of the motor with specified cleaning agents/tools & material - Check the windings insulation resistance with insulation tester - Rewind the motor as per requirement - Check the insulation resistance of repaired motor as per standard - Carry out the No load and load tests as per standard practices |
| CU3. Carry out Preventive Service/ Maintenance of Generators | - Carry out routine maintenance of the generator as specified in the manufacturer’s manual - Check out the switch/ timer, relay, protective device and wire connection - Update the maintenance/service records |
| CU4. Diagnose and Repair Electric Generator | - Dismantle the electric Generator for internal tests/servicing/repairs according to manufacturer’s instructions - Clean the parts of the generator with specified cleaning agents/tools & material - Check the control equipment of generators. - Find out the root cause of fault according to the signs of fault by applying the working principles of control equipment of generators: Speed controller, Magnetic field control circuit and equipment of the generator. - Check Load regulator of the generator and its control circuit. - Control circuit of the auxiliary generator. - Check Control circuit of the battery unit - Check Circuit breaker and relay. - Repair the fault after finding out the faulty equipment or component - Check the windings insulation resistance with insulation tester as per standard - Repair the generator as per diagnosed fault. - Carry out the No load and load tests as per standard practices |

Knowledge & Understanding

- Explain type and construction and working principal of DC motors, DC Series Motor, DC shunt Motor, D.C. compound motor,

- Discuss working principal of D.C generator.

- What is dis-assembling & re-assembling of electric motors.

- Discuss common faults found in electric motors/generators and troubleshooting methods

- Explain the method preventive maintenance on motors and stand by generators.

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | Voltmeters |
| 2 | Wire Striper |
| 3 | Vernier Caliper |
| 5 | Thermometer |
| 7 | Test lamp |
| 8 | Soldering wire |
| 10 | DC Motors |
| 11 | Soldering Iron |
| 12 | Set of Nose Plier |
| 13 | Set of Screw Drivers |
| 16 | DC generators |
| 19 | Tachometer |
| 20 | Ring Spanner Set |
| 21 | Philips Screw drivers Set |
| 22 | Phase Sequence Meter |
| 24 | Medgar (Insulation Tester) |
| 25 | Hammer |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Troubleshooting of starting system of DC motor and generator

- Replacement of bearings of motor and generator

### 0713E&E15 Verify Basic Laws of Electrical Machines

Overview:

This competency standard covers the skills and knowledge required to verify Faraday’s law by moving permanent magnet inside the coil, verify Faraday’s law by moving coil near the magnet field, verify Faraday’s law using relative motion of coil and magnet, verify Faraday’s Law using simple loop generator, verify EMF through induction, verify Torque induce in a current carrying loop and verify Mutual induction.

| Competency Units | Performance Criteria |
| --- | --- |
| - Verify Faraday’s law by moving permanent magnet inside the coil. | - Construct a coil. - Connect Galvanometer with coil. - Move permanent magnet inside the coil fast and slow. - Record the effect of movement of magnet on reading of Galvanometer. - Hold the magnet inside the coil and do not move. - Record the effect on reading of Galvanometer again. |
| - Verify Faraday’s law by moving coil near the magnet field. | - Construct a coil. - Connect Galvanometer with coil. - Fix permanent magnet and move the coil fast and slow on it. - Record the effect of movement of coil on reading of Galvanometer. - Hold the coil near the magnetic field and do not move. - Record the effect on reading of Galvanometer. |
| - Verify Faraday’s law using relative motion of coil and magnet. | - Construct a coil. - Connect Galvanometer with coil. - Make relative motion of coil and magnet. - Record the effect of movement on reading of Galvanometer. |
| - Verify Faraday’s Law using simple loop generator. | - Identify single loop generator and its parts. - Select field winding. - Connect Galvanometer with single loop coil. - Rotate the single loop coil of generator in the field - Measure the voltage induce in loop generator. - Record the effect of movement on reading of Galvanometer. |
| - Verify Torque induce in a current carrying loop | - Construct a coil. - Apply DC voltage to this coil. - Place a current carrying loop in this coil. - Check the direction of force to verify the torque produce. |
| - Verify Mutual induction | - Construct 2 coils on two different legs of single core. - Apply AC voltage to the any one coil. - Check the volts induce in other coil by voltmeter. |

Knowledge & Understanding

- What is Faraday’s first law of Electro-Magnetic Induction?

- What is second law of Faraday?

- How can we find the direction of induced EMF?

- What is the role of magnetic strength in Faraday’s Law?

- What is the role of conductor’s length or turns in Faraday’s Law?

- What is Lenz’s Law

- What is torque?

- Why force produce on current carrying conductor?

- What is core?

- What is induction?

- What is meaning of coupled coils?

- Why EMF induced in 2nd coil, when voltage is applied to 1st coil?

- What is mutual induction?

- Which machine work on the principle of mutual induction?

Tools and Equipment

.

| SN | Tools |
| --- | --- |
| 1 | Coil |
| 2 | Galvanometer. |
| 3 | Voltmeter |
| 4 | Transformer |
| 5 | Connecting leads. |
| 6 | Magnet. |
| 9 | DC source |
| 10 | Rheostat |
| 12 | Loop |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

### 0713E&E16 Operate Single Phase Transformer

Overview:

This competency standard identifies the competencies required to identify types of transformer, Measure transformer ratio of single phase transformer, make transformer winding, determine voltage regulation of single phase transformer with different load and perform parallel operation of single phase transformer.

| Competency Units | Performance Criteria |
| --- | --- |
| - Identify types of transformer. | - Identify no of windings on primary and secondary side of transformer. - Identify the types of transformer cores. - Identify connection of the windings. - Separate single phase, three phase and auto transformer. |
| - Measure transformation ratio of single phase transformer. | - Connect volt meters on both side of the transformer. - Apply rated voltage to the primary. - Note the volt meter reading. - Calculate transformation ratio using ratio formula. - Repeat P1 to P4 for different secondary tapping. |
| - Make transformer winding | - Design the core of transformer according to the transformer rating. - Select the wire for primary and secondary winding. - Perform winding according to require turns with the help of winding machine. - Cover the winding with insulating paper. - Adjust the core pieces into the winding. - Remove the insulation of enameled wire. - Cover the ends of wire with sleeve. - Perform continuity test with ohm meter. |
| - Determine voltage regulation of single phase transformer with different load. | - Connect watt meter, ampere meter, and volt meter to the primary side of transformer. - Connect the voltmeter to secondary side of transformer. - Connect resistive load to secondary side. - Apply rated voltage to primary side of transformer. - Calculate voltage drop and voltage regulations (%) from meter readings. - Repeat from P1 to P5 for inductive load. - Repeat from P1 to P5 for capacitive load. |
| - Perform parallel operation of single phase transformer. | - Identify two transformer having same rated voltage, frequency and transformation ratio. - Mark the terminals with the help of polarity test. - Connect secondary sides of both transformers in parallel. - Connect secondary sides of both transformers with load bus bar having ampere meters between them. - Connect the primary side of both transformers in parallel. - Connect primary side of transformer with generation bus bar. - Adjust the ammeter readings to zero by adding inductor and resistor in series with secondary side of the transformer. |

Knowledge and understanding

- Define primary and secondary windings of transformer.

- Explain the types of transformer cores.

- Explain connection of the windings.

- Differentiate single phase, three phase, and auto and instrument transformer.

- How to connect volt meters on both side of the transformer.

- How to apply voltage to the primary?

- Explain transformer ratio.

- How to select the wire for primary and secondary winding?

- Explain winding procedure.

- Explain winding insulation with insulating paper.

- How to adjust the core pieces into the winding?

- How to remove the insulation of enameled wire?

- What is sleeve?

- Explain continuity test with ohm meter.

- Explain voltage regulation of the transformer.

- Explain the word voltage drop

- What is transformation ratio?

- Explain polarity test of transformer.

- Describe the conditions for paralleling of the transformer.

- What is indicator?

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | Winding wire |
| 2 | Winding machine |
| 3 | Ohmmeter |
| 4 | Sleeve |
| 5 | Hammer |
| 6 | Insulating pap |
| 7 | Single phase transformer |
| 8 | Voltmeters |
| 9 | Connecting wires |
| 12 | Core |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

### 0713E&E17 Perform Tests on Single Phase Transformer

Overview:

This competency standard identifies the competencies required to perform polarity test of single phase transformer, Perform Open circuit test of transformer, Perform Short Circuit Test of Transformer, calculate efficiency of transformer by direct method and calculate efficiency of transformer by Back to Back test.

| Competency Units | Performance Criteria |
| --- | --- |
| - Perform polarity test of single phase transformer. | - Connect the circuit according to the diagram. - Note the reading of both volt meter. - Identify additive and subtractive polarity. - Mark the terminal according to the additive and subtractive polarity. |
| - Perform Open circuit test of transformer. | - Connect watt meter, ampere meter, and volt meter to the lower voltage side of transformer. - Open the high voltage side of transformer. - Apply rated voltage to lower voltage side of transformer. - Read the instrument/meters reading. - Calculate iron losses and excitation branch component (Ro, Xo). |
| - Perform Short circuit Test of Transformer. | - Connect watt meter, ampere meter, and volt meter to the higher voltage side of transformer. - Connect lower voltage side terminal with each other (Short Circuit) - Apply 10 % to 15 % of rated voltage to higher voltage side of transformer. - Read the instrument/meters readings. - Calculate copper losses and winding component (Ro1, Xo1) |
| - Calculate efficiency of transformer by direct method. | - Perform open circuit test. - Perform short circuit test. - Calculate iron losses and copper losses from above tests. - Calculate input power and output power. - Calculate efficiency from calculated powers. |
| - Calculate efficiency of transformer by Back to Back test. | - Identify two transformer of same rating. - Connect both transformer in parallel. - Connect another variable voltage transformer to secondary side of parallel transformers. - Connect a switch, watt meter and ammeter between variable voltage transformer and parallel transformer. - Connect voltmeter and watt meter to primary side of transformer parallel transformer. - Apply rated voltage to the primary side of the parallel transformers. - Open the switch. - Calculate iron losses. - Close the switch - Adjust the voltage of variable voltage transformer until the full load current passes from secondary side of parallel transformers. - Calculate copper losses. - Calculate efficiency from above calculated losses. |

Knowledge and understanding

- Differentiate between additive and subtractive polarity.

- How to use watt meter?

- What is iron loss?

- How many components of excitation branch?

- Why we connect watt meter, ampere meter and volt meter to the higher voltage side of transformer in short circuit test.

- Why we short Circuit the low voltage side of the transformer in short circuit test.

- Explain the reason to apply 10 % to 15 % of rated voltage to higher voltage side of transformer.

- What is copper loss?

- How many components of transformer winding?

- Why we perform open circuit test in transformer?

- Why we perform short circuit test in transformer.

- What is iron loss?

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | Voltmeters |
| 2 | Wattmeter |
| 3 | Ohmmeters |
| 4 | connecting wires |
| 5 | Ammeters |
| 7 | Single phase transformer |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

### 0713E&E18 Operate Auto Transformer

Overview:

This competency standard identifies the competencies required to identify the connection of auto transformer, Calculate Voltage ratio of Auto Transformer and calculate Current ratio of Auto Transformer.

| Competency Units | Performance Criteria |
| --- | --- |
| - Identify the connection of auto transformer | - Connect voltmeters to the both side of the transformer. - Apply rated voltage to the primary side. - Identify the lower voltage and higher voltage side with the help of volt meter reading. - Identify step up and step down transformer. |
| - Calculate Voltage ratio of Auto Transformer. | - Connect voltmeters to the both side of the transformer. - Apply rated voltage to the primary side. - Read the voltmeters reading. - Calculate voltage ratio with the help of voltmeters reading. |
| - Calculate Current ratio of Auto Transformer. | - Connect ammeters to the both side of the transformer. - Connect a resistive load to secondary side of the transformer. - Apply rated voltage to the primary side. - Read the ammeters reading. - Calculate current ratio with the help of ammeters reading. |

Knowledge and understanding

- What is the different between auto transformer and ordinary transformer?

- How to identify step up and step down transformer.

- Explain voltage ratio of auto transformer.

- Explain the connection of ammeter and volt meter

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | Voltmeter |
| 2 | Auto transformer |
| 3 | Ohmmeter |
| 4 | connecting wires |
| 5 | Ammeter |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

### 0713E&E19 Perform Tests on Auto Transformer.

Overview:

This competency standard identifies the competencies required to Perform open circuit Test of auto transformer, perform short circuit test of auto transformer and calculate efficiency of auto transformer.

| Competency Units | Performance Criteria |
| --- | --- |
| - Perform open circuit Test of auto transformer. | - Connect watt meter, ampere meter, and volt meter to the lower voltage side of transformer. - Open the high voltage side of transformer. - Apply rated voltage to lower voltage side of transformer. - Read the instrument/meters reading. - Calculate iron losses and excitation branch component (Ro, Xo). |
| - Perform short circuit test of auto transformer | - Connect watt meter, ampere meter, and volt meter to the higher voltage side of transformer. - Connect lower voltage side terminal with each other (Short Circuit) - Apply 10 % to 15 % of rated voltage to higher voltage side of transformer. - Read the instrument/meters readings. - Calculate copper losses and winding component (Ro1, Xo1) |
| CU3. Calculate efficiency of auto transformer. | - Perform open circuit test. - Perform short circuit test. - Calculate iron losses and copper losses from above tests. - Calculate input power and output power. - Calculate efficiency from calculated powers. |

Knowledge and understanding

- How to use watt meter?

- Why we connect wattmeter, ampere meter and volt meter to the lower voltage side of transformer.

- Why we open high voltage side of transformer instead of low voltage side.

- How many components of excitation branch

- Why we connect watt meter, ampere meter, and volt meter to the higher voltage side of transformer in short circuit test.

- Why we short Circuit the low voltage side of the transformer in short circuit test.

- Explain the reason to apply 10 % to 15 % of rated voltage to higher voltage side of transformer.

- What is copper loss?

- How many components of transformer winding.

- Why we perform open circuit test in transformer.

- Why we perform short circuit test in transformer.

- What is iron loss?

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | Voltmeters |
| 2 | Wattmeter |
|  | Ammeters |
| 3 | Ohmmeters |
| 4 | connecting wires |
| 5 | Auto transformer |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

### 0713E&E20 Operate Three Phase Transformer

Overview:

This competency standard identifies the competencies required to Make Three Phase Transformer Winding, Perform Star to Star Connection of Three Phase Transformer, Perform Star to Delta Connection of Three Phase Transformer, Perform Delta to Delta Connection of Three Phase Transformer, Perform Delta to Star Connection of Three Phase Transformer, Connect Two Single Phase Transformers in Open Delta and perform Parallel operation of 3 phase transformers.

| Competency Units |  | Performance Criteria |
| --- | --- | --- |
| - Make Three Phase Transformer Winding. | - Design the core of transformer according to the transformer rating. - Select the connection of primary and secondary. - Select the wire for primary and secondary winding. - Perform winding according to require turns with the help of winding machine. - Cover the winding with insulating paper. - Adjust the core pieces into the winding. - Remove the insulation of enameled wire. - Cover the ends of wire with sleeve. - Connect the winding terminals in star or delta. - Perform continuity test with ohm meter. |  |
| - Perform Star To Star Connection Of Three Phase Transformer. | - Select three same rated single phase transformers. - Mark the polarity of the winding terminals. - Connect primary terminals and secondary terminals in star. - Connect voltmeters with primary and secondary. - Apply rated voltage to the primary. - Calculate and Verify line voltage and phase voltage |  |
| - Perform Star To Delta Connection Of Three Phase Transformer. | - Select three same rated single phase transformers. - Mark the polarity of the winding terminals. - Connect primary terminals in star and secondary terminals in delta. - Connect voltmeters with primary and secondary. - Apply rated voltage to the primary. - Calculate and Verify line voltage and phase voltage |  |
| - Perform Delta To Delta Connection Of Three Phase Transformer | - Select three same rated single phase transformers. - Mark the polarity of the winding terminals. - Connect primary terminals and secondary terminals in delta. - Connect voltmeters with primary and secondary. - Apply rated voltage to the primary. - Calculate and Verify line voltage and phase voltage |  |
| - Perform Delta To Star Connection Of Three Phase Transformer | - Select three same rated single phase transformers. - Mark the polarity of the winding terminals. - Connect primary terminals in Delta and secondary terminals in Star. - Connect voltmeters with primary and secondary. - Apply rated voltage to the primary. - Calculate and Verify line voltage and phase voltage. |  |
| - Connect Two Single Phase Transformers In Open Delta. | - Select two same rated single phase transformers. - Mark the polarity of the winding terminals. - Connect opposite terminals of primary winding of both transformers - Take out connected terminals and other two terminals. - Repeat P2 to P4 of CU6 for secondary side of transformer. - Connect voltmeters with primary and secondary sides of transformers. - Apply rated voltage to the primary. - Calculate and Verify line voltage and phase voltage. |  |
| - Perform Parallel operation of 3 phase transformers. | - Identify two three phase transformer having same rated voltage, frequency and transformation ratio. - Connect the primary of each transformer in star connection. - Connect the secondary of each transformer in Delta connection. - Connect secondary sides of both transformers in parallel. - Connect secondary sides of both transformers with load bus bar having indicator between them. - Connect the primary side of both transformers in parallel. - Connect primary side of transformer with generation bus bar. - Adjust the series resistance and series inductance to off the indicator. |  |

Knowledge and understanding

- How to select the wire for primary and secondary winding?

- Explain winding procedure.

- Explain winding insulation with insulating paper.

- How to adjust the core pieces into the winding?

- How to remove the insulation of enameled wire?

- What is sleeve?

- What is star connection?

- What is delta connection?

- Explain continuity test with ohm meter.

- What is star to star connection?

- How to mark the polarity of the winding terminals?

- Explain the relation between line voltage and phase voltage in star connection.

- Explain the relation between line current and phase current in star connection

- What is star to Delta connection?

- Explain the relation between line current and phase current in delta connection

- What is delta to delta connection?

- What is delta to star connection?

- Explain open delta connection.

- Explain the relation between line voltage and phase voltage in open delta connection.

- Explain polarity test of transformer.

- Describe the conditions for paralleling of the transformer.

- What is indicator?

- What is transformation ratio?

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | Voltmeter |
| 2 | Wattmeter |
| 3 | Ohmmeter |
| 4 | connecting wires |
| 5 | Ammeter |
| 10 | Three phase transformer |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

### 0713E&E21 Identify Vector Group of Three Phase Transformer

Overview:

This competency standard identifies the competencies required to Connect 3-phase transformers as per given vector group Yyo and Yy6, connect 3-phase transformers as per given vector group Ddo and Dd6 and connect 3-phase transformers as per given vector group Yd11 and Yd1

| Competency Units | Performance Criteria |
| --- | --- |
| - Connect 3-phase transformers as per given vector group Yyo and Yy6 | - Connect three identical single phase transformer in star to star connection. - Connect phase angle meter between primary and secondary. - Apply rated voltage to the primary side of transformer. - Measure the phase angle between primary and secondary with the help of phase angle meter (00 and 1800). |
| - Connect 3-phase transformers as per given vector group Ddo and Dd6 | - Connect three identical single phase transformer in delta to delta connection. - Connect phase angle meter between primary and secondary. - Apply rated voltage to the primary side of transformer. - Measure the phase angle between primary and secondary with the help of phase angle meter (00 and 1800). |
| - Connect 3-phase transformers as per given vector group Yd11 and Yd1 | - Connect three identical single phase transformer in star to delta connection. - Connect phase angle meter between primary and secondary. - Apply rated voltage to the primary side of transformer. - Measure the phase angle between primary and secondary with the help of phase angle meter (300 lag or 300 lead). |

Knowledge and understanding

- Explain star to star connection.

- Explain the term phase angle.

- How to connect phase angle meter between primary and secondary of the transformer.

- What is the phase angle of vector group Yy0.

- How to connect phase angle meter between primary and secondary of the transformer.

- What is the phase angle of vector group Yy6.

- What is the phase angle of vector group Dd0.

- What is the phase angle of vector group Dd6.

- Explain star to delta connection.

- What is the phase angle of vector group Yd11?

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | Single phase transformers |
| 2 | Phase angle meter |
| 3 | Connecting wires |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

### 0713E&E22 Repair/ Maintenance and Service of Transformer

Overview

This Competency Standard identifies the competencies required to Diagnose Fault in transformer, carry out Service/Repair of Transformer and Carry out Preventive Service/ Maintenance of transformer. Carry out repair and maintenance of transformer in accordance with the manufacturer’s instructions. The underpinning knowledge regarding repair and maintenance of transformer will be sufficient to provide the basis for your work.

| Competency Units | Performance Criteria |
| --- | --- |
| - Diagnose Fault in transformer | - Identify the type of transformer from data / name plate or manual and transformer rating. - Check the tripping/continuity of protective device using specified test - Dismantle the transformer, inspect visual mechanical defect such as, leakage of tank, noisy running, short circuit, LT, HT, coil, insulation and temperatures. - Before dissembling the transformer from assembly take snapshot ore draw layout diagram. - Check the transformer by using specified test instruments to detect electrical defects such as loose/or burnt electrical connections, burnt coil. Transformer oil, low insulation resistance etc. - Test Coil by humming method ore any standard method. - Check core lamination - Check continuity of Buchholz relay. - Check Short circuit faults such as inter turn faults, incipient winding faults, and core faults may occur due to the impulse breakdown of the insulating transformer oil. - Diagnose the faulty area |
| - Carry out Service/Repair of Transformer | - Clean the parts of the transformer with specified cleaning agents/tools & material. - Repair the transformer as per diagnosed fault - Rewind and assembled the transformer as per requirement - Check the insulation resistance of repaired transformer as per standard - Carry out the No load and load tests as per standard practices - Carry out de-hydration of oil - Carry out de-hydration of silica gel - Check and repair leakage in transformer tank |
| - Carry out Preventive Service/ Maintenance of transformer | - Carry out routine maintenance of the transformer as specified in the manufacturer’s manual - Check out the, protective relay and other allied device and wire connection - Update the maintenance/service records |

Knowledge & Understanding

- Explain the types of electrical measuring instruments used in testing

- Discuss the transformer faults.

- Explain preventive maintenance work on transformer.

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | Various type of transformers |
| 2 | Ammeters |
| 11 | Cable Knife |
| 16 | Combination Plier Set |
| 27 | Meager |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

:

- Perform insulation, frequency, no-load, load testing etc.

Troubleshooting of transformer fault.

### 0713E&E23 Maintain Tools & Equipment for Three Phase

Overview:

This Competency Standard covers the skills and knowledge required to arrange tools/equipment, maintain tool box, insulate tools/equipment, calibrate measuring tools and manage proper inventory of used/unused tools/equipment. Trainee will be expected to follow the procedures to maintain the tools/equipment.

| Competency Units | Performance Criteria |
| --- | --- |
| - Arrange Tools and Equipment | - Identify tools and equipment - Interpret job card - Prepare list of tools and equipment as per requirement - Collect tools and equipment from store |
| - Maintain Tool Box | - Check physical conditions of tools and equipment before use - Perform preventive maintenance as per standards - Perform corrective maintenance of tools as per requirements - Clean tools and equipment after use - Place tools and equipment at appropriate place |
| - Insulate Tools and Equipment | - Collect the required tools and equipment - Check insulation of Tools and Equipment - Perform insulation of the faulty tool/equipment |
| - Calibrate measuring tools | - Check calibration status of the - measuring tools - Perform calibration of measuring - tools as per standards - Record calibration test results |
| - Manage Inventory of tools and equipment | - Check tools and equipment as - per record - Report for faulty tools and - equipment to supervisor - Generate demand for deficit - tools and equipment - Maintain all records of tools and - Equipment |

Knowledge & Understanding

- Explain Various tools and equipment and their functions

- Define Job card/work order

- How an Arrangement of tools/equipment as per job is required?

- Differentiate between corrective and preventive maintenance

- Arrange tools and equipment in tool box

- Storage methods of tools and equipment

- Insulation procedure

- Types of insulation

- Methods of insulated tools and equipment.

- Types of calibration

- Methods of equipment calibration

- Methods of tools and equipment inventory

- Report writing of faulty tools and equipment:

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Describe functions of various tools and equipment

- Define job card

- Storage methods of tools and equipment

- Types of insulation

- Use of insulated tools and equipment

- Method of equipment calibration

- Report writing

### 0713E&E24 Make Cable/Wire Joints for Three Phase

Overview:

This Competency Standard covers the skills and knowledge required to. Make Cross/Twist joint, Make Straight/Married joint,Make T- Joint, Make Rat tail joint and Make Britannia joint, make joints of cable during wiring and breakage of wires, Make Cross/Twist joint, Make Straight/Married joint, Make T- Joint, Make Rat tail joint, Make Britannia joint.

| Competency Units | Performance Criteria |
| --- | --- |
| - Make Cross/Twist joint | - Select the cable. - Strip the wire according to 50mm. - Twist the conductors. - Solder the conductor - Insulate the joint |
| - Make Straight/Married joint | - Select the cable. - Strip wire to 75mm according to joint requirement. - Intermingle the conductors to 60mm into each other. - Twist 60mm conductors leaving behind 15mm of each cable. - Solder the conductor. - Insulate the joint. |
| - Make T- Joint | - Select the cable. - Remove the insulation of cable 1 to 50mm from where a connection is required. - Separate conductors of cable 1 equally. - Take another 12 mm stripped wire 2. - Insert between two equally half conductors of cable 1 and twist. Half conductors clockwise and half anti-clock wise of cable 2. - Solder the joint. |
| - Make Rat tail joint | - Select the single conductor wires. - Strip both the wires to 5mm. - Twist the conductor. - Insulate the joint. - Solder the joint. |
| - Make Britannia joint | - Select the cable. - Strip both cables to 75mm. - Bend the tips of both cable right angle about to 6mm. - Hold the two cables overlap 50mm with tips in opposite direction. - Take another bare conductor of 1mm and wrap around in both directions to 6mm. - Solder the joint. - Insulate the joint. |

Knowledge & Understanding

- Define conductor.

- Differentiate between cable and wire.

- Describe the type of soldering.

- Differentiate between stripping and insulation removing.

- Describe the type of joints.

- Describe the procedure of jointing & soldering.

- Explain the composition of solder and soldering flux.

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | Cables. |
| 2 | Wires |
| 3 | Wire stripper. |
| 4 | Plier. |
| 5 | Nose plier. |
| 6 | Insulation remover |
| 7 | Soldering paste |
| 8 | Solder. |
| 9 | Solder wire. |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

Striping of cables.

Twisting of cable.

Soldering of joints

### 0713E&E25 Install Three Phase Complex Electrical Wiring

Overview:

This Competency Standard covers the skills and knowledge required to Make and Install 3 phase motor connection Reverse/ forward by Magnetic contactor, Make and Install 3 phase motor connection Reverse/ forward by Magnetic contactor with indicator, Make and Install 3 phase motor connection Star/Delta (Manual) by Magnetic contactor, Make and Install 3 phase motor connection Star/Delta (Manual) by Magnetic contactor with indicator, Make and Install 3 phase motor connection Star/Delta (Auto) by Magnetic contactor, Make and Install 3 phase motor connection Star/Delta (Auto) by Magnetic contactor with indicator, Make and Install 3 phase motor connection 2 speed by Magnetic contactor, Make and Install 3 phase motor connection 2 speed by Magnetic contactor with indicator.

| Competency Units | Performance Criteria |
| --- | --- |
| - Make and Install 3 phase motor connection Reverse/ forward by Magnetic contactor. | - Draw power diagram of circuit. - Draw control diagram of circuit. - Draw installation diagram of circuit. - Mark on exercise board according to installation/layout diagram. - Install accessories according to layout diagram. - Lay wires in duct/pipe according to layout diagram. - Make connections according to wiring diagram. - Check the circuit before connect the main supply. - Make connection with main supply. - Check the function of circuit after connect the main supply |
| - Make and Install 3 phase motor connection Reverse/ forward by Magnetic contactor with indicator. | - Draw power diagram of circuit. - Draw control diagram of circuit. - Draw installation diagram of circuit. - Mark on exercise board according to installation/layout diagram. - Install accessories according to layout diagram. - Lay wires in duct/pipe according to layout diagram. - Make connections according to wiring diagram. - Check the circuit before connect the main supply. - Make connection with main supply. - Check the function of circuit after connect the main supply |
| - Make and Install 3 phase motor connection Star/Delta (Manual) by Magnetic contactor. | - Draw power diagram of circuit. - Draw control diagram of circuit. - Draw installation diagram of circuit. - Mark on exercise board according to installation/layout diagram. - Install accessories according to layout diagram. - Lay wires in duct/pipe according to layout diagram. - Make connections according to wiring diagram. - Check the circuit before connect the main supply. - Make connection with main supply. - Check the function of circuit after connect the main supply |
| - Make and Install 3 phase motor connection Star/Delta (Manual) by Magnetic contactor with indicator. | - Draw power diagram of circuit. - Draw control diagram of circuit. - Draw installation diagram of circuit. - Mark on exercise board according to installation/layout diagram. - Install accessories according to layout diagram. - Lay wires in duct/pipe according to layout diagram. - Make connections according to wiring diagram. - Check the circuit before connect the main supply. - Make connection with main supply. - Check the function of circuit after connect the main supply |
| - Make and Install 3 phase motor connection Star/Delta (Auto) by Magnetic contactor. | - Draw power diagram of circuit. - Draw control diagram of circuit. - Draw installation diagram of circuit. - Mark on exercise board according to installation/layout diagram. - Install accessories according to layout diagram. - Lay wires in duct/pipe according to layout diagram. - Make connections according to wiring diagram. - Check the circuit before connect the main supply. - Make connection with main supply. - Check the function of circuit after connect the main supply |
| - Make and Install 3 phase motor connection Star/Delta (Auto) by Magnetic contactor with indicator | - Draw power diagram of circuit. - Draw control diagram of circuit. - Draw installation diagram of circuit. - Mark on exercise board according to installation/layout diagram. - Install accessories according to layout diagram. - Lay wires in duct/pipe according to layout diagram. - Make connections according to wiring diagram. - Check the circuit before connect the main supply. - Make connection with main supply. - Check the function of circuit after connect the main supply |
| - Make and Install 3 phase motor connection 2 speed by Magnetic contactor. | - Draw power diagram of circuit. - Draw control diagram of circuit. - Draw installation diagram of circuit. - Mark on exercise board according to installation/layout diagram. - Install accessories according to layout diagram. - Lay wires in duct/pipe according to layout diagram. - Make connections according to wiring diagram. - Check the circuit before connect the main supply. - Make connection with main supply. - Check the function of circuit after connect the main supply |
| - Make and Install 3 phase motor connection 2 speed by Magnetic contactor with indicator | - Draw power diagram of circuit. - Draw control diagram of circuit. - Draw installation diagram of circuit. - Mark on exercise board according to installation/layout diagram. - Install accessories according to layout diagram. - Lay wires in duct/pipe according to layout diagram. - Make connections according to wiring diagram. - Check the circuit before connect the main supply. - Make connection with main supply. - Check the function of circuit after connect the main supply. |
| - Perform Electrical Work in Industrial Area. | - Install wire for grounding - Perform pre Installation module based Tests - Perform integrated tests. - Install conduits or Trunks in the building as per drawing - Install electrical wiring, as per drawing and standard. - Install centralize lighting, heating, cooling system and other appliances - Install main box and distribution box. - Install Electric Breakers and switch gears in main box, as per load for safety measures - Install fire alarm and smoke detector system - Install the appliances used in electric work as per drawing - Install emergency light system in main box, as per standard color scheme - Install Main Distribution Board. - Test and certify the Installed system. |

Knowledge & Understanding

- Define three phase supply.

- Describe three phase motor.

- Describe types of three phase motor.

- Define power diagram.

- Define installation diagram.

- Describe Control Diagram.

- Describe types of three phase motor connection

- Describe Star/Delta connection.

- Explain reverse/forward circuit and its uses.

- Define three phase supply.

- Describe three phase motor.

- Describe types of three phase motor.

- Define power diagram.

- Define installation diagram.

- Describe Control Diagram.

- Describe types of three phase motor connection.

- Describe Star/Delta connection.

- Describe the purpose of indicator.

- Explain reverse/forward  circuit and its uses

- Define three phase supply.

- Describe three phase motor.

- Describe types of three phase motor.

- Define power diagram.

- Define installation diagram.

- Describe Control Diagram.

- Describe types of three phase motor connection.

- Describe Star/Delta connection.

- Describe the purpose of indicator.

- Explain reverse/forward circuit and its uses.

- Define star/delta circuit.

- Compare star/delta circuit.

- Describe purpose and uses of star/delta circuit.

- Define three phase supply.

- Describe three phase

- Motor.

- Describe types of three phase motor.

- Define power diagram.

- Define installation diagram.

- Describe Control Diagram.

- Describe types of three phase motor connection.

- Describe Star/Delta connection.

- Describe the purpose of indicator.

- Explain reverse/forward circuit and its uses.

- Define star/delta circuit.

- Compare star/delta circuit.

- Describe purpose and uses of star/delta circuit.

- Describe purpose of indicator.

- Define three phase supply.

- Describe three phase motor

- Describe types of three phase motor.

- Define power diagram.

- Define installation diagram.

- Describe Control Diagram.

- Describe types of three phase motor connection.

- Describe Star/Delta connection.

- Describe the purpose of indicator.

- Explain reverse/forward circuit and its uses.

- Define star/delta circuit.

- Compare star/delta circuit.

- Describe purpose and uses of star/delta circuit.

- Describe the function of timer.

- Define timer.

- Define three phase supply.

- Describe three phase

- Motor.

- Describe types of three phase motor.

- Define power diagram.

- Define installation diagram.

- Describe Control Diagram.

- Describe types of three phase motor connection.

- Describe Star/Delta connection.

- Describe the purpose of indicator.

- Explain reverse/forward circuit and its uses.

- Define star/delta circuit.

- Compare star/delta circuit.

- Describe purpose and uses of star/delta circuit.

- Describe the function of timer.

- Define timer.

- Define three phase supply.

- Describe three phase motor.

- Describe types of three phase motor.

- Define power diagram.

- Define installation diagram.

- Describe Control Diagram.

- Describe types of three phase motor connection.

- Describe 2 speed motor connection and its uses.

- Describe the purpose of 2 speed motor circuit.

- Define three phase supply.

- Describe three phase motor.

- Describe types of three phase motor.

- Define power diagram.

- Define installation diagram

- Describe Control Diagram.

- Describe types of three phase motor connection.

- Describe 2 speed motor connection and its uses.

- Describe the purpose of 2 speed motor circuit.

- Describe purpose of indicator.

- Describe the safety of installation

- Explain the Protective measures against too high voltage and electrical accidents.

- Explain Fundamentals of installation engineering

- Describe the Power overhead-line systems.

- Explain Low-voltage switchgear and distribution systems.

- Explain Mounting and connection of motor testing of installed plant.

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | hammer |
| 2 | wire cutters |
| 3 | linesman pliers |
| 4 | Tape |
| 5 | Power saws |
| 6 | Power drills and drivers, hammer/drills |
| 7 | Labeling machines |
| 8 | Fishing tools |
| 9 | Wire strippers |
| 10 | Measuring devices |
| 11 | Electrician Tool kit |
| 12 | Push button |
| 13 | Screw |
| 14 | PVC Connector Bar |
| 15 | PVC Wire. |
| 16 | PVC Pipe/Duct. |
| 17 | PVC Board. |
| 18 | Three phase induction motor |
| 19 | Magnetic Contactor |
| 20 | Electrician Tool kit |
| 21 | Three phase induction motor. |
| 22 | Fuse. |
| 23 | Over load relay |
| 24 | Three phase induction motor |
| 25 | Magnetic Contactor. |
| 26 | Fuse. |
| 27 | PVC Board. |
| 28 | PVC Wire. |
| 29 | PVC Connector Bar |
| 30 | Screw. |
| 31 | Push button. |
| 32 | PVC Pipe/Duct. |
| 33 | Fuse. |
| 34 | PVC Board. |
| 35 | PVC Wire |
| 36 | PVC Wire |
| 37 | PVC Connector Bar |
| 38 | PVC Connector Bar |
| 39 | Indicator |
| 40 | Phase induction motor. |
| 41 | Timer with base. |
| 42 | Three |
| 43 | Over load relay. |
| 44 | PVC Pipe/Duct. |
| 45 | Screw. |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

### 0713E&E26 Perform Testing of Electrical Three Phase Wiring

Overview:

This competency standard covers the skills and knowledge required to perform earth leakage test, perform open circuit test, perform short circuit test, perform continuity/loop test, perform visual test, perform insulation test, perform polarity test, perform earth resistance test, perform Murray loop test, and perform Blavier & earth loop test

| Competency Units | Performance Criteria |
| --- | --- |
| - Perform Earth leakage Test. | - Ensure the socket must be at least 13A for RCD plug in. - Adjust the sensitivity of RCD. - Ensure tripping time must not exceed than 200msec. - Plug the RCD in socket and switch on socket outlet. - Ensure P-N and P-E light up. - Press the test button. - Ensure that RCD will trip and reading must be shown less than 200msec. - Move the selection switch to 180o. - Press the test button. - Ensure that RCD will trip and reading must be shown less than 200msec. - Move selection switch to 00. - Press the test button. - Ensure that RCD will trip and reading must be shown less than 200msec. |
| - Perform Open Circuit Test | - Disconnect the supply source, neutral conductor and earth conductor from distribution fuse box. - Connect ohmmeter between starting and ending terminals of live conductor. - Ensure the reading must be near to 0 ohm. - Repeat P2 to P3 for neutral and earth conductor. |
| - Perform Short Circuit Test | - Disconnect the supply source, neutral conductor and earth conductor from distribution fuse box. - Connect one terminal of ohmmeter with live conductor. - Connect the other terminals of ohmmeter with neutral conductor. - Ensure the reading must be infinity (No reading). - Connect ohmmeter between live conductor and earth conductor. - Ensure the reading must be infinity (No reading) |
| - Perform Continuity/Loop Test | - Connect all metal clad switches, metal parts, conduits with earth. - Open the main switch. - Turn on all the switches. - Connect earth continuity tester with conduit and independent earth. - Measure the resistance value with tester which must not be more than 1 ohm. |
| - Perform Visual Test | - Check the wires joints. - Check if there is any spark. - Check over heating of wires. - Check all the appliances are On or fluctuating. - Check all indications of meters. - Check Voltages on all phases. - Check the DB and Breakers. - Check the main supply is coming properly. |
| - Perform Insulation Test | - Switch off the main breaker. - Short circuit all lighting connections. - Connect merger between line and earth. - Rotate the merger to generate the voltage. - Read the merger dial and ensure that the insulation resistance should not be less than 50M ohm of any single circuit. |
| - Perform Polarity Test | - Switch off the main switch. - Disconnect all loads. - Switch on all circuit control switches. - Connect ohm meter between main line and all the terminal of sockets and load points one by one. - Ensure meter reading value must be less than 1 ohm. |
| - Perform Earth Resistance Test | - Place 3 electrodes in earth at distance of 10m apart between every electrode. - Connect terminal E of earth resistance tester to first electrode. - Connect terminal P of earth resistance tester to the second electrode. - Connect terminal C of the earth resistance tester with third electrode. - Measure the resistances after applying specific voltage. - Repeat P1 to P5 by interacting earthling regions. - Ensure 3 readings must be equal. |
| - Perform Murray Loop Test | - Connect galvanometer with Wheatstone bridge. - Connect positive terminal of DC source with Wheatstone bridge. - Connect negative terminal of DC source with ground. - Connect remaining two terminal of Wheatstone bridge with two cores of the cable. - Short the other end of these two cores of cable. - Note the reading of galvanometer. - Calculate Rx. - Repeat the P1 to P7 for other cores of the cable. |
| - Perform BLAVIER & Earth Loop Test | - Connect galvanometer with Wheatstone bridge. - Connect positive terminal of DC source with Wheatstone bridge. - Connect negative terminal of DC source with ground. - Connect remaining two terminal of Wheatstone bridge with two cores of the cable. - Short the other end of these two cores of cable. - Note the reading of galvanometer. - Calculate Rx. - Repeat the P1 to P7 for other cores of the cable. |

Knowledge & Understanding

- Define RCD

- Define sensitivity

- Explain P-E and P-N

- Explain the function of RCD.

- Explain the function of distribution box.

- Explain the function of fuse.

- Explain which meters are used for open circuit and short circuits test.

- What does it mean by 0 ohm reading in open circuit test?

- What does it mean by no

- Reading in short circuit test?

- Explain the purpose of continuity test.

- Explain the purpose of polarity test.

- Explain the purpose of insulation resistance test.

- Explain the purpose of Murray loop test.

- Explain the purpose of BLAVIER’s loop test.

- Describe why Wheatstone bridge is attached in Murray loop test.

- What is the purpose of DC supply in Murray loop test?

- Explain the purpose of Galvanometer.

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | Sockets |
| 2 | Magger |
| 3 | Breakers |
| 4 | Earth resistance tester |
| 5 | RCD |
| 6 | Continuity tester |
| 7 | Ohmmeter |
| 8 | Supply source |
| 9 | Distribution Box |
| 10 | Loads |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

### 0713E&E27 Diagnose Electrical Fault in Generator

Overview:

This Competency Standard covers the skills and knowledge required to inspect and service ignition system, inspect and service alternator, inspect and service display panel, inspect and service governor /Actuator System, inspect and service charging system, inspect and service warning system, to diagnose electric fault (s) in generator.

| Competency Units | Performance Criteria |
| --- | --- |
| CU1: Inspect and service Ignition system | P1. Identify the tools and equipment P2. Check DC power supply of ignition coil and distributor P3. Check HT leads P4. Check spark plug |
| CU2: Inspect and service alternator | P1. Identify the tools and equipment P2. Check DC output voltage P3. Check belt P4. connections of alternator |
| CU3:Inspect and service display panel | P1. Identify tools and equipment P2. Check gauges, circuit breakers, relays and wiring as per standard parameters |
| CU4: Inspect and service governor /Actuator System | P1. Identify the tools and equipment P2. Check actuator card supply P3. Check magnetic pick up P4. Check power supply on actuator/governor |
| CU5: Inspect and service charging system | P1. Identify the tools and equipment P2. Check battery power leads P3. Check charging circuit of alternator |
| CU6: Inspect and service warning system | P1. Identify tools and equipment P2. Check oil sensor P3. Check temperature sensor P4. Check fuel sensor P5. Check over/under load module |
| CU7: Inspect and repair wiring | - Identify the tools and equipment for checking of supply. - Check the main supply. - Check the breakers. - Ensure the supply off. - Perform open circuit and short Circuit test with meter. - Identify the faults. - Remove the fault. |

Knowledge & Understanding

- Define Ignition coil and Distributor

- Define High Tension (HT) Leads

- Functions of Spark Plug.

- Describe DC alternator

- Knowledge about voltage of Alternator

- Knowledge about Display Panel

- / Data Screen

- Types of circuit breakers / Relays

- Describe Governor / Actuator

- Describe charging system

- Describe Warning system

- Describe open circuit test

- Describe short circuit test.

- Define fuse, miniature circuit breaker.

- Distinguish between fuse and miniature circuit breaker.

- List the parts of fuse & M.C.B.

- Compare the advantages & disadvantages of fuse & M.C.B.

- Classify cables with respect to insulation, core, voltage grade

- & current carrying capacity.

- Explain different systems for

- Calculation of cable size.

- Find size of cable for respective load.

- Name components of earthling system.

- Define earth electrode, earth continuity conductor &earthling lead.

- Explain the earth fault current.

- Find size of earth continuity conductor, earthling lead & earth electrodes.

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | DC power supply. |
| 2 | Spark plugs. |
| 3 | Alternator |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Ignition coil and Distributor

- High Tension (HT) Leads

- Functions of Spark Plug

- Display Panel / Data Screen

- Governor / Actuator

- Remove the fault in wiring.

### 0713E&E28 Repair/ Maintenance of Electrical Installations for Three Phase

Overview:

This Competency Standard identifies the competencies required to. Analyze Fault in Electrical Installations, troubleshooting of electrical equipment’s and Carry out Preventive Maintenance. Carry out repair and maintenance of electrical installations at workplace in accordance with the manufacturer’s instructions and organizational requirements.

| Competency Units | Performance Criteria |
| --- | --- |
| - Analyze Fault in Electrical Installations | - Inspect visually the electrical wiring, fixtures, equipment, soldering, connection, appliances and machinery for discovering the faults and defects - Check the installation for consistency with the electrical drawing - Draw the layout of equipment's before disassembling of electrical components - Check the faulty components with scope or meter - Re assembled the components as per drawing and the installation - Check the fault indication at relay for HT installation - Test electrical equipment as specified in the manufacturer’s manual and record the results |
| - Repair Fault in Electrical Installations | - Prepare a list of items/material(s) required for repair /replacement as per specifications - Draw circuit diagram of electrical equipment's be disassembling - Make necessary adjustments in the control and protective switchgear - Replace defective control & protective switch gear, cables and accessories with standard items - Replace defective earth electrode & faulty/damaged earthling conductors - Test installed electrical equipment for safe and optimum performance according to standards & regulations - Record the results of the test performed on a standard format |

Knowledge & Understanding

- The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard.

- Interpretation of layout diagrams, technical sketches, graphic symbols and wiring diagrams, and manufacturer’s specifications etc.

- Types of electrical tools used for troubleshooting and preventive maintenance purposes

- Types of electrical measuring instruments used in testing electrical installations

- Types of electrical wiring systems for domestic& industrial purposes

- Methods of tracing the fault

- Types of electrical control and protective switchgear and accessories used in electrical circuits

- Types and principles of operation of circuit breakers used in electrical installations and their applications

- Types of electrical wires and cables and their ratings

- Types of electrical accessories and their applicant

- electrical installations

- Types of earthling systems used in domestic& industrial

Electrical installations

- Importance of testing electrical installations Importance of corrective & preventive maintenance

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | Ammeter |
| 2 | Cable Cutter |
| 3 | Clamp Meter |
| 4 | Circuit Boards |
| 5 | Cells tester |
| 6 | Chisel |
| 7 | Cable Knife |
| 8 | Battery Cleaning Kit |
| 9 | Bearing Puller |
| 10 | Bench Vice |
| 11 | Cable / Wire Gauge |
| 12 | Battery Charger |
| 13 | Hammer |
| 14 | Hacksaw |
| 15 | Gloves |
| 16 | Grinder |
| 17 | Combination Plier Set |
| 18 | Disk Grinder |
| 19 | Duct Rod |
| 20 | Files (set |
| 21 | Earth Tester |
| 22 | Filler gauge |
| 23 | Flux |
| 24 | Generator |
| 25 | Handsaw |
| 26 | Hole saw |
| 27 | Hydrometer |
| 28 | IR Temperature Gun |
| 29 | L Scale |
| 30 | Lugs Punching Machine (Hydraulic and Manual) |
| 31 | Lux Meter |
| 32 | Magnetic Conductor |
| 33 | Megara (Insulation Tester) |
| 34 | Micron Meter |
| 35 | Mini Hydraulic Press Machine |
| 36 | Multimeter |
| 37 | OTDR Meter |
| 38 | Overall Combination |
| 39 | Phase Sequence Meter |
| 0 | Ring Spanner Set |
| 41 | Philips Screw drivers Set |
| 42 | RPM Meter |
| 43 | Safety Belt |
| 44 | Safety Goggles |
| 45 | Safety Helmet |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

  - Trace out the fault and take corrective action

  - Carry out the preventive maintenance

  - Update the service/repair record

### 0713E&E29 Carry Out Manual Electrical Planning and Estimation

Overview:

This Competency Standard identifies the competencies required to analyse customer requirement and specification, Plan and estimate domestic electric work, Plan and estimate commercial   electric work and Plan and estimate industrial electric work This competency standard covers the skills and knowledge in the estimation, planning and installation of an electrical work, keeping in view the necessary rules and regulations.

| Competency Units | Performance Criteria |
| --- | --- |
| - Analyze customer requirement and specification | - Draw the general value chain of the end user. - Highlight the various stages and set of activities in the value chain drawing - Enlist the electrical appliances/materials required in electrical development process - Identify critical stages in the development - Identify the safety aspect required in the critical stages of the development - Enlist the possible energy efficient appliance/devices and global trends in electrical works - Analyse the client requirement at broad level. - Generate a report of various stages of electrical works. - Proposed appliance/materials as per global trends and clarify the technical specification. |
| - Plan and estimate domestic electric work | - Plan the client requirement at broad level from the proposal. - Plan the electrical appliances/materials / used in different stages of the electrical development process - Plan and estimate electrical wiring - Estimate Installation and Material Cost - Estimate Power consumption for lighting and appliances - Plan a main and sub distribution board. - Ensure safety system. |
| - Plan and estimate commercial electric work. | - Analyze the client requirement at broad level from the proposal. - Plan the electrical equipment /appliances/materials / used in different stages of the commercial electrical work - Plan and estimate electrical wiring - Estimate Installation and Material Cost - Estimate Power consumption for centralize lighting, heating, cooling system and other appliances. - Plan a main and sub distribution board. - Ensure safety measures in development process. - Enlist the material required. |
| - Plan and estimate industrial electric work. | - Analyze the client requirement at broad level from the proposal. - Plan the electrical equipment /appliances/materials / used in different stages of the commercial electrical work - Plan and estimate electrical wiring - Estimate Installation and Material Cost - Estimate Power consumption for centralize lighting, heating, cooling system and other appliances. - Plan 3-phase line for heavy loads - Plan a separate control panel box for heavy machinery. - Plan a main and sub distribution board. - Design a safety system for protection of machinery and labor. - Assign color scheme for safety measures |

Knowledge & Understanding

- Define the term ‘value chain’ of industry.

- Define human machine interface (HMI)

- Define the Standard operating procedure (SOP) of the organization for control panel development process.

- How to prepare a General arrangement drawing?

- Define IEC standards

- Describes the standard of electrical components.

- Understanding and knowledge of equipment properties.

- Understanding and knowledge of input and output of tool of electronic.

- Describe the rules and regulation of work.

- Understanding and knowledge of Electrical Specifications component used.

- Describes the standard of electrical components.

- Understanding and knowledge of equipment properties.

- Understanding and knowledge of input and output of tool of electronic.

- Describe the rules and regulation of work

- Describes the standard of electrical components.

- Understanding and knowledge of equipment properties.

- Understanding and knowledge of input and output of tool of electronic

- Describe the rules and regulation of work

Tools and Equipment

| SN | Tools |
| --- | --- |
| 1 | Site Visit |
| 2 | Reference Material |
| 3 | Clipboard |
| 4 | Calculation Sheet |
| 5 | Calculator |
| 6 | Psychometric Charts and Tables |
| 7 | Protective gear such as helmets, goggles, gloves, rubber shoes, etc. |

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Installation techniques

### 0713E&E30 Develop Workplace Policy and Procedures for Sustainability

Overview:

This unit describes the skills and knowledge required to develop and implement a workplace sustainability policy and to modify the policy to suit changed circumstances. It applies to individuals with managerial responsibilities who undertake work developing approaches to create, monitor and improve strategies and policies within workplaces and engage with a range of relevant stakeholders and specialists.

| Unit of Competency | Performance Criteria |
| --- | --- |
| - Develop workplace sustainability policy | - Define scope of sustainability policy - Gather information from a range of sources to plan and develop policy - Identify and consult stakeholders as a key component of the policy development process - include appropriate strategies in policy at all stages of work for minimizing resource use, reducing toxic material and hazardous chemical use and employing life cycle management approaches - Make recommendations for policy options based on likely effectiveness, timeframes and cost - Develop policy that reflects the organization s commitment to sustainability as an integral part of business planning and as a business opportunity - Agree to appropriate methods of implementation, outcomes and performance indicators |
| - Communicate workplace sustainability policy | - Promote workplace sustainability policy, including its expected outcome, to key stakeholders - Inform those involved in implementing the policy about expected outcomes, activities to be undertaken and assigned responsibilities |
| - Implement workplace sustainability policy | - Develop and communicate procedures to help implement workplace sustainability policy - Implement strategies for continuous improvement in resource efficiency - Establish and assign responsibility for recording systems to track continuous improvements in sustainability approaches |
| - 4. Review workplace sustainability policy implementation | - Review workplace sustainability policy implementation Investigate successes or otherwise of policy - Monitor records to identify trends that may require remedial action and use to promote continuous improvement of performance - Modify policy and or procedures as required to ensure improvements are made |

Knowledge and understanding

- outline the environmental or sustainability legislation, regulations and codes of practice applicable to the organization identify internal and external sources of information and explain how they can be used to plan and develop the organization s sustainability policy

- explain policy development processes and practices

- outline organizational systems and procedures that relate to sustainability

- Outline typical barriers to implementing policies and procedures in an organization and possible strategies to address them.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

:

A person who demonstrates competency in this unit must be able to provide evidence of the ability to develop workplace policy and procedures for sustainability. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

- scope and develop organizational policies and procedures that comply with legislative requirements and support the organization s sustainability goals covering at a minimum:

  - minimizing resource use

  - resource efficiency

  - reducing toxic material and hazardous chemical use

  - employing life cycle management approaches

  - continuous improvement

- plan and implement sustainability policy and procedures including:

  - agreed outcomes

  - performance indicators

  - activities to be undertaken

  - assigned responsibilities

  - record keeping, review and improvement processes

- consult and communicate with relevant stakeholders to generate engagement with sustainability policy development, implementation and continuous improvement

- Review and improve sustainability policies.

### 0713E&E31 Maintain Professionalism in the Workplace

Overview:

This unit of competency describes the outcomes required maintain a professional image in the workplace, including behaving ethically, demonstrating motivation, respecting timeframes and maintaining personal appearance.

| Unit of Competency | Performance Criteria |
| --- | --- |
| - Respect work timeframes | - Demonstrate punctuality in meeting, set working hours and times. - Utilize working hours only for working and follow company regulations. - Complete work tasks within deadlines according to order of priority - Supervisors are informed of any potential delays in work times or projects. |
| - Maintain personal appearance and hygiene | - Clean hair, body and nails regularly. - Wear suitable cloths for the workplace, and respect local and cultural contexts - Meet specific company dress code requirements |
| - Maintain adequate distance with colleagues and clients | - Respect personal space of colleagues and clients with reference to local customs and cultural contexts. - Keep sufficient distance from others - Avoid cross transmission of infections (especially through respiration). |
| - Work in an ethical manner | - Follow company values/ethics codes of ethics and/or conduct, policies and guidelines. - Use company resources in accordance with company ethical standards. - Conduct personal behavior and relationships in accord with ethical standards and company policies. - Undertake work practices in compliance with company ethical standards, organizational policy and guidelines. - Instruct co-workers on ethical, lawful and reasonable directives. - Share Company values/practices with co-workers using appropriate behavior and language. - Report work incidents/situations and/or resolved in accordance with company protocol/guidelines. |

Knowledge and understanding

- Explain application of good manners and right conduct

- Explain basic practices for oral and personal hygiene

- Describe common products used for oral and personal hygiene

- Outline the company code of conduct/values

- Outline the Company regulations, performance and ethical standards

- Explain work responsibilities/job functions

- Describe communication skills

- State workplace hygiene standards

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

A person who demonstrates competency in this unit must be able to provide evidence of the ability to maintain professionalism in the workplace The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Evidence of the following is essential:

- clarify and affirm work values/ethics/concepts consistently in the workplace;

- comply with required working times;

- conduct work practices satisfactorily and consistently, in compliance with work ethical standards, organizational policy and guidelines;

- Develop suitable hygiene

Keep adequate distance while interacting with colleagues and clients.

### 0713E&E32 Manage Personal Work Priorities and Professional Development

Overview:

This unit describes the skills and knowledge required to create systems and process to organize information and prioritize tasks. It applies to individuals working in managerial positions who have excellent organizational skills. The work ethic of individuals in this role has a significant impact on the work culture and patterns of behavior of others as managers at this level are role models in their work environment.

| Unit of Competency | Performance Criteria |
| --- | --- |
| - Establish personal work goals | - Serve as a positive role model in the workplace through personal work planning - Ensure personal work goals, plans and activities reflect the organization s plans, and own responsibilities and accountabilities - Measure and maintain personal performance in varying work conditions, work contexts and when contingencies occur |
| - Set and meet own work priorities | - Take initiative to prioritize and facilitate competing demands to achieve personal, team and organizational goals and objectives - Use technology efficiently and effectively to manage work priorities and commitments - Maintain appropriate work-life balance, and ensure stress is effectively managed and health is attended to |
| - 3. Develop and maintain professional competence | - Assess personal knowledge and skills against competency standards to determine development needs, priorities and plans - Seek feedback from employees, clients and colleagues and use this feedback to identify and develop ways to improve competence - P3 Identify, evaluate, select and use development opportunities suitable to personal learning style/s to develop competence - Participate in networks to enhance personal knowledge, skills and work relationships - Identify and develop new skills to achieve and maintain a competitive edge |

Knowledge and understanding

- explain principles and techniques involved in the management and organization of:

- performance measurement

- personal behavior, self-awareness and personality traits identification

- personal development plan

- personal goal setting

- time

- discuss management development opportunities and options for self

- describe methods for achieving a healthy work-life balance

- outline organization s policies, plans and procedures

- explain types of learning style/s and how they relate to the individual

- Describe types of work methods and practices that can improve personal performance.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

A person who demonstrates competency in this unit must be able to provide evidence of the ability to manage personal work priorities and professional development. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

- use business technology to create and use systems and processes to organize and priorities tasks and commitments

- measure and maintain personal work performance including assessing competency against competency standards and seeking feedback

- maintain an appropriate work-life balance to manage personal health and stress

- participate in networks

- develop a personal development plan which includes career objectives and an action plan

- Develop new skills.

### 0713E&E33 Manage Workforce Planning

Overview:

This unit describes the skills and knowledge required to manage planning in relation to an organization’s workforce including researching requirements, developing objectives and strategies, implementing initiatives and monitoring and evaluating trends. It applies to individuals who are human resource managers or staff members with a role in a policy or planning unit that focuses on workforce planning.

| Unit of Competency | Performance Criteria |
| --- | --- |
| - Research workforce requirements | - Review current data on staff turnover and demographics - Assess factors that may affect workforce supply - Establish the organization’s requirements for a skilled and diverse workforce |
| - Develop workforce objectives and strategies | - Review organizational strategy and establish aligned objectives for modification or retention of the workforce - Consider strategies to address unacceptable staff turnover, if required - Define objectives to retain required skilled labor - Define objectives for workforce diversity and cross-cultural management - Define strategies to source skilled labor - Communicate objectives and rationale to relevant stakeholders - Obtain agreement and endorsement for objectives and establish targets - Develop contingency plans to cope with extreme situations |
| - Implement initiatives to support workforce planning objectives | - Implement action to support agreed objectives for recruitment, training, redeployment and redundancy - Develop and implement strategies to assist workforce to deal with organizational change - Develop and implement strategies to assist in meeting the organization’s workforce diversity goals - Implement succession planning system to ensure desirable workers are developed and retained - Implement programs to ensure workplace is an employer of choice |
| - Monitor and evaluate workforce trends | - Review workforce plan against patterns in exiting employee and workforce changes - Monitor labor supply trends for areas of over- or under-supply in the external environment - Monitor effects of labor trends on demand for labor - Survey organizational climate to gauge worker satisfaction - Refine objectives and strategies in response to internal and external changes and make recommendations in response to global trends and incidents - Regularly review government policy on labor demand and supply - Evaluate effectiveness of change processes against agreed objectives |

Knowledge and understanding

- explain current information about external labor supply relevant to the specific industry or skill requirements of the organization

- outline industrial relations relevant to the specific industry

- describe labor force analysis and forecasting techniques

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

A person who demonstrates competency in this unit must be able to provide evidence of the ability to manage workforce planning. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

- review and interpret information from a range of internal and external sources to identify:

  - current staff turnover and demographics

  - labor supply trends factors that may affect workforce supply

  - organization’s workforce requirements objectives and strategies

- manage workforce planning including developing, implementing, monitoring and reviewing strategies to meet workforce needs

- review relevant trends and supply and demand factors that will impact on an organization’s workforce

- Develop a workforce plan that includes relevant research and specific strategies to ensure access to a skilled and diverse workforce.

### 0713E&E34 Undertake Project Work

Overview:

This unit describes the skills and knowledge required to undertake a straightforward project or a section of a larger project. It covers developing a project plan, administering and monitoring the project, finalizing the project and reviewing the project to identify lessons learned for application to future projects. This unit applies to individuals who play a significant role in ensuring a project meets timelines, quality standards, budgetary limits and other requirements set for the project.

| Unit of Competency | Performance Criteria |
| --- | --- |
| - Define project | - Access project scope and other relevant documentation - Define project stakeholders - Seek clarification from delegating authority of issues related to project and project parameters - Identify limits of own responsibility and reporting requirements - Clarify relationship of project to other projects and to the organization’s objectives - Determine and access available resources to undertake project |
| - Develop project plan | - Develop project plan in line with the project parameters - Identify and access appropriate project management tools - Formulate risk management plan for project, including Work Health and Safety (WHS) - Develop and approve project budget - Consult team members and take their views into account in planning the project - Finalize project plan and gain necessary approvals to commence project according to documented plan |
| - Administer and monitor project | - Take action to ensure project team members are clear about their responsibilities and the project requirements - Provide support for project team members, especially with regard to specific needs, to ensure that the quality of the expected outcomes of the project and documented time lines are met - Establish and maintain required recordkeeping systems throughout the project - Implement and monitor plans for managing project finances, resources and quality - Complete and forward project reports as required to stakeholders - Undertake risk management as required to ensure project outcomes are met - Achieve project deliverables |
| - Finalize project | - Complete financial recordkeeping associated with project and check for accuracy - P2 Ensure transition of staff involved in project to new roles or reassignment to previous roles - P3 Complete project documentation and obtain necessary sign-offs for concluding project |

Knowledge and understanding

- give examples of project management tools and how they contribute to a project

- outline types of documents and other sources of information commonly used in defining the parameters of a project

- explain processes for identifying and managing risk in a project

- outline the organization’s mission, goals, objectives and operations and how the project relates to them

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

A person who demonstrates competency in this unit must be able to provide evidence of the ability to undertake project work. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

- define the parameters of the project including:

  - project scope

  - project stakeholders, including own responsibilities

  - relationship of project to organizational objectives and other projects

  - reporting requirements

  - resource requirements

- use project management tools to develop and implement a project plan including:

  - deliverables

  - work breakdown

  - budget and allocation of resources

  - timelines

  - risk management

  - recordkeeping and reporting

- consult and communicate with relevant stakeholders to generate input and engagement in planning, implementing and reviewing the project

- provide support to team members to enable them to achieve deliverables and to transition them as appropriate at completion of the project

- finalize the project including documentation, sign-offs and reporting

Review and document the project outcomes.

### 0713E&E35 Prepare and Implement Negotiation

Overview:

This unit covers the skills, knowledge and attitudes required to prepare for and participate in a process of negotiation.

| Unit of Competency | Performance Criteria |
| --- | --- |
| - Prepare for the negotiation | - Identify objectives and preferred outcome of the negotiation and determine minimum acceptable outcome - Understand in relation to what can be offered and what is needed from the other party - Gather information regarding the other party – objectives, - needs, preferences, resources, what they want to achieve - in - order to determine best negotiating points - List and rank the issues to consider concessions that may be made. - Find examples and refine negotiation argument. - Check information to ensure it is correct and up-to-date. - Develop a negotiation plan that includes information about the other party and its interests and a set of responses and strategies to the anticipated tactics. - Prepare an agenda in advance, which includes discussion topics, participants, location and schedule |
| - Participate in negotiations | - Analyze all aspects of the incident for degree of hazard, priorities, optional outcomes and appropriate strategies - Analyze and determine strategies and priorities on the incident sought from a range of sources - Assess long term objectives against resources and priorities - Apply a range of communication techniques to make and maintain contact with the key people - Provide clear and factual information to enable an honest and realistic assessment of the interests of the key people and their positions - Resolve the conflict and express their likely consequences clearly and do an analysis of the benefits - Reassess points of disagreements for common positive - positions |
| - Coordinate support services | - Assess the need for support services in terms of the determined strategies and priorities - Negotiate the resources of support services according to established procedures and availability - Provide information on strategies to support services and maintain the communication - Delegate roles and responsibilities according to expertise and resources |
| - Restore order | - Assess the incidents for degree of risk and take appropriate action to reduce and remove the impact of the incident and restore order - Take action designed to minimize risk and the preserve the safety and security of all involved - Take action to prevent the escalation of the incident appropriate to the circumstances and agreed procedures. - Carry out the use of force for the restoration of control and the maintenance of security in the least restrictive manner. - Complete reports accurately and clearly provided to the appropriate authority promptly - Review, evaluate and analyze the incident and the organizational response to it and report it promptly and accurately. |
| - Provide leadership. direction and guidance to the work group | - Link between the function of the group and the - goals of the organization - Participate in decision making routinely to develop, implement and review work of the group and to allocate responsibilities where appropriate - Give opportunities and encouragement to others to develop new and innovative work practices and strategies - Identify conflict and resolve with minimum disruption to work group function - Provide staff with the support and supervision necessary to perform work safely and without risk to health - Allocate tasks within the competence of staff and support with appropriate authority, autonomy and training - Supervise appropriately the changing priorities and situations and takes into account the different needs of individuals and the requirements of the task |

Knowledge and understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- Explain organization’s policies, guidelines and procedures related to control and surveillance, safety and preventing and responding to incidents and breaches of orders covered in the range of variables.

- Explain organization’s management and accountability systems

- Describe teamwork principles and strategies

- Outline the principles of effective communication

- Outline the guidelines for use of equipment and technology

- Explain code of conduct

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

A person who demonstrates competency in this unit must be able to provide evidence of the ability to solve problems which jeopardize safety and security. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Evidence of the following is essential:

- evidence of effective communication strategies including negotiation, counseling, mediation, advocacy demonstrated under pressure working effectively in a team environment

- evidence of knowledge and application of organizations policies, procedures and

- guidelines for critical incidents

- evidence of accurate and safe use of all emergency equipment

- evidence of managing effective outcomes using strategic planning, team

Leadership and situational analysis

### 0713E&E36 Manage Meetings.

Overview:

This unit describes the skills and knowledge required to manage a range of meetings including overseeing the meeting preparation processes, chairing meetings, organizing the minutes and reporting meeting outcomes. It applies to individuals employed in a range of work environments who are required to organize and manage meetings within their workplace, including conducting or managing administrative tasks in providing agendas and meeting material. They may work as senior administrative staff or may be individuals with responsibility for conducting and chairing meetings in the workplace.

| Unit of Competency | Performance Criteria |
| --- | --- |
| - Prepare for meetings | - Develop agenda in line with stated meeting purpose - Ensure style and structure of meeting are appropriate to its purpose - Identify meeting participants and notify them in accordance with organizational procedures - Confirm meeting arrangements in accordance with requirements of meeting - Dispatch meeting papers to participants within designated timelines |
| - Conduct meetings | - Chair meetings in accordance with organizational requirements, agreed conventions for type of meeting and legal and ethical requirements - Conduct meetings to ensure they are focused, time efficient and achieve the required outcomes - Ensure meeting facilitation enables participation, discussion, problem-solving and resolution of issues - Brief minute-taker on method for recording meeting notes in accordance with organizational requirements and conventions for type of meeting |
| - Follow up meetings | - Check transcribed meeting notes to ensure they reflect a true and accurate record of the meeting and are formatted in accordance with organizational procedures and meeting conventions - Distribute and store minutes and other follow-up documentation within designated timelines, and according to organizational requirements - Report outcomes of meetings as required, within designated timelines |

Knowledge and understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- outline meeting terminology, structures, arrangements

- outline responsibilities of the chairperson and explain group dynamics in relation to managing meetings

- describe options for meetings including face-to-face, teleconferencing, web-conferencing and using webcams

- Identify the relevant organizational procedures and policies regarding meetings, chairing and minutes including identifying organizational formats for minutes and agendas.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

A person who demonstrates competency in this unit must be able to provide evidence of the ability to manage meetings. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

- apply conventions and procedures for formal and informal meetings including:

  - developing and distributing agendas and papers

  - identifying and inviting meeting participants

  - organizing and confirming meeting arrangements

  - running the meeting and following up

- organize, take part in and chair a meeting

- record and store meeting documentation

Follow organizational policies and procedures.

### 0713E&E37 Organize Schedules

Overview:

This unit describes the skills and knowledge required to manage appointments and diaries for personnel within an organization, using manual and electronic diaries, schedules and other appointment systems. It applies to individuals employed in a range of work environments who provide administrative support to teams and individuals.

| Unit of Competency | Performance Criteria |
| --- | --- |
| - Establish schedule requirements | - Identify organizational requirements and protocols for diaries and staff planning tools - identify organizational procedures for different types of appointments - Determine personal requirements for diary and schedule items for individual personnel - Establish appointment priorities and clarify in discussion with individual personnel |
| - Manage schedules | - Identify recurring appointments and deadlines, and schedule these in accordance with individual and organizational requirements - Establish availability of attendees, and schedule new appointments in accordance with required timelines and diary commitments - Negotiate alternative arrangements and confirm when established appointments are changed - Record appointments and manage schedules in accordance with organizational policy and procedures |

Knowledge and understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out the tasks covered in this competency standard. This includes the knowledge of:

- identify the key provisions of relevant legislation, standards and codes that affect aspects of business operations or the achievement of team goals

- describe organizational requirements for managing appointments for personnel within the organization

- summarize the range of appointment systems that could be used

- Outline important considerations when managing the schedules of others.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

A person who demonstrates competency in this unit must be able to provide evidence of the ability to organize schedules. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

- Appropriately manage the schedules of various individuals through a process of careful planning and negotiation.

### 0713E&E38 Identify and Communicate Trends in Career Development

Overview:

His unit describes the skills and knowledge required to research and confirm career trends, Assess and confirm ongoing career development needs of target group and Maintain quality of career development services and professional practice conduct research to identify and communicate career trends. It establishes the need to interact professionally with others in assessing career needs, to effectively assist clients identify competencies they require for a career and employability in a given context. It also examines how to maintain quality of career development services and professional practice. It applies to individuals seeking to identify and communicate trends in career development.

| Unit of Competency | Performance Criteria |
| --- | --- |
| - Research and confirm career trends | P1 Apply knowledge of changing organizational structures, lifespan of careers and methods of conducting work search, recruitment and selection processes P2 Analyze changing worker and employer issues, rights and responsibilities in context of changing work practices P3 Examine importance of quality careers development services P4 Maintain all research, documentation, sources and references (electronic or physical) to a high degree of currency and relevance P5 Analyze implications of relevant policy, legislation, professional codes of practice and national standards relating to worker and employer issues P6 Research changes and trends in theory of career development counseling and practice P7 Confirm clusters, levels and combinations of transferable employability skills and preferences that may open employment options spanning more than one occupation or career pathway |
| - Assess and confirm ongoing career development needs of target group | P1 Analyze history and records in assessing needs of target group P2 Assess success of previous career development services and techniques used for individual or target group P3 Deploy other means to investigate appropriate care and counseling approaches as required P4 Maintain privacy and security of all data, research and personal records according to relevant policy, legislation, professional codes of practice and national standards P5 Establish existing work-life balance requirements, issues and needs |
| - Maintain quality of career development services and professional practice | P1 Analyze and review relevance of career theories, models, frameworks and research for target group P2 Incorporate into career development services and professional practice, major changes and trends influencing workplace and career-related options and choices P3 Comply with all relevant policy, legislation, professional codes of practice and national standards that influence delivery of career development services |

Knowledge and understanding:

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- explain client care and counseling techniques and processes in the context of career development services

- describe diversity and its potential effects on career choices

- outline human psychological development and needs in relation to careers development

- outline relevant policy, legislation, codes of practice and standards relevant to career development

- explain recruitment and selection processes in the context of career development services

- describe a range of data gathering and research techniques

- Explain techniques used to analyze trends.

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

A person who demonstrates competency in this unit must be able to provide evidence of the ability to identify and communicate trends in career development. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance Requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints.

Demonstrated evidence is required of the ability to:

- research and analyze current economic, labor market, employment, career and vocational, educational and training trends

- identify choices and career development needs for individuals and target groups within a given context

- report and document management of research and career development materials

- Comply with all relevant local, state/territory and national legislation, policies and practices.

### 0713E&E39 Apply Specialist Interpersonal and Counselling Interview Skills

Overview:

This unit describes the skills and knowledge required to communicate effectively, Use specialized counseling interviewing skills and use advanced and specialized communication skills in the client-counselor relationship. This unit applies to individuals whose job role involves working with clients on personal and psychological issues within established policies, procedures and guidelines.

| Unit of Competency | Performance Criteria |
| --- | --- |
| - Communicate effectively | P1 Identify communication barriers and use strategies to overcome these barriers in the client-counselor relationship P2 Facilitate the client-counselor relationship through selection and use of micro skills P3 Integrate the principles of effective communication into work practices P4 Observe and respond to non-verbal communication cues P5 Consider and respond to the impacts of different communication techniques on the client-counselor relationship in the context of individual clients P6 Integrate case note taking with minimum distraction |
| - Use specialized counseling interviewing skills | P1 Select and use communication skills according to the sequence of a counseling interview P2 Identify points at which specialized counseling interviewing skills are appropriate for inclusion P3 Use specialized counseling communication techniques based on their impacts and potential to enhance client development and growth P4 Identify and respond appropriately to strong client emotional reactions |
| - Evaluate own communication | P1 Reflect on and evaluate own communication with clients P2 Recognize the effect of own values and beliefs on communication with clients P3 Identify and respond to the need for development of own skills and knowledge |

Knowledge and understanding:

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Legal and ethical considerations for communication in counseling practice, and how these are applied in individual practice:

- codes of conduct/practice

- discrimination

- duty of care

- human rights

- practitioner/client boundaries

- privacy, confidentiality and disclosure

- rights and responsibilities of workers, employers and clients

- work role boundaries responsibilities and limitations of the counselor role

- work health and safety

- principles of person-centered practice

- key objectives of counseling interviewing

- stages of a counseling interview

- potential impacts of using different communication skills and techniques in counseling contexts

- communication techniques and micro-skills including:

- attending behaviors active listening, reflection of content feeling, summarizing

- questioning skills open, closed, simple and compound questions

- client observation skills

- noting and reflecting skills

- providing client feedback

- specialized counseling communication techniques, and how they are used, including:

- challenging

- reframing

- focusing

- components of the communication process including:

- encoder

- decoder

- primary factors that impact on the communication process including:

- context

- participants

- rules

- messages

- channels

- noise

- feedback

- communication barriers and resolution strategies, including:

- environmental

- physical

- individual perceptions

- cultural issues

- language

- age issues

- disability

- observational techniques including:

- facial expressions

- non-verbal behavior

- posture

- silence

- ways in which different people absorb information, including:

- visual

- auditory

- kinesthetic

- obstacles to the counseling process

- impacts of trauma and stress on the communication process, including on:

- concentration and attention

- memory

- use of verbal and written language

- use of body language

- challenging within the counseling session

- self-evaluation practices, including:

- how to recognize own biases

- Impact of own values on the counseling relationship

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

:

A person who demonstrates competency in this unit must be able to provide evidence of the ability to apply specialist interpersonal and counseling interview skills. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance Requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints.  Demonstrated evidence is required of the ability to:

- interviewed at least 3 different clients using specialized interpersonal communication and counseling interviewing skills, including:

- micro-skills and communication techniques, including:

  - attending behaviors active listening,

  - reflection of content, summarizing

  - questioning skills open, closed, simple and compound questions

  - client observation skills

  - noting and reflecting skills

  - providing client feedback

- specialized counseling interviewing skills, including:

  - challenging

  - reframing

  - focusing

- integrated clear case note taking into the interview process

- Completed a structured process of self-reflection and evaluation of own communication used during the 3 interviews.