|  | Assessment Evidence Guide For HVACR Technician Level-3 Level-3 Equivalent to G-II Formative Assessment - Install Residential Air Conditioner |
| --- | --- |
|  | Date 13-14 April, 2023 Lahore |
|  | National Vocational & Technical Training Commission |

Instruction Sheet for the Candidate

| Qualification | HVACR Technician Level-3 |
| --- | --- |
| Competency Standard(s) | Install Residential Air Conditioner |
| Purpose of Assessment | Formative Assessment |

| Candidate Details | Name________________________________________________________ Registration/Roll Number_____________________________________________ |
| --- | --- |
| Guidance for Candidate | To meet this standard, you are required to complete the following within the given time frame 8 Hrs. (for practical demonstration & assessment): - Install Window Air Conditioner. - Install Split Air Conditioner / DC Inverter. |
| Time: 8 Hrs. | During a practical assessment, under observation by an assessor, you are required to - Select tools, equipment and related accessories according to job requirements - Mark the location on the wall where Window Air Conditioner to be installed according to Unit specifications and client requirements - Make opening at the marked area on the wall. - Fix Iron / wooden frame in the opening firmly and insert in it the Air Conditioner cover according to the instructional manual and standards. - Install the Air conditioner in the framed opening with standard slope so that condensate water drops outside. - Cover / Seal side air gaps of opening with insulation material. - Fix the fancy wooden border around the Air conditioner grill as per client’s requirement. - Fix the Air Conditioner condensate drain pipe and put it into main sewerage line. - Arrange power supply with circuit breaker near the Air Conditioner. - Make sure that all packing materials - Cardboard, Styrofoam, Tape and Plastic Film, have been removed from the site after installation. - Switch on the Air Conditioner and check Air Conditioner performance as per capacity and specifications. - Select tools, equipment and related accessories according to job requirements. - Select and mark the area on the wall where Indoor and Outdoor units are to be installed according to Unit specifications and client requirements. |
| Minimum Evidence Required |  |

|  | - Perform physical inspection of indoor and outdoor unit according to unit specifications. - Make opening for the refrigerant pipes, condensate pipe and control wires to pass through. - Mount the Indoor unit wall mounting plate according to manufacturer specifications and install the Indoor unit on it. - Prepare base for the fixing of Outdoor unit according to manufacturer specifications and fix the Outdoor unit there. - Connect the refrigerant pipes amongst both indoor and outdoor units, supply and control wires according to manufacturer manual. - Add additional refrigerant for additional piping according to manufacturer recommendations. - Make oil trap in copper pipe as per site requirement. - Perform leak test, evacuation procedure, charge refrigerant and open the service valves. - Insulate the joints and refrigerant pipes according to standards and manufacturer installation manual. - Make sure that all packing materials - Cardboard, Styrofoam, Tape and Plastic film, have been removed. - Switch on the Air Conditioner and check performance as per capacity and specifications. |
| --- | --- |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | HVACR Technician Level-3 |
| Competency Standards | Install Residential Air Conditioner |
| Assessment Task | - Install Window Air Conditioner. - Install Split Air Conditioner / DC Inverter. |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Select tools, equipment and related accessories according to job requirements |  |  |
| - Mark the location on the wall where Window Air Conditioner to be installed according to Unit specifications and client requirements |  |  |
| - Make opening at the marked area on the wall |  |  |
| - Fix Iron / wooden frame in the opening firmly and insert in it the Air Conditioner cover according to the instructional manual and standards |  |  |
| - Install the Air conditioner in the framed opening with standard slope so that condensate water drops outside |  |  |
| - Cover / Seal side air gaps of opening with insulation material |  |  |
| - Fix the fancy wooden border around the Air conditioner grill as per client’s requirement |  |  |
| - Fix the Air Conditioner condensate drain pipe and put it into main sewerage line |  |  |
| - Arrange power supply with circuit breaker near the Air Conditioner |  |  |
| - Make sure that all packing materials - Cardboard, Styrofoam, Tape and Plastic Film, have been removed from the site after installation |  |  |
| - Switch on the Air Conditioner and check Air Conditioner performance as per capacity and specifications |  |  |
| - Select tools, equipment and related accessories according to job requirements |  |  |
| - Select and mark the area on the wall where Indoor and Outdoor units are to be installed according to Unit specifications and client requirements |  |  |
| - Perform physical inspection of indoor and outdoor unit according to unit specifications |  |  |
| - Make opening for the refrigerant pipes, condensate pipe and control wires to pass through |  |  |
| - Mount the Indoor unit wall mounting plate according to manufacturer specifications and install the Indoor unit on it |  |  |
| - Prepare base for the fixing of Outdoor unit according to manufacturer specifications and fix the Outdoor unit there |  |  |
| - Connect the refrigerant pipes amongst both indoor and outdoor units, supply and control wires according to manufacturer manual |  |  |
| - Add additional refrigerant for additional piping according to manufacturer recommendations. |  |  |
| - Make oil trap in copper pipe as per site requirement |  |  |
| - Perform leak test, evacuation procedure, charge refrigerant and open the service valves |  |  |
| - Insulate the joints and refrigerant pipes according to standards and manufacturer installation manual |  |  |
| - Make sure that all packing materials - Cardboard, Styrofoam, Tape and Plastic film, have been removed |  |  |
| - Switch on the Air Conditioner and check performance as per capacity and specifications |  |  |

Candidate’s Signature___________________		Assessor’s Signature_____________________

Date: ________________________________

Assessors Judgment Guide

| Qualification | HVACR Technician Level-3 |
| --- | --- |
| Competency Standard(s) | Install Residential Air Conditioner |
| Candidate Details | Name: ____________________________________________________________________ Registration/Roll Number: ________________________Signature: ____________________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor________________________ Assessor’s code: ___________________ Signature: __________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Install Window Air Conditioner. - Install Split Air Conditioner / DC Inverter. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Select tools, equipment and related accessories according to job requirements |  |  |  |  |  |
|  | Mark the location on the wall where Window Air Conditioner to be installed according to Unit specifications and client requirements |  |  |  |  |  |
|  | Make opening at the marked area on the wall |  |  |  |  |  |
|  | Fix Iron / wooden frame in the opening firmly and insert in it the Air Conditioner cover according to the instructional manual and standards |  |  |  |  |  |
|  | Install the Air conditioner in the framed opening with standard slope so that condensate water drops outside |  |  |  |  |  |
|  | Cover / Seal side air gaps of opening with insulation material |  |  |  |  |  |
|  | Fix the fancy wooden border around the Air conditioner grill as per client’s requirement |  |  |  |  |  |
|  | Fix the Air Conditioner condensate drain pipe and put it into main sewerage line |  |  |  |  |  |
|  | Arrange power supply with circuit breaker near the Air Conditioner |  |  |  |  |  |
|  | Make sure that all packing materials - Cardboard, Styrofoam, Tape and Plastic Film, have been removed from the site after installation |  |  |  |  |  |
|  | Switch on the Air Conditioner and check Air Conditioner performance as per capacity and specifications |  |  |  |  |  |
|  | Select tools, equipment and related accessories according to job requirements |  |  |  |  |  |
|  | Select and mark the area on the wall where Indoor and Outdoor units are to be installed according to Unit specifications and client requirements |  |  |  |  |  |
|  | Perform physical inspection of indoor and outdoor unit according to unit specifications |  |  |  |  |  |
|  | Make opening for the refrigerant pipes, condensate pipe and control wires to pass through |  |  |  |  |  |
|  | Mount the Indoor unit wall mounting plate according to manufacturer specifications and install the Indoor unit on it |  |  |  |  |  |
|  | Prepare base for the fixing of Outdoor unit according to manufacturer specifications and fix the Outdoor unit there |  |  |  |  |  |
|  | Connect the refrigerant pipes amongst both indoor and outdoor units, supply and control wires according to manufacturer manual |  |  |  |  |  |
|  | Add additional refrigerant for additional piping according to manufacturer recommendations. |  |  |  |  |  |
|  | Make oil trap in copper pipe as per site requirement |  |  |  |  |  |
|  | Perform leak test, evacuation procedure, charge refrigerant and open the service valves |  |  |  |  |  |
|  | Insulate the joints and refrigerant pipes according to standards and manufacturer installation manual |  |  |  |  |  |
|  | Make sure that all packing materials - Cardboard, Styrofoam, Tape and Plastic film, have been removed |  |  |  |  |  |
|  | Switch on the Air Conditioner and check performance as per capacity and specifications |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | HVACR Technician Level-3 |
| --- | --- |
| Competency Standard(s) | Install Residential Air Conditioner |
| Candidate Details | Name: _____________________________________________________________________ Registration/Roll Number: __________________ Candidate Signature: ________________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor: __________________________Assessor’s code: _________________ Signature of the Assessor: _______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | What type of refrigerant used in air conditioner? |  |  |
|  | Define air conditioning. |  |  |
|  | Define selector switch. |  |  |
|  | Define split air conditioner. |  |  |
|  | Define compressor. |  |  |
|  | What is the function of over load? |  |  |
|  | Write name any three types of relays. |  |  |
|  | What is the function of drier? |  |  |
|  | Which type of meter used for checking Ampere? |  |  |
|  | Which type of refrigerant used in split DC inverter? |  |  |

Feedback to the Candidate

Candidate’s Signature ________________________Assessor’s Signature_________________________