|  | Assessment Evidence Guide For HVACR Technician Level-3 Equivalent to G-II Summative Assessment |
| --- | --- |
|  | Date 13-14 April, 2023 Lahore |
|  | National Vocational & Technical Training Commission |

Instruction Sheet for the Candidate

| Qualification | National Vocation Certificate Level 03 Technician in HVACR Technology |
| --- | --- |
| Competency Standard | - Install Residential Air Conditioner - Maintain Residential Air Conditioner - Perform Installation Commercial Refrigeration Units - Maintain Commercial Refrigeration Units - Perform Installation Commercial Air Conditioning Systems - Maintain Commercial Air Conditioning System |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within 04 Hrs. time frame (for practical demonstration & assessment): Install the split Air conditioner and also servicing the Fan Coil Unit (FCU) |
| Time: 04 Hrs. | During a practical assessment, under observation by an assessor, you are required to |
| Minimum Evidence Required | - Select tools, equipment and related accessories according to job requirements - Mark the location on the wall where Split Air Conditioner to be installed according to Unit specifications. - Select and mark the area on the wall where Indoor and Outdoor units are to be installed according to Unit specifications and client requirements. - Perform physical inspection of indoor and outdoor unit according to unit specifications. - Make opening for the refrigerant pipes, condensate pipe and control wires to pass through. - Mount the Indoor unit wall mounting plate according to manufacturer specifications and install the Indoor unit on it. - Prepare base for the fixing of Outdoor unit according to manufacturer specifications and fix the Outdoor unit there. - Perform leak test, evacuation procedure, charge refrigerant and open the service valves. - Insulate the joints and refrigerant pipes according to standards and manufacturer installation manual. - Switch on the Air Conditioner and check performance as per capacity and specifications. - Shutdown the FCU unit as per the job requirement. - Check each component of the unit. - Follow operational manual to diagnose the fault. - Select tools, equipment, and related accessories according to the requirements and standard. - Rectify the diagnosed fault by repairing / replacing the components. - Switch ON the FCU unit and recheck the performance after repair. - Record all the parameters as per standard. |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | National Vocation Certificate Level 03 Technician in HVACR |
| Competency Standard | - Install Residential Air Conditioner - Maintain Residential Air Conditioner - Perform Installation Commercial Refrigeration Units - Maintain Commercial Refrigeration Units - Perform Installation Commercial Air Conditioning Systems - Maintain Commercial Air Conditioning System |
| Purpose of Assessment | Summative Assessment |
| Assessment Task | Install the split Air conditioner and also servicing the Fan Coil Unit (FCU) |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Select tools, equipment and related accessories according to job requirements |  |  |
| - Mark the location on the wall where Split Air Conditioner to be installed according to Unit specifications. |  |  |
| - Select and mark the area on the wall where Indoor and Outdoor units are to be installed according to Unit specifications and client requirements. |  |  |
| - Perform physical inspection of indoor and outdoor unit according to unit specifications. |  |  |
| - Make opening for the refrigerant pipes, condensate pipe and control wires to pass through. |  |  |
| - Mount the Indoor unit wall mounting plate according to manufacturer specifications and install the Indoor unit on it. |  |  |
| - Prepare base for the fixing of Outdoor unit according to manufacturer specifications and fix the Outdoor unit there. |  |  |
| - Perform leak test, evacuation procedure, charge refrigerant and open the service valves. |  |  |
| - Insulate the joints and refrigerant pipes according to standards and manufacturer installation manual. |  |  |
| - Switch on the Air Conditioner and check performance as per capacity and specifications. |  |  |
| - Shutdown the FCU unit as per the job requirement. |  |  |
| - Check each component of the unit. |  |  |
| - Follow operational manual to diagnose the fault. |  |  |
| - Select tools, equipment, and related accessories according to the requirements and standard. |  |  |
| - Rectify the diagnosed fault by repairing / replacing the components. |  |  |
| - Switch ON the FCU unit and recheck the performance after repair. |  |  |
| - Record all the parameters as per standard |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: ______________________________

Assessors Judgment Guide

| Qualification | National Vocation Certificate Level 03 Technician in HVACR |
| --- | --- |
| Competency Standard | - Install Residential Air Conditioner - Maintain Residential Air Conditioner - Perform Installation Commercial Refrigeration Units - Maintain Commercial Refrigeration Units - Perform Installation Commercial Air Conditioning Systems - Maintain Commercial Air Conditioning System |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________ Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________________________________ Assessor’s code: ____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | Joint copper tube using permanent method and perform compressor test |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Select tools, equipment and related accessories according to job requirements |  |  |  |  |  |
|  | Mark the location on the wall where Split Air Conditioner to be installed according to Unit specifications. |  |  |  |  |  |
|  | Select and mark the area on the wall where Indoor and Outdoor units are to be installed according to Unit specifications and client requirements. |  |  |  |  |  |
|  | Perform physical inspection of indoor and outdoor unit according to unit specifications. |  |  |  |  |  |
|  | Make opening for the refrigerant pipes, condensate pipe and control wires to pass through. |  |  |  |  |  |
|  | Mount the Indoor unit wall mounting plate according to manufacturer specifications and install the Indoor unit on it. |  |  |  |  |  |
|  | Prepare base for the fixing of Outdoor unit according to manufacturer specifications and fix the Outdoor unit there. |  |  |  |  |  |
|  | Perform leak test, evacuation procedure, charge refrigerant and open the service valves. |  |  |  |  |  |
|  | Insulate the joints and refrigerant pipes according to standards and manufacturer installation manual. |  |  |  |  |  |
|  | Switch on the Air Conditioner and check performance as per capacity and specifications. |  |  |  |  |  |
|  | Shutdown the FCU unit as per the job requirement. |  |  |  |  |  |
|  | Check each component of the unit. |  |  |  |  |  |
|  | Follow operational manual to diagnose the fault. |  |  |  |  |  |
|  | Select tools, equipment, and related accessories according to the requirements and standard. |  |  |  |  |  |
|  | Rectify the diagnosed fault by repairing / replacing the components. |  |  |  |  |  |
|  | Switch ON the FCU unit and recheck the performance after repair. |  |  |  |  |  |
|  | Record all the parameters as per standard. |  |  |  |  |  |
| Competent |  |  | Not yet Competent |  |  |  |

Knowledge Assessment

| Qualification | National Vocation Certificate Level 03 Technician in HVACR |
| --- | --- |
| Competency Standard | - Install Residential Air Conditioner - Maintain Residential Air Conditioner - Perform Installation Commercial Refrigeration Units - Maintain Commercial Refrigeration Units - Perform Installation Commercial Air Conditioning Systems - Maintain Commercial Air Conditioning System |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature: ___________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor: ________________________________________________ Assessor’s code: ____________________________________________________ Signature of the Assessor: _______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Which gas is used in split air conditioner? |  |  |
|  | Describe the procedure of Vaccum? |  |  |
|  | What does stand for FCU? |  |  |
|  | What is the gas charging pressure in split air conditioner? |  |  |
| 5 | What is the purpse of compressor in split air conditioner? |  |  |
| 6 | What is the purpse of FCU in commercial unit? |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature _________________