Instruction Sheet for the Candidate

| Qualification | National Vocation Certificate Level 02 Assistant Technician in HVACR Technology |
| --- | --- |
| Competency Standard(s) | 0715M&MT09 – Perform Welding Process 1022OHS01 - Apply Occupational Health and Safety (OHS) in HVAC System 1022OHS02 - Maintain safe work environment |
| Purpose of Assessment | Formative / Devlopmental Assessment |

| Candidate Details | Name________________________________________________________ Registration/Roll Number_____________________________________________ |
| --- | --- |
| Guidance for Candidate | To meet this standard you are required to complete the following within the given time frame 5Hrs. (for practical demonstration & assessment): - Identify and make Oxy-Acetylene Flames - Perform Flange joint - Make MS Butt joint by Oxy Acetylene flame - Perform MS Lap joint by Oxy Acetylene flame - Make brazing joints - Prepare soldering joint - Make Arc Welding joint |
| Time: 5Hrs | During a practical assessment, under observation by an assessor, you are required to - Identify & Report OHS Hazards expected to be occurred during operation & maintenance at work place - Observe & maintain safe working environment - Open oxygen/acetylene cylinder valve - Check leakage of regulators - Open valve of acetylene from welding torch valve and light the torch and make neutral flame - Bend of MS sheet at 90o to make flange - Put both pieces on welding table without gap - Grip the pieces from flange - Make flame and melt the edges and continue this process to complete joint - Clean and straight the both edges of base metal - Adjust both gases pressures - Set base metals with required gap - Melt the edge of base metal and fill the gap by fusing filler rod on both sides - Complete the bead after tacking continue the puddle making and filling of gap - Clean and straight the both edges of base metal - Adjust both gases pressure - Set base metals in overlap position - Make puddle at edges and fill the gap by molten filler metal - Complete the joint by using filler rod - Clean and straight the both edges of base metal - Heat up the edges of base metal up to red hot condition - Use the brazing flux with cleaned nonferrous filler rod - Complete the process of joint by using fore hand technique - Make permanent joint of different materials using nonferrous filler metal having melting point less than 800oF with the help of soldering iron. - Heat copper made soldering iron in furnace - Heat copper made soldering iron by electricity - Clean the surfaces to be joint - Apply soldering flux on the clean surfaces - Clean and straight the edges of base metal - Set the both pieces of base metal with sufficient gap - Switch on the welding machine and set required current - Tack both ends of base metal and clean the slag from both tacks - Start bead from one end of base metal with proper length of Arc and proper speed - Remove slag from bead |
| Minimum Evidence Required |  |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | National Vocation Certificate Level 02 Assistant Technician in HVACR Technology |
| Competency Standards | 0715M&MT09 – Perform Welding Process |
|  | 1022OHS01 - Apply Occupational Health and Safety (OHS) in HVAC System |
|  | 1022OHS02 - Maintain safe work environment |
| Assessment Task | 1. Identify and make Oxy-Acetylene Flames 2. Perform Flange joint 3. Make MS Butt joint by Oxy Acetylene flame 4. Perform MS Lap joint by Oxy Acetylene flame 5. Make brazing joints 6. Prepare soldering joint 7.Make Arc Welding joint |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Identify & Report OHS Hazards expected to be occurred during operation & maintenance at work place |  |  |
| - Observe & maintain safe working environment |  |  |
| - Open oxygen/acetylene cylinder valve |  |  |
| - Check leakage of regulators |  |  |
| - Open valve of acetylene from welding torch valve and light the torch and make neutral flame |  |  |
| - Bend of MS sheet at 90o to make flange |  |  |
| - Put both pieces on welding table without gap |  |  |
| - Grip the pieces from flange |  |  |
| - Make flame and melt the edges and continue this process to complete joint |  |  |
| - Clean and straight the both edges of base metal |  |  |
| - Adjust both gases pressures |  |  |
| - Set base metals with required gap |  |  |
| - Melt the edge of base metal and fill the gap by fusing filler rod on both sides |  |  |
| - Complete the bead after tacking continue the puddle making and filling of gap |  |  |
| - Clean and straight the both edges of base metal |  |  |
| - Adjust both gases pressure |  |  |
| - Set base metals in overlap position |  |  |
| - Make puddle at edges and fill the gap by molten filler metal |  |  |
| - Complete the joint by using filler rod |  |  |
| - Clean and straight the both edges of base metal |  |  |
| - Heat up the edges of base metal up to red hot condition |  |  |
| - Use the brazing flux with cleaned nonferrous filler rod |  |  |
| - Complete the process of joint by using fore hand technique |  |  |
| - Make permanent joint of different materials using nonferrous filler metal having melting point less than 800oF with the help of soldering iron. |  |  |
| - Heat copper made soldering iron in furnace |  |  |
| - Heat copper made soldering iron by electricity |  |  |
| - Clean the surfaces to be joint |  |  |
| - Apply soldering flux on the clean surfaces |  |  |
| - Clean and straight the edges of base metal |  |  |
| - Set the both pieces of base metal with sufficient gap |  |  |
| - Switch on the welding machine and set required current |  |  |
| - Tack both ends of base metal and clean the slag from both tacks |  |  |
| - Start bead from one end of base metal with proper length of Arc and proper speed |  |  |
| - Remove slag from bead |  |  |

Candidate’s Signature___________________   Assessor’s Signature_____________________

Date: ________________________________

Assessors Judgment Guide

| Qualification | National Vocation Certificate Level 02 Assistant Technician in HVACR Technology |
| --- | --- |
| Competency Standard(s) | 0715M&MT09 – Perform Welding Process 1022OHS01 - Apply Occupational Health and Safety (OHS) in HVAC System 1022OHS02 - Maintain safe work environment |
| Candidate Details | Name:______________________________________________________________________ Registration/Roll Number: ________________________ Signature: ____________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________ Assessor’s code:___________________ Signature:__________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task | 1. Identify and make Oxy-Acetylene Flames 2. Perform Flange joint 3. Make MS Butt joint by Oxy Acetylene flame 4. Perform MS Lap joint by Oxy Acetylene flame 5. Make brazing joints 6. Prepare soldering joint 7. Make Arc Welding joint |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  | Yes | No | Remarks |
| - Identify & Report OHS Hazards expected to be occurred during operation & maintenance at work place |  |  |  |  |  |
| - Observe & maintain safe working environment |  |  |  |  |  |
| - Open oxygen/acetylene cylinder valve |  |  |  |  |  |
| - Check leakage of regulators |  |  |  |  |  |
| - Open valve of acetylene from welding torch valve and light the torch and make neutral flame |  |  |  |  |  |
| - Bend of MS sheet at 90o to make flange |  |  |  |  |  |
| - Put both pieces on welding table without gap |  |  |  |  |  |
| - Grip the pieces from flange |  |  |  |  |  |
| - Make flame and melt the edges and continue this process to complete joint |  |  |  |  |  |
| - Clean and straight the both edges of base metal |  |  |  |  |  |
| - Adjust both gases pressures |  |  |  |  |  |
| - Set base metals with required gap |  |  |  |  |  |
| - Melt the edge of base metal and fill the gap by fusing filler rod on both sides |  |  |  |  |  |
| - Complete the bead after tacking continue the puddle making and filling of gap |  |  |  |  |  |
| - Clean and straight the both edges of base metal |  |  |  |  |  |
| - Adjust both gases pressure |  |  |  |  |  |
| - Set base metals in overlap position |  |  |  |  |  |
| - Make puddle at edges and fill the gap by molten filler metal |  |  |  |  |  |
| - Complete the joint by using filler rod |  |  |  |  |  |
| - Clean and straight the both edges of base metal |  |  |  |  |  |
| - Heat up the edges of base metal up to red hot condition |  |  |  |  |  |
| - Use the brazing flux with cleaned nonferrous filler rod |  |  |  |  |  |
| - Complete the process of joint by using fore hand technique |  |  |  |  |  |
| - Make permanent joint of different materials using nonferrous filler metal having melting point less than 800oF with the help of soldering iron. |  |  |  |  |  |
| - Heat copper made soldering iron in furnace |  |  |  |  |  |
| - Heat copper made soldering iron by electricity |  |  |  |  |  |
| - Clean the surfaces to be joint |  |  |  |  |  |
| - Apply soldering flux on the clean surfaces |  |  |  |  |  |
| - Clean and straight the edges of base metal |  |  |  |  |  |
| - Set the both pieces of base metal with sufficient gap |  |  |  |  |  |
| - Switch on the welding machine and set required current |  |  |  |  |  |
| - Tack both ends of base metal and clean the slag from both tacks |  |  |  |  |  |
| - Start bead from one end of base metal with proper length of Arc and proper speed |  |  |  |  |  |
| - Remove slag from bead |  |  |  |  |  |
| - Identify & Report OHS Hazards expected to be occurred during operation & maintenance at work place |  |  |  |  |  |
| Competent |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | National Vocation Certificate Level 02 Assistant Technician in HVACR Technology |
| --- | --- |
| Competency Standard(s) | 0715M&MT09 – Perform Welding Process 1022OHS01 - Apply Occupational Health and Safety (OHS) in HVAC System 1022OHS02 - Maintain safe work environment |
| Candidate Details | Name:_______________________________________________________________________ Registration/Roll Number: __________________ Candidate Signature:__________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor:__________________________ Assessor’s code:_________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Enlist 5 Personal Protective Equipment`s (PPE`s). |  |  |
|  | Is that the carburizing flame add small quantity of oxygen gas? |  |  |
|  | Explain the carburizing and Oxidizing flame? |  |  |
|  | Explain the neutral flame? |  |  |
|  | Explain the angle of welding torch be kept at 45O? |  |  |
|  | Explain the Inner core of carburizing flame? |  |  |
|  | Is it the neutral flame for thin sheets an oxidizing flame for welding of thick sheet? |  |  |

Feedback to the Candidate

Candidate’s Signature________________________ Assessor’s Signature _________________________