Instruction Sheet for the Candidate

| Qualification | National Vocation Certificate Level 02 Assistant Technician in HVACR Technology |
| --- | --- |
| Competency Standard(s) | 0715M&MT06 – Perform Threading with Tap & Die 1022OHS01 - Apply Occupational Health and Safety (OHS) in HVAC System 1022OHS02 - Maintain safe work environment |
| Purpose of Assessment | Formative / Devlopmental Assessment |

| Candidate Details | Name________________________________________________________ Registration/Roll Number_____________________________________________ |
| --- | --- |
| Guidance for Candidate | To meet this standard you are required to complete the following within the given time frame 2Hrs. (for practical demonstration & assessment): - 1 Plan and prepare for Thread work - Cut internal threads using Tap - Cut external threads using Die - Complete the work |
| Time: 2Hrs | During a practical assessment, under observation by an assessor, you are required to - Identify & Report OHS Hazards expected to be occurred during operation & maintenance at work place - Observe & maintain safe working environment - Identify job material and tools required - Plan sequence of procedural steps to complete the job - Clamp the work piece in bench vice jaws - Prepare the work piece using file up to required sizes - Mark the location of central point of hole - Punch the center of hole - Drill the hole as per required tap drill size - Hold the tap in tap handle - Re-clamp the work piece in vice jaws for taping - Locate the tap in hole and rotate it clockwise direction carefully - Use tap no 1, 2 and 3 after one and another - Check the thread with thread plug gauge - Cut the job material in to required size - Chamfer the sharp edge of job by filing - Select the proper threading die as per requirement of work - Locate the threading die in the diestock - Clamp the work piece in V-block - Hold the V-block in vice - Cut threads up to required length - Check the threads with thread ring gauge - Complete the work in accordance with requirement - Clean and clear your work area |
| Minimum Evidence Required |  |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | National Vocation Certificate Level 02 Assistant Technician in HVACR Technology |
| Competency Standards | 0715M&MT06 – Perform Threading with Tap & Die |
|  | 1022OHS01 - Apply Occupational Health and Safety (OHS) in HVAC System |
|  | 1022OHS02 - Maintain safe work environment |
| Assessment Task | - Plan and prepare for Thread work - Cut internal threads using Tap - Cut external threads using Die - Complete the work |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Identify & Report OHS Hazards expected to be occurred during operation & maintenance at work place |  |  |
| - Observe & maintain safe working environment |  |  |
| - Identify job material and tools required |  |  |
| - Plan sequence of procedural steps to complete the job |  |  |
| - Clamp the work piece in bench vice jaws |  |  |
| - Prepare the work piece using file up to required sizes |  |  |
| - Mark the location of central point of hole |  |  |
| - Punch the center of hole |  |  |
| - Drill the hole as per required tap drill size |  |  |
| - Hold the tap in tap handle |  |  |
| - Re-clamp the work piece in vice jaws for taping |  |  |
| - Locate the tap in hole and rotate it clockwise direction carefully |  |  |
| - Use tap no 1, 2 and 3 after one and another |  |  |
| - Check the thread with thread plug gauge |  |  |
| - Cut the job material in to required size |  |  |
| - Chamfer the sharp edge of job by filing |  |  |
| - Select the proper threading die as per requirement of work |  |  |
| - Locate the threading die in the diestock |  |  |
| - Clamp the work piece in V-block |  |  |
| - Hold the V-block in vice |  |  |
| - Cut threads up to required length |  |  |
| - Check the threads with thread ring gauge |  |  |
| - Complete the work in accordance with requirement |  |  |
| - Clean and clear your work area |  |  |

Candidate’s Signature________________________ Assessor’s Signature _________________________

Date: ________________________________

Assessors Judgment Guide

| Qualification | National Vocation Certificate Level 02 Assistant Technician in HVACR Technology |
| --- | --- |
| Competency Standard(s) | 0715M&MT06 – Perform Threading with Tap & Die 1022OHS01 - Apply Occupational Health and Safety (OHS) in HVAC System 1022OHS02 - Maintain safe work environment |
| Candidate Details | Name:______________________________________________________________________ Registration/Roll Number: ________________________ Signature: ____________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________ Assessor’s code:___________________ Signature:__________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task | - Plan and prepare for Thread work - Cut internal threads using Tap - Cut external threads using Die - Complete the work |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  | Yes | No | Remarks |
| - Identify & Report OHS Hazards expected to be occurred during operation & maintenance at work place |  |  |  |  |  |  |
| - Observe & maintain safe working environment |  |  |  |  |  |  |
| - Identify job material and tools required |  |  |  |  |  |  |
| - Plan sequence of procedural steps to complete the job |  |  |  |  |  |  |
| - Clamp the work piece in bench vice jaws |  |  |  |  |  |  |
| - Prepare the work piece using file up to required sizes |  |  |  |  |  |  |
| - Mark the location of central point of hole |  |  |  |  |  |  |
| - Punch the center of hole |  |  |  |  |  |  |
| - Drill the hole as per required tap drill size |  |  |  |  |  |  |
| - Hold the tap in tap handle |  |  |  |  |  |  |
| - Re-clamp the work piece in vice jaws for taping |  |  |  |  |  |  |
| - Locate the tap in hole and rotate it clockwise direction carefully |  |  |  |  |  |  |
| - Use tap no 1, 2 and 3 after one and another |  |  |  |  |  |  |
| - Check the thread with thread plug gauge |  |  |  |  |  |  |
| - Cut the job material in to required size |  |  |  |  |  |  |
| - Chamfer the sharp edge of job by filing |  |  |  |  |  |  |
| - Select the proper threading die as per requirement of work |  |  |  |  |  |  |
| - Locate the threading die in the diestock |  |  |  |  |  |  |
| - Clamp the work piece in V-block |  |  |  |  |  |  |
| - Hold the V-block in vice |  |  |  |  |  |  |
| - Cut threads up to required length |  |  |  |  |  |  |
| - Check the threads with thread ring gauge |  |  |  |  |  |  |
| - Complete the work in accordance with requirement |  |  |  |  |  |  |
| - Clean and clear your work area |  |  |  |  |  |  |
| Competent |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | National Vocation Certificate Level 02 Assistant Technician in HVACR Technology |
| --- | --- |
| Competency Standard(s) | 0715M&MT06 – Perform Threading with Tap & Die 1022OHS01 - Apply Occupational Health and Safety (OHS) in HVAC System 1022OHS02 - Maintain safe work environment |
| Candidate Details | Name:_______________________________________________________________________ Registration/Roll Number: __________________ Candidate Signature:__________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor:__________________________ Assessor’s code:_________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Enlist 5 Personal Protective Equipment`s (PPE`s). |  |  |
|  | Explain the thread cutting methods? |  |  |
|  | Enlist the process of pitch & lead of thread? |  |  |
|  | What is a function of lubricant in threading process? |  |  |
|  | Enlist the thread and their uses? |  |  |
|  | Enlist the cutting tools which are used in metal work? |  |  |

Feedback to the Candidate

Candidate’s Signature________________________ Assessor’s Signature _________________________