Instruction Sheet for the Candidate

| Qualification | # National Vocational Certificate Level 2 in Instrumentation Technology |
| --- | --- |
| Competency Standard | Operate Measuring Instruments(Level 2) |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within time frame (for practical demonstration & assessment): - Operate Ampere meter - Operate Voltmeter - Operate ohm meter |
| Time: 03 Hrs. | During a practical assessment, under observation by an assessor, you are required to |
| Minimum Evidence Required | Operate Ampere meter - Identify ampere meter. - Make a parallel circuit with three different resistors on breadboard and connect with the power supply. - Adjust proper range setting of ampere meter as per load. - Connect ampere meter in series to each resistor to measure the current. - Turn on the supply and note the reading of current against each resistor. Operate Voltmeter - Identify voltmeter. - Make a series circuit with three different resistors on bread board. - Adjust proper range setting of Voltmeter as per load. - Connect voltmeter across any resistor in circuit - Turn on the supply and note the reading of voltage drop against each resistor. Operate ohm meter - Identify Ohm meter. - Adjust proper range of ohm meter as per resistance value. - Disconnect the resistor from the circuit - Connect ohm meter with resistor in series and parallel combination. - Record the reading. |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | # National Vocational Certificate Level 2 in Instrumentation Technology |
| Competency Standard | Operate Measuring Instruments (Level 2) |
| Purpose of Assessment | Formative Assessment |
| Assessment Task | - Operate Ampere meter - Operate Voltmeter - Operate ohm meter |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Identify ampere meter. |  |  |
| - Make a parallel circuit with three different resistors on breadboard and connect with the power supply. |  |  |
| - Adjust proper range setting of ampere meter as per load. |  |  |
| - Connect ampere meter in series to each resistor to measure the current. |  |  |
| - Turn on the supply and note the reading of current against each resistor. |  |  |
| - Identify voltmeter. |  |  |
| - Make a series circuit with three different resistors on bread board. |  |  |
| - Adjust proper range setting of Voltmeter as per load. |  |  |
| - Connect voltmeter across any resistor in circuit |  |  |
| - Turn on the supply and note the reading of voltage drop against each resistor. |  |  |
| - Identify Ohm meter. |  |  |
| - Adjust proper range of ohm meter as per resistance value. |  |  |
| - Disconnect the resistor from the circuit |  |  |
| - Connect ohm meter with resistor in series and parallel combination. |  |  |
| - Record the reading. |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | # National Vocational Certificate Level 2 in Instrumentation Technology |
| --- | --- |
| Competency Standard | Operate Measuring Instruments (Level 2) |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor________________________________________________ Assessor’s code:____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Operate Ampere meter - Operate Voltmeter - Operate ohm meter |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Identified ampere meter. |  |  |  |  |  |
|  | Made a parallel circuit with three different resistors on breadboard and connect with the power supply. |  |  |  |  |  |
|  | Adjusted proper range setting of ampere meter as per load. |  |  |  |  |  |
|  | Connected ampere meter in series to each resistor to measure the current. |  |  |  |  |  |
|  | Turned on the supply and note the reading of current against each resistor. |  |  |  |  |  |
|  | Identified volt meter. |  |  |  |  |  |
|  | Made a series circuit with three different resistors on bread board. |  |  |  |  |  |
|  | Adjusted proper range setting of Volt meter as per load. |  |  |  |  |  |
|  | Connected volt meter across any resistor in circuit |  |  |  |  |  |
|  | Turned on the supply and note the reading of voltage drop against each resistor. |  |  |  |  |  |
|  | Identified Ohm meter. |  |  |  |  |  |
|  | Adjusted proper range of ohm meter as per resistance value. |  |  |  |  |  |
|  | Disconnected the resistor from the circuit |  |  |  |  |  |
|  | Connected ohm meter with resistor in series and parallel combination. |  |  |  |  |  |
|  | Recorded the reading. |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | # National Vocational Certificate Level 2 in Instrumentation Technology |
| --- | --- |
| Competency Standard | Operate Measuring Instruments (Level 2) |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name:______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature:___________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor:________________________________________________ Assessor’s code:____________________________________________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | The unit of current is ________ |  |  |
|  | - Ampere - volt - ohm - farad |  |  |
|  | Galvano meter is used to ……………… |  |  |
|  | - Measure voltage - Sense current - Measure impedance - sense capacitance |  |  |
|  | Volt meter is used in series with load to measure voltage |  |  |
|  | - TRUE - FALSE |  |  |
|  | Ohm meter is used to measure |  |  |
|  | a) voltage b) current c) resistance d) capacitance |  |  |
|  | Describe the electric current and its formula |  |  |
|  | Describe voltage and its formula |  |  |
|  | State Ohm’s Law and express mathematical formula |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature _________________