Instruction Sheet for the Candidate

| Qualification | National Vocational Certificate Level 2 in Instrumentation Technology |
| --- | --- |
| Competency Standard | Make Cable/Wire Joints for single phase wiring (Level 2) |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within time frame (for practical demonstration & assessment): - Make Cross/Twist joint - Make Straight/Married joint - Make T-Joint - Make Rat tail joint - Make Britannia joint |
| Time: 03 Hrs. | During a practical assessment, under observation by an assessor, you are required to |
| Minimum Evidence Required | Make Cross/Twist joint - Select the cable. - Strip the wire according to 50mm. - Twist the conductors. - Solder the conductor - Insulate the joint Make Straight/Married joint - Select the cable. - Strip wire to 75mm according to joint requirement. - Intermingle the conductors to 60mm into each other. - Twist 60mm conductors leaving behind 15mm of each cable. - Solder the conductor. - Insulate the joint. Make T-Joint - Select the cable - Remove the insulation of cable 1 to 50mm from where a connection is required. - Separate conductors of cable 1 equally. - Take another 12 mm stripped wire 2. - Insert between two equally half conductors of cable 1 and twist. Half conductors clockwise and half anti-clock wise of cable 2 - Solder the joint Make Rat tail joint - Select the single conductor wires - Strip both the wires to 5mm - Twist the conductor - Insulate the joint - Solder the joint Make Britannia joint - Select the cable. - Strip both cables to 75mm. - Bend the tips of both cable right angle about to 6mm. - Hold the two cables overlap 50mm with tips in opposite direction. - Take another bare conductor of 1mm and wrap around in bothdirections to 6mm. - Solder the joint. - Insulate the joint. |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | National Vocational Certificate Level 2 in Instrumentation Technology |
| Competency Standard | Make Cable/Wire Joints for single phase wiring (Level 2) |
| Purpose of Assessment | Formative Assessment |
| Assessment Task | - Make Cross/Twist joint - Make Straight/Married joint - Make T-Joint - Make Rat tail joint - Make Britannia joint |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Select the cable. |  |  |
| - Strip the wire according to 50mm. |  |  |
| - Twist the conductors. |  |  |
| - Solder the conductor |  |  |
| - Insulate the joint |  |  |
| - Select the cable. |  |  |
| - Strip wire to 75mm according to joint requirement. |  |  |
| - Intermingle the conductors to 60mm into each other. |  |  |
| - Twist 60mm conductors leaving behind 15mm of each cable. |  |  |
| - Solder the conductor. |  |  |
| - Insulate the joint. |  |  |
| - Select the cable |  |  |
| - Remove the insulation of cable 1 to 50mm from where a connection is required. |  |  |
| - Separate conductors of cable 1 equally. |  |  |
| - Take another 12 mm stripped wire 2. |  |  |
| - Insert between two equally half conductors of cable 1 and twist. Half conductors clockwise and half anti-clock wise of cable 2 |  |  |
| - Solder the joint |  |  |
| - Select the single conductor wires |  |  |
| - Strip both the wires to 5mm |  |  |
| - Twist the conductor |  |  |
| - Insulate the joint |  |  |
| - Solder the joint |  |  |
| - Select the cable. |  |  |
| - Strip both cables to 75mm. |  |  |
| - Bend the tips of both cable right angle about to 6mm. |  |  |
| - Hold the two cables overlap 50mm with tips in opposite direction. |  |  |
| - Take another bare conductor of 1mm and wrap around in both directions to 6mm. |  |  |
| - Solder the joint. |  |  |
| - Insulate the joint. |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | National Vocational Certificate Level 2 in Instrumentation Technology |
| --- | --- |
| Competency Standard | Make Cable/Wire Joints for single phase wiring (Level 2) |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor________________________________________________ Assessor’s code:____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Make Cross/Twist joint - Make Straight/Married joint - Make T-Joint - Make Rat tail joint - Make Britannia joint |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Selected the cable. |  |  |  |  |  |
|  | Striped the wire according to 50mm. |  |  |  |  |  |
|  | Twisted the conductors. |  |  |  |  |  |
|  | Soldered the conductor |  |  |  |  |  |
|  | Insulated the joint |  |  |  |  |  |
|  | Selected the cable. |  |  |  |  |  |
|  | Striped wire to 75mm according to joint requirement. |  |  |  |  |  |
|  | Intermingled the conductors to 60mm into each other. |  |  |  |  |  |
|  | Twisted 60mm conductors leaving behind 15mm of each cable. |  |  |  |  |  |
|  | Soldered the conductor. |  |  |  |  |  |
|  | Insulated the joint. |  |  |  |  |  |
|  | Selected the cable |  |  |  |  |  |
|  | Removed the insulation of cable 1 to 50mm from where a connection is required. |  |  |  |  |  |
|  | Separated conductors of cable 1 equally. |  |  |  |  |  |
|  | Took another 12 mm stripped wire 2. |  |  |  |  |  |
|  | Inserted between two equally half conductors of cable 1 and twist. Half conductors clockwise and half anti-clock wise of cable 2 |  |  |  |  |  |
|  | Soldered the joint |  |  |  |  |  |
|  | Selected the single conductor wires |  |  |  |  |  |
|  | Striped both the wires to 5mm |  |  |  |  |  |
|  | Twisted the conductor |  |  |  |  |  |
|  | Insulated the joint |  |  |  |  |  |
|  | Soldered the joint |  |  |  |  |  |
|  | Selected the cable. |  |  |  |  |  |
|  | Striped both cables to 75mm. |  |  |  |  |  |
|  | Bended the tips of both cable right angle about to 6mm. |  |  |  |  |  |
|  | Hold the two cables overlap 50mm with tips in opposite direction. |  |  |  |  |  |
|  | Took another bare conductor of 1mm and wrap around in both directions to 6mm. |  |  |  |  |  |
|  | Soldered the joint. |  |  |  |  |  |
|  | Insulated the joint. |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | National Vocational Certificate Level 2 in Instrumentation Technology |
| --- | --- |
| Competency Standard | Make Cable/Wire Joints for single phase wiring (Level 2) |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name:______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature:___________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor:________________________________________________ Assessor’s code:____________________________________________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Explain the composition of solder and soldering flux |  |  |
|  | Describe the types of soldering |  |  |
|  | Plier is used to stripe the wire |  |  |
|  | - TRUE - FALSE |  |  |
|  | Differentiate between cable and wire |  |  |
|  | Define conductor |  |  |
|  | Enlist the types of wire joints |  |  |
|  | Define Britannia joint |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature _________________