Instruction Sheet for the Candidate

| Qualification | National Vocational Certificate Level 2 in Instrumentation Technology |
| --- | --- |
| Competency Standard | Prepare and Install Distribution Boards for single phase wiring (Level 2) |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name_____________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within time frame (for practical demonstration & assessment): - Prepare estimate for wiring material. - Prepare Distribution Board - Install Distribution Board and wiring |
| Time: 03 Hrs. | During a practical assessment, under observation by an assessor, you are required to |
| Minimum Evidence Required | Prepare estimate for wiring material. - Develop Basic Drawing - Perform Estimation of Materials - Calculate Labour Cost Prepare Distribution Board - Select Distribution board w.r.t. size, current rating, voltage, No. of C.Bs and phases. - Select Main Incoming Residual Current Circuit Breaker (RCCB) having minimum sensitivity. - Select outgoing Miniature Circuit Breaker (MCB) according to load - Provide space for future MCB - Select Voltmeter, ampere meter and indicator according to load and phases. - Select wire according to load and for wiring in Distribution Board (DB). - Make neutral common for all load - Connect accessories according to the circuit diagram Install Distribution Board and wiring - .Fix the distribution board - Take wire from DB to load without joint - Distribute load equally on all phases |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | National Vocational Certificate Level 2 in Instrumentation Technology |
| Competency Standard | Prepare and Install Distribution Boards for single phase wiring (Level 2) |
| Purpose of Assessment | Formative Assessment |
| Assessment Task | - Prepare estimate for wiring material. - Prepare Distribution Board - Install Distribution Board and wiring |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Develop Basic Drawing |  |  |
| - Perform Estimation of Materials |  |  |
| - Calculate Labour Cost |  |  |
| - Select Distribution board w.r.t. size, current rating, voltage, No. of C.Bs and phases. |  |  |
| - Select Main Incoming Residual Current Circuit Breaker (RCCB) having minimum sensitivity. |  |  |
| - Select outgoing Miniature Circuit Breaker (MCB) according to load |  |  |
| - Provide space for future MCB |  |  |
| - Select Voltmeter, ampere meter and indicator according to load and phases. |  |  |
| - Select wire according to load and for wiring in Distribution Board (DB). |  |  |
| - Make neutral common for all load |  |  |
| - Connect accessories according to the circuit diagram |  |  |
| - Fix the distribution board |  |  |
| - Take wire from DB to load without joint |  |  |
| - Distribute load equally on all phases |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | National Vocational Certificate Level 2 in Instrumentation Technology |
| --- | --- |
| Competency Standard | Prepare and Install Distribution Boards for single phase wiring (Level 2) |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor________________________________________________ Assessor’s code:____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Prepare estimate for wiring material. - Prepare Distribution Board - Install Distribution Board and wiring |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Developed Basic Drawing |  |  |  |  |  |
|  | Performed Estimation of Materials |  |  |  |  |  |
|  | CalculatedLabor Cost |  |  |  |  |  |
|  | Selected Distribution board w.r.t. size, current rating, voltage, No. of C.Bs and phases. |  |  |  |  |  |
|  | Selected Main Incoming Residual Current Circuit Breaker (RCCB) having minimum sensitivity. |  |  |  |  |  |
|  | Selected outgoing Miniature Circuit Breaker (MCB) according to load |  |  |  |  |  |
|  | Provide space for future MCB |  |  |  |  |  |
|  | Selected Voltmeter, ampere meter and indicator according to load and phases. |  |  |  |  |  |
|  | Selected wire according to load and for wiring in Distribution Board (DB). |  |  |  |  |  |
|  | made neutral common for all load |  |  |  |  |  |
|  | Connected accessories according to the circuit diagram |  |  |  |  |  |
|  | Fixed the distribution board |  |  |  |  |  |
|  | Took wire from DB to load without joint |  |  |  |  |  |
|  | Distributed load equally on all phases |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | National Vocational Certificate Level 2 in Instrumentation Technology |
| --- | --- |
| Competency Standard | Prepare and Install Distribution Boards for single phase wiring (Level 2) |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name:______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature:___________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor:________________________________________________ Assessor’s code:____________________________________________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Enlist the main wiring accessories (any five) |  |  |
|  | Differentiate between Fuse and Circuit breaker |  |  |
|  | RCCB are used for over load protection electrical wiring systems. |  |  |
|  | - TRUE - FALSE |  |  |
|  | Define distribution board |  |  |
|  | Fuses are always place in___________ wire |  |  |
|  | Explain the importance of earthing in wiring system |  |  |
|  | Define MCB |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature _________________