Instruction Sheet for the Candidate

| Qualification | National Vocational Certificate Level 3 in Instrumentation Technology |
| --- | --- |
| Competency Standard | Measure dimensions with VernierCalliper(Level 3) |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within time frame (for practical demonstration & assessment): - Take measurement with Vernier caliper - Take measurement with height gauge - Take measurement with Vernier depth gauge |
| Time: 03 Hrs. | During a practical assessment, under observation by an assessor, you are required to |
| Minimum Evidence Required | Take measurement with Vernier caliper - Clean the gauge and surface of work piece. - Place the work piece on surface plate. - Open the lock screws. - Slide the jaws up to the width or size of jaws. - Open slightly larger than the part to be measured. - Set anvil squarely against reference surface of part - Maintain the proper pressure on the jaws with screw. - Lock the movable jaw with knurled screw - Measure and record the dimensions Take measurement with height gauge - Clean the gauge and surface of work piece - Hold the work piece on angle plate. - Open the lock screws. - Set the height gauge on stud to check it with fine adjusting screw. - Lock the measuring head. - Record the reading. Take measurement with Vernier depth gauge - Clean the gauge and surface of work piece - Hold the work piece on sample plate. - Open the lock screws. - Insert the gauge inside the work piece with fine adjusting screw. - Lock the measuring head. - Record the reading |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | National Vocational Certificate Level 3 in Instrumentation Technology |
| Competency Standard | Measure dimensions with VernierCalliper(Level 3) |
| Purpose of Assessment | Formative Assessment |
| Assessment Task | - Take measurement with Vernier caliper - Take measurement with height gauge Take measurement with Vernier depth gauge |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Clean the gauge and surface of work piece |  |  |
| - Place the work piece on surface plate. |  |  |
| 3.Open the lock screws |  |  |
| - Slide the jaws up to the width or size of jaws |  |  |
| 5.Open slightly larger than the part to be measured |  |  |
| 6Set anvil squarely against reference surface of part |  |  |
| - Maintain the proper pressure on the jaws with screw. |  |  |
| - Lock the movable jaw with knurled screw |  |  |
| - Measure and record the dimensions |  |  |
| 10 .Clean the gauge and surface of work piece |  |  |
| 11.Hold the work piece on angle plate. |  |  |
| 12. Open the lock screws. |  |  |
| 13.Set the height gauge on stud to check it with fine adjusting screw. |  |  |
| 14. Lock the measuring head |  |  |
| 15 Record the reading. |  |  |
| 16 Clean the gauge and surface of work piece |  |  |
| 17. Hold the work piece on sample plate |  |  |
| 18. Open the lock screws |  |  |
| 19. Insert the gauge inside the work piece with fine adjusting screw. |  |  |
| 20. Lock the measuring head |  |  |
| 21. Record the reading |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | National Vocational Certificate Level 3 in Instrumentation Technology |
| --- | --- |
| Competency Standard | Measure dimensions with VernierCalliper(Level 3) |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor________________________________________________ Assessor’s code:____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Take measurement with Vernier caliper - Take measurement with height gauge - Take measurement with Vernier depth gauge |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Cleaned the gauge and surface of work piece |  |  |  |  |  |
|  | Placed the work piece on surface plate |  |  |  |  |  |
|  | Opened the lock screws |  |  |  |  |  |
|  | Slided the jaws up to the width or size of jaws |  |  |  |  |  |
|  | Opened slightly larger than the part to be measured |  |  |  |  |  |
|  | Set anvil squarely against reference surface of part |  |  |  |  |  |
|  | Maintained the proper pressure on the jaws with screw |  |  |  |  |  |
|  | Locked the movable jaw with knurled screw |  |  |  |  |  |
|  | Measured and record the dimensions |  |  |  |  |  |
|  | Cleaned the gauge and surface of work piece |  |  |  |  |  |
|  | Holded the work piece on angle plate. |  |  |  |  |  |
|  | Opened the lock screws |  |  |  |  |  |
|  | Set the height gauge on stud to check it with fine adjusting screw. |  |  |  |  |  |
|  | Locked the measuring head |  |  |  |  |  |
|  | Recorded the reading |  |  |  |  |  |
|  | Cleaned the gauge and surface of work piece |  |  |  |  |  |
|  | Holded the work piece on sample plate |  |  |  |  |  |
|  | Opened the lock screws |  |  |  |  |  |
|  | Inserted the gauge inside the work piece with fine adjusting screw. |  |  |  |  |  |
|  | Locked the measuring head |  |  |  |  |  |
|  | Recorded the reading |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | National Vocational Certificate Level 3 in Instrumentation Technology |
| --- | --- |
| Competency Standard | Measure dimensions with VernierCalliper(Level 3) |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name:______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature:___________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor:________________________________________________ Assessor’s code:____________________________________________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | What is the Least count of Vernier caliper |  |  |
|  | What is the use of Vernier caliper |  |  |
|  | How to explain positive zero error & negative zero error of a vernier caliper with the help of diagram |  |  |
|  | How do you calculate vernier caliper error |  |  |
|  | Set anvil squarely against reference surface of part |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature _________________