Instruction Sheet for the Candidate

| Qualification | National Vocational Certificate Level 3 in Instrumentation Technology |
| --- | --- |
| Competency Standard | Perform Measurements Through Micrometer(Level 3) |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within time frame (for practical demonstration & assessment): - . Take measurement with outside micro-meter - Take measurement with inside micrometer - Take measurement with depth micrometer - Measure threads with micrometer - Take measurement with Vernier micrometer |
| Time: 03 Hrs. | During a practical assessment, under observation by an assessor, you are required to |
| Minimum Evidence Required | Take measurement with outside micro-meter - Clean the gauge and surface of work piece - Open slightly larger than the part to be measured - Set anvil squarely against reference surface of part - Using ratchet, slowly until it click once - Record reading and make average. Take measurement with inside micrometer - Clean the gauge and surface of work piece. - Open gauge slightly smaller than the part to be measured - Set anvil squarely against reference surface of part with ratchet, slowly until it click once. - Record reading and make average. Take measurement with depth micrometer - Clean the gauge and surface of work piece. - Access the depth of groove by steel rule - Insert the suitable insertion rod - Place the depth micrometer gauge on the deeper groove and take reading. Measure threads with micrometer - Clean the gauge and the threads of work piece. - Use the suitable set of thread shape tips according to thread size which is to be measured. - Record the reading. Take measurement with Vernier micrometer - Clean the surface. - Place specimen on the surface plate - Take reading accurately. - Calculate mean reading. |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | National Vocational Certificate Level 3 in Instrumentation Technology |
| Competency Standard | Perform Measurements Through Micrometer (Level 3) |
| Purpose of Assessment | Formative Assessment |
| Assessment Task | - . Take measurement with outside micro-meter - Take measurement with inside micrometer - Take measurement with depth micrometer - Measure threads with micrometer - Take measurement with Vernier micrometer |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Clean the gauge and surface of work piece |  |  |
| - Open slightly larger than the part to be measured |  |  |
| - Set anvil squarely against reference surface of part |  |  |
| - Using ratchet, slowly until it click once |  |  |
| - Record reading and make average. |  |  |
| - Clean the gauge and surface of work piece. |  |  |
| - Open gauge slightly smaller than the part to be measured |  |  |
| - Set anvil squarely against reference surface of part with ratchet, slowly until it click once. |  |  |
| - Record reading and make average. |  |  |
| - Clean the gauge and surface of work piece. |  |  |
| - Access the depth of groove by steel rule |  |  |
| - Insert the suitable insertion rod |  |  |
| - Place the depth micrometer gauge on the deeper groove and take reading. |  |  |
| - Clean the gauge and the threads of work piece. |  |  |
| - Use the suitable set of thread shape tips according to thread size which is to be measured. |  |  |
| - Record the reading. |  |  |
| - Clean the surface. |  |  |
| - Place specimen on the surface plate |  |  |
| - Take reading accurately. |  |  |
| - Calculate mean reading. |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | National Vocational Certificate Level 3 in Instrumentation Technology |
| --- | --- |
| Competency Standard | Perform Measurements Through Micrometer (Level 3) |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor________________________________________________ Assessor’s code:____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Take measurement with outside micro-meter - Take measurement with inside micrometer - Take measurement with depth micrometer - Measure threads with micrometer |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Clean the gauge and surface of work piece |  |  |  |  |  |
|  | Open slightly larger than the part to be measured |  |  |  |  |  |
|  | Set anvil squarely against reference surface of part |  |  |  |  |  |
|  | Using ratchet, slowly until it click once |  |  |  |  |  |
|  | Record reading and make average. |  |  |  |  |  |
|  | Clean the gauge and surface of work piece. |  |  |  |  |  |
|  | Open gauge slightly smaller than the part to be measured |  |  |  |  |  |
|  | Set anvil squarely against reference surface of part with ratchet, slowly until it click once. |  |  |  |  |  |
|  | Record reading and make average. |  |  |  |  |  |
|  | Clean the gauge and surface of work piece. |  |  |  |  |  |
|  | Access the depth of groove by steel rule |  |  |  |  |  |
|  | Insert the suitable insertion rod |  |  |  |  |  |
|  | Place the depth micrometer gauge on the deeper groove and take reading. |  |  |  |  |  |
|  | Clean the gauge and the threads of work piece. |  |  |  |  |  |
|  | Use the suitable set of thread shape tips according to thread size which is to be measured. |  |  |  |  |  |
|  | Record the reading. |  |  |  |  |  |
|  | Clean the surface. |  |  |  |  |  |
|  | Place specimen on the surface plate |  |  |  |  |  |
|  | Take reading accurately. |  |  |  |  |  |
|  | Calculate mean reading. |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | National Vocational Certificate Level 3 in Instrumentation Technology |
| --- | --- |
| Competency Standard | Perform Measurements Through Micrometer (Level 3) |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name:______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature:___________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor:________________________________________________ Assessor’s code:____________________________________________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | What is the Least count of Micro meter |  |  |
|  | What is the use of Ratchet stop in Micro meter |  |  |
|  | What is the Total error in Micrometer |  |  |
|  | Range of a Micrometer screw gauge is |  |  |
|  | - More than 1 cm - 2 cm - Less than meter - Less than cm |  |  |
|  | How many divisions are graduated on Thimble |  |  |
|  | - 20 - 40 - 50 - 45 |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature _________________