Instruction Sheet for the Candidate

| Qualification | National Vocational Certificate Level 3 in Instrumentation Technology |
| --- | --- |
| Competency Standard | - Measure Electrical Power, Energy, Power Factor & Determine Phase Sequence - Implement Electromagnet to See Various Effects & Verify Faradays Law |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within time frame (for practical demonstration & assessment): - Measure consumed energy with Energy meter - Measure power factor with voltmeter, ampere meter and watt meter - Verify Faradays law by moving magnet inside coil. |
| Time: 04Hrs. | During a practical assessment, under observation by an assessor, you are required to |
| Minimum Evidence Required | Measure consumed energy with Energy meter 1. Connect phase line of AC supply to the energy meter. 2. Connect AC load to the output terminals of energy meter. 3. Power up the supply and take reading of energy in terms of unit after few minutes form display. Measure power factor with voltmeter, ampere meter and watt meter 1. Connect voltmeter & ampere meter with inductive load (Motor) 2. Connect watt meter and measure. 3. Connect AC supply and take the readings of volt, ampere and watt meter. 4. Utilize the above readings and calculate power factor using power formula P=VI Cos Ø Verify Faradays law by moving magnet inside coil. 1. Construct a coil with hollow iron cylinder (approximately 3 inches in length 1.5 inch in diameter.) and make 150 to 200 turns on it. 2. Connect Galvanometer with coil. 3. Move permanent magnet inside the coil fast and slow and record the effect on reading of Galvanometer. 4. Hold the magnet inside the coil and do not move, now record the effect on reading of Galvanometer. |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | National Vocational Certificate Level 3 in Instrumentation Technology |
| Competency Standard | - Measure Electrical Power, Energy, Power Factor & Determine Phase Sequence - Implement Electromagnet to See Various Effects & Verify Faradays Law |
| Purpose of Assessment | summative Assessment |
| Assessment Task | - Measure consumed energy with Energy meter - Measure power factor with voltmeter, ampere meter and watt meter - Verify Faradays law by moving magnet inside coil.. |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Connect phase line of AC supply to the energy meter. |  |  |
| - Connect AC load to the output terminals of energy meter. |  |  |
| - Power up the supply and take reading of energy in terms of unit after few minutes form display. |  |  |
| - Connect voltmeter & ampere meter with inductive load (Motor) |  |  |
| - Connect watt meter and measure. |  |  |
| - Connect AC supply and take the readings of volt, ampere and watt meter. |  |  |
| - Utilize the above readings and calculate power factor using power formula P=VI Cos Ø |  |  |
| - Construct a coil with hollow iron cylinder (approximately 3 inches in length 1.5 inch in diameter.) and make 150 to 200 turns on it. |  |  |
| - Connect Galvanometer with coil. |  |  |
| - Move permanent magnet inside the coil fast and slow and record the effect on reading of Galvanometer. |  |  |
| - Hold the magnet inside the coil and do not move, now record the effect on reading of Galvanometer. |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | National Vocational Certificate Level 3 in Instrumentation Technology |
| --- | --- |
| Competency Standard | - Measure Electrical Power, Energy, Power Factor & Determine Phase Sequence - Implement Electromagnet to See Various Effects & Verify Faradays Law |
| Purpose of Assessment | summative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor________________________________________________ Assessor’s code:____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Measure consumed energy with Energy meter - Measure power factor with voltmeter, ampere meter and watt meter - Verify Faradays law by moving magnet inside coil. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Connected phase line of AC supply to the energy meter. |  |  |  |  |  |
|  | Connected AC load to the output terminals of energy meter. |  |  |  |  |  |
|  | Powered up the supply and took reading of energy in terms of unit after few minutes form display. |  |  |  |  |  |
|  | Connected voltmeter & ampere meter with inductive load (Motor) |  |  |  |  |  |
|  | Connected watt meter and measure. |  |  |  |  |  |
|  | Connected AC supply and take the readings of volt, ampere and watt meter. |  |  |  |  |  |
|  | Utilized the above readings and calculated power factor using power formula P=VI Cos Ø |  |  |  |  |  |
|  | Construct a coil with hollow iron cylinder (approximately 3 inches in length 1.5 inch in diameter.) and make 150 to 200 turns on it. |  |  |  |  |  |
|  | Connected Galvanometer with coil. |  |  |  |  |  |
|  | Moved permanent magnet inside the coil fast and slow and recorded the effect on reading of Galvanometer. |  |  |  |  |  |
|  | Hold the magnet inside the coil and did not move, now record the effect on reading of Galvanometer. |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | National Vocational Certificate Level 3 in Instrumentation Technology |
| --- | --- |
| Competency Standard | - Measure Electrical Power, Energy, Power Factor & Determine Phase Sequence - Implement Electromagnet to See Various Effects & Verify Faradays Law |
| Purpose of Assessment | summative Assessment |
| Candidate Details | Name:______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature:___________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor:________________________________________________ Assessor’s code:____________________________________________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |  |
| --- | --- | --- | --- | --- |
|  | The unit of power is ________ |  |  |  |
|  | - Ampere - volt - ohm - Watt |  |  |  |
|  | Unit of Power factor ……………… |  |  |  |
|  | - moh - Farad - Volt- Ampere - no units |  |  |  |
|  | Wattmeter is used in measure Electrical power drawn by load |  |  |  |
|  | - TRUE - FALSE |  |  |  |
|  | Ohm meter is used to measure |  |  |  |
|  | a) voltage b) current c) resistance d) capacitance |  |  |  |
|  | Describe the formula of electric power consumed by load from AC source. |  |  |  |
|  | Describe the formula of Electrical Energy dissipation in resistor |  |  |  |
|  | State Faradays’ Law of electromagnetic Induction |  |  |  |
|  | Electric Motors and generators works on principal of faradays law |  |  |  |
|  | - TRUE - FALSE |  |  |  |
|  | KVA is unit used for Real power consumed by load |  |  |  |
|  | - TRUE - FALSE |  |  |  |
|  | Galvanometer is used to detect the current. |  |  |  |
|  | - TRUE - FALSE |  |  |  |
| Feedback to the Candidate |  |  |  |  |
| Candidate’s Signature__________________ Assessor’s Signature _________________ |  |  |  |  |