Instruction Sheet for the Candidate

| Qualification | National Vocational Certificate Level 3 in Instrumentation Technology |
| --- | --- |
| Competency Standard | - Analyze The Gas Chromatography - Measure Temperature and Humidity - Measure The Concentration Of Environmental Gases - Measure The Density & Power of Hydrogen Ion (pH) - Perform The Spectroscopic Characterization |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within time frame (for practical demonstration & assessment): Operate and analyze Chromatography instruments. |
| Time: 04Hrs. | During a practical assessment, under observation by an assessor, you are required to |
| Minimum Evidence Required | Operate and analyze Chromatography instrument. - Identify burner and column assemblyin given chromatograph (GC/LC) and its characteristic parameters. Also, understand theirworking mechanism and principle. - Identify and understand the working and standard operation for carrier /reference (liquid or) gas.Identify and understand the working mechanism of reference solutions (carrier gas) with sample addition through assembly. - Perform measurements for spectrograph for various gases and record the data. And identify the type of used Detector assembly and detection mechanism, i.e. Photo multiplier tube (PMT) etc. - Perform measurements by given samples column(according to type).Obtained gas spectrograph for various gaseous and record the data. Perform measurements (spectrograph) for various Liquid/ gases and record the data. - Identify Signal processing /conditioning circuits. Identify Data processing and control software parameters and settings. - Differentiate between the data measured by “Gas Chromatograph” and “column type chromatograph” and write a report. - Compare, analyze and interpret all measured data and comments on the deviation and discrepancies in relation with the reference database stored in the machine. - Construct an output report - Differentiate between heat and temperature. Identity measuring units.Perform conversion between scales i.e. Kelvin, Celsius and Fahrenheit scales. Identify three reference point of temperatures - Identify and differentiate various types of temperature sensors. Operate various temp sensors. Compare Mercury bulb thermometer with sensor’s base thermometers. - Identify Hygrometer (transducer type) components, Sensor type, Output & Input. Setup wet and dry bulb psychrometer/ hygrometer. - Identify the type of sensor for humidity vs temperature Perform relative and absolute measurement using wet and dry bulb thermometer. - Perform measurement for Humidity and temperature inside close container - Compare the measured humidity results - Focus sensor and note light parameters, Input - output blocks for Luminous meter and identify signal processing or conditioning units. - Prepare or arrange sample (bio material) and reference.Measure the data and record the spectra. - Compare, analyze and interpret all measured data and comments on the deviation and discrepancies in relation with the reference database.Develop aconcise report. |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | National Vocational Certificate Level 3 in Instrumentation Technology |
| Competency Standard | - Analyze The Gas Chromatography - Measure Temperature and Humidity - Measure The Concentration Of Environmental Gases - Measure The Density & Power of Hydrogen Ion (pH) - Perform The Spectroscopic Characterization |
| Purpose of Assessment | Summative Assessment |
| Assessment Task | - Operate and analyze Chromatography instruments - Measure Temperature and Humidity and perform units conversion. - Analyze the spectra using luminance meter. |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Identify burner and column assembly in given chromatograph (GC/LC) and its characteristic parameters. Also, understand their working mechanism and principle. |  |  |
| - Identify and understand the working and standard operation for carrier /reference (liquid or) gas. Identify and understand the working mechanism of reference solutions (carrier gas) with sample addition through assembly. |  |  |
| - Perform measurements for spectrograph for various gases and record the data. And identify the type of used Detector assembly and detection mechanism, i.e. Photo multiplier tube (PMT) etc. |  |  |
| - Perform measurements by given samples column(according to type).Obtained gas spectrograph for various gaseous and record the data. Perform measurements (spectrograph) for various Liquid/ gases and record the data. |  |  |
| - Identify Signal processing /conditioning circuits. Identify Data processing and control software parameters and settings. |  |  |
| - Differentiate between the data measured by “Gas Chromatograph” and “column type chromatograph” and write a report. |  |  |
| - Compare, analyze and interpret all measured data and comments on the deviation and discrepancies in relation with the reference database stored in the machine. |  |  |
| - Construct an output report |  |  |
| - Differentiate between heat and temperature. Identity measuring units.Perform conversion between scales i.e. Kelvin, Celsius and Fahrenheit scales. Identify three reference point of temperatures |  |  |
| - Identify and differentiate various types of temperature sensors. Operate various temp sensors. Compare Mercury bulb thermometer with sensor’s base thermometers. |  |  |
| - Identify Hygrometer (transducer type) components, Sensor type, Output & Input. Setup wet and dry bulb psychrometer/ hygrometer. |  |  |
| - Identify the type of sensor for humidity vs temperature Perform relative and absolute measurement using wet and dry bulb thermometer. |  |  |
| - Perform measurement for Humidity and temperature inside close container |  |  |
| - Compare the measured humidity results |  |  |
| - Focus sensor and note light parameters, Input - output blocks for Luminous meter and identify signal processing or conditioning units. |  |  |
| - Prepare or arrange sample (bio material) and reference. Measure the data and record the spectra. |  |  |
| - Compare, analyze and interpret all measured data and comments on the deviation and discrepancies in relation with the reference database. Develop a concise report. |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | National Vocational Certificate Level 3 in Instrumentation Technology |
| --- | --- |
| Competency Standard | - Analyze The Gas Chromatography - Measure Temperature and Humidity - Measure The Concentration Of Environmental Gases - Measure The Density & Power of Hydrogen Ion (pH) - Perform The Spectroscopic Characterization |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor________________________________________________ Assessor’s code:____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | Operate and analyze Chromatography instruments |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Identify burner and column assembly in given chromatograph (GC/LC) and its characteristic parameters. Also, understand their working mechanism and principle. |  |  |  |  |  |
|  | Identify and understand the working and standard operation for carrier /reference (liquid or) gas. Identify and understand the working mechanism of reference solutions (carrier gas) with sample addition through assembly. |  |  |  |  |  |
|  | Perform measurements for spectrograph for various gases and record the data. And identify the type of used Detector assembly and detection mechanism, i.e. Photo multiplier tube (PMT) etc. |  |  |  |  |  |
|  | Perform measurements by given samples column(according to type).Obtained gas spectrograph for various gaseous and record the data. Perform measurements (spectrograph) for various Liquid/ gases and record the data. |  |  |  |  |  |
|  | Identify Signal processing /conditioning circuits. Identify Data processing and control software parameters and settings. |  |  |  |  |  |
|  | Differentiate between the data measured by “Gas Chromatograph” and “column type chromatograph” and write a report. |  |  |  |  |  |
|  | Compare, analyze and interpret all measured data and comments on the deviation and discrepancies in relation with the reference database stored in the machine. |  |  |  |  |  |
|  | Construct an output report |  |  |  |  |  |
|  | Differentiate between heat and temperature. Identity measuring units.Perform conversion between scales i.e. Kelvin, Celsius and Fahrenheit scales. Identify three reference point of temperatures |  |  |  |  |  |
|  | Identify and differentiate various types of temperature sensors. Operate various temp sensors. Compare Mercury bulb thermometer with sensor’s base thermometers. |  |  |  |  |  |
|  | Identify Hygrometer (transducer type) components, Sensor type, Output & Input. Setup wet and dry bulb psychrometer/ hygrometer. |  |  |  |  |  |
|  | Identify the type of sensor for humidity vs temperature Perform relative and absolute measurement using wet and dry bulb thermometer. |  |  |  |  |  |
|  | Perform measurement for Humidity and temperature inside close container |  |  |  |  |  |
|  | Compare the measured humidity results |  |  |  |  |  |
|  | Focus sensor and note light parameters, Input - output blocks for Luminous meter and identify signal processing or conditioning units. |  |  |  |  |  |
|  | Prepare or arrange sample (bio material) and reference. Measure the data and record the spectra. |  |  |  |  |  |
|  | Compare, analyze and interpret all measured data and comments on the deviation and discrepancies in relation with the reference database. Develop a concise report. |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | National Vocational Certificate Level 3 in Instrumentation Technology |
| --- | --- |
| Competency Standard | - Analyze The Gas Chromatography - Measure Temperature and Humidity - Measure The Concentration Of Environmental Gases - Measure The Density & Power of Hydrogen Ion (pH) - Perform The Spectroscopic Characterization |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name:______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature:___________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor:________________________________________________ Assessor’s code:____________________________________________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | What is the basic principle of Gas Chromatography? |  |  |
|  | Differentiate between gas chromatography and liquid chromatography. |  |  |
|  | What are the main characteristics of Column and Flame in Gas Chromatography? |  |  |
|  | Define Liquid Chromatography. |  |  |
|  | What are the main characteristics of Column and reference solution in Liquid Chromatography? |  |  |
|  | Explain Electronics for sensors and detection mechanism of Gas Chromatography. |  |  |
|  | Explain Electronics for sensors and detection mechanism of Liquid Chromatography. |  |  |
|  | Interpret and discuss the result of Gas Chromatograph. |  |  |
|  | Interpret and discuss the result of Liquid Chromatograph. |  |  |
|  | Explain heat and temperature and their units. |  |  |
|  | Interpret formulae for temperature unit conversion. |  |  |
|  | How many basic types of temperature sensors? |  |  |
|  | Define Humidity and its units. |  |  |
|  | Define Temperature dependency on Humidity. |  |  |
|  | What is the practical range for humidity? |  |  |
|  | What is mono-chromelight? |  |  |
|  | What is function of Lens and mirror? |  |  |
|  | Draw a Block diagram dual beam signal. |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature _________________