Instruction Sheet for the Candidate

| Qualification | National Vocational Certificate Level 4 in Instrumentation Technology |
| --- | --- |
| Competency Standard | - Charactrize diode applications - Charactrize BJT amplifiers - Charactrize various FET amplifiers - Design an operational apmplifier - Charactrize various oscillators |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name______________________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within time frame (for practical demonstration & assessment): - To draw I-V characteristic curve in forward/reverse bias configuration and verify rectification process (half-wave) of PN junction diode with the help of fig given below. Fig:forward/reverse configuration Fig: Circuit Diagram for Half-Wave Rectifier - To design and set up an RC phase shift oscillator using BJT and to observe the sinusoidal output waveform. |
| Time: 04 Hrs. | During a practical assessment, under observation by an assessor, you are required to |
| Minimum Evidence Required | - Connect the circuit as shown in Fig. - Initially vary Regulated Power Supply (RPS) voltage Vsin steps of 0.1 V. Once the current starts increasing vary Vsin steps of 0.02V and note down the corresponding readings Vfand If. - Tabulate different forward currents obtained for different forward voltages. - Plot the V-I characteristics and calculate the resistance levels - Using oscilloscope, determine Vin (Channel 1) and Vout (channel 2) - Sketch the input waveform, Vin and the output waveform, Vout Record the Vpp values for both waveforms. - Using Digital Millimeter (DMM), measure and record Vin and Vout, Measure for both DC and AC mode of the DMM. - Identify the pin details of BC107 Transistor (or equivalent silicon Transistor such as BC108/547) and test it using a millimeter. Set up the circuit on breadboard as shown in figure. - A 12V Supply Voltage is given by using Regulated power supply and output is taken from collector of the Transistor. - By using CRO the output time period and voltage are noted. - Plot all the readings curves on a single graph sheet. |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | (Instrumentation Technology Level-5) |
| Competency Standard | Semiconductor Devices & Electronic Circuits |
| Purpose of Assessment | Summative Assessment |
| Assessment Task | - To draw I-V characteristic curve in forward/reverse bias configuration and verify rectification process (half-wave) of PN junction diode with the help of fig given below. Fig: Circuit Diagram for Half-Wave Rectifier Fig:forward/reverse configuration - To design and set up an RC phase shift oscillator using BJT and to observe the sinusoidal output waveform. |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Connect the circuit as shown in Fig. |  |  |
| - Initially vary Regulated Power Supply (RPS) voltage Vsin steps of 0.1 V. Once the current starts increasing vary Vsin steps of 0.02V and note down the corresponding readings Vfand If. |  |  |
| - Tabulate different forward currents obtained for different forward voltages. |  |  |
| - Plot the V-I characteristics and calculate the resistance levels |  |  |
| - Using oscilloscope, determine Vin (Channel 1) and Vout (channel 2) |  |  |
| - Sketch the input waveform, Vin and the output waveform, Vout Record the Vpp values for both waveforms. |  |  |
| - Using Digital Millimeter (DMM), measure and record Vin and Vout, Measure for both DC and AC mode of the DMM. |  |  |
| - Identify the pin details of BC107 Transistor (or equivalent silicon Transistor such as BC108/547) and test it using a millimeter. Set up the circuit on breadboard as shown in figure. |  |  |
| - A 12V Supply Voltage is given by using Regulated power supply and output is taken from collector of the Transistor. |  |  |
| - By using CRO the output time period and voltage are noted. |  |  |
| - Plot all the readings curves on a single graph sheet. |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | National Vocational Certificate Level 4 in Instrumentation Technology |
| --- | --- |
| Competency Standard | - Charactrize diode applications - Charactrize BJT amplifiers - Charactrize various FET amplifiers - Design an operational apmplifier - Charactrize various oscillators |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor________________________________________________ Assessor’s code:____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - To draw I-V characteristic curve in forward/reverse bias configuration and verify rectification process (half-wave) of PN junction diode with the help of fig given below. Fig: Circuit Diagram for Half-Wave Rectifier Fig:forward/reverse configuration - To design and set up an RC phase shift oscillator using BJT and to observe the sinusoidal output waveform. |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Connect the circuit as shown in Fig. |  |  |  |  |  |
|  | Initially vary Regulated Power Supply (RPS) voltage Vsin steps of 0.1 V. Once the current starts increasing vary Vsin steps of 0.02V and note down the corresponding readings Vfand If. |  |  |  |  |  |
|  | Tabulate different forward currents obtained for different forward voltages. |  |  |  |  |  |
|  | Plot the V-I characteristics and calculate the resistance levels |  |  |  |  |  |
|  | Using oscilloscope, determine Vin (Channel 1) and Vout (channel 2) |  |  |  |  |  |
|  | Sketch the input waveform, Vin and the output waveform, Vout Record the Vpp values for both waveforms. |  |  |  |  |  |
|  | Using Digital Millimeter (DMM), measure and record Vin and Vout, Measure for both DC and AC mode of the DMM. |  |  |  |  |  |
|  | Identify the pin details of BC107 Transistor (or equivalent silicon Transistor such as BC108/547) and test it using a millimeter. Set up the circuit on breadboard as shown in figure. |  |  |  |  |  |
|  | A 12V Supply Voltage is given by using Regulated power supply and output is taken from collector of the Transistor. |  |  |  |  |  |
|  | By using CRO the output time period and voltage are noted. |  |  |  |  |  |
|  | Plot all the readings curves on a single graph sheet. |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | National Vocational Certificate Level 4 in Instrumentation Technology |
| --- | --- |
| Competency Standard | - Charactrize diode applications - Charactrize BJT amplifiers - Charactrize various FET amplifiers - Design an operational apmplifier - Charactrize various oscillators |
| Purpose of Assessment | Summative Assessment |
| Candidate Details | Name:______________________________________________________________ Registration/Roll Number: _________________ Candidate Signature:___________ |
| Assessment Outcome | COMPETENT NOT YETCOMPETENT Name of the Assessor:________________________________________________ Assessor’s code:____________________________________________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | What is a semiconductor |  |  |
|  | What is meant by intrinsic semiconductor? |  |  |
|  | What is the order of energy gap in a pure semiconductor? |  |  |
|  | What is meant by forward bias? |  |  |
|  | What is meant by Reverse bias? |  |  |
|  | What is the function of transistor? |  |  |
|  | What is the function of oscillator? |  |  |

Feedback to the Candidate

Candidate’s Signature__________________ Assessor’s Signature _________________