National Vocational Certificate Level 3 in Instrumentation Technology

(Assistant Technician)

Quality Assessed by:

Mr. Muqeem-ul-Islam

Director General (SS&C NAVTTC HQ, Islamabad)

CONTENTS

| Sr. No | TABLE OF CONTENTS | Pages |
| --- | --- | --- |
| 1. | Introduction | 4 |
| 2. | Purpose of the qualification | 4 |
| 3. | Date of Validation | 5 |
| 4. | Date of Review | 5 |
| 5. | Code of Qualification | 5 |
| 6. | Qualification Development Committee | 6 |
| 7. | Qualification Validation Committee | 7 |
| 8. | Entry Requirements | 7 |
| 9. | Regulations for the Qualification and Schedule of Units | 7 |
| 10. | Packaging of the qualification/Mapping/Common | 8-13 |
| CORE COMPETENCY STANDARDS |  |  |

0714E&A1.	Install computer operating systems and hardware	12

0714E&A2.	Operate word-processing applications	14

0714E&A3.	Operate spreadsheet applications	16

0714E&A4.	Operate presentation packages	18

0714E&A5.	Perform writing and editing tasks	20

0714E&A6.	Perform computer operations	22

0714E&A7.	Use computer applications	24

0714E&A8.	Create user documentation	27

0714E&A9.	Create technical documentation	29

0714E&A10.	Create basic databases	31

0714E&A11.	Operate digital media technology	33

0714E&A12.	Use social media tools for collaboration and engagement	35

0714E&A13.     Apply basic CAD software for Designing & Drawing	37

0714E&A14.     Install Temperature & Pressure Measuring Instruments/Devices	41

0714E&A15.     Install Level Measuring Instruments/Device	44

0714E&A16.     Install the Flow Measuring Instruments/Devices	46

0714E&A17.	Take measurements with graduated tools	48

0714E&A18.	Take measurements with combination set	49

0714E&A19.	Take measurements through various gauges	50

0714E&A20.	Perform measurements through Micrometre	54

0714E&A21.	Measure dimensions with Vernier Calliper	56

0714E&A22.	Perform different measurements	57

0714E&A23.	Operate Measuring Instruments	60

0714E&A24.	Verify Ohm’s Law & Kirchhoff’s Law by Implementing Series/Parallel Circuits	61

0714E&A25.	Measure Electrical Power,Energy, Power Factor & Phase SequencE………………63

0714E&A26.	Implement Electromagnet to see Various Effects & Verify Faraday’s Laws.	66

0714E&A27.	Design a Rectifier using Diode	70

0714E&A28.	Carry out Diode Application	72

0714E&A29.	Implement Bipolar Junction Transistor (BJT) in Different Applications	73

0714E&A30.	Implement Field Effect Transistor (FET) in Different Applications	76

0714E&A31.	Implement Thyristor Family in Various Application	79

0714E&A32.	Applications of Operation Amplifier	82

- INTRODUCTION

Instrumentation Technology is specifically designed for students of Electronics/Electrical and touches upon a variety of fundamental topics. This course will cover a major portion of Instrumentation Technology. This training program will also build upon these general concepts to cover object-oriented Instrumentation terminology.

This specialization(Level 1-5) covers topics ranging from basic Electrical & Electronics Measuring Instrumentation, basic engineering drawing & CAD, Instrumentation drawing, Instrumentation servicing & calibration,  Industrial Instrumentation . You will learn fundamental concepts of Microprocessor applications. We also get practical exposure of Boiler Installation & operation.     By the end of this course, trainee will understand the basics of Instrumentation Technology & able to work in an Industry.

The National Skills Strategy (NSS) aims at achieving a paradigm shift from time-bound   and supply led to competency-based and demand driven training in Pakistan. Competency-based training approach focuses on the demonstration of actual skills required in the workplace/industry. To achieve this strategy, NAVTTC in collaboration with GIZ is involved in the development process of CBT program to ensure competent skilled labor in demand driven vocational trades.

- National Vocational Qualification Framework (NVQF) as designed in consultation with the stakeholders including academia, researchers, industry, chambers and TEVTAs. Its aim is to identify knowledge & skills required for a qualification, determine equivalence, provide guidelines for Recognition of Prior Learning (RPL) and assure quality of training. NVQF designed for the Vertical and horizontal progression for learners, recognition of qualification, meeting with the national and international standards, facilitate conversion of informal training to formal training through RPL, improvement in quality of training and increased options for learners for selecting training programs in different trades. Competency Standards are performance specification that identify the knowledge and competencies an individual need to succeed in the workplace.

2. PURPOSE OF THE QUALIFICATION

The purpose of this qualification is to give the student a thorough understanding and skills of the Instrumentation Technology in three years training program. The Instrumentation technology industry needs skilled labour for meeting the national and international standards. It is therefore important to stress the need for a multidisciplinary approach to meet the challenges within the sector. Upon successful completion of this course the trainee should be able to know:

- Basic principles of Electricity & Electronics

- Installation of process variable measuring Instruments

- Instrumentation servicing & Calibration.

- Interface sensors & Transducer

- Improve the Digital & soft skills  of the trainees

Install & operate Boiler Instrumentation system

- Improve the quality and effectiveness of training and assessment for information technology    industry skilled labour

- Enhance Entrepreneurial skills of the trainee & will be able to start their own Business.

3. DATE OF VALIDATION

These national qualifications have been validated by the Qualification Development Committee (QVC) on 27-29 May, 2019 in Lahore and will remain currency until 28 May 2029.

4. DATE OF REVIEW

These national qualifications shall be reviewed in May, 2022

5. CODE OF QUALIFICATION

The International Standard Classification of Education (ISCED) is a framework for assembling, compiling and analyzing cross-nationally comparable statistics on education and training. ISCED codes for these qualifications are assigned as follows:

| ISCED Classification for “Instrumentation Technology” level 5 (Diploma) |  |
| --- | --- |
| Code | Description |
| 0714E&A(5) | National Vocational Certificate level 5, in “Instrumentation Technology” |
| 0714E&A(4) | National Vocational Certificate level 4, in “Instrumentation Technology” |
| 0714E&A(3) | National Vocational Certificate level 3, in “Instrumentation Technology” |
| 0714E&A(2) | National Vocational Certificate level 2, in “Instrumentation Technology” |
| 0714E&A(1) | National Vocational Certificate level 1, in “Instrumentation Technology” |

6. QUALIFICATION DEVELOPMENT COMMITTEE

The following members participated in the qualifications development workshop 2019 in Karachi

| S. No. | Name & Designation | Organization |
| --- | --- | --- |
|  | Mr. Danish Attiq, Project Manager | Skill Tech International, Karachi |
|  | Engr Bilal Mehmood, Senior Instructor | Skill Tech International, Karachi |
|  | Engr Auragzeb Ansari, Assistant Professor | Usman Institute of Technology, Karachi |
|  | Engr Hassan Ali Rizvi, Assistant Manager | Lucky Cement Limited Karachi |
|  | Mr. Umair, Maintenance Engineer | SAPT, Karachi |
|  | Mr. Babar Sayeed, Chief Instructor | PSTC Karachi |
|  | Mr. Adeel Ahmed, Manager Production | Habib Oil Mills, Karachi |
|  | Mr. Sohail Akhtar, SSO | PCSIR, Karachi |
|  | Mr. Syed Farhan Hamid, Senior Instructor | PSTC Karachi |
|  | Mr. Muhammad Yasir, Assistant Director NAVTTC | NAVTTC, Islamabad |
|  | Mr. Ayub Nasir, DACUM facilitator | FWO |
|  | Dr. Abid Karim, Senior Scientific Officer | PCSIR, Karachi |
|  | Dr. Zeeshan , HOD, electronics | Dawood Engineering University Karachi |

7. QUALIFICATION VALIDATION COMMITTEE

The following members participated in the qualification validation workshop from 27-29 May 2019, in Lahore:

| S.No. | Name & Designation | Organization |
| --- | --- | --- |
|  | Hafiz ur Rehman Associate Professor | KP- TEVTA |
|  | Aijaz Ahmed Zia D&A Engineer | INTECH Process Automation |
|  | Abdur Rehman Research Assistant | PBTE Lahore |
|  | Syed Farhan Hamid Ali Sr.Instructor/Assistant DACUM Facilitator | Pak Swiss Training Centre Karachi |
|  | Pir Bakhush khan HoD/SSO | Centre for Development of Laboratory Equipment Lahore, Lahore Laboratory complex |
|  | Engr. Liaquat Ali Jamroo Director Academics | STEVTA |
|  | Muhammad Arshad Principal | PSTC Lahore |
|  | Muhammad Yasir Assitant Director | NAVTTC HQ |

8. ENTRY REQUIREMENTS

Entry requirements of this qualification are National Vocational Certificate Level 1&2 in Instrumentation Technology

9. REGULATIONS FOR THE QUALIFICATION AND SCHEDULE OF UNITS

Not applicable

10. PACKAGING OF QUALIFICATION

The national vocational qualifications are packaged as per following:

| Competency Standards | Occupation | NVQF Level | Category | Estimated Contact Hours |  |  | Cr Hr |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Th | Pr | Total |  |
| Level 3 |  |  |  |  |  |  |  |
| Install Computer Operating Systems And Hardware | Digital Skills | Level 3 | Generic | 3 | 12 | 15 | 1.5 |
| Operate Word-Processing Applications |  | Level 3 | Generic | 3 | 12 | 15 | 1.5 |
| Operate Spreadsheet Applications |  | Level 3 | Generic | 3 | 12 | 15 | 1.5 |
| Operate Presentation Packages |  | Level 3 | Generic | 3 | 12 | 15 | 1.5 |
| Perform Writing And Editing Tasks |  | Level 3 | Generic | 3 | 12 | 15 | 1.5 |
| Perform Computer Operations |  | Level 3 | Generic | 3 | 12 | 15 | 1.5 |
| Use Computer Applications |  | Level 3 | Generic | 3 | 12 | 15 | 1.5 |
| Create User Documentation |  | Level 3 | Generic | 3 | 12 | 15 | 1.5 |
| Create Technical Documentation |  | Level 3 | Generic | 3 | 12 | 15 | 1.5 |
| Create Basic Databases |  | Level 3 | Generic | 3 | 12 | 15 | 1.5 |
| Operate Digital Media Technology |  | Level 3 | Generic | 3 | 12 | 15 | 1.5 |
| Use Social Media Tools For Collaboration And Engagement |  | Level 3 | Generic | 3 | 12 | 15 | 1.5 |
| Use Digital Devices |  | Level 3 | Generic | 3 | 12 | 15 | 1.5 |
| Apply basic CAD, Solid works software for designing and drawing | CAD Draftman | Level 3 | Technical | 2 | 18 | 20 | 2 |
| Install Temperature & Pressure measuring instruments/devices | Instrument Technician | Level 3 | Technical | 5 | 15 | 20 | 2 |
| Install level measuring instruments/devices |  | Level 3 | Technical | 3 | 15 | 18 | 1.8 |
| Install the flow measuring instruments/devices |  | Level 3 | Technical | 3 | 15 | 18 | 1.8 |
| Take measurements with graduated tools | Metrology | Level 3 | Technical | 3 | 12 | 15 | 1.5 |
| Take measurements with combination set |  | Level 3 | Technical | 3 | 12 | 15 | 1.5 |
| Take measurements through various gauges |  | Level 3 | Technical | 3 | 12 | 15 | 1.5 |
| Perform measurements through Micrometer |  | Level 3 | Technical | 3 | 15 | 18 | 1.8 |
| Measure dimensions with Vernier tools |  | Level 3 | Technical | 3 | 15 | 18 | 1.8 |
| Perform different measurements |  | Level 3 | Technical | 3 | 15 | 18 | 1.8 |
| Operate Measuring Instruments. | Basic Electrical Engineering | Level 3 | Technical | 3 | 15 | 18 | 1.8 |
| Verify Ohm’s Law & Kirchhoff’s Law by Implementing Series/Parallel Circuits. |  | Level 3 | Technical | 3 | 15 | 18 | 1.8 |
| Measure Electrical Power, Energy, Power Factor & Determine Phase Sequence |  | Level 3 | Technical | 3 | 15 | 18 | 1.8 |
| Implement Electromagnet to See Various Effects & Verify Faradays Laws. |  | Level 3 | Technical | 3 | 15 | 18 | 1.8 |
| Verify Law of Combination of Capacitor and Determine Break down voltage of Capacitor. |  | Level 3 | Technical | 3 | 15 | 18 | 1.8 |
| Identify Basic Electronics Components | Basic Electronics (Analog) | Level 3 | Technical | 5 | 15 | 20 | 2 |
| Design a Rectifier using Diode |  | Level 3 | Technical | 5 | 15 | 20 | 2 |
| Carry out Diode Application |  | Level 3 | Technical | 5 | 15 | 20 | 2 |
| Implement Bipolar Junction Transistor (BJT) in Different Applications |  | Level 3 | Technical | 5 | 15 | 20 | 2 |
| Implement Field Effect Transistor (FET) in Different Applications |  | Level 3 | Technical | 5 | 15 | 20 | 2 |
| Implement Thyristor Family in Various Application |  | Level 3 | Technical | 5 | 15 | 20 | 2 |
| Applications of Operation Amplifier |  | Level 3 | Technical | 5 | 15 | 20 | 2 |
| Total |  |  |  | 120 | 480 | 600 | 60 |
| Percentage |  |  |  | 20 | 80 |  |  |

11. COMMON COURSES WITH RESPECTIVE LEVELS

LEVEL2

LEVEL3

LEVEL4

LEVEL5

12. MAPPING OF THE QUALIFICATION

# Install computer operating systems and hardware

Overview: This unit describes the performance outcomes, skills and knowledge required to select, configure and use computer operating systems and basic computer hardware.

| Unit of Competency | Performance Criteria |
| --- | --- |
| - Identify operating system and hardware components | - Determine ICT organizational requirements and specifications - Identify and select the operating system - Identify appropriate external hardware components - Identify internal hardware components |
| - Install and configure operating system and application software with hardware components | - Install and configure operating system to meet organizational requirements - Identify the functions associated with the operating system and associated boot process - Configure power-management settings to minimize power consumption as an environmentally sustainable measure - Use both the graphical user interface and the command line interface to perform basic tasks - Install or upgrade application software onto the operating system and hardware configuration ?? no need - Determine the relationship between an application program, the operating system and hardware - Identify general differences between the different computer platforms and their respective operating systems |
| - Optimize operating system and hardware components | - Optimize operating system using included tools or third-party utilities - Customize the graphical user interface - Use techniques unique to the command line interface - Set up and configure external hardware components and check functionality - Install drivers as appropriate and check functionality |

Knowledge & Understanding

K1: Basic knowledge of current industry-accepted operating system, hardware and software products

K2: Compatibility of an operating system, in respect to other versions

K3: Function of single-user and multi-user operating systems

K4: Interoperability between operating systems

K5: OHS principles and responsibilities, including ergonomic principles to avoid injury associated with using computer systems.

Critical Evidence(s) Required

The candidate needs to produce following Critical Evidence(s) to be competent in this competency standard:

A person who demonstrates competency in this unit must be able to provide evidence of the ability to select, configure and use computer operating systems and basic computer hardware. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

- Use an operating system in a variety of scenarios and across functions, including:

  - scheduling, loading, initiating, and supervising the execution of programs

  - allocating storage

  - initiating and controlling input and output operations

  - handling errors

- Identify and install suitable hardware components

- Install and upgrade application software.

# Operate word-processing applications

Overview: This unit describes the skills and knowledge required to operate word- processing applications and perform basic operations, including creating and formatting documents, creating tables and printing labels. It applies to individuals in the workplace using fundamental knowledge of word-processing under direct supervision or with limited responsibility.

| Unit of Competency | Performance Criteria |
| --- | --- |
| 1. Apply workplace health and safety (WHS) practices | 1.1 Use workplace ergonomic work practices and strategies 1.2 Organize work area to ensure an ergonomic work environment |
| 2. Create documents | 2.1 Open word-processing application, create document and add data according to information requirements 2.2 Use document templates as required 2.3 Use simple formatting tools when creating the document 2.4 Save document to directory |
| 3. Customize basic settings to meet page layout conventions | 3.1 Adjust page layout to meet information requirements 3.2 Open and view various toolbars 3.3 Change font format to suit document purpose 3.4 Change alignment and line spacing according to document information requirements 3.5 Modify margins to suit the document purpose 3.6 Open and switch between several documents |
| 4. Format documents | 4.1 Use formatting features and styles as per required 4.2 Highlight and copy text from another area in the document or from another active document 4.3 Insert headers and footers to incorporate necessary data 4.4 Save document in another file format 4.5 Save and close document to a storage device |
| 5. Create tables | 5.1 Insert standard table into document 5.2 Change the cells to meet the required information 5.3 Insert and delete columns and rows as necessary 5.4 Use formatting tools according to the style of requirements |
| 6. Add images | 6.1 Insert appropriate images into document and customize as necessary 6.2 Position and resize images to meet document formatting needs |
| 7. Print documents | 7.1 Preview the document in print preview mode 7.2 Select basic print settings 7.3 Print document or part of document from printer |

Knowledge & Understanding

K1: describe formatting styles and their effect on formatting, readability and appearance of documents

K2: identify organizational requirements for ergonomics, including work periods and breaks

K3: select organizational style guide to use

K4: outline purpose, use and function of word-processing software.

Critical Evidence(s) Required

A person who demonstrates competency in this unit must be able to provide evidence of the ability to create word documents that is clear to the target audience and easy to navigate. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

- follow organizational ergonomic work health and safety (WHS) requirements and practices

- create, open and retrieve documents using customized basic settings

- format documents by creating tables and adding text, objects and images

- Save and prints documents.

# Operate spreadsheet applications

Overview: This unit describes the skills and knowledge required to operate word- processing applications and perform basic operations, including creating and formatting documents, creating tables and printing labels. It applies to individuals in the workplace using fundamental knowledge of word-processing under direct supervision or with limited responsibility.

| Unit of Competency | Performance Criteria |
| --- | --- |
| 1. Create presentations | 1.1 Open presentation package and create a simple design for a presentation according to organizational requirements 1.2 Open the blank presentation and add text and graphics 1.3 Apply existing styles within a presentation 1.4 Use presentation template and slides to create a presentation 1.5 Use various tools to improve the look of the presentation 1.6 Save presentation to the appropriate storage device and folder |
| 2. Customize basic settings | 2.1 Adjust display to meet user requirements 2.2 Open and view different toolbars to view options 2.3 Ensure font settings are appropriate for the presentation purpose 2.4 View multiple slides at once |
| 3. Format presentations | 3.1 Use and incorporate organizational charts and bulleted lists, and modify as required 3.2 Add objects and manipulate to meet presentation purposes 3.3 Import objects and modify for presentation purposes 3.4 Modify slide layout, including text and colors, to meet presentation requirements 3.5 Use formatting tools as required within the presentation 3.6 Duplicate slides within and across a presentation 3.7 Reorder sequence of slides and delete slides for presentation purposes 3.8 Save presentation in another format 3.9 Save to storage device and close presentation |
| 4. Add slide show effects | 4.1 Incorporate pre-set animation and multimedia effects into presentation as required to enhance the presentation 4.2 Add slide transition effects to presentation to ensure smooth progression through the presentation 4.3 Test presentation for overall effect 4.4 Use onscreen navigation tools to start and stop slide show or move between different slides as required |
| 5. Print presentation and notes | 5.1 Select appropriate print format for presentation 5.2 Select preferred slide orientation 5.3 Add notes and slide numbers 5.4 Preview slides and run spell check before presentation 5.5 Print selected slides and submit presentation to appropriate person for feedback |

Knowledge & Understanding

K1: list basic technical terminology to read help files and prompts

K2: outline the different types of:

- formal and informal presentations

- audience

K3: explain the effect of design and formatting on the readability and usability of presentations

K4: outline presentation pitfalls

K5: identify suitable presentation effects for different audiences.

Critical Evidence(s) Required

A person who demonstrates competency in this unit must be able to provide evidence of the ability to presentations that is clear to the target audience and easy to navigate. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

- create, format and prepare presentations for distribution and display

- customize basic settings

- Add slide show effects.

# Operate presentation packages

Overview: This unit describes the skills and knowledge required to operate word- processing applications and perform basic operations, including creating and formatting documents, creating tables and printing labels. It applies to individuals in the workplace using fundamental knowledge of word-processing under direct supervision or with limited responsibility.

| Unit of Competency | Performance Criteria |
| --- | --- |
| 1. Create spreadsheets | 1.1 Open the spreadsheet application, create spreadsheet files and enter numbers, text and symbols into cells according to information requirements 1.2 Enter simple formulae and functions using cell referencing when required 1.3 Correct formulae when error messages occur 1.4 Use a range of common tools during spreadsheet development 1.5 Edit columns and rows within the spreadsheet 1.6 Use the auto-fill function to increment data where required 1.7 Save the spreadsheet to a folder on a storage device |
| 2. Customize basic settings | 2.1 Adjust page layout to meet user requirements or special needs 2.2 Open and view different tool bars 2.3 Change font settings so they are appropriate for the document purpose 2.4 Change alignment options and line spacing according to spreadsheet formatting features 2.5 Format cell to display different styles as required 2.6 Modify margin sizes to suit the purpose of the spreadsheets 2.7 View multiple spreadsheets concurrently |
| 3. Format spreadsheet | 3.1 Use formatting features as required 3.2 Copy selected formatting features from another cell in the spreadsheet or from another active spreadsheet 3.3 Use formatting tools as required within the spreadsheet 3.4 Align information in a selected cell as required 3.5 Insert headers and footers using formatting features 3.6 Save spreadsheet as another file type 3.7 Save to storage device and close the spreadsheet |
| 4. Incorporate object and chart in spreadsheet | 4.1 Import an object into an active spreadsheet 4.2 Manipulate the imported object by using formatting features 4.3 Create a chart using selected data in the spreadsheet 4.4 Display selected data in a different chart 4.5 Modify chart using formatting features |
| 5. Print spreadsheet | 5.1 Preview spreadsheet in print preview mode 5.2 Select basic printer options 5.3 Print spreadsheet or selected part of spreadsheet 5.4 Submit the spreadsheet to appropriate person for approval or feedback |

Knowledge & Understanding

K1: list basic technical terminology related to reading help files and prompts

K2: explain the effect of formatting and appearance on the readability and usability of spreadsheets

K3: outline log-in procedures relating to accessing a personal computer (PC)

K4: describe the purpose, use and function of spreadsheet applications

Critical Evidence(s) Required

A person who demonstrates competency in this unit must be able to provide evidence of the ability to create technical documentation that is clear to the target audience and easy to navigate. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

- create spreadsheets

- customize basic settings

- format spreadsheets

- create basic formulas

- insert objects and charts in spreadsheets

- Save and print spreadsheets.

# Perform writing and editing tasks

Overview: This unit describes the skills and knowledge required to apply the conventions of plain English to writing and editing tasks of different forms. It also includes editing and proofreading techniques. It applies to individuals in various writing contexts who write and edit texts using appropriate language, style, grammar, spelling, and standard conventions for editing and proofreading.

| Unit of Competency | Performance Criteria |
| --- | --- |
| 1. Apply clear and appropriate language and style to writing and editing tasks | 1.1 Use safe work practices including addressing ergonomic requirements when undertaking writing tasks 1.2 Use clear, concise and plain English in writing and editing tasks 1.3. Write / Use the active form in the sentences and avoid the Passive and Indirect form in the sentences in order to avoid ambiguities. 1.3 Use / Apply the appropriate connecting words and paragraph’s structures in the written materials to ensure the clarity, ease and reading comprehension 1.4 Make a clear and logical connections between sentences, paragraphs and sections with the follow and proper sequence of the reveiling knowledge and logic. 1.5 . Speak / Express / Incorporate the language and style of the audience |
| 2. Apply the appropriate voice, tone and tense | 2.1 Determine appropriate voice, tone and tense of the written materials according to audience requirements 2.2 Maintain consistent voice, tone and tense throughout written material |
| 3. Apply appropriate grammar, spelling and punctuation | 3.1 Apply appropriate grammar conventions to a range of written contexts including use of numbers, quotations, and tables 3.2 Apply appropriate spelling and punctuation conventions in writing and editing tasks. |
| 4. Perform editing and proofreading tasks to meet requirements | 4.1 Edit written material to ensure clear meaning through language and paragraphs, consistent voice, tone and tense 4.2 Copyedit written material by checking grammar, spelling and punctuation using standard editing conventions 4.3 Proofreading using style guides and by monitoring written material for errors |

Knowledge & Understanding

K1: main features of clear, concise and plain English language for written material

K2: grammar, punctuation and spelling conventions that meet the task requirements

K3: editing conventions used in substantive editing and copyediting of written material

K4: basic software used to write and collect feedback

Critical Evidence(s) Required

A person who demonstrates competency in this unit must be able to provide evidence of the ability to perform writing and editing tasks that is clear to the target audience and easy to navigate. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

- write and edit at least one written material (2500-4000 words) and edit another material written by another author (2500-4000 words), of different forms (e.g. blog, journal, book) that demonstrate use of:

  - clear and concise language

  - clear and logical paragraph structures

  - appropriate voice, tone, tense and language

  - plain English grammar, spelling and punctuation

  - accepted grammar conventions for a range of written contexts including use of numbers, quotation and tables

  - standard editing conventions

- complete editing and proofreading tasks using handwritten and digital methods

- accurately follow a style guide where required

follow relevant health and safety practices for writing tasks

# Perform computer operations

Overview: This unit covers the knowledge, skills and attitudes and values needed to perform computer operations which include inputting, accessing, producing and transferring data using the appropriate hardware and software.

| Unit of Competency | Performance Criteria |
| --- | --- |
| - Plan and prepare for task to be undertaken | - Requirements of task are determined as per standard Start with action word !!! - operating the procedures - Appropriate hardware and software is selected according to task assigned and required outcome - Task is planned to ensure |
| - Input data into computer | - Data are entered into the computer using appropriate Start with action word !!! - program/application in accordance with company procedures - Accuracy of information is checked and information is saved in accordance with standard operating procedures - Inputted data are stored in storage media according to requirements - Work is performed within ergonomic guidelines |
| - Access information using computer | - Correct program/application is selected based on job requirements - Program/application containing the information required is accessed according to company procedures - Desktop icons are correctly selected, opened and - closed for navigation purposes - Keyboard techniques are carried out in line with OH & S requirements for safe use of keyboards |
| - Produce/output data using computer system | - Entered data are processed using appropriate software commands - Data are printed out as required using computer hardware/peripheral devices in accordance with standard operating procedures - Files and data are transferred between compatible - systems using computer software, hardware/ eripheral - devices in accordance with standard operating - Procedures ?? Start with action word !!! |
| - Maintain computer equipment and systems | - Systems for cleaning, minor maintenance and replacement of consumables are implemented correct as above - Procedures for ensuring security of data, including regular back-ups and virus checks are implemented in accordance with standard operating procedures - Basic file maintenance procedures are implemented in line with the standard operating procedures |

Knowledge & Understanding

K1: Basic ergonomics of keyboard and computer use

K2: Main types of computers and basic features of different operating systems

K3: Main parts of a computer

K4: Storage devices and basic categories of memory

K5: Relevant types of software

K6: General security

K7: Viruses

K8: OH & S principles and responsibilities

K9: Calculating computer capacity

Critical Evidence(s) Required

The candidate needs to produce following Critical Evidence(s) to be competent in this competency standard:

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

- Selected and used hardware components correctly and according to the task requirement

- Identified and explain the functions of both hardware and software used, their general features and capabilities

- Produced accurate and complete data in accordance with the requirements

- Used appropriate devices and procedures to transfer files/data accurately

- Maintained computer system

# Use computer applications

Overview: This unit describes the performance outcomes, skills and knowledge required to identify, select and operate three commercial software packages, including a word-processing, a spreadsheet and presentation application package.

| Unit of Competency | Performance Criteria |
| --- | --- |
| 1. Use appropriate OHS office work practices | 1.1 Use safe work practices to ensure ergonomic work ?? not clear organization, energy and resource conservation requirements are addressed 1.2 Use appropriate wrist rests and document holders 1.3 Use monitor anti-glare and radiation reduction screens where appropriate |
| 2. Install and remove software | 2.1 Select software to be installed 2.2 Follow installation instructions 2.3 Delete unwanted software |
| 3. Use appropriate word-processing software | 3.1 Select word-processing software appropriate to perform activity 3.2 Identify document purpose, audience and presentation requirements, and clarify with the concern personnel 3.3 Identify organizational requirements for text-based business documents and design document structure and layout to ensure consistency of style and image 3.4 Match document requirements with software functions to provide efficient production of documents 3.5 Use technical functions, other data and formatting to finalize documents 3.6 Ensure the naming and storing of documents in appropriate directories or folders and the printing of documents to the required specifications |
| 4. Use appropriate spreadsheet software | 4.1 Select spreadsheet software appropriate to perform activity 4.2 Identify document purpose, audience and presentation requirements, and clarify with personnel as required 4.3 Enter simple formulas and functions using cell referencing where required 4.4 Customize spreadsheet settings and format documents to meet requirements 4.5 Ensure the naming and storing of documents in appropriate directories or folders and the printing of documents to the required specifications |
| 5. Use appropriate presentation software | 5.1 Select software application package appropriate to perform activity 5.2 Identify purpose, audience and presentation requirements, and clarify with personnel as required 5.3 Use technical functions, other data and formatting to finalize documents 5.4 Ensure documents are named and stored in appropriate directories or folders and printed to required specifications 5.5 Make a presentation |

Knowledge & Understanding

K1: Application software packages used by the organization

K2: Basic technical terminology related to reading help files and responding to system help prompts

K3: Basic knowledge of system usage

K4: Current business practices related to using software to prepare reports

K5: Features and functions of commercial computing packages

K6: Import and export software functions

K7: Linking documents

K8: OHS principles and responsibilities for ergonomics, such as work periods and breaks

K9: Purpose, use and functions of applications

K10: Use of input and output devices

K11: Functions and uses of word processing, spreadsheet and presentation software

Critical Evidence(s) Required

A person who demonstrates competency in this unit must be able to provide evidence of the ability to identify, select and operate three commercial software packages, including a word-processing and a spreadsheet application package. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Evidence of the ability to:

- Produce workplace documents using a minimum of three different software application packages

- Open, amend and save files and documents according to organizational requirements

- Use OHS principles and responsibilities for ergonomics, such as work periods and breaks

- Use help manuals and online help.

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

- Use an operating system in a variety of scenarios and across functions, including:

  - scheduling, loading, initiating, and supervising the execution of programs

  - allocating storage

  - initiating and controlling input and output operations

  - handling errors

- Identify and install suitable hardware components

- Install and upgrade application software.

# Create user documentation

Overview: This unit describes the performance outcomes, skills and knowledge required to create user documentation that is clear to the target audience and easy to navigate.

| Unit of Competency | Performance Criteria |
| --- | --- |
| 1. Determine documentation standards and requirements | 1.1 Determine documentation requirements 1.2 Investigate documentation and industry standards for requirements and determine appropriate application to user documentation 1.3 Design documentation templates using appropriate software and obtain approval from appropriate person |
| 2. Produce user documentation | 2.1 Conduct a review of the subject system, program, network or application in order to understand its functionality 2.2 Gather existing technical, design or user specifications and supporting documentation 2.3 Create user documentation based on template to record the operation of the subject system, program, network or application |
| 3. Review and obtain sign-off | 3.1 Submit user documentation to target audience for review 3.2 Gather and analyze feedback 3.3 Make changes to user documentation 3.4 Submit user documentation to appropriate person for approval |

Knowledge & Understanding

K1: Content features, including clarity and readability

K2: Document design, web design and usability

K3: Functions and features of templates and style guides

K4: Instructional design principles

Critical Evidence(s) Required

A person who demonstrates competency in this unit must be able to provide evidence of the ability to create user documentation that is clear to the target audience and easy to navigate. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to create user documentation that:

- Meets business requirements

- Caters for a diverse readership

- Is clear to the target audience

- Is easy to navigate.

# Create technical documentation

Overview: This unit describes the performance outcomes, skills and knowledge required to create technical documentation that is clear to the target audience and easy to navigate.

| Unit of Competency | Performance Criteria |
| --- | --- |
| 1. Identify and analyze documentation requirements and client needs | 1.1 Consult with client to identify documentation requirements 1.2 Interpret and evaluate documentation requirements and confirm details with client 1.3 Investigate industry and documentation standards for requirements 1.4 Define and document the scope of work to be produced 1.5 Consult with client to validate and confirm the scope of work |
| 2. Design documentation | 2.1 Identify information requirements with reference to layout and document structure 2.2 Create document templates and style guides consistent with information requirements 2.3 Conduct a review of the system in order to understand its functionality 2.4 Extract content that meets information requirements according to copyright restrictions 2.5 Develop the structure of the technical documentation giving focus to the flow of information, style, tone and content format 2.6 Validate the technical documentation structure with the client |
| 3. Develop documentation | 3.1 Write technical documentation based on the template and scope of work using the information gathered 3.2 Translate technical terminology into simple / plain English where appropriate 3.3 Apply content format and style according to documentation standards and templates |
| 4. Evaluate and edit documentation | 4.1 Submit technical documentation to appropriate person for review 4.2 Gather and analyze feedback 4.3 Incorporate alterations into the technical documentation 4.4 Edit the technical documentation for technical and grammatical errors. |
| 5. Prepare documentation for publication | 5.1 Check that the completed technical documentation meets client requirements and scope of work 5.2 Submit the technical documentation to appropriate person for approval correct as previous 5.3 Prepare the technical documentation for publication and distribution using appropriate channels |

Knowledge & Understanding

K1: Content features, such as clarity and readability

K2: Document design, web design and usability

K3: Functions and features of templates and style guides

K4: Instructional design principles

K5: Organizational policies, procedures and standards that cover document design.

Critical Evidence(s) Required

A person who demonstrates competency in this unit must be able to provide evidence of the ability to create technical documentation that is clear to the target audience and easy to navigate. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

- Establish customer needs

- Design and develop technical documentation, such as system, procedures, training material and user guides, incorporating appropriate standards

- Update document with client feedback

- Prepare documentation for publication.

# Create basic databases

Overview: This unit describes the skills and knowledge required to design, develop and test a database in order to meet a specification. It applies to individuals who may be either database, or web designers, required to create a simple database to store information for an online application, using a simple entity relational database.

| Unit of Competency | Performance Criteria |
| --- | --- |
| 1. Analyze the requirements for the database | 1.1 Determine the information that the database is required to hold 1.2 Develop a written requirement report for the functionality of the database 1.3 Complete the documentation, and submit it to an appropriate person in order to be approved |
| 2. Use data modeling to design the database to suit requirements | 2.1 Design an entity-relationship (ER) diagram to model the relationships between the entities and the attributes that the database will hold 2.2 Develop primary and foreign keys to link the entities 2.3 Develop a data dictionary 2.4 Complete the documentation, and submit it to the appropriate person for approval correct as above |
| 3. Create a database on a web or database server | 3.1 Use the appropriate language on a web or database server to create few databases 3.2 Use the appropriate language on a web or database server to create few tables 3.3 Populate the database fields |
| 4. Test the database and debug | 4.1 Test the database on the web or a database server 4.2 Ensure that the information represented matches the requirements |

Knowledge & Understanding

K1: outline the principles of open platforms, including browsers and databases

K2: list the processes associated with the creation of entities, attributes, and I populating fields, using both software solutions and script- based input

K3: describe data-modeling techniques to design a database

K4: outline the steps in database design, modeling and implementation

K5: describe the internet operation related to web servers and clients

K6: identify the naming conventions appropriate to database design

K7: identify security restrictions on servers, incorporating some theoretical concepts

K8: describe best practice communication, and accessibility, for audiences with special needs.

Critical Evidence(s) Required

A person who demonstrates competency in this unit must be able to provide evidence of the ability to create technical documentation that is clear to the target audience and easy to navigate. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

- research client requirements for a database solution

- design a database that meets client requirements

- create a database on a web hosting service or server to meet client requirements by a due date

- Test and debug the database.

# Operate digital media technology

Overview: This unit describes the performance outcomes, skills and knowledge required to identify, select and use a digital media package and supporting technologies.

| Unit of Competency | Performance Criteria |
| --- | --- |
| - Use appropriate OHS office work practices | - Use safe work practices to ensure ergonomic, ??? work organization, energy and resource conservation requirements are addressed - Use wrist rests and document holders where appropriate - Use monitor anti-glare and radiation reduction screens where appropriate |
| - Identify and select appropriate digital media package | - Identify the basic requirements of a design brief, including user environment - Research and review suitable available digital media packages - Select an appropriate digital media package to meet design brief requirements |
| - Use digital media package | - Procure or create suitable data to meet requirements of the brief - Manipulate data using digital media package tools - Ensure naming and storing of documents in appropriate file format in directories or folders |
| - Review digital media design | - Evaluate design for creative, dramatic and technical quality, file size, and suitability to meet the brief - Test and run any incorporated graphics, video or sound as part of a digital media presentation and present designs in the appropriate format - Review final product against the design |

Knowledge & Understanding

K1: Basic principles of visual design

K2: Functions and features of digital media packages and technologies

K3: Graphic design and stylistic language conventions

K4: OHS principles and responsibilities for ergonomics, such as work periods and breaks

K5: Principles of digital imaging and file formats, video and sound file formats, file management and transfer systems

K6: Vendor product directions in digital media hardware and software

K7: Visualization and interpreting creative information, scripts (text) and images

Critical Evidence(s) Required

A person who demonstrates competency in this unit must be able to provide evidence of the ability to identify, select and use a digital media package and supporting technologies. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorised work practices, safety requirements and environmental constraints. Demonstrated evidence of the ability to:

- Identify basic requirements of a design brief

- Use digital media package to meet organizational requirements

- Use OHS principles and responsibilities for ergonomics, such as work periods and breaks

- Use help manuals and online help when appropriate

- Use digital media technologies to support design brief requirements.

# Use social media tools for collaboration and engagement

Overview: This unit describes the performance outcomes, skills and knowledge required to establish a social networking presence using social media tools and applications. The unit specifically identifies the requirement to review, compare and use different types of social networking tools and applications.

| Unit of Competency | Performance Criteria |
| --- | --- |
| 1. Describe different types of social media tools and applications | 1.1 Explain characteristics of the term social media 1.2 Identify different types of social-media tools and applications 1.3 Illustrate various issues associated with the use of social media tools and applications |
| 2. Compare different types of social media tools and applications | 2.1 Select one social media type for review 2.2 Review most popular tools and applications within that social media 2.3 Itemize benefits across a range of the most popular tools and applications 2.4 Select most appropriate social media tool or application |
| 3. Set up and use popular social media tools and applications | 3.1 Identify social media tools and applications for possible implementation 3.2 Initiate preferred social media tools and applications ?? 3.3 Establish social media interface using text and file content 3.4 Initiate social network interaction 3.5 Test and evaluate tools and applications for ease of use 3.6 Report and elaborate the findings |

Knowledge & Understanding

K1: Basic technical terminology in relation to social networking and social media applications and tools

K2: Basic knowledge of uploading images, text files, pdf files, audio files, video files and link associated files

K3: Features and functions of social media applications

K4: Import and export software functions

K5: Linking documents

K6: OHS principles and responsibilities for ergonomics, including work periods and breaks

K7: Tagging to facilitate collaborative folksonomy

K8: Social media applications and procedures for connecting to social networking sites

K9: Use of input and output devices

K10: Use of RSS feeds to connect a social network.

Critical Evidence(s) Required

A person who demonstrates competency in this unit must be able to provide evidence of the ability to create technical documentation that is clear to the target audience and easy to navigate. The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other circumstances and environments.

Performance requirements

This competency is to be assessed using standard and authorized work practices, safety requirements and environmental constraints. Demonstrated evidence is required of the ability to:

- Establish customer needs

- Design and develop technical documentation, such as system, procedures, training material and user guides, incorporating appropriate standards

- Update document with client feedback

- Prepare documentation for publication.

# 0714E&A13.      Apply basic CAD software for Designing & Drawing

Overview:

After completing this competency, the student will be able to draft drawings of architecture, machines, parts and layouts using CAD software as required in the industry.

| Competency Units | Performance Criteria |
| --- | --- |
| - Identify drawing requirements. | - Determine the Requirements and purpose of drawing. - Record the data related to the drawing task. - Verify the data related to the drawing task. |
| - Create and Edit engineering drawing manually. | - Identify and apply different drafting principles to produce drawing. - Select appropriate drawing method for drafting. - Verify drawing in accordance with safety standards and operating procedures (SOP). |
| - Identify engineering parts and related drawing issues. | - Identify and organize list of engineering components. - Draw and record part list according to standard operating procedures. - Verify part list according to standard operating procedures. |
| - Identify elements of the CAD software interface and Plot a basic level drawing. | - Create new drawing document using latest CAD software, Identifying file extensions, file locations. - Sketch drawing using drawing tools, edit tools and modify tools. - Draw the sketch using the Command line, Ribbon, Dynamic input and Menu browser. |
| - Create CAD drawing using multilayer and object editing tools | - Create and managing multiple layers that define line color, line width, line type etc. - Create drawing using object editing tools (such as fillet, chamfer, break, join, trim, extend, lengthen, and scale, arranging and patterning objects with move, copy, mirror, rotate, align, and array, Obtaining and editing object information through quick properties and Editing object properties. - Create polylines, revision clouds, splines, editing and exploding polylines. |
| - Create CAD drawing using Text and non-pictorial information. | - Differentiate the oblique, axonometric, and isometric drawings. - Create isometric drawings, including isometric ellipses and arcs. - Create section views (e.g., full, offset, half, aligned, revolved, removed, and broken out) and applying the hatch tool to depict material surfaces and solids. - Insert tables and fields into a CAD drawing. - Identify different industry standards for dimensioning |
| - Create basic 2D drawings in CAD software. | - Draw lines, curves, circles, ellipses, rectangles, polygons, and donuts in CAD software. - Identify the use of various types of Object Snaps and Auto tracking. - Construct drawing points, construction, lines and rays. - Create multi view drawings of an object - Create partial and full auxiliary view |
| - Create 3D models of physical parts and assemblies. | - Perform the modeling of the components and useful solid geometry. - Produce assemblies from 3D models of component - Draw first angle and third angle orthographic projections manually. - Draw isometric projection manually. - Convert 2D drawings to poly-lines and poly-lines to 3D object. - Observe different views of 3D object using view ports - Plot the multi views and shows the hidden features of a 3D object. |
| - Exporting CAD drawing to different media. | - Identify the difference between model and paper space - Create multiple layouts for a drawing. - Use floating viewports using the correct paper size, orientation - Plot drawings to scale, Outputting to different media |
| - Create photo rendered images | - Create photo rendered images of components or assemblies, Save and label images to access for further use. - Interpret source information - Apply material properties using information supplied from source drawings - Create photo rendered images of components or assemblies |
| - Apply reverse engineering of a physical model. | - Create free hand sketches of industry accepted instruments - Create part and assembly technical drawings in standard format. - Insert written information with more than one column using annotation styles that meet ISO standards. - Produce print outs of drawings in sizes A0 to A4 to ISO standard. |

Knowledge and understanding

- Basic understanding of the customer requirement.

- Describe the CAD software tools, file generation and GUI.

- Explain basics of computer operating systems

- Explain Drafting principles and procedure to produce a drawing.

- Explain knowledge in the use of the CAD software interface for the purpose to Create 3D CAD models of parts and assemblies

- Explain the materials and processes for obtaining unprocessed work pieces

- Explain the output drawings in CAD

- Explain the process to transfer a real object to a 3D image & a technical drawing.

- Explain the rendering

- Explain the rules of technical drawing and the prevailing latest ISO standards that govern those rules.

- Explain the uses of lighting, scenes and decals to produce photo rendered images

- Knowledge and understanding of Issue related to final drawing draft.

- Learn the knowledge about different industrial standards for dimensioning.

- Learn the knowledge to be able to configure the parameters of the software.

- Understand the basics of drawing & its requirement.

- Understand the concept to modify a CAD drawing.

- Understand the key terms, concepts associated with drafting and the drafting profession.

- Understand the principles of technical drawing.

- Understand the usage of drafting in industries as an important aspect in their work.

- Understand the working with text and conveying no pictorial information in CAD software.

- Understanding of the skills necessary to create basic 2D CAD drawings

Tools & Equipment

| SN |  |
| --- | --- |
|  | Cloth, |
|  | Computer installed with CAD software. |
|  | Drafting machine, |
|  | Drafting paper, |
|  | Drawing materials, |
|  | French Curves, Rulers, Compass, |
|  | Pencil, Drafting board, |
|  | Perspective machines, |
|  | Printer/ plotter with all accessories |
|  | Templates |
|  | Thick draft paper, |
|  | Tracing paper, |
|  | Tracing tube, Inks |
|  | T-square, |

.

Critical Evidence(s) Required

The student needs to produce following critical evidence(s) in order to be competent in this competency standard:

Evidence of the following is essential:

- Create 2D/3D drawing

- Perform Rendering

# 0714E&A14.      Install Temperature & Pressure Measuring Instruments/Devices

Overview:

After completing this competency standard, the student will be able to understand the functions, uses, Maintenance & Installation of temperature & Pressure measuring devices.

| Competency Units | Performance Criteria |
| --- | --- |
| - Install the Temperature/Pressure measuring instruments. | P1. Analyze the type of Instrument to be installed P2. Select the appropriate type of instrument and the Scale of measurement P3. Record the output values |
| - Measure the Temperature using Thermocouple & Resistance Thermocouple | P1. Analyze the type of thermocouple to be installed P2. Install the required thermocouple as per installation manual P3. Analyze accuracy, power and flexibility of thermocouples P4. Plot the temperature graph as required. |
| - Install Pressure Transducer & Transmitters. | P1. Analyze the type of Pressure measuring instrument P2. Install the Pressure transducer as per the installation guidelines P3. Install the pressure Transmitter as per Installation guidelines P4. Test & Calibrate the installed pressure measuring instrument |
| - Install Pressure Switch | P1. Analyze the type of pressure switch to be installed P2. Install the pressure switch as per instructions manual P3. Test and Calibrate the pressure switch |

Knowledge and understanding

- Explain the principle of heat transfer

- Explain temperature and scales.

- Explain pressure, types, its units and the formulae.

- Differentiate the Gauge & Atmospheric pressure

- Differential Pressure controller

- And temperature controller

- Different types of thermocouples, heat sensing devices

- Explain thermocouple basic, its working principles and uses in industry.

- Explain the PTC & NTC resistance thermocouple basics, its working principles and uses in industry

- Explain the advantages and disadvantages of Thermocouples

- Explain pressure, transducer, Transmitter and their working principles

- Describe the uses of Pressure Transmitters and Transducers in industry

- Describe the advantages of pressure transducer & Transmitter

- Explain pressure, transmitter, its working and principle

- Describe the importance and requirement of pressure switch

- Describe the advantages of pressure switch

| SN |  |
| --- | --- |
|  | 4-20 map Devices |
|  | Different types of thermocouples, heat sensing devices |
|  | J type thermocouple |
|  | K type thermocouple |
|  | Mass flow controller |
|  | Multi meter |
|  | NTC thermocouple |
|  | Pressure gauge |
|  | Pressure monitor |
|  | Pressure switch |
|  | Pressure transducer |
|  | Pressure transmitter |
|  | PT-100, 1000 and RTD Thermocouple |
|  | PTC thermocouple |
|  | S type thermocouple |
|  | Temperature Gun |
|  | Temperature measuring Graphs |
|  | Toolkit |

Critical Evidence(s) Required

The student needs to produce following critical evidence(s) in order to be competent in this competency standard:

Evidence of the following is essential:

- Install pressure switch in a circuit

- Use Thermocouple in a circuit

# 0714E&A15.      Install Level Measuring Instruments/Device

Overview:

After completion of this competency standard the student will be able to grasp the basic concepts of level measurement, its uses and installation.

| Competency Units | Performance Criteria |
| --- | --- |
| 1. Install the Level Measurement device | P1. Analyze the type of level measurement Instrument P2. Classify the level measurement sensors P3. Install the level measuring instruments as per the installations guidelines P4. Test & Calibrate the level measuring device |
| 2. Install the optical Instrument for Level Measurement | P1. Analyze the type of Optical instrument for level measurement P2. Install the optical instrument as per installations guidelines P3. Test and calibrate the optical instrument |
| 3. Install the Ultrasonic Level measuring Instruments | P1. Analyze the type of Ultrasonic instrument for level measurement P2. Install the Ultrasonic instrument as per installations guidelines P3. Test and calibrate the Ultrasonic instrument |
| Install the float instruments for Level measurement | P1. Analyze the type of float level measuring device P2. Install the float level measuring device as per installations manual P3. Check & Calibrate the float level measuring device |
| Install the capacitive Level measuring device | P1. Analyze the type of capacitive level measuring device P2. Install the capacitive level measuring device as per the installations manual P3. Check and calibrate the capacitive level measuring device |

Knowledge and understanding

- Define the units of level and their conversion.

- Describe calibration and  advantages of optical level measurement

- Describe the advantages of capacitive level measurement

- Describe the advantages of float level measurement

- Describe the advantages of ultrasonic level measurement

- Describe the importance and requirement of capacitive level measurement

- Describe the importance and requirement of float level measurement

- Describe the importance and uses optical level measurement

- Describe the uses of Ultrasonic measuring instruments in industry

- Explain capacitive level measurement &its working principle

- Explain float level measurement &its working principle

- Explain optical level measurement, its working and principle

- Explain the Direct & Indirect level measurement.

- Explain the importance of level measurement in industry

- Explain ultrasonic level measurement &its working principle

Tools & Equipment

| SN |  |
| --- | --- |
|  | Analog Level sensor |
|  | capacitive level Devices and sensors |
|  | Digital Level sensor |
|  | float level sensor |
|  | Horizontal and vertical |
|  | Multi meter |
|  | Optical level sensor |
|  | Sonicator |
|  | Toolkit |
|  | Ultrasonic level sensor |

Critical Evidence(s) Required

The student needs to produce following critical evidence(s) in order to be competent in this competency standard:

Evidence of the following is essential:

- Working principal of level sensors

- Connection of optical sensors

# 0714E&A16.      Install the Flow Measuring Instruments/Devices

Overview:

After completion of this competency standard the student will be able to grasp basic concepts, perform the uses and installation of Flow measurement.

| Competency Units | Performance Criteria |
| --- | --- |
| - Install the Flow - Measuring instrument | P1. Analyze the type of flow measuring device P2. Install the flow measuring device(Air and Liquid) as per the installations manual P3. Check and calibrate the flow measuring device |
| - Install Differential Pressure Flow Meters | P1. Analyze the type of differential pressure flow meter P2.Install the differential pressure flow meter as per installations guideline P3. Check and calibrate the differential pressure flow meter |
| - Implement the Piston & Gear Flow Meters | P1. Analyze the type of Piston flow meter P2.Install the Piston flow meter as per installations guideline P3. Check and calibrate the Piston flow meter |
| - Install the Sluice Gate & Weir Flow Meters | P1. Analyze the type of Sluice Gate & Weir flow meter P2.Install the Sluice Gate & Weir flow meter as per installations guideline P3. Check and calibrate the Sluice Gate & Weir flow meter |

Knowledge and understanding

- Define flow and its types and measurements

- Define Mass, weight, Density, Velocity & their units

- Describe the function of  Venturi Tubes and its working principle

- Describe the function of flow nozzle and its working principle

- Describe the function of gear flow meter and its working principle

- Describe the function of orifice plate and its working principle

- Describe the function of piston flow meter and its working principle

- Describe the function of Rota-meters (Variable Area) and its working principle

- Describe the function of Sluice Gate and its working principle

- Describe the function of weir and its working principle

- Describe the Reynold number & Reynold Equation.

- Enlist chemical and physical properties of fluids.

- Enlist flow rate meters.

- Explain differential pressure measurement flow meter &its working principle

- Explain open channel  flow meter, its working and principle

- Explain positive displacement flow meter, its working and principle

Tools & Equipment

| SN |  |
| --- | --- |
|  | Air Flow meter |
|  | Liquid flow meter |
|  | Mass flow meter |
|  | Differential Pressure Flow meter |
|  | Displacement |
|  | Pressure Flow meter |
|  | Multi meter |
|  | Toolkit |
|  | Calibrator• |
|  | Open channel Pressure Flow meter |

Critical Evidence(s) Required

The student needs to produce following critical evidence(s) in order to be competent in this competency standard:

Evidence of the following is essential:

- Working principal of flow meter

- Connection of  Differential Pressure Flow Meters

# Take measurements with graduated tools

Overview: This competency standard covers the skills and knowledge required to take measurements Steel rule, Take measurements Hook rule, Take measurements Folding rule and Take measurements with Trammels.

| Competency Unit | Performance Criteria |
| --- | --- |
| CU1. Take measurements Steel rule | - Place ruler on different work pieces. - Take dimension and record multiple readings on each work piece. - Take average of readings. - Record the results. |
| CU2. Take measurements Hook rule | - Place hook rule on given different work piece. - Take dimension and record multiple readings on each work piece. - Take average of readings. - Record the results. |
| CU3. Take measurements Folding rule | - Take suitable work piece for measurement. - Measure the dimensions of work pieces. - Compute surface area, volume, of given work pieces. - Record the results in data table. |
| CU4. Take measurements with Trammels | - Open the trammel according to required dimension. - Draw circle or arc by fixing one point in the dot and rotate. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Define Decimal and  fraction of an inch

- Define Measurement techniques

- Describe Calculation of area, volume of geometric shapes.

- Define Arc and circle

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Perform measurement and take a 4 readings on each work piece and calculate the average reading.

- Draw circle or arc by fixing one point in the dot and rotate.

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Steel rule |
|  | Surface plate |
|  | Steps and collars |
|  | Hook rule |
|  | Folding rule |
|  | Trammel |

# Take measurements with combination set

Overview: This competency standard covers the skills and knowledge required to take measurements with Combination Set.

| Competency Unit | Performance Criteria |
| --- | --- |
| CU1. Take Measurement with Square head | - Place the combination set on given drawing sheet. - Draw layout of a component with the help of combination square. |
| CU2. Perform leveling with square head as spirit level | - Place the spirit level on horizontal work piece. - Check the deviation of bubbles - Level the surface by inserting shims. - Place square head on vertical work piece. - Note the deviation and correct the level. - Check the other geometrical shapes. |
| CU3. Measure depth with square head as depth gauge | - Place the square head on work piece. - Apply force at the head to retain contact with the component. - Set square head and record the depth. |
| CU4. Measure height with square head as height gauge | - Place the square head on work piece. - Measure height of work piece with and without the help of height gauge attachment. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Define Measurement techniques

- Define dimensioning of drawings and layouts

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Draw layout of a component with the help of combination square.

- Perform leveling /measure angle of surfaces with protractor Head

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Combination set |
|  | Surface plate |
|  | Scriber |
|  | Tri square |
|  | Divider |
|  | Round stock |

# Take measurements through various gauges

Overview: This competency standard covers the skills and knowledge required to take measurements with various type of gauges.

| Competency Unit | Performance Criteria |
| --- | --- |
| CU1. Take measurement with fixed gauge and plug gauge. | - Check/Compare the dimension of provided work piece against reference standard with the help of fixed gauge. - Insert plug gauge into the work piece to determine either dimension is of acceptable size or over size. |
| CU2. Take measurement with adjustable gauge | - Insert both “go” and “not go” gauges in the work piece. - Check the size and record the results. |
| CU3. Take measurement with small hole gauge | - Insert the small hole gauge of require size into the work piece - Record the result. |
| CU4. Take measurement with telescope gauge | - Insert the telescope gauge into the work piece - Adjust the gauge and determine the size of work piece. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Define  dimensioning and measurement techniques

- Define and differentiate the gauges type

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Draw layout of a component with the help of combination square.

- Perform leveling /measure angle of surfaces with protractor Head

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Fix gauge |
|  | Surface plate |
|  | Surface gauge |
|  | Dial indicator |
|  | Adjustable gauge |
|  | Hole gauge |
|  | Telescope |

# Perform measurements through Micrometre

Overview: This competency standard covers the skills and knowledge required to take measurements with micrometre.

| Competency Unit | Performance Criteria |
| --- | --- |
| CU1. Take measurement with outside micro-meter | - Clean the gauge and surface of work piece - Open slightly larger than the part to be measured - Set anvil squarely against reference surface of part - Using ratchet, slowly until it click once - Record reading and make average. |
| CU2. Take measurement with inside micrometer | - Clean the gauge and surface of work piece. - Open gauge slightly smaller than the part to be measured - Set anvil squarely against reference surface of part with ratchet, slowly until it click once. - Record reading and make average. |
| CU3. Take measurement with depth micrometer | - Clean the gauge and surface of work piece. - Access the depth of groove by steel rule - Insert the suitable insertion rod - Place the depth micrometer gauge on the deeper groove and take reading. |
| CU4. Measure threads with micrometer | - Clean the gauge and the threads of work piece. - Use the suitable set of thread shape tips according to thread size which is to be measured. - Record the reading. |
| CU5. Take measurement with Vernier micrometer | - Clean the surface. - Place specimen on the surface plate - Take reading accurately. - Calculate mean reading. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Define Measurement techniques

- Describe dimensioning of drawings and layouts

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Perform measurement with inside micrometer

- Perform the use of hole gauge

- Perform to operate telescope gauge

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Inside Micrometer |
|  | Outside Micrometer |
|  | Depth Micrometer |
|  | Screw thread Micrometer |

# Measure dimensions with Vernier Calliper

Overview: This competency standard covers the skills and knowledge required to take measurements with Vernier caliper.

| Competency Unit | Performance Criteria |
| --- | --- |
| CU1. Take measurement with Vernier caliper | - Clean the gauge and surface of work piece. - Place the work piece on surface plate. - Open the lock screws. - Slide the jaws up to the width or size of jaws. - Open slightly larger than the part to be measured. - Set anvil squarely against reference surface of part - Maintain the proper pressure on the jaws with screw. - Lock the movable jaw with knurled screw - Measure and record the dimensions. |
| CU2. Take measurement with height gauge | - Clean the gauge and surface of work piece - Hold the work piece on angle plate. - Open the lock screws. - Set the height gauge on stud to check it with fine adjusting screw. - Lock the measuring head. - Record the reading. |
| CU3. Take measurement with Vernier depth gauge | - Clean the gauge and surface of work piece - Hold the work piece on sample plate. - Open the lock screws. - Insert the gauge inside the work piece with fine adjusting screw. - Lock the measuring head. - Record the reading |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Define Vernier calipers, height gauge and Vernier depth gauge

- Define different type of Measurement techniques

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Perform measurement (inside, outside & depth) with Vernier caliper

- Identify Vernier scale and Main scale

- Define least count

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Vernier caliper |
|  | Height Gauge |
|  | Vernier Depth Gauge |

# Perform different measurements

Overview: This competency standard covers the skills and knowledge required to take measurements with Dial caliper, Dial thickness gauge, Dial indicator, Gauge blocks, Tool maker’s microscope, Coordinate measuring machine

| Competency Unit | Performance Criteria |
| --- | --- |
| CU1. Take measurement with dial calliper | - Set the pointer on zero properly - Set the work piece correctly - Read the dial scale accurately - Record the results |
| CU2. Take measurement with dial thickness gauge | - Remove zero error - Set the work piece properly - Measure the size accurately - Record the results |
| CU3. Take measurement with dial Indicator | - Set the Indicator on magnetic stand - Set the pointer on zero - Set the pointer on work piece - Read the dial scale |
| CU4. Exercise on gauge blocks | - Adjust the gauge block as per desired size - Hold the specimen in position - Measure the angle accurately sine bar and gauge blocks |
| CU5. Exercise on tool makers microscope | - Prepare the microscope for examining the job - Hold the job on anvil - Observe the size and profile of the job by setting eye piece - Record the observations |
| CU6. Practice on Profile Projector | - Make complete setting of work piece on Profile Projector - P2. Set the axis - P3. Observe the size |
| CU7. Practice Of Digital Instruments | - Select digital instrument and specimen for measurement. - Take measurement of specimen with digital instrument |
| CU8. Measure with coordinate measuring machine | - Hold the job properly. - Set and locate zero point. - Take the dimensions for different coordinates properly. - Take point to point/linear measurements. - Take measurements profile in 2D and 3D. |
| CU9. Calculate tolerance and allowances | - Measure the job for upper and lower limits. - Calculate the maximum and minimum size. - Calculate tolerance and allowance - Represent tolerance in results. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Define Measurement techniques

- Define Digital measuring instruments

- Describe Knowledge of dimensioning and layouts

- Define ISO standards of fits and tolerance

- Describe Knowledge of components e.g. shafts, holes etc

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Define different type of measuring techniques

- Perform to operate dial indicator

- Define how to operate Vernier caliper

- Define how to adjust the gauge block as per desired size

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Vernier caliper |
|  | Dial thickness gauge |
|  | Dial indicator |
|  | Gauge blocks |
|  | Tool makers microscope |
|  | Coordinate measuring machines |
|  | ISO tables of fits and tolerance |
|  | Measurement tools |

# Operate Measuring Instruments

Overview: This competency standard covers the skills and knowledge related to Calibration in measurement technology and metrology is the comparison of measurement values delivered by a device under test with those of a calibration standard of known accuracy. The term calibration means just the act of comparison, and does not include any subsequent adjustment.

| Competency Unit | Performance Criteria |
| --- | --- |
| - CU1. Operate Ampere meter | - Identify ampere meter. - Make a parallel circuit with three different resistors on breadboard and connect with the power supply. - Adjust proper range setting of ampere meter as per load. - Connect ampere meter in series to each resistor to measure the current. - Turn on the supply and note the reading of current against each resistor. |
| - CU2. Operate Volt meter | - Identify volt meter. - Make a series circuit with three different resistors on bread board. - Adjust proper range setting of Volt meter as per load. - Connect volt meter across any resistor in circuit - Turn on the supply and note the reading of voltage drop against each resistor. |
| - CU3. Operate ohm meter. | - Identify Ohm meter. - Adjust proper range of ohm meter as per resistance value. - Disconnect the resistor from the circuit - Connect ohm meter with resistor in series and parallel combination. - Record the reading. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Define current and its unit.

- Describe series circuit.

- Enlist the name of various meters

- Describe purpose of ampere meter

- Describe how ampere meter is connected in the circuit.

- Define voltage and its unit.

- Describe parallel circuit.

- Describe purpose of volt meter

- Describe how voltmeter is connected in the circuit.

- Define resistance and its unit

- Describe purpose of ohm meter

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Perform turn on the supply and note the reading of current/voltage against each resistor.

- Perform to Adjust the scale for measurements.

- Perform probe selection for appropriate measurement.

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Voltmeters |
|  | Ammeter |
|  | Ohmmeters |
|  | Multi meters. |
|  | Resistors of different values. |

# Verify Ohm’s Law & Kirchhoff’s Law by Implementing Series/Parallel Circuits

Overview: This competency standard covers the skills and knowledge related to Make series circuit and measure voltage and verify KVL, make parallel circuit and measure current and verify KCL, verify resistance of a resistor, Find unknown value of ohm’s law.

| Competency Unit | Performance Criteria |
| --- | --- |
| - CU1. Make series circuit and measure voltage and verify KVL | - Construct series circuit with different resistors on bread board and give supply voltage. - Measure the voltage across each resistor. - Record the reading. - Sum the voltage drop across each resistor. - Compare the total voltage with the applied voltage |
| - CU2. Make parallel circuit and measure current and verify KCL | - Construct parallel circuit with different resistors on bread board. - Connect ampere meter in series to each resistor to measure the current. - Record the reading. - Sum the current of each resistor - Compare the sum with total consumed current. |
| - CU3. Verify resistance of a resistor using color codes | - Select the 4 colour band resistor. - Determine the value of resistor using colour code. - Connect ohm meter across the resistor. - Record reading form ohm meter. - Compare both reading. |
| - CU4. Measure resistance of incandescent lamp. | - Take 40 W lamp and 1 digital or analogy ohmmeter. - Adjust zero with shortening the ohmmeter leads. - Connect the ohmmeter with lamp terminal and note resistance - Repeat this method for 60 & 100 W lamp |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Define Describe characteristics of series circuit

- Describe the use of volt meter

- Describe characteristics of parallel circuit

- Describe the use of ampere meter

- Describe color codes and its use.

- Describe the purpose of ohm meter

- Explain Kirchhoff’s voltage law.

- State Kirchhoff’s current law

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Adjust zero error with shortening the analog ohmmeter leads.

- Perform series circuit and measure voltage across each resistor and verify KVL

- Perform parallel circuit and measure current of each resistor and verify KCL

- Verify resistance of a resistor using color code and verify using ohmmeter.

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Voltmeters |
|  | Ammeter |
|  | Ohmmeters |
|  | Multi meters. |
|  | Resistors of different values. |
|  | Battery |
|  | Copper wire. |
|  | Rheostat |
|  | Incandescent lamp |

# Measure Electrical Power, Energy, Power Factor & Determine Phase Sequence

Overview: This competency standard covers the skills and knowledge required to Measure single phase and poly-phase  electrical para meters using Volt-Ampere meter, watt meter & Measure consumed energy with Energy meter.

| Competency Unit | Performance Criteria |
| --- | --- |
| - CU1. Measure power using Multimeter method | - Construct Select the load to calculate the power and give supply. - Measure the voltage and current by using Multimeter. - Find power using power formula. |
| - CU2. Measure power using watt meter | - Construct Select a load to calculate the power and connect watt meter across the load. - Power-ON the supply and measure the power. |
| - CU3. Measure single phase AC Power | - Select a load to calculate the power and appropriate supply. - Measure the voltage and current using Multimeter. - Calculate the power using power. |
| - CU4. Measure three phase AC Power | - Select a load to calculate the power with appropriate supply. - Measure the voltage, current using Multimeter for each phase. - Calculate the power using power formula. |
| - CU5. Measure consumed energy with Energy meter | - Connect phase line of AC supply to the energy meter. - Connect AC load to the output terminals of energy meter. - Power up the supply and take reading of energy in terms of unit after few minutes form display. |
| - CU6. Measure power factor with voltmeter, ampere meter and watt meter | - Connect voltmeter & ampere meter with inductive load (Motor) - Connect watt meter and measure. - Connect AC supply and take the readings of volt, ampere and watt meter. - Utilize the above readings and calculate power factor using power formula P=VI Cos Ø |
| - CU7. Measure power factor with power factor meter | - Connect current coil of power factor meter in series to the load. - Voltage coil of power factor meter in parallel to the load. - Power the supply and measure the value of power factor form the meter. |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Define AC/D.C power.

- Explain how to measure power of the circuit with meter.

- Differentiate between electrical and mechanical power.

- Define three phase system

- Define wattmeter

- Define relationship between the individual wattmeter readings and the total three phase power

- Use of wattmeter to measure three phase load.

- Explain advantages of three wattmeter method.

- Explain measurement of power in delta and star connection

- Define power factor

- Effect of improved power factor on load Current

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Adjust Measure single phase power using Volt-Ampere meter method, watt meter & Measure consumed energy with Energy meter.

- Perform electrical connection verification as per observation.

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Voltmeter. |
|  | Ammeter |
|  | Wattmeter. |
|  | Connecting Leads. |
|  | Power Supply. |
|  | Screw Driver & Plier. |
|  | Single Phase Load or 100 watt lamp. |
|  | Single phase Energy Meter with connecting leads. |
|  | Ammeter & Voltmeter |
|  | Capacitor bank |
|  | Single phase power factor meter |
|  | Connecting leads as per required. |
|  | Three phase supply source. |
|  | Safety switch |
|  | Phase sequence meter |
|  | Change over switch |
|  | Three phase supply |

# Implement Electromagnet to see Various Effects & Verify Faraday’s Laws.

Overview: This competency standard covers the skills and knowledge required to Implement Electromagnet, Implement circuit to determine the effect on current carrying conductor in magnetic field, Determine the effect on conductor by varying the current with the help of rheostat, plot magnetic lines of forces of bar magnet, Verify Faradays law by moving magnet in side coil, Verify Faradays law by moving coil near the magnet field, Verify EMF through induction.

| Competency Unit | Performance Criteria |
| --- | --- |
| - CU1. Demonstrate Electromagnetism | - Take iron nail (approximately 3 inches in length) as iron core and make 30 to 40 turns of thin coated copper wire to form a coil. - Connect dry cell battery with coil wound on the iron nail. - Bring iron nail near the iron pieces and demonstrate the observation. |
| - CU2. Implement circuit to determine the effect on current carrying conductor in magnetic field. | - Take copper rod (5 cm in length) and connect wires across it. - Give DC supply to copper rod through rheostat. - Place current carrying copper rod inside the horse shoe magnet. - De assembles the Simple DC motor and connects the power leads directly to the electrodes of the armature and observes the rotation. |
| - CU3. Determine the effect on conductor by varying the current with the help of rheostat.single phase AC Power | - Reduce the rheostat resistance - Record the effect on copper rod. - Increase the rheostat resistance - Record the effect on copper rod. |
| - CU4. Plot magnetic lines of forces of bar magnet. | - Place a bar magnet on paper and outline its boundary with the help of lead pencil. - Place a compass needle at one side the magnet. - Mark points on paper where the compass needle stop. - Repeat the same procedure till compass reach at the other end of magnet. - Change the position of compass needle near the magnetic pole and repeat the procedure for P3 to P4. |
| - CU5. Verify Faradays law by moving magnet in side coil. | - Construct a coil with hollow iron cylinder (approximately 3 inches in length 1.5 inch in diameter.) and make 150 to 200 turns on it. - Connect Galvanometer with coil. - Move permanent magnet inside the coil fast and slow and record the effect on reading of Galvanometer. - Hold the magnet inside the coil and do not move, now record the effect on reading of Galvanometer. |
| - CU6. Verify Faradays law by moving coil near the magnet field. | - Construct a coil with hollow iron cylinder (approximately 3 inches in length 1.5 inch in diameter.) and make 150 to 200 turns on it - Connect Galvanometer with coil - Fix permanent magnet and move the coil fast and slow on it and record the effect on reading of Galvanometer - Hold the coil near the magnetic field do not move, now record the effect on reading of Galvanometer |
| - CU7. Verify EMF through induction. | - Take step down transformer and connect its secondary with a Galvanometer and primary winding with a DC battery through a rheostat - Continuously variate the rheostat and observe the reading on the Galvanometer |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Describe magnet and magnetism

- Describe function of iron core

- Explain how to find the movement of current caring conductor which is placed in magnetic field

- Explain Fleming’s left-hand rule

- Define magnetic lines of force.

- Define magnetic field.

- Explain How magnetic lines of force travel with respect to each other

- Explain first law of Michal Faraday

- Explain law of Faraday

- Explain how to find the direction of induced EMF

- Describe role of magnetic strength in Faraday’s Law

- Describe role of conductor’s length or turns in Faraday’s Law

- Define Lenz’s Law

- Explain Faraday’s first law of Electro-Magnetic Induction

Critical Evidence(s) Required

The candidate needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Measure the AC voltage, current, frequency, time period with oscilloscope, RMS value and average value of AC signal with oscilloscope

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Iron nail (as core) |
|  | Thin coated copper wire |
|  | Rheostat |
|  | Current carrying conductor |
|  | Horse shoe magnet. |
|  | Dry cell battery |
|  | Magnet. |
|  | Connecting leads. |
|  | Transformer |
|  | Compass needle |
|  | Coil |
|  | Bar Magnet |
|  | Paper |
|  | Lead pencil. |
|  | Galvanometer |

# Design a Rectifier using Diode

Overview: This competency standard covers the skills and knowledge required to the parameter of Diode and characteristic curve of Diode, designing of Full Wave Rectifier (two diode rectifiers) and designing of Full Wave Rectifier using Diode Bridge. This competency standard will help the candidate in construction of Diode rectifier and its uses in industry.

| Competency Unit | Performance Criteria |
| --- | --- |
| CU1. Identify The parameter of Diode and Draw the characteristic curve of Diode | - Identify the Diodes and their terminal (Anode and Cathode) with the Help of Datasheet - Measure different parameters (Current, Voltage, and power rating) of Diode using multimeter - Implement the Diode in forward and Revers Configuration - Perform the forward and reveres biases operation - Monitor the Output waveform on oscilloscope - Draw the characteristic curves in forward and reverse Biased - Generate the Lab report |
| CU2. Design half wave and Full Wave Rectifier (two diode rectifier | - Select Full Wave and half wave Rectifier components - Draw circuit Diagram of half wave Rectifier - Draw circuit Diagram of Full Wave Rectifier - Observe the Input and Output wave form on oscilloscope and multimeter - Calculate the ripple Factor - Measure output voltage using proper formulas |
| CU3. Design Full Wave Rectifier using Diode Bridge | - Select Full Wave Rectifier components - Draw circuit Diagram of Full Wave Rectifier - Observe the Input and Output wave form on oscilloscope and multimeter - Calculate the ripple Factor - Measure output voltage |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Study the basic of diodes, & their applications in circuits

- Understand Multimeter& power Supply

- Understand the data sheets

- Explain basics of diodes

- Explain the uses of multimeter

- Explain the uses oscilloscope

- Explain the basics of AC & DC voltages

Critical Evidence(s) Required

The student needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Observe the Input and Output wave form of Half wave and Full wave Rectifier on oscilloscope and multimeter

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Oscilloscope |
|  | Diode |
|  | Resistor |
|  | Variable DC power supply |
|  | Soldering iron |
|  | Connecting wire |
|  | Breadboard |
|  | Multimeter |
|  | Wire cutter |
|  | Wire stripper |
|  | Soldering wire |

# Carry out Diode Application

Overview: This competency standard covers the skills and knowledge required to make voltage regulator using Zener diode and make Seven Segment Using Light Emitting Diode. After completing this competency standard, the student will be able to design a voltage regulator circuit using a Zener diode to maintain a constant DC output voltage across the load in spite of variations in the input voltage or changes in the load current.

| Competency Unit | Performance Criteria |
| --- | --- |
| CU1. Make voltage regulator using Zener diode | - Draw the voltage Regulator circuit - Select the Zener diode and components as per requirement for voltage regulator - Placed the components on bread board for voltage regulator circuits. - Measure Input and outputs of the voltage regulator - Verify the required output - Generate the output report |
| CU2. Make Seven Segment Using Light Emitting Diode | - Draw the Seven Segment Display Circuit - Select required components for seven segment display - Placed the components for Seven Segment Display Circuit - Perform basic operations of Seven Segment Display and Verify the required output - Generate the output report |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Learn basic knowledge of Diode & its applications

- Learn to use Multimeter& power Supply

- Understand the data sheets

- Learn adequate knowledge of hand tools

- Learn to Solder the Components

- Learn basic knowledge of LED & its applications

Critical Evidence(s) Required

The student needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Make voltage regulator using Zener diode

- Make Seven Segment Using Light  Emitting Diode

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Oscilloscope |
|  | Zener diode |
|  | Resistor |
|  | Variable DC power supply |
|  | Soldering iron |
|  | Connecting wire |
|  | Breadboard |
|  | Light Emitting diode |
|  | Multimeter |
|  | Seven Segment |

# Implement Bipolar Junction Transistor (BJT) in Different Applications

Overview: This competency standard covers the skills and knowledge required to perform the Biasing of Transistors, Implement Transistor as an amplifier using CB Configuration, Implement Transistor as an amplifier using CC Configuration, Implement Transistor as an amplifier using CE Configuration, Design the circuit of Class A Power Amplifier and Implement BJT as a switch After completion of this competency standard the student will be able to regulate the current or voltage flow and implement a switch for electronic signals.

| Competency Unit | Performance Criteria |
| --- | --- |
| CU1. Perform the Biasing of Transistors | - Identify the Transistor & its types. - Identify the base collector & Emitter of transistors - Perform the standard Biasing of PNP & NPN Transistor |
| CU2. Implement Transistor as an amplifier using CB Configuration. | - Draw the Circuit of CB configuration of transistor - Select the components for CB configurations. - Place the components on breadboard for CB amplifier - Calculate the gain of transistor in CB modes. - Draw VI characteristics curve for CB |
| CU3. Implement Transistor as an amplifier using CC Configuration. | - Draw the Circuit of CC configuration of transistor - Select the components for CC configurations. - Place the components on breadboard for CC amplifier - Calculate the gain of transistor in CC modes. - Draw VI characteristics curve for CC |
| CU4. Implement Transistor as an amplifier using CE Configuration. | - Draw the Circuit of CE configuration of transistor - Select the components for CE configurations. - Place the components on breadboard for CE amplifier - Calculate the gain of transistor in CE modes. - Draw VI characteristics curve for CE |
| CU5. Design the circuit of Class A Power Amplifier | - Identify the Class of a Power Amplifier - Select the component for Class A Power Amplifier - Implement the circuit of PNP OR NPN transistor in Class A Power Amplifier Configuration - Analyse the different parameter of Class A Power Amplifier - Monitor the Output waveform on oscilloscope - Draw the characteristic curves of Class A Power Amplifier - Calculate the Voltage gain and Power Gain of Class A Power Amplifier - Generate the Lab report |
| CU6. Implement BJT as a switch. | - Draw the Circuit of transistor in switching configuration. - Select the components for switching circuits - Place the components on breadboard. - Operate an LED using transistor as a switch - Measure the output and generate the report |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Learn basic concepts of transistor & Biasing

- Study semiconductor theory

- Study the datasheet of transistor

- Learn basics of Coupling Capacitor.

- Study the basic of BJTs, & their applications in circuits

- Learn the V-I Characteristics

- Understand Multimeter, Oscilloscope & power Supply

Critical Evidence(s) Required

The student needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Operate Transistor as an amplifier using CB, CE and CC Configuration.

- Operate Transistor as Switch.

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Oscilloscope |
|  | Transistor |
|  | Resistor |
|  | Variable DC power supply, Multimeter |
|  | Soldering iron |
|  | Connecting wire |
|  | Breadboard |
|  | Light Emitting diode |

# Implement Field Effect Transistor (FET) in Different Applications

Overview: This competency standard covers the skills and knowledge required to perform the Biasing of FET, Implement MOSFET as a switch, Draw the VI characteristics curves for FETs, Design the circuit of Common Drain (CD) Amplifier, Design the circuit of Common Gate (CG) Amplifier, design a switching Circuit Using MOSFET and Design a Low voltage transistor based regulated power supply. After completion of this competency standard the student will be able to implement the FET (Field Effect Transistor) to control the current flow through the device.

| Competency Unit | Performance Criteria |
| --- | --- |
| CU1. Perform the Biasing of FET | - Identify the FET & its types. - Identify the Drain, Gate & Source terminal of FET - Perform the standard Biasing of MOSFET (N-channel, P-channel) - Measure the Gate-Source voltage (Vgs) & Threshold Voltage (Vth) |
| CU2. Implement MOSFET as a switch. | - Draw the Circuit of MOSFET in switching configuration. - Select the components for switching circuits - Place the components on breadboard - Operate an LED using MOSFET as a switch - Measure the output and generate the report |
| CU3. Draw the VI characteristics curves for FETs | - Construct an amplifier circuit using FETs - P2. Apply Vds&Vgs - P2. Measure the drain current - P4. Draw VI characteristic curves |
| CU4. Design the circuit of Common Drain (CD) Amplifier | - Identify the FET and there terminal (gate, drain and Sources) whit the Help of Datasheet - Select the components for Common Drain (CD) amplifier - Implement the circuit of Common Drain (CD) amplifier - Analyse the different parameter of Common Drain (CD) amplifier - Monitor the Output waveform on oscilloscope - Draw the characteristic curves of Common Drain (CD) amplifier - Generate the Lab report |
| CU5. Design the circuit of Common Gate (CG) amplifier | - Identify the FET and there terminal (gate, drain and Sources) whit the Help of Datasheet - Select the components for Common Gate (CG) amplifier - Implement the circuit of Common Gate (CG) amplifier - Analyse the different parameter of Common Gate (CG) amplifier - Monitor the Output waveform on oscilloscope - Draw the characteristic curves of Common Gate (CG) amplifier - Generate the Lab report |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Learn the basics of FET

- Learn the concept of FET Biasing.

- Learn the power rating of FET

- Study the datasheet of FET

- Learn the behavior of current and voltage in FET’s

- Learn the Vgs, VDs, Idss&Rds as per datasheet.

- Understand Multimeter& power Supply

- Understand the data sheets

- Study the basic of FETs, & their applications in circuits

- Understand Multimeter, Oscilloscope & power Supply

Critical Evidence(s) Required

The student needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Operate MOSFET as a switch

- Operate MOSFET as Amplifier

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Oscilloscope |
|  | FET Transistor |
|  | Resistor |
|  | Variable DC power supply |
|  | Soldering iron |
|  | Connecting wire |
|  | Breadboard |
|  | Multimeter |
|  | Wire cutter |
|  | Wire Stripper |
|  | Soldering Wire |

# Implement Thyristor Family in Various Application

Overview: This competency standard covers the skills and knowledge required to Implement the UJT in electronic circuits as switch, Implement the SCR in electronic circuits as switch and Construct the dimmer circuit using Diac&Triac. After the completion of this standard the candidate will be able to use Uni junction Transistor (UJT), Silicon-controlled rectifier (SCR) in power Control Application.

| Competency Unit | Performance Criteria |
| --- | --- |
| CU1. Implement the UJT in electronic circuits as switch | - Identify the UJT terminal - Draw the circuit of switch using UJT. - Select the components for the relaxation oscillator circuits - Construct the relaxation oscillator circuits using UJT - Measure the input and output voltage - Generate the lab report |
| CU2. Implement the SCR in electronic circuits as switch | - Identify the SCR terminals - Draw the circuit of switch using SCR. - Select the components for SCR switching circuits. - Construct the SCR switching circuit. - Apply the trigger Pulse and Check out the desired outputs |
| CU3. Construct the dimmer circuit using Diac &Triac. | - Identify the terminals of Diac&Triac. - Draw the dimmer circuit using Diac&Triac. - Select the components for the dimmer circuit. - Construct the dimmer circuits. - Control the load using dimmer - Generate the lab report |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Learn the basics of UJT

- Understand the data sheets

- Learn adequate knowledge of hand tools

- Learn basics of SCR

- Learn the basics of diac&triac

- Learn the uses of oscilloscope, power Supply &Multimeter

Critical Evidence(s) Required

The student needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Implement the UJT and SCR in electronic circuits as switch

- Make the dimmer circuit using Diac&Triac.

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Oscilloscope |
|  | UJT, |
|  | SCR |
|  | DIAC |
|  | TRIAC |
|  | Variable DC power supply, |
|  | Multimeter |
|  | Soldering iron, wire |
|  | Breadboard or trainer |
|  | Connecting wire |
|  | Wire Stripper |
|  | Wire Cutter |

# Applications of Operation Amplifier

Overview: This competency standard covers the skills and knowledge required to construct a Non-inverting amplifier, an Inverting amplifier, differentiator and comparator circuit using operational amplifier.

| Competency Unit | Performance Criteria |
| --- | --- |
| CU1. Construct a Non-inverting amplifier using operational amplifier | - Identify the Operational amplifier terminals (Inverting, Non-inverting Inputs & Outputs) with the Help of Datasheet - Identify different parameters (Current, Voltage, and power rating) of Op-Amp using datasheet. - Draw the Schematic diagram of non-Inverting Op-Amp. - Select the components for Non-Inverting Op-Amp. - Implement Non-Inverting Op-Amp circuit. - Perform the operations of Non-Inverting Op-Amp circuit. - Measure the output voltage & gain - Generate the Output report |
| CU2. Construct an Inverting amplifier using operational amplifier | - Identify the Operational amplifier terminals (Inverting, Non-inverting Inputs & Outputs) with the Help of Datasheet - Identify different parameters (Current, Voltage, and power rating) of Op-Amp using datasheet. - Draw the Schematic diagram of Inverting Op-Amp. - Select the components for Inverting Op-Amp. - Implement Inverting Op-Amp circuit. - Perform the operations of Inverting Op-Amp circuit. - Measure the output voltage & gain - Generate the Output report |
| CU3. Construct a differentiator circuit using operational amplifier | - Draw the Schematic diagram of differentiator circuit using Op-Amp. - Select the components for differentiator circuit. - Implement differentiator circuit. - Perform the operations of differentiator circuit. - Measure the output waveform - Draw the characteristic curves of differentiator circuit. - Generate the Output report |
| CU4. Construct a integrator circuit using operational amplifier | - Draw the Schematic diagram of integrator circuit using Op-Amp. - Select the components for integrator circuit. - Implement integrator circuit. - Perform the operations of integrator circuit. - Measure the output waveform - Draw the characteristic curves of integrator circuit. - Generate the Output report |
| CU5. Construct a Comparator circuit using operational amplifier | - Draw the Schematic diagram of Comparator circuit using Op-Amp. - Select the components for Comparator circuit. - Implement Comparator circuit. - Perform the operations of Comparator circuit. - Check the output for different input conditions |

Knowledge & Understanding

The candidate must be able to demonstrate underpinning knowledge and understanding required to carry out tasks covered in this competency standard. This includes the knowledge of:

- Study the basics of Op-Amp & their applications in circuits

- Understand Multimeter, Oscilloscope & power Supply & their applications

- Understand the data sheets

Critical Evidence(s) Required

The student needs to produce following critical evidence(s) in order to be competent in this competency standard:

- Perform to use Operational Amplifier IC in different applications

Tools and Equipment

The tools and equipment required for this competency standard are given below:

| S. No. | Items |
| --- | --- |
|  | Functions Generator |
|  | Multimeter |
|  | Digital Oscilloscope |
|  | Capacitors |
|  | Resistors |
|  | Op-Amp |
|  | DC Power supply |
|  | Breadboard |
|  | Connecting leads |