National Vocation Certificate Level-3 in Instrumentation Technology

(Curriculum)

National Vocational and Technical Training Commission (NAVTTC)

Government of Pakistan

Table of Contents

1. 	Introduction	6

1.1.	Purpose of the training program:	6

1.2.	Overall objectives of training program:	7

1.3.	Competencies to be gained after completion of course:	9

1.4.	Entry level of trainees:	15

1.5.	Minimum qualification for teachers:	15

1.6.	Medium of instruction:	15

1.7.	Duration of the course:	16

1.8.	Description and structure of the course	17

Health and Safety	26

Module 0714E&A1: Maintain Occupation Health and Safety.	26

Module 0714E&A2: Adopt Safety Regulation, Labour Protection Laws, Environmental Protection Laws at Workplace.	31

Principles of Electrical & Electronics	36

MODULE 0714E&A3.	VERIFY THE DC FUNDAMENTALS	36

MODULE 0714E&A4.	APPLY THE AC FUNDAMENTALS	41

MODULE 0714E&A5.	INSTALL RELAYS AND SWITCHES IN VARIOUS DOMESTIC AND INDUSTRIAL CIRCUITS	43

MODULE 0714E&A6.	INSTALL DIFFERENT TYPES OF TRANSFORMERS	45

Computer Application	47

MODULE 0714E&A7.	APPLY BASIC COMPUTER SKILLS TO CREATE A VARIETY OF DOCUMENT	47

Basic Electrical Wiring (Single Phase):	52

MODULE 0714E&A8.	MAINTAIN TOOLS & EQUIPMENT.	52

MODULE 0714E&A9.	MAKE CABLE/WIRE JOINTS	54

MODULE 0714E&A10.	PREPARE AND INSTALL DISTRIBUTION BOARDS	58

MODULE 0714E&A11.	CARRYOUT BASIC ELECTRICAL INSTALLATION	61

MODULE 0714E&A12.	INSTALL SIMPLE ELECTRICAL WIRING	64

MODULE 0714E&A13.	PERFORM TESTING OF ELECTRICAL WIRING	69

MODULE 0714E&A14.	REPAIR/ MAINTENANCE OF ELECTRICAL INSTALLATIONS	74

MODULE 0714E&A15.	CARRY OUT ELECTRICAL PLANNING AND ESTIMATION	76

Process Variable Measurements	80

MODULE 0714E&A33.	INSTALL TEMPERATURE & PRESSURE MEASUREMENT INSTRUMENT/DEVICES	80

MODULE 0714E&A34.	INSTALL LEVEL MEASURING INSTRUMENT/DEVICES	83

MODULE 0714E&A35.	INSTALL FLOW LEVEL MEASURING INSTRUMENT/DEVICES	86

Metrology	89

MODULE 0714E&A36.	TAKE MEASUREMENTS WITH GRADUATED TOOLS	89

MODULE 0714E&A37.	TAKE MEASUREMENTS WITH COMBINATION SET	92

MODULE 0714E&A38.	TAKE MEASUREMENTS WITH GAUGES	96

MODULE 0714E&A39.	PERFORM MEASUREMENTS THROUGH MICROMETER	98

MODULE 0714E&A40.	MEASURE DIMENSIONS WITH VERNIER TOOLS	101

MODULE 0714E&A42.	MEASURE THREADS AND GEARS	106

Basic Electrical Engineering:	109

MODULE 0714E&A42.	OPERATE MEASURING INSTRUMENTS.	109

MODULE 0714E&A43.	VERIFY OHM’S LAW & KIRCHHOFF’S LAW BY IMPLEMENTING SERIES/PARALLEL CIRCUITS	111

MODULE 0714E&A44.	MEASURE ELECTRICAL POWER, ENERGY, POWER FACTOR & DETERMINE PHASE SEQUENCE	115

MODULE 0714E&A45.	DEMONSTRATE ELECTROMAGNET TO SEE VARIOUS EFFECTS & VERIFY FARADAYS LAW	118

MODULE 0714E&A46.	VERIFY SERIES AND PARALLEL COMBINATION OF CAPACITORS & DETERMINE BREAK DOWN VOLTAGE OF CAPACITOR	122

Basic Electronics (Analog)	125

MODULE 0714E&A47.	IDENTIFY BASIC ELECTRONICS COMPONENTS	125

MODULE 0714E&A48.	DESIGN RECTIFIER USING DIODE	129

MODULE 0714E&A49.	CARRY OUT DIODE APPLICATION	131

MODULE 0714E&A50.	IMPLEMENT BIPOLAR JUNCTION TRANSISTOR (BJT) IN DIFFERENT APPLICATIONS	133

MODULE 0714E&A51.	IMPLEMENT FIELD EFFECT TRANSISTOR (FET) IN DIFFERENT APPLICATIONS	137

MODULE 0714E&A52.	IMPLEMENT THYRISTOR FAMILY IN VARIOUS APPLICATION	141

MODULE 0714E&A53.	APPLICATION OF OPERATIONAL AMPLIFIER	143

Electrical & Electronic Measuring Instruments	147

MODULE 0714E&A64.	OPERATE MEASURING INSTRUMENTS	147

Instrumentation Workshop Practice	153

MODULE 0714E&A65.	Connect Tubing and Fittings	153

MODULE 0714E&A66.	Perform Basic Machining Operation.	156

Electrical Circuit Analysis	159

MODULE 0714E&A67.	APPLY FUNDAMENTAL DC LAWS IN ELECTRIC CIRCUITS	159

Instrumentation Drawing	163

MODULE 0714E&A69- USE ENGINEERING DRAWING TECHNIQUES TO PRODUCE INSTRUMENTATION DRAWINGS	163

Instrument Servicing & Calibration	172

MODULE 0714E&A70.	CALIBRATE THE MEASURING INSTRUMENTS	172

Sensor & Transducers	181

MODULE 0714E&A72.	SENSOR & TRANSDUCERS	181

Semi-Conductor Devices and Electronic Circuit	186

MODULE 0714E&A73.	CHARACTERIZE BJT AMPLIFIERS.	186

MODULE 0714E&A74.	Characterize Diodes Application	195

MODULE 0714E&A75.	Characterize Various FET Amplifiers.	198

MODULE 0714E&A76.	DESIGN AN OPERATION AMPLIFIER	205

MODULE 0714E&A77.	CHARACTERIZE VARIOUS OSCILLATORS.	208

Analytical Instruments	215

MODULE 0714E&A78.	MEASURE TEMPERATURE & HUMIDITY	215

MODULE 0714E&A79.	MEASURE THE DENSITY & POWER OF HYDROGEN	218

MODULE 0714E&A80.	MEASURE THE CONCENTRATION OF ENVIRONMENTAL GASES	221

MODULE 0714E&A82.	PERFORM THE SPECTRO SCOPIC CHARACTERIZTION	229

Industrial Instrumentation & Control	243

MODULE 0714E&A84.	INSTALL THE TRANSDUCERS & INDUSTRIAL TRANSMITTERS	243

MODULE 0714E&A85.	INSTALL THE VARIABLE FREQUENCY DRIVE	245

Digital Circuits & Microprocessor Applications	246

MODULE 0714E&A88.	Classify the Digital Logic Gates	246

MODULE 0714E&A89.	Design Microprocessor applications	253

MODULE 0714E&A90.	DESIGN MICROCONTROLLER APPLICATIONS	256

Advance Control System:	260

MODULE 0714E&A91.	CONSTRUCT AN ADVANCE AUTOMATION & CONTROL SYSTEM	260

Boiler & Instrumentation:	262

MODULE 0714E&A92.	INSTALL THE BOILER CONTROL SYSTEM	262

MODULE 0714E&A93.	OPERATE THE BOILER SYSTEM	265

4.	List of Tools and Equipment	270

5.	Members of the Curriculum Development Committee	283

# 1. 	Introduction

Instrumentation Technology is specifically designed for students of Electronics/Electrical and touches upon a variety of fundamental topics. This course will cover a major portion of Instrumentation Technology. This training program will also build upon these general concepts to cover object-oriented Instrumentation terminology.

This specialization covers topics ranging from basic Electrical & Electronics Measuring Instrumentation, basic engineering drawing & CAD, Instrumentation drawing, Instrumentation servicing & calibration,  Industrial Instrumentation . You will learn fundamental concepts of Microprocessor applications. We also get practical exposure of Boiler Installation & operation.     By the end of this course, trainee will understand the basics of Instrumentation Technology & able to work in an Industry

The National Skills Strategy (NSS) aims at achieving a paradigm shift from time-bound   and supply led to competency-based and demand driven training in Pakistan. Competency-based training approach focuses on the demonstration of actual skills required in the workplace/industry. To achieve this strategy, NAVTTC in collaboration with GIZ is involved in the development process of CBT program to ensure competent skilled labor in demand driven vocational trades.

National Vocational Qualification Framework (NVQF) as designed in consultation with the stakeholders including academia, researchers, industry, chambers and TEVTAs. Its aim is to identify knowledge & skills required for a qualification, determine equivalence, provide guidelines for Recognition of Prior Learning (RPL) and assure quality of training. NVQF designed for the Vertical and horizontal progression for learners, recognition of qualification, meeting with the national and international standards, facilitate conversion of informal training to formal training through RPL, improvement in quality of training and increased options for learners for selecting training programs in different trades. Competency Standards are performance specification that identify the knowledge and competencies an individual need to succeed in the workplace.

## Purpose of the training program:

The purpose of the training is to provide skilled manpower to improve the existing capacity of Instrumentation sector. This training will provide the requisite skills to the trainees to deal with Instrumentation field. Further, to improve the skill level of the Instrumentation technician and prepare them for the Electrical and Electronics industry to meet the market competition nationally and internationally.

  - Overall objectives of training program:

The Instrumentation qualification level 2-5 consists of theoretical and practical details required to learn operational techniques of Calibration, Operation, and Maintenance of Instruments.

The main objectives of the qualification are as follows:

1.	Health and Safety

2.	Computer Application

3.	Principles of Electrical & Electronics

4.	Soft skills

5.	Electrical & Electronics Measuring Instruments

6.	Basic Engineering drawing and CAD

7.	Instrumentation workshop practice

8.	Process Variable Measurement

9.	Electrical Circuit Analysis

10.	Instrumentation drawing

11.	Semi-Conductor devices and Electronics circuit

12.	Instruments servicing and calibration

13.	Digital Skills

14.	Analytical Instruments

15.	Industrial Instrumentation and control

16.	Digital Circuits and Microprocessor application

17.	Advance control system

18.	Sensor and Transducer

19.	Boiler Instrumentation

20.	Entrepreneurship

## Competencies to be gained after completion of course:

The detail of the competency standards included in this qualification is given below:

National Certificate level 2 in “Instrumentation Technology”

- Maintain Occupational Health & Safety

- Adopt safety Regulations, Labour Protection Laws, Environment

- Develop Professionalism

- Apply the AC Fundamentals

- Verify the DC Fundamentals

- Operate the Measuring Instruments

- Connect Tubing and Fittings

- Perform Basic Machining Operation

- Install Relays & Switches in Domestic and Industrial Applications

- Install different types of Transformers

- Apply Basic Computer Skills to create variety of documents

- Maintain tools & Equipment

- Make Cable / Wire Joints

- Prepare and Install Distribution Boards

- Carryout Basic Electrical Installation

- Install Simple Electrical Wiring

- Repair / Maintenance of Electrical Installations

National Certificate level 3, in “Instrumentation Technology”

- Install Computer Operating Systems and Hardware

- Operate Word-Processing Applications

- Operate Spreadsheet Applications

- Operate Presentation Packages

- Perform Writing And Editing Tasks

- Perform Computer Operations

- Use Computer Applications

- Create User Documentation

- Create Technical Documentation

- Create Basic Databases

- Operate Digital Media Technology

- Use Social Media Tools for Collaboration And Engagement

- E-Commerce

- Use Digital Devices

- Apply basic CAD, Solid works software for designing and drawing

- Install Temperature & Pressure measuring instruments/devices

- Install level measuring instruments/devices

- Install the flow measuring instruments/devices

- Characterize Diodes Application

- Characterize various BJT amplifiers

- Characterize various FET amplifiers

- Characterize various Oscillators

- Design an Operational Amplifier

- Take measurements with graduated tools

- Take measurements with combination set

- Take measurements through various gauges

- Perform measurements through Micrometer

- Measure dimensions with Vernier tools

- Perform different measurements

- Operate Measuring Instruments.

- Verify Ohm’s Law & Kirchhoff’s Law by Implementing Series/Parallel Circuits.

- Measure Electrical Power, Energy, Power Factor & Determine Phase Sequence

- Operate Oscilloscope

- Implement Electromagnet to See Various Effects & Verify Faradays Laws.

- Verify Law of Combination of Capacitor and Determine Break down voltage of Capacitor.

- Basic Electronics (Analog)

- Identify Basic Electronics Components

- Design a Rectifier using Diode

- Carry out Diode Application

- Implement Bipolar Junction Transistor (BJT) in Different Applications

- Implement Field Effect Transistor (FET) in Different Applications

- Implement Thyristor Family in Various Application

- Applications of Operation Amplifier

National Certificate level 4, in “Instrumentation Technology”

- Develop Workplace Policy and Procedures for Sustainability

- Maintain Professionalism in the Workplace

- Manage Personal Work Priorities and Professional Development

- Manage Workforce Planning

- Undertake Project Work

- Prepare and Implement Negotiation

- Manage Meetings

- Organize Schedules

- Identify and Communicate Trends in Career Development

- Apply Specialist Interpersonal and Counseling Interview Skills

- Apply fundamental laws of DC in electrical circuits

- Apply fundamental laws of AC in electrical circuits

- Use engineering drawing techniques to produce Instrumentation Drawings

- Interface the Sensor & Transducers

National Certificate level 5, in “Instrumentation Technology”

- Measure the Temperature & humidity

- Measure the Density and Power of Hydrogen Ion (pH)

- Measure the concentration of environmental gases

- Calibrate the Measuring instruments (Metrological Instruments)

- Analyze the gas chromatography

- Perform the spectroscopic characterization

- Install Transducer & Transmitter in Industrial Application

- Install the process Regulators

- Install the Variable Frequency Drive ( VFD )

- Install Pneumatic & Hydraulic systems in industry

- Installation and setup procedure for Digital Pressure, Temperature, Level and Flow Controllers

- Classify the Digital logic Gates

- Design Microprocessor applications

- Design Microcontroller applications

- Construct an Advance Automation and Control system

- Install the advance control system

- Install the Boiler Control System

- Operate a Boiler Control System

- Investigate Microbusiness Opportunities

- Develop A Micro Business Proposal

- Develop A marketing Plan

- Develop And Review A Business Plan

- Organize Finances For The Micro Business

- Manage Human Resources

- Market Products And Services

- Monitor And Review Business Performance

- Negotiate For Resolving Business Issues

- Manage Personal Finances

- Coordinate A Work Team

- Lead Small Teams

## Entry level of trainees:

The entry for National Vocational Certificate level 2-5, in (Instrumentation Sector) “Electrical & Electronics” are given below:

| Title | Entry requirements |
| --- | --- |
| National Certificate level 2, in “Instrumentation Technology” | Entry for assessment for this qualification is open. However, entry into formal training institutes, based on this qualification may require skills and knowledge equivalent to middle (Grade 8). |
| National Certificate level 3, in “Instrumentation Technology” | Entry for assessment for this qualification is open. However, entry into formal training institute for this qualification is person having National Vocational Certificate level 2, in (Instrumentation) “Electrical & Electronics” |
| National Certificate level 4, in “Instrumentation Technology” | Entry for assessment for this qualification is open. However, entry into formal training institute for this qualification is person having National Vocational Certificate level 3, in (Instrumentation) “Electrical & Electronics” |
| National Certificate level 5, in “Instrumentation Technology” | Entry for assessment for this qualification is open. However, entry into formal training institute for this qualification is person having National Vocational Certificate level 4, in (Instrumentation) “Electrical & Electronics” |

## Minimum qualification for teachers:

    - B.Sc. / BE Engineering in Electronics/Mechatronics/Instrumentation.

    - B.Tech-Hons (4 Years) in Electronics/Mechatronics/ Instrumentationv along with 3 years teaching/industry post qualification experience.

    - Diploma in Electronics/Mechatronics/Instrumentation (Level 5) along with 7 years teaching/industry post qualification experience.

## Medium of instruction:

English/Urdu

## Duration of the course:

The proposed curriculum is composed of 20 modules that will be covered in 4825 learning hours. It is proposed that the course may be delivered in three years period.

The distribution of contact hours is given below:

Total		-	4825 hours.

Theory	-	1343.5 hours (27.84%)

Practical	-	3481.5 hours (72.15%)

## Description and structure of the course

Following is the structure of the course:

| Level 3 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Install Computer Operating Systems And Hardware | Digital Skills | Level 3 | Generic | 3 | 12 | 15 | 1.5 |
| Operate Word-Processing Applications |  | Level 3 | Generic | 3 | 12 | 15 | 1.5 |
| Operate Spreadsheet Applications |  | Level 3 | Generic | 3 | 12 | 15 | 1.5 |
| Operate Presentation Packages |  | Level 3 | Generic | 3 | 12 | 15 | 1.5 |
| Perform Writing And Editing Tasks |  | Level 3 | Generic | 3 | 12 | 15 | 1.5 |
| Perform Computer Operations |  | Level 3 | Generic | 3 | 12 | 15 | 1.5 |
| Use Computer Applications |  | Level 3 | Generic | 3 | 12 | 15 | 1.5 |
| Create User Documentation |  | Level 3 | Generic | 3 | 12 | 15 | 1.5 |
| Create Technical Documentation |  | Level 3 | Generic | 3 | 12 | 15 | 1.5 |
| Create Basic Databases |  | Level 3 | Generic | 3 | 12 | 15 | 1.5 |
| Operate Digital Media Technology |  | Level 3 | Generic | 3 | 12 | 15 | 1.5 |
| Use Social Media Tools For Collaboration And Engagement |  | Level 3 | Generic | 3 | 12 | 15 | 1.5 |
| Use Digital Devices |  | Level 3 | Generic | 3 | 12 | 15 | 1.5 |
| Apply basic CAD, Solid works software for designing and drawing | CAD Draftman | Level 3 | Technical | 2 | 18 | 20 | 2 |
| Install Temperature & Pressure measuring instruments/devices | Instrument Technician | Level 3 | Technical | 5 | 15 | 20 | 2 |
| Install level measuring instruments/devices |  | Level 3 | Technical | 3 | 15 | 18 | 1.8 |
| Install the flow measuring instruments/devices |  | Level 3 | Technical | 3 | 15 | 18 | 1.8 |
| Take measurements with graduated tools | Metrology | Level 3 | Technical | 3 | 12 | 15 | 1.5 |
| Take measurements with combination set |  | Level 3 | Technical | 3 | 12 | 15 | 1.5 |
| Take measurements through various gauges |  | Level 3 | Technical | 3 | 12 | 15 | 1.5 |
| Perform measurements through Micrometer |  | Level 3 | Technical | 3 | 15 | 18 | 1.8 |
| Measure dimensions with Vernier tools |  | Level 3 | Technical | 3 | 15 | 18 | 1.8 |
| Perform different measurements |  | Level 3 | Technical | 3 | 15 | 18 | 1.8 |
| Operate Measuring Instruments. | Basic Electrical Engineering | Level 3 | Technical | 3 | 15 | 18 | 1.8 |
| Verify Ohm’s Law & Kirchhoff’s Law by Implementing Series/Parallel Circuits. |  | Level 3 | Technical | 3 | 15 | 18 | 1.8 |
| Measure Electrical Power, Energy, Power Factor & Determine Phase Sequence |  | Level 3 | Technical | 3 | 15 | 18 | 1.8 |
| Implement Electromagnet to See Various Effects & Verify Faradays Laws. |  | Level 3 | Technical | 3 | 15 | 18 | 1.8 |
| Verify Law of Combination of Capacitor and Determine Break down voltage of Capacitor. |  | Level 3 | Technical | 3 | 15 | 18 | 1.8 |
| Identify Basic Electronics Components | Basic Electronics (Analog) | Level 3 | Technical | 5 | 15 | 20 | 2 |
| Design a Rectifier using Diode |  | Level 3 | Technical | 5 | 15 | 20 | 2 |
| Carry out Diode Application |  | Level 3 | Technical | 5 | 15 | 20 | 2 |
| Implement Bipolar Junction Transistor (BJT) in Different Applications |  | Level 4 | Technical | 5 | 15 | 20 | 2 |
| Implement Field Effect Transistor (FET) in Different Applications |  | Level 4 | Technical | 5 | 15 | 20 | 2 |
| Implement Thyristor Family in Various Application |  | Level 4 | Technical | 5 | 15 | 20 | 2 |
| Applications of Operation Amplifier |  | Level 4 | Technical | 5 | 15 | 20 | 2 |
| Total |  |  |  | 120 | 480 | 600 | 60 |
| Percentage |  |  |  | 20 | 80 |  |  |

## Digital Skills

### Install computer operating systems and hardware

Objective: This module describes the performance outcomes, skills and knowledge required to select, configure and use computer operating systems and basic computer hardware.

Duration: 15 Hours							Theory: 03 Hours				Practice: 12 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Identify operating system and hardware components | Trainee will be able to: - Determine ICT organizational requirements and specifications - Identify and select operating system - Identify appropriate external hardware components - Identify internal hardware components | - Basic knowledge of current industry-accepted operating system, hardware and software products Practical activity - Identify which operating system can be installed on the given hardware system | Theory-01 Hrs Practice-04 Hrs Total- 05 Hrs | Desktop Computer/Laptop | Computer Lab Class room |
| - Install and configure operating system and application software with hardware components | Trainee will be able to: - Install and configure operating system to meet organizational requirements - Identify the functions associated with the operating system and associated boot process - Configure power-management settings to minimize power consumption as an environmentally sustainable measure - Use both the graphical user interface and the command line interface to perform basic tasks - Install or upgrade application software onto the operating system and hardware configuration - Determine the relationship between an application program, the operating system and hardware - Identify general differences between the different computer platforms and their respective operating systems | - Compatibility of an operating system, in respect to other versions Practical activity - Install Microsoft office application in the given system. | Theory-01 Hrs Practice-04 Hrs Total- 05 Hrs | Desktop Computer/Laptop | Computer Lab Class room |
| - Optimize operating system and hardware components | - Optimize operating system using included tools or third-party utilities - Customize the graphical user interface - Use techniques unique to the command line interface - Set up and configure external hardware components and check functionality - Install drivers as appropriate and check functionality | - Function of single-user and multi-user operating systems - Interoperability between operating systems Practical activity Check compatibility of the required drivers, install and check their functionality. | Theory-01 Hrs Practice-04 Hrs Total- 05 Hrs | Desktop Computer/Laptop | Computer Lab Class room |

### Perform word-processing applications

Objective: This unit describes the skills and knowledge required to operate word- processing applications and perform basic operations, including creating and formatting documents, creating tables and printing labels. It applies to individuals in the workplace using fundamental knowledge of word-processing under direct supervision or with limited responsibility.

Duration: 15 Hours							Theory: 03 Hours				Practice: 12 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Apply workplace health and safety (WHS) practices | Trainee will be able to: - Use workplace ergonomic work practices and strategies - Organize work area to ensure an ergonomic work environment | - OHS principles and responsibilities for ergonomics, such as work periods and breaks Practical activity - Identify potential hazards in computer lab and use appropriate actions to mitigate them* | Theory-0.5 Hrs Practice-02 Hrs Total- 2.5 Hrs | Desktop Computer/Laptop | Computer Lab Class room |
| - Create documents | Trainee will be able to: - Open word-processing application, create document and add data according to information requirements - Use document templates as required - Use simple formatting tools when creating the document - Save document to directory | - Create and save new file - Use of appropriate template as required Practical activity - Develop a documents in MS word using format tools as per the requirement of the task. | Theory-0.5 Hrs Practice-02 Hrs Total- 2.5 Hrs | Desktop Computer/Laptop | Computer Lab Class room |
| - Customize basic settings to meet page layout conventions | Trainee will be able to: - Adjust page layout to meet information requirements - Open and view different toolbars - Change font format to suit document purpose - Change alignment and line spacing according to document information requirements - Modify margins to suit the document purpose - Open and switch between several documents | - Understand and edit page layout Practical activity - Open MS word and change select appropriate layout of the page as per the requirement adding header and footer in the document. | Theory-0.5 Hrs Practice-02 Hrs Total- 2.5 Hrs | Desktop Computer/Laptop | Computer Lab Class room |
| - Format documents | Trainee will be able to: - Use formatting features and styles as required - Highlight and copy text from another area in the document or from another active document - Insert headers and footers to incorporate necessary data - Save document in another file format - Save and close document to a storage device | Practical activity - Formatting word document | Theory-0.5 Hrs Practice-02 Hrs Total- 2.5 Hrs | Desktop Computer/Laptop | Computer Lab Class room |
| - Create tables | Trainee will be able to: - Insert standard table into document - Change cells to meet information requirements - Insert and delete columns and rows as necessary - Use formatting tools according to style requirements | Practical activity - Create and edit table | Theory-0.5 Hrs Practice-02 Hrs Total- 2.5 Hrs | Desktop Computer/Laptop | Computer Lab Class room |
| - Add images | Trainee will be able to: - Insert appropriate images into document and customize as necessary - Position and resize images to meet document formatting needs | Practical activity - Insert, position and resize images | Theory-0.5 Hrs Practice-01 Hrs Total- 1.5 Hrs | Desktop Computer/Laptop | Computer Lab Class room |
| - Print documents | Trainee will be able to: - Preview document in print preview mode - Select basic print settings - Print document or part of document from printer | Practical activity - Printing document according to organization need | Theory-00 Hrs Practice-01 Hrs Total- 01 Hrs | Desktop Computer/Laptop | Computer Lab Class room |

### Operate Spreadsheet Application

Objective: This module describes the skills and knowledge required to operate Spreadsheet and perform basic operations, including creating and formatting spreadsheet, creating tables, incorporating chart and object in it, and printing labels.

Duration: 15 Hours							Theory: 03 Hours				Practice: 12 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Create spreadsheets | Trainee will be able to: - Open the spreadsheet application, create spreadsheet files and enter numbers, text and symbols into cells according to information requirements - Enter simple formulas and functions using cell referencing when required - Correct formulas when error messages occur - Use a range of common tools during spreadsheet development - Edit columns and rows within the spreadsheet - Use the auto-fill function to increment data where required - Save the spreadsheet to a folder on a storage device | Practical activity - basic technical terminology related to reading help files and prompts - purpose, use and function of spreadsheet applications - create, edit and save spread sheet | Theory-0.5 Hrs Practice-02 Hrs Total- 2.5 Hrs | Desktop Computer/Laptop | Computer Lab Class room |
| - Customize basic settings | - Adjust page layout to meet user requirements or special needs - Open and view different toolbars - Change font settings so they are appropriate for the document purpose - Change alignment options and line spacing according to spreadsheet formatting features - Format cell to display different styles as required - Modify margin sizes to suit the purpose of the spreadsheets - View multiple spreadsheets concurrently | Practical activity - Formatting spread sheet | Theory-0.5 Hrs Practice-2.5 Hrs Total- 3 Hrs | Desktop Computer/Laptop | Computer Lab Class room |
| - Format spreadsheet | Trainee will be able to: - Use formatting features as required - Copy selected formatting features from another cell in the spreadsheet or from another active spreadsheet - Use formatting tools as required within the spreadsheet - Align information in a selected cell as required - Insert headers and footers using formatting features - Save spreadsheet as another file type - Save to storage device and close spreadsheet | Practical activity - effect of formatting and appearance on the readability and usability of spreadsheets | Theory-0.5 Hrs Practice-2.5 Hrs Total- 3 Hrs | Desktop Computer/Laptop | Computer Lab Class room |
| - Incorporate object and chart in spreadsheet | - Import an object into an active spreadsheet - Manipulate imported object by using formatting features - Create a chart using selected data in the spreadsheet - Display selected data in a different chart - Modify chart using formatting features | Practical activity - Inserting and editing object in spread sheet - Inserting and modifying chart in spreadsheet | Theory-0.5 Hrs Practice-03 Hrs Total- 3.5 Hrs | Desktop Computer/Laptop | Computer Lab Class room |
| - Print spreadsheet | - Preview spreadsheet in print preview mode - Select basic printer options - Print spreadsheet or selected part of spreadsheet - Submit the spreadsheet to appropriate person for approval or feedback | Practical activity - Preview and Print spread sheet | Theory-00 Hrs Practice-03 Hrs Total- 03 Hrs | Desktop Computer/Laptop | Computer Lab Class room |

### Operate Presentation Packages

Objective: This module describes the skills and knowledge required to operate power point processing applications and perform basic operations, including creating and formatting presentations, adding slide show effects and printing presentations and notes. It applies to individuals in the workplace using fundamental knowledge of PowerPoint processing under direct supervision or with limited responsibility.

Duration: 15 Hours							Theory: 03 Hours				Practice: 12 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Create Presentations | Trainee will be able to: - Open presentation package and create a simple design for a presentation according to organizational requirements - Open blank presentation and add text and graphics - Apply existing styles within a presentation - Use presentation template and slides to create a presentation - Use various tools to improve the look of the presentation - Save presentation to the appropriate storage device and folder | Practical activity - Basic technical terminology to read help files and prompts | Theory-0.5 Hrs Practice-02 Hrs Total- 2.5 Hrs | Desktop Computer/Laptop | Computer Lab Class room |
| - Customize basic settings | Trainee will be able to: - Adjust display to meet user requirements - Open and view different toolbars to view options - Ensure font settings are appropriate for the presentation purpose - View multiple slides at once | Practical activity - effect of formatting and appearance on the readability and usability of presentation | Theory-0.5 Hrs Practice-2.5 Hrs Total- 03 Hrs | Desktop Computer/Laptop | Computer Lab Class room |
| - Format presentations | Trainee will be able to: - Use and incorporate organizational charts and bulleted lists, and modify as required - Add objects and manipulate to meet presentation purposes - Import objects and modify for presentation purposes - Modify slide layout, including text and colors, to meet presentation requirements - Use formatting tools as required within the presentation - Duplicate slides within and across a presentation - Reorder sequence of slides and delete slides for presentation purposes - Save presentation in another format - Save to storage device and close presentation | Practical activity - Insert objects and charts | Theory-0.5 Hrs Practice-2.5 Hrs Total- 03 Hrs | Desktop Computer/Laptop | Computer Lab Class room |
| - Add slide show effects | Trainee will be able to: - Incorporate pre-set animation and multimedia effects into presentation as required to enhance the presentation - Add slide transition effects to presentation to ensure smooth progression through the presentation - Test presentation for overall effect - Use onscreen navigation tools to start and stop slide show or move between different slides as required | Practical activity - suitable presentation effects for different audiences | Theory-0.5 Hrs Practice-03 Hrs Total- 3.5 Hrs | Desktop Computer/Laptop | Computer Lab Class room |
| - Print presentation and notes | Trainee will be able to: - Select appropriate print format for presentation - Select preferred slide orientation - Add notes and slide numbers - Preview slides and run spell check before presentation - Print selected slides and submit presentation to appropriate person for feedback | Practical activity - Preview and print presentations | Theory-00 Hrs Practice-03 Hrs Total- 03 Hrs | Desktop Computer/Laptop | Computer Lab Class room |

### Perform writing and editing skills

Objective: This module describes the skills and knowledge required to apply the conventions of plain English to writing and editing tasks of different forms. It also includes editing and proofreading techniques. It applies to individuals in various writing contexts who write and edit texts using appropriate language, style, grammar, spelling, and standard conventions for editing and proofreading.

Duration: 15 Hours							Theory: 03 Hours				Practice: 12 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Apply clear and appropriate language and style to writing and editing tasks | Trainee will be able to: - Use safe work practices including addressing ergonomic requirements when undertaking writing tasks - Use clear, concise and plain English in writing and editing tasks - Apply appropriate paragraph structure to written material to ensure clarity of meaning and ease of reading - Make clear and logical connections between sentences, paragraphs and sections - Determine and incorporate the language and style of the audience | Practical activity - Main features of clear, concise and plain English language for written material | Theory-0.5 Hrs Practice-03 Hrs Total- 3.5 Hrs | Desktop Computer/Laptop | Computer Lab Class room |
| - Apply the appropriate voice, tone and tense | Trainee will be able to: - Determine appropriate voice, tone and tense of the written materials according to audience requirements - Maintain consistent voice, tone and tense throughout written material | Practical activity - Appropriate use of tenses required in written material | Theory-0.5 Hrs Practice-03 Hrs Total- 3.5 Hrs | Desktop Computer/Laptop | Computer Lab Class room |
| - Apply appropriate grammar, spelling and punctuation | Trainee will be able to: - Apply appropriate grammar conventions to a range of written contexts including use of numbers, quotations, and tables - Apply appropriate spelling and punctuation conventions in writing and editing tasks. | Practical activity - Grammar, punctuation and spelling conventions that meet the task requirements | Theory-0.5 Hrs Practice-03 Hrs Total- 3.5 Hrs | Desktop Computer/Laptop | Computer Lab Class room |
| - Perform editing and proofreading tasks to meet requirements | Trainee will be able to: - Edit written material to ensure clear meaning through language and paragraphs, consistent voice, tone and tense - Copyedit written material by checking grammar, spelling and punctuation using standard editing conventions - Proofreading using style guides and by monitoring written material for errors | Practical activity - Editing conventions used in substantive editing and copyediting of written material - Basic software used to write and collect feedback | Theory-0.5 Hrs Practice-03 Hrs Total- 3.5 Hrs | Desktop Computer/Laptop | Computer Lab Class room |

# 0714E&A32- Apply basic CAD software for Designing & Drawing

Objective: This competency standard covers the skills and knowledge required to Take Measurement with Square head, Perform levelling with square head as spirit level, Measure depth with square head as depth gauge, Measure height with square head as height gauge, Perform levelling /measure angle of surfaces with protractor Head, Perform cantering with centre head and Find diameter of round bar.

Duration: 20Hours			Theory: 2 Hours			Practice: 18 Hours			Credit Hours: 2.0

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Identify drawing requirements. | Trainee would be able to - Determine the Requirements and purpose of drawing. - Record the data related to the drawing task. - Verify the data related to the drawing task. | - Explain combination set. - Describe level, head, square, and gauge. - Explain deviation. - Measurement techniques. - Explain procedures of dimensioning of drawings and layouts. - Explain Measurement techniques. Practical Activity: Measure depth and height with square head and draw layout of a component with the help of combination square. | Total 02 Hrs. Theory: 0.5 Hrs. Practical: 1.5 Hrs | - Combination set. - Scriber. - Tri square. - Divider. - Paper - Pencil - Drawing board - Drawing sheets | Class Room and Workshop |
| - Create and Edit engineering drawing manually. | Trainee would be able to - Identify and apply different drafting principles to produce drawing. - Select appropriate drawing method for drafting. - Level the surface by inserting shims. | - Explain combination set. - Describe level, head, square, and gauge. - Explain deviation. - Explain Measurement techniques. - Explain procedures of dimensioning of drawings and layouts. Practical Activity: Correct the level of surfaces of a given work piece by using spirit level of combination sets. | Total 02 Hrs. Theory: 0.5 Hrs. Practical: 1.5 Hrs | - Combination set. - Surface plate. - Scriber. - Tri square. - Divider. - Drawing sheets | Class Room and Workshop |
| - Identify engineering parts and related drawing issues. | Trainee would be able to - Identify and organize list of engineering components. - Draw and record part list according to standard operating procedures. - Verify part list according to standard operating procedures. | - Explain combination set. - Describe level, head, square, and gauge. - Explain deviation. - Explain Measurement techniques. - Explain procedure of dimensioning of drawings and layouts. Practical Activity: Measure depth of a given work piece with square head of combination set. | Total 2 Hrs. Theory: 0.5 Hrs. Practical: 1.5 Hrs | - Combination set. - Surface plate. - Scriber. - Tri square. - Divider. - Round stock. | Class Room and Workshop |
| - Identify elements of the CAD software interface and Plot a basic level drawing. | Trainee would be able to - Create new drawing document using latest CAD software, Identifying file extensions, file locations. - Sketch drawing using drawing tools, edit tools and modify tools. - Draw the sketch using the Command line, Ribbon, Dynamic input and Menu browser. | - Explain combination set. - Describe level, head, square, and gauge. - Explain deviation. - Explain Measurement techniques. - Explain procedure of dimensioning of drawings and layouts. Practical Activity: Measure height of a given work piece by using square head of combination set. | Total 2 Hrs. Theory: 0.5 Hrs. Practical: 1.5 Hrs | - Combination set. - Surface plate. - Scriber. - Tri square. - Divider. - Round stock. | Class Room and Workshop |
| - Create CAD drawing using multilayer and object editing tools | Trainee would be able to - Create and managing multiple layers that define line color, line width, line type etc. - Create drawing using object editing tools (such as fillet, chamfer, break, join, trim, extend, lengthen, and scale, arranging and patterning objects with move, copy, mirror, rotate, align, and array, Obtaining and editing object information through quick properties and Editing object properties. - Create polylines, revision clouds, splines, editing and exploding polylines. | - Explain combination set. - Describe level, head, square, and gauge. - Explain deviation. - Explain Measurement techniques. - Explain procedure of dimensioning of drawings and layouts Practical Activity: Perform leveling of a given work piece and measure angle of surfaces with the help of Protractor head of combination set. | Total 1.5 Hrs. Theory: 00 Hrs. Practical: 1.5 Hrs | - Combination set. - Surface plate. - Scriber. - Tri square. - Divider. - Round stock. | Class Room and Workshop |
| - Create CAD drawing using Text and non-pictorial information . | Trainee would be able to - Differentiate the oblique, axonometric, and isometric drawings. - Create isometric drawings, including isometric ellipses and arcs. - Create section views (e.g., full, offset, half, aligned, revolved, removed, and broken out) and applying the hatch tool to depict material surfaces and solids. - Insert tables and fields into a CAD drawing - Identify different industry standards for dimensioning. | - Explain combination set. - Describe level, head, square, and gauge. - Explain deviation. - Explain Measurement techniques. - Explain procedure of dimensioning of drawings and layouts Practical Activity: Locate centre of a bar stock or a circular job and perform centering of a given work piece by using center Head of combination set. | Total 1.5 Hrs. Theory: 00 Hrs. Practical: 1.5 Hrs | - Combination set. - Surface plate. - Scriber. - Tri square. - Divider. - Round stock. | Class Room and Workshop |
| - Create basic 2D drawings in CAD software. | Trainee would be able to - Draw lines, curves, circles, ellipses, rectangles, polygons, and donuts in CAD software. - Identify the use of various types of Object Snaps and Auto tracking. - Construct drawing points, construction, lines and rays. - Create multi view drawings of an object. - Create partial and full auxiliary view. | - Explain combination set. - Describe level, head, square, and gauge. - Explain deviation. - Explain Measurement techniques. - Explain procedure of dimensioning of drawings and layouts Practical Activity: Determine the diameter of round bar held against the centre Head of combination set. | Total 1.5 Hrs. Theory: 00 Hrs. Practical: 1.5 Hrs | - Combination set. - Surface plate. - Scriber. - Tri square. - Divider. - Round stock. | Class Room and Workshop |
| - Create 3D models of physical parts and assemblies. | - Perform the modeling of the components and useful solid geometry. - Produce assemblies from 3D models of component. - Draw first angle and third angle orthographic projections manually. - Draw isometric projection manually. - Draw isometic projection manually. - Observe different views of 3D object using view ports. - Plot the multi views and shows the hidden features of a 3D object. | - Explain the basic tools bar of software - Define the commands of each tools box - Define the number of views of 2D sketch on - Define Isometric views - Follow the drawings and mark dimensions - What is the solid works Practical Activity: - Make the drawing of sample gear in 2D and 3D using available software. | Total 1.5 Hrs. Theory: 00 Hrs. Practical: 1.5 Hrs | - CAD Software - Computer - Multimedia | Class Room and Workshop |
| - Exporting CAD drawing to different media. | - Identify the difference between model and paper space. - Create multiple layouts for a drawing. - Use floating viewports using the correct paper size, orientation. - Plot drawings to scale, Outputting to different media | - Explain the basic tools bar of software - Define the commands of each tools box - Define the number of views of 2D sketch on - Define Isometric views - Follow the drawings and mark dimensions - What is exploded view in solid works Practical Activity: - Make a 3D CAD model and convert it into step,Iges,stl etc. - Convert Solid Works model file and open it in Autocad. | Total 2. Hrs. Theory: 00 Hrs. Practical: 2. Hrs | - CAD Software - Computer - Multimedia | Class Room and Workshop |
| - Create photo rendered images | - Create photo rendered images of components or assemblies, Save and label images to access for further use. - Interpret source information - Apply material properties using information supplied from source drawings - Create photo rendered images of components or assemblies | - Explain the rules of technical drawing and the prevailing latest ISO standards that govern those rules - Explain the uses of lighting, scenes decals to produce and photo rendered images - Explain the working with text and conveying no pictorial information in CAD software. Practical Activity: Create a drawing of photo rendered images of components or assemblies and save it in the drive. | Total 2. Hrs. Theory: 00 Hrs. Practical: 2. Hrs | - CAD Software - Computer - Multimedia - CAD Software - Computer - Multimedia | Class Room and Workshop |
| - Apply reverse engineering of a physical model. | - Create free hand sketches of industry accepted instruments - Create part and assembly technical drawings in standard format - Insert written information with more than one column using annotation styles that meet ISO standards - Produce print outs of drawings in sizes A0 to A4 to ISO standard. | - Explain the principles of technical drawings. - What are What Are The Geometric Modeling Technique - What Is Applications Cad Software Package - What is the difference between spine and spline? Practical Activity: Create an assembly drawing of an Engine and take print on A2 & A3 and mark annotations as per ISO standards. | Total 2. Hrs. Theory: 00 Hrs. Practical: 2. Hrs | - CAD Software - Computer - Multimedia | Class Room and Workshop |

# Process Variable Measurements

## INSTALL TEMPERATURE & PRESSURE MEASUREMENT INSTRUMENT/DEVICES

Objective: After completing this competency standard, the student will be able to understand the functions, uses, Maintenance & Installation of temperature & Pressure measuring devices.

Duration: 20 Hours				Theory: 05 Hours						Practice: 15 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU-1: Install the Temperature/Pressure measuring instruments. | Trainee will be able to: - Identify the type of Instrument to be installed - Select the appropriate type of instrument and the Scale of measurement - Record the output values | - Explain types & color coding of Thermocouple. - Describe working principle of thermocouple - Explain SOP for the installation of thermocouple. Practical task: Install the Temperature/Pressure measuring instrument as per requirement. | Theory- 1.5Hrs Practical- 4Hrs Total- 5.5Hrs | - Different types of thermocouples - Heat sensing devices - Temperature Gun - RTD | Class Room & Instrumentation Lab |

| LU-2: Measure the Temperature using Thermocouple & Resistance Thermocouple | Trainee will be able to: - Identify the type of thermocouple to be installed - Install the required thermocouple as per installation manual - Analyze accuracy, power and flexibility of thermocouples - Plot the temperature graph as required. | - Explain types & color coding of Thermocouple. - Describe working principle of thermocouple Practical task: - Select the Thermocouple to be installed as per drawing. Confirm the specifications as per requirement. - Install the Thermocouple in the system and check the output as per standard. | Theory- 1.5Hrs Practical- 4Hrs Total- 5.5Hrs | - J-type thermocouple - K-type thermocouple - Multi meter | Class Room & Instrumentation Lab |
| --- | --- | --- | --- | --- | --- |
| LU-3: Install Pressure Transducer & Transmitters. | Trainee will be able to: - Identify the type of Pressure measuring instrument - Install the Pressure transducer as per the installation guidelines - Install the pressure Transmitter as per Installation guidelines - Test & Calibrate the installed pressure measuring instrument | - Explain types and working principle of transducer - Explain types and working principle of transmitter Practical task: - Select the Transmitter to be installed as per drawing. Confirm the specifications as per requirement. - Install the Transmitter in the system and check the output as per standard. | Theory- 1Hrs Practical- 04Hrs Total- 05Hrs | - Pressure gauge - Pressure monitor - Pressure transducer - Pressure Transmitter - Multimeter | Class Room & Instrumentation Lab |
| LU-4: Install Pressure Switch | Trainee will be able to: - Identify the type of pressure switch to be installed - Install the pressure switch as per instructions manual - Test and Calibrate the pressure switch. | - Explain types and working principle of pressure switch - Describe application of pressure switch. Practical task: - Select the pressure switch to be installed as per drawing. Confirm the specifications as per requirement. - Install the pressure switch in the system and check the output as per standard. | Theory- 1Hrs Practical- 3Hrs Total- 04Hrs | - Pressure switch - Toolkit - Pressure gauge - Compressed Air Source | Class Room & Instrumentation Lab |

## INSTALL LEVEL MEASURING INSTRUMENT/DEVICES

Objective: After completion of this competency standard the student will be able to grasp the basic concepts of level measurement, its uses and installation.

Duration: 18 Hours				Theory: 03 Hours						Practice: 15 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU-1: Install the Level Measurement device | Trainee will be able to: - Identify the type of level measurement Instrument - Classify the level measurement sensors - Install the level measuring instruments as per the installations guidelines - Test & Calibrate the level measuring device | - Explain types of Level sensor - Describe working principle of Level sensor Practical task: Install the Level measuring Instrument as per standard and verify its output | Theory- 1Hrs Practical- 3Hrs Total- 4Hrs | - Level Sensor - Multi meter | Class Room & Instrumentation Lab |
| LU-2: Install the optical Instrument for Level Measurement | Trainee will be able to: - Identify the type of optical instrument for level measurement - Install the optical instrument as per installations guidelines - Test and calibrate the optical instrument | - Explain types of Optical Level sensor - Describe working principle of optical Level sensor - Explain alignment of optical sensor Practical task: - Select the type of optical Instrument to be installed as per drawing requirement. - Install the Optical Instrument in the system and check the output as per standard. | Theory- 1Hrs Practical- 3Hrs Total- 4Hrs | - Optical level sensor - Multi meter | Class Room & Instrumentation Lab |
| LU-3: Install the Ultrasonic Level measuring Instruments | Trainee will be able to: - Identify the type of ultrasonic instrument for level measurement - Install the Ultrasonic instrument as per installations guidelines - Test and calibrate the Ultrasonic instrument | - Explain types of Ultrasonic Level sensor - Describe working principle of ultrasonic Level sensor Practical task: - Select the type of Ultrasonic Level meter to be installed as per drawing. - Install the Ultrasonic Level meter in the system and check the output as per standard. | Theory- 0.5Hrs Practical- 3Hrs Total- 3.5Hrs | - Ultrasonic level sensor - Multi meter | Class Room & Instrumentation Lab |
| LU-4: Install the float instruments for Level measurement | Trainee will be able to: - Identify the type of float level measuring device - Install the float level measuring device as per installations manual - Check & Calibrate the float level measuring device | - Explain types of Float Level sensor - Describe working principle of Float Level sensor Practical task - Select the type of Float Level to be installed as per drawing. - Install the Float Level in the system and check the output as per standard. | Theory- 0.5Hrs Practical- 03Hrs Total- 3.5Hrs | - float level sensor - Multi meter | Class Room & Instrumentation Lab |
| LU-5: Install the capacitive Level measuring device | Trainee will be able to: - Identify the type of capacitive level measuring device - Install the capacitive level measuring device as per the installations manual - Check and calibrate the capacitive level measuring device | - Explain types of Capacitive Level sensor - Describe working principle of Capacitive Level sensor Practical task - Select the type of Capacitive Level measuring device to be installed as per drawing. - Install the Capacitive Level measuring device in the system and check the output as per standard | Theory- 00Hrs Practical- 03Hrs Total- 03Hrs | - capacitive level Devices and sensors - Multi meter | Class Room & Instrumentation Lab |

## INSTALL FLOW LEVEL MEASURING INSTRUMENT/DEVICES

Objective: After completion of this competency standard the student will be able to grasp basic concepts, perform the uses and installation of Flow measurement.

Duration: 18 Hours				Theory: 03 Hours						Practice: 15 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU-1: Install the Flow Measuring instrument | Trainee will be able to: - Identify the type of flow measuring device - Install the flow measuring device(Air and Liquid) as per the installations manual - Check and calibrate the flow measuring device | - Explain types of Flow sensor - Describe working principle of Flow sensor Practical task: Install the Flow measuring Instrument as per standard and verify output | Theory- 1Hrs Practical- 3.5Hrs Total- 4.5Hrs | - Flow Sensors - Multi meter - Air Compressor | Class Room & Instrumentation Lab |
| LU-2: Install Differential Pressure Flow Meters | - Identify the type of differential pressure flow meter - Install the differential pressure flow meter as per installations guideline - Check and calibrate the differential pressure flow meter | - Explain types of Differential Pressure flow sensor - Describe working principle of differential Pressure flow sensor Practical task: - Select the type of Differential pressure flow meter to be installed as per drawing. - Install the Differential pressure flow meter in the system and check the output as per standard. | Theory- 1Hrs Practical- 3.5Hrs Total- 4.5Hrs | - Differential Pressure Flow meter - Multimeter - Air compressor | Class Room & Instrumentation Lab |
| LU-3: Implement the Piston & Gear Flow Meters | Trainee will be able to: - Identify the type of Piston flow meter - Install the Piston flow meter as per installations guideline - Check and calibrate the Piston flow meter | - Explain types of piston flow meter - Explain types of Gear flow meter - Describe working principle of Piston & Gear flow meter Practical task: - Select the type of Piston flow meter to be installed as per drawing. - Install the Piston flow meter in the system and check the output as per standard. | Theory- 0.5Hrs Practical- 4Hrs Total- 4.5Hrs | - Piston flow meter - Gear flow meter - 4-20 mA Source | Class Room & Instrumentation Lab |
| LU-4: Install the Sluice Gate & Weir Flow Meters | Trainee will be able to: - Identify the type of Sluice Gate & Weir flow meter - Install the Sluice Gate & Weir flow meter as per installations guideline - Check and calibrate the Sluice Gate & Weir flow meter | - Explain types of sluice gate - Explain types of Weir flow meters - Describe working principle of Sluice gate & Weir flow meter. Practical task: - Select the type of Weir flow meter to be installed as per drawing. - Install the Weir flow meter in the system and check the output as per standard. | Theory- 0.5Hrs Practical- 4Hrs Total- 4.5Hrs | - Sluice gate flow meter - Weir flow meter - Multimeter | Class Room & Instrumentation Lab |

# Metrology

## TAKE MEASUREMENTS WITH GRADUATED TOOLS

Objective: This competency standard covers the skills and knowledge required to take measurements Steel rule, Take measurements Hook rule, Take measurements Folding rule and Take measurements with Trammels.

Duration: 15 Hours			Theory: 3 Hours			Practice: 12 Hours			Credit Hours:

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Take measurements Steel rule. | Trainee would be able to - Place the ruler on various work pieces. - Take the dimension and record the multiple readings on each of work piece. - Take an average of readings. - Record the results. | - Explain basic terms in measurements. - Explain Dimensions and quantities. - Classify linear measurement tools. - Describe types of measurements and its units. - Explain the basic principles of design of linear measuring instruments. - Explain area and volume of geometric shapes. - Describe line, arc and circle measurements. Practical Activity: Measure the linear dimensions of work piece with the help of Steel rule. | Total 4 Hrs. Theory: 1 Hrs. Practical: 3 Hrs | - Steel rule. - Work piece. - Surface plate. - Steps and collars. | Class Room and Workshop |
| - Take measurements Hook rule. | Trainee would be able to - Place hook ruler at the given work piece. - Take the dimension and record multiple readings on each work piece. - Take averages of the recorded measurements or results. - Record the results. | - Explain measurements. - Explain Dimensions and quantities. - Explain linear measurement tools. - Describe types of measurements and its units. - Explain Measurement techniques. - Explain area, volume of geometric shapes. - Describe line, arc and circle measurements. Practical Activity: Measure the linear dimensions of work piece with the help of Hook rule. | Total 4 Hrs.Theory: 1 Hrs. Practical: 3 Hrs | - Hook rule. - Work piece. - Surface plate. - Steps and collars. | Class Room and Workshop |
| - Take measurements Folding rule. | Trainee would be able to - Take the suitable work piece for measurement. - Measure the dimensions of work pieces. - Compute surface area, volume, of given work pieces. - Record the results in data table. | - Explain measurements. - Explain Dimensions and quantities. - Explain linear measurement tools. - Describe types of measurements and its units. - Explain Measurement techniques. - Explain of area, volume of geometric shapes. - Describe line, arc and circle measurements. Practical Activity: Measure the linear dimensions of work piece with the help of Folding rule and calculate and surface area & volume. | Total 4 Hrs.Theory: 1 Hrs. Practical: 3 Hrs | - Work piece. - Surface plate. - Steps and collars. - Folding rule | Class Room and Workshop |

| - Take measurements with Trammels. | Trainee would be able to - Open the trammel according to required dimension. - Draw circle or arc by fixing one point in the dot and rotate. | - Explain measurements. - Explain Dimensions and quantities. - Explain linear measurement tools. - Describe types of measurements and its units. - Explain Measurement techniques. - Explain of area, volume of geometric shapes. - Describe line, arc and circle measurements. Practical Activity: Draw circle & arc by fixing one point in the dot and rotate trammels to bisect lines and angles as per required dimensions on a given work piece. | Total 03 Hrs. Theory: 00 Hrs. Practical: 03 Hrs | - Trammel. - Work piece. - Surface plate. - Steps and collars. - scriber | Class Room and Workshop |
| --- | --- | --- | --- | --- | --- |

## TAKE MEASUREMENTS WITH COMBINATION SET

Objective: This competency standard covers the skills and knowledge required to Take Measurement with Square head, Perform levelling with square head as spirit level, Measure depth with square head as depth gauge, Measure height with square head as height gauge, Perform levelling /measure angle of surfaces with protractor Head, Perform cantering with centre head and Find diameter of round bar.

Duration: 15 Hours			Theory: 3 Hours			Practice: 12 Hours			Credit Hours:

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Take Measurement with Square head. | Trainee would be able to - Place the combination set on given drawing sheet. - Draw layout of a component with the help of combination square. | - Explain combination set. - Describe level, head, square, and gauge. - Explain deviation. - Measurement techniques. - Explain procedures of dimensioning of drawings and layouts. - Explain Measurement techniques. Practical Activity: Measure depth and height with square head and draw layout of a component with the help of combination square. | Total 02 Hrs. Theory: 0.5 Hrs. Practical: 1.5 Hrs | - Combination set. - Scriber. - Tri square. - Divider. - Paper - Pencil - Drawing board | Class Room and Workshop |
| - Perform leveling with square head as spirit level. | Trainee would be able to - Place the spirit level on horizontal work piece. - Check the deviation of bubbles. - Level the surface by inserting shims. - Place square head on vertical work piece. - Note the deviation and correct the level. - Check the other geometrical shapes. | - Explain combination set. - Describe level, head, square, and gauge. - Explain deviation. - Explain Measurement techniques. - Explain procedures of dimensioning of drawings and layouts. Practical Activity: Correct the level of surfaces of a given work piece by using spirit level of combination sets. | Total 02 Hrs. Theory: 0.5 Hrs. Practical: 1.5 Hrs | - Combination set. - Surface plate. - Scriber. - Tri square. - Divider. | Class Room and Workshop |
| - Measure depth with square head as depth gauge. | Trainee would be able to - Place the square head on work piece. - Apply force at the head to retain contact with the component. - Set square head and record the depth. | - Explain combination set. - Describe level, head, square, and gauge. - Explain deviation. - Explain Measurement techniques. - Explain procedure of dimensioning of drawings and layouts. Practical Activity: Measure depth of a given work piece with square head of combination set. | Total 2 Hrs. Theory: 0.5 Hrs. Practical: 1.5 Hrs | - Combination set. - Surface plate. - Scriber. - Tri square. - Divider. - Round stock. | Class Room and Workshop |
| - Measure height with square head as height gauge. | Trainee would be able to - Place the square head on work piece. - Measure height of work piece with and without the help of height gauge attachment. | - Explain combination set. - Describe level, head, square, and gauge. - Explain deviation. - Explain Measurement techniques. - Explain procedure of dimensioning of drawings and layouts. Practical Activity: Measure height of a given work piece by using square head of combination set. | Total 2 Hrs. Theory: 0.5 Hrs. Practical: 1.5 Hrs | - Combination set. - Surface plate. - Scriber. - Tri square. - Divider. - Round stock. | Class Room and Workshop |
| - Perform leveling /measure angle of surfaces with Protractor head. | Trainee would be able to - Place protractor head of combination set on work piece. - Loose screws of revolving circular disk and adjust the bubble of level to minimize deviation. - Study the angle between slopes of a component. | - Explain combination set. - Describe level, head, square, and gauge. - Explain deviation. - Explain Measurement techniques. - Explain procedure of dimensioning of drawings and layouts Practical Activity: Perform leveling of a given work piece and measure angle of surfaces with the help of Protractor head of combination set. | Total 2 Hrs. Theory: 0.5 Hrs. Practical: 1.5 Hrs | - Combination set. - Surface plate. - Scriber. - Tri square. - Divider. - Round stock. | Class Room and Workshop |
| - Perform centering with center head. | Trainee would be able to - Place the center head on round stock. - Trace the line with scriber. - Revolve the center head by 90o and draw another line. - Find center of round stock by bisecting point of lines. | - Explain combination set. - Describe level, head, square, and gauge. - Explain deviation. - Explain Measurement techniques. - Explain procedure of dimensioning of drawings and layouts Practical Activity: Locate centre of a bar stock or a circular job and perform centering of a given work piece by using center Head of combination set. | Total 2 Hrs. Theory: 0.5 Hrs. Practical: 1.5 Hrs | - Combination set. - Surface plate. - Scriber. - Tri square. - Divider. - Round stock. | Class Room and Workshop |
| - Find diameter of round bar. | Trainee would be able to - Place the center head on round stock. - Note the reference point and measure point by avoiding parallax. - Find the difference between reference point and measuring point. | - Explain combination set. - Describe level, head, square, and gauge. - Explain deviation. - Explain Measurement techniques. - Explain procedure of dimensioning of drawings and layouts Practical Activity: Determine the diameter of round bar held against the centre Head of combination set. | Total 03 Hrs.Theory: 00 Hrs.Practical: 3 Hrs 3.5 Hrs | - Combination set. - Surface plate. - Scriber. - Tri square. - Divider. - Round stock. | Class Room and Workshop |

## TAKE MEASUREMENTS WITH GAUGES

Objective:This competency standard covers the skills and knowledge required to take measurement with fixed gauge, plug gauge, adjustable gauge, small hole gauge and telescope gauge.

Duration: 15 Hours			Theory: 3 Hours			Practice: 12 Hours			Credit Hours:

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Take measurement with fixed gauge and plug gauge. | Trainee would be able to - Check / Compare the dimension of provided work piece against the reference standard with the help of fixed gauge. - Insert the plug gauge into the work piece to determine either dimension is of acceptable size or over size. | - Define fixed and plug guage. - Explain procedure of dimensioning and measurement techniques. - Describe standards and references. - Explain inner dia. outer dia. and wall thickness. Practical Activity: Check the dimension of provided work piece against reference standard with the help of fixed gauge. | Total 4 Hrs. Theory: 01 Hrs. Practical: 3 Hrs | - Fix gauge. - Surface plate. - Surface gauge. - Dial indicator. - Plug gauge | Class Room and Workshop |
| - Take measurement with Go Not Go gauge. | Trainee would be able to - Insert both “go” and “not go” gauges in the work piece. - Check the size and record the results. | - Define go/not go gauge. - Explain procedure of dimensioning and measurement techniques. - Explain go and not go gauges. - Describe standards and references. - Explain inner dia. outer dia. and wall thickness. Practical Activity: Check the size of a given work piece by using “go” and “not go” gauges and record the results. | Total 4 Hrs. Theory: 01 Hrs. Practical: 3 Hrs | - Surface plate. - GO NOT GO Gauge. | Class Room and Workshop |
| - Take measurement with small hole gauge. | Trainee would be able to - Insert the small hole gauge of require size into the work piece. Record the result. | - Define small hole gauge - Explain procedure of dimensioning and measurement techniques. - Describe standards and references. - Explain inner dia. outer dia. and wall thickness. Practical Activity: - Determine the dimensions of small holes of a given work piece with the help of with small hole gauge. | Total 3.5 Hrs. Theory: 0.5 Hrs. Practical: 3 Hrs | - Surface plate. - Hole gauge. | Class Room and Workshop |
| - Take measurement with telescope gauge. | Trainee would be able to - Insert the telescope gauge into the work piece. Adjust the gauge and determine the size of work piece. | - Define telescope gauge. - Explain procedure of dimensioning and measurement techniques - Describe standards and references. - Explain inner dia. outer dia. and wall thickness. Practical Activity: - Determine the size of a given work piece by taking measurements with the help of telescope gauge. | Total 3.5 Hrs. Theory: 0.5 Hrs. Practical: 3 Hrs | - Surface plate - Telescope gauge | Class Room and Workshop |

## PERFORM MEASUREMENTS THROUGH MICROMETER

Objective: This competency standard covers the skills and knowledge required to take measurement with outside micro-meter, inside micrometer, depth micrometer, Measure threads with micrometer and Vernier micrometer.

Duration: 18 Hours			Theory: 3 Hours			Practice: 15 Hours			Credit Hours:

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Take measurement with outside micro-meter. | Trainee would be able to - Clean the gauge and surface of work piece. - Open slightly larger than the part to be measured. - Set anvil squarely against reference surface of part. - Using ratchet, slowly until it click once. - Record the reading and calculate the average. | - Define micrometer - Describe how Abbe’s law applies to micrometer measurement. - Explain principle of design of outside micrometer. - Describe types of micrometers. - Classify measurement techniques. - Describe geometric shapes thickness. Practical Activity: Take measurements of a given work piece with outside micrometer. | Total 4 Hrs. Theory: 01 Hrs. Practical: 03 Hrs. | - Outside micrometer. | Class Room and Workshop |
| - Take measurement with inside micrometer. | Trainee would be able to - Clean the gauge and surface of work piece. - Open gauge slightly smaller than the part to be measured. - Set anvil squarely against reference surface of part with ratchet, slowly until it click once. - Record readings and calculate the average. | - Explain principle of design of inside micrometer. - Describe types of Micrometers. - Classify measurement techniques. - Geometric shapes thickness. Practical Activity: Take measurements of a given work piece with inside micrometer. | Total 3.5 Hrs. Theory: 0.5 Hrs. Practical: 3.0Hrs. | - Inside Micrometer. | Class Room and Workshop |
| - Take measurement with depth micrometer. | Trainee would be able to - Clean the gauge and surface of work piece. - Access the depth of groove by steel rule. - Insert the suitable insertion Rod. - Place the depth micrometer gauge on the deeper groove and record the reading. | - Explain principle of design of depth micrometer. - Classify measurement techniques. - Describe geometric shapes thickness. Practical Activity: Access the depth of groove of a given work piece with depth micrometer gauge. | Total 3.5 Hrs. Theory: 0.5 Hrs. Practical: 3.0 Hrs. | - Depth micrometer. | Class Room and Workshop |
| - Measure threads with micrometer. | Trainee would be able to - Clean the gauge and the threads of work piece. - Use the suitable set of thread shape tips according to thread size which is to be measured. - Record the reading. | - Explain design and angles of screw threads. - Explain basic principles of measurement of screw threads. - Describe geometry of screw threads. - Explain measurement principles of major diameter, minor diameter, effective diameter, pitch, angle, and form of screw threads. - Explain tools usage. Practical Activity: Measure the dimensions of threads on a given work piece by using micrometer. | Total 3.5 Hrs. Theory: 0.5 Hrs. Practical: 3.0 Hrs. | - Inside Micrometer. - Outside micrometer. - Depth micrometer. | Class Room and Workshop |
| - Take measurement with Vernier micrometer. | Trainee would be able to - Clean the surface. - Place specimen on the surface plate. - Take reading accurately. - Calculate mean reading. | - Define Vernier micrometer - Explain principle of measurement of a Vernier micrometer - Classify measurement techniques. - Geometric shapes thickness. - Explain tools usage. Practical Activity: Take measurements of a given specimen with Vernier micrometer. | Total 3.5 Hrs. Theory: 0.5 Hrs. Practical: 3.0 Hrs.. | - Vernier micrometer. | Class Room and Workshop |

## MEASURE DIMENSIONS WITH VERNIER TOOLS

Objective: This competency standard covers the skills and knowledge required to take measurement with Vernier calliper, height gauge and Vernier depth gauge.

Duration: 18 Hours			Theory: 3 Hours			Practice: 15 Hours			Credit Hours:

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Take measurement with Vernier caliper | Trainee would be able to - Clean the gauge and surface of work piece. - Place the work piece on surface plate. - Open the lock screws. - Slide the jaws up to the width or size of jaws. - Open slightly larger than the part to be measured. - Set anvil squarely against reference surface of part. - Maintain the proper pressure on the jaws with screw. - Lock the movable jaw with knurled screw. - Measure and record the dimensions. | - Define vernier caliper - Classification of Vernier instruments. - Design of linear measurement instruments. - Explain basic principles of design of Vernier calipers. - Understand dimensioning. Practical Activity: Take measurements on a given work piece with Vernier caliper. | Total 6.0 Hrs. Theory: 1.0 Hrs. Practical: 5.0 Hrs. | - Vernier Caliper. | Class Room and Workshop |
| - Take measurement with height gauge | Trainee would be able to - Clean the gauge and surface of work piece. - Hold the work piece on angle plate. - Open the lock screws. - Set the height gauge on stud to check it with fine adjusting screw. - Lock the measuring head. - Record the reading. | - Classification of Vernier instruments. - Design of linear measurement instruments. - Explain basic principles of design of Vernier height gauge. - Understand dimensioning. Practical Activity: Measure height on a given work piece by using Vernier depth gauge. | Total 6.0 Hrs. Theory: 1.0 Hrs. Practical: 5.0 Hrs. | - Height Vernier. | Class Room and Workshop |
| - Take measurement with Vernier depth gauge | Trainee would be able to - Clean the gauge and surface of work piece. - Hold the work piece on sample plate. - Open the lock screws. - Insert the gauge inside the work piece with fine adjusting screw. - Lock the measuring head. - Record the reading. | - Classification of Vernier instruments. - Design of linear measurement instruments. - Explain basic principles of design of Vernier depth gauge. - Understand dimensioning. Practical Activity: Measure depth on a given work piece by using Vernier depth gauge. | Total 6.0 Hrs. Theory: 1.0 Hrs. Practical: 5.0 Hrs. | - Depth Vernier. | Class Room and Workshop |

## MEASURE ANGLES WITH ANGLE MEASURING INSTRUMENTS.

Objective:This competency standard covers the skills and knowledge required to take measurement with Bevel protractor, Measure angles with Vernier Bevel protractor, Dial protractor, Steel protractor and Sine bar.

Duration: 18 Hours			Theory: 3 Hours			Practice: 15 Hours			Credit Hours:

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Take measurement with Bevel protractor. | Trainee would be able to - Clean the instrument and work piece. - Set the work piece and protractor as per requirement. - Make adjustment of the protractor for accurate measurement. - Record and tabulate the results. | - Define Bevel Protector - Explain the Angles and their supplements. - Describe Angular measurement techniques. - Explain basic requirements of angular measurements. - Classify instruments available for Angular Measurement. - Explain basic principle of a protractor and its extension as the universal bevel protractor. Practical Activity: Measure the angle of a work piece using a universal bevel protractor in clockwise direction, by reading angle directly from the scale | Total 4.0 Hrs. Theory: 01 Hrs. Practical: 03 Hrs. | - Bevel protractor. | Class Room and Workshop |
| - Measure angles with Vernier Bevel protractor. | Trainee would be able to - Set the protractor on work piece properly. - Read the Vernier scale. - Measure and record the angle for clock wise and counter clock wise directions. | - Explain basic principle of vernier bevel protractor. - Explain basic requirements of angular measurements. - Describe use angle gauges to set them accurately to the required angle. Practical Activity: Measure the angle on a given work piece with Vernier Bevel protector. | Total 3.5 Hrs. Theory: 0.5 Hrs. Practical: 03 Hrs. | - Vernier bevel protractor. | Class Room and Workshop |
| - Measure angles with Dial protractor. | Trainee would be able to - Set the protractor on work piece properly. - Read the dial scale. - Measure the angle accurately. | - Explain basic principle of Dial protractor. - Explain Angular measurement techniques. - Define Angles. Practical Activity: Measure the angle on given work piece with Dial protectors. | Total 3.5 Hrs. Theory: 0.5 Hrs. Practical: 3.0 Hrs. | - Dial protractor. | Class Room and Workshop |
| - Measure angles with Steel protractor. | Trainee would be able to - Set the protractor on work piece. - Measure the angle. | - Explain basic principle of steel protractor. - Explain Angular measurement techniques. - Define Angles. Practical Activity: Measure the angle on a given work piece with steel protectors. | Total 3.5 Hrs. Theory: 0.5 Hrs. Practical: 3.0 Hrs. | - Steel protractor. | Class Room and Workshop |
| - Measure angles with Sine bar. | Trainee would be able to - Construct a coil with hollow iron. Select proper size Sine Bar. - Set the Sine Bar with gauge block to measure taper. - Calculate tapper angle accurately. | - Describe measure of angles using the sine principle. - Describe Measuring of Unknown Angles with Sine Bar. - Explain Sine Blocks, Sine Plates, and Sine Tables - Explain the use of sine bar, sine block, sine plate, and sine centre. Practical Activity: What is the maximum recommended angle to which a Sine bar can be set? Measure angle with sine bar and discuss the relationship between the angle being set and the error of measurement for a Sine bar. | Total 3.5 Hrs. Theory: 0.5 Hrs. Practical: 3.0 Hrs | - Sine bar. - Sine block. - Sine plate. - Sine table. - Sine centre. | Class Room and Workshop |

## MEASURE THREADS AND GEARS

Objective: This competency standard covers the skills and knowledge required to Measure threads with thread gauges, Measure gear dimensions with gear tooth calliper and Measure gear dimensions with gear testing machine.

Duration: 18 Hours			Theory: 3 Hours			Practice: 15 Hours			Credit Hours:

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Measure threads with thread gauges. | Trainee would be able to - Place suitable sized wire in thread of work piece. - Take the dimensions. - Note pitch of thread and calculate depth of thread by using formula. | - Define Threads and their types. - Explain basic principles of measurement of screw threads. - Describe geometry of screw threads. - Explain measurement principles of major diameter, minor diameter, effective diameter, pitch, angle, and form of screw threads. - Describe thread-measuring instruments such as bench micrometer, floating carriage micrometer, and pitch-measuring machine. - Discuss types and use of thread gauges for screw thread inspection. - Explain formulas for thread depth Practical Activity: Inspect a metric screw thread by using two-wire method in order to measure its effective diameter, pitch, diameter of the best-size wire and distance over the wires. Determine the effective diameter of the screw thread. | Total 6.0 Hrs. Theory: 01 Hrs. Practical: 05 Hrs. | - Thread gauge. - Plug screw gauges. - Ring screw gauges. - Working gauges. - Inspection gauges. - Master gauges. | Class Room and Workshop |
| - Measure gear dimensions with gear tooth caliper. | Trainee would be able to - Adjust the jaws of calliper into the tooth of gear work piece. - Measure the tooth thickness, cordials addendum and cordials thickness. - Compare the measured values with theoretical results. | - Describe measurement of tooth thickness. - Explain basic principles of measurement of gears. - Describe geometry of spur gears. - Explain techniques used for the measurement of spur gears. - Classify gear-measuring instruments/ testers. - Explain Gears, terminologies related to gears and its types. Practical Activity: Determine chordal height of a gear for which number of teeth, addendum circle diameter, and pressure angle data is given: by using gear tooth caliper during measurement. | Total 6.0 Hrs. Theory: 01 Hrs. Practical: 05 Hrs. | - Gear tooth calliper. | Class Room and Workshop |
| - Measure gear dimensions with gear testing machine. | Trainee would be able to - Identify components of gear inspection machines and draw schematic. - Measure the dimensions of spur, bevel, helical with gear testing machine. | - Describe geometry of gears. - Composite method of gear inspection. - Classify gear-measuring instruments/ testers. - Explain Gears, terminologies related to gears and its types. Practical Activity: Use The Parkinson gear tester (i.e. a composite gear testing machine) to measure the composite errors, such as: errors due to run out, tooth-to-tooth spacing, and profile variations. | Total 6.0 Hrs. Theory: 01 Hrs. Practical: 05 Hrs. | - Gear Inspection machine. - Gear testing machine. - Parkinson Gear Tester. | Class Room and Workshop |

# Basic Electrical Engineering:

## OPERATE MEASURING INSTRUMENTS.

Objective: This competency standard covers the skills and knowledge required to Operate Ampere meter, Volt meter & Ohm meter.

Duration:  18 Hours			Theory: 3 Hours			Practical: 15 Hours			Credit Hours:

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |  |
| --- | --- | --- | --- | --- | --- | --- |
| LU1. Operate Ampere meter. | Trainee would be able to - Identify Ampere meter. - Make a parallel circuit with three different resistors on breadboard. - Adjust proper range of Ampere meter as per load. - Connect Ampere meter in series to each resistor to measure the current. - Turn on the supply and note the reading of current against each resistor. | - Define current and its units. - Define resistance and its units. - Describe series circuit. - Describe parallel circuit. - Enlist the name of various meters. - Describe purpose of Ampere meter. - Explain how Ampere meter is connected in the circuit. - Explain the Meter adjustment as per the desired scale. - How to use breadboard for circuit development? Practical Activity: Measure current across each resistor (connected in series) in an active circuit with the help of Ampere meter. | Total 06 Hrs. Theory: 01 Hrs. Practical: 05 Hrs. | - Multi meters. - Resistors of different values. - Jumper wire. - Breadboard. | Class Room and Workshop |  |
| LU2. Operate Volt meter. | Trainee would be able to: - Identify volt meter. - Make a series circuit with three different resistors on bread board. - Adjust proper range of Volt meter as per load. - Connect volt meter in circuit. - Take the reading. | - Define voltage and its units. - Define resistance and its units. - Describe series circuit. - Describe purpose of Volt meter - Explain the Meter adjustment as per the desired scale. - Explain how voltmeter is connected in the circuit. Practical Activity: Measure voltage across each resistor (connected in series) in an active circuit with the help of Volt meter. | Total 06 Hrs. Theory: 01 Hrs. Practical: 05 Hrs. | - Multi meters. - Resistors of different values. - Jumper wire. - Breadboard. | Class Room and Workshop |  |
| LU3. Operate Ohm meter. | Trainee would be able to: - Identify Ohm meter. - Adjust proper range of Ohm meter as per resistance value. - Connect Ohm meter with resistor. - Take the reading. | - Define resistance and its units. - Define Ohm’s Law. - Define variable resistor. Potentiometer. Rheostat. - Explain the Meter adjustment as per the desired scale. - Describe purpose of Ohm meter. Practical Activity: Measure resistance across different resistors in an active circuit with the help of Ohm meter. | Total 06 Hrs. Theory: 01 Hrs. Practical: 05 Hrs. | - Multi meters. - Resistors of different values. - Jumper wires. - Breadboard. - Variable Power supply |  | Class Room and Workshop |

## VERIFY OHM’S LAW & KIRCHHOFF’S LAW BY IMPLEMENTING SERIES/PARALLEL CIRCUITS

Objective:This competency standard covers the skills and knowledge required to Make series circuit and measure voltage and verify KVL, make parallel circuit and measure current and verify KCL, verify resistance of a resistor, Find unknown value of ohms law.

Duration:  18 Hours			Theory: 3 Hours			Practical: 15 Hours			Credit Hours:

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1.Make series circuit and measure voltage and verify KVL. | Trainee would be able to - Construct series circuit with different resistors on breadboard and give supply voltage. - Measure the voltage across each resistor. - Note the value of voltage against each resistor. - Sum the voltage of each resistor and verify is it equal to applied voltage. | - Describe characteristics of series circuit. - Describe relation between current, voltage and resistance - Explain Kirchhoff’s Voltage law (KVL). - State laws of resistance. Practical Activity: Construct series and parallel circuit of resisters and Practically verify Kirchhoff's Voltage Law (KVL). | Total 2.5 Hrs. Theory: 0.5 Hrs. Practical: 02 Hrs. | - Multi meters. - Resistors of different values. - DC power supply. - Connecting wires. - Rheostat. - Incandescent lamp of different power. | Class Room and Workshop |
| LU2. Make parallel circuit and measure current and verify KCL. | Trainee would be able to - Construct parallel circuit with different resistors on bread board. - Measure the total current drawn by circuit. - Measure the value of current through each resistor individually. - Verify the law by putting all measured value into equation. | - Describe characteristics of series circuit. - Describe relation between current, voltage and resistance - Explain Kirchhoff’s current law (KCL). - Define Rheostat. - State laws of resistance Practical Activity: Construct series and parallel circuit of resisters and Practically verify Kirchhoff's Current Law (KCL). | Total 2.5 Hrs. Theory: 0.5 Hrs. Practical: 02 Hrs . | - Multi meters. - Resistors of different values. - DC power supply - Connecting wire. - Rheostat - Incandescent lamp of different power. | Class Room and Workshop |
| LU3. Verify resistance of a resistor. | Trainee would be able to - Select the 4 colour band resistor. - Determine the nominal value of resistor using colour code including tolerances value. - Connect Ohm meter across the resistor and measure the value of resistance. - Compare both reading. | - Describe colour code and its use. - Describe the purpose of Ohm meter. - Describe how current, voltage and resistor interact with one another. - Define Rheostat. - State laws of resistance. Practical Activity: Practically verify Ohms law. | Total 2.5 Hrs. Theory: 0.5 Hrs. Practical: 02 Hrs | - Multi meters. - Resistors of different values. - DC power supply - Connecting wire. - Rheostat - Incandescent lamp of different power. | Class Room and Workshop |
| LU4.Verify Ohm’s Law | Trainee would be able to - Construct the complex circuit using different resistors. - Find I, V with proper meter and determine R using Ohms law. - Find I, R with proper meter and determine V using Ohm law. - Find R, V with proper meter and determine I using Ohm law. | - Explain Current, Voltage and resistance quantities - Explain Ohm’s law. - Describe mathematical expression of ohm’s law. Practical Activity: Calculate unknown values of I,V & R and verify Ohms law. | Total 2.5 Hrs. Theory: 0.5 Hrs. Practical: 02 Hrs | - Multi meters. - Resistors of different values. - DC power supply - Connecting wire. - Rheostat - Incandescent lamp of different power. | Class Room and Workshop |
| LU5.Measure resistance of Rheostat. | Trainee would be able to - Take sliding Rheostat of 17 Ω and Ohmmeter. (0 to 20 or more than 17 Ω) which is available. - Adjust zero with shortening the Ohm meter leads. - Take reading of rheostat when slide is full placed at end, (whole resistance of rheostat) with the Ohm meter like diagram shown above. - Note this result 17 Ω in your note book. - Repeat these reading when sliding exact in half of rheostat, and note this reading. | - Define Rheostat. - Explain procedure for measuring resistance of rheostat Practical Activity: Practically verify Ohms law by measuring resistance of Rheostat . | Total 2.5 Hrs. Theory: 0.5 Hrs. Practical: 02 Hrs | - Multi meters. - Resistors of different values. - DC power supply - Connecting wire. - Rheostat - Incandescent lamp of different power. | Class Room and Workshop |
| LU6.Measure resistance by using multi-meter. | Trainee would be able to - Take multi-meter of 0 to 10 M-Ohms or any range of multi-meter, which is available at your lab store. - Adjust zero with shortening the multimeter leads. - Take the reading of multimeter like shown above, and note it on your note book. | - Describe the use of multi meter. - State laws of resistance. Practical Activity: Measure resistance by using multimeter. | Total 2.5 Hrs. Theory: 0.5 Hrs. Practical: 02 Hrs | - Multi meters. - Resistors of different values. - DC power supply - Connecting wire. - Rheostat - Incandescent lamp of different power. | Class Room and Workshop |
| LU7.Measure resistance of incandescent lamp. | Trainee would be able to - Take 40 W, lamp and multimeter. - Adjust zero with shortening the Ohm meter leads. - Connect the Ohm meter with its terminal and note its resistance. - Repeat this method for 60 & 100 W lamp. | - Describe characteristics of incandescent lamp circuit. - Define the connection procedure for lamp testing. - Discuss the instrument adjustments as per the testing standard. Practical Activity: Measure resistance of incandescent lamp by using Ohm meter. | Total 03 Hrs. Theory: 00 Hrs. Practical: 03 Hrs | - Multi meters. - Resistors of different values. - DC power supply - Connecting wire. - Rheostat - Incandescent lamp of different power. | Class Room and Workshop |

## MEASURE ELECTRICAL POWER, ENERGY, POWER FACTOR & DETERMINE PHASE SEQUENCE

Objective: This competency standard covers the skills and knowledge required to Measure single phase and poly-phase electrical para meters using Volt-Ampere meter, watt meter & Measure consumed energy with Energy meter.

Duration:  18 Hours			Theory: 3 Hours			Practical: 15 Hours			Credit Hours:

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Measure power using Multimeter method | Trainee would be able to - Construct Select the load to calculate the power and give supply. - Measure the voltage and current by using Multimeter. - Find power using power formula. | - Define AC/D.C power. - Explain how to measure power of the circuit with meter. - Differentiate between electrical and mechanical power - Describe the use of Volt meter and Ampere meter. Practical Activity: Measure single phase apparent power using Volt-Ampere meter method. | Total 2.5 Hrs. Theory: 0.5 Hrs. Practical: 2 Hrs. | - Voltmeter. - Ammeter. - Connecting Leads. - Power Supply. - Screw Driver & Pliers. - Single Phase Load or 100 watt lamp. | Class Room and Workshop |
| - Measure power using watt meter | Trainee would be able to - Construct Select a load to calculate the power and connect watt meter across the load. - Power-ON the supply and measure the power. | - Define watt meter - Explain construction of watt meter - Describe procedure of measuring power through watt meter Practical Activity: Calculate real power of single phase load with the help of watt meter. | Total 2.5 Hrs. Theory: 0.5 Hrs. Practical: 2.5 Hrs. | - Wattmeter. - Connecting Leads. - Power Supply. - Screw Driver & Pliers. - Single Phase Load or 100 watt lamp. | Class Room and Workshop |
| - Measure single phase AC Power | Trainee would be able to - Select a load to calculate the power and appropriate supply. - Measure the voltage and current using Multimeter. - Calculate the power using power. | - Explain single phase network - Describe calculation of measuring power by power formula - Explain measurement procedure of voltage and current with multimeter Practical Activity: Demonstrate the two wattmeter method of measuring the power in single phase networks. | Total 2.5 Hrs. Theory: 0.5 Hrs. Practical: 2 Hrs. | - Voltmeter - Ampere meter - Multimeter - Combination plier - Phase tester - Cutter plier | Class Room and Workshop |
| - Measure three phase AC Power. | Trainee would be able to - Select a load to calculate the power with appropriate supply. - Measure the voltage, current using Multimeter for each phase. - Calculate the power using power formula. | - Explain three phase network - Describe calculation of measuring power by power formula - Explain measurement procedure of voltage and current with multimeter. Practical Activity: Demonstrate the two wattmeter method of measuring the power in 3-phase networks. | Total 2.5 Hrs. Theory: 0.5 Hrs. Practical: 2 Hrs. | - Wattmeter. - Connecting Leads. - Power Supply. - Screw Driver & Plier. - Three Phase Load (3 phase Motor) | Class Room and Workshop |
| - Measure consumed energy with Energy meter | Trainee would be able to - Connect phase line of AC supply to the energy meter. - Connect AC load to the output terminals of energy meter. - Power up the supply and take reading of energy in terms of unit after few minutes form display. | - Explain energy meter in detail - Explain the procedure for measuring consumed energy with energy meter - Describe formula to calculate energy Practical Activity: Connect energy meter with domestic load of single story building | Total 2.5 Hrs. Theory: 0.5 Hrs. Practical: 2 Hrs. | - Connecting Leads. - Power Supply. - Screw driver & pliers. - Energy meter | Class Room and Workshop |
| - Measure power factor with voltmeter, ampere meter and watt meter | Trainee would be able to - Connect voltmeter & ampere meter with inductive load (Motor) - Connect watt meter and measure. - Connect AC supply and take the readings of volt, ampere and watt meter. - Utilize the above readings and calculate power factor using power formula P=VI Cos Ø | - Explain power factor - Describe formula for calculating power factor - Explain procedure for calculating power factor with voltmeter, Ampere meter and watt meter Practical Activity: Measure power factor by connecting voltmeter, ampere meter and watt meter. | Total 2.5 Hrs. Theory: 0.5 Hrs. Practical: 2 Hrs. | - Power factor meter - Voltmeter - Ampere meter - Watt meter - Load (Lamp) - Connecting leads - Power Source | Class Room and Workshop |
| - Measure power factor with power factor meter. | Trainee would be able to - Connect current coil of power factor meter in series to the load. - Voltage coil of power factor meter in parallel to the load. - Power the supply and measure the value of power factor form the meter. | - Explain the reasons of power factor lagging - Describe procedures to improve power factor of an A.C inductive circuit - Explain the leading power factor - Explain function of power factor meter - Explain procedure for calculating power factor with power factor meter. Practical Activity: Measure power factor with power factor meter.and not the readings. | Total 3Hrs. Theory: 00 Hrs. Practical: 3 Hrs. | - Power factor meter - Voltmeter - Ampere meter - Watt meter - Load (Lamp) - Connecting leads - Power Source | Class Room and Workshop |

## DEMONSTRATE ELECTROMAGNETISM TO SEE VARIOUS EFFECTS & VERIFY FARADAYS LAW

Objective::This competency standard covers the skills and knowledge related to Implement Electromagnet, Implement circuit to determine the effect on current carrying conductor in magnetic field, Determine the effect on conductor by varying the current with the help of rheostat, plot magnetic lines of forces of bar magnet, Verify Faradays law by moving magnet in side coil, Verify Faradays law by moving coil near the magnet field, Verify EMF through induction.

Duration:  18 Hours			Theory: 3 Hours			Practical: 15 Hours			Credit Hours:

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Implement Electromagnet. | Trainee would be able to - Take iron nail (approximately 3 inches in length) as iron core and make 30 to 40 turns of thin coated copper wire to form a coil. - Connect dry cell battery with coil wound on the iron nail. - Bring iron nail near the iron pieces and demonstrate the observation. | - Define magnet and magnetism. - Define electromagnet. - Define function of iron core. - Define magnetic lines of force. - Define magnetic field. Practical Activity: Observe how current can create a magnetic field. | Total 2.5 Hrs. Theory: 0.5 Hrs. Practical: 02 Hrs. | - Iron nail as core. - Thin coated copper wire. - Rheostat. - Current carrying conductor. - Horse shoe magnet. - Dry cell battery. | Class Room and Workshop |
| - Implement circuit to determine the effect on current carrying conductor in magnetic field. | Trainee would be able to - Take copper rod (5cm in length) and connect wires across it. - Give DC supply to copper rod through rheostat. - Place current carrying copper rod inside the horse shoe magnet. - Observe the rod movement. - Change the polarity of supply and observe the rod again. | - Explain Fleming’s left-hand rule. - Define the term induction - Explain Faraday’s law of electro-magnetic induction. Practical Activity: Calculate the magnetic force on a current-carrying conductor. | Total 2.5 Hrs. Theory: 0.5 Hrs. Practical: 2.0 Hrs. | - Iron nail as core - Thin coated copper wire - Rheostat - Current carrying conductor - Horse shoe magnet. - Dry cell battery - Magnet. - Connecting leads. - Transformer - Compass needle - Coil - Bar Magnet - Paper - Lead pencil. - Galvanometer | Class Room and Workshop |
| - Determine the effect on conductor by varying the current with the help of rheostat. | Trainee would be able to - Reduce the rheostat resistance. - Record the effect on copper rod. - Increase the rheostat resistance. - Record the effect on copper rod. | - Define function of iron core. - Describe procedure of making magnetic strength of electromagnet. Practical Activity: Measure the effects of a magnetic field on a current carrying conductor with the help of rheostat. | Total 2.5 Hrs. Theory: 0.5 Hrs. Practical: 2 Hrs. | - Rheostat. - Current carrying conductor. - Power supply | Class Room and Workshop |
| - Plot magnetic lines of forces of bar magnet. | Trainee would be able to - Place a bar magnet on paper and outline its boundary with the help of lead pencil. - Place a compass needle at one side the magnet. - Mark points on paper where the compass needle stops. - Repeat the same procedure till compass reach at the other end of magnet. - Change the position of compass needle near the magnetic pole and repeat the procedure for 3 to 4. | - Explain Fleming’s left-hand rule - Explain the direction of magnetic lines of forces Practical Activity: Observe the magnetic field associated with a bar magnet and draw the magnetic field lines graph | Total 2.5 Hrs. Theory: 0.5 Hrs. Practical: 02 Hrs | - Iron nail as core - Thin coated copper wire - Rheostat - Current carrying conductor - Horse shoe magnet. - Dry cell battery - Magnet. - Connecting leads. - Transformer. - Compass needle. - Coil. - Bar Magnet. - Paper. - Lead pencil | Class Room and Workshop |
| - Verify Faradays law by moving magnet in side coil. | Trainee would be able to - Construct a coil with hollow iron cylinder (approximately 3 inches in length 1.5 inch in diameter.) and make 150 to 200 turns on it. - Connect Galvanometer with coil. - Move permanent magnet inside the coil fast and slow and record the effect on reading of Galvanometer. - Hold the magnet inside the coil and do not move, now record the effect on reading of Galvanometer. | - Explain procedure of connecting galvanometer with the coil - Explain Faraday’s law of electromagnetic induction Practical Activity: Verify Farad’s law with the help of galvanometer. | Total 2.5 Hrs. Theory: 0.5 Hrs. Practical: 2 Hrs. | - Iron nail as core. - Thin coated copper wire. - Rheostat. - Current carrying conductor. - Horse shoe magnet. - Dry cell battery. - Magnet. - Connecting leads. - Transformer. - Compass needle. - Coil. - Bar Magnet. - Paper. - Lead pencil. - Galvanometer. | Class Room and Workshop |
| - Verify Faraday’s law by moving coil near the magnet field. | Trainee would be able to - Construct a coil with hollow iron cylinder (approximately 3 inches in length 1.5 inch in diameter.) and make 150 to 200 turns on it. - Connect Galvanometer with coil. - Fix permanent magnet and move the coil fast and slow on it and record the effect on reading of Galvanometer. - Hold the coil near the magnetic field do not move, now record the effect on reading of Galvanometer. | - Explain Lenz’s Law - Explain procedure of moving coil near static magnetic field Practical Activity: Determine the direction of the current induced in a coil as a result of a change in the magnetic flux inside the coil. | Total 2.5 Hrs. Theory: 0.5 Hrs. Practical: 02 Hrs | - Iron nail as core. - Thin coated copper wire. - Rheostat. - Current carrying conductor. - Horse shoe magnet. - Dry cell battery - Magnet. - Connecting leads. - Transformer. - Compass needle. - Coil. - Bar Magnet. - Paper. - Lead pencil. - Galvanometer. | Class Room and Workshop |
| - Verify EMF through induction. | Trainee would be able to - Take step down transformer and connect its secondary with a Galvanometer and primary winding with a DC battery through a rheostat. - Continuously vary the rheostat and observe the reading on the Galvanometer. | - Define term induction - Explain Lenz’s Law for the direction of induced EMF - Explain reason of high and low EMF induction in coil. Practical Activity: Verify Farady’s law by changing magnetic flux and induced current associated with Faraday’s Law of Induction. | Total 3 Hrs. Theory: 00 Hrs. Practical: 3 Hrs. | - Iron nail as core. - Thin coated copper wire. - Rheostat. - Current carrying conductor. - Horse shoe magnet. - Dry cell battery. - Magnet. - Connecting leads. - Transformer. - Compass needle. - Coil. - Bar Magnet. - Paper. - Lead pencil. - Galvanometer. | Class Room and Workshop |

## VERIFY SERIES AND PARALLEL COMBINATION OF CAPACITORS & DETERMINE BREAK DOWN VOLTAGE OF CAPACITOR

Objective: This competency standard covers the skills and knowledge related to Implement a series circuit of capacitors, implement a parallel circuit of capacitors, and determines the breakdown voltage of low voltage capacitor.

Duration:  18 Hours			Theory: 3 Hours			Practical: 15 Hours			Credit Hours:

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Implement a series circuit of capacitors. | Trainee would be able to - Assemble a series circuit of capacitors with 3 different values. - Connect the battery and apply 10 voltages to the combination of capacitors - Measure the voltage across each capacitor connected in series - Add the voltage of each capacitor and verify is it equal to applied voltage. - Use voltage of each capacitor and its value to find the charge on each capacitor. | - Define the capacitance of capacitors. - Explain the principles of capacitance. - Describe the basic units of capacitance. - Explain different types of capacitors. - Determine total capacitance in series circuits. Practical Activity: Connect up-to 3 capacitors in series and find the value of total capacitance using LCR meter. | Total 06 Hrs. Theory: 01 Hrs. Practical: 05 Hrs. | - Capacitors, 4.7 μF - Capacitors, 10, 100 & 8.2 μF. - Voltmeter. - D.C. power supply unit. - Bridging plugs. - Connecting leads. - Multimeter. (With option of Capacitance measurement). | Class Room and Workshop |
| - Implement a parallel circuit of capacitors. | Trainee would be able to - Assemble a parallel circuit of capacitors with 3 different values. - Connect the battery and apply 10 voltages to the combination of capacitors. - Use applied voltage and value of capacitor to find the charge on each capacitor. - Use total voltage and total capacitance to verify the net charge on the capacitor. | - Define the capacitance of capacitors. - Explain the principles of capacitance. - Describe the basic units of capacitance. - Explain different types of capacitors. - Determine total capacitance in parallel circuits. Practical Activity: Connect up-to 3 capacitors in parallel and find the value of total capacitance using LCR meter. | Total 08 Hrs. Theory: 01 Hrs. Practical: 05 Hrs. | - Capacitors, 4.7 μF - Capacitors, 10, 100 & 8.2 μF. - Voltmeter. - D.C. power supply unit. - Bridging plugs. - Connecting leads. - Multimeter. (With option of Capacitance measurement). | Class Room and Workshop |
| - Determine the breakdown voltage of low voltage capacitor. | Trainee would be able to - Take a capacitor of 6.3 rated volts. - Connect it across a 0-30 volts D.C variable supply - Very slowly increased the voltage of variable supply from zero to rated 6.3v, and then higher than rated, and observed carefully the capacitor and to voltmeter reading. - After gradualincrease at critical position of voltage, the capacitor will be burnt, and smoked. | - Explain breakdown voltage of capacitors? - Explain factor can affect the life of a capacitor? - Define charging of capacitor? - Explain factors affecting capacitance. Practical Activity: Construct a RC circuit to measure breakdown voltage | Total 06 Hrs. Theory: 01 Hrs. Practical: 05 Hrs | - Capacitors, 4.7 μF - Capacitors, 10, 100 & 8.2 μF. - Voltmeter. - Connecting leads. - D.C. power supply unit. - Bridging plugs - Multimeter. (With option of Capacitance measurement). - Variable D.C powers supply 0-30 Voltas. - Capacitor (Sanyo Operated Volts 6.3 V, SE 30) - D.C Analog or Preferably Digital voltmeter Range 0-50V. - L.E.D (3 to 9 V) Optional. | Class Room and Workshop |

# Basic Electronics (Analog)

## IDENTIFY BASIC ELECTRONICS COMPONENTS

Overview: This competency standard covers the skills and knowledge required to Identify Various Diodes, Resistors, Capacitors and Inductor in the circuit. After this competency standard the candidate will be able to identify variety of basic electronic components and their usage in industry.

Duration:  20 Hours			Theory: 5 Hours			Practical: 15 Hours			Credit Hours:

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Identify Resistors in circuit | Trainee would be able to - Identify Resistor & its types - Recognize numeric coding & colour coding of resistor - Design series & Parallel circuit of Resistor - Use formulas for Series & parallel circuit of resistors | - Define Resistor & its types with respect to its rating, tolerance and power. - Explain the colour coding reading technique. - Explain series & Parallel circuit. Practical Activity: Draw resistros in series and parallel network. | Total 04 Hrs. Theory: 1Hrs. Practical: 03 Hrs | - Digital Multimeter - Power supply - Resistor - Bread board - Data sheets | Class Room and Electronics Lab |
| - Identify Capacitor in circuit | Trainee would be able to - Identify Capacitor & its types - Recognize polarity & rating of Capacitor - Design Parallel and series circuit of Capacitor - Use formulas for Series & parallel circuit of Capacitor. | - Define Capacitor & its types with respect to its rating, polarity and value. - Explain the capacitor codes to determine its value. - Explain series & Parallel circuit of capacitor. Practical Activity: Draw series and parallel circuit using different capacitors. | Total 04 Hrs. Theory: 01Hrs. Practical: 03 Hrs | - Digital Multimeter - Power supply - Resistor - Capacitor - Bread board - Data sheets | Class Room and Electronics Lab |
| - Identify Inductor in circuit | Trainee would be able to - Identify an Inductor - Recognize Coding & Rating of Inductor - Use formulas for Series & Parallel circuit of Inductor - Analyze Circuit of Inductor | - Define Inductor & its types with respect to its rating and value. - Explain the Inductor codes to determine its value. - State different applications of inductor. Practical Activity: Draw series and parallel circuit using different inductors. | Total 04 Hrs. Theory: 01Hrs. Practical: 03 Hrs | - Digital Multimeter - Power supply - Inductor - Bread board - Data sheets | Class Room and Electronics Lab |
| - Identify Various Diodes | Trainee would be able to - Identify the types of Diodes - Identify the polarities of diode. Check the voltage across the diode in forward and reverse Biased | - Define diode. - State its various types, polarities approximations. - Explain various diode testing methods. Classify the diodes w.r.t its application. - Define Multimeter and its different types. - Explain the operation of multimeter for diode testing. - Explain the data sheets. - State uses and applicaiton of diodes. Practical Activity: - Test the diode to identify its polarity and operation as per data sheet specification. | Total 04 Hrs. Theory: 1Hrs. Practical: 03 Hrs | - Digital Multimeter - Power supply - Diode - Resistor - Bread board - Data sheets | Class Room and Electronics Lab |
| - Identify transistor and its types | Trainee would be able to - Identify types of transistors - Analyze switching functions of transistor - Analyze transistors circuit | - Define transistor and its types - Describe transistor operation modes. - Describe V-I characteristic curve of transistor. - State the uses and application of transistor Practical Activity: - Perform checking procedure of various transistors. - Test the transistor to verify its polarity and voltage current characteristics. | Total 04 Hrs. Theory: 1Hrs. Practical: 03 Hrs | - Multimeter - Power supply - Resistor - Transistors - Bread board - Data sheets | Class Room and Electronics Lab |

## DESIGN RECTIFIER USING DIODE

Overview: This competency standard covers the skills and knowledge required to the parameter of Diode and characteristic curve of Diode, designing of Full Wave Rectifier (two diode rectifiers) and designing of Full Wave Rectifier using Diode Bridge. This competency standard will help the candidate in construction of Diode rectifier and its uses in industry.

Duration:  20 Hours			Theory: 5 Hours			Practical: 15 Hours			Credit Hours:

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Identify The parameter of Diode and Draw the characteristic curve of Diode | - Trainee would be able to - Identify the Diodes and their terminal (Anode and Cathode) with the help of Datasheet - Measure different parameters (Current, Voltage, and power rating) of Diode using digital multimeter - Implement the Diode in forward and Revers Configuration - Perform the forward and reveres biases operation - Monitor the Output waveform on oscilloscope - Draw the characteristic curves in forward and reverse Biased - Generate the Lab report | - Define the basic of diodes, & their applications in circuits - Define Diode Forward Biase / Reverse Biase operation. - Explain the diode configurations and its operation. - Explain the waveform of diode biasing behavior. Practical Activity: Test the forward bias and reverse bias operation using oscilloscope. | Total 07 Hrs. Theory: 02 Hrs. Practical: 05 Hrs | - Oscilloscope - Diode - Resistor - Variable DC power supply - Connecting wire - Breadboard - Multimeter - Wire cutter - Wire stripper | Class Room and Electronics Lab |
| - Design half wave and Full Wave Rectifier (two diode rectifier | Trainee would be able to - Select Full Wave and half wave Rectifier components - Draw circuit Diagram of half wave Rectifier - Draw circuit Diagram of Full Wave Rectifier - Observe the Input and Output wave form on oscilloscope and multimeter - Calculate the ripple Factor - Measure output voltage using proper formulas | - Define the rectifier. - Explain various circuit components used in half wave and full wave rectifier circuit. - Differentiate half wave and full wave rectifier circuit. Practical Activity: Develop a clipper circuit and observe its output waveform on oscilloscope | Total 6.5 Hrs. Theory: 1.5 Hrs. Practical: 05 Hrs | - Oscilloscope Diode - Resistor - Variable DC power supply - Connecting wire - Breadboard - Multimeter - Wire cutter - Wire stripper | Class Room and Electronics Lab |
| - Design Full Wave Bridge rectifier | Trainee would be able to - Select Full Wave Rectifier components - Draw circuit Diagram of Full Wave Rectifier - Observe the Input and Output wave form on oscilloscope - Calculate the ripple Factor - Measure output voltage | - Explain full wave bridge rectifier circuit - Explain the components of full wave bridge rectifier. - Explain waveform for full wave bridge rectifier Practical Activity: Develop a full wave bridge rectifier using 4 diodes. Display the output on oscilloscope. | Total 6.5 Hrs. Theory: 1.5 Hrs. Practical: 05 Hrs | - Oscilloscope Diode - Resistor - Variable DC power supply - Connecting wire - Breadboard - Multimeter - Wire cutter - Wire stripper | Class Room and Electronics Lab |

## CARRY OUT DIODE APPLICATION

Overview: This competency standard covers the skills and knowledge required to make voltage regulator using Zener diode and make Seven Segment Using Light Emitting Diode. After completing this competency standard, the student will be able to design a voltage regulator circuit using a Zener diode to maintain a constant DC output voltage across the load in spite of variations in the input voltage or changes in the load current.

Duration:  20 Hours			Theory: 5 Hours			Practical: 15 Hours			Credit Hours: 2.0

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Make voltage regulator using Zener diode | Trainee would be able to - Draw the voltage Regulator circuit - Select the Zener diode and components as per requirement for voltage regulator - Placed the components on bread board for voltage regulator circuits. - Measure Input and outputs of the voltage regulator - Verify the required output - Generate the output report | - Define the term regulator. - Explain the characteristics and working of a zener regulator. - Discuss the reverse breakdown voltage. - Discuss the application of zener diodes with examples. - Define the output waveform of the zener voltage regulator. Practical Activity: Develop a circuit using zener diode to regulate the voltage at 6VDC. | Total 12 Hrs. Theory: 03 Hrs. Practical: 09 Hrs | - Oscilloscope - Zener diode - Resistor - Variable DC power supply - Connecting wire - Breadboard - Light Emitting diode - Multimeter | Class Room and Electronics Lab |
| - Make Seven Segment Using Light Emitting Diode | Trainee would be able to - Draw the Seven Segment Display Circuit - Select required components for seven segment display - Placed the components for Seven Segment Display Circuit - Perform basic operations of Seven Segment Display and Verify the required output - Generate the output report | Define LEDs. Explain the working and uses of LED. Describe the 7 segment display circuit. Discuss the driver circuit of seven segment. Discuss the control circuit associated with hardware. Explain uses of 7 segment display circuit Practical Activity: Construct a display board using seven segment, displaying the word “ELECTRONICS” | Total 08 Hrs. Theory: 02 Hrs. Practical: 06 Hrs | - Oscilloscope - Resistor - Variable DC power supply - Soldering iron - Connecting wire - Breadboard - Light Emitting diodes - Digital Multimeter | Class Room and Electronics Lab |

## IMPLEMENT BIPOLAR JUNCTION TRANSISTOR (BJT) IN DIFFERENT APPLICATIONS

Overview: This competency standard covers the skills and knowledge required to perform the Biasing of Transistors, Implement Transistor as an amplifier using CB Configuration, Implement Transistor as an amplifier using CC Configuration, Implement Transistor as an amplifier using CE Configuration, BJT as a switch, Design the circuit of Class A Power Amplifier and Implement After completion of this competency standard the student will be able to regulate the current or voltage flow and implement a switch for electronic signals.

Duration:  20 Hours			Theory: 5 Hours			Practical: 15 Hours			Credit Hours:

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Perform the Biasing of Transistors | Trainee would be able to - Identify the Transistor & its types. - Identify the base collector & Emitter of transistors - Perform the standard Biasing of PNP & NPN Transistor. | - Define the transistor construction, its type & identification. - Introduce various transistor biasing methods. - Describe transistor operation. - Explain the purpose of transistor biasing. - Explain transistor operating regions. Practical Activity: Measure the output current for forward biased and reverse biased regions of the transistors. | Total 3.5 Hrs. Theory: 01 Hrs. Practical: 2.5 Hrs | - Oscilloscope - Transistor - Resistor - Variable DC power supply, Multimeter - Connecting wire - Breadboard - Light Emitting diode | Class Room and Electronics Lab |
| - Implement Transistor as an amplifier using CB Configuration. | Trainee would be able to - Draw the Circuit of CB configuration of transistor - Select the components for CB configurations. - Place the components on breadboard for CB amplifier - Calculate the gain of transistor in CB mode - Draw VI characteristics curve for CB | - Define the term amplifier. - Explain the types of amplifier. - Explain CB (common base) amplifier configuration. - Explain the gain variation in amplifiers. - Discuss the Q point factor in amplifiers. - Explain VI characteristic curve for CB configuration Practical Activity: Develop a common base amplifier to verify its parameters. | Total 3.5 Hrs. Theory: 01 Hrs. Practical: 2.5 Hrs | - Oscilloscope - Transistor - Resistor - Variable DC power supply, Multimeter - Connecting wire - Breadboard - Light Emitting diode | Class Room and Electronics Lab |
| - Implement Transistor as an amplifier using CC Configuration. | Trainee would be able to - Draw the Circuit of CC configuration of transistor - Select the components for CC configurations. - Place the components on breadboard for CC amplifier - Calculate the gain of transistor in CC modes. - Draw VI characteristics curve for CC | - Define the term amplifier. - Explain the types of amplifier. - Explain CC (common collector) amplifier configuration. - Describe the gain variation in amplifiers. - Explain the Q point (quiescent) factor in amplifiers. - Explain VI characteristic curve for CC configuration. Practical Activity: Develop a common collector amplifier to verify its parameters. | Total 3.5 Hrs. Theory: 01 Hrs. Practical: 2.5 Hrs | - Oscilloscope - Transistor - Resistor - Variable DC power supply, - Multimeter - Connecting wire - Breadboard - Light emitting diode | Class Room and Electronics Lab |
| - Implement Transistor as an amplifier using CE Configuration. | Trainee would be able to: - Draw the Circuit of CE configuration of transistor - Select the components for CE configurations. - Place the components on breadboard for CE amplifier - Calculate the gain of transistor in CE modes. - Draw VI characteristics curve for CE | - Explain CE (Common Emitter) amplifier configuration. - Describe the gain variation in amplifiers. - Explain the Q point (quiescent) factor in amplifiers. - Explain VI characteristic curve for CE configuration Practical Activity: Develop a common emitter amplifier to verify its parameters. | Total 3.5 Hrs. Theory: 01 Hrs. Practical: 2.5 Hrs | - Oscilloscope - Transistor - Resistor - Variable DC power supply, - Multimeter - Connecting wire - Breadboard - Light emitting diode | Class Room and Electronics Lab |
| - Implement BJT as a switch. | Trainee would be able to: - Draw the Circuit of transistor in switching configuration. - Select the components for switching circuits - Place the components on breadboard. - Operate an LED using transistor as a switch - Measure the output and generate the report | - Define the switch. - Explain the use of transistor used as switch. - Discuss the gain variation in BJT (Bi-polar junction transistor) amplifier. - Discuss the Q point factor in BJT amplifiers. - Discuss the operating regions (Cutoff, Active and Saturation) of BJT transistor. Practical Activity: Develop a BJT transistor based controlled relay as switch. | Total 03 Hrs. Theory: 0.5 Hrs. Practical: 2.5 Hrs | - Oscilloscope - Transistor - Resistor - Variable DC power supply, - Multimeter - Connecting wire - Breadboard - Light emitting diode | Class Room and Electronics Lab |
| - Design the circuit of Class A Power Amplifier | Trainee would be able to: - Identify the Class of a Power Amplifier - Select the component for Class A Power Amplifier - Implement the circuit of PNP OR NPN transistor in Class A Power Amplifier Configuration - Analyze the different parameter of Class A Power Amplifier - Monitor the Output waveform on oscilloscope - Draw the characteristic curves of Class A Power Amplifier - Calculate the Voltage gain and Power Gain of Class A Power Amplifier - Generate the Lab report | - Define the term Power Amplifier - Explain Class A power amplifier and its configuration in detail. - Describe voltage gain and power gain variation in amplifier. - Describe Characteristic curve of class A Power amplifier. - Explain amplifier stability. Practical Activity: Find the gain of a class A amplifier using small signal AC input using oscilloscope. | Total 03 Hrs. Theory: 0.5 Hrs. Practical: 2.5 Hrs | - Oscilloscope - Transistor - Resistor - Variable DC power supply, - Multimeter - Connecting wire - Breadboard - Light emitting diode | Class Room and Electronics Lab |

## IMPLEMENT FIELD EFFECT TRANSISTOR (FET) IN DIFFERENT APPLICATIONS

Overview: This competency standard covers the skills and knowledge required to perform the Biasing of FET, Implement MOSFET as a switch, Draw the VI characteristics curves for FETs, Design the circuit of Common Drain (CD) Amplifier, Design the circuit of Common Gate (CG) Amplifier. After completion of this competency standard the student will be able to implement the FET (Field Effect Transistor) to control the current flow through the device.

Duration:  20 Hours			Theory: 5 Hours			Practical: 15 Hours			Credit Hours:

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Perform the Biasing of FET | Trainee would be able to: - Identify the FET & its types. - Identify the Drain, Gate & Source terminal of FET - Perform the standard Biasing of MOSFET (N-channel, P-channel) - Measure the Gate-Source voltage (Vgs) & Threshold Voltage | - Define the FET transistor, its type & construction. - Introduce various FET transistor biasing methods. - Describe FET transistor operation. - Explain the purpose of FET transistor biasing. - Define the datasheet reading method for FET. - Describe gate-source voltage - Explain threshold voltage Practical Activity: Design a voltage controlled FET amplifier and observe the I/O waveform using oscilloscope. | Total 04 Hrs. Theory: 01 Hrs. Practical: 03 Hrs | - Oscilloscope - FET Transistor - Resistor - Variable DC power supply - Connecting wire - Breadboard - Multimeter - Wire cutter - Wire Stripper | Class Room and Electronics Lab |
| - Implement MOSFET as a switch. | Trainee would be able to: - Draw the Circuit of MOSFET in switching configuration. - Select the components for switching circuits - Place the components on breadboard - Operate an LED using MOSFET as a switch - Measure the output and generate the report | - Define the term MOSFET. - Define the channels of MOSFET. - Explain the MOSFET operation as a switch. Practical Activity: Design a relay using MOSFET and record its output. | Total 04 Hrs. Theory: 01 Hrs. Practical: 03 Hrs | - Oscilloscope - MOSFET Transistor - Resistor - Variable DC power supply - Connecting wire - Breadboard - Multimeter - Wire cutter - Wire Stripper | Class Room and Electronics Lab |
| - Draw the VI characteristics curves for FETs | Trainee would be able to: - Construct an amplifier circuit using FET - Apply Vds &Vgs - Measure the drain current - Draw VI characteristic curves | - Define the term amplifier - Explain the characteristic curve of MOSFET. - Describe the drain source voltage. - Describe the gate source voltages. - Differentiate between Vds (Drain-Source Voltage) and Vgs (Gate-Source Voltage). Practical Activity: Design a circuit to analyze FET characteristic VI curve using oscilloscope. | Total 04 Hrs. Theory: 01 Hrs. Practical: 03 Hrs | - Oscilloscope - MOSFET Transistor - Resistor - Variable DC power supply - Connecting wire - Breadboard - Multimeter - Wire cutter - Wire Stripper | Class Room and Electronics Lab |
| - Design the circuit of Common Drain (CD) Amplifier | Trainee would be able to: - Identify the FET and there terminal (gate, drain and Sources) whit the Help of Datasheet - Select the components for Common Drain (CD) amplifier - Implement the circuit of Common Drain (CD) amplifier - Analyse the different parameter of Common Drain (CD) amplifier - Monitor the Output waveform on oscilloscope - Draw the characteristic curves of Common Drain (CD) amplifier - Generate the Lab report | - Describe FET transistor operation. - Explain the purpose of FET transistor biasing. - Explain common drain (CD) configuration of FET - Discuss transistor operating regions and configuration. - Explain characteristic curve for CD amplifier Practical Activity: Design a voltage controlled FET amplifier in common gain configuration, and observe the I/O waveform using oscilloscope | Total 04 Hrs. Theory: 01 Hrs. Practical: 03 Hrs | - Oscilloscope - MOSFET Transistor - Resistor - Variable DC power supply - Connecting wire - Breadboard - Multimeter - Wire cutter - Wire Stripper | Class Room and Electronics Lab |
| - Design the circuit of Common Gate (CG) amplifier | Trainee would be able to: - Identify the FET and there terminal (gate, drain and Sources) whit the Help of Datasheet - Select the components for Common Gate (CG) amplifier - Implement the circuit of Common Gate (CG) amplifier - Analyze the different parameter of Common Gate (CG) amplifier - Monitor the Output waveform on oscilloscope - Draw the characteristic curves of Common Gate (CG) amplifier - Generate the Lab report | - Describe FET transistor operation. - Explain the purpose of FET transistor biasing. - Define the datasheet reading method for FET. - Discuss transistor operating regions and configuration. Practical Activity: Design a voltage controlled FET amplifier and observe the I/O waveform using oscilloscope | Total 04 Hrs. Theory: 01 Hrs. Practical: 03 Hrs | - Oscilloscope - MOSFET Transistor - Resistor - Variable DC power supply - Connecting wire - Breadboard - Multimeter - Wire cutter - Wire Stripper | Class Room and Electronics Lab |

## IMPLEMENT THYRISTOR FAMILY IN VARIOUS APPLICATION

Overview: This competency standard covers the skills and knowledge required to, Implement the SCR in electronic circuits as switch, Implement the UJT in electronic circuits as switch and Construct the dimmer circuit using DIAC & TRIAC. After the completion of this standard the candidate will be able to use Uni junction Transistor (UJT), Silicon-controlled rectifier (SCR) in power Control Application.

Duration:  20 Hours			Theory: 5 Hours			Practical: 15 Hours			Credit Hours:

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Implement the SCR in electronic circuits as switch | Trainee would be able to: - Identify the SCR terminals - Draw the circuit of switch using SCR. - Select the components for SCR switching circuits. - Construct the SCR switching circuit. - Apply the trigger Pulse and Check out the desired outputs | - Define SCR (Silicon Controlled Rectifier). - Explain SCR and its switching operation. - Explain the Thyristor uni-directional operation. - Explain usage of data sheet. - Discuss the 3 phase circuit applications of SCRs. Practical Activity: Design speed control circuit for a motor using SCR. | Total 07 Hrs. Theory: 02 Hrs. Practical: 05 Hrs | - Oscilloscope - UJT, - SCR - Variable DC power supply, - Multimeter - wire - Breadboard or trainer - Connecting wire - Wire Stripper - Wire Cutter | Class Room and Electronics Lab |
| - Implement the UJT in electronic circuits as switch | Trainee would be able to: - Identify the UJT terminal - Draw the circuit of switch using UJT. - Select the components for the relaxation oscillator circuits - Construct the relaxation oscillator circuits using UJT - Measure the input and output voltage - Generate the lab report. | - Define term UJT (Uni-junction Transistor). - Explain the UJT operation and working modes. - Differentiate UJT with SCR. - Explain usage of data sheet. - Explain working operation of relaxation oscillator circuit. Practical Activity: Implement a circuit of UJT as a light (bulb) intensity controller. | Total 07 Hrs. Theory: 02 Hrs. Practical: 05 Hrs | - Oscilloscope - UJT, - SCR - Variable DC power supply, - Multimeter - wire - Breadboard or trainer - Connecting wire - Wire Stripper - Wire Cutter | Class Room and Electronics Lab |
| - Construct the dimmer circuit using Diac &Triac | Trainee would be able to: - Identify the terminals of Diac & Triac. - Draw the dimmer circuit using Diac & Triac. - Select the components for the dimmer circuit. - Construct the dimmer circuits. - Control the load using dimmer - Generate the lab report | - Explain the DIAC (Diode AC) and TRIAC (Triode AC). - Explain data sheet. - Explain uses of DIAC and TRIAC. - Discuss the control circuits applications of DIAC and TRIAC - Explain dimmer circuit. Practical Activity: - Design chopper circuit using TRIAC. - Design furnace heat control circuit using diac. | Total 06 Hrs. Theory: 01 Hrs. Practical: 05 Hrs | - Oscilloscope - DIAC - TRIAC - Variable DC power supply, - Multimeter - wire - Breadboard or trainer - Connecting wire - Wire Stripper - Wire Cutter | Class Room and Electronics Lab |

## APPLICATION OF OPERATIONAL AMPLIFIER

Overview: This competency standard covers the skills and knowledge required to construct a Non-inverting amplifier, an Inverting amplifier, differentiator and comparator circuit using operational amplifier.

Duration:  20 Hours			Theory: 5 Hours			Practical: 15 Hours			Credit Hours:

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Construct a Non-inverting amplifier using operational amplifier | Trainee would be able to: - Identify the Operational amplifier terminals (Inverting, Non-inverting Inputs & Outputs) with the Help of Datasheet - Identify different parameters (Current, Voltage, and power rating) of Op-Amp using datasheet. - Draw the Schematic diagram of non-Inverting Op-Amp. - Select the components for Non-Inverting Op-Amp. - Implement Non-Inverting Op-Amp circuit. - Perform the operations of Non-Inverting Op-Amp circuit. - Measure the output voltage & gain. - Generate the Output report. | - Define term operational amplifier (OP-AMP). - Explain working construction & identification. - Explain pin configuration of OP-AMP. - Explain the mode of operation of Operational amplifier. - Describe Non-inverting amplifier with its applications - Explain data sheet of OP-AMP Practical Activity: Design a circuit using operational amplifier with non-inverting configuration to analyze it’s output signal behavior. Use an oscilloscope to read the waveform. | Total 04 Hrs. Theory: 01 Hrs. Practical: 03 Hrs | - Functions Generator - Multimeter - Digital Oscilloscope - Capacitors - Resistors - Op-Amp - DC Power supply - Breadboard - Connecting leads | Class Room and Electronics Lab |
| - Construct an Inverting amplifier using operational amplifier | Trainee would be able to: - Identify the Operational amplifier terminals (Inverting, Non-inverting Inputs & Outputs) with the Help of Datasheet - Identify different parameters (Current, Voltage, and power rating) of Op-Amp using datasheet. - Draw the Schematic diagram of Inverting Op-Amp. - Select the components for Inverting Op-Amp. - Implement Inverting Op-Amp circuit. - Perform the operations of Inverting Op-Amp circuit. - Measure the output voltage & gain - Generate the Output report. | - Define term operational amplifier (OP-AMP). - Explain working construction & identification. - Explain pin configuration of OP-AMP. - Explain the mode of operation of Operational amplifier. - Describe Inverting amplifier with its applications - Explain data sheet of OP-AMP Practical Activity: Design a circuit using operational amplifier with inverting input to analyze its output signal using an oscilloscope for signal analysis. | Total 04 Hrs. Theory: 01 Hrs. Practical: 03 Hrs | - Functions Generator - Multimeter - Digital Oscilloscope - Capacitors - Resistors - Op-Amp - DC Power supply - Breadboard - Connecting leads | Class Room and Electronics Lab |
| - Construct a differentiator circuit using operational amplifier | Trainee would be able to: - Draw the Schematic diagram of differentiator circuit using Op-Amp. - Select the components for differentiator circuit. - Implement differentiator circuit. - Perform the operations of differentiator circuit. - Measure the output waveform - Draw the characteristic curves of differentiator circuit. - Generate the Output report | - Explain the operation of Op-Amp as differentiator circuit. - Describe differentiator circuit with its applications - Explain data sheet of OP-AMP - Explain characteristic curve for differentiator circuit Practical Activity: Design a differentiator circuit using operational amplifier. Use an oscilloscope for signal analysis. | Total 04 Hrs. Theory: 01 Hrs. Practical: 03 Hrs | - Functions Generator - Multimeter - Digital Oscilloscope - Capacitors - Resistors - Op-Amp - DC Power supply - Breadboard - Connecting leads | Class Room and Electronics Lab |
| - Construct a integrator circuit using operational amplifier | Trainee would be able to: - Draw the Schematic diagram of integrator circuit using Op-Amp. - Select the components for integrator circuit. - Implement integrator circuit. - Perform the operations of integrator circuit. - Measure the output waveform - Draw the characteristic curves of integrator circuit. - Generate the Output report | - Explain the operation of Op-Amp as Integrator circuit. - Describe integrator circuit with its applications - Explain data sheet of OP-AMP - Explain characteristic curve for integrator circuit Practical Activity: Use an oscilloscope for signal analysis, design a voltage integrator using operational amplifier. | Total 04 Hrs. Theory: 01 Hrs. Practical: 03 Hrs | - Functions Generator - Multimeter - Digital Oscilloscope - Capacitors - Resistors - Op-Amp - DC Power supply - Breadboard - Connecting leads | Class Room and Electronics Lab |
| - Construct a Comparator circuit using operational amplifier | Trainee would be able to: - Draw the Schematic diagram of Comparator circuit using Op-Amp. - Select the components for Comparator circuit. - Implement Comparator circuit. - Perform the operations of Comparator circuit. - Check the output for different input conditions. | - Explain the operation of Op-Amp as Comparator circuit. - Describe Comparator circuit with its applications - Explain data sheet of OP-AMP - Explain the implementation procedure of comparator circuit. Practical Activity: Use an oscilloscope for signal analysis, design an voltage integrator using operational amplifier. | Total 04 Hrs. Theory: 01 Hrs. Practical: 03 Hrs | - Functions Generator - Multimeter - Digital Oscilloscope - Capacitors - Resistors - Op-Amp - DC Power supply - Breadboard - Connecting leads | Class Room and Electronics Lab |

# 4.	List of Tools and Equipment

(FOR A CLASS OF 25 STUDENTS)

| Name of Trade |  | Instrumentation Technology |  |
| --- | --- | --- | --- |
| Sr. # |  | Description |  | Quantity |
| 1. |  | Steel-toed footwear, |  | 25 Nos. |
| 2. |  | hard hat, |  | 25 Nos. |
| 3. |  | safety gloves, |  | 25 Sets |
| 4. |  | appropriate safety glasses, |  | 25 Nos. |
| 5. |  | high visibility vest, |  | 25 Nos. |
| 6. |  | hearing protection, |  | 25 Nos. |
| 7. |  | respirators, masks, |  | 25 Nos. |
| 8. |  | De-electric boots and gloves for protection from electrical shock. |  | 25 Nos. |
| 9. |  | First Aid Box (Include all necessary bandages and medicines) |  | 02 Sets |
| 10. |  | fire extinguishers, |  | 04 Nos. |
| 11. |  | fire blankets, |  | 04 Nos. |
| 12. |  | Sand Bucket with stand |  | 02 Sets |
| 13 |  | basic tools, such as grease gun, air pump etc |  | 10 Sets |
| 14 |  | Allen Key |  | 10 Sets |
| 15 |  | Work Bench (able to hold 06 bench wises) |  | 05 Nos. |
| 16 |  | Bench Wise |  | 30 Nos. |
| 17. |  | hammer, |  | 05 each size |
| 18 |  | screwdrivers, |  | 05 each size |
| 19. |  | pliers, |  | 05 each size |
| 20. |  | self-locking pliers, |  | 05 each size |
| 21. |  | adjustable wrench, |  | 05 each size |
| 22. |  | assorted other wrenches, measuring tape(100m) |  | 05 each size |
| 23. |  | Basic supplies, such as grease, oil, window cleaner, rags, ice scraper, whisk broom. |  | 05 each |
| 24. |  | Color-code cards, utility documentation. Logbooks Service Manuals, OHS Regulation, |  | 10 sets |
| 25 |  | Fix Safe guards |  | 04 Nos |
| 26 |  | Moveable Safe guards |  | 04 N0s. |
| 27 |  | Solder |  | 04 N0s. |
| 28 |  | Wire stripper |  | 04 N0s. |
| 29 |  | Nose plier |  | 04 N0s. |
| 30 |  | Solder wire |  | 04 N0s. |
| 31 |  | Insulation remover |  | 04 N0s. |
| 32 |  | Flat File |  | 25 Nos. |
| 33 |  | Half round file |  | 10 Nos. |
| 34 |  | Round File |  | 10 Nos. |
| 35 |  | Mild Steel Sheet (4 x 8 ft) |  | 02 Nos |
| 36 |  | Mild Steel round work piece (Dia 30) |  | 10 ft |
| 37 |  | Mild Steel flat bar (W:50xL:100xT:20mm) |  | 25 Nos. |
| 38 |  | GI pipe 1”, 2” |  | 20 Feet each |
| 39 |  | Lathe machine (5 ft) |  | 02 Nos |
| 40 |  | Revolving center |  | 02 Nos |
| 41 |  | Dead center |  | 02 Nos |
| 42 |  | Draw bar |  | 02 Nos |
| 43 |  | Face plate |  | 02 Nos |
| 44 |  | Knurling Tool |  | 02 Nos |
| 45 |  | HSS Tool bit |  | 10 Nos |
| 46 |  | Pedestal Drill machine |  | 02 Nos |
| 47 |  | Hand Grinder |  | 02 Nos |
| 48 |  | Drill bits |  | 02 Set |
| 49 |  | Drill shank |  | 02 Set |
| 50 |  | Drill chuck |  | 02 Nos |
| 51 |  | Counter sink tool |  | 02 Nos |
| 52 |  | Counter bore tool |  | 02 Nos |
| 53 |  | Machine reamer set |  | 02 Set |
| 54 |  | Hand Reamer set |  | 02 Set |
| 55 |  | Universal Milling machine |  | 01 Nos. |
| 56 |  | Machine wise |  | 01 Nos. |
| 57 |  | Tail stock with dead center |  | 01 Set |
| 58 |  | Indexing head |  | 01 Nos. |
| 59 |  | Indexing plate set |  | 01 Set |
| 60 |  | Gear cutting tool set |  | 01 Set |
| 61 |  | T-slot cutting tool |  | 02 Nos. |
| 62 |  | Woodruff cutter |  | 02 Nos |
| 63 |  | End mill cutter |  | 02 Nos |
| 64 |  | Portable welding machine 3 Phase 300 Amps. |  | 02 Nos, |
| 65 |  | Welding table |  | 03 Nos, |
| 66 |  | Toggle |  | 02 Nos, |
| 67 |  | Leather gloves |  | 01 Nos, |
| 68 |  | Welding apron |  | 05 Nos, |
| 69 |  | Oxygen Cylinder with protector cap |  | 01 Nos, |
| 70 |  | Acetylene Cylinder with protector cap |  | 01 Nos. |
| 71 |  | Welding Torch |  | 01 Nos. |
| 72 |  | Welding Tip (Nozzle) |  | 01 Set |
| 73 |  | Pressure Regulators |  | 01 Nos. |
| 74 |  | Hose & Hose fittings |  | 30 ft. |
| 75 |  | Filler Rod |  | 01 Box |
| 76 |  | Flux Material |  | 500 gm. |
| 77 |  | Surface Table |  | 01 Nos. |
| 78 |  | - Surface plate. |  | 02 Nos. |
| 79 |  | - Steps and collars. |  | 02 Set |
| 80 |  | Folding rule |  | 02 Nos. |
| 81 |  | Combination set. |  | 02 Nos. |
| 82 |  | Scriber. |  | 02 Nos. |
| 83 |  | Tri square. |  | 02 Nos. |
| 84 |  | Divider. |  | 02 Nos. |
| 85 |  | Spirit level |  | 02 Nos. |
| 86 |  | Radius gauge |  | 02 Nos. |
| 87 |  | Filler gauge |  | 02 Nos. |
| 88 |  | Plug gauge |  | 02 Set |
| 89 |  | Ring gauge |  | 02 Set |
| 90 |  | Outside micrometer |  | 02 Nos. |
| 91 |  | Inside micrometer |  | 02 Nos. |
| 92 |  | Depth micrometer. |  | 02 Nos. |
| 93 |  | Bevel protractor. |  | 02 Nos. |
| 94 |  | Vernier Bevel protractor. |  | 02 Nos. |
| 95 |  | Dial protractor. |  | 02 Nos. |
| 96 |  | Sine bar. |  | 02 Nos. |
| 97 |  | Sine block |  | 02 Nos. |
| 98 |  | Sine plate. |  | 02 Nos. |
| 99 |  | Sine table. |  | 02 Nos. |
| 100 |  | Sine center. |  | 02 Nos. |
| 101 |  | Gear tooth caliper |  | 02 Nos. |
| 102 |  | Gear Inspection machine. |  | 01 Nos. |
| 103 |  | Gear testing machine. |  | 01 Nos. |
| 104 |  | Parkinson Gear Tester. |  | 01 Nos. |
| 105 |  | Multi meters |  | 02 Nos. |
| 106 |  | Resistors of different values |  | 04 Nos. (Each) |
| 107 |  | Small pieces of copper wire |  | 01 roll |
| 108 |  | Breadboard. |  | 04 Nos. |
| 109 |  | Connecting wires |  | 02 Rolls |
| 110 |  | Rheostat. |  | 04 Nos. |
| 111 |  | Incandescent lamp of different watts. |  | 04 Nos. |
| 112 |  | Voltmeter |  | 04 Nos. |
| 113 |  | Ampere meter |  | 04 Nos. |
| 114 |  | Multimeter |  | 04 Nos. |
| 115 |  | Combination plier |  | 04 Nos. |
| 116 |  | Phase tester |  | 04 Nos. |
| 117 |  | Energy meter |  | 04 Nos. |
| 118 |  | Thin coated copper wire. |  | 02 Rolls |
| 119 |  | Dry cell battery |  | 04 Nos. |
| 120 |  | Horse shoe magnet. |  | 04 Nos. |
| 121 |  | Transformer. |  | 02 Nos. |
| 122 |  | Compass needle. |  | 02 Nos. |
| 123 |  | Bar Magnet. |  | 04 Nos. |
| 124 |  | Galvanometer. |  | 02 Nos. |
| 125 |  | Capacitors, 4.7 μF |  | 04 Nos. |
| 126 |  | Capacitors, 10, 100 & 8.2 μF. |  | 04 Nos. (each) |
| 127 |  | Voltmeter. |  | 04 Nos. |
| 128 |  | D.C. power supply unit. |  | 02 Nos. |
| 129 |  | Bridging plugs. |  | 04 Nos. |
| 130 |  | Connecting leads. |  | 04 |
| 131 |  | Multimeter. (With option of Capacitance measurement) |  | 04 Nos. |
| 132 |  | D.C. power supply unit |  | 02 Nos. |
| 133 |  | Variable D.C powers supply 0-30 Voltas |  | 02 |
| 134 |  | Capacitor (Sanyo Operated Volts 6.3 V, SE 30) |  | 02 Nos. (each) |
| 135 |  | D.C Analog or Preferably Digital voltmeter Range 0-50V. |  | 02 Nos. (each) |
| 136 |  | L.E.D (3 to 9 V) |  | 02 Nos. (each) |
| 137 |  | Diode |  | 04 Nos. |
| 138 |  | Resistor |  | 04 Nos. |
| 139 |  | Bread board |  | 04 Nos. |
| 140 |  | Digital Multimeter |  | 04 Nos. |
| 141 |  | Oscilloscope Diode |  | 04 Nos. |
| 142 |  | Wire cutter |  | 04 Nos. |
| 143 |  | Wire stripper |  | 04 Nos. |
| 144 |  | Zener diode |  | 04 Nos. |
| 145 |  | Light Emitting diode |  | 04 Nos. |
| 146 |  | Soldering iron |  | 04 Nos. |
| 147 |  | FET Transistor |  | 04 Nos. |
| 148 |  | MOSFET Transistor |  | 04 Nos. |
| 149 |  | UJT |  | 04 Nos. |
| 150 |  | SCR |  | 04 Nos. |
| 151 |  | Breadboard or trainer |  | 01 Nos. |
| 152 |  | DIAC |  | 04 Nos. |
| 153 |  | TRIAC |  | 04 Nos. |
| 154 |  | Functions Generator |  | 01 Nos. |
| 155 |  | Op-Amp |  | 04 Nos. |
| 156 |  | White chart paper |  | 100 Nos. |
| 157 |  | Graphics and drawing sheets |  | 200 Nos. |
| 158 |  | Pencil |  | 30 Nos. |
| 159 |  | Eraser |  | 30 Nos. |
| 160 |  | Sharpener |  | 30 Nos. |
| 161 |  | Tee |  | 25 Nos. |
| 162 |  | Drawing Board |  | 25 Nos. |
| 163 |  | Set Square |  | 25 Nos. |
| 164 |  | Geometry set |  | 25 Nos. |
| 165 |  | French Curve |  | 25 Nos. |
| 166 |  | Printer |  | 01 Nos. |
| 167 |  | Multimedia projector |  | 01 Nos. |
| 168 |  | Computer |  | 25 Nos. |
| 169 |  | CAD software |  | 01 |
| 170 |  | IDE Software (C Language) |  | 01 |
| 171 |  | Power supply |  | 02 |
| 172 |  | AND gate |  | 02 Nos. |
| 173 |  | OR gate |  | 02 Nos. |
| 174 |  | NAND gate |  | 02 Nos. |
| 175 |  | XOR gate |  | 02 Nos. |
| 176 |  | NOR gate |  | 02 Nos. |
| 177 |  | NOT gate |  | 02 Nos. |
| 178 |  | Tool set |  | 02 Nos. |
| 179 |  | Bread board |  | 02 Nos. |
| 180 |  | Wire 0.5 mm |  | 02 Rolls |
| 181 |  | Digital clock |  | 04 Nos. |
| 182 |  | 555 IC |  | 02 |
| 183 |  | - Connecting leads. |  | 02 Rolls |
| 184 |  | Coil |  | 02 Rolls |
| 185 |  | - Armature |  | 02 Nos. |
| 186 |  | - Winding machine |  | 02 Nos. |
| 187 |  | - Insolating paper |  | 02 Nos. |
| 188 |  | - Carbon brushes |  | 04 Nos. |
| 189 |  | - Soldering wire |  | 02 Rolls |
| 190 |  | Stator frame |  | 02 Nos. |
| 191 |  | - Spring balance |  | 02 Nos. |
| 192 |  | - Ohm meter |  | 02 Nos. |
| 193 |  | - DC compound motor |  | 02 Nos. |
| 194 |  | Electro dynamo meter |  | 02 Nos. |
| 195 |  | - AC series motor |  | 02 Nos. |
| 196 |  | - Reluctance motor |  | 02 Nos. |
| 197 |  | - Hysteresis motor |  | 02 Nos. |
| 198 |  | - Stepper motor |  | 02 Nos. |
| 199 |  | - Servo motor |  | 02 Nos. |
| 200 |  | Control circuit |  | 02 Nos. |
| 201 |  | Temperature sensors. |  | 02 Nos. |
| 202 |  | IDE Software (C++ Language) |  | 01 |
| 203 |  | Sound sensors. |  | 02 Nos. |
| 204 |  | Proximity sensors. |  | 02 Nos. |
| 205 |  | Pressure sensors. |  | 02 Nos. |
| 206 |  | Light sensors. |  | 02 Nos. |
| 207 |  | Position sensors. |  | 02 Nos. |
| 208 |  | Voltage sensors. |  | 02 Nos. |
| 209 |  | Current sensors. |  | 02 Nos. |
| 210 |  | Vision sensors. |  | 02 Nos. |
| 211 |  | Microcontroller |  | 02 Nos. |
| 212 |  | - Arduino Uno |  | 02 Nos. |
| 213 |  | - Arduino Mega |  | 02 Nos. |
| 214 |  | Raspberry pi |  | 01 |
| 215 |  | Software of various microcontroller |  | 01 |
| 216 |  | - Links |  | 02 |
| 217 |  | - Joints |  | 02 |
| 218 |  | - End effectors |  | 02 |
| 219 |  | - Nuts |  | 02 Dozen (each size) |
| 220 |  | - Bolts |  | 02 Dozen (each size) |
| 221 |  | Screws |  | 02 Dozen (each size) |
| 222 |  | - Grippers |  | 02 Nos. |
| 223 |  | Conveyor belt |  | 02 Nos. |
| 224 |  | Sensor |  | 02 Nos. |
| 225 |  | Actuator |  | 02 Nos. |
| 226 |  | - Jumpers |  | 02 Nos. |
| 227 |  | - SD card |  | 02 Nos. |
| 228 |  | - LED |  | 02 Nos. |
| 229 |  | - Power Adaptor |  | 02 Nos. |
| 230 |  | HDMI cable |  | 02 Rolls |
| 231 |  | Humidity sensor |  | 02 Nos. |
| 232 |  | Robotic fastening tool kit |  | 02 Nos. |
| 233 |  | Measuring tool kit |  | 02 Nos. |
| 234 |  | Manipulator links and joints |  | 02 Set |
| 235 |  | Complete design of the Robot |  | 02 Nos. |
| 236 |  | Complete design of the sensor assembly |  | 02 Nos. |
| 237 |  | Robot operating software |  | 01 |
| 238 |  | Camera |  | 02 Nos. |
| 239 |  | Actuator (Robotics Arm) |  | 02 Nos. |
| 240 |  | Potentiometer |  | 02 Nos. |
| 241 |  | Calculator |  | 02 Nos. |
| 242 |  | - Basic ROV components |  | 02 Nos. |
| 243 |  | - Complete design of the ROV |  | 02 Nos. |
| 244 |  | - Remotely Operated Robot (ROR) Trainer |  | 01 Nos. |
| 245 |  | calibration standards measuring devices |  | 01 Nos. |
| 246 |  | - calibration angles measuring devices |  | 01 Nos. |
| 247 |  | - Robots geometry/Length angle alignment measuring device |  | 01 Nos. |
| 248 |  | Robotic Arm |  | 02 Nos. |
| 249 |  | - OHS Standard Manual |  | 01 |
| 250 |  | - friction Measuring Device |  | 01 Nos. |
| 251 |  | Model error detection devices |  | 01 Nos. |
| 252 |  | Strain gauge |  | 02 Nos. |
| 253 |  | Capacitive pressure sensor |  | 02 Nos. |
| 254 |  | Electromagnetic pressure sensor |  | 02 Nos. |
| 255 |  | Optical pressure sensor |  | 02 Nos. |
| 256 |  | Force balancing |  | 02 Nos. |
| 257 |  | Pressure sensor |  | 02 Nos. |
| 258 |  | Lm35 ic solid state temperature sensor |  | 02 Nos. |
| 259 |  | Negative temperature coefficient (ntc) thermistor |  | 02 Nos. |
| 260 |  | Resistance temperature detector (rtd) |  | 02 Nos. |
| 261 |  | Thermocouple |  | 02 Nos. |
| 262 |  | Semiconductor-based sensors |  | 02 Nos. |
| 263 |  | Retro-reflective sensors |  | 02 Nos. |
| 264 |  | Diffuse reflection sensors |  | 02 Nos. |
| 265 |  | BLDC motor |  | 02 Nos. |
| 266 |  | Solenoid valve |  | 02 Nos. |
| 267 |  | Robot Installation Manual |  | 02 Nos. |
| 268 |  | - Error detection tool kit |  | 02 Nos. |
| 269 |  | - Inclinometer |  | 02 Nos. |
| 270 |  | - Mechanical strength measuring tools |  | 01 Nos. |
| 271 |  | - Coordinate calibration tools |  | 01 Nos. |
| 272 |  | Tool Box |  | 02 Nos. |
| 273 |  | - ISO9283 |  | 01 |
| 274 |  | - Vibration sensors |  | 02 Nos. |
| 275 |  | - Jumpers |  | 02 Nos. |
| 276 |  | Turbo C, C++, Code block |  | 01 |
| 277 |  | Line following trainer kit |  | 01 |
| 278 |  | Jumper wires |  | 02 Rolls |
| 279 |  | Chassis for line following robot |  | 01 |
| 280 |  | Bluetooth transceiver |  | 02 Nos. |
| 281 |  | Smart phone |  | 02 Nos. |
| 282 |  | Relay control board |  | 02 Nos. |
| 283 |  | WiFi module |  | 01 Nos. |
| 284 |  | - Door Lock Mechanism |  | 01 Nos. |
| 285 |  | - Controller (Arduino) |  | 01 Nos. |
| 286 |  | - Android Application (Dabble) |  | 01 Nos. |
| 287 |  | - Security System Mechanism |  | 01 Nos. |
| 288 |  | Water level indication Mechanism |  | 01 Nos. |
| 289 |  | - 28 pin DIP IC Socket |  | 01 Nos. |
| 290 |  | - 16MHz Crystal Oscillator |  | 01 Nos. |
| 291 |  | - USB to Serial adapter |  | 01 Nos. |
| 292 |  | - ICs |  | 01 Nos. |
| 293 |  | - Mini Servo Motor |  | 01 Nos. |
| 294 |  | - DVD/CD Drives |  | 01 Nos. |
| 295 |  | - PCB Circuit Board |  | 01 Nos. |
| 296 |  | - 2pins Screw Terminal |  | 01 Nos. |
| 297 |  | 2x 4 pins Screw Terminal Connector |  | 01 Nos. |
| 298 |  | Mobile robot base |  | 01 Nos. |
| 299 |  | Power diode |  | 01 Nos. |
| 300 |  | Resistor of 1 K ohms |  | 01 Nos. |
| 301 |  | Centre tapped transformer |  | 01 Nos. |
| 302 |  | Inductor 2mH |  | 01 Nos. |
| 303 |  | Resistor of 10 K & 100 K ohms |  | 01 Nos. |
| 304 |  | Power MOSFET IRF 840 |  | 01 Nos. |
| 305 |  | Power Diode 1N4007 |  | 01 Nos. |
| 306 |  | DC Source 12V |  | 01 Nos. |
| 307 |  | Resistor 10K |  | 01 Nos. |
| 308 |  | - IC 7404 |  | 01 Nos. |
| 309 |  | Controller IC |  | 02 Nos. |
| 310 |  | Hydraulic bench |  | 02 Nos. |
| 311 |  | Control Valve |  | 02 Nos. |
| 312 |  | Centrifugal pump |  | 02 Nos. |
| 313 |  | Sump tank |  | 02 Nos. |
| 314 |  | Vertical pipe |  | 02 Nos. |
| 315 |  | Channel |  | 02 Nos. |
| 316 |  | Drain Valve |  | 02 Nos. |

# 5.	Members of the Curriculum Development Committee

| Sr. # | Name | Designation | Organization |
| --- | --- | --- | --- |
|  | Engr. Shazia Feroz | Assistant Professor | Benazir Bhutto Shaheed University of Technology |
|  | Engr. Liaqat Ali Jamro | Director Academics | STEVTA |
|  | Mr. Baber Sayeed | Chief Instructor. | Pak Swiss Training Center, Karachi |
|  | Mr. Tariq Aziz | CEO | Megafactor Engineering, Karachi |
|  | Engr. Syed Farhan Hamid Ali | Sr. Instructor. | Pak Swiss Training Center, Karachi |
|  | Engr. Safwan | Automation Engineer | Dewan Cement |
|  | Engr. Altaf Hussain Solangi | Sr. Instructor. | Amantech |
|  | Engr. Hassan | Instrumentation Engineer | Dewan Cement |
|  | Engr. Umair | Automation Engineer | Port Qasim Power Plant |
|  | Engr. Aijaz Ahmed Zia | Automation Engineer | Intech Process Automation |
|  | Engr. Muhammad Yasir | Deputy Director SS&C | NAVTTC |