National Vocation Diploma Level- 5 in Instrumentation Technology

(Curriculum)

National Vocational and Technical Training Commission (NAVTTC)

Government of Pakistan

Table of Contents

1. 	Introduction	6

1.1.	Purpose of the training program:	6

1.2.	Overall objectives of training program:	7

1.3.	Competencies to be gained after completion of course:	9

1.4.	Entry level of trainees:	15

1.5.	Minimum qualification for teachers:	15

1.6.	Medium of instruction:	15

1.7.	Duration of the course:	16

1.8.	Description and structure of the course	17

Health and Safety	26

Module 0714E&A1: Maintain Occupation Health and Safety.	26

Module 0714E&A2: Adopt Safety Regulation, Labour Protection Laws, Environmental Protection Laws at Workplace.	31

Principles of Electrical & Electronics	36

MODULE 0714E&A3.	VERIFY THE DC FUNDAMENTALS	36

MODULE 0714E&A4.	APPLY THE AC FUNDAMENTALS	41

MODULE 0714E&A5.	INSTALL RELAYS AND SWITCHES IN VARIOUS DOMESTIC AND INDUSTRIAL CIRCUITS	43

MODULE 0714E&A6.	INSTALL DIFFERENT TYPES OF TRANSFORMERS	45

Computer Application	47

MODULE 0714E&A7.	APPLY BASIC COMPUTER SKILLS TO CREATE A VARIETY OF DOCUMENT	47

Basic Electrical Wiring (Single Phase):	52

MODULE 0714E&A8.	MAINTAIN TOOLS & EQUIPMENT.	52

MODULE 0714E&A9.	MAKE CABLE/WIRE JOINTS	54

MODULE 0714E&A10.	PREPARE AND INSTALL DISTRIBUTION BOARDS	58

MODULE 0714E&A11.	CARRYOUT BASIC ELECTRICAL INSTALLATION	61

MODULE 0714E&A12.	INSTALL SIMPLE ELECTRICAL WIRING	64

MODULE 0714E&A13.	PERFORM TESTING OF ELECTRICAL WIRING	69

MODULE 0714E&A14.	REPAIR/ MAINTENANCE OF ELECTRICAL INSTALLATIONS	74

MODULE 0714E&A15.	CARRY OUT ELECTRICAL PLANNING AND ESTIMATION	76

Process Variable Measurements	80

MODULE 0714E&A33.	INSTALL TEMPERATURE & PRESSURE MEASUREMENT INSTRUMENT/DEVICES	80

MODULE 0714E&A34.	INSTALL LEVEL MEASURING INSTRUMENT/DEVICES	83

MODULE 0714E&A35.	INSTALL FLOW LEVEL MEASURING INSTRUMENT/DEVICES	86

Metrology	89

MODULE 0714E&A36.	TAKE MEASUREMENTS WITH GRADUATED TOOLS	89

MODULE 0714E&A37.	TAKE MEASUREMENTS WITH COMBINATION SET	92

MODULE 0714E&A38.	TAKE MEASUREMENTS WITH GAUGES	96

MODULE 0714E&A39.	PERFORM MEASUREMENTS THROUGH MICROMETER	98

MODULE 0714E&A40.	MEASURE DIMENSIONS WITH VERNIER TOOLS	101

MODULE 0714E&A42.	MEASURE THREADS AND GEARS	106

Basic Electrical Engineering:	109

MODULE 0714E&A42.	OPERATE MEASURING INSTRUMENTS.	109

MODULE 0714E&A43.	VERIFY OHM’S LAW & KIRCHHOFF’S LAW BY IMPLEMENTING SERIES/PARALLEL CIRCUITS	111

MODULE 0714E&A44.	MEASURE ELECTRICAL POWER, ENERGY, POWER FACTOR & DETERMINE PHASE SEQUENCE	115

MODULE 0714E&A45.	DEMONSTRATE ELECTROMAGNET TO SEE VARIOUS EFFECTS & VERIFY FARADAYS LAW	118

MODULE 0714E&A46.	VERIFY SERIES AND PARALLEL COMBINATION OF CAPACITORS & DETERMINE BREAK DOWN VOLTAGE OF CAPACITOR	122

Basic Electronics (Analog)	125

MODULE 0714E&A47.	IDENTIFY BASIC ELECTRONICS COMPONENTS	125

MODULE 0714E&A48.	DESIGN RECTIFIER USING DIODE	129

MODULE 0714E&A49.	CARRY OUT DIODE APPLICATION	131

MODULE 0714E&A50.	IMPLEMENT BIPOLAR JUNCTION TRANSISTOR (BJT) IN DIFFERENT APPLICATIONS	133

MODULE 0714E&A51.	IMPLEMENT FIELD EFFECT TRANSISTOR (FET) IN DIFFERENT APPLICATIONS	137

MODULE 0714E&A52.	IMPLEMENT THYRISTOR FAMILY IN VARIOUS APPLICATION	141

MODULE 0714E&A53.	APPLICATION OF OPERATIONAL AMPLIFIER	143

Electrical & Electronic Measuring Instruments	147

MODULE 0714E&A64.	OPERATE MEASURING INSTRUMENTS	147

Instrumentation Workshop Practice	153

MODULE 0714E&A65.	Connect Tubing and Fittings	153

MODULE 0714E&A66.	Perform Basic Machining Operation.	156

Electrical Circuit Analysis	159

MODULE 0714E&A67.	APPLY FUNDAMENTAL DC LAWS IN ELECTRIC CIRCUITS	159

Instrumentation Drawing	163

MODULE 0714E&A69- USE ENGINEERING DRAWING TECHNIQUES TO PRODUCE INSTRUMENTATION DRAWINGS	163

Instrument Servicing & Calibration	172

MODULE 0714E&A70.	CALIBRATE THE MEASURING INSTRUMENTS	172

Sensor & Transducers	181

MODULE 0714E&A72.	SENSOR & TRANSDUCERS	181

Semi-Conductor Devices and Electronic Circuit	186

MODULE 0714E&A73.	CHARACTERIZE BJT AMPLIFIERS.	186

MODULE 0714E&A74.	Characterize Diodes Application	195

MODULE 0714E&A75.	Characterize Various FET Amplifiers.	198

MODULE 0714E&A76.	DESIGN AN OPERATION AMPLIFIER	205

MODULE 0714E&A77.	CHARACTERIZE VARIOUS OSCILLATORS.	208

Analytical Instruments	215

MODULE 0714E&A78.	MEASURE TEMPERATURE & HUMIDITY	215

MODULE 0714E&A79.	MEASURE THE DENSITY & POWER OF HYDROGEN	218

MODULE 0714E&A80.	MEASURE THE CONCENTRATION OF ENVIRONMENTAL GASES	221

MODULE 0714E&A82.	PERFORM THE SPECTRO SCOPIC CHARACTERIZTION	229

Industrial Instrumentation & Control	243

MODULE 0714E&A84.	INSTALL THE TRANSDUCERS & INDUSTRIAL TRANSMITTERS	243

MODULE 0714E&A85.	INSTALL THE VARIABLE FREQUENCY DRIVE	245

Digital Circuits & Microprocessor Applications	246

MODULE 0714E&A88.	Classify the Digital Logic Gates	246

MODULE 0714E&A89.	Design Microprocessor applications	253

MODULE 0714E&A90.	DESIGN MICROCONTROLLER APPLICATIONS	256

Advance Control System:	260

MODULE 0714E&A91.	CONSTRUCT AN ADVANCE AUTOMATION & CONTROL SYSTEM	260

Boiler & Instrumentation:	262

MODULE 0714E&A92.	INSTALL THE BOILER CONTROL SYSTEM	262

MODULE 0714E&A93.	OPERATE THE BOILER SYSTEM	265

4.	List of Tools and Equipment	270

5.	Members of the Curriculum Development Committee	283

# 1. 	Introduction

Instrumentation Technology is specifically designed for students of Electronics/Electrical and touches upon a variety of fundamental topics. This course will cover a major portion of Instrumentation Technology. This training program will also build upon these general concepts to cover object-oriented Instrumentation terminology.

This specialization covers topics ranging from basic Electrical & Electronics Measuring Instrumentation, basic engineering drawing & CAD, Instrumentation drawing, Instrumentation servicing & calibration,  Industrial Instrumentation . You will learn fundamental concepts of Microprocessor applications. We also get practical exposure of Boiler Installation & operation.     By the end of this course, trainee will understand the basics of Instrumentation Technology & able to work in an Industry

The National Skills Strategy (NSS) aims at achieving a paradigm shift from time-bound   and supply led to competency-based and demand driven training in Pakistan. Competency-based training approach focuses on the demonstration of actual skills required in the workplace/industry. To achieve this strategy, NAVTTC in collaboration with GIZ is involved in the development process of CBT program to ensure competent skilled labor in demand driven vocational trades.

National Vocational Qualification Framework (NVQF) as designed in consultation with the stakeholders including academia, researchers, industry, chambers and TEVTAs. Its aim is to identify knowledge & skills required for a qualification, determine equivalence, provide guidelines for Recognition of Prior Learning (RPL) and assure quality of training. NVQF designed for the Vertical and horizontal progression for learners, recognition of qualification, meeting with the national and international standards, facilitate conversion of informal training to formal training through RPL, improvement in quality of training and increased options for learners for selecting training programs in different trades. Competency Standards are performance specification that identify the knowledge and competencies an individual need to succeed in the workplace.

## Purpose of the training program:

The purpose of the training is to provide skilled manpower to improve the existing capacity of Instrumentation sector. This training will provide the requisite skills to the trainees to deal with Instrumentation field. Further, to improve the skill level of the Instrumentation technician and prepare them for the Electrical and Electronics industry to meet the market competition nationally and internationally.

  - Overall objectives of training program:

The Instrumentation qualification level 2-5 consists of theoretical and practical details required to learn operational techniques of Calibration, Operation, and Maintenance of Instruments.

The main objectives of the qualification are as follows:

1.	Health and Safety

2.	Computer Application

3.	Principles of Electrical & Electronics

4.	Soft skills

5.	Electrical & Electronics Measuring Instruments

6.	Basic Engineering drawing and CAD

7.	Instrumentation workshop practice

8.	Process Variable Measurement

9.	Electrical Circuit Analysis

10.	Instrumentation drawing

11.	Semi-Conductor devices and Electronics circuit

12.	Instruments servicing and calibration

13.	Digital Skills

14.	Analytical Instruments

15.	Industrial Instrumentation and control

16.	Digital Circuits and Microprocessor application

17.	Advance control system

18.	Sensor and Transducer

19.	Boiler Instrumentation

20.	Entrepreneurship

## Competencies to be gained after completion of course:

The detail of the competency standards included in this qualification is given below:

National Certificate level 2 in “Instrumentation Technology”

- Maintain Occupational Health & Safety

- Adopt safety Regulations, Labour Protection Laws, Environment

- Develop Professionalism

- Apply the AC Fundamentals

- Verify the DC Fundamentals

- Operate the Measuring Instruments

- Connect Tubing and Fittings

- Perform Basic Machining Operation

- Install Relays & Switches in Domestic and Industrial Applications

- Install different types of Transformers

- Apply Basic Computer Skills to create variety of documents

- Maintain tools & Equipment

- Make Cable / Wire Joints

- Prepare and Install Distribution Boards

- Carryout Basic Electrical Installation

- Install Simple Electrical Wiring

- Repair / Maintenance of Electrical Installations

National Certificate level 3, in “Instrumentation Technology”

- Install Computer Operating Systems and Hardware

- Operate Word-Processing Applications

- Operate Spreadsheet Applications

- Operate Presentation Packages

- Perform Writing And Editing Tasks

- Perform Computer Operations

- Use Computer Applications

- Create User Documentation

- Create Technical Documentation

- Create Basic Databases

- Operate Digital Media Technology

- Use Social Media Tools for Collaboration And Engagement

- E-Commerce

- Use Digital Devices

- Apply basic CAD, Solid works software for designing and drawing

- Install Temperature & Pressure measuring instruments/devices

- Install level measuring instruments/devices

- Install the flow measuring instruments/devices

- Characterize Diodes Application

- Characterize various BJT amplifiers

- Characterize various FET amplifiers

- Characterize various Oscillators

- Design an Operational Amplifier

- Take measurements with graduated tools

- Take measurements with combination set

- Take measurements through various gauges

- Perform measurements through Micrometer

- Measure dimensions with Vernier tools

- Perform different measurements

- Operate Measuring Instruments.

- Verify Ohm’s Law & Kirchhoff’s Law by Implementing Series/Parallel Circuits.

- Measure Electrical Power, Energy, Power Factor & Determine Phase Sequence

- Operate Oscilloscope

- Implement Electromagnet to See Various Effects & Verify Faradays Laws.

- Verify Law of Combination of Capacitor and Determine Break down voltage of Capacitor.

- Basic Electronics (Analog)

- Identify Basic Electronics Components

- Design a Rectifier using Diode

- Carry out Diode Application

- Implement Bipolar Junction Transistor (BJT) in Different Applications

- Implement Field Effect Transistor (FET) in Different Applications

- Implement Thyristor Family in Various Application

- Applications of Operation Amplifier

National Certificate level 4, in “Instrumentation Technology”

- Develop Workplace Policy and Procedures for Sustainability

- Maintain Professionalism in the Workplace

- Manage Personal Work Priorities and Professional Development

- Manage Workforce Planning

- Undertake Project Work

- Prepare and Implement Negotiation

- Manage Meetings

- Organize Schedules

- Identify and Communicate Trends in Career Development

- Apply Specialist Interpersonal and Counseling Interview Skills

- Apply fundamental laws of DC in electrical circuits

- Apply fundamental laws of AC in electrical circuits

- Use engineering drawing techniques to produce Instrumentation Drawings

- Interface the Sensor & Transducers

National Certificate level 5, in “Instrumentation Technology”

- Measure the Temperature & humidity

- Measure the Density and Power of Hydrogen Ion (pH)

- Measure the concentration of environmental gases

- Calibrate the Measuring instruments (Metrological Instruments)

- Analyze the gas chromatography

- Perform the spectroscopic characterization

- Install Transducer & Transmitter in Industrial Application

- Install the process Regulators

- Install the Variable Frequency Drive ( VFD )

- Install Pneumatic & Hydraulic systems in industry

- Installation and setup procedure for Digital Pressure, Temperature, Level and Flow Controllers

- Classify the Digital logic Gates

- Design Microprocessor applications

- Design Microcontroller applications

- Construct an Advance Automation and Control system

- Install the advance control system

- Install the Boiler Control System

- Operate a Boiler Control System

- Investigate Microbusiness Opportunities

- Develop A Micro Business Proposal

- Develop A marketing Plan

- Develop And Review A Business Plan

- Organize Finances For The Micro Business

- Manage Human Resources

- Market Products And Services

- Monitor And Review Business Performance

- Negotiate For Resolving Business Issues

- Manage Personal Finances

- Coordinate A Work Team

- Lead Small Teams

## Entry level of trainees:

The entry for National Vocational Certificate level 2-5, in (Instrumentation Sector) “Electrical & Electronics” are given below:

| Title | Entry requirements |
| --- | --- |
| National Certificate level 2, in “Instrumentation Technology” | Entry for assessment for this qualification is open. However, entry into formal training institutes, based on this qualification may require skills and knowledge equivalent to middle (Grade 8). |
| National Certificate level 3, in “Instrumentation Technology” | Entry for assessment for this qualification is open. However, entry into formal training institute for this qualification is person having National Vocational Certificate level 2, in (Instrumentation) “Electrical & Electronics” |
| National Certificate level 4, in “Instrumentation Technology” | Entry for assessment for this qualification is open. However, entry into formal training institute for this qualification is person having National Vocational Certificate level 3, in (Instrumentation) “Electrical & Electronics” |
| National Certificate level 5, in “Instrumentation Technology” | Entry for assessment for this qualification is open. However, entry into formal training institute for this qualification is person having National Vocational Certificate level 4, in (Instrumentation) “Electrical & Electronics” |

## Minimum qualification for teachers:

    - B.Sc. / BE Engineering in Electronics/Mechatronics/Instrumentation.

    - B.Tech-Hons (4 Years) in Electronics/Mechatronics/ Instrumentationv along with 3 years teaching/industry post qualification experience.

    - Diploma in Electronics/Mechatronics/Instrumentation (Level 5) along with 7 years teaching/industry post qualification experience.

## Medium of instruction:

English/Urdu

## Duration of the course:

The proposed curriculum is composed of 20 modules that will be covered in 4825 learning hours. It is proposed that the course may be delivered in three years period.

The distribution of contact hours is given below:

Total		-	4825 hours.

Theory	-	1343.5 hours (27.84%)

Practical	-	3481.5 hours (72.15%)

## Description and structure of the course

Following is the structure of the course:

| Level 5 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Analyze the gas chromatography | Instrumentation supervisor | Level 5 | Technical | 26 | 39 | 65 | 6.5 |
| Perform the spectroscopic characterization |  | Level 5 | Technical | 26 | 39 | 65 | 6.5 |
| Design & Implement a Process Control |  | Level 5 | Technical | 26 | 39 | 65 | 6.5 |
| Install Transducer & Transmitter in Industrial Application |  | Level 5 | Technical | 26 | 39 | 65 | 6.5 |
| Install the process Regulators |  | Level 5 | Technical | 26 | 39 | 65 | 6.5 |
| Install the Variable Frequency Drive ( VFD ) |  | Level 5 | Technical | 26 | 39 | 65 | 6.5 |
| Install Pneumatic & Hydraulic systems in industry |  | Level 5 | Technical | 26 | 39 | 65 | 6.5 |
| Classify the Digital logic Gates |  | Level 5 | Technical | 26 | 30 | 56 | 5.6 |
| Design Microprocessor applications |  | Level 5 | Technical | 26 | 39 | 65 | 6.5 |
| Design Microcontroller applications |  | Level 5 | Technical | 26 | 39 | 65 | 6.5 |
| Design Advance Automation and Control system |  | Level 5 | Technical | 26 | 39 | 65 | 6.5 |
| Install the advance control system | Boiler Operator | Level 5 | Technical | 26 | 39 | 65 | 6.5 |
| Install the Boiler Control System |  | Level 5 | Technical | 26 | 39 | 65 | 6.5 |
| Operate a Boiler Control System |  | Level 5 | Technical | 26 | 39 | 65 | 6.5 |
| Investigate Microbusiness Opportunities | Entrepreneur | Level 5 | Generic | 10 | 18 | 28 | 2.8 |
| Develop A Micro Business Proposal |  | Level 5 | Generic | 10 | 15 | 25 | 2.5 |
| Develop A marketing Plan |  | Level 5 | Generic | 10 | 15 | 25 | 2.5 |
| Develop And Review A Business Plan |  | Level 5 | Generic | 10 | 15 | 25 | 2.5 |
| Organize Finances For The Micro Business |  | Level 5 | Generic | 10 | 15 | 25 | 2.5 |
| Manage Human Resources |  | Level 5 | Generic | 10 | 15 | 25 | 2.5 |
| Market Products And Services |  | Level 5 | Generic | 10 | 15 | 25 | 2.5 |
| Monitor And Review Business Performance |  | Level 5 | Generic | 10 | 15 | 25 | 2.5 |
| Negotiate For Resolving Business Issues |  | Level 5 | Generic | 10 | 15 | 25 | 2.5 |
| Manage Personal Finances |  | Level 5 | Generic | 10 | 15 | 25 | 2.5 |
| Coordinate A Work Team |  | Level 5 | Generic | 10 | 15 | 25 | 2.5 |
| Lead Small Teams |  | Level 5 | Generic | 6 | 15 | 21 | 2.1 |
| Total |  |  |  | 480 | 720 | 1200 | 120 |
| Percentage |  |  |  | 40 | 60 |  |  |

# Analytical Instruments

- ANALYSE THE GAS CHROMATOGRAPHY

Objective:

After the completion of this competency standard the student will be able to apply the Chromatographs in wide range of industries for detection of various concentrations in liquid and gases during the production or analytical routine procedures.

Duration:  Hours	60				Theory:  30 Hours 						Practical:  30 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Lu1 : . Operate Gas chromatograph by (GC) instrument | Trainee will be able to: - Identify Burner assembly - Identify column and its characteristic parameters. - Understand the mechanism and working principle of Burner assembly. - Identify and understand the working mechanism of carrier gas assembly. - Identify the type of used Detector assembly and read it application parameters and detection mechanism, i.e. Photo multiplier tube (PMT) etc - Identify Signal processing /conditioning circuits - Identify Data processing and control software parameters and settings. - Perform measurements by Gas spectrograph for various gases and record the data. - Perform measurements by Column type Gas spectrograph for various gaseous and record the data. - Differentiate between the data measured by “Gas Chromatograph” and “column type chromatograph” and write a report. - Compare, analyze and interpret the all measured data and comments on the deviation and discrepancies in relation with the reference database stored in the machine. - Construct an output report. | - Define basic Gas chromatography techniques and it applications. - Describe the working mechanism, operation and data analysis procedures of the Gas Chromatograph. Practical Activity Operate Gas chromatograph by (GC) instrument | Theory-15Hrs Practical-15Hrs Total-30Hrs | - Gas chromatograph instrument - Known concentration of the reference and sample gaseous. - Column type Gas spectrograph | Class Room Instrumentation Lab. |
| Lu2: Characterize Liquid chromatography instrument | - Identify Burner assembly - Identify column and it characteristic parameters. - Understand the mechanism and working principle of Burner assembly. - Identify and understand the working mechanism of carrier gas assembly. - Identify the type of used Detector assembly and read it application parameters and detection mechanism, i.e. Photo multiplier tube (PMT) etc - Identify Signal processing /conditioning circuits - Identify Data processing and control software parameters and settings. - Identify Data comparison, analysis and interpretation. - Perform measurements by chromatograph spectrograph for various gaseous and record the data. - Compare, analyzed and interpret the data and comments on the deviation and discrepancies in relation with the reference database stored in the machine. - Create an output report. | - Define basic liquid chromatography techniques and it applications. - Define the working mechanism, operation and data analysis procedures of the liquid Chromatograph Practical Activity operation and analysis procedures of the liquid Chromatograph | Theory15Hrs Practical15Hrs Total30Hrs | - Gas chromatograph instrument - Know n concentration of the reference and sample gaseous | Class Room and Instrumentation Laboratory |

## PERFORM THE SPECTRO SCOPIC CHARACTERIZTION

Objective:

After the completion of this competency standard the student will be able to characterize various techniques of spectroscopy that are widely used for analytical purpose in Pharmaceutical, chemical, food and Pathological labs.

Duration:  Hours	60				Theory:  30 Hours 						Practical:  30 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| Lu1 : Operate the Dispersive type Spectrometer in (Visible) | Trainee will be able to: - Identify Optical box containing Prism or grating, lenses and mirrors, detector type, signal conditioning unit, sample compartment and data processing unit. - Setup the spectrometer in visible range and observe the visible beam path. - Identify the sample and or reference compartment - Locate the dispersed spectra after grating or prism and change it position by moving the mirrors positions or prism or grating rotation degree. - Move back or forth and or up-down in order to change the position of colors on the slit entering the sample or reference compartment. - Observe the beam entering in the sample compartment after passing through the slits and observe the change in colors with rotation changing. Identify column and it characteristic parameters. - Observe the beam exiting from the sample compartment and reaching to the Detector and check the output of the Detector or circuit and record the milli-volt. - Prepare or arrange the sample and reference material for the spectrometer. - Measure the reference and data or spectra separately and calculate the results. - Compare, analyzed and interpret the all measured data and comments on the deviation and discrepancies in relation with the reference database. - Conclude and write a report. | - Define basic Gas chromatography techniques and it applications. - Describe the working mechanism, operation and data analysis procedures of the Gas Chromatography. Practical Activity . Operate the Dispersive type Spectrometer | Theory- 06Hrs Practical 06Hrs Total- 12Hrs | - G chromatograph intrument - Known concentration of the reference and sample gaseous | Class Room and Workshop |
| Lu2: Operate the Interferometer type spectrometer | - Identify Optical box containing Interferometer, lenses and mirrors, detector type, signal conditioning unit, sample compartment and data processing unit. - Setup the spectrometer in visible range and observe the visible beam path. - Locate the output spectra from Interferometer and change it movable mirror position and observe the change in output. - Observe the beam entering in the sample compartment after passing through the slits and observe the change in colors with rotation changing. - Observe the beam exiting from the sample compartment and reaching to the Detector and check the output of the Detector or circuit and record the millivolt. - Prepare or arrange the sample and reference material for the spectrometer. - Measure the reference and data or spectra separately and calculate the processed spectra. - Compare the experimental time, accuracy and resolution of the data with the Dispersive type spectrometer. - Compare, analyzed and interpret the all measured data and comments on the deviation and discrepancies in relation with the reference database. - . Create output report. | - Describe basic of the visible dispersive spectrometer a it basic building and functional blocks and it working principles, techniques and its applications Practical Activity Operate the Interferometer type spectrometer | Theory- 06Hrs Practical 06Hrs Total- 12Hrs | - Gas chromatograph instrument - Known concentration of the reference and sample gaseous | Class Room and Workshop |
| Lu3: Operate the FTIR (Four Transform Infrared) Spectrometer | - Identify Optical box containing Interferometer, lenses and mirrors, detector type, signal conditioning unit, sample compartment and data processing unit. - Locate or arrange infrared detector device and observe it working principle. - Setup the spectrometer in visible range and observe the visible beam path. - Locate the output spectra from Interferometer by using Infrared detecting device and change it movable mirror position and observe the change in output. - Observe the beam entering in the sample compartment after passing through the slits and observe the change in colors with rotation changing. - Observe the beam exiting from the sample compartment and reaching to the Detector and check the output of the Detector or circuit and record the millivolt. - Identify and observer Fourier transformation of the signal or spectra. - Prepare or arrange the sample and reference material for the spectrometer. - Measure the reference and data or spectra separately and calculate the processed spectra. - Compare, analyzed and interpret the all measured data and comments on the deviation and discrepancies in relation with the reference database. - Compare the experimental time, accuracy and resolution with the dispersive type infrared spectrometer (if available) - Conclude and write a report. | Define Fourier Transformation and its advantages over conventional data processing techniques Practical Activity Operate the FTIR (Four Transform Infrared) Spectrometer | Theory- 06Hrs Practical 06Hrs Total- 12Hrs | - FTIR (Four Transform Infrared) Spectrometer | Class Room and Workshop |
| Lu4: Operate the Refracto-Meter | - Identify Sensor and its type parameters, Input - output blocks, sample compartment, signal processing or conditioning units, processing units. - Locate and observe the beam path (if possible) in relation with the sample compartment and detector circuit. - Prepare or arrange sample (liquid) and reference materials and literature database. - Measure the data or spectra for the various samples and references and calculate the resultant data or spectra. - Compare, analyzed and interpret the all measured data and comments on the deviation and discrepancies in relation with the reference database. - Conclude and write a report | Define the operation of Refrectro meter. Practical Activity Operate the Refracto-Meter | Theory- 06Hrs Practical 06Hrs Total- 12 Hrs | - Refracto- Meter - Known concentration of the reference and sample materials | Class Room and Workshop |
| Lu5: Analyze the spectra using luminance meter | - Identify Sensor and its type parameters, Input - output blocks, signal processing or conditioning units, data processing units. - Prepare or arrange sample (bio material) and reference material and their literature’s database. - Measure the data or spectra for the various bio-materials and record the data or spectra. - Compare, analyzed and interpret the all measured data and comments on the deviation and discrepancies in relation with the reference database. - . Create an output | - Define basic of the visible infrared light and it detection mechanism. . Practical Activity Analyze the spectra using luminance meter | Theory- 06Hrs Practical 06Hrs Total- 12Hrs | - luminance meter | Class Room and Workshop |

# Industrial Instrumentation & Control

- DESIGN & IMPLEMENT A PROCESS CONTROL

Objective: After completing this competency standard the student will be able to understand the functions, uses and design process control.

Duration: 69Hours				Theory: 30 Hours						Practice: 39 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU-1: Design an Open loop Process Control circuit | Trainee will be able to: - Determine measurement requirements from specifications. - Select appropriate instrument for the measurement - Apply correct measuring techniques as required. - Record the measurements as per requirements. - Generate an output report. | - Explain open loop control. - Describe the working of instruments in open loop control - Explain measuring techniques of open loop control - Describe Advantages and disadvantages of open loop control Practical task: Construct a open loop control system from provided components. | Theory- 07Hrs Practical- 12Hrs Total- 19Hrs | - Sensing element - Final control element - Multimeter - Mili ampere and mili volt source - Connecting wires and cables | Class Room and Instrumentation Laboratory |
| LU-2: Design a Closed Loop Process control circuit | Trainee will be able to: - Select the type of process control - Classify the work output to be taken - Sketch a paper circuit for the required process control - Construct the Closed loop process control circuit - Construct a Feedback circuit - Generate an output report | - Explain Close loop control. - Explain the working of instruments in close loop control Practical task: Construct a close loop control system from provided components. | Theory- 07Hrs Practical- 09Hrs Total- 16Hrs | - Sensing element - Summing amplifier - Final control element - Multimeter - Mili ampere and millivolt source | Class Room and Instrumentation Laboratory |
| LU-3: Implement the ON/OFF Control Mode | Trainee will be able to: - Analyze the type of control mode to be installed - Select the appropriate type of control mode - Install the desired control mode type as per the installation instructions - Perform a test run on the ON/OFF control mode - Generate the Output report | - Explain ON/OFF control mode. - Explain types of control modes - Advantages and disadvantages of ON/OFF control - Explain test run of ON/OFF control. Practical task: Design an open loop control using on/off temperature controller, temperature sensor and heating element. Connect the elements and carry out test run. | Theory- 07Hrs Practical- 09Hrs Total- 16Hrs | - On/Off Temperature controller - Temperature sensor - Heater - Test & measuring Instruments - Connecting Wires and Cables | Class Room and Instrumentation Laboratory |
| LU-4:Implement the PID Control Mode | Trainee will be able to: - Analyze the type of control modeto be installed - Select the appropriate type of control mode - Install the desired control mode type as per the installation instructions - Perform a test run on the P control mode - Perform a test run on the PI control mode - Perform a test run on the PID control mode - Generate the Output report. | - Explain PID control mode. - Describe Working principle of PID control mode. - Describe the test run procedure for PID control system Practical task: Design a close loop control system using PID controller. Connect the elements and carry out test run. | Theory- 09Hrs Practical- 09Hrs Total- 18Hrs | - PID controller - Temperature sensor. - Flow sensor - Pressure sensor - Summing amplifier. - Final control element. | Class Room and Instrumentation Laboratory |

## INSTALL THE TRANSDUCERS & INDUSTRIAL TRANSMITTERS

Objective: After completion of this competency standard the student will be able to grasp the uses, basic concepts and installation of transducers and industrial Transmitters.

Duration: 69 Hours				Theory: 30 Hours						Practice: 39 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU-1: Apply the P/I and I/P Transducers. | Trainee will be able to: - Analyze the type of transducer to be installed - Install the desired type of transducer as per the installations manual - Perform the post installation measures - Generate an output report for the transducer | - Explain P/I(pressure to current) and I/P(current to pressure) Transducers - Describe working of P/I and I/P Transducers Practical task: Identify and connect I/P converter to the input of P/I converter. Operate and check the output of P/I converter. | Theory- 10 Hrs Practical- 12Hrs Total- 22Hrs | - Calibrator - Air Compressor - I/P Transducers - P/I Transducers | Class Room and Workshop |
| LU-2: Install the Differential Pressure Transmitters. | Trainee will be able to: - Analyze the type of DP Transmitter to be installed - Locate the point of installation for DP transmitter - Install the DP Transmitter as per the installations manual. - Perform Post Installation measures. - Generate an output report for the transmitter | - Explain differential pressure transmitters - Describe working of differential pressure transmitters Practical task: Install a differential pressure transmitter high pressure taping and low pressure taping of the process line Apply different pressure and verify the output. Record the result in tabular form | Theory- 10Hrs Practical- 12Hrs Total- 22Hrs | - Calibrator - Pneumatic test bench - Orifice - Air Compressor - Differential Pressure Transmitters | Class Room and Workshop |
| LU-3: Install the Pneumatic Transmitter in Industry | Trainee will be able to: - Analyze the type of Pneumatic Transmitter to be installed - Locate the point of installation for Pneumatic transmitter - Perform Post Installation measures. - Install the Pneumatic Transmitter as per the installations manual. - Generate an output report for the transmitter | - Explain Pneumatic Transmitter. - Describe working principle of Pneumatic Transmitter Practical task: Install a Pneumatic transmitter to the system. Apply pressure and verify the output. Record the result in tabular form | Theory- 10Hrs Practical- 15Hrs Total- 25Hrs | - Pneumatic Transmitter - Pneumatic test bench - Air Compressor - 4-20 mA Source | Class Room and Workshop |

- INSTALL THE PROCESS REGULATORS & ACTUATORS

Objective: After completion of this competency standard the student will be able to grasp the uses, basic concepts and installation of regulator and actuators.

Duration: 90 Hours				Theory: 33 Hours						Practice: 57 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU-1: Install the regulator in application | Trainee will be able to: - Analyze the type of regulator to be installed - Select the appropriate type of regulator - Install the desired regulator type as per the installation instructions - Perform a test run on the regulator installed - Generate the Output report | - Explain types & construction of regulator - Describe the working of regulator Practical task: identify and install the regulator in the system as per sop. Apply input to the regulator and verify the output. | Theory- 08Hrs Practical- 12Hrs Total- 20Hrs | - 4-20 mA source - Pressure Regulator - Compressed air source - Pressure Gauge - Shut-Off Valves | Class Room and Instrumentation Laboratory |
| LU-2: Install the Flow Control Actuators in application | Trainee will be able to: - Analyze the type of Actuator to be installed - Select the appropriate type of Actuator - Install the desired Actuator type as per the installation instructions - Perform a test run on the Actuator installed - Generate the Output report | - Describe characteristics of flow - Explain types & construction of Flow control actuator/valve. - Describe working of Flow control actuator Practical task: Identify the flow control valve and install as per SOP to the system. Operate the system and check the working of flow control actuator. | Theory- 08Hrs Practical- 12Hrs Total- 20Hrs | - 4-20 mA source - Flow control Valve - 3-15 psi compressed air source. - Flow meter - Pipes & Tubing - Assorted fittings - Actuater | Class Room and Instrumentation Laboratory |
| LU-3:Analyze the Power Controlling Devices | Trainee will be able to: - Analyze the type of Power Controlling Device to be installed - Select the appropriate type of Power Controlling Device - Install the desired Power Controlling Device type as per the installation instructions - Perform a test run on the Power Controlling Device - Generate the Output report | - Explain the construction of Power controlling devices - Describe working of Power controlling devices Practical task: Select power controlling devices and install to the system. Operate the system and check the working of power control device. | Theory- 08Hrs Practical- 09Hrs Total- 17Hrs | - Differential Valve - 4-20 mA source - TRIAC Module - DIAC Module - VFD | Class Room and Workshop |
| LU-4: Install the Motor in Application | Trainee will be able to: - Analyze the type of motor to be installed - Select the appropriate type of motor - Install the desired motor type as per the installation manual - Perform a test run on the motor - Generate the Output report | - Describe operation and working principle of motor. - Explain Types & construction of Motor. - Describe applications of motor in industry Practical task: Select the type of motor to be installed according to system requirement. Operate the motor with the help of soft starter switch and overload. | Theory- 06Hrs Practical- 06Hrs Total- 12Hrs | - Rotary Motorized Valve. - Relays - Solid state Relays - Test & Measurement equipment - Connecting Wires and cables | Class Room and Electrical Laboratory |

## INSTALL THE VARIABLE FREQUENCY DRIVE

Objective: After completion of this competency standard the student will be able to grasp the uses, basic concepts and installation of VFD..

Duration: 90 Hours				Theory: 33 Hours						Practice: 57 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU-1: Install the Variable Frequency Drive | Trainee will be able to: - Analyze the type of VFD to be installed - Select the appropriate type of VFD - Install the desired VFD type as per the installation instructions - Perform a test run on the VFD - Generate the Output report | - Explain VFD. - Explain types of VFD - Describe working of VFD - Explain installation of VFD Practical task: Construct a Start/Stop three phase motor circuit through Variable Frequency Drive (VFD). | Theory- 16Hrs Practical- 27Hrs Total- 43Hrs | - VFD with standard acessories - Electrical Test bench - Toolkit - 3-phase Motor - Multi-meter - Product manual | Class Room and Workshop |
| LU-2 Perform the Parameterization process. | Trainee will be able to: - Analyze the required parameter - Select the appropriate parameter - Perform Parameter downloading - Perform a test run with parameter - Generate the output report | - Explain parameters in VFD - Set the Parameters of VFD - Explain test run procedure Practical task:: Construct a Start/Stop three phase motor circuit through Variable Frequency Drive (VFD).Set the parameter for set-point to run on a frequency of 30 Hz clockwise and match the process value (Feedback). | Theory- 17Hrs Practical- 30Hrs Total- 47Hrs | - VFD with standard acessories - Electrical Test bench - Toolkit - 3-phase Motor - Multi-meter - Product Manual | Class Room and Workshop |

## Module 0714E&A137: Install Pneumatic & Hydraulic systems

Objective: This module covers the knowledge and  skills required to install hydraulic equipment, perform equipment maintenance and install pneumatic equipment.

Duration:90  			Hours			Theory: 	33 Hours				Practice: 	57	 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Install Hydraulic Equipment | Trainee will be able to: - Specify the type of Hydraulic system to be installed, such as Jack, press, pump, etc - Select the appropriate type of hydraulic system - Install or mount the desired hydraulic type as per the installation instructions - Perform a test run on the hydraulic equipment - Generate the Output report | - What is Hydraulic system? - Basic components of Hydraulic system - Working principle of Hydraulic operation. - Types of Hydraulic system - Practical applications of Hydraulic system - Pre and Post Installation procedures. - System testing procedures Practical Activity - Install Hydraulic Jack | Theory-11 Hrs Practical-18 Hrs Total-29 Hrs | - Hydraulic Jack - Hydraulic fittings - Solenoid Valve - Pressure Gauge - Hydraulic Station (pump) - Hoses - Installation Manual - Wrench set - Mechanical Tool Kit | Class Room and workshop |
| - Perform equipment maintenance | Trainee will be able to: - Analyze the type of hydraulic maintenance required - Select the appropriate procedure of hydraulic maintenance - Inspect the Pressure Seal, Spring, Piston and oil level - Service the equipment as per the servicing manual | - Types of hydraulic maintenance - Importance of fluid selection, level, temperature and filtration. - Fluid Power principles - Things to monitor during preventive maintenance. - Cleaning and service of components - Importance of Oil and filter changes - Methods for checking pressure Practical Activity - Change the hydraulic filter | Theory-11 Hrs Practical-18 Hrs Total-29 Hrs | - Hydraulic Filter - Hydraulic fittings - Pressure Gauge - Hydraulic Station (pump) - Oil Level - Hoses - Service Manual - Hydraulic Oil - Hydraulic O Rings & Seals - Wrench set - Mechanical Tool Kit | Class Room and workshop |
| - Install Pneumatic Equipment | Trainee must be able to: - Specify the type of pneumatic system to be installed, such as valve, jack, cylinder, etc - Select the appropriate type of Pneumatic - Install the desired Pneumatic type equipment as per the installation instructions - Perform a test run on the Pneumatic equipment - Generate the Output report | - What is Pneumatic system? - Basic components of Pneumatic system - Working principle of Pneumatic operation. - Types of Pneumatic system - Practical applications of Pneumatic system - Working Mechanism of Pneumatic Valve - Pre and Post Installation procedures. - System testing procedures Practical Activity - Install and operate pneumatic paper cutting machine | Theory-11 Hrs Practical-21 Hrs Total-32 Hrs | - Solenoid Valve - Pressure Gauge - Pneumatic Cylinder - Air Compressor - Air Filter - Installation Manual - Mechanical Tool Kit | Class Room and workshop |

# Digital Circuits & Microprocessor Applications

## Classify the Digital Logic Gates

Overview:

After this competency standard the student will be able to identify variety of digital logic gates ICs and their usage in industry.

Duration:  53 Hours					Theory: 26 Hours						Practice: 27 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Identify Various Logic gates ICs | Trainee will be able to: - Identify the Logic gate ICs - Identify its types & polarities - Draw the different circuits with the help of logic gates ICs | - Define Logic gates - Enlist the types of gates - Explain Truth Table of each gate - Explain the logic gates ICs - Explain logic gates parameter - Describe Logic gate IC application Practical Activity: - Separate Logic ICs (7408,7432,7400 etc.) from other ICs | Theory- 05Hrs Practical- 1.5Hrs Total- 6.5Hrs | - Test Bench - Breadboard - Logic gates IC - Datasheet | Class Room and Workshop |
| - Design a circuit based on logic gates using Boolean Expressions. | Trainee will be able to: - Summarize the prescribed Boolean Expression. - Implement on logic gates. - Check input/ouptut waveforms of task on Oscilloscope | - Explain Boolean as digital unit - Describe Boolean voltage levels for 5V and 3.3V ICs - Explain the waveforms in terms of logic levels Practical Activity: Covert the boolean expression into a circuit using logic gates. | Theory- 05 Hrs Practical- 1.5Hrs Total- 6.5Hrs | - Test Bench - Breadboard - Logic gate IC - Dupont Wires - LED & 7Seg. - Relay - Sensors - Logic Tester - IC remover tool - Power supply - Datasheet | Class Room and Workshop |
| - Implement OR gate IC | Trainee will be able to: - Identify OR gate IC - Design different circuits with the help of OR gate IC - Operate DC motor,Relay,7segment,Decoder, and Led with the aid of OR gate IC - Interface sensors(IR Sensor, Proximity Sensor, LDR Sensor, Limit Switch and more) at the inputs of OR gate IC for understanding Automation - Check input/ouptut waveforms of IC on Oscilloscope | - Describe function of OR gate - Explain OR gate construction - Explain the use of OR gate with real world examples - Explain pin functions of logic IC - Describe interlocking with help of OR Logic - Explain OR gate interfacing with other devices (Sensors, Limits switches etc.) Practical Activity: - Construct a circuit using multi input OR gate with real world example (i.e. Elevator Call Button – Four floors) | Theory- 02Hrs Practical- 03Hrs Total- 05Hrs | - Test Bench - Breadboard - OR gate IC - Dupont Wires - LED & 7Seg. - Relay - Sensors - Logic Tester - IC remover tool - Power supply - Datasheet - Oscilloscope | Class Room and Workshop |
| - Implement AND gate IC | Trainee will be able to: - Identify AND gate IC - Design different circuits with the help of AND gate IC - Operate DC motor,Relay,7segment,,Decoder,and Led with the aid of AND gate IC - Interface sensors at the inputs of AND gate IC for understanding Automation - Check input/output waveforms of IC on Oscilloscope | - Describe function of AND gate - Explain AND gate construction - Explain use of AND gate with real world examples - Explain pin functions of logic IC - Describe interlocking with help of AND Logic - Explain AND gate interfacing with other devices (Sensors, Limits switches etc.) Practical Activity: - Construct a circuit using multi input AND gate with real world example (i.e. Elevator Doors – Four floors) | Theory- 02Hrs Practical- 03Hrs Total- 05Hrs | - Test Bench - Breadboard - AND gate IC - Dupont Wires - LED & 7Seg. - Relay - Sensors - Logic Tester - IC remover tool - Power supply - Datasheet - Oscilloscope | Class Room and Workshop |
| - Implement NAND gate IC | Trainee will be able to: - Identify NAND gate IC - Design different circuits with the help of NAND gate IC - Operate DC motor,Relay,7segment,,Decoder,and Led with the aid of NAND gate IC - Interface sensors(IR Sensor, Proximity Sensor, LDR Sensor, Limit Switch and more) at the inputs of NAND gate IC for understanding Automation - Check input/output waveforms of IC on Oscilloscope | - Describe function of NAND gate - Explain NAND gate construction - Explain use of NAND gate with real world examples - Explain pin functions of logic IC - Describe interlocking with help of NAND Logic - Explain NAND gate interfacing with other devices (Sensors, Limits switches etc.) Practical Activity: - Construct a circuit using multi input NAND gate with real world example (i.e. Elevator Example – Four floors) | Theory- 02Hrs Practical- 03Hrs Total- 05Hrs | - Test Bench - Breadboard - NAND gate IC - Dupont Wires - LED & 7Seg. - Relay - Sensors - Logic Tester - IC remover tool - Power supply - Datasheet - Oscilloscope | Class Room and Workshop |
| - Implement NOR gate IC | Trainee will be able to: - Identify NOR gate IC - Design different circuits with the help of NOR gate IC - Operate DC motor,Relay,7segment,,Decoder, and Led with the aid of NOR gate IC - Interface sensors(IR Sensor, Proximity Sensor, LDR Sensor, Limit Switch and more) at the inputs of NOR gate IC for understanding Automation - Check input/output waveforms of IC on Oscilloscope | - Describe function of NOR gate - Explain NOR gate construction - Explain use of NOR gate with real world examples - Explain pin functions of logic IC - Describe interlocking with help of NOR Logic - Explain NOR gate interfacing with other devices (Sensors, Limits switches etc.) Practical Activity: - Construct a circuit using multi input NOR gate with real world example (i.e. Elevator Example – Four floors) | Theory- 02Hrs Practical- 03Hrs Total- 05Hrs | - Test Bench - Breadboard - NOR gate IC - Dupont Wires - LED & 7Seg. - Relay - Sensors - Logic Tester - IC remover tool - Power supply - Datasheet - Oscilloscope | Class Room and Workshop |
| - Implement XOR gate IC | Trainee will be able to: - Identify XOR gate IC - Design different circuits with the help of XOR gate IC - Operate DC motor, Relay and LED with the help of XOR gate IC - Interface sensors(IR Sensor, Proximity Sensor, LDR Sensor, Limit Switch and more) at the inputs of XOR gate IC for understanding Automation - Check input/output waveforms of IC on Oscilloscope | - Describe function of NOR gate - Explain NOR gate construction - Explain use of NOR gate with real world examples - Explain pin functions of logic IC - Describe interlocking with help of NOR Logic - Explain NOR gate interfacing with other devices (Sensors, Limits switches etc.) Practical Activity: Construct a circuit using multi input XOR gate with real world example (i.e. Elevator Example – Four floors) | Theory- 02Hrs Practical- 03Hrs Total- 05Hrs | - Test Bench - Breadboard - XOR gate IC - Dupont Wires - LED & 7Seg. - Relay - Sensors - Logic Tester - IC remover tool - Power supply - Datasheet - Oscilloscope | Class Room and Workshop |
| - Implement XNOR gate IC | Trainee will be able to: - Identify XNOR gate IC - Design different circuits with the help of XNOR gate IC - Operate DC motor,Relay,7segment,,Decoder, and Led with the aid of XNOR gate IC - Interface sensors(IR Sensor, Proximity Sensor, LDR Sensor, Limit Switch and more) at the inputs of XNOR gate IC for understanding Automation - Check input/output waveforms of IC on Oscilloscope | - Describe function of NOR gate - Explain NOR gate construction - Explain use of NOR gate with real world examples - Explain pin functions of logic IC - Describe interlocking with help of NOR Logic - Explain NOR gate interfacing with other devices (Sensors, Limits switches etc.) Practical Activity: - Construct a circuit using multi input XNOR gate with real world example (i.e. Elevator Example – Four floors) | Theory- 02Hrs Practical- 03Hrs Total- 05Hrs | - Test Bench - Breadboard - XNOR gate IC - Dupont Wires - LED & 7Seg. - Relay - Sensors - Logic Tester - IC remover tool - Power supply - Datasheet - Oscilloscope | Class Room and Workshop |
| - Design a circuit by using FLIP FLOP(SR/JK/D) IC | Trainee will be able to: - Identify FLIP FLOP IC Design different circuits with the help of FLIP FLOP IC - Operate DC motor, Relay and Led with the help of FLIP FLOP IC - Interface sensors(IR Sensor, Proximity Sensor, LDR Sensor, Limit Switch and more) at the inputs of FLIP FLOP IC for understanding Automation - Construct counter, digital clock with aid of FLIP FLOP ICs - Serial and Parallel transfer of information from one register to others with the help of FLIP FLOP ICs - Binary up and down counter with the aid of FLIP FLOP ICs - Design a traffic light controller. - Check input/output waveforms of IC on Oscilloscope | - Describe function of flip-flop circuit - XNOR gate circuit design - Describe the types of flip-flips and their usage - Describe parameter associated with flip-flips, (i.e. edge triggering, inverted output) - Describe the real world examples of flip flips and their circuit behavior Practical Activity: - Construct a traffic signal using flip flop circuit | Theory- 02Hrs Practical- 03Hrs Total- 05Hrs | - Test Bench - Breadboard - Flip Flop ICs - Dupont Wires - LED & 7Seg. - Relay - Sensors - Logic Tester - IC remover tool - Power supply - Datasheet - Oscilloscope | Class Room and Workshop |
| - Design a circuit by using Multiplexer and De-multiplexer | Trainee will be able to: - Construct a circuit for operating led by using Multiplexer - Construct a circuit for operating LED’s by using De-Multiplexer - Check input/output waveforms of Mux and De-Mux on Oscilloscope | - Explain multiplexing and de-multiplexing concept and their need - Explain construction and operation of MUX & DEMUX circuit - Explain the logic expression Practical Activity: Construct a combination lock circuit using four input numbers. | Theory- 02Hrs Practical- 03Hrs Total- 05Hrs | - Test Bench - Breadboard - MUX IC - DEMUX IC - Dupont Wires - LED & 7Seg. - Relay - Sensors - Logic Tester - IC remover tool - Power supply - Datasheet - Oscilloscope | Class Room and Workshop |

## Design Microprocessor applications

Overview

After completion of this competency standard the student will be able to perform a variety of tasks on a microprocessor and will be able to integrate the programming knowledge into the microprocessor.

Duration:  65 Hours					Theory: 26 Hours						Practice: 39 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Identify microprocessors | Trainee will be able to: - Identify processor features and Pin-Configuration - Draw the Microprocessor architecture - Compare and contrast the characteristics of Microprocessor & Microcontroller - Implement micro-processor selection criteria | - Explain functions and parameters of microprocessor - Explain Microprocessor elements - Explain Microprocessor architecture - Describe Microprocessor selection criteria Practical Activity: - Separate microprocessor IC (i.e. 8051,89C51,STM32 etc.) from other ICs and micro-controllers | Theory- 02Hrs Practical- 03Hrs Total- 05Hrs | - Test Bench - Microprocessor - Microcontroller - ICs - Development Kit - Logic Tester - Breadboard - IC remover tool - Datasheet | Class Room and Workshop |
| - Identify function of microprocessors and their associated pins | Trainee will be able to: - Identify functions and their allocation - Identify Arithmetic and Logic functions - Identify the bus structure of microprocessor - Identify the IOs and their application | - Explain memory mapping and their functionality - Explain basic/logic arithmetic operation - Explain relationship Hardware pins and their function - Describe the limitation in features of a microprocessor - Explain Datasheet Practical Activity: - Select microprocessor IC according to given criteria (Bus speed, No of IOs, Memory) | Theory- 06Hrs Practical- 09Hrs Total- 15Hrs | - Test Bench - Microprocessor - Development Kit - Logic Tester - Breadboard - IC remover tool - Power supply - Datasheet | Class Room and Workshop |
| - Program a microprocessor using basic instruction set | Trainee will be able to: - Implement Basic instruction set and explain any - Implement addressing mode - Implement data transfer via accumulator | - Explain the use of programming station - Explain Instruction set - Define bus addressing procedure and their use - Describe the basic operation of microprocessor Practical Activity: - Program a microprocessor to perform basic data operation via various addressing modes | Theory- 06Hrs Practical- 09Hrs Total- 15Hrs | - Test Bench - Microprocessor - Development Kit - Breadboard - IC remover tool - Logic Tester - Programming Computer - Datasheet - Instruction Set | Class Room and Workshop |
| - Program a microprocessor to perform basic arithmetic and logical operation | Trainee will be able to: - Implement arithmetic and logical operation. - Addition & Subtraction - Multiplication & Division - AND, OR, NOT operation - Jump operation | - Explain basic arithmetic operations - Explain basic logical operations - Describe conditional and non-condition operations - Describe jump operation Practical Activity: - Program a microprocessor to perform calculator functions (Addition, Subtraction, memory, clear etc.) | Theory- 06Hrs Practical- 09Hrs Total- 15Hrs | - Test Bench - Microprocessor - Development Kit - Breadboard - IC remover tool - Logic Tester - Programming Computer - Datasheet | Class Room and Workshop |
| - Program a microprocessor to perform special operations | Trainee will be able to: - Implement special function operations - Loops - Input / Output - Subroutines and interrupt - Explain Debugging and its procedure | - Define the loops - Explain types of loops - Explain IO operations and their limitations - Explain the subroutine/Interrupt in microprocessor and its need - Describe the debugging tool and its utilization Practical Activity: - Program a microprocessor to develop complex set of instruction including multiple function of microprocessor. (i.e. Clock) | Theory- 06Hrs Practical- 09Hrs Total- 15Hrs | - Test Bench - Microprocessor - Development Kit - Debugging Tool - Breadboard - IC remover tool - Logic Tester - Programming Computer - Datasheet | Class Room and Workshop |

## DESIGN MICROCONTROLLER APPLICATIONS

Overview

After completion of this competency standard the student will be able to perform a variety of tasks on a microcontroller and will be able to integrate the programming knowledge into the microcontroller.

Duration:  65 Hours					Theory: 26Hours						Practice: 39 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Identify microcontrollers | Trainee will be able to: - Draw the microcontroller architecture - Implement microcontroller selection criteria | - Define microcontroller - Explain functions and parameters of microcontroller - Explain microcontroller architecture - Describe microcontroller selection criteria - Describe hardware pins and their function Practical Activity: - Separate microcontroller IC from other ICs. | Theory- 02Hrs Practical- 03Hrs Total- 05Hrs | - Test Bench - Microcontroller - Development Kit - Breadboard - IC remover tool - Datasheet | Class Room and Workshop |
| - Perform BCD addition and subtraction using Microcontroller | Trainee will be able to: - Draw the schematic diagram of microcontroller for BCD addition and subtraction operation - Select the components of microcontroller for BCD addition and subtraction operation - Implement the circuit of microcontroller for BCD addition and subtraction operation - Program the microcontroller for BCD addition Functions - Apply the Inputs and verify the output, - Program the microcontroller for subtraction Functions - Apply the Inputs and verify the output,. - Generate Lab report | - Explain BCD operation and its calculation - Describe the BCD arithmetic operation - Explain the IOs binary operation - Explain the IO voltage levels - Describe the programming station Practical Activity: Program a microcontroller to perform arithmetic operations - Addition (4-bit) - Subtraction (4-bit) - Addition (8-bit) - Subtraction (8-bit) | Theory- 06Hrs Practical- 09Hrs Total- 15Hrs | - Test Bench - Microcontroller - LED - Resistors - Dupont wires - Development Kit - Breadboard - Logic Tester - IC remover tool - Datasheet | Class Room and Workshop |
| - Perform multiplication and division using Microcontroller | Trainee will be able to: - Draw the schematic diagram of microcontroller for multiplication and division operation - Select the components of microcontroller for multiplication and division operation - Implement the circuit of microcontroller for multiplication and division operation - Program the microcontroller for multiplication Functions - Apply the Inputs and verify the output, - Program the microcontroller division - Apply the Inputs and verify the output. - Generate Lab report | - Explain multiplication / division and its calculation. - Describe the multiplication/division operation - Explain the IOs binary operation - Explain the IO voltage levels - Describe the programming kit functions Practical Activity: Program a microcontroller to perform arithmetic operations - Multiplication (4-bit) - Division (4-bit) - Multiplication (8-bit) - Division (8-bit) | Theory- 06Hrs Practical- 09Hrs Total- 15Hrs | - Test Bench - Microcontroller - LED - Resistors - Dupont wires - Development Kit - Breadboard - Logic Tester - IC remover tool - Datasheet | Class Room and Workshop |
| - Perform the operation to convert Centigrade in to Fahrenheit using Microcontroller | Trainee will be able to: - Draw the schematic diagram of microcontroller for the conversion of Centigrade to Fahrenheit - Select the components of microcontroller for the conversion of Centigrade to Fahrenheit - Implement the circuit of microcontroller for the conversion of Centigrade to Fahrenheit - Program the microcontroller for the conversion of Centigrade to Fahrenheit - Apply the Inputs and verify the output - Generate Lab report | - Describe Analog to Digital converter function - Explain the ADC input limitations and parameters - Describe the calculation in the code - Explain the results in BCD data Practical Activity: Program a microcontroller to make a temperature indicator (BCD Based) - Centigrade to Fahrenheit - Fahrenheit to Centigrade | Theory- 06Hrs Practical- 09Hrs Total- 15Hrs | - Test Bench - Microcontroller - LED - Thermistor - Dupont wires - Development Kit - Breadboard - Logic Tester - IC remover tool - Datasheet | Class Room and Workshop |
| - Interface LCD with microcontroller | Trainee will be able to: - Identify the pins of LCD and its function - Draw the schematic diagram of LCD interfacing with microcontroller - Select the components of LCD interfacing with microcontroller - Implement the circuit of LCD interfacing with microcontroller - Interface the LCD with Microcontroller via programming - Generate the hex file and download in microcontroller - Monitor the output - Generate Lab report | - Explain LCD and its function - Explain the pins configuration of LCD - Describe the LCD screen parameters (size, Line, backlight, sleep mode etc.) - Explain the data transfer from sensor to LCD - Explain the LCD related functions (Font size, Screen change, Side Shifting, result update etc.) - Explain code building, compiling and burning process Practical Activity: Program a microcontroller to make a temperature indicator (LCD based) - Centigrade to Fahrenheit - Fahrenheit to Centigrade | Theory- 06Hrs Practical- 09Hrs Total- 15Hrs | - Test Bench - Microcontroller - LED - Thermistor - Dupont wires - LCD Screen - Development Kit - Breadboard - Logic Tester - IC remover tool - Datasheet | Class Room and Workshop |

# Advance Control System:

## DESIGN ADVANCE AUTOMATION AND CONTROL SYSTEM

Objective: This module covers the skills and knowledge required to identify, maintain, develop, program, modify, simulate and perform maintenance and diagnostics of variety of control system and its components.

Duration:  45 Hours					Theory: 15 Hours						Practice: 30 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Construct an advance Automation & Control System | Trainee will be able to: - Program a PLC to implement different programmable functions. - Implement industrial application using instruments and communication system. | - Explain PLC Programming - Ladder - FBD (Function Block Code) - STL (Statement List) - Explain Arithmetic and Logical functions. - Explain Open/Close loop control. - Describe the Communication protocols used in Control System. Practical Activity - Design a DOL (Direct Online) Starter program to run a motor using PLC . | Theory-07Hrs Practical-15Hrs Total- 22Hrs | - Programming Station - PLC Trainer - Industrial Instrument - Datasheet/Manuals - Electrical Tool Box - Magnetic Contactor - Motor - Computer - Programming Station | Class Room and Workshop |
| - Install the Advance Control System | Trainee will be able to: - Differentiate Modules of control systems. - Install / Modify PLC and its components. - Install field instruments | - Describe PLC Components and their working. - Define and explain I/O List. - Explain Hardware Configuration. - Explain Module Selection Criteria. - Describe working principles of Field instruments. - Explain offline/online testing procedures. - Describe the I/O configuration by reading its manual/datasheet. - Explain Communication Topologies. Practical Activity - Install and perform wiring of PLC Rack and components. - Implement the hardware configuration in programming software. | Theory-08Hrs Practical-15Hrs Total- 23Hrs | - Datasheet/Manuals - Electrical Tool Box - Digital Multi meter - PLC Trainer - Power Supply - 4-20Milli-Amp Source. | Class Room and Workshop |

## INSTALL ADVANCE CONTROL SYSTEM

Objective: This module covers the skills and knowledge required to identify, maintain, develop, program, modify, simulate and perform maintenance and diagnostics of variety of control system and its components.

Duration:  45 Hours					Theory: 15Hours						Practice: 30 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Differentiate Modules of control systems | Trainee will be able to: - Identify the standalone/ module - Demonstrate its functions & capabilities - Extract information from device part number | - Identify field instruments - Install the wiring from Module to instrument location - Install the instrument - Perform field testing and calibration (if required) - Generate the field testing reports Practical Activity | Theory-05Hrs Practical-09Hrs Total- 14Hrs | - Programming Station - PLC Trainer - Industrial Instrument - Datasheet/Manuals - Electrical Tool Box - Magnetic Contactor - Motor - Computer - Programming Station | Class Room and Workshop |
| - Install / Modify PLC and its components | Trainee will be able to: - Implement hardware configuration as per given design - Perform post installation test - Disassemble the hardware in accordance user’s manual | - Describe PLC Components and their working. - Define and explain I/O List. - Explain Hardware Configuration. - Explain Module Selection Criteria. - Describe working principles of Field instruments. - Explain offline/online testing procedures. - Describe the I/O configuration by reading its manual/datasheet. - Explain Communication Topologies. Practical Activity Implement the hardware configuration in programming software. | Theory-05Hrs Practical-09Hrs Total- 14Hrs | - Datasheet/Manuals - Electrical Tool Box - Digital Multi meter - PLC Trainer - Power Supply - 4-20Milli-Amp Source. | Class Room and Workshop |
| - Install field instruments | Trainee will be able to: - Identify field instruments - Install the wiring from Module to instrument location - Install the instrument - Perform field testing and calibration (if required) - Generate the field testing reports | - Explain offline/online testing procedures - Explain the data sheets of instruments - Explain the importance of hardware testing (monitoring, diagnose real time data/faults) - Explain the module on basis of their specification Practical Activity - Perform field testing | Theory-05Hrs Practical-12Hrs Total- 17Hrs |  |  |

# Boiler & Instrumentation:

## INSTALL THE BOILER CONTROL SYSTEM

Objective:After this competency standard the student will be able to implement, troubleshoot and perform maintenance and diagnostics of boiler instruments and its control system in various industries

Duration: 90 Hours	Theory: 30 Hours	Practical:  60 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| - Select appropriate fittings and instruments according to requirement | Trainee will be able to: - Select appropriate fittings and instruments according to requirement - Install/remove the orifice plate. - Perform boiler route/ pipe line testing. - Identify/remove the hazard that may affect equipment / maintenance personnel during maintenance - Perform lockout/tag out before starting maintenance work | - Describe the Working principle, method and Type of boiler. - Describe the installation and calibration method of Current to Pneumatic (I/P) and Pneumatic to Current (P/I) converters - Describe the types of mounting fittings and their working principles. - Define instruments as per need and environment. - Explain applications of Bernoulli's Theorem. - Describe the heating system and its effectiveness. - Explain the importance and procedure of flow, pressure and temperature compensation. Practical Activity: Select given fittings & Instruments according to the specification. | Total- 45Hrs Theory – 15 Hrs Practical-30Hrs | - Fixed Spanner Set - Ring Spanner Set - Plier Set - Screw Driver Set Phillips - Screw Drive Set Flat - Grip Plier - C Clamps - Bench Vise - Special types of tools (specified by the manufacturer, If) - Testing instruments - PPE’s - Digital Multi-meter - I/P Converter - P/I Converter - Orifice Plate - Boiler & Instrumentation Trainer | - Class Room - Boiler Room |
| LU2: Install the Instruments | Trainee will be able to: - Install and calibrate the transmitter - Remove the transmitter. - Calibrate the sensor and process variable switches. - Verify the physical value to the controller value by using specific tools. - Install/remove/Calibrate the differential pressure transmitter (ΔP). - Change the operating pressure safety valve (PSV) and Test the operation. - Demonstrate the working of gas regulator by changing the line pressure as specified. - Perform UV flame detector maintenance/ alignment/ manual test its functionality - Install/remove air blower - Perform manual testing/maintenance of an air blower - Install/remove ignition transformer for any abnormality - Visually inspect the electric discharge density and distance/physical condition of electrodes. - Demonstrate the phenomenon of flow, pressure, temperature compensation. | - Explain the installation and calibration method of Pressure, flow and differential Pressure transmitter (ΔP). Also calculation for conversion of ΔP to flow measurement. - Explain the installation of Level and density transmitters/controllers. - Define the mounting and understanding of pressure and flow gauges with pig-tail siphon. - Explain the selection of digital and analog actuators. - Define ventilation system and its importance - Explain the working principles of motor operated valves. (MOV), regulators, air blower, ignition transformers knowledge of their impact on overall operation of a boiler. - Explain the use of Orifice plate for fluid and gas. Practical Activity: Install given Instruments according to the SOP | Total- 45Hrs Theory – 15 Hrs Practical-30Hrs | - Digital pressure gauge - Digital thermometer - Electrical Tester - Electrical Tool Box - Function generator - Jumper wire set - Milliamps Source - Multi-Meter - O&M Manuals - Part Manuals - Power supply - Spanner Set - Special Tools - Tag out/lockout tools - Trainer - PPEs - Boiler & Instrumentation Trainer | Class Room and Workshop |

## OPERATE THE BOILER Control SYSTEM

Objective: After the following competency standard, the student will be able to operate, perform operational shutdowns, monitor, analyze and record boiler data in various industries.

Duration: 90 Hours	Theory: 30 Hours	Practical:  60 Hours

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1: Prepare the boiler for operation | Trainee will be able to: - Identify Health and safety hazards & report to responsible personnel. - Purge the boiler according to workplace procedure - Perform pre-operational checks & confirm operational status of boiler and related equipment | - Describe the Boiler operation checks - Describe Health & safety hazards Practical Activity: Prepare a boiler for operation | Total- 14Hrs Theory – 05 Hrs Practical-09Hrs | - Datasheets - Digital pressure gauge - Digital thermometer - Electrical Tester - Electrical Tool Box - Function generator - Jumper wire set - Milliamp Source - Multi-Meter - PPEs - Boiler with Instrumentation | - Class Room - Boiler room |
| LU2:Start and monitor boiler operation | Trainee will be able to: - Start the boiler safely as per manufacturer's specifications - Operate the plant within limits of manufacturer's specifications - Monitor the Equipment to confirm operating condition - Test the water quality and adjust as required - Ensure that the workplace meets housekeeping standards | - Describe the services associated with boilers - Explain the boiler system layout and steam/oil cycle - Explain the importance and procedure of ash removal - Explain the limitation of a boiler and the process data which need to be monitored Practical Activity: - Start and monitor boiler operation - Start the boiler and fill Monitoring sheet. | Total- 14Hrs Theory – 05 Hrs Practical-09Hrs | - Datasheets - Digital pressure gauge - Digital thermometer - Electrical Tester - Electrical Tool Box - Function generator - Jumper wire set - Milliamp Source - Multi-Meter - Boiler with Instrumentation | - Class Room - Boiler room |
| LU3:Analyze and respond to abnormal performance | Trainee will be able to: - Analyze operating data and plant operating conditions to identify causes of abnormal performance - Take corrective action in response to Hazards - Implement emergency procedures as required | - Explain the parameter which impacts the shutdown. - Explain the pre-handover procedures/checklist and their impact on overall operation. - Explain the relationships of process variables and data curves - Explain the basic working principles of combustion operation - Explain the cleaning/purging procedure (internal/external) during shutdown - Define the data and their operating ranges. Practical Activity: Performboiler operation and observe for abnormal performance. | Total- 14Hrs Theory – 05 Hrs Practical-09Hrs | - Datasheets - Digital pressure gauge - Digital thermometer - Electrical Tester - Electrical Tool Box - Function generator - Jumper wire set - Milliamp Source - Multi-Meter - Boiler with Instrumentation | - Class Room - Boiler room |
| LU4: Handover the boiler operations | Trainee will be able to: - Maintain the workplace in accordance with statutory requirements and workplace procedures - Carry out Handing over according to workplace procedure | - Define the importance of PPEs and safe working procedures - Explain the procedure of Handover the boiler operation - Explain Handover the boiler operations Practical Activity: Carryout handing over procedure as per SOP. | Total- 14Hrs Theory – 05 Hrs Practical-09Hrs | - Datasheets - Digital pressure gauge - Digital thermometer - Electrical Tester - Electrical Tool Box - Function generator - Jumper wire set - Milliamp Source - Multi-Meter | - Class Room - Boiler room |
| LU5:Carry out an operational shutdown | Trainee will be able to: - Shut down the boiler according to workplace procedures - Identify the maintenance requirements & report according to workplace reporting procedure | - Explain the purpose of purging, efficiency of fuel, tag out/lockout. - Explain the operational and emergency shutdown procedure. - Define the operational data and its importance - Explain the procedures for responding to emergency situations including safe shutdown procedure Practical Activity: Perform operational shutdown | Total- 14Hrs Theory – 05 Hrs Practical-09Hrs | - Datasheets - Digital pressure gauge - Digital thermometer - Electrical Tester - Electrical Tool Box - Function generator - Jumper wire set - Milliamp Source - Multi-Meter - Boiler with Instrumentation | - Class Room - Boiler room |
| LU6: Prepare the boiler for an internal inspection | Trainee will be able to: - Shut down the boiler according to workplace procedures - Clean the boiler is internally and externally according to workplace procedures - Remove the valves and fittings - Place the boiler in the appropriate storage mode according to manufacturer's recommendations - Generate a record report | - Explain inspection procedure as per SOP - Describe valve dismental procedure - Explain the importance of periodic inspection of boilers - Explain inspection report Practical Activity Perform internal inspection | Total- 20 Hrs Theory- 05 Hrs Practical-15 Hrs | - Datasheets - Digital pressure gauge - Digital thermometer - Electrical Tester - Electrical Tool Box - Function generator - Jumper wire set - Milliamp Source - Multi-Meter - Boiler with Instrumentation | - Class Room - Boiler room |

# Entrepreneurship

## Module: Develop Entrepreneurial Skills

Objective:

After the completion of this competency standard, the Trainee will be expected to develop a business plan, collect information regarding funding sources, develop a marketing plan and develop basic business communication skills. Trainee’ss underpinning knowledge regarding entrepreneurial skills will be sufficient to provide you the basis for your work.

Duration:  30 Hours			Theory:  12 Hours			Practice:  18 Hours          	        Credit Hours:

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Develop a business plan | Trainee will be able to: - Conduct a market survey to collect following information: - Customer /demand - Tools, equipment, machinery and furniture with rates - Raw material - Supplier - Credit / funding sources - Marketing strategy - Market trends - Overall expenses - Profit margin - Select the best option in terms of cost, service, quality, sales, profit margin, overall expenses - Compile the information collected through the market survey, in the business plan format | - Main elements of business plan - Filling the business plan format - Enlist specific business terms used in the industry - Describe 7Cs of business communication | Theory- 03 Hr Practical- 03Hrs Total- 06 Hr | - Internet - Laptop/Computer - White board - Projector screen - Multimedia projector | Classroom |
| LU2. Collect information regarding funding sources | Trainee will be able to: - Identify the available funding sources based on their terms and conditions, maximum loan limit, payback time, interest rate - Choose the best available option according to investment requirement - Prepare documents according to the loan agreement requirement - Include the information of funding sources in the business plan | - Enlist the available funding sources - Explain how to get loan to start a new business - Explain market survey and its tools e.g.: questionnaire, interview, observation etc | Theory Theory- 03 Hr Practical- 03Hrs Total- 06 Hr | - Internet - Laptop/Computer - White board - Projector screen - Multimedia projector | Classroom |
| LU3. Develop a marketing plan | Trainee will be able to: - Make a marketing plan for the business including product, price, placement, promotion, people, packaging and positioning - Include the information of marketing plan in the business plan | - 7ps of marketing including product, price, placement, promotion, people, packaging and positioning | Theory- 03 Hr Practical- 03Hrs Total- 06 Hr | - Internet - Laptop/Computer - White board - Projector screen - Multimedia projector | Classroom |
| LU4. Develop basic business communication skills | Trainee will be able to: - Communicate with internal customers e.g.: labor, partners and external customers e.g.: suppliers, customers etc., using effective communication skills - Use different modes of communication to communicate internally and externally e.g.: presentation, speaking, writing, listening, visual representation, reading etc. - Use specific business terms used in the market | - Description of the market trends for specific product offering - Different modes of communication and their application in the industry | Theory- 03 Hr Practical- 09Hrs Total- 12 Hr | - Internet - Laptop/Computer - White board - Projector screen - Multimedia projector | Classroom |

## Module: Apply management and communications techniques

Objective:

After the completion of this competency standard, the Trainee will be able to contribute to communications planning, Conduct information management activities, Communicate project information, Contribute to assessing effectiveness of communication.

Duration:  30 Hours		Theory:  12Hours			Practice:  18 Hours          	        Credit Hours:

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Contribute to communications planning | Trainee will be able to: - Identify, source and contribute relevant information requirements to initial project documentation. - Contribute to developing and implementing the project communications plan and communications networks. | - Outline methods of reviewing communication outcomes | Theory- 03 Hr Practical- 03Hrs Total- 06 Hr | - Internet - Laptop/Computer - White board - Projector screen - Multimedia projector | Classroom |
| LU2. Conduct information management activities | Trainee will be able to: - Act on and process project information according to agreed procedures as directed, to aid decision-making processes throughout project life cycle. - Maintain information to ensure data is secure and auditable. | - Summarize models and methods of communications management in context of project life cycle and other project management functions | Theory- 03 Hr Practical- 03Hrs Total- 06 Hr Min | - Internet - Laptop/Computer - White board - Projector screen - Multimedia projector | Classroom |
| LU3. Communicate project information | Trainee will be able to: - Communicate with clients and other stakeholders during project using agreed networks, processes and procedures to ensure flow of necessary information. - Ensure reports are prepared and released according to authorization, or produced for release by others. - Seek information and advice from appropriate project authorities as required. | - Identification of Project Information, policies and procedures relevant - Summarize models and methods of communications management in context of project life cycle and other project management functions | Theory- 03 Hr Practical- 03Hrs Total- 06 Hr | - Internet - Laptop/Computer - White board - Projector screen - Multimedia projector | Classroom |
| LU4. Contribute to assessing effectiveness of communication | Trainee will be able to: - Assist in ongoing review of project outcomes to determine effectiveness of communications-management activities. - Report communications-management issues and responses to higher project authorities for application of lessons learned to future projects. | - Assessment of effectiveness of project communication | Theory- 03 Hr Practical- 09Hrs Total- 12 Hr | - Internet - Laptop/Computer - White board - Projector screen - Multimedia projector | Classroom |

## Module: Create Human Resources Management Plan

Objective:

After the completion of this competency standard, the Trainee will be able to develop a plan for a telecommunications project, including assessing project requirements and planning for all stages to completion and final documentation. It applies to individuals who are likely to have responsibility for the provision of installations, maintenance, upgrades and new services.

Duration:  30 Hours		Theory:  12 Hours			Practice:  18 Hours          	        Credit Hours:

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Prepare project management plan | Trainee will be able to: - Evaluate and assess project brief and related documents. - Produce document on project tasks and associated timelines, including installation processes and test requirements. - Assess and produce document on resource requirements to assist allocation of appropriate resources. - Produce training plan assessing training needs and associated timelines for efficient project implementation. - Determine and document budgetary requirements. - Discuss roles of all identified parties associated with project to ensure their involvement. - Produce project verification document, including monitoring and control processes, and review processes such as quality audits. - Consult with all relevant parties prior to finalizing draft plan and make changes as appropriate. | - Preparation of Project Plans and their management - Evaluation and assessment of project related documentation and tasks - Project Budget preparation | Theory- 03 Hr Practical- 03Hrs Total- 06 Hr | - Internet - Laptop/Computer - White board - Projector screen - Multimedia projector | Classroom |
| LU2. Develop and evaluate management plan | Trainee will be able to: - Produce preliminary plan for consultation, including identified factors that may impact on realization of project and observance of relevant legislation, codes, regulation and standards. - Consult with client and clarify any amendments. - Develop final plan with recommendations. | - Development and evaluation of project management plans - Client dealing for clarification and amendments in project plans - Finalization of plans | Theory- 03 Hr Practical- 03Hrs Total- 06 Hr | - Internet - Laptop/Computer - White board - Projector screen - Multimedia projector | Classroom |
| LU3. Communicate project information | Trainee will be able to: - Produce and document final plan to include implementation details and training needs. - Present plan to client and obtain sign off. | - Presentation of Project plan to client | Theory- 03 Hr Practical- 03Hrs Total- 06 Hr | - Internet - Laptop/Computer - White board - Projector screen - Multimedia projector | Classroom |
| LU4. Contribute to assessing effectiveness of communication | Trainee will be able to: - Assist in ongoing review of project outcomes to determine effectiveness of communications-management activities. - Report communications-management issues and responses to higher project authorities for application of lessons learned to future projects. | - Review of project outcomes and asses effectiveness of business communication | Theory- 03 Hr Practical- 09Hrs Total- 12 Hr | - Internet - Laptop/Computer - White board - Projector screen - Multimedia projector | Classroom |

## Module: Plan And Implement Business-To-Business Marketing

Objective:

After the completion of this competency standard, the Trainee will be able to plan and implement business-to-business (B2B)
marketing. It applies to individuals who work in a supervisory capacity in a team environment, who possess a sound theoretical knowledge base and demonstrate a range of managerial skills to ensure business activities are conducted effectively. In this role, individuals may work in small, medium or large enterprises across a variety of industries.

Duration:  30 Hours		Theory:  12 Hours			Practice:  18 Hours          	        Credit Hours:

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Identify and evaluate business -to-business marketing strategies | Trainee will be able to: - Identify B2B markets in an industry context - Research characteristics of business markets in an industry context - Identify and analyze factors influencing business buyers in an industry context - Analyze the business buying process and its implications in the industry context - Research and analyze a range of B2B marketing strategies appropriate for the organization - Identify key personnel in buying decision process in the organization s business markets | - Identification of Business to Business markets | Theory- 03 Hr Practical- 03Hrs Total- 06 Hr | - Internet - Laptop/Computer - White board - Projector screen - Multimedia projector | Classroom |
| LU2. Select business-to business marketing strategies | Trainee will be able to: - Analyze trends within business markets and identify B2B marketing opportunities for the organization - Identify and analyze success of the organization s previous B2B marketing strategies - Select most appropriate B2B marketing strategies and activities that fit with organization s strategic & marketing plans | - Preparation of business to business strategies | Theory- 03 Hr Practical- 03Hrs Total- 06 Hr | - Internet - Laptop/Computer - White board - Projector screen - Multimedia projector | Classroom |
| LU3. Plan and develop business-to-business marketing activities | Trainee will be able to: - Record B2B marketing objectives and purpose - Calculate costs of B2B marketing activities with assistance of appropriate personnel - Select methods to report and measure effectiveness of B2B marketing activities - Assign responsibilities to team members for B2B marketing activities - Record B2B marketing plan and present to relevant stakeholders - Assemble required resources to implement B2B marketing plan | - Plan the business to business strategies | Theory- 03 Hr Practical- 03Hrs Total- 06 Hr | - Internet - Laptop/Computer - White board - Projector screen - Multimedia projector | Classroom |
| LU4. Implement and monitor business-to business marketing plan | Trainee will be able to: - Schedule work on each B2B marketing campaign element, according to lead times required and marketing plan - Brief staff and suppliers on their budgets, timelines, roles and responsibilities, and legal and ethical requirements - Plan implementation of B2B marketing activities according to marketing plan - Identify & use evaluation criteria and evaluation methods to determine effectiveness of marketing plan - Analyze success indictors of B2B marketing plan and record performance according to organizational reporting requirements | - Trends in B2B marketing, corporate sector and business sector and preparation of plans | Theory- 03 Hrs Practical- 09Hrs Total- 12 Hrs | - Internet - Laptop/Computer - White board - Projector screen - Multimedia projector | Classroom |

## Module: Manage  Finances

Objective:

After the completion of this competency standard, the Trainee will be able to develop, implement and monitor a personal
budget in order to plan regular savings and manage debt effectively.

Duration:  30 Hours		Theory:  12 Hours			Practice:  18 Hours          	        Credit Hours: 0.5

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Develop a personal budget | Trainee will be able to: - Calculate current living expenses using available information to prepare a personal budget. - Keep a record of all income and expenses for a short period of time to help estimate ongoing expenses. - Subtract total expenses from total income to determine a surplus or deficit budget for the specified period. - Find reasons for a deficit budget and ways to reduce expenditure identified. - Identify ways to increase income, if possible | - Preparing cash flow statement - Basic concepts of Time value of money - Understanding of current and fixed assets - Understanding of elements of equity and liabilities (long term and short term) - Identification of available financial resources - Step by step budget plan preparation - Budget review techniques | Theory- 4 Hrs Practical- 06Hrs Total- 10 Hrs | - Internet - Laptop/Computer - White board - Projector screen - Multimedia projector | Classroom |
| LU2. Develop longer term personal budget | Trainee will be able to: - Analyze income and expenditure and set longer term personal, work and financial goals. - Develop a longer-term budget based on the outcomes of short-term budgeting, and adjust to meet living, work and future career requirements. - Identify obstacles that might affect finances such as job loss, sickness or unexpected expenses contingency savings - Formulate a regular savings plan based on budget, using secure savings products and services. - Monitor expenditure against budget and identify areas of possible expenditure saving | - Develop a personal budget based on analysis of expenditure and income - Monitor expenditure for a period of up to 2 weeks. | Theory- 4 Hrs Practical- 06Hrs Total- 10 Hrs | - Internet - Laptop/Computer - White board - Projector screen - Multimedia projector | Classroom |
| LU3. Identify ways to maximize future finances | Trainee will be able to: - Determine sources and ways to maximize personal income, including from work, investments or available government payments/allowances. - Get further education or training to maintain or improve future income. - Identify the need for debt to finance living and other expenses, and determine the appropriate levels of debt and repayment. - Consolidate existing debt, where possible, to minimize interest costs and fees. - Seek professional money management services, where available, to ensure financial plans are effective and achievable | - Formulate goals and identify financial contingency plans | Theory- 4 Hrs Practical- 06Hrs Total- 10 Hrs | - Internet - Laptop/Computer - White board - Projector screen - Multimedia projector | Classroom |

## Module: Develop A Project Management Plan

Objective:

After the completion of this competency standard, the Trainee will be able to develop a plan for a telecommunications project, including assessing project requirements and planning for all stages to completion and final documentation. It applies to individuals who are likely to have responsibility for the provision of installations, maintenance, upgrades and new services.

Duration:  30 Hours		Theory:  12 Hours			Practice:  18 Hours          	        Credit Hours:

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Prepare project management plan | Trainee will be able to: - Evaluate and assess project brief and related documents. - Produce document on project tasks and associated timelines, including installation processes and test requirements. - Assess and produce document on resource requirements to assist allocation of appropriate resources. - Produce training plan assessing training needs and associated timelines for efficient project implementation. - Determine and document budgetary requirements. - Discuss roles of all identified parties associated with project to ensure their involvement. - Produce project verification document, including monitoring and control processes, and review processes such as quality audits. - Consult with all relevant parties prior to finalizing draft plan and make changes as appropriate. | - Preparation of Project Plans and their management - Evaluation and assessment of project related documentation and tasks - Project Budget preparation | Theory- 03 Hrs Practical- 03Hrs Total- 06 Hrs | - Internet - Laptop/Computer - White board - Projector screen - Multimedia projector | Classroom |
| LU2. Develop and evaluate management plan | Trainee will be able to: - Produce preliminary plan for consultation, including identified factors that may impact on realization of project and observance of relevant legislation, codes, regulation and standards. - Consult with client and clarify any amendments. - Develop final plan with recommendations. | - Development and evaluation of project management plans - Client dealing for clarification and amendments in project plans - Finalization of plans | Theory- 03 Hrs Practical- 03Hrs Total- 06 Hrs | - Internet - Laptop/Computer - White board - Projector screen - Multimedia projector | Classroom |
| LU3. Communicate project information | Trainee will be able to: - Produce and document final plan to include implementation details and training needs. - Present plan to client and obtain sign off. | - Presentation of Project plan to client | Theory- 03 Hrs Practical- 03Hrs Total- 06 Hrs | - Internet - Laptop/Computer - White board - Projector screen - Multimedia projector | Classroom |
| LU4. Contribute to assessing effectiveness of communication | Trainee will be able to: - Assist in ongoing review of project outcomes to determine effectiveness of communications-management activities. - Report communications-management issues and responses to higher project authorities for application of lessons learned to future projects. | - Review of project outcomes and asses effectiveness of business communication | Theory- 03 Hrs Practical- 09Hrs Total- 12 Hrs | - Internet - Laptop/Computer - White board - Projector screen - Multimedia projector | Classroom |

## Module: Develop A Sales Plan

Objective:

After the completion of this competency standard, the Trainee will be able to develop a sales plan for a product or service for a team covering a specified sales territory based on strategic objectives and in accordance with established performance targets. It applies to individuals working in a supervisory or managerial sales role who develop a sales plan for a product or service.

Duration:  30 Hours		Theory:  12 Hours			Practice: 18 Hours          	        Credit Hours:

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Identify organizational strategic direction | Trainee will be able to: - Obtain and analyze assessment of market needs and strategic planning documents - Review previous sales performance and successful approaches to identify factors affecting performance - Analyze information on market needs, new opportunities, customer profiles and requirements as a basis for decision making | - Assessment of market needs and strategic planning documents preparation - Knowledge of business opportunities, customer profiles and decision making | Theory- 02 Hrs Practical- 3Hrs Total- 05 Hrs | - Internet - Laptop/Computer - White board - Projector screen - Multimedia projector | Classroom |
| LU2. Establish performance targets | Trainee will be able to: - Determine practical and achievable sales targets - Establish realistic time lines for achieving targets - Determine measures to allow for monitoring of performance - Ensure objectives of the sales plan and style of the campaign are consistent with organizational strategic objectives and corporate image | - Statistical techniques for analyzing sales and market trends | Theory- 02 Hrs Practical- 3Hrs Total- 05 Hrs | - Internet - Laptop/Computer - White board - Projector screen - Multimedia projector | Classroom |
| LU3. Develop a sales plan for a product | Trainee will be able to: - Determine approaches to be used to meet sales objectives - Identify additional expertise requirements and allocate budgetary resources accordingly - Identify risks and develop risk controls - Develop advertising and promotional strategy for product - Identify appropriate distribution channels for product - Prepare a budget for the sales plan - Present documented sales plan to appropriate personnel for approval | - Principles and techniques for selling Sources of internal and external information - Identification of organizational strategic direction and developing a product sales plan - Preparation of sales plan and its approval | Theory- 02 Hrs Practical- 3Hrs Total- 05 Hrs | - Internet - Laptop/Computer - White board - Projector screen - Multimedia projector | Classroom |
| LU4. Identify support requirements | Trainee will be able to: - Identify and acquire staff resources to implement sales plan - Develop an appropriate selling approach - Train staff in the selling approach selected - Develop and assess staff knowledge of product to be sold | - Resources requirement for sales plan | Theory- 02 Hrs Practical- 3Hrs Total- 05 Hrs | - Internet - Laptop/Computer - White board - Projector screen - Multimedia projector | Classroom |
| LU5. Monitor and review sales plan | Trainee will be able to: - Monitor implementation of the sales plan - Record data measuring performance versus sales targets - Make adjustments to sales plan as required to ensure required results are obtained | - Principles, monitoring and techniques for selling Sources of internal and external information | Theory- 04 Hrs Practical- 06Hrs Total- 10 Hrs | - Internet - Laptop/Computer - White board - Projector screen - Multimedia projector | Classroom |

## Module: Conduct research for Customer Needs & Satisfaction

Objective:

After the completion of this competency standard, the Trainee will be able to manage an ongoing relationship with a customer
over a period of time. This includes helping customers articulate their needs and managing networks to ensure customer needs are addressed. It applies to individuals who are expected to have detailed product knowledge in order to recommend customized solutions. In this role, individuals would be expected to apply organizational procedures and be aware of, and apply as appropriate, broader factors involving ethics, industry practice and relevant government policies and regulations.

Duration:  30 Hours		Theory:  12Hours			Practice:  18Hours          	        Credit Hours:

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Assist customer to articulate needs | Trainee will be able to: - Ensure customer needs are fully explored, understood and agreed - Explain and match available services and products to customer needs - Identify and communicate rights and responsibilities of customers to the customer as appropriate | - Explain organizational procedures and standards for establishing and maintaining customer service relationships. - Explain consumer rights and responsibilities | Theory- 03 Hrs Practical- 03Hrs Total- 06 Hrs | - Internet - Laptop/Computer - White board - Projector screen - Multimedia projector | Classroom |
| LU2. Satisfy complex customer needs | Trainee will be able to: - Explain possibilities for meeting customer needs - Assist customers to evaluate service and/or product options to satisfy their needs - Determine and prioritize preferred actions - Identify potential areas of difficulty in customer service delivery and take appropriate actions in a positive manner | - Describe ways to establish effective regular communication with customers - Variations within a limited product and service range. | Theory- 03 Hrs Practical- 03Hrs Total- 06 Hrs | - Internet - Laptop/Computer - White board - Projector screen - Multimedia projector | Classroom |
| LU3. Manage networks to ensure customer needs are addressed | Trainee will be able to: - Establish effective regular communication with customers - Establish, maintain and expand relevant networks to ensure appropriate referral of customers to products and services from within and outside the organization - Ensure procedures are in place to ensure that decisions about targeting of customer services are based on up-to-date information about the customer and the products and services available - Ensure procedures are put in place to ensure that referrals are based on the matching of the assessment of customer needs and availability of products and services - Maintain records of customer interaction in accordance with organizational procedures | - Outline details of products or services including with reference to possible alternative products and services | Theory- 03 Hrs Practical- 03Hrs Total- 06 Hrs | - Internet - Laptop/Computer - White board - Projector screen - Multimedia projector | Classroom |
| LU4. Convert customer enquiries into sales | Trainee will be able to: - Use information provided by customers or accessed from customer relationship management (CRM) system to identify any need - Identify suitable products /services to meet needs - Make convincing sales pitches to customers following standard scripts - Handle customer queries, objections and rebuttals following standard scripts - Adapt your approach and style to customer preferences, within limits of your competence and authority - Refer issues outside your area of competence and authority to appropriate people, following your organization’s procedures - Identify and act on opportunities to up-sell or cross-sell other products/services to customers - Confirm customer wishes and needs in order to close sales - Obtain required financial information from customers, following your organization’s procedures - Complete your organization’s post-sales procedures in order to complete/fulfill sales - Comply with relevant standards, policies, procedures and guidelines when converting customer enquiries into sales | - Organizational standards for maintaining customer relationships to convert convincing sales pitches to customers need | Theory- 03 Hrs Practical- 09Hrs Total- 12 Hrs | - Internet - Laptop/Computer - White board - Projector screen - Multimedia projector | Classroom |

## Module: Identify & resolve Problems

Objective: After the completion of this competency standard, the Trainee will be able to apply the process of problem solving for problems beyond those associated directly with the process unit.

Duration:  30 Hours		Theory:  12 Hours			Practice:  18 Hours          	        Credit Hours:

| Learning Unit | Learning Outcomes | Learning Elements | Duration | Materials Required | Learning Place |
| --- | --- | --- | --- | --- | --- |
| LU1. Analyze the problem | Trainee will be able to: - Evaluate issues/concerns are evaluated based on data gathered. - Identify possible causes of problem are identified within the area of responsibility as based on experience and the use of problem solving tools/analytical techniques. - Develop possible cause statements based on findings. - Use analogies to support reasoning. - Identify cause and effects are identified based on the criteria or information provided to support reasoning. | - Explain Analytical skills and broad knowledge of diagnostic tools - Explain broad knowledge of the client business domain - Elaborate broad understanding of organizational systems and functions Outline communication skills according to the type of audience - Explain the methods of questioning and active listening employed to clarify general information | Theory- 02 Hrs Practical- 03Hrs Total- 05Hrs | - Internet - Laptop/Computer - White board - Projector screen - Multimedia projector | Classroom |
| LU2. Identify possible solutions | Trainee will be able to: - Consider all possible options for solution of the problem in accordance with safety and operating procedures. - Determine strengths and weaknesses of possible options. - Take corrective action to solve the problem and determine its possible future causes. - Analyze past experience - Provide samples to support generalization. - Implement simulations as needed. | - Describe the ways to develop critical thinking - Explain methods and techniques for decision making within a limited range of options - Describe general customer service skills | Theory- 02 Hrs Practical- 03Hrs Total- 05Hrs | - Internet - Laptop/Computer - White board - Projector screen - Multimedia projector | Classroom |
| LU3. Recommend solution to higher management | Trainee will be able to: - Prepare report or documentation. - Present recommendations to appropriate personnel. - Follow up recommendations, if required. | - Explain the ways to improve teamwork in reference to personal responsibility | Theory- 02 Hrs Practical- 03Hrs Total- 05Hrs | - Internet - Laptop/Computer - White board - Projector screen - Multimedia projector | Classroom |
| LU4. Implement solution | Trainee will be able to: - Identify measurable objectives - Identify resource needs - Prepare timelines in accordance with plan. | - Develop teams and individuals and ask for individual feedback | Theory- 02 Hrs Practical- 03Hrs Total- 05Hrs | - Internet - Laptop/Computer - White board - Projector screen - Multimedia projector | Classroom |
| LU5. Evaluate / monitor results and outcome | Trainee will be able to: - Identify processes and improvements based on evaluative assessment of problem. - Prepare recommendations and submit to superiors | - The evidence should integrate employability skills with workplace tasks and job roles and verify competency is able to be transferred to other situations | Theory- 04 Hrs Practical- 06Hrs Total- 10Hrs | - Internet - Laptop/Computer - White board - Projector screen - Multimedia projector | Classroom |

# 4.	List of Tools and Equipment

(FOR A CLASS OF 25 STUDENTS)

| Name of Trade |  | Instrumentation Technology |  |
| --- | --- | --- | --- |
| Sr. # |  | Description |  | Quantity |
| 1. |  | Steel-toed footwear, |  | 25 Nos. |
| 2. |  | hard hat, |  | 25 Nos. |
| 3. |  | safety gloves, |  | 25 Sets |
| 4. |  | appropriate safety glasses, |  | 25 Nos. |
| 5. |  | high visibility vest, |  | 25 Nos. |
| 6. |  | hearing protection, |  | 25 Nos. |
| 7. |  | respirators, masks, |  | 25 Nos. |
| 8. |  | De-electric boots and gloves for protection from electrical shock. |  | 25 Nos. |
| 9. |  | First Aid Box (Include all necessary bandages and medicines) |  | 02 Sets |
| 10. |  | fire extinguishers, |  | 04 Nos. |
| 11. |  | fire blankets, |  | 04 Nos. |
| 12. |  | Sand Bucket with stand |  | 02 Sets |
| 13 |  | basic tools, such as grease gun, air pump etc |  | 10 Sets |
| 14 |  | Allen Key |  | 10 Sets |
| 15 |  | Work Bench (able to hold 06 bench wises) |  | 05 Nos. |
| 16 |  | Bench Wise |  | 30 Nos. |
| 17. |  | hammer, |  | 05 each size |
| 18 |  | screwdrivers, |  | 05 each size |
| 19. |  | pliers, |  | 05 each size |
| 20. |  | self-locking pliers, |  | 05 each size |
| 21. |  | adjustable wrench, |  | 05 each size |
| 22. |  | assorted other wrenches, measuring tape(100m) |  | 05 each size |
| 23. |  | Basic supplies, such as grease, oil, window cleaner, rags, ice scraper, whisk broom. |  | 05 each |
| 24. |  | Color-code cards, utility documentation. Logbooks Service Manuals, OHS Regulation, |  | 10 sets |
| 25 |  | Fix Safe guards |  | 04 Nos |
| 26 |  | Moveable Safe guards |  | 04 N0s. |
| 27 |  | Solder |  | 04 N0s. |
| 28 |  | Wire stripper |  | 04 N0s. |
| 29 |  | Nose plier |  | 04 N0s. |
| 30 |  | Solder wire |  | 04 N0s. |
| 31 |  | Insulation remover |  | 04 N0s. |
| 32 |  | Flat File |  | 25 Nos. |
| 33 |  | Half round file |  | 10 Nos. |
| 34 |  | Round File |  | 10 Nos. |
| 35 |  | Mild Steel Sheet (4 x 8 ft) |  | 02 Nos |
| 36 |  | Mild Steel round work piece (Dia 30) |  | 10 ft |
| 37 |  | Mild Steel flat bar (W:50xL:100xT:20mm) |  | 25 Nos. |
| 38 |  | GI pipe 1”, 2” |  | 20 Feet each |
| 39 |  | Lathe machine (5 ft) |  | 02 Nos |
| 40 |  | Revolving center |  | 02 Nos |
| 41 |  | Dead center |  | 02 Nos |
| 42 |  | Draw bar |  | 02 Nos |
| 43 |  | Face plate |  | 02 Nos |
| 44 |  | Knurling Tool |  | 02 Nos |
| 45 |  | HSS Tool bit |  | 10 Nos |
| 46 |  | Pedestal Drill machine |  | 02 Nos |
| 47 |  | Hand Grinder |  | 02 Nos |
| 48 |  | Drill bits |  | 02 Set |
| 49 |  | Drill shank |  | 02 Set |
| 50 |  | Drill chuck |  | 02 Nos |
| 51 |  | Counter sink tool |  | 02 Nos |
| 52 |  | Counter bore tool |  | 02 Nos |
| 53 |  | Machine reamer set |  | 02 Set |
| 54 |  | Hand Reamer set |  | 02 Set |
| 55 |  | Universal Milling machine |  | 01 Nos. |
| 56 |  | Machine wise |  | 01 Nos. |
| 57 |  | Tail stock with dead center |  | 01 Set |
| 58 |  | Indexing head |  | 01 Nos. |
| 59 |  | Indexing plate set |  | 01 Set |
| 60 |  | Gear cutting tool set |  | 01 Set |
| 61 |  | T-slot cutting tool |  | 02 Nos. |
| 62 |  | Woodruff cutter |  | 02 Nos |
| 63 |  | End mill cutter |  | 02 Nos |
| 64 |  | Portable welding machine 3 Phase 300 Amps. |  | 02 Nos, |
| 65 |  | Welding table |  | 03 Nos, |
| 66 |  | Toggle |  | 02 Nos, |
| 67 |  | Leather gloves |  | 01 Nos, |
| 68 |  | Welding apron |  | 05 Nos, |
| 69 |  | Oxygen Cylinder with protector cap |  | 01 Nos, |
| 70 |  | Acetylene Cylinder with protector cap |  | 01 Nos. |
| 71 |  | Welding Torch |  | 01 Nos. |
| 72 |  | Welding Tip (Nozzle) |  | 01 Set |
| 73 |  | Pressure Regulators |  | 01 Nos. |
| 74 |  | Hose & Hose fittings |  | 30 ft. |
| 75 |  | Filler Rod |  | 01 Box |
| 76 |  | Flux Material |  | 500 gm. |
| 77 |  | Surface Table |  | 01 Nos. |
| 78 |  | - Surface plate. |  | 02 Nos. |
| 79 |  | - Steps and collars. |  | 02 Set |
| 80 |  | Folding rule |  | 02 Nos. |
| 81 |  | Combination set. |  | 02 Nos. |
| 82 |  | Scriber. |  | 02 Nos. |
| 83 |  | Tri square. |  | 02 Nos. |
| 84 |  | Divider. |  | 02 Nos. |
| 85 |  | Spirit level |  | 02 Nos. |
| 86 |  | Radius gauge |  | 02 Nos. |
| 87 |  | Filler gauge |  | 02 Nos. |
| 88 |  | Plug gauge |  | 02 Set |
| 89 |  | Ring gauge |  | 02 Set |
| 90 |  | Outside micrometer |  | 02 Nos. |
| 91 |  | Inside micrometer |  | 02 Nos. |
| 92 |  | Depth micrometer. |  | 02 Nos. |
| 93 |  | Bevel protractor. |  | 02 Nos. |
| 94 |  | Vernier Bevel protractor. |  | 02 Nos. |
| 95 |  | Dial protractor. |  | 02 Nos. |
| 96 |  | Sine bar. |  | 02 Nos. |
| 97 |  | Sine block |  | 02 Nos. |
| 98 |  | Sine plate. |  | 02 Nos. |
| 99 |  | Sine table. |  | 02 Nos. |
| 100 |  | Sine center. |  | 02 Nos. |
| 101 |  | Gear tooth caliper |  | 02 Nos. |
| 102 |  | Gear Inspection machine. |  | 01 Nos. |
| 103 |  | Gear testing machine. |  | 01 Nos. |
| 104 |  | Parkinson Gear Tester. |  | 01 Nos. |
| 105 |  | Multi meters |  | 02 Nos. |
| 106 |  | Resistors of different values |  | 04 Nos. (Each) |
| 107 |  | Small pieces of copper wire |  | 01 roll |
| 108 |  | Breadboard. |  | 04 Nos. |
| 109 |  | Connecting wires |  | 02 Rolls |
| 110 |  | Rheostat. |  | 04 Nos. |
| 111 |  | Incandescent lamp of different watts. |  | 04 Nos. |
| 112 |  | Voltmeter |  | 04 Nos. |
| 113 |  | Ampere meter |  | 04 Nos. |
| 114 |  | Multimeter |  | 04 Nos. |
| 115 |  | Combination plier |  | 04 Nos. |
| 116 |  | Phase tester |  | 04 Nos. |
| 117 |  | Energy meter |  | 04 Nos. |
| 118 |  | Thin coated copper wire. |  | 02 Rolls |
| 119 |  | Dry cell battery |  | 04 Nos. |
| 120 |  | Horse shoe magnet. |  | 04 Nos. |
| 121 |  | Transformer. |  | 02 Nos. |
| 122 |  | Compass needle. |  | 02 Nos. |
| 123 |  | Bar Magnet. |  | 04 Nos. |
| 124 |  | Galvanometer. |  | 02 Nos. |
| 125 |  | Capacitors, 4.7 μF |  | 04 Nos. |
| 126 |  | Capacitors, 10, 100 & 8.2 μF. |  | 04 Nos. (each) |
| 127 |  | Voltmeter. |  | 04 Nos. |
| 128 |  | D.C. power supply unit. |  | 02 Nos. |
| 129 |  | Bridging plugs. |  | 04 Nos. |
| 130 |  | Connecting leads. |  | 04 |
| 131 |  | Multimeter. (With option of Capacitance measurement) |  | 04 Nos. |
| 132 |  | D.C. power supply unit |  | 02 Nos. |
| 133 |  | Variable D.C powers supply 0-30 Voltas |  | 02 |
| 134 |  | Capacitor (Sanyo Operated Volts 6.3 V, SE 30) |  | 02 Nos. (each) |
| 135 |  | D.C Analog or Preferably Digital voltmeter Range 0-50V. |  | 02 Nos. (each) |
| 136 |  | L.E.D (3 to 9 V) |  | 02 Nos. (each) |
| 137 |  | Diode |  | 04 Nos. |
| 138 |  | Resistor |  | 04 Nos. |
| 139 |  | Bread board |  | 04 Nos. |
| 140 |  | Digital Multimeter |  | 04 Nos. |
| 141 |  | Oscilloscope Diode |  | 04 Nos. |
| 142 |  | Wire cutter |  | 04 Nos. |
| 143 |  | Wire stripper |  | 04 Nos. |
| 144 |  | Zener diode |  | 04 Nos. |
| 145 |  | Light Emitting diode |  | 04 Nos. |
| 146 |  | Soldering iron |  | 04 Nos. |
| 147 |  | FET Transistor |  | 04 Nos. |
| 148 |  | MOSFET Transistor |  | 04 Nos. |
| 149 |  | UJT |  | 04 Nos. |
| 150 |  | SCR |  | 04 Nos. |
| 151 |  | Breadboard or trainer |  | 01 Nos. |
| 152 |  | DIAC |  | 04 Nos. |
| 153 |  | TRIAC |  | 04 Nos. |
| 154 |  | Functions Generator |  | 01 Nos. |
| 155 |  | Op-Amp |  | 04 Nos. |
| 156 |  | White chart paper |  | 100 Nos. |
| 157 |  | Graphics and drawing sheets |  | 200 Nos. |
| 158 |  | Pencil |  | 30 Nos. |
| 159 |  | Eraser |  | 30 Nos. |
| 160 |  | Sharpener |  | 30 Nos. |
| 161 |  | Tee |  | 25 Nos. |
| 162 |  | Drawing Board |  | 25 Nos. |
| 163 |  | Set Square |  | 25 Nos. |
| 164 |  | Geometry set |  | 25 Nos. |
| 165 |  | French Curve |  | 25 Nos. |
| 166 |  | Printer |  | 01 Nos. |
| 167 |  | Multimedia projector |  | 01 Nos. |
| 168 |  | Computer |  | 25 Nos. |
| 169 |  | CAD software |  | 01 |
| 170 |  | IDE Software (C Language) |  | 01 |
| 171 |  | Power supply |  | 02 |
| 172 |  | AND gate |  | 02 Nos. |
| 173 |  | OR gate |  | 02 Nos. |
| 174 |  | NAND gate |  | 02 Nos. |
| 175 |  | XOR gate |  | 02 Nos. |
| 176 |  | NOR gate |  | 02 Nos. |
| 177 |  | NOT gate |  | 02 Nos. |
| 178 |  | Tool set |  | 02 Nos. |
| 179 |  | Bread board |  | 02 Nos. |
| 180 |  | Wire 0.5 mm |  | 02 Rolls |
| 181 |  | Digital clock |  | 04 Nos. |
| 182 |  | 555 IC |  | 02 |
| 183 |  | - Connecting leads. |  | 02 Rolls |
| 184 |  | Coil |  | 02 Rolls |
| 185 |  | - Armature |  | 02 Nos. |
| 186 |  | - Winding machine |  | 02 Nos. |
| 187 |  | - Insolating paper |  | 02 Nos. |
| 188 |  | - Carbon brushes |  | 04 Nos. |
| 189 |  | - Soldering wire |  | 02 Rolls |
| 190 |  | Stator frame |  | 02 Nos. |
| 191 |  | - Spring balance |  | 02 Nos. |
| 192 |  | - Ohm meter |  | 02 Nos. |
| 193 |  | - DC compound motor |  | 02 Nos. |
| 194 |  | Electro dynamo meter |  | 02 Nos. |
| 195 |  | - AC series motor |  | 02 Nos. |
| 196 |  | - Reluctance motor |  | 02 Nos. |
| 197 |  | - Hysteresis motor |  | 02 Nos. |
| 198 |  | - Stepper motor |  | 02 Nos. |
| 199 |  | - Servo motor |  | 02 Nos. |
| 200 |  | Control circuit |  | 02 Nos. |
| 201 |  | Temperature sensors. |  | 02 Nos. |
| 202 |  | IDE Software (C++ Language) |  | 01 |
| 203 |  | Sound sensors. |  | 02 Nos. |
| 204 |  | Proximity sensors. |  | 02 Nos. |
| 205 |  | Pressure sensors. |  | 02 Nos. |
| 206 |  | Light sensors. |  | 02 Nos. |
| 207 |  | Position sensors. |  | 02 Nos. |
| 208 |  | Voltage sensors. |  | 02 Nos. |
| 209 |  | Current sensors. |  | 02 Nos. |
| 210 |  | Vision sensors. |  | 02 Nos. |
| 211 |  | Microcontroller |  | 02 Nos. |
| 212 |  | - Arduino Uno |  | 02 Nos. |
| 213 |  | - Arduino Mega |  | 02 Nos. |
| 214 |  | Raspberry pi |  | 01 |
| 215 |  | Software of various microcontroller |  | 01 |
| 216 |  | - Links |  | 02 |
| 217 |  | - Joints |  | 02 |
| 218 |  | - End effectors |  | 02 |
| 219 |  | - Nuts |  | 02 Dozen (each size) |
| 220 |  | - Bolts |  | 02 Dozen (each size) |
| 221 |  | Screws |  | 02 Dozen (each size) |
| 222 |  | - Grippers |  | 02 Nos. |
| 223 |  | Conveyor belt |  | 02 Nos. |
| 224 |  | Sensor |  | 02 Nos. |
| 225 |  | Actuator |  | 02 Nos. |
| 226 |  | - Jumpers |  | 02 Nos. |
| 227 |  | - SD card |  | 02 Nos. |
| 228 |  | - LED |  | 02 Nos. |
| 229 |  | - Power Adaptor |  | 02 Nos. |
| 230 |  | HDMI cable |  | 02 Rolls |
| 231 |  | Humidity sensor |  | 02 Nos. |
| 232 |  | Robotic fastening tool kit |  | 02 Nos. |
| 233 |  | Measuring tool kit |  | 02 Nos. |
| 234 |  | Manipulator links and joints |  | 02 Set |
| 235 |  | Complete design of the Robot |  | 02 Nos. |
| 236 |  | Complete design of the sensor assembly |  | 02 Nos. |
| 237 |  | Robot operating software |  | 01 |
| 238 |  | Camera |  | 02 Nos. |
| 239 |  | Actuator (Robotics Arm) |  | 02 Nos. |
| 240 |  | Potentiometer |  | 02 Nos. |
| 241 |  | Calculator |  | 02 Nos. |
| 242 |  | - Basic ROV components |  | 02 Nos. |
| 243 |  | - Complete design of the ROV |  | 02 Nos. |
| 244 |  | - Remotely Operated Robot (ROR) Trainer |  | 01 Nos. |
| 245 |  | calibration standards measuring devices |  | 01 Nos. |
| 246 |  | - calibration angles measuring devices |  | 01 Nos. |
| 247 |  | - Robots geometry/Length angle alignment measuring device |  | 01 Nos. |
| 248 |  | Robotic Arm |  | 02 Nos. |
| 249 |  | - OHS Standard Manual |  | 01 |
| 250 |  | - friction Measuring Device |  | 01 Nos. |
| 251 |  | Model error detection devices |  | 01 Nos. |
| 252 |  | Strain gauge |  | 02 Nos. |
| 253 |  | Capacitive pressure sensor |  | 02 Nos. |
| 254 |  | Electromagnetic pressure sensor |  | 02 Nos. |
| 255 |  | Optical pressure sensor |  | 02 Nos. |
| 256 |  | Force balancing |  | 02 Nos. |
| 257 |  | Pressure sensor |  | 02 Nos. |
| 258 |  | Lm35 ic solid state temperature sensor |  | 02 Nos. |
| 259 |  | Negative temperature coefficient (ntc) thermistor |  | 02 Nos. |
| 260 |  | Resistance temperature detector (rtd) |  | 02 Nos. |
| 261 |  | Thermocouple |  | 02 Nos. |
| 262 |  | Semiconductor-based sensors |  | 02 Nos. |
| 263 |  | Retro-reflective sensors |  | 02 Nos. |
| 264 |  | Diffuse reflection sensors |  | 02 Nos. |
| 265 |  | BLDC motor |  | 02 Nos. |
| 266 |  | Solenoid valve |  | 02 Nos. |
| 267 |  | Robot Installation Manual |  | 02 Nos. |
| 268 |  | - Error detection tool kit |  | 02 Nos. |
| 269 |  | - Inclinometer |  | 02 Nos. |
| 270 |  | - Mechanical strength measuring tools |  | 01 Nos. |
| 271 |  | - Coordinate calibration tools |  | 01 Nos. |
| 272 |  | Tool Box |  | 02 Nos. |
| 273 |  | - ISO9283 |  | 01 |
| 274 |  | - Vibration sensors |  | 02 Nos. |
| 275 |  | - Jumpers |  | 02 Nos. |
| 276 |  | Turbo C, C++, Code block |  | 01 |
| 277 |  | Line following trainer kit |  | 01 |
| 278 |  | Jumper wires |  | 02 Rolls |
| 279 |  | Chassis for line following robot |  | 01 |
| 280 |  | Bluetooth transceiver |  | 02 Nos. |
| 281 |  | Smart phone |  | 02 Nos. |
| 282 |  | Relay control board |  | 02 Nos. |
| 283 |  | WiFi module |  | 01 Nos. |
| 284 |  | - Door Lock Mechanism |  | 01 Nos. |
| 285 |  | - Controller (Arduino) |  | 01 Nos. |
| 286 |  | - Android Application (Dabble) |  | 01 Nos. |
| 287 |  | - Security System Mechanism |  | 01 Nos. |
| 288 |  | Water level indication Mechanism |  | 01 Nos. |
| 289 |  | - 28 pin DIP IC Socket |  | 01 Nos. |
| 290 |  | - 16MHz Crystal Oscillator |  | 01 Nos. |
| 291 |  | - USB to Serial adapter |  | 01 Nos. |
| 292 |  | - ICs |  | 01 Nos. |
| 293 |  | - Mini Servo Motor |  | 01 Nos. |
| 294 |  | - DVD/CD Drives |  | 01 Nos. |
| 295 |  | - PCB Circuit Board |  | 01 Nos. |
| 296 |  | - 2pins Screw Terminal |  | 01 Nos. |
| 297 |  | 2x 4 pins Screw Terminal Connector |  | 01 Nos. |
| 298 |  | Mobile robot base |  | 01 Nos. |
| 299 |  | Power diode |  | 01 Nos. |
| 300 |  | Resistor of 1 K ohms |  | 01 Nos. |
| 301 |  | Centre tapped transformer |  | 01 Nos. |
| 302 |  | Inductor 2mH |  | 01 Nos. |
| 303 |  | Resistor of 10 K & 100 K ohms |  | 01 Nos. |
| 304 |  | Power MOSFET IRF 840 |  | 01 Nos. |
| 305 |  | Power Diode 1N4007 |  | 01 Nos. |
| 306 |  | DC Source 12V |  | 01 Nos. |
| 307 |  | Resistor 10K |  | 01 Nos. |
| 308 |  | - IC 7404 |  | 01 Nos. |
| 309 |  | Controller IC |  | 02 Nos. |
| 310 |  | Hydraulic bench |  | 02 Nos. |
| 311 |  | Control Valve |  | 02 Nos. |
| 312 |  | Centrifugal pump |  | 02 Nos. |
| 313 |  | Sump tank |  | 02 Nos. |
| 314 |  | Vertical pipe |  | 02 Nos. |
| 315 |  | Channel |  | 02 Nos. |
| 316 |  | Drain Valve |  | 02 Nos. |

# 5.	Members of the Curriculum Development Committee

| Sr. # | Name | Designation | Organization |
| --- | --- | --- | --- |
|  | Engr. Shazia Feroz | Assistant Professor | Benazir Bhutto Shaheed University of Technology |
|  | Engr. Liaqat Ali Jamro | Director Academics | STEVTA |
|  | Mr. Baber Sayeed | Chief Instructor. | Pak Swiss Training Center, Karachi |
|  | Mr. Tariq Aziz | CEO | Megafactor Engineering, Karachi |
|  | Engr. Syed Farhan Hamid Ali | Sr. Instructor. | Pak Swiss Training Center, Karachi |
|  | Engr. Safwan | Automation Engineer | Dewan Cement |
|  | Engr. Altaf Hussain Solangi | Sr. Instructor. | Amantech |
|  | Engr. Hassan | Instrumentation Engineer | Dewan Cement |
|  | Engr. Umair | Automation Engineer | Port Qasim Power Plant |
|  | Engr. Aijaz Ahmed Zia | Automation Engineer | Intech Process Automation |
|  | Engr. Muhammad Yasir | Deputy Director SS&C | NAVTTC |