|  | Assessment Evidence Guide For National Vocational Certificate in Marble Sector “Marble Technician (Block cutting)” Formative Assessment “Perform Block Cutting Operations” |
| --- | --- |
|  | Date: 19-24 August, 2024 GATTC, Peshawar |
|  | National Vocational & Technical Training Commission |

Instruction Sheet for the Candidate

| Qualification | National Vocational Certificate in Marble Sector “Marble Technician (Block cutting)” |
| --- | --- |
| Competency Standard | 8. Perform Block cutting Operations |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name_____________________________________________ Registration/Roll Number_______________________________________ |
| Guidance for Candidate | To meet this standard, you are required to complete the following within 04 Hrs.time frame (for practical demonstration & assessment): - Prepare the Gangsaw for Operation - Load and Secure Marble Blocks on Gangsaw - Operate the Gangsaw - Conduct Post-Cutting Operations on Gangsaw - Conduct Post-Cutting Operations on Gangsaw - Load and Secure Marble Blocks - Operate the Bridge Cutter, Vertical Saw, and Cross Cutter Operations - Conduct Post-Cutting Operations |
| Time: 4 Hrs. | During a practical assessment, under observation by an assessor, you are required to |
| Minimum Evidence Required | 1. Prepare the Gangsaw for Operation - Identify different block-to-slabs cutting machine - Change blade size according to block size. - Calibrate the gangsaw - Check the alignment and tension of the belts and blades. - Lubricate all moving components. - Conduct a pre-operation inspection to identify issues (if any). 2. Load and Secure Marble Blocks on Gangsaw - Load marble blocks onto the carriage with lifting equipment. - Align and secure the marble block position. - Use wedges or clamps to prevent movement during cutting. - Double-check the stability and alignment of the blocks. 3. Operate the Gangsaw - Start cutting the block according to the requirement. - Observe the operation, adjust feed rate and water flow as per requirement. - Adjust the cutting process as per block hardness for efficiency - Keep the detailed records of machine operation and maintenance. - Maintain detailed records of production 4. Conduct Post-Cutting Operations on Gangsaw - Remove marble slabs from the carriage safely. - Sorting the cut slabs as per quality and grading. - Perform finishing touches, such as smoothing edges when required. - Clean the gangsaw and surrounding area. - Dispose off waste materials according to environmental regulations and company policies. - Conduct a post-operation review to identify areas which needs attention for further operation 5. Prepare the Bridge Cutter, Vertical Saw, and Cross Cutter for Operations - Calibrate the bridge cutter, vertical saw, and cross cutter - Check the alignment and tension of the belts and blades. - Lubricate all moving components. - Conduct a pre-operation inspection to identify issues. - Adjust positions to ensure precise cuts. 6. Load and Secure Marble Blocks - Load marble blocks onto the carriage with lifting equipment. - Align and secure the marble block position. - Use wedges or clamps to prevent movement during cutting. - Double-check the stability and alignment of the blocks 7. Operate the Bridge Cutter, Vertical Saw, and Cross Cutter Operations - Start cutting according to the instructions and safety protocols. - Observe the operation, adjust feed rate and water flow as per requirement. - Adjust the cutting process for efficiency and change blade size according to block size. - Keep detailed records of machine operation and maintenance. - Maintain detailed records of production. 8. Conduct Post-Cutting Operations - Remove marble slabs from the carriage safely. - Inspect the cut slabs for quality and defects. - Perform finishing touches, such as smoothing edges when required. - Clean the bridge cutter, vertical saw, and cross cutter and surrounding area. - Dispose off waste materials according to environmental regulations and company policies. - Conduct a post-operation review to identify areas for process improvement. |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | National Vocational Certificate in Marble Sector “Marble Technician (Block cutting)” |
| Competency Standard | 8. Perform Block cutting Operations |
| Purpose of Assessment | Formative Assessment |
| Assessment Task | - Prepare the Gangsaw for Operation - Load and Secure Marble Blocks on Gangsaw - Operate the Gangsaw - Conduct Post-Cutting Operations on Gangsaw - Conduct Post-Cutting Operations on Gangsaw - Load and Secure Marble Blocks - Operate the Bridge Cutter, Vertical Saw, and Cross Cutter Operations - Conduct Post-Cutting Operations |

I can……………….

|  | Performance Criteria | Yes | No |
| --- | --- | --- | --- |
|  | Identify different block-to-slabs cutting machine |  |  |
|  | Change blade size according to block size. |  |  |
|  | Calibrate the gangsaw |  |  |
|  | Check the alignment and tension of the belts and blades. |  |  |
|  | Lubricate all moving components. |  |  |
|  | Conduct a pre-operation inspection to identify issues (if any). |  |  |
|  | Load marble blocks onto the carriage with lifting equipment. |  |  |
|  | Align and secure the marble block position. |  |  |
|  | Use wedges or clamps to prevent movement during cutting. |  |  |
|  | Double-check the stability and alignment of the blocks. |  |  |
|  | Start cutting the block according to the requirement. |  |  |
|  | Observe the operation, adjust feed rate and water flow as per requirement. |  |  |
|  | Adjust the cutting process as per block hardness for efficiency |  |  |
|  | Keep the detailed records of machine operation and maintenance. |  |  |
|  | Maintain detailed records of production |  |  |
|  | Remove marble slabs from the carriage safely. |  |  |
|  | Sorting the cut slabs as per quality and grading. |  |  |
|  | Perform finishing touches, such as smoothing edges when required. |  |  |
|  | Clean the gangsaw and surrounding area. |  |  |
|  | Dispose off waste materials according to environmental regulations and company policies. |  |  |
|  | Conduct a post-operation review to identify areas which needs attention for further operation |  |  |
|  | Calibrate the bridge cutter, vertical saw, and cross cutter |  |  |
|  | Check the alignment and tension of the belts and blades. |  |  |
|  | Lubricate all moving components. |  |  |
|  | Conduct a pre-operation inspection to identify issues. |  |  |
|  | Adjust positions to ensure precise cuts. |  |  |
|  | Load marble blocks onto the carriage with lifting equipment. |  |  |
|  | Align and secure the marble block position. |  |  |
|  | Use wedges or clamps to prevent movement during cutting. Double-check the stability and alignment of the blocks |  |  |
|  | Start cutting according to the instructions and safety protocols. |  |  |
|  | Observe the operation, adjust feed rate and water flow as per requirement. |  |  |
|  | Adjust the cutting process for efficiency and change blade size according to block size. |  |  |
|  | Keep detailed records of machine operation and maintenance. |  |  |
|  | Maintain detailed records of production |  |  |
|  | Remove marble slabs from the carriage safely. |  |  |
|  | Inspect the cut slabs for quality and defects. |  |  |
|  | Perform finishing touches, such as smoothing edges when required. |  |  |
|  | Clean the bridge cutter, vertical saw, and cross cutter and surrounding area. |  |  |
|  | Dispose off waste materials according to environmental regulations and company policies. |  |  |
|  | Conduct a post-operation review to identify areas for process improvement. |  |  |

Candidate’s Signature__________________ Assessor’s Signature____________________

Date: _______________________________

Assessors Judgment Guide

| Qualification | National Vocational Certificate in Marble Sector “Marble Technician (Block cutting)” |
| --- | --- |
| Competency Standard | 8. Perform Block cutting Operations |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: ______________________________________________________________ Registration/Roll Number: ______________________Signature: _______________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________________________________ Assessor’s code: ____________________________________________________ Signature: ________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | - Prepare the Gangsaw for Operation - Load and Secure Marble Blocks on Gangsaw - Operate the Gangsaw - Conduct Post-Cutting Operations on Gangsaw - Conduct Post-Cutting Operations on Gangsaw - Load and Secure Marble Blocks - Operate the Bridge Cutter, Vertical Saw, and Cross Cutter Operations - Conduct Post-Cutting Operations |  |  |  |
| --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  | Yes | No | Remarks |
|  | Identified different block-to-slabs cutting machine |  |  |  |  |
|  | Change blade size according to block size. |  |  |  |  |
|  | Calibrated the gangsaw |  |  |  |  |
|  | Checked the alignment and tension of the belts and blades. |  |  |  |  |
|  | Lubricated all moving components. |  |  |  |  |
|  | Conducted a pre-operation inspection to identify issues (if any). |  |  |  |  |
|  | Loaded marble blocks onto the carriage with lifting equipment. |  |  |  |  |
|  | Align and secure the marble block position. |  |  |  |  |
|  | Used wedges or clamps to prevent movement during cutting. |  |  |  |  |
|  | Double-checked the stability and alignment of the blocks. |  |  |  |  |
|  | Started cutting the block according to the requirement. |  |  |  |  |
|  | Observed the operation, adjust feed rate and water flow as per requirement. |  |  |  |  |
|  | Adjusted the cutting process as per block hardness for efficiency |  |  |  |  |
|  | Kept the detailed records of machine operation and maintenance. |  |  |  |  |
|  | Maintain detailed records of production |  |  |  |  |
|  | Removed marble slabs from the carriage safely. |  |  |  |  |
|  | Sorted the cut slabs as per quality and grading. |  |  |  |  |
|  | Performed finishing touches, such as smoothing edges when required. |  |  |  |  |
|  | Cleaned the gangsaw and surrounding area. |  |  |  |  |
|  | Disposed off waste materials according to environmental regulations and company policies. |  |  |  |  |
|  | Conducted a post-operation review to identify areas which needs attention for further operation |  |  |  |  |
|  | Calibrated the bridge cutter, vertical saw, and cross cutter |  |  |  |  |
|  | Checked the alignment and tension of the belts and blades. |  |  |  |  |
|  | Lubricated all moving components. |  |  |  |  |
|  | Conducted a pre-operation inspection to identify issues. |  |  |  |  |
|  | Adjusted positions to ensure precise cuts. |  |  |  |  |
|  | Loaded marble blocks onto the carriage with lifting equipment. |  |  |  |  |
|  | Aligned and secure the marble block position. |  |  |  |  |
|  | Used wedges or clamps to prevent movement during cutting. Double-check the stability and alignment of the blocks |  |  |  |  |
|  | Started cutting according to the instructions and safety protocols. |  |  |  |  |
|  | Observed the operation, adjust feed rate and water flow as per requirement. |  |  |  |  |
|  | Adjusted the cutting process for efficiency and change blade size according to block size. |  |  |  |  |
|  | Kept detailed records of machine operation and maintenance. |  |  |  |  |
|  | Maintained detailed records of production |  |  |  |  |
|  | Removed marble slabs from the carriage safely. |  |  |  |  |
|  | Inspected the cut slabs for quality and defects. |  |  |  |  |
|  | Performed finishing touches, such as smoothing edges when required. |  |  |  |  |
|  | Cleaned the bridge cutter, vertical saw, and cross cutter and surrounding area. |  |  |  |  |
|  | Disposed off waste materials according to environmental regulations and company policies. |  |  |  |  |
|  | Conducted a post-operation review to identify areas for process improvement. |  |  |  |  |

Knowledge Assessment

| Qualification | National Vocational Certificate in Marble Sector “Marble Technician (Block cutting)” |
| --- | --- |
| Competency Standard | 8. Perform Block cutting Operations |
| Purpose of Assessment | Formative Assessment |
| Candidate Details | Name: _____________________________________________________ Registration/Roll Number: ___________ Candidate Signature: ___________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor: ___________________________________ Assessor’s code: _______________________________________ Signature of the Assessor: _______________________ |

| Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application. |  |  |  |
| --- | --- | --- | --- |
| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  |  | Satisfactory | Not Satisfactory |  |
| 1 |  | Name any two block and slab marble cutting machines? |  |  |  |
| 2 |  | What is the calibration of a machine? |  |  |  |
| 3 |  | Name any four(4) common lubricant material? |  |  |  |
| 4 |  | What are the basic functions of water flow during marble cutting? |  |  |  |
| 5 |  | How to increase efficiency of marble cutting blade? |  |  |  |
| 6 |  | Safety precautions/measurements for block cutting are? |  |  |  |

Feedback to the Candidate

Candidate’s Signature_______________ Assessor’s Signature _________________

|  | Correct Answers |
| --- | --- |
| 1 | Block cutting machines are Gangsaw, Wiresaw and block cutter Slab cutting machines are table cutter, resize machine, and edge cutter |
| 2 | Calibration of a machine involves adjusting it to ensure it operates accurately and consistently according to specified standards |
| 3 | Lubricant materials are mineral oil, synthetic oil, grease and silicon lubricants. |
| 4 | Water flow plays several critical roles during marble cutting like cooling, cleaning, dust suppression, lubrication and improving cutting accuracy. |
| 5 | Regular Maintenance and Inspection, Use Proper Cutting Techniques, Maintain Proper Cooling and Use High-Quality Blades |
| 6 | 1. Personal Protective Equipment (PPE) 2. Machine Safety 3. Pre-Operation Checks 4. Safe Handling and Storage 5. Inspection and Reporting |