Instruction Sheet for the Candidate

| Qualification | Electrical Workshop |
| --- | --- |
| Competency Standard(s) | ### Overhaul the Lead Acid Batteries & implement their series & Parallel circuits |
| Purpose of Assessment | Formative Assessment |

| Candidate Details | Name________________________________________________________ Registration/Roll Number_____________________________________________ |
| --- | --- |
| Guidance for Candidate | To meet this standard, you are required to complete the following within 02 Hrs. time frame (for practical demonstration & assessment): - Maintain Lead Acid Batteries - Implement series & parallel circuit for battery |
| Time: 02 Hrs. | During a practical assessment, under observation by an assessor, you are required to “ Maintain Lead Acid Batteries “demonstrating the following criteria: - Select the proper container & Pour H2SO4 ,water in the container as per specification.. - Mix the mixture with wooden/glass rod so that two solutions mix completely. - Open the vent plug of battery and Fill the hydrometer with electrolyte - Keep the hydrometer vertical and take reading form hydrometer - Turn the switch off and take the reading of voltmeter and note its value as “E”. - Turn on the switch and again take the reading of voltmeter and note its value as “V”. - Note the reading of ammeter as well. - Use above measured values and find out internal resistance using appropriate formula. - Set proper voltage and current setting of charger. - Connect the battery , turn on the supply and note the current reading of ammeter from charger. - Remain connect the battery with charger till the ammeter show minimum or zero reading. - Disconnect the battery and check its specific gravity with hydrometer. During a practical assessment, under observation by an assessor, you are required to “Implement series & parallel circuit for battery “demonstrating the following criteria: - Select the specific quantity cells & Connect them in series , parallel with proper polarity. - Measure the voltage of series , parallel combined cell. - Increase the number of cells,note the voltage & Interpret the effect. |
| Minimum Evidence Required |  |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | Electrical Workshop |
| Competency Standards | Overhaul the Lead Acid Batteries & implement their series & Parallel circuits |
| Assessment Task | Maintain Lead Acid Batteries |
|  | Implement series & parallel circuit for battery |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Select the proper container & Pour H2SO4 ,water in the container as per specification.. |  |  |
| - Mix the mixture with wooden/glass rod so that two solutions mix completely. |  |  |
| - Open the vent plug of battery and Fill the hydrometer with electrolyte - Keep the hydrometer vertical and take reading form hydrometer |  |  |
| - Turn the switch off and take the reading of voltmeter and note its value as “E”. |  |  |
| - Turn on the switch and again take the reading of voltmeter and note its value as “V”. |  |  |
| - Note the reading of ammeter as well. |  |  |
| - Use above measured values and find out internal resistance using appropriate formula. |  |  |
| - Set proper voltage and current setting of charger. |  |  |
| - Connect the battery , turn on the supply and note the current reading of ammeter from charger. |  |  |
| - Remain connect the battery with charger till the ammeter show minimum or zero reading. |  |  |
| - Disconnect the battery and check its specific gravity with hydrometer |  |  |
| - Select the specific quantity cells & Connect them in series, parallel with proper polarity. |  |  |
| - Measure the voltage of series, parallel combined cell. |  |  |
| - Increase the number of cells, note the voltage & Interpret the effect |  |  |

Candidate’s Signature___________________		Assessor’s Signature_____________________

Date: ________________________________

Assessors Judgment Guide

| Qualification | Electrical Workshop |
| --- | --- |
| Competency Standard(s) | Overhaul the Lead Acid Batteries & implement their series & Parallel circuits |
| Candidate Details | Name:______________________________________________________________________ Registration/Roll Number: ________________________ Signature: ____________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________ Assessor’s code:___________________ Signature:__________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | Maintain Lead Acid Batteries Implement series & parallel circuit for battery |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Select the proper container & Pour H2SO4, water in the container as per specification. |  |  |  |  |  |
|  | Mix the mixture with wooden/glass rod so that two solutions mix completely. |  |  |  |  |  |
|  | Open the vent plug of battery and Fill the hydrometer with electrolyte |  |  |  |  |  |
|  | Keep the hydrometer vertical and take reading form hydrometer |  |  |  |  |  |
|  | Turn the switch off and take the reading of voltmeter and note its value as “E”. |  |  |  |  |  |
|  | Turn on the switch and again take the reading of voltmeter and note its value as “V”. |  |  |  |  |  |
|  | Note the reading of ammeter as well. |  |  |  |  |  |
|  | Use above measured values and find out internal resistance using appropriate formula. |  |  |  |  |  |
|  | Set proper voltage and current setting of charger. |  |  |  |  |  |
|  | Connect the battery , turn on the supply and note the current reading of ammeter from charger. |  |  |  |  |  |
|  | Remain connect the battery with charger till the ammeter show minimum or zero reading. |  |  |  |  |  |
|  | Disconnect the battery and check its specific gravity with hydrometer |  |  |  |  |  |
|  | Select the specific quantity cells & Connect them in series, parallel with proper polarity. |  |  |  |  |  |
|  | Measure the voltage of series, parallel combined cell. |  |  |  |  |  |
|  | Increase the number of cells, note the voltage & Interpret the effect |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | Electrical Workshop |
| --- | --- |
| Competency Standard(s) | Overhaul the Lead Acid Batteries & implement their series & Parallel circuits |
| Candidate Details | Name: _______________________________________________________________________ Registration/Roll Number: __________________ Candidate Signature:__________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor: __________________________ Assessor’s code: _________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | What is difference between primary and secondary cell? |  |  |
|  | What is the difference between cell and battery |  |  |
|  | Why batteries are used in series? |  |  |
|  | Why batteries are used in parallel? |  |  |
|  | What is specific gravity? |  |  |
|  | What is hydrometer? |  |  |

Feedback to the Candidate

Candidate’s Signature________________________ Assessor’s Signature _________________________