Instruction Sheet for the Candidate

| Qualification | Electrical Workshop |
| --- | --- |
| Competency Standard(s) | ### Implement Electromagnet to observe various effects & verify Faraday’s law |
| Purpose of Assessment | Formative Assessment |

| Candidate Details | Name________________________________________________________ Registration/Roll Number_____________________________________________ |
| --- | --- |
| Guidance for Candidate | To meet this standard, you are required to complete the following within 02 Hrs. time frame (for practical demonstration & assessment): Construct & Implement Electromagnet to observe (a) effects of current carrying conductor in magnetic field (b)effect of varying the current with the help of rheostat (c) Plot magnetic lines of forces of bar magnet verify Faraday’s law & EMF through induction |
| Time: 02 Hrs. | During a practical assessment, under observation by an assessor, you are required to “ Construct & Implement Electromagnet to observe various effects “ demonstrating the following criteria: - Take iron nail (approximately 3 inches in length) as iron core and make 30 to 40 turns of thin coated copper wire to form a coil. - Connect dry cell battery with coil,bring iron nail near the iron pieces and record the observation Effects of current carrying conductor in magnetic field - Take copper rod (5cm in length) and connect wires across it. - Give DC supply to copper rod through rheostat. - Place current carrying copper rod inside the horseshoe magnet. Effect of varying the current with the help of rheostat - Reduce the rheostat resistance & record the effect on copper rod. - Increase the rheostat resistance & record the effect on copper rod. Plot magnetic lines of forces of bar magnet - Place a bar magnet on paper and outline its boundary with the help of lead pencil. - Place a compass needle at one side the magnet, mark points on paper where the compass needle stop & Repeat the same procedure till compass reach at the other end of magnet. - Change the position of compass needle near the magnetic pole and repeat the procedure for P3 to P4 During a practical assessment, under observation by an assessor, you are required to “ Verify Faraday’s Law & EMF through induction “demonstrating the following criteria: - Connect Galvano meter with coil,Move permanent magnet inside the coil, vary its speed and record the effect on reading of Galvanometer. - Hold the magnet inside the coil and do not move, now record the effect on reading of Galvano meter. - Fix permanent magnet and move the coil fast and slow on it and record the effect on reading of Galvano meter. - Hold the coil near the magnetic field do not move, now record the effect on reading of Galvano meter. - Take step down transformer and connect its secondary with a Galvano meter and primary winding with a DC battery through a rheostat. - Continuously variate the rheostat and observe the reading on the Galvano meter |
| Minimum Evidence Required |  |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | Electrical Workshop |
| Competency Standards | Implement Electromagnet to observe various effects & verify Faraday’s law |
| Assessment Task | Construct & Implement Electromagnet to observe (a) effects of current carrying conductor in magnetic field (b)effect of varying the current with the help of rheostat (c) Plot magnetic lines of forces of bar magnet |
|  | verify Faraday’s law & EMF through induction |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Take iron nail (approximately 3 inches in length) as iron core and make 30 to 40 turns of thin coated copper wire to form a coil. |  |  |
| - Connect dry cell battery with coil,bring iron nail near the iron pieces and record the observation |  |  |
| - Take copper rod (5cm in length) and connect wires across it. |  |  |
| - Give DC supply to copper rod through rheostat. |  |  |
| - Place current carrying copper rod inside the horseshoe magnet |  |  |
| - Reduce the rheostat resistance & record the effect on copper rod. |  |  |
| - Increase the rheostat resistance & record the effect on copper rod. |  |  |
| - Place a bar magnet on paper and outline its boundary with the help of lead pencil. |  |  |
| - Place a compass needle at one side the magnet, mark points on paper where the compass needle stop & Repeat the same procedure till compass reach at the other end of magnet. |  |  |
| - Change the position of compass needle near the magnetic pole and repeat the procedure for P3 to P4 |  |  |
| - Connect Galvano meter with coil,Move permanent magnet inside the coil, vary its speed and record the effect on reading of Galvanometer. Hold the magnet inside the coil and do not move, now record the effect on reading of Galvano meter. |  |  |
| - Fix permanent magnet and move the coil fast and slow on it and record the effect on reading of Galvano meter. |  |  |
| - Hold the coil near the magnetic field do not move, now record the effect on reading of Galvano meter. |  |  |
| - Take step down transformer and connect its secondary with a Galvano meter and primary winding with a DC battery through a rheostat. |  |  |
| - Continuously variate the rheostat and observe the reading on the Galvano meter |  |  |

Candidate’s Signature___________________		Assessor’s Signature_____________________

Date: ________________________________

Assessors Judgment Guide

| Qualification | Electrical Workshop |
| --- | --- |
| Competency Standard(s) | Implement Electromagnet to observe various effects & verify Faraday’s law |
| Candidate Details | Name:______________________________________________________________________ Registration/Roll Number: ________________________ Signature: ____________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________ Assessor’s code:___________________ Signature:__________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | Construct & Implement Electromagnet to observe (a) effects of current carrying conductor in magnetic field (b)effect of varying the current with the help of rheostat (c) Plot magnetic lines of forces of bar magnet verify Faraday’s law & EMF through induction |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Take iron nail (approximately 3 inches in length) as iron core and make 30 to 40 turns of thin coated copper wire to form a coil. |  |  |  |  |  |
|  | Connect dry cell battery with coil,bring iron nail near the iron pieces and record the observation |  |  |  |  |  |
|  | Take copper rod (5cm in length) and connect wires across it. |  |  |  |  |  |
|  | Give DC supply to copper rod through rheostat. |  |  |  |  |  |
|  | Place current carrying copper rod inside the horseshoe magnet |  |  |  |  |  |
|  | Reduce the rheostat resistance & record the effect on copper rod. |  |  |  |  |  |
|  | Increase the rheostat resistance & record the effect on copper rod. |  |  |  |  |  |
|  | Place a bar magnet on paper and outline its boundary with the help of lead pencil. |  |  |  |  |  |
|  | Place a compass needle at one side the magnet, mark points on paper where the compass needle stop & Repeat the same procedure till compass reach at the other end of magnet. |  |  |  |  |  |
|  | Change the position of compass needle near the magnetic pole and repeat the procedure for P3 to P4 |  |  |  |  |  |
|  | Connect Galvano meter with coil,Move permanent magnet inside the coil, vary its speed and record the effect on reading of Galvanometer. Hold the magnet inside the coil and do not move, now record the effect on reading of Galvano meter. |  |  |  |  |  |
|  | Fix permanent magnet and move the coil fast and slow on it and record the effect on reading of Galvano meter. |  |  |  |  |  |
|  | Hold the coil near the magnetic field do not move, now record the effect on reading of Galvano meter. |  |  |  |  |  |
|  | Take step down transformer and connect its secondary with a Galvano meter and primary winding with a DC battery through a rheostat. |  |  |  |  |  |
|  | Continuously variate the rheostat and observe the reading on the Galvano meter |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | Electrical Workshop |
| --- | --- |
| Competency Standard(s) | Implement Electromagnet to observe various effects & verify Faraday’s law |
| Candidate Details | Name: _______________________________________________________________________ Registration/Roll Number: __________________ Candidate Signature:__________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor: __________________________ Assessor’s code: _________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Define Magnet? |  |  |
|  | Different types of magnet? |  |  |
|  | Define Faraday’s law? |  |  |
|  | Define induced e.m.f? |  |  |
|  | What is induction? |  |  |
|  | Define Turn Ratio? |  |  |

Feedback to the Candidate

Candidate’s Signature________________________ Assessor’s Signature _________________________