Instruction Sheet for the Candidate

| Qualification | Electrical Workshop |
| --- | --- |
| Competency Standard(s) | ### Make & Solder the Joints |
| Purpose of Assessment | Formative Assessment |

| Candidate Details | Name________________________________________________________ Registration/Roll Number_____________________________________________ |
| --- | --- |
| Guidance for Candidate | To meet this standard, you are required to complete the following within 02 Hrs. time frame (for practical demonstration & assessment): Make different types of joints - Cross/Twist joint - Straight/Married joint - T- Joint - Rat tail joint - Britannia joint |
| Time: 02 Hrs. | During a practical assessment, under observation by an assessor, you are required to “Make different types of joints “ demonstrating the following criteria: - Select the cable - Strip the wire according to 50mm. - Twist, Solder the conductor & Insulate the joint - Strip wire to 75mm according to joint requirement. - Intermingle the conductors to 60mm into each other. - Twist 60mm conductors leaving behind 15mm of each cable. - Solder the conductor & Insulate the joint - Remove the insulation of cable 1 to 50mm from where a connection is required. - Separate conductors of cable 1 equally. - Take another 12 mm stripped wire 2. - Insert between two equally half conductors of cable 1 and twist. - Half conductors clockwise ,half anti-clock wise of cable 2 & Solder the joint. - Select the single conductor wires. - Strip both the wires to 5mm. - Twist the conductor, Solder & Insulate the joint. - Strip both cables to 75mm & Bend the tips of both cable right angle about to 6mm. - Hold the two cables overlap 50mm with tips in opposite direction. - Take another bare conductor of 1mm , wrap around in both directions to 6mm ,Solder & Insulate the joint. |
| Minimum Evidence Required |  |

Self-Assessment Checklist

| Candidate Name |  |
| --- | --- |
| Registration No. |  |
| Qualification | Electrical Workshop |
| Competency Standards | Make & Solder the Joints |
| Assessment Task | Make different types of joints - Cross/Twist joint - Straight/Married joint - T- Joint - Rat tail joint - Britannia joint |

I can……………….

| Performance Criteria | Yes | No |
| --- | --- | --- |
| - Select the cable |  |  |
| - Strip the wire according to 50mm. |  |  |
| - Twist, Solder the conductor & Insulate the joint |  |  |
| - Strip wire to 75mm according to joint requirement. |  |  |
| - Intermingle the conductors to 60mm into each other. |  |  |
| - Twist 60mm conductors leaving behind 15mm of each cable. |  |  |
| - Solder the conductor & Insulate the joint |  |  |
| - Remove the insulation of cable 1 to 50mm from where a connection is required. |  |  |
| - Separate conductors of cable 1 equally. |  |  |
| - Take another 12 mm stripped wire 2. |  |  |
| - Insert between two equally half conductors of cable 1 and twist. |  |  |
| - Half conductors clockwise ,half anti-clock wise of cable 2 & Solder the joint. |  |  |
| - Select the single conductor wires. |  |  |
| - Strip both the wires to 5mm. |  |  |
| - Twist the conductor, Solder & Insulate the joint. |  |  |
| - Strip both cables to 75mm & Bend the tips of both cable right angle about to 6mm. |  |  |
| - Hold the two cables overlap 50mm with tips in opposite direction. |  |  |
| - Take another bare conductor of 1mm , wrap around in both directions to 6mm ,Solder & Insulate the joint |  |  |

Candidate’s Signature___________________		Assessor’s Signature_____________________

Date: ________________________________

Assessors Judgment Guide

| Qualification | Electrical Workshop |
| --- | --- |
| Competency Standard(s) | Make & Solder the Joints |
| Candidate Details | Name:______________________________________________________________________ Registration/Roll Number: ________________________ Signature: ____________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor________________________ Assessor’s code:___________________ Signature:__________________________________ |

| Assessment Summary (to be filled by the assessor) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Activity | Method |  |  |  |  | Result |  |
| Nature of Activity | Written | Oral | Observation | Portfolio | Role Play | Competent | Not Yet Competent |
| Practical Skill Demonstration |  |  |  |  |  |  |  |
| Knowledge Assessment |  |  |  |  |  |  |  |
| Other Requirement |  |  |  |  |  |  |  |

Observation Checklist

| Assessment Task |  | Make different types of joints - Cross/Twist joint - Straight/Married joint - T- Joint - Rat tail joint - Britannia joint |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| During the practical assessment, candidate demonstrated the following: |  |  |  | Yes | No | Remarks |
|  | Select the cable |  |  |  |  |  |
|  | Strip the wire according to 50mm. |  |  |  |  |  |
|  | Twist, Solder the conductor & Insulate the joint |  |  |  |  |  |
|  | Strip wire to 75mm according to joint requirement. |  |  |  |  |  |
|  | Intermingle the conductors to 60mm into each other. |  |  |  |  |  |
|  | Twist 60mm conductors leaving behind 15mm of each cable. |  |  |  |  |  |
|  | Solder the conductor & Insulate the joint |  |  |  |  |  |
|  | Remove the insulation of cable 1 to 50mm from where a connection is required. |  |  |  |  |  |
|  | Separate conductors of cable 1 equally. |  |  |  |  |  |
|  | Take another 12 mm stripped wire 2. |  |  |  |  |  |
|  | Insert between two equally half conductors of cable 1 and twist. |  |  |  |  |  |
|  | Half conductors clockwise ,half anti-clock wise of cable 2 & Solder the joint. |  |  |  |  |  |
|  | Select the single conductor wires. |  |  |  |  |  |
|  | Strip both the wires to 5mm. |  |  |  |  |  |
|  | Twist the conductor, Solder & Insulate the joint. |  |  |  |  |  |
|  | Strip both cables to 75mm & Bend the tips of both cable right angle about to 6mm. |  |  |  |  |  |
|  | Hold the two cables overlap 50mm with tips in opposite direction. |  |  |  |  |  |
|  | Take another bare conductor of 1mm , wrap around in both directions to 6mm ,Solder & Insulate the joint |  |  |  |  |  |
| Competent |  |  | Not Yet Competent |  |  |  |

Knowledge Assessment

| Qualification | Electrical Workshop |
| --- | --- |
| Competency Standard(s) | Make & Solder the Joints |
| Candidate Details | Name: _______________________________________________________________________ Registration/Roll Number: __________________ Candidate Signature:__________________ |
| Assessment Outcome | COMPETENT NOT YET COMPETENT Name of the Assessor: __________________________ Assessor’s code: _________________ Signature of the Assessor:_______________________ |

Candidate’s response is not required to be identical, but similar concepts and/or keywords must be used. Oral questioning may be used to clarify candidate understanding of topic and its application.

| Questions (Candidate confidently answered questions correctly and demonstrated understanding of the topics and their application) |  | Satisfactory | Not Satisfactory |
| --- | --- | --- | --- |
|  | Why Insulation is required at the jointed point? |  |  |
|  | When Cross/Twist joint are used? |  |  |
|  | When Straight/Married joint are used? |  |  |
|  | When T- Joint are used? |  |  |
|  | When Rat tail joint are used? |  |  |
|  | When the Britannia joint are used? |  |  |

Feedback to the Candidate

Candidate’s Signature________________________ Assessor’s Signature _________________________